Source: 	MCC Support

Title:	Final Report of 3GPP TSG RAN WG1 #92bis v1.0.0

(Sanya, China, 16th – 20th April 2018)

Document for:	Approval

Table of contents

1	Opening of the meeting	7

1.1	Call for IPR	7

1.2	Competition law statement	7

1.3	Network usage conditions	7

1.4	Check-in for Registered Delegates	8

1.5	Aspects related to RAN1 Meeting Management	8

2	Approval of Agenda	8

3	Highlights from RAN plenary	8

4	Approval of Minutes from previous meeting	8

5	Incoming Liaison Statements	8

6	E-UTRA	12

6.1	Maintenance of E-UTRA Releases 8 – 14	12

6.1.1	Maintenance of E-UTRA Release 8 – 13	12

6.1.2	Maintenance of Release 14 Enhanced Licensed-Assisted Access to Unlicensed Spectrum	12

6.1.3	Maintenance of Release 14 V2V/V2X services based on LTE sidelink	12

6.1.4	Maintenance of Release 14 Full-Dimension MIMO for LTE	12

6.1.5	Maintenance of Release 14 Further Enhanced MTC for LTE	13

6.1.6	Maintenance of Release 14 Enhancements of NB-IoT for LTE	13

6.1.7	Other	14

6.2	LTE Release 15	15

6.2.1	Maintenance of Release 15 Shortened TTI and processing time for LTE	15

6.2.1.1	Maintenance for shortened processing time for 1ms TTI	15

6.2.1.2	Maintenance for shortened TTI with shortened processing time	16

6.2.1.2.1	Aspects related to interaction between different TTI lengths	16

6.2.1.2.2	Aspects related to DL control channel	16

6.2.1.2.3	Aspects related to UL control channel	17

6.2.1.2.4	Aspects related to DL data channel	17

6.2.1.2.5	Aspects related to UL data channel	19

6.2.1.2.6	Aspects related to FS2	19

6.2.1.2.7	Other	19

6.2.1.3	Other	20

6.2.2	Maintenance of Release 15 Enhancements to LTE operation in unlicensed spectrum	20

6.2.2.1	TPs for Multiple starting and ending positions in a subframe for UL	21

6.2.2.2	TPs for Autonomous uplink access with Frame Structure type 3	21

6.2.2.3	Other	21

6.2.3	Maintenance of Release 15 Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE	21

6.2.3.1	Text Proposals	22

6.2.3.2	Other	22

6.2.4	Maintenance of Release 15 Further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE	22

6.2.4.1	Text Proposals	22

6.2.5	3GPP V2X Phase 2 - WID in RP-171740	22

6.2.5.1	Carrier Aggregation (up to 8 PC5 carriers)	22

6.2.5.1.1	Mode-4 support	22

6.2.5.1.2	Synchronization	23

6.2.5.1.3	Other	24

6.2.5.2	Support for 64-QAM	24

6.2.5.3	Feasibility and gain of PC5 operation with Transmit Diversity	24

6.2.5.3.1	Transmit diversity solutions	25

6.2.5.3.2	Evaluation results	25

6.2.5.3.3	Other	25

6.2.5.4	Resource pool sharing between mode-3 and mode-4 users	25

6.2.5.5	Maximum time reduction between packet arrival at layer 1 and resource selection for transmission	26

6.2.5.6	Other	26

6.2.6	Even further enhanced MTC for LTE - WID in RP-172811	26

6.2.6.1	Reduced system acquisition time	26

6.2.6.2	Early data transmission	27

6.2.6.3	Downlink channel power efficiency	28

6.2.6.4	Uplink HARQ-ACK feedback	29

6.2.6.5	PUSCH sub-PRB allocation	29

6.2.6.6	Flexible PDSCH/PUSCH starting PRB	31

6.2.6.7	Other	31

6.2.7	Further enhancements of NB-IoT - WID in RP-172063.	31

6.2.7.1	Latency and power consumption reduction	31

6.2.7.1.1	Power consumption reduction for paging and connected-mode DRX	31

6.2.7.1.2	Data transmission during the random access procedure	33

6.2.7.1.3	Physical layer scheduling request	34

6.2.7.1.4	Semi-persistent scheduling	35

6.2.7.2	Reduced system acquisition time	35

6.2.7.2.1	Cell search	35

6.2.7.2.2	System Information	35

6.2.7.3	Narrowband measurement accuracy improvements	35

6.2.7.4	NB-IoT small cell support	36

6.2.7.5	NPRACH reliability and range enhancements	36

6.2.7.5.1	Reliability enhancements for NPRACH	36

6.2.7.5.2	Range enhancements for NPRACH	36

6.2.7.6	TDD	37

6.2.7.6.1	Downlink aspects	37

6.2.7.6.2	Uplink aspects	38

6.2.7.6.3	Common aspects	39

6.2.7.7	Other	40

6.2.8	Ultra Reliable Low Latency Communication for LTE - WID in RP-172845	40

6.2.8.1	Blind/HARQ-less Repetition for Scheduled DL-SCH Operation	40

6.2.8.2	Repetition Enhancements for UL SPS Operation	41

6.2.8.3	Other	41

6.2.9	Enhanced LTE Support for Aerial Vehicles - WID in RP-172826	42

6.2.9.1	UL power control enhancements	42

6.2.9.2	Other	42

6.2.10	Enhancing CA Utilization - WID in RP-180561	42

6.2.11	Other	43

7	NR	43

7.1	Maintenance of NR - WID in RP-172834	43

7.1.1	Initial access and mobility	44

7.1.1.1	Synchronization signal	44

7.1.1.2	Broadcast signal/channel	46

7.1.1.2.1	NR-PBCH	46

7.1.1.2.2	Remaining minimum system information	47

7.1.1.2.3	Other system information delivery	49

7.1.1.3	Paging design	49

7.1.1.4	Physical random access channel and random access procedure	50

7.1.1.4.1	PRACH formats	50

7.1.1.4.2	RACH procedure	57

7.1.1.5	Mobility procedure	61

7.1.1.5.1	Measurement for mobility management	61

7.1.1.5.2	Radio link monitoring for mobility management	64

7.1.1.6	Other	65

7.1.2	MIMO	66

7.1.2.1	Multi-antenna scheme	66

7.1.2.1.1	Codeword mapping	66

7.1.2.1.2	Codebook based transmission for UL	67

7.1.2.1.3	Non-codebook based transmission for UL	68

7.1.2.1.4	PRB bundling for DL	69

7.1.2.1.5	Other	70

7.1.2.2	CSI acquisition and beam management	70

7.1.2.2.1	CSI measurement	70

7.1.2.2.2	CSI reporting	71

7.1.2.2.3	Beam measurement and reporting	74

7.1.2.2.4	Mechanism to recover from beam failure	75

7.1.2.2.5	CQI and MCS	76

7.1.2.2.6	Other	78

7.1.2.3	Reference signals and QCL	79

7.1.2.3.1	Multiplexing of different types of RSs	79

7.1.2.3.2	CSI-RS	80

7.1.2.3.3	DMRS	82

7.1.2.3.4	PT-RS	86

7.1.2.3.5	SRS	87

7.1.2.3.6	TRS	90

7.1.2.3.7	QCL	91

7.1.2.3.8	Other	92

7.1.2.4	Other	92

7.1.3	Scheduling/HARQ aspects	92

7.1.3.1	Physical downlink control channel	92

7.1.3.1.1	PDCCH structure	92

7.1.3.1.2	Search space	93

7.1.3.1.3	Group-common PDCCH	95

7.1.3.1.4	DCI contents and formats	97

7.1.3.1.5	Other	100

7.1.3.2	Physical uplink control channel	100

7.1.3.2.1	PUCCH structure in short-duration	100

7.1.3.2.2	PUCCH structure in long-duration	102

7.1.3.2.3	UCI multiplexing	103

7.1.3.2.4	Resource allocation for PUCCH	106

7.1.3.2.5	Other	109

7.1.3.3	DL/UL data scheduling and HARQ procedure	109

7.1.3.3.1	DL/UL resource allocation	109

7.1.3.3.2	DL/UL scheduling and HARQ management	112

7.1.3.3.3	CBG-based (re)transmission	115

7.1.3.3.4	UL data transmission procedure	115

7.1.3.3.5	Soft-buffer management for NR	116

7.1.3.3.6	Multiplexing data with different transmission durations	116

7.1.3.3.7	Other	117

7.1.3.4	Other aspects on carrier aggregation and bandwidth parts	117

7.1.3.4.1	Other aspects on bandwidth Parts	117

7.1.3.4.2	Other aspects on carrier aggregation	119

7.1.3.5	Rate matching aspects for NR DL and UL	120

7.1.3.6	Other	121

7.1.4	Channel coding	121

7.1.5	NR-LTE co-existence	122

7.1.6	UL power control	123

7.1.6.1	NR UL power control - non-CA aspects	123

7.1.6.2	NR UL power control – CA aspects	124

7.1.6.3	Other	124

7.1.7	UE features	125

7.1.8	Other	125

7.2	NR URLLC in Release 15	125

7.2.1	Support of separate CQI and MCS table(s) for URLLC	125

7.2.2	Study of necessity of a new DCI format	128

7.2.3	Study of necessity of PDCCH repetition	130

7.2.4	Study of handling UL multiplexing of transmissions with different reliability requirements	132

7.2.5	Other	134

7.3	Study on enhanced NR to support Non-Terrestrial Networks - SID in RP-171450	135

7.3.1	Channel Model	135

7.3.2	Other	149

7.4	Study on non-orthogonal multiple access for NR - SID in RP-171043	150

7.4.1	Transmitter side signal processing schemes for NOMA	150

7.4.2	Receivers for NOMA	151

7.4.3	Procedures related to NOMA	153

7.4.4	Link and system level performance evaluation for NOMA	154

7.4.5	Other	158

7.5	Study on evaluation methodology of new V2X use cases - SID in RP-170837	158

7.5.1	Evaluation scenarios	159

7.5.2	Channel Model	161

7.5.3	Other	165

7.6	Study on NR-based Access to Unlicensed Spectrum - SID in RP-172021	165

7.6.1	Simulation Methodology for NR-U operation	165

7.6.2	Frame structure for NR-U operation	167

7.6.3	Potential physical layer channel design	167

7.6.4	Potential physical layer procedures	168

7.6.5	Other	169

7.7	Study on Integrated Access and Backhaul for NR - SID in RP-172290	169

7.7.1	Enhancements to support NR backhaul links	169

7.7.2	Evaluation methodology for NR IAB	171

7.7.3	Other	174

7.8	Study on Self Evaluation towards IMT-2020 Submission	174

7.8.1	Evaluation methodology	174

7.8.2	Other	174

7.9	Other	175

8	Closing of the meeting	175

Annex A:	List of Tdocs at RAN1 #92bis	176

Annex B:	List of CRs agreed at RAN1 #92bis	177

Annex C-1:	List of Outgoing LSs from RAN1 #92bis	180

Annex C-2:	List of Incoming LSs from RAN1 #92bis	182

Annex D:	List of Approved updated WIDs	185

Annex E:	List of draft TSs/TRs agreed at RAN1 #92bis	186

Annex F:	List of actions	187

Annex G:	List of participants at RAN1 #92bis	194

Annex H:	TSG RAN WG1 meetings in 2018 – 2019	195

Main facts summary

The 3GPP TSG WG RAN1#92bis meeting, hosted by Chinese Friends of 3GPP, was held at the Westin Sanya Haitang Bay Resort, Sanya, China.

The meeting started at 9:00 on Monday 16th April and finished at 16:31 on Friday 20th April 2018.

The number of attending delegates, having confirmed their participation through the electronic check-in application, was 418.

Note that the number of registered was 554.

The schedule of the week was as follows:

The list of action points that required RAN1 close follow-up is listed in Annex F (end of document).

The number of contributions for this meeting was 2145.

Note: The amount of documents includes those discussed during the email discussion session post meeting.

|  | Contribution is agreed/approved |
| --- | --- |
|  | Contribution is not pursued (no consensus) |
|  | Further discussion is required – come-back needed |
|  | Working assumption |
|  | Specific action needed from MCC |
|  | Contribution is for email discussion/approval |

# Opening of the meeting

Mr Wanshi Chen (RAN1 Chairman) welcomed the participants of the RAN WG1 #92 bis meeting and opened the meeting at 09:00.

Mr Xueming Pan from vivo on behalf of the Chinese Friends of 3GPP, Apple Computer Trading Co., Ltd., 	BEIJING SAMSUNG TELECOM R&D CENTER, China Academy of Information and Communications Technology, China Academy of Telecommunication Technology, China Information Technology Design & Consulting Institute Co., Ltd., China Mobile Communications Corporation, China Telecommunications Corporation, China Unicom, Guangdong OPPO Mobile Telecommunications Corp., Ltd., Huawei Technologies Co.,Ltd., Intel China Ltd., Nanjing Ericsson Panda Com Ltd., Qualcomm Wireless Communication Technologies (China) Ltd., vivo Mobile Communication Co., Ltd., ZTE Corporation, welcomed the delegates, and detailed the domestic arrangements for the full week.

Before any other duties, RAN1 paid a tribute to Gabriel Lin who passed away on April 6th. Gabriel was MediaTek's 3GPP lead, and has been a great person to work with in the past years.

## Call for IPR

The attention of the members of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The members take note that they are hereby invited:

-	to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

-	to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms (e.g. see the ETSI IPR forms http://webapp.etsi.org/Ipr/).

## Competition law statement

The Chairman also drew Member’s attention to the fact that 3GPP activities are subject to antitrust and competition laws and that compliance with said laws is therefore required of any participant of this WG meeting including the Chairman and Vice Chairmen. In case of question, please contact your legal counsel.

The present meeting will be conducted with strict impartiality and in the interests of 3GPP.

Furthermore, the Chairman reminded Members that timely submission of work items/contributions in advance of WG meetings is important to allow for full and fair consideration of such matters.

## Network usage conditions

The PCG has laid down the following network usage conditions:

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network. Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting chairman should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself. 1. DON’T place your WiFi device in ad-hoc mode 2. DON’T set up a personal hotspot in the meeting room 3. DO try 802.11a if your WiFi device supports it 4. DON’T manually allocate an IP address 5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

## Check-in for Registered Delegates

The attention of the delegates to this meeting was drawn to the fact that it is not permitted to check in other delegates on their behalf. In the event of technical difficulties preventing check-in, delegates should present themselves in person to the Secretary.

## Aspects related to RAN1 Meeting Management

Delegates are encouraged to check R1-1718766 for some thoughts on RAN1 meeting management.

# Approval of Agenda

R1-1803570	Draft Agenda of RAN1#92bis meeting	RAN1 Chair

Wanshi Chen (RAN1 Chair) proposed the agenda for the meeting, as well the schedule of the week.

Discussion: No comments.

Decision: The agenda is approved.

# Highlights from RAN plenary

R1-1803572	Highlights from RAN#79	RAN1 Chair

The document was presented by Wanshi Chen (RAN1 Chair) and provides the highlights relevant to RAN1 from the last plenary.

Discussion: All LTE CRs were approved, except "Early contention resolution CRs – 36.213 CR 1044/1045", which were postponed due to incompletion in RAN2. MCC will re-submit the two CRs to RAN#80

Decision: The document is noted.

# Approval of Minutes from previous meeting

R1-1803571	Report of RAN1#92 meeting	ETSI

The document was presented by Patrick Merias from MCC (ETSI Mobile Competence Center) and provides the report from last RAN1#92 meeting in Athens.

Discussion: MCC recalled the decision to use .docx format for RAN1 specs – ahead of all other WGs – with the intention to solving the equation editor issue in Word.

CR0273 to 36.212 on deriving number of available symbols for PUSCH – implemented in Rel-14 but not properly reflected in Rel-15 - MCC will take the necessary corrective action to solve the issue – no need for a new CR.

Decision: The document is approved.

# Incoming Liaison Statements

LTE

R1-1803580	Reply LS on HARQ-ACK feedback for efeMTC	RAN2, Qualcomm

- RAN2 assumes that with “early termination of MPDCCH monitoring” in the below sentence, RAN1 refers to “early termination of MPDCCH monitoring and early termination of any ongoing PUSCH transmission”.

  - “For early termination of MPDCCH monitoring, RAN1 assumed that this may be used in the following cases:”

- For case 1; RAN2 thinks that uplink explicit HARQ-ACK for early termination of MPDCCH monitoring and early termination of any ongoing PUSCH transmission can be used to acknowledge the receipt of the RRCConnectionRelease message. RAN2 assumes no changes in RAN2 specification to capture this agreement.

- For case 2; RAN2 thinks that using an explicit HARQ-ACK feedback to acknowledge all pending UL HARQ processes is not efficient with respect to UE power consumption. RAN2 agreed that this use case is not supported.

Decision: The document is noted. Details to be handled 6.2.6

R1-1803573	Reply LS to RAN1 on wake-up signal	RAN2, Huawei

From RAN2 perspective, it is feasible to apply one wake-up signal to multiple POs in a PTW. The number of POs mapped to one WUS should be configurable by the eNB.

Decision: The document is noted. Details to be handled 6.2.6 & 6.2.7

R1-1803598	LS on NSSS based NRSRP measurement definition	RAN4, Nokia

Decision: The document is noted. Details to be handed under 6.2.7

R1-1803585	LS on RAN2 agreements for LTE URLLC	RAN2, Ericsson

Decision: The document is noted. Details to be handled 6.2.8

R1-1803589	LS on RAN2 agreements for euCA	RAN2, Nokia

RAN2 requests RAN1 to take the enclosed agreements into consideration when defining the CQI feedback for the Dormant Scell state.

Decision: The document is noted. Details to be handled under 6.2.10

NR

(Including EN-DC)

R1-1803584	LS on Frequency Band list	RAN2, Huawei

RAN2 agreed to add a list of frequency band indicators in both FrequencyInfoDL and FrequencyInfoUL IEs.  When the IEs are used in dedicated signalling the list will have only a single entry; when used in SIB1 it may have multiple entries and the UE applies the first band in the list that it supports.

Decision: The document is noted.

R1-1803603	Reply LS on NR SS Raster Shift for Frequency Range 0-2700MHz	RAN4, ZTE

RAN4 has discussed the NR SS raster shift for the frequency range of 0-2700MHz, and has made the following working agreements:

- Large shift is used for the SS raster in re-farming bands in the range of 70 - 100 kHz.

- No RMSI signaling is defined for the shift.

- Detailed decision on the exact shift is made at RAN4#86bis.

Note:  The large shift may impact on the GSCN values

Decision: The document is noted. Details to be handled under 7.1.1.1

R1-1803586	Response LS on Maximum TBS for PDSCH containing RMSI/OSI/Paging	RAN2, Huawei

RAN2 asks RAN1 to provide feedback on:

- Whether NR can support approximately 1700 bits RMSI in one TB in all cases.

- If necessary, whether L1 parameters size in RMSI can be reduced.

Decision: The document is noted.

Details to be handled under 7.1.1.2.2/7.1.1.2.3/7.1.1.3. Reply LS is necessary

R1-1803575	LS on messagePowerOffsetGroupB	RAN2, Ericsson

RAN2 asks RAN1 to confirm the values for parameter messagePowerOffsetGroupB.

Decision: The document is noted. Detailed to be handled under 7.1.1.4.2. Reply LS is necessary.

Friday: Unfortunetaly no time to prepare a reply  such reply postponed to next meeting.

R1-1803579	LS to RAN1 on handling parallel RA procedure and SR procedure	RAN2, AsusTek

- In the current RAN2 specs, a UE could perform random access procedure and scheduling request procedure in parallel for single cell, CA, and DC. The UE may need to perform PRACH transmission and SR transmission in the same slot.

- RAN2 doesn’t expect any changes in RAN2 specification and would like to inform RAN1 of this possible situation.

Decision: The document is noted. Details to be handled under 7.1.1.4.2. Reply LS is necessary

R1-1803582	LS on Message 3 size for NR	RAN2, NTT DOCOMO

RAN2 asks RAN1 to provide the possible transport block size of CCCH SDU + MAC header (2 octets) delivered as message 3 for NR.

Decision: The document is noted. Details to be handled under 7.1.1.4.2. Reply LS is necessary.

R1-1805029	Draft reply LS on Message 3 size for NR	NTT DOCOMO, INC.

R1-1805030	Discussion for NR Message 3 size	NTT DOCOMO, INC.

R1-1805214	Draft LS reponse to RAN2 on Message 3 size for NR	Ericsson

R1-1803590	LS on UE measurements for demodulation test setup in FR2	RAN4, Intel

Decision: The document is noted.

R1-1805264	LS on RAN2 agreements on RRM	RAN2, Huawei

Decision: The document is noted.

Details to be handled under 7.1.1.5.1

R1-1803577	LS to RAN1 on Beam- and Cell- Radio-Link-Monitoring	RAN2, Nokia

RAN2 requests RAN1 to provide feedback on two aspects:

- Based on the information provided by RAN1 previously, RAN2 understands that up to 8 RLM-RS(s) can be configured for the UE. However, there is no maximum number of BFD RS resources provided yet.

Question 1: Should the maximum number be applicable per RS resource purpose (i.e. separate maximum number for RLM and for BFD) or should the maximum cover both purposes (i.e. a single maximum number value of RLM+BFD RS(s) )? What is the maximum number of BFD-RS(s) in the first case or the maximum number of sum of BFD and RLM RS(s) in the second case?

- RAN2 understanding is that it is up to the network configuration how to configure RLM-RS and BFD-RS resources, i.e. they can overlap entirely (i.e. the same RS resources are used for both purposes), overlap partially (i.e. some of RS(s) configured for RLM and for BFD are the same) or they can be completely orthogonal (i.e. all of the resources configured for RLM are different from the ones configured for BFD). However, RAN2 was not sure whether the last option is possible.

Question 2: Can the RS resources for BFD and RLM be completely orthogonal?

Decision: The document is noted. Details to be handled under 7.1.1.5.2 (to lead to the discussion) & 7.1.2.2.3. Reply LS is necessary.

R1-1803722	Draft reply LS to RAN2 on beam- and cell- radio link monitoring	CATT

R1-1804446	[Draft] Reply LS on Beam- and Cell- Radio-Link-Monitoring	Huawei, HiSilicon

R1-1805099	Draft LS Response to RAN2 on Beam- and Cell- Radio-Link-Monitoring	Nokia, Nokia Shanghai Bell

R1-1805213	Draft LS reponse to RAN2 on Beam- and Cell- Radio-Link-Monitoring	Ericsson

R1-1803576	LS response to RAN1 on NR MIMO MAC CE	RAN2, Huawei

From RAN2 perspective there is no additional latency introduced and this issue regarding latency of MAC CE message application for NR MIMO should be finally decided by RAN1 and/or RAN4.

Decision: The document is noted. Details to be handled under 7.1.2

R1-1803601	LS on PA EVM for pi/2 BPSK and DMRS	RAN4, Qualcomm

- Observation 1: For un-shaped waveforms, when ZC DMRS accompanies pi/2 BPSK, PA output may be backed off compared to the PA’s capability with pure pi/2 BPSK.

- Observation 2: For shaped waveforms, when ZC DMRS accompanies pi/2 BPSK, PA output may have to be backed off further compared to non-shaped waveforms. Reader is referred to [1] for details. RAN4 believes alternative shaping filter implementations are possible.

Decision: The document is noted. Details to be handed under 7.1.2.3.3

R1-1803574	LS on LTE peak data-rate calculation for EN-DC	RAN2, Intel

RAN2 asks RAN1 on the validity of the provided formula for the calculation of the LTE peak data-rate in EN-DC and if needed, requests a corrected formula for RAN2 to use.

Decision: The document is noted.

Detailed to be handled under 7.1.3.3.1 (along with R1-1801324). Reply LS is necessary

R1-1803648	Discussion on UE peak data rate	Huawei, HiSilicon

R1-1803649	Draft reply LS on NR UE peak data rate	Huawei, HiSilicon

R1-1803699	Draft reply LS on LTE peak data rate calculation for EN-DC	Huawei, HiSilicon

R1-1804681	Discussion on L1 data rate	Intel Corporation

R1-1804682	DRAFT Reply LS on formula or table for L1 data rate	Intel Corporation

R1-1803581	LS on maximum number of CCs for NR CA	RAN2, NTT DOCOMO

RAN2 respectfully asks RAN1 to check if RAN1 specs support 32 CCs for NR CA.

Decision: The document is noted. Details to be handled under 7.1.3.4.2. Reply LS is necessary.

R1-1803702	LS reply on maximum number of CCs for NR CA	Huawei, HiSilicon

Proposed conclusion:

- From RAN1 perspective, the previous agreements of supporting up to 16 DL carriers in CA and up to 16 UL carriers in CA still hold

- RAN1 does not intend to change the above agreements in Rel-15

- Note: whether or not there is any RAN1 spec update if increasing from 16 to 32 CCs in CA is a separate issue

Friday conclusion:

- From RAN1 perspective, the previous agreements of supporting up to 16 DL carriers in CA and up to 16 UL carriers in CA still hold

- RAN1 does not intend to change the above agreements in Rel-15

- Email discussion/approval of reply LS to R1-1803581 till 4/27  – Fred (NTT DOCOMO)

R1-1803588	Reply LS on SI reception in BWP		RAN2, LG Electronics

Question 1:	Can SI be broadcast in UE’s active BWP if the subcarrier spacing of UE’s active BWP is different than the subcarrier spacing of initial BWP?

Question 2:	What is the subcarrier spacing (i.e. the subcarrier spacing of UE’s active BWP or the subcarrier spacing of initial BWP) used for broadcasting SI in UE’s active BWP, if the answer to the Question 1 is YES?

Question 3:	Can UE support reception of broadcast SI within UE’s active BWP if the answer to question 1 is YES?

In a different scenario, a BWP where paging and SI are broadcast may be overlapped (in frequency) with a UE’s active BWP, and both BWPs use the same subcarrier spacing. In this case, if UE’s active BWP does not provide paging and broadcast SI, the UE may be able to receive paging and SI from the overlapping BWP that provides paging and SI broadcast, while staying in UE’s active BWP.

Question 4:	Can UE support reception of broadcast SI from non-active BWP while staying in UE’s active BWP, when SI is provided in a non-active BWP part overlapping with UE’s active BWP?

Question 5:	If the answer to Question 4 is yes, RAN2 would like to also know whether reception of broadcast SI from a non-active BWP while staying in UE’s active BWP is mandatory for the UE to support for the case where the BWPs are overlapping

Decision: The document is noted.

Details to be handled under 7.1.3.4.1 (related to also 7.1.1.2/7.1.1.3). Reply LS is necessary

R1-1804506	[draft] Reply LS on SI reception in BWP	LG Electronics

R1-1804953	Discussion and draft LS response to SI reception in BWP	Motorola Mobility, Lenovo

R1-1804962	[Draft] Reply LS on SI Reception in BWP	MediaTek Inc.

R1-1803602	LS on BWP switching delay	RAN4, Intel

Decision: The document is noted. Details to be handled under 7.1.3.4.1

R1-1803600	LS on SFTD measurement before EN-DC	RAN4, Huawei

Decision: The document is noted. Details to be handled under 7.1.5

R1-1803595	LS reply to RAN1 on P_0 ranges on UL power control	RAN4, Huawei

Decision: The document is noted. Details to be handled under 7.1.6

R1-1803596	LS on NR UE feature list	RAN, NTT DOCOMO

Decision: The document is noted.

R1-1803604	LS on UE feature list	RAN4, NTT DOCOMO, Intel

Decision: The document is noted.

Details to be handled under 7.1.7

R1-1803578	Reply to LS on eV2X evaluation methodology	5GAA WG4, Intel

Decision: The document is noted. Details to be handled under 7.5

R1-1805523	LS on the transmission of video over PC5 interface	SA4, Samsung

RAN1 chair: unclear whether these issues relates to LTE only, NR only or both?

Decision: The document is noted. Details to be handled under 7.5. Reply LS is necessary

Friday: No reply available  such reply postponed to next meeting.

R1-1803597	LS on Summary of email discussion “[ITU-R AH 01] Calibration for self-evaluation”	ITU-R Ad-Hoc, Huawei

Decision: The document is noted. Details to be handled under 7.8

The following LSs are noted (not to be handled online unless requested)

R1-1803583	Reply LS on required information for NSA on X2	RAN2, Nokia Shanghai Bell

R1-1803587	LS on UL timing difference in EN-DC	RAN2, Qualcomm

R1-1803591	Reply LS on clarification in MAC	RAN4, Huawei

R1-1803592	LS reply on UE RF related parameters, capabilities and features for NR standalone	RAN4, NTT DOCOMO

R1-1803593	LS reply on UE baseband processing capability	RAN4, Intel

R1-1803594	Reply LS on NR Idle Mode Measurements	RAN4, CMCC

R1-1803599	LS on EARFCN provisioning for Release 15 MTC and Release 15 NB-IOT UE	RAN4, Qualcomm

R1-1803605	Acquisition of N_(TA_Offset) for Uplink Transmission	RAN4, Ericsson

R1-1805520	LS on L2 measurement specification for NR in R15	SA5, Ericsson

LS received during the week:

R1-1805744	Reply LS on eV2X evaluation methodology	SAE C-V2X TC Advanced Application Task-Force, AT&T

Decision: The document is noted.

R1-1805579	Reply LS on handling of multiple numerologies in FeMBMS	RAN2, Qualcomm	(see 6.1.7)

R1-1805576	LS on NPRS design enhancement for colliding NPRS	RAN4, Qualcomm	(see 6.1.6)

# E-UTRA

## Maintenance of E-UTRA Releases 8 – 14

R1-1805421	Chairman's notes of AI 6.1 Maintenance of E-UTRA Releases 8 - 14	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

### Maintenance of E-UTRA Release 8 – 13

36.212

R1-1804500	Clarification on CRC attachment for DL-SCH and PCH transport channels	NEC Corporation

Decision: The intention of the CR is agreed in principle. Offline discussion whether a CR is necessary or not (particularly the need to fine-tuning “reasons for change”) and if so, which release the CR will start to apply.

R1-1805327	Clarification on CRC attachment for DL-SCH and PCH transport channels in NB-IoT

Decision: CR0285 to 36.212 (Rel-15, F) is agreed.

R1-1804891	Ordering of P-CSI plus SR in eCA	Qualcomm Incorporated

Decision: The draft is CR is endorsed. Final CRs are agreed in R1-1805298 (Rel-13, CR0281), R1-1805299 (Rel-14, CR0282), R1-1805300 (Rel-15, CR0283).

36.213

R1-1804964	Correction on number of subframes for semistatic codebook determination and transmission over PUSCH	Qualcomm Incorporated

Decision: The draft is CR is endorsed by updating “a PUSCH transmission” to “on PUSCH”. Final CR is agreed in R1-1805301 (Rel-13, CR1058), R1-1805302 (Rel-14, CR1059), R1-1805303  (Rel-15, CR1060).

### Maintenance of Release 14 Enhanced Licensed-Assisted Access to Unlicensed Spectrum

None.

### Maintenance of Release 14 V2V/V2X services based on LTE sidelink

None

### Maintenance of Release 14 Full-Dimension MIMO for LTE

36.211

R1-1805280	Correction on DMRS scrambling ID for semi-open loop transmission	Huawei, HiSilicon

(Revision of R1-1803955)

Decision: CR0429r1 to 36.211 Rel-14, F is agreed.

R1-1805281	Correction on DMRS scrambling ID for semi-open loop transmission	Huawei, HiSilicon

(Revision of R1-1803956)

Decision: CR0430r1 to 36.211 Rel-15, A is agreed.

R1-1804892	On support of advanced CSI	Qualcomm Incorporated

Decision: The document is noted.

Agreements:

- Introduce a new UE capability csi-ReportingAdvancedMaxPorts-r14 that indicates the maximum number of CSI-RS ports supported for advanced CSI.

  - The range of this capability is ENUMERATED {[4], [8], 12, 16, 20, 24, 28}

    - Further offline discussion to finalize the ones in brackets

  - Introduce this capability per BoBC.

  - To keep backward compatibility with current Rel-14 ASN.1:

    - if the UE does not include this capability and indicates csi-ReportingAdvanced-r14 as supported for a BoBC, then the UE supports advanced CSI with 32 ports in that BoBC

    - if the UE includes this capability for a BoBC, then the UE does not indicate csi-ReportingAdvanced-r14 for a BoBC

- Send an LS to RAN2 in R1-1805304 – (after the set of values are finalized)

From maintenance session, the above agreement is modified as:

  - The range of this capability is ENUMERATED {[8], 12, 16, 20, 24, 28}

    - RAN1 will confirm whether the value 8 is included in RAN1#93.

R1-1805304	Draft LS on Capabilities for advanced CSI in FD-MIMO	Qualcomm Incorporated

Decision: Final LS is approved in R1-1805402.

### Maintenance of Release 14 Further Enhanced MTC for LTE

R1-1804893	Clarification on CSS monitoring for 5MHz operation	Qualcomm Incorporated

R1-1805273	Correction on TBS determination for msg 3 retransmission and CSS for fallback	Samsung

Late submission

Decision: Further discussion offline (including the issues in x5273).

Decision: Agreed in principle. Final CR to be prepared in R1-1805361 (36.213 CR1062, Rel-14) and R1-1805362 (36.213 CR1063, Rel-15)

R1-1805353	Fallback DCI for eMTC	Qualcomm

Decision: Agreed. Final CR in R1-1805359 (36.212 CR 0287, Rel-14) and R1-1805360 (36.212 CR 0288, Rel-15)

R1-1804894	Correction to timing advance	Qualcomm Incorporated

Decision: Further discussion offline. Revisit in RAN1#93

R1-1804150	Correction on TPC field in DCI format 6-1A	Huawei, HiSilicon

Decision: Further discussion offline

Decision: Agreed. Final CR in R1-1805329 (36.212 CR 0289, Rel-14) and R1-1805330 (36.212 CR 0286, Rel-15)

### Maintenance of Release 14 Enhancements of NB-IoT for LTE

UCG with 2 HARQ processes

R1-1803881	On NPUSCH uplink compensation gaps for 2 HARQ processes	Huawei, HiSilicon

R1-1804145	NPUSCH UL gap for 2 HARQ processes	MediaTek Inc.

R1-1804899	UL Compensation gaps for multiple HARQ	Qualcomm Incorporated

Decision: Further discussion offline

R1-1805358	Clarification of NPUSCH uplink compensation gaps for 2 HARQ processes	Huawei, HiSilicon

R1-1805398	Clarification of NPUSCH uplink compensation gaps for 2 HARQ processes	Huawei, HiSilicon

Decision: Agreed. Final CRs in R1-1805403 (TS36.213, CR1071, Rel-14, F) and R1-1805404 (TS36.213, CR1072, Rel-15, A)

DL Channel Quality Reporting in Msg 3

R1-1804087	Discussion on remaining issues for DL CE Level determination  in NB-IoT	CMCC

R1-1804144	CSI Report in Msg3 in Random Acces Procedure in NB-IoT	MediaTek Inc.

R1-1804146	On downlink channel quality measurement and report in Msg3	Huawei, HiSilicon

R1-1804168	Remaining issues on DL measurement reporting in NB-IoT	Ericsson

R1-1804315	Discussion on measurement report for NB-IoT	Samsung

R1-1804897	Considerations for DL quality reporting in msg3 for NB-IoT	Qualcomm Incorporated

Decision: Further discussion offline

R1-1805401	WF on channel quality report in Msg3 for NB-IoT	CMCC, ZTE, Sanechips, Samsung, MediaTek

Agreement:

For the downlink channel quality reporting in msg3:

- RAN1 assumes that the UE is not required to measure additional subframes for this feature  (e.g., the measured subframes used for cell reselection before random access can be reused).

  - RAN1 does not intend to define the subframes used for measurement for DL channel quality reporting.

- In Rel-14, this feature is only supported for the anchor carrier on which the UE received msg2.

- RAN1 will not define a reference resource for NPDCCH (i.e., the location in time of the “virtual PDCCH”)

- RAN1 considers that the indicated hypothetical NPDCCH repetition number (R) should be derived based on averaging the DL quality during a period of time (to average fading out) without incurring in additional wake-ups for measurement.

- RAN1 leaves the decision on the number and value of candidates of R to be decided by RAN2 and RAN4.

R1-1805405	Draft LS on downlink channel quality report in Msg3 for NB-IoT	CMCC

Decision: The document is noted. Final LS is approved in R1-1805407 with the following modification:

Change the Action to “RAN1 kindly requests RAN2 and RAN4 to determine the number and value of candidates of R”

NPRS

R1-1805284	On NPRS for OTDOA	Huawei, HiSilicon	(Revision of R1-1804147)

R1-1804167	On NPRS performance	Ericsson

R1-1804896	On NPRS performance	Qualcomm Incorporated

Decision: Further discussion offline

R1-1805576	LS on NPRS design enhancement for colliding NPRS		RAN4

Decision: The document is noted.

Agreement: A new NPRS sequence should be designed

- Working assumption: The new NPRS sequence can operate in a backward compatible manner with the Rel-14 NPRS by configuration

Others

R1-1804151	Correction on Msg3 power control for NB-IoT	Huawei, HiSilicon

Decision: The intention of the CR is agreed in principle. Offline discussion whether a RAN1 CR is necessary or not including refining the wording/parameter name of the change

Decision: Final CR in R1-1805364 (36.213 CR1064, Rel-14, F) and R1-1805365 (36.213 CR1065, Rel-15, A)

R1-1804195	Clarification on NPUSCH postponement	ZTE, SaneChips

Conclusion:

- The two proposals in R1-1804195 are aligned with RAN1 understanding

  - No RAN1 spec change is necessary

R1-1804895	Collision with NPRACH in non-anchor carriers	Qualcomm Incorporated

Decision: Further discussion offline especially related to parameter multiCarrier-NPRACH

R1-1805279	Collision with NPRACH in non-anchor carriers	Qualcomm Incorporated

Decision: Agreed. Final CRs in R1-1805367 (36.211 CR0436, Rel-14, F) and R1-1805368 (36.211 CR0437, Rel-15, A)

R1-1804898	On NRS presence in non-anchor carriers	Qualcomm Incorporated

Conclusion:

- RAN1 to clarify that the UE can assume that the eNB always transmits NRS in the Type-2 CSS configured by higher layers, as well as 10 NB-IoT DL subframes before and 4 NB-IoT DL subframes after each Type-2 CSS, regardless of whether the eNB detected an NPRACH transmission or not

  - Further discussion offline on the corresponding necessary spec changes

Decision: The text proposal in R1-1804898 is endorsed. Final CRs in R1-1805369 (36.211 CR0438, Rel-14, F) and R1-1805370 (36.211 CR0439, Rel-15, A).

### Other

R1-1804196	Correction on PUSCH hopping parameter for eVoLTE	ZTE, CMCC

Decision: The draft CR is endorsed in principle. Further check offline the exact parameter name in 36.331 for alignment

R1-1805331	Correction on PUSCH hopping parameter for eVoLTE	ZTE, CMCC

Decision: Agreed. Final CRs to be prepared and agreed in R1-1805332 (36.211 CR 0433, Rel-14, F) and R1-1805333 (36.211 CR 0434, Rel-15, A)

R1-1805294	Discussion on the configuration of UL 256QAM	Huawei, HiSilicon	(Revision of R1-1804440)

R1-1805295	On the configuration of UL 256QAM	Huawei, HiSilicon	(Revision of R1-1804441)

Decision: The draft CR in x5295 is endorsed in principle. Further check offline on the detaild wording

R1-1805296	On the configuration of UL 256QAM	Huawei, HiSilicon	(Revision of R1-1804442)

Decision: Discuss further offline for the Rel-15 CR in x5296 especially for the sTTI related aspects

R1-1805338	On the configuration of UL 256QAM	Huawei, HiSilicon

Decision: Agreed. Final CRs in R1-1805371 (36.213, CR1066, Rel-14, F) and R1-1805372 (36.213, CR1067, Rel-15, A)

R1-1804900	Issues for SRS Carrier switching	Qualcomm Incorporated

Decision: The document is noted.

Agreements:

- For the issue identified as in proposal “In case an SRS transmission in subframe N in a CC without PUSCH/PUCCH is dropped due to collision with a higher priority transmission in subframe N+1, and SRS would have been otherwise transmitted (in case the higher priority transmission in subframe N+1 had not occurred), the UE is not required to transmit any physical signal or channel in subframe N”, RAN1 acknowledges the issue

  - Furhter discussion offline to refine the wording of the proposal

    - Note: this may or may not have RAN1 spec impact

- Introduce new UE capability for support of 6-symbol SRS in UpPTS.

- Add a UE capability (per band combination) on the number of simultaneously configurable target CC for SRS switching.

- LS to send to RAN2 for the above two bullets (R1-1805307)

R1-1805307	Draft LS on Capabilities for SRS carrier switching	Qualcomm Incorporated

Decision: The document is endorsed and final LS is approved in R1-1805376.

The text proposal in R1-1804900 is endorsed.

Final CR in R1-1805373 (36.213, CR1068, Rel-14, F) and R1-1805374 (36.213, CR1069, Rel-15, A)

R1-1805579	Reply LS on handling of multiple numerologies in FeMBMS	RAN2

Email discussion on candidate values for Maximum supported bandwidth T and scaling factors until May 3, 2018 (Alberto, Qualcomm).

## LTE Release 15

### Maintenance of Release 15 Shortened TTI and processing time for LTE

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

R1-1805422	Chairman's notes of AI 6.2.1 Maintenance of Release 15 Shortened TTI and processing time for LTE	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

Friday

Editors to prepare CRs capturing the agreements till April 27th, for comments and endorsement by May 3rd.

R1-1805420	[DRAFT] LS on additional agreements for shortened TTI and processing time for LTE	Ericsson

Decision: The document is endorsed and final LS is approved in R1-1805435.

R1-1803901	Corrections for shortened processing time and shortened TTI in 36.213, s00-s05	Motorola Mobility Germany GmbH

R1-1803902	Corrections for shortened processing time and shortened TTI in 36.213, s06-s09	Motorola Mobility Germany GmbH

R1-1803903	Corrections for shortened processing time and shortened TTI in 36.213, s10-s13	Motorola Mobility Germany GmbH

R1-1803904	Corrections for shortened processing time and shortened TTI in 36.213, s14-xx	Motorola Mobility Germany GmbH

Decision: Above CRs to 36.213 were endorsed post#92 over email. They were based on previous version of the spec as v15.1.0 was not available. CR needs to be revised in RAN1#93.

R1-1803954	Corrections on shortened processing time and shortened TTI in 36 212	Huawei

Decision: CR0280 to 36.212 (Rel-15, F) is agreed (was endorsed post#92 over email agreement).

R1-1805146	Missing agreements from agreed text proposals at RAN1#92	Ericsson, LG Electronics

The proposed agreements in section 2 of R1-1805146 except proposal 8 and 11 reflect agreed text proposals

Agreement:

Different UL sTTI lengths can be configured for the serving cells across different PUCCH groups for which sTTI operation is configured.

R1-1803861	Corrections to sTTI and SPT implementation	Ericsson

CR based on previous version of the spec as v15.1.0 was not available. CR needs to be revised.

R1-1805171	Corrections to sTTI and SPT implementation	Ericsson

R1-1805394	Corrections to sTTI and SPT implementation	Ericsson

Decision: CR0428r2 to 36.211 (Rel-15, F) is agreed.

#### Maintenance for shortened processing time for 1ms TTI

R1-1804507	TDD HARQ-ACK resource determination for SPT (36.213)	LG Electronics, Ericsson, Nokia, Nokia Shanghai Bell

The text proposal in R1-1804507 is endorsed.

R1-1804871	Correction on shortened processing time for 1ms TTI	Sharp

Text proposal 2 in R1-1804871 is endorsed

R1-1805118	HARQ ID definition to TDD UL index based PUSCH scheduling for SPT in 36.213 - TP#15	Nokia, Nokia Shanghai Bell, Ericsson

The text proposals in R1-1805118 are endorsed.

Agreement:

In case shortProcessingTime is configured in a cell when multiple PUSCHs are transmitted by one UL grant in FS2, consecutive HARQ process IDs are used for PUSCH transmission.

R1-1805119	Retaining UL index based scheduling flexibility for SPT without UpPTS PUSCH (36.213) - TP#16	Nokia, Nokia Shanghai Bell, Ericsson

Endorse the text proposals in R1-1805119.

Agreement:

Retain the legacy flexibility for TDD Configuration 0 without PUSCH on UpPTS with SPT to schedule PUSCH in subframes 2 and 7 with UL index LSB=1.

Agreement:

Enable uplink index based scheduling for TDD Configuration 6 and SSC 10 with SPT in subframes 5, 6 and 9.

R1-1805387	UL index based scheduling flexibility for SPT for UL/DL configuration #6 and SSC#10	ZTE, Sanechips

Endorse the text proposal in R1-1805387

R1-1805120	Needed correction in 36.213 of applicability of TDD UL PUSCH timing for SPT - TP#17	Nokia, Nokia Shanghai Bell, Qualcomm

The text proposals in R1-1805120 are endorsed.

R1-1805121	Support for RRC configurable DM-RS CS for 1ms TTI UL SPS - TP#19	Nokia, Nokia Shanghai Bell, Ericsson

Agreement:

For 1ms TTI UL SPS operation, different Cyclic shift for DM RS can be configured by RRC (3-bit CS).

- No changes to the SPS activation / release DCI (i.e. DM-RS CS field set to ‘000’).

- Independent UE capability signaling (36.331 impact)

- Independent RRC configuration (i.e. independent from shortProcessingTime or shortTTI configuration, 36.331 impact)

R1-1805150	Clarification on HARQ sequence for sPT, FS1 and fixed codebook size (36.212)	Ericsson, Qualcomm

#### Maintenance for shortened TTI with shortened processing time

R1-1805415	Notes from the offline session on LTE sTTI	Ericsson

##### Aspects related to interaction between different TTI lengths

R1-1803718	Handling of collision between PRACH and sTTI	Huawei, HiSilicon

R1-1804901	Aspects related to interaction between different TTI lengths	Qualcomm Incorporated

Revisit text proposals in sections other than section 5 in R1-1804901 prioritizing the TP in section 4 in RAN1#93.

R1-1805406	Corrections for supporting two TBs in case of sTTI-to-TTI dynamic switching (TP#57)	Qualcomm

The text proposal in R1-1805406 is endorsed.

R1-1805147	Text proposal on 1ms HARQ bits inclusion in UL sTTI in case of SPS and HARQ process sharing	Ericsson

Endorse the text proposal in section 2 of R1-1805147

R1-1805354	Text proposal on 1ms HARQ bits inclusion in UL sTTI in case of SPS and HARQ process	Ericsson

The text proposals in R1-1805354 are endorsed.

R1-1805260	Corrections related to interaction between different TTI lengths	LG Electronics, Ericsson

Late submission

##### Aspects related to DL control channel

R1-1803714	Aspects related to DL control channel	Huawei, HiSilicon

R1-1804137	Clarification on sREG to sCCE mapping for sTTI	MediaTek Inc.

The text proposal in R1-1804137 is endorsed.

R1-1804683	RE mapping of sPDCCH	Intel Corporation, Ericsson, Nokia, Qualcomm

The text proposal in R1-1804683 is endorsed.

R1-1804902	Aspects related to DL control channel	Qualcomm Incorporated

Agreement:

Support for monitoring sPDCCH on RB sets with different RS types within a TTI is a UE capability.

R1-1805087	Clarification for sPDSCH rate matching behavior with two SPDCCH PRB set	MediaTek Inc.

Withdrawn

R1-1805088	Clarification for assumption in SPT with CSS and USS overlap	MediaTek Inc.

R1-1805129	EPDCCH monitoring restrictions for shorter TTI operation (36.213) - TP#45	Nokia, Nokia Shanghai Bell, Huawei, LG Electronics, Mediatek, Intel

Agreement:

The following text proposal for section 9.1.4 of TS 36.213 is endorsed.

--- Start of TP ---

9.1.4 	EPDCCH assignment procedure

For each serving cell, higher layer signalling can configure a UE with one or two EPDCCH-PRB-sets for EPDCCH monitoring. The PRB-pairs corresponding to an EPDCCH-PRB-set are indicated by higher layers as described in Subclause 9.1.4.4. Each EPDCCH-PRB-set consists of set of ECCEs numbered from 0 to where is the number of ECCEs in EPDCCH-PRB-set  of subframe . Each EPDCCH-PRB-set can be configured for either localized EPDCCH transmission or distributed EPDCCH transmission.

The UE shall monitor a set of EPDCCH candidates on one or more activated serving cells as configured by higher layer signalling for control information, where monitoring implies attempting to decode each of the EPDCCHs in the set according to the monitored DCI formats.

A BL/CE UE is not required to monitor EPDCCH.

For a serving cell, a A UE configured with higher layer parameters shortProcessingTime and/or shortTTI for a serving cell, is not required to monitor EPDCCH for that serving cell or on that serving cell.

A UE is not required to monitor EPDCCH in an MBSFN subframe with zero-size non-MBSFN region.

The set of EPDCCH candidates to monitor are defined in terms of EPDCCH UE-specific search spaces.

For each serving cell, the subframes in which the UE monitors EPDCCH UE-specific search spaces are configured by higher layers.

--- End of TP ---

Agreement:

UE can indicate its capability to support the following separately

- EPDCCH and sTTI on different serving cells

- EPDCCH and Shortened processing time on different serving cells

R1-1805149	TP to 36.211 Excluding TDM support for subslot CRS-based SPDCCH and DMRS-based PDSCH	Ericsson, Nokia, Nokia Shanghai Bell, Intel, Huawei, HiSilicon, Qualcomm, LG Electronics

The text proposals in R1-1805149 are endorsed.

##### Aspects related to UL control channel

R1-1803720	Aspects related to UL control channel	Huawei, HiSilicon

R1-1804903	Aspects related to UL control channel	Qualcomm Incorporated

R1-1805310 (revision of R1-1805161)	Correction to UE sounding procedures (36.213)	Ericsson

The text proposals in sections 2 and 3 of R1-1805310 are endorsed.

##### Aspects related to DL data channel

R1-1803716	Aspects related to DL data channel	Huawei, HiSilicon

R1-1804508	Corrections related to DL data channel for sTTI operation	LG Electronics, Ericsson, Nokia, Nokia Shanghai Bell, Intel Corporation

R1-1805399	Corrections related to DL data channel for sTTI operation	LG Electronics, Ericsson, Nokia, Nokia Shanghai Bell, Intel Corporation

Agreement:

The following modifications to Section 7.1 of TS36.213 are agreed.

--- Start of TP ---

7.1 UE procedure for receiving the physical downlink shared channel

<Unchanged parts are omitted>

For a UE configured with higher layer parameter shortTTI, the UE is not expected to receive

-	PDSCH data transmissions signalled via PDCCH/SPDCCH with CRC scrambled by the C-RNTI/SPS C-RNTI and DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G for a transport block corresponding to a HARQ process with NDI not toggled if the previous PDSCH transmission of the transport block was signalled via PDCCH/EPDCCH with CRC scrambled by the C-RNTI/SPS C-RNTI with DCI format other than DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G when the number of codewords for the previous PDSCH transmissions is two or the transport block size is more than the maximum transport block size supported for slot/subslot-PDSCH transmission.

- PDSCH data transmissions signalled via PDCCH/SPDCCH with CRC scrambled by the C-RNTI/SPS C-RNTI and DCI format 7-1F/7-1G in subslot n, if the PDCCH/SPDCCH indicates the absence of the UE-specific reference signal in subslot n and if the UE has not received a PDCCH/SPDCCH with CRC scrambled by the C-RNTI/SPS C-RNTI and DCI format 7-1F/7-1G in subslot n-1. The UE shall provide the HARQ-ACK response associated with the PDCCH/SPDCCH received in subslot n.

For a UE configured with higher layer parameter shortTTI, and for PDSCH data transmissions in subslot n signalled via PDCCH/SPDCCH of a serving cell with DCI format 7-1F/7-1G, the UE is not expected to decode the PDSCH

- If the DCI associated with subslot-PDSCH indicates the absence of the UE-specific reference signal in subslot n-1, and if the DCI associated with subslot-PDSCH indicates the absence of the UE-specific reference signal in subslot n, or

- If the DCI in subslot n-1 indicates the presence of DMRS and the DCI in subslot n indicates the absence of DMRS for a subslot-PDSCH received on PRGs other than PRGs on which the subslot-PDSCH was received in subslot n-1, or

- If the DCI associated with subslot-PDSCH indicates the presence of the UE-specific reference signal in subslot n-1, and if the DCI associated with subslot-PDSCH indicates the absence of the UE-specific reference signal in subslot n with greater number of antenna ports than those indicated in subslot n-1.

--- End of TP ---

R1-1805269	Aspects related to DL data channel	Qualcomm Incorporated	(Revision of R1-1804904)

Agreement:

The determination of the shifted DMRS pattern is based on the collision between the number of configured layers for the UE and other RS types.

Agreement:

The Text proposal affecting clause 6.4 of TS36.211 in Section 4 “Corrections to Mapping of Symbols to REs for SFBC-based PDSCH (TP#55)”) of R1-1805269 is endorsed.

Agreement:

The Text proposal affecting clause 5.3.1 of TS36.211 in Section 4 “Corrections for DL and UL Scrambling Sequence (TP#56)” of R1-1805269 is endorsed.

R1-1805122	Enabling transmit diversity fallback for DL TM 6 for shorter TTI (36.212, 36.213) - TP#20	Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics, Intel, Qualcomm

R1-1805123	Enabling transmit diversity fallback for DL TM 8 for shorter TTI (36.212, 36.213) - TP#21	Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics

R1-1805124	Enabling transmit diversity fallback for DL TM 9 and 10 for shorter TTI (36.212, 36.213) - TP#22	Nokia, Nokia Shanghai Bell, LG Electronics

Agreement:

Define a UE capability for the support of TX diversity transmission using ports 7 and 8 for TM9/10 separately for slot and subslot PDSCH.

Agreement:

Support dynamic TX diversity fallback for TM9 & 10 (through DCI Format 7-1F/7-1G) for up to 2-layer SPDSCH MIMO operation. The alternative Antenna port(s), scrambling identity and number of layers indication table for TX fallback & SU-MIMO up to 2 layers is configured by higher layers.

Agreement:

Support dynamic TX diversity fallback for TM9 & 10 (through DCI Format 7-1F/7-1G) for up to 4-layer SPDSCH MIMO operation. The alternative Antenna port(s), scrambling identity and number of layers indication table for TX fallback & SU-MIMO up to 4 layers is configured by higher layers.

R1-1805126	Corrections on DM-RS for slot PDSCH (36.211) - TP#24	Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics

R1-1805160	Correction to UE behavior in case simultaneous subslot/slot and subframe PDSCH decoding is not supported (36.213)	Ericsson, Nokia, Huawei

The text proposal in R1-1805160 is endorsed.

R1-1805412	On modulation for sTTI operation	Huawei, HiSilicon

##### Aspects related to UL data channel

R1-1803715	Additional DMRS position pattern for subslot #5 	Huawei, HiSilicon, Ericsson

R1-1805363	Text proposal on TBS determination for uplink DMRS position pattern DDD	Huawei, HiSilicon

R1-1804509	Corrections related to UL data channel for sTTI operation	LG Electronics

R1-1805270	Aspects related to UL data channel	Qualcomm Incorporated	(Revision of R1-1804905)

R1-1805117	Correction of number of UL HARQ processes for shorter TTI and shortened processing time (36.213) - TP#14	Nokia, Nokia Shanghai Bell, Ericsson, Intel

R1-1805148	Text proposal on TPC timing with DCI format 3/3A and short TTI operation	Ericsson, LG Electronics, Nokia, Nokia Shanghai Bell, Qualcomm

R1-1805417	On UL TPC initialization after UL shorter TTI configuration	Nokia, Nokia Shanghai Bell	(Revision of R1-1805317)

##### Aspects related to FS2

R1-1803717	Aspects related to FS2	Huawei, HiSilicon

R1-1805125	Changes to FS2 SRS triggering for shorter TTI (36.212, 36.213) - TP#23	Nokia, Nokia Shanghai Bell, Ericsson, Huawei, LG Electronics

R1-1805127	Corrections and simplifications of K_PUSCH for FS2 slot-PUSCH in 36.213 - TP#25	Nokia, Nokia Shanghai Bell, Ericsson

R1-1805128	Correction in 36.213 to TDD UL index based slot PUSCH scheduling  - TP#26	Nokia, Nokia Shanghai Bell

R1-1805375	Correcting wrong entry in K_PUSCH Table for FS2 slot-PUSCH in 36.213	Nokia, Nokia Shanghai Bell, Ericsson	(Revision of R1-1805127)

##### Other

R1-1804410	Correction on mapping to physical resources for slot-SPUCCH format 1/1a/1b with frequency hopping	Huawei, HiSilicon, Ericsson, LG Electronics

Text proposal in R1-1804410 is endorsed.

R1-1804411	Discussion on subslot-SPUCCH repetition	Huawei, HiSilicon

R1-1804412	Corrections on timing from sPDSCH to HARQ-ACK for SSC3,4,8	Huawei, HiSilicon, Ericsson, LG Electronics

R1-1804413	Corrections on HARQ-ACK reporting procedure for FS2 with sTTI	Huawei, HiSilicon, Ericsson

R1-1804414	Corrections on TBS determination for more than 1 layer	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell

R1-1804415	Corrections on DAI in 36.212	Huawei, HiSilicon, Ericsson, LG Electronics, Qualcomm

R1-1804416	Correction on antenna ports quasi co-location for DMRS based SPDCCH	Huawei, HiSilicon, Nokia

Endorse the text proposal in R1-1804416

R1-1804417	Correction on DCI format 7-1E in 36.212	Huawei, HiSilicon, Ericsson, Nokia

Endorse the text proposal in R1-1804417

R1-1804418	TP on VRB assignment of DL resource allocation type 2	Huawei, HiSilicon, Nokia, Mediatek, Qualcomm

Agreement:

The text proposal in R1-1804418 is endorsed.

Agreement:

A slot/subslot-based PDSCH only support localized VRB assignment in Resource Allocation Type 2.

R1-1804419	Correction on sequence generation for UE-specific reference signals associated with slotsubslot based PDSCH	Huawei, HiSilicon, Nokia

Agreement:

The text proposal in R1-1804419 is endorsed.

R1-1804420	Correction on UCI on PUSCH in 36212	Huawei, HiSilicon, LG Electronics, Qualcomm, Ericsson

Endorse the text proposal in R1-1804420

R1-1804421	Discussion and text proposal on UL resource allocation	Huawei, HiSilicon

R1-1804422	Correction on DL subslot pattern determination	Huawei, HiSilicon

R1-1805366	Text proposal on TBS determination for downlink pattern DDD	Huawei, HiSilicon

R1-1805411	Corrections on DAI in 36.212	Huawei, HiSilicon

#### Other

Including aspects related to maximum TA and processing time, if any

R1-1803719	Remaining issues on SPS	Huawei, HiSilicon

### Maintenance of Release 15 Enhancements to LTE operation in unlicensed spectrum

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

R1-1805423	Chairman's notes of AI 6.2.2 Maintenance of Release 15 Enhancements to LTE operation in unlicensed spectrum	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

Friday

Editors to prepare CRs capturing the agreements till April 27th, for comments and endorsement by May 3rd.

R1-1803960	Introduction of FeLAA into 36.212	Huawei

Decision: The document is endorsed. Final CR is agreed in R1-1805418 (TS36.212, CR0290, Rel-15, B).

R1-1804409	Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.201	Nokia, Nokia Shanghai Bell

Endorsed in principle with the following suggested modifications to the editor:

- Change “A downlink transmission may or may not start at the subframe boundary, and may or may not end at the subframe boundary” to “A downlink or uplink transmission may start at the subframe boundary or later, and may end at the subframe boundary or earlier.

- Delete “An uplink transmission may or may not start at the subframe boundary, and may or may not end at the subframe boundary.”

Decision: Final CR is agreed in R1-1805335 (36.201 CR0024r1, Rel-15, B).

R1-1804453	3GPP TS 37.213 Physical layer procedures for shared spectrum channel	Ericsson

Ericsson to prepare the following document for discussion:

R1-1805336	Additions to TS 36.213 (Rel-14) to capture agreements for Rel-15 FeLAA

R1-1805351	3GPP TS 37.213 v0.0.2 Physical layer procedures for shared spectrum channel	Ericsson

R1-1805352	3GPP TS 37.213 v0.0.3 Physical layer procedures for shared spectrum channel	Ericsson

Decision: The document is endorsed as V0.1.0 in R1-1805416. Further modifications reflecting comments received will be made using R1-1805416 as a reference.

R1-1804484	Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s00-s05	Motorola Mobility Germany GmbH

Decision: The document is endorsed in principle with the following suggested modifications.

- Align terminology between all the specifications by using “Autonomous Uplink” or similar consistently.

R1-1805389	Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s00-s05	Motorola Mobility

Decision: The document is endorsed and final CR is agreed in R1-1804489 (36.213 CR1054, Rel-15, B)

R1-1804485	Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s06-s09	Motorola Mobility Germany GmbH

R1-1805337	Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s06-s09			Motorola Mobility Germany GmbH

Decision: The document is endorsed and final CR is agreed in R1-1804490 (36.213 CR1055, Rel-15, B)

R1-1804486	Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s10-s13	Motorola Mobility Germany GmbH

Decision: The CR (CR1056) is withdrawn as there is no changes for the sections covered by R1-1804491 (Tdoc# is withdrawn).

R1-1804488	Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s14-xx	Motorola Mobility Germany GmbH

Decision: The document is endorsed with an update to remove section 15. Final CR is agreed in R1-1804492 (36.213 CR1057, Rel-15, B)

R1-1805265	Introduction of enhancements to operation in unlicensed spectrum	Ericsson

Decision: Editor to submit an update to RAN1#93. R1-1805340 (CR 0435, Rel-15) is withdrawn.

#### TPs for Multiple starting and ending positions in a subframe for UL

R1-1804316	Discussion on UCI transmission without UL-SCH for PUSCH mode 1	Samsung

R1-1805239	Remaining issues on channel access for UL partial subframe on LAA Scell	WILUS Inc.

#### TPs for Autonomous uplink access with Frame Structure type 3

R1-1804317	Discussion on UCI transmission on AUL PUSCH	Samsung

#### Other

R1-1805153	TBS determination for partial UL subframes	Lenovo, Motorola Mobility

R1-1805154	Collision of AUL and granted transmissions	Lenovo, Motorola Mobility

R1-1805159	On UE Capabilities for LTE_unlic	Nokia, Nokia Shanghai Bell

Email discussion until May 3, 2018 on UE capabilities for FeLAA and associated LS to RAN2 (Timo, Nokia)

### Maintenance of Release 15 Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

R1-1805424	Chairman's notes of AI 6.2.3 Maintenance of Release 15 Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

R1-1803864	Introduction of enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE	Ericsson

R1-1805172	Introduction of enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE	Ericsson

Decision: The draft CR to 36.211 is endorsed. Final CR is agreed in R1-1805314 (CR 0431, Rel-15, B).

R1-1803961	Introduction of high capacity stationary wireless link and 1024QAM into 36.212	Huawei

Decision: The draft CR to 36.212 is endorsed. Final CR is agreed in R1-1805315 (CR 0284, Rel-15, B).

R1-1804477	Introduction of enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE to 36.213	Motorola Mobility Germany GmbH

R1-1804478	Introduction of enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE to 36.213	Motorola Mobility Germany GmbH

Decision: CR1053 to 36.213 (Rel-15, B) is agreed.

R1-1804496	Introduction of Downlink 1024QAM into 36.201	Nokia, Nokia Shanghai Bell

Decision: The draft CR to 36.201 is endorsed. Final CR is agreed in R1-1805316 (CR 0025 rev 1, Rel-15, B).

#### Text Proposals

R1-1804685	Corrections to 1024QAM support in LTE	Intel Corporation

#### Other

R1-1804906	UE Categories for 1024QAM	Qualcomm Incorporated

R1-1805355	WF on DL Categories	Intel, Qualcomm

Decision: Email discussion on DL categories until May 3, 2018 (Qualcomm)

### Maintenance of Release 15 Further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

R1-1805425	Chairman's notes of AI 6.2.4 Maintenance of Release 15 Further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

#### Text Proposals

R1-1803721	Corrections on feCoMP	Huawei, HiSilicon

Decision: Text proposals are endorsed. Final CRs are agreed in R1-1805318 (TS36.211, CR0432, Rel-15, F) and R1-1805319 (TS36.213, CR1061, Rel-15, F).

### 3GPP V2X Phase 2 - WID in RP-171740

R1-1805426	Chairman's notes of AI 6.2.5 3 GPP V2X Phase 2	Ad-Hoc chair (Samsung)

Decision: The document is endorsed, content incorporated below.

Friday

Rapporteur to prepare the list of agreements by April 23rd

Editors to prepare CRs incorporating all the agreements by 5/4, to be commented and endorsed by 5/11

R1-1805380	Reply LS on PC5 transmission mechanism selection	SA2

Reply LS to be handled in RAN1#93

R1-1805397	[DRAFT] LS on RRC parameters for V2X Phase 2	Huawei

Agreement

The LS to RAN2 is endorsed with the following changes:

- T2min_Set should be changed from ‘carrier specific’ to resource ‘pool specific’

- For SL_V2X_Carriers_RX and SL_V2X_Carriers_TX, it is up to RAN2 to make it ‘UE specific’ or ‘UE specific / cell specific’

- For SLSS_MultiCarrierTx and SL_V2X_Carriers_PotentialSync, ‘UE specific’ should be changed to ‘UE specific / cell specific’

- For rows 3 through 7, the RRC parameters are applicable only for carrier aggregation only

Decision: The final LS is approved in R1-1805620.

#### Carrier Aggregation (up to 8 PC5 carriers)

##### Mode-4 support

Agreement:

- If there is overlap in one TTI and UE is not able to transmit simultaneously on multiple carrier due to limitation in available power, then UE should prioritise transmission on higher priority packets.

- If there is overlap in one TTI of same priority packets in different carriers then it should be left to UE implementation to perform transmission if it is constrained in terms of available power.

- In case of conflict with uplink transmission, Rel-14 rules are used with respect to uplink transmissions

R1-1803723	Remaining details on carrier aggregation for mode 4 in V2X phase 2	CATT

R1-1803890	Remaining issues for resource selection in mode 4 CA	ZTE, Sanechips

R1-1803983	Carrier and resource selection in eV2X carrier aggregation	OPPO

R1-1804018	V2X Sidelink CA Scheduling issues in Mode 4	ITRI

R1-1804130	Power sharing for UE with limited TX capability	Huawei, HiSilicon

R1-1804204	Discussion on Mode 4 support for V2X carrier aggregation	Lenovo, Motorola Mobility

R1-1804256	Remaining Details for Mode 4 support for V2X carrier aggregation	Nokia, Nokia Shanghai Bell

R1-1804318	mode-4 support in V2X CA	Samsung

R1-1804510	Discussion on carrier aggregation in sidelink mode 4 operation	LG Electronics

R1-1804629	On Mode-4 resource selection for PC5 CA	Ericsson

R1-1804686	Remaining physical layer aspects of sidelink carrier aggregation for mode-4 LTE V2V communication	Intel Corporation

R1-1804907	Mode 4 support with V2X PC5 Carrier Aggregation	Qualcomm Incorporated

R1-1805022	On carrier aggregation using mode 4 resource selection	NTT DOCOMO, INC.

R1-1805393	Summary of Mode 4 CA	LG Electronics

##### Synchronization

R1-1805382	Summary of offline discussion on PC5 CA synchronization	Ericsson

Agreement

- For UEs operating with CA

  - RAN1 assumes a UE may be configured a non-synchronization carrier by defining the location of the SLSS resources and by configuring the UE to not transmit SLSS on that carrier.

    - Rel. 14 RRC signalling is not sufficient.

    - Include an RRC parameter to introduce such mechanism.

      - A Rel.15 UE using the carrier without CA does not apply this parameter.

      - It is up to RAN2 to design the signalling to support this feature

Agreement

The working assumption from RAN1#92 is confirmed with following corrections

- The UE is configured one of the following options based on UE capability:

  - SLSS is transmitted (based on Rel-14 procedure) on selected sync carrier from Set-B

  - SLSS is transmitted on all carriers from Set-B

- Each option is an independent UE capability

- On top of this, Release-14 configuration applies to each carrier individually

Agreement

- For the case of limited TX capabilities, for UE SLSS transmission, it is up to UE implementation on which synchronization carrier(s) from Set B UE transmits SLSS

  - The above applies for the case when SLSS is transmitted on all carriers from Set-B

Agreement

- PSBCH content other than bandwidth, TDD configuration, reserved bits are generated following the Rel. 14 procedure following the selected synchronization reference.

  - Note if there is an issue with reserved bits, it will be addressed in RAN1#93

- SLSS ID is derived from the selected synchronization source.

Agreement

- When synchronization is lost, synchronization carrier reselection is up to UE implementation.

R1-1803724	Remaining details on synchronization for carrier aggregation in V2X Phase 2	CATT

R1-1803891	Remaining issues for synchronization in Sidelink CA	ZTE, Sanechips

R1-1803984	Synchronization in eV2X carrier aggregation	OPPO

R1-1804019	Synchronization for Sidelink Carrier Aggregation in V2X Phase 2 	ITRI

R1-1804133	Remaining details for synchronization for carrier aggregation on sidelink	Huawei, HiSilicon

R1-1804257	Remaining details of SL CA synchronization	Nokia, Nokia Shanghai Bell

R1-1804319	Synchronization in V2X CA	Samsung

R1-1804511	Discussion on synchronization for sidelink CA operation	LG Electronics

R1-1804630	On Synchronization Aspects for PC5 CA	Ericsson

R1-1804687	Remaining synchronization details for LTE V2V sidelink carrier aggregation	Intel Corporation

R1-1804908	Synchronization for V2X PC5 Carrier Aggregation	Qualcomm Incorporated

R1-1805091	Remaining issues on synchronization for CA in V2X Phase 2	ITL

##### Other

R1-1804631	Spectrum considerations for PC5 carrier aggregation	Ericsson

#### Support for 64-QAM

R1-1805328	Summary of offline discussion on support of 64-QAM for LTE V2V communication	Intel

Agreement

The following working assumption is confirmed

- Single TBS scaling factor value is applied (i.e. independently of ITBS/IMCS values) to all numbers in the table.

Working Assumption

- Scaling factor is applied to the number of PRBs derived from SCI

  - Actual TBS value is derived by using scaled number of PRBs defined by the following equation

,

  - where NPRB' is the original total number of allocated PRBs according to 7.1.6 from 3GPP 36.213 LTE R14.

- No new TBS values are introduced.

Agreement

- TBS scaling factor value is equal to =0.8

Agreement

- Agree to use reserved bit(s) in SCI format to indicate R15 PSSCH transmission format/features

- Further discuss whether it is needed to separately indicate specific features (e.g. rate-matching, 64-QAM support) or transmission format

R1-1803725	Discussion on 64QAM support in V2X phase 2	CATT

R1-1803894	Support for 64 QAM	ZTE, Sanechips

R1-1804129	Remaining details for supporting 64-QAM	Huawei, HiSilicon

R1-1804258	Discussions on support of 64QAM for LTE V2X sidelink	Nokia, Nokia Shanghai Bell

R1-1804320	High order modulation in V2X	Samsung

R1-1804512	Discussion on 64QAM support	LG Electronics

R1-1804632	Supporting 64QAM on PC5	Ericsson

R1-1804688	On demodulation enhancements for LTE V2V sidelink communication	Intel Corporation

#### Feasibility and gain of PC5 operation with Transmit Diversity

R1-1804259	Transmit diversity schemes and evaluations for V2X sidelink	Nokia, Nokia Shanghai Bell

R1-1804689	On support of transmit diversity schemes for LTE V2V sidelink communication	Intel Corporation

Agreement:

- Transmit diversity is not specified in RAN1 specifications

  - Inform RAN4 that RAN1 assumes the use of SD-CDD when >1 antenna. It is up to RAN4 to discuss and/or specify any performance related requirements (including bounds for the delay)

Note: it is assumed that there is a UE capability to indicate the support of SD-CDD

Send LS to RAN4, (and cc RAN2)

R1-1805334	[DRAFT] LS on small-delay CDD for V2X Phase 2	Huawei

Decision: The document is endorsed and final LS is approved in R1-1805619.

Agreement:

The following working assumption is reverted

- Assuming the previous WA of introducing non-transparent transmit diversity is confirmed, for two-port non-transparent transmit diversity for PSSCH, downselect option 1 as WA among the following candidate schemes

  - Working assumption: Option 1: SFBC-based scheme (including PAPR preserving)

    - FFS whether to apply slot-level PVS

  - Option 2: STBC-based (including half symbol)

##### Transmit diversity solutions

For both PSSCH and PSCCH

R1-1803726	Discussion on Tx diversity schemes in PC5	CATT

R1-1803892	Discussion on transmit diversity for PC5	ZTE, Sanechips

R1-1803985	Transmit diversity scheme in eV2X	OPPO

R1-1804038	Low PAPR SFBC for V2X transmit diversity	Mitsubishi Electric RCE

R1-1804132	Transmit diversity solutions for PSSCH and PSCCH	Huawei, HiSilicon

R1-1804321	Transmit diversity schemes for PSSCH	Samsung

R1-1804513	Discussion on transparent transmit diversity	LG Electronics

R1-1804633	Transmit diversity solutions for Rel-15 PSCCH and PSSCH transmissions	Ericsson

R1-1804909	Transmit Diversity for V2X Phase 2	Qualcomm Incorporated

R1-1805023	Transmission diversity solutions	NTT DOCOMO, INC.

##### Evaluation results

For both PSSCH and PSCCH

R1-1803893	Evaluation results of TxD	ZTE, Sanechips

R1-1804040	Low PAPR SFBC evaluations for PSSCH	Mitsubishi Electric RCE

R1-1804131	Performance evaluation of transmit diversity for eV2X	Huawei, HiSilicon

R1-1804634	Evaluation of transmit diversity for V2X	Ericsson

##### Other

R1-1804635	DMRS design for two-port PSSCH transmission	Ericsson

#### Resource pool sharing between mode-3 and mode-4 users

R1-1805326	Summary of offline discussion for resource pool sharing between Mode-3 UE and Mode-4 UE	NTT DOCOMO

R1-1805392	Summary of offline discussion for resource pool sharing between Mode-3 UE and Mode-4 UE	NTT DOCOMO

R1-1805391	WF on Reporting Mechanism for Resource Pool Sharing by Mode-3/Mode-4 UEs	ZTE, Sanechips, Fraunhofer HHI, InterDigital, ITRI

R1-1805390	Resource Pool Sharing (Option 1-4)	OPPO

R1-1803727	Discussion on resource pool sharing between mode 3 and mode 4	CATT

R1-1803895	Resource pool sharing between mode 3 and mode 4	ZTE , Sanechips

R1-1803986	Discussion of resource pool sharing between mode 3 and mode 4	OPPO

R1-1804020	Discussion on Resource Pool Sharing between Mode 3 and Mode 4 in V2X commuincation	ITRI

R1-1804127	Solutions for Rel-14 and Rel-15 UE Pool sharing	Huawei, HiSilicon

R1-1804205	Discussion on Pool sharing for V2X Phase 2	Lenovo, Motorola Mobility

R1-1804233	Sharing Mode 3 and Mode 4 resource pools	NEC

R1-1804260	On Resource pool sharing between mode-3 and mode-4	Nokia, Nokia Shanghai Bell

R1-1804322	Resource pool sharing between mode 3 and mode 4 Ues	Samsung

R1-1804514	Discussion on resource pool sharing between UEs using mode 3 and 4	LG Electronics

R1-1804628	Discussion on resource pool sharing between UEs in mode 3 and UEs in mode 4	Panasonic

R1-1804636	Resource pool sharing between mode 3 and mode 4 UEs	Ericsson

R1-1804679	Open Issues on Mode 3 and Mode 4 Pool Sharing	InterDigital, Inc.

R1-1804690	Sidelink resource pool sharing for eNB-controlled and UE-autonomous V2V transmission modes	Intel Corporation

R1-1804910	Resource pool sharing between Mode 3 and Mode 4	Qualcomm Incorporated

R1-1805024	Resource pool sharing between Mode-3 UE and Mode-4 UE	NTT DOCOMO, INC.

R1-1805248	Resource Pool Sharing between V2X Mode 3 and Mode 4 UEs	Fraunhofer HHI

#### Maximum time reduction between packet arrival at layer 1 and resource selection for transmission

R1-1805283	Summary on Maximum time reduction between packet arrival at layer 1 and resource selection for transmission	CATT

Further revised in R1-1805395.

Agreement

- The minimum (pre-)configurable T2min is [10]ms.

- The maximum (pre-)configurable T2min is 20ms.

- The determination of T2min

  - For each PPPP, the T2min is (pre-)configured by RRC.

Note: The actual value of T2 (>=T2min) is left to UE implementation.

R1-1803728	Discussion on latency reduction between packet arrival and resource selection	CATT

R1-1803896	Consideration for maximum time reduction between packet arrival at layer 1 and resource selection for transmission	ZTE, Sanechips

R1-1803987	Latency reduction for eV2X	OPPO

R1-1804021	Considerations of Latency Reduction between Packet Arrival and Resource Selection	ITRI

R1-1804128	Discussion on latency reduction for eV2X	Huawei, HiSilicon

R1-1804206	Discussion on Latency Reduction for V2X Phase 2	Lenovo, Motorola Mobility

R1-1804261	Discussion on latency reduction for eV2X	Nokia, Nokia Shanghai Bell

R1-1804323	Support of shorter latency in V2X	Samsung

R1-1804515	Discussion on maximum time reduction between packet arrival and selected transmission resource	LG Electronics

R1-1804637	Reducing time-to-transmit for V2X	Ericsson

R1-1804691	Resource selection latency reduction for LTE V2V sidelink communication	Intel Corporation

R1-1804911	Reduction of time between packet arrival and transmission	Qualcomm Incorporated

R1-1805025	Reducing the maximum time between packet arrival and selected resource for data transmission	NTT DOCOMO, INC.

#### Other

R1-1804627	Introducing sTTI to V2X mode 3	Panasonic

### Even further enhanced MTC for LTE - WID in RP-172811

R1-1805427	Chairman's notes of AI 6.2.6 Even further enhanced MTC for LTE	Ad-hoc chair (NTT DOCOMO)

Decision: The document is endorsed, content incorporated below.

Friday

Rapporteur to prepare the list of agreements by April 23rd

Editors to prepare CRs incorporating all the agreements by 5/4 (for 211, based on x3862/x5173; for 212, based on x4149), to be commented and endorsed by 5/11

R1-1803862	Introduction of PUSCH sub-PRB allocation	Ericsson

R1-1804119	Draft RRC parameter list for L1 configuration of Rel-15 LTE-MTC	Ericsson

Email discussion including the agreements at RAN1#92bis meeting by 4/27(Fri)

R1-1804149	Introduction of FeMTC into 36.212	Huawei, HiSilicon

R1-1805173	Draft CR 36.211, Introduction of increased PUSCH spectral efficiency	Ericsson

#### Reduced system acquisition time

Physical layer aspects of improved cell search and/or system information acquisition performance

R1-1805306	Feature summary for Reduced system acquisition time in MTC	Ericsson

Agreements

- Resynchronization signal (RSS) configuration information is provided in SIB.

- The frequency location[s] of the RSS are configurable. Details are FFS.

- The UE can assume that the transmission of RSS in subframes (2n, 2n+1) is from the same antenna port at least when TxD is used.

  - FFS if TX diversity is signaled to UE.

- The periodicity of the first subframe of RSS is configurable to 160, 320, 640 and 1280 ms.

  - In addition, time offset is configurable

    - FFS: Details

- The RSS provides information about at least cell id.

Working assumption

- The RSS bandwidth is 2 PRBs.

  - FFS whether the 2PRB sequence is repetitions in the frequency domain of 1 PRB sequence.

  - Confirmed when analysis shows enough sequences with good cross correlation, without significantly increasing the time duration compared to 6 PRBs, can be found.

Agreement

- The RSS should be designed taking into account the correlation properties with other LTE signals.

- The RSS transmission length is configurable in SIB

- The RSS base sequence duration is down selected from 1 and 11 symbols.

R1-1805385	WF on Reduced system acquisition time for MTC	Ericsson, Nokia, NSB, Sony, Intel, Sierra Wireless

R1-1803883	Cell search and system information acquisition improvements in eFeMTC	Huawei, HiSilicon

R1-1804120	Reduced system acquisition time for MTC	Ericsson

R1-1804174	Remaining issues on system acquisition time reduction for MTC	ZTE, SaneChips

R1-1804238	Reducing system acquisition time for efeMTC	Nokia, Nokia Shanghai Bell

R1-1804324	Discussion on enhanced synchronization signals and resynchronization signals	Samsung

R1-1804471	Re-Sync Signal Design Considerations	Sierra Wireless, S.A.

R1-1804602	Considerations on eSS for efeMTC	Sony

R1-1804692	System acquisition time reduction for efeMTC	Intel Corporation

R1-1804912	Reduced system acquisition time	Qualcomm Incorporated

#### Early data transmission

Physical layer aspects of support for data transmission during the random access procedure

R1-1805287	Feature lead summary of EDT in eMTC	Huawei, HiSilicon

Agreement

For EDT scheduled by RAR grant,

- For CE Mode A, 2–6 PRBs are supported with 4 bits resource allocation field.

  - Whether to support 1 PRB can be reconsidered if there is an available spare bit to expand resource allocation field.

- For CE Mode B, 1-2 PRBs are supported with at least 3 bits resource allocation field.

  - Resource allocation may or may not be modified, picking at least 8 resource allocations.

  - Whether to expand resource allocation can be reconsidered if there is an available spare bit to expand resource allocation field.

Agreement

- The 8 maximum TBS values in CE mode A (i.e. PRACH levels 0 and 1) are {328 408 504 600 712 808 936 1000}.

  - Only values of NPRB and values of TBS in legacy Rel-13 PUSCH table can be used for EDT

- The 8 maximum TBS values in CE mode B (i.e. PRACH levels 2 and 3) are {328 408 456 504 600 712 808 936}.

  - Only values of NPRB and values of TBS in legacy Rel-13 PUSCH table can be used for EDT

- For each of the 8 maximum TBS values

  - The number of repetitions changes depending on the actual TBS, potentially including numbers of repetitions which are not included in legacy PUSCH repetition numbers (FFS how)

Agreement

For EDT Msg. 3 transmission, 16QAM modulation is not supported.

Agreement

- The use of TBS smaller than the maximum configured is configured per CE level in SIB.

- Per cell, in the below, Ti < Ti+1, eNB can configure that the UE chooses from:

  - When there are 4 permitted actual transmitted TBS {T1, T2, T3, T4}

    - T2 or T4

    - T1, or T2, or T3, or T4

  - When there are 3 permitted actual transmitted TBS {T1, T2, T3}

    - T2 or T3

    - T1 or T2 or T3

  - When there are 2 permitted actual transmitted TBS {T1, T2}

    - T1 or T2

Agreement

The possible smaller TBS values are predefined from the configured maximum TBS and the configured maximum number of blind decodes.

Email discussion to decide the exact table by 4/27 – Yuzheng (Huawei)

Agreement

Use the ‘R’ bit in RAR to differentiate EDT and non-EDT.

R1-1804121	Early data transmission for MTC	Ericsson

R1-1804191	On early data transmission for eMTC	ZTE, SaneChips

R1-1804239	Data transmission during random access procedure	Nokia, Nokia Shanghai Bell

R1-1804325	Early data transmission for eMTC	Samsung

R1-1804474	EDT Design Consideration	Sierra Wireless, S.A.

R1-1804516	Data transmission during random access procedure in MTC	LG Electronics

R1-1804603	On Early Data Transmission over Message 3	Sony

R1-1804693	Early data transmission for efeMTC	Intel Corporation

R1-1804872	Early data transmission for MTC	Sharp

R1-1804913	Physical layer aspects of early data transmission	Qualcomm Incorporated

R1-1803884	Early data transmission for eFeMTC	Huawei, HiSilicon

#### Downlink channel power efficiency

Physical signal/channel that can be efficiently decoded or detected prior to decoding MPDCCH/PDSCH

R1-1805384	Remaining issues of 6.2.6.3 Downlink channel power efficiency	Qualcomm

Agreement

At least WUS/DTX is supported at least for paging for RRC_IDLE UEs

Agreement

Confirm Working assumption

- WUS transmission relative to associated PO of subgroup of UEs is aligned to the start of the configured maximum duration of WUS.

  - Note: the above applies to at least the case where the gap is large enough for scheduling UE

  - Note: the above does not imply that subgroup of UEs is introduced and that subgroup is TDM.

Agreement

- [2] bits are used to indicate the scaling factors between maximum duration of WUS and Rmax associated with type 1 CSS

Agreement

- WUS can be time-varying from subframe to subframe.

- WUS signal is designed based on the following information

  - [Full] Cell ID information

  - UE group ID (if introduced)

  - time information of the starting subframe of the WUS or PO (Paging Occasion)

  - FFS: (part of) the SFN information

Agreement

- WUS subframe is postponed

  - when colliding with SIB1-BR PRBs

  - in non-BL/CE subframes.

  - FFS: when colliding with the PRBs that carry SIs other than SIB1.

- Note: “Postpone” means the corresponding subframes are not counted as configured maximum WUS transmission duration and actual WUS transmission duration.

- Note: This does not imply that the minimum gap between the end of actual WUS duration and the first associated PO is reduced

Agreement

- The gap between the end of configured maximum WUS duration and the first associated PO is equal or larger than the minimum value, which is implicitly or explicitly configured and is an absolute number of subframes

  - Note: UE can assume CRS between the end of configured maximum WUS duration and the first associated PO

  - Note: The gap should be large enough to warm up tracking loops.

FFS: whether to define UE capability for wake-up time

R1-1803804	Remaining aspects on wake-up signals for efeMTC	vivo

R1-1803885	On 'wake-up signal' for eFeMTC	Huawei, HiSilicon

R1-1803995	On downlink channel power efficiency for efeMTC	OPPO

R1-1804122	Downlink channel power efficiency for MTC	Ericsson

R1-1804189	Power consumption reduction for physical channels for MTC	ZTE, SaneChips

R1-1804243	Wake-up signal for efeMTC	Nokia, Nokia Shanghai Bell

R1-1804326	DL power consumption reduction for eMTC	Samsung

R1-1804475	WUS Design Considerations	Sierra Wireless, S.A.

R1-1804517	Discussion on wake up signal in MTC	LG Electronics

R1-1804604	Considerations of using WUS in efeMTC	Sony

R1-1804694	Power saving signal for efeMTC	Intel Corporation

R1-1804914	Efficient monitoring of DL control channels	Qualcomm Incorporated

R1-1805308	Summary of 6.2.6.3 Downlink channel power efficiency	Qualcomm

#### Uplink HARQ-ACK feedback

Design of MPDCCH-based explicit HARQ-ACK feedback

R1-1805311	Summary of Uplink HARQ-ACK Feedback for MTC	ZTE, SaneChips

Agreement

The working assumption from RAN1#91 is replaced by the following agreement

- One unused DCI state in each CE mode is used for indicating:

  - Early termination of any ongoing PUSCH transmission (without early termination of MPDCCH monitoring except for the case of acknowledging RRC Connection Release, for which MPDCCH monitoring is terminated)

Agreement

For the UE-specific UL HARQ-ACK feedback, ACK is indicated by DCI format 6-0A and 6-0B for CE mode A and CE mode B, respectively, by setting all fields except the ones below to default fixed value

- ‘Flag for format 6-0A/6-1A differentiation’ in CE mode A and ‘Flag for format 6-0B/6-1B differentiation’ in CE mode B

- ‘DCI subframe repetition number’ field

- ‘Resource block assignment’ field in DCI format 6-0A and ‘MCS’ field in DCI format 6-0B

  - Values; TBD

Note Exact default fixed value is up to specification editor

R1-1803886	Further considerations on HARQ-ACK feedback for PUSCH in eFeMTC	Huawei, HiSilicon

R1-1804123	Uplink HARQ-ACK feedback for MTC	Ericsson

R1-1804175	Remaining issues on UL HARQ-ACK feedback for MTC	ZTE, SaneChips

R1-1804200	Remaining issue for uplink HARQ-ACK feedback in efeMTC	Lenovo, Motorola Mobility

R1-1804244	Uplink HARQ-ACK feedback in efeMTC	Nokia, Nokia Shanghai Bell

R1-1804327	Group DCI for uplink HARQ-ACK feedback	Samsung

R1-1804518	Discussion on early termination of uplink repetitions for MTC	LG Electronics

R1-1804605	Remaining issues in explicit uplink HARQ-ACK feedback for efeMTC	Sony

R1-1804695	HARQ-ACK feedback for efeMTC UL transmission	Intel Corporation

R1-1804915	Uplink HARQ-ACK feedback	Qualcomm Incorporated

R1-1805026	Views on UL HARQ-ACK feedback design	NTT DOCOMO, INC.

#### PUSCH sub-PRB allocation

Design of PUSCH sub-PRB allocation

R1-1805305	efeMTC Key Issue Summary for PUSCH Sub-PRB Allocation	Sierra Wireless

Agreement

Each DMRS symbol is length-2 BPSK with DFT-S-OFDM.

Agreement

- For CE Mode B, only 2 and 4 RUs can be allocated.

- For CE Mode A, 1, 2 and 4 RUs can be allocated.

Agreement

- For FDD, RU Size of 2 of 3 sub-carriers pi/2 BPSK option = 8 ms for CE Mode A and CE Mode B

Agreement

For sub-PRB transmissions in TDD, down-select between the following at RAN1#93

- Alt. 1: In TDD, the RU length depends on TDD UL/DL configuration.

- Alt. 2: In TDD, the same RU lengths supported in FDD mode are re-used.

Agreement

For subPRB allocation, the maximum transmission power is used in CE Mode B and for power control in CE Mode A, MPUSCH,c(i) ={1/6, 1/4, 1/2} for 2-subcarrier, 3-subcarrier and 6-subcarrrier allocation, respectively and for PRB level allocation, there are no changes to transmit power control.

Agreement

Cyclic repetition is not supported for sub-PRB allocation for CE Mode A

- FFS: For CE Mode B

Agreement

The Sub-PRB DCI design for Mode A and B should not increase the DCI size by more than 3 bits

R1-1805410	PUSCH Sub-PRB DCI Possible Agreements	Sierra Wireless, Nokia, NSB, Huawei, HiSilicon, Qualcomm, Sony, Sharp, Samsung, Sane Chips, ZTE

Agreement

- For sub-PRB allocation in CE Mode B, the PRB location(s) within the [system bandwidth or narrowband] is configured by RRC.

Agreement

- For sub-PRB allocation in CE Mode A, there is no RRC configuration needed to indicate the

  - PRB location in the system bandwidth

  - Sub-PRB location within PRB

  - # of sub-carriers

  - # of RUs

  - MCS Index

Agreement

- The Sub-PRB DCI design will not limit scheduling options for Full-PRB legacy allocations for CE Mode A and B

R1-1803887	On Sub-RB resource allocation for MTC PUSCH	Huawei, HiSilicon

R1-1804124	PUSCH sub-PRB allocation for MTC	Ericsson

R1-1804176	Remaining issues on PUSCH sub-PRB allocation for MTC	ZTE, SaneChips

R1-1804245	Design of PUSCH Sub-PRB Allocation	Nokia, Nokia Shanghai Bell

R1-1804328	Discussion on sub-PRB allocation for eFeMTC	Samsung

R1-1804472	Sub-PRB Design Considerations	Sierra Wireless, S.A.

R1-1804606	Remaining issues in sub-PRB transmission	Sony

R1-1804696	Design of sub-PRB PUSCH for efeMTC	Intel Corporation

R1-1804873	Remaining issues for PUSCH sub-PRB allocation	Sharp

R1-1804916	Sub-PRB allocation for eFeMTC	Qualcomm Incorporated

R1-1805027	Views on PUSCH sub-PRB allocation for MTC	NTT DOCOMO, INC.

#### Flexible PDSCH/PUSCH starting PRB

Design of more flexible starting PRB for PDSCH/PUSCH resource allocation in connected mode

R1-1805272	Summary on Flexible PDSCH/PUSCH starting PRB	Samsung

Agreement

- At least for BL/CE UE configured without other Rel-14/15 features, DCI size after padding is not increased in order to support flexible starting PRB.

  - Note: For PUSCH and PDSCH, the solution may or may not be the same

  - Note: The solution may or may not be the same for FDD and TDD

R1-1803888	On more flexible starting PRB for PDSCH/PUSCH resource allocation	Huawei, HiSilicon

R1-1804125	Flexible PDSCH/PUSCH starting PRB for MTC	Ericsson

R1-1804177	Discussion on Flexible starting PRB of PUSCH/PDSCH for MTC	ZTE, SaneChips

R1-1804246	Support for flexible PDSCH/PUSCH starting PRB	Nokia, Nokia Shanghai Bell

R1-1804329	Discussion on Flexible PDSCH/PUSCH starting PRB	Samsung

R1-1804607	Flexible PDSCH/PUSCH Resource Allocation for efeMTC	Sony

R1-1805286	On more flexible PDSCH/PUSCH resource allocation	Sharp	(rev of R1-1804874)

R1-1804917	Flexible allocation for PDSCH and PUSCH	Qualcomm Incorporated

#### Other

Including any remaining details of PDSCH 64QAM support

R1-1805341	Summary of section 6.2.6.7 on CQI for 64QAM for efeMTC	Sony

Agreement

- Confirm the RAN1 working assumption that eNodeB can optionally configure the UE to use the option B CQI table non-64QAM capable UE if the UE supports option B CQI table and that in this case only QPSK and 16QAM entries are reported by UE.

R1-1804126	Remaining details of PDSCH 64QAM support for MTC	Ericsson

R1-1804247	Remaining issue on supporting DL 64QAM for efeMTC	Nokia, Nokia Shanghai Bell

R1-1804608	Use of option B CQI table by non-64QAM capable UEs	Sony

### Further enhancements of NB-IoT - WID in RP-172063.

R1-1805428	Chairman's notes of AI 6.2.7 Further enhancements of NB-IoT	Ad-hoc chair (NTT DOCOMO)

Decision: The document is endorsed, content incorporated below.

Friday

Rapporteur to prepare the list of agreements by April 23rd

Editors to prepare CRs incorporating all the agreements by 5/4 (for 211, based on 3863; for 212, based on x4148), to be commented and endorsed by 5/11

R1-1803863	Introduction Rel-15 Further NB-IoT enhancements	Ericsson

R1-1803882	List of L1 parameters for further enhancements of NB-IoT	Huawei, HiSilicon

Email discussion including the agreements at RAN1#92bis meeting by 4/27(Fri)

R1-1804148	Introduction of further enhancements of NB-IoT	Huawei, HiSilicon

R1-1805174	Introduction Rel-15 Further NB-IoT enhancements	Ericsson

#### Latency and power consumption reduction

##### Power consumption reduction for paging and connected-mode DRX

R1-1804248	Functions of wake-up signal	Nokia, Nokia Shanghai Bell

###### Wake-up signal functions

R1-1805288	Feature lead summary on functions of power saving signal	Huawei, HiSilicon

Agreement

At least WUS/DTX is supported at least for paging for RRC_IDLE UEs

R1-1803866	On functions of power saving signal	Huawei, HiSilicon

R1-1804138	Wake Up Signal Functionsfor NB-IoT	MediaTek Inc.

R1-1804152	Wake-up signal functions for NB-IoT	Ericsson

R1-1804186	Wake up signal function for NB-IoT	ZTE, SaneChips

R1-1804519	Discussion on wake up signal function in NB-IoT	LG Electronics

R1-1804697	The function scope of wake-up signal for feNB-IoT	Intel Corporation

R1-1804918	Wake-up signal functions	Qualcomm Incorporated

###### Wake-up signal configurations and procedures

R1-1805289	Feature lead summary of Wake-up signal configurations and procedures in NB-IoT	Huawei, HiSilicon

Agreement

Confirm the following working assumption

- WUS transmission relative to associated PO of subgroup of UEs is aligned to the start of the configured maximum duration of WUS.

- Note: the above applies to at least the case where the gap is large enough for scheduling UE

- Note: the above does not imply that subgroup of UEs is introduced and that subgroup is TDM

Agreement

- 3 bits are used to indicate the scaling factors between maximum duration of WUS and Rmax associated with type 1 CSS

Note: Ericsson, Samsung, and Sony consider 3bits are waste.

Agreement

- The gap between the end of configured maximum WUS duration and the first associated PO is equal or larger than the minimum value which is implicitly or explicitly configured and is an absolute number of subfames

  - There are at least 10 valid subframes between the end of configured maximum WUS duration and the first associated PO.

- FFS: whether to define UE capability for wake-up time

Agreement

WUS is postponed in subframes that are not NB-IoT DL subframes and is not transmitted [FFS dropped/postponed] in the subframes that carries SIs other than SIB1.

- Note: “Postpone” means the corresponding subframes are not counted as configured maximum WUS transmission duration and actual WUS transmission duration.

- Note: This does not imply that the minimum gap between the end of actual WUS duration and the first associated PO is reduced

R1-1803805	Remaining aspects on wake-up signals for feNB-IoT	vivo

R1-1803867	On configurations and procedures of power saving signal	Huawei, HiSilicon

R1-1804139	Wake Up Signal Configuration for NB-IoT	MediaTek Inc.

R1-1804153	Wake-up signal configurations and procedures for NB-IoT	Ericsson

R1-1804187	Remaining issues on wake up signal configuration for NB-IoT	ZTE, SaneChips

R1-1804249	Wake-up signal configurations and procedures	Nokia, Nokia Shanghai Bell

R1-1804330	Discussion on Wake up signal configuraion	Samsung

R1-1804520	Discussion on wake up signal configurations and procedures in NB-IoT	LG Electronics

R1-1804698	Configurations of wake-up signal for feNB-IoT	Intel Corporation

R1-1804919	Wake-up signal configurations and procedures	Qualcomm Incorporated

###### Detailed design of wake-up signal

R1-1805290	Feature lead summary on detailed design of Wake-up signal in NB-IoT	Huawei, HiSilicon

Agreement

- WUS can be time-varying from subframe to subframe.

- Besides information on cell ID, WUS signal is designed based on the following information:

  - UE group ID (if introduced)

  - time information of the starting subframe of the WUS or PO (Paging Occasion)

  - FFS: (part of) the SFN information

Agreement

- When generating power saving signal in a subframe where NRS is assumed to be transmitted, the power saving signal in the subframes shall be generated and mapped in the same way as in other valid subframes where NRS is not assumed to be transmitted;

- WUS is punctured in RE-level by NRSs.

R1-1805320	WF on WUS Sequence Design	Qualcomm, Samsung, LGE, ZTE, SaneChips, Ericsson, MediaTek

Agreement

WUS signal in a subframe is as follows,

where LZC = 131 for inband mode (FFS for SA and GB modes)

- FFS RE-level cover codes/RE-level scrambling sequence c(m) using

  - Hadamard codes

  - Gold sequences

  - M sequences

- FFS phase shift

- FFS detailed design for time-varying

- 11 symbols for inband mode and 14 symbols for SA and GB modes

  - Strive toward as much as possible commonality between SA/GB and inband

R1-1803868	On detailed design and evaluations of power saving signal	Huawei, HiSilicon

R1-1803996	On wake-up signal design	OPPO

R1-1804140	Wake Up Signal Design for NB-IoT	MediaTek Inc.

R1-1804154	Detailed design of wake-up signal for NB-IoT	Ericsson

R1-1804188	Details design of wake up signal for NB-IoT	ZTE, SaneChips

R1-1804331	Wake up signal Design	Samsung

R1-1804521	Wake up signal design in NB-IoT	LG Electronics

R1-1804920	Wake-up signal design	Qualcomm Incorporated

R1-1805343	Detailed design of wake-up signal for NB-IoT	Ericsson

##### Data transmission during the random access procedure

R1-1805291	Feature lead summary of EDT during RACH in feNB-IoT	Huawei, HiSilicon

Agreement

- The 8 maximum TBS is {1000, 936, 808, 680, 584, 504, 408, 328}.

  - Only values of NRU and values of TBS in legacy Rel-13 NPUSCH table can be used for EDT

  - For each of the 8 maximum TBS values

      - For each of the Rel-13 reserved ‘MCS index’ states there is one number of RUs

        - The number of repetitions changes depending on the actual TBS, potentially including numbers of repetitions which are not included in legacy NPUSCH repetition numbers (FFS how)

          - Potential new numbers of repetitions may include multiple of 4 or 8

- The 8 maximum TBS values are: 1000, 936, 808, 680, 584, 504, 408, 328 bits

  - For 1000 bits max TBS: I_RU = 3, 4, 5, 6, 7

  - For 936 bits max TBS: I_RU = 3, 4, 5, 6, 7

  - For 808 bits max TBS: I_RU = 3, 4, 5, 6, 7

  - For 680 bits max TBS: I_RU = 2, 3, 4, 6, 7

  - For 584 bits max TBS: I_RU = 2, 3, 4, 5, 6

  - For 504 bits max TBS: I_RU = 2, 3 4, 5, 6

  - For 408 bits max TBS: I_RU = 2, 3, 4, 5, 6

  - For 328 bits max TBS: I_RU = 2, 3, 4, 5, 6

- The 3 legacy MCS indices are used for fallback to non-EDT transmission;

Agreement

- The use of TBS smaller than the maximum configured is configured per CE level in SIB.

- Per cell, in the below, Ti < Ti+1, eNB can configure that the UE chooses from:

  - When there are 4 permitted actual transmitted TBS {T1, T2, T3, T4}

    - T2 or T4

    - T1, or T2, or T3, or T4

  - When there are 3 permitted actual transmitted TBS {T1, T2, T3}

    - T2 or T3

    - T1 or T2 or T3

  - When there are 2 permitted actual transmitted TBS {T1, T2}

    - T1 or T2

Agreement

- Ti with the maximum value of i is the maximum TBS in SIB.

Agreement

- Up to 4 TBS values are defined based on only the maximum broadcast TBS

R1-1805409	Updated Feature lead summary of EDT during RACH in feNB-IoT	Huawei, HiSilicon

Agreement (including post RAN1#92b clarification over email)

- The following table is agreed:

|  | 328 | 408 | 504 | 584 | 680 | 808 | 936 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 328 | 328 | 328 | 328 | 328 | 328 | 328 | 328 |
| T2 |  | 408 | 408 | 408 | 456 | 504 | 504 | 536 |
| T3 |  |  | 504 | 504 | 584 | 680 | 712 | 776 |
| T4 |  |  |  | 584 | 680 | 808 | 936 | 1000 |

Conclusion

Further study aspects related to Msg. 3 retransmission until RAN1#93 meeting

R1-1803871	Early data transmission in RACH for NB-IoT	Huawei, HiSilicon

R1-1804143	Early Data Transmission TBS Determination	MediaTek Inc.

R1-1804155	Data transmission during random access procedure for NB-IoT	Ericsson

R1-1804190	On early data transmission for NB-IoT	ZTE, SaneChips

R1-1804251	Data transmission during random access procedure	Nokia, Nokia Shanghai Bell

R1-1804332	Early data transmission for NB-IOT	Samsung

R1-1804522	Data transmission during random access procedure in NB-IoT	LG Electronics

R1-1804699	Early data transmission for feNB-IoT	Intel Corporation

R1-1804875	Data transmission during random access procedure for NB-IoT	Sharp

R1-1804921	Physical layer aspects of data transmission during random access procedure	Qualcomm Incorporated

R1-1805132	TBS Determination for Early Data Transmission	III

##### Physical layer scheduling request

R1-1805292	Feature lead summary of physical layer SR in feNB-IoT	Huawei, HiSilicon

Agreement

- If the working assumption is confirmed and when SR is transmitted without HARQ-ACK, it is transmitted in NPRACH resource using NPRACH based signal.

Agreement

- If the working assumption is confirmed, UE transmits the physical layer SR at the first opportunity unless there is a collision with another physical layer transmission/reception:

  - Collision between SR without HARQ-ACK resource and NPDSCH - SR is not transmitted, and remains pending

    - FFS: Collision with NPDCCH search space

R1-1803869	Design for physical layer scheduling request	Huawei, HiSilicon

R1-1804141	Physical Layer Scheduling Request	MediaTek Inc.

R1-1804156	Physical layer scheduling request for NB-IoT	Ericsson

R1-1804178	Details on physical layer SR without HARQ-ACK for NB-IoT	ZTE, SaneChips

R1-1804250	Considerations for design of wake-up signal	Nokia, Nokia Shanghai Bell

R1-1804252	Design of physical layer scheduling request	Nokia, Nokia Shanghai Bell

R1-1804333	Discussion on scheduling request for NB IoT	Samsung

R1-1804523	Discussion on Scheduling request in NB-IoT	LG Electronics

R1-1804700	Scheduling request for feNB-IoT	Intel Corporation

R1-1804922	Physical layer scheduling request	Qualcomm Incorporated

##### Semi-persistent scheduling

R1-1805293	Feature lead summary of SPS in NB-IoT	Huawei, HiSilicon

R1-1803870	On support of semi-persistent scheduling	Huawei, HiSilicon

R1-1804142	L1 Scheduling Request by  Semi Persistent Scheduling	MediaTek Inc.

R1-1804157	Semi-persistent scheduling for NB-IoT	Ericsson

R1-1804179	Considerations on physical layer aspects on SPS in NB-IoT	ZTE, SaneChips

R1-1804253	Support for semi-persistent scheduling in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1804334	New SPS mechanism for power saving	Samsung

R1-1804524	SPS support in NB-IoT	LG Electronics

R1-1804923	Support of SPS	Qualcomm Incorporated

R1-1805028	Views on SPS activation and deactivation mechanism	NTT DOCOMO, INC.

R1-1805324	WF on SPS for NB-IoT in Rel-15	NTT DOCOMO

#### Reduced system acquisition time

R1-1805309	Summary of 6.2.7.2 Reduced system acquisition time	Qualcomm

##### Cell search

R1-1803877	Reduction of NB-IoT synchronization time	Huawei, HiSilicon

R1-1804158	Cell search time reduction for NB-IoT	Ericsson

R1-1804180	Cell search enhancement for NB-IoT	ZTE, SaneChips

R1-1804254	Reducing cell search time for feNB-IoT	Nokia, Nokia Shanghai Bell

R1-1804924	Enhancements to cell search	Qualcomm Incorporated

##### System Information

R1-1803878	Reduction of NB-IoT system information acquisition time	Huawei, HiSilicon

R1-1804159	System information acquisition time reduction for NB-IoT	Ericsson

R1-1804181	Remaining issues on system information acquisition improvement for NB-IoT	ZTE, SaneChips

R1-1804255	Reducing system acquisition time for feNB-IoT	Nokia, Nokia Shanghai Bell

R1-1804925	Enhancements to system information acquisiton	Qualcomm Incorporated

#### Narrowband measurement accuracy improvements

R1-1805346	Feature lead summary Narrowband measurement accuracy improvements	Ericsson

Agreement

The eNB signals the number of consecutive NSSS occasions, with possible values of 2, 4, [8], that use different precoders for NSSS transmission, and the UE may take such information into account when using NSSS for measurement. If eNB does not signal this, the UE makes no assumptions of the antenna port(s), i.e., “the UE shall not assume that the transmissions of the narrowband secondary synchronization signal in a given subframe use the same antenna port, or ports, as the narrowband secondary synchronization signal in any other subframe”. The signalling does not mandate how, when, and over how many subframes the UE should measure the NSSS, and the measurement accuracy should not assume the UE measures consecutive NSSS occasions. LS RAN4 about the agreements.

Agreement

LS RAN4 (CC RAN2) on whether and if so, how the update of NRSSI measurement in NRSRQ is needed when NSSS is used for NRSRQ measurements

Send LS related to the above agreement to RAN4 (CC: RAN2) – Yutao (Ericsson)

R1-1805419	Draft LS on Narrowband measurement accuracy improvements	Ericsson

Decision: The LS is endorsed and final LS is approved in R1-1805434.

R1-1803875	Remaining details of NB-IoT measurements improvement	Huawei, HiSilicon

R1-1804160	Narrowband measurement accuracy improvements for NB-IoT	Ericsson

R1-1804264	Measurement accuracy improvement in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1804926	Improvement of PHY measurements	Qualcomm Incorporated

#### NB-IoT small cell support

R1-1805345	Feature lead summary NB-IoT small cell support	Ericsson

R1-1803876	On the support of NB-IoT small cell	Huawei, HiSilicon

R1-1804161	NB-IoT small cell support	Ericsson

R1-1804192	On small cell support for NB-IoT	ZTE, SaneChips

R1-1804265	Small cell support in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1804335	Power control for extended coverage UE	Samsung

R1-1804525	UL power control enhancement for NB-IoT small cell	LG Electronics

#### NPRACH reliability and range enhancements

R1-1805344	Feature lead summary NPRACH reliability and range enhancements	Ericsson

##### Reliability enhancements for NPRACH

Agreement

Confirm the following working assumption made in RAN1#92 with the Note.

- Sharing the same NPRACH resources as Rel-13 NPRACH formats, with symbol or symbol-group level scrambling; maintaining feasibility of FFT processing and orthogonality of preambles on different tones.

- Down-select the following alternatives at RAN1#92bis:

  - Symbol level scrambling

  - Symbol group scrambling

Note: Qualcomm has storng concerns about the above agreement and thinks it is not based on technical merits.

R1-1803880	On NPRACH false alarm reduction due to inter-cell interference	Huawei, HiSilicon

R1-1804162	NPRACH false alarm reduction for NB-IoT	Ericsson

R1-1804194	NPRACH reliability enhancement for NB-IoT	ZTE, SaneChips

R1-1804266	NPRACH reliability enhancement in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1804336	Reliability enhancements for NPRACH	Samsung

R1-1804526	Remaining issues on reliability enhancements for NPRACH	LG Electronics

R1-1805297	NPRACH Reliability Enhancement	Qualcomm Incorporated	(rev of R1-1804928)

##### Range enhancements for NPRACH

Agreement

- Configurable NPRACH bandwidth is integer multiple of 45 kHz.

Agreement

- For Rel-15 UEs, in case NPUSCH transmission collides with any configured NPRACH resources in the cell, the NPUSCH transmission is postponed.

Agreement

- For one NPRACH symbol group for subcarrier spacing of 1.25 kHz, one NPRACH symbol group consists of CP+3 Symbols

- FFS the number of symbol groups per NPRACH preamble repetition.

Agreement

- For frequency hopping within one NPRACH preamble, down-select the following alternatives at RAN1#93 meeting

  - Alt 1: 2-level  -/+ 1.25 kHz -/+ 22.5 kHz

  - Alt. 2: 3-level -/+ 1.25 kHz -/+ 3.75 kHz 22.5 kHz

Agreement

Specification supports that the new NPRACH format resouces can be configured to overlap or not overlap with the NPRACH resources of Rel-13/Rel-14.

R1-1804193	NPRACH range enhancement	ZTE, SaneChips

R1-1803879	NPRACH enhancement for cell radius extension	Huawei, HiSilicon

R1-1804163	NPRACH range enhancements for NB-IoT	Ericsson

R1-1804268	NPRACH cell range enhancement in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1804527	Resource configuration for NPRACH range enhancement	LG Electronics

Late submission

R1-1804927	NPRACH support for large cell access	Qualcomm Incorporated

#### TDD

R1-1804932	Coexistence with NR	Qualcomm Incorporated

##### Downlink aspects

R1-1805312	Summary of DL aspects for TDD NB-IoT	ZTE, SaneChips

Agreement

- For SIB1-NB transmission in TDD on an anchor carrier, the subframe index and TBS are indicated by schedulingInfoSIB1 as follows:

| Value of schedulingInfoSIB1 | Number of NPDSCH repetitions | Subframe | TBS |
| --- | --- | --- | --- |
| 0 | 4 | 0 | 208 |
| 1 | 8 | 0 | 208 |
| 2 | 16 | 0 | 208 |
| 3 | 4 | 0 | 328 |
| 4 | 8 | 0 | 328 |
| 5 | 16 | 0 | 328 |
| 6 | 4 | 0 | 440 |
| 7 | 8 | 0 | 440 |
| 8 | 16 | 0 | 440 |
| 9 | 4 | 0 | 680 |
| 10 | 8 | 0 | 680 |
| 11 | 16 | 0 | 680 |
| 12 | 16 | 4 | 208 |
| 13 | 16 | 4 | 328 |
| 14 | 16 | 4 | 440 |
| 15 | 16 | 4 | 680 |

Agreement

- For 16 and 8 repetitions of SIB1-NB on a non-anchor carrier, the starting radio frame is as follows:

| Number of SIB1-NB repetitions | PCID | Starting radio frame number for SIB1-NB repetitions (nf mod 256) |
| --- | --- | --- |
| 16 | PCID mod 2 = 0 | 0 |
|  | PCID mod 2 = 1 | 1 |
| 8 | PCID mod 2 = 0 | 0 |
|  | PCID mod 2 = 1 | 16 |

R1-1805348	WF on SIB1-NB transmission on a non-anchor carrier in TDD	LG Electronics, Ericsson, Lenovo, Motorola mobility

Agreement

- When SIB-NB is transmitted on a non-anchor carrier, two subframes, i.e., subframe #0 and #5 in the same radio frame, are used for SIB1-NB transmission in every other radio frame

  - One transport block of SIB1-NB is transmitted over 16 SIB1-NB subframes

  - 16 SIB1-NB subframes carry encoded bits of SIB1-NB continuing reading them from circular buffer.

Agreement

For non-anchor carrier transmission of SIB1-NB

- MIB-NB has 3 bits to indicate the number of repetitions/TBS

- MIB-NB has 1 bit to indicate the frequency domain location of SIB1-NB non-anchor carrier

  - For In-band anchor + in-band non-anchor, either the adjacent lower PRB relative to anchor PRB, or the adjacent higher PRB relative to anchor PRB

  - FFS guard-band combinations

  - For standalone+standalone, to indicate one of the 2 adjacent carriers relative to anchor carrier. FFS signaling of frequency offset of the carrier.

Agreement

- For normal subframes, NRS resource mapping is the same as FDD.

- For special subframe configurations #3, #4 and #8, NRS is mapped to the 3rd and 4th symbols of each slot

- For special subframe configurations #9 and #10, NRS is mapped to the 3rd and 4th symbols of the first slot.

- For special subframe configurations #1, #2, #6 and #7, NRS is mapped to

  - The 6th and 7th symbols of the first slot

- For special subframe configuration #0 and #5, NRS is not transmitted

R1-1803872	Common aspects for TDD NB-IoT	Huawei, HiSilicon

R1-1804164	DL aspects of TDD for NB-IoT	Ericsson

R1-1804182	Remaining issues on downlink aspects to support TDD NB-IoT	ZTE, SaneChips

R1-1804201	Views on TDD downlink aspect	Lenovo, Motorola Mobility

R1-1804269	Downlink aspects of TDD support in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1804337	Discussion on DL common channel/signal for TDD NB-IoT	Samsung

R1-1804528	Discussion on DL aspects in TDD NB-IoT	LG Electronics

R1-1804929	Downlink aspects of TDD	Qualcomm Incorporated

##### Uplink aspects

R1-1805400	Summary of UL aspects for TDD NB-IoT	ZTE, SaneChips

Agreement

Mapping table for TDD UL/DL configuration and TDD NPRACH formats is list in Table X

- FFS configuration 6 if it is supported

- Transmit Format 0 and Format 0-a in all NPRACH subframes, and aligned with subframe boundary

Table X Mapping table for UL/DL configuration and NPRACH format

| UL/DL Configuration index | Format 0 | Format 1 | Format 2 | Format 0-a | Format 1-a |
| --- | --- | --- | --- | --- | --- |
| 1 | √ | √ |  | √ | √ |
| 2 | √ |  |  | √ |  |
| [3] | [√] |  | [√] | [√] |  |
| 4 | √ | √ |  | √ | √ |
| 5 | √ |  |  | √ |  |

Agreement

- For G=2, P=4  (format 0, 1, 2)

  - when the repetition number =1

    - Tone index of the first and third symbol groups in the preamble repetition unit is chosen by [SFN and cell specific pseudo-random sequence (FFS details)]. The initial tone index to hopping pattern mapping is

| Index of the tone used by the 1st symbol group | Deterministic hopping length for the 2nd within a repetition unit |
| --- | --- |
| 0, 2, 4, 6, 8, 10 | +3.75kHz |
| 1, 3, 5, 7, 9, 11 | -3.75kHz |
| Index of the tone used by the 3rd symbol group | Deterministic hopping length for the 4th within a repetition unit |
| 0, 1, 2, 3, 4, 5 | +22.5kHz |
| 6, 7, 8, 9, 10, 11 | -22.5kHz |

Table 16

  - when repetition number >=2

    - The initial tone index to hopping pattern mapping follows table above.

    - Tone index of the first symbol groups in the odd preamble repetition unit is chosen by [SFN and cell specific pseudo-random sequence (FFS details)] .

Aiming at cancelling phase errors, for an odd preamble that has been transmitted with a given tone index for its first and third symbol groups, the candidate tone index for the first and third symbol groups in the even preamble is chosen by [SFN and ]cell specific pseudo-random sequence and will be confined to one of the tone indexes in the opposite half of the bandwidth as follows:

| Odd Preamble Repetition Unit | Even Preamble Repetition Unit |
| --- | --- |
| Index of the tone used by the 1st symbol group | Candidate Indexes for the tone to be used by the 1st symbol group |
| 0, 2, 4, 6, 8, 10 | 1, 3, 5, 7, 9, 11 |
| 1, 3, 5, 7, 9, 11 | 0, 2, 4, 6, 8, 10 |
| Index of the tone used by the 3rd symbol group | Candidate Indexes for the tone to be used by the 3rd symbol group |
| 0, 1, 2, 3, 4, 5 | 6, 7, 8, 9, 10, 11 |
| 6, 7, 8, 9, 10, 11 | 0, 1, 2, 3, 4, 5 |

Table 17

- For G=3, P=6 (format 0-a, format 1-a)

  - Tone index of the first and fourth symbol groups in the preamble repetition unit is chosen by [SFN and cell specific pseudo-random sequence (FFS details)]. The initial tone index of the first and fourth symbol groups to hopping pattern mapping is

Table 18 Hopping patterns for 2nd and 3rd symbol group within a repetition unit

| Index of the tone used by the 1st symbol group | Hopping pattern for the 2nd and 3rd symbol group within a repetition unit |
| --- | --- |
| 0, 2, 4,6, 8, 10 | +3.75kHz，-3.75kHz |
| 1, 3, 5,7, 9, 11 | -3.75kHz，+3.75kHz |

Table 19 Hopping patterns for 5th and 6th symbol group within a repetition unit

| Index of the tone used by the 4th symbol group | Hopping pattern for the 5th and 6th symbol group within a repetition unit |
| --- | --- |
| 0, 1, 2, 3, 4, 5 | +22.5kHz，-22.5kHz |
| 6, 7, 8,9, 10, 11 | -22.5kHz，+22.5kHz， |

Agreement

For NPUSCH transmission with 15 kHz subcarrier spacing, the number of subcarriers supported in TDD UL/DL configuration #3 is 1, 3, 6 and 12.

- RU length reuses FDD

Agreement

- For TDD NB-IoT with 15 kHz subcarrier spacing, the resource unit of NPUSCH format 2 is single tone with RU length 2 ms.

- The previous agreement above refers to NPUSCH format 1

Agreement

3.75kHz subcarrier spacing is not supported for TDD UL/DL configuration #3.

R1-1803873	On downlink TDD NB-IoT	Huawei, HiSilicon

R1-1804165	UL aspects of TDD for NB-IoT	Ericsson

R1-1804183	Remaining issues on uplink aspects to support TDD NB-IoT	ZTE, SaneChips

R1-1804270	Uplink aspects of TDD support in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1804338	Discussion on UL channel for TDD NB-IoT	Samsung

R1-1804529	Discussion on UL aspects in TDD NB-IoT	LG Electronics

R1-1804876	Uplink aspects of TDD NB-IoT	Sharp

R1-1804930	Uplink aspects of TDD	Qualcomm Incorporated

R1-1805350	WF on NPUSCH transmission in UL/DL configuration #3	LG Electronics

R1-1805413	WF on NPUSCH transmission with 3.75kHz subcarrier spacing	LG Electronics

##### Common aspects

E.g. Relaxations of MCL, latency, capacity targets; UL:DL configurations, special subframe, HARQ, etc.

R1-1805349	Summary of TDD Common aspects	LG Electronics

Agreement

DwPTS can be used for NB-IoT transmission in CRS-less special subframe configuration #10 in TDD NB-IoT.

- FFS on how to use the blank REs corresponding to CRSs

Agreement

Downlink scheduling delay is defined by 4 physical subframes + k0, and k0 is based on valid downlink subframes. The scheduling delay values in FDD NB-IoT are reused.

- FFS on whether valid special subframes which include DwPTS can be counted as a part of the scheduling delay k0

Email discussion to decide the details on DL/UL interlacing until 5/4 Changhwan (LGE)

R1-1803874	On uplink TDD NB-IoT	Huawei, HiSilicon

R1-1804166	DL/UL common aspects of TDD for NB-IoT	Ericsson

R1-1804184	Remaining issues on common aspects to support TDD NB-IoT	ZTE, SaneChips

R1-1804271	Common Aspects of NB-IoT TDD Operation	Nokia, Nokia Shanghai Bell

R1-1804339	Discussion on 2 HARQ processes and cross carrier scheduling	Samsung

R1-1804530	Discussion on common aspects in TDD NB-IoT	LG Electronics

R1-1804890	TDD NB-IoT coexistence with NR	SoftBank Corp.

R1-1804931	General considerations on TDD design	Qualcomm Incorporated

R1-1805133	Scheduling Aspects of TDD NB-IoT	III

#### Other

Including extended PHR related aspects.

R1-1804185	Considerations on extended Power Headroom report for NB-IoT	ZTE, SaneChips

R1-1804272	PHR Enhancement for NB-IoT	Nokia, Nokia Shanghai Bell

R1-1804340	Enhanced PHR for NB-IOT	Samsung

### Ultra Reliable Low Latency Communication for LTE - WID in RP-172845

R1-1805429	Chairman's notes of AI 6.2.8 Ultra Reliable Low Latency Communication for LTE	Ad-Hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

R1-1805414	Notes from the offline session on LTE URLLC	Ericsson

Friday

R1-1805433	Draft LS on agreements for ultra reliable low latency communication for LTE	Ericsson

Email discussion on the set of RRC parameters/UE capability for HRLLC based on the agreements so far by 4/27 – Laetitia (Ericsson)

#### Blind/HARQ-less Repetition for Scheduled DL-SCH Operation

- Finalise details of RAN1 agreement to support blind/HARQ-less PDSCH repetition.

- Using legacy (S/E)PDCCH, (S)PUCCH formats (if applicable); any discussion of potential DCI modifications is limited to support of blind/HARQ-less repetition

- All four variants (as identified in RAN1#92) are valid for further discussion.

R1-1805322	Blind/HARQ-less Repetition for Scheduled DL-SCH Operation	Huawei, HiSilicon	(rev of R1-1803711)

R1-1805342	Summary of simulation results regarding HARQ-less repetition for scheduled DL-SCH	Huawei, HiSilicon

R1-1803962	Discussion on  PDSCH repetition for LTE URLLC	ZTE, Sanechips

R1-1804136	Blind/Harq-less repetitions for scheduled DL-SCH in LTE HRLLC	MediaTek Inc.

R1-1804341	Discussion on PDSCH repetition	Samsung

R1-1804531	Discussion on blind/HARQ-less repetition for scheduled DL-SCH operation	LG Electronics

R1-1804587	On blind/HARQ-less PDSCH repetition	Nokia, Nokia Shanghai Bell

R1-1804701	Repetitions for DL operation	Intel Corporation

R1-1805271	Downlink Enhancements for URLLC	Qualcomm Incorporated	(Revision of R1-1804933)

R1-1804958	PDSCH aspects of LTE-URLLC	Motorola Mobility, Lenovo

R1-1805162	URLLC techniques for PDSCH	Ericsson

Agreement:

PDCCH indicates number of PDSCH transmissions associated with the PDCCH. PDCCH may or may not be transmitted with a PDSCH repetition. PDSCH transmissions can be soft combined after a PDCCH is successfully received

- The number of transmissions, k, is the number of PDSCH transmissions associated with the PDCCH starting with the current TTI

  - FFS: Values for k

- FFS: What the UE does with PDCCHs received for the TB after a successfully received PDCCH within the repetition window

Agreement:

Supporting repetition based PDSCH reception is a UE capability.

- FFS: Details of capabilities and how they are signalled.

Agreement:

The UE shall discard any PDSCH assignment for a (s)TTI in a serving cell with CRC scrambled with C-RNTI, if PDSCH by means of blind repetition is being received in that (s)TTI in the same serving cell.

Agreement:

Blind/HARQ-less PDSCH repetitions are enabled by RRC configuration. If configured, a single DCI format is used to schedule PDSCH with a given TTI length. There is a field in the DCI that indicates the number of PDSCH transmissions k associated with the DCI, where k >= 1.

- FFS on DCI content

Working assumption:

For HARQ for repeated PDSCH transmissions, the UE shall report HARQ feedback with the timing given by the last PDSCH repetition.

#### Repetition Enhancements for UL SPS Operation

Secondary priority (best effort only). Finalise details to support UL SPS repetition configuration (both sTTI and TTI)

R1-1803712	Repetition Enhancements for UL SPS Operation	Huawei, HiSilicon

R1-1803963	On UL SPS repetition for LTE URLLC	ZTE, Sanechips

R1-1804342	Discussion on Repetition for UL SPS	Samsung

R1-1804498	On UL SPS enhancements for URLLC	Nokia, Nokia Shanghai Bell

R1-1804532	Discussion on repetition enhancements for UL SPS operation	LG Electronics

R1-1804934	Uplink Enhancements for URLLC	Qualcomm Incorporated

R1-1805163	UL SPS  PUSCH performance and HARQ ambiguity resolution	Ericsson

Proposal:

Any further discussion on UL SPS should focus on the following options:

Option 1: K repetitions, less or equal to the SPS periodicity P. Transmission starts at the beginning of the P windows. RV sequence is configurable.

Option 2: P=1, K repetitions are guaranteed and can start at any point.

Option 3: P configurable, UE can start at any point within P, but K transmission configured but Tx stops at P boundary (i.e. K tx are not garanteed).

#### Other

R1-1804436	Necessity for specifying UL SPS with repetition to support URLLC in Rel-15	Huawei, HiSilicon

R1-1804437	Preliminary summary of specification impact for supporting UL SPS with repetition	Huawei, HiSilicon

R1-1804588	On higher layer configured PCFICH for URLLC	Nokia, Nokia Shanghai Bell

R1-1804612	DL Control Techniques for LTE URLLC	Fraunhofer HHI

R1-1804935	PCFICH Enhancements for URLLC	Qualcomm Incorporated

Agreement:

Support an optional, CFI configuration per TTI length through UE and serving cell specific semi-static configuration.

- If CFI is semi-statically configured for TTIs of different lengths, the UE does not expect the configured CFI values to be different.

- The semi-static CFI value could be configured separately for MBSFN and non-MBSFN subframes over each cell.

R1-1805164	Latency for URLLC	Ericsson

R1-1805165	Reliability for URLLC	Ericsson

R1-1805166	On Pre-emption in DL	Ericsson

R1-1805167	On Pre-emption in UL	Ericsson

R1-1805168	On PUCCH/SPUCCH reliability for URLLC	Ericsson

### Enhanced LTE Support for Aerial Vehicles - WID in RP-172826

R1-1805430	Chairman's notes of AI 6.2.9 Enhanced LTE Support for Aerial Vehicles	Ad-Hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

#### UL power control enhancements

R1-1803889	Uplink Power control for aerial UEs	Huawei, HiSilicon

R1-1804088	Discussion on UL power control for Aerial Vehicles	CMCC

R1-1804237	Enhancements on power control for aerial vehicle	ZTE, Sanechips

R1-1804609	Discussion on UL transmission for aerial vehicle	Sony

R1-1804702	On UL power control enhancements for Aerial Vehicles	Intel Corporation

R1-1804936	UL power control enhancements for aerial vehicles	Qualcomm Incorporated

R1-1805019	Discussion on UL power control enhancements for aerial vehicles	Ericsson

R1-1805356	WF on UL Power Control Enhancements for Aerials	Ericsson, ZTE, Sanechips, Nokia, NSB, Sony, CMCC

Agreement:

If accumulation is enabled, the aerial UE shall reset the power control adjustment state for PUSCH when the value of alpha used by UE for PUSCH is changed

Agreement:

The range of UE-specific P0 parameter is extended to (-16,…,15)

Agreement:

- The aerial UE can be RRC configured with a single UE-specific alpha parameter for PUSCH

- The UE-specific alpha overrides the cell-specific alpha once it is configured.

- The support of UE-specific alpha and the extended range of UE-specific P0 are optional with capability signaling

  - Note: Whether the capability signaling is separate for the two aspects should be discussed in the LTE feature list discussion

R1-1805378	WF on altitude threshold for Aerials	Sony, Ericsson, ZTE

Agreement:

- RAN1 has discussed whether it is useful to have multiple altitude thresholds from a power control perspective.

- Send an LS to RAN2 requesting them to introduce multiple altitude thresholds.

R1-1805379	LS on altitude threshold for Aerials	Sony

R1-1805383	[Draft] LS on RAN1 Agreements from RAN1#92bis and on the introduction of multiple altitude thresholds	Ericsson

Decision: The document is endorsed and final LS is approved in R1-1805388.

#### Other

R1-1805020	Evaluation results on extended range of UE specific P0 parameter	Ericsson

R1-1805021	On positioning for aerial vehicles	Ericsson

R1-1805169	Resource allocation for aerial vehicle	Sony

### Enhancing CA Utilization - WID in RP-180561

- Including draft CR, to be prepared by Rapporteur, capturing previous RAN1 agreements.

  - Note: no other CRs are allowed. Individual text proposals are allowed, which once approved, are to be consolidated into a single jumbo CR

R1-1805431	Chairman's notes of AI 6.2.10 Enhancing CA Utilization	Ad-Hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

R1-1804438	Discussion on CSI feedback for CA SCell New State	Huawei, HiSilicon

R1-1804454	Introduction of Enhancing CA Utilization into 36.213		Nokia, Nokia Shanghai Bell

R1-1805357	Introduction of Enhancing CA Utilization into 36.213		Nokia, Nokia Shanghai Bell

Decision: Endorsed. Final CR is agreed in R1-1805377 (36.213, CR1070, Rel-15, B).

### Other

Including modulation enhancements, 2Tx/4Tx SRS switching, CBSR & CP latency reduction

R1-1805432	Chairman's notes of AI 6.2.11 Other	Ad-Hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

CP Latency Reduction

R1-1803713	Views on LTE Control Plane Latency Reduction	Huawei, HiSilicon

Proposal 1: CP latency of no more than 20ms needs to be supported in LTE in Rel-15.

Proposal 2: At least 4ms is supported for step 5 for 1ms TTI based CP latency reduction.

R1-1804134	Discussion on CP Latency	MediaTek Inc.

R1-1804939	Control plane latency reduction	Qualcomm Incorporated

R1-1805210	Introduction of reduced control plane latency	Ericsson

R1-1805325	Introduction of reduced control plane latency	Ericsson

R1-1805396	Introduction of reduced control plane latency	Ericsson

Decision: Endorsed. Final CR is agreed in R1-1805408 (TS36.213, CR1073, Rel-15, B).

R1-1805211	Control Plane latency reduction	Ericsson

R1-1805212	[DRAFT] Reply LS on CP latency reduction	Ericsson

Decision: The document is endorsed and final LS is approved in R1-1805321 with the following modification. Change “…3ms corresponding to a n+4 timing” to “4 ms corresponding to a n+5 timing”.

Decision: The document is revised and approved in R1-1805347 (to delete a duplicate sentence)

SRS Antenna Switching

R1-1803957	On support of SRS antenna switching for 1T4R and 2T4R antenna configurations	Huawei, HiSilicon

R1-1803958	On support of PUSCH antenna selection for 1T4R and 2T4R antenna configurations	Huawei, HiSilicon

R1-1803959	Introduction of SRS antenna switching for 4 ports in 36.213	Huawei, HiSilicon

R1-1805278	SRS antenna switching for 1T4R and 2T4R	Qualcomm Incorporated	(Revision of R1-1804937)

Agreement:

Support SRS antenna switching for 1T4R antenna configuration when frequency hopping is disabled as anSRS=nSRS mod 4

R1-1805323	Introduction of SRS antenna switching for 4 ports in TS36.213	Huawei, HiSilicon, Qualcomm

Decision: Email discussion until May 9, 2018 (Le, Qualcomm)

Modulation Enhancements

R1-1804135	Modulation Enhancements	MediaTek Inc.

R1-1804439	Discussion on modulation enhancement for 256QAM	Huawei, HiSilicon

R1-1804684	On TBS selection for LTE	Intel Corporation

R1-1804938	Discussion on modulation enhancements	Qualcomm Incorporated

Revisit this topic in RAN1#93

# NR

R1-1805141	Draft CR to 38.300 on functionality related to 38.21x	Editor (Nokia)

Decision: The document is endorsed. Send an LS to RAN2 – R1-1805558.

Friday

R1-1805558	[DRAFT] LS on time domain resource allocation	Nokia

Discussion: Huawei fine with the content, title may require some update?

Decision: The document is endorsed and final LS is approved in R1-1805765, assuming fixing with a proper title.

## Maintenance of NR - WID in RP-172834

Only essential corrections. Notes:

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

- Editors are encouraged to submit editor’s CR before the meeting reflecting additional comments received after the endorsed & frozen NR specifications. If submitted, please submit under agenda 7.

R1-1803865	Draft CR to 38.211	Ericsson

Discussion:

Ericsson: Intent is to raise issue that has not been correctly captured – suggests moving the table used for SFI signaling into the SFI section of 213

Huawei: draft CR was submitted very late

Ericsson: no intention to agree on it, for info moving forward

Decision: The document is noted. The change in 7.4.1.1.1 in R1-1803865 is endorsed (to be included in the next editor CR update)

Friday

R1-1805751	[DRAFT] LS on RRC parameters for NR	Ericsson

Decision: The document is endorsed and final LS is approved in R1-1805766.

Editors to prepare CRs capturing the agreements till April 27th, for comments and endorsement by May 3rd.

### Initial access and mobility

#### Synchronization signal

R1-1805501	Summary of 7.1.1.1 Synchronization signal	Ericsson

Decision: The document is noted.

For the issue of potentially define an offset range between SSS and PDCCH, discuss further offline, aim to conclude at this meeting

Conclusion:

- For the discrepancy in carrier definition in 38.211 and 38.331, 38.211 editor will align the definition with 38.331

Proposed conclusion:

- Agree that k_0 signalling is no longer necessary

  - To inform RAN2 about the above

- In 38.211, define k_0 based on existing RRC parameters

Offline discussion to prepare how k_0 is to be defined in 38.211 if k_0 signalling is removed (note: k_0 is also used for PRACH)

Agreement:

- To adopt the following TP to 38.212, Section 7.1.1:

Generate the following additional timing related PBCH payload bits , where:

-	 are the 4th, 3rd, 2nd, and 1st LSB of SFN, respectively;

-	 is the half radio frame bit ;

-	if

are the 6th, 5th, and 4th bits of SS/PBCH block index, respectively.

else

is the MSB of as defined in Subclause 7.4.3.1 of [4, TS 38.211].

are reserved.

end if

Agreement:

- The working assumption made in RAN1 AdHoc#1 on OFDM signal generation is confirmed.

Thursday

R1-1805595	Summary of 7.1.1.1 Synchronization signal	Ericsson

Decision: The document is noted.

Agreement:

- Agree the text proposal below

For a half frame with SS/PBCH blocks, the first symbol indexes for candidate SS/PBCH blocks are determined according to the subcarrier spacing of SS/PBCH blocks as follows, where index 0 is the first symbol of the first slot in a half-frame.

Agreements:

- To adopt the following TP to 38.213, Section 4.1:

--- Start of TP ---

For a half frame with SS/PBCH blocks, the first symbol indexes for candidate SS/PBCH blocks are determined according to the subcarrier spacing of SS/PBCH blocks as follows.

-	Case A - 15 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes of {2, 8} + 14*n. For carrier frequencies smaller than or equal to 3 GHz, n=0, 1. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1, 2, 3.

-	Case B - 30 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8, 16, 20} + 28*n. For carrier frequencies smaller than or equal to 3 GHz, n=0. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1.

-	Case C - 30 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {2, 8} + 14*n. For carrier frequencies smaller than or equal to 3 GHz, n=0, 1. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1, 2, 3.

-	Case D - 120 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8, 16, 20} + 28*n. For carrier frequencies larger than 6 GHz, n=0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

-	Case E - 240 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {8, 12, 16, 20, 32, 36, 40, 44} + 56*n. For carrier frequencies larger than 6 GHz, n=0, 1, 2, 3, 5, 6, 7, 8.

Which case(s) of Case A through E to be used on a carrier depends on frequency band(s) which is given by section of 5.4.3.3 of [8-1, TS 38.101-1] and [8-2, TS 38.101-2], and is the same case for all SS/PBCH blocks on a carrier.

--- End of TP ---

Proposal:

- The value of k0μ  for any numerology µ is obtained using

where μ0 is the numerology with highest SCS supported in the carrier, and the value of k0μ0∈{-6,0,+6} is obtained by RRC signaling

FFS EPRE offset between SSS and PDCCH

Friday

R1-1805748	Summary of 7.1.1.1 Synchronization signal	Ericsson

Decision: The document is noted.

Agreements:

- In initial cell selection and idle mode, the UE may assume that the EPRE offset between PDCCH DMRS and SSS in the QCL’ed SSB is in the range [X, Y]dB when PDDCH with CRC scrambled by SI-RNTI, P-RNTI and RA-RNTI

  - FFS: value of X and Y and dependence on numerology

    - |X| & Y are no more than 12

Conclusion:

- It is RAN1’s understanding that the f0 does not have to be on the channel raster nor the center of the RF filter.

R1-1803606	Remaining details of Synchronization Signal Design	ZTE, Sanechips

R1-1803625	Remaining details on synchronization signal	Huawei, HiSilicon

R1-1803729	Remaining Issues on Synchronization Signal	CATT

R1-1803806	Remaining issues on synchronization signal	vivo

R1-1803998	Power offset between SS/PBCH block and PDCCH	OPPO

R1-1804050	Remaining issues on synchronization signals	MediaTek Inc.

R1-1804343	Corrections on Synchronization Signaling	Samsung

R1-1804501	Symbol index definition for SSB time location	NEC Corporation

R1-1804533	Remaining Details on Synchronization signal	LG Electronics

R1-1804703	Remaining details of SS/PBCH block	Intel Corporation

R1-1804774	Remaining details on synchronization signal design	Qualcomm Incorporated

R1-1805031	Remaining issues on Synchronization signal	NTT DOCOMO, INC.

R1-1805134	Open issues related to Synchronisation Signal	Nokia, Nokia Shanghai Bell

R1-1805215	Remaining details on Synchronization signal	Ericsson

R1-1805723	Derivation of k0	Qualcomm

#### Broadcast signal/channel

##### NR-PBCH

R1-1805502	Summary of 7.1.1.2.1 NR-PBCH	Ericsson

Decision: The document is noted.

Proposal:

- From RAN1 perspective, the signalling of M in RMSI is not necessary in Rel-15

Agreement:

- Confirm working assumption on signalling tables for indication of cell-defining SSB

Agreement:

- Confirm working assumption on step size X=1 (for cell-defining SSB)

Proposal:

- To adopt the following TP to 38.213, Section 13:

If a UE detects a first SS/PBCH block and determines that a control resource set for Type0-PDCCH common search space is not present, and for  for FR1 or for  for FR2, the UE determines the closest global synchronization channel number (GSCN) of a second SS/PBCH block having a control resource set for an associated Type0-PDCCH common search space as .  is the GSCN of the first SS/PBCH block and  is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2.

Thursday

R1-1805596	Summary of 7.1.1.2.1 NR-PBCH	Ericsson

Decision: The document is noted.

Agreement: (RRC update)

- From RAN1 perspective, the signalling of M in RMSI with parameter frequencyOffsetSSB is not necessary in Rel-15

Agreement:

- To adopt the following TP to 38.213, Section 13:

If a UE detects a first SS/PBCH block and determines that a control resource set for Type0-PDCCH common search space is not present, and for  for FR1 or for  for FR2, the UE determines the nearest (in the corresponding frequency direction) global synchronization channel number (GSCN) of a second SS/PBCH block having a control resource set for an associated Type0-PDCCH common search space as .  is the GSCN of the first SS/PBCH block and  is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2.

Agreements:

- To adopt the following TP for 7.4.1.4.1 of 38.211

The UE shall assume the reference-signal sequence  for an SS/PBCH block is defined by

where  is given by clause 5.2. The scrambling sequence generator shall be initialized at the start of each SS/PBCH block occasion with

where

-	for, ,  is the number of the half-frame in which the PBCH is transmitted in a frame with  for the first half-frame in the frame and  for the second half-frame in the frame, and  is the two least significant bits of the SS/PBCH block index as defined in [5, TS 38.213]

-	for  or ,  and  is the three least significant bits of the SS/PBCH block index as defined in [5, TS 38.213]

with  being the maximum number of SS/PBCH beams in an SS/PBCH period for a particular band as given by [38.104].

Friday: no further progress regarding the following issues:

Timing Alignment between Cells to Avoid Unnecessary Reading of PBCH?

Alignment of scrambling for FR1 vs. FR2?

R1-1803607	Remaining details of NR-PBCH Design	ZTE, Sanechips

R1-1803626	Remaining details of NR PBCH design	Huawei, HiSilicon

R1-1803807	Remaining aspects on NR-PBCH	vivo

R1-1803997	On synchronization raster indication	OPPO

R1-1804051	Remaining issues on NR-PBCH	MediaTek Inc.

R1-1804089	Discussion on power offset for PBCH and PDCCH	CMCC

R1-1804344	Corrections on NR-PBCH	Samsung

R1-1804534	Remaining Details on PBCH design and contents	LG Electronics

R1-1804648	Remaining detail on the indication of cell-defining SS/PBCH block	Xiaomi Communications

R1-1804704	Remaining details of NR-PBCH	Intel Corporation

R1-1804775	Remaining details on NR-PBCH	Qualcomm Incorporated

R1-1804840	Remaining Details for Physical Broadcast Channel 	InterDigital, Inc.

R1-1805032	Remaining issues on NR-PBCH	NTT DOCOMO, INC.

R1-1805135	Remaining details related to SS/PBCH block	Nokia, Nokia Shanghai Bell

R1-1805216	Remaining details of PBCH	Ericsson

##### Remaining minimum system information

R1-1803731	Summary of Offline Discussion on RMSI	CATT

Decision: The document is noted.

Agreements:

- To adopt the following TP of 38.213, Section 13:

---Start of TP---

Table 13-1: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz

...

Table 13-2: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz

...

Table 13-3: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

...

Table 13-4: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

...

Table 13-5: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 15} kHz for frequency bands with minimum channel bandwidth 40MHz

...

Table 13-6: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 30} kHz for frequency bands with minimum channel bandwidth 40MHz

---End of TP---

Tuesday's session

R1-1805598	Offline summary on Max TBS for broadcast PDSCH	Huawei

Decision: The document is noted.

Agreements:

- Regarding the questions in LS R1-1803586:

  - 1st question: whether NR can support approximately 1700 bits RMSI in one TB in all cases.

    - Answer:

      - Yes, NR can support RMSI size of approximately 1700 bits in one TB for both FR1 and FR2 with an appropriate RMSI configuration. RAN1 agrees that max TBS of 2976 bits is supported for PDSCH by SI-RNTI from physical layer perspective.

  - 2nd question: If necessary, whether L1 parameters size in RMSI can be reduced.

    - Answer:  Since the answer to the 1st question is yes, RAN1 sees no urgency to reduce L1 parameters size in RMSI. However, RAN1 is considering the possibility of doing so and will update RAN2 upon new progress.

- To send the reply LS

R1-1805597	Draft LS on Max TBS for PDSCH containing RMSI	Huawei

Decision: The document is noted. To be revised in R1-1805617 to take into account above agreement

Wednesday

R1-1805617	Draft LS on Max TBS for PDSCH containing RMSI	Huawei

Decision: The draft LS is endorsed and approved in R1-1805653.

Thursday session

R1-1805600	Summary of Offline Discussion on RMSI	CATT

Decision: The document is noted.

Conclusion:

- Editor to make updates regarding the wording of “subcarrier k” in 38.211 when necessary and applicable.

Issues for {SSB SCS, PDCCH SCS} = {240, 120} kHz with 2 symbols

R1-1805676	[Draft] LS on RMSI CORESET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth	CATT

Decision: The document is endorsed and final LS is approved in R1-1805712.

Friday

R1-1805741	Summary of Offline Discussion on RMSI	CATT

Decision: The document is noted.

Agreements:

For {SSB SCS, PDCCH SCS} = {240, 120} kHz, remove the configurations with =2 for Pattern 2 in Table 13-10 of TS 38.213.

---- Start of Test Proposal (TS 38.213) ----

Table 13-10: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {240, 120} kHz

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 0 |
| 1 | 1 | 48 | 1 | 8 |
| 2 | 1 | 48 | 2 | 0 |
| 3 | 1 | 48 | 2 | 8 |
| 4 | Reserved |  |  |  |
| 5 | Reserved |  |  |  |
| 6 | Reserved |  |  |  |
| 7 | Reserved |  |  |  |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

---- End of Test Proposal (TS 38.213) ----

R1-1803608	Remaining details of RMSI	ZTE, Sanechips

R1-1803627	Remaining  on RMSI configurations	Huawei, HiSilicon

R1-1803730	Remaining Issues on RMSI	CATT

R1-1803808	Discussion on Remaining Minimum System Information	vivo

R1-1803991	Remaing issues on RMSI	OPPO

R1-1804052	Remaining issues on RMSI	MediaTek Inc.

R1-1804090	Discussion on remaining issues for RMSI	CMCC

R1-1804345	Corrections on Remaining Minimum System Information	Samsung

R1-1804535	RMSI delivery and CORESET configuration	LG Electronics

R1-1804705	Remaining details of RMSI	Intel Corporation

R1-1804776	Remaining system information delivery consideration	Qualcomm Incorporated

R1-1805033	Remaining issues on RMSI	NTT DOCOMO, INC.

R1-1805136	Remaining issues on RMSI	Nokia, Nokia Shanghai Bell

R1-1805217	Remaining minimum system information	Ericsson

R1-1805605	RMSI CORESET Configuration Tables for SS SCS = 15 kHz and min CH BW = 10 MHz	Samsung

##### Other system information delivery

R1-1805516	Summary of 7.1.1.2.3 Other system information delivery	Samsung

Decision: The document is noted.

R1-1804053	Remaining issues on OSI	MediaTek Inc.

R1-1804346	Corrections on Other System Information Delivery	Samsung

R1-1804536	Other system information delivery	LG Electronics

R1-1805137	On BWP for Other System Information Delivery	Nokia, Nokia Shanghai Bell

R1-1805218	On other system information delivery	Ericsson

#### Paging design

R1-1805508	Summary on Paging design	Huawei

Decision: The document is noted.

Thursday

R1-1805679	Summary on Paging design	Huawei

Decision: The document is noted.

Continue further discussion on the following proposal (TP for 38.213, Section 12):

(For the case when initial DL BWP is defined to match to SSB+CORESET)

---

For SS/PBCH block and control resource set multiplexing pattern 1, Aan initial active DL BWP is defined by a location and number of contiguous PRBs, a subcarrier spacing, and a cyclic prefix, for the control resource set for Type0-PDCCH common search space. For SS/PBCH block and control resource set multiplexing pattern 2 and 3, number of contiguous PRBs, NRBinit, for an initial active DL BWP is defined as NRBinit=max⁡((offset+NRBSSB),(NRBCORESET-offset)), where offset is defined in defined in Tables 13-7 to 13-10 in Section 13 and NRBSSB is the SS/PBCH block bandwidth in number of RB assuming the subcarrier spacing for Type0-PDCCH common search space. For SS/PBCH block and control resource set multiplexing pattern 2 and 3, the start of the initial active DL BWP is defined with PRB offset with respect to the smallest RB index of the initial active DL BWP to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block and is given as max⁡(0, offset).

For operation on the primary cell, a UE is provided by higher layer parameter initial-UL-BWP an initial active UL BWP for a random access procedure. If the UE is configured with a secondary carrier on the primary cell, the UE can be configured with an initial BWP for random access procedure on the secondary carrier.

---

R1-1803992	Remaining details on NR paging	OPPO

R1-1804054	Remaining issue on Paging	MediaTek Inc.

R1-1804347	Corrections on Paging Design	Samsung

R1-1804537	Paging design in NR	LG Electronics

R1-1804777	Paging design consideration	Qualcomm Incorporated

R1-1804841	Remaining Details for Paging Channel 	InterDigital, Inc.

R1-1805034	Remaining issues on Paging design	NTT DOCOMO, INC.

R1-1805138	BWP determination for paging message delivery	Nokia, Nokia Shanghai Bell

R1-1805219	Remaining details on paging design	Ericsson

#### Physical random access channel and random access procedure

##### PRACH formats

R1-1805533	Summary of rmaining details on PRACH format	Convida Wireless

Decision: The document is noted.

Proposals:

- For FR2, the working assumption is updated as follows:

  - For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part in any RACH configuration period are valid as long as it does not precede or collide with an SSB in the RACH slot in any RACH configuration period

- UE is not expected to receive DL signals during any valid RACH occasion

- Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured

- FFS how to handle the gap necessary in between SS/PBCH block or DL part and PRACH tx

Proposal:

- To adopt the following TP to 5.3.2 of 38.211:

-	 is the PRACH transmission occasion within the PRACH slot, numbered in increasing order from 0 to  within a RACH slot where  is given Tables 6.3.3.2-2 to 6.3.3.2-4 for short sequences and fixed to 1 for long sequences;

-	 is given by Tables 6.3.3.2-2 to 6.3.3.2-4 for short sequences; is fixed to 0 for long sequences.

Tuesday session

R1-1805577	Summary of remaining details on PRACH format	Convida Wireless

Agreements:

- For FR2, the previous working assumption is replaced with the following working assumption (no change of the previous working assumption regarding FR1)

  - For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot, and it is more than N symbols after DL part and/or the last symbol of an SSB.

    - N = 2 for Msg1 SCS = 60kHz

    - N = 3 for Msg1 SCS = 120kHz

      - UE is not expected to receive DL signals during any valid RACH occasion

      - The SSB to RO association pattern period shall repeat at most every 160 ms

        - If there are leftover ROs after an integer number of SSB to RO association period(s) within the SSB to RO association pattern period. Those leftover ROs are invalid.

      - Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured

      - FFS how to handle the gap necessary in between SS/PBCH block or DL part and PRACH tx

Thursday session

R1-1805648	Summary of remaining details on PRACH format	Convida Wireless

Decision: The document is noted.

Agreements:

- Update the WA (remains as WA) on FR1 configuration Table 6.3.3.2-3 in 38.211 section 6.3.3.2, with the following update for configuration index 166 for format B4:

Table 6.3.3.2-3: Random access configurations for FR1 and unpaired spectrum.

| PRACH Configuration Index | Preamble format |  |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a RACH slot |  |  | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 166 | B4 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 3,4,8,9 | 0 | 2 |  | 1 | 12 |  |

Agreements:

- Replace the previous working assumption (remains WA) for FR2 RACH configuration table, with the following table.

  - The ones in yellow background are not final

  - Note: RACH configuration indexing will be up to editor to update.

--- Text proposal for 38.211, section 6.3.3 ---

Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum.

| PRACH Config. Index | Preamble format |  |  | Slot number | Starting symbol | Number of PRACH slots within a 60 kHz slot | , number of time-domain PRACH occasions within a RACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 |
| 1 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 3 | 2 |
| 2 | A1 | 1 | 0 | 9,19,29,39 | 0 | 2 | 6 | 2 |
| 3 | A1 | 1 | 0 | 24,29,34,39 | 7 | 1 | 3 | 2 |
| 4 | A1 | 1 | 0 | 17,19,37,39 | 0 | 1 | 6 | 2 |
| 5 | A1 | 1 | 0 | 19,39 | 7 | 1 | 3 | 2 |
| 6 | A1 | 1 | 0 | 9,19,29,39 | 7 | 2 | 3 | 2 |
| 7 | A1 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 3 | 2 |
| 8 | A1 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 9 | A1 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 3 | 2 |
| 10 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 11 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 3 | 2 |
| 12 | A1 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 6 | 2 |
| 13 | A1 | 1 | 0 | 3,5,7 | 0 | 1 | 6 | 2 |
| 14 | A1 | 1 | 0 | 3,5,7,9,11,13 | 7 | 1 | 3 | 2 |
| 15 | A1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 16 | A1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 17 | A1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 18 | A1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 19 | A1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 20 | A1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 21 | A1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 22 | A1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 23 | A1 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 6 | 2 |
| 24 | A1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 |
| 25 | A1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 |
| 26 | A1 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 | 2 | 3 | 2 |
| 27 | A1 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 6 | 2 |
| 28 | A1 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 6 | 2 |
| 0 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 |
| 1 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 5 | 1 | 2 | 4 |
| 2 | A2 | 1 | 0 | 9,19,29,39 | 0 | 2 | 3 | 4 |
| 3 | A2 | 1 | 0 | 24,29,34,39 | 5 | 1 | 2 | 4 |
| 4 | A2 | 1 | 0 | 17,19,37,39 | 0 | 1 | 3 | 4 |
| 5 | A2 | 1 | 0 | 19,39 | 5 | 1 | 2 | 4 |
| 6 | A2 | 1 | 0 | 9,19,29,39 | 5 | 2 | 2 | 4 |
| 7 | A2 | 1 | 0 | 0,1,2,…,39 | 5 | 1 | 2 | 4 |
| 8 | A2 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 9 | A2 | 1 | 0 | 23,27,31,35,39 | 5 | 1 | 2 | 4 |
| 10 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 11 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 5 | 1 | 2 | 4 |
| 12 | A2 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 3 | 4 |
| 13 | A2 | 1 | 0 | 3,5,7 | 0 | 1 | 3 | 4 |
| 14 | A2 | 1 | 0 | 3,5,7,9,11,13 | 5 | 1 | 2 | 4 |
| 15 | A2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 16 | A2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 17 | A2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 18 | A2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 19 | A2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 20 | A2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 21 | A2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 22 | A2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 23 | A2 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 3 | 4 |
| 24 | A2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 |
| 25 | A2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 |
| 26 | A2 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 5 | 2 | 2 | 4 |
| 27 | A2 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 3 | 4 |
| 28 | A2 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 3 | 4 |
| 0 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 1 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 1 | 6 |
| 2 | A3 | 1 | 0 | 9,19,29,39 | 0 | 2 | 2 | 6 |
| 3 | A3 | 1 | 0 | 24,29,34,39 | 7 | 1 | 1 | 6 |
| 4 | A3 | 1 | 0 | 17,19,37,39 | 0 | 1 | 2 | 6 |
| 5 | A3 | 1 | 0 | 19,39 | 7 | 1 | 1 | 6 |
| 6 | A3 | 1 | 0 | 9,19,29,39 | 7 | 2 | 1 | 6 |
| 7 | A3 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 1 | 6 |
| 8 | A3 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 9 | A3 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 1 | 6 |
| 10 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 11 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 1 | 6 |
| 12 | A3 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 2 | 6 |
| 13 | A3 | 1 | 0 | 3,5,7 | 0 | 1 | 2 | 6 |
| 14 | A3 | 1 | 0 | 3,5,7,9,11,13 | 7 | 1 | 1 | 6 |
| 15 | A3 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 16 | A3 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 17 | A3 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 18 | A3 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 19 | A3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 20 | A3 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 21 | A3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 22 | A3 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 23 | A3 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 2 | 6 |
| 24 | A3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 25 | A3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 26 | A3 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 | 2 | 1 | 6 |
| 27 | A3 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 2 | 6 |
| CMCC | A3 | 1 | 0 | 7,15,23,31,39 or 9,11,13 | 2 | 1 | 2 | 6 |
| 0 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 1 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 | 2 |
| 2 | B1 | 1 | 0 | 9,19,29,39 | 2 | 2 | 6 | 2 |
| 3 | B1 | 1 | 0 | 24,29,34,39 | 8 | 1 | 3 | 2 |
| 4 | B1 | 1 | 0 | 17,19,37,39 | 2 | 1 | 6 | 2 |
| 5 | B1 | 1 | 0 | 19,39 | 8 | 1 | 3 | 2 |
| 6 | B1 | 1 | 0 | 9,19,29,39 | 8 | 2 | 3 | 2 |
| 7 | B1 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 | 2 |
| 8 | B1 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 9 | B1 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 10 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 11 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 12 | B1 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 6 | 2 |
| 13 | B1 | 1 | 0 | 3,5,7 | 2 | 1 | 6 | 2 |
| 14 | B1 | 1 | 0 | 3,5,7,9,11,13 | 8 | 1 | 3 | 2 |
| 15 | B1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 16 | B1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 17 | B1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 18 | B1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 19 | B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 20 | B1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 21 | B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 22 | B1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 23 | B1 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 6 | 2 |
| 24 | B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 25 | B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 26 | B1 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 8 | 2 | 3 | 2 |
| 27 | B1 | 8 | 1,2 | 9,19,29,39 | 2 | 2 | 6 | 2 |
| 0 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 1 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 or 2 | 1 | 12 |
| 2 | B4 | 1 | 0 | 9,19,29,39 | 0 | 2 | 1 | 12 |
| 3 | B4 | 1 | 0 | 24,29,34,39 | 2 | 1 | 1 | 12 |
| 4 | B4 | 1 | 0 | 17,19,37,39 | 0 | 1 | 1 | 12 |
| 5 | B4 | 1 | 0 | 19,39 | 2 | 1 or 2 | 1 | 12 |
| 6 | B4 | 1 | 0 | 9,19,29,39 | 2 | 2 | 1 | 12 |
| 7 | B4 | 1 | 0 | 0,1,2,…,39 | 2 | 1 | 1 | 12 |
| 8 | B4 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 9 | B4 | 1 | 0 | 23,27,31,35,39 | 2 | 1 or 2 | 1 | 12 |
| 10 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 11 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 or 2 | 1 | 12 |
| 12 | B4 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 1 | 12 |
| 13 | B4 | 1 | 0 | 3,5,7 | 0 | 1 | 1 | 12 |
| 14 | B4 | 1 | 0 | 3,5,7,9,11,13 | 2 | 1 | 1 | 12 |
| 15 | B4 | 16 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| FO | B4 | 16 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 17 | B4 | 8 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 18 | B4 | 8 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 19 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 20 | B4 | 4 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 21 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 22 | B4 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 23 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 1 or 2 | 1 | 12 |
| 24 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 25 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 26 | B4 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 2 | 2 | 1 | 12 |
| 27 | B4 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 1 | 12 |
| 28 | B4 | 2 | 1 | 7,15,23,31,39 | 2 | 2 | 1 | 12 |
| 0 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 1 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 | 2 |
| 2 | C0 | 1 | 0 | 9,19,29,39 | 0 | 2 | 7 | 2 |
| 3 | C0 | 1 | 0 | 24,29,34,39 | 8 | 1 | 3 | 2 |
| 4 | C0 | 1 | 0 | 17,19,37,39 | 0 | 1 | 7 | 2 |
| 5 | C0 | 1 | 0 | 19,39 | 8 | 1 | 3 | 2 |
| 6 | C0 | 1 | 0 | 9,19,29,39 | 8 | 2 | 3 | 2 |
| 7 | C0 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 | 2 |
| 8 | C0 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 9 | C0 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 10 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 11 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 12 | C0 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 7 | 2 |
| 13 | C0 | 1 | 0 | 3,5,7 | 0 | 1 | 7 | 2 |
| 14 | C0 | 1 | 0 | 3,5,7,9,11,13 | 8 | 1 | 3 | 2 |
| 15 | C0 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 or 2 | 7 | 2 |
| EF | C0 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 17 | C0 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 or 2 | 7 | 2 |
| 18 | C0 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 19 | C0 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 |
| 20 | C0 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 21 | C0 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 |
| 22 | C0 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 23 | C0 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 7 | 2 |
| 24 | C0 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 25 | C0 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 26 | C0 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 8 | 2 | 3 | 2 |
| 27 | C0 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 7 | 2 |
| 28 | C0 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 7 | 2 |
| 0 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 1 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 or 2 | 1 | 6 |
| 2 | C2 | 1 | 0 | 9,19,29,39 | 2 | 2 | 2 | 6 |
| 3 | C2 | 1 | 0 | 24,29,34,39 | 7 | 1 | 1 | 6 |
| 4 | C2 | 1 | 0 | 17,19,37,39 | 0 | 1 | 2 | 6 |
| 5 | C2 | 1 | 0 | 19,39 | 2 | 1 | 2 | 6 |
| 6 | C2 | 1 | 0 | 9,19,29,39 | 7 | 2 | 1 | 6 |
| 7 | C2 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 1 | 6 |
| JF | C2 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 9 | C2 | 1 | 0 | 23,27,31,35,39 | 7 | 1 or 2 | 1 | 6 |
| 10 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 11 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 or 2 | 1 | 6 |
| 12 | C2 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 2 | 6 |
| 13 | C2 | 1 | 0 | 3,5,7 | 0 | 1 | 2 | 6 |
| 14 | C2 | 1 | 0 | 3,5,7,9,11,13 | 7 | 1 | 1 | 6 |
| 15 | C2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 16 | C2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 17 | C2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 18 | C2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 19 | C2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 20 | C2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 21 | C2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 22 | C2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 23 | C2 | 1 | 0 | 7,15,23,31,39 | 2 | 1 or 2 | 2 | 6 |
| 24 | C2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 25 | C2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 26 | C2 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 | 2 | 1 | 6 |
| 27 | C2 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 2 | 6 |
| 28 | C2 | 2 | 1 | 7,15,23,31,39 | 2 | 2 | 2 | 6 |
| 0 | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| HK | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 | 2 |
| 2 | A1/B1 | 1 | 0 | 9,19,29,39 | 2 | 2 | 6 | 2 |
| 3 | A1/B1 | 1 | 0 | 9,19,29,39 | 8 | 1 | 3 | 2 |
| 4 | A1/B1 | 1 | 0 | 17,19,37,39 | 2 | 1 | 6 | 2 |
| 5 | A1/B1 | 1 | 0 | 19,39 | 8 | 1 | 3 | 2 |
| 6 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 7 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 8 | A1/B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 9 | A1/B1 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 6 | 2 |
| 10 | A1/B1 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 6 | 2 |
| 11 | A1/B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 12 | A1/B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 13 | A1/B1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 14 | A1/B1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 15 | A1/B1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 16 | A1/B1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 17 | A1/B1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 0 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 1 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 6 | 1 | 2 | 4 |
| 2 | A2/B2 | 1 | 0 | 9,19,29,39 | 2 | 2 | 3 | 4 |
| 3 | A2/B2 | 1 | 0 | 9,19,29,39 | 6 | 1 | 2 | 4 |
| 4 | A2/B2 | 1 | 0 | 17,19,37,39 | 2 | 1 | 3 | 4 |
| 5 | A2/B2 | 1 | 0 | 19,39 | 6 | 1 | 2 | 4 |
| 6 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 7 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 6 | 1 | 2 | 4 |
| 8 | A2/B2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 9 | A2/B2 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 3 | 4 |
| 10 | A2/B2 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 3 | 4 |
| 11 | A2/B2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 12 | A2/B2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 13 | A2/B2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 14 | A2/B2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 15 | A2/B2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 16 | A2/B2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 17 | A2/B2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 0 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 1 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 2 | 6 |
| 2 | A3/B3 | 1 | 0 | 9,19,29,39 | 2 | 2 | 2 | 6 |
| 3 | A3/B3 | 1 | 0 | 9,19,29,39 | 2 | 1 | 2 | 6 |
| 4 | A3/B3 | 1 | 0 | 17,19,37,39 | 2 | 1 | 2 | 6 |
| 5 | A3/B3 | 1 | 0 | 19,39 | 2 | 1 | 2 | 6 |
| 6 | A3/B3 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 7 | A3/B3 | 1 | 0 | 23,27,31,35,39 | 2 | 2 | 2 | 6 |
| 8 | A3/B3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 9 | A3/B3 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 2 | 6 |
| 10 | A3/B3 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 2 | 6 |
| 11 | A3/B3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 12 | A3/B3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 13 | A3/B3 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 14 | A3/B3 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 15 | A3/B3 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 16 | A3/B3 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 17 | A3/B3 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |

--- End Text proposal for 38.211, section 6.3.3 ---

Agreements:

- To adopt the following TP to 6.3.3.1 of 38.211

There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift  of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter PRACHRootSequenceIndex. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic: the logical index 0 is consecutive to 837 when  and is consecutive to 137 when . The sequence number  is obtained from the logical root sequence index according to Tables 6.3.3.1-3 and 6.3.3.1-4.

Friday

R1-1805738	Summary of remaining details on PRACH format	Convida Wireless

Agreements:

- For both FR1 and FR2, the previous working assumption is replaced with the following working assumption

  - Note: The value with yellow background is not final

- For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot, and it is more than at least N symbols after DL part and/or the last symbol of an SSB.

  - N = 0 for Msg1 SCS = 1.25kHz, 5kHz

  - N = 2 for Msg1 SCS = 15kHz

  - N = 2 for Msg1 SCS = 30kHz

  - N = 2 for Msg1 SCS = 60kHz

  - N = 2 or 3 for Msg1 SCS = 120kHz

  - Note for format B4, N = 0 irrespectively of Msg1 SCS

- UE is not expected to receive DL signals during any valid RACH occasion

- The SSB to RO association pattern period shall repeat at most every 160 ms

  - If there are leftover ROs after an integer number of SSB to RO association period(s) within the SSB to RO association pattern period. Those leftover ROs are invalid.

- Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured

- FFS how to handle the gap necessary in between SS/PBCH block or DL part and PRACH tx

R1-1803609	Remaining details of PRACH Resource Configuration	ZTE, Sanechips

R1-1803628	Remaining details of RACH Preambles	Huawei, HiSilicon

R1-1803732	Further details on NR RACH format	CATT

R1-1804091	Discussion on remaining issues for PRACH configuration table	CMCC

R1-1804348	Corrections on PRACH Formats	Samsung

R1-1804456	Remaining details on PRACH formats	Nokia, Nokia Shanghai Bell

R1-1804538	Discussion on PRACH preamble format details	LG Electronics

R1-1804706	NR PRACH formats	Intel Corporation

R1-1804778	Remaining details on PRACH formats	Qualcomm Incorporated

R1-1804842	Remaining Details for Physical Random Access Channel 	InterDigital, Inc.

R1-1805035	Remaining issues on PRACH formats	NTT DOCOMO, INC.

R1-1805220	Remaining details of RACH configurations	Ericsson

##### RACH procedure

R1-1805566	Summary of remaining details on RACH procedure	Qualcomm

Decision: The document is noted.

Agreements:

- Confirm the following working assumption:

  - Minimum time gap between the end of PDSCH for Msg4 and the start of HARQ-ACK equals to the duration of N1 + L2.

    - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

    - L2=500 usec refers to the MAC layer processing time

    - Note: the reason for working assumption is due to potential concern over satisfying ITU requirement (to be checked)

Agreement:

- For the parameter new, new =250us

Thursday

Proposals: Update the previous agreements as follows:

- If a received Msg2 does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence after no later than the duration of N1 + new + L2.

  - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

  - new>=0, FFS

  - L2=500 usec refers to the MAC layer processing time

Proposal:

- For the parameter Delay for PDCCH ordered CFRA, Delay=0us

Proposals: Update previous agreements as follows:

- Minimum time gap between Msg2 and Msg3 if Msg2 and Msg3 have the same SCS

- Duration of N1 + duration of N2 + L2 + TA

  - N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1

  - N2 also refers to the value determined in control session with UE capability #1

  - TA is given by a 2-bit indicator in msg 2, with 4 values

    - Value 1:

    - Value 2:

    - Value 3:

    - Value 4: equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the SCS of Msg3

  - L2 refers to the MAC processing latency and it does not depend on subcarrier spacing

    - L2=500us as a working assumption

  - Note: If Msg2 and Msg3 have different SCS, value of N1 and N2 will refer to the ones determined in control session.

Proposals:

- When the UE transmits a PRACH, it considers all TCI states associated with any CORESET or any PDSCH configuration deactivated.

Agreements:

Modify the previous agreement with following updates:

- For PDCCH ordered CFRA, the minimum timing gap between PDCCH order reception and Msg1 transmission is

  - N2+BWPSwitching+ Delay,

    - If BWP switching is not required, BWPSwitching=0; otherwise, BWPSwitching is up to RAN4

    - N2 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

    - N2 is dependent on the SCS of Msg1

      - Working assumption: For Msg1 with 1.25 kHz or 5 kHz SCS, calculation of N2 uses 15 kHz SCS.

    - Delay includes at least MAC layer delay in initializing PRACH

      - Value of Delay is FFS (to be decided in RAN1) Delay = 250 us for FR2, Delay = 500 us for FR1

Agreements:

UE does not simultaneously transmit PRACH and PUSCH/PUCCH/SRS in, at least, single CC and in intra-band CA, during any of the following scenarios:

- Same slot

- When the gap between the end of PRACH (PUSCH/PUCCH/SRS) and the start of PUSCH/PUCCH/SRS (PRACH) is less than N symbols

  - N = 2 for 15 kHz and 30 kHz SCS.

  - N = 4 for 60 kHz and 120 kHz SCS.

  - Working assumption: Reference SCS for determining N is the SCS for UL BWP.

- FFS: inter-band CA.

- Transmission of PRACH or SR (on PUCCH) if any, is up to UE implementation.

To draft reply LS based on the above agreements in R1-1805713

Friday

R1-1805713	[Draft] Reply LS response on handling parallel RA procedure and SR procedure	Qualcomm

Decision: The document is endorsed and final LS is approved in R1-1805759.

Agreements:

- To adopt the following TP to 8.1 of 38.213:

[…] The UE is provided a number of contention based preambles per SS/PBCH block per PRACH occasion by the value of higher layer parameter cb-preamblePerSSB and the UE determines a total number of preambles per SSB per PRACH occasion as the multiple of the value of SSB-perRACH-Occasion and the value of cb-preamblePerSSB.

Agreement:

- For the case that N SSBs are associated with one RACH occasion, where N >= 1, the subset of cb-preamblePerSSB consecutive CBRA preambles associated with SSB i (i=0, …, N-1) starts from preamble index i*(64/N).

Working assumption:

For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where

- N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1

- N2 also refers to the value determined in control session with UE capability #1

- L2 refers to the MAC processing latency and is equal to 0.5 msec.

- TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the UL SCS

- Note: Spec captures the value k for different SCS

- UE is not expected that UL transmissions in the same slot shall apply different TA values.

- If this happens, it is up to UE’s implementation.

- FFS: the reference SCS of slot n+k and the time duration of (k-1) slots when UL and DL have different SCSs.

Agreement:

- The parameter cb-preamblePerSSB indicates the number of CBRA preambles per SSB per RACH occasion.

R1-1804457	Remaining details on RACH procedure	Nokia, Nokia Shanghai Bell

Decision: The document is noted.

Tuesday session

R1-1805614	Summary of remaining details on RACH procedure	Qualcomm

Decision: The document is noted.

Agreements:

Change the previous working assumption:

- Time period for SSB to RO association is multiple of PRACH configuration periods

  - The number of multiple PRACH configuration periods in the time period is the smallest value in the set {1, 2, 4} such that the number is ≥ Actually transmitted SSBs / SSBs that can be mapped to one PRACH config period

  - The time period starts from frame 0

And replace it with the following:

- Time period for SSB to RO association is multiple of PRACH configuration periods

  - The number of multiple PRACH configuration periods in the time period is the smallest value in the set determined by the RACH configuration period from following table such that the number is

≥ Actually transmitted SSBs / SSBs that can be mapped to one PRACH config period  such that all actually transmitted SSBs can be mapped, at least once, to the ROs within the time period.

  - The time period starts from frame 0

| RACH configuration period (ms) | Mapping period set (# of RACH configuration period ) |
| --- | --- |
| 10 | {1，2，4，8，16} |
| 20 | {1，2，4，8} |
| 40 | {1，2，4} |
| 80 | {1，2} |
| 160 | {1} |

Agreements: Update the previous agreement on Msg1 retransmission as follows:

- If a received Msg2 does not contain a response to the transmitted preamble sequence, or no RAR is received by the end of the RAR window, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence after no later than the duration of N1 + new + L2.

  - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

  - new = 250 us

  - L2=500 usec refers to the MAC layer processing time

  - Note: UE is not mandated to measure each SSB prior to every Msg1 retransmission

Agreements: Update the previous agreement in the following way:

- From UE perspective, minimum time gap between Msg2 over-the-air reception and Msg3 transmission if Msg2 and Msg3 have the same SCS

- Duration of N1 + duration of N2 + L2 + N_TA + TA

  - N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1

  - N2 also refers to the value determined in control session with UE capability #1

  - TA is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the SCS of Msg3

  - L2 refers to the MAC processing latency and it does not depend on subcarrier spacing

    - L2=500us as a working assumption

  - Note: If Msg2 and Msg3 have different SCS, value of N1 and N2 will refer to the ones determined in control session.

  - Note: If Msg2 and Msg3 have different SCS, value of N1 and N2 – in terms of absolute time - are determined by lower SCS between Msg2 and Msg3

  - Note: Network schedules Msg3 while ensuring the minimum time gap and considering the TA command conveyed in Msg2

Wednesday

R1-1805647	Draft reply LS on Message 3 size for NR	NTT DOCOMO, INC., CMCC, Orange, Deutsche Telekom, Vodafone, AT&T

Decision: The LS is endorsed with the following updates:

- 3 to hundreds of bytes

- Q2: NR PUSCH, based on existing RAN1 evaluations, TB size…

Final LS is approved in R1-1805657.

Thursday

R1-1805701	Summary of remaining details on RACH procedure	Qualcomm

Decision: The document is noted.

Agreements:

- PDCCH order for RACH procedure includes the following fields:

  - Random Access Preamble index – 6 bits. Indicating which Random access preamble to use in case of contention-free random access procedure, or the value 000000 in case of contention-based random access procedure

  - For CFRA only:

    - FFS BWP index. Indicating which BWP to transmit the Random access preamble on

    - SUL indicator – 1 bit. Indicating whether to transmit the Random access preamble on SUL or normal uplink carrier

    - SSB index - 6 bits are used to indicate an SSB index, which, in turn, identifies a group of RACH occasions

    - RACH occasion index - 3 bits are used to indicate the relative RACH occasion index that corresponds to the indicated SSB index identified group of RACH occasions

R1-1805634	Summary of MSG3 timing discussion	Ericsson

Decision: The document is noted.

Proposal for working assumption:

- For the MSG3 PUSCH time domain resource assignment in RAR, the UE uses the default table or table indicated by RMSI for MSG3 PUSCH for the value of K2

  - Note:  FFS: Numerology for K2

- Additional delay is added to K2 to account for N1+L2. The delay is subcarrier specific and given by the specification

  - Note: If Msg2 and Msg3 have different SCS, value of N1 – in terms of absolute time - are determined by lower SCS between Msg2 and Msg3

- Note: Working assumption can be revisited once default table and/or support for RMSI signaling has been decided

Check later whether the above is necessary or not considering the discussion in other sessions

Friday: Revisit till next meeting

R1-1805754	Summary of remaining details on RACH procedure	Qualcomm

Decision: The document is noted. Companies are encouraged to check the default search space for RACH procedure proposals as in R1-1805754

R1-1803610	Remaining details of RACH procedure	ZTE, Sanechips

R1-1803629	Remaining details of RACH Procedures	Huawei, HiSilicon

R1-1803733	Further details on NR 4-step RA Procedure	CATT

R1-1803809	Remaining issues on RACH procedure	vivo

R1-1804032	Collision between PRACH transmission and SR transmission	ASUSTEK COMPUTER (SHANGHAI)

R1-1804092	Discussion on time period for SSB to RO association	CMCC

R1-1804225	Remaining issues on RACH	Spreadtrum Communications

R1-1804349	Corrections on RACH Procedure	Samsung

R1-1804539	RACH Procedure	LG Electronics

R1-1804590	Remaining details on RACH timing for NR	ITL

R1-1804652	Remaining details of RACH Procedures	AT&T

R1-1804707	NR RACH procedures	Intel Corporation

R1-1804877	Remaining issues on RACH procedure	Sharp

R1-1804954	Remaining details on random access procedure	Motorola Mobility, Lenovo

R1-1805036	Remaining issues on RACH procedure	NTT DOCOMO, INC.

R1-1805221	Remaining details of RACH procedure	Ericsson

R1-1805524	Remaining details on RACH procedure	MediaTek Inc.	(revision of R1-1804047)

R1-1805527	Remaining details on RACH procedure	Qualcomm Incorporated	(revision of R1-1804779)

R1-1805573	RACH MSG2 to MSG3 delay	Ericsson

#### Mobility procedure

##### Measurement for mobility management

R1-1805512	Summary of Discussion for NR RRM measurements	Intel

Decision: The document is noted.

Agreement:

- To adopt the following TP for 38.215

| Definition | CSI reference signal received power (CSI-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements that carry CSI reference signals configured for RSRP measurements within the considered measurement frequency bandwidth in the configured CSI-RS occasions. For CSI-RSRP determination CSI reference signals transmitted on antenna port 3000 or antenna ports 3000, 3001 according to 3GPP TS 38.211 [4] shall be used. If CSI-RSRP is used for L1-RSRP, CSI reference signals transmitted on antenna ports 3000, 3001 can be used for CSI-RSRP determination. For frequency range 1, the reference point for the CSI-RSRP shall be the antenna connector of the UE. For frequency range 2, CSI-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported CSI-RSRP value shall not be lower than the corresponding CSI-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | If CSI-RSRP is used for L1-RSRP, RRC_CONNECTED intra-frequency. Otherwise, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

Wednesday session

R1-1805632	Summary of Offline Discussion for NR RRM measurements	Intel

Decision: The document is noted.

Agreements:

Send an LS to RAN4 – R1-1805649

- RAN1 has discussed following proposals regarding Rx beam selection for RRM measurements in the context of requiring the UE to provide more stable measurements and request that RAN4 take the discussion into account in their work.

  - Measurement to be reported is the best among the measurements based on each RX beam in the selected set

  - Measurement to be reported shall be greater than average of measurements based on each RX beam in the selected set

  - The selection of Rx beam set to perform measurement on carrier is left to the UE implementation with the limitation that the same Rx beam set is used to measure the same carrier

- RAN1 believes the selection and/or definition of the Rx beam for measurement should be determined by RAN4, and have not concluded on a specific proposal.

- The following are related agreement from RAN1 2017 Adhoc #3:

Friday

R1-1805649	[DRAFT] LS on Rx beam selection for RRM measurements	Intel

Decision: The document is endorsed and final LS is approved in R1-1805760.

Agreements:

- For a SS-RSRQ measurement, the same RX beam shall be applied between RSSI measurement and RSRP measurement

- For a CSI-RSRQ measurement, the same RX beam shall be applied between RSSI measurement and RSRP measurement

- It is up to UE implementation how to select a set of RX beams to perform RRM measurement on a carrier

  - Different sets of RX beams can be used in measurements based on different measurement objects

- Same set of RX beams shall be used in measurement of each TX beam based on a measurement object

- FFS

  - Alt.1: Measurement to be reported shall be greater than average of measurements based on each RX beam in the selected set

  - Alt.2: Measurement to be reported shall be the best among measurements based on each RX beam in the selected set

  - Other alternatives are not precluded

Agreement:

- NR does not support different measurement BWs for SS-RSRP and SS-RSSI for Rel-15.

Agreements:

- For reception of SSB for the target PSCell, frequency indication will be required for SSB not on the synchronization signal raster.

- Send LS to RAN2 (R1-1805651) and ask RAN2 to provide feedback if any

R1-1805651	[DRAFT] LS on SSB frequency location of Pscell	Intel

Decision: The document is endorsed and final LS is approved in R1-1805717, with a note asking RAN2 to provide feedback if any.

Proposals:

- For intra-frequency measurement and if measurement gap is not configured, UE is not expected to

  - Measure the CSI-RS resources outside the active DL BWP when the SCS for CSI-RS and the SCS for the active DL BWP are the same, and

  - Measure the CSI-RS resources whose SCS is different from that of active DL BWP

- It is RAN1’s understandings that when the UE performs CSI-RS for L3 mobility measurement with CSI-RS numerology different from the numerology of active DL BWP, UE requires measurement gap configuration in order perform measurements of the CSI-RS for L3 mobility

Conclusion:

- It is up to RAN4 to define associated UE behavior in context of reception of PUSCH/PDSCH during measurements of CSI-RS resource with same/different numerology with PUSCH/PDSCH.

Thursday session

R1-1805696	Summary of Offline Discussion for NR RRM measurements	Intel

Decision: The document is noted.

Agreement:

- For intra-frequency measurement and if measurement gap is not configured, UE is not expected to measure the CSI-RS resources outside the active DL BWP.

Conclusion:

- It is RAN1’s understandings that UE, which does not support simultaneous reception of multiple numerologies, requires measurement gap configuration in order to perform measurements of the CSI-RS for L3 mobility for intra-frequency measurements when the numerology of CSI-RS resource is different from the active DL BWP numerology.

Agreements:

- For FR1: support alt 1 with total maximum number of CSI-RS for L3 mobility as 64.

- For FR2: Support alt 2.

- Alt 1 & Alt 2

  - Alt 1) NR supports configuration of mixture of CSI-RS resources with associated SSB and without associated SSB in the same measurement object (e.g. same carrier frequency)

  - Alt 2) NR does not supports configuration of mixture of CSI-RS resources with associated SSB and without associated SSB in the same measurement object (e.g. same carrier frequency)

Agreements:

- For intra-frequency measurements, SS-RSSI configured by higher layers is measured with timing reference from serving cell in the frequency layer.

- For inter-frequency measurements, SS-RSSI configured by higher layers is measured with timing reference from any one of the cells in the target frequency layer

Friday

R1-1805736	Summary of Offline Discussion for NR RRM measurements	Intel

Decision: The document is noted.

Agreements:

- Send Reply LS to RAN2 and inform that agreement 5 of R1-1805264 is not aligned with RAN1 understanding and therefore should be reconsidered.

  - It is RAN1 understanding that useServingCellTimingForSync parameter is only applicable for SSB based RRM measurements and should not be coupled with configuration of CSI-RS based RRM measurements.

  - Additionally, the configuration of SSB based RRM measurement should not be a condition for configuring CSI-RS based RRM measurement with associated SSB.

  - In case assocatedSSB is configured for CSI-RS based RRM measurement, frequency information of the SSB (to be associated with CSI-RS) is required. For example, ssbFrequency in MO.

R1-1805756	Draft reply LS on RAN2 agreements on RRM	Huawei

Decision: The document is endorsed assuming updates to fix the typo “assocated”, & “agreement 5 in R2-1804103”. Final LS is approved in R1-1805762

R1-1803611	Remaining details of RRM measurements	ZTE, Sanechips

R1-1803630	Remaining details on NR RRM	Huawei, HiSilicon

R1-1803734	Remaining Issues on NR Mobility Management	CATT

R1-1803810	Remaining issues on RRM measurement	vivo

R1-1803993	Remaining details on NR RRM	OPPO

R1-1804041	Remaining issues on RRM	MediaTek Inc.

R1-1804350	Corrections on Measurements for Mobility Management	Samsung

R1-1804540	Remaining Details on L3 measurement and mobility management	LG Electronics

R1-1804708	Remaining details of RRM measurements	Intel Corporation

R1-1804780	Remaining details on measurement for mobility management	Qualcomm Incorporated

R1-1805037	Remaining issues on Measurement for mobility management	NTT DOCOMO, INC.

R1-1805139	Remaining issues on mobility measurements	Nokia, Nokia Shanghai Bell

R1-1805222	Remaining issues on mobility measurements	Ericsson

##### Radio link monitoring for mobility management

R1-1805513	Summary of Discussion for NR Radio Link Monitoring	Intel

Decision: The document is noted.

Wednesday

R1-1805608	Summary of Offline Discussion for NR Radio Link Monitoring	Intel

Decision: The document is noted.

Agreements:

- To reply the LS from RAN2 in R1-1803577

Answer 1:

- The maximum number of BFD-RS(s) is 2 per BWP.

- The maximum number of RLM-RS(s) and BFD-RS(s) should depend on whether same RS(s) is shared between RLM and BFD. The maximum number of unique RS(s), each RS using different set of resources, for both RLM-RS(s) and BFD-RS(s) are:

  - X RS(s) per BWP for below 3 GHz,

      - FFS X – to conclude this week - X=2(working assumption)

  - Y RS(s) per BWP for above 3 GHz and below 6 GHz,

      - FFS Y – to conclude this week - Y=6(working assumption)

  - Z RS(s) per BWP for above 6 GHz,

      - FFS Z – to conclude this week - Z=8(working assumption)

  - where maximum number of BFD-RS(s) is 2 RSs per BWP and maximum number of RLM-RS(s) is 2 RSs per BWP for below 3 GHz, 4 RSs per BWP for above 3 GHz and below 6 GHz, 8 RSs per BWP for above 6 GHz.

- Please note that support of 8 RLM-RSs and 2 BFD-RS for above 6 GHz is feasible if the 2 BFD-RS are a subset of 8 RLM-RSs.

Answer 2:

- Yes. They can be completely orthogonal, depending on NW configuration.

Draft reply LS to R1-1803577 in R1-1805718.

Friday

R1-1805718	[DRAFT] Reply LS on RAN1 on Beam- and Cell- Radio-Link-Monitoring	Intel

Decision: The document is endorsed and final LS is approved in R1-1805761.

Agreements:

- Further clarification of RAN2 agreement (will require additional physical layer text proposals):

  - Working assumption: If the TCI-states refer to CSI-RS for tracking, it is up to UE to select a NZP-CSI-RS resource from the configured resources for CSI-RS for tracking for RLM

  - FFS on the UE behavior when TCI-states indicate a combination of SSB, CSI-RS, and CSI-RS for tracking

Proposal: Agree to the following text proposal

=== Text Proposal Starts 38.213 section 5 ===

--- parts that are not affected are omitted ---

A UE can be configured for each SpCell [11, TS 38.321] with a set of resource indexes for radio link monitoring by higher layer parameter RadioLinkMonitoringConfig RLM-RS-List. The UE is provided by higher layer parameter failureDetectionResources RLM-RS an association between a resource index, from the set of resource indexes, with either a CSI-RS resource configuration or a SS/PBCH block. For a CSI-RS resource configuration, the UE is provided a corresponding index by higher layer parameter csi-RS-IndexRLM-CSIRS. The higher layer parameters CSI-IM-RE-pattern, CSI-IM-Resource, CSI-IM-ResourceId, CSI-IM-timeConfig, CSI-IM-FreqBand, CSI-IM-ResourceMapping, and Pc_SS in the CSI-RS configuration are not applicable. In CSI-RS resource configuration, a UE expects to be provided only ‘No CDM’ from higher layer parameter CDM-Type, only ‘1’ and ‘3’ from higher layer parameter density, and only ‘1 port’ from higher layer parameter nrofPorts [6, TS 38.214]. For a SS/PBCH block, the UE is provided a corresponding index by higher layer parameter ssb-Index RLM-SSB.

If the UE is not provided with higher layer parameter RadioLinkMonitoringRS list for the purpose of radio link monitoring, the UE determines the set of failureDetectionResources to include SS/PBCH block indexes and CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by the active TCI states for respective control resource sets that the UE is configured for monitoring PDCCH.

Thursday

R1-1805697	Summary of Offline Discussion for NR Radio Link Monitoring	Intel

Decision: The document is noted.

Conclusion:

- It is RAN1 understanding that each CSI-RS resource can be configured with QCL-info independently, and in general UE may not assume that different reference signals have QCL relationship unless configured. No further clarification is needed on the QCL assumption for RLM-RSs.

Friday

R1-1805737	Summary of Offline Discussion for NR Radio Link Monitoring	Intel

Decision: The document is noted.

R1-1803612	Remaining details of RLM	ZTE, Sanechips

R1-1803631	Remaining details on NR RLM	Huawei, HiSilicon

R1-1803735	Remaining Issues on NR Radio Link Monitoring	CATT

R1-1803994	Remaining details on NR RLM	OPPO

R1-1804042	Remaining issues on RLM	MediaTek Inc.

R1-1804351	Corrections on Radio Link Monitoring for Mobility Management	Samsung

R1-1804541	Discussion on Radio Link Monitoring in NR	LG Electronics

R1-1804653	Impact of beam failure and recovery on RLM procedures	AT&T

R1-1804709	Remaining details of RLM	Intel Corporation

R1-1804781	Radio link monitoring consideration	Qualcomm Incorporated

R1-1804955	Remaining details on radio link monitoring	Motorola Mobility, Lenovo

R1-1805038	Remaining issues on RLM for mobility management	NTT DOCOMO, INC.

R1-1805140	Remaining Details on Radio Link Monitoring	Nokia, Nokia Shanghai Bell

R1-1805223	Remaining issues for radio link monitoring	Ericsson

#### Other

Including TA related aspects, if any leftover issues

R1-1805745	Summary of TA related aspects	Panasonic

R1-1805757	Summary of TA related aspects	Panasonic

Proposal:

- to revise the working assumption as following.

  - For the timing advance in MAC-CE in the case with multiple configured UL BWPs in the TAG, the granularity depends on the maximum SCS of all the activated UL BWPs including SUL within the TAG.

    - When the TA is adjusted based on the timing advance in MAC-CE in higher SCS, the TA in lower SCS may be rounded in order to align with lower SCS TA granularity while satisfying the timing advance accuracy requirements in [10, TS38.133].

    - UE keeps track of the accumulation of rounding errors. UE adjusts its uplink timing to compensate for the accumulated rounding error and to satisfy the requirement.

    - This may result in different timing across UL carriers within the same TAG

Agreement:

- For TDD+FDD CA, in a TAG without PCell or PSCell, UE should adjust its uplink transmission timing of all SCells in the TAG using the largest TA offset value within the TAG

R1-1803650	On RMSI PRB grid	Huawei, HiSilicon

R1-1803651	On k0 for OFDM signal generation	Huawei, HiSilicon

R1-1803703	Remaining issues on TA aspects	Huawei, HiSilicon

R1-1804048	Remaining TA related issues	MediaTek Inc.

R1-1804310	Draft CR on TA of Pcell of EN-DC with SUL on chapter 4.2 in TS38.213	Huawei, HiSilicon

Withdrawn

R1-1804352	Corrections on Timing Advance	Samsung

R1-1804782	Remaining details on Timing advance granularity and adjustment consideration	Qualcomm Incorporated

R1-1804949	What is "initial access" in RAN1?	Panasonic

R1-1805224	On multiple preamble transmissions for contention free random access	Ericsson

R1-1805225	L1 reporting on other cells	Ericsson

### MIMO

R1-1805711	Chairman's notes of AI 7.1.2 NR MIMO	Ad-Hoc chair (Samsung)

Decision: The document is endorsed, content incorporated below.

R1-1804965	On DMRS groups in Rel-15	Ericsson

#### Multi-antenna scheme

R1-1804966	Corrections for UL Antenna Port Identification	Ericsson

##### Codeword mapping

R1-1804354	Summary of open issues on codeword mapping	Samsung

Agreement

Text proposal for TS38.211 section 6.3.1.1

- equals the higher-layer parameter Data-scrambling-Identity if configured and the RNTI equals the C-RNTI or CS-RNTI,

Agreement

Text proposal for TS38.211 section 7.3.1.1

- equals the higher-layer parameter Data-scrambling-Identity if configured and the RNTI equals the C-RNTI or CS-RNTI

Agreement

Text proposal for Section 7.3.1.2.2 of 38.212

------------------------------------------- Start of Text Proposal -------------------------------------------

7.3.1.2.2	Format 1_1

< Unchanged parts are omitted >

-	Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports  shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4.

< Unchanged parts are omitted >

Agreement

For TB to CW mapping:

- 2-TB: TB1  CW0, TB2  CW1

- 1-TB: always mapped to CW0

Section 5.1.3.2 of TS38.214

In case the higher layer parameter Number-MCS-HARQ-DL-DCI indicates that two codeword transmission is enabled, then a transport block is disabled by DCI format 1_1 if IMCS=26 and if rvid=1 for the corresponding transport block, otherwise the transport block is enabled.

For the PDSCH assigned by a PDCCH with DCI format 1_0/1_1 with CRC scrambled by C-RNTI, if the higher layer parameter MCS-Table-PDSCH is set to '256QAM' is configured and  , or the higher layer parameter MCS-Table-PDSCH is not set to '256QAM' configured and  , the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below:

R1-1803632	Remaining issues on CW mapping	Huawei, HiSilicon

R1-1803736	Remaining issues on codeword mapping	CATT

R1-1803811	Remaining issues on layer mapping	vivo

R1-1804353	Issues on codeword mapping	Samsung

R1-1804654	Remaining issue for MIMO Codeword mapping	AT&T

R1-1804878	Open issues on codeword mapping	Sharp

R1-1804967	Remaining issues on CW mapping and data scrambling	Ericsson

R1-1805100	Remaining Issue on DMRS Port Mapping and its Impact on CQI/MCS	Nokia, Nokia Shanghai Bell

##### Codebook based transmission for UL

R1-1805564	Summary of Codebook Based UL Transmission	Intel

Agreement

Adopt the following TP.

/************************* Start of Text Proposal for 38.212 **************************/

7.3.1.1.2		Format 0_1

-	SRS resource indicator – or  bits, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter SRS-SetUse of value 'CodeBook' or 'NonCodeBook', and  is the maximum number of supported layers for the PUSCH.

-	 bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter ulTxConfig = NonCodeBook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter SRS-SetUse of value 'NonCodeBook';

-	 bits according to Tables 7.3.1.1.2-32 if the higher layer parameter ulTxConfig = CodeBook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter SRS-SetUse of value 'CodeBook'.

-	Precoding information and number of layers – number of bits determined by the following:

-    0 bits if the higher layer parameter ulTxConfig = NonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter ulTxConfig = Codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if ulTxConfig = Codebook, and according to the values of higher layer parameters PUSCH-tp, ULmaxRank, and ULCodebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if ulTxConfig = Codebook, and according to the values of higher layer parameters PUSCH-tp, ULmaxRank, and ULCodebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if ulTxConfig = Codebook, and according to the values of higher layer parameters ULmaxRank and ULCodebookSubset;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if ulTxConfig = Codebook, and according to the values of higher layer parameters ULmaxRank and ULCodebookSubset.

-	Antenna ports – number of bits determined by the following

-	2 bits as defined by Tables 7.3.1.1.2-6, if PUSCH-tp=Enabled, UL-DMRS-config-type=1, and UL-DMRS-max-len=1;

-	4 bits as defined by Tables 7.3.1.1.2-7, if PUSCH-tp=Enabled, UL-DMRS-config-type=1, and UL-DMRS-max-len=2;

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if PUSCH-tp=Disabled, UL-DMRS-config-type=1, and UL-DMRS-max-len=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter ulTxConfig = NonCodeBook and according to the Precoding information and number of layers field if the higher layer parameter ulTxConfig = CodeBook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if PUSCH-tp=Disabled, UL-DMRS-config-type=1, and UL-DMRS-max-len=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter ulTxConfig = NonCodeBook and according to the Precoding information and number of layers field if the higher layer parameter ulTxConfig = CodeBook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if PUSCH-tp=Disabled, UL-DMRS-config-type=2, and UL-DMRS-max-len=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter ulTxConfig = NonCodeBook and according to the Precoding information and number of layers field if the higher layer parameter ulTxConfig = CodeBook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if PUSCH-tp=Disabled, UL-DMRS-config-type=2, and UL-DMRS-max-len=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter ulTxConfig = NonCodeBook and according to the Precoding information and number of layers field if the higher layer parameter ulTxConfig = CodeBook

/************************* End of Text Proposal for 38.212 **************************/

Agreement

Text alignment between section 6.1.2.3 of 38214 and the UL MIMO parts in section 6.1.1 in terminology.  Remove references to TPMI, TRI, etc, and use the DCI field names from 38.212 as section 6.1.2.3 does now. 38.214 editor to make necessary changes.

R1-1803738	Remaining issues on codebook based UL transmission	CATT

R1-1803812	Remaining issues on codebook based UL transmission	vivo

R1-1803967	Remaining issues on codebook based UL transmission	OPPO

R1-1804202	Remaining issues on codebook based UL transmission	Lenovo, Motorola Mobility

R1-1804278	Summary of remaining issues for codebook based transmission for UL MIMO	Huawei, HiSilicon

R1-1804355	Issues on codebook based UL transmisison	Samsung

R1-1804710	On Codebook based transmission for UL	Intel Corporation

R1-1804783	Maintenance for codebook based UL transmission	Qualcomm Incorporated

R1-1804968	Corrections for Codebook Based UL MIMO	Ericsson

R1-1805101	Remaining issues on UL codebook based transmission	Nokia, Nokia Shanghai Bell

##### Non-codebook based transmission for UL

R1-1805549	FL Summary of issues on UL non-codebook based transmission	Nokia, NSB

R1-1805603	Proposal on aperiodic CSI-RS for non-codebook-based transmission	Samsung, Huawei, HiSilicon, Nokia, NSB, CATT, NTT DOCOMO, CHTTL, Verizon Wireless, KDDI, Spreadtrum, KT, WILUS Inc., ASUSTek

Agreement

Text proposal for TS38.214 Section 6.1.1.2

6.1.1.2	Non-Codebook based UL transmission

For non-codebook based transmission, the UE can determine its PUSCH precoder and transmission rank based on the wideband SRI field from the DCI. The UE shall use one or multiple SRS resources for SRS transmission, where the number of SRS resources which can be configured to the UE for simultaneously transmission in the same RBs is being part of UE capability signalling Only one SRS port for each SRS resource is configured. Only one SRS resource set can be configured with higher layer parameter srsSetUse set to ‘nonCodebook’. The maximum number of SRS resources that can be configured for non-codebook based uplink transmission is 4. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI before slot n.

For non-codebook based transmission, the UE can measure NZP CSI-RS resource to calculate the precoder used for the transmission of precoded SRS. A UE can be configured with only one NZP CSI-RS resource for the SRS resource set.

-	If aperiodic SRS resource is configured, the [CSI-RS information in the same slot TBD] for UL channel measurement is indicated via DCI, where the association among aperiodic SRS triggering state, triggered SRS resource(s) SRS-ResourceConfigId, CSI-RS resource ID NZP-CSI-RS-ResourceConfigID are higher layer configured by AperiodicSRS-ResourceTrigger. A UE may receive the dynamic SRS transmission request for aperiodic SRS transmission in the same slot as the reception of the DL CSI-RS resource. A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the AP-CSI-RS resource and the first symbol of the AP-SRS transmission is less than 42 OFDM symbols. A UE may receive the SRI which shall be associated with the most recent SRS transmission.

-	If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceConfigID for measurement is indicated via higher layer parameter SRS-AssocCSIRS per set.

R1-1805722	Offline discussion summary on non-codebook based transmission	Nokia, NSB

Agreement

When the UE is configured with AP-SRS and AP-CSI-RS associated with the AP-SRS for non-codebook based UL transmission,

- Any of the TCI states in the scheduled CC should not be configured with QCL Type D.

- AP-CSI-RS is located in the same slot as the AP-SRS trigger/request.

- The presence of AP-CSI-RS is (implicitly) indicated via the SRS request field in the DCI.

- The QCL assumption for receiving the associated AP-CSI-RS for non-codebook-based transmission is up to UE implementation

- Note: This does not preclude the use of the CSI request field to trigger this AP CSI-RS for DL CSI acquisition

R1-1803633	Summary of remaining issues for UL non-codebook based transmission	Huawei, HiSilicon

R1-1803739	Remaining issues on non-codebook based UL transmission	CATT

R1-1803813	Remaining issues on non-codebook based UL transmission	vivo

R1-1803905	Remaining issues on non-codebook based UL transmission	ZTE, Sanechips

R1-1803968	Remaining issues on non-codebook based UL transmission	OPPO

R1-1804056	Non-codebook based transmission for UL	MediaTek Inc.

R1-1804203	Remaining issues on non-codebook based UL transmission	Lenovo, Motorola Mobility

R1-1805510	Open issues on aperiodic CSI-RS for non-codebook-based transmission	Samsung	(revision of R1-1804356)

R1-1804542	Corrections on non-codebook-based UL	LG Electronics

R1-1804711	On non-codebook based transmission for UL	Intel Corporation

R1-1804784	Maintenance for non-codebook based UL transmission	Qualcomm Incorporated

R1-1804843	On Operation of Non-CB based UL Transmission 	InterDigital, Inc.

R1-1804969	Corrections for Non-Codebook Based UL MIMO	Ericsson

R1-1805039	Remaining details on non-codebook based UL transmission	NTT DOCOMO, INC.

R1-1805102	Remaining issues on non-codebook based UL-MIMO transmission	Nokia, Nokia Shanghai Bell

Late submission

##### PRB bundling for DL

R1-1805582	Summary on Issues of PRB bundling for DL	vivo

Agreement

- In case UE is scheduled by DCI 1_0, the higher layer parameter prb-BundlingType is set to 'static', and when receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI,

  - the value of  is fixed as 2 PRBs.

Agreement

Text Proposal for TS 38.214

5.1.2.3	Physical resource block (PRB) bundling

< Unchanged parts are omitted >

A UE may assume that precoding granularity is  consecutive resource blocks in the frequency domain.  can be equal to one of the values among {2, 4, wideband}.

If  is determined as one of the values among {2, 4}, Precoding Resource Block Group (PRGs) partition the carrier bandwidth part i with  consecutive PRBs  can be equal to one of the values among {2, 4, wideband}. Actual number of consecutive PRBs in each PRG could be one or more.

for each carrier bandwidth part is equal to 2 PRBs unless configured by the higher layer parameter prb-BundlingType. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that  is equal to 2 PRBs.

If  is determined as "wideband", the UE is not expected to be scheduled with non-contiguous resource allocation and the UE may assume that the same precoding is applied to the allocated resource.

When  is determined as one of the values among {2, 4}, the first PRG size is given by  and the last PRG size given by  if  , and the last PRG size is if .

The UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a PRG partitioned above.

For PDSCH carrying SIB1 scheduled by PDCCH with CRC scrambled by SI-RNTI, PRG is partitioned from the lowest numbered resource block of the CORESET signalled in PBCH.

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI,  for each carrier bandwidth part is equal to 2 PRBs unless configured by the higher layer parameter prb-BundlingType. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that  is equal to 2 PRBs. In case UE is scheduled by DCI 1_0, the higher layer parameter prb-BundlingType is set to 'static', and when receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, the value of  is fixed as 2 PRBs.

If the higher layer parameter prb-BundlingType is set to 'dynamic', and the UE is scheduled a PDSCH with DCI format 1_0 scrambled with C-RNTI and CS-RNTI, then the UE shall assume that  is equal to 2 PRBs.

If the higher layer parameter prb-BundlingType is set to 'dynamic', the higher layer parameter bundleSizeSet1 and bundleSizeSet2 configures two sets of  values, the first set includes one or two  values among {2, 4, wideband}, and the second set includes one  value. The UE is not expected to be configured with (2, 4) in the first set.

If the PRB bundling size indicator signalled in DCI format 1_1 as defined in Subclause 7.3.1.2.2 of [2, TS 38.212]

-	is set to '0', the UE shall use the  value from the second set of PRG  values when receiving PDSCH scheduled by the same DCI.

-	is set to '1' and one value is configured for the first set of  values, the UE shall use this  value when receiving PDSCH scheduled by the same DCI

-	is set to '1' and two values are configured for the first set of  values as ‘n2-wideband’ (corresponding to two  values 2 and wideband) or ‘n4-wideband’ (corresponding to two  values 4 and wideband), the UE shall use the value when receiving PDSCH scheduled by the same DCI as follows:

-	If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than ,  is the same as the scheduled bandwidth, otherwise  is set to the remaining configured value of 2 or 4, respectively.

If the higher layer parameter prb-BundlingType is set to 'static', the  value is configured with the single value indicated by the higher layer parameter bundleSize.

When a UE is configured with RBG = 2  for bandwidth part i  according to Section 5.1.2.2.1, or when a UE is configured with resource block bundle size of 2 for VRB to PRB mapping provided by the higher layer parameter VRB-to-PRB-interleaver for bandwidth part i, the UE is not expected to be configured with = 4.

< Unchanged parts are omitted >

Agreement

The following text proposal for TS38.212 section 7.3.1.2.2 Format 1_1

< Unchanged parts are omitted >

-	PRB bundling size indicator – 0 bit if the higher layer parameter prb-BundlingType PRB_bundling is not configured or is set to ‘static’, or 1 bit if the higher layer parameter prb-BundlingType PRB_bundling is set to ‘dynamic’, according to Subclause 5.1.2.3 of [6, TS38.214].

< Unchanged parts are omitted >

R1-1803740	Remaining issues on PRB bundling	CATT

R1-1803814	Remaining issues on PRB bundling	vivo

R1-1803906	Remaining issues on PRB bundling	ZTE, Sanechips

R1-1803999	Text Proposal for DL PRB Bundling	OPPO

R1-1804072	Remaining issues on PRB bundling	MediaTek Inc.

R1-1804970	Remaining issues on DL PRB bundling	Ericsson

##### Other

R1-1804447	Antenna selection for PUSCH	Huawei, HiSilicon

R1-1804971	UL precoder classification	Ericsson

#### CSI acquisition and beam management

R1-1803741	On multiplexing of signals and physical channels	CATT

##### CSI measurement

R1-1805552	Summary of remaining issues on CSI measurement	ZTE, Sanechips

Agreement:

- Support 3ms as MAC CE latency for activation/deactivation of SP CSI on PUCCH, SP CSI-RS and trigger states of AP-CSI/CSI-RS. Note that MAC CE latency is the time interval from the slot UE transmits the ACK corresponding to PDSCH carrying the MAC-CE message to the time that the MAC-CE message is applied.

Agreement:

- Periodicities/slot offsets for P/SP CMR and IMR are configured independently

Agreement:

- From a UE’s perspective, the same CSI-RS/CSI-IM resource ID cannot be configured in multiple CSI Resource settings with different time-domain behaviors.

R1-1805661	[Draft] LS on time domain behavior of CSI-RS and CSI-IM	ZTE, Sanechips

Decision: The document is endorsed and final LS is approved in R1-1805621.

Conclusion:

- 1 slot periodicity for CSI-IM is not supported in NR Rel-15.

R1-1805606	Summary of offline discussion on CSI measurement	ZTE, Sanechips

Agreement

- Aperiodic triggering offset of CSI-IM follows triggering offset value of the associated NZP CSI-RS for channel measurement.

R1-1803634	Summary of remaining issues on CSI measurement	Huawei, HiSilicon

R1-1803742	Remaining issues on CSI measurement	CATT

R1-1803815	Remaining issues on CSI measurement	vivo

R1-1803907	Remaining issues on CSI measurement	ZTE, Sanechips

R1-1803981	Remaining issues on CSI measurement	OPPO

R1-1804712	Remaining issues on interference measurement for CSI	Intel Corporation

R1-1805267	Remaining  issues  on CSI measurement	Ericsson	(revision of R1-1804972)

R1-1805521	Maintenance for CSI measurement	Qualcomm Incorporated	(revision of R1-1804785)

##### CSI reporting

R1-1805551	Summary of views on CSI reporting v2	Ericsson

Agreement

What is the time offset of the CSI reference resource of P / SP CSI report?

- A fixed time offset,  slots, is used for a given numerology

  - N=4 for 15kHz, N=8 for 30kHz, N=16 for 60kHz, N=32 for 120kHz in case of CSI report without CRI

  - N=5 for 15kHz, N=10 for 30kHz, N=20 for 60kHz, N=40 for 120kHz in case of CSI report with CRI

  - FFS: Applicability of smaller value of N depending on UE capability

Agreement

- For CSI latency requirement when an A-CSI trigger state triggers multiple CSI reports, up to X CSI reports are assumed to be calculated in a given time period.

  - FFS: A UE is not expected to update a CSI report if the PUSCH scheduling offset does not give enough CSI calculation time according to the Z’ values for that report

  - UE ignores a DCI scheduling a PUSCH with scheduling offset smaller than the any of the Z values in different reports

    - FFS: Details in case A-CSI is triggered for different numerologies

  - Value of X depends on UE capability and takes into account all the CCs

  - The above applies for the case of CSI only transmission

Agreement

- For PUSCH-based SP-CSI reporting triggered by DCI, the timing offset of the initial report after the trigger should meet the PUSCH preparation procedure time (Z) for A-CSI reporting

R1-1805692	Summary of views on CSI reporting v8	Ericsson

Agreement:

- A UE capable of X simultaneous CSI calculations (according to capability 2-35) is said to have X CSI processing units. For aperiodic CSI report using AP CSI-RS (with a single CSI-RS resource in the resource set for channel measurement).

  - the CSI processing unit remains occupied from the first OFDM symbol after the PDCCH trigger until the last OFDM symbol of the PUSCH carrying the CSI report (working assumption)

- If N AP CSI reports (each with a single CSI-RS resource in the resource set for channel measurement) are triggered in a slot, but the UE only has M un-occupied CSI processing units, UE is only required to update M of the N CSI reports

  - FFS if a rule is needed which CSI reports are required to be update or if it’s up to the UE

- FFS if a CSI report linked with Ks > 1 CSI-RS resources for channel measurement occupies Ks CSI processing units or one CSI processing units

Agreement

- Introduce a new UE capability on support of either “Type A CSI processing capability” or “Type B CSI processing capability” with regard to the number of simultaneous CSI calculations X

  - For CSI latency requirement when an A-CSI trigger state triggers N CSI reports (where each report n is associated with (Zn, Z’n)) and have M un-occupied CSI processing units:

    - For Type A CSI processing capability:

      - A UE is not expected to update any of the triggered CSI reports if the time gap between the first symbol of PUSCH and the last symbol of the associated ap-CSI-RS / ap-CSI-IM does not give enough CSI calculation time according to  Z'TOT=n=1MZn'

        - FFS how to index the M reports in this case to form Z'TOT

      - UE may ignore a DCI scheduling a PUSCH with scheduling offset smaller than ZTOT=n=1MZn

        - This applied to CSI only case, FFS for CSI+UL-SCH case

    - For Type B CSI processing capability:

      - A UE is not expected to update a CSI report if the PUSCH scheduling offset does not give enough CSI calculation time according to the Z’ values for that report

      - UE may ignore a DCI scheduling a PUSCH with scheduling offset smaller than the any of the Z values in different reports

        - This applied to CSI only case, FFS for CSI+UL-SCH case

  - CSI reports based on P / SP CSI-RS are assigned to CSI processing units as follows:

    - Type A:

      - For P / SP CSI reports, the CSI processing unit is occupied from the first symbol of the CSI reference resource of the P / SP report until the first symbol of the physical channel carrying the report

      - For A CSI reports, the CSI processing unit is occupied from the first symbol after the PDCCH triggering the report until the [first or last] symbol of the PUSCH carrying the report

    - Type B:

      - A periodic or aperiodic CSI reporting setting associated with P/SP CSI-RS is assigned one (or Ks) CSI processing unit and always occupy them

      - An activated SP-CSI report setting is assigned one (or Ks) CSI processing unit and occupies them until deactivated

        - Once the SP-CSI report is deactivated, the CSI processing unit can be used for other CSI report

  - Note: Type A assumes serial CSI processing implementation while Type B assume parallel CSI processing implementation. Note that this will not be captured in specification.

Agreement:

- For Type A CSI processing capability, a UE is not expected to update P/SP CSI reports if the number of CSI processing units occupied by P/SP CSI reports exceeds the number of simultaneous CSI calculations X according to the UE capability

Agreement

- When a CSI report is linked with Ks > 1 CSI-RS resources for channel measurement is triggered

  - For Type A CSI processing capability:

    - The CSI report occupies a single CSI processing unit and the latency requirement is calculated based on (Zn, Z’n) according to the High latency CSI class

  - For Type B CSI processing capability, downselect between the following in RAN1#93:

    - Alt1: The CSI report occupies a single CSI processing unit and the latency requirement is calculated based on (Zn, Z’n) according to the High latency CSI class

    - Alt2: The CSI report occupies Ks CSI processing units and the latency requirement for each unit is calculated based on (Zn, Z’n) according to the High latency CSI class

    - Companies are encouraged to provide their preference on either of the above alternatives and relevant (Zn, Z’n) values

  - Note: If more than one CSI-RS resource for CM is configured for a CSI report, the report belongs to the High Latency CSI class

- Assuming Type A CSI processing capability or Type B if Alt 1 above is agreed:

  - If a CSI report is linked with Ks > 1 CSI-RS resources for channel measurement

    - For Ks=2, at most 16 CSI-RS ports per CSI-RS resource can be configured

    - For 8>=Ks>2, at most 8 CSI-RS ports per CSI-RS resource can be configured

Agreement

- Adopt the following (Z,Z’) values for Low Latency CSI class for normal UE capability for the Type A and Type B UE CSI processing capabilities:

For normal UE capability (assuming single CSI report, single CMR/IMR)

|  |  |  | For Type A CSI processing capability |  | For Type B CSI processing capability |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CSI only | CSI+UL-SCH | CSI only | CSI+UL-SCH |
| Low latency CSI | 15 kHZ | Z | 5 | 6 | 22 | 32 |
|  |  | Z' | 3 | 4 | 15 | 25 |
|  | 30 kHZ | Z | 9 | 10 | 25 | 37 |
|  |  | Z' | 6 | 7 | 16 | 28 |
|  | 60 kHZ | Z | 17 | 18 | 34 | 57 |
|  |  | Z' | 12 | 13 | 25 | 48 |
|  | 120 kHZ | Z | 27 | 30 | 49 | 85 |
|  |  | Z' | 22 | 25 | 37 | 73 |
| High latency CSI | 15 kHZ | Z |  |  |  |  |
|  |  | Z' |  |  |  |  |
|  | 30 kHZ | Z |  |  |  |  |
|  |  | Z' |  |  |  |  |
|  | 60 kHZ | Z |  |  |  |  |
|  |  | Z' |  |  |  |  |
|  | 120 kHZ | Z |  |  |  |  |
|  |  | Z' |  |  |  |  |

Agreement

- When DL BWP is switched, PUCCH-based SP-CSI report setting in activated state stays in its activated state. SP-CSI reporting is simply suspended until the DL BWP is switched back, whereon it resumes

R1-1803635	Summary of remaining issues on CSI reporting	Huawei, HiSilicon

R1-1803743	Remaining issues on CSI reporting	CATT

R1-1803816	Remaining issues on CSI reporting	vivo

R1-1804014	Remaining issues on CSI reporting	OPPO

R1-1804057	Remaining issues in CSI reporting	MediaTek Inc.

R1-1804093	Discussion on remaining issues on semi-persistent CSI	CMCC

R1-1804223	Remaining issues on CSI reporting	Spreadtrum Communications

R1-1804357	Issues on CSI reporting	Samsung

R1-1804499	Remaining details on CSI Reporting	MTI

R1-1804543	Remaining issues on CSI reporting	LG Electronics

R1-1804655	Remaining issue for CSI Reporting	AT&T

R1-1804713	Remaining issues on CSI reporting	Intel Corporation

R1-1804786	Maintenance for CSI reporting	Qualcomm Incorporated

R1-1804844	Remaining issues on CSI reporting 	InterDigital, Inc.

R1-1804879	Clarification on aperiodic CSI reporting in CA case	Sharp

R1-1804973	Remaining  issues  on CSI reporting	Ericsson

R1-1805040	Remaining issues on CSI reporting	NTT DOCOMO, INC.

R1-1805103	Remaining Details on the CSI Framework	Nokia, Nokia Shanghai Bell

R1-1805669	WF on Type I CSI for rank 8	LG Electronics, Ericsson

##### Beam measurement and reporting

R1-1805514	Feature lead summary on beam measurement and reporting	Ericsson

Agreement:

The following text change is agreed:

- In Section 5.2.1 of TS 38.214, add SSBRI (SSB Resource Indicator) to the contents of CSI reporting. [6]

>>> Text Proposal for 38.214 Section 5.2.1 >>>

5.2.1 Channel state information framework

The time and frequency resources that can be used by the UE to report CSI are controlled by the gNB. CSI may consist of Channel Quality Indicator (CQI), precoding matrix indicator (PMI), CSI-RS resource indicator (CRI), SS/PBCH Block Resource indicator (SSBRI), layer indication (LI), rank indication (RI) and/or and L1-RSRP.

For CQI, PMI, CRI, SSBRI, LI, RI, L1-RSRP, a UE is configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings, M≥1 CSI-ResourceConfig Resource Settings, and a list of trigger states ReportTriggerList containing a list of associated CSI-ReportConfigs indicating the Resource Set IDs for channel and optionally for interference.

>>> End text proposal >>>

Agreement

For the case of single CC case, to determine the “lowest CORESET-ID” for determining default spatial QCL assumption for PDSCH, only consider CORESETs in active BWP

R1-1805574	Feature lead summary 2 on beam measurement and reporting	Ericsson

Agreement:

- For PUSCH scheduled by DCI format 0_0, the UE shall use a default spatial relation corresponding to the spatial relation, if applicable, used by the PUCCH resource with the lowest ID configured in the active UL BWP

  - Above applies for a cell configured with PUCCH

  - Note: the UE is configured with a list of spatial relations in PUCCH-SpatialRelationInfo. MAC-CE indicates a single selected spatial relation from the list on a per-PUCCH resource basis if the list has more than one element.

R1-1805640	WF on UE capability for SP-SRS	Samsung, KDDI, Ericsson, ZTE, Sanechips, LGE, Verizon Wireless, InterDigital, vivo, KT, NTT DOCOMO, SK Telecom, CHTTL

Agreement:

- Support SP-SRS as mandatory with UE capability signalling

R1-1805616	WF on MAC CE indication delay	Intel, Spreadtrum, MediaTek, OPPO, NEC, InterDigital, CATT, vivo

Agreement:

The latency for MAC CE activation is 12 slots for 60kHz SCS and 24 slots for 120kHz SCS. The latency is calculated from the ending symbol of UE’s ACK transmission for PDSCH carrying MAC-CE activation message to when the MAC CE message is applied.

Agreement:

- Support uplink cross-carrier beam indication for PUCCH and SRS

- Add Cell index and BWP information in SpatialRelation configuration

R1-1805739	[DRAFT] LS on UL cross carrier beam indication	Samsung

Decision: The LS is endorsed  with the following additional sentences: “RAN1 has agreed on cross carrier indication of spatial relations with SRS and PUCCH. For the case of SRS, changes are needed to the MAC CE message used for activating semi-persistent SRS to enable cross carrier indication. RAN1 suggests that RAN2 take this into their work.”

Final LS is approved in R1-1805627.

R1-1803636	Remaining issue for beam management	Huawei, HiSilicon

R1-1803744	Remaining issues on beam management	CATT

R1-1803817	Remaining issues on beam measurement and reporting	vivo

R1-1803908	Remaining issues on beam management	ZTE, Sanechips

R1-1803939	Ambiguities about PUCCH beam indication	Fujitsu

R1-1804000	Text Proposal for Beam Management	OPPO

R1-1804035	Remaining issues on beam management	ASUSTEK COMPUTER (SHANGHAI)

R1-1804058	Remaining issues in beam management	MediaTek Inc.

R1-1804210	Discussion of beam management for carrier aggregation	Lenovo, Motorola Mobility

R1-1804358	Corrections on Beam reporting and Indication	Samsung

R1-1804544	Remaining issues on beam management	LG Electronics

R1-1804593	Remaining issues on downlink beam management	Sony

R1-1804714	Remaining Issues on beam management	Intel Corporation

R1-1804770	Discussion on beam measurement and reporting	Apple Inc.

R1-1804787	Beam management for NR	Qualcomm Incorporated

R1-1804845	Remaining issues on beam management 	InterDigital, Inc.

R1-1804974	Remaining details of beam management	Ericsson

R1-1805041	Remining Issues on Beam Management	NTT DOCOMO, INC.

R1-1805104	Remaining issues on beam management	Nokia, Nokia Shanghai Bell

R1-1805639	WF on default PDCCH and PUCCH beams	LG Electronics, InterDigital, Samsung, vivo

##### Mechanism to recover from beam failure

Including beamFailureREcoveryTimer related discussion per RAN plenary guidance as in RP-180597

R1-1805583	Revised Summary 1 on Remaing issues on Beam Failure Recovery	MediaTek

R1-1805604	WF on beamFailureRecoveryTimer	InterDigital, MediaTek, Ericsson, ZTE, SaneChips, Huawei, HiSilicon, NTT DOCOMO, Intel, Samsung, CATT

R1-1805565	WF on remaining issues for beam failure recovery	Intel, MediaTek, ZTE, Sanechips, Huawei, HiSilicon

Agreement:

- BFI indication interval is lower bounded by 2ms

Agreement:

Agree to the following text change for TS38.213 section 6

A UE is configured with one control resource set by higher layer parameter Beam-failure-Recovery-Response-CORESET and with an associated search space provided by higher layer parameter search-space-config, as described in subcaluse 10.1, for monitoring PDCCH in the control resource set. The UE may receive from higher layers, by parameter Beam-failure-recovery-request-RACH-Resource, a configuration for a PRACH transmission as described in Subclause 8.1. For PRACH transmission in slot  and according to antenna port quasi co-location parameters associated with periodic CSI-RS configuration or SS/PBCH block with index  provided by higher layer [5, TS 38.321], the UE monitors PDCCH for detection of a DCI format with CRC scrambled by C-RNTI starting from slot  within a window configured by higher layer parameter Beam-failure-recovery-request-window. For PDSCH reception, the UE assumes the same antenna port quasi-collocation parameters as for monitoring PDCCH until the UE receives by higher layers an activation for a TCI state or a parameter TCI-StatesPDCCH.

R1-1805689	Summary 2 on Remaing issues on Beam Failure Recovery	MediaTek

Agreement:

- Send an LS to RAN2 with following information

  - RAN1 confirms the need for beamFailureRecoveryTimer and keep its uses as current RAN1 agreement

    - Note: unsuccessful CFRA based BFR upon expiry of beamFailureRecoveryTimer means that UE shall not use CFRA for BFR after beamFailureRecoveryTimer expired and no indication to higher layer is required

  - The following use case will be captured in the LS

    - Candidate beam selection for contention-free PRACH-based BFR is based on L1-RSRP. For qualified candidate beam but poor SINR, the timer enables UE to try contention-based PRACH resources after timer expires

  - Note: CBRA resource can be used when there is no candidate beam identified from candidate-beam-RS-list, as described by TS 38.321 section 5.1.2.

    - beamFailureRecoveryTimer does not apply to the use of CBRA resources.

R1-1805716	[DRAFT] LS on beamFailureRecoveryTimer	InterDigital

Decision: The draft LS is endorsed and final version is approved in R1-1805622

Agreement:

Text proposal for TS38.213 section 6

------------------------------Start Text Proposal---------------------------------

The physical layer in the UE shall, in slots where the radio link quality according to the set  is assessed, provide an indication to higher layers when the radio link quality for all corresponding resource configurations in the set  that the UE uses to assess the radio link quality is worse than the threshold Qout,LR. The physical layer informs the higher layers when the radio link quality is worse than the threshold Qout,LR with a periodicity determined by the maximum between the shortest periodicity of periodic CSI-RS configurations or SS/PBCH blocks in the set  and X.

------------------------------End Text Proposal---------------------------------

R1-1805730	Summary 3 on Remaing issues on Beam Failure Recovery	MediaTek

R1-1805714	[Draft] LS on NR BFR	Huawei, HiSilicon

Decision: The draft LS is endorsed and final version is approved in R1-1805623

R1-1803622	Remaining issues on beam failure recovery	NEC

R1-1803637	Summary of remaining issues on beam failure recovery	Huawei, HiSilicon

R1-1803745	Remaining issues on beam failure recovery	CATT

R1-1803818	Remaining issues on mechanism to recover from beam failure	vivo

R1-1803909	Remaining issues on beam recovery	ZTE, Sanechips

R1-1804001	Text Proposal for Beam Failure Recovery	OPPO

R1-1804036	Remaining issues on beam failure recovery procedure	ASUSTEK COMPUTER (SHANGHAI)

R1-1804059	Remaining issues in beam failure recovery	MediaTek Inc.

R1-1804094	Discussion on remaining issues on beam failure recovery	CMCC

R1-1804211	Discussion of beam failure recovery for carrier aggregation	Lenovo, Motorola Mobility

R1-1804359	Corrections on beam failure recovery	Samsung

R1-1804494	Corrections on beam failure recovery	ETRI

R1-1804545	Remaining issues on beam failure recovery	LG Electronics

R1-1804715	Remaining Issues on beam failure recovery	Intel Corporation

R1-1804788	Beam recovery procedures	Qualcomm Incorporated

R1-1804846	Remaining issues on beam failure recovery 	InterDigital, Inc.

R1-1804975	Remaining issues on beam recovery	Ericsson

R1-1805042	Remaining issues on beam recovery	NTT DOCOMO, INC.

R1-1805105	Remaining Details on Beam Recovery	Nokia, Nokia Shanghai Bell

R1-1805675	WF on Beam Failure Recovery	Samsung, CATT, Intel, MediaTek, OPPO, Spreadtrum, ASUSTEK, NEC, InterDigital, Qualcomm, ZTE, Sanechips, Huawei, HiSilicon, CHTTL,CMCC

##### CQI and MCS

R1-1805545	Summary for CQI and MCS	AT&T

Agreement

Text proposals for TS 38.214 v15.0.0 Section 6.1.4.1

< Unchanged parts are omitted >

- for MCS index 0,1 and 28, q=1 if UE has reported to support pi/2 BPSK modulation; and q=2 in other cases

else

- the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

end

Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate x 1024 R | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 240/q | 0.2344 |
| 1 | q | 314/q | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 466 | 2.7305 |
| 18 | 6 | 517 | 3.0293 |
| 19 | 6 | 567 | 3.3223 |
| 20 | 6 | 616 | 3.6094 |
| 21 | 6 | 666 | 3.9023 |
| 22 | 6 | 719 | 4.2129 |
| 23 | 6 | 772 | 4.5234 |
| 24 | 6 | 822 | 4.8164 |
| 25 | 6 | 873 | 5.1152 |
| 26 | 6 | 910 | 5.3320 |
| 27 | 6 | 948 | 5.5547 |
| 28 | 1q | reserved |  |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

< Unchanged parts are omitted >

Agreement

Text proposals for TS 38.214 v15.0.0 Section 5.2.2.1.1

The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Subclause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,...,1000 1008+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1], as given by

where  is a vector of PDSCH symbols from the layer mapping defined in Subclause 7.3.1.4 of [4, TS 38.211],  is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1, otherwise W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Subclause 4.1.

Agreement:

- For PDSCH scheduled by DCI format 1_0, UE assumes the default 64QAM table regardless of RRC configuration

- For PUSCH scheduled by DCI format 0_0, UE assumes the default 64QAM table regardless of RRC configuration

Agreement:

- For PDSCH scheduled with other RNTIs than C-RNTI & CS-RNTI, UE assumes 64 QAM MCS table

- For PUSCH scheduled with other RNTIs than C-RNTI & CS-RNTI, SP-CSI C-RNTI, UE assumes 64 QAM MCS table

Agreement

Text proposals for TS 38.214 v15.0.0 Section 5.2.2.1

< Unchanged parts are omitted >

If a UE is not configured with higher layer parameter MeasRestrictionConfig-time-channel, the UE shall derive the channel measurements for computing CQI CSI value reported in uplink slot n based on only the NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting.

If a UE is configured with higher layer parameter MeasRestrictionConfig-time-channel, the UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

If a UE is not configured with higher layer parameter MeasRestrictionConfig-time-interference, the UE shall derive the interference measurements for computing CQI CSI value reported in uplink slot n based on only the CSI-IM and/or NZP CSI-RS for interference measurement no later than the CSI reference resource associated with the CSI resource setting.

If a UE is configured with higher layer parameter MeasRestrictionConfig-time-interference the UE shall derive the interference measurements for computing the CQI CSI value reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM and/or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) associated with the CSI resource setting.

< Unchanged parts are omitted >

Agreement

Text proposals for TS 38.214 v15.0.0 Section 5.2.2.1

------------------------------------------------------Start text proposal-----------------------------------------------------------

5.2.2.1.1 CSI reference resource definition

-	the PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Subclause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000 3000+P-1], as given by

where  is a vector of PDSCH symbols from the layer mapping defined in Subclause 7.3.1.4 of [4, TS 38.211],  is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1,. otherwise If the higher layer parameter reportQuantity for the CSI-ReportConfig for which the CQI is reported is set to either ‘CRI/RI/PMI/CQI’ or ‘CRI/RI/LI/PMI/CQI’,  W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity for the CSI-ReportConfig for which the CQI is reported is set to ‘CRI/RI/CQI’, W(i) is the precoding matrix corresponding to the procedure described in Subclause 5.2.1.4.2. If the higher layer parameter reportQuantity for the CSI-ReportConfig for which the CQI is reported is set to ‘CRI/RI/i1/CQI’, W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Subclause 5.2.1.4. The corresponding PDSCH signals transmitted on antenna ports [3000,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Subclause 4.1.

R1-1803819	Remaining issues for CQI and MCS	vivo

R1-1804360	Remaining issues on CQI and MCS	Samsung

R1-1804444	Remaining issues on CQI and MCS	Huawei, HiSilicon

R1-1804716	Corrections to CQI definition in NR	Intel Corporation

##### Other

Including handling simultaneous transmission and reception of different channels and reference signals

R1-1805575	Summary of simultaneous transmission and reception of channels/signals	Intel Corporation

For further discussion in RAN1#93

- RAN4 agreement exists for SSB-RRM + PDSCH multiplexing for non-CA and CA cases

- Consider at least the following alternatives. At least for cases outside SMTC window (SSB-RLM+PDSCH, SSB-BM+PDSCH) including CA:

  - Alt 1: (multiplexing restriction) Extend the RAN4 agreement to the case of SSB+PDSCH multiplexing for both CA and non-CA without PDSCH associated with SI-RNTI, P-RNTI. This means that the UE is not expected to receive PDSCH on SSB symbols (for same numerology in FR2)

  - Alt 2: gNB will ensure at least spatial QCL

  - Alt 3: The UE is not expected to perform Rx beam refinement on SSB locations (Rx beam refinement may be associated with BM, RLM, BFD)

Note: The above applies to the case with same numerology for SSB and PDSCH. The case with different numerologies for SSB and PDSCH is to be addressed.

R1-1803701	QCL and spatial-relation for simultaneous reception and transmission	Huawei, HiSilicon

R1-1803820	Remaining issues on simultaneous reception of DL/UL physical channels and reference signals	vivo

R1-1803910	Simultaneous transmission and reception of channels/signals	ZTE, Sanechips

R1-1803964	Discussion on QCL across CCs	ITRI

R1-1804002	Discussion on simultaneous receptions/transmissions of multiple signals/channels	OPPO

R1-1804060	Views on multi-CC QCL assumptions	MediaTek Inc.

R1-1804085	PHY notification for beam failure instance	ASUSTEK COMPUTER (SHANGHAI)

R1-1804118	Enhancements on Type-II CSI Reporting Scheme	Fraunhofer IIS, Fraunhofer HHI

R1-1804361	Aperiodic beam reporting	Samsung

R1-1804362	Corrections on UL beam management	Samsung

R1-1804363	Simultaneous reception of physical channels and reference signals	Samsung

R1-1804445	Further enhancements on CSI reporting and codebook design	Huawei, HiSilicon

R1-1804717	Spatial QCL considerations for multiplexing of physical channels and RS	Intel Corporation

R1-1804718	On UE capability signalling for Type II CSI	Intel Corporation

R1-1804789	Details on  simultaneous reception of PHY channels and RS in FR2	Qualcomm Incorporated

R1-1804976	Multi-cell beam recovery	Ericsson

R1-1804977	On simultaneous reception of physical and reference signals across CCs	Ericsson

R1-1804978	Remaining issues on SP CSI reporting on PUSCH	Ericsson

R1-1804979	Miscellaneous corrections and clarifications on CSI acquisition	Ericsson

R1-1804980	On OFDM symbol level PDSCH power boosting	Ericsson

R1-1805043	On Type II codebook enhancement	NTT DOCOMO, INC.

R1-1805106	Remaining Issue on Type II CSI Omission	Nokia, Nokia Shanghai Bell

R1-1805107	On channels multiplexing	Nokia, Nokia Shanghai Bell

#### Reference signals and QCL

##### Multiplexing of different types of RSs

R1-1805544	Feature lead summary of remaining issues for RS multiplexing	Huawei, HiSilicon

Agreement

Text proposal in Section 5.1.6.1 of TS 38.214

The CSI-RS defined in Subclause 7.4.1.5 of [4, TS 38.211], may be used for time/frequency tracking, CSI computation,  L1-RSRP computation and mobility.

If the UE is configured with a CSI-RS resource and a CORESET in the same OFDM symbol(s), the UE may assume that the CSI-RS and a PDCCH DM-RS transmitted in the CORESET are quasi co-located with ‘QCL-TypeD’, if ‘QCL-TypeD’ is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the CORESET.

Agreement

Text proposal in Section 6.2.1 of TS38.214

A UE shall not transmit simultaneously SRS resource(s) and PRACH. If a UE is configured with an SRS resource and PRACH in the same OFDM symbols in a slot, SRS is not transmitted in the overlapping symbols. If SRS and PRACH would occur on overlapping OFDM symbol(s), SRS is not transmitted on the overlapping symbol(s).

R1-1805704	Feature lead summary 2 of remaining issues for RS multiplexing	Huawei, HiSilicon

Agreement

In the case of collision of aperiodic/semi-persistent/periodic SRS and short PUCCH carrying HARQ-ACK, drop SRS.

And related Text Proposal:

TP for Section 6.2.1 in TS 38.214 (treatment of short PUCCH with A/N against SRS)

------Unchanged parts are omitted------

For PUCCH formats 0 and 2, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s) or if aperiodic SRS is configured and PUCCH consists of beam failure request. A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS happens is triggered to be transmitted to overlap in the same symbol with semi-persistent or periodic PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only.

------Unchanged parts are omitted------

R1-1803704	Remaining issues on RS multiplexing	Huawei, HiSilicon

R1-1803821	Remaining issues on multiplexing of different types of RSs	vivo

R1-1804073	Remaining issues on multiplexing	MediaTek Inc.

R1-1804364	Remaining issues on RS multiplexing	Samsung

R1-1804546	Correction on multiplexing between CSI-RS and CORESET	LG Electronics

R1-1804790	Maintenance for Multiplexing of RS	Qualcomm Incorporated

R1-1804948	Discussions on multiplexing of different types of RSs	Mitsubishi Electric Co.

R1-1804981	Remaining issues on RS multiplexing	Ericsson

##### CSI-RS

R1-1805557	Remaining issues on CSI-RS	Huawei, HiSilicon

Agreement:

- Confirm the WA on CSI-RS sequence mapping for the case with 0.5 density

Agreement:

- A UE may assume all CSI-RS ports within one CSI-RS resource are quasi co-located with respect to ‘QCL type A’ and ‘QCL type D’ when applicable.

Agreement:

Text proposal for Section 5.2.2.3.1 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

-	CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211] , and the minimum bandwidth is the minimum between 24 and   resource blocks.

----------------------------------------------------------------------------------------------------------------------------------

Agreement:

Text proposal for Section 5.1.6.1.2 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

If the UE is configured with a CSI-ReportConfig with reportQuantity set to “CRI/RSRP”, or “No Report” and if the Resource Setting for channel measurement contains a CSI-RS Resource Set that is configured with the higher layer parameter CSI-RS-ResourceRep and configured without the higher layer parameter TRS-Info, the UE can only be configured with 1 or 2 ports with the higher layer parameter NrofPorts for all CSI-RS resources within the set, and the UE may be configured with the CSI-RS resource in the same OFDM symbol(s) as an SS/PBCH block, and the UE may assume that the CSI-RS and the SS/PBCH block are quasi co-located with ‘QCL-TypeD’ if ‘QCL-TypeD‘ is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the SS/PBCH block, and the UE shall expect that the same subcarrier spacing is used for both the CSI-RS and the SS/PBCH block.

Agreement:

The number of ports in a resource set for L1-RSRP

- Always have the same number of ports (1 or 2) across all resources

----------------------------------------------------------------------------------------------------------------------------------

Agreement:

For density ½, odd/even PRB allocations is respect to the common resource block grid

Include in LS to RAN2 for CSI-RS Xi (Huawei)

Agreement

Text proposal for Section 5.1.4.2 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

- ZP-CSI-RS-ResourceConfigId determines ZP CSI-RS resource configuration identity.

- NrofPorts defines the number of CSI-RS ports, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

- ZP-CSI-RS-Density defines the zero-power CSI-RS frequency density, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

- CDMType defines CDM values and pattern, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

----------------------------------------------------------------------------------------------------------------------------------

R1-1805685	Remaining issues on CSI-RS #2	Huawei, HiSi

Agreement

Text proposal for Section 5.2.2.3.1 in 38.214

-	CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211]. If the configured bandwidth is larger than the corresponding BWP, UE shall assume that the actual CSI-RS bandwidth is equal to the BWP size.

Agreement

- The UE expects that all the CSI resources of a resource set are configured with the same starting RB and number of RBs and the same CDM-type.

  - FFS: Other parameters

Agreement

Text proposal for Section 5.2.2.3.1 in 38.214

--------------------------------------------------------------------------------------------------------------------------

-	CSI-RS-Density defines CSI-RS frequency density of each CSI-RS port per PRB and CSI-RS PRB offset in case of the density value of 1/2, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211]. For density ½, odd/even PRB allocations is respect to the common resource block grid.

-	CDMType defines CDM values and pattern, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

-	CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211].

-	Pc: which is the assumed ratio of PDSCH EPRE to NZP CSI-RS EPRE when UE derives CSI feedback and takes values in the range of [-8, 15] dB with 1 dB step size.

-	Pc_SS: which is the assumed ratio of SS/PBCH block EPRE to NZP CSI-RS EPRE.

-	ScramblingID: defines scrambling ID of CSI-RS with length of 10 bits.

-	CC_Info defines which component carrier the configured CSI-RS is located in.

- 	BWP_Info defines which bandwidth part the configured CSI-RS is located in.

--------------------------------------------------------------------------------------------------------------------------

Agreement

Text proposal for Section 5.1.4.2 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

-	ZP-CSI-RS-ResourceMapping defines the OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot that are given in Subclause 7.4.1.5 of [4, TS 38.211].

----------------------------------------------------------------------------------------------------------------------------------

R1-1805740	[Draft] LS on NR CSI-RS	Huawei, HiSilicon

Decision: The draft LS is endorsed and final version is approved in R1-1805626.

R1-1803638	Remaining details on CSI-RS design in NR	Huawei, HiSilicon

R1-1803746	Remaining issues on CSI-RS	CATT

R1-1803822	Remaining issues on CSI-RS	vivo

R1-1803969	Remaining issues on CSI-RS design	OPPO

R1-1804074	Remaining issues on CSI-RS	MediaTek Inc.

R1-1804197	Remaining issues on CSI-RS	Lenovo, Motorola Mobility

R1-1804365	Remaining issues on CSI-RS	Samsung

R1-1804719	Remaining issues on CSI-RS	Intel Corporation

R1-1804791	Maintenance for on CSI-RS	Qualcomm Incorporated

R1-1804982	Remaining issues on CSI-RS	Ericsson

R1-1805044	Remaining details on CSI-RS	NTT DOCOMO, INC.

R1-1805108	Remaining issues on CSI-RS design	Nokia, Nokia Shanghai Bell

##### DMRS

R1-1805611	First Summary of Issues for PDSCH/PUSCH’s DM-RS	Qualcomm

Agreement:

Text proposals for TS 38.211 Section 6.4.1.1.1.2

------Unchanged parts are omitted------

6.4.1.1.1.2	Sequence generation when transform precoding is enabled

If transform precoding for PUSCH is enabled, the reference-signal sequence  shall be generated according to

where  is given by clause 5.2.2 with and  for a PUSCH transmission dynamically scheduled by DCI.

------Unchanged parts are omitted------

Agreement

Text proposals for TS 38.211 Section 7.4.1.1.1

------Unchanged parts are omitted------

7.4.1.1	Demodulation reference signals for PDSCH

7.4.1.1.1	Sequence generation

The UE shall assume the sequence  is defined by

.

where the pseudo-random sequence  is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

where  is the OFDM symbol number within the slot,  is the slot number within a frame, and

-	 and  is given by the higher-layer parameter DL-DMRS-Scrambling-ID if provided and the PDSCH is scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI

-	 and  otherwise

------Unchanged parts are omitted------

Agree with the above TP and change 14 to

Agreement

<<<< Text proposal for section 6.4.1.1.3 in TS 38.211 <<<<

6.4.1.1.3	Precoding and mapping to physical resources

[…]

Table 6.4.1.1.3-6: PUSCH DM-RS positions  for single-symbol DM-RS and frequency hopping enabled.

| Duration in symbols for each hop | DM-RS positions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUSCH mapping type A |  |  |  | PUSCH mapping type B |  |  |  |
|  | UL-DMRS-add-pos |  |  |  | UL-DMRS-add-pos |  |  |  |
|  | 0 |  | 1 |  | 0 |  | 1 |  |
|  | First hop | Second hop | First hop | Second hop | First hop | Second hop | First hop | Second hop |
| ≤3 | - | - | - | - |  | 0 | l0 | 0 |
| 4 |  | 0 | l0 | 0 |  | 0 | l0 | 0 |
| 5, 6 |  | 0 | l0 | 0, 4 |  | 0 | , | 0, 4 |
| 7 |  | 0 | , | 0, 4 |  | 0 | , | 0, 4 |

[…]

Agreement:

UE DM-RS transmission procedure for before RRC configuration in case of frequency hopping enabled should be specified in TS38.214 as following:

- If frequency hopping is enabled, the UE shall assume dmrs-AdditionalPosition = 1, and one additional DM-RS can be transmitted according to PUSCH duration.

Text Proposal for this correction shown below:

6.2.2	UE DM-RS transmission procedure

When transmitted PUSCH is not scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining Res not used for DM-RS in the symbols are not used for any PUSCH transmission, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and

If frequency hopping is disabled,

-	For PUSCH with allocation duration of more than 7 symbols, the UE shall assume dmrs-AdditionalPosition=2 and up to two additional DM-RS can be transmitted according to PUSCH duration, or

-	For PUSCH with allocation duration of 7 symbols with mapping type B, the UE shall assume dmrs-AdditionalPosition=1 and one additional DM-RS can be transmitted

-	For PUSCH with allocation duration of less than 7 symbols with mapping type B, the UE shall assume dmrs-AdditionalPosition=0 and not transmit additional DM-RS.

If frequency hopping is enabled,

-	The UE shall assume dmrs-AdditionalPosition=1 and up to one additional DM-RS can be transmitted according to PUSCH duration.

[…]

Agreement

For PUSCH not scheduled by PDCCH format 0_1:

- Support FDM for PUSCH and DMRS when PUSCH duration is N or less OFDM symbols

  - N=2 for CP-OFDM.

TDM for other cases. Note: For DFT-S-OFDM, always TDM.

R1-1805587	WF on group/sequence hopping for DFT-S-OFDM PUSCH DMRS	CMCC, ZTE, Sanechips, Intel, Nokia, Nokia Shanghai Bell, OPPO, NEC, LG Electronics, NTT DOCOMO, MediaTek, vivo, Qualcomm

Agreement

- For DFT-s-OFDM based PUSCH transmission, if group hopping or sequence hopping is used, the sequence of the second DMRS symbol follows the first symbol for each pair of adjacent DMRS symbols.

Agreement

The following text proposal for section 7.4.1.1.2 of TS38.211 is agreed.

---------------------

For PDSCH mapping type B

-	if the PDSCH duration is 2, 4, or 7 OFDM symbols for normal CP, or 2, 4, 6 OFDM symbols for extended CP, and the PDSCH allocation collides with resources reserved for a CORESET,  shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET and

-	if the PDSCH duration is 4 symbols, the UE is not expected to receive a DM-RS symbol beyond the third symbol,

-	if the PDSCH duration is 7 symbols for normal CP, or 6 OFDM symbols for extended CP,

-	the UE is not expected to receive the first DM-RS beyond the fourth symbol, and

-	if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted.

Table 7.4.1.1.2-3: PDSCH DM-RS positions  for single-symbol DM-RS.

| Duration in symbols | DM-RS positions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PDSCH mapping type A |  |  |  | PDSCH mapping type B |  |  |  |
|  | DL-DMRS-add-pos |  |  |  | DL-DMRS-add-pos |  |  |  |
|  | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| 2 | - | - | - | - |  |  |  |  |
| 3 |  |  |  |  | - | - |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | - | - |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 |  | , [7] | , [7] | , [7] | - | - |  |  |
| 9 |  | ,7 | ,7 | ,7 | - | - |  |  |
| 10 |  | ,9 | , 6, 9 | , 6, 9 | - | - |  |  |
| 11 |  | ,9 | , 6, 9 | , 6, 9 | - | - |  |  |
| 12 |  | 9 | , 6, 9 | , 5, 8, 11 | - | - |  |  |
| 13 |  | ,11 | , 7, 11 | , 5, 8, 11 | - | - |  |  |
| 14 |  | ,11 | , 7, 11 | , 5, 8, 11 | - | - |  |  |

Table 7.4.1.1.2-4: PDSCH DM-RS positions  for double-symbol DM-RS

| Duration in symbols | DM-RS positions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PDSCH mapping type A |  |  | PDSCH mapping type B |  |  |
|  | DL-DMRS-add-pos |  |  | DL-DMRS-add-pos |  |  |
|  | 0 | 1 | 2 | 0 | 1 | 2 |
| <4 |  |  |  | - | - |  |
| 4 |  |  |  | - | - |  |
| 5 |  |  |  | - | - |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  | - | - |  |
| 9 |  |  |  | - | - |  |
| 10 |  | , 8 |  | - | - |  |
| 11 |  | , 8 |  | - | - |  |
| 12 |  | , 8 |  | - | - |  |
| 13 |  | , 10 |  | - | - |  |
| 14 |  | , 10 |  | - | - |  |

Agreement

In Section 5.1.6.2 of TS38.214

When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, DL-DMRS-max-len and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in subclause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition

-	For PDSCH with mapping type A, the UE shall assume dmrs-AdditionalPosition=2 and up to two additional single-symbol DM-RS present in a slot according to the PDSCH duration indicated in the DCI as defined in Subclause 7.4.1.1 of [4, TS 38.211], and

-	For PDSCH with allocation duration of 7 symbols for normal CP or 6 symbols for extended CP with mapping type B, the UE shall assume one additional single-symbol DM-RS present in the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol respectively of the PDSCH allocation duration, otherwise the UE shall assume that the additional DM-RS symbol is not present, and

-	For PDSCH with allocation duration of 4 symbols with mapping type B, the UE shall assume that no additional DM-RS are present, and

-	For PDSCH with allocation duration of 2 symbols with mapping type B, the UE shall assume that no additional DM-RS are present, and the UE shall assume that the PDSCH is present in the symbol carrying DM-RS

R1-1805691	Second Summary of Issues for PDSCH/PUSCH’s DM-RS	Qualcomm

Agreement

Add the following text in Section 7.4.1.1.2 of 38.211 to clarify the above:

------Unchanged parts are omitted------

The position(s) of the DM-RS symbols is given by  and

-	for PDSCH mapping type A, the duration is between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PDSCH resources in the slot

-	for PDSCH mapping type B, the duration is the number of OFDM symbols of the scheduled PDSCH resources as signalled

and according to Tables 7.4.1.1.2-3 and 7.4.1.1.2-4. The case DL-DMRS-add-pos equal to 3 is only supported when DL-DMRS-typeA-pos is equal to 2. For PDSCH mapping type A, duration of 3 and 4 symbols in Tables 7.4.1.1.2-3 and 7.4.1.1.2-4 respectively is only applicable for DL-DMRS-typeA-pos equals 2.

------Unchanged parts are omitted------

Agreement

Add the following text in Section 6.4.1.1.3 of 38.211 to clarify the above:

------Unchanged parts are omitted------

The position(s) of the DM-RS symbols is given by  and

-	the signalled duration between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PUSCH resources in the slot for PUSCH mapping type A according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if frequency hopping is not used, or

-	the signalled duration of scheduled PUSCH resources for PUSCH mapping type B according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if frequency hopping is not used, or

-	the signalled duration per hop according to Table 6.4.1.1.3-6 if frequency hopping is used.

For PUSCH mapping type A, the case UL-DMRS-add-pos equal to 3 is only supported when DL-DMRS-typeA-pos is equal to 2. For PUSCH mapping type A, duration of 4 symbols in Table 6.4.1.1.3-4 is only applicable for DL-DMRS-typeA-pos equals 2.

------Unchanged parts are omitted------

Agreement

Text proposal for Section 5.1.6.2, 38.214

------Unchanged parts are omitted------

When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, or receiving PDSCH scheduling DCI format 1_0 by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, DL-DMRS-max-len and dmrs-Type,

------Unchanged parts are omitted------

When receiving PDSCH scheduled by DCI format 1_1 by PDCCH with CRC scrambled by C-RNTI or CS-RNTI,

-	the UE may be configured with the higher layer parameter dmrs-Type, and the configured DM-RS configuration type is used for receiving PDSCH in as defined in Subclause 7.4.1.1 of [4, TS 38.211].

-	the UE may be configured with the maximum number of front-loaded DM-RS symbols for PDSCH by higher layer parameter DL-DMRS-max-len.

-	if DL-DMRS-max-len is equal to 1, single-symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be 0, 1, 2 or 3.

-	if DL-DMRS-max-len equal to 2, both single-symbol DM-RS and double symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be 0 or 1.

-	and the UE shall assume to receive additional DM-RS as specified in Table 7.4.1.1.2-3 and Table 7.4.1.1.2-4 as described in Subclause 7.4.1.1.2 of [4, TS 38.211].

------Unchanged parts are omitted------

Agreement

Text proposal for TS38.211

6.4.1.1.3	Precoding and mapping to physical resources

[…]

The time-domain index  and the supported antenna ports  are given by Tables 6.4.1.1.3-3 through 6.4.1.1.3-6:

-	if the higher-layer parameter UL-DMRS-max-len is equal to 1 and frequency hopping is disabled, the tables shall be used according to single-symbol DM-RS

-	if the higher-layer parameter UL-DMRS-max-len is equal to 2 and frequency hopping is disabled, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used

-	if higher-layer parameter UL-DMRS-add-pos is equal to or more than 1 and frequency hopping is enabled according to clause 7.3.1.1.2 in [5, TS 38.212], Table 6.4.1.1.3-6 shall be used assuming UL-DMRS-add-pos is 1 for each hop. If , only UL-DMRS-add-pos equal to 0 is supported is assumed to be 0.

[…]

R1-1805610	WF on DM-RS scrambling with fallback DCIs	Huawei, HiSilicon, vivo, China Telecom, NEC, CEWiT, China Unicom, AT&T, IITH, Deutsche Telekom, Tejas Networks, Softbank, Sony, Fujitsu, IITM, ASUSTeK, Lenovo, Motorola Mobility, CMCC, ZTE, Sanechips, Intel, OPPO

R1-1805668	WF on DMRS scrambling ID for fall-back DCIs	Samsung, LG Electronics, Spreadtrum, KT, ITL, KRRI, WILUS

Agreement

For DMRS scrambling ID for PDSCH scheduled by PDCCH format 1_0 with CRC scrambled by C-RNTI, or PUSCH scheduled by PDCCH format 0_0 with CRC scrambled by C-RNTI

  - If a PUSCH/PDSCH of a UE is scheduled by DCI format 0_0 or DCI format 1_0, the UE would assume the first DMRS scrambling ID if configured, otherwise   is used.

R1-1803705	Remaining issues on DMRS design	Huawei, HiSilicon

R1-1803747	Remaining issues on DMRS	CATT

R1-1803823	Remaining issues on DMRS	vivo

R1-1803911	Remaining issues on DMRS	ZTE, Sanechips

R1-1803970	Remaining issues on DMRS design	OPPO

R1-1804075	Remaining issues on DMRS	MediaTek Inc.

R1-1804095	Discussion on remaining issues on DMRS	CMCC

R1-1804198	Remaining issues on DMRS	Lenovo, Motorola Mobility

R1-1804366	Remaining issues on DMRS	Samsung

R1-1804647	DMRS for pi/2 BPSK with spectrum shaping	IITH, CEWIT, IITM, Tejas Networks, Reliance Jio

R1-1804720	Remaining issues on DM-RS	Intel Corporation

R1-1804792	Maintenance for DMRS	Qualcomm Incorporated

R1-1804946	Discussions on DMRS	Mitsubishi Electric Co.

R1-1804983	Remaining issues on DMRS	Ericsson

R1-1805045	Remaining details on DM-RS	NTT DOCOMO, INC.

R1-1805109	Further comments on DM-RS for NR physical data channels	Nokia, Nokia Shanghai Bell

R1-1805548	Remaining issues on DMRS	KT Corp.

##### PT-RS

R1-1805542	Feature lead summary 2 of PT-RS	Ericsson

Working assumption:

Support variable time density LPT-RS for UCI-only PUSCH where information (e.g. MCS) in DCI is used to determine LPT-RS.

R1-1805584	WF on UL PTRS Port Indication	vivo, Intel, NEC, Lenovo, Motorola Mobility, ZTE, Sanechips, OPPO, Samsung, LG Electronics, MediaTek, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, CMCC, NTT DOCOMO

Agreement

- For the following case, the field of PTRS-DMRS association is 0 bit for both codebook based UL transmission and non-codebook based UL transmission

  - High layer parameter maxRank is configured as 1.

- Update the following Table 7.3.1.1.2-26 for PTRS-DMRS association for UL PTRS ports 0 and 1, UL-PTRS-ports = 2

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port transmitting layers corresponding to SRS port 0 and 2, the 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port transmitting layers corresponding to SRS port 1 and 3, 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port transmitting layers corresponding to SRS port 0 and 2, 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port transmitting layers corresponding to SRS port 1 and 3, 2nd DMRS port which shares PTRS port 1 |

Agreement

Agree on the following text proposal TS 38.211 v15.0.0 Section 6.4.1.2.1.2 to implement the previous agreement by changing m’ in the exponential to m:

< Unchanged parts are omitted >

If transform precoding is enabled, the phase-tracking reference signal  to be mapped in position  before transform precoding, where  depends on the number of PT-RS groups , the number of samples per PT-RS group , and   according to Table 6.4.1.2.2.2-1, shall be generated according to

< Unchanged parts are omitted >

R1-1803623	Remaining issues on PTRS configurations	NEC

R1-1803706	Remaining issues on PTRS	Huawei, HiSilicon

R1-1803748	Remaining issues on PT-RS	CATT

R1-1803824	Remaining issues on PT-RS	vivo

R1-1803912	Remaining issues on PT-RS	ZTE, Sanechips

R1-1803940	Discussion on remaining details on PT-RS	Fujitsu

R1-1804076	Remaining issues on PT-RS	MediaTek Inc.

R1-1804096	Discussion on remaining issues on PT-RS	CMCC

R1-1804199	Remaining issues on PTRS	Lenovo, Motorola Mobility

R1-1804216	Remaining issues on PT-RS	Spreadtrum Communications

R1-1804273	Remaining issues on PT-RS	Panasonic Corporation

R1-1804367	Remaining issues on PTRS	Samsung

R1-1804469	Remaining issues on PTRS	Mitsubishi Electric RCE

R1-1804547	Text proposals on PT-RS	LG Electronics

R1-1804721	Remaining issues On PT-RS	Intel Corporation

R1-1804793	PTRS considerations	Qualcomm Incorporated

R1-1804847	Remaining issues on PT-RS 	InterDigital, Inc.

R1-1804984	Remaining issues on PT-RS	Ericsson

R1-1805046	Remaining details on PTRS	NTT DOCOMO, INC.

R1-1805110	Further comments on PT-RS	Nokia, Nokia Shanghai Bell

##### SRS

R1-1805522	Summary of SRS	Sony

Agreement

The following working assumption is confirmed.

- NR supports the case that different ports of one 4-port SRS resource have different comb offsets for comb 2 and 4.

Agreement

Text proposal for section 6.2.1.3, 38.214):

For an aperiodic SRS triggered in DCI format 2_3 and UE configured with higher layer parameter typeA-SRS-TPC-PDCCH-Group without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers.

For an aperiodic SRS triggered in DCI format 2_3 and UE configured with higher layer parameter typeB-SRS-TPC-PDCCH-Group without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells with aperiodic SRS triggered in the DCI.

Agreement

Text proposal for section 7.3.1.3.4, 38.212:

If the UE is configured with higher layer parameter typeA-SRS-TPC-PDCCH-Group for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one block is configured for the UE by higher layers, with the following fields defined for the block:

-	SRS request – 0 or 2 bits. The presence of this field is according to the definition in Subclause x.x of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-5.

-	TPC command number –2 bits-     TPC command number 1, TPC command number 2, …, TPC command number n

where each TPC command applies to a respective serving cell index provided by higher layer parameter cc-IndexInOneCC-Set.

If the UE is configured with higher layer parameter typeB-SRS-TPC-PDCCH-Group for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one or more blocks is configured for the UE by higher layers where each block applies to a serving cell, with the following fields defined for the block:

-	SRS request – 0 or 2 bits. The presence of this field is according to the definition in Subclause x.x of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-5.

-	TPC command number –2 bits

Agreement:

If TS38.214 does not capture that slot-level offset is for aperiodic SRS resource set, the spec editor should capture this

Agreement

The configuration of SRS starting position offset and symbol number  should satisfy .

The following text proposal for TS38.211 section 7.1.2.4 captures the above agreement.

< Unchanged parts are omitted >

An SRS resource consists of

-	 antenna ports , , given by the higher layer parameter NrofSRS-Ports

-	 consecutive OFDM symbols contained in the higher layer parameter SRS-ResourceMapping

-	, the starting position in the time domain given by  where the offset counts symbols backwards from the end of the slot and is contained in the higher layer parameter SRS-ResourceMapping, where

-	, the frequency-domain starting position of the sounding reference signal

< Unchanged parts are omitted >

Agreement

Inform RAN2 that aperiodicSRS-ResourceTrigger should start from 1 not 0. LS to be prepared by Steve (Ericsson)

Agreement

>>> Text proposal for 38.212 Section 7.3.1.1.2 >>>

Table 7.3.1.1.2-24: SRS request

| Value of SRS request field | Triggered aperiodic SRS resource set(s) |
| --- | --- |
| 00 | No aperiodic SRS resource set triggered |
| 01 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 |
| 10 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 2 |
| 11 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 3 |

>>> End text proposal >>>

Agreement

To avoid duplication between 38.212 and 38.214 of the table mapping the codepoints of the SRS request field in DCI to the triggered aperiodic SRS resource set(s), remove Table 6.2.1-1 from 38.214. Text proposal is provided below.

>>> Text proposal for 38.214 Section 6.2.1  >>>

For aperiodic SRS, at least one state of the DCI field is used to select at least one out of the configured SRS resource set.

The 2-bit SRS request field [5, TS38.212] in DCI format 0_1, 1_1 indicates the triggered SRS resource set given in Table 6.2.1-1 7.3.1.1.2-24 of [5, TS38.212]. The 2-bit SRS request field [5, TS38.212] in DCI format 2_3 indicates the triggered SRS resource set given in Subclause 11.4 of [6, TS 38.213].

Table 6.2.1-1: SRS request value for aperiodic SRS

| Value of SRS request field | Description |
| --- | --- |
| '00' | No aperiodic SRS trigger |
| '01' | The 1st SRS resource set(s) configured by higher layers |
| '10' | The 2nd SRS resource set(s) configured by higher layers |
| '11' | The 3rd SRS resource set(s) configured by higher layers |

If a UE is configured with the higher layer parameter SRS-AssocCSIRS and with the higher layer parameter ulTxConfig set to 'NonCodebook', the UE may be configured with a NZP CSI-RS resource where a NZP-CSI-RS-ResourceConfigId is associated with an SRS resource set.

>>> End text proposal >>>

R1-1805654	WF on SRS antenna switching	CMCC, ZTE, Sanechips, Ericsson, CATT, Sony, OPPO, NEC, Nokia, Nokia Shanghai Bell

R1-1805695	Summary of SRS	Sony

Agreement

- 1T4R antenna switching for SRS transmission is supported by 2 aperiodic SRS resource sets

  - two aperiodic SRS resource sets with total four SRS resources transmitted in different symbols of two different slots, each SRS resource consisting of a single SRS port, and where the SRS port of each resource is associated with a different UE antenna port

  - each of the two SRS resource sets consists of two SRS resources, or one SRS resource set consists of a single SRS resource and the other SRS resource set consists of three SRS resources

  - the UE shall expect the same value for the higher layer parameters alpha-srs, p0-srs, srs-pathlossReference-rs-config and srs-pcadjustment-state-config in the two SRS resource sets

  - FFS: On UE ambiguity if two DCI triggers separately triggers two SRS resource sets

- FFS: The gap before or after any SRS resource set for antenna switching

- FFS: The number of SRS resource sets for other cases

R1-1805700	Draft LS on aperiodic SRS triggering	Ericsson

Decision: The draft LS is endorsed and final version is approved  in R1-1805625.

Agreement

>>> Text proposal for 38.214 Section  6.2.1.2 >>>

6.2.1.2            UE antenna switching

When UE antenna switching is enabled by the higher layer parameter SRS-SetUse set as 'antenna switching' for a UE that supports transmit antenna switching, a UE may be configured with one of the following configurations depending on the UE capability:

-     SRS resource set with two SRS resources transmitted in different symbols, each SRS resource consisting of a single SRS port, being associated with different UE antenna ports, or

-     SRS resource set with two SRS resources transmitted in different symbols, each SRS resource consisting of two SRS ports where the port pair of the second resource is associated with a different UE antenna pair than the port pair of the first resource, or

-     SRS resource set with four SRS resources transmitted in different symbols, each SRS resource consisting of a single SRS port being associated with different UE antenna port, or

-     SRS resource set with one SRS resource, where the number of SRS ports is equal to 1, 2, or 4

and a guard period where UE does not transmit any other signal of Y symbols in-between the SRS resources of the set, is used in case the SRS resources are transmitted in the same slot.

The value of Y is defined by Table 6.2.1.2-1.

>>> End text proposal >>>

R1-1803639	Remaining details of SRS design	Huawei, HiSilicon

R1-1803749	Remaining issues on SRS	CATT

R1-1803825	Remaining issues on SRS	vivo

R1-1803913	Remaining issues on SRS	ZTE, Sanechips

R1-1803971	Remaining issues on SRS design	OPPO

R1-1804061	Remaining issues in SRS	MediaTek Inc.

R1-1804097	Discussion on remaining issues on SRS	CMCC

R1-1804368	Remaining issues on SRS	Samsung

R1-1804548	Remaining issues on SRS	LG Electronics

R1-1804594	Remaining issues on SRS	Sony

R1-1804722	Remaining issues on SRS	Intel Corporation

R1-1804794	Maintenance for SRS	Qualcomm Incorporated

R1-1804880	Discussions on SRS resource transmission for BWP switch	Sharp

R1-1804985	Remaining issues on SRS	Ericsson

R1-1805111	Remaining details on SRS design in NR	Nokia, Nokia Shanghai Bell

##### TRS

R1-1805550	Summary #1 for TRS	MediaTek

Agreement

The following text proposals are agreed.

Text proposal 5.1.6.1.2 38.214

A UE does not expect to be configured with a Reporting Setting, which is linked to a CSI-RS resource set for tracking, where the higher layer parameter MeasRestrictionConfig-time-channel for the Reporting Setting set to ‘ON’.

Text proposal 5.1.6.1.1 38.214

The periodic CSI-RS resources in the CSI-RS resource set configured with higher layer parameter TRS-Info have the same periodicity, bandwidth and subcarrier location.

A UE does not expect to be configured with a reporting setting, which is linked to a CSI-RS resource set for tracking, where the higher layer parameter MeasRestrictionConfig-time-channel for the reporting setting set to ‘ON’.

Agreement

A CSI-RS resource set should not be configured with TRS-info and CSI-RS-ResourceRep simultaneously.

R1-1805694	Summary #2 for TRS	MediaTek

Working Assumption

Agreement

- Rel-15 supports aperiodic TRS for FR2 as an optional UE feature with UE capability signalling

  - (Working assumption) A UE does not expect to be triggered to aperiodic TRS unless it has been configured with an associated periodic TRS with the same burst length X slots on the same BWP

    - The aperiodic TRS and the associated periodic TRS has the same BW, [same symbol position, same subcarrier location] and the aperiodic TRS will be QCL type A+D (if applicable) with this periodic TRS

    - FFS on DCI trigger timing for SCell activation and BWP switching

    - The UE does not expect the scheduling offset between DCI to the first symbol of TRS be smaller than the threshold UE reported

    - The aperiodic TRS is triggered by UL DCI (same as CSI-RS)

- No new/additional RRC parameters are introduced for this feature except UE capability

Note: (from Huawei) Aperiodic TRS is a new feature in Rel-15 but Huawei is willing to accept this for progress

Note: AT&T has concerns to make this an UE optional feature since this is a forward compatibility issue

Friday

Huawei confirmed the above note and worried that the new feature was lately introduced – nevertheless it should be clarified that this is clearly an optional feature

MIMO session's chair: lengthly discussed, many companies believe it is essential.

RAN1 chair: let's take it as an agreement – warned the group for introducing new feature at this stage of the WI

Intel suggested to liaise to RAN2 asking them whether a new feature group can still be added

Samsung requested preparing the corresponding CRs by June

Debate regarding the timeline for CRs as the new feature has not been incorporated in the current UE feature list spreadsheet and the impact to RAN2 work.

Conclusion: Above working assumption is changed to Agreement.

Send an LS to RAN2 regarding the above agreements and ask RAN2 to take into account in their work – R1-1805758 (Xiaoyi, ATT)

R1-1805758	[draft] LS for supporting aperiodic TRS	AT&T

Decision: The document is endorsed and final LS is approved in R1-1805763.

The above optional UE feature is to be included in the current UE feature spreadsheet for RAN1#93.

R1-1803826	Remaining issues on TRS	vivo

R1-1803914	Remaining issues on TRS	ZTE, Sanechips

R1-1804003	Text Proposal for CSI-RS for Time/frequency Tracking	OPPO

R1-1804077	Remaining issues on TRS	MediaTek Inc.

R1-1805261	Remaining details on TRS design	Spreadtrum Communications	(revision of R1-1804232)

R1-1804369	Remaining issues on TRS	Samsung

R1-1804443	Further discussion on introducing aperiodic TRS	Huawei, HiSilicon

R1-1804549	Text proposals on TRS	LG Electronics

R1-1804723	Remaining Issues On TRS	Intel Corporation

R1-1804795	Remaining issues on TRS	Qualcomm Incorporated

R1-1804986	Remaining issues on TRS	Ericsson

R1-1805047	Remaining details on TRS	NTT DOCOMO, INC.

R1-1805112	Remaining details of TRS design	Nokia, Nokia Shanghai Bell

##### QCL

R1-1805660	Feature lead summary on QCL	Nokia, NSB

R1-1805690	Offline discussion summary on QCL	Nokia, NSB

Agreement

Text proposal for TS38.211

----------------------------begin text change------------------------

4.4.1 Antenna ports

An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed.

-	For DM-RS associated with PDSCH the channel over which a symbol on one of these antenna ports is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols are within the same scheduled PDSCH, same slot and in the same PRG as described in 3GPP TS 38.214 Section 5.1.2.3.

---------------------------- end text change------------------------

Note that the above text change does not imply the support of time domain PRB bundling.

Working assumption on ‘same’ slot in Rel 15. If there is no consensus to revisit the working assumption in RAN1#93, it is automatically confirmed.

R1-1803640	Remaining issues and TP  for QCL assumptions	Huawei, HiSilicon

R1-1803750	Remaining issues on QCL	CATT

R1-1803827	Remaining issues on QCL	vivo

R1-1803915	Remaining issues on QCL	ZTE, Sanechips

R1-1804004	Text Proposal for QCL	OPPO

R1-1804724	Corrections to QCL for NR	Intel Corporation

R1-1804796	Remaining details on QCL	Qualcomm Incorporated

R1-1804987	Remaining issues on QCL	Ericsson

##### Other

R1-1803982	Evaluation on DMRS performance in high mobility scenario	OPPO

R1-1804281	Draft CR on PDSCH/PUSCH DMRS position for ECP in TS38.211	Huawei, HiSilicon

R1-1804282	Draft CR on PDSCH/PUSCH DMRS position for ECP in TS38.214	Huawei, HiSilicon

R1-1804284	SRS carrier switching involving SUL	Huawei, HiSilicon

R1-1804448	Remaining issues for SRS switching among CCs	Huawei, HiSilicon

R1-1804988	Analysis of periodic vs aperiodic TRS	Ericsson

R1-1804989	Benefits of resource specific DMRS mapping	Ericsson

R1-1804990	Evaluations of TRS periodicity	Ericsson

R1-1804991	Lossless compression of scheduling BW thresholds for PT-RS	Ericsson

R1-1804992	On correlation of DMRS Gold sequences	Ericsson

R1-1804993	On UE TX frequency accuracy with TRS based synchronization	Ericsson

R1-1804994	TP for TS 38.212 related to aperiodic and semi-persistent TRS	Ericsson

R1-1804995	TP for TS 38.214 related to DMRS sequence	Ericsson

R1-1804996	Type 1 mapping of halp-pi BPSK DFTS DMRS	Ericsson

R1-1804997	On a resource specific sequence mapping for CSI-RS	Ericsson

R1-1804998	On correlation of CSI-RS Gold sequences	Ericsson

R1-1804999	On additional periodicities for CSI-RS and SRS	Ericsson

#### Other

R1-1805000	NR UE feature list for MIMO and RS	Ericsson

R1-1805001	On RRC parameter alignment between 38.214 and 38.331	Ericsson

### Scheduling/HARQ aspects

#### Physical downlink control channel

R1-1805176	Further Notes for DCI Size Discussion	Ericsson

##### PDCCH structure

R1-1805537	Offline summary AI 7.1.3.1.1 PDCCH structure	NTT DOCOMO

Decision: The document is noted.

Proposals:

- For wideband RS CORESET,

  - The UE shall rate-match around PDCCH RBs containing the scheduling DCI as well as the DMRS across all the resource-element groups within the set of contiguous resource blocks in the CORESET.

  - The UE is not expected to be configured with non-contiguous RBs for the CORESET.

  - FFS interaction with BD/CCE limitations

From Thursday's offline

Agreements:

- NW and UE maintain the same understanding on SSB/CORESET#0/SS#0 in connected_mode at least for non-broadcast PDCCH

  - Solutions FFS

- For the broadcast PDCCH, it is up to UE which common search space to monitor based on which SSB in both connected, in-active, and idle modes

- Unicast PDSCH can be scheduled by a DCI associated with the CORESET #0

Agreement:

- The size of DCI 1_1 is the same (by zero-padding when necessary) across CORESETs of a BWP of a serving cell

Friday

Agreements:

- For WB RS CORESET, the UE is expected to be configured with the CORESET only with the following possibilities:

  - Up to 4 clusters, each cluster consisting of contiguous RBs.

- It is clarified that the counting rule of CCE for channel estimation is the same between NB RS CORESET and WB RS CORESET, which implies following:

  - The number of CCEs for channel estimation in case of WB RS CORESET is counted as the union of the sets of CCEs for PDCCH candidates to be monitored and hence BW of the WB RS CORESET is not restricted by CCE limit for channel estimation.

  - Note: it is up to RAN4 whether UE may perform channel estimation within a BW smaller than the maximum number of contiguous RBs within the WB RS CORESET

R1-1803790	PDCCH structure remaining issues	ZTE, Sanechips

R1-1804279	Remaining issues on PDCCH structure	Huawei, HiSilicon

R1-1804550	Remaining issues on PDCCH structure	LG Electronics

R1-1804759	Remaining details on PDCCH structure	Nokia, Nokia Shanghai Bell

R1-1804797	Remaining issues on PDCCH structure	Qualcomm Incorporated

R1-1804950	Collision between SSB and PDCCH	Panasonic

R1-1805177	On remaining issues of PDCCH structure	Ericsson

##### Search space

R1-1805568	Summary of offline for search space	NTT DOCOMO

Decision: The document is noted.

Agreement:

- For Rel.15 December 2017 version of Case 2, number of CCEs for channel estimation per slot is {56, 56, 48, 32} CCEs for SCS {15kHz, 30kHz, 60kHz, 120kHz}

Nokia, LGE raised concern to take it as an agreement, would prefer as a working assumption

Agreement:

- UE is not expected to be configured with PDCCH monitoring in CSS(s) for more than what UE can monitor in terms of numbers of BDs/CCEs

Proposals:

- For the non-CA case, in mapping PDCCH candidates in a slot, start with CSS monitored by the UE, then for USS, across search-space-sets:

  - Alt 1: SS with lower ID take precedence over SS with higher ID

  - Alt 2: alternating decoding candidates across SS sets?

  - FFS the case of CA

Wednesday

R1-1805655	Summary of offline for search space	NTT DOCOMO

Decision: The document is noted.

Agreements:

- Following working assumption is made:

  - For a UE supporting CA with up to X DL-CCs with the same numerology with X <= 4, maximum number of PDCCH blind decodes per slot the UE shall support is X*M, and;

  - For a UE supporting CA with up to Y DL-CCs with the same numerology Y > 4, maximum number of PDCCH blind decodes per slot the UE shall support is y*M, where;

    - M = {44, 36, 22, 20} for SCS = {15kHz, 30kHz, 60kHz, 120kHz}

    - y is an integer from {4, …, 16} and is reported as UE capability to the network.

  - For a UE supporting CA with up to X DL-CCs with the same numerology with X <= 4, maximum number of CCEs for channel estimation per slot the UE shall support is X*N, and;

  - For a UE supporting CA with up to Y DL-CCs with the same numerology with Y > 4, maximum number of CCEs for channel estimation per slot the UE shall support is y*N, where;

    - N = {56, 56, 48, 32} for SCS = {15kHz, 30kHz, 60kHz, 120kHz}

    - y is an integer from {4, …, 16} and is reported as UE capability to the network.

Agreement:

- The working assumption as part of the agreements below is confirmed:

  - Change Y_{p, kp} to Y_{p, ns,f }  in the search space hashing function in subclause 10.1 of 38.213, where the index ns,f  is the slot number.

    - (Working assumption) The reset of the update is per radio frame

Agreement:

- A UE is not expected to be configured to monitor PDCCH across a slot boundary.

Working assumption:

- For a common search space configured with RMSI-PDCCH-Config, osi-searchSpace, paging-searchSpace, and ra-searchSpace, DCI format 0_0/1_0 with C-RNTI is monitored in non-DRX occasions after C-RNTI is available.

- For a common search space configured with RMSI-PDCCH-Config, osi-searchSpace, paging-searchSpace, and ra-searchSpace, DCI format 0_0/1_0 with CS-RNTI is monitored in non-DRX occasions after CS-RNTI is available.

Agreements:

- Following is to be clarified in 213:

  - Type0-CSS is configured by RMSI-PDCCH-Config provided by MIB or is configured by searchSpace-SIB1 provided by PDCCH-ConfigCommon

  - Type0A-CSS is configured by searchSpace-OSI provided by PDCCH-ConfigCommon

  - Type1-CSS is configured by ra-SearchSpace provided by PDCCH-ConfigCommon

  - Type2-CSS is configured by pagingSearchSpace provided by PDCCH-ConfigCommon

  - Type3-CSS is configured by a SearchSpace with the searchSpaceType “common” provided by PDCCH-Config

From Thursday's offline

Working assumption:

- At least for Case 1-1 and Case 1-2, map all candidates of USS search-space-set with lower SS set ID before candidates of USS with higher ID

  - If all candidates in a SS set can’t be mapped, any candidates in the SS set and in any subsequent SS sets are dropped (not mapped)

- Case 2 FFS

Proposal:

- Update RRC signaling searchSpace such that it includes the set of periodicities defined for ssb-PeriodicityServingCell

  - These added periodicities are used only for searchSpaceSIB1

Friday

Email discussion till May 4th to come-up with identifying necessary information (e.g., periodicities, offsets, occasions) for updating RRC signaling searchSpace such that it includes the set of periodicities defined for ssb-PeriodicityServingCell.

Aim to target providing an LS to RAN2 – Fred (NTT DOCOMO)

R1-1803642	Remaining issues on search space	Huawei, HiSilicon

R1-1803751	Remaining details of NR PDCCH search space	CATT

R1-1803828	Remaining issues on PDCCH search space	vivo

R1-1803988	Remaining issues on Search Space	OPPO

R1-1804031	Remaining details on search space	ASUSTEK COMPUTER (SHANGHAI)

R1-1804043	Remaining issues on search space	MediaTek Inc.

R1-1804098	Discussion on PDCCH blind decoding dropping rule	CMCC

R1-1804214	On PDCCH candidate priority rules	Spreadtrum Communications

R1-1804240	Discussion on search space	Potevio

R1-1804370	Corrections on Search Space Design	Samsung

R1-1804551	Remaining issues on search space	LG Electronics

R1-1804725	NR PDCCH search spaces: BDs and channel estimation requirements	Intel Corporation

R1-1804760	On NR operation under PDCCH channel estimation and BD limits	Nokia, Nokia Shanghai Bell

R1-1804798	Remaining issues on control resource set and search space	Qualcomm Incorporated

R1-1804848	Remaining issues of search spaces 	InterDigital, Inc.

R1-1804951	Channel estimation complexity and search space	Panasonic

R1-1805048	Search space	NTT DOCOMO, INC.

R1-1805178	On remaining issues of search space	Ericsson

R1-1805263	Search space design and related issues	ZTE, Sanechips	(revision of R1-1803791)

R1-1805517	Remaining details on search space	Nokia, Nokia Shanghai Bell	(revision of R1-1804761)

##### Group-common PDCCH

R1-1805559	Offline discussion summary for GC-PDCCH carrying SFI	Qualcomm

Decision: The document is noted.

Agreement:

- Adopt the TP in R1-1805569 for 38.213 section 11.1

  - Note that it reflects the agreements reached before RAN1 #92bis related to TDD UL/UL configurations and SFI

Agreements:

- UE does not expect the reference SCS in TDD UL/DL configuration common and common2 to be different

- UE does not expect the reference SCS in cell-specific UL/DL configuration in a cell to be larger than the SCS of any BWP configured for the cell

Wednesday

R1-1805656	Offline discussion summary on GC-PDCCH carrying SFI	Qualcomm

Decision: The document is noted.

Agreement:

- Limit the size of the UE-specific SFI table to a max total of 512 values across all entries in Rel 15.

Proposal:

- In the slot format table in TS38.211, the entry 255 is defined such that the slot format is according to the RRC configuration

Agreements:

- On slot format indication for ECP, reuse the slot format indication for NCP, and determine the direction of an ECP symbol using the indicated direction of (partially) overlapping NCP symbols

  - If all (partially) overlapping NCP symbols are D, the ECP symbol is D

  - If all (partially) overlapping NCP symbols are U, the ECP symbol is U

  - If one of (partially) overlapping NCP symbols is X, the ECP symbol is X

Agreement:

- Regarding cancellation of RRC configured DL reception with a DCI granted UL transmission, or the cancellation of RRC configured UL transmission with a DCI granted DL reception, the cancellation is subject to a minimum time constraint, which follows N2 timeline

Proposal:

- Clarify that for the N2 for SFI or DCI based RRC configured UL transmission cancellation, the N2 is measured at the UE side, from the end of the OFDM symbol carrying the SFI or DCI to the beginning of the intended RRC configured UL transmission time.

Agreement:

- After active BWP switching, the SFI received before the BWP switching is still applicable to the new active BWP after switching.

Agreement:

- UE is not expected to monitor GC-PDCCH for SFI for a first cell in another cell with larger SCS than the first cell in Rel-15.

Proposals:

- For cancellation of RRC configured transmission or reception by SFI, the cancellation is for a unit of transmission/reception if any OFDM symbol within the unit is cancelled by SFI.

  - For RRC configured CSI-RS resource set, the cancellation unit is the CSI-RS resource set

    - Note that the CSI-RS resource set definition for the case of TRS vs. for the case of not-for-TRS is a separate issue

  - For RRC configured PDSCH and PUSCH with slot aggregation, the cancellation unit is the whole PDSCH or PUSCH within a slot

  - For RRC configured PDSCH, PUCCH, and PUSCH without slot aggregation, the cancellation unit is the whole PDSCH, PUCCH, and PUSCH

  - For RRC configured SRS transmission, the cancellation unit is OFDM symbol

Thursday

R1-1805720	Offline discussion summary on GC-PDCCH carrying SFI	Qualcomm

Decision: The document is noted.

Agreements:

- In UE-specific SFI table configuration, it is possible for the length of an entry to be longer than the configured monitoring period of the SFI

- For a slot covered by multiple SFIs transmitted at different slots, the UE does not expect to receive different slot format indicated by different SFIs.

  - If UE receives different slot formats for the same slot from different SFI, the UE behaviour is not defined.

Agreement

- In the slot format table in TS38.211, the entry 255 is defined such that when a slot format for a slot is indicated as 255, the UE does not use this information in deciding the cancellation of UE-specific RRC configured DL receptions or UE-specific RRC configured UL transmissions

Agreement

- When an RRC configured UL transmission is cancelled by SFI or DCI, the UE is not expected to cancel the part of RRC configured UL transmission that is to be transmitted over the OFDM symbols within N2 OFDM symbols after the end of the OFDM symbol carrying the SFI or DCI from UE perspective.

Agreements:

- For cancellation of RRC configured transmission or reception by SFI, the cancellation is for a unit of transmission/reception if any OFDM symbol within the unit is cancelled by SFI.

  - For RRC configured CSI-RS resource set, the cancellation unit is the CSI-RS resource set

  - For RRC configured PDSCH and PUSCH with slot aggregation, the cancellation unit is the whole PDSCH or PUSCH within a slot

  - For RRC configured PDSCH, PUCCH, and PUSCH without slot aggregation, the cancellation unit is the whole PDSCH, PUCCH, and PUSCH

  - For RRC configured SRS transmission, the cancellation unit is OFDM symbol

Agreements:

- For a grant based PDSCH, rate matching around RRC configured CSI-RS, if the CSI-RS is cancelled by setting SFI to “flexible” or the UE does not detect the SFI for the slot, the PDSCH still rate match around the CSI-RS RE locations.

  - This may not have spec impact

Agreement: (Friday)

- UE does not expect to be UE-specific RRC configured to perform UL transmission or DL reception in the same symbol

Proposal:

- For the reception of PDSCH granted by DCI scrambled by P-RNTI, SI-RNTI, RA-RNTI, and TC-RNTI(?), same behaviour as if the UE-specific RRC configuration is not configured (i.e, SIB1 configuration is available only)

Friday

R1-1805725	Offline discussion summary on GC-PDCCH carrying SFI	Qualcomm

Decision: The document is noted.

Agreement:

- In 38.213 section 11.1, when PDSCH granted by DCI with CRC scrambles by C-RNTI is mentioned, add CS-RNTI, P-RNTI, SI-RNTI, RA-RNTI, and TC-RNTI to the list of RNTIs as well

Agreements:

- For a UE-specific RRC configured UL transmission, if one OFDM symbol of the configured transmission falls on the semi-static DL symbol or a symbol SSB is transmitted as indicated, the UE shall cancel the transmission.

- For a UE-specific RRC configured DL reception, if one OFDM symbol of the configured reception falls on the semi-static UL symbol or a symbol PRACH is configured, the UE shall cancel the reception.

R1-1803643	Remaining issues on group-common PDCCH	Huawei, HiSilicon

R1-1803752	Open issues for semi-static and dynamic slot format indication	CATT

R1-1803792	Remaining issues for slot format	ZTE, Sanechips

R1-1803829	Remaining issues on GC-PDCCH	vivo

R1-1803989	Remaining issues on GC-PDCCH	OPPO

R1-1804044	Remaining issues in GC-PDCCH	MediaTek Inc.

R1-1804371	Corrections on UE-Group Common PDCCH	Samsung

R1-1804552	Remaining issues on group common PDCCH	LG Electronics

R1-1804613	Remaining issues for the Slot Format Indication	Nokia, Nokia Shanghai Bell

R1-1804726	GC PDCCH: Some remaining details	Intel Corporation

R1-1804799	Remaining issues for slot format indication	Qualcomm Incorporated

R1-1804881	Remaining issues on slot format for NR	Sharp

R1-1805049	Remaining issues on group-common PDCCH	NTT DOCOMO, INC.

R1-1805179	On remaining issues of GC-PDCCH	Ericsson

R1-1805240	Remaining issues on group-common PDCCH for NR	WILUS Inc.

##### DCI contents and formats

R1-1805601	Outcome of offline discussion on 7.1.3.1.4	Ericsson

Decision: The document is noted.

Wednesday

R1-1805638	Summary of offline discussion on 7.1.3.1.4 part II	Ericsson

Agreement:

- Confirm the following working assumption:

  - (working assumption) At most 4 different DCI sizes are monitored by the UE per slot

    - At most 3 different DCI sizes are monitored per C-RNTI per slot

Conclusion:

- It is understood that DCI sizes to monitor do not vary dynamically from slot-to-slot (other than impact due to BWP switching) but are based on RRC configuration, CSS vs. USS, and/or active BWP.

Agreements:

- To confirm the following working assumption with update

Working assumption:

  - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by

    - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWP

    - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled

      - FFS: Otherwise, for format 0-0/1-0, the size is given by the initial DL BWP

Agreements:

- For DCI format 1-0 in CSS with P-RNTI, SI-RNTI, RA-RNTI, C-RNTI, CS-RNTI, or TC-RNTI:

  - the RB numbering for the scheduled PDSCH starts from the lowest RB in the CORESET the DCI is received in

  - the maximum number of RBs possible to indicate in the DCI is given by the size of the initial DL BWP.

  - The case of TC-RNTI is a working assumption

  - The case of C-RNTI/CS-RNTI is at least applicable to non-CA case and Pcell in CA

    - FFS other cases

Agreements:

- Confirm the following working assumption:

  - (working assumption) Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 as defined by the initial BWP

Proposals:

- Confirm the following working assumption:

  - Sizes of all DCI bitfields in DCI formats 0-1 and 1-1 in USS determined by current BWP. Data transmitted on the BWP indicated by the BWP index. If the BWP index activates another BWP, transform as follows:

    - Zero-pad too small bitfields to match the new BWP

    - Truncate too large bitfields to match the new BWP

- Support null assignment in BWP switching, i.e., switching without scheduling data

Friday

R1-1805735	Summary of offline discussions on 7.1.3.1.4 (DCI contents and formats) – part III	Ericsson

Decision: The document is noted.

Agreements:

When a valid DCI with P-RNTI is detected, the following fields apply:

| Field | Bits | Comment |
| --- | --- | --- |
| Short Messages Indicator | 1 | Already captured in 38.212. Need to add interpretation. 0 = short message ([8] bits defined by RAN2 + padding to meet the DCI size) 1 = paging message scheduled (following fields) |
| In case of short message service |  |  |
| Short message | [8] | Defined by RAN2. The number of bits to be checked with RAN2. |
| Reserved bits |  | To meet overall DCI size |
| In case of paging message |  |  |
| Frequency domain resource assignment |  |  |
| Time domain resource assignment | 4 | Note: applicability of the currently agreed default table to paging or whether an additional default table or SIB1 signaled table is FFS. |
| VRB-to-PRB mapping | 1 |  |
| Modulation and coding scheme | 5 | Same MCS table as for “normal” transmission without 256QAM |
| TB scaling | 2 | To scale TB size |
| Reserved bits | (fixed value) | To meet overall DCI size |

- Note: If initial access session defines the possibility for simultaneous short message service and paging, the DCI design may need to be updated. From a DCI payload perspective, concurrent transmission of short messages and paging is possible.

- Note: The TB size and RB allocation are dependent and there may be a need to apply TB size scaling to obtain low code rates for small payloads.

Agreements:

- For PDSCH scheduled by P-RNTI (working assumption), and RA-RNTI, scaling factor is applied in the calculation of Ninfo of TBS determination

  - (Working assumption) values of scaling factor: 1, ½, ¼

    - FFS the possibility of adding another value, e.g., 1/8 or 1/16

Agreements:

When a valid DCI with SI-RNTI is detected in either CSS type 0 or CSS type 0a, the following fields apply:

| Field | Bits | Comment |
| --- | --- | --- |
| Frequency domain resource assignment |  |  |
| Time domain resource assignment | 4 | Index into default table |
| VRB-to-PRB mapping | 1 | FFS whether this field is needed or not |
| Modulation and coding scheme | 5 | Same MCS table as for “normal” transmission without 256QAM, only lowest part used. |
| Redundancy version | 2 | FFS whether this field is needed or not |
| Reserved bits | (fixed value) | To meet fallback DCI size |

- Note: Need to check whether interleaved VRB-to-PRB mapping is possible for SI-RNTI as the UE does not know the CRB grid.

- Note: Need to check with initial access session whether RV field is needed or not. From a DCI perspective, there is no problem of providing 2 bits for signaling the redundancy version.

Agreements:

- 4 bits are used for time-domain allocation field for RA-RNTI

- RV0 is used for transmissions using RA-RNTI

- 4 bits are used for time-domain resource allocation field for TC-RNTI

Working assumption:

- Interpret RIV in RAR for initial msg3 according to the same method as in LTE

Agreements:

Time domain allocation field in RAR for initial msg3 interpreted according to

- use table provided by RMSI (if provided)

- otherwise use default PUSCH table

Agreements:

- SUL indicator for DCI format 0-0 is located last in the DCI (including padding)

- FH bit is optional in DCI format 0-1

Conclusion:

- Remove UL VRB-to-PRB interleaver from Rel-15

R1-1803707	Remaining issues on DCI contents and format	Huawei, HiSilicon

R1-1803753	Open issues on DCI formats	CATT

R1-1803793	Remaining issues on DCI content	ZTE, Sanechips

R1-1803830	Remaining issues on DCI formats	vivo

R1-1803941	Frequency domain field interpretation for fallback DCI in DL	Fujitsu

R1-1803965	Discussion on DCI format 0-0/0-1	ITRI

R1-1804033	Discussion about number of DCI format size	ASUSTEK COMPUTER (SHANGHAI)

R1-1804045	Remaining issues on DCI contents and formats	MediaTek Inc.

R1-1804099	Discussion on remaining issues for DCI	CMCC

R1-1804219	Remaining issues on DCI contents and formats	Spreadtrum Communications

R1-1804372	DCI Contents and Formats	Samsung

R1-1804553	Remaining issues on DCI contents and formats	LG Electronics

R1-1804614	On the alignment of DCI format sizes	Nokia, Nokia Shanghai Bell

R1-1804656	DCI size reduction for eMBB	AT&T

R1-1804727	Further considerations on DCI sizes	Intel Corporation

R1-1804800	Discussion on DCI related issues	Qualcomm Incorporated

R1-1805050	DCI contents and formats	NTT DOCOMO, INC.

R1-1805080	DCI format 1_0 with X-RNTIs	Panasonic Corporation

R1-1805180	On remaining issues of DCI	Ericsson

R1-1805241	Discussion on DCI contents and formats	WILUS Inc.

##### Other

R1-1803652	Discussion and TP on slot format for ECP	Huawei, HiSilicon

R1-1803708	DCI interpretation for different BWP	Huawei, HiSilicon

R1-1803710	Summary of remaining issues on  PDCCH monitoring with TP	Huawei, HiSilicon

R1-1803831	On compact DCI	vivo

R1-1804426	DCI contents and formats for RMSI/OSI/Paging/RAR	Huawei, HiSilicon

R1-1805181	TP for remaining issues of GC-PDCCH	Ericsson

R1-1805182	TP for remaining issues of DCI	Ericsson

R1-1805251	Modifications to the specification for UE procedure for receiving control information	Ericsson

R1-1805569	TP for 38.213 on SFI section 11.1	Qualcomm Incorporated	(revision of R1-1804801)

#### Physical uplink control channel

##### PUCCH structure in short-duration

R1-1805560	Summary of Partially Overlapped PUCCH Resources	Ericsson

Decision: The document is noted.

Proposals:

- When SR PF1 resource partially overlap with an HARQ-ACK PF 3 or 4 resource with a starting symbol before the SR PUCCH resource, the HARQ-ACK and SR are multiplexed using the agreed multiplexing method when the SR PF1 resource and HARQ-ACK PUCCH resource had the same stating time.

- When SR PF0 resource partially overlap with an HARQ-ACK PF0 or 2 resource with a starting symbol before the SR PUCCH resource, the HARQ-ACK and SR are multiplexed using the agreed multiplexing method when the SR PF1 resource and HARQ-ACK PUCCH resource had the same stating time.

- Consider potential impact of UE processing time

R1-1805561	Summary of Remaining Issues on Short PUCCH	Ericsson

Decision: The document is noted.

Proposals:

- To adopt the following TP for 38.213, Section 9.2.3:

A UE may transmit one or more two PUCCHs on a serving cell in different symbols within a slot of  symbols as defined in [4, TS 38.211]. In case of two PUCCHs in the slot, the possible combinations are given by Table xxx

- Offline discussion to finalize the table based on previous agreements

Check futher offline the agreements (tx vs. configuration)

Agreements:

- TS38.212 to catpure that in case of multiplexing CSI with SR in a same PUCCH format, bit sequence order is that SR bit(s) are mapped to the UCI bit sequence first and CSI bits are mapped to the UCI bit sequence second.

  - The corresponding description is to be removed from 38.213

Friday

Agreement:

- To adopt the following TP for 38.213, Section 9.2.3:

A UE may transmit one or more two PUCCHs on a serving cell in different symbols within a slot of  symbols as defined in [4, TS 38.211]. In case of two PUCCHs in the slot, at least one of them is based on PUCCH format 0 or 2.

R1-1805636	Partially overlapped PUCCH resources	Ericsson

R1-1805698	Multiplexing rules for PUCCH resources with the same starting symbols	Ericsson

Decision: The document is noted.

Agreements:

- When HARQ-ACK PUCCH resource with PUCCH F1 overlaps with SR PUCCH resource with PUCCH F0 with the same starting symbol in a slot,

  - The UE transmits only HARQ-ACK in the HARQ-ACK PUCCH resource ( i.e the UE does not transmit SR if it is positive).

Agreements:

- When AN/SR and configured CSI PUCCH resources based on PF3 or 4 have the same starting symbols in a slot, when the UE determines the PUCCH resource set is based on the total number of AN/SR and CSI and the PUCCH resource based on ARI, if the PUCCH resource based on ARI for transmitting HARQ-ACK/SR and CSI is determined to PF2,

  - Drop CSI part 2 (if present). Transmit jointly encoded AN/SR and CSI part 1 on the determined PUCCH format 2 resource indicated by ARI using the mapping rules for PF2.

Agreements:

- When AN/SR and CSI PUCCH resources have the same starting symbols in a slot, for a wideband semi-persistent/periodic CSI reporting based on a PUCCH format 2, if the PUCCH resource based on ARI for transmitting HARQ-ACK/SR and CSI is based on format 3:

  - CSI (only CSI part 1) is jointly encoded with HARQ-ACK/SR, following the existing procedure of encoding and mapping on resources for PUCCH format 3 or 4 to transmit the UCI.

Agreements:

- When a configured CSI PUCCH resource overlaps with one or more configured SR PUCCH resources in a slot,

  - The UE multiplexes all UCIs on the CSI PUCCH resource using the corresponding UCI multiplexing rules for the overlapping PUCCH resources with the same starting symbol.

  - FFS: When one SR PUCCH resource overlaps with two non-overlapping CSI PUCCH resources

R1-1805764	Summary of offline discussion	Ericsson

Decision: The document is noted.

Proposals

- When HARQ-ACK/SR PUCCH resource determined by ARI and configured CSI PUCCH resources have the same starting symbols in a slot, if the UE is configured with only one PUCCH resource set and simultaneous transmission of HARQ-ACK/SR and CSI is enabled, select from one of the following alternatives:

  - Alt 1: The UE is not expected to transmit the corresponding HARQ-ACK/SR and the CSI report in a PUCCH resource in that slot.

    - Supportive companies: Ericsson, Intel, OPPO, Qualcomm, Huawei, Samsung, Panasonic

  - Alt 2: HARQ-ACK/SR and CSI are transmitted together by PUCCH resource for CSI

    - Supportive companies:  Nokia, MediaTek, Lenovo, Motorola, ZTE, CATT, NTT DOCOMO

  - Alt 3: HARQ-ACK/SR is transmitted on the HARQ-ACK/SR PUCCH resource and CSI reporting is dropped in that slot.

    - Supportive companies: LG, Vivo

Discuss further till next meeting

Proposals:

- Select one of the following two alternatives:

  - Alt 1: If a HARQ-ACK transmission from a UE using PUCCH format 1 determined by ARI overlaps with K SR occasions each using PUCCH format 1, in case of negative SR, the HARQ-ACK is transmitted on the HARQ-ACK resource. In case of positive SR, the HARQ-ACK is transmitted on the SR resource corresponding to the positive SR.

    - Supportive companies: Nokia, Ericsson, CATT, LG, NTT DOCOMO, Huawei, Vivo, Panasonic

  - Alt 2: PUCCH format 2 or 3 or 4 is used to carry L = 1 or 2 bits HARQ-ACK and K > 1 SR.  That implies when a UE has K SRs configured in a same time position,

    - For 1 or 2 bits HARQ-ACK, if K=0 or 1, ARI field in DCI indicates PUCCH format 0 or 1; otherwise, ARI field in DCI indicates PUCCH format 2 or 3 or 4.

    - Supportive companies: Samsung, ZTE

Discuss till next meeting

R1-1803644	Remaining issues on PUCCH structure in short-duration	Huawei, HiSilicon

R1-1803754	On multiplexing of different UCI transmissions	CATT

R1-1803794	Remaining issues on short PUCCH	ZTE, Sanechips

R1-1803832	Remaining issues on PUCCH with short-duration	vivo

R1-1804005	Multiplexing of HARQ-ACK and SR	OPPO

R1-1804373	Corrections on PUCCH for up to 2 Symbols	Samsung

R1-1804458	Remaining open items on short PUCCH	Nokia, Nokia Shanghai Bell

R1-1804481	Discussion on partial overlap between HARQ-ACK and SR	Panasonic Corporation

R1-1804554	Remaining issues on short-duration PUCCH	LG Electronics

R1-1804728	Handling collision between HARQ-ACK/SR and CSI	Intel Corporation

R1-1804802	Remaining issues for short PUCCH	Qualcomm Incorporated

R1-1805051	PUCCH structure in short-duration	NTT DOCOMO, INC.

R1-1805131	Remaining Issues on Overlapping of HARQ-ACK and SR	III

R1-1805183	On UCI Multiplexing on PUCCH	Ericsson

R1-1805699	SR ID for multiple overlapping SR PUCCHs with UCI PUCCH	Ericsson

##### PUCCH structure in long-duration

R1-1805526	Review Summary for Al 7.1.3.2.2 PUCCH structure in long-duration	Huawei

Decision: The document is noted.

Proposals:

- The meaning of the capability of FH (frequency-hopping) is UE is configured the higher layer parameter intraSlotFrequencyHopping as “enabled”, regardless of whether the values of startingPRB and secondHopPRB are the same or not.

- The meaning of the capability of Non-frequency hopping is UE is configured the higher layer parameter intraSlotFrequencyHopping as “disabled”.

- In the table 6.3.2.4.1-1 of TS38.211,

  - No intra-slot hopping is replaced as intraSlotFrequencyHopping = “disabled”.

  - Intra-slot hopping is replaced as intraSlotFrequencyHopping = “enabled”.

Proposals:

  - When AN/SR and CSI PUCCH resources have the same starting symbols, UE assumes a fixed rank for CSI part 2 in determination of PUCCH resource set from which one PUCCH resource is used for transmission of AN/SR and CSI

    - FFS: the value of fixed rank

- Also consider the case of PUSCH

Wednesday

R1-1805615	Outcome of the offline discussion for PUCCH structure in long-duration	Huawei

Decision: The document is noted.

Proposed conclusion:

- The meaning of the capability of FH (frequency-hopping) is UE is configured the higher layer parameter intraSlotFrequencyHopping as “enabled” including the case where the values of startingPRB and secondHopPRB are the same.

- When intraSlotFrequencyHopping is enabled, UE is not required to keep phase coherence or use the same antenna port between the two hops even when the values of startingPRB and secondHopPRB are the same

  - FFS the issue of phase coherent within one hop (including the case when hopping is disabled)

- No spec impact for the above two bullets

Proposals:

- When AN/SR and SP-CSI PUCCH resources have the same starting symbols, the following options can be considered (down selected)

  - Opt1: Drop SP-CSI and only transmit AN/SR

  - Opt2: Drop CSI part2 for SP-CSI and only transmit AN/SR and CSI part1

  - Opt3: Multiplex AN/SR and SP-CSI on a PUCCH resource. The reference payload for CSI part 2 could be estimated based on one of the following options (down selected)

    - Opt1: Assume rank 1

      - Ericsson, MediaTek, Samsung, Intel, Qualcomm, LGE

    - Opt2: Assume maximum size of CSI part 2

      - OPPO, Huawei, Lenovo

Friday

R1-1805733	Outcome of the offline discussion for PUCCH structure in long-duration	Huawei

Decision: The document is noted.

Agreements:

- To adopt the following TP to 9.2.5.2 of 38.213

---- Start of TP ----

- else, the UE selects  CSI report(s) for transmission together with HARQ-ACK/SR in ascending order of , where  is determined according to [6, TS 38.214]; the value of  satisfies OACK+OSR+n=1NCSIreportedOCSI,n+OCRCOCRC,N≤MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH∙Qm∙r and  OACK+OSR+n=1NCSIreported+1OCSI,n+OCRCOCRC,N+1>MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH∙Qm∙r, where  is the number of CSI report bits for the  CSI report in ascending order of , OCRC,N is the number of CRC bits corresponding to OACK+OSR+n=1NCSIreportedOCSI,n and OCRC,N+1 is the number of CRC bits corresponding to OACK+OSR+n=1NCSIreported+1OCSI,n.

---- End of TP ----

Agreement:

- When AN/SR and SP-CSI PUCCH resources have the same starting symbol, multiplexing AN/SR and SP-CSI on a PUCCH resource. The reference payload for CSI part 2 is based on assuming by rank 1.

Proposal: Support CGS ordering for DFT-s- for length-12, length-18 and length-24 CGS

Discussion: No consensus regarding the timeline for introducing the new CGS – proponents believe it should be done now in NR – opponents believe it is kind of optimization

Huawei: re-ordering the CGS tables should not be considered as a big issue and requested objecting companies going forward

Objected by Qualcomm and Samsung

R1-1803645	Remaining issues on PUCCH structure in long-duration	Huawei, HiSilicon

R1-1803795	Issues related to Long PUCCH	ZTE, Sanechips

R1-1803833	Remaining issues on PUCCH with long-duration	vivo

R1-1804006	Remaining issues on CSI reporting using PUCCH	OPPO

R1-1804066	Remaining issues of long-PUCCH	MediaTek Inc.

R1-1804100	Discussion on PUCCH repetition	CMCC

R1-1804212	Multiplexing of different type of UCIs	Lenovo, Motorola Mobility

R1-1804374	Corrections on PUCCH for more than 2 Symbols	Samsung

R1-1804459	Remaining open items on long PUCCH	Nokia, Nokia Shanghai Bell

R1-1804493	Remaining issues for UCI repetition	ETRI

R1-1804555	Remaining issues on long-duration PUCCH	LG Electronics

R1-1804729	PUSCH-PUCCH and PUCCH-PUCCH collision handling	Intel Corporation

R1-1804803	Remaining issues for long PUCCH	Qualcomm Incorporated

R1-1804882	Discussions on RB selection for PUCCH resource	Sharp

R1-1805052	PUCCH structure in long-duration	NTT DOCOMO, INC.

R1-1805184	On PUCCH Remaining Issues	Ericsson

R1-1805242	Remaining issues on long PUCCH over multiple slots	WILUS Inc.

##### UCI multiplexing

R1-1805554	Summary of remaining issues for UCI multiplexing on PUSCH	Qualcomm

Decision: The document is noted.

Proposals:

- For SP-CSI on PUSCH or A-CSI-only on PUSCH colliding with other UL transmissions with the same starting symbol, adopt the proposals in the table below

| SP-CSI on PUSCH | A-CSI-only on PUSCH | HARQ-ACK | SR | PUSCH with UL-SCH | Proposal |
| --- | --- | --- | --- | --- | --- |
|  | * |  | * |  | Drop A-CSI-only on PUSCH, transmit SR. |
|  | * | * | * |  | Drop A-CSI-only on PUSCH, transmit SR and HARQ-ACK |
| * |  |  | * |  | Drop SP-CSI on PUSCH, transmit SR |
| * |  | * |  |  | Multiplex HARQ-ACK on SP-CSI on PUSCH |
| * |  | * | * |  | Drop SP-CSI on PUSCH, transmit HARQ-ACK and SR |
| * |  |  |  | * | Drop SP-CSI on PUSCH |

- FFS when the starting symbols among UL transmissions are different in the above scenarios.

Tuesday session

R1-1805599	Summary of remaining issues for UCI multiplexing on PUSCH	Qualcomm

Decision: The document is noted.

Agreements:

- For SP-CSI on PUSCH or A-CSI-only on PUSCH colliding with SR/HARQ-ACK with the same starting symbol within the same CC, adopt the proposals in the table below

| SP-CSI on PUSCH without UL-SCH | A-CSI-only on PUSCH without UL-SCH | HARQ-ACK | SR | Proposal |
| --- | --- | --- | --- | --- |
|  | * |  |  | Drop A-CSI-only on PUSCH, transmit SR on PUCCH. (follow LTE approach) |
|  | * | * | * | Drop A-CSI-only on PUSCH, transmit SR and HARQ-ACK on PUCCH. (follow LTE approach) |
| * |  |  | * | Drop SP-CSI on PUSCH, transmit SR on PUCCH (follow LTE approach) |
| * |  | * | * | Drop SP-CSI on PUSCH, transmit HARQ-ACK and SR on PUCCH (follow LTE approach) |

- The symbol “*” in SR means when SR is positive

- FFS when the starting symbols among UL transmissions are different in the above scenarios.

Agreements:

- For SP-CSI on configured PUSCH without UL-SCH colliding with a PUSCH with UL-SCH with overlapped symbols in time

  - When starting symbols between the two channels are aligned, drop SP-CSI (following MIMO agreements made in last meeting)

  - When starting symbols between the two channels are not aligned

    - FFS

    - Note: this replaces the MIMO agreements made earlier

Agreement:

- Adopt the following TP in Section 6.2.7 of 38.212:

for  to

;

;

;

;

end for

Conclusion:

- It is clarified that based on this sentence of “A UE is not expected to receive more than one aperiodic CSI report request for a given slot” in Section 5.2.1.5.1 in 38.214 V15.0.1, for a given slot, UE is not expected to transmit CSI report triggered by more than one CSI requests received in multiple DCIs, in both CA and non-CA scenarios, unless for a given CC where it is possible that two or more DCIs may trigger the same CSI report

- No spec update is necessary.

Wednesday

R1-1805666	Summary of remaining issues for UCI multiplexing on PUSCH	Qualcomm

Decision: The document is noted.

Proposals:

- When single-slot PUCCH overlaps with single-slot PUCCH or single-slot PUSCH in slot n for a PUCCH group,

  - If the first symbol of the earliest PUCCH/PUSCH starts no earlier than symbol N1+X after the last symbol of PDSCH(s) and no earlier than N2+Y after the last symbol of PDCCHs scheduling UL transmissions including HARQ-ACK and PUSCH(only applicable to grant based PUSCH) for slot n, the UE multiplex all UCIs on either one PUCCH or one PUSCH, using the same UCI multiplexing rule defined with the same starting symbols.

  - Otherwise, it is considered as an error case.

- The definition of N1 and N2 follows the same definition in current NR spec.

- X and Y are non-negative integer values.

- FFS on values of X and Y

- FFS different Y values for grant-based vs grant free PUSCH

- FFS multiplexing rule when AN PUCCH resource with F1 overlaps with SR PUCCH resource with F0.

- FFS how to handle ACK-bundling when partial HARQ-ACK bits cannot fulfil the timeline requirement.

- FFS how to handle one PUCCH overlap with multiple PUSCHs which satisfies timeline requirement.

- FFS How to handle two PUCCH groups when the two PUCCH groups are on contiguous intra-band CCs

- Note: The above proposal does not override the dropping rules defined for ACK/SR colliding with A-CSI-only on PUSCH without UL-SCH, or ACK/SR colliding with SP-CSI on PUSCH without UL-SCH,  and SP-CSI on PUSCH colliding with PUSCH with UL-SCH.

Friday

Working assumption:

- When single-slot PUCCH overlaps with single-slot PUCCH or single-slot PUSCH in slot n for a PUCCH group,

  - The UE multiplex all UCIs on either one PUCCH or one PUSCH, using the existing UCI multiplexing rule, if both following conditions are satisfied:

    - If the first symbol of the earliest PUCCH(s)/PUSCH(s) among all the overlapping channels starts no earlier than symbol N1+X after the last symbol of PDSCH(s)

    - If the first symbol of the earliest PUCCH(s)/PUSCH(s) among all the overlapping channels starts no earlier than N2+Y after the last symbol of PDCCHs scheduling UL transmissions including HARQ-ACK and PUSCH (if applicable) for slot n

  - If at least one pair of overlapping channels does not meet the above timeline requirements, UE consider it is an error case for all UL channels in the group of overlapping channels. UE behavior is not specified.

- The definition of N1 and N2 follows the same definition in current NR spec.

- X and Y are non-negative integer values.

- FFS on values of X and Y

- FFS on timeline requirement for multiplexing UCIs on PUSCH with A-CSI.

- FFS how to handle one PUCCH overlap with multiple PUSCHs which satisfy timeline requirement.

- FFS: how to handle HARQ-ACK for semi-static PDSCH.

- FFS multiplexing rule when AN PUCCH resource with F1 overlaps with SR PUCCH resource with F0.

- FFS: how to handle semi-statically configured PUCCH overlap with semi-statically configured PUCCH or PUSCH.

- Note: The above proposal does not override the dropping rules defined for ACK/SR colliding with A-CSI-only on PUSCH without UL-SCH, or ACK/SR colliding with SP-CSI on PUSCH without UL-SCH.

- Note: Consider how to handle PUCCH colliding with other UL channels in NR Rel. 15 June drop when URLLC is taking into account.

Agreements:

- For HARQ-ACK, CSI part 1, and CSI part 2 (if exists) transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK, CSI part 1, and CSI part 2 (exists), are determined as follows:

  - SE is the spectrum efficiency which is code rate * modulation order

  - FFS: UE is not expected to transmit A-CSI on PUSCH without UL-SCH, if the actual coding rate for CSI-part 1 is smaller than T_m for a modulation order m.

    - When A-CSI is dropped, HARQ-ACK if exists is transmitted on PUCCH resource.

    - FFS the value of T_m for each modulation order m

R1-1803646	Remaining issues on UCI multiplexing	Huawei, HiSilicon

R1-1803755	Remaining aspects of UCI multiplexing on PUSCH	CATT

R1-1803834	Remaining issues on UCI multiplexing	vivo

R1-1804007	Remaining details on UCI multiplexing	OPPO

R1-1804067	Remaining issues of UCI on PUSCH	MediaTek Inc.

R1-1804213	Multiplexing between PUCCH and PUSCH	Lenovo, Motorola Mobility

R1-1804375	Corrections on UCI Multiplexing in PUSCH	Samsung

R1-1804460	Remaining open items on UCI multiplexing	Nokia, Nokia Shanghai Bell

R1-1804482	Discussion on partial overlap between PUCCH and PUSCH	Panasonic Corporation

R1-1804556	Remaining issues on UCI multiplexing	LG Electronics

R1-1804730	UCI on PUSCH	Intel Corporation

R1-1804804	Remaining issues for multiplexing UCI on PUSCH	Qualcomm Incorporated

R1-1804883	UCI multiplexing with NUCI based PUCCH resource set selection	Sharp

R1-1805053	UCI multiplexing on PUSCH	NTT DOCOMO, INC.

R1-1805185	On UCI Multiplexing on PUSCH	Ericsson

R1-1805252	Remaining issues on UCI multiplexing on PUSCH	CATR

##### Resource allocation for PUCCH

R1-1805578	Summary of offline discussion on PUCCH resource allocation	OPPO

Decision: The document is noted.

Proposal from offline

For PUCCH resource allocation with NUCI≤2 after a UE has a dedicated PUCCH configuration, when >8 resources are configured, down-select from following two options for determining the PUCCH resource in the resource set:

- Option 1: ARI-indicated resource sub-set + CCE-index-based implicit mapping within sub-set

- Support: Nokia, NSB, Qualcomm, OPPO, LGE, Ericsson, ZTE

  - Resource sub-set m is indicated by ARI

  - The resource index r within the resource sub-set m is implicitly derived based on the equation:

rm= CCEstartRmNCCE,p

      - NCCE,p is the total number of CCEs in CORESET p

      - Rm is the size of resource sub-set m

- Option 2: CCE-index-based implicit determination of a PUCCH resource + ARI-indicated offset (EPDCCH-like approach)

- Support: Samsung, Huawei, HiSilicon, Lenovo, MotM

  - The resource index r within the resource set is implicitly derived based on the equation:

r=Mod( CCEstartRNCCE,p+ARI, R)

      - NCCE,p is the total number of CCEs in CORESET p

      - R is the size of the resource set

Samsun's concern with option 1 and granularity (1 resource or 4 resources?)

Agreements:

For PUCCH resource allocation with NUCI≤2 after a UE has a dedicated PUCCH configuration, when >8 resources are configured, the following is used to determine the PUCCH resource in the resource set:

- ARI-indicated resource sub-set + CCE-index-based implicit mapping within sub-set

  - Resource sub-set m is indicated by ARI

  - The resource index r within the resource sub-set m is implicitly derived based on at least the following two parameters:

    - NCCE,p is the total number of CCEs in CORESET p

    - Rm is the size of resource sub-set m

Wednesday

R1-1805612	Summary of offline discussion on PUCCH resource allocation	OPPO

Decision: The document is noted.

Above agreements replaced by following agreements:

- For PUCCH resource allocation with NUCI≤2 after a UE has a dedicated PUCCH configuration, when >8 resources are configured,

  - The resource index r within the resource set is derived by:

r=i=0mRi-Rm+  CCEstartRmNCCE,p

    - m is the ARI value

    - NCCE,p is the total number of CCEs in CORESET p of the corresponding PDCCH

    - R is the resource set

    - |Ri| is given by:

Ri=R8,  if i<ModR, 8R8,  if i≥ModR, 8

    - CCEstart is the starting CCE index of the last received DCI within the ACK bundling window

      - In case of CA, CCEstart is the starting CCE index of the DCI received on the CC with the smallest CC index in the last slot where a DCI is received within the ACK bundling window

        - FFS the case of different SCS of CCs in CA

    - Note: for m>0, the summation in the equation is equivalent to having the upper bound as m-1 without substracting |Rm| - up to editor how to capture

Proposals:

For PUCCH resource allocation with before a UE has a dedicated PUCCH configuration,

- The resource index r within the resource set is derived by:

r=2m+  CCEstartRmNCCE,p

- m is the ARI value

- NCCE,p is the total number of CCEs in CORESET p

- |Ri| is given by:

Ri=R8,  if i<ModR, 8R8,  if i≥ModR, 8

- CCEstart is the starting CCE index of the DCI containing the ARI

Agreements: Confirm the following working assumption:

- For HARQ-ACK resource allocation before a UE has a dedicated PUCCH configuration,

  - Frequeny hopping is always enabled for PUCCH transmission for FR1 and FR2.

Proposals:

For PUCCH resource allocation before a UE has a dedicated PUCCH configuration,

- For each PUCCH duration in the RMSI table,

  - 4 RMSI values are used to indicate 4 PUCCH resource sets respectively

    - The same 4 cell-specific PRB offsets from edge of initial UL BWP are specified

      - Option 1: Defined as x PRBs (using the outermost PRBs)

      - Option 2: Defined based on the initial BWP size, e.g. {0, 18NBWP, 28NBWP, 38NBWP}

    - FFS: Necessity of cell-specific Intial CS indication in RMSI table

| RMSI | PUCCH duration | Starting symbol | Cell-specific PRB offset |
| --- | --- | --- | --- |
| 0 | 2-symbol | #12 | Offset #1 |
| 1 |  |  | Offset #2 |
| 2 |  |  | Offset #3 |
| 3 |  |  | Offset #4 |
| 4 | 4-symbol | #10 | Offset #1 |
| 5 |  |  | Offset #2 |
| 6 |  |  | Offset #3 |
| 7 |  |  | Offset #4 |
| 8 | 10-symbol | #4 | Offset #1 |
| 9 |  |  | Offset #2 |
| 10 |  |  | Offset #3 |
| 11 |  |  | Offset #4 |
| 12 | 14-symbol | #0 | Offset #1 |
| 13 |  |  | Offset #2 |
| 14 |  |  | Offset #3 |
| 15 |  |  | Offset #4 |

Friday

R1-1805667	Summary of offline discussion on PUCCH resource allocation	OPPO

Decision: The document is noted.

Agreements:

For PUCCH resource allocation before a UE has a dedicated PUCCH configuration,

- One RMSI value is used for indicating cell-specific PRB offset = 0 for PUCCH duration = 2, 4, 10,14 symbols respectively.

- (working assumption) One RMSI value is used for indicating cell-specific PRB offset = Floor(NBWP/4) for PUCCH duration = 14 symbols.

  - NBWP is the size of initial UL BWP

Agreements:

For PUCCH resource allocation before a UE has a dedicated PUCCH configuration,

- Up to two sets of initial CS shift indices can be specified for different RMSI values for a specific PUCCH format

Agreements:

For PUCCH resource allocation with NUCI≤2 after a UE has a dedicated PUCCH configuration, when >8 resources are configured,

- Adopt the following equation for 38.213 subclause 9.2.3 for resource index r within the resource set:

r= m*RPUCCH8+CCEstartRPUCCH8NCCE,pif m<RPUCCH mod 8m*RPUCCH8+RPUCCH mod 8+CCEstartRPUCCH/8NCCE,pif m ≥RPUCCH mod 8

Where, m is the PUCCH resource indicator field value, RPUCCH is the number of the PUCCH resources given by higher layer parameter maxNrofPUCCH-ResourcesPerSet. CCEstart is the starting CCE index of the last received DCI within the ACK bundling window, In case of CA, CCEstart is the starting CCE index of the DCI received on the CC with the largest CC index in the last slot where a DCI is received within the ACK bundling window . And NCCE,p is the number of CCEs in control resource set p. r is the PUCCH resource index within the PUCCH resource set. r=0… RPUCCH-1.

Agreements:

For PUCCH resource determination within a resource set before a UE has a dedicated PUCCH configuration, besides 3-bit ARI, the additional 1-bit indication is based on

- CCE-index-based implicit mapping same as that after UE has a dedicated PUCCH configuration

R1-1805742	Summary of offline discussion on PUCCH resource allocation	OPPO

R1-1803647	Remaining issues on resource allocation for PUCCH	Huawei, HiSilicon

R1-1803756	Remaining issues on PUCCH resource allocation	CATT

R1-1803835	Remaining issues PUCCH resource allocation	vivo

R1-1804015	Remaining issues on resource allocation for PUCCH	OPPO

R1-1804115	PUCCH resource allocation prior to RRC configuration	NEC

R1-1804220	Remaining issues on resource allocation for PUCCH	Spreadtrum Communications

R1-1804376	Corrections on Resource Allocation for PUCCH	Samsung

R1-1804483	Discussion on remaining issues on PUCCH resource allocation	Panasonic Corporation

R1-1804557	Remaining issues on PUCCH resource allocation	LG Electronics

R1-1804657	Remianing issues of multi-slot PUCCH	AT&T

R1-1804731	Resource allocation before RRC setup	Intel Corporation

R1-1804805	Remaining issues for resource allocation for PUCCH	Qualcomm Incorporated

R1-1804956	Remaining details on PUCCH resource allocation	Motorola Mobility, Lenovo

R1-1805054	Resource allocation for PUCCH	NTT DOCOMO, INC.

R1-1805186	On PUCCH Resource Allocation	Ericsson

R1-1805509	Remaining open items on PUCCH resource allocation	Nokia, Nokia Shanghai Bell	(revision of R1-1804461)

##### Other

R1-1804287	TP on UCI multiplexing between UL and SUL	Huawei, HiSilicon

R1-1804427	Discussion on partial overlap between PUCCHs	Huawei, HiSilicon

R1-1804432	Discussion and text proposal for HARQ-ACK on PUCCH format 0	Huawei, HiSilicon

R1-1804434	Discussion on aperiodic CSI feedback on PUCCH in NR	Huawei, HiSilicon

R1-1804451	Correction on PUCCH multiplexing in 38.213	Huawei, HiSilicon

R1-1805187	TP on UCI Multiplexing on PUSCH	Ericsson

R1-1805732	On CGS ordering for NR	Huawei, HiSilicon, ZTE, Sanechips	(revision of R1-1804433)

#### DL/UL data scheduling and HARQ procedure

##### DL/UL resource allocation

Including handling LS on formula or table for L1 data rate as in R1-1801324

R1-1805602	Outcome of offline discussion on 7.1.3.3.1	Ericsson

Decision: The document is noted.

Agreements:

- (Working assumption) adopt the tables of R1-1805602 for RMSI time domain resource allocation

  - The previously agreed 4-bit indicator in DCI is not changed

  - Note:

    - Some entries in the tables are to be merged by using the known DM-RS position and/or CORESET duration/location signaled in PBCH, or removed

    - New entries may also be added

    - However, no more than 16 entries in total for pattern 1

  - FFS: Need for “gaps” in time domain allocation, K0 offset, or rate matching information to handle collisions with other SSBs.

  - FFS the applicability to OSI/Paging/RAR/Msg4

Wednesday

R1-1805663	Summary of offline discussion on 7.1.3.3.1 part II	Ericsson

Decision: The document is noted.

Above agreements are modified as follows:

Agreements:

- Adopt the tables below for RMSI time domain resource allocation

  - The previously agreed 4-bit indicator in DCI is not changed

Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 1

| L | DL-DMRS-typeA-pos | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | Type A | 0 | 2 | 12 |
|  | 3 | Type A | 0 | 3 | 11 |
| 1 | 2 | Type A | 0 | 2 | 10 |
|  | 3 | Type A | 0 | 3 | 9 |
| 2 | 2 | Type A | 0 | 2 | 9 |
|  | 3 | Type A | 0 | 3 | 8 |
| 3 | 2 | Type A | 0 | 2 | 7 |
|  | 3 | Type A | 0 | 3 | 6 |
| 4 | 2 | Type A | 0 | 2 | 5 |
|  | 3 | Type A | 0 | 3 | 4 |
| 5 | 2 | Type B | 0 | 9 | 4 |
|  | 3 | Type B | 0 | 10 | 4 |
| 6 | 2 | Type B | 0 | 4 | 4 |
|  | 3 | Type B | 0 | 6 | 4 |
| 7 | x | Type B | 0 | 5 | 7 |
| 8 | x | Type B | 0 | 5 | 2 |
| 9 | x | Type B | 0 | 9 | 2 |
| 10 | x | Type B | 0 | 12 | 2 |
| 11 | x | Type A | 0 | 1 | 13 |
| 12 | x | Type A | 0 | 1 | 6 |
| 13 | x | Type A | 0 | 2 | 4 |
| 14 | x | Type B | 0 | 4 | 7 |
| 15 | x | Type B | 0 | 8 | 4 |

Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 2

| I | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type B | 0 | 2 | 2 |
| 1 | Type B | 0 | 4 | 2 |
| 2 | Type B | 0 | 6 | 2 |
| 3 | Type B | 0 | 8 | 2 |
| 4 | Type B | 0 | 10 | 2 |
| 5 | Type B | 1 | 2 | 2 |
| 6 | Type B | 1 | 4 | 2 |

Note: From a table size perspective, it is possible to add 7 more rows with same K0 and S as above but with L=4 as well as 2 more rows S=0 and L=4.

Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 3

| i | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type B | 0 | 4 | 2 |
| 1 | Type B | 0 | 6 | 2 |
| 2 | Type B | 0 | 8 | 2 |
| 3 | Type B | 0 | 10 | 2 |

Note: From a table size perspective, additional entries are possible.

Proposals:

- If SIB1 configures a table with values for time-domain allocation, use this table for

    - at least for CSS configured by MIB/RMSI: RA-RNTI and TC-RNTI (msg4)

    - FFS: OSI, P-RNTI

- If SIB1 does not provide a table, use the RMSI table above.

    - FFS: which pattern (Same pattern as used for SIB1? Always pattern 1? New pattern? …)

Thursday

R1-1805702	Summary of offline discussion on 7.1.3.3.1 (resource allocation) – part III	Ericsson

Decision: The document is noted.

Working assumption:

- For PUSCH tx, 4-bit to indicate the resource allocation, as shown in Table below

Table 6.1.2.1-x: Default resource allocation for PUSCH

| i | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type A | j | 0 | 14 |
| 1 | Type A | j | 0 | 12 |
| 2 | Type A | j | 0 | 10 |
| 3 | Type B | j | 2 | 10 |
| 4 | Type B | j | 4 | 10 |
| 5 | Type B | j | 4 | 8 |
| 6 | Type B | j | 4 | 6 |
| 7 | Type A | j+1 | 0 | 14 |
| 8 | Type A | j+1 | 0 | 12 |
| 9 | Type A | j+1 | 0 | 10 |
| 10 | Type B | j+2 | 0 | 14 |
| 11 | Type A | j+2 | 0 | 12 |
| 12 | Type A | j+2 | 0 | 10 |
| 13 | Type B | j | 8 | 6 |
| 14 | Type A | j +3 | 0 | 14 |
| 15 | Type A | j +3 | 0 | 10 |

Table 6.1.2.1-x: Definition of value j in determination for K2

| µ | j for msg 3 initial tx | j for other cases |
| --- | --- | --- |
| 0 | 3 | 1 |
| 1 | 3 | 1 |
| 2 | 6 | 2 |
| 3 | 9 | 3 |

Agreements:

For time-domain signaling for PUSCH scheduled by DCI in USS

- if dedicated RRC signaling has provided a table

  - Use this table for PUSCH transmission scheduled by DCI in USS

- else if RMSI signaling has provided a table

  - Use this table for PUSCH transmission scheduled by DCI in USS

- else

  - Use the default PUSCH table

For time-domain signaling for PDSCH scheduled by DCI in USS

- if dedicated RRC signaling has provided a table

  - Use this table for PDSCH transmission scheduled by DCI in USS

- else if RMSI signaling has provided a table

  - Use this table for PDSCH transmission scheduled by DCI in USS

- else

  - Use pattern 1 from the agreed default table

There is a WF in the inbox (5641). Interested companies are encouraged to check the WF and discuss to try to come up with a common proposal.

Friday

R1-1805641	Way Forward on NR Peak Data Rate Formula	Intel, Samsung, MediaTek, Huawei, HiSilicon, Apple, Vivo, OPPO

Decision: The document is noted for email discussion/approval including the related RAN2 LS(s) till 4/27 – Daewon (Intel)

R1-1803709	Remaining issues on resource allocation and TBS	Huawei, HiSilicon

R1-1803757	Remaining issues on PDSCH/PUSCH resource allocation	CATT

R1-1803796	Remaining issues for data resource allocation	ZTE, Sanechips

R1-1803836	Remaining issues on resource allocation	vivo

R1-1804016	Remaining issues on multi-slot resource allocation	OPPO

R1-1804082	Remaining Issues on DL/UL Resource Allocation	MediaTek Inc.

R1-1804221	Remaining issues on time-domain scheduling	Spreadtrum Communications

R1-1804377	Corrections on DL/UL Resource Allocation	Samsung

R1-1804558	Remaining issues on Resource allocation	LG Electronics

R1-1804650	Further details of PDSCH time-domain resource allocation before RRC connection setup	AT&T

R1-1804732	On peak rates and remaining details on DL/UL RA	Intel Corporation

R1-1804806	Remaining Issues on DL/UL resource allocation	Qualcomm Incorporated

R1-1805055	Resource allocation and TBS determination	NTT DOCOMO, INC.

R1-1805142	Open issues on time domain resource allocation	Nokia, Nokia Shanghai Bell

R1-1805266	Discussion on remaining issues for time domain resource allocation	CMCC	(revision of R1-1804101)

R1-1805505	Summary of 7.1.3.3.1 (resource allocation)	Ericsson

R1-1805506	On DL/UL resource allocation	Ericsson	(revision of R1-1805188)

R1-1805607	Discussion on Approximate Peak Data Rate and Scaling Factor Use Cases	Intel

R1-1805618	Default PDSCH table	Ericsson

R1-1805721	Way Forward on LTE Peak Data Rate Calculation for EN-DC	Huawei, HiSilicon, Intel

R1-1805743	Summary of EN-DC LTE and NR data rate	Intel

##### DL/UL scheduling and HARQ management

R1-1805540	Summary of DL/UL scheduling and HARQ management	Qualcomm

Decision: The document is noted.

Agreements:

The following text proposal is adopted to clarify that N1 processing time at least follows the semi-static configuration for DMRS, and not that resulting from the allocation.

------ Text Proposal for TS 38.214 Subclause 5.3 ------

-	N1 is based on µ of table 5.3-1 that corresponds to the min (µDL, µUL) where the µDL corresponds to the subcarrier spacing of the downlink with which the PDSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted. If the higher layer parameter dmrs-AdditionalPosition is configured with a value greater than 0, or if the high layer parameter is not configured, then N1  value follows from the second column of table 5.3.1; otherwise, the N1 value follows from the first column of table 5.3-1.

Agreements:

The following text proposal is adopted to clarify the start position of the physical channel carrying HARQ-ACK.

------ Text Proposal for TS 38.214 Subclause 5.3 ------

If the first uplink symbol of the physical channel which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1 then the UE shall provide a valid HARQ-ACK message, where L1 is defined as the next uplink symbol with its CP starting after  after the end of the last symbol of the PDSCH carrying the TB being acknowledged.

<text omitted which is unchanged>

-	If the PDSCH is mapping type A as given in subclause 7.4.1.1 of [4, TS 38.211], and the last symbol of PDSCH is on the ith symbol of the slot where i<7, then , otherwise d1,3 = 0

------ End of Text Proposal ------

Agreement:

- Confirm the following working assumption:

  - For broadcast PDSCH, MCS is limited to QPSK, rank is limited to 1.

Agreements:

- Adopt the corresponding text proposals:

----- Text proposals for 38.214, 5.1.1.1 -----

For transmission scheme 1 of the PDSCH, the UE may assume that a gNB transmission on the PDSCH would be performed with up to 8 transmission layers on antenna ports 1000-1011 as defined in Subclause 7.3.1.4 of [4, TS 38.211], subject to the DMRS reception procedures in Subclause 5.1.6.2.

<unchanged text omitted>

The UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel scheduled with other RNTIs than C-RNTI. The UE is not expected to decode a PDSCH scheduled with P-RNTI, RA-RNTI, or SI-RNTI and (Qm) > 2.

----- End of Text Proposal -----

Agreement:

- Confirm the following working assumption:

  - P-RNTI PDSCH is not required to be decoded by UE in RRC_CONNECTED mode

    - Note: P-RNTI PDCCH may not schedule a P-RNTI PDSCH but rather carries the message by itself, which may be used for e.g. systemInfoModification, cmas-Indication, and etws-Indication

Working assumption:

- For baseline (Capability 1) UE processing times, for PDSCH mapping type B with 2-symbol duration, the minimum UE processing times are defined as the following where N1 is the corresponding value from Table 5.3-1 of TS 38.214:

  - N1 + 3 + d symbols when there is a time-domain overlap of ‘d’ symbols (d = {0,1,2}) between the scheduling PDCCH and the scheduled PDSCH

Thursday

R1-1805686	Simultaneous Decoding of System Information	Qualcomm

Decision: The document is noted.

Agreements:

The previous WA is confirmed with updates:

- While UE acquires SI upon being triggered by Paging DCI

  - UE is not required to decode C-RNTI PDSCH if the SI-RNTI PDSCH is overlapped with at least one symbol

- In case UE autonomously monitors SI-RNTI PDCCH while monitoring C-RNTI PDCCH, and both SI-RNTI PDSCH and C-RNTI PDSCH are overlapped with at least one symbol, the UE is not required to decode SI-RNTI PDSCH

- (Working assumption)The first two bullets apply unless TBS of SI-RNTI PDSCH ≤ 2976 for FR1, then UE decodes both SI-RNTI PDSCH and C-RNTI PDSCH

- The first two bullets always apply in FR2

R1-1805687	Baseline Capability #1 Remaining Issues	Qualcomm

Decision: The document is noted.

Conclusion:

Consider whether further handling on UE processing time may be needed in the following cases

- L1 is the length (in symbols) of a first PDSCH allocation

  - L1-dmrs is location of last DMRS in the allocation

- L2 is the length (in symbols) of a second PDSCH allocation

  - L2-dmrs is location of last DMRS in the allocation

- g is the delay (in symbols) between the first and second PDSCH allocation

- In case where L1 – L2 – g > 0

- The DMRS locations between the first and second PDSCH allocation are different

R1-1805688	Aggressive Capability #2 Processing Time	Qualcomm

Decision: The document is noted.

Conclusion:

At least for the following conditions below, the processing times listed in Table 2 have been considered as potential candidates for specification of Capability #2, although there has been no conclusion on the final value.

- Single numerology for PDCCH, PDSCH, and PUSCH and no UCI multiplexing

- Case 1-1: PDCCH monitoring on up to three OFDM symbols at the beginning of a slot

- PDSCH allocation with at least 7 symbols is supported

- One unicast PDSCH received and/or one unicast PUSCH transmitted within the same slot

  - FFS: More than one PDSCH and/or PUSCH case

- Single CC

  - FFS: CA case with and without cross-carrier scheduling

- For C-RNTI only

  - FFS: also applicable to the cases when C-RNTI and with other broadcast RNTIs are processed simultaneously by the UE

- FFS: whether Capability #2 relaxation is applied dynamically depending on conditions

- Note: Companies are also encouraged to provide processing times for 60kHz (FR1)

- Note: Companies are also encouraged to provide conditions under which more aggressive processing times (within the range) could be enabled

Table 2. UE Processing Times for Capability #2

| Configuration | HARQ Timing Parameter | Units | 15 KHz SCS | 30 KHz SCS |
| --- | --- | --- | --- | --- |
| Front-loaded DMRS only | N1 | Symbols | [2.5-4] | [2.5-6] |
| Frequency-first RE-mapping | N21 | Symbols | [2.5-6] | [2.5-6] |
| 1If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2 in table. |  |  |  |  |

Friday

R1-1805746	Summary of DL/UL scheduling and HARQ management	Qualcomm

Decision: The document is noted.

Conclusion:

The existing agreements were made before RAN2 defined SUL and non-SUL as a single serving cell

- One TB is mapped to one DL/UL carrier.

- Re-transmission of a TB cannot take place on different carrier than the initial transmission.

So far this implies for some companies that for SUL, the re-transmission of a TB should not take place on a different carrier than the initial transmission. Other companies believe re-transmission may take place on a different carrier in case of SUL and UL.

- If intention for previous agreement was to be worded “per cell” then it would be restricted to CA

- RAN1 should discuss whether to clarify the existing agreements

- Note: in SUL there is only one cell and one HARQ entity

Working assumption:

- The processing requirement for HARQ-ACK in response to DL SPS release is N OFDM symbols from last symbol of the corresponding PDCCH for UE Capability #1:

  - 15 kHz case: 8 + 5

  - 30 kHz case: 10 + 5

  - 60 kHz case: 17 + 5

  - 120 kHz case: 20 + 5

Proposals:

- UE behavior in RRC_IDLE: If any two PDSCH among SI-RNTI PDSCH, P-RNTI PDSCH, and RA-RNTI/TC-RNTI PDSCH are overlapped with at least one symbol for a given UE from the primary cell, the UE is not required to decode both of PDSCH simultaneously

  - FFS: the prioritization of the PDSCH to be decoded

Discuss till next meeting

R1-1805747	TP to 38.214 on UE processing time definitions	Qualcomm

Decision: The proposal in R1-1805747 is agreed.

R1-1803758	Remaining issues on NR HARQ timing	CATT

R1-1803837	Remaining issues on HARQ management	vivo

R1-1804068	Remaining issues of DL/UL scheduling and HARQ management	MediaTek Inc.

R1-1804102	Discussion on HARQ timing for DL SPS	CMCC

R1-1804207	Remaining issues for HARQ-ACK feedback timing indication	Lenovo, Motorola Mobility

R1-1804312	CR on semi-static HARQ-ACK codebook fallback operation in 38.213	Huawei, HiSilicon

R1-1804378	Corrections on HARQ Management and Feedback	Samsung

R1-1804431	Remaining issues on scheduling and HARQ	Huawei, HiSilicon

R1-1804559	Remaining issues on DL/UL scheduling and HARQ	LG Electronics

R1-1804733	Some remaining details on DL/UL scheduling and HARQ management	Intel Corporation

R1-1804807	Remaining Issues on DL/UL Scheduling, Processing Time and HARQ management	Qualcomm Incorporated

R1-1805056	PUSCH time-domain resource allocation	NTT DOCOMO, INC.

R1-1805081	Discussions on HARQ-ACK bundling and multiplexing	China Telecommunications

Late submission

R1-1805189	On remaining issues of scheduling and HARQ management	Ericsson

R1-1805249	TP to 38.214 on UE processing time definitions	Nokia, Nokia Shanghai Bell

##### CBG-based (re)transmission

R1-1805570	Summary of remaining issues on CBG based retransmission	LG Electronics

Decision: The document is noted.

For the first two issues identified in R1-1803759 (CATT), prepare a TP – revisit later

Notes: # CBGs is configured per TB

For the third issue identified in R1-1803759 (CATT), check further offline

Proposal:

- To adopt the following TP (Section 9.1.1, 38.213)

---Start of TP---

If a UE is configured with higher layer parameter HARQ-ACK-codebook=semi-static, tThe HARQ-ACK codebook includes the  HARQ-ACK information bits and, if  for a transport block, the UE generates a NACK value for the last  HARQ-ACK information bits for the transport block in the HARQ-ACK codebook.

---End of TP---

R1-1803759	Corrections to CBG-based transmission	CATT

Friday

R1-1805731	Summary of text proposals on CBG based retransmission	LG Electronics

Decision: The document is noted. The four TPs in R1-1805731 are endorsed in principle, up to editor to refine the wording where necessary

R1-1804379	Corrections on CBG-Based UL/DL Retransmissions	Samsung

R1-1804430	Remaining issues on CBG-based (re)transmission	Huawei, HiSilicon

R1-1805190	On remaining issues for CBG based HARQ	Ericsson

##### UL data transmission procedure

R1-1805539	Offline summary for AI 7.1.3.3.4 UL data transmission procedure	NTT DOCOMO

Decision: The document is noted.

Agreement:

- Adopt following TP for 38.213 subclause 8.3 Msg3 PUSCH:

---Start of TP---

If the UE applies transform precoding to an Msg3 PUSCH transmission with frequency hopping, the frequency offset for the second hop [6, TS38.214] is given in Table 8.3-1.

| Number of PRBs in initial active UL BWP | Value of Hopping Bits | Frequency offset for 2nd hop |
| --- | --- | --- |
|  | 0 | floor() |
|  | 1 | floor() |
|  | 00 | floor() |
|  | 01 | floor() |
|  | 10 | －floor() |
|  | 11 | Reserved |

---End of TP---

Agreement:

- To adopt the following TP for Section 6.3 of 38.214

---Start of TP---

In case of intra-slot frequency hopping is configured for PUSCH without repetitions, the number of symbols in the first hop is given by , the number of symbols in the second hop is given by , where NsymbolPUSCH,s/2 NsymbolPUSCH,s is the length of the PUSCH transmission in OFDM symbols in one slot.

---End of TP---

Conclusion:

- Confirmation of the activation/deactivation signalling for SP-CSI on PUSCH is not explicitly supported in the specs

  - Up to gNB implementation

Wednesday

R1-1805646	Repetition for PUSCH mapping Type B	NTT DOCOMO, INC.

Agreement:

- For Rel.15 Dec. drop, for both Type 1 and Type 2 PUSCH transmissions with a configured grant, when a UE is configured with data mapping Type B and K> 1, the same symbol allocation is applied across the K consecutive slots.

R1-1803797	Remaining issues for UL data transmission procedure	ZTE, Sanechips

R1-1803838	Remaining issues on UL data transmission procedure	vivo

R1-1803900	Underlay SR: a complementary solution to overcome the limitations of periodic PUCCH-SR	Idaho National Laboratory

R1-1804013	Remaining issues on grant free	OPPO

R1-1804103	Discussion on PUSCH repetition for grant-free transmission	CMCC

R1-1804228	correction of PUSCH frequency hopping	Spreadtrum Communications

R1-1804295	Summary of remaining issues on UL data transmission procedure	Huawei, HiSilicon

R1-1804560	Remaining issues on UL data transmission procedure	LG Electronics

R1-1804734	Remaining issues of UL data transmission procedures	Intel Corporation

R1-1805057	UL data transmission procedure	NTT DOCOMO, INC.

R1-1805507	On UL data transmission procedure	Ericsson	(Revision of R1-1805191)

##### Soft-buffer management for NR

R1-1803938	Remaining Issues of Soft Buffer Management	Ericsson

R1-1805664	Summary of offline for 7.1.3.3.5	Ericsson

Decision: The document is noted.

Agreements:

- Adopt the following TP regarding the number of layers in LBRM operation.

----Start of TP ---

5.4.2.1	Bit selection

The bit sequence after encoding  from Subclause 5.3.2 is written into a circular buffer of length  for the -th coded block, where  is defined in Subclause 5.3.2.

For the -th code block, let  if  and  otherwise, where, ,  is determined according to Subclause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Subclause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following:

-	maximum number of layers for one TB supported by the UE for the serving cell;

-	maximum modulation order configured for the serving cell, if configured by higher layers; otherwise a maximum modulation order  is assumed for DL-SCH ;

---End of TP---

Conclusion:

- It is RAN1 understanding that UE is not expected to handle cases where the Nref value (in 5.4.2.1) changes across transmissions for the same transport block.

  - No spec impact.

##### Multiplexing data with different transmission durations

R1-1805529	Summary of Multiplexing data with different transmission durations	vivo

Decision: The document is noted.

Agreement:

- Following change is adopted for TS 38.213 section.

------------------------------------------------start of text proposal------------------------------------------------

If a UE detects a DCI format 2_1 in a PDCCH transmitted in a control resource set in a slot, the set of symbols indicated by a field in DCI format 2_1 includes the last   symbols prior to the first symbol of the control resource set in the slot  where  is the value of higher layer parameter Monitoring-periodicity-PDCCH-slot,  is the number of symbols per slot, is the subcarrier spacing configuration for a serving cell with mapping to a respective field in the DCI format 2_1,  is the subcarrier spacing configuration of the DL BWP where the UE receives the PDCCH conveying the DCI format 2_1. If the UE is configured with higher layer parameters UL-DL-configuration-common or UL-DL-configuration-common-Set2, symbols indicated as uplink by UL-DL-configuration-common or UL-DL-configuration-common-Set2 are excluded from the last  symbols prior to the first symbol of the control resource set in the slot. The resulting set of symbols includes a number of symbols that is denoted as.

-------------------------------------------end of text proposal---------------------------------------------------------

R1-1803662	Summary of remaining issues on pre-emption indication with TP	Huawei, HiSilicon

R1-1803760	Remaining details of DL pre-emption indication	CATT

R1-1803839	Remaining issues on Multiplexing data with different transmission durations	vivo

R1-1804083	Remaining issues on pre-emption indication	MediaTek Inc.

R1-1804086	Remaining issues for preemption indication	ASUSTEK COMPUTER (SHANGHAI)

R1-1804808	Remaining issues on downlink preemption indication	Qualcomm Incorporated

R1-1805192	Multiplexing data with different durations	Ericsson

##### Other

R1-1804280	TP on HARQ-ACK reporting with different numerology	Huawei, HiSilicon

R1-1804292	Support of 60 kHz subcarrier spacing	Huawei, HiSilicon

R1-1804307	Draft CR on resource allocation	Huawei, HiSilicon

R1-1804308	Draft CR on TBS	Huawei, HiSilicon

R1-1804309	Draft CR on HARQ-ACK codebook	Huawei, HiSilicon

R1-1804311	Correction on scheduling and HARQ procedure for case 1 HARQ timing of EN-DC in TS36.213	Huawei, HiSilicon

R1-1804452	Draft CR on UL data transmission procedure	Huawei, HiSilicon

R1-1804771	Discussion on CBG based retransmissions	Apple Inc.

R1-1804952	Inconsistent PDCCH detection	Panasonic

R1-1805193	[DRAFT] Reply LS on formula or table for L1 data rate	Ericsson

R1-1805194	TP on RAR grant resource allocation	Ericsson

R1-1805195	TP on default resource allocation table and MSG3 table	Ericsson

R1-1805196	TP for remaining issues of scheduling and HARQ management	Ericsson

R1-1805197	TP for DL and UL semi-persistent scheduling	Ericsson

R1-1805198	[DRAFT] Reply LS on LTE peak data-rate calculation for EN-DC	Ericsson

#### Other aspects on carrier aggregation and bandwidth parts

##### Other aspects on bandwidth Parts

R1-1805543	Summary of bandwidth part remaining issues	MediaTek

Decision: The document is noted.

Conclusion:

- Regarding the discrepancy in RAN1 and RAN2 agreements w.r.t. initial active BWP vs. the up to configured 4 BWPs, to adopt the RAN2 agreements and make necessary RAN1 spec update

Wednesday

R1-1805658	Summary of bandwidth part remaining issues	MediaTek

Decision: The document is noted.

Agreements:

- Regarding the discrepancy in RAN1 and RAN2 agreements w.r.t. initial active BWP vs. the up to configured 4 BWPs, to adopt the RAN2 agreements and make necessary RAN1 spec update

- The corresponding TP is shown below:

---Start of Text Proposal Sec 7.3.1.1.2 of 38.212 ---

< Omitted >

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL bandwidth parts configured by higher layersdefined in Table 7.3.1.1.2-1. The bitwidth for this field is determined as bits, where

-	 and the bandwidth part indicator is equivalent to the higher layer parameter BWP-Id if  the higher layer parameter BandwidthPart-Config configures up to 3 bandwidth parts and the initial bandwidth part is not included in higher layer parameter BandwidthPart-Config;

-	otherwise  and the bandwidth part indicator is defined in Table 7.3.1.1.2-1.;

-	 is the number of configured bandwidth parts according to higher layer parameter BandwidthPart-Config.

< Omitted >

Table 7.3.1.1.2-1: Bandwidth part indicator

| Value of BWP indicator field |  | Bandwidth part |
| --- | --- | --- |
| 1 bit | 2 bits |  |
| 0 | 00 | First bandwidth part configured by higher layers |
| 1 | 01 | Second bandwidth part configured by higher layers |
|  | 10 | Third bandwidth part configured by higher layers |
|  | 11 | Fourth bandwidth part configured by higher layers |

---End of Text Proposal Sec 7.3.1.1.2 of 38.212 ---

---Start of Text Proposal Sec 7.3.1.2.2 of 38.212 ---

< Omitted >

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of DL bandwidth parts  configured by higher layersdefined in Table 7.3.1.1.2-1. The bitwidth for this field is determined as bits, where

-	 and the bandwidth part indicator is equivalent to the higher layer parameter BWP-Id if  the higher layer parameter BandwidthPart-Config configures up to 3 bandwidth parts and the initial bandwidth part is not included in higher layer parameter BandwidthPart-Config;

-	otherwise  and the bandwidth part indicator is defined in Table 7.3.1.1.2-1.;

-	 is the number of configured bandwidth parts according to higher layer parameter BandwidthPart-Config.

< Omitted >

---End of Text Proposal Sec 7.3.1.2.2 of 38.212 ---

Agreements:

- To adopt the following TP:

---Start of Text Proposal Sec 12 of 38.213 ---

< Omitted >

For each DL BWP or UL BWP in a set of DL BWPs or UL BWPs, respectively, the UE is configured the following parameters for the serving cell as defined in [4, TS 38.211] or [6, TS 38.214]:

- a subcarrier spacing provided by higher layer parameter DL-BWP-mu or UL-BWP-mu;

- a cyclic prefix provided by higher layer parameter DL-BWP-CP or UL-BWP-CP;

- a PRB offset with respect to the PRB determined by higher layer parameters offset-pointA-low-scs and ref-scs and a number of contiguous PRBs provided by higher layer parameter DL-BWP-BW or UL-BWP-BW;

- a starting PRB and a number of contiguous PRBs indicated by higher layer parameter locationAndBandwidth which is interpreted as RIV according to [5.1.2.2.2, TS 38.213], setting =275 and the starting PRB is a PRB offset with respect to the PRB indicated by higher layer parameters offsetToCarrier and subcarrierSpacing.

- an index in the set of DL BWPs or UL BWPs by respective higher layer parameters DL-BWP-index or UL-BWP-index;

- DCI format 1_0 or DCI format 1_1 detection to a PDSCH reception timing values by higher layer parameter DL-data-time-domain, PDSCH reception to a HARQ-ACK transmission timing values by higher layer parameter DL-data-DL-acknowledgement, and DCI format 0_0 or DCI format 0_1 detection to a PUSCH transmission timing values by higher layer parameter UL-data-time-domain;

< Omitted >

---End of Text Proposal Sec 12 of 38.213 ---

Thursday

R1-1805693	Summary of bandwidth part remaining issues	MediaTek

Decision: The document is noted.

R1-1805662	[Draft] Reply LS on SI Reception in BWP	LGE, MediaTek

Decision: The document is endorsed and final LS is approved (accepting all change marks) in R1-1805726

Agreements (RAN2 impact)

- The existing higher-layer parameter SCS-SpecificCarrier provided to a UE via a dedicated RRC signal is also applicable to Scell & SUL.

Agreements:

- It’s RAN1’s understanding that the active DL (or UL) BWP switching delay (i.e. T μs) for two types of UE capability will be specified in RAN4 specs.

  - Send LS to RAN4 – R1-1805727

- For DCI-based active DL (or UL) BWP switching, a UE is not expected to switch its active DL (or UL) BWP based on an active DL (or UL) BWP switching DCI which is considered as  invalid where invalidity is defined as when the signaled K0 (or K2) value in the DCI can’t guarantee the active DL (or UL) BWP switching delay equal to or larger than the one specified in RAN4 specs to the UE based on the UE’s capability.

Friday

R1-1805727	[Draft] LS on active BWP switching delay specification	MediaTek

Decision: The document is endorsed and final LS is approved in R1-1805755

R1-1805750	Summary of bandwidth part remaining issues	MediaTek

Decision: The document is noted.

Agreements:

Confirm the following working assumption with updates:

- Sizes of all DCI bitfields in DCI formats 0-1 and 1-1 in USS determined by current BWP. Data transmitted on the BWP indicated by the BWP index. If the BWP index activates another BWP, transform as follows:

  - Zero-pad too small bitfields to match the new BWP

  - Truncate too large bitfields to match the new BWP

  - The truncation is done from MSB (including the bit indicating the resource allocation type)

  - Zero-padding is done for MSB

R1-1803653	Remaining issues on bandwidth part and wideband operation	Huawei, HiSilicon

R1-1803761	Corrections to BWP operation	CATT

R1-1803798	Issues for scell BWP	ZTE, Sanechips

R1-1803840	Remaining issues on BWP operation	vivo

R1-1803966	Discussion on bandwidth part operation	ITRI

R1-1804017	Other aspects on bandwidth parts	OPPO

R1-1804049	Remaining details on bandwidth part operation	MediaTek Inc.

R1-1804226	Remaining issues on BWP	Spreadtrum Communications

R1-1804380	Corrections on Bandwidth Part Operation	Samsung

R1-1804502	Remaining details on common PRB indexing and RRC parameters	NEC Corporation

R1-1804561	Remaining issues on bandwidth part operation	LG Electronics

R1-1804674	Remaining issues on dynamic BWP switching	InterDigital, Inc.

R1-1804762	On remaining details of BWPs	Nokia, Nokia Shanghai Bell

R1-1804809	Remaining issues on BWP	Qualcomm Incorporated

R1-1805058	Other aspects on bandwidth Parts	NTT DOCOMO, INC.

R1-1805199	On BWP adaptation	Ericsson

R1-1805244	Remaining issues on BWP switching	WILUS Inc.

##### Other aspects on carrier aggregation

Including up to two different numerologies per PUCCH group

R1-1805277	Summary on CA Aspects	Samsung

Decision: The document is noted.

Conclusion:

- It is confirmed that when a UE is configured with semi-static HARQ-ACK codebook, the HARQ association set is determined based on the semi-static (via RMSI/UE-specific) UL/DL configurations for PDSCH occasions in addition to HARQ-ACK in response to SPS release

  - This is already reflected in the current specs

Agreements:

- Confirm WA from RAN1#92 with the following update

  - When a UE is configured with semi-static HARQ-ACK Codebook, on a per cell basis:

    - If the UE indicates capability to receive more than one unicast PDSCH per slot, it assumes a max number of non-overlapping candidate unicast PDSCH occasions per slot as determined by the SLIV in the configured pdsch-symbolAllocation table.

    - Otherwise, the UE is expected to receive only one unicast PDSCH per slot, and HARQ-ACK association set assumes one unicast PDSCH per slot

      - The UE is not expected to receive SPS PDSCH release and unicast PDSCH in a same slot

  - Handling HARQ-ACK for PDCCH for SPS release follows the same way as in LTE

Thursday

R1-1805719	Offline summary on CA aspects	Samsung

Decision: The document is noted.

Agreement:

- For unpaired spectrum and DL/UL BWP switching

  - UE does not transmit HARQ-ACK for PDSCH reception(s) on the DL BWP prior to DL/UL BWP switching

R1-1803655	Remaining issues on NR CA	Huawei, HiSilicon

R1-1803762	Remaining issues on NR CA operation	CATT

R1-1803841	Other aspects on carrier aggregation	vivo

R1-1804008	Remaining details on HARQ-ACK transmission	OPPO

R1-1804034	Discussion about semi static codebook	ASUSTEK COMPUTER (SHANGHAI)

R1-1804046	Remaining issues in carrier aggregation	MediaTek Inc.

R1-1804208	Remaining issues for CA with different numerologies	Lenovo, Motorola Mobility

R1-1804227	Remaining issues on HARQ-ACK codebook	Spreadtrum Communications

R1-1804381	Corrections on CA Operation	Samsung

R1-1804562	Remaining issues on CA and HARQ-ACK codebook	LG Electronics

R1-1804735	Further discussion UCI mapping for carrier aggregation	Intel Corporation

R1-1804763	Remaining aspects of NR CA	Nokia, Nokia Shanghai Bell

R1-1804810	Remaining issues on CA	Qualcomm Incorporated

R1-1805059	Remaining issues on HARQ-ACK codebook and CA	NTT DOCOMO, INC.

R1-1805200	On remaining issues of carrier aggregation	Ericsson

R1-1805245	Remaining issues on HARQ-ACK codebook construction	WILUS Inc.

#### Rate matching aspects for NR DL and UL

R1-1805518	Summary of open issues related to rate-matching in NR	Nokia, Nokia Shanghai Bell

Decision: The document is noted.

Proposals:

- Clarify in the TS 38.214 that

  - a PDSCH conveying [RAR, OSI, Paging, Msg4] is rate-matched only around SSB-transmitted-SIB1 and the PDCCH that scheduled the PDSCH.

  - a PDSCH conveying [SIB1] is rate-matched only around the PDCCH that scheduled the PDSCH

- To check offline (also with initial access experts/DL control concepts e.g. w.r.t. wideband RS), to prepare TP if necessary

Proposals:

For resources of a bitmap pair configured with rate-match-PDSCH-bitmap3 and of a CORESET included in one or two groups of resource sets, the dynamic indication of availability for PDSCH applies

- Alt 1: to a set of slot(s) where the bitmap pair/CORESET actually exists among the slots of PDSCH scheduled.

- Alt 2: to the first slot among the slots of PDSCH scheduled (if bitmap pair / CORESET is present in that slot).

Agreements:

- When UE is to rate-match a PDSCH around PDCCH scheduling the PDSCH, and PDCCH is received in the CORESET configured with WB RS, the UE shall rate-match around

  - The union of REs of the PDCCH scheduling the PDSCH and the WB RS REs for the configured CORESET

Proposals:

- Update the agreement from the last meeting:

  - When non-interleaved CORESET which is 1 symbol long and AL8 and AL16 PDCCH candidates have the same starting CCE, when the UE successfully decodes a PDCCH with AL8 or AL16 satisfying the ambiguity condition, the UE performs rate-matching for PDSCH by assuming AL 16.

    - Note: the actual spec text wording can be refined

    - Applies to at least non-interleaved CORESET which is 1 symbol long

    - Decide on other scenarios by the end of RAN1#92bis

Agreement:

- To adopt the following TP to 5.1.4 of 38.214

When receiving PDSCH not conveying [RAR, OSI, Paging, Msg4, SIB1], Tthe REs corresponding to the union of configured or dynamically indicated resources in Subclauses 5.1.4.1, 5.1.4.2 and resources corresponding to SS/PBCH are declared as not available for PDSCH in Subclause 7.3.1.5 of [4, TS 38.211].

Agreements:

- When an PDCCH candidate collides with any RB-level resource (FFS RE-level), where PDSCH is semi-statically not mapped, as described in TS38.214 Section 5.1.4.1 then

  - The UE is not required to monitor the PDCCH candidate overlapping with the resource even only by one RE

Friday

Agreements: Clarify in the specification that the periodicity of bitmap-pair, when bitmap-3 is not configured, is equal to the length of the bitmap-2.

- For bitmap-pair not included in one or two groups of resource sets (semi-static RMR) and associated with two-slot long bitmap-2, first slot of bitmap-2 corresponds to even slots, second slot corresponds to odd slots.

- For bitmap-pair not included in one or two groups of resource sets (semi-static RMR) and associated with one slot long bitmap-2, corresponds to every slot.

Agreements:

- For resources of a bitmap-pair configured with bitmap-3 and of a CORESET included in one or two groups of resource sets (dynamic RMR), the dynamic indication of availability for PDSCH applies to a set of slot(s) where the bitmap-pair/CORESET actually exists among the slots of PDSCH scheduled.

R1-1803654	Remaining issues on reserved resources and rate-matching	Huawei, HiSilicon

R1-1803799	Remaining aspects for rate matching	ZTE, Sanechips

R1-1804215	On PDSCH rate-matching when UE configured with WB RS	Spreadtrum Communications

R1-1804267	Handling of PDCCH on rate matching resource	vivo

R1-1804382	Corrections on Rate Matching	Samsung

R1-1804563	Remaining issues on rate matching	LG Electronics

R1-1804764	Remaining details on rate-matching in NR	Nokia, Nokia Shanghai Bell

R1-1805250	Remaining issues on PDSCH rate matching in CORESET	Qualcomm Incorporated

#### Other

R1-1804283	Consideration on power saving for NR CA	Huawei, HiSilicon

R1-1804773	Potential enhancements for NR UE power saving	Apple Inc.

R1-1805201	TP on remaining issues of carrier aggregation	Ericsson

### Channel coding

R1-1805710	Chairman's notes of AI 7.1.4 Channel coding	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

R1-1803937	Remaining Issues of Channel Coding	Ericsson

R1-1804383	Corrections on Channel Coding	Samsung

R1-1804849	Remaining Issues on URLLC Data Channel Coding 	InterDigital, Inc.

R1-1804889	Consideration on TBS determination	ZTE, Sanechips

Discussed VoIP support of TBS determination procedure in 38.214. There is no consensus on modifying the TBS table. Further discussion in Scheduling/HARQ session when DOCOMO’s contribution is discussed in that agenda item

R1-1805556	TBS determination for HARQ re-transmission, Intel

R1-1805645	Text Proposals on Polar Codes	Samsung, Ericsson, MediaTek, Nokia

Agreement:

The UE is not expected to transmit or receive the bit sequence if K+npc>E, where npc is the number of parity check bits as defined in Section 5.3.1.2;

- Editor to capture this in Section 5.3 of TS 38.212

Agreement:

Polar code segmentation is applied if ( and ) or A >= 1013

- Editor to capture this in Section 6.3.1.2.1 of TS 38.212

Conclusion:

It is RAN1’s understanding that a UE is not expected to receive a retransmission with a TB size that is different from the last valid TB size signalled for this TB.

Note: This does not have any RAN1 specification impact. A note similar to the one in TS36.321, Subclause 5.3.2.2 can be added to 38.321.

### NR-LTE co-existence

R1-1805709	Chairman's notes of AI 7.1.5 NR-LTE co-existence	Ad-Hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

Including architecture option 4 related issues for both NR and LTE, if any

R1-1803656	Discussion on remaining issues for SUL and EN-DC	Huawei, HiSilicon

R1-1804384	Corrections on LTE-NR Coexistence	Samsung

R1-1804455	On NR-LTE coex design	Ericsson GmbH, Eurolab

R1-1804564	Remaining issues on NR-LTE DC/Coexistence	LG Electronics

Conclusion:

Placement of the SUL CIF field in DCI needs to be corrected.

- Details should be handled in the DCI discussions in scheduling and HARQ AI

R1-1804811	Remaining aspects of NR-LTE coexistence	Qualcomm Incorporated

R1-1805143	Power control for EN-DC option 4	Nokia, Nokia Shanghai Bell

Agreement:

In Rel-15, Case 1 HARQ timing is supported for the LTE serving cells only for the following cases

- A single FDD cell

- DL CA with FDD-FDD CA and FDD-TDD CA, where the LTE PCell is FDD and there is only one UL carrier

  - The UE is not configured for cross-carrier scheduling

Agreement:

For a UE operating in EN-DC when configured with Case 1 HARQ timing on an FDD PCell, the following is supported for the LTE serving cells

- For an FDD SCell, the same DL HARQ timing as the PCell is applied for the FDD SCell,

- For a TDD SCell, the same DL HARQ timing as the PCell is applied restricted to the set of DL/special subframes on the TDD SCell

- The UL HARQ timing for the FDD PCell is the same for the CA and non-CA cases

### UL power control

R1-1805708	Chairman's notes of AI 7.1.6 UL Power control	Ad-Hoc chair (Samsung)

Decision: The document is endorsed, content incorporated below.

#### NR UL power control - non-CA aspects

R1-1805553	Summary for Al 7.1.6.1 NR UL power control in non-CA aspects	ZTE, Sanechips

Agreement

Definition of PUSCH open-loop parameter j=0 should also include Msg3 transmission in the RRC_CONNECTED state

Agreement

For PUSCH Msg3 in the RRC_CONNECTED state, UE shall use the SSB or CSI-RS associated with the PRACH for the pathloss measurement

Agreement

For PUSCH Msg3 and UE in RRC_CONNECTED state, if the UE is configured with more than one closed-loop process for PUSCH, the CL-PC index shall be fixed to l=0.

Agreement

For PUCCH without high layer parameter PUCCH-Spatial-relation-info, the closed loop index l should also be set to a fixed value, i.e., l=0.

Agreement:

Single K1 value is supported for PUCCH power control

- The value will be decided in RAN1#93

Agreement

Single K2 value is supported for PUCCH power control

- The value will be decided in RAN1#93

R1-1805665	Summary 2 for Al 7.1.6.1 NR UL power control in non-CA aspects	ZTE, Sanechips

Agreement:

UE uses one selected SSB or CSI-RS resource among SSB(s) and CSI-RS resource(s) associated with indicated RACH resource(s) in the handover message for pathloss estimation for PRACH during handover.

Agreement:

- For group-common TPC command in DCI format 2-2, both closed loops can be adjusted dynamically

Working Assumption

For group-common TPC command in DCI format 2-2,

One bit closedloopindex field is present along with the TPC command in the DCI format 2_2 to indicate which closed loop the TPC command applies to

- When a UE is configured with 1 closed loop, the bit closedloopindex field is NOT present.

- When a UE is configured with 2 closed loops, the bit location of closedloopindex field follows the 2 bit TPC command.

Notes: UE assumes that the start bit of the TPC command is X*tpc_index, where X=3 if configured with 2 closed-loop process, otherwise X=2.

R1-1805637	Way Forward on UL Power Control Regarding NR Scheduling	MediaTek

Conclusion

For further discussion in RAN1#93

- K value for group common TPC is the minimum of the configured K2 values plus one.

  - Applies for both PUCCH, PUSCH, SRS

- Power scaling

R1-1803641	Remaining details of UL power control design	Huawei, HiSilicon

R1-1803763	Remaining issues of Non-CA based power control	CATT

R1-1803842	Remaining issues on non-CA UL power control	vivo

R1-1803916	Remaining issues on NR power control in non-CA aspects	ZTE, Sanechips

R1-1803972	Remaining issues on uplink power control for non-CA	OPPO

R1-1804069	Remaining issues of UL power control	MediaTek Inc.

R1-1804104	Discussion on power control signalling	CMCC

R1-1804385	Corrections on UL Power Control	Samsung

R1-1804503	RRC parameter alignment for EN-DC power control	NEC Corporation

R1-1804566	Remaining issues on non-CA aspect of UL power control	LG Electronics

R1-1804736	On Power Control framework	Intel Corporation

R1-1804812	Remaining issues on power control for NR	Qualcomm Incorporated

R1-1804850	On Pathloss Correction during Beam Switch 	InterDigital, Inc.

R1-1804959	Remaining Details on non-CA NR UL power control	Motorola Mobility, Lenovo

R1-1805060	Remaining issues on PHR	NTT DOCOMO, INC.

R1-1805156	Remaining details on NR PUSCH power control	Nokia, Nokia Shanghai Bell

R1-1805202	Remaining issues for NR power control	Ericsson

#### NR UL power control – CA aspects

R1-1805571	Summary of NR UL power control – CA aspects	Samsung

Agreement

Default parameter setting for virtual PHR

How to set {j, qd, l}

- For j, P0alphasetindex = 0 of p0-pusch-alpha-setconfig

- For qd, pusch-pathlossreference-index = 0 of pusch-pathloss-Reference-rs

- For l, l =0

- Note: If the UE is configured with multiple UL BWPs, j, qd, l corresponding to lowest BWP ID are used

Conclusion: Type 3 PHR is supported only in case PUSCH is not configured

Agreement

- At least from RAN1 perspective, it is not needed to include Pcmax,c together with PH in case of virtual PHR

Agreement: Priority rule for SUL

- For simultaneous uplink transmissions that are power-limited, when one or multiple serving cell(s) are configured with both UL and SUL carriers

  - Existing priority rule based on signals/channels content

  - In case transmissions with the same priority level on the two UL carriers,

    - the UL carrier which is configured for PUCCH has higher priority

R1-1803700	Remaining details of UL power control design for CA	Huawei, HiSilicon

R1-1803764	Parameters setting on virtual PHR for CA	CATT

R1-1803843	Remaining issues on CA UL power control	vivo

R1-1803917	Remaining issues on NR power control in CA aspects	ZTE, Sanechips

R1-1804039	Virtual PHR derivation in some situations	ASUSTEK COMPUTER (SHANGHAI)

R1-1804386	Corrections on UL Power Control for EN-DC	Samsung

R1-1804567	Remaining issues on CA aspect of UL power control	LG Electronics

R1-1804737	Power headroom report for NR carrier aggregation	Intel Corporation

R1-1804813	Remaining Issues on power control  for NR CA	Qualcomm Incorporated

R1-1804960	On CA-related NR UL power control	Motorola Mobility, Lenovo

R1-1805203	Remaining issues for CA and power sharing	Ericsson

#### Other

R1-1804286	Discussion on PHR for SUL	Huawei, HiSilicon

R1-1804288	TP on power control for concurrent transmission of SRS on both UL and SUL	Huawei, HiSilicon

R1-1804387	Corrections on PHR calculation for CA	Samsung

R1-1804449	Independent Power control for SRS	Huawei, HiSilicon

R1-1804675	Timing and Asynchronous Transmissions with Power Control for NR DC	InterDigital, Inc.

R1-1804676	Power Control for NR DC	InterDigital, Inc.

R1-1805204	Remaining issues on closed loop power control	Ericsson

R1-1805205	Power control for UL MIMO	Ericsson

### UE features

R1-1805707	Chairman's notes of AI 7.1.7 on NR - UE features	Ad-hoc chair (NTT DOCOMO, AT&T)

Decision: The document is endorsed, content incorporated below.

R1-1803844	Discussion on Rel-15 NR UE features	vivo

R1-1804055	Discussion on NR UE Feature List	MediaTek Inc.

R1-1804305	Remaining issues of NR UE features	Huawei, HiSilicon

R1-1804388	UE Features	Samsung

R1-1804568	Views on UE feature (scheduling, BWP)	LG Electronics

R1-1804651	Further details on NR UE Feature List	AT&T

R1-1805546	On NR UE feature list	Intel Corporation	(Revision of R1-1804738)

R1-1804814	NR Features and Capabilities	Qualcomm Incorporated

R1-1804815	Signaling aspects for UE capabilities	Qualcomm Incorporated

R1-1804961	On DL 4-layer MIMO supporting	China Unicom

R1-1805002	Discussion on UE capability list	Ericsson

R1-1805061	Views on NR UE features	NTT DOCOMO, INC.

Agreement

Feature 0-13 “Phase coherence across non-contiguous UL symbols in slot in the transmission of one channel” deleted from the NR UE feature list as this feature is not supported in Rel-15.

Note :

Create the following new feature if the following new feature is supported in Rel-15 after other discussion related to R1-1803374.

1-13: Simultaneous Transmission of PRACH and SR (new): Optional with capability signaling

Conclusion

NR UE feature related to June version of Rel-15 specification will be discussed separately.

Conclusion:

“Optional” in the UE feature list means it’s optional with UE capaiblity signaling

Friday

R1-1805728	NR UE feature list	NTT DOCOMO, INC., AT&T

Discussion: Late agreement made in the MIMO session not included in

Decision: The document is agreed.

R1-1805729	Draft LS on NR UE features	NTT DOCOMO, INC., AT&T

Decision: The document is endorsed and final LS is approved in R1-1805752.

### Other

R1-1805206	Corrections to 38.212	Ericsson

R1-1805207	Corrections to 38.213	Ericsson

R1-1805208	Corrections to 38.214	Ericsson

## NR URLLC in Release 15

### Support of separate CQI and MCS table(s) for URLLC

Focus on:

- N separate CQI table(s) are supported for URLLC

  - Downselect the value of N between 1 or 2

- Two target BLER are supported for URLLC

  - Note: RRC signalling is used by gNB to select one of the two target BLER

  - Note: The configuration of target BLER or CQI table is part of CSI report setting

R1-1803661	MCS/CQI design for URLLC	Huawei, HiSilicon

- Observation: The fine granularity CQI/MCS table has a significant improvement on spectrum efficiency which translates directly to improved URLLC system capacity.

- Proposal 1: The CQI/MCS table with target BLER of 1e-3 and 1e-5 should be supported for URLLC.

- Proposal 2: CQI/MCS table design for URLLC should support the fine SNR granularity, as 1dB SNR CQI value granularity and 0.5dB SNR MCS value granularity.

- Proposal 3: Adopt Table 3 as the CQI tables with target BLER 1e-3 and 1e-5 for URLLC.

- Proposal 4: Adopt Table 4 as the MCS tables with target BLER 1e-3 and 1e-5 for URLLC.

- Proposal 5: Selection of MCS table for URLLC should be dynamically indicated.

Decision: The document is noted.

R1-1805515	Considerations on CQI /MCS table(s) and related aspects for URLLC	ZTE, Sanechips	(rev of R1-1803800)

- Proposal 1: One-shot transmission should be supported by URLLC for its lower latency.

- Proposal 2: Two-shot transmission can also be supported by URLLC for its higher transmission efficiency.

- Proposal 3:  The BLER targets of URLLC CQI reporting should be 10-1 and 10-5.

- Proposal 4: The lowest code rate x 1024 of URLLC CQI table should be 40 or 30.

- Proposal 5: The highest code rate x 1024 of URLLC CQI table should be 666 or 873.

- Proposal 6: The CQI table of Option A or B should be taken as URLLC CQI table for BLER target =10-5.

- Proposal 7: The CQI table of Option C or D should be taken as URLLC CQI table for BLER target =10-1.

- Proposal 8: The 5-bit MCS table for URLLC should consist of all the CQI entries and the interpolation between adjacent CQI entries.

- Proposal 9: If the CQI table of Option A is adopted, its corresponding MCS table for CP-OFDM should be the one in Table 8.

- Proposal 10: Some entries with QPSK modulation should be changed into pi/2 BPSK with the same efficiencies for DFT-s-OFDM MCS table.

- Proposal 11: If the CQI table of Option A is adopted, its corresponding MCS table for DFT-s-OFDM should be the one in Table 9.

- Proposal 12: If the CQI table of Option A is adopted, its corresponding 4-bit MCS table for CP-OFDM should be the one in Table 10.

- Proposal 13: If the CQI table of Option A is adopted, its corresponding 4-bit MCS table for DFT-s-OFDM should be the one in Table 11.

- Proposal 14: Support gNB configuration of at least the number of DL symbols per slot to UE for URLLC CQI derivation.

- Observation 1: The accuracy of SNR estimation can be much improved by OLLA in EMBB.

- Observation 2: The fact that OLLA is not suitable in URLLC scenario makes it more important to reduce the difference between the real and the target BLER.

- Observation 3: The accuracy of SNR estimation have an impact on the transmission efficiency.

- Observation 4:  For low SNR range, the CQI report has low accuracy; for medium or high SNR range, the CQI report has high accuracy.

- Observation 5: One-shot transmission scheme has an overall lower latency, and the average latency of different two-shot transmission schemes are comparable. In the case when the initial transmission fails, the latency in most cases is beyond the URLLC user plane latency of 1 ms for two-shot transmission.

- Observation 6: For two shot-transmission, the first transmission BLER target impacts the occupied resource most and a lower target occupies more RE.

- Observation 7: If 10-1 is set as the BLER target for the initial transmission, a BLER target of 10-5 or 10-4 for retransmission occupies almost the same resources.

- Observation 8: there is a about 2dB SNR offset between LTE 16QAM table and LTE 64QAM table at BLER=0.1 for the same CQI index.

- Observation 9: For pi/2 BPSK and QPSK, a similar performance is achieved in MCS 0~3 and the gap becomes large from MCS4 above.

Decision: The document is noted.

R1-1803919	CQI and MCS Tables for URLLC	Ericsson

- Observation 1: In many scenarios physical layer has time for only one or two HARQ transmissions.

- Observation 2: Testing time for CQI reporting at BLER target 10-5 is equivalent to NB-IoT NPDCCH.

- Observation 3: Low rate MCSs is more efficient compare to automatic repetitions in case of contiguous RA.

- Proposal 1: CQI reporting target BLERs supported for URLLC are 10-3 and 10-5 (Option C).

- Proposal 2: Selection of CQI table for URLLC should be done based on RRC configured target BLER.

- Proposal 3: Highest spectral efficiency in any/all CQI table is not greater than 873/1024 * 6 (Option C).

- Proposal 4: Use Table 2 as the CQI table for URLLC, with one set of CQI index for BLER=10-3 and another set for BLER=10-5.

- Proposal 5: Highest spectral efficiency in any/all MCS table is not greater than 873/1024 * 6 (Option C).

- Proposal 6: Selection of MCS table for URLLC should be UE-specific RRC configured.

- Proposal 7: Use Table 3 as the MCS table for PDSCH and PUSCH with CP-OFDM for URLLC.

- Proposal 8: Use Table 4 as the MCS table for PUSCH with DFT-s-OFDM for URLLC.

- Proposal 9: If compact DCI is to be adopted, consider limiting the size of MCS table(s) for URLLCs to be smaller than 5 bits, while taking into account scheduling flexibility.

- Proposal 10: TBS determination for URLLC follows the same procedure as eMBB.

Decision: The document is noted.

R1-1804739	CQI and MCS tables for NR URLLC	Intel Corporation

- Proposal 1: Select 10% and 0.001% (1e-1, 1e-5) as configurable BLER targets for CQI reporting targeting URLLC use cases

- Proposal 2: Single separate CQI table is defined for the second (i.e. lowest) BLER target while the legacy 64QAM CQI table is reused for the first (i.e. higher) BLER target

- Proposal 3

  - Minimum SE point in the new CQI table designed specifically for URLLC is 50 * 2/1024.

  - Maximum SE point in the new CQI table designed specifically for URLLC for 64 QAM table is 873 * 6/1024

- Proposal 4: Adopt Table 2 of current document for CQI table designed for configured BLER 10-5

- Proposal 5

  - Adopt Table 4 of current document as the MCS table designed specifically for URLLC

  - Minimum SE point of MCS table designed specifically for URLLC is 50 * 2 /1024

  - Maximum SE point of MCS table designed specifically for URLLC is 772 * 6 /1024

- Proposal 6

  - MCS Table is independently configured from BLER target and CQI Table

  - MCS Table for UL and DL can be separately configured

Decision: The document is noted.

R1-1805511	Considerations for CQI and MCS for URLLC	Qualcomm Incorporated	(rev of R1-1804817)

- Proposal 1: The minimum SE in the URLLC CQI table should be 2*39/1024 (half of that of the EMBB table).

- Observation 1: The number of allocated RBs is very sensitive to variation in SE in the low SE region, but not in the high SE region.

- Observation 2: The CQI table for EMBB has coarse granularity in low SE, which can result in either inefficient resource allocation or more transmission failures due to insufficient resource allocation.

- Proposal 2: The CQI table for URLLC should take into consideration the sensitivity of RB allocation with respect to SE.

- Proposal 3: The CQI table for URLLC should have finer SE granularity in the low SE region and coarser granularity in the high SE region.

- Proposal 4: The minimum SE entry in the CQI table is included in the MCS table in URLLC.

- Observation 3: A low BLER target will cause the UE to underreport the achievable spectral efficiency.

- Observation 4: A BLER target of 1e-5 can lower the average reported SE by up to 20-50%.

- Observation 5: A first transmission BLER target between 10% and 1% minimizes amount of used resources.

- Observation 6: CQI report with low BLER target can only be practical use under static channel condition and with reasonably

- Proposal 5: UE should not be expected to achieve the reported the very low target BLER if channel and interference varies from where CSI measurement takes place to where data transmission occurs or CSI measurement cannot be done with sufficient accuracy (FFS the level of accuracy).

- Observation 7: Not using 1e-1 as a BLER target for URLLC increases CSI reporting complexity and UL resource.

- Proposal 6: For URLLC, 1e-4 can be considered as the low target BLER in addition to the 1e-1 BLER target for regular 1st transmission BLER target considering all the practical challenges in the accuracy of reporting. Higher reliability can be achieved via gNB scheduling and HARQ retransmissions.

Decision: The document is noted.

Tuesday's wrap-up: Continue discussion offline – Yufei (Ericsson)

Wednesday

R1-1805613	Offline Discussion on Support of Separate CQI and MCS table(s) for URLLC	Ericsson

Decision: The document is noted.

Agreements:

- The two BLER targets that are configurable for URLLC for CSI reporting are:

  - Option B. (10-1, 10-5)

  - Note: The definition of the test case for the BLER target of 10-5 should take into account channel and interference variations and estimation errors.

Agreements:

- Highest spectral efficiency for CQI based on 10-5 BLER target for URLLC is no more than 772/1024*6

- Highest spectral efficiency for CQI based on 10-1 BLER target for URLLC is no more than 873/1024*6

  - It doesn’t necessarily mean that the CQI table introduced for eMBB can not be directly reused for URLLC – it’s still a separate discussion

- Note that

  - Whether or not to have two tables or a single table covering both BLER targets is a separate issue

Thursday

R1-1805642	Summary of Wednesday offline discussion on Support of Separate CQI and MCS table(s) for URLLC	Ericsson

Decision: The document is noted.

Agreements:

- In total, there are two CQI tables for URLLC CQI reporting

  - The first table for URLLC CQI reporting is the same as the existing 64QAM CQI table without any change, which is for BLER target 10-1 for URLLC

    - Note: this means the agreement on “Highest spectral efficiency for CQI based on 10-1 BLER target for URLLC is no more than 873/1024*6” is overturned

  - The new table will have entries corresponding to BLER target 10-5

- For CSI reporting, the CQI field is 4-bit.

Agreements:

- For BLER 10-5,

- Companies are encouraged to perform simulations for the new CQI table for URLLC, including

  - The lowest SE entry

    - E.g., 30~50/1024*2

  - Note that the highest SE entry of no more than 772/1024*6 is already agreed

  - Consider using approximately equally spaced SNR values

    - Other options are not precluded

    - Whether or not some existing CQI entries for BLER 10-1 can be reused

      - Consider exsiting CQI entires when applicable

  - In total 15 CQI entries (+1 OOR entry)

- In performing the simulations, consider

  - Fading channel (TDL-A, 30ns) & (TDL-C, 300ns)

    - Other options are not precluded

  - Payload of 32 bytes

    - Other payload sizes can also be considered, up to each company

  - SNR at 5% geometry for the lowest SE entry

    - Other options are not precluded

  - For other simulation assumptions, refer to agreements from RAN1#92

- Similar considerations are also applicable to the MCS table evaluations

R1-1803765	CQI and MCS design for NR URLLC	CATT

R1-1803845	Discussion on CSI report for URLLC	vivo

R1-1804009	CQI and MCS design for URLLC	OPPO

R1-1804078	On CQI and MCS Design for URLLC	MediaTek Inc.

R1-1804105	Discussion on BLER requirement for URLLC	CMCC

R1-1804569	Discussion on CQI and MCS table design for URLLC	LG Electronics

R1-1804615	Remaining details of CQI and MCS for URLLC	Nokia, Nokia Shanghai Bell

R1-1804641	On details of CQI and MCS tables for NR URLLC	Panasonic

R1-1804658	CQI/MCS table design for URLLC	AT&T

R1-1804851	Remaining Details of CQI and MCS Tables for URLLC 	InterDigital, Inc.

R1-1805519	CQI and MCS Tables for URLLC	Samsung	(revision of R1-1804389)

### Study of necessity of a new DCI format

Whether or not to define a new DCI format(s) that has a smaller DCI payload size than DCI format 0-0 and DCI format 1-0 unicast data

R1-1803657	Compact DCI for URLLC	Huawei, HiSilicon

- Observation 1: For 1e-5 target BLER, around 1dB gain can be achieved by reducing the DCI payload from 40 bits to 24 bits without CRC for AL 16.

- Observation 2: A compact DCI with a smaller DCI payload size than DCI format 0_0 and DCI format 1_0 requires more than 20% less resources in order to meet BLER of 1e-5.

- Proposal 1: For transmission without HARQ-ACK feedback, the operating BLER of the PDCCH for URLLC should be on the order of 1e-5 or smaller.

- Proposal 2: Support a new DCI format(s) design that has a smaller DCI payload size than DCI format 0_0 and DCI format 1_0 for URLLC.

- Proposal 3: Consider the following DL compact DCI format for URLLC

| DCI field | Header | Frequency domain resource allocation | Time domain resource allocation | HARQ process | MCS | NDI | RV | HARQ-ACK timing | TPC | PUCCH resource | A-CQI | CRC | Total payload size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # bits | 1 | 5 | 2 | 3 | 4 | 1 | 2 | 2 | 2 | 1 | 1 | 24 | 48 |

- Proposal 4: Consider the following UL compact DCI format for URLLC

| DCI field | Header | Frequency domain resource allocation | Time domain resource allocation | HARQ process | MCS | NDI | RV | Frequency hopping flag | TPC | CRC | Total payload size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # bits | 1 | 5 | 2 | 3 | 4 | 1 | 1 | 1 | 2 | 24 | 44 |

Decision: The document is noted.

R1-1804390	On Need for Additional DCI Format	Samsung

- Observation 1: In case of 2 Rx and 700MHz, all DCI payloads with AL 16 satisfy URLLC reliability requirements for all channel models.

- Observation 2: In case of 2 Rx and 700MHz, all DCI payloads with AL 4 does not satisfy URLLC reliability requirements for all channel models

- Observation 3: In case of 2 Rx and 700MHz, all DCI payloads with AL 8 satisfy URLLC reliability requirements in TDL-B and TDL-C.

- Observation 4: In case of 2 Rx and 700MHz, only DCI payloads of 30bits and 24bits with AL 8 satisfy URLLC reliability requirements in TDL-A.

- Observation 5: In case of 4 Rx and 4GHz, all DCI payloads with AL 4, 8 and 16 satisfy URLLC reliability requirements for all channel models.

- Observation 6: Compact DCI might not support full scheduling flexibility as Normal DCI does

- Proposal: It does not need to consider compact DCI because current NR specification can sufficiently satisfy URLLC reliability requirement by using normal DCI.

Decision: The document is noted.

R1-1805268	On further study of compact DCI for URLLC	Nokia, Nokia Shanghai Bell	(rev of R1-1804616)

- Observation 1: With the agreed RAN1 link level simulation assumptions, 0.4~0.8 dB gain can be achieved with the compact DCI (30 bits vs. 40 bits) with AL 8 at BLER=10-4.

- Observation 2: With the agreed RAN1 link level simulation assumptions, 0.3~0.4 dB gain can be achieved with the compact DCI (30 bits vs. 40 bits) with AL 16 at BLER=10-4.

- Proposal: Do not specify compact DCI in NR Rel-15, considering both the limited benefit and the resulted impact from compact DCI.

Decision: The document is noted.

R1-1804943	Discussion on compact DCI for NR URLLC	Beijing Xiaomi Mobile Software

- Observation: Around 1 dB performance gain can be achieved by reducing the DCI payload from 40bits to 24bits

- Proposal: The number of the monitored DCI formats should be limited for a URLLC UE.

Decision: The document is noted.

Tuesday's wrap-up: Continue discussion offline – Xinghua (Huawei)

Wednesday

R1-1805630	Summary of 7.2.2 Study of necessity of a new DCI format	Huawei

Decision: The document is noted.

R1-1803766	Necessity of a compact DCI format for NR URLLC	CATT

R1-1803801	NR compact DCI format for URLLC	ZTE, Sanechips

R1-1803846	Discussion on compact DCI for URLLC	vivo

R1-1803920	On Compact DCI for URLLC	Ericsson

R1-1804010	DCI design for URLLC	OPPO

R1-1804079	On Compact DCI Design for URLLC	MediaTek Inc.

R1-1804497	Discussion on compact DCI format for NR URLLC	Panasonic Corporation

R1-1804570	Discussion on compact DCI format design	LG Electronics

R1-1804595	Compact DCI for NR URLLC operation	Sony

R1-1804659	Compact DCI and DM-RS design for URLLC	AT&T

R1-1804740	On compact DCI format for NR URLLC	Intel Corporation

R1-1804818	Considerations on new DCI format	Qualcomm Incorporated

R1-1804852	Evaluation of Compact DCI for URLLC 	InterDigital, Inc.

R1-1804940	Consideration of compact DCI design for URLLC	China Unicom

R1-1805062	Necessity of compact DCI	NTT DOCOMO, INC.

R1-1805096	DCI format for URLLC	China Telecommunications

### Study of necessity of PDCCH repetition

Whether or not for a given carrier, PDCCH repetitions over same or multiple PDCCH monitoring occasion(s) of the same or multiple CORESET and search space

R1-1803921	On PDCCH Repetition for URLLC	Ericsson

- Observation 1: Existing PDCCH design and improved UE capability in terms of increased number of receive antennas provide a strong basis for PDCCH reliability enhancement fulfilling URLLC requirements.

- Observation 2: PDCCH repetition provides extra robustness for DL control information in general but also requires extra resources in time, or frequency.

- Observation 3: PDCCH repetition over multiple CORESETs and search spaces can lead to additional UE complexity regarding PDCCH monitoring and blind decoding.

- Observation 4: Gain from PDCCH repetition over multiple monitoring occasions is unclear.

- Proposal 1	If compact DCI is not adopted, PDCCH repetition associated with PDSCH repetition is considered for improving URLLC reliability.

- Proposal 2	Existing PDCCH design potentially together with compact DCI size are sufficient for fulfilling URLLC reliability requirement.

Decision: The document is noted.

R1-1804071	On PDCCH Repetition for URLLC	MediaTek Inc.

- Observation 1: From IMT-2020 self evaluation results, 5 percentile SINR target can be set as -3.1dB for carrier frequency 4GHz and as -3.0dB for carrier frequency 700MHz.

- Observation 2: For carrier frequency 4GHz, NR PDCCH fulfils the reliability requirement of URLLC for UEs equipped with 4 receive antennas.

- Observation 3: For carrier frequency 700MHz with 2 Rx at the UE side, the current NR PDCCH can attain a BLER less than 10-5 at SNR -2.5dB.

- Observation 4: Compact DCI is a more suitable solution to provide proper gain than PDCCH repetition.

- Observation 5: PDCCH repetition can enhance reliability but is unnecessary for the existing NR-PDCCH.

- Observation 6: In terms of latency, PDCCH repetition provides little benefit to the existing NR-PDCCH.

- Observation 7: Under stringent latency and reliability requirements of URLLC, PDCCH repetition cannot improve spectral efficiency.

- Observation 8: Comparing with PDCCH repetition, HARQ retransmission is a more efficient way to lower the blocking probability.

- Observation 9: PDCCH repetition can enhance scheduling flexibility but with increased monitoring and blind decoding complexity.

- Observation 10: For carrier frequency 700MHz, NR PDCCH with a payload size of 30 bits fulfils the reliability requirement of URLLC for UEs equipped with 2 receive antennas.

- Proposal: PDCCH repetition is not adopted in NR.

Decision: The document is noted.

R1-1804644	On PDCCH repetition for NR URLLC	Sequans Communications

- Observation 1: For URLLC PDCCH, keeping AL 16 or higher as the only mechanism to reach reliability target is inefficient in terms of CCE usage and PDCCH blocking probability, while AL 8 is inadequate.

- Observation 2: Compact DCI provides minor gain for AL of 8 or 16.

- Proposal 1: For NR URLLC, RAN1 should decide on the necessity of compact DCI and PDCCH repetition by taking into account the Y<10-5 PDCCH BLER target to support the one-shot data transmission case, i.e. where only a single PDSCH transmission is possible.

- Proposal 3: PDCCH repetition is supported to achieve URLLC reliability.

- Proposal 4: PDCCH repetition in time, i.e. in different monitoring occasions, where the additional DCI transmission is conditional to the successful reception of the initial DCI by the UE, is supported.

Decision: The document is noted.

R1-1804853	Evaluation of PDCCH Repetition for URLLC	InterDigital, Inc.

- Observation 1: PDCCH repetition is needed for achieving the block error rate of less than 10-5 for URLLC with one-shot transmission.

- Observation 2: For the case of 2 receive antennas at the UE and the lack of frequency selectivity, PDCCH repetition with the highest aggregation level of 16 is not sufficient to satisfy URLLC requirement with one-shot transmission.

- Proposal 1: Introduce URLLC DCI diversity transmission by exploring DCI diversity based on:

  - Uncorrelated spatial resource assignment

  - Disjoint frequency allocations with large frequency distance, e.g. carrier assignment for DCI transmission for the same UE.

- Proposal 2: NR should support paired search spaces over multiple CORESETs

  - FFS: One-to-one correspondence between the candidates of the paired search spaces.

Decision: The document is noted.

Tuesday's wrap-up: Continue discussion offline – Chih-Ping (Qualcomm)

Wednesday

R1-1805633	Offline summary on the study of necessity of PDCCH repetition	Qualcomm

Decision: The document is noted. Continue offline discussion (Xinghua/Chih-Ping) focusing on:

- Whether or not the case of 700MHz, TDL-A, 30ns has issues or not in terms PDCCH reliability, and if so, whether such a case should be addressed by spec

  - Including checking/discussing the target SINR operating point for URLLC

- Whether or not to support one or both of the following:

  - Compact DCI targeting minimal spec impact

    - At least 10 bits reduction?

    - Target to have a list of information fields for reduction

  - PDCCH repetition with minimal spec impact, e.g.,

    - Up to 4 repetitions?

    - Also in freq domain?

Thursday

R1-1805683	Offline summary on compact DCI and PDCCH repetition	Qualcomm

Decision: The document is noted.

- Alt 1: No support of compact DCI and PDCCH repetition

  - Yes: AT&T, MediaTek, Qualcomm, Samsung, Nokia, NSB, Fujitsu, Ericsson, Intel, NTT DOCOMO

  - No: Huawei, HiSilicon, InterDigital, ZTE

- Alt.2: Support compact DCI but not PDCCH repetetion

  - Yes: Vivo, CATT, OPPO, Huawei, HiSilicon, Qualcomm, InterDigital

  - No: Nokia, NSB, Samsung, LGE, Intel, Ericsson, KT

- Alt.3: not supporting compact DCI but support PDCCH repetetion

  - Yes: Nokia, NSB, LGE, ZTE, Sequans, NTT DOCOMO, Intel, SONY, KT

  - No: Qualcomm, MediaTek, CATT, vivo, InterDigital, OPPO, AT&T, Samsung

- Alt.4: support both

  - Yes: Huawei, HiSilicon, ZTE

  - No: Qualcomm, Samsung, MediaTek, AT&T, CATT, Ericsson, Nokia, NSB, vivo, OPPO, NTT DOCOMO

Conclusion:

- There is no consensus in Rel-15 to support:

  - Defining a new DCI format(s) that has a smaller DCI payload size than DCI format 0-0 and DCI format 1-0 unicast data, and/or

  - For a given carrier, PDCCH repetitions over same or multiple PDCCH monitoring occasion(s) of the same or multiple CORESET and search space

R1-1803658	PDCCH repetition for URLLC	Huawei, HiSilicon

R1-1803767	Discussion on enhanced PDCCH for NR URLLC	CATT

R1-1803802	Discussion on PDCCH repetition for URLLC	ZTE, Sanechips

R1-1803847	Discussion on PDCCH repetition for URLLC	vivo

R1-1803943	PDCCH repetition for URLLC transmissions	Fujitsu

R1-1804011	PDCCH repetition for URLLC	OPPO

R1-1804224	On URLLC PDCCH simulation	Spreadtrum Communications

R1-1804234	On enhancing reliability of PDCCH	NEC

R1-1804391	On Need for PDCCH Repetitions	Samsung

R1-1804571	Discussion on PDCCH repetition for URLLC	LG Electronics

R1-1804599	PDCCH repetition for NR URLLC	Sony

R1-1804617	Discussion on reliability enhancement for PDCCH	Nokia, Nokia Shanghai Bell

R1-1804660	On PDCCH repetition for URLLC	AT&T

R1-1804741	On NR PDCCH repetitions for URLLC	Intel Corporation

R1-1804819	Considerations on PDCCH repetition	Qualcomm Incorporated

R1-1804944	Discussion on PDCCH repetition for NR URLLC	Beijing Xiaomi Mobile Software

R1-1805063	Necessity of PDCCH repetition	NTT DOCOMO, INC.

R1-1805547	On PDCCH Reliability	KT Corp.

### Study of handling UL multiplexing of transmissions with different reliability requirements

Study of handling UL multiplexing of transmission with different reliability requirements (including the potential need for UL UE pre-emption)

R1-1804106	Discussion on handling for UL URLLC and eMBB collision issue	CMCC

- Proposal 1: UE decides which one is transmitted when multiple UL transmissions with different traffic priority collide and are not allowed simultaneously. Traffic priority could be taken into account by UE to make the decision.

- Specifically, in case of overlapping between URLLC UL transmission (for both grant-based and grant-free) and eMBB transmissions, URLLC transmission is prioritized and eMBB UL transmission is cancelled.

- Proposal 2: Semi-static power control mechanism is supported in Rel-15. Specifically, multiple sets of power control parameters should be configured by RRC signaling and UE can select one set of parameters according to the traffic priority.

Decision: The document is noted.

R1-1804600	Considerations on UL pre-emption	Sony

- Observation 1: Intra-UE grant based uplink pre-emption of an ongoing eMBB transmission by a grant based URLLC transmission can be avoided via mini-slot level scheduling.

- Observation 2: An intra-UE uplink grant based eMBB transmission can collide with an uplink grant free URLLC transmission.

- Observation 3: Intra-UE uplink grant free URLLC repetitions can collide with another grant free URLLC transmission.

- Observation 4: Since the gNB is unaware when a UE may use the grant free resource for URLLC transmission, transmitting a grant free URLLC transmission using resources originally granted for eMBB would require the gNB to blind detect for possible URLLC or mini-slot transmission for every scheduled eMBB transmission.

- Observation 5: A Pre-emption Indicator can only be transmitted to an eMBB UE if the gNB receives an SR prior to the slot (PDCCH) where the eMBB is transmitted.

- Observation 6: Increasing URLLC transmission power and/or reducing eMBB transmission power would lead to poor SNR for the eMBB transmission and cause spikes of inter-cell interferences.

- Proposal 1: For eMBB scheduled at slot level, the eMBB transmission is terminated if the UE receives an UL grant for a mini-slot PUSCH.

- Proposal 2: For intra-UE pre-emption, when an earlier ongoing (eMBB or URLLC repetition) is pre-empted by a later grant free URLLC transmission, the earlier transmission is terminated and the later grant free URLLC transmission will proceed.

- Proposal 3: For grant based inter-UE URLLC and eMBB pre-emption, a Pre-emption Indicator (PI) can be transmitted with a DCI using a PDCCH prior to the transmission of the eMBB data to terminate its eMBB transmission prior to the URLLC pre-emption.

- Proposal 4: For inter-UE URLLC and eMBB pre-emption where the URLLC is transmitted using uplink grant free that overlaps the eMBB transmission, the eMBB is transmitted in a DTX manner.

Decision: The document is noted.

R1-1804742	Handling UL transmissions with different reliability requirements	Intel Corporation

- Observation 1: There is no need for additional handling of configured grant and dynamic grant prioritization since RAN2 already considered the use cases of URLLC and eMBB multiplexing when taking the decision about current overriding behaviour

- Observation 2: NR provides sufficient mechanisms to configure different power control parameters for different services either dynamically or semi-statically

- Observation 3: Dynamic reduction of eMBB power can be better classified as a generalized form of interruption indication and should be discussed in that context rather than power control techniques

- Observation 4: Different approaches may be considered to support cancellation of UL transmissions in favor of higher priority URLLC traffic in terms of either transmission interruption or continuation signaling and related UE behavior.

- Proposal 1: For a UE supporting URLLC traffic, if the UE receives a dynamic grant scheduling PUSCH overlapping with another PUSCH scheduled by an earlier received grant, the UE follows the latter-received grant and the previously scheduled PUSCH is dropped in the overlapping symbols

  - UE is not expected to continue transmission of the lower priority earlier scheduled PUSCH after dropping the overlapped symbols

- Proposal 2: For a UE supporting URLLC traffic, if the UE is instructed to trigger an UL transmission overlapping with a previously triggered UL transmission regardless whether this is PUSCH or UCI/PUCCH, SRS, the previously triggered UL transmission is dropped starting from the overlapping symbols

- Proposal 3: It is not essential to specify UL interruption indication in NR

Decision: The document is noted.

R1-1804820	eMBB and URLLC dynamic multiplexing and preemption indication on the uplink	Qualcomm Incorporated

- Proposal 1: NR supports dynamic resource sharing between different latency and/or reliability requirements for the eMBB and URLLC services on the uplink.

- Observation 1: Boosting transmission power of URLLC UEs is infeasible for cell-edge UEs that have limited power headroom, and is ineffective in the interference-limited regime for all URLLC UEs.

- Observation 2: Static or semi-static reduction of the transmission power of eMBB UEs significantly impacts the eMBB performance. Dynamic power reduction is similar to the pre-emption indication scheme.

- Observation 3: In intra-cell eMBB and URLLC multiplexing on the uplink, semi-static power control of eMBB UEs significantly degrades the URLLC performance, unless the target received data SNR of eMBB is very low, resulting in significantly degraded eMBB performance.

- Observation 4: Uplink pre-emption indication maximizes the URLLC performance by muting eMBB transmissions that interfere with the URLLC ones. Compared to semi-static power control, the eMBB performance is substantially improved since eMBB transmissions are only suspended when URLLC transmission is present.

- Observation 5: The monitoring and reacting to ULPI, per UE capability, takes less processing time than monitoring UL grants and preparing for uplink data transmissions by URLLC UEs.

- Observation 6: The timing alignments of UEs with different TAs for ULPI processing can be done the same way as how they are handled in SFI (which are signalled similarly in GC-PDCCH).

- Proposal 2: NR should support uplink pre-emption indication for eMBB and URLLC dynamic multiplexing on the uplink. Monitoring of ULPI by eMBB UEs, per UE capability, should be non-slot-based to align with non-slot-based UL URLLC transmissions. ULPI needs to be a current indication scheme and is received by eMBB UEs before the scheduled URLLC PUSCH resources.

- Proposal 3: The discussions of inter-UE multiplexing of eMBB and URLLC should be prioritized over that of intra-UE multiplexing.

Decision: The document is noted.

Tuesday's wrap-up: Continue discussion offline – Xueming (vivo)

Wednesday

R1-1805629	Summary of handling UL multiplexing of transmission with different reliability requirements	vivo

Decision: The document is noted. Continue discussion offline (Xueming) focusing on:

- Whether or not to have any enhancements

- For any potential enhancements, aim to converge on a high-level summary of impacts

  - Aim to have minimal spec impact

Thursday

R1-1805684	URLLC Wednesday Offline summary	vivo

Decision: The document is noted.

Proposal:

- Support of handling inter-UE UL multiplexing of transmission with different reliability requirements (inc. the potential need UL UE pre-emption)

  - No: Huawei, HiSilicon, CATT, Intel, MediaTek

  - Yes: Qualcomm, ZTE, Sharp, NTT DOCOMO, CMCC, NEC, Ericsson, Fujitsu, Mitsubishi, vivo, OPPO, LGE, Sequans, MoTM, Lenovo, Nokia, NSB, Panasonic

General comment from the "No" companies: Only one company has provided simulation results (Qualcomm), how can a decision be made supporting such the above proposal without results showing the benefits/drawbacks?

Conclusion:

- There is no concensus in Rel-15 to support handling inter-UE UL multiplexing of transmission with different reliability requirements (including the potential need for UL UE pre-emption).

R1-1803659	UL multiplexing between URLLC and eMBB	Huawei, HiSilicon

R1-1803768	Multiplexing of UL transmissions of different reliability requirements	CATT

R1-1803803	UL multiplexing of transmissions with different reliability requirements	ZTE, Sanechips

R1-1803848	Discussion on handling UL multiplexing of transmissions with different reliability requirements	vivo

R1-1803922	On Pre-emption in Uplink	Ericsson

R1-1803944	On UL multiplexing of transmissions with different reliability requirements	Fujitsu

R1-1804012	Low-latency transmission for uplink	OPPO

R1-1804080	On UL Multiplexing for URLLC	MediaTek Inc.

R1-1804084	Power control details for multiplexing UL eMBB and URLLC	ASUSTEK COMPUTER (SHANGHAI)

R1-1804222	Discussion on UL multiplexing of transmissions with different reliability requirements	Spreadtrum Communications

R1-1804392	Multiplexing of UL transmissions with different reliability requirements	Samsung

R1-1804450	Uplink Pre-emption for URLLC Reliability	III

R1-1804572	Discussion on multiplexing UL transmission with different requirements	LG Electronics

R1-1804618	On UL multiplexing between eMBB and URLLC	Nokia, Nokia Shanghai Bell

R1-1804642	Discussion on URLLC UL multiplexing	Panasonic

R1-1804772	Discussion on handling of UL multiplexing of transmissions with different reliability requirement	Apple Inc.

R1-1804854	On Dynamic Resource Sharing between eMBB and URLLC in UL 	InterDigital, Inc.

R1-1804945	On UL multiplexing of transmission with different reliability requirements	Beijing Xiaomi Mobile Software

R1-1804947	Discussions on UL multiplexing	Mitsubishi Electric Co.

R1-1804957	Multiplexing of URLLC and eMBB uplink channels	Motorola Mobility, Lenovo

R1-1804963	 Disscussion on UL multiplexing of transmissions with different reliability requirements	China Unicom

R1-1805064	Handling UL multiplexing of transmissions with different reliability requirements	NTT DOCOMO, INC.

R1-1805098	UL Multiplexing for Transmissions of different Reliabilities	TCL Communication

R1-1805155	On uplink multiplexing of transmissions with different reliability requirements	KDDI Corporation

R1-1805158	Considerations for UL multiplexing with different reliability requirements	Sequans Communications

R1-1805246	UCI multiplexing considering URLLC data in PUSCH	WILUS Inc.

### Other

R1-1803660	PDSCH reliability for URLLC	Huawei, HiSilicon

R1-1803698	Discussion on semi-static HARQ-ACK codebook for slot aggregation	Huawei, HiSilicon

R1-1803769	PUCCH enhancement for URLLC	CATT

R1-1803849	Other aspects for URLLC	vivo

R1-1803923	Channel Coding Techniques for URLLC	Ericsson

R1-1803924	On PUCCH for Ultra-Reliable Transmission	Ericsson

R1-1803925	On Performance of PUCCH Format 0 for URLLC Use Cases	Ericsson

R1-1803926	On Performance of PUCCH Format 2 for URLLC Use Cases	Ericsson

R1-1803927	On Frequency Hopping for Ultra-reliable Transmission	Ericsson

R1-1803928	On Transmit Diversity for Ultra-high Reliability Use Cases	Ericsson

R1-1803929	URLLC Downlink System Level Simulation Results	Ericsson

R1-1803930	On Supporting Reliable HARQ Feedback for UL Transmission with Configured Grant	Ericsson

R1-1803931	UCI Transmission for URLLC	Ericsson

R1-1803932	Evaluation of Reliability in NR	Ericsson

R1-1803933	Evaluation of UP latency in NR	Ericsson

R1-1803936	Indication of URLLC Configuration	Ericsson

R1-1803945	UCI piggyback on PUSCH with URLLC data	Fujitsu

R1-1804081	On Grant-Free Transmission Framework for URLLC	MediaTek Inc.

R1-1804173	UCI multiplexing for URLLC	ZTE, Sanechips

R1-1804217	Discussion on possible open issues for PDCCH in URLLC	Spreadtrum Communications

R1-1804289	CSI feedback enhancements for URLLC	Huawei, HiSilicon

R1-1804290	UCI piggyback on PUSCH for URLLC	Huawei, HiSilicon

R1-1804291	PUCCH reliability	Huawei, HiSilicon

R1-1804293	PUCCH resource allocation for non-slot scheduling	Huawei, HiSilicon

R1-1804294	Considerations on DMRS to support URLLC	Huawei, HiSilicon

R1-1804296	Reliability enhancement for grant-free transmission	Huawei, HiSilicon

R1-1804297	Discussion on partially overlapped PUCCH and PUSCH for URLLC	Huawei, HiSilicon

R1-1804428	Discussion on the feasibility of High Accuracy Time Synchronization for NR	Huawei, HiSilicon

R1-1804429	Discussion on the motivation of High Accuracy Time Synchronization for URLLC	Huawei, HiSilicon

R1-1804619	Simultaneous configuration of grant and grant-free based UL transmission for URLLC UEs	Nokia, Nokia Shanghai Bell

R1-1804643	On support of faster inter-bandwidth part hopping for increased reliability	Panasonic

R1-1804677	Transmission of UCI with different reliability requirements	InterDigital, Inc.

R1-1804678	Power Control for Transmissions with Different Service Requirements	InterDigital, Inc.

R1-1804765	Precoder Update for SPS UL	Ericsson Inc.

R1-1804821	Impact of UE processing timeline on the URLLC performance	Qualcomm Incorporated

R1-1805157	Considerations for PUCCH reliability	Sequans Communications

## Study on enhanced NR to support Non-Terrestrial Networks - SID in RP-171450

R1-1805706	Chairman's notes of 7.3 SI on NR to support non-terrestrial networks	Ad-Hoc chair (Samsung)

Decision: The document is endorsed, content incorporated below.

R1-1805652	Summary of NTN offline discussions	Thales

### Channel Model

Agreement: Besides rural and sub-urban environments, two urban environments are included in the channel model for NTN: urban and dense urban.

Agreement: Use the formula  to calculate the coherence bandwidth from the x percentile rms delay spread for each environment and elevation

Working Assumption

- α = 10 and x = 95

Agreement

The two-state ITU model (flat fading model) can be used as a simplified alternative to the TR38.901 methodology for:

- Omnidirectionnal UE antenna patterns

- If for all elevatation angles under consideration in an environment the UE bandwidth is lower than the coherence bandwidth

Agreement

For rural environments, the coherence bandwidth should be the same as that of suburban environment.

Agreement:

For dense urban environment, the following LOS probability is assumed:

| Elevation | LOS probability |
| --- | --- |
| 10° | 28.2% |
| 20° | 33.1% |
| 30° | 39.8% |
| 40° | 46.8% |
| 50° | 53.7% |
| 70° | 73.8% |
| 90° | 98.1% |

Agreement:

For urban environment, the following LOS probability is assumed:

| Elevation | LoS prob. Urban |
| --- | --- |
| 10° | 24.58% |
| 20° | 38.56% |
| 30° | 49.34% |
| 40° | 61.33% |
| 50° | 72.59% |
| 60° | 80.51% |
| 70° | 91.94% |
| 80° | 96.79% |
| 90° | 99.21% |

Agreement

For suburban environment, the LOS probability is as follows:

| Elevation | LOS probability |
| --- | --- |
| 10° | 78,2% |
| 20° | 86,9% |
| 30° | 91,9% |
| 40° | 92,9% |
| 50° | 93,5% |
| 60° | 94% |
| 70° | 94.9% |
| 80° | 95.2% |
| 90° | 99.8% |

Agreement

The tropospheric scintillation is only considered for satellite links above 6GHz, and is defined by the ITU-R P.618-13 model.

Agreement

The ionospheric scintillation is only considered for satellite links below 6GHz, and is defined by the  ITU-R P.531-11 model.

Working Assumption

O2I loss is considered based on the ITU-R P.2109-0 recommendation

Agreement

Atmospheric attenuation is only considered for above 10GHz, and is defined by the ITU-R P.676-11 model.

R1-1805670	TP for chapter 6 of TR38.811	Huawei, HiSilicon, Thales, ESA, Nokia, Nokia Shanghai Bell

Agreement

The text proposal for TR38.811 is agreed.

------- start of Text Proposal -------

6	Non-Terrestrial Networks channel models

6.1. Status/expectation of existing information for satellite/HAPS channels

6.1.1 Channel modeling works outside of 3GPP

6.1.2 Targeted user environment (Satellite: outdoor only; HAPS: outdoor & indoor)

6.1.3 Modeling objectives

6.2 Differences between satellite/HAPS and terrestrial channel modelling approaches

Figures 6.2-1 and 6.2-2 compare the macro-cellular and satellite access links in NLOS and LOS cases, respectively. The terrestrial propagation is quite similar: multipath propagation is caused by objects near the user. However, the angular spread from satellite is almost zero (due to its distant location) while it can still be several degrees from the base station.

Figure 6.2-1: Macro-cellular vs. satellite channel, NLOS

Figure 6.2-2: Macro-cellular vs. satellite channel, LOS

Figure 6.2-3: Combined satellite and terrestrial channels (conceptual drawing).

6.3 Coordinate system

6.4 Antenna modelling

6.4.1 HAPS/Satellite antenna

6.4.2 UE antenna

6.5 Methodology to define channel models

6.5.1 for system-level simulations

6.5.2 for link-level simulations

6.6 Large scale model

6.6.1 LOS probability

6.6.2 Path loss and Shadow fading

6.6.3 O2I penetration loss

6.6.4 Atmospheric absorption

6.6.5 Rain and cloud attenuation

6.6.6 Scintillation

6.7 Fast fading model

6.7.1 Flat fading

6.7.2 Frequency selective fading

In the fast fading model, the process in 7.5 of TR38.901 is used. This section is not a stand-alone description of the fast fading model, but it describes the differences between the channel models used for terrestrial and satellite/HAPS communications.

The channel model parameter tables (Table 7.5-6 Part-1 and Part-2 in TR38.901) are replaced by the following tables.

Table 6.7-6 Part-1: Channel model parameters for Urban Scenario (LOS).

| Scenarios |  | Urban LOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| - Delay spread (DS) - lgDS=log10(DS/1s) | - lgDS |  |  |  |  |  |  |  |  |  |
|  | - lgDS |  |  |  |  |  |  |  |  |  |
| - AOD spread (ASD) - lgASD=log10(ASD/1) | - lgASD |  |  |  |  |  |  |  |  |  |
|  | - lgASD |  |  |  |  |  |  |  |  |  |
| - AOA spread (ASA) - lgASA=log10(ASA/1) | - lgASA |  |  |  |  |  |  |  |  |  |
|  | - lgASA |  |  |  |  |  |  |  |  |  |
| - ZOA spread (ZSA) - lgZSA=log10(ZSA/1) | - lgZSA |  |  |  |  |  |  |  |  |  |
|  | - lgZSA |  |  |  |  |  |  |  |  |  |
| - ZOD spread (ZSD) - lgZSA=log10(ZSD/1) | - lgZSD |  |  |  |  |  |  |  |  |  |
|  | - lgZSD |  |  |  |  |  |  |  |  |  |
| Shadow fading (SF) [dB] | - SF |  |  |  |  |  |  |  |  |  |
| K-factor (K) [dB] | - K |  |  |  |  |  |  |  |  |  |
|  | - K |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ASD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs SF |  |  |  |  |  |  |  |  |  |
|  | - DS vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ASD vs  |  |  |  |  |  |  |  |  |  |
|  | - ASA vs  |  |  |  |  |  |  |  |  |  |
|  | - DS vs  |  |  |  |  |  |  |  |  |  |
|  | - SF vs  |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ZSD vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ZSA |  |  |  |  |  |  |  |  |  |
| - Delay scaling parameter r |  |  |  |  |  |  |  |  |  |  |
| - XPR [dB] | - XPR |  |  |  |  |  |  |  |  |  |
|  | - XPR |  |  |  |  |  |  |  |  |  |
| - Number of clusters |  |  |  |  |  |  |  |  |  |  |
| - Number of rays per cluster |  |  |  |  |  |  |  |  |  |  |
| - Cluster DS () in [ns] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASD () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ZSA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Per cluster shadowing std  [dB] |  |  |  |  |  |  |  |  |  |  |
| - Correlation distance in the horizontal plane [m] | - DS |  |  |  |  |  |  |  |  |  |
|  | - ASD |  |  |  |  |  |  |  |  |  |
|  | - ASA |  |  |  |  |  |  |  |  |  |
|  | - SF |  |  |  |  |  |  |  |  |  |
|  | -  |  |  |  |  |  |  |  |  |  |
|  | - ZSA |  |  |  |  |  |  |  |  |  |
|  | - ZSD |  |  |  |  |  |  |  |  |  |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values - NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values |  |  |  |  |  |  |  |  |  |  |

Table 6.7-6 Part-2: Channel model parameters for Urban Scenario (NLOS).

| Scenarios |  | Urban NLOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| - Delay spread (DS) - lgDS=log10(DS/1s) | - lgDS |  |  |  |  |  |  |  |  |  |
|  | - lgDS |  |  |  |  |  |  |  |  |  |
| - AOD spread (ASD) - lgASD=log10(ASD/1) | - lgASD |  |  |  |  |  |  |  |  |  |
|  | - lgASD |  |  |  |  |  |  |  |  |  |
| - AOA spread (ASA) - lgASA=log10(ASA/1) | - lgASA |  |  |  |  |  |  |  |  |  |
|  | - lgASA |  |  |  |  |  |  |  |  |  |
| - ZOA spread (ZSA) - lgZSA=log10(ZSA/1) | - lgZSA |  |  |  |  |  |  |  |  |  |
|  | - lgZSA |  |  |  |  |  |  |  |  |  |
| - ZOD spread (ZSD) - lgZSA=log10(ZSD/1) | - lgZSD |  |  |  |  |  |  |  |  |  |
|  | - lgZSD |  |  |  |  |  |  |  |  |  |
| Shadow fading (SF) [dB] | - SF |  |  |  |  |  |  |  |  |  |
| K-factor (K) [dB] | - K |  |  |  |  |  |  |  |  |  |
|  | - K |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ASD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs SF |  |  |  |  |  |  |  |  |  |
|  | - DS vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ASD vs  |  |  |  |  |  |  |  |  |  |
|  | - ASA vs  |  |  |  |  |  |  |  |  |  |
|  | - DS vs  |  |  |  |  |  |  |  |  |  |
|  | - SF vs  |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ZSD vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ZSA |  |  |  |  |  |  |  |  |  |
| - Delay scaling parameter r |  |  |  |  |  |  |  |  |  |  |
| - XPR [dB] | - XPR |  |  |  |  |  |  |  |  |  |
|  | - XPR |  |  |  |  |  |  |  |  |  |
| - Number of clusters |  |  |  |  |  |  |  |  |  |  |
| - Number of rays per cluster |  |  |  |  |  |  |  |  |  |  |
| - Cluster DS () in [ns] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASD () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ZSA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Per cluster shadowing std  [dB] |  |  |  |  |  |  |  |  |  |  |
| - Correlation distance in the horizontal plane [m] | - DS |  |  |  |  |  |  |  |  |  |
|  | - ASD |  |  |  |  |  |  |  |  |  |
|  | - ASA |  |  |  |  |  |  |  |  |  |
|  | - SF |  |  |  |  |  |  |  |  |  |
|  | -  |  |  |  |  |  |  |  |  |  |
|  | - ZSA |  |  |  |  |  |  |  |  |  |
|  | - ZSD |  |  |  |  |  |  |  |  |  |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values - NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values |  |  |  |  |  |  |  |  |  |  |

Table 6.7-6 Part-3: Channel model parameters for Dense Urban Scenario (LOS)

| Scenarios |  | Dense Urban LOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| - Delay spread (DS) - lgDS=log10(DS/1s) | - lgDS |  |  |  |  |  |  |  |  |  |
|  | - lgDS |  |  |  |  |  |  |  |  |  |
| - AOD spread (ASD) - lgASD=log10(ASD/1) | - lgASD |  |  |  |  |  |  |  |  |  |
|  | - lgASD |  |  |  |  |  |  |  |  |  |
| - AOA spread (ASA) - lgASA=log10(ASA/1) | - lgASA |  |  |  |  |  |  |  |  |  |
|  | - lgASA |  |  |  |  |  |  |  |  |  |
| - ZOA spread (ZSA) - lgZSA=log10(ZSA/1) | - lgZSA |  |  |  |  |  |  |  |  |  |
|  | - lgZSA |  |  |  |  |  |  |  |  |  |
| - ZOD spread (ZSD) - lgZSA=log10(ZSD/1) | - lgZSD |  |  |  |  |  |  |  |  |  |
|  | - lgZSD |  |  |  |  |  |  |  |  |  |
| Shadow fading (SF) [dB] | - SF |  |  |  |  |  |  |  |  |  |
| K-factor (K) [dB] | - K |  |  |  |  |  |  |  |  |  |
|  | - K |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ASD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs SF |  |  |  |  |  |  |  |  |  |
|  | - DS vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ASD vs  |  |  |  |  |  |  |  |  |  |
|  | - ASA vs  |  |  |  |  |  |  |  |  |  |
|  | - DS vs  |  |  |  |  |  |  |  |  |  |
|  | - SF vs  |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ZSD vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ZSA |  |  |  |  |  |  |  |  |  |
| - Delay scaling parameter r |  |  |  |  |  |  |  |  |  |  |
| - XPR [dB] | - XPR |  |  |  |  |  |  |  |  |  |
|  | - XPR |  |  |  |  |  |  |  |  |  |
| - Number of clusters |  |  |  |  |  |  |  |  |  |  |
| - Number of rays per cluster |  |  |  |  |  |  |  |  |  |  |
| - Cluster DS () in [ns] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASD () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ZSA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Per cluster shadowing std  [dB] |  |  |  |  |  |  |  |  |  |  |
| - Correlation distance in the horizontal plane [m] | - DS |  |  |  |  |  |  |  |  |  |
|  | - ASD |  |  |  |  |  |  |  |  |  |
|  | - ASA |  |  |  |  |  |  |  |  |  |
|  | - SF |  |  |  |  |  |  |  |  |  |
|  | -  |  |  |  |  |  |  |  |  |  |
|  | - ZSA |  |  |  |  |  |  |  |  |  |
|  | - ZSD |  |  |  |  |  |  |  |  |  |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values - NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values |  |  |  |  |  |  |  |  |  |  |

Table 6.7-6 Part-3: Channel model parameters for Dense Urban Scenario (NLOS)

| Scenarios |  | Dense Urban NLOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| - Delay spread (DS) - lgDS=log10(DS/1s) | - lgDS |  |  |  |  |  |  |  |  |  |
|  | - lgDS |  |  |  |  |  |  |  |  |  |
| - AOD spread (ASD) - lgASD=log10(ASD/1) | - lgASD |  |  |  |  |  |  |  |  |  |
|  | - lgASD |  |  |  |  |  |  |  |  |  |
| - AOA spread (ASA) - lgASA=log10(ASA/1) | - lgASA |  |  |  |  |  |  |  |  |  |
|  | - lgASA |  |  |  |  |  |  |  |  |  |
| - ZOA spread (ZSA) - lgZSA=log10(ZSA/1) | - lgZSA |  |  |  |  |  |  |  |  |  |
|  | - lgZSA |  |  |  |  |  |  |  |  |  |
| - ZOD spread (ZSD) - lgZSA=log10(ZSD/1) | - lgZSD |  |  |  |  |  |  |  |  |  |
|  | - lgZSD |  |  |  |  |  |  |  |  |  |
| Shadow fading (SF) [dB] | - SF |  |  |  |  |  |  |  |  |  |
| K-factor (K) [dB] | - K |  |  |  |  |  |  |  |  |  |
|  | - K |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ASD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs SF |  |  |  |  |  |  |  |  |  |
|  | - DS vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ASD vs  |  |  |  |  |  |  |  |  |  |
|  | - ASA vs  |  |  |  |  |  |  |  |  |  |
|  | - DS vs  |  |  |  |  |  |  |  |  |  |
|  | - SF vs  |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ZSD vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ZSA |  |  |  |  |  |  |  |  |  |
| - Delay scaling parameter r |  |  |  |  |  |  |  |  |  |  |
| - XPR [dB] | - XPR |  |  |  |  |  |  |  |  |  |
|  | - XPR |  |  |  |  |  |  |  |  |  |
| - Number of clusters |  |  |  |  |  |  |  |  |  |  |
| - Number of rays per cluster |  |  |  |  |  |  |  |  |  |  |
| - Cluster DS () in [ns] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASD () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ZSA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Per cluster shadowing std  [dB] |  |  |  |  |  |  |  |  |  |  |
| - Correlation distance in the horizontal plane [m] | - DS |  |  |  |  |  |  |  |  |  |
|  | - ASD |  |  |  |  |  |  |  |  |  |
|  | - ASA |  |  |  |  |  |  |  |  |  |
|  | - SF |  |  |  |  |  |  |  |  |  |
|  | -  |  |  |  |  |  |  |  |  |  |
|  | - ZSA |  |  |  |  |  |  |  |  |  |
|  | - ZSD |  |  |  |  |  |  |  |  |  |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values - NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values |  |  |  |  |  |  |  |  |  |  |

Table 6.7-6 Part-1: Channel model parameters for Suburban and Rural Scenarios (LOS).

| Scenarios |  | Suburban and Rural LOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| - Delay spread (DS) - lgDS=log10(DS/1s) | - lgDS |  |  |  |  |  |  |  |  |  |
|  | - lgDS |  |  |  |  |  |  |  |  |  |
| - AOD spread (ASD) - lgASD=log10(ASD/1) | - lgASD |  |  |  |  |  |  |  |  |  |
|  | - lgASD |  |  |  |  |  |  |  |  |  |
| - AOA spread (ASA) - lgASA=log10(ASA/1) | - lgASA |  |  |  |  |  |  |  |  |  |
|  | - lgASA |  |  |  |  |  |  |  |  |  |
| - ZOA spread (ZSA) - lgZSA=log10(ZSA/1) | - lgZSA |  |  |  |  |  |  |  |  |  |
|  | - lgZSA |  |  |  |  |  |  |  |  |  |
| - ZOD spread (ZSD) - lgZSA=log10(ZSD/1) | - lgZSD |  |  |  |  |  |  |  |  |  |
|  | - lgZSD |  |  |  |  |  |  |  |  |  |
| Shadow fading (SF) [dB] | - SF |  |  |  |  |  |  |  |  |  |
| K-factor (K) [dB] | - K |  |  |  |  |  |  |  |  |  |
|  | - K |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ASD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs SF |  |  |  |  |  |  |  |  |  |
|  | - DS vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ASD vs  |  |  |  |  |  |  |  |  |  |
|  | - ASA vs  |  |  |  |  |  |  |  |  |  |
|  | - DS vs  |  |  |  |  |  |  |  |  |  |
|  | - SF vs  |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ZSD vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ZSA |  |  |  |  |  |  |  |  |  |
| - Delay scaling parameter r |  |  |  |  |  |  |  |  |  |  |
| - XPR [dB] | - XPR |  |  |  |  |  |  |  |  |  |
|  | - XPR |  |  |  |  |  |  |  |  |  |
| - Number of clusters |  |  |  |  |  |  |  |  |  |  |
| - Number of rays per cluster |  |  |  |  |  |  |  |  |  |  |
| - Cluster DS () in [ns] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASD () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ZSA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Per cluster shadowing std  [dB] |  |  |  |  |  |  |  |  |  |  |
| - Correlation distance in the horizontal plane [m] | - DS |  |  |  |  |  |  |  |  |  |
|  | - ASD |  |  |  |  |  |  |  |  |  |
|  | - ASA |  |  |  |  |  |  |  |  |  |
|  | - SF |  |  |  |  |  |  |  |  |  |
|  | -  |  |  |  |  |  |  |  |  |  |
|  | - ZSA |  |  |  |  |  |  |  |  |  |
|  | - ZSD |  |  |  |  |  |  |  |  |  |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values - NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values |  |  |  |  |  |  |  |  |  |  |

Table 6.7-6 Part-2: Channel model parameters for Suburban and Rural Scenarios (NLOS).

| Scenarios |  | Suburban and Rural NLOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| - Delay spread (DS) - lgDS=log10(DS/1s) | - lgDS |  |  |  |  |  |  |  |  |  |
|  | - lgDS |  |  |  |  |  |  |  |  |  |
| - AOD spread (ASD) - lgASD=log10(ASD/1) | - lgASD |  |  |  |  |  |  |  |  |  |
|  | - lgASD |  |  |  |  |  |  |  |  |  |
| - AOA spread (ASA) - lgASA=log10(ASA/1) | - lgASA |  |  |  |  |  |  |  |  |  |
|  | - lgASA |  |  |  |  |  |  |  |  |  |
| - ZOA spread (ZSA) - lgZSA=log10(ZSA/1) | - lgZSA |  |  |  |  |  |  |  |  |  |
|  | - lgZSA |  |  |  |  |  |  |  |  |  |
| - ZOD spread (ZSD) - lgZSA=log10(ZSD/1) | - lgZSD |  |  |  |  |  |  |  |  |  |
|  | - lgZSD |  |  |  |  |  |  |  |  |  |
| Shadow fading (SF) [dB] | - SF |  |  |  |  |  |  |  |  |  |
| K-factor (K) [dB] | - K |  |  |  |  |  |  |  |  |  |
|  | - K |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ASD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ASA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs SF |  |  |  |  |  |  |  |  |  |
|  | - DS vs SF |  |  |  |  |  |  |  |  |  |
|  | - ASD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ASD vs  |  |  |  |  |  |  |  |  |  |
|  | - ASA vs  |  |  |  |  |  |  |  |  |  |
|  | - DS vs  |  |  |  |  |  |  |  |  |  |
|  | - SF vs  |  |  |  |  |  |  |  |  |  |
| Cross-Correlations | - ZSD vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs SF |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs K |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs DS |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASD |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSA vs ASA |  |  |  |  |  |  |  |  |  |
|  | - ZSD vs ZSA |  |  |  |  |  |  |  |  |  |
| - Delay scaling parameter r |  |  |  |  |  |  |  |  |  |  |
| - XPR [dB] | - XPR |  |  |  |  |  |  |  |  |  |
|  | - XPR |  |  |  |  |  |  |  |  |  |
| - Number of clusters |  |  |  |  |  |  |  |  |  |  |
| - Number of rays per cluster |  |  |  |  |  |  |  |  |  |  |
| - Cluster DS () in [ns] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASD () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ASA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Cluster ZSA () in [deg] |  |  |  |  |  |  |  |  |  |  |
| - Per cluster shadowing std  [dB] |  |  |  |  |  |  |  |  |  |  |
| - Correlation distance in the horizontal plane [m] | - DS |  |  |  |  |  |  |  |  |  |
|  | - ASD |  |  |  |  |  |  |  |  |  |
|  | - ASA |  |  |  |  |  |  |  |  |  |
|  | - SF |  |  |  |  |  |  |  |  |  |
|  | -  |  |  |  |  |  |  |  |  |  |
|  | - ZSA |  |  |  |  |  |  |  |  |  |
|  | - ZSD |  |  |  |  |  |  |  |  |  |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values - NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values |  |  |  |  |  |  |  |  |  |  |

6.8 Additional modelling components

6.8.1 Time-varying Doppler shift

6.8.2 Faraday Rotation

6.9 Channel models for link level simulations

6.9.1 CDL models

6.9.2 TDL models (3GPP TR38.901 based and ITU-R based)

6.10 Channel model calibration

------- end of Text Proposal -------

R1-1805680	NR-NTN Channel Modeling – Text proposal for the 6th clause of TR38.811	Thales	(Revision of R1-1805116)

Agreement: The text proposal in R1-1805680 for TR38.811 is endorsed.

Agreement: The clear-air basic path loss for a single unwanted transmitter and victim receiver over an Earth-space path is composed of losses as follows:

Lb=Lbfs+Ag+As+Lc+Lbe                                               (dB)

where:

:	 Free space basic path loss

:	 Attenuation due to atmospheric gasses

: 	Attenuation due to either ionospheric or troposcatter scintillation

: 	Earth-space and Aeronautical statistical clutter loss

: 	Building entry/exit loss

Note : No indoor conditions are considered for satellite links.

Agreement: The clutter loss = 0dB in LOS conditions

R1-1803675	General considerations on NTN Channel Modeling	Huawei, HiSilicon

R1-1804235	Discussion on the large scale channel model for NTN	ZTE, Sanechips,CATR

R1-1804393	Remaining issues on NTN Channel Modeling	Samsung

R1-1804470	Fast Fading Model of GEO S-Band Satellite 	Dish Network

R1-1804479	Path loss model for NTN	Nokia, Nokia Shanghai Bell

R1-1804480	Fast fading characteristics for NTN	Nokia, Nokia Shanghai Bell

R1-1804855	Modeling of Tropospheric Scintillation Effects in NTN Channel Model 	InterDigital, Inc.

R1-1805083	Ionospheric scintillations for NTN channel	ESA

R1-1805084	Tropospheric scintillations for NTN channel	ESA

R1-1805114	NR-NTN channel modeling - Flat fading criteria	THALES

R1-1805115	NR-NTN Channel Modeling – Large scale parameters	THALES

### Other

R1-1803673	Consideration on GEO and LEO channel models	Huawei, HiSilicon

R1-1803674	HAPS channel modeling	Huawei, HiSilicon

R1-1803696	Ray Tracing simulation result for NTN	Huawei, HiSilicon

R1-1803697	On the multipath propagation in satellite to UE link	Huawei, HiSilicon

R1-1804236	Discussion on the NR impacts on random access for NTN	ZTE, Sanechips,CATR

R1-1804304	TP for chapter 6 of TR38.811	Huawei, HiSilicon

R1-1804394	PT-RS design consideration for NTN	Samsung

R1-1804395	Considerations on random access for NTN	Samsung

R1-1804467	TDD and FDD for NTN Applications	HUGHES Network Systems Ltd

R1-1804495	Considerations on NR impacts for Non-Terrestrial Networks	ETRI

R1-1804610	Considerations on NTN Relay use cases	Sony

R1-1804640	Phase noise correction for non-terrestrial networks	Mitsubishi Electric RCE

R1-1804822	On NTN initial access	Qualcomm Incorporated

R1-1804856	Simulation Results for PTRS Performance in Non-Terrestrial Networks 	InterDigital, Inc.

R1-1804857	Deactivating HARQ for Non-Terrestrial Networks 	InterDigital, Inc.

R1-1804858	Considerations on Timing Advance for NTN 	InterDigital, Inc.

R1-1805078	NR-NTN: Link budget analysis	THALES

R1-1805079	NR-NTN: Potential areas of impact, chapter structure	THALES

R1-1805089	NR-NTN: Impact of channel frequency selectivity on DM-RS distribution (frequency)	THALES

R1-1805094	NR-NTN: Impact of Doppler variation rate on DM-RS distribution (time)	THALES

R1-1805095	NR-NTN: Impact on Initial TA during random access procedure	THALES

R1-1805130	NTN NR impacts due to phase noise	Fraunhofer IIS

## Study on non-orthogonal multiple access for NR - SID in RP-171043

R1-1803613	Text proposal for LLS parameters in TR 38.812 (NOMA)	ZTE, Sanechips

Decision: The TP is endorsed.

R1-1803614	Updated work plan for Rel-15 SI on NR NOMA	ZTE, Sanechips

Decision: The document is noted. For information.

### Transmitter side signal processing schemes for NOMA

R1-1803615	Key processing modules at transmitter side for NOMA	ZTE, Sanechips

- Proposal 1: Further clarify the following simulation parameters:

  - Number of TBS value does not include the CRC overhead which is 16 bits for NR LDPC when TBsize < 3824 bits.

  - For the ideal channel estimation for calibration purpose, REs of one every 7 symbols for DMRS will not be used to carry the data.

  - Using 8 PRB for high payload sizes of 60 and 75 Bytes in mMTC, 16 PRB for 60 and 75 Bytes in URLLC, and 16 PRB for 120 and 150 Bytes in eMBB.

- Observation 1: NOMA with QAM-sequence based symbol-level linear spreading can achieve excellent overloading capability with very limited per UE performance loss in all the interested use scenarios.

- Observation 2: Low code rate is beneficial when the overloading is not very high, while low inter-user interference is beneficial for high overloading cases.

- Observation 3: Trade-off of spreading factor between per UE performance and overloading capability should be considered for the design of symbol-level linear spreading based NOMA.

- Observation 4: Different schemes have similar performance at moderate system overloading and low per UE spectral efficiency. For higher overloading or per UE spectral efficiency, more comprehensive evaluations should be performed to have better understanding on the performances of different schemes.

Decision: The document is noted.

R1-1803953	Considerations on NOMA schemes	Google, Inc.

- Proposal 1: A flexible NOMA transmitter is desired to support mMTC, eMBB, and URLLC.

- Proposal 2: Evaluate and compare NOMA features with single spreading ingredient first.

- LDS: PDMA

  - Full-length symbol spreading: RSMA, GOCA, NCMA, NCOA, (LDS-SVE without LDS)

  - LCRS: LCRS

  - Bit spreading: MUSA

- Proposal 3: The PAPR of LDS based features needs for further investigation.

- Proposal 4: Consider multiple features to NOMA scheme regarding different use scenarios.

- Proposal 5: Companies should study the potential of combining multiple NOMA features under the consideration of the mutual interference within the signature resource pool.

Decision: The document is noted.

R1-1804823	Transmitter side signaling processing schemes for NOMA	Qualcomm Incorporated

Proposal 1: The standardization of NOMA UL transmission should consider the best trade-off among the following design objectives:

- error performance [4-5]

  - conditioned on per UE SE and the number of multiplexing UEs

- sum throughput vs SNR for a given BLER target [4-5]

  - conditioned on per UE SE and the number of multiplexing UEs

- scalability

  - easy adaptation of spreading and/or scrambling codes configuration to accommodate N NOMA UEs with spreading factor K, where N and K can be configured dynamically

- complexity and latency [4-5]

  - transmitter side and receiver side processing, including computation and memory requirements for successful data decoding

  - latency of advanced receiver

- flexibility

  - joint support of DFT-s-OFDM waveform and CP-OFDM waveform

  - joint support for different operation modes and use cases

- PAPR and ACLR

Proposal 2: NR NOMA SI needs to consider linear hybrid spreading and scrambling as the UL transmission scheme.

Proposal 3: The design, evaluation and optimization of linear spreading codebook needs to consider a comprehensive set of overloading configurations for (K, N), where K denotes the spreading factor and N denotes the codebook size.

Proposal 4: Depending on the use cases of NOMA UL transmission, the following options of linear hybrid spreading and scrambling need to be considered:

- Option A: Apply UE-specific short spreading code only, wherein the configuration of spreading code can be made symbol-dependent;

- Option B: Apply UE-specific long scrambling codes only, wherein the configuration of scrambling codes can be made symbol-dependent and can be combined with repetition of modulation symbols;

- Option C: Joint use of UE-specific short spreading code and cell-specific long scrambling code, wherein the configuration of spreading codes and/or scrambling codes can be made symbol-dependent.

Proposal 5:  To reduce the signalling overhead of NOMA systems, as well as the complexity and memory size of transceivers, it is desirable to employ the following schemes for NOMA transmission:

- a closed form design for the short spreading codes that achieves Welch bound equality (WBE) for arbitrary spreading factor (K) and overloading ratio (N/K>1);

- a closed from design for the long scrambling codes that is compliant with Rel-15 NR specification;

- reuse the same spreading codes and scrambling codes for both DFT-s-OFDM and CF-OFDM waveforms;

- reuse linear modulation of NR Tx chain.

Proposal 6: The design of linear spreading scheme for NOMA should consider the inclusion of symbol-wise scrambling to improve the PAPR performance. The PAPR distribution of QPSK can be used as a baseline.

Proposal 7:  In NR NOMA SI, the complexity, configuration flexibility and scalability of transmitter configuration, as well as the memory requirements, should be evaluated and compared.

Decision: The document is noted.

Tuesday's wrap-up: Continue discussion offline – Li (ZTE)

R1-1803663	Discussion on the design of SCMA	Huawei, HiSilicon

R1-1803770	PDMA transmitter side signal processing	CATT

R1-1803850	Discussion on NOMA transmitters	vivo

R1-1804107	Considerations on Rel-15 NoMA transmitter design	CMCC

R1-1804116	Discussion on NoMA Schemes for NR	NEC

R1-1804396	Transmitter side signal processing schemes for NOMA	Samsung

R1-1804462	Considerations on NOMA transmitter	Nokia, Nokia Shanghai Bell

R1-1804573	Transmitter side signal processing schemes for NCMA	LG Electronics

R1-1804601	Consideration on the transmitter side signal processing for NOMA	Sony

R1-1804667	On Transnmitter side processing for NOMA	AT&T

R1-1804743	NOMA transmission scheme: LCRS	Intel Corporation

R1-1804859	Implementation Aspects of Low Code Rate NOMA Schemes 	InterDigital, Inc.

R1-1804941	Tx side processing for UL NOMA	NICT

R1-1805003	Signature design for NoMA	Ericsson

R1-1805065	Transmitter design for uplink NOMA	NTT DOCOMO, INC.

### Receivers for NOMA

R1-1803664	Discussion on the design of NoMA receiver	Huawei, HiSilicon

- Observation 1: Different NoMA receivers adopt a common iterative structure that iterates between the SU/MU detector and the FEC decoders to separate mutually interfered UEs at the receiver.

- Observation 2: An iterative receiver can improve the BLER performance of the NoMA transmissions. The higher the number of users, the more iterations may be needed.

- Observation 3: The implementation of MPA supports full parallelism and its complexity can be greatly reduced by low projection in constellation design and ordered group SIC decoding.

- Observation 4: ESE-PIC receiver heavily relies on the coding gain from the FEC and thus may have slower convergence, especially at high spectral efficiency and high overloading.

- Observation 5: A Block-wise MMSE has much higher complexity than a chip-by-chip MMSE.

- Observation 6: EPA can achieve similar performance as MPA in many cases of interest with lower complexity and has better convergence performance compared with ESE.

- Proposal 1: The iterative receiver structure with configurable number of outer-loop iterations between MU detector and channel decoders is supported for NoMA transmissions.

- Proposal 2: Further study the performance limits, application scenarios, and the implementation costs of the iterative NoMA receivers with different MU detectors and different IC methods.

Decision: The document is noted.

R1-1804744	Receiver structure for NOMA	Intel Corporation

- Observation 1:

  - Baseline SIC shows higher BLER compared to modified SIC and PIC

  - Modified SIC shows comparable BLER performance with PIC while it needs much less decoding trials than PIC

- Observation 2: IC-based channel estimation algorithm can improve the channel estimation and hence overall decoding performance, especially for the support of a relatively large number of UEs.

- Observation 3: The complexity of MMSE equalizer is increased by the order of the SF2 if spreading based NOMA scheme is used

- Proposal 1: IC-based receiver (hard IC or soft IC) should be a baseline receiver of NOMA

  - Modified SIC is considered for hard IC based receiver

- Proposal 2: Enhanced channel estimation algorithm should be studied for NOMA receivers.

Decision: The document is noted.

R1-1805004	NoMA receiver design	Ericsson

- Observation 1: A CW-level successive IC receiver processing overhead in a gNB is mainly due to re-encoding each detected user and the associated buffering. Symbol-level IC obviates the need for re-encoding, with minor expected performance penalty in short-TBS scenarios.

- Observation 2: Sequential receiver structures, detecting one signal at a time, have reduced decoding and signal regeneration complexity compared to iterative receivers.

- Observation 3: JD and iterative receivers are less suited for NOMA reception in scenarios with unknown MCS, compared to sequential receivers.

- Observation 4: Symbol-level MMSE-SIC type of receivers, e.g. OMMSE-SIC, are versatile and robust for WSMA-based NOMA reception.

- Observation 5: A commercial gNB preferably implements a single UL MU receiver architecture for different multiple-access signal demodulation scenarios, including regular MU-MIMO and NOMA.

- Observation 6: NOMA evaluations should be based on receiver structures that are likely to be implemented in gNB products.

- Proposal 1: NOMA designs should target receiver architectures that suitable for both MU-MIMO and NOMA reception.

- Proposal 2: NOMA signature design should target the lowest possible receiver complexity needed to attain performance gains, while being scalable to a wide variety of MCS states.

- Proposal 3: Both OMA and MU-MIMO should be baseline schemes for comparing NOMA performance. Performance evaluation of all schemes should be done at similar receiver complexities.

Decision: The document is noted.

Tuesday's wrap-up: Continue discussion offline – Li (ZTE)

Thursday

R1-1805589	Summary of offline discussion on receiver types	ZTE, Sanechips

Decision: The document is noted.

Agreements:

Adopt Figure 1 as the general block diagram of multi-user receiver for UL data transmissions.

- The algorithms for the detector block (for data) can be e.g. MMSE, MF, ESE, MAP, MPA, EPA.

- The interference cancellation can be hard, soft, or hybrid, and can be implemented in serial, parallel, or hybrid.

  - Note: the IC block may consist of an input of the received signal for some types of IC implementations

- The interference cancellation block may or may not be used.

  - Note: if not used, an input of interferene estimation to the decoder may be required for some cases.

- The input to interference cancellation may come directly from the Detector for some cases

Figure 1 A high-level block diagram of multi-user receiver

R1-1803616	Typical multi-user receivers for NOMA	ZTE, Sanechips

R1-1803771	Discussion on PDMA receivers	CATT

R1-1803851	Receiver design considerations for NOMA	vivo

R1-1804397	Considerations on receiver types for NoMA schemes	Samsung

R1-1804463	Receiver considerations for UL NOMA	Nokia, Nokia Shanghai Bell

R1-1804574	Considerations on the receiver types and NoMA schemes	LG Electronics

R1-1804668	Low complexity Receiver design for NOMA	AT&T

R1-1804824	Receivers for NOMA	Qualcomm Incorporated

R1-1804860	DMRS CHEST Error Model for NOMA Evaluation 	InterDigital, Inc.

R1-1804942	Receivers for UL NOMA	NICT

R1-1805066	Receiver candidate for uplink NOMA transmission	NTT DOCOMO, INC.

R1-1805082	Multistage MMSE scheme for low complexity NOMA receivers	ESA, AIRBUS DS SLC

### Procedures related to NOMA

R1-1803852	Discussion on NOMA procedures	vivo

- Proposal 1: For asynchronous UL transmission, both cell-specific and UE specific signalling should be allowed for MA signature pool configuration.

- Proposal 2: The preambles used for UL transmission may need to be associated to UE ID.

  - Mapping between preamble and DMRS could be limited to reduce Rx complexity.

  - Mapping between DMRS and MA signature could be one to one to reduce Rx complexity.

- Proposal 3: To reduce false alarm, UE ID could be piggybacked in transmitted data in the following way

- CRC check

- MAC CE

- Proposal 4: Group-common DCI signalling could be used for HARQ ACK/NACK for NOMA transmission

- The DCI field applicable to a specific UE is based on preambles selected.

- Proposal 5:  For pool-based MA signature allocation and selection, without explicit ACK/NACK signaling, UE should not assume correct transmission.

  - OMA resources could be scheduled for retransmission and share the same HARQ process ID as previous NOMA transmission.

- Proposal 6: For synchronous UL transmission, pool-based based MA signature allocation and selection should also be supported.

- Proposal 7:  For synchronous UL transmission, Preambles like what is used by asynchronous UL transmission could also be used for synchronous UL transmission for the network to identify the UE.

  - DMRS and MA signature are both pool based and MA signature could be indexed by DMRS port or configuration index

- Proposal 8:  For pool-based MA signature allocation and selection, if preambles are not transmitted DMRS should be one to one mapped to a specific UE ID for UL detection.

  - To reduce Rx complexity, the mapping between MA signature and UE ID could be limited through configuration.

- Proposal 9:  Group common DCI signaling for ACK/NACK and OMA based retransmission could also be used for synchronous UL transmission.

Decision: The document is noted.

R1-1804398	Procedures related consideration to NOMA	Samsung

- Observation 1: the Alt.1 of grant free transmission (viewed as being similar to SPS transmission, where multiple access (MA) resources are pre-assigned/pre-configured to UEs, and the MA signatures pool shared by UEs are actually transparent to UE) can have gNB controllable collision of MA signature.

- Observation 2: HARQ retransmission can be supported for contention based non-orthogonal multiple access, e.g. by explicit signalling or by resource partitioning and mapping, for identifying initial transmission and associated retransmission(s).

- Observation 3: Link adaptation with explicit/implicit MCS signalling can be supported by, e.g. resource partitioning, for grant-free non-orthogonal multiple access.

- Observation 4: Enhanced DMRS design should be studied for improving channel estimation performance and the UE-detection.

- Observation 5: Dynamic switching between OMA and NoMA can be considered for grant-based non-orthogonal multiple access.

- Observation 6: Dynamic resource allocation is group-common information for grant-based non-orthogonal multiple access.

- Proposal 1: Means to support grant-free alt.1 can be studied for grant-free/contention based non-orthogonal multiple access.

- Proposal 2: Means to support HARQ re-transmission should be studied for grant-free/contention based non-orthogonal multiple access.

- Proposal 3: Means to support link adaptation should be studied for grant-free/contention based non-orthogonal multiple access.

Decision: The document is noted.

R1-1805067	Initial views on procedures related to NOMA	NTT DOCOMO, INC.

- Proposal 1:

  - Grant-free based NOMA is supported at least for mMTC scenario

  - Grant-free based NOMA can be considered for eMBB with small data transmission

- Proposal 2: The contents of resource pool including the time/frequency resource, candidate signatures and the DM-RS for the grant-free should be further considered for grant-free transmission

- Proposal 3: NOMA SI should assume UL synchronization  is maintained by certain manners

  - Further study the exact manner to maintain the UL synchronization

- Proposal 4: Support of HARQ can be considered

  - How to support the UE identification and differentiation of the initial transmission and retransmission needs further investigation

- Proposal 5: Functionalities to achieve link adaptation should be considered for UL grant free transmission

  - The supported set of MCS levels needs further study

  - The detailed design for MCS notification needs further study

- Proposal 6: Further investigate solution to control the signature collision and DM-RS collision

  - Joint selection of signature and DM-RS for the collision reduction can be considered

Decision: The document is noted.

Tuesday's wrap-up: Continue discussion offline – Li (ZTE)

R1-1803617	Procedures related to NOMA	ZTE, Sanechips

R1-1803669	Discussion on NoMA related procedures	Huawei, HiSilicon

R1-1803772	Discussion on NOMA procedures	CATT

R1-1803946	Considerations on NOMA related procedures	Fujitsu

R1-1804117	Considerations on NOMA related procedures	NEC

R1-1804229	Considerations on NOMA procedures	Spreadtrum Communications

R1-1804464	Procedures to be considered for NOMA operation	Nokia, Nokia Shanghai Bell

R1-1804575	Discussion on procedures related to NOMA	LG Electronics

R1-1804646	Discussion on procedures related to NOMA	Google, Inc.

R1-1804745	NOMA related procedure	Intel Corporation

R1-1804825	Procedures related to NOMA	Qualcomm Incorporated

R1-1804861	On Procedures Related to Non-orthogonal Multiple Access 	InterDigital, Inc.

R1-1805005	Procedures related to NoMA	Ericsson

R1-1805151	Considerations of DMRS design with large number of UEs	ETRI

R1-1805255	Consideration on a basic receiver for NOMA schemes	CATR

### Link and system level performance evaluation for NOMA

R1-1803773	System evaluation assumptions for NOMA	CATT

Decision: The document is noted.

R1-1804108	Discussion on SLS Evaluation Methodology and Assumptions for R15 NoMA	CMCC

- Proposal 1: The curve of packet drop rate (PDR) vs packet arrival rate (PAR) and the curve of resource utilization (RU) v.s. PAR for OMA should be used as the evaluation metrics in mMTC scenario. The RU is defined as “RU= Number of RB per cell used by traffic during observation time / Total number of RB per cell available for traffic over observation time”, where one RB allocated to one (e.g., SU-MIMO) or more (e.g., MU-MIMO or NOMA) users within a cell is counted as used only one time, which is different from the definition in TR36.814.

- Proposal 2: The curve of the reliability which can be achieved by 95% UEs v.s. PAR and the curve of resource utilization (RU) v.s. PAR for OMA should be used as the evaluation metrics in URLLC scenarios.

- Proposal 3: For grant-based eMBB case, user perceived throughput (UPT) should be taken as the evaluation metric.

- Proposal 4: For grant-free eMBB case, the same evaluation metrics as in mMTC could be used, which are the curve of PDR vs PAR and the curve of RU vs PAR for OMA.

- Proposal 5: BS antenna configurations with 16 ports for URLLC and eMBB scenarios should be considered in SLS assumptions for NoMA performance evaluation.

Decision: The document is noted.

R1-1804465	Considerations on NOMA evaluation	Nokia, Nokia Shanghai Bell

- Proposal 1: The NOMA evaluation should in the first place focus on UL grant-free for each usage scenario. If NOMA for grant-based is evaluated, the baseline scheme for comparison should be UL MU-MIMO.

- Proposal 2: Scalable spreading factors need to be evaluated for spreading based scheme.

- Proposal 3: The evaluation should consider same number of UEs occupying same number of total resources, and use same TBS for evaluation of different schemes. Based on these, two methods could be considered

  - Method 1: same MCS for each case with per-UE occupied different number of PRBs

  - Method 2: different MCS for each case with same per-UE occupied number of PRBs.

- Proposal 4: DMRS details need to be reported in the evaluation.

Decision: The document is noted.

R1-1805209	Considerations on NoMA Performance Evaluation	Vodafone Group Plc

Proposal 1: NoMA SI should not target any Low Power Wide Area (LPWA) use cases and shall focus on only “normal coverage” use cases which corresponds to UE spectral efficiency values greater than 0.1 bps/Hz for establishing the feasibility of NoMA.

Proposal 2: To evaluate the system level gain of NoMA over existing NR, the baseline reference system configuration shall include MU-MIMO (for grant-based operation) and the existing grant-free operation. This baseline configuration should be mandatory for SLS.

Proposal 3: In terms of the antenna configurations included for the evaluated deployment scenarios, the following baseline is proposed:

- for 700MHz carrier frequency: 1 UE Tx and 2 BS Rx antennas

- for 4GHz carrier frequency: 1 UE Tx and 16 BS Rx for 4GHz

Proposal 4:  The following should be agreed upon as an input to system level simulations:

- Traffic generation: UE transmit power control modelling

  - Non-full-buffer traffic model should be the focus of the NoMA SI, with full buffer traffic model being used as a baseline for eMBB.

- How resource allocation is modelled for scheduled and grant-free operation

- NoMA performance gains should be evaluated as a function of packet size variation

Proposal 5: The output metrics of total latency and throughput for system-level simulations should include cumulative distribution functions (CDFs) of average throughput and average latency for each user in the system, derived for both NoMA and the baseline performance, instead of just a single average throughput and latency value averaged across all users in the system.

Proposal 6: Companies should focus on a uniform spatial distribution of UE drops within a cell when running system-level simulations.

Decision: The document is noted.

Tuesday's wrap-up: Continue discussion offline – Li (ZTE)

Thursday

R1-1805681	Summary of offline email discussion on SLS assumptions	ZTE, Sanechips

Decision: The document is noted.

Agreements:

- Adopt the parameters in the following table for system-level evaluations of NOMA study

Table I: System-level evaluation assumptions

| Parameters | mMTC | URLLC | eMBB | Further specified values |
| --- | --- | --- | --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |  |
| Inter-BS distance | [1732]m | [500m] | 200m |  |
| Carrier frequency | 700MHz | [4GHz. 700MHz] | 4GHz |  |
| Simulation bandwidth | [6] PRBs | 12 PRBs | 12 PRBs |  |
| Number of UEs per cell | Companies report |  |  |  |
| Channel model | UMa in TR 38.901 |  |  |  |
| UE Tx power | Max 23 dBm |  |  |  |
| BS antenna configurations | 2 Rx or 4 Rx for 700MHz; 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), 2 TXRU; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; dH = dV = 0.5λ; BS antenna downtilt: companies to report, FFS a single value 4 Rx or 16 Rx for 4GHz; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; 16 ports: (M, N, P, Mg, Ng) = (10, 8, 2, 1, 1), 16 TXRU; dH = 0.5λ, dV = 0.8λ; BS antenna downtilt: companies to report, FFS a single value |  |  |  |
| BS antenna height | 25m |  |  |  |
| BS antenna element gain + connector loss | 8 dBi, including 3dB cable loss |  |  |  |
| BS receiver noise figure | 5dB |  |  |  |
| UE antenna configuration | 1Tx as starting point |  |  |  |
| UE antenna height | Follow the modelling of TR 38.901 |  |  |  |
| UE antenna gain | 0dBi as starting point |  |  |  |
| UE distribution | For mMTC: [20%] of users are outdoors (3km/h), [80%] of users are indoor (3km/h); Users dropped uniformly in entire cell Companies are encouraged to check whether the percentage of UEs whose CL > 144 dB is significant (e.g., 5%) and the CDF of the CL. Further discuss the percentage of outdoor UEs, to be finalized in May meeting. For URLLC [20%] of users are outdoors (3km/h), [80%] of users are indoor (3km/h); Users dropped uniformly in entire cell For eMBB 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell |  |  |  |
| UE power control | Open loop PC for mMTC. Companies report the PC mechanisms used for eMBB and URLLC. |  |  |  |
| HARQ/repetition | Companies report (including HARQ mechanisms). |  |  |  |
| Channel estimation | Realistic |  |  |  |
| BS receiver | Advanced receiver, with baseline scheme is MU-MIMO (e.g., has the capability of spatial differentiation) Companies to provide analysis of complexity between baseline vs. advanced receivers |  |  |  |
| Packet dropping criterion |  |  |  |  |

Note: other values can be considered.

Agreements:

- The traffic model below is used for NOMA evaluations in mMTC scenario:

  - Packet arrival per UE: Poisson arrival with arrival rate λ;

  - Packet size: 20~200 bytes Pareto + higher layer protocol overhead of [29] bytes, as defined in TR 45.820 to be the starting point

    - Other packet sizes are not precluded.

- The traffic model for NOMA evaluations in URLLC scenario is to be decided in May meeting.

- The traffic model for NOMA evaluations in eMBB scenario is to be decided in May meeting.

Agreements:

- Adopt the following performance metrics for NOMA study from system level point of view.

For mMTC

- Focus on normal coverage.

- The performance metrics for mMTC include the following:

  - Higher layer packet drop rate (PDR) vs. offered load. The definition of PDR is FFS:

    - Offered load can be at least

      - Higher layer packet arrival rate (PAR) per cell for massive connectivity

  - CDF of packet drop rate per UE is optional.

  - CDF of transmission latency is optional.

  - CDF of the inter-cell interference-over-thermal (IOT) is optional.

- Note: companies are encouraged to provide the curve of resource utilization (RU) vs. offered load.

For URLLC

- The baseline for performance comparison is UL transmission without dynamic link adaptation (i.e., using configured grant type 1 or type 2)

- The performance metrics for URLLC include at least the following:

- Percentage of users satisfying reliability and latency requirements vs. packet arrival rate (PAR).

  - CDF of reliability per UE is optional.

  - CDF of the inter-cell interference-over-thermal (IOT) is optional.

    - Note: companies are encouraged to provide the curve of resource utilization (RU) vs. PAR.

For eMBB

- The performance metrics for eMBB include the following:

  - Metric 1: Higher layer packet drop rate (PDR) vs. offered load. The definition of PDR is FFS:

    - Offered load can be at least

      - Higher layer packet arrival rate (PAR) per cell

    - CDF of packet drop rate per UE is optional.

    - CDF of transmission latency is optional.

    - CDF of the inter-cell interference-over-thermal (IOT) is optional.

    - Note: companies are encouraged to provide the curve of resource utilization (RU) vs. offered load.

  - Metric 2: UPT vs. offered load.

    - CDF of the inter-cell interference-over-thermal (IOT) is optional.

    - CDF of UE perceived throughput is optional

  - FFS whether or not to have signalling overhead as one performance metric

R1-1805682	Summary of offline discussion on LLS clarifications	ZTE, Sanechips

Decision: The document is noted.

Agreements: Further clarify the LLS parameters:

- For ideal channel estimation, DMRS overhead is 1/7 for #OS 7 and 14, and 1/4 for #OS 4.

- For a=[3], companies are encouraged to check RAN4 power control rerquirements  aim to conclude in RAN1#93

- FFS timing offset for grant-free without perfect TA,

- FFS frequency offset

R1-1803618	System-level evaluation methodology and assumptions for NOMA	ZTE, Sanechips

R1-1803668	Discussion on LLS and SLS evaluation metrics and parameters	Huawei, HiSilicon

R1-1803774	Link level evaluation results of PDMA	CATT

R1-1803853	Evaluations for NOMA	vivo

R1-1804169	ACMA-OFDM with NR LDPC Code for Uplink NOMA	HUGHES Network Systems Ltd

R1-1804170	Low PAPR ACMA (SC-FDE) for Uplink NOMA	HUGHES Network Systems Ltd

R1-1804171	ACMA-OFDM for Uplink NOMA 	HUGHES Network Systems Ltd

R1-1804399	Considerations on evaluation for NoMA study	Samsung

R1-1804576	Initial LLS evaluation for NCMA	LG Electronics

Late submission

R1-1804670	Views On NOMA procedures	AT&T

R1-1805528	Initial LLS results and SLS evaluation assumptions	Intel Corporation	(rev of R1-1804746)

R1-1804826	Link and system level  evaluation for NOMA	Qualcomm Incorporated

R1-1804862	IDMA Link Level Simulation Results 	InterDigital, Inc.

R1-1805006	Evaluation methodology and metrics for NoMA	Ericsson

R1-1805068	Discussion on comparison methodology and system-level evaluations for NOMA	NTT DOCOMO, INC.

R1-1805152	Initial LLS results for low-rate code and signature based shared access NOMA scheme	ETRI

R1-1805256	Preliminary link level simulation results of non-orthogonal multiple-access schemes	CATR

### Other

R1-1803619	Channel estimation error modeling for NOMA receiver	ZTE, Sanechips

R1-1803665	Preliminary LLS evaluation results for mMTC scenario	Huawei, HiSilicon

R1-1803666	Preliminary LLS evaluation results for URLLC scenario	Huawei, HiSilicon

R1-1803667	Preliminary LLS evaluation results for eMBB scenario	Huawei, HiSilicon

R1-1804466	Consideration on NOMA study	Nokia, Nokia Shanghai Bell

R1-1804468	Initial link level simulation results for NOCA	Nokia, Nokia Shanghai Bell

R1-1804487	Coding schemes for NOMA in mMTC	Google Inc.

R1-1804863	Discussion on System Level NOMA Evaluation 	InterDigital, Inc.

R1-1804864	On Operations for Non-orthogonal Multiple Access 	InterDigital, Inc.

R1-1805007	Observations on NOMA traffic models	Ericsson

R1-1805535	Summary of offline email discussion on Tx side processing and receiver types for clarification	ZTE, Sanechips

R1-1805588	Summary of offline email discussion on system-level evaluation methodology and assumptions for NOMA	ZTE, Sanechips

## Study on evaluation methodology of new V2X use cases - SID in RP-170837

R1-1804589	Draft TR 37.885 v0.1.0 Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR	LG Electronics Inc.

Decision: The document is endorsed.

R1-1805581	Summary of offline discussion on eV2X evaluation methodology	LG Electronics

Decision: The document is noted.

Proposals:

- Three vehicle types are defined as follows.

  - Type 1 (passenger vehicle with lower antenna position): length 5 meters, width X meters, height 1.6 meters, antenna height Y meters

    - X & Y FFS

  - Type 2 (passenger vehicle with higher antenna position): length 5 meters, width 2.6 meters, height 1.6 meters, antenna height 1.6 meters

  - Type 3 (truck): length 13 meters, width 2.6 meters, height 3 meters, antenna height 3 meters

- The difference of the vehicle type does not change the channel model potentially except the following aspects:

  - Pathloss equation where the antenna height is set according to the vehicle type

  - Loss caused by vehicle blockage (details to be discussed in the vehicle blockage modeling)

  - Radiation pattern

Agreements:

- At least one “clustered UE dropping model” is defined.

  - A cluster consists of a number vehicle UEs located in the same lane and having the same direction/speed. Two closest UEs belonging to the same cluster are separated with a fixed distance and no other UEs can be located between them.

  - The distance between a platoon and a vehicle not belonging to the platoon follows the statistics of the distance between two vehicles not belonging to any platoon.

  - Only Type 3 vehicles form a cluster.

  - Clustered UE dropping is used only in Freeway scenario.

Proposals:

- Adopt the following metric for persistent collision

  - Packet Inter-Reception (PIR)

    - Time elapsed between two successive successful receptions of two different packets transmitted from node A to node B for the same application. Results collected in an area of interest are to be used in a form of [CDF].

      - FFS the area of interest

Continue offline discussion – Hanbyul (LGE)

Thursday

R1-1805671	Summary of offline discussion on eV2X evaluation methodology	LG Electronics

Decision: The document is noted.

### Evaluation scenarios

Including scenarios, performance metric, vehicle dropping, traffic model

R1-1803775	Discusion on evaluation scenarios of new V2X use cases	CATT

- Proposal 1: It is necessary to include mixed scenarios in new V2X use cases evaluation.

- Proposal 2: From the perspective of generation and generation interval, the following two traffic models are proposed to be evaluated:

  - Messages are periodically generated and the message generation interval is fixed like the Release14 periodic traffic.

  - Message generation starts when certain conditions such as time/location/probability are met, periodic or non-periodic transmission for a certain duration (details are FFS).

- Proposal 3: Message size of a traffic model shall be dependent of the use cases for evaluation accordingly and needs further study.

- Proposal 4: Platoon-type dropping is proposed to add to UE dropping model.

- Proposal 5: UEs are dropped on the roads according to the Poisson process as Release 14.

Decision: The document is noted.

R1-1804638	Simulation scenarios and performance metrics for V2X	Ericsson

Decision: The document is noted.

R1-1805590	Evaluation scenarios for NR V2X use cases	Qualcomm Incorporated	(rev of R1-1804827)

Decision: The document is noted.

Thursday

Agreements:

- Three vehicle types are defined as follows.

  - Type 1 (passenger vehicle with lower antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 0.75 meters

  - Type 2 (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters

  - Type 3 (truck): length 13 meters, width 2.6 meters, height 3 meters, antenna height 3 meters

  - FFS how to drop different vehicle types

- The difference of the vehicle type does not change the channel model potentially except the following aspects:

  - Pathloss equation where the antenna height is set according to the vehicle type

  - Loss caused by vehicle blockage (details to be discussed in the vehicle blockage modeling)

  - Radiation pattern

Agreements:

- Vehicles are dropped according to the following process.

  - The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {1 meter, an exponential random variable with the average of the speed * x sec}.

    - FFS for x sec.

- All the vehicles in the same lane have the same speed.

- The following options are supported for freeway:

  - Option A

    - Homogeneous vehicle types: 100% vehicle type 2

    - Non-clustered dropping

    - Same vehicle density in all the directions: Speed is [140 and/or 70] km/h in all the lanes.

  - Option B

    - Heterogeneous vehicle types: [20]% vehicle type 1, [60]% vehicle type 2, [20]% vehicle type 3

    - Non-clustered dropping

    - Different vehicle density in different lanes:

      - Speed in Lane 1: 80km/h

      - Speed in Lane 2: 100km/h

      - Speed in Lane 3: 140km/h

      - Speed in Lane 4: 40km/h

      - Speed in Lane 5: 30km/h

      - Speed in Lane 6: 20km/h

  - Option C

    - Heterogeneous vehicle types: 0% vehicle type 1, [67]% vehicle type 2, [33]% vehicle type 3

    - Clustered dropping: Each cluster consists of [6] Type 3 vehicles with a gap of [2] meters

      - FFS how to drop multiple clusters

    - Same vehicle density in all the directions: Speed is [140] km/h in all the lanes.

Agreements:

- The following options are supported for urban case:

  - Option A

    - Homogeneous vehicle types: 100% vehicle type 2

    - Non-clustered dropping

    - Same vehicle density in all the directions: Speed is [60 and/or 15] km/h in all the lanes.

    - In the intersection, a UE goes straight, turns left, turns right with the probability of 0.5, 0.25, 0.25, respectively.

  - Option B

    - Heterogeneous vehicle types: [20]%, [60]%, [20]% for vehicles types 1, 2, 3, respectively

    - Non-clustered dropping

    - Different vehicle density in different directions:

      - In the East-West direction:

        - Speed in Lane 1: 60km/h

        - Speed in Lane 2: 50km/h

        - Speed in Lane 3: 25km/h

        - Speed in Lane 4: 15km/h

      - In the North-South direction:

        - 0 km/h in all the lanes.

    - FFS how to handle the vehicle dropping and direction change at the intersection.

    - FFS whether to consider a reduced layout (e.g., covering a single intersection)

Agreements:

- Adopt the following metric for persistent collision

  - Packet Inter-Reception (PIR)

    - Time elapsed between two successive successful receptions of two different packets transmitted from node A to node B for the same application.

      - FFS how to collect results of PIR

Agreements:

- “Alt. 2” (agreed in [85-15] and RAN1#86) is supported and can be used as performance metric in scenarios such as multicast/unicast or to see the performance of links in a certain condition (e.g., links blocked by a building). When used, the company needs to clarify how the intended set of receivers is determined and what the motivation is.

  - i.e., Alt. 2: (1-Y) is the packet reception ratio calculated on a subset of UEs:

    - For one Tx packet, 1-Y is calculated by S/Z, where Z is the number of UEs in the intended set of receivers, and S is the number of UE with successful reception among Z.

      - Unicast is the special case where Z includes a single UE, where the PRR is average of packets of the unicast link

Agreements:

- Two options are supported as follows:

  - Periodic traffic based on Option 1

    - FFS on which option(s) is(are) supported:

      - Message size varies in time in a deterministic manner.

      - Message size varies in time in a random manner.

  - Aperiodic Traffic based on Option 3

    - Working assumption: Inter-packet arrival time = a non-negative constant value + a random variable following an exponential distribution

    - Message size varies in time in a random manner.

  - Other options are not precluded if a relevant use case is identified.

- Further discussion till next meeting whether both options have equal priority or one of them has a higher priority

R1-1803672	On eV2X Evaluation Methodology	Huawei, HiSilicon

R1-1803854	Discussion on eV2X evaluation scenarios	vivo

R1-1803897	NR eV2X evaluation methodology	ZTE, Sanechips

R1-1804022	Considerations on NR-eV2X evaluation scenarios	OPPO

R1-1804262	Evaluation scenarios of new V2X use cases	Nokia, Nokia Shanghai Bell

R1-1804400	Evaluation scenarios for NR V2X	Samsung

R1-1804577	Discussion on evaluation scenarios for new V2X use cases	LG Electronics

R1-1804596	Discussion on remaining issues of eV2X evaluation methodology	Sony

R1-1804671	Evaluation scenario and simulation assumptions	AT&T

R1-1804747	On scenarios, traffic models and performance metrics for eV2X evaluations	Intel Corporation

R1-1804865	On V2X Evaluation Methodology 	InterDigital, Inc.

R1-1805069	Evaluation assumption for V2X phase 3	NTT DOCOMO, INC.

R1-1805257	V2X NR evaluation criterions	OnStar

Late submission

R1-1805258	V2X NR evaluation scenarios	OnStar

Late submission

### Channel Model

R1-1805567	Three state V2X sidelink channel model	Huawei, HiSilicon, Spirent Communications, Panasonic, Cohere Technologies, ITRI, Intel, InterDigital, CATT, NEC, Keysight Technologies, Qualcomm, ZTE, Sanechips, LGE, Fraunhofer HHI, General Motors, Nokia, Nokia Shanghai Bell, AT&T, Sony, Vodafone	(rev of R1-1803670)

- Proposal : At least for above 6 GHz, the V2V sidelink channel is modeled according to the following three states:

  - LOS

  - NLOS: LOS path blocked by static objects (e.g., building, tree)

  - NLOSv: LOS path blocked by mobile objects (i.e., vehicle)

- FFS details including at least the following aspects:

  - How to determine the different states (e.g., statistically, deterministically),

  - Whether the blockage loss is dependent on the blocker size, number, and/or the distance between Tx, Rx, blocker, etc.

  - Small and large scale parameters for the different state

Decision: The document is noted.

R1-1804578	Discussion on channel model for new V2X use cases	LG Electronics

Decision: The document is noted.

R1-1804748	On channel models for eV2X evaluation methodology	Intel Corporation

Discussion: Huawei: Proposal 5 is already part of R1-1805567 – others required more discussion

Decision: The document is noted.

Thursday

Agreements:

- At least for above 6 GHz, the V2V sidelink channel is modeled according to the following three states:

  - LOS

  - NLOS: LOS path blocked by buildings

  - NLOSv: LOS path blocked by vehicles

Agreements:

- The following pathloss equation applies to both below and above 6 GHz

| LOS/NLOS | Pathloss [dB] |
| --- | --- |
| LOS | For Freeway case, PL = 32.4 + 20log10(d) + 20log10(fc) (fc is in GHz and d is in meters) For Urban case, PL= 38.77 + 16.7log10(d) + 18.2log10(fc) (fc is in GHz and d is in meters) |
| NLOS | FFS including Option 1: where and Option 2: NLOS pathloss equation in R1-1803671 Other options are not precluded. |

- LOS pathloss equation is used when the two vehicles are in the same street.

  - Additional vehicle blockage loss is added the link between the two vehicles is blocked by other vehicles.

- NLOS pathloss equation is used when the two vehicles are in different streets.

  - No vehicle blockage is considered in this case.

Agreements:

- A link between two vehicles is considered blocked (i.e., in NLOSv)

  - Option 1: if a line connecting the antennas of the two vehicles in the same street intersects any vehicle (including either of the two vehicles) in the 3-dimensional space.

  - Option 2: with a probability

    - Distribution is FFS. Note that vehicle width and height will not affect this option once the probability is defined.

- In NLOSv , a random variable is added to the pathloss equation.

  - FFS the distribution of the random variable

- Vehicle blockage is applied to below 6 GHz.

Agreements:

- The link state (LOS, NLOS, or NLOSv) is updated for each link during the SLS runtime.

  - FFS whether the state is updated by Option 1 or Option 2 w.r.t the link between two vehicles is considered blocked (i.e., in NLOSv)

- At each state, each link uses pathloss, shadowing, and fast fading parameters corresponding to the state.

  - FFS how to model smooth transition between different states

- UE location is updated every [100] ms.

Agreements:

- Use LOS propagation type for V2B and B2R links in Freeway scenario

- Use LOS/NLOS propagation types for V2B, P2B and B2R links in Urban scenario and maintain spatial consistency following procedure in Subclause 7.6.3.3 of the 3GPP TR 38.901

  - Derive propagation type based on probability formula

Pathloss models of V2B, P2B, B2R for Freeway and Urban scenarios

|  | Below 6 GHz |  | Above 6 GHz |  |
| --- | --- | --- | --- | --- |
|  | LOS | NLOS | LOS | NLOS |
| B2V B2P B2R | Urban: TR 38.901 UMa LOS Freeway: TR 38.901 RMa LOS | Urban: TR 38.901 UMa NLOS Freeway: N/A | Urban: TR 38.901 UMa LOS Freeway: FFS | Urban: TR 38.901 UMa NLOS Freeway: N/A |

Agreements:

Large scale parameters of fast fading for V2B, P2B, B2R

|  | B2V/B2P/B2R |  |
| --- | --- | --- |
|  | LOS | NLOS |
| Freeway | TR 38.901 RMa LOS | N/A |
| Urban | TR 38.901 UMa LOS | TR 38.901 UMa NLOS |

Agreements:

- For sidelink in Urban and freeway, the fast fading parameters of “UMi-Street Canyon in TR 38.901” is the starting point and to be modified considering sidelink characteristics.

  - FFS fading parameters in Urban and Freeway

| - Scenarios |  | UMi - Street Canyon |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | NLOSv |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | [-0.24 log10(1+ fc) - 7.14] | [-0.24 log10(1+ fc) - 6.83] | FFS |
|  | lgDS | [0.38] | [0.16 log10(1+ fc) + 0.28] | FFS |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | [-0.05 log10(1+ fc) + 1.21] | [-0.23 log10(1+ fc) + 1.53] | FFS |
|  | lgASD | [0.41] | [0.11 log10(1+ fc) + 0.33] | FFS |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | [-0.08 log10(1+ fc) + 1.73] | [-0.08 log10(1+ fc) + 1.81] | FFS |
|  | lgASA | [0.014 log10(1+ fc) + 0.28] | [0.05 log10(1+ fc) + 0.3] | FFS |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | [-0.1 log10(1+ fc) + 0.73] | [-0.04 log10(1+ fc) + 0.92] | FFS |
|  | lgZSA | [-0.04 log10(1+ fc) + 0.34] | [-0.07 log10(1+ fc) + 0.41] | FFS |
| Shadow fading (SF) [dB] | SF | [4] | [7.82] | FFS |
| K-factor (K) [dB] | K | [9] | N/A | FFS |
|  | K | [5] | N/A | FFS |
| Cross-Correlations | ASD vs DS | [0.5] | [0] | FFS |
|  | ASA vs DS | [0.8] | [0.4] | FFS |
|  | ASA vs SF | [-0.4] | [-0.4] | FFS |
|  | ASD vs SF | [-0.5] | [0] | FFS |
|  | DS vs SF | [-0.4] | [-0.7] | FFS |
|  | ASD vs ASA | [0.4] | [0] | FFS |
|  | ASD vs  | [-0.2] | N/A | FFS |
|  | ASA vs  | [-0.3] | N/A | FFS |
|  | DS vs  | [-0.7] | N/A | FFS |
|  | SF vs  | [0.5] | N/A | FFS |
| Cross-Correlations 1) | ZSD vs SF | [0] | [0] | FFS |
|  | ZSA vs SF | [0] | [0] | FFS |
|  | ZSD vs K | [0] | N/A | FFS |
|  | ZSA vs K | [0] | N/A | FFS |
|  | ZSD vs DS | [0] | [-0.5] | FFS |
|  | ZSA vs DS | [0.2] | [0] | FFS |
|  | ZSD vs ASD | [0.5] | [0.5] | FFS |
|  | ZSA vs ASD | [0.3] | [0.5] | FFS |
|  | ZSD vs ASA | [0] | [0] | FFS |
|  | ZSA vs ASA | [0] | [0.2] | FFS |
|  | ZSD vs ZSA | [0] | [0] | FFS |
| Delay scaling parameter r |  | [3] | [2.1] | FFS |
| XPR [dB] | XPR | [9] | [8.0] | FFS |
|  | XPR | [3] | [3] | FFS |
| Number of clusters |  | [12] | [19] | FFS |
| Number of rays per cluster |  | [20] | [20] | FFS |

| Scenarios |  | LOS | NLOS | NLOSv |
| --- | --- | --- | --- | --- |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1) | lgZSD | [max[-0.21, -14.8(d2D/1000) + 0.01¦hUT-hBS¦ + 0.83]] | [max[-0.5, -3.1(d2D/1000) + 0.01 max(hUT-hBS,0) +0.2]] | FFS |
|  | lgZSD | [0.35] | [0.35] | FFS |
| ZOD offset | µoffset,ZOD | [0] | [-10^{-1.5log10(max(10, d2D))+3.3}] | FFS |

Agreements:

- Rel-14 dual mobility is modified such that a random Doppler shift is added to each reflected path.

  - FFS details of the additional random Doppler shift

Agreements:

- gNB antenna element pattern and array configuration follows TR 38.901.

  - FFS which gNB configuration is used in which cases

Agreements:

Antenna element pattern for UE-Type RSU

| Parameter | Below 6 GHz | Above 6 GHz |
| --- | --- | --- |
| Antenna element gain vertical pattern | FFS | FFS |
| Antenna element gain horizontal pattern |  |  |
| Pattern combining method for 3D | FFS | FFS |
| Max direct. gain of the antenna element | FFS | FFS |

Antenna array configuration for UE-Type RSU

|  | Below 6 GHz | Above 6 GHz |
| --- | --- | --- |
| TXRU mapping | FFS | FFS |
| Number of antenna elements across all panels | FFS | FFS |
| Antenna array configuration (M, N, P, Mg, Ng) | FFS | FFS |
| Antenna array spacing (dH,dV,dH,g,dV,g) | FFS | FFS |
| Antenna tilt, deg | FFS | FFS |

Agreements:

Antenna element pattern for pedestrian UE

|  | Pedestrian UE |  |
| --- | --- | --- |
|  | For 6 GHz | For 63 GHz (according to the UE antenna radiation pattern parameters specified in Table A.2.1-8 of TR 38.802) |
| Antenna element gain vertical pattern | Omni-directional |  |
| Antenna element gain horizontal pattern |  |  |
| Pattern combining method for 3D |  |  |
| Max direct. gain of the antenna element | 0 dBi | 5 dBi |

Antenna array configuration for pedestrian UE

|  | Pedestrian UE |  |
| --- | --- | --- |
|  | For 6 GHz | For 63 GHz |
| TXRU mapping | Up to proponents decision | Up to proponents decision |
| Number of antenna elements across all panels | Up to 8 Tx /Rx antenna elements | Up to 32 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | FFS TR 38.802, Table A.1.6-1 | (2, 4, 2, 1, 2) Panel bearing angle: Ω0,1=Ω0,0+180° TR 38.802, Table A.1.6-2 |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.5)λ TR 38.802, Table A.1.6-1 | (dH, dV) = (0.5, 0.5)λ; (dH,g, dV,g) = (0, 0)λ TR 38.802, Table A.1.6-2 |
| Antenna tilt, deg | 0 | 90 |

Note: Antenna pattern can be different in different carrier frequencies.

Agreements:

Antenna element pattern for vehicle UE

|  | Vehicle UE |  |
| --- | --- | --- |
|  | For 6 GHz | For 63 GHz |
| Antenna element gain vertical pattern | FFS | FFS |
| Antenna element gain horizontal pattern | FFS | FFS |
| Pattern combining method for 3D | FFS | FFS |
| Max direct. gain of the antenna element | FFS | FFS |

Antenna array configuration for vehicle UE

|  | Vehicle UE |  |
| --- | --- | --- |
|  | For 6 GHz | For 63 GHz |
| TXRU mapping | FFS | FFS |
| Number of antenna elements across all panels | FFS | FFS |
| Antenna array configuration (M, N, P, Mg, Ng) | FFS | FFS |
| Antenna array spacing (dH,dV,dH,g,dV,g) | FFS | FFS |
| Antenna tilt, deg | FFS | FFS |

R1-1803776	Discussion on channel models of new V2X use cases	CATT

R1-1803898	Consideration of Blockage in eV2X Channel Modeling	ZTE, Sanechips

R1-1804023	On some remaining issues in channel modelling	OPPO

R1-1804263	V2X channel modelling and other issues	Nokia, Nokia Shanghai Bell

R1-1804401	Channel model for NR V2X	Samsung

R1-1804639	On channel models for eV2X evaluation	Ericsson

R1-1804672	Channel model for V2X	AT&T

R1-1804866	V2X Sidelink Channel Model with Vehicle Blockage 	InterDigital, Inc.

R1-1805070	Channel model for V2X phase 3	NTT DOCOMO, INC.

R1-1805077	On channel models for eV2X	Cohere Technologies

R1-1805259	V2X NR channel model	OnStar

Late submission

R1-1805659	Modelling of the Doppler effect in V2V sidelink channel	Huawei, HiSilicon

### Other

R1-1804673	Integrated access and sidelink: A unified concept for V2X	AT&T

Decision: The document is noted.

R1-1803899	NR eV2X link level simulation assumptions	ZTE , Sanechips

R1-1805090	Discussion on eV2X use cases	Ericsson LM

## Study on NR-based Access to Unlicensed Spectrum - SID in RP-172021

R1-1805705	Chairman's notes of AI 7.6 SI on NR-based access to unlicensed spectrum	Ad-Hoc chair (Ericsson)

Decision: The document is endorsed, content incorporated below.

R1-1805677	Summary of offline discussion for NR-U	Qualcomm

Agreement:

In the discussions in the NR-U study item, references to sub-7 GHz are intended to include unlicensed bands in the 6 GHz region that are being discussed in regulatory discussions which may have some region exceeding 7 GHz (e.g., 7.125 GHz).

### Simulation Methodology for NR-U operation

R1-1803777	Remaining Issues of Simulation Methodology for NR Unlicensed operations	CATT

R1-1803918	Updated Coexistence Evaluation Assumptions	CableLabs

R1-1803948	Considerations for Simulation Methodology for NR-U Operation	ZTE, Sanechips

R1-1804062	On simulation methodology for NR-U operation	MediaTek Inc.

R1-1804109	Discussion on evaluation assumption for NR-U	CMCC

R1-1804274	Simulation methodology and assumptions for NR-U evaluation	Nokia, Nokia Shanghai Bell

R1-1804300	Simulation assumptions and preliminary results for NR unlicensed bands	Huawei, HiSilicon

R1-1804402	Simulation Methodology for NR-U	Samsung

R1-1804579	Simulation methodology for NR unlicensed operation	LG Electronics

R1-1804664	Simulation assumptions and scenarios for NR Unlicensed	AT&T

R1-1804749	On the simulation methodology for NR unlicensed	Intel Corporation

R1-1804828	Simulation methodology for NR unlicensed	Qualcomm Incorporated

R1-1804867	Consideration of NR-U Simulation Evaluation on 60 GHz Channels 	InterDigital, Inc.

R1-1805008	NR-U evaluation methodology	Ericsson

R1-1805071	Simulation Methodology for NR-U operation	NTT DOCOMO, INC.

R1-1805093	Discussion on network topology for sub-7GHz coexistence evaluations between NR-U and Wi-Fi 	Broadcom, Cisco, HPE

R1-1805555	Discussion on network topology for sub-7GHz coexistence evaluations between NR-U and Wi-Fi

R1-1805585	WF on Simulation Scenarios for NR-U	Qualcomm, Nokia, NSB

Agreement:

- For sub7 indoor simulation evaluation:

  - Scenario: Option 2 (3+3) with indoor mixed office model

    - Target to reach 10%-15% serving links below -72dBm

    - Further layout parameter fine tuning may be needed. An example procedure for fine tuning is the following sequence.

      - Currently a-b-a=15-20-15

      - If not reaching target, try a-b-a=15-30-15 and a-b-a=20-40-20

      - If not reaching target, apply a scaling factor to the layout with a-b-a=20-40-20

  - Other parameters: Default is NR parameters in 38.901 and 38.802 with the exception of the following

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline , 80MHz optional |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| SCS | To be reported together simulation results |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

Email discussion on further layout parameter fine tuning until May 3, 2018 (Jing, Qualcomm)

Agreement:

- For sub7 outdoor simulation evaluation:

  - Select one of the following for the Outdoor sub-7 GHz scenario

    - Alt 1: Each operator randomly drop [1 or 2] micro-layer TRPs within each macro cell with minimum dibstance between gNBs as in NR

      - Use NR dense Urban option 1 (gNB dropped at the center of the hot-spot)

      - Independent dropping between two operators

      - Use the NR current [57.9] meters intra-operator minimum distance

      - Use [10] meters as the inter-operator minimum distance

      - UE randomly dropped within [28.9] meters within the serving cell

    - Alt 2: Drop [1 or 2 or 3] hot spots as in NR urban option 1

      - Within each hot-spot, randomly drop one gNB from each operator within a circle of radius [10] meters centered at the center of the hot-spot

        - The minimum inter-gNB distance is [10] meters

      - Within each hot-spot, drop UE within [28.9] meters from the hot-spot center

  - Parameters: Use the indoor sub7 table as baseline, with further fine tunes possible

Agreement:

- For calibration for sub-7 GHz indoor and outdoor scenarios, companies should submit for the baseline scenario:

  - Cdf of received signal power from serving cell

  - Optional: Cdf of received signal power from each of the all non-serving cells (including the cells from the other operator)

### Frame structure for NR-U operation

R1-1803678	NR frame structure and scheduling on unlicensed bands	Huawei, HiSilicon

R1-1803778	Frame Structure for NR Unlicencensed Operations	CATT

R1-1803855	Frame structure for NR-U operation	vivo

R1-1803947	Flexible starting transmission point for NR-U	Fujitsu

R1-1803949	Discussion on frame structure and scheduling for NR-U	ZTE, Sanechips

R1-1803973	Frame structure for NR-U	OPPO

R1-1804063	On frame structure design for NR-U operation	MediaTek Inc.

R1-1804275	Frame structure for NR-U operation	Nokia, Nokia Shanghai Bell

R1-1804403	Frame structure for NR-U	Samsung

R1-1804580	Frame structure for NR unlicensed operation	LG Electronics

R1-1804592	Frame structure and numerology for NR-U	ITL

R1-1804598	Considerations on frame structure for NR unlicensed operations	Sony

R1-1804750	On the flexibility of NR to operate in unlicensed spectrum	Intel Corporation

R1-1804766	Frame Structure Design for NR-U	Charter Communications

R1-1804829	Frame structure for NR-U operation	Qualcomm Incorporated

R1-1804868	On Frame Structure for NR-U Operation 	InterDigital, Inc.

R1-1805009	On Frame Structure for NR in Unlicensed Spectrum	Ericsson

R1-1805253	Frame structure design in NR-U	CATR

R1-1805586	WF on Frame Structure for NR-U	Qualcomm, Ericsson

Agreement:

- NR-U supports both Type-A and Type-B mapping already supported in NR

  - Additional starting positions and durations are not precluded

- For sub-7 GHz, NR-U study the SCSs, 15/30/60KHz

  - Study performance difference between different SCS

  - Study if changes to UL design are needed to meet the PSD and OCB requirements

  - Study if an SS block design/RMSI/OSI with 60KHz SCS is needed

  - Impact on MIB and SIB1 content

  - Need for use of ECP for 60KHz

  - RACH design with 60KHz SCS in addition to options currently part of NR

  - Other considerations are not precluded.

  - Impact on support of different BWs with different SCS

- Study supporting more than one switching points within a TxOP

  - FFS the LBT requirement for each DL/UL data/control burst in the TxOP

Proposal:

- Study FBE (as in the ETSI BRAN specifications) based frame structure

### Potential physical layer channel design

R1-1803624	Discusses on waveform and bandwidth of NR-U	NEC

R1-1803676	Design princples of physcial channels on NR unlicensed band	Huawei, HiSilicon

R1-1803779	Physical Channel Design for NR Unlicensed Operations	CATT

R1-1803856	Discussion on physical layer channel design in unlicensed spectrum	vivo

R1-1803951	Considerations on RS/channel design and measurement for NR-U	ZTE, Sanechips

R1-1803974	Views on physical layer channel design for NR-U	OPPO

R1-1804064	On physical later channel design for NR-U operation	MediaTek Inc.

R1-1804218	Discussion on SS/PBCH block for NR-U operation	Spreadtrum Communications

R1-1804276	On physical layer channel design for NR unlicensed	Nokia, Nokia Shanghai Bell

R1-1804404	Potential physical layer channel design for NR-U	Samsung

R1-1804581	Physical layer channel design for NR unlicensed operation	LG Electronics

R1-1804649	Considerations on SS/PBCH blocks for NR unlicensed	Xiaomi Technology

R1-1804665	Closed loop LBT for Licensed Assisted NR	AT&T

R1-1804751	Design consideration for NR unlicensed physical layer channels	Intel Corporation

R1-1804752	Enhancements to NR to operate in unlicensed spectrum	Intel Corporation

R1-1804768	NR Unlicensed physical channel design considerations	Apple Inc.

R1-1804830	Potential phy channel design for NR unlicensed	Qualcomm Incorporated

R1-1804869	On UL Physical Layer Channel Design for NR-U 	InterDigital, Inc.

R1-1805010	Physical layer channel design for NR unlicensed operation	Ericsson

R1-1805092	Waveform considerations for NR-U in the 60GHz band	Mitsubishi Electric RCE

R1-1805591	WF on PHY Channel Design for NR-U	Qualcomm, Ericsson

Agreements:

- Study the design changes needed to support the following channels /signals in NR-U

  - PDCCH/PDSCH

  - PUCCH/PUSCH

  - PSS/SSS/PBCH

  - PRACH

  - DL and UL reference signals applicable to the operational frequency range

### Potential physical layer procedures

R1-1803679	Coexistence and channel access for NR unlicensed band operations	Huawei, HiSilicon

R1-1803780	Physical Layer Procedures for NR Unlicensed Operations	CATT

R1-1803857	Discussion on the channel access procedures	vivo

R1-1803952	Design of Channel access mechanism for NR-U	ZTE, Sanechips

R1-1803975	Views on physical layer procedures for NR-U	OPPO

R1-1804037	Potential solutions and techniques for NR unlicensed	ASUSTEK COMPUTER (SHANGHAI)

R1-1804065	Discussion on potential physical layer procedures in NR-U	MediaTek Inc.

R1-1804209	HARQ-ACK transmission on unlicensed carriers	Lenovo, Motorola Mobility

R1-1804230	Considerations on the LBT procedures for NR unlicensed band	Spreadtrum Communications

R1-1804241	Discussion on NR unlicensed band operation	Potevio

R1-1804277	Physical layer procedure enhancements for NR unlicensed	Nokia, Nokia Shanghai Bell

R1-1804405	Potential physical layer procedures for NR-U	Samsung

R1-1804582	Physical layer procedures for NR unlicensed operation	LG Electronics

R1-1804591	Broadcast signals and synchronization for NR-U	ITL

R1-1804597	Considerations on physical layer procedures for NR unlicensed operations	Sony

R1-1804666	Physical layer enhancements for NR Unlicensed	AT&T

R1-1804753	Consideration for NR unlicensed channel access mechanism	Intel Corporation

R1-1804767	Design considerations for standalone NR-U	Charter Communications

R1-1804769	NR Unlicensed considerations	Apple Inc.

R1-1804831	Potential procedure design for NR unlicensed	Qualcomm Incorporated

R1-1804870	On Physical Layer Procedures for NR-U 	InterDigital, Inc.

R1-1805011	Physical layer procedures for NR unlicensed operation	Ericsson

R1-1805072	Potential physical layer procedures  for NR-U operation	NTT DOCOMO, INC.

R1-1805113	Channel Access Procedure for NR on Unlicensed Band	Shenzhen Coolpad Technologies

R1-1805254	Considerations on LBT in NR-based access to unlicensed spectrum	CATR

Agreement:

- Study possible enhancements for HARQ operation

- Study changes needed for Configured Grant support in NR-U

  - Baseline for study: If absence of Wi-Fi cannot be guaranteed (e.g. by regulation) in the band (sub-7 GHz) where NR-U is operating, the NR-U operating bandwidth is an integer multiple of 20MHz

- At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

  - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

- Study whether or not the following techniques enhance performance beyond the baseline LBT mechanisms

  - Techniques to cope with directional antennas/transmissions

  - Receiver assisted LBT : RTS/CTS type mechanism

    - On-demand receiver assisted LBT: For example receiver assisted LBT enabled only when needed

  - Techniques to enhance spatial reuse

  - Preamble detection

  - Enhancements to baseline LBT mechanisms above 7 GHz

- Note: LTE-LAA LBT mechanism are assumed as baseline for evaluations for 5GHz.

- Note: Other aspects are not precluded from being included

### Other

R1-1803677	Numerology and wideband operation in NR unlicensed	Huawei, HiSilicon

R1-1803680	SSB/DRS in NR unlicensed	Huawei, HiSilicon

R1-1803681	Random access in NR unlicensed	Huawei, HiSilicon

R1-1803682	UCI transmission in NR unlicensed	Huawei, HiSilicon

R1-1803858	Design considerations for standalone operation in NR unlicensed spectrum	vivo

R1-1803976	Channel access mechanisms on NR-U	OPPO

R1-1803977	SS/PBCH block transmission on NR-U	OPPO

R1-1803978	Discussion on SS/PBCH block waveform for NR-U	OPPO

R1-1803979	Paging transmission on NR-U	OPPO

R1-1803980	HARQ procedure on NR-U	OPPO

R1-1804231	Considerations on Autonomous UL transmission for NR unlicensed band	Spreadtrum Communications

R1-1804242	Discussion on initial access based on beam sweeping in NR-U	Potevio

R1-1804313	Channel access and co-existence for NR-U operation	Nokia, Nokia Shanghai Bell

R1-1804314	Interlace structures for NR-U uplink	Nokia, Nokia Shanghai Bell

R1-1804680	BWP operation in unlicensed spectrum	InterDigital, Inc.

R1-1804754	Initial thoughts foron technology neutral design for greenfield	Intel Corporation

R1-1804832	NR-U AUL	Qualcomm Incorporated

R1-1804833	Waveform options for 60GHz band	Qualcomm Incorporated

R1-1804834	Waveform under PSD limitation for NR-U	Qualcomm Incorporated

R1-1804884	On Potential Waveform Solutions for NR-U 	InterDigital, Inc.

R1-1804885	On LBT for Beam-Based Transmission for NR-U 	InterDigital, Inc.

R1-1804886	On Synchronization Signal and PBCH for NR Unlicensed Spectrum  	InterDigital, Inc.

R1-1804887	On Random Access for NR Unlicensed Spectrum  	InterDigital, Inc.

R1-1804888	On Paging for NR Unlicensed Spectrum 	InterDigital, Inc.

R1-1805012	On SRS design for NR unlicensed operation	Ericsson

R1-1805013	On the Introduction of a Discovery Reference Signal	Ericsson

R1-1805014	On NR operation in 60GHz unlicensed band	Ericsson

R1-1805015	Numerology for NR operation in unlicensed spectrum	Ericsson

R1-1805016	On Autonomous UL Transmissions for NR in Unlicensed Spectrum	Ericsson

R1-1805017	On PRACH for NR in Unlicensed Spectrum	Ericsson

R1-1805018	On interlace design for NR-U uplink	Ericsson

R1-1805247	Discussion on coexistence between NR-U and 802.11ax	Broadcom

## Study on Integrated Access and Backhaul for NR - SID in RP-172290

R1-1805275	Work plan for Integrated Access and Backhaul Study Item 	Samsung, Qualcomm, AT&T, KDDI	(rev of R1-1804473)

Decision: The document is noted.

R1-1804476	TR 38.874, Version 0.1.1	Samsung, Qualcomm, AT&T, KDDI

Decision: The document is noted. For information only.

Huawei: far from being a skeleton TR – need to be revised during the week.

### Enhancements to support NR backhaul links

R1-1804620	Enhancements to support NR backhaul links	Nokia, Nokia Shanghai Bell

- Observation 1: From RAN1 point of view, NR Rel-15 provides a very good baseline for IAB operation.

- Observation 2: Depending on the allocation scheme of BH/Access slots, there could be a shortage of DL Access slots available for IAB Node’s SS/PBCH block transmission.

- Observation 3: To enable monitoring between all IAB Nodes, IAB measurement gaps must be introduced: RX in a slot that is normally used for transmission or TX in a slot that is normally for reception.

- Observation 4: GC-PDCCH can enable interference coordination between DgNB cell and RN cell(s) with dynamic split between Access/BH in such that the network can be operated free from cross-link interference.

- Observation 5: Synchronization requirements can be met in typical IAB deployment scenario according to current knowledge about the specified TA adjustment granularity (separately for each numerology), UE TX timing error limits and TA adjustment accuracy.

- Proposal 1: The number of hops is determined by the propagation environment where the topology is constructed to maximize system spectral efficiency and minimize latency. There should be no specified limit on the number of hops, but instead, it should be governed by the propagation environment.

- Proposal 2: Prioritize half-duplex scenario with TDM between access and backhaul in 3GPP studies.

- Proposal 3: GC-PDCCH enhancements are needed to facilitate dynamic resource allocation between the backhaul and access links.

- Proposal 4: RN processing time capabilities need to be discussed as part of the IAB studies.

- Proposal 5: IAB Nodes are measuring link quality to candidate nodes besides the active IAB connection(s).

- Proposal 6: Cross-link interference management related to half-duplex operation with SDM/FDM between access and backhaul needs to be considered as part of NR studies.

- Proposal 7: Develop a unified cross-link interference management framework under single WI, e.g. flexible duplexing or MIMO, covering all potential use-cases, e.g. dynamic TDD and IAB, etc.

- Proposal 8. TA based synchronization can be taken as basis for IAB synchronization supporting also multi-hop scenarios.

Decision: The document is noted.

R1-1804662	Overview of Physical layer enhancements for IAB	AT&T

- Proposal 1: IAB should support the same physical layer signals and procedures designed for access UEs performing initial access and link management (e.g. RLM/RRM) for IAB node discovery and topology/route management.

- Proposal 2: The multiplexing of initial access signals (e.g. SSB, RACH, and CSI-RS for RRM) should consider multi-hop topologies and half-duplex constraints at IAB nodes.

- Proposal 3: The resources and respective transmission periods for signals required for initial access and RLM/RRM of backhaul link should be separately configured from the parameters used by access UEs.

- Proposal 4: RAN1 should study mechanisms for efficient multiplexing of access/backhaul traffic across multiple hops considering a per IAB-node half-duplex constraint.

- Proposal 5: Cross link interference measurement and mitigation is critical for the operation of IAB.

- Proposal 6: IAB should make use of different interference measurement categories and components to enable CLI mitigation and cancellation.

- Proposal 7: Consider to support 8 layers of PUSCH transmission in the spec.

Decision: The document is noted.

R1-1804835	Enhancements to support NR backhaul links	Qualcomm Incorporated

- Proposal 1: RAN1 should strive for a common design, as NR access, to support NR backhaul links.

- Observation 1: access and backhaul have several differences at both link-level and system-level.

- Proposal 2: RAN1 should identify the aspects of NR-IAB that require a new design or a modification of the NR access design. RAN1 should deprioritize any design optimization for NR-IAB.

- Observation 2: an IAB-node can act as a UE and follow the same initial access procedure, including cell search, SI acquisition, and random access, in order to initially integrate to an IAB network.

- Observation 3: over a BH link, an IAB-node can adopt UE functionality (UE-F) and leverage the NR access design for various control/data channels, reference signals, and procedures (e.g. RRM, RLM, beam management, mobility management).

- Proposal 3: RAN1 should further study inter-relay discovery procedure subject to half-duplex constraint.

- Proposal 4: Timing alignment across multi-hop NR-IAB network should be studied.

- Observation 4: Over-the-air (OTA) synchronization can be used to synchronize a multi-hop IAB network to the IAB-donors.

- Proposal 5: It should further be investigated if the tolerance of multi-hop OTA synchronization is acceptable.

- Proposal 6: Network should synchronize the IAB-donors using the available techniques, e.g. using GPS/GNSS, Ethernet, etc.

- Observation 5: IAB resource partitioning must be such that the UEs can continue their normal operations with no issue and no need for modifications.

  - Details of the IAB resource partitioning is outside of RAN1 scope.

Decision: The document is noted.

Wednesday's wrap-up: Discuss further offline – Thomas (AT&T)

Thursday

R1-1805673	Summary of 7.7.1 Enhancements to support NR backhaul links	AT&T

Decision: The document is noted.

Agreement:

- The Release 15 NR physical layer should be the starting point for the physical layer of the IAB backhaul link.

Agreements:

- An IAB-node can follow the same initial access procedure as an access UE, including cell search, SI acquisition, and random access, in order to connect to an IAB node/donor and initially integrate to the network.

- Two cases: (1) donor and relay node share the same cell ID and (2) donor and relay maintain separate cell ID can be further studied.

  - Note: The feasibility of (1) may depend on architectures considered in RAN2/3.

- The SSB/CSI-RS based RRM measurement defined in NR R15 are considered as a starting point for IAB node discovery and measurement.

  - How to avoid conflicting SSB configurations among IAB nodes, as well as the feasibility of CSI-RS based IAB node discovery, should be studied.

- RAN1 should further study inter-relay discovery procedure subject to half-duplex constraint and multi-hop topologies.

Agreements:

- Study the feasibility of over-the-air (OTA) synchronization and the impact of timing misalignment on IAB performance (e.g. the number of supportable hops).

- Mechanisms for timing alignment across multi-hop NR-IAB networks should be studied.

Agreements:

- Measurements on multiple backhaul links for link management and route selection should be studied.

- Mechanisms for efficient route switching or transmission/reception on multiple backhaul links simultaneously (e.g. multi-TRP operation and intra-frequency dual connectivity) should be studied.

  - Note: The feasibility of (1) may depend on architectures considered in RAN2/3.

- Mechanisms for scheduling coordination, resource allocation, and route selection across IAB nodes/donors and multiple backhaul hops should be studied.

- Mechanisms for efficient TDM/FDM/SDM multiplexing of access/backhaul traffic across multiple hops considering an IAB-node half-duplex constraint should be studied.

- The impact of cross-link interference on access and backhaul links (including across multiple hops) should be studied.

- Interference measurement and management solutions should be studied.

- 1024QAM for the backhaul link should be studied.

  - FFS: whether solutions should be specified as part of an IAB WI or other NR WI

R1-1803695	Physical layer enhancement consideration on IAB	Huawei, HiSilicon

R1-1803990	Discussion of resource allocation for IAB	OPPO

R1-1804110	Discussion on deployment scenario and requirement for IAB	CMCC

R1-1804406	Necessary Enhancements for NR IAB	Samsung

R1-1804504	Multiplexing of IAB links	ZTE, Sanechips

R1-1804505	IAB enhancements overview	ZTE, Sanechips

R1-1804583	Discussions on NR IAB support	LG Electronics

R1-1804611	NR enhancements to support integrated access and backhaul	Sony

R1-1804755	RAN1 related issues in IAB	Intel Corporation

R1-1805085	Physical-layer aspects related to IAB	Nanjing Ericsson Panda Com Ltd

### Evaluation methodology for NR IAB

Including deployment scenarios, evaluation assumptions, etc.

R1-1805274	Evaluation Methodology for NR IAB	Samsung	(rev of R1-1804407)

Proposal 1: IAB should be evaluated with the following assumptions.

- Physically fixed relays (imply that the benefit of planning should be captured in modelling)

- In-band and out-of-band scenarios

- NR access over NR backhaul

- 30GHz for NR backhaul

- Support of multiple backhaul hops

- Topology adaptation to enable robust operation, e.g., mitigate blockage and load variation on backhaul links

- Maximize the reuse of Rel-15 NR specifications

Observation 1: A potential candidate scenario to evaluate IAB as backhaul for coverage extension solution is the “dense urban” scenario with two layers and 3-sector micro TRPs. Some of the important issues to be further studied are:

- Additonal ISDs of the macro layer (e.g. 200m (dense urban), 500m (urban), 1732m (suburban))

- Sector orientation of the IAB-capable micro TRPs

Observation 2: A potential candidate scenario to evaluate IAB as backhaul for area traffic capacity solution is the urban grid model with 4-sector micro TRPs. The channel model at >6GHz for UMi street canyon particularly for links between nodes not dropped on the same street should be further studied.

Observation 3: For the backhaul links, the LOS probability determination for the large-scale channel model and AoD/AoA modelling for the small-scale channel model need further study.

Proposal 2: For the backhaul channel model, consider “bonus” to path loss formula to capture the benefit of planned deployment for rTRP, the following alternatives are to be studied:

Alt 1:  Pathloss: PL=PLLOS(R), if LOS; PLNLOS(R) - B, if NLOS; where R is the 3-D distance, FFS value for B

LOS probability: Prob(R)’ = 1-(1- Prob(R))N, FFS value for N

Alt 2: LOS probability is set to a constant (representing the target LOS probability via planning)

Decision: The document is noted.

R1-1804584	Discussions on evaluation methodology on NR IAB	LG Electronics

- Proposal 1: For the deployment scenario in IAB SLS simulation,

  - Reuse dense urban scenario in the table A.2.1-1 in TR38.802.

    - Hexagonal deployment for Macro TRP. All with fiber connection

    - RN is deployed as micro TRP

  - Simulate 2 cases:

    - Case-1: Only Macro layer

    - Case-2: Macro layer + Micro layer (using IAB for backhaul)

- Proposal 2: For the multi-hop IAB SLS simulation, 500m ISD is used in dense urban scenario.

- Proposal 3: For the relay drop, equally dropped in the simulations.

  - For the single hop IAB, 3 relays drop per sector

  - For the multi hop IAB, 8 relays drop per sector

    - To reduce the simulation burden, 7 sites can be considered.

- Proposal 4: Considering cell planning well done, the LOS probability is assumed to be ‘1’ for Macro to Micro and Micro to Micro; it is not applied for the interference link.

- Proposal 5: For the same scatterer assumptions, Macro-to-Micro channel model is modified as

  - Below 6GHz:

    - 3D UMa O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD from 3D UMi O-to-O (hUE =25m)

  - Above 6GHz:

    - 5GCM UMa O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD from UMi-Street canyon O-to-O (hUE =25m)

- Proposal 6: For the performance metric in IAB SLS,

  - Outage probability

  - User perceived throughput

Decision: The document is noted.

R1-1805086	IAB Simulation Evaluation Methodology and KPIs 	Nanjing Ericsson Panda Com Ltd

- Proposal 1: The IAB simulation scenario should capture the multi-hop feature of the backhaul links.

- Proposal 2: The baseline performance of the IAB system should be evaluated when radio resources are separated 1) between access and backhaul links, or 2) between successive IAB-Ns (including IAB-DN). Other resource allocation features such as dynamic FDM/TDM/SDM between different links can be evaluated by comparing with the baseline performances.

- Proposal 3: IAB performance evaluation KPIs should include the per-link SNR and SINR, user bitrate (assigned by the scheduler), user perceived throughput, and resource utilization.

Decision: The document is noted.

Wednesday's wrap-up: Discuss further offline – Thomas (AT&T)

R1-1805672	WF on Simulation Scenarios for IAB SI	AT&T, Ericsson, KDDI, Samsung, Qualcomm, ZTE

Thursday

R1-1805674	Summary of 7.7.2 Evaluation Methodology for NR IAB	AT&T

Decision: The document is noted.

Agreements:

- The following two scenarios should be considered for evaluations of IAB:

|  | Homogeneous IAB Scenario | Heterogeneous IAB Scenario |
| --- | --- | --- |
| Node deployment | All nodes (IAB donor and IAB node) are dropped on a hexagonal grid | Only IAB donor are dropped on a hexagonal grid and IAB node are dropped randomly |
| IAB donor | Micro | Macro |
| IAB node | Micro | Micro |
| Number of IAB donor | Ndonor: [1, 3, 7] | Ndonor = 7 |
| Number of IAB node | 19 - Ndonor | Nrelay selected from the following set of values: [1,3]*Ndonor*3 |
| Total number of Nodes | 19 | Ndonor + Nrelay |
| Reference Network | Ndonor donor nodes with 0 relay node | 7 donor nodes |
| Macro ISD | 200m | 2 values: 500m and [FFS] |
| Frequency and Bandwidth (total spectrum access + backhaul) | FR1: 4GHz (100MHz), FR2: 30GHz (400MHz) | FR1: 4GHz (100MHz), FR2: 30GHz (400MHz) |
| Duplex mode | TDD | TDD |

- Note: Further prioritization of these scenarios is not precluded

- Continue discussion on remaining parameters and FFS points until the next RAN1 meeting.

Agreements:

- Take large scale parameters for flexible duplex evaluations in 38.802 as the baseline for IAB evaluations.

- For determine the pathloss for links between the IAB node and other IAB nodes/donors, the following alternatives are considered:

  - Alt. 1: Determine the pathloss for links between the IAB node and candidate serving IAB nodes/donors based on N (value FFS but <= 5) independent large-scale channel realizations (taking into account LOS/NLOS probability and shadow fading).

    - Select the realization that results in the minimum pathloss between the IAB node and the selected serving IAB node/donor.

  - Alt. 2: Determine the pathloss for links between the IAB node and candidate serving IAB nodes/donors based on a LoS probability of 1-(1- Prob(R))^N (N>1, N FFS). An additional “bonus” B (value of B is FFS) is added to the pathloss for links between the IAB node and the serving IAB nodes/donors. For the links between non-serving IAB nodes/donors the pathloss is determined based on the non-modified LoS probability and no bonus is applied.

- Continue to discuss until RAN1#93 the value of B, N, and remaining details of topology selection methodology

- Either Alt. 1 or Alt. 2 to be selected in RAN1#93.

Agreements:

- For IAB donor (UMa) -> IAB node (UMi): Apply UMa fast-fading parameters but replace ASA, ZSA using ASD and ZSD from UMi-SC separately --- follow table A.2.1-11 in TR38.802

- For IAB node/donor (UMi) –> IAB node (UMi), Apply UMi fast-fading parameters but replace ASA, ZSA using ASD and ZSD from UMi-SC separately--- follow table A.2.1-11 in TR38.802

Agreements:

- The following performance metrics should be considered in IAB evaluations:

  - Area traffic capacity

  - Outage for access UEs (details FFS)

  - Per-link SNR and Geometry

    - Detailed definition of per-link SNR FFS

  - Resource utilization (details FFS)

  - User plane latency (from the donor to the access UE)

  - User perceived throughput (UPT) for bursty traffic: the unfinished bursts should be incorporated in the UPT calculation

R1-1803694	Consideration on IAB evaluation methodology and parameters	Huawei, HiSilicon

R1-1804172	IAB scenarios and evaluations	ZTE, Sanechips

R1-1804621	IAB Deployment Scenarios and Evaluation Methodology	Nokia, Nokia Shanghai Bell

R1-1804661	Simulation assumptions, scenarios and channel model for IAB	AT&T

R1-1804756	Evaluation methodology for IAB	Intel Corporation

R1-1804836	Evaluation methodology for NR IAB	Qualcomm Incorporated

### Other

R1-1804303	Evaluation on the dynamic and flexible resouce allocation in IAB	Huawei, HiSilicon

R1-1804622	Dynamic resource allocation between backhaul and access link	Nokia, Nokia Shanghai Bell

R1-1804623	RRM measurements in presence of IAB	Nokia, Nokia Shanghai Bell

R1-1804624	IAB timing	Nokia, Nokia Shanghai Bell

R1-1804625	IAB Topology Discussion	Nokia, Nokia Shanghai Bell

R1-1804626	On Cross-link Interference Management in IA	Nokia, Nokia Shanghai Bell

R1-1804663	System level simulation results of IAB	AT&T

## Study on Self Evaluation towards IMT-2020 Submission

### Evaluation methodology

R1-1805643	Summary of discussion on mobility evaluation method and parameters	Huawei

Decision: The document is noted.

Agreement: The proposals 1, 2, 3 and 4 in R1-1805643 are agreed.

R1-1805644	Summary of discussion on eMBB evaluation method and parameters for FR2	Huawei

Decision: The document is noted.

Agreement: Section 2 and its subsections in R1-1805644 are agreed.

R1-1803683	Consideration on evaluation features and parameters for IMT-2020 self evaluation	Huawei, HiSilicon

R1-1803781	Discussions on evaluation methodologies for link-level and system-level simulations	CATT

R1-1804029	Consideration on Evaluation methodology of IMT-2020 self-evaluation	ZTE, Sanechips

R1-1804408	Evaluation methodology for IMT-2020 Submission	Samsung

R1-1804757	On Simulation Assumptions, Features and Parameters for IMT-2020 self-evaluation	Intel Corporation

R1-1805073	Link to System Mapping for IMT-2020 Self-evaluation	NTT DOCOMO, INC.

R1-1805276	On Evaluation of the Mobility Parameter for IMT-202	Nokia, Nokia Shanghai Bell	(Revision of R1-1805144)

R1-1805226	On IMT-2020 Evaluation methodology	Ericsson

### Other

Including evaluation results

R1-1803684	Initial evaluation results of eMBB spectral efficiency	Huawei, HiSilicon

R1-1803685	Consideration on self evaluation of IMT-2020 for user experienced data rate	Huawei, HiSilicon

R1-1803686	Consideration on self evaluation of IMT-2020 for area traffic capacity	Huawei, HiSilicon

R1-1803687	Consideration on self evaluation of IMT-2020 for high mobility	Huawei, HiSilicon

R1-1803688	Consideration on self evaluation of IMT-2020 for peak spectral efficiency and peak data rate	Huawei, HiSilicon

R1-1803689	Consideration on self evaluation of IMT-2020 for network energy efficiency	Huawei, HiSilicon

R1-1803690	Consideration on self evaluation of IMT-2020 for mMTC connection density	Huawei, HiSilicon

R1-1803691	Consideration on self evaluation of IMT-2020 for reliability	Huawei, HiSilicon

R1-1803692	Consideration on self evaluation of IMT-2020 for control plane latency	Huawei, HiSilicon

R1-1803693	Consideration on self evaluation of IMT-2020 for user plane latency	Huawei, HiSilicon

R1-1803782	Discussions on detailed link budget evaluation methodology	CATT

R1-1803783	Discussions on calculation of shadow fading margin  for link budget	CATT

R1-1803784	Discussions on penetration margin and SD for link budget	CATT

R1-1803785	Discussions on system level simulation configurations and technical features for self evaluation	CATT

R1-1803786	Initial self evaluation on peak data rate and peak spectrum efficiency	CATT

R1-1803787	Initial self evaluation for Indoor Hotspot-eMBB	CATT

R1-1803788	Initial self evaluation for Dense Urban-eMBB	CATT

R1-1803789	Initial self evaluation for Rural-eMBB	CATT

R1-1804024	Updated UMi calibration results for user experience data rate in the multi-layer network deployment	ZTE, Sanechips

R1-1804025	Updated DL spectral efficiency in the eMBB usage scenario for IMT-2020 self evaluation	ZTE, Sanechips

R1-1804026	Link curve performance of LDPC coding in IMT-2020 self evaluation	ZTE, Sanechips

R1-1804027	Considerations on the evaluated details and parameters for IMT-2020 self evaluation	ZTE, Sanechips

R1-1804028	Updated UL spectral efficiency in the eMBB usage scenario for IMT-2020 self evaluation	ZTE, Sanechips

R1-1804030	Initial consideration on Evaluation of mobility and reliability of IMT-2020 self-evaluation	ZTE, Sanechips

R1-1804111	Initial self evaluation results for mobility in eMBB scenario	CMCC

R1-1804301	Consideration on self evaluation of IMT-2020 for UE energy efficiency	Huawei, HiSilicon

R1-1804302	Consideration on self evaluation of IMT-2020 for link budget	Huawei, HiSilicon

R1-1804585	Initial evaluation results of eMBB for ITU IMT-2020 self-evaluation	LG Electronics

R1-1804586	Calibration results for ITU IMT-2020 self-evaluation	LG Electronics

R1-1804758	On the IMT-2020 Self-Evaluation Performance Metrics and Initial Results	Intel Corporation

R1-1805074	Evaluation results of rural-eMBB for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1805075	Evaluation results of dense urban-eMBB for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1805076	Evaluation results of Indoor Hotspot-eMBB for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1805097	Preliminary evaluation of link level performance for LDPC coding	China Telecommunications

R1-1805145	Spectral Efficiency Results for the IMT-2020 Self-Evaluation	Nokia, Nokia Shanghai Bell

R1-1805227	An overview of the IMT-2020 Evaluations	Ericsson

R1-1805229	IMT 2020 Evaluations: On eMBB user experienced data rate with NR	Ericsson

R1-1805230	IMT 2020 Evaluations: On eMBB area traffic capacity with NR	Ericsson

R1-1805231	IMT-2020 self-evaluation: mMTC non-full-buffer connection density	Ericsson

R1-1805232	IMT-2020 self-evaluation: mMTC full buffer connection density	Ericsson

R1-1805233	IMT-2020 self-evaluation: Mobility evaluations for NR	Ericsson

R1-1805234	IMT-2020 self-evaluation: Peak data rate and peak spectrum efficiency evaluations for NR	Ericsson

R1-1805235	IMT-2020 self evaluation: Radio Network Energy Performance	Ericsson

R1-1805236	IMT 2020 Evaluations - CP latency in NR	Ericsson

R1-1805237	IMT 2020 Evaluations - UP latency in NR	Ericsson

R1-1805238	IMT 2020 Evaluations - Reliability in NR	Ericsson

R1-1805503	IMT 2020 Evaluations: On eMBB spectral efficiency with NR	Ericsson

## Other

R1-1803671	V2X sidelink channel model	Huawei, HiSilicon

R1-1803859	Measurement results and analysis on UE power consumption	vivo

R1-1803860	NR UE power saving	vivo

R1-1804070	Considerations of UE power saving for NR	MediaTek Inc.

R1-1804112	Initial self evaluation results for connection density in mMTC scenario	CMCC

R1-1804113	Initial self evaluation results for reliability in URLLC scenario	CMCC

R1-1804114	Initial link-level self evaluation results for Mobility in eMBB scenario	CMCC

R1-1804285	On coexistence between LTE and NR	Huawei, HiSilicon

R1-1804298	V2X sidelink measurement results	Huawei, HiSilicon

R1-1804299	On UE Antenna model in eV2X Evaluation Methodology	Huawei, HiSilicon

# Closing of the meeting

RAN1 chair thanked all the delegates for their great efforts, with a particular attention to the achievement made on UE features list. He also thanked all the feature leads for their implication, their work is greatly appreciated.

Finally he thanked CF3 for hosting the meeting in such a nice place as Sanya.

Meeting was closed at 16:31.

See you all in Busan.

# Annex A:	List of Tdocs at RAN1 #92bis

Please see excel file attached to this report

# Annex B:	List of CRs agreed at RAN1 #92bis

| TS/TR | CR | Rev | Rel | Title | Cat | Vsn | @ Mtg | TD# | Source to WG | Work Item |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36.201 | 24 | 1 | Rel-15 | Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.201 | B | 15.0.0 | R1#92b | R1-1805335 | Nokia, Nokia Shanghai Bell | LTE_unlic-Core |
| 36.201 | 25 | 1 | Rel-15 | Introduction of Downlink 1024QAM into 36.201 | B | 15.0.0 | R1#92b | R1-1805316 | Nokia, Nokia Shanghai Bell | LTE_1024QAM_DL-Core |
| 36.211 | 429 | 1 | Rel-14 | Correction on DMRS scrambling ID for semi-open loop transmission | F | 14.6.0 | R1#92b | R1-1805280 | Huawei, HiSilicon | LTE_eFDMIMO-Core |
| 36.211 | 430 | 1 | Rel-15 | Correction on DMRS scrambling ID for semi-open loop transmission | A | 15.1.0 | R1#92b | R1-1805281 | Huawei, HiSilicon | LTE_eFDMIMO-Core |
| 36.211 | 431 | - | Rel-15 | Introduction of enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE | B | 15.1.0 | R1#92b | R1-1805314 | Ericsson | LTE_1024QAM_DL-Core |
| 36.211 | 432 | - | Rel-15 | Correction on layer-to-port mapping for feCoMP | F | 15.1.0 | R1#92b | R1-1805318 | Huawei, HiSilicon | feCOMP_LTE-Core |
| 36.211 | 433 | - | Rel-14 | Correction on PUSCH hopping parameter for eVoLTE | F | 14.6.0 | R1#92b | R1-1805332 | ZTE, CMCC | LTE_VoLTE_ViLTE_enh-Core |
| 36.211 | 434 | - | Rel-15 | Correction on PUSCH hopping parameter for eVoLTE | A | 15.1.0 | R1#92b | R1-1805333 | ZTE, CMCC | LTE_VoLTE_ViLTE_enh-Core |
| 36.211 | 436 | - | Rel-14 | Collision with NPRACH in non-anchor carriers | F | 14.6.0 | R1#92b | R1-1805367 | Qualcomm Incorporated | NB_IOTenh2-Core |
| 36.211 | 437 | - | Rel-15 | Collision with NPRACH in non-anchor carriers | A | 15.1.0 | R1#92b | R1-1805368 | Qualcomm Incorporated | NB_IOTenh2-Core |
| 36.211 | 438 | - | Rel-14 | On NRS presence in non-anchor carriers | F | 14.6.0 | R1#92b | R1-1805369 | Qualcomm Incorporated | NB_IOTenh2-Core |
| 36.211 | 439 | - | Rel-15 | On NRS presence in non-anchor carriers | A | 15.1.0 | R1#92b | R1-1805370 | Qualcomm Incorporated | NB_IOTenh2-Core |
| 36.212 | 281 | - | Rel-13 | Ordering of P-CSI plus SR in eCA | F | 13.7.1 | R1#92b | R1-1805298 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.212 | 282 | - | Rel-14 | Ordering of P-CSI plus SR in eCA | A | 14.5.1 | R1#92b | R1-1805299 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.212 | 283 | - | Rel-15 | Ordering of P-CSI plus SR in eCA | A | 15.1.0 | R1#92b | R1-1805300 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.212 | 284 | - | Rel-15 | Introduction of high capacity stationary wireless link and 1024QAM into 36.212 | B | 15.1.0 | R1#92b | R1-1805315 | Huawei | LTE_1024QAM_DL-Core |
| 36.212 | 285 | - | Rel-15 | Clarification on CRC attachment for DL-SCH and PCH transport channels in NB-IoT | F | 15.1.0 | R1#92b | R1-1805327 | Huawei | NB_IOTenh2-Core |
| 36.212 | 286 | - | Rel-15 | Correction on TPC field in DCI format 6-1A | A | 15.1.0 | R1#92b | R1-1805330 | Huawei, HiSilicon | LTE_feMTC-Core |
| 36.212 | 287 | - | Rel-14 | Fallback DCI for eMTC | F | 14.5.1 | R1#92b | R1-1805359 | Qualcomm Incorporated | LTE_feMTC-Core |
| 36.212 | 288 | - | Rel-15 | Fallback DCI for eMTC | A | 15.1.0 | R1#92b | R1-1805360 | Qualcomm Incorporated | LTE_feMTC-Core |
| 36.212 | 289 | - | Rel-14 | Correction on TPC field in DCI format 6-1A | F | 14.5.1 | R1#92b | R1-1805329 | Huawei, HiSilicon | LTE_feMTC-Core |
| 36.212 | 290 | 1 | Rel-15 | Introduction of FeLAA into 36.212 | B | 15.1.0 | R1#92b | R1-1805770 | Huawei | LTE_unlic-Core |
| 36.213 | 1075 | - | Rel-15 | Introduction of SRS antenna switching for 4 ports in TS36.213 | B | 15.1.0 | R1#92b | R1-1805441 | Qualcomm, Huawei, HiSilicon | TEI15 |
| 36.213 | 1049 | 1 | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s00-s05 | F | 15.1.0 | R1#92b | R1-1805495 | Motorola Mobility | LTE_sTTIandPT-Core |
| 36.213 | 1050 | 1 | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s06-s07 | F | 15.1.0 | R1#92b | R1-1805496 | Motorola Mobility | LTE_sTTIandPT-Core |
| 36.213 | 1051 | 1 | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s10-s13 | F | 15.1.0 | R1#92b | R1-1805497 | Motorola Mobility | LTE_sTTIandPT-Core |
| 36.213 | 1052 | 1 | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s14-xx | F | 15.1.0 | R1#92b | R1-1805498 | Motorola Mobility | LTE_sTTIandPT-Core |
| 36.213 | 1053 | - | Rel-15 | Introduction of enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE to 36.213 | B | 15.1.0 | R1#92b | R1-1804478 | Motorola Mobility | LTE_1024QAM_DL-Core |
| 36.213 | 1054 | 1 | Rel-15 | Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s00-s05 | B | 15.1.0 | R1#92b | R1-1805798 | Motorola Mobility | LTE_unlic-Core |
| 36.213 | 1055 | 1 | Rel-15 | Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s06-s09 | B | 15.1.0 | R1#92b | R1-1805799 | Motorola Mobility | LTE_unlic-Core |
| 36.213 | 1058 | - | Rel-13 | Correction on number of subframes for semistatic codebook determination and transmission over PUSCH | F | 13.9.0 | R1#92b | R1-1805301 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1059 | - | Rel-14 | Correction on number of subframes for semistatic codebook determination and transmission over PUSCH | A | 14.6.0 | R1#92b | R1-1805302 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1060 | - | Rel-15 | Correction on number of subframes for semistatic codebook determination and transmission over PUSCH | A | 15.1.0 | R1#92b | R1-1805303 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1061 | - | Rel-15 | Correction on the index of codewords for feCoMP | F | 15.1.0 | R1#92b | R1-1805319 | Huawei, HiSilicon | feCOMP_LTE-Core |
| 36.213 | 1062 | - | Rel-14 | Correction on TBS determination for msg 3 retransmission and CSS for fallback | F | 14.6.0 | R1#92b | R1-1805361 | Samsung, Ericsson | LTE_feMTC-Core |
| 36.213 | 1063 | - | Rel-14 | Correction on TBS determination for msg 3 retransmission and CSS for fallback | A | 15.1.0 | R1#92b | R1-1805362 | Samsung, Ericsson | LTE_feMTC-Core |
| 36.213 | 1064 | - | Rel-14 | Clarification of Msg3 power control in NB-IoT | F | 14.6.0 | R1#92b | R1-1805364 | Huawei, HiSilicon | NB_IoTenh-Core |
| 36.213 | 1065 | - | Rel-15 | Clarification of Msg3 power control in NB-IoT | A | 15.1.0 | R1#92b | R1-1805365 | Huawei, HiSilicon | NB_IoTenh-Core |
| 36.213 | 1066 | - | Rel-14 | On the configuration of UL 256QAM | F | 14.6.0 | R1#92b | R1-1805371 | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell | LTE_UL_CAP_enh-Core |
| 36.213 | 1067 | - | Rel-15 | On the configuration of UL 256QAM | A | 15.1.0 | R1#92b | R1-1805372 | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell | LTE_UL_CAP_enh-Core |
| 36.213 | 1068 | - | Rel-14 | Correction on look-ahead for SRS carrier switching | F | 14.6.0 | R1#92b | R1-1805373 | Qualcomm Incorporated | LTE_SRS_switch-Core |
| 36.213 | 1069 | - | Rel-15 | Correction on look-ahead for SRS carrier switching | A | 15.1.0 | R1#92b | R1-1805374 | Qualcomm Incorporated | LTE_SRS_switch-Core |
| 36.213 | 1070 | - | Rel-15 | Introduction of Enhancing CA Utilization into 36.213 | B | 15.1.0 | R1#92b | R1-1805377 | Nokia, Nokia Shanghai Bell | LTE_euCA-Core |
| 36.213 | 1071 | - | Rel-14 | Clarification of NPUSCH uplink compensation gaps for 2 HARQ processes | F | 14.6.0 | R1#92b | R1-1805403 | Huawei, HiSilicon | NB_IoTenh-Core |
| 36.213 | 1072 | - | Rel-15 | Clarification of NPUSCH uplink compensation gaps for 2 HARQ processes | A | 15.1.0 | R1#92b | R1-1805404 | Huawei, HiSilicon | NB_IoTenh-Core |
| 36.213 | 1073 | - | Rel-15 | Introduction of reduced control plane latency | B | 15.1.0 | R1#92b | 1-1805408 | Ericsson | TEI15 |
| 36.213 | 1074 | - | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s08-s09 | F | 15.1.0 | R1#92b | R1-1805499 | Motorola Mobility | LTE_sTTIandPT-Core |
| 36.211 | 440 | - | Rel-15 | Corrections to sTTI and SPT implementation | F | 15.1.0 | R1#92b | R1-1805780 | Ericsson | LTE_sTTIandPT-Core |
| 36.212 | 280 | 1 | Rel-15 | Corrections on shortened processing time and shortened TTI in 36.212 | F | 15.1.0 | R1#92b | R1-1805769 | Huawei | LTE_sTTIandPT-Core |
| 38.202 | 2 | - | Rel-15 | CR to 38.202 capturing RAN1#92bis meeting agreements and comments | F | 15.1.0 | R1#92b | R1-1805792 | Qualcomm Incorporated | NR_newRAT-Core |
| 38.211 | 2 | - | Rel-15 | CR to 38.211 capturing RAN1#92bis meeting agreements | F | 15.1.0 | R1#92b | R1-1805793 | Ericsson | NR_newRAT-Core |
| 38.212 | 2 | - | Rel-15 | CR to 38.212 capturing RAN1#92bis meeting agreements | F | 15.1.1 | R1#92b | R1-1805794 | Huawei | NR_newRAT-Core |
| 38.213 | 2 | - | Rel-15 | CR to 38.213 capturing RAN1#92bis meeting agreements | F | 15.1.0 | R1#92b | R1-1805795 | Samsung | NR_newRAT-Core |
| 38.214 | 2 | - | Rel-15 | CR to 38.214 capturing RAN1#92bis meeting agreements | F | 15.1.0 | R1#92b | R1-1805796 | Nokia | NR_newRAT-Core |
| 38.215 | 3 | - | Rel-15 | CR to 38.215 capturing RAN1#92bis meeting agreements | F | 15.1.0 | R1#92b | R1-1805797 | Intel Corporation | NR_newRAT-Core |
| 36.211 | 441 | - | Rel-15 | Introduction of enhancements to operation in unlicensed spectrum | B | 15.1.0 | R1#92b | R1-1805781 | Ericsson | LTE_unlic-Core |
| 36.213 | 1057 | - | Rel-15 | Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s14-xx | B | 15.1.0 | R1#92b | R1-1804492 | Motorola Mobility | LTE_unlic-Core |

# Annex C-1:	List of Outgoing LSs from RAN1 #92bis

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-1805347 | Reply LS on CP latency reduction | RAN1, Ericsson | Rel-15 | TEI15 |  | RAN2 |  |  |  |
| R1-1805376 | LS on Capabilities for SRS carrier switching | RAN1, Qualcomm Incorporated | Rel-14 | LTE_SRS_switch-Core |  | RAN2 |  |  |  |
| R1-1805388 | LS on RAN1 Agreements from RAN1#92bis and on the introduction of multiple altitude thresholds | RAN1, Ericsson | Rel-15 | LTE_Aerial-Core |  | RAN2 |  |  |  |
| R1-1805402 | LS on Capabilities for advanced CSI in FD-MIMO | RAN1, Qualcomm Incorporated | Rel-14 | LTE_eFDMIMO-Core |  | RAN2 |  |  |  |
| R1-1805407 | LS on downlink channel quality report in Msg3 for NB-IoT | RAN1, CMCC | Rel-14 | NB_IOT-Core, TEI14 |  | RAN2, RAN4 |  |  |  |
| R1-1805434 | LS on Narrowband measurement accuracy improvements | RAN1, Ericsson | Rel-15 | NB_IOTenh2-Core |  | RAN4 | RAN2 |  |  |
| R1-1805435 | LS on additional agreements for shortened TTI and processing time for LTE | RAN1, Ericsson | Rel-15 | LTE_sTTIandPT-Core |  | RAN2 |  |  |  |
| R1-1805619 | LS on small-delay CDD for V2X Phase 2 | RAN1, Huawei | Rel-15 | LTE_eV2X-Perf |  | RAN4 | RAN2 |  |  |
| R1-1805620 | LS on RRC parameters for V2X Phase 2 | RAN1, Huawei | Rel-15 | LTE_eV2X-Core |  | RAN2 |  |  |  |
| R1-1805625 | LS to RAN2 on aperiodic SRS triggering | RAN1, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805653 | LS on Max TBS for PDSCH containing RMSI | RAN1, Huawei | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805657 | Reply LS on Message 3 size for NR | RAN1, NTT DOCOMO | Rel-15 | NR_newRAT-Core | R2-1804073 | RAN2 |  |  |  |
| R1-1805712 | LS on RMSI CORESET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth | RAN1, CATT | Rel-15 | NR_newRAT-Core |  | RAN4, RAN2 |  |  |  |
| R1-1805717 | LS on SSB frequency location of Pscell | RAN1, Intel | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805726 | Reply LS on SI Reception in BWP | RAN1, LGE, MediaTek | Rel-15 | NR_newRAT-Core |  | RAN2 | RAN4 |  |  |
| R1-1805752 | LS on NR UE features | RAN1, NTT DOCOMO, AT&T | Rel-15 | NR_newRAT-Core |  | RAN2, RAN4 | RAN3 |  |  |
| R1-1805755 | LS on active BWP switching delay specification | RAN1, MediaTek | Rel-15 | NR_newRAT-Core |  | RAN4 |  |  |  |
| R1-1805759 | Reply LS response on handling parallel RA procedure and SR procedure | RAN1, Qualcomm | Rel-15 | NR_newRAT-Core | R2-1803856 | RAN2 |  |  |  |
| R1-1805760 | LS on Rx beam selection for RRM measurements | RAN1, Intel | Rel-15 | NR_newRAT-Core |  | RAN4 |  |  |  |
| R1-1805761 | Reply LS on RAN1 on Beam- and Cell- Radio-Link-Monitoring | RAN1, Intel | Rel-15 | NR_newRAT-Core | R2-1804066 | RAN2 |  |  |  |
| R1-1805762 | Reply LS on RAN2 agreements on RRM | RAN1, Huawei | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805763 | LS for supporting aperiodic TRS | RAN1, AT&T | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805765 | LS on RAN1 input to TS38.300 | RAN1, Nokia | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805766 | LS on RRC parameters for NR | RAN1, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805621 | LS on time domain behavior of CSI-RS and CSI-IM | RAN1, ZTE | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805622 | LS on beamFailureRecoveryTimer | RAN1, InterDigital | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805623 | LS on NR BFR | RAN1, Huawei | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805626 | LS on NR CSI-RS | RAN1, Huawei | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805627 | LS on UL cross carrier beam indication | RAN1, Samsung | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1805768 | LS on RRC parameter list for L1 configuration of Rel-15 LTE-MTC | RAN1, Ericsson | Rel-15 |  |  | RAN2, RAN3 |  |  |  |
| R1-1805782 | Reply LS on maximum number of CCs for NR CA | RAN1, NTT DOCOMO | Rel-15 | NR_newRAT-Core | R2-1804052 | RAN2 |  |  |  |
| R1-1805784 | LS on RRC parameter list for Ultra Reliable Low Latency Communication for LTE | RAN1, Ericsson | Rel-15 | LTE_HRLLC-Core |  | RAN2 |  |  |  |
| R1-1805785 | LS on RRC and LPP parameter list for further NB-IoT enhancements | RAN1, Huawei | Rel-15 | NB_IOTenh2-Core |  | RAN2, RAN3 |  |  |  |
| R1-1805791 | LS on UE Capabilities for LTE_unlic | RAN1, Nokia | Rel-15 | LTE_unlic-Core |  | RAN2 |  |  |  |
| R1-1805438 | Reply LS on handling of multiple numerologies in FeMBMS | RAN1, Qualcomm | Rel-14 | MBMS_LTE_enh2-Core | R2-1806298 | RAN2 |  |  |  |
| R1-1805440 | LS on PDCCH common search space configurations | RAN1, NTT DOCOMO | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |

# Annex C-2:	List of Incoming LSs from RAN1 #92bis

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-1803573 | Reply LS to RAN1 on wake-up signal | RAN2, Huawei | Rel-15 | NB_IOTenh2-Core, LTE_eMTC4-Core |  | RAN1 |  | R2-1803798 | R1-1719207 |
| R1-1803574 | LS on LTE peak data-rate calculation for EN-DC | RAN2, Intel | Rel-15 | NR_newRAT-Core |  | RAN1 | RAN4 | R2-1803950 |  |
| R1-1803575 | LS on messagePowerOffsetGroupB | RAN2, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1803860 |  |
| R1-1803576 | LS response to RAN1 on NR MIMO MAC CE | RAN2, Huawei | Rel-15 | NR_newRAT-Core | R1-1801272 | RAN1 |  | R2-1803872 |  |
| R1-1803577 | LS to RAN1 on Beam- and Cell- Radio-Link-Monitoring | RAN2, Nokia | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1804066 |  |
| R1-1803578 | Reply to LS on eV2X evaluation methodology | 5GAA WG4, Intel | Rel-15 | FS_LTE_NR_V2X_eval | R1-1719239 | RAN1 | 5GAA WG2, 5GAA WG5, RAN | S-180039 |  |
| R1-1803579 | LS to RAN1 on handling parallel RA procedure and SR procedure | RAN2, AsusTek | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1803856 |  |
| R1-1803580 | Reply LS on HARQ-ACK feedback for efeMTC | RAN2, Qualcomm | Rel-15 | LTE_eMTC4-Core | R1-1721254 | RAN1 |  | R2-1803886 |  |
| R1-1803581 | LS on maximum number of CCs for NR CA | RAN2, NTT DOCOMO | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1804052 |  |
| R1-1803582 | LS on Message 3 size for NR | RAN2, NTT DOCOMO | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1804073 |  |
| R1-1803583 | Reply LS on required information for NSA on X2 | RAN2, Nokia Shanghai Bell | Rel-15 | NR_newRAT-Core | R3-174964 | RAN3 | RAN1, RAN4 | R2-1804074 |  |
| R1-1803584 | LS on Frequency Band list | RAN2, Huawei | Rel-15 | NR_newRAT-Core | R4-1714295 | RAN1, RAN3, RAN4 |  | R2-1804075 |  |
| R1-1803585 | LS on RAN2 agreements for LTE URLLC | RAN2, Ericsson | Rel-15 | LTE_HRLLC-Core |  | RAN1 |  | R2-1804084 |  |
| R1-1803586 | Response LS on Maximum TBS for PDSCH containing RMSI/OSI/Paging | RAN2, Huawei | Rel-15 | NR_newRAT-Core | R1-1803375 | RAN1 |  | R2-1804098 |  |
| R1-1803587 | LS on UL timing difference in EN-DC | RAN2, Qualcomm | Rel-15 | NR_newRAT-Core |  | RAN4 | RAN1 | R2-1804102 |  |
| R1-1803588 | Reply LS on SI reception in BWP | RAN2, LG Electronics | Rel-15 | NR_newRAT-Core |  | RAN1 | RAN4 | R2-1804109 |  |
| R1-1803589 | LS on RAN2 agreements for euCA | RAN2, Nokia | Rel-15 | LTE_euCA-Core |  | RAN1, RAN4 | RAN3 | R2-1804137 |  |
| R1-1803590 | LS on UE measurements for demodulation test setup in FR2 | RAN4, Intel | Rel-15 | FS_NR_test_methods |  | RAN1, RAN5 |  | R4-1801809 |  |
| R1-1803591 | Reply LS on clarification in MAC | RAN4, Huawei | Rel-15 | NR_newRAT-Core | R2-1801569 | RAN2 | RAN1 | R4-1802690 |  |
| R1-1803592 | LS reply on UE RF related parameters, capabilities and features for NR standalone | RAN4, NTT DOCOMO | Rel-15 | NR_newRAT-Core | R2-1712028 | RAN2 | RAN1, RAN3 | R4-1802708 |  |
| R1-1803593 | LS reply on UE baseband processing capability | RAN4, Intel | Rel-15 | NR_newRAT-Core | R2-1712078 | RAN2 | RAN1 | R4-1803122 |  |
| R1-1803594 | Reply LS on NR Idle Mode Measurements | RAN4, CMCC | Rel-15 | NR_newRAT-Core | R2-1801658 | RAN2 | RAN1 | R4-1803276 |  |
| R1-1803595 | LS reply to RAN1 on P_0 ranges on UL power control | RAN4, Huawei | Rel-15 | NR_newRAT-Core | R1-1721608 | RAN1, RAN2 |  | R4-1803466 |  |
| R1-1803596 | LS on NR UE feature list | RAN, NTT DOCOMO | Rel-15 | NR_newRAT-Core |  | RAN1, RAN2, RAN4 | RAN3 | RP-180596 |  |
| R1-1803597 | LS on Summary of email discussion “[ITU-R AH 01] Calibration for self-evaluation” | ITU-R Ad-Hoc, Huawei | Rel-15 | FS_5G_eval |  | RAN, RAN1, RAN2, RAN4 |  | RT-180011 |  |
| R1-1803598 | LS on NSSS based NRSRP measurement definition | RAN4, Nokia | Rel-15 | NB_IOTenh2-Core |  | RAN1 |  | R4-1802788 |  |
| R1-1803599 | LS on EARFCN provisioning for Release 15 MTC and Release 15 NB-IOT UE | RAN4, Qualcomm | Rel-15 | NB_IOTenh2-Core, LTE_eMTC4-Core |  | RAN2 | RAN1, CT6 | R4-1803078 |  |
| R1-1803600 | LS on SFTD measurement before EN-DC | RAN4, Huawei | Rel-15 | NR_newRAT-Core |  | RAN1 | RAN2 | R4-1803174 |  |
| R1-1803601 | LS on PA EVM for pi/2 BPSK and DMRS | RAN4, Qualcomm | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R4-1803265 |  |
| R1-1803602 | LS on BWP switching delay | RAN4, Intel | Rel-15 | NR_newRAT-Core |  | RAN1 | RAN2 | R4-1803283 |  |
| R1-1803603 | Reply LS on NR SS Raster Shift for Frequency Range 0-2700MHz | RAN4, ZTE | Rel-15 | NR_newRAT-Core | R1-1801182 | RAN1, RAN2 |  | R4-1803441 |  |
| R1-1803604 | LS on UE feature list | RAN4, NTT DOCOMO, Intel | Rel-15 | NR_newRAT-Core |  | RAN1, RAN2 | RAN3 | R4-1803564 |  |
| R1-1803605 | Acquisition of N_(TA_Offset) for Uplink Transmission | RAN4, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN2 | RAN1 | R4-1803570 |  |
| R1-1805264 | LS on RAN2 agreements on RRM | RAN2, Huawei | Rel-15 | NR_newRAT-Core |  | RAN1, RAN4 |  | R2-1804103 |  |
| R1-1805380 | Reply LS on PC5 transmission mechanism selection | SA2, LG Electronics | Rel-15 | LTE_eV2X-Core |  | RAN1 |  | S2-183968 |  |
| R1-1805520 | LS on L2 measurement specification for NR in R15 | SA5, Ericsson | Rel-15 | NETSLICE-ADPM5G |  | RAN2 | RAN1 | S5-182574 |  |
| R1-1805523 | LS on the transmission of video over PC5 interface | SA4, Samsung | Rel-15 | FS_mV2X |  | RAN1 | RAN2, SA1 | S4-180633 |  |
| R1-1805576 | LS on NPRS design enhancement for colliding NPRS | RAN4, Qualcomm | Rel-14 | NB_IOTenh-Core |  | RAN1 |  | R4-1805486 |  |
| R1-1805579 | Reply LS on handling of multiple numerologies in FeMBMS | RAN2, Qualcomm | Rel-14 | MBMS_LTE_enh2-Core | R1-1803116 | RAN1 |  | R2-1806298 |  |
| R1-1805744 | Reply LS on eV2X evaluation methodology | SAE C-V2X TC Advanced Application Task-Force, AT&T | Rel-15 | FS_LTE_NR_V2X_eval | R1-1719239 | RAN, RAN1 |  |  |  |

# Annex D:	List of Approved updated WIDs

None

# Annex E:	List of draft TSs/TRs agreed at RAN1 #92bis

| Tdoc Number | Title | Source | Conclusion/Decision |
| --- | --- | --- | --- |
| R1-1804589 | Draft TR 37.885 v0.1.0 Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR | LG Electronics Inc. | Endorsed as v0.1.0 |
| R1-1805416 | 3GPP TS 37.213 Physical layer procedures for shared spectrum channel | Ericsson | Endorsed as v0.1.0 |
| R1-1805790 | TS 37.213 v0.2.0 Physical layer procedures for shared spectrum channel access | Ericsson | Endorsed as v0.2.0 |

# Annex F:	List of actions

- Outgoing LS

[92b-NR-01]  – Fred (NTT DOCOMO)

R1-1803581	LS on maximum number of CCs for NR CA	RAN2, NTT DOCOMO

Email discussion/approval of reply LS to R1-1803581 till April 27th, incl. reaching the answer to the question: RAN2 asks RAN1 to check if RAN1 specs support 32 CCs for NR CA.

Done: Following the decision posted on April 30th, final LS is approved in R1-1805782.

[92b-NR-03] – Fred (NTT DOCOMO)

Email discussion till May 4th to come-up with identifying necessary information (e.g., periodicities, offsets, occasions) for updating RRC signaling searchSpace such that it includes the set of periodicities defined for ssb-PeriodicityServingCell.

Aim to target providing an LS to RAN2.

Done: Following the decision posted on May 9th, final LS is approved in R1-1805440.

R1-1805439	[92b-NR-03] Necessary information for search space configuration

R1-1805440	LS on PDCCH common search space configurations

[92b-LTE-04] – Timo (Nokia)

Email discussion until May 3rd on UE capabilities for FeLAA and associated LS to RAN2

Done: Following the decision posted on May 4th, final LS is approved in R1-1805791.

[92b-LTE-08] – Johan (Ericsson)

Email discussion for RRC parameters for eMTC including the agreements and working assumptions at RAN1#92bis meeting by April 27th

Done: Following the decision posted on May 1st , following is agreed:

R1-1805787	List of RRC parameters for L1 configuration of Rel-15 LTE-MTC	Ericsson

R1-1805768	LS on RRC parameter list for L1 configuration of Rel-15 LTE-MTC	RAN1, EricssonR1-1805785	LS on RRC and LPP parameter list for further NB-IoT enhancements	RAN1, Huawei

[92b-LTE-11] – Matthew (Huawei)

Email discussion for RRC parameters for NB-IOT including the agreements and working assumptions at RAN1#92bis meeting by April 27th.

Done: Following the decision posted on May 1st , following is agreed:

R1-1805786	List of RRC and LPP parameters for further NB-IoT enhancements	Huawei

R1-1805785	LS on RRC and LPP parameter list for further NB-IoT enhancements	RAN1, Huawei

[92b-LTE-13] – Laetitia (Ericsson)

Email discussion on the set of RRC parameters/UE capability for HRLLC based on the agreements and working assumptions so far by April 27th

Done: Following the decision posted on May 1st , following is agreed:

R1-1805783	List of physical layer RRC parameters for Ultra Reliable Low Latency Communication for LTE	Ericsson

R1-1805784	LS on RRC parameter list for Ultra Reliable Low Latency Communication for LTE	RAN1, Ericsson

- CR approval

[92b-LTE-02-xxx]

6.2.1 sTTI & sPT, xxx – spec #

Editors to prepare CRs incorporating all the agreements and working assumptions by April 27th, for comments and endorsement by May 3rd

Done: The following set of CRs are agreed:

R1-1805780	Corrections to sTTI and SPT implementation	Ericsson	36.211	15.1.0	LTE_sTTIandPT-Core	CR0440, F

R1-1805769	Corrections on shortened processing time and shortened TTI in 36 212	Huawei	36.212	15.1.0	LTE_sTTIandPT-Core	CR0280r1, F

R1-1805495	Corrections for shortened processing time and shortened TTI in 36.213, s00-s05	Motorola Mobility	36.213	15.1.0	LTE_sTTIandPT-Core	CR1049r1, F

R1-1805496	Corrections for shortened processing time and shortened TTI in 36.213, s06-s07	Motorola Mobility	36.213	15.1.0	LTE_sTTIandPT-Core	CR1050r1, F

R1-1805497	Corrections for shortened processing time and shortened TTI in 36.213, s10-s13	Motorola Mobility	36.213	15.1.0	LTE_sTTIandPT-Core	CR1051r1, F

R1-1805498	Corrections for shortened processing time and shortened TTI in 36.213, s14-xx	Motorola Mobility	36.213	15.1.0	LTE_sTTIandPT-Core	CR1052r1, F

R1-1805499	Corrections for shortened processing time and shortened TTI in 36.213, s08-s09	Motorola Mobility	36.213	15.1.0	LTE_sTTIandPT-Core	CR1074, F

[92b-LTE-03-xxx]

6.2.2 Unlic, xxx-spec #

Editors to prepare CRs incorporating all the agreements and working assumptions by April 27th, for comments and endorsement by May 3rd

Done: The following set of CRs are agreed:

R1-1805781	Introduction of enhancements to operation in unlicensed spectrum	Ericsson	36.211	15.1.0	LTE_unlic-Core	CR0441, B

R1-1805770	Introduction of FeLAA into 36.212	Huawei	36.212	15.1.0	LTE_unlic-Core		CR0290r1, B

R1-1805798	Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s00-s05	Motorola Mobility	36.213	15.1.0	LTE_unlic-Core	CR1054r1, B

R1-1805799	Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s06-s09	Motorola Mobility Germany GmbH		36.213	15.1.0	LTE_unlic-Core	CR1055r1, B

In addition, the following is endorsed as v0.2.0 of 37.213

R1-1805790	TS 37.213 v0.2.0 Physical layer procedures for shared spectrum channel access	Ericsson

[92b-LTE-06-xxx] – Rapporteur or spec #

Rapporteur to prepare the list of V2X agreements and working assumptions by April 23rd

Editors to prepare CRs incorporating all the agreements and working assumptions by May 4th, for comments and endorsement by May 11th.

Done:

R1-1805789	RAN1 agreements for LTE V2X Phase 2	Huawei

The following draft CRs (with RAN1#93 Tdoc#) are endorsed.

R1-1807401	Introduction of V2X Phase 2 based on LTE	Huawei	36.212	15.1.0	LTE_eV2X-Core

R1-1807408	Introduction of eV2X	Ericsson	36.211	15.1.0	LTE_eV2X-Core

R1-1807411	Addition of V2X Phase 2 based on LTE in 36.213, s14	Motorola Mobility, LG Electronics	36.213	15.1.0	LTE_eV2X-Core

[92b-LTE-07-xxx] – Rapporteur or spec #

Rapporteur to prepare the list of eMTC agreements and working assumptions by April 23rd

Editors to prepare CRs incorporating all the agreements and working assumptions by May 4th, (for 36.211, based on x3862/x5173; for 36.212, based on x4149), for comments and endorsement by May 11th.

Done:

R1-1805767	Agreement summary for Rel-15 Even further enhanced MTC for LTE	Ericsson

The following draft CRs (with RAN1#93 Tdoc#) are endorsed.

R1-1807310	Introduction of even further enhancements of eMTC	Huawei, HiSilicon	draftCR	36.212	15.1.0	LTE_eMTC4-Core	B

R1-1807409	Introduction of Even Further Enhanced MTC for LTE	Ericsson	draftCR	36.211	15.1.0	LTE_eMTC4-Core	B

[92b-LTE-10-xxx] – Rapporteur or spec #

Rapporteur to prepare the list of agreements and working assumptions April 23rd

Editors to prepare CRs incorporating all the agreements and working assumptions by May 4th, (for 36.211, based on x3863; for 36.212, based on x4148), for comments and endorsement by May 11th.

Done:

R1-1805436	RAN1 agreements for Rel-15 Further NB-IoT enhancements	Ericsson

The following draft CRs (with RAN1#93 Tdoc#) are endorsed.

R1-1807309	Introduction of further enhancements of NB-IoT	Huawei, HiSilicon	draftCR	36.212	15.1.0	NB_IOTenh2-Core	B

R1-1807410	Introduction Rel-15 Further NB-IoT enhancements	Ericsson	draftCR	36.211	15.1.0	NB_IOTenh2-Core	B

[92b-LTE-14] – Le (Qualcomm)

R1-1805323	Introduction of SRS antenna switching for 4 ports in TS36.213	Huawei, HiSilicon, Qualcomm

Email approval until May 9th

Done: According to the decision posted on May 10th, the following is agreed:

R1-1805441	Introduction of SRS antenna switching for 4 ports in TS36.213	Qualcomm, Huawei, HiSilicon	(rev of R1-1805323)	36.213	15.1.0	TEI15	CR1075, B

[92b-NR-02-xxx]

xxx: spec # editors to prepare CRs capturing the agreements and working assumptions till April 27th, for comments and endorsement by May 3rd

Done: The following set of CRs are endorsed:

R1-1805792	CR to 38.202 capturing the RAN1#92bis meeting agreements	Qualcomm	38.202	15.1.0	NR_newRAT-Core	CR0002, F

R1-1805793	CR to 38.211 capturing the RAN1#92bis meeting agreements	Ericsson	38.211	15.1.0	NR_newRAT-Core	CR0002, F

R1-1805794	CR to 38.212 capturing the RAN1#92bis meeting agreements	Huawei		38.212	15.1.1	NR_newRAT-Core	CR0002, F

R1-1805795	CR to 38.213 capturing the RAN1#92bis meeting agreements	Samsung	38.213	15.1.0	NR_newRAT-Core	CR0002, F

R1-1805796	CR to 38.214 capturing the RAN1#92bis meeting agreements	Nokia		38.214	15.1.0	NR_newRAT-Core	CR0002, F

R1-1805797	CR to 38.215 capturing the RAN1#92bis meeting agreements	Intel		38.215	15.1.0	NR_newRAT-Core	CR0003, F

- Miscellaneous

[92b-LTE-01] – Alberto (Qualcomm)

Email discussion on candidate values for Maximum supported bandwidth T and scaling factors until May 3rd

Done: According to Mr Chair's email posted on May 7th, the following is agreed:

Agreement: For the signalling of T, the UE can pick between one of these two options:

- 1.- Implicit (based on UE DL category): Divide max data rate of UE (as given by the column “Maximum number of DL-SCH transport block bits received within a TTI” in TS 36.306 of the highest indicated UE category) by max TBS for the maximum supported modulation scheme (as given by the column “Maximum number of bits of a DL-SCH transport block received within a TTI” for 2 layers in TS 36.306) x 20MHz (e.g. for cat-20 for a 256-QAM capable UE would be capable of round(2019360/100752)x20MHz = 400MHz – 20 layers at 20MHz per layer)

- 2.- Explicit: Separate indication in absolute values. To cover all possible cases (including some future cases), we can cover from 4 layers to 40 layers in steps of 2 layers (i.e., from 80MHz to 800MHz in steps of 40MHz)

Agreement: The possible values for A(1.25 and A(7.5 are:

- A(1.25 :{3, 6, 9, 12}

- A(7.5 :{1, 2, 3, 4}

Inform RAN2 accordingly. LS is approved in R1-1805438.

[92b-LTE-05] – Alberto  (Qualcomm)

R1-1805355	WF on DL Categories	Intel, Qualcomm

Email discussion on DL categories for 1024 QAM until May 3rd

Done: According to Mr Chair's email posted on May 5th, the following is agreed:

Agreements:

- Introduce Rel-15 DL Categories with the following data rates with optional support of 1024QAM

  - Cat 22: ~2.4-2.5Gbps – (20 layer 1024 or 24 layer 256QAM)

  - Cat 23: ~2.75-2.8Gbps – (22 layer 1024 or 28 layer 256QAM)

  - Cat 24: ~3Gbps – (24 layer 1024 or 30 layer 256QAM)

  - Cat 25: ~3.25 – 3.2 Gbps – (26 layer 1024QAM or 32 layer 256QAM)

  - Cat 26: ~3.4 – 3.5Gbps – (28 layer 1024QAM or 34 layer 256QAM)

  - Cat 20 (legacy): Enable use of 1024QAM for this category.

  - Other UE categories for 1024QAM are not precluded to be defined during Rel-15 and later releases

- Details (including soft buffer size/legacy categories signaled/rate matching) to be decided no later than during RAN1#93.

[92b-LTE-09] – Yuzheng (Huawei)

Email discussion to decide the exact TBS table on EDT for eFeMTC by April 27th

Done: According to Mr Chair's email posted on April 30th, the following is agreed:

Agreements: The following Table 1 and Table 2 are supported for CEModeA and CEModeB, respectively.

Table 1: Exact TBS table on EDT, CEModeA

|  | 328 | 408 | 504 | 600 | 712 | 808 | 936 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 328 | 328 | 328 | 328 | 328 | 328 | 328 | 328 |
| T2 |  | 408 | 408 | 408 | 456 | 504 | 504 | 536 |
| T3 |  |  | 456 | 504 | 600 | 712 | 712 | 776 |
| T4 |  |  | 504 | 600 | 712 | 808 | 936 | 1000 |

Table 2: Exact TBS table on EDT, CEModeB

|  | 328 | 408 | 456 | 504 | 600 | 712 | 808 | 936 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 328 | 328 | 328 | 328 | 328 | 328 | 328 | 328 |
| T2 |  | 408 | 408 | 408 | 408 | 456 | 504 | 504 |
| T3 |  |  | 456 | 456 | 504 | 600 | 712 | 712 |
| T4 |  |  |  | 504 | 600 | 712 | 808 | 936 |

[92b-LTE-12] – Changhwan (LGE)

Email discussion to decide the details on DL/UL interlacing (for Rel.15 NB-IoT) until may 4th

Done: According to Mr Chair's email posted on May 5th, the following is agreed:

Agreements:

- For a UE configured with 2 HARQ processes, the minimum timing relationships within one HARQ process for NPDCCH to NPDSCH, NPDCCH to NPUSCH format 1, NPDSCH to corresponding ACK/NACK, NPUSCH format 1 to corresponding ACK/NACK, and NPUSCH format 2 to next DCI for the same process, are the same as Rel-14 FDD.

- The 1 ms minimum gap from end of NPUSCH format 2 for one HARQ process to NPDCCH for the other HARQ process is not specified for TDD NB-IoT.

For 2-HARQ capable UEs:

- For UE configured with 2 HARQ processes, eNB can schedule NPDSCH of a DL HARQ process which begins before the completion of NPUSCH format 1 for an UL HARQ process or format 2 of the other DL HARQ process transmission

- For UE configured with 2 HARQ processes, eNB can schedule NPUSCH format 1 for an UL HARQ process or format 2 of a DL HARQ process which begins before the completion of NPDSCH reception for the other DL HARQ process

- Collisions between NPUSCH format 1 for an UL HARQ process and NPUSCH format 2 corresponding to a DL HARQ process are assumed to be avoided by eNB, with no specified UE behavior in case of collision.

[92b-NR-04]  – Daewon (Intel)

R1-1805641	Way Forward on NR Peak Data Rate Formula	Intel, Samsung, MediaTek, Huawei, HiSilicon, Apple, Vivo, OPPO

Email discussion/approval including the related RAN2 LS(s) till April 27th

Done: According to the decision posted on May 3rd, the following is agreed and should be considered in the reply LS to RAN2:

Agreements : Motivation for Scaling Factor (response to LS R1-1801324 from RAN2)

- Scaling factor is used to reflect the association of capability mismatch between the baseband capability and RF capability for both SA UE and NSA UE.

- Scaling factor is used to scale down maximum throughput of NSA UEs operating in EN-DC scenario that share hardware resources between LTE and NR.

- RAN1 considers the described scaling factor signalled per band in per band combination per UE capability as an important parameter. RAN1 understands that RAN2 is responsible for the overall design of the UE capabilities and that RAN2 has to ensure a reasonable size of the overall UE capability structure. Hence, RAN1 kindly asks RAN2 to aim to include the scaling factor into the capability signalling if possible from a capability signalling perspective.

Agreements: Updates to the scaling factor (update to the LS R1-1721733 to RAN2)

- Scaling factor, f(j) , are {1, 0.8, 0.75, 0.4}

- NR supports separate scaling factor indication for DL and UL

- f(j) is signalled per band in per band combination per UE capability signalling, where f(j) applies to j-th NR component carrier

[92b-NR-05] – Jing (Qualcomm)

Email discussion on further layout parameter fine tuning until May 3rd

Done: According to Mr Chair's email posted on May 5th, the following is agreed:

Agreement:

- Adopt layout as in the following figure with a=20 meters, b=40 meters, c=20 meters, and d=40 meters for indoor sub7GHz NR-U evaluation.

Indoor sub7 simulation office layout

# Annex G:	List of participants at RAN1 #92bis

Please see excel file attached to this report

# Annex H:	TSG RAN WG1 meetings in 2018 – 2019

| TITLE | TYPE | DATES | LOCATION | CTRY |
| --- | --- | --- | --- | --- |
| 3GPPRAN1#93 | WG | 21 – 25 May 2018 | Busan | Korea |
| 3GPPRAN1#94 | WG | 20 – 24 Aug 2018 | Gothenburg | Sweden |
| 3GPPRAN1#94bis | WG | 8 – 12 Oct 2018 | Chengdu (TBC) | China |
| 3GPPRAN1#95 | WG | 12 – 16 Nov 2018 | TBD | US |
| 3GPPRAN1-AH-1901-TBC | AH | 21 - 25 Jan 2019 | TBD |  |
| 3GPPRAN1#96 | WG | 25 Feb - 1 Mar 2019 | Athens | Greece |
| 3GPPRAN1#96bis | WG | 8 - 12 Apr 2019 | China | China |
| 3GPPRAN1#97 | WG | 13 - 17 May 2019 | US |  |
| 3GPPRAN1-AH-1906-TBC | AH | 24 - 28 Jun 2019 | TBD |  |
| 3GPPRAN1#98 | WG | 26 - 30 Aug 2019 | Prague | CZ |
| 3GPPRAN1#98bis | WG | 14 - 18 Oct 2019 | China |  |
| 3GPPRAN1#99 | WG | 18 - 22 Nov 2019 | US |  |

| MEETING TYPES |  |  |
| --- | --- | --- |
| AH = Ad Hoc | ST = Startup Meeting | RG = Rapporteurs Group |
| JM = Joint | WG = Working Group | SG = Steering Group |
| PM = Preparatory Meeting | CM = Chairmen's meeting | TG = Task Group |
| RM = Resolution Meeting | OR = Ordinary | XO = Extraordinary |

End of document