Source: 	MCC Support

Title:	Final Report of 3GPP TSG RAN WG1 #93 v1.0.0

(Busan, South Korea, 21st – 25th May 2018)

Document for:	Approval

Table of contents

1	Opening of the meeting	7

1.1	Call for IPR	7

1.2	Competition law statement	7

1.3	Network usage conditions	7

1.4	Check-in for Registered Delegates	7

1.5	Aspects related to RAN1 Meeting Management	7

2	Approval of Agenda	8

3	Highlights from RAN plenary	8

4	Approval of Minutes from previous meeting	8

5	Incoming Liaison Statements	8

6	E-UTRA	11

6.1	Maintenance of E-UTRA Releases 8 – 14	11

6.1.1	Maintenance of E-UTRA Release 8 – 13	11

6.1.2	Maintenance of Release 14 Enhanced Licensed-Assisted Access to Unlicensed Spectrum	12

6.1.3	Maintenance of Release 14 V2V/V2X services based on LTE sidelink	12

6.1.4	Maintenance of Release 14 Full-Dimension MIMO for LTE	12

6.1.5	Maintenance of Release 14 Further Enhanced MTC for LTE	12

6.1.6	Maintenance of Release 14 Enhancements of NB-IoT for LTE	13

6.1.7	Other	14

6.2	LTE Release 15	14

6.2.1	Maintenance of Release 15 Shortened TTI and processing time for LTE	14

6.2.1.1	Maintenance for shortened processing time for 1ms TTI	15

6.2.1.2	Maintenance for shortened TTI with shortened processing time	15

6.2.1.2.1	Aspects related to interaction between different TTI lengths	15

6.2.1.2.2	Aspects related to DL control channel	15

6.2.1.2.3	Aspects related to UL control channel	15

6.2.1.2.4	Aspects related to DL data channel	16

6.2.1.2.5	Aspects related to UL data channel	17

6.2.1.2.6	Aspects related to FS2	17

6.2.1.2.7	Other	18

6.2.2	Maintenance of Release 15 Enhancements to LTE operation in unlicensed spectrum	18

6.2.2.1	TPs for Multiple starting and ending positions in a subframe for UL	18

6.2.2.2	TPs for Autonomous uplink access with Frame Structure type 3	19

6.2.2.3	Other	19

6.2.3	Maintenance of Release 15 Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE	19

6.2.3.1	Text Proposals	19

6.2.3.2	Other	20

6.2.4	Maintenance of Release 15 Further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE	20

6.2.4.1	Text Proposals	20

6.2.5	3GPP V2X Phase 2 - WID in RP-171740	20

6.2.5.1	Carrier Aggregation (up to 8 PC5 carriers)	21

6.2.5.1.1	Mode-4 support	21

6.2.5.1.2	Synchronization	22

6.2.5.1.3	Other	22

6.2.5.2	Support for 64-QAM	22

6.2.5.3	Resource pool sharing between mode-3 and mode-4 users	23

6.2.5.4	Maximum time reduction between packet arrival at layer 1 and resource selection for transmission	24

6.2.5.5	Other	24

6.2.6	Even further enhanced MTC for LTE - WID in RP-172811	25

6.2.6.1	Resynchronization signal	25

6.2.6.2	Early data transmission	27

6.2.6.3	Wake-up signal	27

6.2.6.4	PUSCH sub-PRB allocation	29

6.2.6.5	Flexible PDSCH/PUSCH starting PRB	31

6.2.6.6	Other	32

6.2.7	Further enhancements of NB-IoT - WID in RP-172063.	32

6.2.7.1	Latency and power consumption reduction	33

6.2.7.1.1	Power consumption reduction for paging and connected-mode DRX	33

6.2.7.1.2	Data transmission during the random access procedure	35

6.2.7.1.3	Physical layer scheduling request	36

6.2.7.1.4	Semi-persistent scheduling	37

6.2.7.2	NPRACH reliability and range enhancements	37

6.2.7.2.1	Reliability enhancements for NPRACH	37

6.2.7.2.2	Range enhancements for NPRACH	38

6.2.7.3	TDD	38

6.2.7.3.1	Downlink aspects	38

6.2.7.3.2	Uplink aspects	40

6.2.7.3.3	Common aspects	41

6.2.7.4	Other	42

6.2.8	Ultra Reliable Low Latency Communication for LTE - WID in RP-172845	43

6.2.8.1	Blind/HARQ-less Repetition for Scheduled DL-SCH Operation	44

6.2.8.2	Repetition Enhancements for UL SPS Operation	45

6.2.8.3	Other	47

6.2.9	Other	47

7	NR	50

7.1	Maintenance of NR - WID in RP-172834	50

7.1.1	Initial access and mobility	51

7.1.1.1	Synchronization signal	51

7.1.1.2	Broadcast signal/channel	52

7.1.1.2.1	NR-PBCH	52

7.1.1.2.2	Remaining minimum system information	53

7.1.1.2.3	Other system information delivery	54

7.1.1.3	Paging design	55

7.1.1.4	Physical random access channel and random access procedure	56

7.1.1.4.1	PRACH formats	56

7.1.1.4.2	RACH procedure	60

7.1.1.5	Mobility procedure	65

7.1.1.5.1	Measurement for mobility management	65

7.1.1.5.2	Radio link monitoring for mobility management	66

7.1.1.6	Other	67

7.1.2	MIMO	68

7.1.2.1	Multi-antenna scheme	68

7.1.2.1.1	Codeword mapping	69

7.1.2.1.2	Codebook based transmission for UL	69

7.1.2.1.3	Non-codebook based transmission for UL	70

7.1.2.1.4	PRB bundling for DL	72

7.1.2.1.5	Other	73

7.1.2.2	CSI acquisition and beam management	73

7.1.2.2.1	CSI measurement	73

7.1.2.2.2	CSI reporting	75

7.1.2.2.3	Beam measurement and reporting	81

7.1.2.2.4	Mechanism to recover from beam failure	81

7.1.2.2.5	CQI and MCS	82

7.1.2.2.6	Other	83

7.1.2.3	Reference signals and QCL	85

7.1.2.3.1	Multiplexing of different types of RSs	85

7.1.2.3.2	CSI-RS	85

7.1.2.3.3	DMRS	87

7.1.2.3.4	PT-RS	91

7.1.2.3.5	SRS	93

7.1.2.3.6	TRS	96

7.1.2.3.7	QCL	97

7.1.2.3.8	Other	99

7.1.2.4	Other	100

7.1.3	Scheduling/HARQ aspects	100

7.1.3.1	Physical downlink control channel	100

7.1.3.1.1	PDCCH structure	100

7.1.3.1.2	Search space	100

7.1.3.1.3	Group-common PDCCH	104

7.1.3.1.4	DCI contents and formats	105

7.1.3.1.5	Other	107

7.1.3.2	Physical uplink control channel	107

7.1.3.2.1	PUCCH structure in short-duration	107

7.1.3.2.2	PUCCH structure in long-duration	109

7.1.3.2.3	UCI multiplexing	113

7.1.3.2.4	Resource allocation for PUCCH	115

7.1.3.2.5	Other	119

7.1.3.3	DL/UL data scheduling and HARQ procedure	119

7.1.3.3.1	DL/UL resource allocation	119

7.1.3.3.2	DL/UL scheduling and HARQ management	122

7.1.3.3.3	CBG-based (re)transmission	127

7.1.3.3.4	UL data transmission procedure	127

7.1.3.3.5	Soft-buffer management for NR	128

7.1.3.3.6	Multiplexing data with different transmission durations	128

7.1.3.3.7	Other	128

7.1.3.4	Other aspects on carrier aggregation and bandwidth parts	129

7.1.3.4.1	Other aspects on bandwidth Parts	129

7.1.3.4.2	Other aspects on carrier aggregation	131

7.1.3.5	Rate matching aspects for NR DL and UL	133

7.1.3.6	Other	134

7.1.4	Channel coding	134

7.1.5	NR-LTE co-existence	134

7.1.6	UL power control	135

7.1.6.1	NR UL power control - non-CA aspects	135

7.1.6.2	NR UL power control – CA aspects	136

7.1.6.3	Other	137

7.1.7	UE features	137

7.1.8	Other	140

7.2	NR URLLC in Release 15	140

7.2.1	Support of separate CQI and MCS table(s) for URLLC	140

7.2.2	Other	145

7.3	Study on enhanced NR to support Non-Terrestrial Networks - SID in RP-171450	145

7.3.1	Channel Model	145

7.3.2	Other	146

7.4	Study on non-orthogonal multiple access for NR - SID in RP-171043	147

7.4.1	Transmitter side signal processing schemes for NOMA	147

7.4.2	Receivers for NOMA	148

7.4.3	Procedures related to NOMA	150

7.4.4	Link and system level performance evaluation for NOMA	151

7.4.5	Other	155

7.5	Study on evaluation methodology of new V2X use cases - SID in RP-170837	155

7.5.1	Evaluation scenarios	156

7.5.2	Channel Model	159

7.5.3	Other	162

7.6	Study on NR-based Access to Unlicensed Spectrum - SID in RP-172021	162

7.6.1	Simulation Methodology for NR-U operation	163

7.6.2	Frame structure for NR-U operation	164

7.6.3	Potential physical layer channel design	165

7.6.3.1	DL Signals and Channels	165

7.6.3.2	UL Signals and Channels	165

7.6.4	Potential physical layer procedures	166

7.6.4.1	Channel access procedures	166

7.6.4.2	Initial access and mobility	167

7.6.4.3	Potential HARQ enhancements	168

7.6.4.4	Potential enhancements to configured grants	168

7.6.4.5	Other	169

7.6.5	Other	169

7.7	Study on Integrated Access and Backhaul for NR - SID in RP-172290	169

7.7.1	Enhancements to support NR backhaul links	169

7.7.2	Evaluation methodology for NR IAB	173

7.7.3	Other	175

7.8	Study on Self Evaluation towards IMT-2020 Submission	175

7.8.1	Evaluation methodology	175

7.8.2	Other	175

7.9	Other	177

8	Closing of the meeting	177

Annex A:	List of Tdocs at RAN1 #93	178

Annex B:	List of CRs agreed at RAN1 #93	179

Annex C-1:	List of Outgoing LSs from RAN1 #93	180

Annex C-2:	List of Incoming LSs from RAN1 #93	181

Annex D:	List of Approved updated WIDs	183

Annex E:	List of draft TSs/TRs agreed at RAN1 #93	184

Annex F:	List of actions	185

Annex G:	List of participants at RAN1 #93	193

Annex H:	TSG RAN WG1 meetings in 2018 – 2019	194

Main facts summary

The 3GPP TSG WG RAN1#93 meeting, hosted by Samsung Electronics Co., Ltd., was held at the Haeundae Grand Hotel, Busan, South Korea.

The meeting started at 9:00 on Monday 21st May and finished at 17:30 on Friday 25th May 2018.

The number of attending delegates, having confirmed their participation through the electronic check-in application, was 430.

Note that the number of registered was 600.

The schedule of the week was as follows:

The list of action points that require RAN1 post meeting follow-up is listed in Annex F (end of document).

The number of contributions for this meeting was xxxx.

Note: The amount of documents includes those discussed during the email discussion session post meeting.

|  | Contribution is agreed/approved |
| --- | --- |
|  | Contribution is not pursued (no consensus) |
|  | Further discussion is required – come-back needed |
|  | Working assumption |
|  | Specific action needed from MCC |
|  | Contribution is for email discussion/approval |

# Opening of the meeting

Mr Wanshi Chen (RAN1 Chairman) welcomed the participants of the RAN WG1 #93 meeting and opened the meeting at 09:00.

Mr Yousun Kim on behalf of Samsung Electronics Co., Ltd., welcomed the delegates, and detailed the domestic arrangements for the full week.

## Call for IPR

The attention of the members of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The members take note that they are hereby invited:

-	to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

-	to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms (e.g. see the ETSI IPR forms http://webapp.etsi.org/Ipr/).

## Competition law statement

The Chairman also drew Member’s attention to the fact that 3GPP activities are subject to antitrust and competition laws and that compliance with said laws is therefore required of any participant of this WG meeting including the Chairman and Vice Chairmen. In case of question, please contact your legal counsel.

The present meeting will be conducted with strict impartiality and in the interests of 3GPP.

Furthermore, the Chairman reminded Members that timely submission of work items/contributions in advance of WG meetings is important to allow for full and fair consideration of such matters.

## Network usage conditions

The PCG has laid down the following network usage conditions:

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network. Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting chairman should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself. 1. DON’T place your WiFi device in ad-hoc mode 2. DON’T set up a personal hotspot in the meeting room 3. DO try 802.11a if your WiFi device supports it 4. DON’T manually allocate an IP address 5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

## Check-in for Registered Delegates

The attention of the delegates to this meeting was drawn to the fact that it is not permitted to check in other delegates on their behalf. In the event of technical difficulties preventing check-in, delegates should present themselves in person to the Secretary.

## Aspects related to RAN1 Meeting Management

Delegates are encouraged to check R1-1718766 for some thoughts on RAN1 meeting management.

# Approval of Agenda

R1-1805800	Draft Agenda of RAN1#93 meeting	RAN1 Chair

Wanshi Chen (RAN1 Chair) proposed the agenda for the meeting, as well the schedule of the week.

Discussion: No comments.

Decision: The agenda is approved.

RAN1 chair commented that RAN1#93 was the last meeting for Rel-15. With regard to the nbr of TUs, RAN1 spent a lot of time on CRs since Dec 2017– Chair's hope is that all can be finalized and no more than 10 TUs be allocated to CRs by August meeting.

Qualcomm: RAN1 still need to finalize the features list – what about URLLC?

RAN1 chair: Focus is on eMBB and LTE features – there is a need to discuss how to capture and differentiate URLLC versus eMBB in the specs. Plan is to set-some oflline up for such discussion.

# Highlights from RAN plenary

This section is intentionally left void.

# Approval of Minutes from previous meeting

R1-1805801	Report of RAN1#92bis meeting	ETSI

The document was presented by Patrick Merias from MCC (ETSI Mobile Competence Center) and provides the report from last RAN1#92bis meeting in Sanya.

Discussion: No comments.

Decision: The document is approved.

# Incoming Liaison Statements

LTE

R1-1805806	LS on feLAA agreements	RAN2, Nokia

Decision: The document is noted. Details to be handled under 6.2.2.

R1-1805823	Reply LS on PC5 transmission mechanism selection	SA2, LG Electronics

Decision: The document is noted. Details to be handled under 6.2.5. Reply LS is necessary.

R1-1805820	Reply LS on CRS muting in eFeMTC	RAN4, Huawei

Decision: The document is noted. Details to be handled under 6.2.6.

R1-1805805	LS on DL channel quality reporting in MSG3	RAN2, Ericsson

RAN2 asks RAN4 if a suitable mapping with (only) 3 measured values is feasible.

Decision: The document is noted.

R1-1805815	LS response on serving cell measurement relaxation for NB-IOT	RAN4, Ericsson

It is feasible to reduce the serving cell measurement rate for low-mobility UEs under certain criteria which is FFS.

Decision: The document is noted.

R1-1805819	Reply LS on narrowband measurement	RAN4, Huawei

RAN4 asks RAN1 and RAN2 to check if it is possible to keep the spare bits static across NPBCH transmission periods.

Decision: The document is noted. Details to be handled under 6.2.7

R1-1805816	LS on RLM Qin/Qout Targets for URLLC	RAN4, Ericsson

Question 1:  Are existing Qin and Qout thresholds (2% and 10% respectively) feasible enough for URLLC to meet the high reliability targets agreed by RAN1?

Question 2: If answer to question # 1 is YES then RAN4 would like to know if there would be specific techniques/solutions to ensure that the existing Qin/Qout values will still enable the UE to meet the high reliability targets?

Question 3: If answer to question # 1 is NO then RAN4 would like to know feasible/recommended values of Qin and Qout for RLM for URLLC?

Decision: The document is noted. Details to be handled under 6.2.8. Reply LS is necessary

R1-1806388	On LTE HRLLC RLM Qin/Qout Targets	Nokia, Nokia Shanghai Bell

R1-1806389	[Draft] Reply LS on RLM Qin/Qout Targets for URLLC	Nokia, Nokia Shanghai Bell

R1-1805804	LS on further RAN2 agreements for euCA	RAN2, vivo

Decision: The document is noted. Details to be handled under 6.2.9.

R1-1807614	LS on Removal of LTE specific terminology from Group Communication System Enablers TS 22.468	SA1, Nokia

Decision: The document is noted.

The following LSs are noted (not to be presented online unless specifically requested)

R1-1805822	LS on network-based CRS interference mitigation	RAN4, Ericsson

Decision: The document is noted.

NR (including coex)

R1-1805818	LS on UE Rx/Tx Transition Times	RAN4, Qualcomm

Decision: The document is noted. Details to be handled under 7.1.

R1-1805812	LS on subcarrier spacing for BWPs and TDD configurations	RAN2, Nokia

Decision: The document is noted. Details to be handled under 7.1.1 & 7.1.3.1.3. Reply LS is necessary.

R1-1806777	LS reply on subcarrier spacing for BWPs and TDD configurations	MediaTek Inc.

R1-1806885	Draft reply LS on subcarrier spacing for BWPs and TDD configurations	Huawei, HiSilicon

R1-1805810	Reply LS on Simultaneous PDSCH Reception with Paging	RAN2, Qualcomm

Decision: The document is noted. Details to be handled under 7.1.1.3 & 7.1.3.3.2. Reply LS is necessary.

R1-1806416	Draft Reply LS on Simultaneous PDSCH Reception with Paging	Ericsson

R1-1805803	LS on the design of PRACH table	RAN2, Huawei

Decision: The document is noted. Details to be handled 7.1.1.4. Reply LS is necessary.

R1-1806415	Draft Reply LS on the design of PRACH table	Ericsson

R1-1806908	Draft reply LS on the design of PRACH table	Huawei, HiSilicon

R1-1805814	Reply LS on MSG3 size for NR	RAN2, Huawei

Decision: The document is noted. Details to be handled under 7.1.1.4.2. Reply LS is necessary.

R1-1806418	Draft Reply LS on MSG3 size for NR	Ericsson

R1-1807308	Draft reply LS on MSG3 size for NR	Huawei, HiSilicon

R1-1806414	Draft Reply LS on messagePowerOffsetGroupB RAN2	Ericsson

Decision: The document (oversight from last meeting) is noted and final LS is approved in R1-1807650.

R1-1805807	LS on SSB-based Beam Management, Beam Failure Detection, RLM	RAN2, ZTE

Decision: The document is noted. Reply LS is necessary?

R1-1805811	LS to RAN1 on Removing RLM-RS Restriction in TS 38.331 ASN.1	RAN2, Samsung

Decision: The document is noted. Details to be handled under 7.1.1.5 & 7.1.2.2. Reply LS is necessary?

Huawei: RAN2 LS is in contradiction with RAN1 agreement – following draft reply LS has been prepared in that context.

R1-1807132	[Draft] Reply LS on Removing RLM-RS Restriction	Huawei, HiSilicon

Friday: No need to reply.

R1-1805802	Reply LS on beam failure recovery	RAN2, Ericsson

Decision: The document is noted. Details to be handled under 7.1.2.2.4. Reply LS is necessary.

R1-1806208	Draft LS reply on beam failure recovery	Ericsson

R1-1806478	Draft Response to LS on Beam Failure Recovery	Intel Corporation

R1-1807129	[Draft] Reply LS on beam failure recovery	Huawei, HiSilicon

R1-1807143	[DRAFT] LS reply on beam failure recovery	NTT DOCOMO, INC.

R1-1807180	Draft LS Response to RAN2 on beam failure recovery	Nokia, Nokia Shanghai Bell

R1-1805808	LS on Removal of csi-RS-for-tracking from TCI-State	RAN2, ZTE

Decision: The document is noted. Details to be handled under 7.1.2. Reply LS is necessary.

R1-1805838	[Draft] Reply LS on removal of CSI-RS-for-tracking from TCI-State	ZTE

R1-1806207	Draft LS reply to RAN2 on removal of CSI-RS-for-tracking from TCI-State	Ericsson

R1-1807133	[Draft] Reply LS on Removal of csi-RS-for-tracking from TCI-State	Huawei, HiSilicon

R1-1805939	Discussion on LTE peak data rate for EN-DC	Huawei, HiSilicon

R1-1805940	Draft reply LS on LTE peak data rate for EN-DC	Huawei

RAN1 chair: To be handled under 7.1.3.3.1 – striving for a common reply for both LTE and NR. Reply LS is necessary.

Intel: Reply to the NR part is in

R1-1806477	[DRAFT] Reply LS on formula or table for L1 data rate	Intel Corporation

Decision: The draft LS is endorsed with the following updates “per band in per band combination” (two occurences), & “per in UE capability”. Final LS is approved in R1-1807651.

Friday

R1-1807895	Draft reply LS on LTE peak data rate for EN-DC	Huawei

Decision: The draft LS is endorsed and final version is approved in R1-1807908.

R1-1805809	LS on Defining reserved resources in NR-SIB1	RAN2, Ericsson

Decision: The document is noted. Details to be handled under 7.1.3.1.3 & 7.1.3.5. Reply LS is necessary.

R1-1806417	Draft Reply LS on Defining reserved resources in NR-SIB1	Ericsson

R1-1806886	Draft reply LS on defining reserved resources in NR-SIB1	Huawei, HiSilicon

R1-1806022	Coexistence of NR-NR and NR-LTE networks with synchronised, unsynchronised and semi-synchronised operation in 3400-3800 MHz	CEPT ECC PT1

Decision: The document is noted. Details to be handled under 7.1.5. Reply LS is necessary.

R1-1805869	Draft LS reply on Coexistence of NR-NR and NR-LTE networks with synchronised, unsynchronised and semi-synchronised operation in 3400-3800 MHz	Huawei, HiSilicon

R1-1805870	Discussion on Coexistence of NR-NR and NR-LTE networks with synchronized, unsynchronized and semi-synchronized operation	Huawei, HiSilicon

From R1-1807787 [Chairman's notes of AI 7.1.5 NR-LTE co-existence Ad-Hoc chair (Ericsson)]

R1-1807655	Draft LS reply on Coexistence of NR-NR and NR-LTE networks with synchronised, unsynchronised and semi-synchronised operation in 3400-3800 MHz	Huawei, HiSilicon

R1-1807397	Draft Response LS on Coexistence of NR-NR and NR-LTE networks	Qualcomm Incorporated

Late submission

R1-1807762	Draft LS reply on Coexistence of NR-NR and NR-LTE networks with synchronised, unsynchronised and semi-synchronised operation in 3400-3800 MHz	Huawei	(Revision of R1-1807655)

R1-1807844	Draft LS reply on Coexistence of NR-NR and NR-LTE networks with synchronised, unsynchronised and semi-synchronised operation in 3400-3800 MHz	Huawei

Decision: The final version to RAN is approved in R1-1807868 with the following changes:

- a modification of “The study made no conclusion on the coexistence feasibility with interference mitigation mechanisms for scenario 7” to “There was no study or conclusion in [1] on the coexistence feasibility with interference mitigation mechanisms for scenario 7. The study was completed in RAN56 (June 2012) and did not include other techniques specified subsequently, e.g., AAS. There have been no further studies on coexistence feasibility of inter-operator macro cell deployments with unsynchronized or semi-synchronized operation on adjacent channels using the techniques specified since June 2012, e.g., AAS.”

- A statement at the beginning stating “RAN1 would like to provide RAN TSG with the following text as a suggested response to CEPT ECC PT1”.

R1-1807048	Draft LS reply on simultaneous configuration of EN-DC and NR PUCCH Scell	NTT DOCOMO, INC.

R1-1807814	Proposals on EN-DC and NR PUCCH SCell		NTT DOCOMO

Agreement:

EN-DC with two NR PUCCH groups is supported as UE optional capability in Rel-15.

- UE cannot be configured with more than one NR PUCCH groups in the same FR.

- The same numerology is used within the NR PUCCH group in FR2 if two NR PUCCH groups are configured.

- Note: This agreement supersedes an agreement from RAN1#92 on NR PUCCH SCell

R1-1807835	Draft reply LS on simultaneous configuration of EN-DC and NR PUCCH SCell	NTT DOCOMO, INC.

Decision: The draft LS is endorsed and final version is approved in R1-1807871 with a modification to include RAN4 in the Actions section.

R1-1807805	LS on RAN4 agreement on intraband EN-DC A-MPR	RAN4, Nokia

RAN4 requests RAN1 to consider whether the RAN4 definition of MPR and A-MPR where the calculation of transmission power for LTE may take into consideration NR transmission and vice versa is consistent with RAN1 power control design. This is a similar approach that has been taken in E-UTRA intraband CA.

Decision: The document is noted. For email discussion on whether any response is needed to the RAN4 LS (R1-1807805) and, if needed, the response LS details until June 29th, 2018	Qualcomm (Peter).

R1-1805821	LS reply to RAN1 on P_0 ranges on UL power control	RAN4, ZTE

Decision: The document is noted. Details to be handled under 7.1.6.

R1-1805813	LS on Type 3 UE capabilities	RAN2, NTT DOCOMO

Details to be handled under 7.1.7.

R1-1805817	Reply LS on SRS switching	RAN4, Huawei

Details to be handled under 7.1.7

R1-1806674	Discussion on the transmission of video over PC5 interface	Samsung

R1-1806904	[Draft] Reply LS on the transmission of video over PC5 interface	Huawei, HiSilicon

To be handled under 7.5

The following LSs are noted (not to be presented online unless specifically requested)

Qualcomm requested looking at:

R1-1806456	LS on minimum number of RX antennae for NR vehicular UEs	5GAA WG2, LG Electronics

5GAA WGs asks RAN and RAN4 to allow the use of 2 RX antennae exceptionally for Uu interface of vehicular NR UEs operating on bands below 6GHz.

Discussion: Qualcomm concern

Huawei: 4RX should be mandatory.

Decision: The document is noted. The LS is not to be addressed by RAN1#93.

R1-1807139	On minimum number of RX antennae for NR vehicular UEs	Huawei, HiSilicon

R1-1807627	LS on L2 measurement specification for NR in R15	SA5, Ericsson

The following LSs were received during the week:

R1-1807524	LS response on NB-IoT downlink channel quality determination and report	RAN4, Ericsson

R1-1807683	LS on updates for TS 36.300	RAN2, Huawei	see 6.2.7

R1-1807732	Reply LS on PDCCH common search space configurations	RAN2, NTT DOCOMO	see 7.1.3.1.2

R1-1807731	LS on Bandwidth configuration for initial BWP	RAN2, NTT DOCOMO	see 7.1.3.4.1

R1-1807737	Reply LS on signalling	RAN3,

R1-1807804	LS on the feasibility of 1 symbol GP for 60KHz in FR1 and 120KHz in FR2	RAN4, MediaTek

R1-1807806	LS on RMSI CORESET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth	RAN4, CATT, Samsung

Friday

R1-1807898	LS on Lead Time for Beam Switching	RAN4, Qualcomm

Decision: The document is noted.

# E-UTRA

## Maintenance of E-UTRA Releases 8 – 14

R1-1807530	Chairman's notes of AI 6.1 Maintenance of E-UTRA Releases 8 – 14	Ad-Hoc chair (Ericsson)

Decision: Contents incorporated below, endorsed with update.

### Maintenance of E-UTRA Release 8 – 13

CA

R1-1807088	Clarification on beta offset for more than 22 HARQ-Ack bits	Qualcomm Incorporated

Decision: The draft CR is endorsed. Final Rel-13 CR approved in R1-1807432 (CR 1079), Rel-14 in R1-1807433 (CR 1080), & Rel-15 in R1-1807434 (CR 1081).

R1-1807089	Clarification on simultaneousAckNackAndCQI for format 4-5 and format 3	Qualcomm Incorporated

Decision: The draft CR is endorsed. Final Rel-13 CR is approved in R1-1807435 (CR 1082), Rel-14 in R1-1807436 (CR 1083), & Rel-15 in R1-1807437 (CR 1084).

R1-1807090	Clarification on PUCCH formats	Qualcomm Incorporated

Decision: The draft CR is endorsed. Final Rel-13 CR is approved in R1-1807438 (CR 1085), Rel-14 in R1-1807439 (CR 1086), & Rel-15 in R1-1807440 (CR 1087).

R1-1807423	Correction of a typo further to CR0594 faulty implementation	ETSI (MCC)

Decision: CR1076 to 36.213 (Rel-13, F) is agreed.

R1-1807424	Correction of a typo further to CR0594 faulty implementation	ETSI (MCC)

Decision: CR1077 to 36.213 (Rel-14, A) is agreed.

R1-1807425	Correction of a typo further to CR0594 faulty implementation	ETSI (MCC)

Decision: CR1078 to 36.213 (Rel-15, A) is agreed.

MIMO

R1-1807091	Clarification on CSI-RS resource aggregation	Qualcomm Incorporated

Decision: Potential impact to release 14 - offline needed

Decision: The draft CR to 36.211 (Rel-13, F) is noted. Discuss further offline

R1-1807451	Clarification on CSI-RS resource aggregation	Qualcomm Incorporated	(Revision of R1-1807091)

R1-1807462	Clarification on CSI-RS resource aggregation	Qualcomm Incorporated

Decision: The draft CR is endorsed. Final CRs agreed in R1-1807476 (TS36.211, CR0412, Rel-13), in R1-1807477 (TS36.211, CR0413, Rel-14) and in R1-1807478 (TS36.211, CR0414, Rel-15).

Post meeting: Wrong CR# were provided in the online session – MCC to revise them as follows:

Final CRs are agreed in R1-1807587 (TS36.211, CR0442r1, Rel-13), in R1-1807588 (TS36.211, CR0443r1, Rel-14) and in R1-1807589 (TS36.211, CR0444r1, Rel-15).

MTC

R1-1807296	Correction on PUCCH configuration with CEModeA or CEModeB	Huawei Tech.(UK) Co., Ltd

Decision: The draft CR is endorsed. Final Rel-13 CR is approved in R1-1807441 (CR 1088), Rel-14 in R1-1807442 (CR 1089), & Rel-15 in R1-1807443 (CR 1090).

### Maintenance of Release 14 Enhanced Licensed-Assisted Access to Unlicensed Spectrum

None.

### Maintenance of Release 14 V2V/V2X services based on LTE sidelink

None.

### Maintenance of Release 14 Full-Dimension MIMO for LTE

R1-1807092	On support of advanced CSI	Qualcomm Incorporated

Proposal: 8 CSI-RS ports shall be included also in the new capability signalling for the maximum number of ports for the advanced CSI.

- Send LS to RAN2 to confirm the value of 8. The range of the capability is {8, 12, 16, 20, 24, 28}

Decision: The document is noted.

R1-1807421	UE capability on number of antenna ports for advanced CSI	Ericsson Inc.	(Revision of R1-1807402)

Observation: Advanced CSI codebook does not provide either significant cell edge throughput gains or mean throughput gains over Class A codebook for 4 and 8 antenna ports

Proposal: The range of the capability csi-ReportingAdvancedMaxPorts-r14 is ENUMERATED {12, 16, 20, 24, 28}

Decision: The document is noted.

Discuss further offline

Agreement:

- CSI-RS ports shall be included also in the new capability signalling for the maximum number of ports for the advanced CSI.

- Send LS to RAN2 to confirm the value of 8. The range of the capability is {8, 12, 16, 20, 24, 28}.

R1-1807500	Draft LS on capability signalling for maximum number of ports for advanced CSI	Qualcomm Incorporated

Decision: The draft LS is endorsed and final version is approved in R1-1807543.

### Maintenance of Release 14 Further Enhanced MTC for LTE

R1-1806381	Introduce definitions of the terms BL/CE and non-BL/CE in 36.211	Ericsson

R1-1806382	Introduce definitions of the terms BL/CE and non-BL/CE in 36.213	Ericsson

Decision: The document is noted. Discuss further offline.

R1-1807453	Introduce definition of non-BL/CE in Definitions section	Ericsson

Decision: The draft CR is endorsed and final CR is agreed in R1-1807499 (TS36.212, CR0291, Rel-15).

R1-1806675	Corrections in Physical Resource Block numbering to RA Type0 for FeMTC UEs	Samsung

Decision: The document is noted. Discuss further offline.

Agreed in R1-1807480 (TS36.213, CR1095, Rel-14), in R1-1807481 (TS36.213, CR1096, Rel-15) with a modification to make the last equation rendered with change marks.

R1-1806991	Discussion of SR collision with MPDCCH for CE Mode B	Nokia, Nokia Shanghai Bell

R1-1806992	SR collision with MPDCCH for CE Mode B	Nokia, Nokia Shanghai Bell

Decision: The document is noted. Discuss further offline.

Note:	For a BL/CE UE in half-duplex FDD operation, in case an MPDCCH candidate partially or fully overlaps with an uplink SR transmission, the prioritization between the two is up to UE implementation

R1-1807093	Correction to timing advance	Qualcomm Incorporated

Decision: The draft CR is endorsed and final Rel-14 CR is agreed in R1-1807446 (CR 1091), & Rel-15 CR is agreed in R1-1807447 (CR 1092).

### Maintenance of Release 14 Enhancements of NB-IoT for LTE

R1-1805976	On NPRS for OTDOA	Huawei, HiSilicon

Proposal 1: For new NPRS sequence generation, change the Rel-14 NPRS sequence initialization to:

Proposal 2: Given the additional complexity of conforming the working assumption, strong reasons for backward compatibility to existing Rel-14 NPRS should be explained by proponents.

Proposal 3: RAN1 does not confirm the working assumption, and proceeds to design new NPRS for Rel-14 OTDOA on the basis that eNB only needs to transmit the new NPRS.

Proposal 4: When the new NPRS collides LTE PRS in-band, the colliding part is transmitted as LTE PRS on those PRB(s).

Decision: The document is noted.

R1-1806909	On NPRS performance	Ericsson

Observation 1: None-backward compatible changes should be introduced with cautions, especially at this late stage.

Observation 2: The solution should not generate c_init values larger than 31 bits, and all generated values should be unique

Proposal: Change resource mapping according to a_(k,l)^((p))=r_(l,n_s ) ((m^'+2×SFN)mod 220).

Decision: The document is noted.

R1-1807094	NPRS enhancements	Qualcomm Incorporated

Proposal 1: Confirm the working assumption

- The new NPRS sequence can operate in a backward compatible manner with the Rel-14 NPRS by configuration

Proposal 2: Design the enhanced NPRS sequence by modifying the initialization seed as

cinit=228NIDNPRS512+210⋅7⋅ns+20∙nf mod NNPRS10+1+l+1⋅2⋅NIDNPRSmod 512+1+2⋅NIDNPRSmod 512+NCP

Proposal 3: Agree the TP for TS36.211 in Appendix.

Proposal 4: Send in LS to RAN2 about the configuration of enhanced NPRS.

Decision: The document is noted.

Monday's conclusion on NPRS enhancements: Discuss further offline

Agreement: For the guard band and standalone cases, the UE is not expected to be configured with a legacy NPRS sequence

Agreement: For the in-band case, the new NPRS sequence that is designed can operate in a backward compatible manner with the Rel-14 NPRS by configuration

R1-1807550	WF on the new NPRS sequence design	Ericsson, Qualcomm, Huawei, HiSilicon

Decision: The document is agreed.

Email approval for CR on new NPRS sequence design until May 30, 2018	Ericsson (Yutao Sui)

R1-1807546 	Draft LS on Rel-14 NPRS Enhancements	Qualcomm

Decision: The draft LS is noted. Final LS is approved in R1-1807567 with inclusion of the RAN4 LS number in the “Reply To” field.

R1-1806910	draft CR on 2 NPUSCH timing in NB-IoT HARQ processes	Ericsson

Discussion: According to Huawei, proposed text is already captured into 36.211.

Decision: The document is noted. Discuss further offline

R1-1807541	Clarification of NPUSCH reception in 2 NB-IoT HARQ processes	Huawei, HiSilicon

Decision: The draft CR is endorsed with the title modified as “Clarification of DL reception in 2 NB-IoT HARQ processes”.

Final CR is agreed in R1-1807547 (TS36.213, CR1098, Rel-14) and R1-1807548 (TS36.213, CR1099, Rel-15).

### Other

R1-1806454	Correction on RV determination for eVoLTE	Huawei, HiSilicon, CMCC

Decision: The draft CR is endorsed. Final Rel-14 CR is approved in R1-1807448 (CR 1093).

R1-1807311	Correction on RV determination for eVoLTE	Huawei, HiSilicon, CMCC

Decision: The draft shadow CR is endorsed. Final Rel-15 CR is approved in R1-1807449 (CR 1094).

## LTE Release 15

Friday

Drafts for LTE CRs to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

sTTI: Updated cat-F editor CRs for 211, 212, 213 based on endorsed TPs, agreements, and working assumptions.

FeLAA: Updated cat-F editor CRs for 211, 212, 36.213, and 37.213 based on endorsed TPs, agreements, and working assumptions.

HCS: New cat-F editor CRs for 212, 213 based on endorsed TPs

FeCoMP: New cat-F editor CR for 213 based on endorsed TPs

(initial versions may be Draft CR, final agreed version CR)

eMTC: Updated cat-B editor CRs for 211, 212, 213 based on agreements and working assumptions.

NB-IoT: Updated cat-B editor CRs for 211, 212, 213, 214 based on agreements and working assumptions.

eV2X: Updated cat-B editor CRs for 212, 213 based on agreements and working assumptions.

(Initial versions may be Draft CR, final agreed version CR)

(Rapporteur to provide updated list of agreements and working assumptions by Saturday May 26)

HRLLC: New cat-B editor CRs for 211, 212, 213 based on agreements and working assumptions.

(Initial versions may be Draft CR, final agreed version CR)

(Rapporteur to provide list of agreements and working assumptions by Saturday May 26)

Aerials: New cat-B editor CR for 213 based on endorsed TPs.

EN-DC: 36.213 CRs

R1-1807458	Rel-15 LTE UE features	NTT DOCOMO, INC.

### Maintenance of Release 15 Shortened TTI and processing time for LTE

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

R1-1807531	Chairman's notes of AI 6.2.1 Maintenance of Release 15 Shortened TTI and processing time for LTE Ad-Hoc chair (Ericsson)

Decision: Contents incorporated below, endorsed with update.

R1-1807463	Draft LS on shortened TTI and processing time for LTE	Ericsson

Decision: The document is endorsed and final LS is approved in R1-1807488.

R1-1807565	Draft LS on shortened TTI and processing time for LTE	Ericsson

Decision: The document is endorsed and final LS is approved in R1-1807566.

#### Maintenance for shortened processing time for 1ms TTI

R1-1806936	Corrections on sPT for FDD-TDD CA with FDD PCell	Sharp, Ericsson

Decision: The text proposal in Section 3 of R1-1806936 is endorsed.

#### Maintenance for shortened TTI with shortened processing time

R1-1807320	Correction on DL subslot pattern determination	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell, LG Electronics

Decision: The text proposals in R1-1807320 are endorsed.

##### Aspects related to interaction between different TTI lengths

R1-1806575	Corrections related to interaction between different TTI lengths	LG Electronics

Decision: The text proposal in R1-1806575 is endorsed. The text proposal is also applicable to SPS and editor can incorporate text accordingly.

R1-1806857	Aspects related to collision handling for subslot-PUSCH	OPPO

R1-1807095	Aspects related to interaction between different TTI lengths	Qualcomm Incorporated

R1-1807487	Handling sTTI SR and 1 ms TTI collision	Qualcomm Incorporated	(Revision of R1-1807095)

Decision: The text proposal in R1-1807487 is endorsed.

Agreement:

To handle UL collision between subslot-TTI conveying SR only and subframe-based PUSCH or PUCCH with more than 2 bits of ACK/NAK, a UE is configured with up to two subslot-PUCCH format 4 resources. The maximum coding rate for subslot-PUCCH format 4 transmission is configured by a higher layer signalling.

Agreement:

To handle UL collision between slot-TTI conveying SR only and subframe-based PUSCH or PUCCH with more than 2 bits of ACK/NAK, a UE can be configured with a slot-PUCCH format 3 resource, and /or up to two slot-PUCCH format 4 resources.

- The maximum coding rate for slot-PUCCH format 4 transmission is configured by a higher layer signalling

##### Aspects related to DL control channel

R1-1806576	Corrections related to DL control channel for sTTI	LG Electronics

Decision: The text proposals in Sections 2 and 3 of R1-1806576 are endorsed.

R1-1807096	Aspects related to DL control channel	Qualcomm Incorporated

Decision: The text proposal in R1-1807096 is endorsed.

R1-1807317	Corrections on DAI in 36.212	Huawei, HiSilicon

R1-1807485	Corrections on DAI in 36.212	Huawei, HiSilicon	(Revision of R1-1807317)

Decision: The text proposal in R1-1807485 is endorsed.

##### Aspects related to UL control channel

R1-1807097	Aspects related to UL control channel	Qualcomm Incorporated

Decision: The proposal in Section 2 of R1-1807097 is endorsed.

R1-1807219	On UL TPC initialization after UL shorter TTI configuration - TP#9	Nokia, Nokia Shanghai Bell, LG Electronics

R1-1807545	On UL TPC initialization after UL shorter TTI configuration – TP#9	Nokia, Nokia Shanghai Bell, LG Electronics	(revision of R1-1807219)

Decision: The text proposals in R1-1807545 are endorsed.

Agreement:

Update a previous agreement as follows:

If a UE is (re)configured with sTTI operation for a serving cell, the initial value of fc(0) for slot/sublot PUSCH of a serving cell c is determined as the current value fc(*) for subframe PUSCH of serving cell c (i.e. fc,slot-sublot(0)=fc_subframe(*))the zero, and the UE shall reset accumulation, regardless of whether P_0_UE_PUSCH,c is changed or not.

Agreement:

Update a previous agreement as follows:

If a UE is (re)configured with sTTI operation for the primary cell, the initial value g(0) for SPUCCH on the primary cell is determined as the current (or latest) value g(*) of PUCCH of the primary cellzero, and the UE shall reset accumulation, regardless of whether P_0_UE_PUCCH is changed or not.

.

R1-1807297	Miscellaneous corrections to SPUCCH (36.213)	Ericsson, LG Electronics, Qualcomm

Decision: The text proposal in R1-1807297 is endorsed.

R1-1807298	Correction to the derivation of the number of bits for Rank Indication	Ericsson

Decision: The text proposal in R1-1807298 is endorsed.

R1-1807322	Correction on SPUCCH coverage	Huawei, HiSilicon, Qualcomm

##### Aspects related to DL data channel

R1-1806479	Correction on 256QAM for DL sTTI operation	Intel Corporation

Agreement:

Introduce a new signaling to configure the 256-QAM MCS table for sPDSCH transmissions (e.g., altCQI-Table-STTI).

R1-1807564	Correction on 256QAM for DL sTTI operation	Intel Corporation

Decision: The text proposal in R1-1807564 is endorsed

R1-1807426	Corrections related to DL data channel for sTTI	LG Electronics	(Revision of R1-1806577)

Decision: The text proposal in R1-1807426 is endorsed.

Agreement:

- Define UE capability on support of DMRS sharing in consecutive two subslots across subframe boundary for subslot-PDSCH.

- Define UE capability on support of back-to-back 3/4-layer DMRS reception in consecutive two subslots across subframe boundary for subslot-PDSCH.

R1-1807006	Clarification on Xp value determination for sTTI	InterDigital, Inc.

R1-1807098	Aspects related to DL data channel	Qualcomm Incorporated

R1-1807461	Aspects related to DL data channel	Qualcomm Incorporated	(Revision of R1-1807098)

Agreement:

Define separate optional UE capabilities for supporting 1024QAM for slot TTI/PDSCH, subslot TTI operation with TA set1 and subslot TTI operation with TA set 2.

Agreement:

A separate configuration of altCQI-Table-1024QAM parameter (e.g. altCQI-Table-1024QAM-STTI_r15) is supported for sTTI.

R1-1807562	Aspects related to DL data channel	Qualcomm Incorporated	(Revision of R1-1807461)

Decision: The text proposals in R1-1807562 are endorsed.

Agreement:

Introduce separate RRC configurations, tbsIndexAlt-STTI, tbsIndexAlt2-STTI and tbsIndexAlt3-STTI for the support of higher order modulation in the DL sTTI.

- tbsIndexAlt-STTI indicates the applicability of alternative TBS index 33A for PDSCHs scheduled by DCI format 7-1F and 7-1G.

- tbsIndexAlt2-STTI indicates the applicability of alternative TBS index 33B for PDSCHs scheduled by DCI format 7-1B, 7-1C and 7-1D.

- tbsIndexAlt3-STTI indicates the applicability of alternative TBS index 37A for PDSCHs scheduled by DCI format 7-1F and 7-1G.

R1-1807213	Enabling transmit diversity fallback for DL TM 6 for shorter TTI (36.212, 36.213) - TP#2	Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics, Qualcomm, Intel

Decision: The text proposals in R1-1807213 are endorsed.

R1-1807214	Enabling transmit diversity fallback for DL TM 8 for shorter TTI (36.212, 36.213) - TP#3	Nokia, Nokia Shanghai Bell, Ericsson

R1-1807515	Enabling transmit diversity fallback for DL TM 8 for shorter TTI (36.212, 36.213) - TP#3	Nokia, Nokia Shanghai Bell, Ericsson	(Revision of R1-1807214)

Decision: The text proposals in R1-1807515 are endorsed.

Agreement:

Support a UE capability for dynamic TX diversity fallback using port 7 & 8 for TM8 / DCI Format 7-1E.

R1-1807215	Enabling transmit diversity fallback for DL TM 9 and 10 for shorter TTI (36.211, 36.212, 36.213) - TP#4	Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm, LG Electronics

Decision: The text proposals in R1-1807215 are endorsed.

Agreement:

Support an independent higher layer configuration of semi-open loop operation for TM9/10 for shorter TTI PDSCH (e.g. example RRC parameter name semiOpenLoop-STTI).

R1-1807217	Corrections on DM-RS for slot PDSCH (36.211) - TP#6	Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, Qualcomm

The text proposals in R1-1807217 are endorsed

R1-1807315	Text proposal on TBS determination for downlink pattern DDD	Huawei, HiSilicon

Late submission

R1-1807430	Correction of transmit diversity precoding using port 7 & 8 for shorter TTI (36.211) – TP#47	Nokia, Nokia Shanghai Bell, LG Electronics

Decision: The text proposal in R1-1807430 is endorsed.

##### Aspects related to UL data channel

R1-1806449	Aspects related to UL data channel	Huawei, HiSilicon

R1-1806578	Corrections related to UL data channel for sTTI	LG Electronics

R1-1807468	Corrections related to UL data channel for sTTI	LG Electronics	(Revision of R1-1806578)

Decision: The text proposal in R1-1807468 is endorsed.

R1-1806877	UL DMRS pattern for subslot #5	ZTE

R1-1807099	Aspects related to UL data channel	Qualcomm Incorporated

Decision: The text proposals in Sections 3 and 4 in R1-1807099 are endorsed.

R1-1807212	Correction of number of UL HARQ processes for shorter TTI and shortened processing time (36.213) - TP#1	Nokia, Nokia Shanghai Bell, Ericsson

Decision: The text proposals in R1-1807212 are endorsed. Editor to correrct RRC parameter name shortTTI.

R1-1807299	Text proposal on TPC timing with DCI format 3/3A and short TTI operation	Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm

Decision: The text proposals in R1-1807299 are endorsed.

##### Aspects related to FS2

R1-1806356	Correction on HARQ timing issues for TDD specific sTTI	CMCC

R1-1806450	Aspects related to FS2	Huawei, HiSilicon

R1-1806878	UL index based scheduling flexibility for TDD UL/DL configuration #6 with slot-TTI	ZTE

Decision: The text proposals in R1-1806878 are endorsed.

R1-1807216	Changes to FS2 SRS triggering for shorter TTI (36.212, 36.213) - TP#5	Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, LG Electronics, Qualcomm

Decision: The text proposals in R1-1807216 are endorsed.

R1-1807218	Correction in 36.213 to TDD UL index based slot PUSCH scheduling  - TP#8	Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics

Decision: The text proposals in R1-1807218 are endorsed.

##### Other

R1-1806446	Remaining issues on SPS	Huawei, HiSilicon

R1-1806579	Corrections related to SPS for sTTI	LG Electronics

R1-1807312	Corrections on timing from sPDSCH to HARQ-ACK for SSC3,4,8	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, LG Electronics

Decision: The text proposal in R1-1807312 is endorsed.

R1-1807313	Corrections on HARQ-ACK reporting procedure for FS2 with sTTI	Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell, LG Electronics

Decision: The text proposal in R1-1807313 is endorsed.

R1-1807314	Text proposal on TBS determination for uplink DMRS position pattern DDD	Huawei, HiSilicon, Qualcomm

R1-1807316	Corrections on TBS determination for more than 1 layer	Huawei, HiSilicon, Ericsson

R1-1807514	Corrections on TBS determination for more than 1 layer	Huawei, HiSilicon, Ericsson	(Revision of R1-1807316)

Decision (Friday main online): The text proposals in R1-1807514 are endorsed.

R1-1807318	Additional DMRS position pattern for subslot #5	Huawei, HiSilicon, Qualcomm

R1-1807484	Text proposals on uplink DMRS position pattern DDD in subslot #5	Huawei, HiSilicon

Agreement:

Support of uplink DMRS position pattern ‘D D D’ in subslot #5 is a UE capability with application of the 1/6 as the TBS scaling factor.

R1-1807553	Text proposals on uplink DMRS position pattern DDD in subslot #5	Huawei, HiSilicon

Decision: The text proposals in R1-1807553 are endorsed with a modification to change “applied” to “applicable” in the TP in Section 2.

R1-1807319	Text proposal on UL resource allocation	Huawei, HiSilicon

Decision: The text proposal in R1-1807319 is endorsed.

Agreement:

An RB resource allocation offset of 1 or 2 PRBs optionally configured by RRC is introduced for sPUSCH

R1-1807321	On modulation for sTTI operation	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, LG Electronics

Agreement:

- Support of 256 QAM for subslot PUSCH is a UE capability.

- Support of 256 QAM for slot PUSCH is a UE capability.

- The 256 QAM subslot/slot PUSCH is independently configurable for each subframe set.

R1-1807513	On modulation for sTTI operation	Huawei, HiSilicon

Decision: The text proposals in R1-1807513 are endorsed.

R1-1807323	Corrections on HARQ-ACK reporting procedure for FS2 with sTTI	Huawei, HiSilicon

R1-1807490	Corrections on HARQ-ACK reporting procedure for FS2 with sTTI	Huawei, HiSilicon	(

Revision of R1-1807323)

### Maintenance of Release 15 Enhancements to LTE operation in unlicensed spectrum

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

R1-1807532	Chairman's notes of AI 6.2.2 Maintenance of Release 15 Enhancements to LTE operation in unlicensed spectrum	Ad-Hoc chair (Ericsson)

Decision: Contents incorporated below, the document is endorsed.

#### TPs for Multiple starting and ending positions in a subframe for UL

R1-1805865	Correction for multiple starting and ending positions	Huawei, HiSilicon

Decision: The text proposals in R1-1805865 is endorsed.

R1-1806676	Discussion on UCI transmission without UL-SCH for PUSCH mode 1	Samsung

Agreement:

UCI transmission without UL-SCH is not supported for PUSCH Mode 1.

R1-1807496	Text proposal on UCI transmission without UL-SCH for PUSCH mode 1	Samsung

R1-1807525	Text proposal on UCI transmission without UL-SCH for PUSCH mode 1	Samsung

Decision: The text proposals in R1-1807525 are endorsed. Editor can align wording for “PUSCH mode 1”.

R1-1807232	Discussion on channel access for UL transmission with Mode-1 under UE’s power limitation	WILUS Inc.

R1-1807450	TBS determination for partial UL subframes	Lenovo, Motorola Mobility	(Revision of R1-1807007)

Agreement:

When configured with partial subframe modes 2 and 3

- TBS Scaling is according to the scheduled subfame length if MCS 0-28 are indicated. TBS and modulation order are obtained as for the most recent transmissions granted with MCS 0-28 if MCS 30 is indicated.

R1-1807508	TBS determination for partial UL subframes	Lenovo, Motorola Mobility

Decision: The text proposals in R1-1807508 are endorsed.

#### TPs for Autonomous uplink access with Frame Structure type 3

R1-1805866	Corrections for AUL control channels	Huawei, HiSilicon

Decision: The text proposal in Section 2.1 of R1-1805866 is endorsed.

R1-1807540	Corrections and text proposals for AUL control channels	Huawei, HiSilicon

Decision: The text proposal in Section 2.2 is endorsed. The text proposal in Section 2.3 is endorsed with a modification to promote the bullet to main level.

R1-1806580	Discussion on remaining issues for AUL	LG Electronics

R1-1807502	Correction for spatial bundling in AUL-DFI	LG Electronics

R1-1807509	Corrections for spatial bundling in AUL-DFI	LG Electronics	(Revision of R1-1807502)

Decision: The text proposal in R1-1807509 is endorsed with a change of “should” to “shall” in the inserted text.

R1-1807510	Clarification for HARQ-ACK delivery to higher layers for AUL-DFI	Nokia

Decision: The text proposal in R1-1807510 is endorsed with a modification to change “autonomous transmission” to “autonomous uplink transmission”.

#### Other

R1-1806445	Corrections for channel access mechanism	Huawei, HiSilicon

Decision: The text proposal in R1-1806445 are endorsed.

R1-1807507	Capturing an agreement on back-to-back AUL-SUL transmissions in FeLAA	Broadcom

Decision: This text proposal can be discussed as part of the email approval for 37.213.

### Maintenance of Release 15 Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

R1-1087533	Chairman's notes of AI 6.2.3 Maintenance of Release 15 Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE	Ad-Hoc chair (Ericsson)

Decision: Contents incorporated below, the document is endorsed.

#### Text Proposals

R1-1807101	Corrections for 1024QAM	Qualcomm Incorporated

Agreement:

The text proposals in R1-1807101 are endorsed.

R1-1806480	Remaining issues of 1024QAM support in LTE	Intel Corporation

R1-1807493	Corrections for 1024QAM	Qualcomm Incorporated	(revision of R1-1807101)

Decision: The text proposal in Section 2 of R1-1807493 is endorsed.

The proposal in Section 1 of R1-1807493 is endorsed by RAN1.

R1-1807503	Draft LS on details of new categories for 1024QAM	Qualcomm Incorporated, Intel Corporation

Decision: The draft LS is endorsed and final version is approved in R1-1807549.

#### Other

R1-1807100	UE Categories for 1024QAM	Qualcomm Incorporated

### Maintenance of Release 15 Further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

R1-1807534	Chairman's notes of AI 6.2.4 Maintenance of Release 15 Further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE	Ad-Hoc chair (Ericsson)

Decision: Contents incorporated below, the document is endorsed.

#### Text Proposals

R1-1806448	Correction on QCL Type C for feCoMP in 36.213	Huawei, HiSilicon

R1-1807464	Text proposals on QCL Type C for feCoMP in 36.213	Huawei, HiSilicon, Intel Corporation

Decision: The text proposals in R1-1807464 are endorsed with a modification to the first proposal to change “….for Type C quasi co-location are not presented…” to “for Type C quasi co-location are not present…”.

### 3GPP V2X Phase 2 - WID in RP-171740

R1-1807535	Chairman’s notes of AI 6.2.5 3GPP V2X Phase 2	Ad-Hoc chair (Samsung)

Decision: Contents incorporated below, the document is endorsed with update.

Friday - eV2X CR

R1-1807414	Introduction of V2X Phase 2 enhancements into 36.201	Nokia, Nokia Shanghai Bell

Decision: CR0026 to 36.201 (Rel-15, B) is agreed.

R1-1807401	Introduction of V2X Phase 2 based on LTE	Huawei

Decision: Draft CR to 36.212 endorsed by email (May 12th) as result of email thread [92b-LTE-06-212].

R1-1807408	Introduction of eV2X	Ericsson

Decision: Draft CR to 36.211 endorsed by email (May 12th) as result of email thread [92b-LTE-06-211].

R1-1807411	Addition of V2X Phase 2 based on LTE in 36.213, s14	Motorola Mobility, LG Electronics	(revision of R1-1805991)

Decision: Draft CR to 36.213 endorsed by email (May 13th) as result of email thread [92b-LTE-06-213].

R1-1805823	Reply LS on PC5 transmission mechanism selection	SA2, LG Electronics

R1-1806557	[Draft] Reply LS on PC5 transmission mechanism selection	Huawei

R1-1807482	[Draft] Reply LS on PC5 transmission mechanism selection	LG Electronics

Decision: The LS to SA2/RAN2 is approved in R1-1807718 with the following change on the support of 64QAM:

- Support of 64QAM reception is mandatory for Rel-15 UEs supporting rate matching feature

- Support of 64QAM transmission is optional for Rel-15 UEs supporting rate matching feature

R1-1807544	UE capability for V2X Phase 2	Huawei, HiSilicon

Agreement

| Features | # | Feature group | Components | Prerequisite feature groups (listed in this sheet only) | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) | Consequences if the feature is not supported by the UE | Need of FDD/TDD differentiation | RAN5 implication | Note | Responsible WG | RAN WG recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| eV2X | 4-1 | Enhancements to resource allocation with CA | Resource selection enhancements for UE with limited TX capabilities |  | Yes |  | No |  |  |  | Optional |  |
|  | 4.2 | Synchronization enhancements for CA with single carrier SLSS transmission |  |  |  |  |  |  |  |  | Optional |  |
|  | 4.3 | Synchronization enhancements for CA with multiple carrier SLSS transmission |  |  |  |  |  |  |  |  | Optional |  |
|  | 4-4 | Support of 64-QAM RX |  | 4-6 |  |  |  |  |  |  | Mandatory |  |
|  | 4-5 | Support of 64-QAM TX |  | 4-6 |  |  |  |  |  |  | Optional |  |
|  | 4-6 | Support of rate matching and TBS scaling |  |  |  |  |  |  |  |  | Mandatory |  |
|  | 4-7 | Support of transmit diversity | (note: pending RAN4 specification) |  | Yes | single antenna port transmission is assumed by the UE | No |  |  |  | TBD |  |
|  | 4-8 | Support of low T2 min |  |  | Yes | Radio latency of V2X lower than 20ms cannot be guaranteed | No |  |  |  | Mandatory |  |

#### Carrier Aggregation (up to 8 PC5 carriers)

##### Mode-4 support

R1-1807460	Summary of Mode 4 CA	LG Electronics

R1-1807526	Summary of further offline discussion on A.I. 6.2.5.1.1	LG Electronics

Agreement

Carrier resource selection order is according to the ascending value of PPPP.

Agreement

- When random selection is configured by upper layers, resources within a selection window of a resource pool are considered as candidate resource set

- When random selection is configured by upper layers, for (a), (b), (c), option 1-1 is applied, otherwise, option 1-2 is applied.

Agreement

Additional resource exclusion procedure is specified in MAC layer spec

Agreement

When a UE with limited TX capability performs resource selection for a certain carrier, there could be ambiguity about the duration for which the current reserved resources of the other carriers are valid.

- Determining the duration for which the current resource reservation on other carriers is valid can be left to UE implementation

Conclusion

Do not specify enhancements to resource (re)selection triggering across aggregated carriers that specifically align simultaneous transmissions on multiple carriers to be on the same TTI.

R1-1805988	Remaining details for supporting carrier aggregation with mode-4	Huawei, HiSilicon

R1-1806098	Remaining issues on resource selection in mode 4 CA	ZTE

R1-1806126	Discuss the remaining issues for resource selection in mode 4 CA	ZTE Wistron Telecom AB, Sanechips

Withdrawn

R1-1806147	On mode-4 support for PC5 CA	Ericsson

R1-1806262	Remaining details on carrier aggregation for mode 4 in V2X phase 2	CATT

R1-1806350	Discussion on Mode 4 support for V2X carrier aggregation	Lenovo, Motorola Mobility

R1-1806481	Remaining physical layer aspects of sidelink carrier aggregation for mode-4 LTE V2V communication	Intel Corporation

R1-1806581	Discussion on carrier aggregation in sidelink mode 4 operation	LG Electronics

R1-1806677	mode-4 support in V2X CA	Samsung

R1-1806816	Remaining Issues on V2X Sidelink CA Scheduling issues in Mode 4	ITRI

R1-1806860	Remaining issues in eV2X carrier aggregation	OPPO

R1-1806994	Remaining Details for Mode 4 support for V2X carrier aggregation	Nokia, Nokia Shanghai Bell

R1-1807045	On carrier aggregation using mode 4 resource selection	NTT DOCOMO, INC.

##### Synchronization

R1-1807427	Summary of synchronization for PC5 CA	Ericsson

Agreement:

BW and TDD configuration and reserved bits are derived from CC where PSBCH and SLSS are transmitted.

Agreement:

When the UE has selected a synchronization reference other than Sync Ref UE (e.g., GNSS or eNB), selection of carrier for transmission of SLSS in Option 1 is up to UE implementation.

R1-1807522	Summary of synchronization reference priority order for PC5 CA	Samsung

R1-1806146	On Synchronization Aspects for PC5 CA	Ericsson

R1-1806263	Remaining details on synchronization for carrier aggregation in V2X Phase 2	CATT

R1-1806482	Remaining sidelink synchronization details for LTE V2V carrier aggregation	Intel Corporation

R1-1806582	Remaining issues on synchronization for sidelink CA	LG Electronics

R1-1806678	Synchronization in V2X CA	Samsung

R1-1806817	Remaining Issues on Synchronization for Sidelink Carrier Aggregation in V2X Phase 2	ITRI

R1-1806995	Remaining details of SL CA synchronization	Nokia, Nokia Shanghai Bell

R1-1807272	Remaining details on synchronization for CA in V2X Phase 2	ITL

##### Other

R1-1807155	On specification for PC5 CA	Ericsson

#### Support for 64-QAM

R1-1807459	Summary of offline discussion on support of 64-QAM for LTE V2V communication	Intel

Agreement:

The following working assumption is confirmed.

- Scaling factor is applied to the number of PRBs derived from SCI

  - Actual TBS value is derived by using scaled number of PRBs defined by the following equation

,NPRB=maxNPRB'∙αTBS,1

  - where  NPRB'is the original total number of allocated PRBs according to 7.1.6 from 3GPP 36.213 LTE R14.

- No new TBS values are introduced.

Agreement

- The following new ITBS entries are introduced for IMCS > 28

| IMCS | ITBS |
| --- | --- |
| … | … |
| 29 | 30 |
| 30 | 31 |
| 31 | 33 |

Agreement

- Use a single reserved bit in SCI Format 1 to indicate the new PSSCH transmission format (i.e. rate-matching of the last symbol of subframe, NPRB scaling for ITBS selection and modifications MCS/TBS table)

R1-1805987	Remaining details for supporting 64-QAM	Huawei, HiSilicon

R1-1806099	Discussion on support of 64QAM	ZTE

R1-1806125	Remaining detail on support for 64-QAM	ZTE Wistron Telecom AB, Sanechips

Withdrawn

R1-1806148	Remaining aspects of 64QAM	Ericsson

R1-1806264	Remaining details on 64QAM support in V2X phase 2	CATT

R1-1806483	Remaining details of demodulation enhancements for LTE V2V sidelink communication	Intel Corporation

R1-1806583	Discussion on 64QAM support	LG Electronics

R1-1806679	High order modulation in V2X	Samsung

R1-1806861	Support of 64 QAM	OPPO

R1-1806961	Indication of Transmission Format for Rel 15 Features	InterDigital, Inc.

R1-1806996	Remaining Details for Support of 64QAM for LTE V2X sidelink	Nokia, Nokia Shanghai Bell

#### Resource pool sharing between mode-3 and mode-4 users

R1-1807445	Summary of offline discussion for resource pool sharing between Mode-3 UE and Mode-4 UE NTT DOCOMO

Agreement

- No additional specification work to support resource pool sharing between Mode-3 UE and Mode-4 UE in RAN1 unless there is request from RAN2

R1-1805985	Solutions for Rel-14 and Rel-15 UE Pool sharing	Huawei, HiSilicon

R1-1806096	Further discussion on resource pool sharing between mode 3 and mode 4 UEs	ZTE Wistron Telecom AB, Sanechips, Fraunhofer HHI

Withdrawn

R1-1806100	Resource pool sharing between mode 3 and mode 4	ZTE

R1-1806149	Resource pool sharing between mode 3 and mode 4 UEs	Ericsson

R1-1806351	Discussion on Pool sharing for V2X Phase 2	Lenovo, Motorola Mobility

R1-1806484	On sidelink resource pool sharing for eNB-controlled and UE-autonomous V2V transmission modes	Intel Corporation

R1-1806574	Resource pool sharing between mode-3 and mode-4 users	NEC

R1-1806584	Discussion on resource pool sharing between UEs using mode 3 and 4	LG Electronics

R1-1806680	Resource pool sharing between mode 3 and mode 4 Ues	Samsung

R1-1806818	Remaining issues on Resource Pool Sharing for Mode 3 and Mode 4 V2X 	ITRI

R1-1806862	Discussion of resource pool sharing between mode 3 and mode 4	OPPO

R1-1806951	Discussion on resource pool sharing between UEs in mode 3 and UEs in mode 4	Panasonic

R1-1806962	Reducing resource collision between mode 3 and mode 4 in pool sharing	InterDigital, Inc.

R1-1806997	Resource pool sharing between mode-3 and mode-4 - remaining details	Nokia, Nokia Shanghai Bell

R1-1807046	Resource pool sharing between Mode-3 UE and Mode-4 UE	NTT DOCOMO, INC.

R1-1807220	Resource Pool Sharing between V2X Mode 3 and Mode 4 UEs	Fraunhofer HHI

#### Maximum time reduction between packet arrival at layer 1 and resource selection for transmission

R1-1807422	Summary on Maximum time reduction between packet arrival at layer 1 and resource selection for transmission	CATT

Agreement:

The minimum (pre-)configurable T2min is 10ms

R1-1805986	Remaining details for latency reduction for eV2X	Huawei, HiSilicon

R1-1806097	Further discussion on maximum time reduction between packet arrival at layer 1 and resource selection for transmission	ZTE Wistron Telecom AB, Sanechips

Withdrawn

R1-1806101	Consideration on maximum time reduction between packet arrival and resource selection	ZTE

R1-1806150	Reducing time-to-transmit for V2X	Ericsson

R1-1806265	Discussion on latency reduction between packet arrival and resource selection	CATT

R1-1806352	Discussion on Latency Reduction for V2X Phase 2	Lenovo, Motorola Mobility

R1-1806485	Remaining details of reduced resource selection time for LTE V2V sidelink communication	Intel Corporation

R1-1806585	Discussion on maximum time reduction between packet arrival and selected transmission resource	LG Electronics

R1-1806681	Support of shorter latency in V2X	Samsung

R1-1806819	Remaining issues on Latency Reduction for V2X	ITRI

R1-1807407	Latency reduction for eV2X	OPPO	(revision of R1-1806863)

R1-1806998	Discussion on latency reduction for eV2X	Nokia, Nokia Shanghai Bell

R1-1807047	Reducing the maximum time between packet arrival and selected resource for data transmission	NTT DOCOMO, INC.

#### Other

R1-1806486	UE Capabilities for LTE Rel.15 V2X Communication	Intel Corporation

Agreement

- Support of 64QAM reception is mandatory for Rel-15 UEs supporting rate matching feature

- Support of 64QAM transmission is optional for Rel-15 UEs supporting rate matching feature

Agreement

Transmission of physical parameter values set by ue-CategorySL-C-TX

| UE SL-C-TX Category | Maximum number of SL-SCH transport block bits transmitted within a TTI | Maximum number of bits of a SL-SCH transport block transmitted within a TTI | Maximum number of supported layers for spatial multiplexing in SL-C | Comments |
| --- | --- | --- | --- | --- |
| SL-C-TX Category 1 | 25456 | 25456 | 1 |  |
| SL-C-TX Category 2 | 31704 | 31704 | 1 | 20MHz aggregated BW 16QAM |
| SL-C-TX Category 3 | [32856+16416] | [32856] | 1 | 30MHz aggregated BW 16QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |
| SL-C-TX Category 4 | [57336] | [57336] | 1 | 20MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz and 20MHz) |
| SL-C-TX Category 5 | [57336+28336] | [57336] | 1 | 30MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |

Agreement

Reception of physical parameter values set by ue-CategorySL-C-RX

| UE SL-C-RX Category | Maximum number of SL-SCH transport block bits received within a TTI | Maximum number of bits of a SL-SCH transport block received within a TTI | Total number of soft channel bits | Comments |
| --- | --- | --- | --- | --- |
| SL-C-RX Category 1 | 25456 | 25456 |  |  |
| SL-C-RX Category 2 | 31704 | 31704 | 737280 | 20MHz aggregated BW 16QAM |
| SL-C-RX Category 3 | [57336] | [57336] | [995328] | 20MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz and 20MHz) |
| SL-C-RX Category 4 | [57336+28336 | [57336] | [1492992] | 30MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |

- No LBRM for Rel-15

Email discussion to finalize on the following by June 1st (Alexey, Intel)

- Transmission of physical parameter values set by ue-CategorySL-C-TX

- Reception of physical parameter values set by ue-CategorySL-C-RX

Email discussion by June 1st on V2X Sidelink Processing Capabilities – Indication per CC (Alexey, Intel)

| Component Carrier RX BW, MHz | V2X High Reception Capability | V2X Default Reception Capability |
| --- | --- | --- |
| 10 (Rel.15) | 10 PSCCHs and 68 RBs | 5 PSCCHs and 50 RBs |
| 20 (Rel.14, Rel.15) | 20 PSCCHs and 136 RBs | 10 PSCCHs and 100 RBs |

- If there is no CA, Rel.15 UE should be able to support Rel.14 UE capabilities

R1-1806950	Introducing sTTI to V2X mode 3	Panasonic

R1-1806999	eV2X UE capabilities	Nokia, Nokia Shanghai Bell

R1-1807000	Introduction of 10 ms sidelink SPS period	Nokia, Nokia Shanghai Bell

### Even further enhanced MTC for LTE - WID in RP-172811

R1-1807536	Chairman's notes of AI 6.2.6 even further enhanced MTC for LTE	Ad-Hoc chair (NTT DOCOMO, INC.)

Decision: Contents are incorporated below. The document is endorsed with updates.

Friday

R1-1807568	Introduction of Rel-15 LTE eMTC physical layer aspects	Ericsson

Decision: The draft CR (36.300, Rel-15, B) is endorsed.

R1-1807556	Introduction of even further enhanced MTC for LTE in 36.213, s00-s05	Motorola Mobility

R1-1807557	Introduction of even further enhanced MTC for LTE in 36.213, s06-s07	Motorola Mobility

R1-1807558	Introduction of even further enhanced MTC for LTE in 36.213, s08-s09	Motorola Mobility

Decision: The above 3 CRs are endorsed. Common understanding is to use these CRs as the basis for next update.

R1-1807310	Introduction of even further enhancements of eMTC	Huawei, HiSilicon

Decision: Draft CR to 36.212 endorsed by email (May 12th) as result of email thread [92b-LTE-07-212].

R1-1807409	Introduction of Even Further Enhanced MTC for LTE	Ericsson

Decision: Draft CR to 36.211 endorsed by email (May 14th) as result of email thread [92b-LTE-07-211].

R1-1807578	L1 parameter list of even further enhancements of eMTC	Ericsson

Decision: The list is agreed assuming the following update:

- Update H15 to “{40, 80, 160, 240} [ms]”

Final list in R1-1807861

R1-1807579	LS on L1 parameter list of even further enhancements of eMTC	Ericsson

Decision: The draft LS is endorsed (by updating the attachment to R1-1807861). Final LS is approved in R1-1807580.

#### Resynchronization signal

R1-1807431	Feature summary of Resynchronization signal for MTC	Ericsson

Agreement

The RSS may be located at any 2 consecutive PRBs throughout the system bandwidth. The lowermost RSS PRB is signaled in SI.

Agreement

(SFN_RSS mod P_RSS) = O_RSS. The first subframe of RSS is subframe 0 of SFN_RSS.

- Frame resolution of O_RSS is 5 bits

Agreement

The RSS duration is configurable among 8, 16, 32, 40 ms. The RSS duration configuration is signaled in SI.

Agreement

The bandwidth of the RSS is 2 PRBs.

The base sequence duration of RSS is 11 symbols, starting with the 4th symbol in the subframe.

Agreement

There is no signaling of RSS TX diversity configuration. If only one CRS port is configured by the eNB, the UE may assume that no transmit diversity is employed for the RSS (i.e., the whole RSS transmission is from the same antenna port); otherwise, the UE may assume that the transmission of RSS in subframes (2n, 2n+1) is from the same antenna port.

Agreement

- The RSS base sequence is based on QPSK mapped, 528-element Gold sequence

- Binary cover code indicates whether the base sequence S or its conjugate S* are transmitted in a subframe

R1-1807517	WF on Detailed RSS Design for efeMTC	Ericsson, Qualcomm, Sierra Wireless, Nokia, NSB, Sony

Agreement

- RSS also includes information according to MIB field systemInfoUnchanged-BR-R15 as set in the starting frame of the RSS transmission.

- The frequency-domain base sequence S is a QPSK sequence of length 264, generated from a Gold sequence cn of length 528 initialized by cinit=NIDcell+29u (where c(n) and cinit are defined in Section 7.2 of TS 36.211, and the update flag u follows systemInfoUnchanged-BR-R15):

Si=1-2c2i2+j1-2c2i+12,  i=0, 1,…, 263

- The sequence S is mapped frequency-first, time-second.

Agreement

- The cover codes to use for different durations are (0=S, 1=S*):

  - ms:{0, 0, 1, 0, 1, 1, 0, 0}

  - 16 ms:{0, 0, 1, 1, 0, 1, 0, 0, 0, 1, 1, 0, 0, 1, 0, 1}

  - 32 ms: {1, 1, 0, 0, 1, 0, 0, 1, 0, 1, 1, 1, 0, 0, 0, 1, 1, 1, 0, 1, 0, 1, 0, 0, 1, 0, 0, 0, 1, 1, 0, 1}

  - 40 ms {0, 1, 1, 0, 1, 1, 0, 0, 0, 1, 0, 1, 0, 0, 1, 1, 1, 0, 1, 1, 1, 0, 0, 0, 0, 1, 1, 1, 0, 1, 0, 0, 1, 1, 0, 1, 0, 1, 1, 0}

Agreement

Information about RSS TX boosting is signalled in SI among {0dB, 3dB, 4.8dB, 6dB} relative to CRS.

R1-1807569	WF on RSS timing offset for efeMTC	Ericsson, Qualcomm, Sierra Wireless, Sony, Intel

Agreement

RSS timing offset has the following resolution:

  - 160 ms: 1 frame

  - 320 ms: 1 frame

  - 640 ms: 2 frames, starting at SFN = 0

  - 1280 ms: 4 frames, starting at SFN = 0

R1-1805849	Resynchronization signal for MTC	Ericsson

R1-1805977	Cell search and system information acquisition improvements in eFeMTC	Huawei, HiSilicon

R1-1806002	Re-Sync Signal Design Considerations	Sierra Wireless, S.A.

R1-1806172	Resynchronization signal for efeMTC	Nokia, Nokia Shanghai Bell

R1-1806185	Remaining details on resynchronization signal for MTC	ZTE

R1-1806487	Resynchronization signal for efeMTC	Intel Corporation

R1-1806558	Remaining issues in RSS	Sony

R1-1806682	Discussion on enhanced synchronization signals and resynchronization signals	Samsung

R1-1807102	Reduced system acquisition time	Qualcomm Incorporated

#### Early data transmission

R1-1807	469	Feature lead summary of EDT in eMTC	Huawei, HiSilicon

Agreement

For CEModeA, 5 bits resource allocation to support 1~6PRBs in RAR UL grant for EDT.

For CEModeB, reuse Rel-13 legacy resource allocation in RAR UL grant for EDT.

Agreement

Scaled down proportionally to the nearest equal or higher legacy repetition number.

- New number can be considered during CR phase.

Agreement

For UL grant in MAC RAR scheduling EDT:

- In CEModeA,  bits for any Msg3/4 MPDCCH narrowband index in CEModeA.

- In CEModeB, 3 bits for Msg3/4 MPDCCH narrowband index, and 3 bits for Msg3 PUSCH narrowband index.

Agreement

DCI for Msg3 re-transmission can instruct physical layer in UE by using one unused state of MCS to indicate EDT Msg. 3 retransmission and the rest to indicate to fallback legacy Msg. 3 transmission.

Note: Spec. editor to choose one unused state of MCS

Note: Whether to introduce higher layer support for this mechanism is up to RAN2.

Agreement

Msg3 retransmission with EDT uses the same TBS as the initial transmission assuming repetition number indicated in the DCI for configured maximum TBS;

Agreement

The timing for monitoring DCI is based on the number of Msg3 PUSCH repetitions indicated in RAR/DCI

R1-1805850	Early data transmission for MTC	Ericsson

R1-1805978	Early data transmission for eFeMTC	Huawei, HiSilicon

R1-1806173	Data transmission during random access procedure	Nokia, Nokia Shanghai Bell

R1-1806186	Remaining issues on early data transmission for MTC	ZTE

R1-1806488	Early data transmission for efeMTC	Intel Corporation

R1-1806559	Transmission of EDT on Message 3	Sony

R1-1806586	Data transmission during random access procedure in MTC	LG Electronics

R1-1806683	Early data transmission for eMTC	Samsung

R1-1807103	Physical layer aspects of early data transmission	Qualcomm Incorporated

#### Wake-up signal

R1-1807444	Summary of 6.2.6.3 Wake-up signal	Qualcomm Incorporated

Agreement

- 2-bit scaling factor for the maximum WUS duration is selected in the range of {132,116,18,14}.

Agreement

- Define candidate values for actual WUS duration for UE blind detection

Agreement

- Higher-layer configured ce-wus-timeOffset-config is the non-zero gap from the end of the configured maximum WUS duration to the associated PO, as a number of absolute subframes.

  - FFS ce-wus-timeOffset-config could be different for DRX and eDRX.

- WUS is dropped in subframes that carry SIB1-BR and other SIs.

  - Note: this reverses the previous agreement of “WUS subframe is postponed when colliding with SIB1-BR PRBs”.

Note that Samsung has concern to unnecessarily increase in UE memory complexity to determine the start subframe of the WUS transmission.

Agreement

- WUS is punctured in CRS REs.

Agreement

- The UE reports one capability for "minimum gap between WUS and associated PO" for eDRX, selected among {Y1, Y2, …, YN}.

- There is no UE capability reporting for “minimum gap between WUS and associated PO” for DRX (i.e, the minimum gap is fixed in specification).

R1-1807528	Remaining issues of 6.2.6.3 Wake-up signal	Qualcomm

Agreement:

- The UE is not required to monitor the actual WUS duration other than power of 2 values from 1ms to max WUS duration.

Agreement: Delete the brackets of [Full] cell ID in the following agreement

- WUS signal is designed based on the following information

  - [Full] Cell ID information

  - UE group ID (if introduced)

  - time information of the starting subframe of the WUS or PO (Paging Occasion)

  - FFS: (part of) the SFN information

Agreement: WUS is in the same narrowband as the (first) PO to which it is associated.

Note: This precludes WUS frequency hopping

Agreement:

- There is no explicit signaling of WUS TX diversity configuration. If only one CRS port is configured by the eNB, the UE may assume the transmission of all WUS subframes is using the same antenna port; otherwise, the UE can assume the transmission of WUS in at least one subframe is using the same antenna port.

  - Note: the meaning of “at least” is clarified during the CR phase.

- UE shall not assume that the WUS is transmitted on the same antenna port as any of the downlink reference signals or synchronization signals.

Agreement: For UE capability:

- The UE reports one capability for "minimum gap between WUS and associated PO" for eDRX, selected among {Y1, Y2, …, YN}.

  - {Y1, Y2, …, YN}={40ms, 240ms, 1s, 2s}.

- There is no UE capability reporting for “minimum gap between WUS and associated PO” for DRX (i.e, the minimum gap is fixed in specification).

  - The “minimum gap between WUS and associated PO” is fixed as 40ms.

Agreement: For eNB configuration:

- For DRX, a gap duration is configured with a value range of {40ms, 80ms, 160ms, 240ms}.

- For eDRX, a short gap is configured with a value range of {40ms, 80ms, 160ms, 240ms} and another longer gap can be optionally configured with a value range of {1s, 2s}.

  - The short gap can be same or larger than the gap configured for DRX.

Agreement

- For eDRX, if a short gap (G1) and a longer gap (G2) are both configured, and the configured gap (G1 for Case 1 and G2 for Case2) is the same as “minimum gap between WUS and associated PO” reported by UE

| UE reported one capability of “minimum gap between WUS and associated PO” | UE expects the WUS with G1 | UE expects the WUS with G2 |
| --- | --- | --- |
| Case 1: {40ms or 240ms} | Y | N |
| Case 2: {1s or 2s} | N | Y |

- If the configured gap (G1 for Case 1 and G2 for Case2) is other than the UE reported capability of “minimum gap between WUS and associated PO”,

  - Case A: if the configured gap is larger than the UE reported capability of “minimum gap between WUS and associated PO”, UE monitors WUS with the configured larger gap

      - UE could wait until the end of the configured gap to detect paging.

  - Case B: if the configured gap is smaller than the UE reported capability of “minimum gap between WUS and associated PO”, UE does not monitor WUS with the configured long gap for eDRX and uses short gap to monitor WUS with the configured short gap.

Agreement

MTC WUS is 2 PRB WUS with the NB-IoT WUS repeated.

Agreement

One of {0, 2, 4} is signalled for the lowest PRB location of WUS in SI.

R1-1805851	Wake-up signal for MTC	Ericsson

R1-1805979	On wake-up signal for eFeMTC	Huawei, HiSilicon

R1-1806001	WUS Design Considerations	Sierra Wireless, S.A.

R1-1806031	Remaining aspects on wake-up signals for efeMTC	vivo

R1-1806174	Wake-up signal for efeMTC	Nokia, Nokia Shanghai Bell

R1-1806187	Discussion on wake-up signal for MTC	ZTE

R1-1806489	Power saving signal for efeMTC	Intel Corporation

R1-1806560	Remaining issues in WUS for efeMTC	Sony

R1-1806587	Discussion on wake up signal in MTC	LG Electronics

R1-1806684	DL power consumption reduction for eMTC	Samsung

R1-1806871	On wake-up precedure for efeMTC	OPPO

R1-1807104	Remaining issues on wake-up signals for MTC	Qualcomm Incorporated

#### PUSCH sub-PRB allocation

R1-1807428	efeMTC Key Issue Summary for PUSCH Sub-PRB Allocation	Sierra Wireless

Agreement: The two DMRS pi/2 BPSK sequences, R1(n) and R2(n) are calculated as:

R1(n) =    B(n)

R2(n) =    B(n) (-1) (n+Physical_Cell_ID)

B(n) is the same as the NB-IOT single tone DRMS sequence

Agreement: For sub-PRB transmissions in TDD, the same RU lengths supported in FDD mode are re-used.

Agreement

- For Sub-PRB, the number of MCS for CE Mode A and CE Mode B is 8 values

- For Sub-PRB, common TBS table is used for CE Mode A and CE Mode B

Agreement:

For DCI design for CE Mode A

Add new 2 bits “# of RUs” field to indicate # of RUs and legacy resource allocation according to the following table:

| 2-bit states | Interpretation |
| --- | --- |
| 00 | Legacy PRB level RA |
| 01 | RU number=1, Sub-RB RA |
| 10 | RU number=2, Sub-RB RA |
| 11 | RU number=4, Sub-RB RA |

When “# of RU” field=0 this means the grant is for a Full-PRB allocations, with the following legacy fields unchanged:

| Field Name | Bits | Description |
| --- | --- | --- |
| PRB Assignment | 5 | Legacy |
| MCS index | 4 | Legacy |
| Total Bits | 9 |  |

Otherwise sub-PRB allocation with the following fields:

| Field Name | Bits | Description |
| --- | --- | --- |
| Allocation of PRB within NB and SC within PRB | 6 | 6 PRBs within a NB and 10 values for the number of SC and their location within a PRB => 60 possible allocations within a NB. |
| MCS index | 3 | 8 values for MCS index |
| Total Bits | 9 |  |

For DCI design for CE Mode B

The PRB location within the narrowband is configured by RRC.

A new 1-bit field to indicate “Sub-PRB Allocation” is added to DCI.

When “Sub-PRB Allocation” field=0 this means grant is for a Full-PRB allocations, with the following legacy fields unchanged:

| Field Name | Bits | Description |
| --- | --- | --- |
| PRB Assignment | 3 | Legacy |
| MCS index | 4 | Legacy |
| Spare | 1 | New |
| Total Bits | 8 |  |

When “Sub-PRB Allocation” field=1 this means grant is for a sub-PRB allocations, with the following fields:

| Field Name | Bits | Description |
| --- | --- | --- |
| PRB Assignment | 0 | Configured by RRC |
| Allocation of SC within PRB | 4 | 10 values for the number of SC and their location within a PRB Six spare states |
| MCS index | 3 | 8 values for MCS index |
| Number of RUs | 1 | 2 or 4 |
| Total Bits | 8 |  |

Aperiodic CSI triggering is not supported for Sub-PRB allocations

Agreement: TBS table for CE Mode A and CE mode B

|  |  | NPRB |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 |  |  |  |
|  | 0 | 16 | 32 | 56 | 88 | 120 | 152 |  |  |  |
|  | 1 | 24 | 56 | 88 | 144 | 176 | 208 |  |  |  |
|  | 2 | 32 | 72 | 144 | 176 | 208 | 256 |  |  |  |
|  | 3 | 40 | 104 | 176 | 208 | 256 | 328 |  |  |  |
|  | 4 | 56 | 120 | 208 | 256 | 328 | 408 |  |  |  |
|  | 5 | 72 | 144 | 224 | 328 | 424 | 504 |  |  |  |
|  | 6 | 328 | 176 | 256 | 392 | 504 | 600 |  |  | 1 RU |
|  | 7 | 104 | 224 | 328 | 472 | 584 | 712 |  |  | 2 RUs |
|  | 8 | 120 | 256 | 392 | 536 | 680 | 808 |  |  | 4 RUs |
|  | 9 | 136 | 296 | 456 | 616 | 776 | 936 |  |  |  |
|  | 10 | 144 | 328 | 504 | 680 | 872 | 10321000 |  |  |  |

Agreement

- For Sub-PRB allocations, for CE mode A, ‘pusch-maxNumRepetitionCEmodeA’ is reused.

- For Sub-PRB allocations, for CE mode B, ‘pusch-maxNumRepetitionCEmodeB’ is reused.

- For sub-PRB allocation, the number of repetitions is calculated as Ceil(“DCI Repetition Number Field”/(RU_length*# of RUs))

R1-1805852	PUSCH sub-PRB allocation for MTC	Ericsson

R1-1805981	On Sub-RB resource allocation for MTC PUSCH	Huawei, HiSilicon

R1-1806000	Remaining Issues for PUSCH Sub-PRB Allocation	Sierra Wireless, S.A.

R1-1806175	Design of PUSCH Sub-PRB Allocation	Nokia, Nokia Shanghai Bell

R1-1806188	Remaining issues on PUSCH sub-PRB allocation for MTC	ZTE

R1-1806490	Design of sub-PRB PUSCH for efeMTC	Intel Corporation

R1-1806561	Remaining issues in Sub-PRB PUSCH transmission in efeMTC	Sony

R1-1806685	Discussion on sub-PRB allocation for eFeMTC	Samsung

R1-1806937	Remaining issues of PUSCH sub-PRB allocation	Sharp

R1-1807043	Views on PUSCH sub-PRB allocation for MTC	NTT DOCOMO, INC.

R1-1807105	Remaining issues for sub-PRB allocation	Qualcomm Incorporated

#### Flexible PDSCH/PUSCH starting PRB

R1-1807498	Summary on Flexible PDSCH/PUSCH starting PRB	Samsung

Agreement

For PDSCH in CE mode A, use all or some of the unused states in DCI format 6-1A to extend RA.

Agreement

For PDSCH in CE mode A, 10 new states are introduced to indicate new resource allocation as the table below.

| 10 states | Index of the starting PRB of the allocated resource (s is start PRB of RBG where the first PRB of the narrow band located; t = end PRB of RBG where the last PRB of the narrow band located.) | Number of PRBs allocated |
| --- | --- | --- |
| State 1 | s | 2 |
| State 2 | s | 3 |
| State 3 | s | 4 |
| State 4 | s | 5 |
| State 5 | s | 6 |
| State 6 | t-5 | 6 (ending at PRB t) |
| State 7 | t-4 | 5 (ending at PRB t) |
| State 8 | t-3 | 4 (ending at PRB t) |
| State 9 | t-2 | 3 (ending at PRB t) |
| State 10 | t-1 | 2 (ending at PRB t) |

Agreement

For PDSCH in CE mode B, introduce 1 bit in UE-specific RRC signaling to enable the shift of narrowband to align with RBG. The shift of narrowband depends on the system bandwidth and the allocated narrowband.

Agreement: The following shift of narrow band is used for PDSCH resource allocation in CE mode B.

| 1.4MHz | No shift |
| --- | --- |
| 3MHz | -1 PRB for NB {#0}; No shift for NB {#1}. |
| 5MHz | No shift for NB {#0, #1}; - 1PRB for NB {#2,#3}. |
| 10MHz | - 1 PRB for all NB. |
| 15MHz | -1 PRB for NB {#0, #1,…, #5}; No shift for NB {#6, #7,…, #11} |
| 20MHz | -2PRB for all NB. |

Agreement

For PUSCH in CE mode A, use DCI to provide more flexible resource allocation for PUSCH.

- FFS on use RA type 2 or use all or some of the unused 11 states in DCI format 6-0A to extend RA.

Agreement

For PUSCH in CE mode A and for the case where DL system BW and UL system BW is the same,  use resource allocation type 2 with starting PRB within the full system BW

Agreement

For PUSCH in CE mode B, adjust the starting PRB based on the resource allocation type 2 for 2 PRBs by {-1, 1, 2, 3} PRBs towards higher PRB index via RRC signaling.

Agreement

Flexible starting PUSCH PRB for BL/CE UE configured with 5 MHz max PUSCH channel bandwidth is not considered further in Rel-15.

R1-1805853	Flexible PDSCH/PUSCH starting PRB for MTC	Ericsson

R1-1805982	On more flexible starting PRB for PDSCH/PUSCH resource allocation	Huawei, HiSilicon

R1-1806176	Support for flexible PDSCH/PUSCH starting PRB	Nokia, Nokia Shanghai Bell

R1-1806189	Flexible starting PRB of PUSCH/PDSCH for MTC	ZTE

R1-1806562	On flexible PDSCH/PUSCH PRB allocation in efeMTC	Sony

R1-1806686	Discussion on Flexible PDSCH/PUSCH starting PRB	Samsung

R1-1807420	On more flexible PDSCH/PUSCH resource allocation	Sharp	(revision of R1-1806938)

R1-1807106	Flexible allocation for PDSCH and PUSCH	Qualcomm Incorporated

#### Other

Including any remaining details of HARQ-ACK feedback

R1-1807465	Summary of Uplink HARQ-ACK Feedback for MTC	ZTE

Agreement

For the UE-specific UL explicit HARQ-ACK feedback in CE Mode A, ACK is indicated by DCI format 6-0A where ‘Resource block assignment’ field is set to be all ‘1’s.

For the UE-specific UL explicit HARQ-ACK feedback in CE Mode B, ACK is indicated by DCI format 6-0B where ‘MCS’ field is set to be ’1111’.

R1-1805980	Remaining details of HARQ-ACK feedback for PUSCH in eFeMTC	Huawei, HiSilicon

R1-1806177	Remaining details of HARQ-ACK feedback for efeMTC	Nokia, Nokia Shanghai Bell

R1-1806190	Remaining issues on UL HARQ-ACK feedback for MTC	ZTE

R1-1806383	Remaining details of CRS muting for MTC	Ericsson

R1-1806588	Discussion on early termination of uplink repetitions for MTC	LG Electronics

R1-1807044	Views on UL HARQ-ACK feedback design for MTC	NTT DOCOMO, INC.

R1-1807245	Remaining issues in explicit uplink HARQ-ACK feedback	Sony

### Further enhancements of NB-IoT - WID in RP-172063.

R1-1807537	Chairman's notes of AI 6.2.7 further enhancements of NB-IoT Ad-Hoc chair (NTT DOCOMO, INC.)

Decision: Contents are incorporated below. The document is endorsed with updates.

Friday

R1-1807576	Introduction of Rel-15 NB-IoT physical layer aspects TS36.300	Huawei	(revision of R1-1806920)

Decision: The draft CR (36.300, Rel-15, B) is endorsed (removing change on changes), final endorsed version is in R1-1807860.

R1-1807559	Introduction of Rel-15 Further NB-IoT enhancements in 36.213, s14	Motorola Mobility

Decision: The above CR is endorsed. Common understanding is to use it as the basis for next update.

R1-1807410	Introduction Rel-15 Further NB-IoT enhancements	Ericsson

Decision: Draft CR to 36.211 endorsed by email (May 14th) as result of email thread [92b-LTE-10-211].

R1-1807309	Introduction of further enhancements of NB-IoT	Huawei, HiSilicon

Decision: Draft CR to 36.212 endorsed by email (May 14th) as result of email thread [92b-LTE-10-212].

R1-1807403	Introduction Rel-15 Further NB-IoT enhancements	Ericsson

Decision: Draft CR to 36.214 endorsed by email (May 12th) as result of email thread [92b-LTE-10-214].

R1-1807577	[DRAFT] Reply LS on physical layer aspects of Rel-15 NB-IoT and eMTC for TS 36.300	Huawei

Decision: The LS is endorsed (to update the attached tdoc #s).and final LS is approved in R1-1807858.

R1-1807573	L1 parameter list of further enhancements of NB-IoT	Huawei, HiSilicon

Decision: The list is agreed.

R1-1807574	LS on L1 parameter list of further enhancements of NB-IoT – Matthew

Decision: The LS is endorsed and final LS is approved in R1-1807575.

#### Latency and power consumption reduction

##### Power consumption reduction for paging and connected-mode DRX

###### Wake-up signal functions

R1-1807470	draft Feature lead summary on functions of power saving signal	Huawei, HiSilicon

Conclusion

The working assumption in RAN1#92 has been automatically confirmed base on the following agreement.

Agreements in RAN1#92:

Send LS Xiaolei- (HiSilicon) to RAN4 (CC: RAN2) on the feasibility of RRM measurement relaxation in the part on NB-IoT WUS.

If RAN4 confirms the feasibility of RRM measurement relaxation in the part on NB-IoT WUS, the working assumption is automatically confirmed.

Agreement

- Support WUS with synchronization of up to the timing/frequency offset resulting from not synchronizing due to the RRM measurement relaxation of N DRX cycles, where N is configured from the list of {1, 2, 4, 8} with the condition that N DRX cycles <= 10.24s.

- Configuration of N is cell specific

R1-1805854	Wake-up signal functions for NB-IoT	Ericsson

R1-1805983	On functions of power saving signal	Huawei, HiSilicon

R1-1805995	Wake Up Signal Functionsfor NB-IoT	MediaTek Inc.

R1-1806158	Functions of wake-up signal	Nokia, Nokia Shanghai Bell

R1-1806191	Discussion on wake up signal function for NB-IoT	ZTE

R1-1806491	The function scope of wake-up signal for feNB-IoT	Intel Corporation

R1-1807107	Further discusion of Wake-up signal functions	Qualcomm Incorporated

###### Wake-up signal configurations and procedures

R1-1807471	Feature lead summary of Wake-up signal configurations and procedures in NB-IoT	Huawei, HiSilicon

Agreement: The scaling factor between maximum duration of WUS and Rmax associated with type 1 CSS is selected from {1/128,1/64,1/32,1/16,1/8,1/4,1/2}.

Note that Ericsson has a concern about values of 1/128 and 1/2.

Agreement

Define candidate values for actual WUS duration for UE blind detection.

Agreement

The UE is not required to monitor the actual WUS duration other than power of 2 values from 1ms to the maximum WUS duration.

Agreement

- Higher-layer configured WUS_offset is the non-zero gap from the end of the configured maximum WUS duration to the associated PO, as a number of absolute subframes.

- FFS WUS_offset could be different for DRX and eDRX.

Agreement

- The UE reports one capability for "minimum gap between WUS and associated PO" for eDRX, selected among {Y1, Y2, …, YN}.

- There is no UE capability reporting for “minimum gap between WUS and associated PO” for DRX (i.e, the minimum gap is fixed in specification).

Agreement

- WUS is dropped in subframes that carry SIB1-NB and other SIs.

- WUS is postponed in subframes that are not NB-IoT DL subframes and does not carry SIB1-NB.

  - Note: this reverses the previous agreement of “WUS is postponed in subframes that are not NB-IoT DL subframes and is not transmitted [FFS dropped/postponed] in the subframes that carries SIs other than SIB1.”.

  - Note: “Postpone” means the corresponding subframes are not counted as configured maximum WUS transmission duration and actual WUS transmission duration.

  - Note: This does not imply that the minimum gap between the end of actual WUS duration and the first associated PO is reduced

Agreement

WUS is punctured in CRS REs for in-band deployment

Agreement

Sub-grouping is not supported in Rel-15.

Note that some companies observe the gains from sub-grouping.

R1-1807560	Updated feature lead summary of Wake-up signal configurations and procedures in NB-IoT	Huawei, HiSilicon

Agreement: For UE capability:

- {Y1, Y2, …, YN}={40ms, 240ms, 1s, 2s} for eDRX.

- The “minimum gap between WUS and associated PO” is fixed as 40ms for DRX.

Note; Qualcomm has concern that 40ms may not always include 10 valid subframes

Agreement: For eNB configuration:

- For DRX, a gap duration is configured with a value range of {40ms, 80ms, 160ms, 240ms}.

- For eDRX, a short gap is configured with a value range of {40ms, 80ms, 160ms, 240ms} and another longer gap can be optionally configured with a value range of {1s, 2s}.

  - The short gap can be same or larger than the gap configured for DRX.

Agreement

- For eDRX, if a short gap (G1) and a longer gap (G2) are both configured, and the configured gap (G1 for Case 1 and G2 for Case2) is the same as “minimum gap between WUS and associated PO” reported by UE

| UE reported one capability of “minimum gap between WUS and associated PO” | UE expects the WUS with G1 | UE expects the WUS with G2 |
| --- | --- | --- |
| Case 1: {40ms or 240ms} | Y | N |
| Case 2: {1s or 2s} | N | Y |

- If the configured gap (G1 for Case 1 and G2 for Case2) is other than the UE reported capability of “minimum gap between WUS and associated PO”,

  - Case A: if the configured gap is larger than the UE reported capability of “minimum gap between WUS and associated PO”, UE monitors WUS with the configured larger gap

      - UE could wait until the end of the configured gap to detect paging.

  - Case B: if the configured gap is smaller than the UE reported capability of “minimum gap between WUS and associated PO”, UE does not monitor WUS with the configured long gap for eDRX and uses short gap to monitor WUS with the configured short gap.

R1-1805855	Wake-up signal configurations and procedures for NB-IoT	Ericsson

R1-1805964	On configurations and procedures of power saving signal	Huawei, HiSilicon

R1-1805996	Wake Up Signal Configuration for NB-IoT	MediaTek Inc.

R1-1806032	Remaining aspects on configurations and procedures of wake-up signals for feNB-IoT	vivo

R1-1806159	Wake-up signal configurations and procedures	Nokia, Nokia Shanghai Bell

R1-1806192	Remaining issues on wake up signal configuration for NB-IoT	ZTE

R1-1806492	Configurations of wake-up signal for feNB-IoT	Intel Corporation

R1-1806589	Discussion on wake up signal configurations and procedures in NB-IoT	LG Electronics

R1-1806687	Discussion on Wake up signal configuraion	Samsung

R1-1806872	On wake-up procedure for feNBIoT	OPPO

R1-1807108	Remaining issues on Wake-up signal configurations and procedures	Qualcomm Incorporated

###### Detailed design of wake-up signal

R1-1807472	Feature lead summary on detailed design of Wake-up signal in NB-IoT	Huawei, HiSilicon

Agreement

- Gold sequence is used as the RE-level cover codes/RE-level scrambling sequence c(m), and phase shift θ=0.

Agreement

- For Guard Band and Standalone modes, the WUS signal in a subframe is generated as follows:

  - The WUS signal in the last 11 OFDM symbols are exactly the same as that under inband mode

  - Copy 3 OFDM symbol from the 8th, 9th, and 10th OFDM symbols to the first 3 OFDM symbols of the subframe

Agreement

- The UE can assume the transmission of WUS in at least one subframe use same antenna port.

  - Note: the meaning of “at least” is clarified during the CR phase.

- UE shall not assume that the WUS is transmitted on the same antenna port as any of the downlink reference signals or synchronization signals.

R1-1807561	Updated feature lead summary on detailed design of Wake-up signal in NB-IoT	Huawei, HiSilicon

Agreement

- c(m) is generated the same as the RE-level scrambling of Rel-14 NPDCCH/NPDSCH;

- c(m) shall be initialized at the start of WUS with

cinit_WUS=(NIDNcell+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDNcell

Agreement

- There is no explicit signaling of WUS TX diversity configuration. If only one NRS port is configured by the eNB, the UE may assume the transmission of all WUS subframes is using the same antenna port; otherwise, the UE can assume the transmission of WUS in at least one subframe is using the same antenna port.

R1-1805856	Detailed design of wake-up signal for NB-IoT	Ericsson

R1-1805965	On detailed design and evaluations of power saving signal	Huawei, HiSilicon

R1-1805997	Wake Up Signal Design for NB-IoT	MediaTek Inc.

R1-1806160	Considerations for design of wake-up signal	Nokia, Nokia Shanghai Bell

R1-1806193	Details design of wake up signal for NB-IoT	ZTE

R1-1806590	Wake up signal design in NB-IoT	LG Electronics

R1-1806688	Wake up signal Design	Samsung

R1-1807109	Detailed design of Wake-up signal sequence	Qualcomm Incorporated

##### Data transmission during the random access procedure

R1-1807473	Feature lead summary of EDT during RACH in feNB-IoT	Huawei, HiSilicon

Agreement

The repetition number of actual TBS（Ti）for initial transmission of Msg3 is determined by scaling the repetition number indicated in the corresponding RAR as following:

- The repetition number indicated in the corresponding RAR is set based on the broadcast maximum TBS;

- The scaling factor is the ratio of the actual TBS(Ti) and the broadcast maximum TBS for Msg3;

- The repetition number of the actual TBS is rounded up to the nearest higher integer number of L after the scaling;

Agreement

For EDT Msg3 transmission/retransmission using single-tone, pi/4 QPSK is used always.

Agreement

The parameter L for cyclic repetition is identical for all 4 actual TBS candidates.

- When the repetition number for the maximum TBS is equal or larger than 8, L = 4.

- When the repetition number for the maximum TBS is smaller than 8, L=1.

Agreement

- For a retransmission of msg3 carrying early data transmission, the UE uses the same TBS as previous transmission of Msg3 scheduled by RAR.

- Note: this does not exclude possibility that DCI can indicate fallback to transmit legacy Msg3;

Agreement

- For retransmission, if the UE selects an actual TBS smaller than the maximum TBS, the number of repetitions is down-scaled in the same way as for the initial transmission;

Agreement

- DCI format for scheduling retransmission of Msg3 can be used to indicate UE to transmit a legacy Msg3 to fallback to non-EDT Msg3 transmission when the indicated TBS is 88 bits;

  - Note: it is up to spec. editor which state to choose for EDT retransmission

  - Note: Whether to introduce higher layer support for this mechanism is up to RAN2.

Agreement

The timing for monitoring DCI is based on the number of Msg3 NPUSCH repetitions indicated in RAR/DCI

R1-1805857	Data transmission during random access procedure for NB-IoT	Ericsson

R1-1805968	Early data transmission in RACH for NB-IoT	Huawei, HiSilicon

R1-1805999	Fallback in Early Data Transmission	MediaTek Inc.

R1-1806161	Data transmission during random access procedure	Nokia, Nokia Shanghai Bell

R1-1806194	Remaining issues on early data transmission for NB-IoT	ZTE

R1-1806493	Early data transmission for feNB-IoT	Intel Corporation

R1-1806591	Data transmission during random access procedure in NB-IoT	LG Electronics

R1-1806689	Early data transmission for NB-IOT	Samsung

R1-1806939	Remaining issues of early data transmission for NB-IoT	Sharp

R1-1807110	Physical layer aspects of data transmission during random access procedure	Qualcomm Incorporated

R1-1807137	Discussion on remaining issues of EDT for NB-IoT	Xiaomi Communications

##### Physical layer scheduling request

R1-1807512	Feature lead summary of physical layer SR in feNB-IoT	Huawei, HiSilicon

Agreement

Confirm the WA on physical layer SR

Agreement

- Physical layer SR without HARQ-ACK does not convey BSR information in Rel-15;

- Physical layer SR without HARQ-ACK is by presence/absence of a NPRACH preamble of preamble format 0/1/2;

Note that NPRACH capacity enhancement may be considered in the future release.

Agreement

- For physical layer SR without HARQ-ACK, Non-contention-based RACH resource is used for SR transmission.

- The configuration is UE-specific as follows which is similar as that in NPDCCH order:

  - Carrier indication, value range is {0, 1, …, 15};

  - Subcarrier indication, value range is {0, 1, … , 47} for NPRACH format 0/1 and the value range is {0, 1, …, 143} for NPRACH format 2;

    - Note: Only the subcarriers within non-contention based RACH resource can be indicated.

  - Repetition number, value range reuses Table 16.3.2-1 in TS 36.213, i.e. {0, 1, 2, 3};

  - NPRACH format 0/1/2 indication is used to indicate which NPRACH resource is used.

R1-1805858	Physical layer scheduling request for NB-IoT	Ericsson

R1-1805966	Design for physical layer scheduling request	Huawei, HiSilicon

R1-1806162	Design of physical layer scheduling request	Nokia, Nokia Shanghai Bell

R1-1806195	Remaining issues on physical layer SR for NB-IoT	ZTE

R1-1806592	Discussion on Scheduling request in NB-IoT	LG Electronics

R1-1806690	Discussion on scheduling request for NB IoT	Samsung

R1-1807111	Physical layer scheduling request	Qualcomm Incorporated

##### Semi-persistent scheduling

R1-1807518	WF on the support of UL SPS in NB-IoT	Ericsson

Agreement

- UL SPS for BSR with skipUplink is introduced for RRC connected mode

  - Periodicity of UL SPS resource, SPS C-RNTI are configured via RRC signaling

  - SPS activation/deactivation is indicated by DCI scrambled by SPS C-RNTI on UE seach space

  - The TBS is 16 bits.

- Define separate UE capability signaling for UL SPS for BSR as optional

Agreement

The fields and their sizes in the DCI scrambled by SPS C-RNTI for UL BSR activation/release are the same as DCI format N0

Agreement

DCI for Semi-Persistent Scheduling Activation NPDCCH Validation

|  | DCI format N0 |
| --- | --- |
| Redundancy version | set to '0' |
| HARQ process number (if 2 HARQ processes are configured) | set to '0' |
| Modulation and coding scheme | set to ‘0000' |
| Resource assignment | Set to ‘000’ |
| Flag for format N0/format N1 differentiation | Set to ‘0’ |

Semi-Persistent Scheduling Release NPDCCH Validation

|  | DCI format N0 |
| --- | --- |
| Redundancy version | set to '0' |
| Repetition number | set to '000' |
| Subcarrier indication | Set to all '1's |
| Modulation and coding scheme | Set to ‘1111’ |
| HARQ process number (if 2 HARQ processes are configured) | set to '0' |
| Flag for format N0/format N1 differentiation | Set to ‘0’ |

R1-1807475	Feature lead summary for SPS in NB-IoT	Huawei, HiSilicon

R1-1805859	Semi-persistent scheduling for NB-IoT	Ericsson

R1-1805967	On support of semi-persistent scheduling	Huawei, HiSilicon

R1-1805998	UL SPS for Scheduling Request and BSR	MediaTek Inc.

R1-1806163	Support for semi-persistent scheduling in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1806196	Discussion on physical layer aspects on SPS in NB-IoT	ZTE

R1-1806691	New SPS mechanism for power saving	Samsung

R1-1807112	Support of SPS	Qualcomm Incorporated

#### NPRACH reliability and range enhancements

R1-1807429	Feature lead summary NPRACH reliability and range enhancements	Ericsson

##### Reliability enhancements for NPRACH

R1-1807479 	NPRACH false alarm reduction for NB-IoT	Ericsson	(Revision of R1-1805860)

R1-1805975	On NPRACH false alarm reduction due to inter-cell interference	Huawei, HiSilicon

R1-1806164	NPRACH reliability enhancement in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1806197	NPRACH reliability enhancement for NB-IoT	ZTE

R1-1806593	Remaining issues on reliability enhancements for NPRACH	LG Electronics

R1-1806692	Reliability enhancements for NPRACH	Samsung

R1-1807114	NPRACH Reliability Enhancement	Qualcomm Incorporated

##### Range enhancements for NPRACH

Agreement

Number of symbol groups per NPRACH preamble repetition is 6 symbol groups per NPRACH preamble repetition.

Agreement

Frequency hopping within one NPRACH preamble is 3-level -/+ 1.25 kHz -/+ 3.75 kHz 22.5 kHz.

Agreement

The parameters and their values for the type2-CSS configurations of RAR corresponding to the Rel-15 NPRACH resources remain the same as in legacy NPRACH.

Agreement

- Reuse the Rel-13 values for nprach-StartTime, numRepetitionsPerPreambleAttemp, npdcch-CarrierIndex, for Rel-15 NPRACH format 2 resources configurations

- Values for NPRACH periodicity are 40, 80, 160, 320, 640, 1280, 2560, 5120 ms

Agreement

Values for nprach-SubcarrierOffset and nprach-NumCBRA-StartSubcarriers are

- nprach-SubcarrierOffset: {0, 36, 72, 108, 6, 54, 102, 42, 78, 90, 12, 24, 48, 84, 60, 18} subcarriers.

- nprach-NumCBRA-StartSubcarriers: 4 bits

  - It is up to RAN2 to choose the exact subcarrier numbers

Agreement

- The new NPRACH format has:

  - 6 symbol groups per preamble.

  - Symbol-group level frequency hopping within one preamble

  - 1.25 kHz hopping gap for 1st to 2nd symbol group and with opposite direction for 5th to 6th symbol group.

  - 3.75 kHz hopping gap for 2nd to 3rd symbol group and with opposite direction for 4th to 5th symbol group.

  - 22. 5 kHz hopping gap for 3rd to 4th symbol group.

Agreement

- nprach-NumSubcarriers: {n36, n72, n108, n144}

- nprach-SubcarrierMSG3-RangeStart: same as legacy

R1-1805861	NPRACH range enhancements for NB-IoT	Ericsson

R1-1805974	NPRACH enhancement for cell radius extension	Huawei, HiSilicon

R1-1806165	NPRACH cell range enhancement in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1806198	Remaining issues on NPRACH range enhancement	ZTE

R1-1806594	Remaining issues on NPRACH range enhancement	LG Electronics

R1-1806693	Range enhancements for NPRACH	Samsung

R1-1807113	NPRACH support for large cell access	Qualcomm Incorporated

#### TDD

##### Downlink aspects

R1-1807466	Summary of DL aspects for TDD NB-IoT	ZTE

Agreement: For guard-band anchor, in-band non-anchor for SIB1-NB transmission can be samePCI mode.

Agreement

When SIB1-NB is transmitted on a non-anchor carrier and the anchor-carrier is guard-band mode, MIB-NB indicates:

- One of the following possible locations for the non-anchor NB-IoT carrier for SIB1-NB transmission is indicated in MIB-NB:

  - The NB-IoT carrier adjacent to anchor carrier in the same guard-band and to the outer side of the guard-band, i.e. the side away from the LTE carrier

  - The NB-IoT carrier in the opposite guard-band side and closest to the edge of the LTE carrier

  - The in-band PRB (samePCI=true) at the edge of the LTE carrier and on the same side as the anchor carrier.

    - For 5 MHz and 15 MHz LTE system bandwidth, the offset between the NB-IoT carriers is 45 kHz.

  - The in-band PRB (samePCI=false) at the edge of the LTE carrier and on the same side as the anchor carrier.

    - For 5 MHz and 15 MHz LTE system bandwidth, the offset between the NB-IoT carriers is 45 kHz.

- If non-anchor PRB is in the opposite guard-band side (symmetric guard-band) or in-band samePCI, the LTE bandwidth between 5 and 15MHz or between 10 and 20MHz

- For in-band samePCI=false case, the number of LTE CRS ports.

Agreement

For standalone anchors, the frequency offset between anchor and SIB1-NB non-anchor carrier is up to RAN4.

Note: there is no RAN1 specification impact.

Agreement

- For an anchor carrier, before obtaining the operation mode information (MIB-NB), the UE may assume NRSs are transmitted in SF #9 and SF #0 not containing NSSS.

- On an anchor carrier for SIB1-NB, before the UE obtains SIB1-NB, the UE may assume NRSs are transmitted in SF#0 not containing NSSS, SF #9, and SF #4 if SF #4 is configured for SIB1-NB transmission. After the UE obtains SIB1-NB, the UE may assume NRS are also transmitted in any other valid DL subframe, including special subframe in which NRS is supported

- On a non-anchor carrier for SIB1-NB, the UE may assume NRS are transmitted in SF #0 and SF #5 before the UE obtains SIB1-NB.

Agreement

Special subframes are not used for NPRS transmission in TDD NB-IoT.

Agreement

In normal DL subframes that are used for NPRS transmission, the NPRS generation is the same as NB-IoT FDD, including any revised NPRS introduced in FDD Rel-14.

Agreement

The Part A NPRS configuration in TDD NB-IoT is the same as FDD, except the bitmap length:

- 8 bits for 10 ms, 32 bits for 40 ms, assuming subframe #1 and #2 are always not used for NPRS.

Agreement

- For TDD NB-IoT, Part B OTDOA configuration is:

  - Extend the value of  to {40, 80, 160, 320, 640, 1280, 2560} subframes

  - Extend the value of  to {160, 320, 640, 1280, 2560} ms

  - is the same as FDD

Agreement

For NPDSCH without repetition, rate matching is used for RE mapping in the special subframe.

Agreement

- For NPDSCH with repetition/cyclic repetition,

  - The resource mapping in special subframe follows the regular full DL subframe, with the transmission on remaining symbols that are not part of DwPTS being punctured.

    - NPDSCH is punctured by NRS REs

  - Valid special subframes are counted for number of NPDSCH repetitions

Agreement

- For NPDCCH candidates without repetition, the rate matching per NPDCCH in a special subframe is done according to the available REs within DwPTS in each transmission subframe.

- For NPDCCH candidates with repetitions, the rate matching in special subframe follows the regular full DL subframe, with the transmission on remaining symbols that are not part of DwPTS being punctured.

Agreement

- For NPDCCH USS and Type 2 CSS,

  - the modified value of G is {4, 8, 16, 32, 48, 64, 96, 128}

  - the value set of  for FDD is resued in TDD NB-IoT.

Agreement

Confirm the working assumption at the 3GPP RAN1#92 meeting as

- For NPDCCH:

  - Transmission in DwPTS is supported for in-band when the number of OFDM symbols in DwPTS is greater than 3.

    - All DwPTS configurations are supported for NPDCCH

  - Transmission in DwPTS is supported for guard-band and standalone for all DwPTS configurations.

Agreement

The NPDCCH/NPDSCH starting position in special subframe is  in TDD LTE where l’DataStart is eturaControlRegionSize provided by system information

Agreement

dl-GapPeriodicity value range is doubled in TDD i.e. {128, 256, 512, 1024} subframes.

dl-GapThreshold and the dl-GapDurationCoeff are the same as FDD.

R1-1805862	DL aspects of TDD for NB-IoT	Ericsson

R1-1805970	On downlink TDD NB-IoT	Huawei, HiSilicon

R1-1806166	Downlink aspects of TDD support in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1806199	Remaining issues on downlink aspects to support TDD NB-IoT	ZTE

R1-1806348	Views on TDD downlink aspect	Lenovo, Motorola Mobility

R1-1806595	Discussion on DL aspects in TDD NB-IoT	LG Electronics

R1-1806694	Discussion on DL common channel/signal for TDD NB-IoT	Samsung

R1-1807115	Remaining issues on downlink aspects of TDD NB-IoT	Qualcomm Incorporated

R1-1807152	Design of DL aspects for TDD support in feNB-IoT	Intel Corporation

##### Uplink aspects

R1-1807467	Summary of UL aspects for TDD NB-IoT 	ZTE

Agreement

For TDD configuration 1 and 4, for NPUSCH format 1 and format 2 with 3.75 kHz subcarrier spacing, if one of the two consecutive UL subframes is configured invalid, postpone the NB-slot.

Agreement

DMRS location in FDD NB-IoT is reused for TDD NB-IoT.

For multi-tone RU(s), DMRS sequence in FDD NB-IoT is reused for TDD NB-IoT.

For single-tone RU(s), DMRS sequence in FDD NB-IoT is reused for TDD NB-IoT.

Agreement

NPRACH Format 0, 2, 0-a are supported for TDD UL/DL configuration 3.

Agreement

- For TDD NPRACH, the proposed configuration parameters are as following:

  - numRepetitionsPerPreambleAttempt, the value range is {1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024}

  - nprach-Periodicity( ), the value range is {80, 160, 320, 640, 1280, 2560, 5120, 10240}ms

  - The value range of nprach-StartTime ,, is 10, 20, 40, 80, 160, 320, 640, 1280, 2560, 5120}ms

  - TDD NPRACH transmission shall start at the first valid UL subframe that is  time units after the start of a radio frame fulfilling .

  - G symbol groups of NPRACH are dropped when there are not enough consecutive valid UL subframes to transmit G symbol groups back-to-back.

Agreement

For TDD NPRACH, the cell specific pseudo random hopping pattern is initialized with  cinit=NIDNcell.

Agreement

For NPRACH formats 0, 1 and 2, the followings are applied to random hopping:

- The tone index of 1st symbol group of 1st preamble repetition is randomly selected by MAC layer.

- The tone index of 1st symbol group and 3rd symbol group of each preamble repetition, except for the tone index of 1st symbol group of 1st preamble repetition, is selected according to the cell specific Pseudo random sequence.

Frequency hopping shall be used within the subcarriers.

Agreement

For NPRACH format 0-a, 1-a, the followings are applied to random hopping:

- The tone index of 1st symbol group of 1st preamble repetition is randomly selected by MAC layer.

- The tone index of 1st symbol group and 4th symbol group of each preamble repetition, except for the tone index of 1st symbol group of 1st preamble repetition, is selected according to the cell specific Pseudo random sequence.

Frequency hopping shall be used within the subcarriers.

Agreement

The value range of ack-NACK-NumRepetitions and ack-NACK-NumRepetitions-Msg4 in TDD NB-IoT is same as FDD NB-IoT.

Agreement

2-bit signaling indicates offset between UL PRB center with respect to DL PRB center to be either 7.5kHz, -7.5kHz, or 0 kHz

R1-1805863	UL aspects of TDD for NB-IoT	Ericsson

R1-1805971	On uplink TDD NB-IoT	Huawei, HiSilicon

R1-1806167	Uplink aspects of TDD support in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1806200	Remaining issues on uplink aspects to support TDD NB-IoT	ZTE

R1-1806596	Discussion on UL aspects in TDD NB-IoT	LG Electronics

R1-1806695	Discussion on UL channel for TDD NB-IoT	Samsung

R1-1806940	Remaining issues of uplink aspects of TDD NB-IoT	Sharp

R1-1807116	Remaining issues on uplink aspects of TDD NB-IoT	Qualcomm Incorporated

##### Common aspects

E.g. Relaxations of MCL, latency, capacity targets; UL:DL configurations, special subframe, HARQ, etc.

R1-1807563	Summary of TDD Common aspects	LG Electronics	(revision of R1-1807527	R1-1807489)

Agreement

- No new UL/DL configurations for standalone operation mode are supported in TDD NB-IoT

Agreement

- Confirm the working assumption to not support UL/DL configuration #6 in Rel-15.

Agreement

- Do not map transmissions to DwPTS and UpPTS in special subframe configuration 10 in Rel-15.

Agreement

For downlink scheduling delay, valid special subframes which include DwPTS to which NPDSCH can be mapped are counted as a part of the scheduling delay k0.

Agreement

- Uplink scheduling delays for NPUSCH format 1 for 3.75kHz and 15kHz, are 8 absolute subframes + k0 valid uplink subframes, where a set of k0 is {0, 8, 16, 32}

- Uplink scheduling delays for NPUSCH format 2 for 3.75kHz and 15kHz, are 12 absolute subframes + k0 valid uplink subframes, where k0 is the same as described in Tables of 16.4.2-1 and 16.4.2-2 respectively in TS36.213, subtracting 12 from the existing values

- For TDD configuration 1 and 4, for NPUSCH format 1 and format 2 with 3.75 kHz subcarrier spacing, if the indicated starting subframe is the 2nd of the two contiguous UL subframes, postpone the NB-slot.

Agreement

- Single-HARQ capable UE does not support UL/DL interlaced transmission and reception

Agreement

- The 1 ms minimum gap from end of NPUSCH format 1 for one HARQ process to NPDCCH for the other HARQ process is not specified for TDD NB-IoT.

Agreement

- For UE configured with 2 HARQ processes, the UE is not required to monitor NPDCCH 2ms before the start of the earliest NPDSCH until the end of the latest NPUSCH format 2

Agreement

- For UE configured with 2 HARQ processes, the UE is not required to monitor NPDCCH 2ms before the start of the first scheduled NPUSCH until the end of the latest NPUSCH format 1

Agreement

- For UE configured with 2 HARQ processes, an explicit guard time is not defined for the minimum gap between NPDCCH/NPDSCH reception and NPUSCH transmission

Agreement

For UE configured with 2 HARQ processes the minimum gap between NPUSCH transmission and NPDCCH/NPDSCH reception is as follows:

- In the case of in-band operation mode, an explicit guard time is not defined

- For 15 kHz SCS, in the case of guard-band and standalone operation modes, UE is allowed to skip at least part of the first OFDM symbol in a downlink subframe right after an uplink subframe where the UE performs uplink transmission.

  - Clarify which part in CR phase

Agreement

For UE configured with 1 HARQ process, minimum timing relationships between channels in TDD NB-IoT are the same as FDD NB-IoT

Agreement

- The number of soft channel bits for Cat. NB1 and Cat. NB2 is the same as Rel-13/Rel-14 (e)NB-IoT FDD systems.

Agreement

- The Rel-14 FDD interference randomization based on RE level rotation is applied to downlink channels for both anchor and non-anchor carrier in TDD NB-IoT.

R1-1805864	DL/UL common aspects of TDD for NB-IoT	Ericsson

R1-1805969	Common aspects for TDD NB-IoT	Huawei, HiSilicon

R1-1806168	Common Aspects of NB-IoT TDD Operation	Nokia, Nokia Shanghai Bell

R1-1806201	Remaining issues on common aspects to support TDD NB-IoT	ZTE

R1-1806349	Views on TDD common aspect	Lenovo, Motorola Mobility

R1-1806597	Discussion on common aspects in TDD NB-IoT	LG Electronics

R1-1806696	Discussion on 2 HARQ processes and cross carrier scheduling	Samsung

R1-1807117	Remaining issues on common aspects of TDD NB-IoT	Qualcomm Incorporated

R1-1807153	Design of common aspects for TDD support in feNB-IoT	Intel Corporation

R1-1807271	Scheduling Aspects of TDD NB-IoT	III

#### Other

Including extended PHR related aspects and any other aspects necessary to complete the WI.

R1-1807542	Way forward on remaining details of NB-IoT measurement improvements	Huawei, HiSilicon

Agreement

- Based on the RAN4 confirmation, confirm the working assumptions on use of NSSS for RRM

- The NSSS-NRS EPRE ratio is signaled per neighbor cell

- nsss-NumOccasionDifferentPrecoder = {2, 4, 8}

- NPBCH for RRM is not specified until or unless it is confirmed by RAN4

  - Send an LS to RAN4 to communicate this situation and strongly encourage them to send an LS to RAN1 by the end of RAN1#94

  - This implies the current 36.214 text is to be removed.

Draft LS on NPBCH for RRM measurements (R1-1807570) to RAN4 – Matthew (Huawei)

R1-1807570	[DRAFT] LS on NPBCH for RRM measurements	Huawei

Decision: The document is endorsed and final LS is approved in R1-1807572.

R1-1805972	Remaining details of NB-IoT measurements improvement	Huawei, HiSilicon

R1-1805973	On the support of NB-IoT small cell	Huawei, HiSilicon

R1-1806169	PHR Enhancement for NB-IoT	Nokia, Nokia Shanghai Bell

R1-1806170	Measurement accuracy improvement in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1806171	Small cell support in NB-IoT	Nokia, Nokia Shanghai Bell

R1-1806202	Considerations on extended Power Headroom report for NB-IoT	ZTE

R1-1806203	UL power control enhancement for NB-IoT small cell	ZTE

R1-1806880	Cell search time reduction for NB-IoT	Ericsson

R1-1806881	System information acquisition time reduction for NB-IoT	Ericsson

R1-1806911	Narrowband measurement accuracy improvements for NB-IoT	Ericsson

R1-1806912	Small-cell support for NB-IoT	Ericsson

R1-1807118	Coexistence with NR	Qualcomm Incorporated

R1-1807119	Enhancements to cell search and system information acquisition	Qualcomm Incorporated

R1-1807120	Improvement of PHY measurements	Qualcomm Incorporated

### Ultra Reliable Low Latency Communication for LTE - WID in RP-172845

R1-1807538	Chairman's notes of AI 6.2.8 Ultra Reliable Low Latency Communication for LTE	Ad-Hoc chair (Ericsson)

Decision: Contents incorporated below, the document is endorsed.

R1-1806555	Missing agreement on system level assumptions for LTE URLLC	Ericsson

Agreement:

For system level simulations, the system bandwidth on the UL is equally split between the number of UEs simulated. Each UE in each TTI/sTTI will be allocated 10 RBs (assuming 10 UE per sector and 100 RB system bandwidth) in a proportional fair fashion.

Note: This does not impact the RB allocations assumed for the link level simulations

R1-1807491	Draft LS on Ultra Reliable Low Latency Communication for LTE	Qualcomm	(Revision of R1-1807486)

Decicion: The LS is approved in R1-1807492 with the following modifications

- Modify the action to say “RAN1 kindly requests RAN2 to take the above agreements into account. RAN1 also kindly requests RAN2 to enable multiple UL SPS configurations for a TTI length for a serving cell.

- Remove the title in the “Work Item” field.

R1-1807494	Draft LS on RRC parameter list for Ultra Reliable Low Latency Communication for LTE	Ericsson

Decision: The document is endorsed and final LS is approved in R1-1807519.

R1-1807511	Notes of the offline session for LTE URLLC	Ericsson

(Response to R1-1805816 treated in AI 5)

R1-1807303	Reply LS on RLM Qin/Qout Targets for URLLC	Ericsson

R1-1807516	[Draft] Reply LS on RLM Qin/Qout Targets for URLLC	Nokia

Final LS agreed in R1-1807520

R1-1807523	Notes of the offline session for LTE URLLC	Ericsson	(Revision of R1-1807511)

R1-1807554	Notes of the offline session for LTE URLLC		Nokia, Nokia Shanghai Bell

#### Blind/HARQ-less Repetition for Scheduled DL-SCH Operation

- Finalise details of RAN1 agreement to support blind/HARQ-less PDSCH repetition.

- Using legacy (S/E)PDCCH, (S)PUCCH formats (if applicable); any discussion of potential DCI modifications is limited to support of blind/HARQ-less repetition

R1-1805867	Blind/HARQ-less Repetition for Scheduled DL-SCH Operation	Huawei, HiSilicon

R1-1805993	Blind/Harq-less repetitions for scheduled DL-SCH in LTE HRLLC	MediaTek Inc.

R1-1806385	On remaining details of blind/HARQ-less PDSCH repetition	Nokia, Nokia Shanghai Bell

R1-1806494	Remaining details of PDSCH repetitions	Intel Corporation

R1-1806598	Discussion on blind/HARQ-less PDSCH repetition for URLLC	LG Electronics

R1-1806697	Discussion on PDSCH repetition	Samsung

R1-1806769	Remaining issues on PDSCH repetition for LTE URLLC	ZTE

R1-1807121	Blind and HARQ-less Repetition for Scheduled DL-SCH Operation	Qualcomm Incorporated

R1-1807273	PDSCH aspects of LTE-URLLC	Motorola Mobility, Lenovo

R1-1807302	Remaining details on URLLC techniques for PDSCH	Ericsson

R1-1807452	Notes of the offline session on PDSCH repetition for LTE URLLC	Ericsson

Agreement:

For PDSCH repetitions, the number of PDSCH transmissions, k, associated with the PDCCH starting with the current TTI can take the following values and is indicated in a 2-bit DCI field as follows.

| DCI bit field | Value of k for subframe, slot and subslot |
| --- | --- |
| 00 | 1 |
| 01 | 2 |
| 10 | 3 |
| 11 | a value configured over RRC from {4, 6} |

Agreement:

For PDSCH repetition, a field of 2 bits is amended to the PDSCH related DCI (i.e. subframe PDSCH DCI and DCI 7-1x).

Change the previous agreement as follows:

- Blind/HARQ-less PDSCH repetitions are enabled by RRC configuration. If configured, a single DCI format is used to schedule PDSCH with a given TTI length. There is a field in the DCI that indicates the number of PDSCH transmissions k associated with the DCI, where k >= 1.

- If configured for slot or subslot PDSCH, a single DCI format containing this field is used to schedule slot or subslot PDSCH

- If configured for subframe PDSCH, the UE (as in legacy) continues to monitor for DCI Format 1A and the DL TM specific DCI format. The field is included only in DCI format 1A on USS

Agreement:

For slot/subslot PDSCH repetition, the rate-matching around SPDCCH resources for the PDSCH transmissions within a repetition window follows the rate-matching around SPDCCH resources for the first PDSCH transmission of the repetition window.

Agreement:

- For TDD, if a UE is scheduled with K repetitions for slot PDSCH, the UE assumes the PDSCH to be present in the next available K DL slots

  - UL slots are not counted in the repetition count

- For TDD, if a UE is scheduled with K repetitions for subframe PDSCH, the UE assumes the PDSCH to be present in the next available K DL subframes or flexible subframes

  - UL subframes are not counted in the repetition count

- In case different DL transmission modes are configured for MBSFN and non-MBSFN subframes for slot or subslot PDSCH, a UE is only expecting PDSCH repetitions up to the boundary between MBSFN and non-MBSFN subframes in case of a larger indicated K (larger than the number of remaining TTIs of the same subframe type).

- For TM10 blind/HARQ-less PDSCH repetition, the UE is to assume the same PQI to be applied to all PDSCH repetitions.

Agreement:

- Slot & subslot PDSCH repetition across subframe boundaries is supported

  - For subslot PDSCH, in case sTTI#0 cannot contain PDSCH (i.e CFI=1 or 2 / PDCCH length of 2 or 3) then sTTI#0 is not counted as valid transmission/repetition and the next repetition resumes from sTTI#1

Agreement:

- When a UE is configured with a semi-static CFI for a given TTI length, the UE is not expected to decode PCFICH for that TTI length.

Agreement:

- PDSCH repetitions associated with a single DL assignment assume the same RB allocation.

Agreement:

For slot/subslot PDSCH repetition and DMRS based PDSCH, the UE may assume that the same precoder is kept between PDSCH repetitions. For TDD, the UE cannot assume coherent DMRS channel estimation across a UL/DL switching point.

- Note: this allows the UE to do coherent channel estimation filtering.

Conclusion:

Agreements referring to PDCCH in the LTE_HRLLC WI are intended to refer to the DL assignment which may be via PDCCH/SPDCCH/EPDCCH on a slot/subslot/subframe as applicable.

Agreement:

For subslot PDSCH repetitions DMRS sharing is not supported for blind/HARQ-less PDSCH repetition with k > 1, i.e., DMRS is present in each subslot.

- For k > 1, the value of DMRS position field is fixed by specification to 0

Agreement:

For slot/subslot PDSCH, with k > 1, the maximum transmission rank is 2 for reception of PDSCH.

Agreement:

For PDSCH repetitions, different RV can be used in different PDSCH transmissions within the repetition window. (i,.e. RV cycling is supported).

Agreement:

For PDSCH repetitions and RV cycling, the RV field in the DCI scheduling a repetition sequence of k PDSCH transmissions identifies the starting RV in the cycling sequence that is used for the first transmission of PDSCH within the repetition window.

Agreement:

For PDSCH repetitions and RV cycling, the following options are considered.

- The RV cycling sequence applied for PDSCH repetition when PDSCH repetition is configured is configured by higher layer signalling and selected between {0, 0, 0, 0} or {0, 2, 3, 1}.

Agreement:

- The number of HARQ processes for PDSCH repetition applicable when k > 1 can be configured. The value range is 1 to the maximum number of DL HARQ processes.

- The x MSB of the 5-bit MCS field for PDSCH repetitions and k>1 are set to 0. The value of x is configurable and the value range is {0,1}

Agreement:

Confirm the working assumption: For HARQ for repeated PDSCH transmissions, the UE shall report HARQ feedback with the timing given by the last PDSCH repetition.

#### Repetition Enhancements for UL SPS Operation

Secondary priority (best effort only). Finalise details to support UL SPS repetition configuration (both sTTI and TTI)

R1-1805868	Repetition Enhancements for UL SPS Operation	Huawei, HiSilicon

R1-1805994	UL SPS in LTE HRLLC	MediaTek Inc.

R1-1806457	On UL SPS enhancements for URLLC	Nokia, Nokia Shanghai Bell

R1-1806599	Repetition Enhancements for UL SPS Operation	LG Electronics

R1-1806698	Discussion on Repetition for UL SPS	Samsung

R1-1806770	On UL SPS repetition for LTE URLLC	ZTE

R1-1807122	Repetition Enhancements for UL SPS Operation	Qualcomm Incorporated

R1-1807166	PUSCH repetitions enhancements for UL SPS	Intel Corporation

R1-1807301	Analysis of Uplink SPS solutions for URLLC	Ericsson

R1-1807521	Summary on HRLLC UL DMRS sharing	Huawei, HiSilicon

R1-1807529	Proposal on UCI transmission for collision with UL SPS repetition	LG Electronics

R1-1807456	Notes of the offline session on UL SPS repetition for LTE URLLC	Ericsson

Agreement:

The total number of UL transmissions that can be configured for UL SPS repetition is from {2, 3, 4, 6}.

Agreement:

The following UL SPS repetition scheme with SPS periodicity P is supported with N SPS configurations on a serving cell for subframe/slot/subslot operation, where N<=6. The support of the UL SPS repetition scheme in the RAN1 specifications is subject to RAN2 defining the support of multiple SPS configurations on a serving cell in this WI.

The following is defined per SPS configuration.

- The total number of UL transmissions K obtained with the UL SPS repetition scheme is RRC configured and can take any value from {2, 3, 4, 6} that satisfies K <= P.

- The initial transmission of a TB shall start at the first transmission occasion of the K transmissions. Single HARQ process/TB per transmission window.

- RV sequence is configurable and can take one of the following values

  - Value 1: {0, 0, 0, 0, 0, 0}

  - Value 2: {0, 2, 3, 1, 0, 2}

  - Value 3: {0, 3, 0, 3, 0, 3}

- The Cyclic shift for UL DMRS is configured by RRC (3-bit CS).

- When N > 1 is configured for UL SPS, the 3 LSB of the bit field for HARQ process ID in the SPS activation/deactivation DCI indicate which SPS configuration is activated/deactivated.

- The UE shall not transmit PUSCH simultaneously using different UL SPS configurations on a serving cell.

Agreement:

For UL SPS repetition with subframe/slot/subslot operation with multiple UL SPS configurations on a serving cell for a given TTI length:

- In one TTI only one UL SPS configuration can be activated

- The initial transmission of a TB for different UL SPS configurations is on different TTIs.

- Different HARQ process IDs are used for different UL SPS configurations.

Agreement:

Separate UE capability is defined for the support of UL SPS repetitions for subframe, slot and subslot.

FFS: Whether UE indicates the maximum number of supported UL SPS configurations per serving cell or across all serving cells.

Agreement:

For UL SPS repetition with subslot operation, the TBS scaling factor of subslot PUSCH transmitted using an UL SPS configuration is configurable per UL SPS configuration and can take a value from {1/6, 1/12} independently of the number of PUSCH data symbols.

R1-1807495	Proposal on collision handling for UL SPS repetition	LG Electronics

Agreement:

In case of collision between non-SPS PUSCH and a SPS PUSCH (that is part of a SPS repetition window) of the same TTI length in the same subframe/slot/subslot on a given carrier, the UE shall transmit non-SPS PUSCH transmission and drop the colliding SPS PUSCH transmission.

- Note: Not necessarily captured in RAN1 specifications

Agreement:

For UL SPS repetition with subframe/slot/subslot operation with multiple UL SPS configurations on a serving cell for a given TTI length:

- Each UL SPS configuration is activated by individual SPS activation DCI.

- The UE is not expected to receive more than one activation DCI in a TTI.

Agreement:

UL SPS repetition across subframe boundary is supported

Agreement:

If the UE is configured with UL SPS and the configured number of SPS PUSCH transmission k>1, simultaneous transmission of PUSCH and PUCCH is not configured.

Agreement:

In case of collision between (S)PUCCH and a SPS PUSCH (that is part of a SPS repetition window) of the same TTI length in the same subframe/slot/subslot on a given carrier,

- the UE shall transmit SPS PUSCH transmission and drop/stop the (S)PUCCH transmission

  - HARQ-ACK of (S)PUCCH is transmitted via the SPS PUSCH that collides with (S)PUCCH.

  - CSI of (S)PUCCH, if any, is dropped.

Agreement:

In case of collision between SPS subframe-PUSCH (that is part of a SPS repetition window) and a non-SPS slot/subslot-PUSCH in the same subframe on a given carrier, the UE shall transmit non-SPS slot/subslot-PUSCH transmission and drop the colliding subframe SPS PUSCH transmission.

Agreement:

In case of collision between SPS subframe-PUSCH (that is part of a SPS repetition window) and a slot/subslot-PUCCH in the same subframe on a given carrier, the UE shall transmit slot/subslot-PUCCH transmission and drop the colliding subframe SPS PUSCH transmission.

Agreement:

For each UL subslot SPS repetition configuration, the UL DMRS pattern for UL SPS is reused.

Agreement:

From RAN1 perspective to guarantee the number of repetitions, on a serving cell an ongoing UL SPS repetition transmission should not be interrupted by another UL SPS repetition configuration having new data arriving (i.e. UE should wait with a SPS transmission of another TB till having finished the repetitions of an ongoing UL SPS repetition transmission)

- Note: This may not have RAN1 specification impact.

#### Other

R1-1806386	On remaining details of CFI configuration operation	Nokia, Nokia Shanghai Bell

R1-1806451	UE capability on PDSCH/PUSCH repetition	Huawei, HiSilicon

R1-1806452	CFI configuration for LTE URLLC	Huawei, HiSilicon

R1-1806556	Summary of the email discussion [92b-LTE-13] on UE capability for LTE_HRLLC	Ericsson

R1-1807300	RLM for URLLC	Ericsson

### Other

Including any leftovers from Enhanced LTE Support for Aerial Vehicles, Enhancing CA Utilization, modulation enhancements, 2Tx/4Tx SRS switching, CP latency reduction, etc.

R1-1807539	Chairman's notes of AI 6.2.9 Others	Ad-Hoc chair (Ericsson)

Decision: Contents are incorporated below. The document is endorsed with updates.

Modulation Enhancements

R1-1805992	Modulation Enhancements		MediaTek Inc.

R1-1806387	On TEI-15 LTE Modulation Enhancements	Nokia, Nokia Shanghai Bell

R1-1806495	On MCS selection for LTE	Intel Corporation

R1-1807124	Discussion on modulation enhancements	Qualcomm Incorporated

R1-1807400	Rel-15 LTE modulation enhancements	Ericsson

R1-1807571	Conclusion for modulation enhancements	Qualcomm Incorporated

Agreement:

For the solution to modulation enhancements for PDSCH, introduce a 6-bit MCS table as below:

- For the entries with “Scaling = NO”, the UE follows legacy procedure to determine the TBS.

- For the entries with “Scaling = YES”, the UE selects the TBS by scaling the number of allocated PRBs by a factor α, where α is RRC configured

  - Note: This is applied in addition to any other applicable scaling factors

Email approval to decide the value range of α, the contents for rows 42-58, and whether entries 56-58 have scaling applied or not, agree to CR, and send LS to RAN2 until June 7th, 2018	Qualcomm (Alberto)

| MCS Index | Modulation Order | Modulation Order | TBS Index | Scaling |
| --- | --- | --- | --- | --- |
| 0 | 2 | 2 | 0 | No |
| 1 | 2 | 2 | 1 | No |
| 2 | 2 | 2 | 2 | No |
| 3 | 2 | 2 | 3 | No |
| 4 | 2 | 2 | 4 | No |
| 5 | 2 | 4 | 5 | No |
| 6 | 2 | 4 | 6 | No |
| 7 | 2 | 4 | 7 | No |
| 8 | 2 | 4 | 8 | No |
| 9 | 2 | 4 | 9 | No |
| 10 | 4 | 6 | 9 | No |
| 11 | 4 | 6 | 10 | No |
| 12 | 4 | 6 | 11 | No |
| 13 | 4 | 6 | 12 | No |
| 14 | 4 | 6 | 13 | No |
| 15 | 4 | 6 | 14 | No |
| 16 | 4 | 6 | 15 | No |
| 17 | 6 | 6 | 15 | No |
| 18 | 6 | 6 | 16 | No |
| 19 | 6 | 6 | 17 | No |
| 20 | 6 | 6 | 18 | No |
| 21 | 6 | 6 | 19 | No |
| 22 | 6 | 6 | 20 | No |
| 23 | 6 | 6 | 21 | No |
| 24 | 6 | 6 | 22 | No |
| 25 | 6 | 6 | 23 | No |
| 26 | 6 | 6 | 24 | No |
| 27 | 6 | 6 | 25 | No |
| 28 | 8 | 8 | 25 | No |
| 29 | 8 | 8 | 27 | No |
| 30 | 8 | 8 | 28 | No |
| 31 | 8 | 8 | 29 | No |
| 32 | 8 | 8 | 30 | No |
| 33 | 8 | 8 | 31 | No |
| 34 | 8 | 8 | 32 | No |
| 35 | 8 | 8 | 33A | No |
| 36 | 8 | 8 | 33/33B | No |
| 37 | 10 | 10 | 34A | No |
| 38 | 10 | 10 | 35 | No |
| 39 | 10 | 10 | 36 | No |
| 40 | 10 | 10 | 37A | No |
| 41 | 10 | 10 | 37 | No |
| 42 |  |  |  | Yes |
| 43 |  |  |  | Yes |
| 44 |  |  |  | Yes |
| 45 |  |  |  | Yes |
| 46 |  |  |  | Yes |
| 47 |  |  |  | Yes |
| 48 |  |  |  | Yes |
| 49 |  |  |  | Yes |
| 50 |  |  |  | Yes |
| 51 |  |  |  | Yes |
| 52 |  |  |  | Yes |
| 53 |  |  |  | Yes |
| 54 |  |  |  | Yes |
| 55 |  |  |  | Yes |
| 56 |  |  |  | No/Yes |
| 57 |  |  |  | No/Yes |
| 58 |  |  |  | No/Yes |
| 59 | 2 | 2 | Reserved |  |
| 60 | 4 | 4 |  |  |
| 61 | 6 | 6 |  |  |
| 62 | 8 | 8 |  |  |
| 63 | 10 | 10 |  |  |

Control Plane Latency Reduction

R1-1806266	Further Discussion on Control Plane Latency Reduction in LTE	CATT

R1-1806267	Introduction of further reduced control plane latency	CATT

Antenna Selection

R1-1806447	On support of PUSCH antenna selection for 1T4R and 2T4R antenna configurations	Huawei, HiSilicon

R1-1807123	Capability for SRS antenna switching	Qualcomm Incorporated

R1-1807501	Introduction of 1T2R PUSCH antenna selection for 1T4R antenna ports configuration in TS36.213		Huawei, HiSilicon

Decision: The draft CR is Merged to R1-1807504 CR1075r1.

R1-1807504	Introduction of SRS antenna switching for 4 ports in TS36.213	Qualcomm, Huawei, HiSilicon

Decision: Revision of R1-1805441 (CR1075) including R1-1807501 – CR1075r1 to 36.213, Rel-15, B is agreed.

Agreement:

Send a LS to RAN2 to inform them of the configuration parameters that were introduced in CR1075 and ask them to introduce the following independent UE capabilities for SRS antenna switching per BoBC:

- Support of SRS-Antenna-Switching-1T4R: Indicates the UE supports 1T4R in the corresponding band of combination.

- Support of SRS-Antenna-Switching-2T4R with SRS-Antenna-Switching-2T4R-NrofPairs = 2: Indicates the UE supports 2T4R with two antenna pairs (Λ=2 in TS 36.213, Clause 8.2) for the corresponding band of band combination.

- Support of SRS-Antenna-Switching-2T4R with SRS-Antenna-Switching-2T4R-NrofPairs = 3: Indicates the UE supports 2T4R with three antenna pairs (Λ=3 in TS 36.213, Clause 8.2) for the corresponding band of band combination.

R1-1807505	Draft LS on parameters and capabilities for SRS antenna switching	Qualcomm

Decision: The document is endorsed and final LS is approved in R1-1807551.

CBSR

R1-1806955	Draft LS on introduction of advanced CSI CBSR and associated UE capability	Ericsson

Decision: The final LS is approved in R1-1807457 with a modification to change “new IE” to “new field” in the last line of Section 1 of the draft LS.

R1-1806956	Introduction of advanced CSI CBSR in TS 36.213	Ericsson

R1-1807497	Introduction of advanced CSI CBSR in TS 36.213	Ericsson

Decision: Final CR is agreed in R1-1807506 (TS36.213, CR 1097, Rel-15, B) with a modification so that the summary of change field reads as “Addition of two types of CBSR mechanisms for advanced CSI reporting with and without amplitude subset restriction”

Aerial

R1-1807154	On UL power control enhancements for LTE Aerials	Ericsson

Decision: The text proposals in R1-1807154 are endorsed

EuCA

R1-1807291	Short CQI Reporting for EuCA	Nokia, Nokia Shanghai Bell

R1-1807552	Short CQI Reporting for EuCA	Nokia, Nokia Shanghai Bell	(Revision of R1-1807291)

Decision: The CR (TS36.213, CR 1100, Rel-15, B) is agreed.

Others

R1-1806453	Discussion on PRG size for TM9 and TM10	Huawei, HiSilicon

R1-1806455	Introduction of configured PRG size for TM9 and TM10 in 36.213	Huawei, HiSilicon

R1-1807418	Support of CRS muting	Qualcomm Incorporated, MediaTek	(Revision of R1-1807125)

# NR

R1-1806890	Correction on scheduling and HARQ procedure for case 1 HARQ timing of EN-DC in TS36.213	Huawei, HiSilicon

Above CR to be treated under coexistence – 7.1.5

## Maintenance of NR - WID in RP-172834

Only essential corrections. Notes:

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

- Editors are encouraged to submit editor’s CR before the meeting reflecting additional comments received after the endorsed & frozen NR specifications. If submitted, please submit under agenda 7.

R1-1806496	Removal of SRS-RSRP and RSSI measurements from TS 38.215	Intel Corporation

Propose to delete SRS-RSRP and RSSI measurements from Rel-15 TS 38.215 due to lack of follow up work to finalize details of the corresponding measurements. The TP for the proposed changes is also provided.

Decision: The TPs in R1-1806496 are endorsed.

R1-1807242	CR to 38.214: maintenance according to agreed Rel 15 features 	Nokia

Remove the multi-TRP specification text according to RAN plenary guidance in RP-172108.

Nokia's suggestion is to take it under MIMO session

Decision: The document is noted. Companies are encouraged to check R1-1807242, which is to be handled under 7.1.2.

R1-1806573	Miscellaneous Corrections to TS38.213	Editor (Samsung)

The document is for information.

Friday

Editor to prepare CRs (capturing agreements and working assumptions): drafts to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

### Initial access and mobility

#### Synchronization signal

R1-1807607	Summary of 7.1.1.1 Synchronization signal	Ericsson

Decision: The document is noted.

Proposal:

- For connected mode, no explicit EPRE offset between SSS and PDCCH is specified by RAN1 in Rel-15.

  - Note: the EPRE offset may be implicitly derived from other parameters such as SSB vs. CSI-RS & CSI-RS vs. PDCCH as specified in 38.214

Wednesday

Conclusion:

- It is understood that the resource grid of Nsize, ugrid, x is the same as the CarrierBandwidth in 38.331

  - Up to editor how to capture

Thursday

R1-1807775	Summary of 7.1.1.1 Synchronization signal	Ericsson

Decision: The document is noted.

Agreement: Update the previous agreements as follows:

- In initial cell selection and idle mode, the UE may assume that the EPRE offset between PDCCH DMRS and SSS in the QCL’ed SSB is in the range [X, Y]dB when PDDCH with CRC scrambled by SI-RNTI, P-RNTI and RA-RNTI, where X=-8 & Y=8

  - FFS: value of X and Y and dependence on numerology

    - |X| & Y are no more than 12

Agreement:

- Adopt the following text proposal to 38.211 section 7.4.3.1 below:

---- Start of TP ----

The quantity  is the subcarrier offset from subcarrier 0 in common resource block  to subcarrier 0 of the SS/PBCH block, where the 4 least significant bits of  are given by the higher-layer parameter ssb-SubcarrierOffset and for SS/PBCH block type A the most significant bit of  is given by  in the PBCH payload as defined in subclause 7.1.1 of [4, TS 38.212]. When ssb-SubcarrierOffset is not provided, is derived from the frequency difference between the SS/PBCH block and Point A.

---- End of TP ----

Friday

R1-1807862	Summary of 7.1.1.1 Synchronization signal	Ericsson

Decision: The document is noted.

Agreements:

- For connected mode, no explicit EPRE offset between SSS and PDCCH is specified by RAN1 in Rel-15.

  - Note: the EPRE offset between SSS and PDCCH may be implicitly derived from other parameters, e.g. power offset of SSB to NZP CSI-RS and power offset of NZP CSI-RS to PDCCH. This does not mandate any gNB behaviour.

Agreement: k0μ is computed using

k0μ=Ngrid,xstart,μ+Ngrid,xsize,μ/2NscRB -Ngrid,xstart,μ0+Ngrid,xsize,μ0/2NscRB2μ0-μ

where  μ0 is the largest subcarrier spacing configured.

Send an LS to RAN2 to inform the above agreements and removal of parameter k_0. Email approval by May 30th – Abjorn (Ericsson)

R1-1805871	Remaining details on synchronization signal	Huawei, HiSilicon

R1-1805941	Remaining details of Synchronization Signal Design	ZTE

R1-1806033	Remaining issues on synchronization signal	vivo

R1-1806268	Remaining Issues on Synchronization Signal	CATT

R1-1806406	Remaining issues for synchronization signals	Spreadtrum Communications

R1-1806419	Remaining details on Synchronization signal	Ericsson

R1-1806497	Remaining issues in SSB	Intel Corporation

R1-1806600	Remaining Details on Synchronization signal	LG Electronics

R1-1806673	Remaining details of synchronization signal and signal generation	NEC Corporation

R1-1806699	Corrections on Synchronization Signaling	Samsung

R1-1806782	Power offset between PDCCH DMRS and QCL'ed SSS	MediaTek Inc.

R1-1806830	Power offset between SS/PBCH block and PDCCH	OPPO

R1-1807008	On Remaining Details of NR Synchronization Signal 	InterDigital, Inc.

R1-1807049	Remaining issues on Synchronization signal	NTT DOCOMO, INC.

R1-1807170	Open issues related to Synchronisation Signal	Nokia, Nokia Shanghai Bell

R1-1807328	Remaining details on synchronization signal design	Qualcomm Incorporated

R1-1807880	k0 for OFDM signal generation	Huawei

#### Broadcast signal/channel

##### NR-PBCH

R1-1807608	Summary of 7.1.1.2.1 NR-PBCH	Ericsson

Decision: The document is noted.

Agreement:

- Adopt the following change “…to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters.” to sections 7.4.1.1.2, 7.4.1.3.2 and 7.4.3.1 in 38.211.

Thursday

R1-1807776	Summary of 7.1.1.2.1 NR-PBCH	Ericsson

Decision: The document is noted.

Agreement:

- For TDD, the UE may assume that the SFN is aligned among cells of the same frequency layer

Agreements:

- Adopt to below TP on 38.213, section 13:

---- Start of TP ----

If a UE detects a first SS/PBCH block and determines that a control resource set for Type0-PDCCH common search space is not present, and for  for FR1 or for  for FR2, the UE may determine the global synchronization channel number (GSCN) of a second SS/PBCH block having a control resource set for an associated Type0-PDCCH common search space as .  is the GSCN of the first SS/PBCH block and  is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2. If a UE detects the second SS/PBCH block at  according to the indication in the first SS/PBCH block and determines that a control resource set for Type0-PDCCH common search space is not present as specified in sub-clause 4.1, UE may ignore the assistance information related to GSCN of SS/PBCH locations in its SS/PBCH block search.

If a UE detects a SS/PBCH block and determines that a control resource set for Type0-PDCCH common search space is not present, and for  for FR1 or for  for FR2, the UE may assume determines that there is no SS/PBCH block having an associated Type0-PDCCH common search space within a GSCN range . and  are respectively determined by the four most significant bits and the four least significant bits of RMSI-PDCCH-Config. If UE has not detected within a reasonable time any SS/PBCH block with the associated control resource set for Type0-PDCCH common search space being present, UE may ignore the assistance information related to GSCN of SS/PBCH locations in its SS/PBCH block search.

---- End of TP ----

R1-1805872	Remaining details of NR PBCH design	Huawei, HiSilicon

R1-1805942	Remaining details of NR-PBCH Design	ZTE

R1-1806347	Correction on QCL of SS blocks	Lenovo, Motorola Mobility

R1-1806420	Remaining details of PBCH	Ericsson

R1-1806601	Remaining Details on PBCH design and contents	LG Electronics

R1-1806700	Corrections on NR-PBCH	Samsung

R1-1806873	Remaining issues on NR-PBCH	OPPO

R1-1807009	On Remaining Details of NR Physical Broadcast Signal and Channel 	InterDigital, Inc.

R1-1807050	Remaining issues on NR-PBCH	NTT DOCOMO, INC.

R1-1807171	Remaining details related to SS/PBCH block	Nokia, Nokia Shanghai Bell

R1-1807329	Remaining details on NR-PBCH	Qualcomm Incorporated

##### Remaining minimum system information

R1-1806270	Summary of Offline Discussion on RMSI	CATT

Decision: The document is noted.

Tuesday

R1-1807692	Summary of Offline Discussion on RMSI	CATT

Decision: The document is noted.

Agreement:

- NR supports 2-bits RV field in the DCI with SI-RNTI for CSS type 0 and CSS type 0a.

Wednesday

R1-1807735	Summary of Offline Discussion on RMSI	CATT

Decision: The document is noted.

Proposal:

- Make the modification on Table 13-2 of TS 38.213 as suggested in R1-1807735.

Agreements:

- To adopt the following TP to Section 10 of 38.213:

---- Start of TP for 38.213 -----

For monitoring of a PDCCH candidate

If the UE has received ssb-PositionsInBurst in SystemInformationBlockType1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if monitoring for PDCCH candidate is not for a Type0-PDCCH common search space and at least one RE for monitoring a PDCCH candidate for a DCI format with CRC not scrambled by SI-RNTI on the serving cell overlaps with respective at least one RE corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SystemInformationBlockType1, the UE is not required to monitor the PDCCH candidate.

-	If a UE has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if monitoring for PDCCH is not for a Type0-PDCCH common search space and at least one RE for monitoring a PDCCH candidate for a DCI format with CRC not scrambled by SI-RNTI on the serving cell overlaps with respective at least one RE corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate.

-	If the has not received both ssb-PositionsInBurst in SystemInformationBlockType1 and ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if the UE monitors the PDCCH candidate for a Type0-PDCCH common search space on the serving cell according to the procedure described in Subclause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell.

---- End of TP for 38.213 -----

Thursday

R1-1807811	Summary of Offline Discussion on RMSI	CATT

Decision: The document is noted.

Agreement:

- Make the following modification on Table 13-2 in TS 38.213.

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 5 |
| 1 | 1 | 24 | 2 | 6 |
| 2 | 1 | 24 | 2 | 7 |
| 3 | 1 | 24 | 2 | 8 |
| 4 | 1 | 24 | 3 | 5 |
| 5 | 1 | 24 | 3 | 6 |
| 6 | 1 | 24 | 3 | 7 |
| 7 | 1 | 24 | 3 | 8 |
| 8 | 1 | 48 | 1 | 18 |
| 9 | 1 | 48 | 1 | 20 |
| 10 | 1 | 48 | 2 | 18 |
| 11 | 1 | 48 | 2 | 20 |
| 12 | 1 | 48 | 3 | 18 |
| 13 | 1 | 48 | 3 | 20 |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Agreements:

- Adopt Option 2 in LS R1-1805712 for supporting the RMSI CORESET configuration for {SSB SCS, minimum channel bandwidth} = {15kHz, 10MHz}.

- Send a reply LS to RAN4 informing RAN1’s decision to adopt Option 2 – R1-1807819

- Adopt the corresponding TP (Section 13, 38.213) as follows:

---- Start of TP for 38.213 -----

Table 13-1: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

Table 13-2: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

---- End of TP for 38.213 -----

R1-1807819	[Draft] Reply LS on RMSI CORESET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth	CATT

Decision: The document is endorsed and final LS is approved in R1-1807831.

R1-1805873	Remaining details on RMSI configurations	Huawei, HiSilicon

R1-1805943	Remaining details of RMSI	ZTE

R1-1807419	Remaining Issues on RMSI	CATT	(rev of R1-1806269)

R1-1806421	Remaining minimum system information	Ericsson

R1-1806498	Remaining issues in RMSI	Intel Corporation

R1-1806602	RMSI delivery and CORESET configuration	LG Electronics

R1-1806701	Corrections on Remaining Minimum System Information	Samsung

R1-1806783	Remaining issues for RMSI	MediaTek Inc.

R1-1806874	Remaining issues on RMSI configuration	OPPO

R1-1807051	Remaining issues on RMSI	NTT DOCOMO, INC.

R1-1807172	Remaining issues on RMSI	Nokia, Nokia Shanghai Bell

R1-1807330	Remaining system information delivery consideration	Qualcomm Incorporated

R1-1807734	Remaining Issue on RMSI CORESET Configuration	Samsung

##### Other system information delivery

R1-1807619	Summary of 7.1.1.2.3 Other system information delivery	Samsung

Decision: The document is noted.

Conclusion:

- It is understood that for the 5-bit MCS indication in PDCCH DCI for broadcast scheduling, only the entires associated with QPSK modulation are valid entries

  - Up to editor how to capture

R1-1806422	On other system information delivery	Ericsson

R1-1806603	Other system information delivery	LG Electronics

R1-1806702	Corrections on Other System Information Delivery	Samsung

R1-1806784	Remaining issues for OSI	MediaTek Inc.

R1-1807173	On remaining details on Other System Information Delivery	Nokia, Nokia Shanghai Bell

#### Paging design

R1-1807643	Offline summary for Al 7.1.1.3 on Paging	Huawei

Decision: The document is noted.

Agreements:

- NR supports in one PDCCH containing an indication in paging DCI for simultaneous short messages and scheduling information, where the short messages (w/o PDSCH) can be used for all UEs while the scheduling information is intended for Idle UEs only

  - FFS the details for the indication

  - Note: the above support is subject to size-limitations for scheduling information and the actual short messages

  - To send LS to RAN2 later (after more details)

Wednesday: Above agreement is modified as follows:

Agreements:

- NR supports in one PDCCH containing an indication in paging DCI for simultaneous short messages and scheduling information, where the short messages (w/o PDSCH) can be used for all UEs while the scheduling information is intended for Idle UEs only

  - Replace the existing 1-bit indication in TS38.212 with 2 bits with the following indication:

    - 00: Reserved

    - 01: Paging scheduling only

    - 10: Short Message only

    - 11: Paging scheduling and Short Messages

  - Note: The above assumes a single DCI format/structure in which the bit field location is determined based on both paging scheduling and short messages being present. This and other remaining issues related to DCI designs will be finalized in the DCI session.

  - Note: the above support is subject to size-limitations for scheduling information and the actual short messages

  - To send LS to RAN2

R1-1807750	[Draft] LS on simultaneous PDSCH reception with Paging	Huawei

Decision: The document is endorsed with one update “like to ask”. Final LS is approved in R1-1807763.

R1-1807749	Offline summary for Al 7.1.1.3 on Paging	Huawei

Decision: The document is noted.

Thursday

R1-1807829	Offline summary for Al 7.1.1.3 on Paging	Huawei

Decision: The document is noted.

Friday

R1-1807872	Offline summary for Al 7.1.1.3 on Paging	Huawei

Decision: The document is noted.

Agreements:

- When SS/PBCH block and control resource set multiplexing pattern 2 or 3 is configured in PBCH and initial DL BWP is active, for SS/PBCH block based RRM, RLM and [BM] measurement purposes only, UE is expected to be able to measure the SS/PBCH block associated with control resource set configured in PBCH.

- Note: When SS/PBCH block and control resource set multiplexing pattern 2 or 3 is configured in PBCH and initial DL BWP is active, SS/PBCH based RRM, RLM, and [BM] is still possible when the initial active DL BWP does not contain SS/PBCH

R1-1807888	[draft] Reply LS on Initial Active BWP for Pattern 2 and Pattern 3	Huawei

Decision: The draft LS is endorsed with the following updates:

- Remove “Reply” in the title

- RAN4 “to” instead of “cc”

- To RAN2, RAN4

- Action – add RAN4 as well

Final LS is approved in R1-1807893.

R1-1805874	Remaining details on paging	Huawei, HiSilicon

R1-1806034	Remaining issues on Paging	vivo

R1-1806358	Discussion on remaining issues for paging	CMCC

R1-1806423	Remaining details on paging design	Ericsson

R1-1806499	Remaining issues in Paging	Intel Corporation

Withdrawn

R1-1806604	Paging design in NR	LG Electronics

R1-1806703	Corrections on Paging Design	Samsung

R1-1806785	Remaining issues for Paging	MediaTek Inc.

R1-1806875	Remaining issues on paging	OPPO

R1-1807010	On Remaining Details of NR Paging Channel	InterDigital, Inc.

R1-1807052	Remaining issues on Paging design	NTT DOCOMO, INC.

R1-1807174	On open issues on paging message delivery	Nokia, Nokia Shanghai Bell

R1-1807331	Paging design consideration	Qualcomm Incorporated

#### Physical random access channel and random access procedure

##### PRACH formats

R1-1807632	Summary of Remaining details on PRACH formats	Convida Wireless

Decision: The document is noted.

Agreement:

- For the previous working assumption “For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot, and it is at least N symbols after DL part and/or the last symbol of an SSB”, N = 2 for Msg1 SCS = 120kHz

Agreement:

- Confirm the current WA on starting OFDM symbol number for all formats using short sequences in FR1 and unpaired spectrum

Agreements:

- For PRACH configuration table for FR2, for the configuration indices

  - 8

  - 22

  - 37

  - 47

  - 75

  - 104

  - 123

  - 152

  - 163

  - 181

  - 191

slot numbers 7 & 15 are supported.

Proposals:

- For PRACH configuration table for FR2, the working assumption is confirmed with the following updates:

  - Table 4. Proposed RACH configuration table for format A3 for FR2 unpaired spectrum

| PRACH Configuration Index | Preamble format | nSFN mod x = y |  | Slot number (SCS=60kHz) | Starting symbol | Number of PRACH slots within a 60 kHz slot | Number of time domain PRACH occasions within a RACH slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 58 | A3 | 41 | 10 | 4,9,14,19,24,29,34,39 9,11,13 | 02 | 21 | 2 |
| 67 | A3 | 21 | 10 | 4,9,14,19,24,29,34,39 3,5,7,9,11,13 | 0 | 21 | 2 |

  - Table 5. Proposed RACH configuration table for format B4 for FR2 unpaired spectrum

| PRACH Configuration Index | Preamble format | nSFN mod x = y |  | Slot number (SCS=60kHz) | Starting symbol | Number of PRACH slots within a 60 kHz slot | Number of time domain PRACH occasions within a RACH slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128 | B4 | 1 | 0 | 3,5,7,9,11,13 | 02 | 1 | 1 |
| 136 | B4 | 1 | 0 | 3,5,7,9,11,13,15,17,19,21,23,25 | 2 | 1 | 1 |

Tuesday session

R1-1807686	Summary of Remaining details on PRACH formats	Convida Wireless

Decision: The document is noted.

Working assumption:

- For FR1 and unpaired spectrum:

  - Remove 4 to 6 entries from formats 0-3

  - Reduce format B1 with 6 entries

  - These removed/reduced entries are then used to add entries for format A2 (3 entries), A3(3 entries) and format B4 (4 entries)

- For FR2 and unpaired spectrum:

  - Reduce format B1 with 6 entries

  - The 6 entries are then used to add entries for format A2 (2 entries) and B4(4 entries)

Wednesday session

R1-1807733	Summary of Remaining details on PRACH formats	Convida Wireless

Decision: The document is noted.

The above working assumption is obsolete due to the agreements below

Agreement:

- The following entries for FR1 and unpaired spectrum are removed:

| 15 | 0 | 1 | 0 | 1 | 0 | - | - | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | 0 | 1 | 0 | 3,4,8 | 0 | - | - | 0 |
| 57 | 3 | 1 | 0 | 1 | 0 | - | - | 0 |
| 68 | 3 | 1 | 0 | 6,7,8,9 | 0 | - | - | 0 |

Agreement:

- The following 8 entries for format B1 for FR1 and unpaired spectrum are deleted

| 131 | B1 | 16 | 1 | 9 | 2 | 2 | 6 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 132 | B1 | 8 | 1 | 9 | 2 | 2 | 6 | 2 |
| 134 | B1 | 2 | 1 | 2,3,4,7,8,9 | 2 | 1 | 6 | 2 |
| 135 | B1 | 2 | 1 | 8,9 | 2 | 2 | 6 | 2 |
| 137 | B1 | 2 | 1 | 7,9 | 8 | 1 | 3 | 2 |
| 147 | B1 | 1 | 0 | 3,4,8,9 | 2 | 1 | 6 | 2 |
| 148 | B1 | 1 | 0 | 3,4,8,9 | 2 | 2 | 6 | 2 |
| 149 | B1 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 8 | 1 | 3 | 2 |

Agreement:

- The following two entries for format A2 for FR1 and unpaired spectrum are updated:

| 92 | A2 | 4 2 | 1 | 2,3,4,7,8, 9 | 0 | 12 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 94 | A2 | 82 | 1 | 4 ,7,9 | 0 | 12 | 3 | 4 |

Agreement:

- The following entries for format A2, A3 and B4 for FR1 and unpaired spectrum are added:

Format A2:

|  | A2 | 1 | 0 | 2 | 0 | 1 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A2 | 1 | 0 | 7 | 0 | 1 | 3 | 4 |
|  | A2 | 1 | 0 | 2,7 | 0 | 1 | 3 | 4 |

Format A3:

|  | A3 | 1 | 0 | 2 | 0 | 1 | 2 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A3 | 1 | 0 | 7 | 0 | 1 | 2 | 6 |
|  | A3 | 1 | 0 | 2,7 | 0 | 1 | 2 | 6 |

Format B4:

|  | B4 | 2 | 1 | 9 | 0 | 1 | 1 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B4 | 1 | 0 | 9 | 0 | 1 | 1 | 12 |
|  | B4 | 1 | 0 | 2 | 0 | 1 | 1 | 12 |
|  | B4 | 1 | 0 | 1 | 0 | 1 | 1 | 12 |
|  | B4 | 1 | 0 | 7 | 0 | 1 | 1 | 12 |
|  | B4 | 1 | 0 | 4 | 0 | 1 | 1 | 12 |

Agreement:

- The following entries are deleted for format B1 in FR2 and unpaired spectrum:

| 88 | B1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 90 | B1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 92 | B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 94 | B1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 95 | B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |

Agreement:

- The following entries for FR2 and unpaired spectrum are updated:

| 78 | A3 | 1 | 0 | 9,11,13 | 2 | 1 | 2 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 127 | B4 | 1 | 0 | 19,39 | 2 | 2 | 1 | 12 |
| 128 | B4 | 1 | 0 | 3,5,7,9,11,13 | 02 | 1 | 1 | 12 |
| 133 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 1 | 12 |
| 135 | B4 | 1 | 0 | 23,27,31,35,39 | 2 | 2 | 1 | 12 |
| 136 | B4 | 1 | 0 | 3,5,7,9,11,13,15,17,19,21,23,25 | 2 | 1 | 1 | 12 |
| 138 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 1 | 12 |
| 141 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 1 | 12 |
| 144 | C0 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 |
| 146 | C0 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 193 | C2 | 1 | 0 | 23,27,31,35,39 | 7 | 2 | 1 | 6 |
| 195 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 2 | 1 | 6 |
| 198 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 1 | 6 |

Agreement:

- The following entries are added for format A2, A3 and B4 for FR2 unpaired spectrum

Format A2:

|  | A2 | 1 | 0 | 3,5,7,9,11,13 | 0 | 1 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Format A3:

|  | A3 | 1 | 0 | 3,5,7,9,11,13 | 0 | 1 | 2 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Format B4

|  | B4 | 1 | 0 | 9,11,13,15,17,19 | 0 | 1 | 1 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B4 | 1 | 0 | 3,5,7,9,11,13,15,17,19,21,23,25 | 0 | 2 | 1 | 12 |
|  | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 2 | 1 | 12 |  |

Thursday

R1-1807830	Summary of Remaining details on PRACH formats	Convida Wireless

Decision: The document is noted.

Agreements:

- Confirm the following working assumption with updates

  - If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is also provided higher layer parameter tdd-UL-DL-ConfigurationCommon2For the Semi-static UL/DL configuration in RMSI, PRACH occasions in the UL part are always valid, and only a PRACH occasions within the UL part and X part is are valid as long as it does not precede or collide with an SSB in the RACH slot and it is at least N symbols after the DL part and/or the last symbol of an SSB.

    - N = 0 for Msg1 SCS = 1.25kHz, 5kHz

    - N = 2 for Msg1 SCS = 15kHz

    - N = 2 for Msg1 CS = 30kHz

    - N = 2 for Msg1 SCS = 60kHz

    - N = 2 or 3 for Msg1 SCS = 120kHz

    - Note for format B4, N = 0 irrespectively of Msg1 SCS

      - UE is not expected to receive DL signals during any valid RACH occasion and N symbols before valid RO occasions

      - The SSB to RO association pattern period shall repeat at most every 160 ms

        - If there are leftover ROs after an integer number of SSB to RO association period(s) within the SSB to RO association pattern period. Those leftover ROs are invalid.

      - Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured

Agreements:

- To adopt to the following TPs:

38.213 Section 8.1

<Unchanged parts>

If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is also provided higher layer parameter tdd-UL-DL-ConfigurationCommon2, valid PRACH occasions are ones that include uplink symbols or flexible symbols that start at least symbols after a last downlink symbol or a last SS/PBCH block transmission symbol a PRACH occasion is valid if

- It is within the UL symbols or,

- It does not precede or collide with an SS/PBCH block in the RACH slot, and it is at least  symbols after the last downlink symbol right before the PRACH occasion and the last symbol of an SS/PBCH block right before the PRACH occasion,

where  is provided in Table 8.1-2 as a function of the preamble subcarrier spacing value. For preamble format B4 [4, TS 38.211], .

<Unchanged parts>

-----------Text Proposal--------

38.213 Section 11.1

<Unchanged parts>

For a the set of symbols of a slot indicated to a UE by higher layer parameter prach-ConfigurationIndex in RACH-ConfigCommon for PRACH transmissions, occupied by valid PRACH occasions as described in Subclause 8.1 and symbols before valid PRACH occasions where  is provided in Table 8.1-2 and for preamble format B4 [4, TS 38.211], , the UE does not receive PDSCH or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect theis set of symbols of the slot to be indicated as downlink by higher layer parameters tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationCommon2, or tdd-UL-DL-ConfigDedicated.

<Unchanged parts>

Friday

R1-1807855	Summary of Remaining details on PRACH formats	Convida Wireless

Decision: The document is noted.

Agreements:

-------------- Start of Text proposal for 38.211, section 6.3.3.2 ---------------------------

<omitted >

For the purpose of slot numbering in the tables, the following subcarrier spacing shall be assumed:

-	 for PRACH preamble formats 0 – 3

-	 for PRACH preamble formats A1, A2, A3, B1, B2, B3, B4, C0, C2 where  is the configured PRACH subcarrier spacing.

- 15kHz for FR1

- 60kHz for FR2

For paired spectrum and within the same frequency range, and L = 4,

- the UE may for handover purpose assume an absolute value of the relative time difference between radio frame i in the current cell and the target cell is less than 153600∙TS, if the any of the following conditions of the PRACH configurations are met:

  - for PRACH configurations in which x is not equal to 1 in Tables 6.3.3.2-2 and 6.3.3.2-3

  - for PRACH configurations in which x is equal to 1 in Tables 6.3.3.2-2 and 6.3.3.2-3 and the association period in Table 8.1-1 of [38.213] is not equal to 1

-------------- End of Text proposal for 38.211, section 6.3.3.2 ---------------------------

R1-1805875	Remaining details of RACH Preambles	Huawei, HiSilicon

R1-1805944	Remaining details of PRACH Format	ZTE

R1-1806271	Further details on NR RACH format	CATT

R1-1806357	Discussion on remaining issues for PRACH formats	CMCC

R1-1806424	Remaining details of RACH configurations	Ericsson

R1-1806500	Remaining issues in PRACH formats	Intel Corporation

R1-1806605	Discussion on PRACH preamble format details	LG Electronics

R1-1806704	Corrections on PRACH Formats	Samsung

R1-1806922	Remaining details on PRACH formats	Nokia, Nokia Shanghai Bell

R1-1807011	On Remaining Details of NR Physical Random Access Channel 	InterDigital, Inc.

R1-1807053	Remaining issues on PRACH formats	NTT DOCOMO, INC.

R1-1807274	Discussion on valid RACH occasions	Motorola Mobility, Lenovo

R1-1807332	Remaining details on PRACH formats	Qualcomm Incorporated

##### RACH procedure

R1-1807633	Summary of Remaining Details on RACH Procedure	Qualcomm Incorporated

Decision: The document is noted.

Agreements:

Confirm the following working assumption.

- For PDCCH ordered CFRA, the minimum timing gap between PDCCH order reception and Msg1 transmission is

  - N2+BWPSwitching+ Delay,

    - If BWP switching is not required, BWPSwitching=0; otherwise, BWPSwitching is up to RAN4

    - N2 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

    - N2 is dependent on the SCS of Msg1

      - Working assumption: For Msg1 with 1.25 kHz or 5 kHz SCS, calculation of N2 uses 15 kHz SCS.

    - Delay includes at least MAC layer delay in initializing PRACH

      - Delay = 250 us for FR2, Delay = 500 us for FR1

Proposals:

Confirm the following working assumption with updates in red:

For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where

- N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1 and is determined by the reference SCS

- N2 also refers to the value determined in control session with UE capability #1 and is determined by the reference SCS

- L2 refers to the MAC processing latency and is equal to 0.5 msec.

- TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the UL SCS  reference SCS

- Note: Spec captures the value k for different reference SCS

- UE is not expected that UL transmissions in the same slot shall apply different TA values

  - If this happens, it is up to UE’s implementation.

- The reference SCS is the minimum SCS among the subcarrier spacings of all the configured BWPs in all the configured uplink carriers in a TAG and their corresponding configured downlink carriers/BWPs.

  - Slot n, slot n+k, the time duration of (k-1) slots, and slot_duration are defined based on the reference SCS.

Agreements:

Update previous agreements as follows:

- PDCCH order for RACH procedure includes the following fields:

  - Random Access Preamble index – 6 bits. Indicating which Random access preamble to use in case of contention-free random access procedure, or the value 000000 in case of contention-based random access procedure

  - For CFRA only:

    - FFS BWP index. Indicating which BWP to transmit the Random access preamble on

    - SUL indicator – 1 bit. Indicating whether to transmit the Random access preamble on SUL or normal uplink carrier

    - SSB index - 6 bits are used to indicate an SSB index, which, in turn, identifies a group of RACH occasions

    - PRACH occasion Mask index – 3 4 bits are used to indicate the relative RACH occasion PRACH Mask index that corresponds to the RO(s) in the set of RACH occasions identified by the indicated SSB index the indicated SSB index identified group of RACH occasions

      - RAN1 supports RAN2’s agreements regarding the PRACH table as in R1-1805803.

      - The set of RACH occasions identified by the indicated SSB index within one association period are ordered in the following way:

        - In the order of increasing frequency multiplexed RACH occasions and then

        - In the order of increasing time multiplexed RACH occasions within a RACH slot and then

        - In the order of increasing RACH slot indices

To draft reply LS to RAN2 based on the above agreements and RAN2 LS.

Friday

R1-1807889	[draft] reply LS on the design of PRACH table	Qualcomm

Decision: The document is endorsed and final LS is approved in R1-1807892.

Tuesday session

R1-1807706	Summary of Remaining Details on RACH Procedure	Qualcomm

Decision: The document is noted.

Agreements:

Confirm the following working assumption with updates in red:

For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where

For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot m, where slot m is determined starting from the end of slot n with an additional offset of ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where

- N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1 and is determined by the reference SCS

- N2 also refers to the value determined in control session with UE capability #1 and is determined by the reference SCS

- The reference SCS for N1 & N2 is the minimum SCS among the subcarrier spacings of all the configured BWPs in all the configured uplink carriers in a TAG, and their corresponding configured downlink carriers/BWPs.

- L2 refers to the MAC processing latency and is equal to 0.5 msec.

- TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the UL SCS  minimum SCS among the subcarrier spacings of all the configured BWPs in all the configured uplink carriers in a TAG and initial active UL BWP

- Note: Spec captures the value k offset for different SCS

- UE is not expected that UL transmissions in the same slot shall apply different TA values

  - If this happens, it is up to UE’s implementation.

- Slot m and slot_duration are defined based on the minimum SCS among the subcarrier spacings of all the configured BWPs in all the configured uplink carriers in a TAG.

Proposal:

- To adopt the following TP for section 7.3 of 38.213:

If the PRACH transmission from the UE is not in response to a detection of a PDCCH order by the UE which triggers non-contention based random access procedure, referenceSignalPower is provided by ss-PBCH-BlockPower or, when the UE is configured resources for a periodic CSI-RS reception, is obtained by higher layer parameter ss-PBCH-BlockPower and powerControlOffsetSS; where powerControlOffsetSS  provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214], depending on the used DL RS for RACH association and transmission.

If the PRACH transmission from the UE is in response to a detection of a PDCCH order by the UE which triggers non-contention based random access procedure, referenceSignalPower is ss-PBCH-BlockPower or, when the UE is configured resources for a periodic CSI-RS reception, is obtained by higher layer parameter  ss-PBCH-BlockPower and powerControlOffsetSS; where powerControlOffsetSS  that provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214], depending on the DL RS that the DMRS of the PDCCH order is quasi-collocated with as described in Subclause 10.1. The UE is provided by higher layer parameters CFRA-SSB-Resource and CFRA-CSIRS-Resource an association between PRACH preamble indexes and corresponding indexes for SS/PBCH blocks and periodic CSI-RS resource configurations.

Proposals:

- To adopt the following TP to Section 8.1 of 38.213:

A preamble is transmitted using the selected PRACH format with transmission power , as described in Subclause 7.4, on the indicated PRACH resource. UE is not expected to be configured with more than one preamble format and more than one subcarrier spacing for PRACH in a cell.

Agreement:

- To adopt the following TP to section 8.2 of 38.213:

---- Start of TP ----

A UE When receivinges the PDCCH with DCI format 1_0 with the CRC scrambled by the corresponding RA-RNTI and the corresponding PDSCH that includes the DL-SCH transport block, the UE may assume with the same DM-RS antenna port quasi co-location properties, as described in [6, 38.214], as for a detected SS/PBCH block or a received CSI-RS the SS/PBCH block or the CSI-RS resource the UE used for RACH association and transmission. If the UE attempts to detect the PDCCH with the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order which triggers non-contention based random access procedure, the UE may assumes that the PDCCH and the PDCCH order have same DM-RS antenna port quasi co-location properties.

---- End of TP ----

Agreement:

- To adopt the following to section 5.1 of 38.213:

---- Start of TP ----

When receiving PDSCH for Random Access Response (RAR) the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE selected used for RACH association and transmission with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. When receiving a RAR triggered by a PDCCH order which triggers non-contention based random access procedure, the UE may assume that the DM-RS port of the received PDCCH order and the PDSCH of the corresponding RAR are quasi co-located with the same SS/PBCH block or CSI-RS with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable.

---- End of TP ----

Agreement:

- Kindly request RAN2 to adopt the following TP for 38.331 by sending an LS to RAN2 – R1-1807707

---- Start of TP ----

-- ASN1START

-- TAG-RACH-CONFIG-COMMON-START

RACH-ConfigCommon ::= 				SEQUENCE {

rach-ConfigGeneric			RACH-ConfigGeneric,

totalNumberOfRA-Preambles			INTEGER (1..63)																		OPTIONAL,	-- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}																														OPTIONAL,	-- Need M

groupBconfigured 					SEQUENCE {

ra-Msg3SizeGroupA					ENUMERATED { b56, b144, b208, b256, b282, b480, b640,

b800, b1000, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB			ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA			INTEGER (1..64)

ssb-perRACH-OccasionAndnumberOfRA-PreamblesGroupAPerSSBPerRACH-Occasion	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}

}																														OPTIONAL,	-- Need R

ra-ContentionResolutionTimer			ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB						RSRP-Range																		OPTIONAL,	-- Need R

rsrp-ThresholdSSB-SUL					RSRP-Range																		OPTIONAL,	-- Cond SUL

prach-RootSequenceIndex					CHOICE {

l839									INTEGER (0..837),

l139									INTEGER (0..137)

},

msg1-SubcarrierSpacing					SubcarrierSpacing,

restrictedSetConfig						ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoding					ENUMERATED {enabled}															OPTIONAL,	-- Need R

...

}

---- End of TP ----

Wednesday session

R1-1807707	LS on numberOfRA-PreamblesGroupA	Qualcomm

Decision: The document is endorsed and final LS is approved in R1-1807764.

R1-1807747	Summary of Remaining Details on RACH Procedure	Qualcomm

Decision: The document is noted.

Agreements:

- To adopt the following TP for section 7.3 of 38.213 (with changes in red).

---- Start of TP ----

If the PRACH transmission from the UE is not in response to a detection of a PDCCH order by the UE, referenceSignalPower is provided by ss-PBCH-BlockPower if SS block is used for RACH association and transmission.

If the PRACH transmission from the UE is in response to a detection of a PDCCH order by the UE which triggers non-contention based random access procedure, referenceSignalPower is provided by ss-PBCH-BlockPower or, when the UE is configured resources for a periodic CSI-RS reception, is obtained by higher layer parameter  ss-PBCH-BlockPower and powerControlOffsetSS; where powerControlOffsetSS  that provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214], depending on the DL RS that the DMRS of the PDCCH order is quasi-collocated with as described in Subclause 10.1. The UE is provided by higher layer parameters CFRA-SSB-Resource and CFRA-CSIRS-Resource an association between PRACH preamble indexes and corresponding indexes for SS/PBCH blocks and periodic CSI-RS resource configurations.

If the PRACH transmission from the UE is in response to a detection of a PDCCH order by the UE which triggers contention based random access procedure, referenceSignalPower is provided by ss-PBCH-BlockPower.

---- End of TP ----

Conclusion:

- From RAN1 perspective, both 56 bits and 72 bits msg 3 payload sizes can be supported in NR. Up to RAN2 to decide whether or not it’s necessary to support both sizes.

- Send an LS to RAN2, R1-1807766

Thursday

R1-1807766	Draft Reply LS on MSG3 size for NR	Ericsson

Decision: The document is endorsed and final LS is approved in R1-1807842.

R1-1807832	Summary of Remaining Details on RACH Procedure	Qualcomm

Decision: The document is noted.

Agreements:

- For CSI-RS based Msg1 transmission, from RAN1 perspective, 0dB power offset of CSI-RS resource RE to SS RE is assumed by the UE without any explicit RRC configuration

  - Up to RAN2 to discuss whether or not a new RRC configuration parameter is necessary

Agreement:

- From a UE’s perspective, if a Msg2/3/4 search space overlaps in time with UL part and/or valid RACH occasions, the search space in that particular slot is invalid for the UE

R1-1807833	LS on power offset of CSI-RS resource RE to SS RE for CSI-RS based Msg1	Qualcomm

Not treated – no need to send this LS to RAN2 due to 0 Db power offset assumption.

Friday

R1-1807897	Summary of Remaining Details on RACH Procedure	Qualcomm

Decision: The document is noted.

Working assumption:

- In single CC and intra-band CA, if the SSB/CSI-RS that UE selects for RACH association and Msg1 (re)transmission is not quasi-co-located (QCLed) - with respect to type D - with the DM-RS antenna port associated with PDCCH reception in reference search spaces, UE is not expected to decode the PDCCH and or PDSCH in the reference search space scrambled by an RNTI other than those associated with Type1-PDCCH common search space, only if they are it is overlapped with PDCCH or PDSCH scrambled by the RNTIs associated with Type1-PDCCH common search space in at least one symbol.

- Reference search spaces denote the Type0-PDCCH common search space, the Type0A-PDCCH common search space, the Type2-PDCCH common search space, or Type3-PDCCH common search space.

Agreements:

To reply RAN2 LS:

- Whether tci-StatesPDCCH is ignored during the RA procedure (for BFR it seems already clear) or the tci-StatePDCCH should have been in the SearchSpace (e.g. if it is meant to be used for USS only and not for CSS)?

  - While monitoring Type 1 PDCCH common search space during the RA procedure, UE can ignore the tci-StatesPDCCH of the associated CORESET, if any tci-StatesPDCCH is configured with the associated CORESET.

  - Note: In case of PDCCH ordered CFRA, UE follows the same procedure that got agreed in RAN1 92 RACH procedure session.

- Whether it is allowed to configure UEs with CSI-RS based RA and send the RAR on CORESET #0 (associated with SSBs)?

  - RAN1 is still discussing this.

Agreement: To adopt the following TP, section 10.1 of 38.213

---- Start of TP ----

f a UE is not provided higher layer parameter ra-SearchSpace for Type1-PDCCH common search space set, the control resource set and the association between PDCCH monitoring occasions for Type1-PDCCH common search space set and the SS/PBCH block index are same as the control resource set and the association of PDCCH monitoring occasions for Type0-PDCCH common search space set as described in Subclause 13. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level are given in Table 10.1-1.

---- End of TP ----

R1-1805876	Remaining details of RACH Procedures	Huawei, HiSilicon

R1-1805945	Remaining details of RACH procedure	ZTE

R1-1806035	Remaining issues on RACH procedure	vivo

R1-1806272	Further details on NR 4-step RA Procedure	CATT

R1-1806425	Remaining details of RACH procedure	Ericsson

R1-1806501	Remaining issues in RACH procedure	Intel Corporation

R1-1806606	RACH Procedure	LG Electronics

R1-1806705	Corrections on RACH Procedure	Samsung

R1-1807417	On remaining issues of RACH procedure	MediaTek Inc.	(R1-1806771)

R1-1806923	Remaining details on RACH procedure	Nokia, Nokia Shanghai Bell

R1-1806941	Remaining issues on RACH procedure	Sharp

R1-1807603	Remaining issues on RACH procedure	NTT DOCOMO, INC.	(R1-1807054)

R1-1807275	Remaining details for Msg/2/3/4 transmission and reception	Motorola Mobility, Lenovo

R1-1807333	Remaining details on RACH procedure	Qualcomm Incorporated

#### Mobility procedure

##### Measurement for mobility management

R1-1807628	Summary of Discussion for NR RRM measurements	Intel Corporation

Decision: The document is noted.

Agreement:

- Confirm that only periodic CSI-RS is supported for CSI-RS based RRM in Rel-15

Tuesday session

R1-1807689	Summary of Offline Discussion for NR RRM measurements	Intel Corporation

Decision: The document is noted.

Agreements:

- For RRM, UE may assume that in TDD bands, the half radio frame boundaries of the cells in the same frequency layer are aligned.

- When useServingCellTimingForSync is set to TRUE for measurements, it means the following:

  - For intra-frequency measurements, UE may use the serving cell timing to derive the SSB index of neighbor cells in the same frequency layer.

- For inter-frequency measurements, UE may use timing of any detected cell in the target frequency layer to derive the SSB index of neighbor cells of the target frequency layer

  - Note: cells in different frequency layer are not assumed to be half radio frame aligned.

  - UE may assume that in TDD bands useServingCellTimingForSync is assumed to be set to TRUE

- Note: half radio frame alignment is the minimal requirement in RAN1’s understanding that allows the UE to use timing of one cell to derive the SSB index of another cell. However, it is up to RAN4 to determine the exact requirements for the “useServingCellTimingForSync”. In RAN1 understanding, the agreements above should not impact RAN4 discussion on the requirements for “useServingCellTimingForSync”.

- RAN 1 believes useServingCellTimingForSync information should be also applicable to UEs in IDLE/INACTIVE mode.

- It is up to RAN2 to determine to how to provide/apply useServingCellTimingForSync information in IDLE/INACTIVE mode.

- Send an LS to RAN2/RAN4 to inform the above agreements (also add in the LS the previous RAN1 definition of “frequency layer”)

Conclusion:

- With respect to agreement made in RAN4 2018 Ad-hoc #1 stating “Note: whether or not to define specific behavior for rate matching around above symbols is up to RAN1”, RAN1 will not define any additional rate matching behavior in Rel-15.

Wednesday

R1-1807741	[Draft] LS on useServingCellTimingForSync definition	Intel Corporation

Decision: The document is endorsed with the update that the bullet regarding “UE may assume that in TDD bands useServingCellTimingForSync is assumed to be set to TRUE” should be left-indented twice. Final LS is approved in R1-1807767.

Friday

R1-1807852	Summary of Offline Discussion for NR RRM measurements	Intel Corporation

Decision: The document is noted.

Agreements:

- No dedicated measurement windows for CSI-RS for L3 mobility is supported in Rel-15

- If CSI-RS for L3 mobility with associated SSB is configured, SMTC configuration (including frequency location and SCS of the SSB) should be also provided regardless whether UE is configured to measure SSBs in the same MO.

  - UE is expected to detect the associated SSB (to the CSI-RSs) within the configured SMTC window.

  - Note that when SSB based RRM measurements are configured, UE is always expected to be configured with SMTC information.

LS to RAN2 – R1-1807906

R1-1807906	[DRAFT] LS on CSI-RS for L3 mobility	Intel

Decision: The document is endorsed and final LS is approved in R1-1807909.

R1-1805877	Remaining details on NR RRM	Huawei, HiSilicon

R1-1805946	Remaining details of RRM measurements	ZTE

R1-1806036	Remaining issues on RRM	vivo

R1-1806273	Remaining Issues on NR Mobility Management	CATT

R1-1806407	Remaining issues for CSI-RS based mobility management	Spreadtrum Communications

R1-1806426	Remaining details for mobility measurements	Ericsson

R1-1806502	Remaining issues in RRM measurements	Intel Corporation

R1-1806607	Remaining Details on L3 measurement and mobility management	LG Electronics

R1-1806706	Corrections on Measurements for Mobility Management	Samsung

R1-1806772	Remaining issues on RRM	MediaTek Inc.

R1-1806869	Remaining details on NR RRM	OPPO

R1-1807055	Remaining issues on Measurement for mobility management	NTT DOCOMO, INC.

R1-1807175	Remaining issues on mobility measurements	Nokia, Nokia Shanghai Bell

R1-1807334	Remaining details on measurement for mobility management	Qualcomm Incorporated

##### Radio link monitoring for mobility management

R1-1807629	Summary of Discussion for NR Radio Link Monitoring	Intel Corporation

Decision: The document is noted.

Agreements:

Confirm the following working assumptions regarding the values of X/Y/Z:

- The maximum number of RLM-RS(s) and BFD-RS(s) should depend on whether same RS(s) is shared between RLM and BFD. The maximum number of unique RS(s), each RS using different set of resources, for both RLM-RS(s) and BFD-RS(s) are:

  - X RS(s) per BWP for below 3 GHz,

    - X=2(working assumption)

  - Y RS(s) per BWP for above 3 GHz and below 6 GHz,

    - Y=6 (working assumption)

  - Z RS(s) per BWP for above 6 GHz,

    - Z=8(working assumption)

Agreements:

- Remove csi-IM-ResourceElementPattern, csi-IM-ResourceElementPattern, csi-IM-ResourceId, periodicityAndOffset and freqBand from the list of ‘not applicable parameters’ from section 5 in 38.213.

  - Note: there is no RAN2 impact

Tuesday session

R1-1807690	Summary of Offline Discussion for NR Radio Link Monitoring	Intel Corporation

Decision: The document is noted.

Proposal:

- No aperiodic IS or OOS indication based on successful/unsuccessful beam failure recovery is supported in Rel-15.

Agreement:

- Confirm WA on RLM based on CSI-RS for tracking: If the TCI-states refer to CSI-RS for tracking, it is up to UE to select a NZP-CSI-RS resource from the configured resources for CSI-RS for tracking for RLM

Agreement:

- UE is not expected to monitor more than NRLM RadioLinkMonitoringRS for radio link monitoring when UE monitors RSs based on TCI states of PDCCH.

Wednesday session

R1-1807740	Summary of Offline Discussion for NR Radio Link Monitoring	Intel Corporation

Decision: The document is noted.

Conclusion:

- Aperiodic IS and aperiodic OOS indication based on successful/unsuccessful beam failure recovery is not supported in Rel-15.

Agreements:

- In case of no explicit RLM-RS configuration, when TCI-states indicate a combination of SSB, CSI-RS, and CSI-RS for tracking, UE RLM behavior is as follows:

  - When an active TCI state contains single RS, the UE is expected to perform RLM measurement using RS configured by the active TCI state

  - When an active TCI state contains two RSs, the UE is expected to use the RS associated with QCL Type D as RLM-RS.

    - For RLM, it is assumed that a TCI state that contains two RS should have one and only one RS with TypeD QCL.

  - UE is not required to perform RLM measurements with aperiodic RS configured in the TCI state.

Friday

R1-1807853	Summary of Offline Discussion for NR Radio Link Monitoring	Intel Corporation

R1-1805878	Remaining details on NR RLM	Huawei, HiSilicon

R1-1805947	Remaining details of RLM	ZTE

R1-1806037	Remaining issues on RLM	vivo

R1-1806274	Remaining Issues on NR Radio Link Monitoring	CATT

R1-1806427	Remaining details for radio link monitoring	Ericsson

R1-1806503	Remaining issues in RLM	Intel Corporation

R1-1806608	Discussion on Radio Link Monitoring in NR	LG Electronics

R1-1806707	Corrections on Radio Link Monitoring	Samsung

R1-1806773	Remaining issues on RLM	MediaTek Inc.

R1-1806870	Remaining details on NR RLM	OPPO

R1-1807056	Remaining issues on RLM for mobility management	NTT DOCOMO, INC.

R1-1807176	Remaining Details on Radio Link Monitoring	Nokia, Nokia Shanghai Bell

R1-1807276	Remaining details for radio link monitoring	Motorola Mobility, Lenovo

R1-1807335	Radio link monitoring consideration	Qualcomm Incorporated

#### Other

Including TA related aspects, if any leftover issues

R1-1807700	Summary of TA related aspects	Panasonic

Decision: The document is noted.

Agreements:

- To adopt the following TP to section 4.2 of TS36.213

---- Start of TP ----

Upon reception of a timing advance command for a TAG containing the primary cell or PSCell, the UE adjusts uplink transmission timing for PUCCH/PUSCH/SRS of the primary cell or PSCell based on the received timing advance command.

The UL transmission timing for PUSCH/SRS/PUCCH(if any) of a secondary cell is the same as the primary cell if the secondary cell and the primary cell belong to the same TAG. If the primary cell in a TAG operates with paired DL/UL spectrum and a secondary cell in the same TAG operates with unpaired DL/UL spectrum, a UE may assume that  for FR1 and  for FR2 [10, TS 38.133].

If the UE is configured with a SCG, the UL transmission timing for PUSCH/SRS of a secondary cell other than the PSCell is the same as the PSCell if the secondary cell and the PSCell belong to the same TAG.

Upon reception of a timing advance command or a timing adjustment indication for a TAG not containing the primary cell or PSCell, if all the serving cells in the TAG have the same duplex mode type, the UE adjusts uplink transmission timing for PUSCH/SRS/PUCCH(if any) of all the secondary cells in the TAG based on the received timing advance command or a timing adjustment indication where the UL transmission timing for PUSCH /SRS/PUCCH(if any) is the same for all the secondary cells in the TAG.

Upon reception of a timing advance command or a timing adjustment indication for a TAG not containing the primary cell or PSCell, if a serving cell in the TAG has a different duplex mode type compared to the duplex mode type of another serving cell in the same TAG, the UE adjusts uplink transmission timing for PUSCH/SRS/PUCCH(if any) of all the secondary cells in the TAG by using  for FR1 and  for FR2 regardless of the duplex mode type of the serving cells and based on the received timing advance command or a timing adjustment indication where the UL transmission timing for PUSCH /SRS/PUCCH(if any) is the same for all the secondary cells in the TAG.

………………

In other cases, a timing advance command [11, TS 38.321], , for a TAG indicates adjustment of the current  value, , to the new  value, , by index values of  = 0, 1, 2,..., 63, where for a subcarrier spacing of  kHz, . If a UE has multiple active UL BWPs in a same TAG, including UL BWPs in two UL carriers of a serving cell, the timing advance command value is relative to the largest larger subcarrier spacing of the multiple active UL BWPs. The applicable  value for an UL BWP with lower subcarrier spacing may be rounded to align with the timing advance granularity for the UL BWP with the lower subcarrier spacing while satisfying the timing advance accuracy requirements in [10, TS38.133].

---- End of TP ----

R1-1805879	Remaining issues on TA aspects	Huawei, HiSilicon

R1-1805890	On RMSI PRB grid	Huawei, HiSilicon

R1-1805891	On k0 for OFDM signal generation	Huawei, HiSilicon

R1-1806428	On multiple preamble transmissions for contention free random access	Ericsson

R1-1806429	L1-RSRP reporting on non-serving cells	Ericsson

R1-1806774	Remaining TA related issues	MediaTek Inc.

R1-1806914	What is "initial access" in RAN1?	Panasonic

R1-1806924	PUCCH Groups and Timing Advance Groups	Nokia, Nokia Shanghai Bell

R1-1807057	Remaining issues on TA related aspects	NTT DOCOMO, INC.

R1-1807177	Initial active BWP definition for RMSI multiplexing pattern 2 and 3	Nokia, Nokia Shanghai Bell

R1-1807336	Remaining details on Timing advance granularity and adjustment consideration	Qualcomm Incorporated

### MIMO

R1-1807785	Chairman’s notes of AI 7.1.2 MIMO	Samsung

Decision: Contents are incorporated below. The document is endorsed with updates.

#### Multi-antenna scheme

R1-1807242	CR to 38.214: maintenance according to agreed Rel 15 features 	Nokia

Agreement

The text proposal in R1-1807242 is agreed with the following revision. Delete the following text:

-	If the number of DL PT-RS ports associated to a tci-State in DCI is set to 2,

-	when one codeword is scheduled, the scheduled number of PT-RS ports is one, and the PT-RS port is associated with the lowest indexed DM-RS port.

-	when two codewords are scheduled, the scheduled number of PT-RS ports is two, and the first PT-RS antenna port is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the first codeword. The second PT-RS antenna port is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the second codeword.

Note: Rel-15 agreements of supporting multi-TRPs/panels/beams related to e.g. DMRS/PTRS/TCI states can be a starting point for future discussions.

R1-1806209	Corrections for UL Antenna Port Identification	Ericsson

##### Codeword mapping

R1-1806708	Summary of open issues on codeword mapping	Samsung

Agreement:

Section 7.3.1.2.2 of TS38.212

…

- DMRS sequence initialization – 1 bit if transform precoding is enabled

If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. In case one of the transport blocks is disabled as specified in Subclause 5.1.3.2 of [6, TS 38.214], the enabled transport block is mapped to codeword 0.

Agreement

The term “rate matched” is not defined, hence we suggest replacing it with “not available for PDSCH” to align with section 5.1.4 terminology of TS 38.214.

- Editor to make appropriate revision

Agreement

Section 5.1.3.2 of TS38.214

…

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then a transport block is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1for the corresponding transport block, otherwise the transport block is enabled. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

Agreement

For PDSCH/PUSCH scrambling,

- For DCI formats 1_1 and 0_1, the scrambling sequence initialization is done using the configured ID if the ID is provided by higher layers, otherwise using the default ID

- For DCI formats 0_0 and 1_0 with C-RNTI/CS-RNTI in USS, the scrambling sequence initialization is done using the configured ID if the ID is provided by higher layers, otherwise using the default ID

- For DCI 0_0 and 1_0 in CSS, the scrambling sequence initialization is done using the default ID

R1-1806038	Remaining issues on layer mapping	vivo

R1-1806210	Remaining issues on codeword mapping and data scrambling	Ericsson

R1-1806275	Remaining issues on codeword mapping	CATT

R1-1806942	Open issue on codeword mapping	Sharp

R1-1806975	Remaining Issues of Codeword Mapping	AT&T

##### Codebook based transmission for UL

R1-1807669	Summary on Codebook Based UL Transmission	Intel

Agreement

- Support number of SRS resources for codebook based transmission as a UE capability

R1-1807768	Way Forward on PUSCH Power Scaling in UL Power Control	Ericsson, Samsung, DOCOMO, CATT

Also supported by KDDI

Agreement

Adopt the following TP for 38.214

6.1.1.1	Codebook based UL transmission

For codebook based transmission, if PUSCH is scheduled by DCI format 0_1, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank from the DCI, given by DCI fields of SRS resource indicator and Precoding information and number of layers in subclause 7.3.1.1.2 of [TS 38.212], where the TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} that correspond to the SRS resource. The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Subclause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI before slot n.

Agreement

Adopt the following TP for 38.214.

6.1.1.1	Codebook based UL transmission

A UE shall not expect to be configured with the higher layer parameter codebookSubset set to 'partialAndNonCoherent' when higher layer parameter nrofSRS-Ports in SRS-ResourceSet with usage set to ”codeBook” SRS-Config indicates two SRS antenna ports are configured.

For codebook based transmission, the UE may be configured with a single SRS-ResourceSet with usage set to “codeBook” SRS resource set and only one SRS resource can be indicated based on the SRI from within the SRS resource set. The maximum number of configured SRS resources for codebook based transmission is 2. If AP- aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of AP- aperiodic SRS resources.

When multiple SRS resources are configured in SRS-ResourceSet with usage set to ”codeBook”, the UE shall expect that higher layer parameter nrofSRS-Ports in the SRS-ResourceSet SRS-Config shall be configured with the same value in all SRS resources, and the higher layer parameter resourceType in SRS-ResourceSet shall be configured with the same value in all SRS resources.

R1-1806039	Remaining issues on codebook based UL transmission	vivo

R1-1806211	Corrections for Codebook Based UL MIMO	Ericsson

R1-1806276	Remaining issues on codebook based UL transmission	CATT

R1-1806338	Remaining issues on codebook based UL transmission	Lenovo, Motorola Mobility

R1-1806339	Remaining issues on non-Codebook based UL transmission	Lenovo, Motorola Mobility

R1-1806504	Remaining Issues on Codebook based UL Transmission	Intel Corporation

R1-1806709	Issues on codebook based UL transmission	Samsung

R1-1806824	Remaining issues on codebook based UL transmission	OPPO

R1-1807127	Remaining issues for codebook based transmission for UL MIMO	Huawei, HiSilicon

R1-1807181	Remaining issues on UL codebook based transmission	Nokia, Nokia Shanghai Bell

R1-1807337	Maintenance for codebook based UL transmission	Qualcomm Incorporated

##### Non-codebook based transmission for UL

R1-1807671	Summary of issues on UL non-codebook based transmission	 Nokia, Nokia Shanghai Bell

Agreement

TP to 38.214 Section 6.1.1.2:

-------------------------------------------------------Start of TP for 38.214--------------------------------------------------------------

6.1.1.2	Non-Codebook based UL transmission

For non-codebook based transmission, the UE can calculate the precoder used for the transmission of precoded SRS based on measurement of an associated NZP CSI-RS resource. A UE can be configured with only one NZP CSI-RS resource for the SRS resource set.

-	If aperiodic SRS resource set is configured, the [CSI-RS information in the same slot TBD] associated NZP CSI-RS for UL channel measurement is indicated via SRS request field in DCI format 0_1 and 1_1, where AperiodicSRS-ResourceTrigger  (indicating the association among aperiodic SRS triggering state), triggered SRS resource(s) srs-ResourceSetId, NZP-CSI-RS-ResourceId, csi-RS (indicating the associated NZP-CSI-RS-ResourceId for UL channel measurement) are higher layer configured by AperiodicSRS-ResourceTrigger in SRS-ResourceSet SRS-Config. A UE may receive the dynamic SRS transmission request for aperiodic SRS transmission in the same slot as the reception of the DL CSI-RS resource A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the AP-NZP CSI-RS resource and the first symbol of the AP-aperiodic SRS transmission is less than 42 OFDM symbols.

  - If the UE is configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of SRS request field is not “00” as in Table 7.3.1.1.2-24 of [5, TS 38.212]. The CSI-RS is located in the same slot as the SRS request field, while any of the tci-States for aperiodic CSI-RS shall not be configured with ‘QCL-TypeD’.

- If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceConfigID for measurement is indicated via higher layer parameter associatedCSI-RS in SRS-ResourceSet SRS-Config  per set.

-----------------------------------------------------------------End of TP ----------------------------------------------------------------

Agreement

-------------------------------------------------------Start of TP for 38.214--------------------------------------------------------------

6.2.1	UE sounding procedure

The 2-bit SRS request field [5 TS38.212] in DCI format 0_1, 1_1 indicates the triggered SRS resource set given in Table 7.3.1.1.2-24 of [5, TS 38212]. The 2-bit SRS request field [5, TS38.212] in DCI format 2_3 indicates the triggered SRS resource set given in Subclause 11.4 of [6, TS 38.213].

-----------------------------------------------------------------End of TP ----------------------------------------------------------------

Agreement

TP proposal to 38.214 Section 6.1.1.2:

-------------------------------------------------------Start of TP for 38.214--------------------------------------------------------------

6.1.1.2	Non-Codebook based UL transmission

For non-codebook based transmission, the UE can determine its PUSCH precoder and transmission rank based on the wideband SRI given by SRS resource indicator field from the DCI. The UE shall use one or multiple SRS resources for SRS transmission, where the number of SRS resources which can be configured to the UE for simultaneously transmission in the same RBs is a UE capability. Only one SRS port for each SRS resource is configured. Only one SRS resource set can be configured with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook'. The maximum number of SRS resources that can be configured for non-codebook based uplink transmission is 4. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI before slot n.

For non-codebook based transmission, the UE can calculate the precoder used for the transmission of precoded SRS based on measurement of an associated NZP CSI-RS resource. A UE can be configured with only one NZP CSI-RS resource for the SRS resource set with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook'.

-	If aperiodic SRS resource is configured, the [CSI-RS information in the same slot TBD] UL channel measurement is indicated via DCI, where the association among aperiodic SRS triggering state, triggered SRS resource(s) srs-ResourceSetId, NZP-CSI-RS-ResourceId, csi-RS are higher layer configured by AperiodicSRS-ResourceTrigger in SRS-Config. A UE may receive the dynamic SRS transmission request for aperiodic SRS transmission in the same slot as the reception of the DL CSI-RS resource A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the AP-CSI-RS resource and the first symbol of the AP-SRS transmission is less than 42 OFDM symbols.

-	If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceConfigID for measurement is indicated via higher layer parameter associatedCSI-RS in SRS-Config  per set.

The UE shall perform one-to-one mapping from the indicated SRI(s) to the indicated DM-RS ports(s) in the DCI format 0_1 in increasing order.

For non-codebook based transmission, the UE does not expect to be configured with both spatialRelationInfo for SRS resource and associatedCSI-RS in SRS-Config for SRS resource set.

For non-codebook based transmission, the UE can be scheduled with DCI format 0_1 when at least one SRS resource is configured in SRS-ResourceSet with usage set to ‘nonCodebook'.

-----------------------------------------------------------------End of TP ----------------------------------------------------------------

R1-1807824	Summary of offline discussion on UL non-codebook based transmission	Nokia, Nokia Shanghai Bell

Conclusion

For non-codebook based UL transmission,

- gNB may assume that UE applies a single precoder throughout the transmission bandwidth for PUSCH and SRS

Working Assumption

TP to 38.214, Section 6.1.1.2:

-------------------------------------------------------Start of TP for 38.214--------------------------------------------------------------

6.1.1.2	Non-Codebook based UL transmission

For non-codebook based transmission, the UE can determine its PUSCH precoder and transmission rank based on the wideband SRI given by SRS resource indicator field from the DCI when multiple SRS resources are configured, where the SRI is given by the SRS resource indicator in DCI according to subclause 7.3.1.1.2 of [TS 38.212], or the SRI is given by srs-ResourceIndicator according to subclause 6.1.2.3.

-----------------------------------------------------------------End of TP ----------------------------------------------------------------

Note: Whether non-codebook based UL transmission can be used for configured grant type-1/type-2 PUSCH transmission is determined under the scheduling agenda item

R1-1805825	Remaining issues on non-codebook based UL transmission	ZTE

Withdrawn

R1-1805948	Remaining issues for UL non-codebook based transmission	Huawei, HiSilicon

R1-1806040	Remaining issues on non-codebook based UL transmission	vivo

R1-1806212	Corrections for Non-codebook Based UL MIMO	Ericsson

R1-1806277	Remaining issues on non-codebook based UL transmission	CATT

R1-1806391	Remaining issues on non-codebook based UL	Spreadtrum Communications

R1-1806505	Remaining Issues on Non-Codebook based UL Transmission	Intel Corporation

R1-1806672	Remaining issues on non-codebook based UL transmission	NEC

R1-1806710	On non-codebook based UL transmission	Samsung

R1-1806825	Remaining issues on non-codebook based UL transmission	OPPO

R1-1807012	Remaining Details on Non-CB UL Transmission 	InterDigital, Inc.

R1-1807182	Remaining issues on non-codebook based UL-MIMO transmission	Nokia, Nokia Shanghai Bell

R1-1807338	Maintenance for non-codebook based UL transmission	Qualcomm Incorporated

##### PRB bundling for DL

R1-1807647	Summary on Issues of PRB bundling for DL	vivo

Agreement

----Text Proposal for TS 38.214------------------------------------------------------------------

5.1.2.3	Physical resource block (PRB) bundling

< Unchanged parts are omitted >

A UE may assume that precoding granularity is  consecutive resource blocks in the frequency domain for bandwidth part i.  can be equal to one of the values among {2, 4, wideband}.

If  is determined as one of the values among {2, 4}, Precoding Resource Block Group (PRGs) partition the bandwidth part i with  consecutive PRBs. Actual number of consecutive PRBs in each PRG could be one to or more. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that  is equal to 2 PRBs.

If  is determined as "wideband", the UE is not expected to be scheduled with non-contiguous resource allocation and the UE may assume that the same precoding is applied to the allocated resource.

If  is determined as one of the values among {2, 4}, Precoding Resource Block Group (PRGs) partition the bandwidth part i with  consecutive PRBs. Actual number of consecutive PRBs in each PRG could be one to or more. When  is determined as one of the values among {2, 4}, The first PRG size is given by and the last PRG size given by  if , and the last PRG size is if .

The UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a PRG partitioned above.

For PDSCH carrying SIB1 scheduled by PDCCH with CRC scrambled by SI-RNTI, PRG is partitioned from the lowest numbered resource block of the CORESET signalled in PBCH.

If a UE is scheduled a PDSCH with DCI format 1_0, then the UE shall assume that  is equal to 2 PRBs.

When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI, for each carrier bandwidth part i is equal to 2 PRBs unless configured by the higher layer parameter prb-BundlingType given by PDSCH-Config. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that  is equal to 2 PRBs. In case UE is scheduled by DCI 1_0, the higher layer parameter prb-BundlingType is set to 'static', and when receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, the value of  is fixed as 2 PRBs.

If a UE is scheduled a PDSCH with DCI format 1_0 scrambled with C-RNTI and CS-RNTI, then the UE shall assume that  is equal to 2 PRBs.

When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI, if the higher layer parameter prb-BundlingType is set to 'dynamicBundling', the higher layer parameters bundleSizeSet1 and bundleSizeSet2 configure two sets of  values, the first set include can take one or two  values among {2, 4, wideband}, and the second set include can take one  value among {2, 4, wideband}. The UE is not expected to be configured with (2, 4) in the first set.

If the PRB bundling size indicator signalled in DCI format 1_1 as defined in Subclause 7.3.1.2.2 of [2, TS 38.212]

-	is set to '0', the UE shall use the  value from the second set of  values when receiving PDSCH scheduled by the same DCI.

-	is set to '1' and one value is configured for the first set of  values, the UE shall use this  value when receiving PDSCH scheduled by the same DCI

-	is set to '1' and two values are configured for the first set of  values as n2-wideband' (corresponding to two  values 2 and wideband) or n4-wideband' (corresponding to two  values 4 and wideband), the UE shall use the value when receiving PDSCH scheduled by the same DCI as follows:

-	If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than ,  is the same as the scheduled bandwidth, otherwise  is set to the remaining configured value of 2 or 4, respectively.

When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI, if the higher layer parameter prb-BundlingType is set to 'staticBundling', the  value is configured with the single value indicated by the higher layer parameter bundleSize.

When a UE is configured with RBG = 2 for bandwidth part i according to Section 5.1.2.2.1, or when a UE is configured with interleaving unit of 2 for VRB to PRB mapping provided by the higher layer parameter vrb-ToPRB-Interleaver given by PDSCH-Config for bandwidth part i, the UE is not expected to be configured with = 4.

< Unchanged parts are omitted >

R1-1806041	Remaining issues on PRB bundling	vivo

R1-1806213	Remaining issue on PRB bundling	Ericsson

R1-1806711	Remaining issues on DL PRB bundling	Samsung

R1-1806839	Text Proposal for DL PRB Bundling	OPPO

##### Other

R1-1805954	Antenna selection for PUSCH	Huawei, HiSilicon

R1-1807130	Discussion on multi-TRP/panel transmission in NR	Huawei, HiSilicon

R1-1807131	Discussion on diversity based multi-TRP transmission in NR	Huawei, HiSilicon

#### CSI acquisition and beam management

R1-1805811	LS to RAN1 on Removing RLM-RS Restriction in TS 38.331 ASN.1	RAN2, Samsung

R1-1807132	[Draft] Reply LS on Removing RLM-RS Restriction	Huawei, HiSilicon

##### CSI measurement

R1-1807654	Summary of remaining issues on CSI measurement	ZTE

Agreement

If interference measurement is performed on aperiodic NZP CSI-RS, UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.

Agreement:

For TS38.214, the following text proposal is agreed:

5.2.1.4.2	Report Quantity Configurations

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to ‘cri-RSRP’, 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', and multiple resources are configured in the corresponding resource set for channel measurement, then the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where CRI k (k≥0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding nzp-CSI-RS-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resources in the corresponding csi-IM-ResourceSet (if configured).

Agreement:

For TS38.214, the following text proposal is agreed:

5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS

---unchanged texts are ommitted-----

-	For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Subclause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State’s for the aperiodic CSI-RS resources associated with the CSI triggering state. If a TCI- State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP.

-	A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the CSI triggering state ID in the up to   trigger states with codepoint ‘1’ mapped to the triggering state having the smallest triggering state ID.

Agreement:

For TS38.214, the following text proposal is agreed:

5.1.4.2	PDSCH resource mapping with RE level granularity

---unchanged texts are ommitted-----

The UE may be configured with a DCI field for triggering the aperiodic ZP-CSI-RS. A list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config, is configured for aperiodic triggering. The maximum number of aperiodic ZP-CSI-RS-ResourceSet(s) configured per BWP is 3. The bit-length of DCI field ZP CSI-RS trigger depends on the number of aperiodic ZP-CSI-RS-ResourceSet(s) configured (up to 2 bits).   Each triggering state none-zero codepoint of ZP CSI-RS trigger in DCI is to triggers one aperiodic ZP-CSI-RS-ResourceSet in the list aperiodic-ZP-CSI-RS-ResourceSetsToAddModList by indicating the aperiodic ZP CSI-RS resource set ID. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetIds =1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetIds=2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetIds=3. The first state Codepoint ‘00’ is reserved for not triggering aperiodic ZP CSI-RS.

Agreements

- When DL BWP is switched, the following resources defined in the BWP in activated state stays in its activated state and is suspended until the DL BWP is switched back.

  - Semi-persistent CSI-RS/CSI-IM resource

  - Semi-persistent ZP CSI-RS resource set

- When UL BWP is switched, the following resources defined in the BWP in activated state stays in its activated state and is suspended until the UL BWP is switched back.

  - Semi-persistent SRS

Agreement

In DRX mode operation, PUSCH-based SP-CSI is not reported when UE is not in Active Time.

R1-1807695	Summary of offline discussion on CSI measurement	ZTE

Agreement:

- Send an LS to RAN2 capturing the following details for non-PMI feedback to correct inaccurate implementation of RAN1 agreements

  - RI restriction is supported

  - For rank-R port index, the sequence size shall be R, e.g., rank 2-8 includes 2-entry PortIndex8

R1-1807797	[Draft] LS on configuration of non-PMI port index	ZTE

Decision: Draft LS is endorsed and final LS is approved in R1-1807727.

Agreement

An NZP CSI-RS resource set for interference measurement can contain more than one resource.

- The entire resource set is used for interference measurement

- For all the resources configured in an NZP CSI-RS resource set for interference measurement, the total number of port shall be no larger than 18

R1-1805826	Remaining issues on CSI measurement	ZTE

R1-1805949	Remaining issues on CSI measurement	Huawei, HiSilicon

R1-1806042	Remaining issues on CSI measurement	vivo

R1-1806215	Corrections and clarifications for CSI measurement	Ericsson

R1-1806278	Remaining issues on CSI measurement	CATT

R1-1806392	Remaining issues on CSI measurement	Spreadtrum Communications

R1-1806712	Remaining issues on CSI measurement	Samsung

R1-1806786	Remaining issues in CSI measurement	MediaTek Inc.

R1-1806864	Remaining issues on CSI measurement	OPPO

R1-1807339	Maintenance for CSI measurement	Qualcomm Incorporated

##### CSI reporting

R1-1807648	Summary of views on CSI reporting v2	Ericsson

Agreement

- To select which reference SCS µ is used to determine Z and Z’ for a CSI report

  - Let µDL correspond to the subcarrier spacing of the PDCCH with which the DCI was transmitted, µUL corresponds to the subcarrier spacing of the PUSCH with which the CSI report is to be transmitted and

    - For Type B processing capability, µCSI-RS correspond to the subcarrier spacing of the ap-CSI-RS for the CSI report

    - For Type A processing capability, µCSI-RS correspond to the minimum subcarrier spacing of the all aperiodic CSI-RS triggered by the DCI

  - To determine Z, the reference SCS µ corresponds to min(min(µDL, µCSI-RS), µUL)

  - To determine Z’, the reference SCS µ corresponds to min(min(µDL, µCSI-RS), µUL)

Agreement

- Make the following corrections in TS 38.214 and 38.212

  - Define CSI report mapping order for PUCCH and PUSCH (including report ordering when applying CSI omission rules) according to CSI reporting priority rule in Section 5.2.5 of 38.214

Agreement

Align CPU occupancy rules so that the CPU is occupied until the last symbol of the PUSCH/PUCCH carrying the report for all cases (except for Type B capability with P/SP CSI-RS where a CPU is always occupied)

Agreement

L1-RSRP reporting is treated as a new Latency Class where the Z’ values follow the reported beam report timing values in capability 2-25

Conclusion

TRS does not occupy a CPU

Agreement

- For the purpose of simultaneous CSI-RS reception in UE features 2-33, 2-36, 2-40, 2-41 and 2-43, CSI-RS ports within one CSI-RS resource, as well as the CSI-RS resource, are counted N times if the CSI-RS resource is referred by N resource Report settings

Agreement

- For PUSCH-based SP-CSI report setting in activated state, when either UL BWP or DL BWP is switched, the SP-CSI report setting transitions to inactivated state. New activation message is required to transition the SP-CSI report setting back to activated state.

Agreement

- When a carrier is deactivated, the following resources configured in the carrier in activated state would also be deactivated and needs new activation message to transit back to activated state.

  - Semi-persistent CSI-RS/CSI-IM resource

  - Semi-persistent CSI reporting on PUCCH

  - Semi-persistent SRS

  - Semi-persistent ZP CSI-RS resource set

Agreement

Adopt the following TP for TS 38.214

/************************ Start of Text Proposal **************************/

For each sub-band index s, a 2-bit sub-band differential CQI is defined as:

-	Sub-band Offset level (s) = sub-band CQI index (s) – wideband CQI index

/************************ Unchanged parts omitted**************************/

/************************ End of Text Proposal **************************/

Agreement

Support CSI report based on 1-port CSI-RS, where the number of bits for RI/LI/PMI is 0. Make the corresponding updates in 38.212.

Agreement

The number of CQI fields in UCI is determined only from rank restriction

Agreement

The most recent CSI measurement occasion occurs in DRX Active time for CSI to be reported

Agreement

- Add the following sentences to 38.214 Section 5.2.1.4.2 as the definition of LI:

  - “The LI indicates which column of the precoder matrix of the reported PMI corresponds to the strongest layer of the codeword corresponding to the largest reported wideband CQI. If two wideband CQIs are reported and have equal value, the LI corresponds to strongest layer of the first codeword.”

Agreement

- WB CSI report, the UCI parameter ordering for LI reporting is as follows: CRI → RI → LI (if present) → Padding bits (if present) → PMI → CQI.

  - Note: This is already captured in specification

R1-1807810	CSI reporting offline session notes Thursday morning	Ericsson

Agreement

- Remove UE capability FG 2-57 on low latency CSI

- Remove UE capability component 6 in FG 2-35 on Type A/B CSI processing capability

- Refine the UE CSI processing capability as follows

  - Define two tables

    - Table 1 for the case where only a single CSI report is aperiodically triggered

    - Table 1 is defined for low-latency CSI only

Table 1: A-CSI timing requirement 1 with single capability

| SCS | Low-latency CSI |  |
| --- | --- | --- |
|  | CSI only (proposal) |  |
|  | Z(1) | Z’(1) |
| 15kHz | FFS: 10 or 9 | FFS: 8 or 7 |
| 30kHz | 13 | 11 |
| 60kHz | 25 | 21 |
| 120kHz | 43 | 36 |

    - Table 2 for all other cases

Table 2: A-CSI timing requirement 2

| SCS | Low-latency CSI |  |
| --- | --- | --- |
|  | CSI-only |  |
|  | Z(1) | Z’(1) |
| 15kHz | 22 | 16 |
| 30kHz | 33 | 30 |
| 60kHz | 44 | 42 |
| 120kHz | 97 | 85 |

Table 5: A-CSI timing requirement 2 (assuming CRI occupies Ks CPU) proposal

| SCS | High-Latency CSI |  |
| --- | --- | --- |
|  | CSI only (proposal) |  |
|  | Z(1) | Z’(1) |
| 15kHz | 40 | 37 |
| 30kHz | 72 | 69 |
| 60kHz | 141 | 140 |
| 120kHz | 152 | 140 |

    - When X CSI processing units are unoccupied and a single report with low latency CSI class without UL-SCH is triggered

    - Select one of the following alternatives:

      - The timing requirements are defined by Z(1) in Table 1

        - There is no “advanced UE CSI processing” capability in Rel-15

      - This report occupies X CSI processing units

      - X is the number of CSI processing units supported by a UE

    - Otherwise, the timing requirements are defined by (Z,Z’) in Table 2 and follows the regular CPU occupancy rules

- When a single DCI triggers N >= 1 A-CSI reports RS (each with single CSI-RS resource in the set for channel measurement) ((Zn, Z'n) for report n), but UE only has M<=X unoccupied CSI processing units, UE is only required to update M of the N CSI reports

  - Additionally, CSI for a report n=0,1,…M-1 is not updated if the PUSCH scheduling offset does not give enough CSI calculation time for that CSI report according to  maxk=0,…,min⁡(M,N)-1(Zk') where the reports are indexed according to the priority rule in Section 5.2.5 of TS38.214

- CPU occupancy rules for Table 2

  - When single DCI triggers N>=1 A-CSI reports based on A-CSI-RS (each with single CSI-RS resource in the set for channel measurement),

    - min(M, N) CPUs are occupied from the first symbol after the PDCCH triggering the report until the last symbol of the PUSCH carrying the report

  - For A/P/SP CSI reporting based on P/SP CSI-RS

    - The CSI reporting setting associated with P/SP CSI-RS/CSI-IM resource is assigned [one or Ks] CSI processing unit and occupy [it or them] from the earliest one of the first symbol of the latest CSI-RS/CSI-IM resource earlier than the corresponding CSI reference resource until the last symbol of the PUSCH/PUCCH carrying the report

  - When triggered with a CSI report with a set containing Ks > 1 CSI-RS resources for channel measurement

    - For Ks=2, at most 16 CSI-RS ports per CSI-RS resource can be configured

    - For 8>=Ks>2, at most 8 CSI-RS ports per CSI-RS resource can be configured

    - The CSI report occupies Ks CSI processing units and the latency requirement for each unit is calculated based on (Z, Z') according to the High latency CSI class with the Z values in Table 5

- Simultaneous resources in FG 2-36, 2-40, 2-41, 2-43, 2-33

  - In any slot, the number of “active” CSI-RS resources/ports should not exceed the numbers reported in FG2-36, 2-40, 2-41, 2-43, 2-33

  - An NZP CSI-RS resource is active in a duration of time defined as follows:

    - For A-CSI-RS, starting from the end of the PDCCH containing the request, and ending at the end of the PUSCH containing the report associated with this A-CSI-RS.

    - For SP-CSI-RS, starting from the end of the activation command is applied, and ending at the end of the deactivation command is applied.

    - For P-CSI-RS, the activate duration ends when the P-CSI-RS configuration is released.

- Modify UE feature group 2-33 as follows:

| Features |  |
| --- | --- |
| # | 2-33 |
| Feature group | CSI-RS and CSI-IM reception for CSI feedback |
| Components | 1. Supported max # of configured NZP-CSI-RS resources per CC 2. Supported max # of ports across all configured NZP-CSI-RS resources per CC 3. Supported max # of configured CSI-IM resources per CC 4. Supported max # simultaneous CSI-RS resources in active BWPs across all CCs 5. Supported max total # of CSI-RS ports in simultaneous CSI-RS resources in active BWPs across all CCs |
| Prerequisite feature groups | 2-32 |
| Need for gNB to know whether the | Yes |
| Consequences if the feature | CSI acquisition is not supported |
| Type (see R2-1712078) | Type 3 |
| Need of FDD/TDD differentiation | N.A. |
| Need of FR1/FR2 differentiation | N.A. |
| RAN5 implication |  |
| Note | Note: all the candidate values are the range of capability signaling which doesn’t determine whether UE is mandatory to support all the signaling values. |
| Responsible WG |  |
| RAN WG recommendation | Component-1 candidate values: {from 1 to 32} Component-2 candidate values: , further down-select between: Alt.1: {from 4 to 64}, Alt.2: {from 4 to 256}, Alt.3: {from 4 to 256} Component-3: candidate values: {1,2,4,8,16,32} Component-4: candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component-5: candidate values {8, 16, 24, …, 248, 256} |
| TSG-RAN decision |  |

Agreement:

If a UE only has M CPUS available and is triggered with N>M reports, UE shall be required to update the M reports with highest priority according to priority rule in Section 5.2.5

Agreement

- If UE supports X simultaneous CSI calculations, a UE is not expected to be configured with an aperiodic CSI trigger state containing more than X CSI reports settings across all CCs

Agreement

When single CSI report is not multiplexed with UL-SCH and Z’ timing requirement is not met (but Z condition is met), a UE does not expect to be scheduled with a DCI where the Z’ criterion is not met, and may ignore the DCI

- Agreement

- To select which reference SCS µ is used to determine Z and Z’ for a CSI report

  - Let µDL correspond to the subcarrier spacing of the PDCCH with which the DCI was transmitted, µUL corresponds to the subcarrier spacing of the PUSCH with which the CSI report is to be transmitted and µCSI-RS correspond to the minimum subcarrier spacing of the all aperiodic CSI-RS triggered by the DCI

  - To determine Z and Z’, the reference SCS µ corresponds to min(min(µDL, µCSI-RS), µUL)

- Note that subcarrier spacing corresponding to min(min(µDL, µCSI-RS), µUL) should be used for calculation of time interval

Conclusion

For PUSCH-based SP-CSI, each DCI only activates a single trigger state. One codepoint of the CSI request field triggers a single trigger state. The PUSCH-based SP-CSI trigger state definition is kept as is, each trigger states activates a single CSI report setting.

- The same size of CSI request field is used for SP-CSI as for aperiodic CSI and depends on reportTriggerSize. The number of trigger states for SP-CSI and aperiodic CSI does not need to be the same.

Agreement

For PUSCH-based SP-CSI, multiple SP-CSI trigger states are allowed to be simultaneously active. Deactivation of an SP-CSI trigger state is indicated with DCI format 0_1

- UE is not expected to transmit more than one SP-CSI on PUSCH per slot, for all CCs

Agreement

- Use the following DCI fields for SP-CSI activation/deactivation:

Table A: Special field for SP-CSI Activation PDCCH validation

|  | DCI format 0_1 |
| --- | --- |
| HARQ process number | set to all '0's |
| Redundancy version | set to '00' |

Table B: Special fields for SP-CSI Deactivation PDCCH Validation

|  | DCI format 0_1 |
| --- | --- |
| HARQ process number | set to all '0's |
| Modulation and coding scheme | set to all '1's |
| Resource block assignment | If higher layer configures RA type 0 only, set to all ‘0’s; If higher layer configures RA type 1 only, set to all ‘1’s; If higher layer configures dynamic switch between RA type 0 and 1, then if MSB is’0’, set to all ‘0’s; else, set to all ‘1’s |
| Redundancy version | set to '00' |

Agreement

How to correct CSI reference resource with respect to definition of downlink slot?

- Adopt TP below

<TP for Section 5.2.2.1.1 of TS 38.214>

A slot in a serving cell shall be considered to be a valid downlink slot if:

-	it comprises at least one higher layer configured downlink or flexible symbol is configured as a downlink slot for that UE, and

-	it does not fall within a configured measurement gap for that UE

</TP>

Agreement

- Add the below sentence to section 5.2.1.5.1 of TS 38.214

  - A UE is not expected to receive more than one DCI with CSI request per slot

  - Note: editor of TS 38.214 may change the exact wording to reflect the above sentence.

  - Note: This applies to Rel-15.

Conclusion

WB Type II CSI on PUSCH (comprising both Part 1 and Part 2) reporting is supported

Conclusion

- 38.212/38.213/38.214 editors to align definition of “CSI report” to mean either a set of PMI/RI/CQI/LI corresponding to one CSI report setting or the physical channel carrying possibly multiple CSIs

- 38.212/38.213/38.214 editors to align definition of single/multiple CQI/PMI to wideband/subband CQI/PMI

Agreement

- When (single or multiple) CSI report(s) is multiplexed with UL-SCH and N2’ criterion is met but Z’’ criterion is not met, the UE drops the CSI, but transmit the UL-SCH without CSI multiplexed on PUSCH

R1-1805827	Remaining issues on CSI reporting	ZTE

R1-1805950	Summary of remaining issues on CSI reporting	Huawei, HiSilicon

R1-1806043	Remaining issues on CSI reporting	vivo

R1-1806216	Corrections and clarifications for CSI reporting	Ericsson

R1-1806279	Remaining issues on CSI reporting	CATT

R1-1806340	Remining issues on CSI reporting	Lenovo, Motorola Mobility

R1-1806393	Remaining issues on CSI reporting	Spreadtrum Communications

R1-1806506	Remaining issues on CSI reporting	Intel Corporation

R1-1806609	Remaining issues on CSI reporting	LG Electronics

R1-1806713	On CSI processing and reporting latency	Samsung

R1-1806714	Issues on CSI reporting	Samsung

R1-1806787	Remaining issues in CSI reporting	MediaTek Inc.

R1-1806848	Remaining issues on CSI reporting	OPPO

R1-1806943	Clarification on aperiodic CSI reporting in CA case	Sharp

R1-1807013	Remaining details on CSI reporting 	InterDigital, Inc.

R1-1807144	Remaining issues on CSI reporting	NTT DOCOMO, INC.

R1-1807179	Remaining issues on CSI reporting	China Unicom

R1-1807183	Remaining issues on CSI reporting	Nokia, Nokia Shanghai Bell

R1-1807340	Maintenance for CSI reporting	Qualcomm Incorporated

##### Beam measurement and reporting

R1-1807679	Feature lead summary 2 for beam measurement and reporting	Ericsson

Agreement

Agree to the following text proposal for TS38.213 and TS38.214.

  - When the HARQ/ACK corresponding to the PDSCH carrying the MAC CE command is transmitted in the slot n, the command shall be applied starting from slot n + 3Nslotsubframe, + 1.

R1-1807782	Feature lead summary 3 for beam measurement and reporting	Ericsson

Agreement

>>> Text proposal for 38.214 Section 5.2.1.4.3 >>>

For L1-RSRP reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size, if the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as ‘enabled', the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP uses is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP uses is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. The mapping between the reported L1-RSRP value and the measured quantity is described in [11, TS 38.133].

>>> End text proposal >>>

Agreement

Text proposal for Section 9.2.1 in 38.213

If a UE does not have dedicated PUCCH resource configuration, provided by higher layer parameter PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by higher layer parameter pucch-ResourceCommon in SystemInformationBlockType1 through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial active UL BWP provided by SystemInformationBlockType1. The PUCCH resource set is provided by higher layer parameter PUCCH-Resource-Common and includes sixteen resources, each corresponding to a PUCCH format, a first symbol, a duration, a PRB offset, and a cyclic shift index set for a PUCCH transmission. The UE transmits a PUCCH using frequency hopping. If a UE does not have dedicated PUCCH resource configuration, provided by higher layer parameter PUCCH-ResourceSet in PUCCH-Config, the UE transmits the PUCCH using the same spatial domain transmission filter as for the Msg3 PUSCH transmission.

R1-1805828	Remaining issues on beam management	ZTE

R1-1805952	Remaining issues on beam management	Huawei, HiSilicon

R1-1806044	Remaining issues on beam measurement and reporting	vivo

R1-1806120	Ambiguities about PUCCH beam indication	Fujitsu

R1-1806217	Remaining issues for beam measurement and reporting	Ericsson

R1-1806280	Remaining issues on beam management	CATT

R1-1806353	Discussion of remaining details of beam reporting	Lenovo, Motorola Mobility

R1-1806394	Remaining issues on beam management	Spreadtrum Communications

R1-1806507	Remaining Issues on Beam Management	Intel Corporation

R1-1806563	Remaining issues on beam management	Sony

R1-1807610	Remaining issues on beam management	LG Electronics	(rev of R1-1806610)

R1-1806715	Corrections on beam reporting and indication	Samsung

R1-1806788	Remaining issues on beam management	MediaTek Inc.

R1-1806840	Remaining Issues and Text Proposals for Beam Management	OPPO

R1-1806944	Remaining issues on beam management	Sharp

R1-1806983	Remaining details of beam reporting	AT&T

R1-1807014	Remaining issues on beam management 	InterDigital, Inc.

R1-1807126	Discussion on remaining issues of beam management	CHTTL

R1-1807145	Remaining issues on beam management	NTT DOCOMO, INC.

R1-1807184	Remaining issues on beam management	Nokia, Nokia Shanghai Bell

R1-1807210	Remaining issues on beam management	ASUSTEK COMPUTER (SHANGHAI)

R1-1807341	Beam management for NR	Qualcomm Incorporated

R1-1807678	WF on UL beam management	LG Electronics, CATT, Ericsson, InterDigital, KT Corporation, Mitsubishi, OPPO, Sony, Spreadtrum

##### Mechanism to recover from beam failure

Including beamFailureREcoveryTimer related discussion per RAN plenary guidance as in RP-180597

R1-1805802	Reply LS on beam failure recovery	RAN2, Ericsson

R1-1806208	Draft LS reply on beam failure recovery	Ericsson

R1-1806478	Draft Response to LS on Beam Failure Recovery	Intel Corporation

R1-1807129	[Draft] Reply LS on beam failure recovery	Huawei, HiSilicon

R1-1807143	[DRAFT] LS reply on beam failure recovery	NTT DOCOMO, INC.

R1-1807180	Draft LS Response to RAN2 on beam failure recovery	Nokia, Nokia Shanghai Bell

R1-1807834	DRAFT Reply LS on beam failure recovery	Ericsson

Decision: Draft LS is endorsed with the following text revisions, approved in R1-1807725

Replace “Concern: NTT DoCoMo (more details on complexity needed), OPPO (complexity from NW/performance perspective should be included)” with “Some companies showed concerns on complexity from NW/performance perspective and additional specification effort.”

R1-1807661	Summary 1 on Remaing issues on Beam Failure Recovery	MediaTek

R1-1807796	Summary 2 on Remaing issues on Beam Failure Recovery	MediaTek

Agreement

- (Working Assumption) If either CORESET associated with SearchSpace-BFR or SearchSpace-BFR is not configured, UE assumes contention-free PRACH BFR is not performed.

  - In this case, SearchSpace-0 or CORESET-0 is not used for monitoring contention-free PRACH BFR response.

- If either CORESET associated with SearchSpace-BFR or SearchSpace-BFR is not configured, UE can still perform contention-based PRACH BFR based on configuration.

- Send LS to RAN2

R1-1807881	[draft] LS on BFR search space configuration	MediaTek

Decision: Draft LS is endorsed and final LS is approved in R1-1807883

Working Assumption

In case of implicit configuration of BFD set q0, if there are two RS indices in the RS set indicated by TCI states for CORESETs, the RS indices associated with QCL type D in the RS sets indicated by TCI states for CORESETs are used for beam failure detection.

R1-1805829	Remaining issues on beam recovery	ZTE

R1-1805953	Remaining issues on beam failure recovery	Huawei, HiSilicon

R1-1806045	Remaining issues on mechanism to recover from beam failure	vivo

R1-1806218	Remaining issues for beam recovery	Ericsson

R1-1806281	Remaining issues on beam failure recovery	CATT

R1-1806354	Discussion of remaining details of beam failure recovery	Lenovo, Motorola Mobility

R1-1806359	Discussion on remaining issues on beam failure recovery	CMCC

R1-1806395	Remaining issues on beam failure recovery	Spreadtrum Communications

R1-1806508	Remaining Issues on Beam Failure Recovery	Intel Corporation

R1-1806611	Remaining issues on beam failure recovery	LG Electronics

R1-1806716	Corrections on beam failure recovery	Samsung

R1-1806789	Remaining issues on beam failure recovery	MediaTek Inc.

R1-1806841	Remaining issues and Text Proposal for Beam Failure Recovery	OPPO

R1-1807015	Remaining details on beam failure recovery 	InterDigital, Inc.

R1-1807621	Remaining issues on beam recovery	NTT DOCOMO, INC.	( rev of R1-1807146)

R1-1807159	Remaining issues on beam recovery	ITRI

R1-1807185	Remaining Details on Beam Recovery	Nokia, Nokia Shanghai Bell

R1-1807211	Remaining issues on beam failure recovery procedure	ASUSTEK COMPUTER (SHANGHAI)

R1-1807342	Beam recovery procedures	Qualcomm Incorporated

##### CQI and MCS

R1-1807649	Summary of CQI/MCS	AT&T

Agreement (Impact to RAN2 specifications)

Adopt the following text proposal in 38.214

-------------------------------------begin text proposal--------------------------------------

6.1.4.1	Modulation order and target code rate determination

>>> unchanged text omitted <<<

-	for MCS index 0, 1 and 28, q=1 if UE has reported to support pi/2 BPSK modulation; and q=2 in other cases

-     for MCS index 0, 1, and 28, if higher-layer parameter PUSCH-tp-pi2BPSK is configured, q = 1; otherwise q= 2

-------------------------------------end text proposal--------------------------------------

Include as part of LS to RAN2

Agreement (Impact to RAN2 specifications)

A new RRC parameter, PUSCH-tp-pi2BPSK, needs to be introduced to clarify default capability. The table below provides the parameter values and other details. This parameter is UE optional.

| Sub-feature Group | RAN1 Spec | RAN2 Parent IE | Parameter Name | Description | Values (default) | UE/Cell Specific | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH | 38.214 | PUSCH-Config | PUSCH-tp- pi2PBSK | For a UE capable of supporting pi/2-BPSK for PUSCH with transform precoding, this parameter configures whether pi/2-BPSK modulation is enabled or disabled for PUSCH with transform precoding. For a UE not capable of supporting pi/2-BPSK on PUSCH with transform precoding, this parameter is not applicable. | {enabled, disabled} (disabled) | UE specific | Introduced to clarify default capability. |

LS to RAN2 to inform them of the above agreements on pi/2 BPSK.

R1-1807808	[DRAFT] Signalling Aspects Related to Pi/2 BPSK	Qualcomm

Decision: Final LS is approved in R1-1807720.

R1-1805951	Remaining issues on CQI and MCS	Huawei, HiSilicon

R1-1806046	Remaining issues for CQI and MCS	vivo

R1-1806214	Remaining issues on CQI and MCS	Ericsson

R1-1806717	Remaining issues on CQI and MCS	Samsung

R1-1806976	Remaining Issues in CQI/MCS	AT&T

R1-1807343	Remaining Issues in MCS	Qualcomm Incorporated

##### Other

Including handling simultaneous transmission and reception of different channels and reference signals

R1-1807708	Summary of Multiplexing of Physical Channels and RS	Intel

Agreement

For the case of CSIRS+PDCCH on the same OFDM symbols where spatial QCL is configured for CSI-RS

- NW configuration should ensure spatial QCL

  - UE is not expected to be configured with CSI-RS with repetition=ON in the symbols UE is configured to monitor the CORESET

Above agreement applies to both same BWP as well as intra-band CA if spatial QCL is applicable.

For further study until the next meeting

Alt 1: For the case of CSIRS+PDCCH on the same OFDM symbols, if CSI-RS is not configured with spatial QCL, the UE follows the spatial QCL associated with the PDCCH

Alt 2: No need for further definition of UE behavior for the case of CSIRS not configured with spatial QCL overlapping with PDCCH on an OFDM symbol

R1-1807818	Summary of offline on simultaneous transmission and reception of channels/signals	Intel

For further study until the next meeting

For the case of CSIRS+PDSCH on the same OFDM symbols where spatial QCL is configured for CSI-RS

- NW configuration should ensure spatial QCL

  - UE is required to decode PDSCH that is overlapping with one or more symbols configured with CSI-RS with repetition=ON

  - Note: The UE is not expected to perform Rx beam refinement on CSIRS symbols configured with CSI-RS with repetition=ON

- NW configuration should ensure spatial QCL

  - UE is expected to symbol level rate-match PDSCH that is overlapping with one or more symbols on the symbols configured with CSI-RS with repetition=ON

    - DMRS should not be affected by rate matching

- NW configuration should ensure spatial QCL

  - UE is not expected to receive PDSCH that is overlapping with one or more symbols configured with CSI-RS with repetition=ON

    - DMRS should not be affected by rate matching

For further study until the next meeting

For the case of PDSCH+PDCCH simultaneous RX in case spatial RX is configured and is different between the two channels

  - Alt1: CORESET is prioritized

    - spatial QCL associated with CORESET is applied to the PDSCH

    - FFS: PDSCH slot aggregation case

  - Alt2: PDSCH is prioritized depending on scheduling offset

  - Before threshold, CORESET is prioritized

  - After threshold, PDSCH is prioritized

R1-1805830	Simultaneous transmission and reception of channels/signals	ZTE

R1-1805955	Remaining issues on simultaneous reception and transmission over CCs and BWPs	Huawei, HiSilicon

R1-1806047	Remaining issues on simultaneous reception of DL/UL physical channels and reference signals	vivo

R1-1806124	Enhancements on Type II CSI Reporting scheme	Fraunhofer IIS

R1-1806219	On simultaneous reception of physical and reference signals across CCs	Ericsson

R1-1806220	On signalling reduction for beam management	Ericsson

R1-1806221	Multi-cell beam recovery	Ericsson

R1-1806282	Simultaneous Tx/Rx of channels and reference signals	CATT

R1-1806355	Discussion of UE behavior related to QCL assumption in CA	Lenovo, Motorola Mobility

R1-1806509	Simultaneous Tx and Rx of physical channels and RS	Intel Corporation

R1-1806510	On UE capability signalling for Type II CSI	Intel Corporation

R1-1806718	Aperiodic beam reporting	Samsung

R1-1806719	Corrections on UL beam management	Samsung

R1-1806720	Simultaneous transmission and reception of physical channels and reference signals	Samsung

R1-1806790	Views on multi-CC QCL assumptions	MediaTek Inc.

R1-1806842	Discussion on simultaneous receptions/transmissions of multiple signals/channels	OPPO

R1-1806892	CSI feedback enhancements for URLLC	Huawei, HiSilicon

R1-1806952	Miscellaneous corrections and clarifications on CSI acquisition	Ericsson

R1-1806953	On OFDM symbol level PDSCH power boosting	Ericsson

R1-1806954	Remaining details on SP CSI activation/deactivatin  on PUSCH	Ericsson

R1-1807128	Further enhancements on CSI reporting and codebook design	Huawei, HiSilicon

R1-1807147	On Type II codebook enhancement	NTT DOCOMO, INC.

R1-1807148	Simultaneous reception of DL/UL physical channels and reference signals	NTT DOCOMO, INC.

R1-1807186	Remaining Details on the CSI Framework	Nokia, Nokia Shanghai Bell

R1-1807187	Remaining Issue on Type II CSI Omission	Nokia, Nokia Shanghai Bell

R1-1807188	On channels multiplexing	Nokia, Nokia Shanghai Bell

R1-1807189	Decoupling DL and UL beam selection	Nokia, Nokia Shanghai Bell

R1-1807190	Discussion on DL and UL beam tracking	Nokia, Nokia Shanghai Bell

R1-1807191	Discussion on PUCCH Beam management	Nokia, Nokia Shanghai Bell

R1-1807192	Discussion on PUSCH Beam Indication	Nokia, Nokia Shanghai Bell

R1-1807193	Joint L1-RSRP Beam Reporting for SSB and CSI-RS	Nokia, Nokia Shanghai Bell

R1-1807194	On Spatial Relation between UL and DL Beams	Nokia, Nokia Shanghai Bell

R1-1807344	Details on  simultaneous reception of PHY channels and RS in FR2	Qualcomm Incorporated

#### Reference signals and QCL

R1-1806222	On DMRS groups in Rel-15	Ericsson

##### Multiplexing of different types of RSs

R1-1807657	Feature lead summary of remaining issues for RS multiplexing	Huawei, HiSilicon

Agreement:

TP for Section 6.2.1 in TS 38.214

------Unchanged parts are omitted------

For PUCCH formats 0 and 2, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s) or if aperiodic SRS is configured and PUCCH consists of beam failure request. A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with semi-persistent or periodic PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only.

------Unchanged parts are omitted------

Agreement:

If the following change is captured in TS38.213 after Busan meeting, then the red highlighted part can be removed from TS38.214.

6.2.1	UE sounding procedure

<……>

A UE shall not transmit simultaneously SRS resource(s) and PRACH. If SRS and PRACH would occur on overlapping OFDM symbol(s), SRS is not transmitted on the overlapping symbol(s).

R1-1805956	Remaining issues on RS multiplexing	Huawei, HiSilicon

R1-1806048	Remaining issues on multiplexing of different types of RSs	vivo

R1-1806223	On multiplexing of different types of RSs	Ericsson

R1-1806721	Remaining issues on RS multiplexing	Samsung

R1-1806807	Remaining issues on multiplexing	MediaTek Inc.

R1-1807293	Discussions on multiplexing of different types of RSs	Mitsubishi Electric Co.

R1-1807345	Maintenance for Multiplexing of RS	Qualcomm Incorporated

##### CSI-RS

R1-1807656	Feature lead summary of CSI-RS #0	Huawei, HiSilicon

Agreement (Impact to RAN2 specifications)

Text proposal #3 for Section 5.2.2.3.1 in TS38.214

< Unchanged parts are omitted >

The bandwidth and initial CRB index of a CSI-RS resource within a BWP, as defined in Subclause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min24, NBWPsize.

< Unchanged parts are omitted >

Send an LS to RAN2.

Agreement (Impact to RAN2 specifications)

Change the value range of nrofRBs in CSI-RS bandwidth configuration from 24 to maxNrofPhysicalResourceBlocks + 1 and include as part of an LS to RAN2. Ask RAN2 to update the relevant description.

Send an LS to RAN2.

Agreement

UE expects to be configured with same BWP_info for all CSI-RS resources within one set.

UE expects to be configured with same density for all CSI-RS resources within one set.

Agreement

- For P/SP-CSI-RS, UE expects to be configured with same periodicity for all CSI-RS resources within one set (slot offset can still be different for different CSI-RS resources).

Agreement

Text proposal for Section 5.1.6.1.2 in 38.214

If the UE is configured with a CSI-ReportConfig with reportQuantity set to "cri-RSRP", or "none" and if the CSI-ResourceConfig for channel measurement (higher layer parameter resourcesForChannelMeasurement) contains a NZP-CSI-RS-ResourceSet that is configured with the higher layer parameter repetition  and configured without the higher layer parameter trs-Info, the UE can only be configured with the same number (1 or 2) of  ports with the higher layer parameter nrofPorts for all CSI-RS resources within the set,

Agreement

UE expects to be configured with same number of ports for all CSI-RS resources within one set.

Agreement

Text proposal for Section 7.4.1.5.2 in TS38.211

< Unchanged parts are omitted >

where the pseudo-random sequence  is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

at the start of each OFDM symbol where  is the slot number within a radio frame,  is the OFDM symbol number within a slot, and  equals the higher-layer parameter scramblingID.

< Unchanged parts are omitted >

Note: Inside the equation, the number of 14 is replaced as N_symb^slot.

Agreement

Suggest RAN2 to change the value range of firstOFDMSymbolInTimeDomain2 as from 2 to 12 and include as part of an LS to RAN2.

R1-1807843	[Draft] LS on NR CSI-RS	Huawei, HiSilicon

Decision: Draft LS is endorsed and final LS is approved in R1-1807887.

Agreement

Text proposal for Section 7.4.1.5.1 in 38.211

< Unchanged parts are omitted >

Zero-power and non-zero-power CSI-RS are defined

-	for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3

-	for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are not used for PDSCH transmission. UE performs the same measurement/reception on channels/RSs except PDSCH regardless of whether they collide with ZP CSI-RS or not.

< Unchanged parts are omitted >

R1-1807717	Feature lead summary of CSI-RS #2	Huawei, HiSilicon

Working Assumption

Text proposal for TS38.214

If the UE is configured with a CSI-RS resource and a search space set associated with a CORESET in the same OFDM symbol(s), the UE may assume that a PDCCH DM-RS transmitted in the all search space sets associated with CORESET CORESET are quasi co-located with ‘QCL-Type D’, if ‘QCL-Type D’ is applicable. Furthermore, the UE shall not expect to be configured with CSI-RS in PRBs that overlap those of the CORESET in the OFDM symbols occupied by the search space set(s).

R1-1805831	Remaining issues on CSI-RS	ZTE

R1-1805957	Remaining details on CSI-RS design in NR	Huawei, HiSilicon

R1-1806049	Remaining issues on CSI-RS	vivo

R1-1806224	Remaining details on CSI-RS	Ericsson

R1-1806283	Remaining issues on CSI-RS	CATT

R1-1806341	Remaining issues on CSI-RS	Lenovo, Motorola Mobility

R1-1806511	Remaining issues on CSI-RS	Intel Corporation

R1-1806722	Remaining issues on CSI-RS	Samsung

R1-1806808	Remaining issues on CSI-RS	MediaTek Inc.

R1-1806826	Text proposal on CSI-RS design	OPPO

R1-1807195	Common parameters on CSI-RS resource set configuration	Nokia, Nokia Shanghai Bell

R1-1807346	Maintenance for on CSI-RS	Qualcomm Incorporated

R1-1807696	WF on Clarification of report quantity set to "none"	LG Electronics, MediaTek

##### DMRS

R1-1807691	Finalizing PDSCH/PUSCH’s DM-RS for NR Rel-15	Qualcomm

Agreement:

Text proposal for TS38.214:

4.1	Power allocation for downlink

...

For downlink DM-RS associated with PDSCH, the UE may assume the ratio of PDSCH EPRE to DM-RS EPRE (1/[dB]) is given by Table 4.1-1 according to the number of DM-RS CDM groups without data as described in Subclause 5.1.6.2. The DM-RS scaling factor  specified in subclause 7.4.1.1.2 of [4, TS 38.211] is given by .

Table 4.1-1: The ratio of PDSCH EPRE to DM-RS EPRE

| Number of DM-RS CDM groups without data | DM-RS configuration type 1 | DM-RS configuration type 2 |
| --- | --- | --- |
| 1 | 0 dB | 0 dB |
| 2 | -3 dB | -3 dB |
| 3 | - | -4.77 dB |

...

#### 5.1.6.2  DM-RS reception procedure

...

The UE is not expected to assume co-scheduled UE(s) with different DM-RS configuration with respect to the actual number of front-loaded DM-RS symbol(s), the number of additional DM-RS, the DM-RS symbol location, and DM-RS configuration type as described in Subclause 7.4.1.1 of [4, TS 38.211].

When receiving PDSCH scheduled by DCI format 1_1, the UE shall assume that the CDM groups indicated in the configured index from Tables 7.3.1.2.2-1, 7.3.1.2.2-2-4, 7.3.1.2.2-1-3, 7.3.1.2.2-1-4 of [5, TS. 38.212] contain potential co-scheduled downlink DM-RS and are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) in Tables 7.3.1.2.2-1-1, 7.3.1.2.2-1-2, 7.3.1.2.2-1-3, 7.3.1.2.2-1-4 of [5, TS. 38.212] correspond to CDM group 0, {0,1}, {0,1,2}, respectively.

When receiving PDSCH scheduled by DCI format 1_0, the UE shall assume the number of DM-RS CDM groups without data is 1 which corresponds to CDM group 0 for the case of PDSCH with allocation duration of 2 symbols, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases.

...

6.2.2	UE DM-RS transmission procedure

...

For PUSCH scheduled by DCI format 0_1, the UE shall assume the DM-RS CDM groups indicated in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of Subclause 7.3.1.1 of [5, TS38.212] are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) correspond to CDM group 0, {0,1}, {0,1,2}, respectively.

For PUSCH scheduled by DCI format 0_0, the UE shall assume the number of DM-RS CDM groups without data is 1 which corresponds to CDM group 0 for the case of PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases.

For uplink DM-RS with PUSCH, the UE may assume the ratio of PUSCH EPRE to DM-RS EPRE ([dB]) is given by Table 6.2.2-1 according to the number of DM-RS CDM groups without data. The DM-RS scaling factor  specified in subclause 6.4.1.1.3 of [4, TS 38.211] is given by .

Agreement

TP for Section 6.2.2 in 38.214:

### 6.2.2	UE DM-RS transmission procedure

When transmitted PUSCH is not scheduled by PDCCH format 0_1 with CRC scrambled by C-RNTI or CS-RNTI, the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and

If frequency hopping is disabled:

-	The UE shall assume dmrs-AdditionalPosition=’pos2’ and up to two additional DM-RS can be transmitted according to PUSCH duration.

If frequency hopping is enabled:

-	The UE shall assume dmrs-AdditionalPosition=’pos1’ and up to one additional DM-RS can be transmitted according to PUSCH duration.

…

Agreement

Text proposals for TS 38.211 Section 6.4.1.1.3

6.4.1.1.3	Precoding and mapping to physical resources

------Unchanged parts are omitted------

The time-domain index  and the supported antenna ports  are given by Tables 6.4.1.1.3-3 through 6.4.1.1.3-6:

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 1 and frequency hopping is disabled, the tables shall be used according to single-symbol DM-RS

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 2 and frequency hopping is disabled, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used

-	if the higher-layer parameter dmrs-AdditionalPosition is equal to or more than 1 and frequency hopping is enabled according to clause 7.3.1.1.2 in [5, TS 38.212], Table 6.4.1.1.3-6 shall be used assuming dmrs-AdditionalPosition is 1 for each hop.

------Unchanged parts are omitted------

Table 6.4.1.1.3-6: PUSCH DM-RS positions  for single-symbol DM-RS and frequency hopping enabled.

| Duration in symbols for each hop | DM-RS positions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUSCH mapping type A |  |  |  |  |  |  |  | PUSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition (=2) |  |  |  | dmrs-AdditionalPosition (=3) |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 1 |  |
|  | First hop | Second hop | First hop | Second hop | First hop | Second hop | First hop | Second hop | First hop | Second hop | First hop | Second hop |
| ≤3 | - | - | - | - | - | - | - | - |  | 0 |  | 0 |
| 4 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |
| 5, 6 |  | 0 |  | 0, 4 |  | 0 |  | 0, 4 |  | 0 | , | 0, 4 |
| 7 |  | 0 | , | 0, 4 |  | 0 |  | 0, 4 |  | 0 | , | 0, 4 |

Agreement

=========================== Text Proposal for TS38.214   ==============================

5.1.6.2	DM-RS reception procedure

When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, or receiving PDSCH scheduling DCI format 1_0 by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in subclause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition

< Unchanged parts are omitted >

================================= End of Text Proposal ===============================

Agreement

Text proposals for TS 38.214 Section 5.1.6.2

------Unchanged parts are omitted------

5.1.6.2	DM-RS reception procedure

When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, or receiving PDSCH scheduling DCI format 1_0 by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in subclause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition

------Unchanged parts are omitted------

A UE shall assume that the CDM groups indicated in the configured index from Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-4 of [5, TS. 38.212] contain potential co-scheduled downlink DM-RS and are not used for data transmission, where “1”, “2” and “3” for the number of DM-RS CDM group(s) in Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-4 of [5, TS. 38.212] correspond to CDM group 0, {0,1}, {0,1,2}, respectively.

------Unchanged parts are omitted------

Agreement

/************************* Start of Text Proposal for 38.211 ************************/

6.4.1.1.3	Precoding and mapping to physical resources

The time-domain index  and the supported antenna ports  are given by Tables 6.4.1.1.3-3 through 6.4.1.1.3-6:

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 1 and frequency hopping is disabled, the tables shall be used according to single-symbol DM-RS

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 2 and frequency hopping is disabled, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used

-	if the higher-layer parameter dmrs-AdditionalPosition is equal to or more than 1 and frequency hopping is enabled according to clause 7.3.1.1.2 in [4, TS 38.212], Table 6.4.1.1.3-6 shall be used assuming dmrs-AdditionalPosition is 1 for each hop. If , dmrs-AdditionalPosition is assumed to be 0.

/******************************* omitted part *******************************/

Table 6.4.1.1.3-5: PUSCH DM-RS time index .

| DM-RS duration |  | Supported antenna ports |  |
| --- | --- | --- | --- |
|  |  | Configuration type 1 | Configuration type 2 |
| single-symbol DM-RS | 0 | 0 – 3 | 0 – 5 |
| double-symbol DM-RS | 0, 1 | 0 – 7 | 0 – 11 |

/************************** End of Text Proposal for 38.211*************************/

Agreement

For PDSCH/PUSCH mapping type A with 8 symbols duration

- Support the value in the bracket (additional DMRS in symbol 7 is supported)

For PUSCH mapping type B with 5 symbols duration,

- Support the value in the bracket (additional DMRS in symbol 4 is supported)

Send an LS to RAN4 to inform them of the potential issue on different power / different antenna (Peng)

R1-1807757	[DRAFT] LS on DMRS positions of PUSCH	vivo

Decision: Final LS is approved in R1-1807726.

Conclusion:

No consensus in RAN1 for addressing high PAPR in CP-OFDM DMRS for NR Rel-15.

R1-1807745	Finalizing PDSCH/PUSCH’s DM-RS for NR Rel-15	Qualcomm

Agreement

=============================== Text Proposal for TS38.211 ===============================

6.4.1.1	Demodulation reference signal for PUSCH

6.4.1.1.1	Sequence generation

6.4.1.1.1.1	Sequence generation when transform precoding is disabled

If transform precoding for PUSCH is not enabled, the sequence  shall be generated according to

.

where the pseudo-random sequence  is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

where  is the OFDM symbol number within the slot,  is the slot number within a frame, and

-	 and  are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1 not a msg3 PUSCH according to clause 8.3 in [5, TS 38.213];

-	 and  is given by the higher-layer parameter scramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI;

-	 and  otherwise.

is indicated by the DMRS sequence initialization field in the DCI Format 0_1 in 3GPP TS 38.212 [X] associated with the PUSCH transmission.

< Unchanged parts are omitted >

7.4.1.1	Demodulation reference signals for PDSCH

7.4.1.1.1	Sequence generation

The UE shall assume the sequence  is defined by

.

where the pseudo-random sequence  is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

where  is the OFDM symbol number within the slot,  is the slot number within a frame, and

-	 and  are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_1 with the CRC scrambled by C-RNTI or CS-RNTI;

-	 and  is given by the higher-layer parameter scramblingID0 in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_0 with the CRC scrambled by C-RNTI or CS-RNTI;

-	 and  otherwise

is indicated by the DMRS sequence initialization field in the DCI Format 1_1 in 3GPP TS 38.212 [X] associated with the PDSCH transmission.

================================= End of Text proposal  ===============================

R1-1805832	Remaining issues on DMRS	ZTE

R1-1805958	Remaining issues on DMRS design	Huawei, HiSilicon

R1-1806050	Remaining issues on DMRS	vivo

R1-1806225	Remaining issues on DMRS	Ericsson

R1-1806284	Remaining issues on DMRS	CATT

R1-1806396	Reimaining issues on DMRS	Spreadtrum Communications

R1-1806723	Remaining issues on DMRS	Samsung

R1-1806827	Text proposal on DMRS design	OPPO

R1-1806974	Remaining Issues of DMRS for NR	AT&T

Withdrawn

R1-1807149	Remaining details on DM-RS	NTT DOCOMO, INC.

R1-1807196	Further comments on DM-RS for NR physical data channels	Nokia, Nokia Shanghai Bell

R1-1807292	Discussions on DL DMRS	Mitsubishi Electric Co.

R1-1807295	Remaining issues on DL DMRS	KT Corp.

R1-1807347	Maintenance for DMRS	Qualcomm Incorporated

##### PT-RS

R1-1807658	Feature lead summary 1 of PT-RS	Ericsson

Agreement

Text proposal for TS 38.214 v15.1.0 Section 5.1.6.3

A UE shall report the preferred MCS and bandwidth thresholds based on the UE capability at a given carrier frequency, for each subcarrier spacing applicable to data channel at this carrier frequency, assuming the MCS table with the maximum ModOrder as it reported to support.

If a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-DownlinkConfig ,

-    the higher layer parameters timeDensity and frequencyDensity in PTRS-DownlinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2,  respectively.

-	if either or both of the additional higher layer parameters timeDensity and frequencyDensity are configured, and the RNTI equals C-RNTI or CS-RNTI, the UE shall assume the PT-RS antenna ports’ presence and pattern are a function of the corresponding scheduled MCS of the corresponding codeword and scheduled bandwidth in corresponding bandwidth part as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2,

-	if the higher layer parameter timeDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume LPT-RS = 1.

-	if the higher layer parameter frequencyDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume KPT-RS = 2.

-	otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals C-RNTI or CS-RNTI,  the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2 and the UE shall assume The UE shall assume that PT-RS is not present when,

-	the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or

-	the scheduled MCS from Table 5.1.3.1-2 is smaller than 5, or

-	the number of scheduled RBs is smaller than 3, or

-	the RNTI equals RA-RNTI, SI-RNTI or P-RNTI.

-	otherwise, if the RNTI equals RA-RNTI, SI-RNTI, or P-RNTI, the UE shall assume PT-RS is not present

Agreement

Text proposal for clause 6.2.3.2 of TS 38.214

The UE shall assume no PT-RS is present when the number of scheduled RBs is less than or equal to NRB0 if NRB0 > 01 or if the RNTI equals TC-RNTI.

Agreement

Text proposal for 38.214, section 6.2.3.1

<unchanged parts omitted>

For PT-RS, the transmit power of PT-RS is derived from , which is the power ratio between power of PUSCH and power of PT-RS per port.

For codebook based uplink transmission, wWhen the UE is scheduled with Qp={1,2} PT-RS port(s) in uplink and the number of scheduled layers is ,

-	If the UE is configured with higher layer parameter ptrs-Power, the PUSCH to PT-RS power ratio per layer per RE  is given by , where  is shown in the Table 6.2.3.1-3 according to the higher layer parameter ptrs-Power, the PT-RS scaling factor  specified in subclause 6.4.1.2.2.1 of [4, TS 38.211] is given by  and also on the TPMI field in DCI.	The UE shall assume ptrs-Power in PTRS-UplinkConfig  is set to state "00" in Table 6.2.3.1-3 if not configured or in case of non-codebook based PUSCH .

- Table 6.2.3.1-3: Factor related to PUSCH to PT-RS power ratio per layer per RE

| UL-PTRS-power / |  | The number of PUSCH layers ( ) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 |  | 3 |  | 4 |  |  |
|  | All cases | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial coherent | Non-coherent and non-codebook based |
| 00 | 0 | 3 | 3Qp-3 | 4.77 | 3Qp-3 | 6 | 3Qp | 3Qp-3 |
| 01 | 0 | 3 | 3 | 4.77 | 4.77 | 6 | 6 | 6 |
| 10 | Reserved |  |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |  |

Agreement

Text proposal for Section 6.4.1.2.2.1 and 7.4.1.2.2 of TS 38.211

is the C-RNTI, which can be C-RNTI/SP-CSI C-RNTI/CS-RNTI for DCI-based scheduling, and CS-RNTI for grant-free type 1 transmission.

Conclusion

The TS38.211, 38.214 editors are encouraged to check editorial corrections in R1-1805990 related to PTRS

Agreement

Text proposals for TS 38.211 v15.1.0 Section 7.4.1.2.2

< Unchanged parts are omitted >

If present, the UE shall assume the PDSCH PT-RS is scaled by a factor  to conform with the transmission power specified in clause 4.1 of [6, TS 38.214] and mapped to resource elements according to

when all the following conditions are fulfilled

-	 is within the OFDM symbols allocated for the PDSCH transmission

-	resource element  is not used for DM-RS, NZP CSI-RS (except for NZP CSI-RS for mobility), ZP CSI-RS, SS/PBCH block,

R1-1807659	Feature lead summary 2 of PT-RS	Ericsson

R1-1805833	Remaining issues on PT-RS	ZTE

R1-1805960	Remaining issues on PT-RS	Huawei, HiSilicon

R1-1805990	Remaining issues on PTRS	Mitsubishi Electric RCE

R1-1806051	Remaining issues on PT-RS	vivo

R1-1806154	Remaining issues on PT-RS	Panasonic Corporation

R1-1806226	Remaining issues on PT-RS	Ericsson

R1-1806285	Remaining issues on PT-RS	CATT

R1-1806397	Remaining issues on PT-RS	Spreadtrum Communications

R1-1806512	Remaining Issues on PT-RS	Intel Corporation

R1-1806612	Text proposals on PT-RS	LG Electronics

R1-1806724	Issues on PT-RS design	Samsung

R1-1806805	Remaining issues on PTRS	MediaTek Inc.

R1-1807150	Remaining details on PTRS	NTT DOCOMO, INC.

R1-1807197	Further comments on PT-RS	Nokia, Nokia Shanghai Bell

R1-1807605	PTRS considerations	Qualcomm Incorporated	(rev of R1-1807348)

##### SRS

R1-1807602	Summary of SRS	Sony

Agreement

The two aperiodic SRS resource sets for 1T4R antenna switching can only be allowed to be triggered by one single DCI, which means that the UE shall expect that the two SRS resource sets for 1T4R antenna switching are both configured with the same value of the higher layer parameter aperiodicSRS-ResourceTrigger and with different values of the higher layer parameters slotoffset.

Agreement:

RAN1 has identified the following adjacent TDM combinations of SRS resource configured for antenna switching and other physical channels

- SRS-PUSCH

- PUSCH-SRS

- SRS-PUCCH

- PUCCH-SRS

- RAN1 notes that PUCCH may convey payload with different priorities (e.g., CSI, ACK/NACK), or may have different formats, or PUSCH and PUCCH may use different number of UL symbols

- Send an LS to RAN4 request that they address the issue of antenna switching in such adjacent combinations in RAN4 spec

  - RAN1 does not plan to introduce scheduling restrictions on back to back SRS antenna switching and PUSCH/PUCCH channel unless serious concerns are identified in RAN4

R1-1807754	[DRAFT] LS on adjacent TDM combinations of SRS for antenna switching and other UL physical channels	Qualcomm

Decision: Final LS is approved in R1-1807722.

Agreement:

For a SRS resource set,

- a UE is not expected to be configured with multiple SRS resources with different periodicities within a SRS resource set.

Agreement

New text in Section 6.2.1.3 in 38.214:

For n-th (n>=1) aperiodic SRS transmission on a cell c, upon detection of a positive SRS request on a grant, the UE shall commence this SRS transmission on the configured symbol and slot provided

- it is no earlier than the summation of

  - the maximum time duration between the two durations spanned by N OFDM symbols of the numerology of cell c and the cell carrying the grant respectively, and

  - the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL,

- it does not collide with any previous SRS transmissions, or interruption due to UL or DL RF retuning time.

otherwise, n-th SRS transmission is dropped, where N is the reported capability as the minimum time interval in unit of symbols, between the DCI triggering and aperiodic SRS transmission

Agreement

Text proposal for section 6.2.1.3 of 38.214

A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier indicating indicating the switching from carrier c2 which is configured for PUSCH/PUCCH transmission in the case that switching-from carrier is from the serving cell configured with one uplink carrier. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwichFromCarrier indicating the switching from carrier c2 which is configured for PUSCH/PUCCH transmission in the case that switching-from carrier is from the serving cell configured with two uplink carriers. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL), uplink transmission is interrupted the UE temporarily suspends the uplink transmission on carrier c2.

Agreement:

Text proposal for section 6.2.1.3 of 38.214 (remove brackets)

If the UE is not configured for PUSCH/PUCCH transmission on carrier c1 with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on carrier c1 and serving cell c2, the UE is not expected to be configured or indicated with SRS resource(s) such that SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) would collide with the REs corresponding to the SS/PBCH blocks configured for the UE or the slots belonging to a control resource set indicated by [SystemInformationBlockType0] or [SystemInformationBlockType1] on serving cell c2.

  - 38.214 editor to check the higher layers parameters SystemInformationBlockType0, SystemInformationBlockType1

R1-1807716	Summary of SRS	Sony

Agreement

Make the following changes:

- Change the title of Section 6.2.1.2 in 38.214 as follows

6.2.1.2 UE antenna switching UE sounding procedure for DL CSI acquisition

- Support following change in UE capability 2-55 (SRS Tx Switch)

Component-1 is a list of TRx  pairs, candidates are {1T2R, 1T4R, 2T4R, 1T4R/2T4R, T=R}

Agreement

>>> Text proposal for 38.214 Section 6.2.1.2 >>>

When UE antenna switching is enabled by the higher layer parameter SRS-SetUse set as 'antenna switching' for a UE that supports transmit antenna switching, a UE may be configured with one of the following configurations depending on the UE capability:

-	up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each set has with two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and where the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, or

-	up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each SRS resource set has with two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and where the SRS port pair of the second resource in the set is associated with a different UE antenna port pair than the SRS port pair of the first resource in the same set, or

-	up to one SRS resource set configured with higher layer parameter resourceType in SRS-ResourceSet set to “periodic” or “semi-persistent”, withwith four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and where the SRS port of each resource in the set is associated with a different UE antenna port, and or

zero or two SRS resource sets each configured with higher layer parameter resourceType in SRS-ResourceSet set to “aperiodic” and with a total of four SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in given two sets is associated with a different UE antenna port. The two sets are each configured with two SRS resources, or one set is configured with one SRS resource and the other set is configured with three SRS resources. The UE shall expect that the two sets are both configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet, or,

-	up to two SRS resource sets each with one SRS resource, where the number of SRS ports for each resource is equal to 1, 2, or 4

and a guard period of Y symbols, where the UE does not transmit any other signal, is used in the case the SRS resource of a set are transmitted in the same slot. In this case, the guard period is in-between the SRS resources of the set.

If the UE indicates support of “1T4R/2T4R” antenna switching and is configured with SRS sets with SRS-SetUse set as “antenna switching”, the UE is only expected to be configured with same number of SRS ports, either one or two, for all SRS resources in the sets.

The UE does not expect to be configured or triggered more than one SRS resource set with SRS-SetUse set as “antenna switching” in the same slot, if the UE indicates support of “1T2R, 1T4R, 2T4R, or 1T4R/2T4R” antenna switching, otherwise the UE does not expect to be configured or triggered more than one SRS resource set with SRS-SetUse set as “antenna switching” in the same symbol.

Agreement

-------------------------------------------------------Start text proposal-----------------------------------------------------------

38.214 v15.0.0

6.2.1.3 UE sounding procedure between component carriers

…

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) on the carrier of the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in the [SRS_capability [13, TS 38.306].

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to ‘typeA’, and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers., where the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter SRS-SetUse set to ‘antenna switching’ and higher layer parameter resourceType in SRS-ResourceSet set to ‘aperiodic’.

…

--------------------------------------------------------End text proposal----------------------------------------------------------

For Type-B SRS switching across CCs, similar revisions to be made by the editor.

Agreement

Change in Section 6.2.1 of 38.214:

“-	if the UE is configured with the higher layer parameter SRS-SpatialRelationInfo set to 'SSB/PBCH', the UE shall transmit the SRS resource with the same spatial domain transmission filter used for the reception of the SSB/PBCH, if the higher layer parameter SRS-SpatialRelationInfo is set to 'CSI-RS', the UE shall transmit the SRS resource with the same spatial domain transmission filter used for the reception of the periodic CSI-RS or of the semi-persistent CSI-RS, or of the latest aperiodic CSI-RS. If the higher layer parameter SRS-SpatialRelationInfo is set to 'SRS', the UE shall transmit the SRS resource with the same spatial domain transmission filter used for the transmission of the periodic SRS or of the semi-persistent SRS or of the aperiodic SRS. “

Agreement

The following text proposal is agreed.

============= Text Proposal in 6.4.1.4.2 in 38.211 ======================

if SRS-GroupSequenceHopping equals ’groupHopping’, group hopping but not sequence hopping shall be used and

where the pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized with  at the beginning of each radio frame.

============= End of Text Proposals in 38.211 ======================

is replaced with .

Agreement

-------------------------------------------------------Start text proposal-----------------------------------------------------------

38.211 v15.0.0

6.4.1.4.3	Mapping to physical resources

If , frequency hopping is enabled and the frequency position indices  are defined by

where  is given by Table 6.4.1.4.3-1,

and where  regardless of the value of …

--------------------------------------------------------End text proposal----------------------------------------------------------

Agreement

For intra-slot SRS antenna switching, during the gap of Y symbol(s), the same priority rules for SRS and other uplink channels are applicable.

Agreement (RRC impact)

Up to maxNrofSRS-TriggerStates-1 SRS triggering states can be configured for one SRS resource set.

- As part of LS sent to RAN2

R1-1807827	[DRAFT] LS on aperiodic SRS	ZTE

Decision: Final LS is approved in R1-1807882

R1-1805834	Remaining issues on SRS	ZTE

R1-1805959	Remaining details of SRS design	Huawei, HiSilicon

R1-1806052	Remaining issues on SRS	vivo

R1-1806227	Remaining issues on SRS	Ericsson

R1-1806286	Remaining issues on SRS	CATT

R1-1806360	Discussion on remaining issues on SRS	CMCC

R1-1806513	Remaining issues on SRS	Intel Corporation

R1-1806564	Remaining issues on SRS	Sony

R1-1806613	Remaining issues on SRS	LG Electronics

R1-1806725	Remaining issues on SRS	Samsung

R1-1806791	Remaining issues in SRS	MediaTek Inc.

R1-1806828	Remaining issues on SRS design	OPPO

R1-1806945	Remaining issues on SRS	Sharp

R1-1807198	SRS resource configuration for antenna switching	Nokia, Nokia Shanghai Bell

R1-1807349	Maintenance for SRS	Qualcomm Incorporated

##### TRS

R1-1805808	LS on Removal of csi-RS-for-tracking from TCI-State	RAN2, ZTE

R1-1805838	[Draft] Reply LS on removal of CSI-RS-for-tracking from TCI-State	ZTE

Decision: Final LS is approved in R1-1807723.

R1-1807668	Summary #1 for TRS	MediaTek

Agreement

Aperiodic TRS is supported for FR1 as a UE optional feature capability with separate FR1/FR2 indication

  - There are no additional specification details introduced to the existing FR2 design

Note: MediaTek has concerns on the usefulness of this feature for FR1

R1-1807798	Summary #2 for TRS	MediaTek

Agreement

A UE is not expected to be configured with CSI-ReportConfig with the higher layer parameter reportQuantity set to other than ‘none’ for aperiodic CSI-RS resource set configured with TRS info

Agreement

  - CSI-RS resource set with TRS info configured with periodic CSI-RS resources has no linkage to any of the reporting settings

Agreement

Text Proposal for 5.1.6.1.1 38.214:

For the aperiodic CSI-RS resource set if triggered, and if the associated periodic CSI-RS resource set is configured with four periodic CSI-RS resources with two consecutive slots with two periodic CSI-RS resources in each slot, the higher layer parameter aperiodicTriggeringOffset indicates the triggering offset for the first slot.

Agreement

- Periodic CSI-RS resource in one set and aperiodic CSI-RS resources in a second set, with the aperiodic CSI-RS and periodic CSI-RS resource having the same bandwidth (with same RB location) and the aperiodic CSI-RS being ‘QCL-Type-A’ and ‘QCL-TypeD’, where applicable, with the periodic CSI-RS resources

- Note: Huawei has concerns on the UE complexity due to lack of restriction on subcarrier location but for the progress of NR completion, will accept the above compromise

R1-1805835	Remaining issues on TRS	ZTE

R1-1805962	Further discussion on introducing aperiodic TRS	Huawei, HiSilicon

R1-1806053	Remaining issues on TRS	vivo

R1-1806228	Remaining issues on TRS	Ericsson

R1-1806398	Remaining issues on TRS	Spreadtrum Communications

R1-1806514	Remaining Issues on TRS	Intel Corporation

R1-1806614	Text proposals on TRS	LG Electronics

R1-1806726	Remaining issues on TRS	Samsung

R1-1806806	Remaining issues on TRS	MediaTek Inc.

R1-1806843	Remaining Issues and Text Proposal for TRS	OPPO

R1-1807199	Remaining details of TRS design	Nokia, Nokia Shanghai Bell

Withdrawn

R1-1807350	Remaining issues on TRS	Qualcomm Incorporated

##### QCL

R1-1807664	Feature lead summary on QCL	Nokia, Nokia Shanghai Bell

Agreement

The following working assumption on DM-RS antenna ports is confirmed.

---- Unchanged parts omitted----

4.4.1 Antenna ports

An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed.

        - For DM-RS associated with PDSCH the channel over which a symbol on one of these antenna ports is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols are within the same scheduled PDSCH, same slot and in the same PRG as described in 3GPP TS 38.214 Section 5.1.2.3.

---- Unchanged parts omitted----

Agreement

The following text proposal is agreed

---- Unchanged parts omitted, section 7.2 of 38.211----

7.2	Physical resources

The frame structure and physical resources the UE shall assume when receiving downlink transmissions are defined in Clause 4.

The following antenna ports are defined for the downlink:

-	Antenna ports starting with 1000 for PDSCH

-	Antenna ports starting with 2000 for PDCCH

-	Antenna ports starting with 3000 for channel-state information reference signals

-	Antenna ports starting with 4000 for SS/PBCH block transmission

A UE shall not assume that two antenna ports are quasi co-located with respect to any QCL type unless specified otherwise.

The UE may assume that antenna ports within a CSI-RS resource are QCL with QCL Type A, Type D (when applicable) and average gain. The UE may assume that DMRS ports associated with a PDSCH are QCL with QCL Type A, Type D (when applicable) and average gain.

---- Unchanged parts omitted----

Agreement: The following text proposal is agreed.

---- Unchanged parts omitted, section 10.1 of 38.213----

The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the control resource set configured by pdcch-ConfigSIB1 in MasterInformationBlock and for corresponding PDSCH receptions, and the corresponding SS/PBCH block  are quasi co-located with respect to average gain and ‘QCL-typeA’ and ‘QCL-typeD’, , when applicable. The value for the DM-RS scrambling sequence initialization is the cell ID. A subcarrier spacing is provided by higher layer parameter subCarrierSpacingCommon in MasterInformationBlock.

---- Unchanged parts omitted ----

Agreement: The following text proposal is agreed.

---- Unchanged parts omitted ----

The UE receives an activation command [10, TS 38.321] used to map up to 8 TCI states to the codepoints of the DCI field ‘Transmission Configuration Indication’. When the HARQ-ACK corresponding to the PDSCH carrying the activation command is transmitted in slot n, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from slot n+3Nslotsubframe,µ. After a UE receives higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the SSB determined in the initial access procedure with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial Rx parameters, where applicable.

---- Unchanged parts omitted ----

Agreement:

The QCL signalling in red are additionally agreed.

| QCL linkage for above 6GHz after RRC | signalling |
| --- | --- |
| SSB TRS w.r.t average delay, Doppler shift, spatial RX parameters | QCL type: C + D |
| TRS CSI-RS for BM w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A + D |
| TRS CSI-RS for CSI w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| TRS DMRS for PDCCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A + D |
| TRS DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A + D |
| SSB CSI-RS for BM w.r.t. average delay, Doppler shift, spatial RX parameters | QCL type: C+D |
| SSB CSI-RS for CSI w.r.t, spatial RX parameters | QCL type: D |
| SSB DMRS for PDCCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters | QCL type: A+D |
| SSB DMRS for PDSCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters | QCL type: A+D |
| CSI-RS for BM DMRS for PDCCH w.r.t. spatial RX parameters | QCL type: D |
| CSI-RS for BM DMRS for PDSCH w.r.t., spatial RX parameters | QCL type: D |
| CSI-RS for CSI DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; Note: QCL parameters may not be derived directly from CSI-RS for CSI | QCL type: A+D |
| CSI-RS for BM CSI-RS for TRS/BM/CSI w.r.t. spatial RX parameters | QCL type: D |

Agreement

Note: For all the tables below, if a row has the same RS type, same RS ID should be assumed.

If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect the following two possible configurations of the higher layer parameter TCI-State:

| Valid TCI state Configuration | DL RS 1 | qcl-Type1 | DL RS 2 (if configured) | qcl-Type2 (if configured) |
| --- | --- | --- | --- | --- |
| 1* | SS/PBCH Block | QCL-TypeC | SS/PBCH Block | QCL-TypeD |
| 2* | SS/PBCH Block | QCL-TypeC | CSI-RS (BM) | QCL-TypeD |

* If QCL type-D is applicable, DL RS2 and QCL type-2 shall be configured for the UE

If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, the UE should only expect the following three possible configurations of the higher layer parameter TCI-State:

| Valid TCI state Configuration | DL RS 1 | qcl-Type1 | DL RS 2 (if configured) | qcl-Type2 (if configured) |
| --- | --- | --- | --- | --- |
| 1** | TRS | QCL-TypeA | SS/PBCH Block | QCL-TypeD |
| 2** | TRS | QCL-TypeA | CSI-RS (BM) | QCL-TypeD |
| 3* | TRS | QCL-TypeB |  |  |

* If QCL type-D is not applicable

** If QCL type-D is applicable, DL RS2 and QCL type-2 shall be configured for the UE

If a CSI-RS resource in a NZP-CSI-RS-ResourceSet is configured with higher layer parameter repetition, the UE should only expect the following three configurations of the higher layer parameter TCI-State:

| Valid TCI state Configuration | DL RS 1 | qcl-Type1 | DL RS 2 (if configured) | qcl-Type2 (if configured) |
| --- | --- | --- | --- | --- |
| 1 | TRS | QCL-TypeA | TRS | QCL-TypeD |
| 2 | TRS | QCL-TypeA | CSI-RS (BM) | QCL-TypeD |
| 3 | SS/PBCH Block | QCL-TypeC | SS/PBCH Block | QCL-TypeD |

For the next two tables, if QCL type-D is applicable, DL RS2 and QCL type-2 shall be configured for the UE except for the default case (fourth row in the two tables below). If TRS for downlink is used for QCL type-D, the TRS must have an SSB or CSI-RS for BM as source RS for QCL type-D.

For the DM-RS of PDCCH, the UE should only expect the following three configurations of the higher layer parameter TCI-State while the fourth configuration is valid as default, before TRS is configured:

| Valid TCI state Configuration | DL RS 1 | qcl-Type1 | DL RS 2 (if configured) | qcl-Type2 (if configured) |
| --- | --- | --- | --- | --- |
| 1 | TRS | QCL-TypeA | TRS | QCL-TypeD |
| 2 | TRS | QCL-TypeA | CSI-RS (BM) | QCL-TypeD |
| 3** | CSI-RS (CSI) | QCL-TypeA |  |  |
| 4* | SS/PBCH Block* | QCL-TypeA | SS/PBCH Block* | QCL-TypeD |

* Before TRS configured. Note: this is not a TCI state, rather a valid QCL assumption

**Note: Only when QCL type-D is not applicable

For the DM-RS of PDSCH, the UE should only expect the three following configurations of the higher layer parameter TCI-State while the fourth is valid by default, before TRS is configured:

| Valid TCI state Configuration | DL RS 1 | qcl-Type1 | DL RS 2 (if configured) | qcl-Type2 (if configured) |
| --- | --- | --- | --- | --- |
| 1 | TRS | QCL-TypeA | TRS | QCL-TypeD |
| 2 | TRS | QCL-TypeA | CSI-RS (BM) | QCL-TypeD |
| 3** | CSI-RS (CSI) | QCL-TypeA | CSI-RS (CSI) | QCL-TypeD |
| 4* | SS/PBCH Block* | QCL-TypeA | SS/PBCH Block* | QCL-TypeD |

* Before TRS configured. Note: this is not a TCI state, rather a valid QCL assumption

** Note: QCL parameters may not be derived directly from CSI-RS (CSI)

R1-1805836	Remaining issues on QCL	ZTE

R1-1805961	Remaining issues and TP  for QCL assumptions	Huawei, HiSilicon

R1-1806054	Remaining issues on QCL	vivo

R1-1806229	Remaining issues on QCL	Ericsson

R1-1806287	Remaining issues on QCL	CATT

R1-1806342	Remaining issues on QCL	Lenovo, Motorola Mobility

R1-1806515	On the remaining issues of QCL	Intel Corporation

R1-1806727	Remaining issues on QCL	Samsung

R1-1806844	Text Proposal for QCL	OPPO

R1-1807398	Remaining details on QCL	Qualcomm Incorporated	(rev of R1-1807351)

##### Other

R1-1806230	Further analysis of the PAPR issue of DMRS in Rel-15	Ericsson

R1-1806231	Benefits of resource specific DMRS mapping	Ericsson

R1-1806232	On correlation of DMRS Gold sequences	Ericsson

R1-1806233	On high PAPR in rank 2 transmissions with FDM of DMRS ports	Ericsson

R1-1806234	Type 1 mapping of pi/2-BPSK DFTS DM-RS	Ericsson

R1-1806235	Analysis of periodic vs aperiodic TRS	Ericsson

R1-1806236	On the benefits of a resource element specific sequence mapping for CSI-RS	Ericsson

R1-1806237	Evaluations of TRS periodicity	Ericsson

R1-1807134	Remaining issues for SRS switching among CCs	Huawei, HiSilicon

#### Other

R1-1806238	NR UE feature list for MIMO and RS	Ericsson

Withdrawn

### Scheduling/HARQ aspects

Including NR URLLC aspects. Note: contribution specific to URLLC aspects for June release can be a separate contribution from the one for NR CRs in the respective section.

#### Physical downlink control channel

##### PDCCH structure

R1-1807636	Offline summary for AI 7.1.3.1.1 PDCCH structure	NTT DOCOMO

Decision: The document is noted.

Agreement:

- To adopt the TP to 7.3.2.2 of 38.211

---- Start of TP ----

For both interleaved and non-interleaved mapping, the UE may assume

-	the same precoding being used within a REG bundle if the higher-layer parameter precoderGranularity equals sameAsREG-bundle

-	the same precoding being used across the all resource-element groups within the set of contiguous resource blocks in the CORESET if the higher-layer parameter precoderGranularity equals allContiguousRBs

---- End of TP ----

Agreement:

- To adop the TP to 7.4.1.3.2 of 38.211

---- Start of TP ----

-	subcarrier 0 of the lowest-numbered resource block in the CORESET if the CORESET is configured by the PBCH or SIB1,

---- End of TP ----

Agreement:

- To adopt the TP to 10.1 of 38.213

---- Start of TP ----

an indication for a presence or absence of a transmission configuration indication (TCI) field forDCI format 1_1 transmitted by a PDCCH in control resource set , by higher layer parameter TCI-PresentInDCI

---- End of TP ----

R1-1807637	Offline summary for AI 7.1.3.1.1 PDCCH structure for URLLC	NTT DOCOMO

R1-1805880	Remaining issues on PDCCH structure	Huawei, HiSilicon

R1-1806055	Remaining issues on PDCCH CORESET	vivo

R1-1806127	PDCCH structure remaining issues	ZTE

R1-1806141	Remaining details on PDCCH structure	Nokia, Nokia Shanghai Bell

R1-1806288	On PDCCH structure	CATT

R1-1806361	Discussion on remaining issues for unicast PDCCH in CORESET#0	CMCC

R1-1806362	Discussion on PDCCH repetition for URLLC	CMCC

R1-1806408	Remaining issues on PDCCH structure	Spreadtrum Communications

R1-1806615	Remaining issues on PDCCH structure	LG Electronics

R1-1806728	Corrections on PDCCH structure	Samsung

R1-1806778	Reamining issues on PDCCH structure	MediaTek Inc.

R1-1806915	Collision between SSB and PDCCH	Panasonic

R1-1807160	Remaining issues on CORESET#0	ITRI

R1-1807246	Remaining issues of PDCCH	Ericsson

R1-1807413	Remaining issues on PDCCH structure	Qualcomm Incorporated	(Revision of R1-1807352)

##### Search space

R1-1807638	Offline summary for AI 7.1.3.1.2 Search space	NTT DOCOMO

Decision: The document is noted.

Agreements:

Confirm the following working assumptions with updates:

- At least for Dec. Drop, Case 1-1 and Case 1-2, map all candidates of USS search-space-set with lower SS set ID before candidates of USS with higher ID

  - If all candidates in a SS set can’t be mapped, any candidates in the SS set and in any subsequent SS sets are dropped (not mapped)

- Case 2 FFS

Agreements:

- For following, regardless of whether one or more numerologies, the limit of BDs/CCEs per CC per slot is equal to the limit of BDs/CCEs for non-CA case.

  - Self-scheduling with up to 4 DL-CCs

  - Self-scheduling with up to T DL-CCs where the UE reports BD capability of y >= T

    - y is integer from (4, …, 16)

    - T is integer from (1, .., 16)

- For cross-carrier scheduling where the scheduling CC and all the CCs schedulable by the scheduling CC have the same numerology and the number of DL-CCs is up to 4 or with up to T DL-CCs where the UE reports BD capability of y >= T, the limit of BDs/CCEs of the scheduling CC per slot is (the number of CCs schedulable by the scheduling CC) x (the limit of BDs/CCEs for non-CA case)

  - Note: this is inline with the previous agreements at RAN1#92bis meeting

Propsoals:

- For cross-carrier scheduling where the scheduling CC and at least one the CC schedulable by the scheduling CC have different numerologies and the number of DL-CCs is up to 4 or with up to T DL-CCs where the UE reports BD capability of y >= T, the limit of BDs/CCEs of the scheduling CC per slot of the scheduling CC is:

  - Working assumption: (the number of CCs schedulable by the scheduling CC) x (the limit of BDs/CCEs for non-CA case with the numerology of the scheduling CC)

Tuesday session

R1-1807709	Offline summary for AI 7.1.3.1.2 Search space	NTT DOCOMO

Decision: The document is noted.

Agreements:

- For self-scheduling with same numerology, and the number of DL-CCs is more than 4 and with up to T DL-CCs where the UE reports BD capability of y < T, the limit of BDs/CCEs per CC per slot is

  - The total number of BDs/CCEs across CCs is based on UE BD capability. It can be split across CCs, subject to the non-CA limit on each CC.

    - For SCell, NW ensures no overbooking based on non-CA case occurs.

Proposals:

- For UE BD capability reporting y = integer(4, …, 16)

  - For UE not supporting CA with different numerologies, the reported value equally applies to all numerologies

  - For UE supporting CA with different numerologies,

    - When the UE is configured with CA with the same numerology, the reported value applies

    - When the UE is configured with CA with different numerologies, FFS

Agreement:

- TC-RNTI is not monitored on USS

Agreement:

- To adopt the following TP to 10.1 of 38.213:

---- Start of TP ----

If, for a search space set , a CCE aggregation level , and a control resource set , any CCE index for a second PDCCH candidate   overlaps with any CCE index for a first PDCCH candidate with index , where the PDCCH candidates are indexed in ascending order of a respective first CCE index , the UE is not expected to monitor the second PDCCH candidate with index

---- End of TP ----

Proposal:

- The UE is not expected to be configured with PDCCH monitoring occasion/periodicity causing partial time-domain overlapping for a given SS or across SSs within the same CORESET.

Conclusion:

- It is understood that configuration of cross-carrier scheduling for a CC is UE-specific, i.e., applicable to all USS sets containing 0-1/1-1 configured on the active DL BWP of the scheduling cell enabled with cross-carrier scheduling with CIF.

Wedenesday session

R1-1807732	Reply LS on PDCCH common search space configurations	RAN2, NTT DOCOMO

Decision: The document is noted. Reply LS is necessary.

R1-1807772	Offline summary for AI 7.1.3.1.2 Search space	NTT DOCOMO

Decision: The document is noted.

Agreements:

- For UE BD capability reporting y = integer(4, …, 16), at least for self-scheduling,

  - For UE not supporting CA with different numerologies, the reported value equally applies to all numerologies

  - (Working assumption) For UE supporting CA with different numerologies,

    - When the UE is configured with CA with the same numerology, the reported value applies

    - When the UE is configured with CA with different numerologies, the reported value applies to each set of DL-CCs with the same numerology, i.e.:

      - If a UE is configured with DL-CCs of X0, X1, X2, X3, where Xi denotes the number of DL-CCs with the numerology i, the total number of (BDs or CCEs) for the DL-CCs with the numerology i is given by Floor{Xi / (X0 + X1 + X2 + X3) * (Mi or Ni) * y} per slot of the numerology i

        - Where Mi and Ni represent the number of BDs and CCEs per slot specified for non-CA case, respectively

        - Note: some of the values of Xi may be zero depending on the CA configuration

    - For each CC, the non-CA limit still applies

  - FFS the impact of BWP, if any

Agreements:

- For a given SS set or across SS sets, the UE is not expected to be configured with PDCCH monitoring occasion/periodicity causing partial time-domain overlapping for the same CORESET.

  - Partial-overlapping in time-domain for the same CORESET means the case of that the separation between two monitoring occasions is smaller than the CORESET duration but greater than 0.

Agreements:

- PDCCH with CRC scrambled by C-RNTI and CS-RNTI are not monitored on Type3-CSS on SCell

- When a UE is configured with cross-carrier scheduling and CS-RNTI for a CC, the UE monitors DCI format(s) 0_1 and/or 1_1 with CIF with CRC scrambled by CS-RNTI

  - For CS-RNTI, the CIF field is used to indicate cross-carrier activation of SPS/configured grant Type2

Agreements: Update previous agreements as follows:

- NW and UE maintain the same understanding on SSB/CORESET#0/SS#0 in connected_mode at least for non-broadcast PDCCH and for broadcast PDCCH

  - Solutions FFS

- For the broadcast PDCCH, it is up to UE which common search space to monitor based on which SSB in both connected, in-active, and idle modes

- Unicast PDSCH can be scheduled by a DCI associated with the CORESET #0

Thursday (from offline)

R1-1807838	Offline summary on Thursday	NTT DOCOMO

Decision: The document is noted.

Agreements: To reply to LS in R1-1807732

- Regarding RAN2 agreements 1 & 2

  - Agree to the answer in red on slide 4 of R1-1807838, with one update

    - “contains the CORESET#0…”

    - “as long as the CORESET#0…”

- Regarding RAN2 agreements 3a

  - Answer: this is only for CSS configurations

- For other RAN2 agreements, no comments from RAN1

Friday (from offline)

Agreement:

- For self-scheduling with the same numerology or different numerologies, and the number of DL-CCs is <= 4 or with up to T DL-CCs where the UE reports BD capability of y >= T, network ensures the number of BDs/CCEs on any SCell does not exceed the non-CA limit.

Agreements:

- Clarify the agreements at RAN1#91 as follows:

  - Case 1: PDCCH monitoring of all SS sets that are monitored in a slot occurs within 3 consecutive OFDM symbols that have fixed positions in each slot periodicity of 14 or more symbols

    - Case 1-1: PDCCH monitoring on up to three OFDM symbols at the beginning of a slot

    - Case 1-2: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot

      - For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot

  - Case 2: PDCCH monitoring other than Case 1periodicity of less than 14 symbols

    - Note: this includes the PDCCH monitoring of up to three OFDM symbols at the beginning of a slot

Agreements:

- For self-scheduling with different numerologies, and the number of DL-CCs is more than 4 and with up to T DL-CCs where the UE reports BD capability of y < T, the limit of BDs/CCEs per CC per slot is

  - (Working assumption) The total number of BDs/CCEs across CCs per numerology is based on UE BD capability. It can be split across CCs for the given numerology, subject to the non-CA limit on each CC.

    - If a UE is configured with DL-CCs of X0, X1, X2, X3, where Xi denotes the number of DL-CCs with the numerology i, the limit of (BDs or CCEs) for the DL-CCs with the numerology i is given by Floor{Xi / (X0 + X1 + X2 + X3) * (Mi or Ni) * y} per slot of the numerology i

  - For SCell, NW ensures no overbooking based on non-CA case occurs

Agreement:

- For clarification on association b/w CORESET and A0/A1/A2,

  - For a CORESET with ControlResourceSetId, the corresponding Ap that is used in the PDCCH hashing is determined by p = ControlResourceSetId modulo 3.

R1-1807901	TP on TS38.213 Section 10.1	NTT DOCOMO

Agreement:

- The TP in R1-1807901 is endorsed

R1-1807639	Offline summary for AI 7.1.3.1.2 Search space for URLLC	NTT DOCOMO

R1-1805881	Remaining issues on search space	Huawei, HiSilicon

R1-1806019	Search Space Design for URLLC	Ericsson

R1-1806056	Remaining issues on Search Space	vivo

R1-1806128	Remaining issues on search space	ZTE

R1-1806142	Remaining details on search space	Nokia, Nokia Shanghai Bell

R1-1806289	Remaining details of NR PDCCH search space	CATT

R1-1806550	Remaining details on CORESET and search space	Intel Corporation

R1-1806616	Remaining issues on search space	LG Electronics

R1-1806655	On the PDCCH limits on the number of CCEs for channel estimation and BDs for URLLC	Nokia, Nokia Shanghai Bell

R1-1806666	Clarification on UE procedure for receiving OSI	ETRI

R1-1806729	Corrections on Search Space Design	Samsung

R1-1806779	Reamining issues on PDCCH search space	MediaTek Inc.

R1-1806879	Channel estimation complexity and search space	Panasonic Corporation

R1-1807016	On remaining aspects of search space 	InterDigital, Inc.

R1-1807058	Search space	NTT DOCOMO, INC.

R1-1807059	Search space for June drop URLLC	NTT DOCOMO, INC.

R1-1807238	Remaining details on search space	ASUSTEK COMPUTER (SHANGHAI)

R1-1807247	Remaining issues of search space	Ericsson

R1-1807353	Remaining issues on control resource set and search space	Qualcomm Incorporated

##### Group-common PDCCH

Revision of R1-1806777 in:

R1-1807674	[Draft] Reply LS on subcarrier spacing for BWPs and TDD configurations	MediaTek

Decision: The document is endorsed. Final LS is approved in R1-1807676.

From Tuesday'offline

Agreements:

- When a configured SFI monitoring is cancelled under the condition that the symbols are indicated to be UL or flexible by an earlier SFI

  - (Follow earlier agreement on configure DCI format 2_0 not detected) For the slots till the next configured DCI format 2_0 monitoring occasion with slot format not provided by earlier SFI, PDCCH monitoring is performed but  the UE will cancel RRC configured transmission, and assume RRC configured DL transmission is not transmitted, during semi-static configured flexible symbols

    - Spec change may not be needed

- If GC-PDCCH monitoring is skipped due to DRX (treat this the same as if the configured GC-PDCCH monitoring is not detected)

  - (Follow earlier agreement on configure DCI format 2_0 not detected) For the slots in the DRX ON cycle before the next configured DCI format 2_0 monitoring occasion, PDCCH monitoring is performed but  the UE will cancel RRC configured transmission, and assume RRC configured DL transmission is not transmitted, during semi-static configured flexible symbols

    - Spec change may not be needed

Agreement:

- If the SFI monitored in one cell is used to cancel an UE-specific RRC configured UL transmission in another BWP in another cell, the N2 for the cancellation timeline is with respect to the SCS the SFI is monitored with.

Agreements:

- Confirm the working assumption on slot format 55. (Already captured in spec)

- On slot format indication for ECP, reuse the slot format indication for NCP, and determine the direction of an ECP symbol using the indicated direction of (partially) overlapping NCP symbols

  - If one of the (partially) overlapping NCP symbol is D, and the other (partially) overlapping NCP symbol is U, the ECP symbol will be X

Thursday

R1-1807812	Summary of offline discussion on SFI	Qualcomm

Decision: The document is noted.

Agreement:

- When configure the UE-specific SFI table, each entry at least explicitly specifies the slot formats for the slots equals to the configured SFI monitoring periodicity.

Agreement:

- For Rel.15, on semi-static flexible symbol in a TDD cell, UE does not expect simultaneous RRC configurated DL reception and RRC configured UL transmission in the same symbol.

Conclusion:

- For the previous agreement “For a slot covered by multiple SFIs transmitted at different slots, the UE does not expect to receive different slot format indicated by different SFIs”, this include the case 255 is used for the slot format as well.

Agreement:

- For the previous agreement on “RRC configured DL reception and RRC configured UL transmission cancellation based on miss-detection of configured DCI format 2_0 monitoring” only applies for slots when their slot formats are not indicated by any other detected DCI format 2_0.

R1-1807813	TP for 38.213 for SFI	Qualcomm

Decision: The TPs are endorsed.

R1-1805883	Remaining issues on group-common PDCCH	Huawei, HiSilicon

R1-1806057	Remaining issues on SFI	vivo

R1-1806129	Remaining issues for slot format	ZTE

R1-1806290	On UE procedures for SFI reception	CATT

R1-1806363	Discussion on remaining issues for group-common PDCCH	CMCC

R1-1806404	Remaining issues on UE procedure for determining slot format	Spreadtrum Communications

R1-1807624	Remaining details on GC PDCCH and dynamic SFI	Intel Corporation	(R1-1806516)

R1-1806617	Remaining issues on group common PDCCH	LG Electronics

R1-1806656	Proposed corrections for slot configuration in 38.213	Nokia, Nokia Shanghai Bell

R1-1806730	Corrections on UE-Group Common PDCCH	Samsung

R1-1806780	Reamining issues on group-common PDCCH	MediaTek Inc.

R1-1806858	Remaining issues on GC-PDCCH	OPPO

R1-1806913	Remaining issues for SFI	ASUSTEK COMPUTER (SHANGHAI)

R1-1806946	Remaining issues on slot format for NR	Sharp

R1-1807060	Remaining issues on group-common PDCCH	NTT DOCOMO, INC.

R1-1807248	Remaining issues of GC-PDCCH	Ericsson

R1-1807354	Remaining issues for slot format indication	Qualcomm Incorporated

R1-1807645	Summary on proposals for offline discussion on GC-PDCCH for SFI	Qualcomm

##### DCI contents and formats

R1-1807702	Outcome of offline discussion on 7.1.3.1.4 (DCI content) - part I	Ericsson

Decision: The document is noted.

Wednesday

R1-1807738	Outcome of offline discussion on 7.1.3.1.4 (DCI content) - part II	Ericsson

Decision: The document is noted.

Agreements:

- For CS-RNTI DCI sizes

  - For DCI format 1-1 with CS-RNTI, the DCI size are the same as the DCI format 1-1 with C-RNTI in respective search space.

  - For DCI format 0-1 with CS-RNTI, the DCI size are the same as the DCI format 0-1 with C-RNTI in respective search space.

  - For DCI format 1-0 with CS-RNTI, the DCI size are the same as the DCI format 1-0 with C-RNTI in respective search space.

  - For DCI format 0-0 with CS-RNTI, the DCI size are the same as the DCI format 0-0 with C-RNTI in respective search space.

Proposal:

- No need for BWP index in PRACH order was found (but need to check with initial access session and/or RAN2)

Agreements:

- Confirm the working assumptions in the previous agreements as shown below:

  - For PDSCH scheduled by P-RNTI (working assumption), and RA-RNTI, scaling factor is applied in the calculation of Ninfo of TBS determination

    - (Working assumption) values of scaling factor: 1, ½, ¼

      - FFS the possibility of adding another value, e.g., 1/8 or 1/16

- DCI content for paging updated as shown in the table

| Field | Bits | Comment |
| --- | --- | --- |
| - Short Messages Indicator | - 12 | - Already captured in 38.212. Need to add interpretation. |
| In case of short message service |  |  |
| Short message | [8] | Defined by RAN2. The number of bits to be checked with RAN2. |
| Reserved bits |  | To meet overall DCI size |
| In case of paging message |  |  |
| - Frequency domain resource assignment |  |  |
| - Time domain resource assignment | 4 | Note: applicability of the currently agreed default table to paging or whether an additional default table or SIB1 signaled table is FFS. |
| VRB-to-PRB mapping | [1] | Depends on whether interleaved mapping is mandatory or not |
| Modulation and coding scheme | 5 | Same MCS table as for “normal” transmission without 256QAM |
| TB scaling | 2 | To scale TB size |
| Reserved bits | (fixed value) | To meet overall DCI size |

Agreement:

- The 1-bit indicator in DCI 2-x(=0,1,2,3) formats is removed

Proposals:

- If both UL and SUL is configured, zero-padding is used to ensure the same DCI size regardless of whether UL or SUL is scheduled by the DCI at least for the case when the DCI is in USS

  - Note: may already be covered by the spec

Proposals:

- If the size of DCI formats 0-0/1-0 is determined by the initial downlink BWP

  - Determine the size of the resource allocation field for format 0-0 from active uplink BWP

  - If overall DCI size is larger than what is defined by DCI format 1-0, truncate the resource allocation field (MSBs) to match the overall size of formats 0-0 and 1-0

Agreements:

When the DCI size for DCI formats 0-0/1-0 in USS is derived from the initial BWP but applied to another BWP, apply scaling according to

- Derive the resource allocation (start and length) from the RIV field assuming the initial BWP

- Scale start and length by a factor of K

- Apply the scaled start and length to the target BWP

K is the max of

- ratio between the size of the active BWP and the initial BWP, rounded down to the nearest power of 2

- 1

Thursday

R1-1807817	Outcome of offline discussion on 7.1.3.1.4 (DCI content) - part III	Ericsson	noted

Decision: The document is noted.

Agreements:

- Reuse DCI format 1-0 with C-RNTI and with resource allocation field equal to all ones for PDCCH order

  - Note: need to update the spec such that no HARQ ACK is reported for PDCCH order

Agreements:

- If the size of DCI format 0-0/1-0 is determined by the initial downlink BWP

  - Determine the size of the resource allocation field for format 0-0 from respective (UL or SUL) initial uplink BWP

  - If overall DCI size is larger than what is defined by DCI format 1-0, truncate the resource allocation field (MSBs) to match the overall size of formats 0-0 and 1-0

Agreement:

- For DCI format 1-1, the number of bits for the PDSCH-to-HARQ-timing-indicator field is given by ceil(log2(#values configured by RRC))

Agreements:

- If both PDSCH/PUSCH mapping type A and B are configured for a UE, the size of the antenna port field is determined by the maximum required for type A and B.

  - Zero-padding of MSBs is used if necessary to fill the antenna port field.

Agreements:

- Inform RAN2 to remove the comment in the RRC spec that the size for DCI format 2-1 must be a multiple of 14. RAN1 only uses multiples of 14 bits for preemption indication and discards the remaining bits (if any) in DCI format 2-1.

- LS in R1-1807841

Friday

R1-1807841	[Draft] LS on the size of DCI format 2-1	Ericsson

Decision: The document is endorsed and final LS is approved in R1-1807903.

R1-1805882	Remaining issues on DCI contents and format	Huawei, HiSilicon

R1-1805902	DCI design for URLLC	Huawei, HiSilicon

R1-1806020	DCI Contents and Formats for URLLC	Ericsson

R1-1806058	Remaining issues on eMBB DCI format	vivo

R1-1806059	Discussion on DCI format for URLLC	vivo

R1-1806130	Remaining issues on DCI content	ZTE

R1-1806291	Remaining issues on DCI contents	CATT

R1-1806292	On support of URLLC scheduling	CATT

R1-1806364	Discussion on remaining issues for DCI	CMCC

R1-1806365	Discussion on support of compact DCI for URLLC	CMCC

R1-1806399	Remaining issues on DCI contents and formats	Spreadtrum Communications

R1-1806517	Further considerations on DCI formats	Intel Corporation

R1-1806518	On DCI format for NR URLLC	Intel Corporation

R1-1806618	Remaining issues on DCI contents and formats	LG Electronics

R1-1806619	DCI contents and foramts for URLLC	LG Electronics

R1-1806657	On the remaining issues for DCI format sizes and contents	Nokia, Nokia Shanghai Bell

R1-1806731	DCI Contents and Formats	Samsung

R1-1806732	On Need for Additional DCI Format for URLLC	Samsung

R1-1806781	Reamining issues on DCI formats and contents	MediaTek Inc.

R1-1806809	Discussion on new DCI format for URLLC	MediaTek Inc.

R1-1806916	Remaining issues on DCI contents and formats	Panasonic

R1-1807233	Remaining issues on DCI contents and formats	WILUS Inc.

R1-1807239	Remaining details on DCI interpretation	ASUSTEK COMPUTER (SHANGHAI)

R1-1807249	Remaining issues of DCI	Ericsson

R1-1807355	Discussion on DCI related issues	Qualcomm Incorporated

R1-1807356	Considerations on a new DCI format for URLLC	Qualcomm Incorporated

R1-1807623	Summary of 7.1.3.1.4 (DCI content)	Ericsson

##### Other

R1-1805901	Summary of remaining issues on PDCCH monitoring with TP	Huawei, HiSilicon

R1-1806884	DCI size and interpretation for different BWP	Huawei, HiSilicon

R1-1807250	On Improvements to the Search Space Design	Ericsson

R1-1807305	Consideration on the maximum number of DCIs to be monitored	Huawei, HiSilicon

#### Physical uplink control channel

##### PUCCH structure in short-duration

R1-1807662	Summary of overlapped PUCCH resources	Ericsonn

Decision: The document is noted.

Agreements:

- The TP as listed in proposal 3.1 in R1-1807662 is adopted

  - Note: the part “m=0, 1, …” is the one replacing the deleted text for m

Wednesday session

R1-1807774	Wed offline on Overlapped PUCCH Resources	Ericsson

Decision: The document is noted.

Thursday session

R1-1807821	Proposals on remaining issues on overlapping PUCCHs	Ericsson

Decision: The document is noted. Further revised in R1-1807848.

Agreements:

- On a per PUCCH group basis:

  - If a UE is configured with overlapping PUCCH resources for CSI-only reporting in a slot,

    - If  a UE is configured with multi-CSI-PUCCH-Config, the UE multiplexes all the CSI reports corresponding to CSI only PUCCH resources (overlapping or not) in the slot on a single multi-CSI-PUCCH resource, following the agreed priority rules for CSI reporting.

    - If a UE is not configured with multi-CSI-PUCCH-Config, among all the PUCCH resources for CSI reporting, the UE selects maximum two non-overlapping PUCCH resources for CSI reports with the highest priority.

      - If two non-overlapping PUCCH resources are selected, they include at least one with PUCCH format 2.

  - For CSI-only reporting in a slot, if a UE is configured with more than two non-overlapping CSI reports in a slot, the UE selects the maximum two PUCCH resources with the highest priority CSI repots.

    - If two non-overlapping PUCCH resources are selected, they include at least one with PUCCH format 2.

Agreements:

- For step 1, while there are overlapping PUCCH resources in a slot,

  - The UE determines the PUCCH resource (resource A) that overlaps with at least another PUCCH resource with the priority order of earliest followed by largest duration (if applicable)

    - In case of multiple candidates for resource A, the UE can pick one (implementation-specific)

  - The UE determines a set of PUCCH resources (resource set X) overlapping with PUCCH resource A.

  - The UE determines a PUCCH resource and corresponding UCI for multiplexing the PUCCH resource A and PUCCH resources in set X in one shot.

  - The determined PUCCH resource and the corresponding UCI replace resource set X and resource A

- For step 1, when there is no more overlapping PUCCH resources in the slot, the UE determines “the timeline requirements for overlapping UL channels”. If the requirement is met, the UE proceeds to Step 2.

- Note: the above is per PUCCH group

- The above agreements are to replace Step 1 in the agreements under 7.1.3.2.3

Friday

R1-1807857	Proposals on remaining issues on overlapping PUCCHs	Ericsson

Decision: The document is noted. Further revised to R1-1807904.

Agreement:

- When a PUCCH resource carrying SR and other UCI overlaps with PUSCH, if the UCI on PUCCH is going to be multiplexed on PUSCH, the SR bit(s) is dropped from the UCI before multiplexing on PUSCH.

Agreement:

- When a UE is configured for one PUCCH resource set for UCI transmission including HARQ-ACK, the UE is not expected to be provided with simultanouesARQ-ACK-CSI=TRUE.

Agreements:

- If a HARQ-ACK transmission from a UE using PUCCH format 1 overlaps with SR transmission occasions corresponding to K SR PUCCH configurations each using PUCCH format 1 in a slot,

  - In case of negative SR, the HARQ-ACK is transmitted on the HARQ-ACK resource. In case of positive SR, the HARQ-ACK is transmitted on the SR resource corresponding to the positive SR.

Agreements:

- If a HARQ-ACK transmission from a UE using PUCCH format 0 overlaps with SR transmission occasions corresponding to K SR PUCCH configurations, in a slot,

  - If any SR is positive, indicate positive SR. Else indicate negative SR

R1-1807663	Summary of URLLC contributions	Ericsson

R1-1805884	Remaining issues on PUCCH structure in short-duration	Huawei, HiSilicon

R1-1806060	Remaining issues on short PUCCH for eMBB	vivo

R1-1806061	Discussion on short PUCCH for URLLC	vivo

R1-1806131	Remaining issues on short PUCCH	ZTE

R1-1806180	Discussion on PUCCH overlap issues	Panasonic Corporation

R1-1806293	On UCI multiplexing on PUCCH	CATT

R1-1806294	PUCCH enhancement for URLLC	CATT

R1-1806336	Remaining issues  on UCI multiplexing on PUCCH	Lenovo, Motorola Mobility

R1-1806400	Remaining issues on PUCCH structure in short-duration	Spreadtrum Communications

R1-1806519	Multiple UCI types in a PUCCH	Intel Corporation

R1-1806620	Remaining issues on short-duration PUCCH	LG Electronics

R1-1806733	Corrections on PUCCH for up to 2 Symbols	Samsung

R1-1806799	Remaining issues of short-PUCCH	MediaTek Inc.

R1-1806812	ACK/NACK feedback design and reliability for URLLC	MediaTek Inc.

R1-1806834	Remaining issues on UCI multiplexing	OPPO

R1-1806894	PUCCH design for URLLC	Huawei, HiSilicon

R1-1806925	On short PUCCH	Nokia, Nokia Shanghai Bell

R1-1807061	PUCCH structure in short-duration	NTT DOCOMO, INC.

R1-1807251	On UCI Multiplexing on PUCCH	Ericsson

R1-1807357	Remaining issues for short PUCCH	Qualcomm Incorporated

##### PUCCH structure in long-duration

R1-1807630	Review Summary for Al 7.1.3.2.2 PUCCH structure in long-duration	Huawei, HiSilicon

Decision: The document is noted.

Agreement:

- To add OACK & OSR in determining the number of CRC bits as part of Section 9.2.5.2 of 38.213

Proposals:

  - When HARQ-ACK/SR PUCCH resource determined by ARI and configured CSI PUCCH resources have the same starting symbols in a slot, if the UE is configured with only one PUCCH resource set and simultaneous transmission of HARQ-ACK/SR and CSI is enabled, the UE is not expected to transmit the corresponding HARQ-ACK/SR and the CSI report in a PUCCH resource in that slot (i.e., it’s an error case).

From Tuesday'offline session

Agreements:

- To adopt the following TP to 5.2.2.2 of 38.211:

  - Note: the TP is not to change the contents of the tables below, but to re-order the sequences

<Unchanged parts are omitted>

Table 5.2.2.2-2: Definition of  for.

| 0 | -3 | 1 | -3 | -3 | -3 | 3 | -3 | -1 | 1 | 1 | 1 | -3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -3 | 3 | 1 | -3 | 1 | 3 | -1 | -1 | 1 | 3 | 3 | 3 |
| 2 | -3 | 3 | 3 | 1 | -3 | 3 | -1 | 1 | 3 | -3 | 3 | -3 |
| 3 | -3 | -3 | -1 | 3 | 3 | 3 | -3 | 3 | -3 | 1 | -1 | -3 |
| 4 | -3 | -1 | -1 | 1 | 3 | 1 | 1 | -1 | 1 | -1 | -3 | 1 |
| 5 | -3 | -3 | 3 | 1 | -3 | -3 | -3 | -1 | 3 | -1 | 1 | 3 |
| 6 | 1 | -1 | 3 | -1 | -1 | -1 | -3 | -1 | 1 | 1 | 1 | -3 |
| 7 | -1 | -3 | 3 | -1 | -3 | -3 | -3 | -1 | 1 | -1 | 1 | -3 |
| 8 | -3 | -1 | 3 | 1 | -3 | -1 | -3 | 3 | 1 | 3 | 3 | 1 |
| 9 | -3 | -1 | -1 | -3 | -3 | -1 | -3 | 3 | 1 | 3 | -1 | -3 |
| 10 | -3 | 3 | -3 | 3 | 3 | -3 | -1 | -1 | 3 | 3 | 1 | -3 |
| 11 | -3 | -1 | -3 | -1 | -1 | -3 | 3 | 3 | -1 | -1 | 1 | -3 |
| 12 | -3 | -1 | 3 | -3 | -3 | -1 | -3 | 1 | -1 | -3 | 3 | 3 |
| 13 | -3 | 1 | -1 | -1 | 3 | 3 | -3 | -1 | -1 | -3 | -1 | -3 |
| 14 | 1 | 3 | -3 | 1 | 3 | 3 | 3 | 1 | -1 | 1 | -1 | 3 |
| 15 | -3 | 1 | 3 | -1 | -1 | -3 | -3 | -1 | -1 | 3 | 1 | -3 |
| 16 | -1 | -1 | -1 | -1 | 1 | -3 | -1 | 3 | 3 | -1 | -3 | 1 |
| 17 | -1 | 1 | 1 | -1 | 1 | 3 | 3 | -1 | -1 | -3 | 1 | -3 |
| 18 | -3 | 1 | 3 | 3 | -1 | -1 | -3 | 3 | 3 | -3 | 3 | -3 |
| 19 | -3 | -3 | 3 | -3 | -1 | 3 | 3 | 3 | -1 | -3 | 1 | -3 |
| 20 | 3 | 1 | 3 | 1 | 3 | -3 | -1 | 1 | 3 | 1 | -1 | -3 |
| 21 | -3 | 3 | 1 | 3 | -3 | 1 | 1 | 1 | 1 | 3 | -3 | 3 |
| 22 | -3 | 3 | 3 | 3 | -1 | -3 | -3 | -1 | -3 | 1 | 3 | -3 |
| 23 | 3 | -1 | -3 | 3 | -3 | -1 | 3 | 3 | 3 | -3 | -1 | -3 |
| 24 | -3 | -1 | 1 | -3 | 1 | 3 | 3 | 3 | -1 | -3 | 3 | 3 |
| 25 | -3 | 3 | 1 | -1 | 3 | 3 | -3 | 1 | -1 | 1 | -1 | 1 |
| 26 | -1 | 1 | 3 | -3 | 1 | -1 | 1 | -1 | -1 | -3 | 1 | -1 |
| 27 | -3 | -3 | 3 | 3 | 3 | -3 | -1 | 1 | -3 | 3 | 1 | -3 |
| 28 | 1 | -1 | 3 | 1 | 1 | -1 | -1 | -1 | 1 | 3 | -3 | 1 |
| 29 | -3 | 3 | -3 | 3 | -3 | -3 | 3 | -1 | -1 | 1 | 3 | -3 |

Table 5.2.2.2-3: Definition of  for

| 0 | -1 | 3 | -1 | -3 | 3 | 1 | -3 | -1 | 3 | -3 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | -3 | 3 | -1 | 1 | 3 | -3 | -1 | -3 | -3 | -1 | -3 | 3 | 1 | -1 | 3 | -3 | 3 |
| 2 | -3 | 3 | 1 | -1 | -1 | 3 | -3 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 3 | -1 | -3 | -1 |
| 3 | -3 | -3 | 3 | 3 | 3 | 1 | -3 | 1 | 3 | 3 | 1 | -3 | -3 | 3 | -1 | -3 | -1 | 1 |
| 4 | 1 | 1 | -1 | -1 | -3 | -1 | 1 | -3 | -3 | -3 | 1 | -3 | -1 | -1 | 1 | -1 | 3 | 1 |
| 5 | 3 | -3 | 1 | 1 | 3 | -1 | 1 | -1 | -1 | -3 | 1 | 1 | -1 | 3 | 3 | -3 | 3 | -1 |
| 6 | -3 | 3 | -1 | 1 | 3 | 1 | -3 | -1 | 1 | 1 | -3 | 1 | 3 | 3 | -1 | -3 | -3 | -3 |
| 7 | 1 | 1 | -3 | 3 | 3 | 1 | 3 | -3 | 3 | -1 | 1 | 1 | -1 | 1 | -3 | -3 | -1 | 3 |
| 8 | -3 | 1 | -3 | -3 | 1 | -3 | -3 | 3 | 1 | -3 | -1 | -3 | -3 | -3 | -1 | 1 | 1 | 3 |
| 9 | 3 | -1 | 3 | 1 | -3 | -3 | -1 | 1 | -3 | -3 | 3 | 3 | 3 | 1 | 3 | -3 | 3 | -3 |
| 10 | -3 | -3 | -3 | 1 | -3 | 3 | 1 | 1 | 3 | -3 | -3 | 1 | 3 | -1 | 3 | -3 | -3 | 3 |
| 11 | -3 | -3 | 3 | 3 | 3 | -1 | -1 | -3 | -1 | -1 | -1 | 3 | 1 | -3 | -3 | -1 | 3 | -1 |
| 12 | -3 | -1 | -3 | -3 | 1 | 1 | -1 | -3 | -1 | -3 | -1 | -1 | 3 | 3 | -1 | 3 | 1 | 3 |
| 13 | 1 | 1 | -3 | -3 | -3 | -3 | 1 | 3 | -3 | 3 | 3 | 1 | -3 | -1 | 3 | -1 | -3 | 1 |
| 14 | -3 | 3 | -1 | -3 | -1 | -3 | 1 | 1 | -3 | -3 | -1 | -1 | 3 | -3 | 1 | 3 | 1 | 1 |
| 15 | 3 | 1 | -3 | 1 | -3 | 3 | 3 | -1 | -3 | -3 | -1 | -3 | -3 | 3 | -3 | -1 | 1 | 3 |
| 16 | -3 | -1 | -3 | -1 | -3 | 1 | 3 | -3 | -1 | 3 | 3 | 3 | 1 | -1 | -3 | 3 | -1 | -3 |
| 17 | -3 | -1 | 3 | 3 | -1 | 3 | -1 | -3 | -1 | 1 | -1 | -3 | -1 | -1 | -1 | 3 | 3 | 1 |
| 18 | -3 | 1 | -3 | -1 | -1 | 3 | 1 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | 1 | 1 | -1 | -1 |
| 19 | 3 | 3 | 3 | -3 | -1 | -3 | -1 | 3 | -1 | 1 | -1 | -3 | 1 | -3 | -3 | -1 | 3 | 3 |
| 20 | -3 | 1 | 1 | -3 | 1 | 1 | 3 | -3 | -1 | -3 | -1 | 3 | -3 | 3 | -1 | -1 | -1 | -3 |
| 21 | 1 | -3 | -1 | -3 | 3 | 3 | -1 | -3 | 1 | -3 | -3 | -1 | -3 | -1 | 1 | 3 | 3 | 3 |
| 22 | -3 | -3 | 1 | -1 | -1 | 1 | 1 | -3 | -1 | 3 | 3 | 3 | 3 | -1 | 3 | 1 | 3 | 1 |
| 23 | 3 | -1 | -3 | 1 | -3 | -3 | -3 | 3 | 3 | -1 | 1 | -3 | -1 | 3 | 1 | 1 | 3 | 3 |
| 24 | 3 | -1 | -1 | 1 | -3 | -1 | -3 | -1 | -3 | -3 | -1 | -3 | 1 | 1 | 1 | -3 | -3 | 3 |
| 25 | -3 | -3 | 1 | -3 | 3 | 3 | 3 | -1 | 3 | 1 | 1 | -3 | -3 | -3 | 3 | -3 | -1 | -1 |
| 26 | -3 | -1 | -1 | -3 | 1 | -3 | 3 | -1 | -1 | -3 | 3 | 3 | -3 | -1 | 3 | -1 | -1 | -1 |
| 27 | -3 | -3 | 3 | 3 | -3 | 1 | 3 | -1 | -3 | 1 | -1 | -3 | 3 | -3 | -1 | -1 | -1 | 3 |
| 28 | -1 | -3 | 1 | -3 | -3 | -3 | 1 | 1 | 3 | 3 | -3 | 3 | 3 | -3 | -1 | 3 | -3 | 1 |
| 29 | -3 | 3 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | 3 | 3 | -3 | -1 | 1 | 3 | -1 | 3 | -1 |

Table 5.2.2.2-4: Definition of  for

| 0 | -1 | -3 | 3 | -1 | 3 | 1 | 3 | -1 | 1 | -3 | -1 | -3 | -1 | 1 | 3 | -3 | -1 | -3 | 3 | 3 | 3 | -3 | -3 | -3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -1 | -3 | 3 | 1 | 1 | -3 | 1 | -3 | -3 | 1 | -3 | -1 | -1 | 3 | -3 | 3 | 3 | 3 | -3 | 1 | 3 | 3 | -3 | -3 |
| 2 | -1 | -3 | -3 | 1 | -1 | -1 | -3 | 1 | 3 | -1 | -3 | -1 | -1 | -3 | 1 | 1 | 3 | 1 | -3 | -1 | -1 | 3 | -3 | -3 |
| 3 | 1 | -3 | 3 | -1 | -3 | -1 | 3 | 3 | 1 | -1 | 1 | 1 | 3 | -3 | -1 | -3 | -3 | -3 | -1 | 3 | -3 | -1 | -3 | -3 |
| 4 | -1 | 3 | -3 | -3 | -1 | 3 | -1 | -1 | 1 | 3 | 1 | 3 | -1 | -1 | -3 | 1 | 3 | 1 | -1 | -3 | 1 | -1 | -3 | -3 |
| 5 | -3 | -1 | 1 | -3 | -3 | 1 | 1 | -3 | 3 | -1 | -1 | -3 | 1 | 3 | 1 | -1 | -3 | -1 | -3 | 1 | -3 | -3 | -3 | -3 |
| 6 | -3 | 3 | 1 | 3 | -1 | 1 | -3 | 1 | -3 | 1 | -1 | -3 | -1 | -3 | -3 | -3 | -3 | -1 | -1 | -1 | 1 | 1 | -3 | -3 |
| 7 | -3 | 1 | 3 | -1 | 1 | -1 | 3 | -3 | 3 | -1 | -3 | -1 | -3 | 3 | -1 | -1 | -1 | -3 | -1 | -1 | -3 | 3 | 3 | -3 |
| 8 | -3 | 1 | -3 | 3 | -1 | -1 | -1 | -3 | 3 | 1 | -1 | -3 | -1 | 1 | 3 | -1 | 1 | -1 | 1 | -3 | -3 | -3 | -3 | -3 |
| 9 | 1 | 1 | -1 | -3 | -1 | 1 | 1 | -3 | 1 | -1 | 1 | -3 | 3 | -3 | -3 | 3 | -1 | -3 | 1 | 3 | -3 | 1 | -3 | -3 |
| 10 | -3 | -3 | -3 | -1 | 3 | -3 | 3 | 1 | 3 | 1 | -3 | -1 | -1 | -3 | 1 | 1 | 3 | 1 | -1 | -3 | 3 | 1 | 3 | -3 |
| 11 | -3 | 3 | -1 | 3 | 1 | -1 | -1 | -1 | 3 | 3 | 1 | 1 | 1 | 3 | 3 | 1 | -3 | -3 | -1 | 1 | -3 | 1 | 3 | -3 |
| 12 | 3 | -3 | 3 | -1 | -3 | 1 | 3 | 1 | -1 | -1 | -3 | -1 | 3 | -3 | 3 | -1 | -1 | 3 | 3 | -3 | -3 | 3 | -3 | -3 |
| 13 | -3 | 3 | -1 | 3 | -1 | 3 | 3 | 1 | 1 | -3 | 1 | 3 | -3 | 3 | -3 | -3 | -1 | 1 | 3 | -3 | -1 | -1 | -3 | -3 |
| 14 | -3 | 1 | -3 | -1 | -1 | 3 | 1 | 3 | -3 | 1 | -1 | 3 | 3 | -1 | -3 | 3 | -3 | -1 | -1 | -3 | -3 | -3 | 3 | -3 |
| 15 | -3 | -1 | -1 | -3 | 1 | -3 | -3 | -1 | -1 | 3 | -1 | 1 | -1 | 3 | 1 | -3 | -1 | 3 | 1 | 1 | -1 | -1 | -3 | -3 |
| 16 | -3 | -3 | 1 | -1 | 3 | 3 | -3 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 3 | -3 | 1 | -3 | 1 | -1 | -1 | -1 | -3 |
| 17 | 3 | -1 | 3 | -1 | 1 | -3 | 1 | 1 | -3 | -3 | 3 | -3 | -1 | -1 | -1 | -1 | -1 | -3 | -3 | -1 | 1 | 1 | -3 | -3 |
| 18 | -3 | 1 | -3 | 1 | -3 | -3 | 1 | -3 | 1 | -3 | -3 | -3 | -3 | -3 | 1 | -3 | -3 | 1 | 1 | -3 | 1 | 1 | -3 | -3 |
| 19 | -3 | -3 | 3 | 3 | 1 | -1 | -1 | -1 | 1 | -3 | -1 | 1 | -1 | 3 | -3 | -1 | -3 | -1 | -1 | 1 | -3 | 3 | -1 | -3 |
| 20 | -3 | -3 | -1 | -1 | -1 | -3 | 1 | -1 | -3 | -1 | 3 | -3 | 1 | -3 | 3 | -3 | 3 | 3 | 1 | -1 | -1 | 1 | -3 | -3 |
| 21 | 3 | -1 | 1 | -1 | 3 | -3 | 1 | 1 | 3 | -1 | -3 | 3 | 1 | -3 | 3 | -1 | -1 | -1 | -1 | 1 | -3 | -3 | -3 | -3 |
| 22 | -3 | 1 | -3 | 3 | -3 | 1 | -3 | 3 | 1 | -1 | -3 | -1 | -3 | -3 | -3 | -3 | 1 | 3 | -1 | 1 | 3 | 3 | 3 | -3 |
| 23 | -3 | -1 | 1 | -3 | -1 | -1 | 1 | 1 | 1 | 3 | 3 | -1 | 1 | -1 | 1 | -1 | -1 | -3 | -3 | -3 | 3 | 1 | -1 | -3 |
| 24 | -3 | 3 | -1 | -3 | -1 | -1 | -1 | 3 | -1 | -1 | 3 | -3 | -1 | 3 | -3 | 3 | -3 | -1 | 3 | 1 | 1 | -1 | -3 | -3 |
| 25 | -3 | 1 | -1 | -3 | -3 | -1 | 1 | -3 | -1 | -3 | 1 | 1 | -1 | 1 | 1 | 3 | 3 | 3 | -1 | 1 | -1 | 1 | -1 | -3 |
| 26 | -1 | 3 | -1 | -1 | 3 | 3 | -1 | -1 | -1 | 3 | -1 | -3 | 1 | 3 | 1 | 1 | -3 | -3 | -3 | -1 | -3 | -1 | -3 | -3 |
| 27 | 3 | -3 | -3 | -1 | 3 | 3 | -3 | -1 | 3 | 1 | 1 | 1 | 3 | -1 | 3 | -3 | -1 | 3 | -1 | 3 | 1 | -1 | -3 | -3 |
| 28 | -3 | 1 | -3 | 1 | -3 | 1 | 1 | 3 | 1 | -3 | -3 | -1 | 1 | 3 | -1 | -3 | 3 | 1 | -1 | -3 | -3 | -3 | -3 | -3 |
| 29 | 3 | -3 | -1 | 1 | 3 | -1 | -1 | -3 | -1 | 3 | -1 | -3 | -1 | -3 | 3 | -1 | 3 | 1 | 1 | -3 | 3 | -3 | -3 | -3 |

<Unchanged parts are omitted >

Agreement:

  - When single slot PUCCH overlaps with multi-slot PUSCH repetition in a slot, if the timeline requirement is met for the overlapped slot, multiplexing UCI on PUSCH in the overlapped slot; otherwise, it is considered as an error case for the overlapped slot.

Proposal:

  - At least for Dec drop, when multi-slot PUCCH repetition overlap with single/multi-slot PUSCH repetition in time, dropping PUSCH without deferral in overlapped slots if the timeline requirement is met; otherwise; it is considered as the error case for the overlapped slots

Proposal:

  - At least for Dec drop, multi-slot PUCCH repetition is only used for HARQ-ACK

Friday

R1-1807854	Outcome offline discussion on PUCCH structure in long-duration	Huawei, HiSilicon

Decision: The document is noted.

Agreement:

  - At least for Dec drop, when multi-slot PUCCH repetition overlap with single/multi-slot PUSCH repetition in time, dropping PUSCH without deferral in overlapping slots if the timeline requirement within overlapping slots is met; otherwise; it is considered as the error case for overlapping slots

R1-1807899	WF for OCC configuration of PF1	NTT DOCOMO, Qualcomm, OPPO, Panasonic, Nokia, NSB, Samsung, KDDI, Intel, MediaTek, vivo, InterDigital, Huawei, HiSilicon

Decision: The document is noted.

Agreements:

- In 6.3.2.4.1 of TS38.211,

  - No intra-slot frequency hopping is replaced as higher layer parameter PUCCH-frequency-hopping = “disabled”.

  - Intra-slot frequency hopping is replaced as higher layer parameter PUCCH-frequency-hopping = “enabled”.

  - Add the following:

    - The channel over which a symbol on the antenna port used for UL transmission is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols correspond to the same frequency hop, when frequency hop is enabled, irrespective of whether the frequency hop distance is zero or not.

R1-1805885	Remaining issues on PUCCH structure in long-duration	Huawei, HiSilicon

R1-1806062	Remaining issues on long PUCCH for eMBB	vivo

R1-1806132	Issues related to Long PUCCH	ZTE

R1-1806295	Remaining issues for long PUCCH	CATT

R1-1806520	PUSCH-PUCCH and PUCCH-PUCCH collision handling	Intel Corporation

R1-1806621	Remaining issues on long-duration PUCCH	LG Electronics

R1-1806667	Remaining issues for PUCCH structure in long-duration	ETRI

R1-1806734	Corrections on PUCCH for more than 2 Symbols	Samsung

R1-1806800	Remaining issues of long-PUCCH	MediaTek Inc.

R1-1806835	Remaining issues on CSI reporting using PUCCH	OPPO

R1-1806926	Remaining open items on long PUCCH	Nokia, Nokia Shanghai Bell

R1-1806947	Remaining issues on PUCCH in long duration	Sharp

R1-1807062	PUCCH structure in long-duration	NTT DOCOMO, INC.

R1-1807234	Remaining issues on long PUCCH over multiple slots	WILUS Inc.

R1-1807252	On PUCCH Remaining Issues	Ericsson

R1-1807358	Remaining issues for long PUCCH	Qualcomm Incorporated

R1-1807809	Assumptions on the number of CRC bits for PUCCH resource determination	Huawei, HiSilicon, Sharp

##### UCI multiplexing

Including handling URLLC related aspects for overlapped UL transmissions

R1-1807677	Summary of remaining issues for overlapping UL transmissions	Qualcomm

Decision: The document is noted.

Agreements:

For HARQ-ACK, CSI part 1, and CSI part 2 (if exists) transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK, CSI part 1, and CSI part 2 (exists), are determined as follows:

If CSI part 2 exists,

- SE is the spectrum efficiency which is code rate * modulation order

- Note: if HARQ-ACK is up to 2 bits,  in the equation for  calculation is the amount of reserved REs assuming 2 bits HARQ-ACK

Agreement:

- For omission of lower priority information bits for CSI part 2 on PUSCH without UL-SCH, the threshold coding rate is given by where R is the signaled code rate in DCI.

Agreements:

- Within a group of overlapping PUCCH/PUSCH channels satisfying the timeline requirements, adopt the following procedure

  - Step 1: determine a set of non-overlapping (in time)PUCCH resource(s) for UCI multiplexing by considering only the PUCCH resources in the group (irrespective whether or not PUSCH(s) exists), where each PUCCH resource satisfies the timeline requirements

  - Step 2:

    - if the resulting PUCCH resource(s) in step 1 overlapping with PUSCH(s), multiplex UCIs on the overlapping PUSCH(s);

      - FFS: When UCI includes SR

    - Otherwise, multiplex UCI on the determined PUCCH resource(s)

Proposal

- When one PUCCH overlap with multiple PUSCHs which satisfy timeline requirement, the UE multiplex UCIs on earliest PUSCH, using existing UCI multiplexing rule. (supporting companies: ZTE, PANASONIC, CATR, Qualcomm)

  - Also supported by Ericsson, LG Electronics

Revised proposal:

- When one PUCCH overlaps with multiple PUSCHs (across CCs) satisfying the timeline requirement, the UE multiplexes UCI(s) on the earliest (in time) starting PUSCH, using the existing UCI multiplexing rule

  - If there are two or more PUSCHs with the same starting time, the PUSCH with the lowest cell ID is picked

From Tuesday'offline

Agreement:

- DCI format 0_1 includes an ‘UL-SCH indicator’ field of 1 bit. Modulation and code rate are determined by I_MCS field in DCI.

Wednesday session

R1-1807773	Summary of remaining issues for overlapping UL transmissions	Qualcomm

Decision: The document is noted.

Agreements: In intra-band CA, if PRACH transmission and PUSCH/PUCCH/SRS transmission collide.

- how to handle collision is up to UE implementation except when PRACH and SRS collides, PRACH take precedence.

- Note: The definition of collision follows the PRACH procedure agreements in RAN1 92bis.

Agreements:

- In a PUCCH group, within one set of colliding PUCCHs/PUSCHs channels, when the detemined PUCCH resource overlaps with multiple PUSCHs which satisfy timeline requirements for overlapped UL channels,

  - UE first determines CC index for UCI multiplexing based on procedure defined in 38.213 V15.1.0

  - If there are multiple PUSCHs on the determined CC, UE multiplex PUCCH on the PUSCH with earliest starting symbol on that CC.

Thursday session

R1-1807820	Summary of remaining issues for overlapping UL transmissions	Qualcomm

Decision: The document is noted.

Agreements:

- The UE is not expected to be configured to transmit on the same OFDM symbol with an SRS resource and a PUCCH/PUSCH across different CCs in intra-band CA

  - Note: no spec change is needed.

- Parallel SRS and PUCCH/PUSCH transmissions across CCs is supported in inter-band CA.

  - Note: if case parallel SRS and PUCCH/PUSCH is supported, the SRS resource does not belong to a set which is for antenna switching, if the SRS resource set for antenna switching has more than one SRS resource (T < R)

  - Supporting of this feature is subject to UE capability which is a separate capability

Agreements:

- Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs is supported in inter-band CA

  - Supporting of this feature is subject to UE capability which is a separate capability

Friday

Agreements:

- Adopt the following TP in 38.212 Section 7.3.1.1.2

---- Start of TP ----

-	1st downlink assignment index – 1 or 2 bits

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook with single HARQ-ACK codebook.

---- End of TP ----

Working assumption:

- In the WA of timeline requirements on overlapping UL channels made in RAN1 92bis, ,  where T = 1

  - If  is not applicable,

  - If  is not applicable,

  - If  is not applicable,

- Note: T is based on the  as specified in 38.214.

- Note: the definition ofs , , and  follows the same definitions in TS 38.214.

R1-1805887	Remaining issues on UCI multiplexing	Huawei, HiSilicon

R1-1806005	Intra-UE Pre-emption in Uplink	Ericsson

R1-1806063	Remaining issues on UCI multiplexing for eMBB	vivo

R1-1806064	Discussion on eMBB and URLLC UCI multiplexing	vivo

R1-1806121	UCI piggyback on PUSCH with URLLC data	Fujitsu

R1-1806133	Further considerations on remaining UCI multiplexing issues	ZTE

R1-1806181	Discussion on UCI multiplexing	Panasonic Corporation

R1-1806182	Discussion on URLLC UL multiplexing	Panasonic Corporation

R1-1806296	On UCI multiplexing on PUSCH	CATT

R1-1806297	Considerations on UCI multiplexing for NR URLLC	CATT

R1-1807604	Remaining issues  on UCI multiplexing on PUSCH	Lenovo, Motorola Mobility	(R1-1806337)

R1-1806401	Remaining issues on UCI multiplexing	Spreadtrum Communications

R1-1806521	UCI on PUSCH	Intel Corporation

R1-1806622	Discussion on multiplexing UL transmission with different requirements	LG Electronics

R1-1806623	Remaining issues on UCI multiplexing	LG Electronics

R1-1806735	Corrections on UCI Multiplexing in PUSCH	Samsung

R1-1806836	Discussion on PUCCH partial overlap	OPPO

R1-1806837	Discussion on overlapped UL transmissions with URLLC	OPPO

R1-1806893	UL multiplexing for URLLC	Huawei, HiSilicon

R1-1806927	On UCI multiplexing	Nokia, Nokia Shanghai Bell

R1-1806928	URLLC aspects for overlapped UL transmissions	Nokia, Nokia Shanghai Bell

R1-1806948	Remaining issues on UCI multiplexing	Sharp

R1-1806963	URLLC UCI prioritization	InterDigital, Inc.

R1-1807063	Maintenance of UCI multiplexing on PUSCH	NTT DOCOMO, INC.

R1-1807064	UCI multiplexing for June drop URLLC	NTT DOCOMO, INC.

R1-1807168	On Remaining issue of UCI multiplexing 	Xiaomi Communications

R1-1807202	Remaining issues on UCI multiplexing on PUSCH	CATR

R1-1807235	Intra-UE UL multiplexing considering URLLC data in PUSCH	WILUS Inc.

R1-1807253	On UCI Multiplexing on PUSCH	Ericsson

R1-1807359	Remaining issues for overlapping UL transmissions	Qualcomm Incorporated

R1-1807360	Handling UCI on PUSCH for URLLC	Qualcomm Incorporated

##### Resource allocation for PUCCH

R1-1807600	Summary of RAN1#93 Tdocs on PUCCH resource allocation	OPPO

Decision: The document is noted.

Proposals:

- The first TP in Section 3 of R1-1807600 for Section 9.2.3 of 38.213 is adopted with the following update:

  - Also removing the first part of equation on the RHS when PRI < RPUCCH mod 8

Discuss further offline by removing the table but adding a sentence on mapping RRC configuration with the resource indexing

Tuesday session

R1-1807697	Summary of offline discussion on PUCCH resource allocation	OPPO

Decision: The document is noted.

Agreements: Confirm working assumption as:

- For PUCCH resource allocation before a UE has a dedicated PUCCH configuration,

  - One RMSI value is used for indicating cell-specific PRB offset = Floor(NBWP/4) for PUCCH duration = 14 symbols.

    - NBWP is the size of initial UL BWP

  - For 2 symbols, 4 symbols and 10 symbols,

    - Floor(NBWP/4) is not specified.

  - At least one additional offset is specified in number of PRBs for each duration.

    - Note: Offset = 0 has been agreed.

Wednesday session

R1-1807751	Summary of offline discussion on PUCCH resource allocation	OPPO

Decision: The document is noted.

Agreements:

- For PUCCH resource allocation before a UE has a dedicated PUCCH configuration, adopt the following table for specifying cell-specific PUCCH resources/parameters:

  - NBWP is the size of initial UL BWP

| RMSI value | PUCCH format | Starting symbol | Number of symbols | Cell-specific PRB offset | Initial CS index set |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 4 | 10 | 0 | {0, 6} |
| 7 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 8 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 9 | 1 | 0 | 14 | 0 | {0, 6} |
| 10 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 12 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | Floor(NBWP/4) | {0, 3, 6, 9} |
| 14 | FFS |  |  |  |  |
| 15 | FFS |  |  |  |  |

Friday

Agreements:

For PUCCH resource allocation before a UE has a dedicated PUCCH configuration, the PUCCH resource within the is determined as belows.

- PRB offset = cell-specific PRB offset + UE-specific PRB offset

- It is up to the editor to adopt one option among the two below, where

    - nCS is the number of CS indices in the CS index set.

    - m is the PUCCH resource indicator field value,

    - CCEstart is the starting CCE index of the DCI received

    - NCCE,pNCCE,p is the number of CCEs in control resource set p.

  - Option 1: Equation-based expression.

    - PUCCH resource index rPUCCH = 2m+Floor(2*CCEstart/NCCE,p)

    - Hopping direction h = Floor(rPUCCH/8)

    - UE-specific PRB offset p = Floor((rPUCCH-8h)/nCS)

    - CS iIndex in the initial CS index set c = Mod (rPUCCH-8h, nCS)

  - Option 2: Table-based expression.

    - CCE-based implicit bit = Floor(2*CCEstart/NCCE,p)

    - For RMSI value with initial CS index set {0, 3},

| ARI bits | CCE-based implicit bit | Hopping direction | UE specific PRB offset | Initial CS index |
| --- | --- | --- | --- | --- |
| 000 | 0 | 0 | 0 | 0 |
|  | 1 |  |  | 3 |
| 001 | 0 |  | 1 | 0 |
|  | 1 |  |  | 3 |
| 010 | 0 |  | 2 | 0 |
|  | 1 |  |  | 3 |
| 011 | 0 |  | 3 | 0 |
|  | 1 |  |  | 3 |
| 100 | 0 | 1 | 0 | 0 |
|  | 1 |  |  | 3 |
| 101 | 0 |  | 1 | 0 |
|  | 1 |  |  | 3 |
| 110 | 0 |  | 2 | 0 |
|  | 1 |  |  | 3 |
| 111 | 0 |  | 3 | 0 |
|  | 1 |  |  | 3 |

    - For RMSI values with initial CS index set {0, 4, 8},

| ARI | CCE-based implicit bit | Hopping direction | UE-specific PRB offset | Initial CS index |
| --- | --- | --- | --- | --- |
| 000 | 0 | 0 | 0 | 0 |
|  | 1 |  |  | 4 |
| 001 | 0 |  |  | 8 |
|  | 1 |  | 1 | 0 |
| 010 | 0 |  |  | 4 |
|  | 1 |  |  | 8 |
| 011 | 0 |  | 2 | 0 |
|  | 1 |  |  | 4 |
| 100 | 0 | 1 | 0 | 0 |
|  | 1 |  |  | 4 |
| 101 | 0 |  |  | 8 |
|  | 1 |  | 1 | 0 |
| 110 | 0 |  |  | 4 |
|  | 1 |  |  | 8 |
| 111 | 0 |  | 2 | 0 |
|  | 1 |  |  | 4 |

    - For RMSI values with initial CS index set {0, 6},

| ARI | CCE-based implicit bit | Hopping direction | UE specific PRB offset | Initial CS index |
| --- | --- | --- | --- | --- |
| 000 | 0 | 0 | 0 | 0 |
|  | 1 |  |  | 6 |
| 001 | 0 |  | 1 | 0 |
|  | 1 |  |  | 6 |
| 010 | 0 |  | 2 | 0 |
|  | 1 |  |  | 6 |
| 011 | 0 |  | 3 | 0 |
|  | 1 |  |  | 6 |
| 100 | 0 | 1 | 0 | 0 |
|  | 1 |  |  | 6 |
| 101 | 0 |  | 1 | 0 |
|  | 1 |  |  | 6 |
| 110 | 0 |  | 2 | 0 |
|  | 1 |  |  | 6 |
| 111 | 0 |  | 3 | 0 |
|  | 1 |  |  | 6 |

    - For RMSI values with initial CS index set {0, 3, 6, 9},

| ARI | CCE-based implicit bit | Hopping direction | UE specific PRB offset | Initial CS index |
| --- | --- | --- | --- | --- |
| 000 | 0 | 0 | 0 | 0 |
|  | 1 |  |  | 3 |
| 001 | 0 |  |  | 6 |
|  | 1 |  |  | 9 |
| 010 | 0 |  | 1 | 0 |
|  | 1 |  |  | 3 |
| 011 | 0 |  |  | 6 |
|  | 1 |  |  | 9 |
| 100 | 0 | 1 | 0 | 0 |
|  | 1 |  |  | 3 |
| 101 | 0 |  |  | 6 |
|  | 1 |  |  | 9 |
| 110 | 0 |  | 1 | 0 |
|  | 1 |  |  | 3 |
| 111 | 0 |  |  | 6 |
|  | 1 |  |  | 9 |

R1-1807863	Summary of offline discussion on PUCCH resource allocation	OPPO

Decision: The document is noted.

Agreements:

For PUCCH resource allocation before a UE has a dedicated PUCCH configuration, adopt the following table for specifying cell-specific PUCCH resources/parameters:

- NBWP is the size of initial UL BWP

| RMSI value | PUCCH format | Starting symbol | Number of symbols | Cell-specific PRB offset | Initial CS index set |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 | Floor(NBWP/4) | {0, 3, 6, 9} |

R1-1805886	Remaining issues on resource allocation for PUCCH	Huawei, HiSilicon

R1-1806065	Remaining issues on PUCCH resource allocation for eMBB	vivo

R1-1806066	Discussion on PUCCH resource allocation for URLLC	vivo

Withdrawn

R1-1806183	Discussion on remaining issues on PUCCH resource allocation	Panasonic Corporation

R1-1806298	On PUCCH resource allocation	CATT

R1-1806366	Discussion on PUCCH repetition	CMCC

R1-1806402	Remaining issues on resource allocation for PUCCH	Spreadtrum Communications

R1-1806468	Remaining issues of PUCCH RA prior to RRC configuration	NEC

R1-1806624	Remaining issues on PUCCH resource allocation	LG Electronics

R1-1806736	Corrections on Resource Allocation for PUCCH	Samsung

R1-1806866	Remaining issues on resource allocation for HARQ-ACK before RRC connection setup	OPPO

R1-1806899	PUCCH resource allocation for URLLC	Huawei, HiSilicon

R1-1806929	Remaining open items on PUCCH resource allocation	Nokia, Nokia Shanghai Bell

R1-1807065	Resource allocation for PUCCH	NTT DOCOMO, INC.

R1-1807254	On PUCCH Resource Allocation	Ericsson

R1-1807361	Remaining issues for resource allocation for PUCCH	Qualcomm Incorporated

R1-1807362	Considerations for URLLC resource allocation for PUCCH	Qualcomm Incorporated

##### Other

R1-1805899	Intra-UE UL multiplexing between eMBB and URLLC	Huawei, HiSilicon

R1-1806882	Discussion of partial overlap between PUCCH and PUCCH/PUSCH	Huawei, HiSilicon

R1-1806891	TP on UCI multiplexing between UL and SUL	Huawei, HiSilicon

R1-1806895	Discussion on partially overlapped PUCCH and PUSCH for URLLC	Huawei, HiSilicon

R1-1807307	On CGS ordering for NR	Huawei, HiSilicon

#### DL/UL data scheduling and HARQ procedure

##### DL/UL resource allocation

Including handling the leftovers for the LS as in R1-1801324

R1-1807675	Outcome from offline discussion on 7.1.3.3.1 (resource allocation)	Ericsson

Decision: The document is noted.

Agreements:

- Update the default tables for time-domain allocation as shown below.

  - Note: not all the entries are valid for RMSI scheduling

    - For pattern 2, rows 1-7, 9, 10, 11, and 15 are valid for RMSI scheduling

    - For pattern 3, at least rows 2-5 are valid for RMSI scheduling

Table 5.1.2.1.1-3: Default PDSCH time domain resource allocation B

| Row index | dmrs-TypeA-Position | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 2,3 | Type B | 0 | 2 | 2 |
| 2 | 2,3 | Type B | 0 | 4 | 2 |
| 3 | 2,3 | Type B | 0 | 6 | 2 |
| 4 | 2,3 | Type B | 0 | 8 | 2 |
| 5 | 2,3 | Type B | 0 | 10 | 2 |
| 6 | 2,3 | Type B | 1 | 2 | 2 |
| 7 | 2,3 | Type B | 1 | 4 | 2 |
| 8 | 2,3 | Type B | 0 | 2 | 4 |
| 9 | 2,3 | Type B | 0 | 4 | 4 |
| 10 | 2,3 | Type B | 0 | 6 | 4 |
| 11 | 2,3 | Type B | 0 | 8 | 4 |
| 12 | 2,3 | Type B | 0 | 10 | 4 |
| 13 | 2,3 | Type B | 0 | 2 | 7 |
| 14 | 2 | Type A | 0 | 2 | 12 |
|  | 3 | Type A | 0 | 3 | 11 |
| 15 | 2,3 | Type B | 1 | 2 | 4 |

Table 5.1.2.1.1-4: Default PDSCH time domain resource allocation C

| Row index | dmrs-TypeA-Position | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 2,3 | Type B | 0 | 2 | 2 |
| 12 | 2,3 | Type B | 0 | 4 | 2 |
| 23 | 2,3 | Type B | 0 | 6 | 2 |
| 34 | 2,3 | Type B | 0 | 8 | 2 |
| 45 | 2,3 | Type B | 0 | 10 | 2 |
| 6 | Reserved |  |  |  |  |
| 7 | Reserved |  |  |  |  |
| 8 | 2,3 | Type B | 0 | 2 | 4 |
| 9 | 2,3 | Type B | 0 | 4 | 4 |
| 10 | 2,3 | Type B | 0 | 6 | 4 |
| 11 | 2,3 | Type B | 0 | 8 | 4 |
| 12 | 2,3 | Type B | 0 | 10 | 4 |
| 13 | 2,3 | Type B | 0 | 2 | 7 |
| 14 | 2 | Type A | 0 | 2 | 12 |
|  | 3 | Type A | 0 | 3 | 11 |
| 15 | 2 | Type A | 0 | 0 | 6 |
| 16 | 3 | Type A | 0 | 2 | 6 |

Agreements:

- Adopt the following default tables for ECP:

  - For PUSCH time-domain RA table for ECP, j=2 follows the previous agreements on 60kHz, Delta = 4.

Default PDSCH time-domain RA table for ECP

(note: this case is not applicable to RMSI since ECP for RMSI is not supported)

| Row index | DL-DMRS-typeA-pos | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | Type A | 0 | 2 | 6 |
|  | 3 | Type A | 0 | 3 | 5 |
| 2 | 2 | Type A | 0 | 2 | 10 |
|  | 3 | Type A | 0 | 3 | 9 |
| 3 | 2 | Type A | 0 | 2 | 9 |
|  | 3 | Type A | 0 | 3 | 8 |
| 4 | 2 | Type A | 0 | 2 | 7 |
|  | 3 | Type A | 0 | 3 | 6 |
| 5 | 2 | Type A | 0 | 2 | 5 |
|  | 3 | Type A | 0 | 3 | 4 |
| 6 | 2 | Type B | 0 | 6 | 4 |
|  | 3 | Type B | 0 | 8 | 2 |
| 7 | 2 | Type B | 0 | 4 | 4 |
|  | 3 | Type B | 0 | 6 | 4 |
| 8 | 2,3 | Type B | 0 | 5 | 6 |
| 9 | 2,3 | Type B | 0 | 5 | 2 |
| 10 | 2,3 | Type B | 0 | 9 | 2 |
| 11 | 2,3 | Type B | 0 | 10 | 2 |
| 12 | 2,3 | Type A | 0 | 1 | 11 |
| 13 | 2,3 | Type A | 0 | 1 | 6 |
| 14 | 2,3 | Type A | 0 | 2 | 4 |
| 15 | 2,3 | Type B | 0 | 4 | 6 |
| 16 | 2,3 | Type B | 0 | 8 | 4 |

Default PUSCH time domain RA table with ECP

| Row index | PUSCH mapping type | K2' | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 8 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 4 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 8 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type B | j+2 | 0 | 6 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 4 |
| 15 | Type A | j+3 | 0 | 8 |
| 16 | Type A | j+3 | 0 | 10 |

Agreements:

- For slot aggregation,

  - the RV field in the DCI scheduling the transmission indicates the RV of the first slot of the PDSCH/PUSCH transmission,

  - slots cycle through the RVs in order 0, 2, 3, 1 starting from the RV indicated in the DCI

Tuesday session

R1-1807712	Outcome of offline discussion on 7.1.3.3.1 (resource allocation) – part II	Ericsson

Decision: The document is noted.

Proposals:

For C-RNTI and CS-RNTI in CSS at least in Pcell (FFS Scell Type 3 CSS)

- If PDSCH-Config has provided a table

  - Use this table for PDSCH transmission scheduled by DCI in CSS

- else if PDSCH-ConfigCommon has provided a table

  - Use this table for PDSCH transmission scheduled by DCI in CSS

- else

  - Use the default PDSCH table A (mux pattern 1)

- Note: Same table is used as in USS

- Note: If some entries in the table before RRC reconfiguration and in the table after RRC reconfiguration are the same, the UE can be schedulable during the RRC reconfiguration period

  - No specification impact

For C-RNTI and CS-RNTI in CSS:

- if PUSCH-Config has provided a table

  - Use this table for PUSCH transmission scheduled by DCI in CSS

- else if PUSCH-ConfigCommon has provided a table for PUSCH

  - Use this table for PUSCH transmission scheduled by DCI in CSS

- else

  - Use the default PUSCH table

- Note: Same table is used as in USS

- Note: If some entries in the table before RRC reconfiguration and in the table after RRC reconfiguration are the same, the UE can be schedulable during the RRC reconfiguration period

  - No specification impact

Agreements: For RAR and msg4:

- if PDSCH-ConfigCommon has provided a table

  - Use this table

- else

  - Use the default PDSCH table A

Agreements:

- For TB size determination, assume Xoh=0 for all transmission scheduled with broadcast RNTIs (SI-RNTI, RA-RNTI, P-RNTI,) and msg3 regardless of UE state.

- Adopt following TP for 38.214:

---- Start of TP ----

6.1.4.1	Modulation order and target code rate determination

For PUSCH scheduled by RAR UL Grant or the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, or CS-RNTI, or SP-CSI-RNTI

6.1.4.2	Transport block size determination

For PUSCH scheduled by RAR UL Grant or the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, CS-RNTI, or SP-CSI-RNTI.

---- End of TP ----

Friday

R1-1807849	Outcome of offline discussion on 7.1.3.3.1 (resource allocation) – part III	Ericsson

Decision: The document is noted.

Agreements:

For C-RNTI and CS-RNTI in CSS at least in Pcell (FFS Scell Type 3 CSS)

- If the CSS is not associated with CORESET#0 and PDSCH-Config has provided a table

  - Use this table for PDSCH transmission scheduled by DCI in CSS

- else if PDSCH-ConfigCommon has provided a table

  - Use this table for PDSCH transmission scheduled by DCI in CSS

- else

  - Use the default PDSCH table A (mux pattern 1)

- Note: Same table is used as in USS

- Note: If some entries in the table before RRC reconfiguration and in the table after RRC reconfiguration are the same, the UE can be schedulable during the RRC reconfiguration period

  - No specification impact

- Note: it is assumed that random access response can be received in CORESET #0

For C-RNTI and CS-RNTI in CSS:

- if the CSS is not associated with CORESET#0 and PUSCH-Config has provided a table

  - Use this table for PUSCH transmission scheduled by DCI in CSS

- else if PUSCH-ConfigCommon has provided a table for PUSCH

  - Use this table for PUSCH transmission scheduled by DCI in CSS

- else

  - Use the default PUSCH table

- Note: Same table is used as in USS

- Note: If some entries in the table before RRC reconfiguration and in the table after RRC reconfiguration are the same, the UE can be schedulable during the RRC reconfiguration period

  - No specification impact

- Note: it is assumed that random access response can be received in CORESET #0

Agreement:

- Increase the number of bits for RIV in RAR from 12 bits to 14 bits.

Agreement:

- Change Delta from 2 to 3 for 30 kHz for msg 3 processing

Agreement:

- In addition to previously agreed rows, also row 8 in Table C can be used for RMSI scheduling

R1-1807886	WF on PUSCH time-domain resource allocation	NTT DOCOMO, Samsung, Panasonic, KDDI, OPPO, Ericsson, CATT, AT&T, vivo, Nokia, NSB, NTT, Sharp, ZTE

Decision: The document is noted.

R1-1805893	Remaining issues on resource allocation and TBS	Huawei, HiSilicon

R1-1805900	PDSCH design for URLLC	Huawei, HiSilicon

R1-1806067	Resource block bundle for RMSI	vivo

R1-1806134	Remaining issues for data resource allocation	ZTE

R1-1806299	Remaining issues on time domain resource allocation	CATT

R1-1806367	Discussion on remaining issues for time domain resource allocation	CMCC

R1-1806625	Remaining issues on Time-domain Resource allocation	LG Electronics

R1-1806737	Corrections on DL/UL Resource Allocation	Samsung

R1-1806815	Remaining Issues on DL/UL Resource Allocation	MediaTek Inc.

R1-1806867	Remaining issues on multi-slot resource allocation	OPPO

R1-1807041	UE Capabilities for Supporting URLLC	Ericsson Inc.

R1-1807066	On PUSCH time-domain resource allocation	NTT DOCOMO, INC.

R1-1807280	Remaining issues on resource allocation	Nokia, Nokia Shanghai Bell

R1-1807412	Remaining issues for DL UL resource allocation	Ericsson

R1-1807622	Summary of 7.1.3.3.1 (resource allocation)	Ericsson

R1-1807652	Remaining Issues on DL/UL resource allocation	Qualcomm Incorporated

R1-1807736	On the Size of Transmit Buffer	Huawei, HiSilicon

##### DL/UL scheduling and HARQ management

Including handling URLLC related aspects for parallel DL receptions

R1-1807681	Summary of Maintenance for DL/UL Scheduling	Qualcomm

Decision: The document is noted.

Agreements:

- For UE behavior in RRC_IDLE, if PDSCHs among SI-RNTI PDSCH, P-RNTI PDSCH, and RA-RNTI/TC-RNTI PDSCH are overlapped with at least one symbol for a given UE from the primary cell

  - NR supports UE to decode up to two PDSCH simultaneously in Pcell, but not more than two (if more than two are received, the decoding prioritization is up to UE implementation)

  - Note: the above assumes that all the PDSCHs to be decoded by the UE have non-overlapped PRBs

Agreements:

The following working assumption as part of the previous agreement is confirmed with updates:

- While UE acquires SI upon being triggered by Paging DCI

  - UE is not required to decode C-RNTI PDSCH if the SI-RNTI PDSCH is overlapped with at least one symbol

- In case UE autonomously monitors SI-RNTI PDCCH while monitoring C-RNTI PDCCH, and both SI-RNTI PDSCH and C-RNTI PDSCH are overlapped with at least one symbol, the UE is not required to decode SI-RNTI PDSCH

- (Working assumption) The first two bullets apply unless TBS of SI-RNTI PDSCH ≤ 2976 for FR1, then UE decodes both SI-RNTI PDSCH and C-RNTI PDSCH

  - Note: the above assumes that the PDSCHs to be decoded by the UE have non-overlapped PRBs

- The first two bullets always apply in FR2

Agreements:

- To adopt the following TP:

------ Start of Text Proposal for TS 38.214 Subclause 5.3 ------

N1 is based on µ of table 5.3-1 that corresponds to the min (µDL, µUL) where the µDL corresponds to the subcarrier spacing of the downlink with which the PDSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in subclause 4.41 of [4, TS 38.211] If the higher layer parameter dmrs-AdditionalPosition is configured with a value greater than 0  (at least for one of PDSCH mapping Type A or B) or if the high layer parameter is not configured (at least for one of PDSCH mapping Type A or B), then N1 value follows from the second column of table 5.3-1; otherwise, the N1 value follows from the first column of table 5.3-1.

------ End of Text Proposal ------

Agreements: The following working assumption is confirmed with updates:

- The processing requirement for HARQ-ACK in response to DL SPS release is N OFDM symbols from last symbol of the corresponding PDCCH for UE Capability #1:

  - 15 kHz case: 8 + 2 5

  - 30 kHz case: 10 + 2 5

  - 60 kHz case: 17 + 5

  - 120 kHz case: 20 + 5

Tuesday session

R1-1807711	Summary of Maintenance for DL/UL Scheduling	Qualcomm

Decision: The document is noted.

Agreements:

In the case of multiplexing CSI with uplink data on PUSCH, the MIMO session should update agreements related to CSI multiplexing with UL-SCH (i.e., data on PUSCH), and NR should adopt the framework below:

- Z’’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the DCI triggering a CSI report on PUSCH to the earliest possible start of the corresponding CSI transmission on PUSCH from UE perspective

  - Z’’ ≥ Z + d where Z is based on CSI-only processing time from the MIMO session

- N2’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the UL grant to the earliest possible start of the corresponding PUSCH from UE perspective

  - N2’ ≥ N2 + d where N2 is based on the UE capability for sending data-only on PUSCH

- d symbols

  - FFS value of d <= N2

    - E.g., d=1

    - E.g., d=1 for 15kHz, 2 for 30kHz, 3 for 60kHz, and 4 for 120kHz

    - E.g., d=N2

- UE is not expected transmit the CSI multiplexed with uplink data if the network set the values of K2 and SLIV without leaving sufficient time for UE processing

Agreements:

- d in the above agreement is updated to the following

- 2 for 15kHz

- 2 for 30kHz

- 3 for 60kHz

- 4 for 120kHz

Conclusion:

- The following limitation in DL control procedure is beneficial (possible even necessary) from a processing time perspective.

  - FFS whether to capture in specification or as capability.

| (If proposed as capability, this would be in addition to feature 3-5, 3-5a) | For type 1 with dedicated RRC configuration, type 3, and UE-SS,, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a ocassion gap | For type 1 with dedicated RRC configuration, type 3 and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2, with minimum time separation between two occasions for any search space, is one or more of the following cases for a given SCS 1OFDM symbol 2OFDM symbols 4OFDM symbols 7OFDM symbols |
| --- | --- | --- |

Wednesday session

R1-1807752	Summary of Maintenance for DL/UL Scheduling	Qualcomm

Decision: The document is noted.

Agreements:

- Cross-carrier scheduling should at least satisfy the causality constraints between scheduling PDCCH and PDSCH as for self-scheduling, also taking into carrier timing difference

  - Note: in the case of mixed numerology, limitations on the number of symbols to buffer need to be taken into account

Agreements:

- When HARQ-ACK for the PDSCH with larger SCS is transmitted on a carrier with smaller SCS

  - K1=0 corresponds to the slot for the smaller SCS which overlaps with the PDSCH

- When HARQ-ACK for the PDSCH with smaller SCS is transmitted on a carrier with larger SCS

  - K1=0 corresponds to the slot for the larger SCS with end boundary aligned to the slot for the corresponding PDSCH

Conclusion:

- Note that below are already in specification

  - When larger SCS schedules smaller SCS, overlapping slot is considered as K0=0 or K2=0

  - When smaller SCS schedules larger SCS, K0=0 or K2=0 is defined for slot with boundary aligned with that of slot containing the corresponding PDCCH

    - Slot timing is defined by the start of the slot

Agreement:

- For cross-carrier scheduling across different numerology

  - FFS: how to specify additional constraints related to K0 to address the number of symbols which may need to be buffered

Agreements:

- For any two HARQ process IDs A and B for a given cell,

- If the scheduling DCI scrambled by C-RNTI for unicast PUSCH transmission A comes before (in time) the scheduling DCI scrambled by C-RNTI for unicast PUSCH transmission B, then for the Dec. 2017 baseline capability

  - UE is not expected to be scheduled such that PUSCH for B is before the PUSCH for A

- For any two HARQ process IDs A and B for a given cell,

- If the scheduling DCI scrambled by C-RNTI for unicast PDSCH transmission A comes before (in time) the scheduling DCI scrambled by C-RNTI for unicast PDSCH transmission B, then for the Dec. 2017 baseline capability

  - UE is not expected to be scheduled such that PDSCH for B is before the PDSCH for A

Thursday

R1-1807825	Summary of Maintenance for DL/UL Scheduling	Qualcomm

Decision: The document is noted.

Agreements:

- To adopt the following TP to Section 5.3 of 38.214:

---- Start of TP ----

If the first uplink symbol of the physical channel which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and the PUSCH or PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1 then the UE shall provide a valid HARQ-ACK message, where L1 is defined as the next uplink symbol with its CP starting after   after the end of the last symbol of the PDSCH carrying the TB being acknowledged.

-	N1 is based on µ of table 5.3-1 that corresponds to the min (µDL,PDCCH, µDL,PDSCH, µUL) where the µDL,PDCCH corresponds to the subcarrier spacing of the downlink with which the PDCCH was transmitted, µDL,PDSCH corresponds to the subcarrier spacing of the downlink with which the PDSCH was transmitted, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in subclause 4.41 of [4, TS 38.211] If the higher layer parameter dmrs-AdditionalPosition is configured with a value greater than 0, or if the high layer parameter is not configured, then N1 value follows from the second column of table 5.3-1; otherwise, the N1 value follows from the first column of table 5.3-1.

---- End of TP ----

Agreements: The Capability #2 for (Aggressive) UE processing time in Rel-15 is supported under the following conditions

- Non-CA

  - Note: this does not preclude EN-DC

- FFS CA case with Capability #2 supported on only one or more of the carriers, and potential handling of some special cases

- Single numerology for PDCCH, PDSCH, and PUSCH for the serving cell

- PDSCH/PUSCH allocation with mapping Type A and Type B

  - For PDSCH mapping type A with last PDSCH symbol ending in symbol ‘i' of a slot, where i < 7

    - N1 processing time is increased by (7-i) relative to the case where i=7.

  - (Working assumption) For PDSCH mapping type B with 4 or 2 symbols

    - N1 processing time is increased by ‘d’ symbols relative to the case of PDSCH with 7 symbols, where ‘d’ is the amount of time-domain overlap in symbols between the scheduling PDCCH and the scheduled PDSCH

      - FFS: handling of 3-symbol CORESET where first 2 symbols of CORESET are overlapped with a 2-symbol PDSCH

- No UCI multiplexing

  - FFS: whether similar multiplexing rule as with Capability #1 may be included

- For C-RNTI

  - FFS: simultaneous reception with broadcast PDSCH

- Note: The UE signals whether Capability #2 is supported for each SCS, and separately for uplink and downlink

Agreements: The Capability #2 UE processing times are given below, where the PDSCH allocation length is assumed to be at least 7 symbols.

- N1

  - 15kHz: N1 = 3

  - 30kHz: N1 = 4.5

    - Note: as part of UE capability,

      - UE may report support with maximum scheduled RB allocation of 136 RBs

        - If RB allocation by scheduling exceeds the maximum signaled, UE defaults to Capability #1 processing time

      - OR UE may report support with no restriction on maximum scheduled RB allocation

  - 60kHz for FR1: 9

- N2

  - 15kHz: N2 = 5 for CP-OFDM & For DFT-S-OFDM

  - 30kHz: N2 = 5.5

  - 60kHz for FR1: 11

  - If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2.

Proposals:

- To adopt the following TP to Section 5.1 of 38.214:

A UE shall upon detection of a PDCCH with a configured DCI format 1_0 or 1_1 decode the corresponding PDSCHs as indicated by that DCI.

In the case when the PDCCH and the corresponding PDSCHs are of the same numerology, a UE is not expected to receive PDSCH type A in the same slot if this PDCCH comes after the first 3 symbols of a slot.

A UE is not expected to receive a PDCCH to schedule a PDSCH of mapping type B for which the first symbol of the PDSCH comes earlier than the first symbol of PDCCH.

The UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Subclause 9.2.3 of [6]. The UE is not expected to receive a PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and another PDSCH in slot after slot i with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j.

Friday

R1-1807902	Summary of Maintenance for DL/UL Scheduling	Qualcomm

Decision: The document is noted.

Agreements:

In the case of multiplexing CSI, HARQ-ACK, and uplink data on PUSCH

- N1’ the number of OFDM symbols required for UE processing from the end of PDSCH to the earliest possible start of the corresponding ACK/NACK transmission on PUSCH from UE perspective

  - N1’ ≥ N1 + d where N1 is based on the UE capability for ACK-only

- Z’’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the DCI triggering a CSI report on PUSCH to the earliest possible start of the corresponding CSI transmission on PUSCH from UE perspective

  - Z’’ ≥ Z + d where Z is based on CSI-only processing time from the MIMO session

- N2’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the UL grant to the earliest possible start of the corresponding PUSCH from UE perspective

  - N2’ ≥ N2 + d where N2 is based on the UE capability for sending data-only on PUSCH

- d symbols

  - 2 for 15kHz

  - 2 for 30kHz

  - 3 for 60kHz

  - 4 for 120kHz

- UE is not expected transmit the CSI multiplexed with HARQ-ACK and with uplink data if the network set the values of K1, K2 and SLIV without leaving sufficient time for UE processing

Agreement:

- The TP in Section 3 of R1-1807902 is endorsed

Agreements:

- UE is not required to receive on a downlink symbol and then transmit on a uplink symbol if those two symbols are not separated by at least Rx2Tx us on unpaired spectrum for a given serving cell, from the UE perspective

  - Discuss further whether it’s an error case or to specify a UE behavior

  - Note that the exact value of Rx2Tx has been specified in RAN4 [R4-1805766]

R1-1805888	Remaining issues on scheduling and HARQ	Huawei, HiSilicon

R1-1806021	Scheduling and HARQ for URLLC	Ericsson

R1-1806068	Remaining issues on HARQ management for eMBB	vivo

R1-1806069	Discussion on DL scheduling for URLLC	vivo

R1-1806343	Remaining issues for HARQ-ACK feedback timing indication	Lenovo, Motorola Mobility

R1-1806368	Discussion on scheduling and HARQ management	CMCC

R1-1806522	Remaining details on scheduling and HARQ	Intel Corporation

R1-1806626	URLLC aspects on DL/UL scheduling and HARQ management	LG Electronics

R1-1806627	Remaining issues on DL/UL scheduling and HARQ	LG Electronics

R1-1806738	Corrections on HARQ Management and Feedback	Samsung

R1-1806801	Remaining issues of DLUL scheduling and HARQ management	MediaTek Inc.

R1-1806814	PUCCH HARQ reporting framework for URLLC	MediaTek Inc.

R1-1806845	Parallel DL reception	OPPO

R1-1806903	Handling of parallel DL receptions with URLLC	Huawei, HiSilicon

R1-1806917	Remaining issues on DL/UL scheduling and HARQ management	Panasonic

R1-1807067	On HARQ-ACK feedback	NTT DOCOMO, INC.

R1-1807162	On the out-of-order HARQ-ACK support for URLLC	Nokia, Nokia Shanghai Bell

R1-1807256	Remaining issues of DL Scheduling and Processing times	Ericsson

R1-1807364	Remaining Issues on DL/UL Scheduling, Processing Time and HARQ management	Qualcomm Incorporated

R1-1807365	On UE Capability #2 Processing Time	Qualcomm Incorporated

##### CBG-based (re)transmission

R1-1807257	Remaining issues for CBG based HARQ	Ericsson

##### UL data transmission procedure

R1-1807640	Offline summary for UL data transmission procedure	NTT DOCOMO

Decision: The document is noted.

Agreements:

- Adopt following TP for 38.213 subclause 10.2.

---- Start of TP ----

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or UL Type 2 configured grant PDCCH only if all the following conditions are met:

- when the UE detects a corresponding DCI format after descrambling the CRC parity bits of the DCI format are scrambled with a CS-RNTI provided by higher layer parameter cs-RNTI

- the new data indicator field is set to ‘0’. In case of DCI formats 1-1, the new data indicator field refers to the one for the enabled transport block.

Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or UL configured grant Type 2. If validation is not achieved, the UE considers the DCI format as having been detected with a non-matching CRC.

Table 10.2-1: Special fields for DL SPS and UL grant Type 2 scheduling activation PDCCH validation

|  | DCI format 0_0/0_1 | DCI format 1_0 | DCI format 1_1 |
| --- | --- | --- | --- |
| HARQ process number | set to all ‘0’s | set to all ‘0’s | set to all ‘0’s |
| Redundancy version | set to ‘00’ | set to ‘00’ | For the enabled transport block: set to ‘00’ |

Table 10.2-2: Special fields for DL SPS and UL grant Type 2 scheduling release PDCCH validation

|  | DCI format 0_0 | DCI format 1_0 | D |
| --- | --- | --- | --- |
| HARQ process number | set to all ‘0’s | set to all ‘0’s | s |
| Redundancy version | set to ‘00’ | set to ‘00’ | F |
| Modulation and coding scheme | set to all ‘1’s | set to all ‘1’s | s |
| Resource block assignment | set to all ‘1’s | set to all ‘1’s | s |

---- End of TP ----

Agreements:

- To adopt the following TP (to reflect a previous agreement) to 6.1.2.3 of 38.214:

---- Start of TP ----

For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or at the last transmission occasion among the K repetitions within the period P, or when a UL grant for scheduling the same TB is received within the period P, whichever is reached first.

UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P.

---- End of TP ----

R1-1807641	Offline summary for UL data transmission procedure for June drop URLLC	NTT DOCOMO

R1-1805824	Underlay SR: a complementary solution to overcome the limitations of periodic PUCCH-SR  	Idaho National Laboratory

R1-1806030	Remaining issues for UL data transmission procedure	ZTE

R1-1806070	Discussion on URLLC UL transmission procedure	vivo

R1-1806369	Discussion on handling collision issues of UL URLLC and eMBB	CMCC

R1-1806523	On slot aggregation for URLLC	Intel Corporation

R1-1806628	Remaining issues on UL data transmission procedure	LG Electronics

R1-1806810	PUSCH repetition in configured-grant for URLLC	MediaTek Inc.

R1-1806811	Remaining issues for UL data transmission procedure	MediaTek Inc.

R1-1806846	Remaining issues for low latency uplink transmission	OPPO

R1-1806901	Summary of remaining issues on UL data transmission procedure	Huawei, HiSilicon

R1-1806902	PUSCH reliability for URLLC	Huawei, HiSilicon

R1-1807068	UL data transmission procedure	NTT DOCOMO, INC.

R1-1807069	UL data transmission procedure for June drop URLLC	NTT DOCOMO, INC.

R1-1807258	Remaining issues of UL data transmission procedure	Ericsson

R1-1807366	Considerations on short periodicities of DL SPS for URLLC	Qualcomm Incorporated

##### Soft-buffer management for NR

R1-1805889	Remaining issues on soft buffer management	Huawei, HiSilicon

R1-1807259	Remaining issues of Soft buffer management	Ericsson

R1-1807698	Summary for 7.1.3.3.5	Ericsson

Decision: The document is noted.

Agreements:

- Update the conclusion from RAN1#92bis as follows:

Conclusion:

- It is RAN1 understanding that UE is not expected to handle cases where the Ncb Nref value (in 5.4.2.1) changes across transmissions for the same transport block.

  - No spec impact.

Friday

R1-1807851	Summary of offline discussion on 7.1.3.3.5	Ericsson

Decision: The document is noted.

Conclusion:

- Performance is not expected to be optimized for the following special cases (while supported in specification) in which retransmissions of a transport block occur at or near peak rate

  - the TB is transmitted with significantly lower code rate (considering initial and retransmission) than R_LBRM(=2/3)

  - the retransmission occurs on a much shorter PDSCH duration than the initial transmission

  - No spec change

Agreements:

- To adopt the TP to Section 5.4.2.1 of 38.212

maximum number of layers for one TB supported by the UE for the serving cell, which for UL-SCH is according to higher layer parameter ULmaxRank if the parameter is configured;

##### Multiplexing data with different transmission durations

R1-1807646	Summary of 7.1.3.3.6 – URLLC related aspects	vivo

Decision: The document is noted.

R1-1806071	Remaining issues on Multiplexing data with different transmission durations	vivo

R1-1806668	Remaining issues on DL data multiplexing with different reliability requirements	ETRI

R1-1806897	Summary of remaining issues on pre-emption indication with TP	Huawei, HiSilicon

R1-1807260	Remaining issues of multiplexing data with different transmission durations	Ericsson

R1-1807367	Considerations on differentiating eMBB and URLLC	Qualcomm Incorporated

##### Other

R1-1806384	[DRAFT] Reply LS on LTE peak data-rate calculation for EN-DC	Ericsson

R1-1806883	TP on HARQ-ACK reporting with different numerology	Huawei, HiSilicon

R1-1806898	Support of 60 kHz subcarrier spacing	Huawei, HiSilicon

R1-1806900	Considerations on DMRS to support URLLC	Huawei, HiSilicon

R1-1806918	Inconsistent PDCCH detection	Panasonic

R1-1807138	Discussion on semi-static HARQ-ACK codebook when slot aggregation for UCI reliability	Huawei, HiSilicon

R1-1807261	TP on remaining issues for DL UL resource allocation	Ericsson

#### Other aspects on carrier aggregation and bandwidth parts

##### Other aspects on bandwidth Parts

R1-1807667	Summary of bandwidth part remaining issues	MediaTek

Decision: The document is noted.

Wednesday

R1-1807731	LS on Bandwidth configuration for initial BWP	RAN2, NTT DOCOMO

Decision: The document is noted. Reply LS is necessary

Friday

R1-1807840	[Draft] Reply LS on bandwidth configuration for initial BWP	MediaTek

Decision: The draft LS is endorsed and final LS is approved in R1-1807873.

R1-1807729	Summary of bandwidth part remaining issues	MediaTek

Decision: The document is noted.

Agreements:

- When DCI format 1_1 is used for active BWP switching,

  - A UE ignores the following DCI bitfields after MSB zero-padding when the old active DL BWP is configured with 1 TB and the indicated DL BWP is configured with 2 TBs

    - Modulation and coding scheme for TB2

    - New data indicator for TB2

    - Redundancy version for TB2

  - FFS For “transmission configuration indication” bitfield, a UE follows the TCI for the CORESET with the lowest CORESE ID of the new active DL BWP in the same slot as the scheduled PDSCH

  - For “frequency domain resource assignment” bifield with the following cases, no further special handling, in addition to MSB truncation and zero-padding, is needed

    - When the indicated DL BWP is configured with Type 0 resource allocation only

    - When the indicated DL BWP is configured with both Type 0 & Type 1 resource allocation

      - In case of MSB zero-padding, Type 0 resource allocation is always applied

    - When the indicated DL BWP is configured with Type 1 resource allocation only & bitfield truncation is applied

  - For “frequency domain resource assignment” bitfield with the following case, continue discussion on whether further special handling, in addition to MSB truncation and zero-padding, is needed

    - When the indicated DL BWP is configured with Type 1 resource allocation only & MSB zero-padding is applied

  - For other DCI bitfields, no further special handling, in addition to MSB truncation and zero-padding, is required

- When DCI format 0_1 is used for active BWP switching,

  - FFS  For “SRS resource indicator” bitfield, a UE follows the spatial relationship of the PUCCH resource with the lowest PUCCH resource ID of the new active UL BWP

  - FFS For “PTRS-DMRS association” bitfield, no further special handling, in addition to MSB truncation and zero-padding, is needed

  - For “frequency domain resource assignment” bitfield with the following cases, no further special handling, in addition to MSB truncation and zero-padding, is required

    - When the indicated UL BWP is configured with Type 0 resource allocation only

    - When the indicated UL BWP is configured with both Type 0 & Type 1 resource allocation

      - In case of MSB zero-padding, Type 0 resource allocation is always applied

    - When the indicated UL BWP is configured with Type 1 resource allocation only & bitfield truncation is applied

  - For “frequency domain resource assignment” bitfield with the following case, continue discussion whether further special handling, in addition to MSB truncation and zero-padding, is required

    - When the indicated UL BWP is configured with Type 1 resource allocation only & bitfield zero-padding is applied

  - For other DCI bitfields, no further special handling, in addition to MSB truncation and zero-padding, is required

Thursday

R1-1807801	Summary of bandwidth part remaining issues	MediaTek

Decision: The document is noted.

Agreements:

- If a UE doesn’t support DCI-based active BWP switching, the UE ignores the “BWP indicator” bitfield in DCI format 1_1 and DCI format 0_1 when the bitfield exists

  - Note: it doesn’t imply that DCI-based active BWP switching is an optional feature

Agreements:

- It was noticed by RAN1 that there is an agreement regarding that a UE should report UL Tx DC location to the network by a semi-static signalling. However, such signalling wasn’t implemented in RAN2’s RRC spec. yet. RAN1 kindly asks RAN2 to implement such signalling.

  - FFS details of the signalling

- Check offline further on DL Tx DC location related issue

- To send an LS to RAN2 – Peikai (MediaTek)

Friday

R1-1807905	[Draft] LS on signalling of transmitter-side DC location	MediaTek

Decision: The draft LS is endorsed and final LS is approved in R1-1807910.

Agreements:

- When DCI format 1_1 is used for active BWP switching,

  - For “frequency domain resource assignment” bitfield with the following case, no further special handling, in addition to MSB truncation and zero-padding, is needed

    - When the indicated DL BWP is configured with Type 1 resource allocation only & MSB zero-padding is applied

  - For “transmission configuration indication” bitfield,

    - When there is no TCI bitfield in the BWP switching DCI, a UE follows the TCI indication defined in the spec, assuming the TCI bitfield disabled, based on the old active DL BWP

    - Otherwise, no further special handling, in addition to MSB truncation and zero-padding, is needed

- When DCI format 0_1 is used for active BWP switching,

  - For “frequency domain resource assignment” bitfield with the following case, no further special handling, in addition to MSB truncation and zero-padding, is required

    - When the indicated UL BWP is configured with Type 1 resource allocation only & bitfield zero-padding is applied

  - (working assumption) For “PTRS-DMRS association” bitfield,

    - When there is no “PTRS-DMRS association” bitfield in the BWP switching DCI, a UE follows the behaviour defined in the spec, assuming “PTRS-DMRS association” bitfield disabled

    - Otherwise, no further special handling, in addition to MSB truncation and zero-padding, is needed

Agreements:

- If there is active UL BWP switching between the time of TA command reception and the corresponding TA applying time, a UE applies the numerology of new active UL BWP

- A UE assumes the same absolute TA value before & after active UL BWP switching, including the case of numerology change

Friday

Above agreements are modified as:

Agreements:

- If there is active UL BWP switching between the time of TA command reception and the corresponding TA applying time, a UE applies the TA command based on the numerology of new active UL BWP for the UL transmission in new active UL BWP

- Otherwise, A UE assumes the same absolute TA value before & after active UL BWP switching, including the case of numerology change

Agreements:

- For UL, a 13-bit indicator is used to indicate the Tx DC location corresponding to a UL BWP within a carrier

  - This information is provided for each UL BWP configuration

  - It’s also RAN1’s understanding that this information can be provided in RRCReconfigurationComplete from a UE to the network when a UL BWP is configured

  - 1 bit is used to indicate whether there is 7.5 KHz shift

    - “0” means no 7.5 KHz shift

    - “1” means 7.5 KHz shift

  - 12 bits are used to indicate the Tx DC location with the following table

| Subcarrier index (12 bits) within the carrier corresponding to the numerology of the UL BWP | Tx DC location |
| --- | --- |
| 0 | 0 |
| ⁞ | ⁞ |
| 3299 | 3299 |
| 3300 | Outside the carrier |
| 3301 | Undetermined position within the carrier |
| 3302 | Reserved |
| 3303 | Reserved |
| ⁞ | Reserved |
| 4095 | Reserved |

- For DL, 12-bit indicator is used to indicate the Tx DC location corresponding to a numerology within a carrier

  - This information is provided for each numerology potentially used for the carrier

  - It’s RAN1’s understanding that this information can be optionally provided in SCS-SpecificCarrier from the network to a UE

  - It’s RAN1’s understanding that the UE can assume that Tx DC location is outside the carrier if this information is not provided – up to RAN4 to handle performance related issues

  - 12 bits are used to indicate the Tx DC location with the following table

| Subcarrier index (12 bits) within the carrier corresponding to a numerology | Tx DC location |
| --- | --- |
| 0 | 0 |
| ⁞ | ⁞ |
| 3299 | 3299 |
| 3300 | Outside the carrier |
| 3301 | Reserved |
| 3302 | Reserved |
| 3303 | Reserved |
| ⁞ | Reserved |
| 4095 | Reserved |

R1-1805892	Remaining issues on bandwidth part and wideband operation	Huawei, HiSilicon

R1-1806072	Remaining issues on bandwidth part switching	vivo

R1-1806135	Remaining issues for BWP	ZTE

R1-1806143	On remaining details of BWPs	Nokia, Nokia Shanghai Bell

R1-1806300	On BWP operation	CATT

R1-1806390	QCL related issues during BWP switching	PANASONIC

R1-1806411	Remaining issues on BWP	Spreadtrum Communications

R1-1806524	Remaining details of BWP adaptation	Intel Corporation

R1-1806629	Remaining issues on bandwidth part operation	LG Electronics

R1-1806739	Corrections on Bandwidth Part Operation	Samsung

R1-1806775	Remaining Details on Bandwidth Part Operation in NR	MediaTek Inc.

R1-1806868	Other aspects on bandwidth parts	OPPO

R1-1806949	Clarification on aperiodic CSI reporting for CA	Sharp

R1-1806964	Remaining issues on dynamic BWP switching	InterDigital, Inc.

R1-1807070	Other aspects on bandwidth Parts	NTT DOCOMO, INC.

R1-1807263	Remaining issues for BWP operation	Ericsson

R1-1807270	Corrections to BWP Operation	III

R1-1807277	Remaining details for BWP operation	Motorola Mobility, Lenovo

R1-1807368	Remaining issues on BWP	Qualcomm Incorporated

##### Other aspects on carrier aggregation

Including up to two different numerologies per PUCCH group

R1-1807699	Summary on CA aspects	Samsung

Decision: The document is noted.

Conclusion:

- It is understood that only TB-level HARQ-ACK is supported for SPS PDSCH

Agreement:

- To clarify in 38.213 that “among the DCI formats 1_0 or DCI formats 1_1 indicating the same slot for HARQ-ACK reporting” is intended for the last DCI.

Wednesday

R1-1807771	Summary on CA aspects	Samsung

Decision: The document is noted.

Agreement:

- For semi-static HARQ codebook, for PDSCH with slot aggregation over R slots, keep current specifications and include that number of slots can be >1.

Proposals:

- For semi-static HARQ codebook, HARQ response is transmitted in every HARQ-ACK codebook for the configured k1 values

  - Note: the current spec already reflects it.

Agreement:

- UE is not expected to transmit more than one PUCCH with HARQ-ACK in a slot at least for Dec. 2017 drop.

Thursday

Agreement:

- UE reports a PDSCH detection outcome (ACK/NACK) for a PDSCH occasion only in HARQ-ACK codebook indicated by PDSCH-to-HARQ timing field in corresponding DL DCI. UE reports NACK/DTX in other HARQ-ACK codebooks where UE reports HARQ-ACK for the PDSCH occasion

Friday

R1-1807878	Third Offline Summary on CA Aspects	Samsung

Decision: The document is noted.

Agreements:

- Semi-static HARQ-ACK codebook size does not depend on the time duration between a last symbol for any PDSCH reception candidate and a first symbol for a PUCCH/PUSCH transmission that includes the HARQ-ACK codebook

  - UE sets to NACK the HARQ-ACK bits in the HARQ-ACK codebook that correspond to PDSCH reception occasions that do not satisfy the above timing requirement

  - No additional specification impact

Agreement:

- Prioritize grant-based PUSCH over grant-free PUSCH for UCI multiplexing

R1-1807879	TP to include total DAI in TS38.213 for determining missed TBs/CBGs	Samsung

R1-1805895	Remaining issues on NR CA	Huawei, HiSilicon

R1-1806073	Remaining issues on NR CA	vivo

R1-1806144	Remaining aspects of NR CA	Nokia, Nokia Shanghai Bell

R1-1806301	Remaining issues on HARQ-ACK codebook	CATT

R1-1806344	Remaining issues for CA with different numerologies	Lenovo, Motorola Mobility

R1-1806525	Remaining aspects for carrier aggregation	Intel Corporation

R1-1806630	Remaining issues on CA and HARQ-ACK codebook	LG Electronics

R1-1806740	Corrections on CA Operation	Samsung

R1-1806776	Remaining issues in carrier aggregation	MediaTek Inc.

R1-1806838	Remaining details on HARQ-ACK transmission	OPPO

R1-1807415	Remaining issues on CA and type1 HARQ-ACK codebook	NTT DOCOMO, INC.

R1-1807236	Remaining issues on HARQ-ACK codebook construction	WILUS Inc.

R1-1807241	Discussion about semi-static codebook	ASUSTEK COMPUTER (SHANGHAI)

R1-1807262	Other aspects on carrier aggregation	Ericsson

R1-1807399	Remaining issues on CA	Qualcomm Incorporated

#### Rate matching aspects for NR DL and UL

R1-1807705	Summary of open issues related to rate-matching in NR	Nokia, Nokia Shanghai Bell

Decision: The document is noted.

Proposal:

- To adopt the following TP,  TS38.214 subclause 5.1.4:

“a pair of reserved resources in numerology of the BWP indicated by an RB level bitmap (higher layer parameter resourceBlocks given by RateMatchPattern) with 1RB granularity and a symbol level bitmap spanning one or two slots (higher layer parameters symbolsInResourceBlock given by RateMatchPattern ) for which the reserved RBs apply. The bit in the RB and symbol level bitmaps indicates that corresponding resource is not available for PDSCH.  For each pair of RB and symbol level bitmaps, a UE may be configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by RateMatchPattern ) corresponding to a unit equal to a duration of the symbol level bitmap, and indicating whether the pair is present in the unit or not.”

Discuss further offline when UE is configured to rate-match around the LTE CRS, whether or not it rate-matches also around the centre sub-carrier provided with the configuration.

Thursday (from offline)

Agreements:

When an PDCCH candidate collides with any RE of LTE CRS configured for NR PDSCH rate-matching, where PDSCH is semi-statically not mapped, as described in TS38.214 Section 5.1.4.1 then

- The UE is not required to monitor the PDCCH candidate overlapping with the resource even only when the overlaps is by one RE

- Note: this is the same behavior as agreed for collision with RB-level in RAN1#92b

Agreements:

- To adop the following TP to 5.1.4 of 38.214:

---- Start of TP ----

“a pair of reserved resources in numerology of the BWP indicated by an RB level bitmap (higher layer parameter resourceBlocks given by RateMatchPattern) with 1RB granularity and a symbol level bitmap spanning one or two slots (higher layer parameters symbolsInResourceBlock given by RateMatchPattern) for which the reserved RBs apply. The bit equal to 1 in the RB and symbol level bitmaps indicates that corresponding resource is not available for PDSCH.  For each pair of RB and symbol level bitmaps, a UE may be configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by RateMatchPattern ) corresponding to a unit equal to a duration of the symbol level bitmap, and bit equal to 1 indicating whether the pair is present in the unit or not.”

….

A configured group rateMatchPatternGroup1 or rateMatchPatternGroup2 contains a list of RB symbol level resource set indices forming a union of resource-sets not available for PDSCH dynamically if corresponding bit in the PDCCH with a scheduling DCI. The REs corresponding to the union of configured RB-symbol level resource-sets that are not included in neither of the two groups are not available for PDSCH.”

---- End of TP ----

Proposals:

In response to RAN2 LS in R1-1805809, include the following in SIB1

- Rate-matching bitmaps (RateMatchPattern)

  - resourceBlocks (275bits)

  - symbolsInResourceBlock (14/28bits)

  - periodicityAndPattern (10 or 40 bits [up to RAN2])

  - Note: bit equal to 1 in periodicityAndPattern indicates whether unit of a slot or two-slots is rate-matched

Friday

R1-1807900	[DRAFT] Reply to LS on Defining reserved resources in NR-SIB1	Nokia

Decision: The draft LS is endorsed (by accepting all changes) and final LS is approved in R1-1807907.

R1-1805894	Remaining issues on reserved resources and rate-matching	Huawei, HiSilicon

R1-1806074	Remaining aspects for rate matching	vivo

R1-1806136	Remaining aspects for rate matching	ZTE

R1-1806145	Remaining details on rate-matching in NR	Nokia, Nokia Shanghai Bell

R1-1807370	Remaining issues on PDSCH rate matching in CORESET	Qualcomm Incorporated

#### Other

R1-1806887	Consideration on power saving for NR CA	Huawei, HiSilicon

R1-1807005	Enhancements for NR UE power saving	Apple Inc.

R1-1807306	Design considerations for UE power saving	Huawei, HiSilicon

### Channel coding

R1-1807786	Chairman's notes of AI 7.1.4 Channel coding	Ad-Hoc chair (Ericsson)

Decision: Contents incorporated below, the document is endorsed.

R1-1806741	Remaining Issues on Channel Coding	Samsung

Agreement:

For transmission on PUCCH, if the number of bits in a CSI part 2 is less than 3 bits, zeros shall be appended to the CSI part 2 sequence until the payload size equals 3.

R1-1807765	Text proposals on channel coding	Intel Corporation

Decision: Text proposal 2 in R1-1807765 is agreed.

R1-1807042	Assumptions on the number of CRC bits for UCI rate matching with segmentation	Huawei Technologies Co. Ltd.

Decision: The text proposals in Section 2.4 of R1-1807042 are endorsed.

Agreement:

In the determination of EUCI, a UE assumes the number of CRC bits to be 11 when the payload size.

FFS: Further check if a similar correction is needed to 36.213 to the procedure for selection of PUCCH resource and the dropping of CSI.

R1-1807405	Remaining Issue of Channel Coding	Ericsson Inc.

R1-1807828	Text Proposal for Polar Code Rate Matching	Ericsson

### NR-LTE co-existence

Including architecture option 4 related issues for both NR and LTE, if any

R1-1807787	Chairman's notes of AI 7.1.5 NR-LTE co-existence Ad-Hoc chair (Ericsson)

Decision: Contents incorporated below (see also AI 5), the document is endorsed.

R1-1805896	Discussion on remaining issues for SUL and EN-DC	Huawei, HiSilicon

R1-1806302	Dynamic Power Sharing for NE-DC	CATT

R1-1806631	Remaining issues for NR LTE coexistence	LG Electronics

R1-1806742	Corrections on LTE-NR Coexistence	Samsung

R1-1807004	Single Tx switched UL support for EN-DC in LTE TDD band	Apple Inc.

R1-1807264	On remaining issues of NR-LTE co-existence	Ericsson

R1-1807281	On single Tx transmission timing in TDD LTE-NR D	Nokia, Nokia Shanghai Bell

R1-1807371	Remaining issues on coexistence	Qualcomm Incorporated

R1-1806890	Correction on scheduling and HARQ procedure for case 1 HARQ timing of EN-DC in TS36.213	Huawei

Decision: The text proposal in R1-1806890 is endorsed. The editor to incorporate the TP into TS36.213.

R1-1807693	Proposals on LTE-NR coexistence	Qualcomm

R1-1807865	Proposals on LTE-NR coexistence	Qualcomm

R1-1807744	Draft LS on switching times	Ericsson

R1-1807816	[Draft] LS on LTE-NR switching time for ‘Single UL operation’	Ericsson

Revision of R1-1807744

Conclusion:

- Current RAN1 assumption is that that the switching between LTE and NR on different carrier frequencies for EN-DC operation is negligible for the cases below

  - Intra-band TDD EN-DC band combination described in TS38.101-3 where ‘single UL allowed’ is applicable

  - Intra-band FDD EN-DC band combination described in TS38.101-3 where ‘single UL allowed’ is applicable

  - Inter-band EN-DC band combination described in TS38.101-3 where ‘single UL allowed’ is applicable

- RAN1 assumes RAN4 is discussing switching times for the above cases

R1-1807713	WF on Single Tx in EN-DC with TDD PCell	Apple

Conclusion:

When the UE is configured with multiple UL carriers on different frequencies, but the UE operates only on one of the UL carriers at a given time among a pair of LTE and NR carriers, LTE operation with an LTE TDD PCell using the same HARQ timing as LTE operation without EN-DC is supported by the current RAN1 specifications at least when no LTE UL CA is used at least for the following cases

- The UE supports dynamic power sharing

- The UE is configured with P_LTE + P_NR not greater than P_cmax

### UL power control

R1-1807788	Chairman’s notes for AI UL power control	Ad-hoc chair (Samsung)

Decision: Contents incorporated below, the document is endorsed.

R1-1805821	LS reply to RAN1 on P_0 ranges on UL power control	RAN4, ZTE

#### NR UL power control - non-CA aspects

R1-1807653	Summary for Al 7.1.6.1 NR UL power control in non-CA aspects	ZTE

Agreement

For large payload size for delta_TF,c, K1 = 2.4

Agreement

For small payload size for delta_TF,c, K2 = 6.0 [linear]

Agreement

The following working assumption is confirmed

For group-common TPC command in DCI format 2-2,

One bit closedloopindex field is present along with the TPC command in the DCI format 2_2 to indicate which closed loop the TPC command applies to

- When a UE is configured with 1 closed loop, the bit closedloopindex field is NOT present.

- When a UE is configured with 2 closed loops, the bit location of closedloopindex field follows the 2 bit TPC command.

Notes: UE assumes that the start bit of the TPC command is X*tpc_index, where X=3 if configured with 2 closed-loop process, otherwise X=2.

Conclusion

No further RAN1 specification support for additional cases of resetting closed-loop accumulation in Rel-15.

R1-1807710	Summary for Al 7.1.6.1 NR UL power control in non-CA aspects	ZTE

Agreement

For PUSCH scheduled by DCI format 0_0, if the PUSCH transmission uses the spatial relation for a PUCCH resource (i.e., as agreed in RAN1#92bis).

- The UE shall use the PL reference associated with the PUCCH resource whose spatial relation is used for the PUSCH transmission;

- P0 and alpha are given by, the values corresponding p0alphasetindex=0 of p0-pusch-alpha-setconfig (i.e., j=2).

- PUSCH closed loop index l = 0.

Agreement:

K value for non-scheduled UL transmission is the minimum of the common configured K2 values of the associated BWP.

- Applies for both PUSCH and SRS

R1-1807800	Summary 3 for Al 7.1.6.1 NR UL power control in non-CA aspects	ZTE

Working Assumption

For PUCCH, K value for non-scheduled UL transmission is the minimum of the common configured K2 values

Working Assumption

For group common TPC

- If group TPC commands for PUSCH are received the K symbols before PUSCH transmission period i, the accumulation is updated according to all the group common TPC commands;

- If group TPC commands for PUCCH are received the K symbols before PUCCH transmission period i, the accumulation is updated according to all the group common TPC commands;

- If group TPC commands for SRS not tied with PUSCH are received the K symbols before SRS transmission period i, the accumulation is updated according to all the group common TPC commands;

Notes: How to capture the above is up to editor, especially for the time unit of i and K.

R1-1805837	Remaining issues on NR power control in non-CA aspects	ZTE

R1-1805963	Remaining details of UL power control design	Huawei, HiSilicon

R1-1806075	Remaining issues on non-CA UL power control	vivo

R1-1806303	Remaining issues of Non-CA based power control	CATT

R1-1806370	Discussion on power control signalling	CMCC

R1-1806526	Remaining Issues on Power Control Framework	Intel Corporation

R1-1806632	Remaining issues on non-CA aspect of UL power control	LG Electronics

R1-1806743	Corrections on UL Power Control	Samsung

R1-1807626	Remaining issues of UL power control	MediaTek Inc.	(R1-1806802)

R1-1806829	Remaining issues on uplink power control for non-CA	OPPO

R1-1807017	Discussion on Pathloss Update during Beam Switch 	InterDigital, Inc.

R1-1807151	Remaining issues on group common TPC command	NTT DOCOMO, INC.

R1-1807265	Remaining issues for NR power control	Ericsson

R1-1807278	Remaining Details on non-CA NR UL power control	Motorola Mobility, Lenovo

R1-1807372	Remaining issues on power control for NR	Qualcomm Incorporated

R1-1807404	Remaining details on NR PUSCH power control	Nokia, Nokia Shanghai Bell

#### NR UL power control – CA aspects

R1-1807660	Summary of NR UL power control – CA/DC aspects	Samsung

Agreement

In case of UL CA with different SCSs, when the slot of the UL transmission carrying the PHR overlaps with the slot on CC with higher SCS, the PH of the first slot with higher SCS that fully overlaps with the slot with the lower SCS is reported.

Agreement

RAN1 changes the text in TS38.213 to be consistent with TS38.321 on how UE determines whether PH value for an activated Serving Cell is based on real transmission or a reference format. The text from TS38.321 is provided below:

“UE determines whether PH value for an activated Serving Cell is based on real transmission or a reference format by considering the downlink control information which has been received until and including the PDCCH occasion in which the first UL grant is received since a PHR has been triggered.”

Working Assumption

For CA, when calculating the PHR of a cell based on a real transmission, transmissions whose DCI come after the first DCI after PHR trigger are ignored.

Conclusion

Editor to note the text proposal below which was agreed in RAN1#92.

To capture the following text proposal:

---------------- Start TP for 38.213 sub clause 7.6.1 ----------------------------------

If the UE indicates a capability for dynamic power sharing between EUTRA and NR and

-	if the UE is not configured for operation with shortened TTI and processing time on the MCG [13, TS 36.213], and

-	if the UE transmission(s) in subframe  of the MCG overlap in time with UE transmission(s) in slot  of the SCG, and

-	if  in any portion of slot  of the SCG,

the UE reduces transmission power in any portion of slot  of the SCG so that  in any portion of slot , where  and  are the linear values of the total UE transmission powers in subframe  of the MCG and in slot  of the SCG, respectively.

-------------------- End TP ----------------------------------------------------------------

Working Assumption

Cross-carrier indication for power control parameters

- For PUCCH, if higher layer parameters “cell” and “bwp-Id” are included in the PUCCH-SpatialRelationInfo configuration:

  - The reference signal corresponding to pathloss index ‘q_d’ included in PUCCH-SpatialRelationInfo is from the serving cell and active BWP indicated by the higher layer parameters “cell” and “bwp-Id”.

Note: No RRC specification impact.

Agreement

Selection of UL carrier for virtual PUSCH PHR

- For the PC setting for virtual PUSCH PHR in case of CA/DC, for a cell configured with non-SUL and SUL, a default UL carrier is selected according to the following rules:

  - Semi-statically configured carrier for PUSCH if present, otherwise

    - The semi-statically configured carrier for PUCCH if present, otherwise the non-SUL carrier

Conclusion

- PUCCH-SCell is included in ‘a secondary cell’ and no update of TS38.213 is needed.

R1-1806527	Further discussion on power sharing for EN-DC	Intel Corporation

R1-1806633	Remaining issues on CA aspect of UL power control	LG Electronics

R1-1806744	Corrections on UL Power Control for EN-DC	Samsung

R1-1807136	Remaining details of UL power control design for CA	Huawei, HiSilicon

R1-1807163	Virtual PHR derivation in some situations	ASUSTEK COMPUTER (SHANGHAI)

R1-1807266	Remaining issues for LTE-NR power sharing	Ericsson

R1-1807279	Remaining Details on CA-related NR UL power control	Motorola Mobility, Lenovo

R1-1807373	Remaining Issues on power control for NR CA	Qualcomm Incorporated

#### Other

R1-1806745	Corrections on PHR calculation for CA	Samsung

R1-1806889	Discussion on PHR for SUL	Huawei, HiSilicon

R1-1806965	Power Control for NE-DC (Option 4/4a)	InterDigital, Inc.

R1-1806966	Timing and Asynchronous Transmissions with Power Control for NR DC	InterDigital, Inc.

R1-1806967	Power Control for NR DC	InterDigital, Inc.

R1-1807135	Independent Power control for SRS	Huawei, HiSilicon

R1-1807267	PUSCH power scaling in UL power control	Ericsson

R1-1807268	SRS power scaling in UL power control	Ericsson

R1-1807269	Remaining issues on closed loop power control	Ericsson

### UE features

R1-1807789	Chairman's notes of AI 7.1.7 UE feature	Ad-Hoc chair (NTT DOCOMO, INC., AT&T)

Decision: Contents are incorporated below. The document is endorsed with updates.

R1-1807847	Draft reply LS on Type 3 UE capabilities	NTT DOCOMO, AT&T

Email discussion to conclude Type 3 UE capabilities and to approve reply LS by 5/31

R1-1807846	Updated NR UE features	NTT DOCOMO, AT&T

R1-1807845	Draft LS on updated NR UE features	NTT DOCOMO, AT&T

Friday

R1-1807890	Draft LS on updated NR UE features	NTT DOCOMO, AT&T

Decision: The draft LS is endorsed and final LS is approved in R1-1807896.

R1-1807891 	Updated NR UE features	NTT DOCOMO, AT&T

Discussion: Intel concerns regarding 4-25, not clear whether the feature is mandatory/optional

Nokia: mandatory

Decision: The list is agreed. Note in R1-1807891 that the editorial change for entry 2-33a should be reverted. However, this change will be reflected in RAN1#94.

R1-1805897	Remaining issues of NR UE features	Huawei, HiSilicon

R1-1807282	On UE features 3-1, 5-1, 5-1a, 6-1, 6-2 and capability to FDM slot and mini-slot	Nokia, Nokia Shanghai Bell

Continue offine discussion on 5-1

Friday: Discuss in RAN plenary

Note: Current discussion so far considers only UE processing capability #1.

Agreement:

For the candidate set value of FG 2-25:

X1={2, 4, 8}, X2= {4, 8, 14}, X4={14, 28, 56}

Candidate value set for X3 is {8, 14, 28}

Agreement:

For 2-28, remove SCS 15 and 30

Agreement

For 2-30, change the candidate value set for component-2:

Component-2, candidate value set is {2, 4, 8, 16}

Agreement

For 2-54a, change the component-1 to:

1. Maximum number of simultaneous transmitted SRS resources per CC at one symbol for NCB PUSCH

Agreement

For 2-14, add a new component:

“Supported max number of SRS resource per set (SRS set use is configured as for codebook).”

With Candidate value: {1, 2}

Agreement

| 2-4 | TCI states for PDSCH | 1. Support number of active TCI states per BWP per CC, including control and data 2. maximum number of configured TCI states per CC for PDSCH | 2-1 | Yes | Only one TCI state can be supported | Type 1 | N.A. | N.A. |  |  |  | Component-1: Candidate value set: {1, 2, 3, 4, 8} Component-2: candidate value set: {4, 8, 16, 32, 64} FFS: mandatory value for component-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-4a | Additional active TCI state for PDCCH | Support one additional active TCI state for control in addition to the supported number of active TCI states for PDSCH | 2-1 | Yes | No additional TCI state for control | Type 1 | N.A. | N.A. |  | Note: Only applicable if Component-1 of 2-4 is set to 1 |  | Mandatory with capability signalling |

Agreement

On 2-1, add a new component as below (for mandatory without capability )

3. Support one active TCI state

Agreement:

For each of Type I, Type I multi-panel, Type II, Type II with port selection codebooks, clarify that the number of CSI-RS resources and total number of ports used are counted multiple times (n times) if the same resource or port is used in multiple (n) simultaneous CSI calculations.

Add a note to clarify the above agreement in the feature table.

Agreement:

For FG2-28, candidate value set:

Alt-1: {14, 28, 48}

Alt-2: {“14-14”, “28-28”, “48-48”, “14-336”, “28-336”, “48-336”}, where the first threshold corresponding to the case when UE doesn’t activate antenna panel (using same panel to receive DCI and CSI-RS), and the second threshold corresponding to the case when UE may activate antenna panel (other than the panel used to receive DCI)

Note: Alt-2 requires some spec change to support more than 4 slots offset between DCI and A-CSI-RS transmission.

Discuss additional spec requirements for Alt-2 in main session in RAN1#93, if support more than 24 slot offsets between DCI and A-CSI-RS transmission are supported, Alt.2 is agreed, otherwise Alt.1 is agreed.

Friday: Discuss in RAN plenary

Agreement:

For FG 2-33, candidate value set for component-2:

{2, 4, 8, 12, 16 : 8 : 256}

Agreement

For 2-15a, 2-53, 2-55, 2-57, keep the signalling granuality as 4) per band per band comb.

Agreement:

For 2-36, 2-40, 2-41 and 2-43, maximal size of the list is 16

Agreement:

For 2-36, Component-2 candidate set is

{“Mode-1 only”, “Mode-1 and Mode-2”}.

Agreement

For 2-36, 2-40, 2-41, and 2-43: add a new component:

- Max # of CSI-RS resource in a resource set, FR1/FR2 differentiation: candidate value is {1:8}

Remove 2-39

Agreement

For 2-41, candidate value set for component-4 is:

{“no amplitude subset restriction”, “support amplitude subset restriction”}

Agreement

For 2-54,

- Remove 2-54 from UE feature list.

- Specify the minimum time interval, N in unit of symbols, between DCI triggering and A-SRS transmission is N2+42.

Agreement

Add a new FG

|  | 2-51a | Aperiodic TRS | DCI triggering Aperiodic TRS associated with periodic TRS | 2-50 | Yes | A-TRS is not supported | Type 1 | N.A. | Yes |  |  |  | Optional with capability signaling |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

2-51a is differentiated by FR1 and FR2.

Proposal:

Add a new FG as below:

| 2-33b | SP CSI-RS | 1. Support SP CSI-RS | 2-1 | Yes |  | Type 4 | No | Yes |  |  | RAN1 | FFS on the mandatory or optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-33c | SP CSI-IM | 2. Support SP CSI-IM | 2-1 | Yes |  | Type 4 | No | Yes |  |  | RAN1 | FFS on the mandatory or optional |

Make final decision in the main session:

On 2-50: TRS bandwidth configuration

Atl.1 BWP;

Alt.2: min(52,BWP)

Alt.3: Both

Friday

Agreement:

- On 2-50: TRS bandwidth configuration = BWP or min(52, BWP)

  - TRS bandwidth configuration does not imply UE processing bandwidth

R1-1806239	Discussion on UE capability list	Ericsson

R1-1806528	On Rel-15 UE feature list	Intel Corporation

R1-1806746	UE Features	Samsung

R1-1807616	NR Features and Capabilities	Qualcomm Incorporated

Revision of R1-1807374

Late submission

R1-1806076	Discussion on Rel-15 NR UE features	vivo

R1-1806184	Discussion on multi-slot PUCCH and PUSCH supporting	Panasonic Corporation

R1-1806792	Discussion on NR UE Feature List	MediaTek Inc.

R1-1806973	Further details on NR UE Feature List	AT&T

R1-1807072	Further views on NR UE features	NTT DOCOMO, INC.

R1-1807178	Consideration on UE feature for DL MIMO layers	China Unicom

R1-1807375	Signaling aspects for UE capabilities	Qualcomm Incorporated

R1-1807618	On selected MIMO UE features	Nokia, Nokia Shanghai Bell

R1-1807761	Remaining issues for MIMO feature list	Qualcomm

### Other

R1-1806240	UE Feature Band Combination Signalling Granularity	Ericsson

## NR URLLC in Release 15

Friday

Editor to prepare CRs: drafts to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

### Support of separate CQI and MCS table(s) for URLLC

Focus on:

- N separate CQI table(s) are supported for URLLC

  - Downselect the value of N between 1 or 2

- Two target BLER are supported for URLLC

  - Note: RRC signalling is used by gNB to select one of the two target BLER

  - Note: The configuration of target BLER or CQI table is part of CSI report setting

R1-1805898	MCS and CQI design for URLLC	Huawei, HiSilicon

Decision: The document is noted.

R1-1806004	CQI and MCS Tables for URLLC	Ericsson

Decision: The document is noted.

R1-1806137	Considerations on CQI /MCS table(s) for URLLC	ZTE

Decision: The document is noted.

R1-1806529	CQI/MCS enhancements for URLLC	Intel Corporation

Decision: The document is noted.

R1-1806747	CQI and MCS Tables for URLLC	Samsung

Decision: The document is noted.

R1-1807376	Considerations for CQI and MCS for URLLC	Qualcomm Incorporated

Decision: The document is noted.

Agreements:

- For CQI table for URLLC with target BLER of 10^-5,

  - At least some entries for CQI table for target BLER of 10^-1 are re-used

  - The highest SE entry is 772/1024*6

  - The lowest SE entry is between (30 ~ 40)/1024*2

Agreements:

- The MCS table for CP-OFDM based PUSCH is separate from the MCS table for DFT-s-OFDM based PUSCH

- The same MCS table is used for PDSCH and CP-OFDM based PUSCH

- At least one new MCS table is introduced for URLLC

Discuss further offline – Yufei (Ericsson)

Wednesday session

R1-1807730	Offline Discussion on Support of Separate CQI and MCS table(s) for URLLC	Ericsson

Decision: The document is noted.

Agreement:

- The CQI table for URLLC with target BLER of 10^-5 is composed of 16 entries, including one entry of “out of range”.

Agreements:

- In CSI report, RRC parameter cqi-Table implicitly provides the BLER target for CQI reporting.

  - New 64QAM table implies BLER target =10^-5 only.

  - Existing 64QAM table and 256QAM table imply BLER target =10^-1 only.

- From RAN1 perspective, RRC parameter bler-Target is not to be used by RAN1 specification in Rel-15. Thus, from RAN1 perspective, this parameter can be removed from 38.331 in Rel-15

  - Spec update is necessary since the parameter already appears in RAN1 specs

- Send an LS to RAN2 to inform the above agreements – R1-1807743

Friday

R1-1807822	[Draft] LS on RRC Parameters for NR URLLC	Ericsson	(rev. of R1-1807743)

Decision: To prepare RRC parameter list in spreadsheet to send to RAN2 (R1-1807866)

R1-1807874	[Draft] LS on RRC Parameters for NR URLLC	Ericsson	(rev. of R1-1807822)

Decision: LS is for email approval till 5/31 – Yufei (Ericsson)

Agreement:

- Lowest SE entry for the new CQI table is 30/1024 * 2

Agreement:

- The CQI table for URLLC with target BLER of 10^-5 is:

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 30 | 0.0586 |
| 2 | QPSK | 50 | 0.0977 |
| 3 | QPSK | 78 | 0.1523 |
| 4 | QPSK | 120 | 0.2344 |
| 5 | QPSK | 193 | 0.3770 |
| 6 | QPSK | 308 | 0.6016 |
| 7 | QPSK | 449 | 0.8770 |
| 8 | QPSK | 602 | 1.1758 |
| 9 | 16QAM | 378 | 1.4766 |
| 10 | 16QAM | 490 | 1.9141 |
| 11 | 16QAM | 616 | 2.4063 |
| 12 | 64QAM | 466 | 2.7305 |
| 13 | 64QAM | 567 | 3.3223 |
| 14 | 64QAM | 666 | 3.9023 |
| 15 | 64QAM | 772 | 4.5234 |

Agreement:

- For both CP-OFDM and DFT-s-OFDM, the new MCS table for URLLC has a size of 32 entries, and the MCS information field in DCI is 5-bit.

Agreements:

- For CP-OFDM, the 32-entry MCS table for URLLC includes at least 3 reserved entries for modulation order.

  - FFS if more reserved entries are needed

- For DFT-s-OFDM, the 32-entry MCS table for URLLC includes at least 4 reserved entries for modulation order (pi/2-BPSK, QPSK, 16QAM, 64QAM).

  - FFS if more reserved entries are needed

- Regarding pi/2-BPSK support for PUSCH, the same signalling and procedure are used for URLLC and eMBB.

Agreement:

- The lowest SE entry in the new MCS table is the same as the lowest SE entry of the CQI table for BLER target of 10^-5.

Discuss further offline regarding whether or not to address the discrepancy in the higest SE(s) in designing MCS table vs. the target packet BLER (e.g., 10^-5)

Discuss further offline whether or not there is a connection between the new CQI table vs. the new MCS table, e.g.,

- Alt 1: the new MCS table can only be used when the new CQI table is configured

  - Qualcomm, AT&T

- Alt 2: the new MCS table can be used even when the new CQI table is NOT configured

  - Ericsson, Nokia, Samsung, vivo, Intel, CATT, MediaTek, NTT DOCOMO, Sharp, Huawei

Also consider UE capability aspects

Proposals:

- For CP-OFDM of URLLC, the MCS table design and signalling is selected from one of the options below.

- Option 1: Two MCS tables are used for URLLC.

  - The first MCS table reuses the existing 64QAM MCS table of eMBB.

  - The second MCS table is newly introduced for URLLC.

  - Signalling is selected from the following:

    - Option 1-A: RRC signalling is used to select the MCS table. The RRC parameter is extension of existing RRC parameter.

      - Supported by (6): Ericsson, Nokia, Intel, CATT, Samsung, MediaTek, Sharp

    - Option 1-B: Dynamic signalling is used to select the MCS table.

      - NOTE: This option requires a new mechanism to provide dynamic signalling.

      - Supported by (5): Samsung, InterDigital, AT&T, OPPO, Huawei

- Option 2: Only one MCS table is used for URLLC. The MCS table is newly introduced for URLLC.

  - Signalling is selected from the following:

    - Option 2-A: RRC signalling is used to select the eMBB vs URLLC MCS table

      - Supported by (3): Huawei, Qualcomm, ZTE

    - Option 2-B: Dynamic signalling is used to select the eMBB vs URLLC MCS table

      - Supported by (4): AT&T, Qualcomm, Huawei, InterDigital

Offline discussion to finalize the MCS tables for CP-OFDM & DFT-s-OFDM

Thursday session

R1-1807742	Summary of offline discussion on eMBB and URLLC data differentiation	Nokia, Nokia Shanghai Bell

Decision: The document is noted.

R1-1807748	Offline Discussion on Support of Separate CQI and MCS table(s) for URLLC	Ericsson

Decision: The document is noted.

Agreement:

- For PDSCH and PUSCH with CP-OFDM, one new MCS table is introduced for URLLC:

| MCS Index | Modulation Order | Code rate R × 1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 30 | 0.0586 |
| 1 | 2 | 40 | 0.0781 |
| 2 | 2 | 50 | 0.0977 |
| 3 | 2 | 64 | 0.1250 |
| 4 | 2 | 78 | 0.1523 |
| 5 | 2 | 99 | 0.1934 |
| 6 | 2 | 120 | 0.2344 |
| 7 | 2 | 157 | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 4 | 340 | 1.3281 |
| 16 | 4 | 378 | 1.4766 |
| 17 | 4 | 434 | 1.6953 |
| 18 | 4 | 490 | 1.9141 |
| 19 | 4 | 553 | 2.1602 |
| 20 | 4 | 616 | 2.4063 |
| 21 | 6 | 438 | 2.5664 |
| 22 | 6 | 466 | 2.7305 |
| 23 | 6 | 517 | 3.0293 |
| 24 | 6 | 567 | 3.3223 |
| 25 | 6 | 616 | 3.6094 |
| 26 | 6 | 666 | 3.9023 |
| 27 | 6 | 719 | 4.2129 |
| 28 | 6 | 772 | 4.5234 |
| 29 | 2 | reserved |  |
| 30 | 4 |  |  |
| 31 | 6 |  |  |

Agreement:

- For PUSCH with transform precoding, one new MCS table is introduced for URLLC:

| MCS Index IMCS | Modulation Order Qm | Code rate R × 1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 60/q | 0.0586 |
| 1 | q | 80/q | 0.0781 |
| 2 | q | 100/q | 0.0977 |
| 3 | q | 128/q | 0.1250 |
| 4 | q | 156/q | 0.1523 |
| 5 | q | 198/q | 0.1934 |
| 6 | 2 | 120 | 0.2344 |
| 7 | 2 | 157 | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 2 | 679 | 1.3262 |
| 16 | 4 | 378 | 1.4766 |
| 17 | 4 | 434 | 1.6953 |
| 18 | 4 | 490 | 1.9141 |
| 19 | 4 | 553 | 2.1602 |
| 20 | 4 | 616 | 2.4063 |
| 21 | 4 | 658 | 2.5703 |
| 22 | 4 | 699 | 2.7305 |
| 23 | 4 | 772 | 3.0156 |
| 24 | 6 | 567 | 3.3223 |
| 25 | 6 | 616 | 3.6094 |
| 26 | 6 | 666 | 3.9023 |
| 27 | 6 | 772 | 4.5234 |
| 28 | q | reserved |  |
| 29 | 2 |  |  |
| 30 | 4 |  |  |
| 31 | 6 |  |  |

Agreement:

- Regarding configuration of CQI reporting and MCS table:

  - The new MCS table can be used even when the new CQI table is NOT configured

Proposals:

- For URLLC, for MCS table signalling for unicast,

  - For grant-based scheduling, dynamic signalling is used to choose MCS table, which is based on a RNTI for URLLC

    - The RNTI is based on “111111” + C-RNTI for the new table, while “000000”+C-RNTI is for the legacy table

  - For grant-free based scheduling (both initial & re-transmissions), by RRC configuration

- For URLLC, for the case of grant-based scheduling, select from one of the options below.

- Option A. by RRC configuration

  - Supported by (8): Ericsson, Nokia, Intel, CATT, Samsung, MediaTek, Sharp, LG

- Option B. Dynamic signalling is used to choose MCS table, which is based a RNTI for URLLC

  - Supported by (11): Qualcomm, Huawei, ZTE, AT&T, InterDigital, OPPO, Spreadtrum, CMCC, Nokia, Orange, Fujitsu

Friday session

R1-1807823	Offline Discussion on Support of Separate CQI and MCS table(s) for URLLC	Ericsson

Decision: The document is noted.

Agreements:

- For URLLC, for grant-based transmissions, introduce one RRC parameter for configuring a new RNTI.

  - When the new RNTI is not configured, existing RRC parameter mcs-table is extended to select from 3 MCS tables (existing 64QAM MCS table, existing 256QAM MCS table, new 64QAM MCS table).

    - When mcs-table indicates the new 64QAM MCS table:

      - For DCI format 0_0/1_0 in CSS, existing 64QAM MCS table is used.

      - For DCI formats 0_0/1_0/0_1/1_1 in USS, new 64QAM MCS table is used.

    - Otherwise, follow existing behaviour.

    - Note: the configuration for DL and UL is separate

  - When the new RNTI (via RRC) is configured, RNTI scrambling of DCI CRC is used to choose MCS table:

    - If the DCI CRC is scrambled with the new RNTI, the new 64QAM MCS table is used; otherwise, follow existing behaviour.

Intel: concerns about impact of introducing new RNTI on false alarm probability increase compared with eMBB

vivo: share the same concern as above

Agreements:

For both initial transmission and re-transmissions for GF scheduling for URLLC,

- For UL configured grant, the MCS table is configured by the existing parameter associated with the RRC configured grant configuration, which is extended to include the new 64QAM MCS table.

- For DL SPS, RRC indicates whether or not the new 64QAM table is configured. The indication for the new MCS table for DL SPS is separate from the one for grant-based DL scheduling.

R1-1807885	Summary of offline discussion on capturing URLLC related agreements	Nokia, Nokia Shanghai Bell

Agreement:

- For Rel-15, capture the functionalities of all the agreements made for URLLC generically in RAN1 specs (i.e. without mentioning “URLLC”).

R1-1806077	Discussion on CSI report for URLLC	vivo

R1-1806140	On details of CQI and MCS tables for NR URLLC	Panasonic

R1-1806304	CQI and MCS design for NR URLLC	CATT

R1-1806371	Discussion on CQI and MCS design for URLLC	CMCC

R1-1806409	Consideration on CQI and MCS tables for NR URLLC	Spreadtrum Communications

R1-1806634	Discussion on CQI and MCS table design for URLLC	LG Electronics

R1-1806658	Remaining details of CQI and MCS for URLLC	Nokia, Nokia Shanghai Bell

R1-1806804	On CQI and MCS design for URLLC	MediaTek Inc.

R1-1806977	CQI/MCS Reporting for NR URLLC	AT&T

R1-1807018	CQI and MCS Tables for NR URLLC 	InterDigital, Inc.

### Other

Including any aspects not covering 7.2.1 and 7.1.3

R1-1806006	Channel Coding Techniques for URLLC	Ericsson

R1-1806007	On PUCCH for Ultra-Reliable Transmission	Ericsson

R1-1806008	On Performance of PUCCH Format 0 for URLLC Use Cases	Ericsson

R1-1806009	On Performance of PUCCH Format 2 for URLLC Use Cases	Ericsson

R1-1806010	On Frequency Hopping for Ultra-reliable Transmission	Ericsson

R1-1806011	On Transmit Diversity for Ultra-high Reliability Use Cases	Ericsson

R1-1806012	URLLC Downlink System Level Simulation Results	Ericsson

R1-1806013	On Supporting Reliable HARQ Feedback for UL Transmission with Configured Grant	Ericsson

R1-1806014	UCI Transmission for URLLC	Ericsson

R1-1806015	Evaluation of Reliability in NR	Ericsson

R1-1806016	Evaluation of UP latency in NR	Ericsson

R1-1806017	Indication of URLLC Configuration	Ericsson

R1-1806018	Precoder Update for SPS UL	Ericsson

R1-1806078	Other aspects for URLLC	vivo

R1-1806138	Discussion on URLLC control	ZTE

R1-1806179	Discussion on uplink power control for NR URLLC	Panasonic Corporation

R1-1806530	On UE features for URLLC	Intel Corporation

R1-1806659	On Intra-UE UL multiplexing between eMBB and URLLC	Nokia, Nokia Shanghai Bell

R1-1806813	On UL Multiplexing for URLLC	MediaTek Inc.

R1-1806847	Study on URLLC scenario and UE	OPPO

R1-1806896	On evaluation assumptions for URLLC and the need of enhanced schemes	Huawei, HiSilicon

R1-1806919	Reliability and latency problems with existing grant free transmission	Sequans Communications

R1-1806921	Considerations for PUCCH reliability	Sequans Communications

R1-1806957	On enhancements for eMBB/URLLC multiplexing in DL	Sequans Communications

R1-1806958	On PDCCH repetition for NR URLLC	Sequans Communications

R1-1806959	On UL multiplexing between grant-based URLLC and eMBB	Sequans Communications

R1-1806978	On  the repetition of NR PDCCH for URLLC	AT&T

R1-1807140	Intra-UE UL multiplexing with different reliability requirements	III

R1-1807228	Discussion on UL pre-emption for URLLC reliability	Potevio

R1-1807294	Discussions on intra-UE multiplexing between URLLC and eMBB	Mitsubishi Electric Co.

R1-1807326	Discussion on the Reliability Enhancement for Grant-free Transmission	China Telecommunications

R1-1807406	Discussion on Multiplexing UL Transmissions with Different Reliabilities	TCL Communication Ltd.

## Study on enhanced NR to support Non-Terrestrial Networks - SID in RP-171450

R1-1807790	Chairman’s notes of AI 7.3 Study on enhanced NR to support NTN	Ad-hoc chair (Samsung)

Decision: Contents incorporated below, the document is endorsed.

### Channel Model

Agreement

In rural environment, consider the K-factor, clutter loss and SF values from suburban environment. In urban environment, consider the K-factor, clutter loss and SF values from suburban/dense urban environment

Agreement

Reuse cluster parameters from TR38.901, with the below adjustments

- The number of clusters for suburban environment is

| Scenarios | LOS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10° | 20° | 30° | 40° | 50° | 60° | 70° | 80° | 90° |
| Number of clusters | 6 | 5 | 4 | 4 | 4 | 3 | 4 | 4 | - |

R1-1807792	NR-NTN - Text proposal for TR 38.811 chap 6 NTN channel modelling	Thales

R1-1807803	NR-NTN - Text proposal for TR 38.811 chap 6 NTN channel modelling	Thales, HNS, Huawei, Hisilicon, Nokia, ZTE, ESA, Dish

Decision: Text proposal in R1-1807803 is agreed.

R1-1807793	[Draft] Liaison Statement to RAN about TP for TR38.811 chapter 6	Thales

Decision: The draft LS is endorsed and final version is approved in R1-1807719 with R1-1807803 as an attachment for information

R1-1805903	Text Proposal for TR38.811	Huawei, HiSilicon

R1-1806155	Discussion on the channel model for sub-urban	ZTE,CATR

R1-1806467	TP for Section 6 of TR38.811 – “Rain + Scintillations”	ESA, THALES

R1-1806748	Remaining issues on NTN Channel Modeling	Samsung

R1-1806969	A Fast Fading Model for S-Band Satellites in GOOD Link State	Dish Network

R1-1806993	Text proposal for LOS probability, path loss and shadow fading in TR38.811	Nokia, Nokia Shanghai Bell

R1-1807019	Rain and Cloud Attenuation in NTN Channel Model 	InterDigital, Inc.

R1-1807165	NR-NTN Channel Modeling - TP for methodology, atmospheric absorption, fast fading model and calibration	THALES

R1-1807714	NR-NTN Channel Modeling - Summary of offline discussions	Thales

R1-1807715	NR-NTN Channel Modeling - TP for remaining sections of the 6th clause of TR38.811	Thales

### Other

R1-1805847	Impact of Doppler Shift on Time Density of DMRS in NTN	Nokia, Nokia Shanghai Bell, Thales

R1-1805904	HAPS channel modeling	Huawei, HiSilicon

R1-1805905	LEO/GEO channel modelling	Huawei, HiSilicon

R1-1805984	Phase noise correction for non-terrestrial networks	Mitsubishi Electric RCE

R1-1806003	TDD and FDD for NTN Applications	HUGHES Network Systems Ltd

R1-1806023	Discussion of PTRS for NR Supporting NTN	Nokia, Nokia Shanghai Bell

R1-1806094	Impact of long propagation to control loops	HUGHES Network Systems Ltd

R1-1806095	Satellite Payload Characteristics and its performance impact on NTN signals	HUGHES Network Systems Ltd

R1-1806156	TP for TR38.811 on random access for NTN	ZTE

R1-1806157	TP for TR38.811 on additional modelling components	ZTE,CATR

R1-1806472	NR-NTN: Impact of Doppler variation rate on DM-RS time distribution	THALES, Nokia, Nokia Shanghai Bell

R1-1806473	NR-NTN: Proposed recommendations	THALES

R1-1806475	NR-NTN: Impact of channel frequency selectivity on DM-RS distribution in frequency	THALES

R1-1806476	NR-NTN: Impact on Initial TA during random access procedure	THALES

R1-1806749	PT-RS consideration for NTN	Samsung

R1-1806750	Considerations on random access for NTN	Samsung

R1-1806768	Considerations on Timing Advance and Random Access for NTN	Nokia, Nokia Shanghai Bell

R1-1807020	Simulation Results for PTRS Performance in NTN	InterDigital, Inc.

R1-1807164	NR-NTN Channel Modeling - Flat fading criteria	THALES

R1-1807243	Discussions on handover in NTN	Sony

R1-1807665	Consideration on HARQ Impact for NTN	Nokia, Nokia Shanghai Bell

R1-1807673	Updated Ray Tracing simulation result for NTN	Huawei, HiSilicon

R1-1807794	Text proposal for TR 38.811 chap 7.3	Thales

R1-1807795	Text proposal for TR 38.811 chap 8	Thales

R1-1807864	NR-NTN: solution principles for NR to support Non-Terrestrial networks	Thales

## Study on non-orthogonal multiple access for NR - SID in RP-171043

R1-1805839	Text proposal to capture agreements made in RAN1#92bis to TR 38.812 (NOMA)	ZTE

Decision: The TP is endorsed.

### Transmitter side signal processing schemes for NOMA

R1-1805840	Key processing modules at transmitter side for NOMA	ZTE

- Observation 1: Low code rate is beneficial when the overloading is not very high, while low inter-user interference is beneficial for high overloading cases.

- Observation 2: Multi-layer transmission can be applied for very high per UE spectral efficiency when the number of UEs is not very large.

- Observation 3: Unequal power assignment with user grouping can facilitate SIC receiver to achieve better performance under high overloading cases.

- Proposal 1: Capture the listed key modules and the MA signature designs in the TR38.812.

- Proposal 2: Trade-off of spreading factor between per UE performance and overloading capability should be considered for the design of symbol-level linear spreading based NOMA.

- Proposal 3: Trade-off of number of layer per UE between per UE performance and overloading capability should be considered for the design of symbol-level linear spreading based NOMA.

- Proposal 4: Support symbol-level linear spreading and multi-layer transmission for NOMA transmitter processing.

Decision: The document is noted.

R1-1806531	NOMA transmission scheme: LCRS	Intel Corporation

- Observation 1: Low Code Rate Spreading (LCRS) is a promising candidate for NOMA from the perspectives of

  - Support of flexible code rate,

  - Support of (comparatively) with full coding gain

  - Support of NOMA without introducing new transmission schemes,

- Observation 2:

  - No (or negligible) specification impact is needed for the LCRS transmission scheme

  - Possible improvement could be needed for PUSCH DMRS design regardless of NOMA transmission schemes

- Observation 3:

  - For sufficiently low code rate, LCRS/MUSA/SCMA shows similar results

  - For comparatively high code rate, LCRS shows better LLS results due to the coding gain

- Observation 4: For OFDM waveform, spreading based NOMA scheme with frequency domain spreading using same spreading sequence increases PAPR substantially.

- Proposal: LCRS is considered as the transmission scheme for NOMA study

Decision: The document is noted.

R1-1807377	Transmitter side signaling processing schemes for NOMA	Qualcomm Incorporated

- Proposal 1: NR NOMA should consider the joint use of linear modulation, linear spreading, scrambling and multi-layer transmission in UL to optimize the tradeoff in performance and transceiver complexity.

- Proposal 2:   The transmission scheme of NR NOMA should support different spectral efficiency per UE via flexible configuration of spreading factor, code rate and number of layers.

- Proposal 3: The transmission scheme of NR NOMA should support scalable configuration of multiplexing capacity and overloading ratios with minimal description complexity.

- Proposal 4:  Depending on the use cases of NOMA, the following options of linear hybrid spreading and scrambling can be considered:

  - Option A: Joint use of UE-specific spreading and cell-specific scrambling, wherein the configuration of spreading codes and/or scrambling sequences can be made symbol-dependent;

  - Option B: Joint use of symbol-level repetition and UE-specific scrambling, wherein the configuration of scrambling sequences can be made symbol-dependent;

  - Option C: Joint use of UE-specific spreading and UE specific scrambling, wherein the configuration of spreading codes and/or scrambling sequences can be made symbol-dependent.

- Proposal 5:  To reduce the signalling overhead of NOMA systems, as well as the complexity and memory size of transceivers, it is desirable to employ the following schemes for NOMA transmission:

  - a closed form design for the short spreading codes that achieves Welch bound equality (WBE) for arbitrary spreading factor (K) and overloading ratio (N/K>1);

  - a closed from design for the long scrambling codes that is compliant with Rel-15 NR specification;

  - reuse the same spreading codes and scrambling codes for both DFT-s-OFDM and CF-OFDM waveforms;

  - reuse linear modulation of NR Tx chain.

- Proposal 6: The design of linear spreading scheme for NOMA should consider the inclusion of symbol-wise scrambling to improve the PAPR performance. The PAPR performance of QPSK can be used as a baseline.

Decision: The document is noted.

Continue offline discussion – Li (ZTE)

Thursday

R1-1807778	Offline summary of transmitter processing for NOMA	ZTE

Decision: The document is noted.

Agreements:

- Detailed transmission schemes particularly MA signature design per scheme will be captured in TR. Performance and complexity comparisons and observation/conclusion should at least be made scheme-wise.

- Transmitter side data processing for NOMA can be based on one or more of the following aspects

  - UE -specific bit-level scrambling

  - UE -specific bit-level interleaving

  - UE -specific symbol-level spreading

    - Can be with NR legacy modulation or modified modulation

  - UE -specific symbol-level scrambling

  - UE -specific symbol-level interleaving, with symbol-level zero padding

  - UE -specific power assignment

  - UE-specific sparse RE mapping

  - Cell-specific MA signature

  - Multi-branch/MA signature transmission (irrespective of rank) per UE

R1-1805907	Discussion on the design of SCMA	Huawei, HiSilicon

R1-1806079	Discussion on NOMA transmitters	vivo

R1-1806241	Signature design for NoMA	Ericsson

R1-1806305	NOMA transmitter side signal processing	CATT

R1-1806372	Considerations on Rel-15 NoMA Transmitter Design	CMCC

R1-1806469	Discussion on NoMA Schemes for NR	NEC

R1-1806565	Consideration on the transmitter side signal processing for NOMA	Sony

R1-1806635	Transmitter side signal processing schemes for NCMA	LG Electronics

R1-1806751	Transmitter side signal processing schemes for NOMA	Samsung

R1-1806876	TX side processing for UL NOMA	NICT

R1-1806930	Considerations on NOMA transmitter	Nokia, Nokia Shanghai Bell

R1-1806979	On Transmitter Side signal Processing for NOMA	AT&T

R1-1807021	Implementation Aspects of Low Code Rate NOMA Schemes 	InterDigital, Inc.

R1-1807073	Transmitter design of UGMA	NTT DOCOMO, INC.

R1-1807601	Discussion on NOMA schemes	Google Inc.	(rev. of R1-1807167)

R1-1807223	Design Considerations for NOMA Transmitter 	Convida Wireless LLC

R1-1807290	On the NOMA schemes	KT Corp.

### Receivers for NOMA

R1-1805908	Discussion on the design of NoMA receiver	Huawei, HiSilicon

- Observation 1: The implementation of MPA supports full parallelism and its complexity can be greatly reduced by low projection in constellation design and ordered group SIC decoding.

- Observation 2: ESE heavily relies on the coding gain from the FEC and thus may have slower convergence, especially at high spectral efficiency and high overloading.

- Observation 3: A Block-wise MMSE has much higher complexity than a chip-by-chip MMSE.

- Observation 4: EPA can achieve similar performance as MPA in many cases of interest with lower complexity and has better convergence performance compared with ESE.

- Observation 5: Hybrid IC can take advantages of both pure soft and pure hard IC schemes and achieves the best performance with low-complexity implementations.

- Observation 6: Compared with SIC, PIC has much lower decoding latency and can avoid the impact of error propagation problem caused by inaccurate SINR sorting.

- Observation 7: Hybrid PIC archives the best performances among different combinations of IC implementation.

- Observation 8: An iterative receiver can improve the BLER performance of the NoMA transmissions. The higher the number of users, the more iterations may be needed.

- Proposal 1: Capture the proposed candidates of the MU detectors into TR and further evaluate the performance and implementation limits.

- Proposal 2: Hybrid PIC should be supported for IC implementation.

- Proposal 3: Iterative advanced receiver with configurable number of outer-loop iterations between MU detector and channel decoders is supported for NoMA transmissions.

- Proposal 4: Further study the performance limits, application scenarios, and the implementation costs of the iterative NoMA receivers with different MU detectors and different IC methods.

Decision: The document is noted.

R1-1806242	Receiver design for NOMA	Ericsson

- Observation 1: A commercial gNB preferably implements a single UL MU receiver architecture for different multiple-access signal demodulation scenarios, including regular MU-MIMO and NOMA, regardless of whether dynamic or configured grant-based scheduling is used.

- Observation 2: Due to the low total capacity impact of NOMA scenarios, the NOMA feature is likely to be implemented only if the receiver complexity increase compared to MU-MIMO is moderate.

- Observation 3: If the incremental cost of NOMA receiver for high overloading conditions is considerable, an operator may prefer avoiding high overloading by allocating additional resources to NOMA mode operations.

- Proposal 1: NOMA signature designs should target receiver architectures that attain performance gains with a limited processing overhead compared to conventional MU-MIMO reception.

- Proposal 2: MMSE-SIC receiver should be used as a baseline receiver for NOMA scheme comparison. Additional performance figures may be provided for additional receiver architectures.

Decision: The document is noted.

R1-1806931	Receiver considerations for UL NOMA	Nokia, Nokia Shanghai Bell

- Observation 1: Linear decorrelation-based receivers can be used for the asynchronous NOMA.

- Observation 2: For a parallel IC receiver architecture, the total processing latency is determined by the maximum number of iterations and processing latency of a single iteration.

- Observation 3: For a parallel IC receiver architecture, the total computation complexity is determined by the average number of iterations per user, the number of users processed in a single iteration and total computation complexity of a single user per iteration.

- Observation 4: MPA receiver shows significant computation complexity compared to linear decorrelation based receiver.

- Observation 5: Channel re-encoding and re-decoding operation will dominate the complexity in iteration based codeword-level based receiver.

- Proposal 1: When evaluating the performance of grant-free access, receiver complexity and receiver type should be explicitly indicated.

- Proposal 2: For NOMA evaluation consider an MMES-based Hard IC receiver.

- Proposal 3: For NOMA evaluation consider an ESE-based Soft IC receiver.

Decision: The document is noted.

Continue offline discussion – Li (ZTE)

Thursday

R1-1807779	Offline summary of receiver for NOMA	ZTE

Decision: The document is noted.

Agreements:

- In performing performance evaluation, companies should provide analsysis of receiver complexity. Particularlly (with details FFS):

  - Detector complexity

  - Decoding complexity

  - Interference cancellation complexity, if any

  - Number of iteration(s), if any

  - Other receiver optimization, if any

  - Complexity for the preamble/DMRS detection

  - Memory requirements

  - Latency

  - FFS which simulation cases to be selected for evaluation

- Discuss further next meeting potential template capturing the complexity analysis, especially regarding the level of details in the analysis

R1-1805841	Multi-user receivers for NOMA	ZTE

R1-1806080	Receiver design considerations for NOMA	vivo

R1-1806306	Discussion on NOMA receivers	CATT

R1-1806532	Receiver structure for NOMA	Intel Corporation

R1-1806636	Considerations on the receiver types and NoMA schemes	LG Electronics

R1-1806752	Considerations on receiver types for NoMA schemes	Samsung

R1-1806980	Receiver techniques for NOMA	AT&T

R1-1807022	IDMA Receiver Performance Evaluation 	InterDigital, Inc.

R1-1807074	Receiver candidates for uplink NOMA transmission	NTT DOCOMO, INC.

R1-1807378	Receivers for NOMA	Qualcomm Incorporated

### Procedures related to NOMA

R1-1806081	Discussion on NOMA procedures	vivo

- Proposal 1: Asynchronous UL transmission should be fully studied in NR NOMA SI.

- Proposal 2: Asynchronous UL transmission is targeting eMBB and mMTC UEs in inactive/idle state and UEs in out-of-sync state.

- Proposal 3: Synchronous and asynchronous UL transmission could be unified under the same framework.

- Proposal 4: Asynchronous procedures could be studied based on R-15 signal structures at least for some scenarios previously agreed.

- Proposal 5: Signal structure could be further optimized considering larger overloading factor and simple Rx design.

Decision: The document is noted.

R1-1806981	On NOMA Procedures	AT&T

- Proposal 1: For grant based transmissions, spreading factor should be indicated in the downlink control channel for smooth transition between NOMA and OMA

- Proposal 2: RAN1 should study techniques for finding the approximate expression SINR for NOAM receivers for performing LA in NOMA scenarios

Decision: The document is noted.

R1-1807075	Considerations on NOMA related procedures	NTT DOCOMO, INC.

- Proposal 1:

  - Both grant-based and grant-free NOMA need to be further studied.

  - Grant-based NOMA is supported at least for eMBB scenario

  - Grant-free NOMA is supported at least for mMTC scenario

  - Grant-free NOMA can be considered for eMBB scenario

- Proposal 2: The contents of resource pool including the time/frequency resource, candidate signatures and the DM-RS for the grant-free should be further considered for grant-free transmission

- Proposal 3: NOMA SI should assume UL synchronization is maintained by certain manners

  - Further study the exact manner to maintain the UL synchronization

- Proposal 4: Support of HARQ can be considered

  - How to support the UE identification and differentiation of the initial transmission and retransmission needs further investigation

- Proposal 5: Functionalities to achieve link adaptation should be considered for UL grant free transmission

  - The supported set of MCS levels needs further study

  - The detailed design for MCS notification needs further study

- Proposal 6: Further investigate solution to control the signature collision and DM-RS collision

  - Joint selection of signature and DM-RS for the collision reduction can be considered

Decision: The document is noted.

Continue offline discussion – Li (ZTE)

Thursday

R1-1807780	Offline summary of procedures for NOMA	ZTE

Decision: The document is noted.

Agreements:

- UL data transmission and detection procedures of Rel-15 configured grant is the starting point for NOMA study.

- Different UL data transmission and detection procedures from Rel-15 configured grant for NOMA study can be considered

  - e.g. Preamble, DMRS, synchronization, resource (physical resource and MA signature) configuration, UE detection, HARQ retransmission and ACK/NACK feedback, link adaptation, adaptation between orthogonal and non-orthogonal multiple access, collision control, etc.

Agreements:

- Synchronous UL data transmission should be the starting point.

- Also considers the asynchronous transmission

  - Timing offset is within [0,  y] as starting point, where y has two values at least for the purpose of evaluation:

    - Case 1: CP/[2] < y <= CP+rms_DS, with detailed value FFS

    - Case 2: 2*CP>=y > CP, with detailed value FFS

    - Additional value(s) for y are not precluded

    - Possible down-selection can still be discussed

  - FFS the channel structure and procedures for asynchronous.

R1-1805842	Procedures related to NOMA	ZTE

R1-1805909	Discussion on NoMA related procedures	Huawei, HiSilicon

R1-1806122	Considerations on procedures related to NOMA	Fujitsu

R1-1806243	Procedures related to NoMA	Ericsson

R1-1806307	Discussion on NOMA procedures	CATT

R1-1806373	Considerations on NoMA related procedures	CMCC

R1-1806405	Considerations on NOMA procedures	Spreadtrum Communications

R1-1806470	Considerations on NOMA related procedures	NEC

R1-1806474	Discussion on HARQ Procedure 	TCL Communication Ltd.

R1-1806533	NOMA related procedure	Intel Corporation

R1-1806637	Discussion on procedures related to NOMA	LG Electronics

R1-1806753	Discussion on NoMA related procedure	Samsung

R1-1806932	Procedures to be considered for NOMA operation	Nokia, Nokia Shanghai Bell

R1-1806960	Discussion on Procedures Related to NOMA	National Taiwan University

R1-1807023	On Procedures for Non-orthogonal Multiple Access 	InterDigital, Inc.

R1-1807207	Considerations on Link Rate Adaptation for NOMA	TCL Communication Ltd.

R1-1807209	Discussion on NOMA procedure	Google, Inc.

R1-1807222	Discussion on procedures related to NOMA	KDDI Corporation

R1-1807379	Procedures related to NOMA	Qualcomm Incorporated

### Link and system level performance evaluation for NOMA

R1-1806093	Multiple Access Capacity for ACMA	HUGHES Network Systems Ltd

- Observation1: Under the same test conditions, truly asynchronous NOMA can roughly double or triple the overloading capability of synchronous NOMA.

- Observation 2: Overloading capacity is especially crucial for successful deployment of NOMA techniques within mMTC applications.

- Proposal: NR should further study and consider the adoption of completely asynchronous NOMA especially for mMTC applications.

Decision: The document is noted.

R1-1806754	Evaluation discussion for NoMA study	Samsung

- Observation 1: For low UE spectral efficiency, almost all evaluated NoMA schemes can provide good capability of user multiplexing (e.g. >=300% overloading) with good BLER performance, even with low complexity detectors.

- Observation 2: In case of supporting high UE spectral efficiency, advanced receiver with higher computational complexity (e.g. MPA/MAP) or enabling of higher order modulation/ multi-layer transmission from single UE is required for NOMA.

- Observation 3: By configuring mapping density, proposed IGMA can achieve good trade-off between sparsity and channel coding gain.

- Observation 4: NOMA based on low coding rate and ESE detection, including IGMA, has great potential to support URLLC scenario.

- Observation 5: Sparsity introduced by grid-mapping is important against inter-cell interference.

- Observation 6: the proposed IGMA can well suit for mMTC, URLLC and eMBB, even with low complexity ESE detector. The performance of IGMA is among the best of evaluated NOMA schemes.

- Observation 7: Multi-layer transmission can help improve the performance of IGMA by achieving lower coding rate.

- Observation 8: Enhanced ESE, i.e., e-ESE, can improve the performance of IGMA by exploiting the multi-antenna receivers without introducing much complexity.

- Observation 9: the IGMA with e-ESE and MAP can satisfy the BLER requirement for URLLC.

- Observation 10: The performance loss caused by realistic channel estimation for IGMA is acceptable.

Decision: The document is noted.

R1-1807024	Link Level Evaluation of IDMA 	InterDigital, Inc.

- Observation 1 – For both AWGN and fading channels, a significant throughput gain can be achieved by IDMA.

- Observation 2 – Frequency diversity plays an important role in multiplexing capability of  IDMA scheme.

- Observation 3 – Unequal SNR have some negative impact on the capacity of IDMA in fading channels, however its performance is still superior to the OMA.

- Observation 4 – IDMA can provide significant multiplexing gains compared to OMA at the cost of small SNR penalty.

Decision: The document is noted.

Continue offline discussion – Li (ZTE)

Thursday

R1-1807781	Offline summary of link and system level evaluations for NOMA	ZTE

Decision: The document is noted.

Agreements:

- For mMTC,

  - the baseline for system-level performance comparison is

    - UL transmission with configured grant type 1 or type 2 in Rel.15 NR.

      - Companies to report the link adaptation assumptions, if any.

      - The DMRS collision, if any, should be taken into account.

  - For the evaluation of NOMA schemes

    - UL transmission with configured grant type 1 or type 2 in Rel.15 NR as staring point

      - Companies to report the link adaptation assumptions, if any.

      - The MA signature (including DMRS) is semi-statically configured.

        - The MA signature collision, if any, should be taken into account.

    - FFS: to demonstrate the potential NOMA gain under grant-free transmission with random selection of MA signatures, where collision of MA signature should be considered.

      - The grant-free definition follows NR SI.

Agreements:

- For eMBB,

  - the baseline for system-level performance comparison can be

    - Configured grant type 1 or type 2 in Rel.15 NR.

      - The DMRS collision, if any, should be taken into account.

      - Companies to report the link adaptation assumptions, if any.

    - UL transmission with dynamic grant

      - Details to be reported.

      - The signalling overhead should be reported.

  - For the evaluation of NOMA schemes

    - Configured grant type 1 or type 2 in Rel.15 NR.

      - The MA signature (including DMRS) is semi-statically configured.

      - The MA signature collision, if any, should be taken into account.

      - Companies to report the link adaptation assumptions, if any.

    - UL transmission with dynamic grant

      - Details to be reported.

      - The signalling overhead should be reported.

    - FFS: to demonstrate the potential NOMA gain under grant-free transmission with random selection of MA signatures, where collision of MA signature should be considered.

      - The grant-free definition follows NR SI.

Agreements:

- For SLS in mMTC and eMBB, the packet drop rate (PDR) is defined as (the number of packets in outage) / (the number of packets generated), where a packet is in outage if this packet failed to be successfully decoded by the receiver beyond

  - “packet dropping timer”, or

    - The packet dropping timer can be set to 1 second as the starting point.

  - “maximum number of HARQ transmission(s)”

    - 1 and 8 as starting point

    - The HARQ timing is FFS

Agreements:

- Simplified system-level evaluations can be used for URLLC scenario as detailed as follows:

  - Mean BLER of a UE can be used to represent the reliability of the UE.

    - Note: Further considerations can be reviewed, e.g. the deviation of BLER about the mean BLER.

- PHY abstraction methods agreed in TR38.802 can be reused as the starting point.

  - Note: Further considerations can be reviewed.

Agreements:

- For mMTC, higher layer protocol overhead can be confirmed to 29 bytes for evaluation purpose.

- The traffic model below is used for NOMA evaluations in URLLC scenario:

  - Packet arrival per UE can be based on either option 1 or option 2

    - Option 1: FTP Model 3 with Poisson arrival;

    - Option 2: Periodic packet arrivals.

  - Packet size:

    - Single fixed value per simulation: 60 bytes and 200 bytes

      - higher layer protocol overhead included

  - The target reliability is 99.999% and the target delay requirement is 1ms (for 60 bytes) and 4ms (for 200bytes) as starting point.

- The traffic model below is used for NOMA evaluations in eMBB scenario:

  - Packet arrival per UE: FTP Model 3 with Poisson arrival

  - Packet size:

    - [40]~[600] bytes Pareto distribution, with shaping parameter alpha = [1.5] as starting point.

      - Further refinement can be further discussed in RAN1#94

Agreements:

- The building penetration model defined in Table 7.4.3-3 in TR 38.901 is used for SLS with frequencies below 6 GHz.

- For mMTC:

  - Inter-BS distance is 1732 m.

  - Simulation bandwidth with 6 PRBs is the starting point.

  - For UE distribution, 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h).

- For URLLC:

  - Carrier frequency can be 4GHz or 700MHz.

    - For 4GHz,

      - 200m ISD, 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h).

      - For 700MHz, 500m ISD, 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h).

        - Other option(s) not precluded, e.g., 500m ISD, 80% of users are outdoors (3km/h), 20% of users are indoor (3km/h).

- Clarify the simulation bandwidth in the SLS assumptions is the bandwidth for uplink transmission.

  - FFS whether or not to introduce system bandwidth in SLS

Agreements:

- For calibration of the CDFs of coupling loss and downlink geometry averaged over two antenna ports.

  - Use the assumption in the following Table.

Table System-level assumptions for calibration purpose

| Parameters | Case 1 | Case 2 |  | Case 3 |  |
| --- | --- | --- | --- | --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |  |  |
| Inter-BS distance | 1732m | 500m |  | 200m |  |
| Carrier frequency | 700MHz | 700MHz |  | 4GHz |  |
| Channel model | UMa in TR 38.901 |  |  |  |  |
| UE Tx power | Max 23 dBm |  |  |  |  |
| BS Tx power | Max 46 dBm |  |  |  |  |
| BS antenna configurations | 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), +-45 Polarization dH = dV = 0.8λ; |  |  |  |  |
| BS antenna downtilt | 92 |  | 98 |  | 102 |
| BS antenna height | 25m |  |  |  |  |
| BS antenna element gain + connector loss | 8 dBi, including 3dB cable loss |  |  |  |  |
| BS receiver noise figure | 5dB |  |  |  |  |
| UE antenna configuration | 1 |  |  |  |  |
| UE antenna height | Follow the modelling of TR 38.901 |  |  |  |  |
| UE antenna gain | 0dBi |  |  |  |  |
| UE distribution | Follow the evaluation assumptions |  |  |  |  |
| UE power control | Open loop PC, P0 = [-90] dBm, alpha = 1. |  |  |  |  |
| HARQ/repetition | 1 |  |  |  |  |
| UE attachment | Refer to 36.873 |  |  |  |  |

Agreements:

- Residual frequency offset for link-level simulation

  - In addition to 0, evaluate uniform distribution between -70 and 70 Hz for 700MHz carrier frequency, and uniform distribution between [-140] and [140] Hz for 4GHz carrier frequency.

Agreements:

- Clarify the definition of SNR in LLS as:

  - The mean received power over the allocated bandwidth per OFDM symbol carrying data, divided by noise power per OFDM symbol within the allocated bandwidth.

Agreements:

- For realistic channel estimation with number of DMRS ports <= 12

  - Reuse the NR design for evaluation purpose

  - Other DMRS designs are not precluded for the NOMA study

- For realistic channel estimation with number of DMRS ports > 12

  - The DMRS overhead should not be less than NR design for evaluation purpose.

  - FFS extending DMRS design for the NOMA study

- Number of PRB is 24 PRBs for URLLC evaluations with 30 kHz SCS.

  - There is no implication to make performance comparison between 60 kHz and 30 kHz.

Agreements:

- For calibration purpose, the link-level evaluation assumptions are given below:

  - Companies to share the calibration results by RAN1 email by August 1st, to be consolidated for RAN1#94 contribution – Li (ZTE)

| Implementation assumptions | Values |
| --- | --- |
| LDPC decoding algorithm (e.g. MaxLogMAP or LogMAP, fully parallel or row parallel) | Companies to report |
| Number of LDPC decoding iteration | Companies to report (e.g., 50 for flooding, 25 for layered) |
| Modulation for 10/20 bytes | QPSK |
| Modulation for 75/150 bytes | QPSK |
| Channel Estimation | Ideal |
| Channel Model | AWGN, TDL-A with 30ns (3km/h), TDL-C with 300ns (3km/h), no spatial correlation Initialize channel realization at each slot |
| Total number of slots | 1000 for eMBB/mMTC AWGN 10000 for eMBB/mMTC fading channel [50000] for URLLC AWGN [100000] for URLLC fading channel |
| System bandwidth | 10 MHz |

Email discussion for the templates of reporting the link-level evaluation results, to start email discussion from June 18, aim to finish by 8/3

R1-1805843	Remaining evaluation methodology and assumptions for NOMA	ZTE

R1-1805910	Further discussion on SLS and LLS evaluations	Huawei, HiSilicon

R1-1806082	Evaluations for NOMA	vivo

R1-1806092	Link-Level Simulation results for ACMA	HUGHES Network Systems Ltd

R1-1806244	Evaluation methodology and metrics for NoMA	Ericsson

R1-1806308	Further link level evaluation results of NOMA	CATT

R1-1806534	Evaluation methodology for NOMA study	Intel Corporation

R1-1806638	LLS evaluation for NCMA	LG Electronics

R1-1806669	Comparison of full-length spreading domains for NOMA transmitter design	ETRI

R1-1806933	Further considerations on NOMA evaluation	Nokia, Nokia Shanghai Bell

R1-1806982	Link system performance for NOMA	AT&T

R1-1807076	Discussion on comparison methodology and evaluations assumption for NOMA	NTT DOCOMO, INC.

R1-1807203	Further link level simulation results of non-orthogonal multiple-access schemes	CATR

R1-1807244	Initial link level simulation result of NOMA	Sony

R1-1807380	System level  evaluation for NOMA	Qualcomm Incorporated

R1-1807381	Link level evaluation for NOMA	Qualcomm Incorporated

### Other

R1-1805844	Channel estimation error modeling for NOMA receiver	ZTE

R1-1805845	Link-level simulation results for NOMA	ZTE

R1-1805846	Calibration results of reference OMA single user cases	ZTE

R1-1805911	LLS evaluation results for mMTC scenario	Huawei, HiSilicon

R1-1805912	LLS evaluation results for URLLC scenario	Huawei, HiSilicon

R1-1805913	LLS evaluation results for eMBB scenario	Huawei, HiSilicon

R1-1806245	Observations on NOMA traffic models	Ericsson

R1-1806246	Impairments model for link and system evaluations	Ericsson

R1-1806247	Link level evaluations for MU-MIMO	Ericsson

R1-1806248	MU-MIMO operation in NR with configured grants	Ericsson

R1-1806309	Link level evaluation results of OMA	CATT

R1-1806310	Remaining evaluation assumptions	CATT

R1-1806535	Initial link level evaluations for NOMA – mMTC	Intel Corporation

R1-1806536	Initial link level evaluations for NOMA – eMBB	Intel Corporation

R1-1806537	Initial link level evaluations for NOMA - URLLC	Intel Corporation

Late submission

R1-1806934	Initial link level simulation results for NOCA	Nokia, Nokia Shanghai Bell

R1-1806935	Initial system level simulation results for NOCA	Nokia, Nokia Shanghai Bell

R1-1807025	DMRS CHEST Error Model for NOMA Evaluation 	InterDigital, Inc.

R1-1807026	Remaining issues on system level simulations for NOMA 	InterDigital, Inc.

R1-1807027	Link Level mMTC Performance Comparison 	InterDigital, Inc.

R1-1807028	On Operations of Non-orthogonal Multiple Access 	InterDigital, Inc.

R1-1807029	On Collision Mitigation and Handling for Non-orthogonal Multiple Access 	InterDigital, Inc.

R1-1807030	On Demodulation Reference Signal for Non-orthogonal Multiple Access 	InterDigital, Inc.

R1-1807077	LLS calibration results for OMA	NTT DOCOMO, INC.

R1-1807169	Coding schemes for NOMA in mMTC	Google Inc.

## Study on evaluation methodology of new V2X use cases - SID in RP-170837

R1-1806178	Draft TR 37.885 v0.2.0 Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR	LG Electronics Inc.

Decision: The document (v0.2.0) is endorsed.

Thursday

TP to capture the agreements to be prepared by Hanbyul (LGE).

Prepare reply LS to S4-180633.

Friday

R1-1807837	Draft TR 37.885 v0.3.0 Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR	LG Electronics

Decision: The document is endorsed as version 1.0.0 in R1-1807869, aiming at one-step approval by next plenary.

R1-1807826	[Draft] Reply LS on the transmission of video over PC5 interface 	Huawei

Decision: The document is endorsed and final LS is approved in R1-1807870.

### Evaluation scenarios

Including scenarios, performance metric, vehicle dropping, traffic model

R1-1807687	Summary of discussion on NR V2X evaluation scenarios	LG Electronics

Decision: The document is noted.

Agreements:

- For sidelink simulation bandwidth for below 6GHz:

  - Both 10 MHz and 20 MHz are the baseline.

  - 100 MHz is optional

    - Note that this bandwidth is available in licensed spectrum at this moment and not supported in the current ITS spectrum allocation.

Agreements:

- Aggregated system bandwidth for above 6 GHz is up to 1 GHz in DL+UL and SL. Simulation bandwidth is 200 MHz for DL+UL and 200 MHz for SL.

  - Further consideration is needed on how to scale the performance of sidelink.

Agreements:

- Max transmit power of vehicle/pedestrian UE for both urban case and highway case:

  - 30GHz: 23dBm

  - 63GHz: 21dBm (baseline) and 27dBm (optional)

  - For both 30Ghz & 63GHz, EIRP should not exceed 43 dBm

Agreements:

- Max transmit power of BS and BS-type RSU

  - Urban scenario

    - 43dBm PA scaled down proportionally with simulation BW when system BW is higher than simulation BW. Otherwise, 43dBm. EIRP should not exceed 78 dBm and is also subject to appropriate scaling

  - Highway scenario

    - 43dBm PA scaled down proportionally with simulation BW when system BW is higher than simulation BW. Otherwise, 43dBm.

- Max transmit power of UE-type RSU for both urban case and highway case

  - 30GHz: 23dBm

  - 63GHz: 21dBm (baseline) and 27dBm (optional)

  - For both 30Ghz & 63GHz, EIRP should not exceed 43 dBm

Agreement:

- Inter-vehicle distance by max {2 m, an exponential random variable with the average of the speed * 2 sec}.

Agreements:

- For Option A in highway scenario

  - Vehicle speed is 140 km/h as baseline and 70 km/h as optional

- For Option A in urban scenario, vehicle speed is 60 km/h.

Agreement:

- Vehicle type distribution is not dependent of the lane.

Agreements:

- In option B of the urban scenario,

  - Do not drop vehicles at the intersections in the North-South directions:

  - No change of direction at the intersection

Agreement: In option A of the urban scenario,

- No specific handling is needed for vehicle overlap at the intersections.

Agreement: In Option B of urban scenario

- No consensus in supporting the reduced layout.

Agreement:

- Reuse the pedestrian UE dropping in TR 36.885.

Agreements: The following cellular UE dropping is defined.

- Cellular UE inside a vehicle

  - In each vehicle of type 1 or type 2, the number of cellular UEs inside the vehicle is uniformly random in the range between 1 and 4

  - In each vehicle of type 3, the number of cellular UEs inside the vehicle is uniformly random in the range between 10 and 20

  - The portion of the active cellular UEs is [20], [50], [80]%.

  - Note that the traffic for the cellular UE is to use licensed spectrum.

Agreements:

- For >6GHz

  - The BS deployment in urban scenario is the same as that for below 6 GHz.

  - The BS deployment in highway scenario uses the option of the highway overlaid with 500 m ISD cell layout (Figure A.1.3-3 of TR 36.885).

Agreement:

- Baseline is to drop the macro BS only. Companies are not precluded to evaluate a scenario with additional micro BS.

Agreement:

- UE-type RSU deployment is the same as that of TR 36.885. Companies are not precluded to evaluate a scenario with additional UE-type RSU.

Agreements:

- To define three options for periodic traffic:

  - One option with low traffic intensity

    - Inter-packet arrival time: 100 ms

    - Packet size: Pattern of {300 bytes, 190 bytes, 190 bytes, 190 bytes, 190 bytes} with random starting point for each UE

    - Latency requirement: 100 ms

  - One option with medium traffic intensity

    - Inter-packet arrival time: 10 ms

      - Other value(s) are not precluded, e.g., 100ms

    - Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8

    - Latency requirement: 10 ms

  - One option with high traffic intensity

    - Inter-packet arrival time: 30 ms

    - Packet size: Uniformly random in the range between 30000 bytes and 60000 bytes with the quantization step of 10000 bytes

    - Latency requirement: 30 ms

- To define two options for aperiodic traffic:

  - One option with medium traffic intensity

    - Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms

    - Packet size: Uniformly random in the range between 200 bytes and 2000 bytes with the quantization step of 200 bytes

    - Latency requirement: 50 ms

  - One option with high traffic intensity

    - Inter-packet arrival time: 10 ms + an exponential random variable with the mean of 10 ms

    - Packet size: Uniformly random in the range between 10000 bytes and 30000 bytes with the quantization step of 4000 bytes

    - Latency requirement: 10 ms

Agreements:

- For evaluations of UE-network relaying or coexistence of sidelink and Uu in licensed spectrum, FTP model 3 is used.

- No new model is needed for event-triggered traffic.

- The baseline is all the vehicles generate messages in evaluating broadcast.

Agreements:

- Baseline is to evaluate unicast, multicast, and broadcast in separate simulations.

- Note: in evaluating technical solutions, scenarios such as mixture of unicast, multicast, and broadcast, mixture of periodic and aperiodic traffic, etc., may be discussed

Agreements:

- For each transmitter for unicast, the target receiver is randomly selected among the UEs within X meters.

  - For the clustered vehicle dropping, the target receiver should be in the same cluster as the transmitter.

  - X:

    - Further discussion offline. If no additional progress, X is to be discussed as a part of technical solutions.

  - This transmitter-receiver association assumption can be revisited if an issue is identified.

- For each transmitter for multicast, the target receivers are all the UEs within Y meters.

  - For the clustered vehicle dropping, the target receivers should be in the same cluster as the transmitter.

  - Y :

    - Further discussion offline. If no additional progress, Y is to be discussed as a part of technical solutions.

Agreements: Define two types of PIR similarly to PRR:

- PIR type 1: For a given distance D, PIR is the time Ti elapsed between two successive successful receptions of two different packets transmitted from node A to node B for the same application, if the distances at the two packets’ receiving time between node A and node B is within the range of (0,D].

  - Average PIR within given distance D, calculated as (T1+T2+T3+…+Tn)/n where n denotes the number of collected PIR in simulation.

  - CDF of PIR with given distance D.

- PIR type 2: PIR is the time Ti elapsed between two successive successful receptions of two different packets transmitted from node A to node B for the same application, if the node B is one of the intended set of receivers of the node A.

  - Average PIR with intended set of receives, calculated as (T1+T2+T3+…+Tn)/n where n denotes the number of collected PIR in simulation.

  - CDF of PIR with intended set of receives.

- PIR type 1 is included as a performance metric at least for the broadcast-type use cases.

- PIR type 2 is supported and can be used as performance metric in scenarios such as multicast/unicast or to see the performance of links in a certain condition (e.g., links blocked by a building) or links in a specific area (e.g., lanes with high congestion in highway scenario). When used, the company needs to clarify how the intended set of receivers is determined and what the motivation is.

Agreement:

- Packet throughput is used as a performance metric.

Agreements:

- Add “Between vehicle/pedestrian UE” and “UE-type-RSU to/from vehicle/pedestrian UE” to the links being evaluated using 30 GHz frequency

  - The channel model is the same as that of 63 GHz except the center frequency.

Thursday

R1-1807756	Summary of offline discussion on NR V2X evaluation scenarios	LG Electronics

Decision: The document is noted.

Agreement:

- TR22.872 can be used as a starting point when necessary for the additional performance metric for latency for positioning evaluation.

R1-1805915	On eV2X Evaluation Methodology	Huawei, HiSilicon

R1-1806027	NR eV2X Evaluation Methodology	ZTE Wistron Telecom AB, Sanechips

Withdrawn

R1-1806102	On NR eV2X evaluation methodology	ZTE

R1-1806151	Simulation scenarios and performance metrics for V2X	Ericsson

R1-1806205	On the Motivation for Periodic Data Traffic Model	OnStar Europe

Withdrawn

R1-1806206	PRI Metric Calculation	OnStar Europe

R1-1806311	Remaining details on evaluation scenarios of new V2X use cases	CATT

R1-1806335	On the Urban Scenario Evaluation Scenario	OnStar Europe

R1-1806538	Scenarios, traffic models and performance metrics for eV2X evaluations	Intel Corporation

R1-1806566	Discussion on remaining issues of eV2X evaluation methodology	Sony

R1-1806639	Discussion on evaluation scenarios for new V2X use cases	LG Electronics

R1-1806755	Evaluation scenarios for NR V2X	Samsung

R1-1806820	Remaining issues on evaluation scenarios for new V2X use cases	China Telecommunications

R1-1806831	Further considerations on evaluation scenarios for NR-V2X	OPPO

R1-1806984	Evaluation scenario and simulation assumptions	AT&T

R1-1807001	Evaluation scenarios of new V2X use cases	Nokia, Nokia Shanghai Bell

R1-1807031	Discussion on Remaining Issues of V2X Evaluation Scenarios	InterDigital, Inc.

R1-1807078	Evaluation assumption for V2X phase 3	NTT DOCOMO, INC.

R1-1807200	Discussion on eV2X evaluation scenarios	vivo

R1-1807382	Traffic models for NR V2X evaluation scenarios	Qualcomm Incorporated

R1-1807666	WF on evaluation methodology for use case ‘tethering via vehicle’	NTT DOCOMO, AT&T

### Channel Model

Thursday

R1-1807755	Summary of discussion for NR V2X channel model	LG Electronics

Decision: The document is noted.

Agreements:

- Reuse gNB cell layout from TR 36.885 including antenna boresight angle definition

- Apply gNB antenna tilts of 102 degree for the case of ISD 500m.

- Apply gNB antenna tilts of 96 degree for the case of ISD 1732m.

Agreements:

- BS antenna configuration is as follows:

|  | gNB (Macro gNB) |  |
| --- | --- | --- |
|  | Low band (2-4 GHz) | High band (30 GHz) |
| TXRU mapping | Up to proponents | Up to proponents |
| Number of antenna elements across all panels | Up to 256 Tx /Rx antenna elements | Up to 256 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | (8, 8, 2, 1, 1) (TR 38.802, Baseline gNB antenna configuration for UMa scenario) | (4, 8, 2, 2, 2) (TR 38.802, Baseline gNB antenna configuration for UMa scenario) |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.8)λ | (dH, dV) = (0.5, 0.5)λ (dH,g, dV,g) = (4.0, 2.0)λ |
| Others | TXRUs within a panel can be assumed to be synchronized and phase-calibrated (at least to the same level as in LTE) |  |

Agreement:

- Use “micro BS” in the TR instead of “BS-type RSU.” Detailed text will be discussed with the TR update.

Agreements:

- UE-type RSU antenna pattern is as follows:

|  | Low band (6 GHz) |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 3 dBi |

|  | High band (63 GHz) |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 5 dBi |

Agreements:

- UE-type RSU antenna configuration is as follows:

|  | Low band (2-6 GHz) | High band (30-63 GHz) |
| --- | --- | --- |
| TXRU mapping | Up to proponents decision | Up to proponents decision |
| Number of antenna elements across all panels | Up to 8 Tx /Rx antenna elements | Up to 32 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | Baseline: (1, 2, 2, 1, 1) Optional: (1, 4, 2, 1, 1) | (1, 4, 2, 1, 4) Panel bearing angle: Ω0,1=Ω0,0+90°; Ω0,2=Ω0,0+180°; Ω0,3=Ω0,0+270°; |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.8)λ TR 38.802, Section A.2.1, Dense Urban micro cell TRP configuration | (dH, dV) = (0.5, 0.5)λ |
| Antenna tilt, deg | 96 | 96 |

Agreements: Two options are defined as follows. Companies are encouraged to provide evaluation results using both options.

- Option 1:

|  | Low band (6 GHz) |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern | Vehicle Type 2: Vehicle Type 1 and Type 3: |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 3 dBi |

|  | High band (63 GHz) |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 5 dBi |

|  | Vehicle UE |  |
| --- | --- | --- |
|  | Low band (2-6 GHz) | High band (30-63 GHz) |
| TXRU mapping | Up to proponents decision | Up to proponents decision |
| Number of antenna elements across all panels | Up to 8 Tx /Rx antenna elements | Up to 32 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | Vehicle Type 1 and Type 3: Front and rear antennas: Baseline: (1, 1, 2, 1, 1) for each location Optional: (1, 2, 2, 1, 1) for each location Vehicle Type 2: Rooftop antenna: Baseline: (1, 2, 2, 1, 1) Optional: (1, 4, 2, 1, 1) | Vehicle Type 1 and 3: [Front and rear antennas: (2, 4, 2, 1, 1) for each location Front antenna array bearing angle: ΩFront = 0° Rear antenna array bearing angle: ΩRear = 180° Vehicle Type 2: Multi-panel at rooftop: (1, 4, 2, 1, 4) Panel bearing angle: Ω0,1=Ω0,0+90°; Ω0,2=Ω0,0+180°; Ω0,3=Ω0,0+270°;] |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.5)λ | (dH, dV) = (0.5, 0.5)λ |
| Antenna tilt, deg | 90 | 90 |

- Option 2:

  - Antenna pattern in R1-1807613 is used.

  - For vehicle type 1,

    - One panel at the front bumper and one panel at the rear bumper

  - For vehicle type 2,

    - One panel at the front edge and one panel at the rear edge of the rooftop

  - For vehicle type 3,

    - One panel at the front edge and one panel at the rear edge of the rooftop

  - Antenna configuration is as follows:

|  | Low band (2-6 GHz) | High band (30-63 GHz) |
| --- | --- | --- |
| TXRU mapping | Up to proponents decision | Up to proponents decision |
| Number of antenna elements across all panels | Up to 8 Tx /Rx antenna elements | Up to 32 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | (1, 1, 2, 1, 1) for each location | (2, 4, 2, 1, 1) for each location |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.5)λ | (dH, dV) = (0.5, 0.5)λ |
| Antenna tilt, deg | 0 | 0 |

Agreement:

- Determination between LOS and NLOSv states is decided per vehicle pair. Note that self-blockage effect is captured Option 2 in the antenna pattern. Self-blockage effect in Option 1 can be further discussed later if necessary.

Agreement:

- Pedestrian antenna height is 1.5 m. A single panel array with (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) is used.

Agreements:

- The first proposal in Section 3 of R1-1807755 is agreed, with the except that the detailed table is agreed as a working assumption

  - If no additional concensus can be reached by Friday, the working assumption automatically becomes agreements

Agreements:

The following statistics of the additional loss value are used when a link is in NLOSv:

- The blocker height is the vehicle height which is randomly selected out of the three vehicle types according to the portion of the vehicle types in the simulated scenario.

- The additional blockage loss is max {0 dB, a log-normal random variable}.

- Case 1: Minimum antenna height value of TX and RX > Blocker height

  - No additional blockage loss

- Case 2: Maximum antenna height value of TX and RX < Blocker height

  - Mean: 12.5 dB

  - STD: 4.5 dB

- Case 3: Otherwise

  - Mean: 5 dB

  - STD: 4 dB

Agreement:

- For Uu interface, use UMa of TR 38.901 for above 6 GHz in highway scenario.

Agreements:

- Set the effective environment height hE to 0.25m.

- Pathloss equation for V2V link in NLOS is PL= 36.85 + 30log10(d) + 18.9log10(fc) (fc is in GHz and d is in meters) where d is the Euclidean distance between TX and RX.

- Pathloss equation of V2V is reused for that of V2P, P2P, V2R, R2R.

- For V2V, V2P, P2P, V2R, R2R links, the shadowing model in TR 36.885 is used. The LOS shadowing model in TR 36.885 applies to NLOSv. For B2V, B2P, B2R links, the shadowing model and fast fading parameters associated with the used pathloss model in TR 38.901 is used.

Agreements:

- The proposal in Section 6 of R1-1807755 is taken as a working assumption

  - If no additional concensus can be reached by Friday, the working assumption automatically becomes agreements

Agreement:

- The proposal in Section 7 of R1-1807755 is agreed.

Agreements:

- Reuse V2P channel model as baseline for the channel model between vehicle UE acting as relay and cellular UE.

  - [3] meter is assumed for the link between antenna of a cellular UE inside a vehicle and the antenna of the same vehicle UE.

- Cellular UE uses the antenna model of pedestrian UE.

Friday

R1-1807836	Offline discussions on working assumptions in channel model	LG Electronics

Decision: The document is noted.

R1-1807672	V2X sidelink channel model	Huawei, HiSilicon, InterDigital, General Motors, Sony	(R1-1805914)

R1-1806028	Further discussion on eV2X Channel Modeling	ZTE Wistron Telecom AB, Sanechips

Withdrawn

R1-1806103	Remaining issues on eV2X channel modeling	ZTE

R1-1806152	Remaining issues on channel models and antenna models for V2X	Ericsson

R1-1807613	Vehicle UE Antenna Radiation Patterns	OnStar Europe	(R1-1806204)

R1-1806312	Remaining details on channel models of new V2X use cases	CATT

R1-1806539	Channel models for eV2X evaluation methodology	Intel Corporation

R1-1806640	Discussion on channel model for new V2X use cases	LG Electronics

R1-1806756	Channel model for NR V2X	Samsung

R1-1806832	On vehicle blocking in channel modelling	OPPO

R1-1806985	Channel model for V2X	AT&T

R1-1807032	On the Remaining Details of V2X Channel Model 	InterDigital, Inc.

R1-1807079	Channel model for V2X phase 3	NTT DOCOMO, INC.

R1-1807612	Discussion on antenna radiation pattern for vehicle UE	LG Electronics, OnStar

### Other

R1-1806029	NR eV2X Link Level Simulation Assumptions	ZTE Wistron Telecom AB, Sanechips

Withdrawn

R1-1806153	Discussion on eV2X use cases	Ericsson

R1-1806986	Autonomous topology management for sidelink operation	AT&T

R1-1807080	Discussion on eV2X operation scenario	NTT DOCOMO, INC.

R1-1807201	On the Motivation for Periodic Data Traffic Model	OnStar Europe

## Study on NR-based Access to Unlicensed Spectrum - SID in RP-172021

R1-1807791	Chairman's notes of AI 7.6 Study on NR-based Access to Unlicensed Spectrum	Ad-Hoc chair (Ericsson)

Discussion: Huawei commented that no Tdocs were opened and treated  would like such situation be avoided for the next meeting.

Decision: Contents incorporated below, the document is endorsed.

R1-1807383	TR38.889 v0.0.2 Study on NR-based Access to Unlicensed Spectrum	Qualcomm Incorporated

Decision: The draft TR is endorsed as v0.0.2.

R1-1807701	TR38.889 v0.0.2 Study on NR-based Access to Unlicensed Spectrum	Qualcomm Incorporated

Revision of R1-1807383

Decision: More time needed for checking.

R1-1807728	Summary of Offline Discussion on NR-U	Qualcomm Incorporated

R1-1807777	Summary of offline discussion on NR-U	Qualcomm Incorporated

Further revised in R1-1807815.

### Simulation Methodology for NR-U operation

R1-1806104	Simulation Methodology and Assumptions for NR-U Evaluations	Nokia, Nokia Shanghai Bell

R1-1806249	NR-U evaluation methodology	Ericsson

R1-1806313	Remaining Issues of Simulation Methodology for NR Unlicensed operations	CATT

R1-1806458	Simulation Methodology for NR-U Operation 	ZTE Corporation

R1-1806540	Simulation methodology and assumptions for NR unlicensed	Intel Corporation

R1-1806641	Simulation methodology for NR unlicensed operation	LG Electronics

R1-1806757	Remaining simulation methodology for NR-U	Samsung

R1-1806793	On simulation methodology for NR-U operation	MediaTek Inc.

R1-1806905	Simulation assumptions and preliminary results for NR unlicensed bands	Huawei, HiSilicon

R1-1807033	Simulation Calibration and Model Selection for NR-Unlicensed 	InterDigital, Inc.

R1-1807156	Outdoor Sub-7 GHz RSSI Evaluation	Charter Communications

R1-1807327	Discussion on outdoor network topology for sub-7GHz coexistence evaluations between NR-U and Wi-Fi	BROADCOM

R1-1807384	Email discussion summary for sub7 indoor evalution calibration	Qualcomm Incorporated

R1-1807385	Simulation methodology for NR unlicensed	Qualcomm Incorporated

Agreement:

- For sub7 GHz outdoor scenario, adopting the following

  - Macro deployment with ISD=200×A meters

  - Each operator randomly drops 1 micro-layer TRP within each macro cell sector with minimum distance between micro-layer TRPs equals 57.9×A meters

  - Independent dropping between two operators

    - Use 10 meters as the inter-operator micro-layer TRP minimum distance

    - For the inter-operator micro-layer TRP maximum distance

      - Outdoor scenario 1: 30

      - Outdoor scenario 2: No limit as long as the TRP is within the macro cell

  - UE randomly dropped within macro cell sector with a minimum serving cell RSSI of -82dBm

  - All UEs dropped outdoor

  - Try A>=1 and find the A that satisfies serving cell received power distribution satisfies (10+X)% to (15+X)%] UEs below -72dBm

  - Other parameters follow the table below

| Parameters | Outdoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline , 80MHz optional |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| SCS | To be reported together simulation results |
| Channel Model | NR UMi street canyon |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |
| gNB to gNB link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |

Email discussion on calibration of parameters A and X targeting a single setting of parameters for both outdoor scenarios until August 2nd, 2018	Qualcomm (Jing).

### Frame structure for NR-U operation

R1-1805917	NR frame structure on unlicensed bands	Huawei, HiSilicon

R1-1806083	Frame structure for NR-U operation	vivo

R1-1806105	Frame Structure for NR-U Operation	Nokia, Nokia Shanghai Bell

R1-1806123	Discussion on flextible starting positions and related PDCCH monitoring for NR-U	Fujitsu

R1-1806250	Frame structure for NR-U	Ericsson

R1-1806314	Frame Structure for NR Unlicensed Operations	CATT

R1-1806345	Frame structure for NR-U operation	Lenovo, Motorola Mobility

R1-1806459	Discussion on frame structure for NR-U	ZTE Corporation

R1-1806541	On the NR frame structure to support unlicensed operation	Intel Corporation

R1-1806567	Considerations on frame structure for NR unlicensed operations	Sony

R1-1806642	Frame structure for NR unlicensed operation	LG Electronics

R1-1806758	Frame structure for NR-U	Samsung

R1-1806794	On frame structure design for NR-U operation	MediaTek Inc.

R1-1806849	Frame structure for NR-U	OPPO

R1-1806987	Frame structure for NR-U operation	AT&T

R1-1807034	On NR-Unlicensed Frame Structure 	InterDigital, Inc.

R1-1807141	Frame structure and numerology for NR-U	ITL

R1-1807204	Frame structure design in NR-U	CATR

R1-1807386	Frame structure for NR-U operation	Qualcomm Incorporated

Agreement:

- Single and multiple DL to UL and UL to DL switching within a shared gNB COT is identified to be beneficial and can be supported

  - LBT requirements to support single or multiple switching points, include

    - For gap of less than 16us: no-LBT can be used

      - Restrictions/conditions on when no-LBT option can be used will be further identified, in consideration of fair coexistence.

    - For gap of above 16us but does not exceed 25us: one-shot LBT can be used

      - Restrictions/conditions on when one-shot LBT option can be used will be further identified, in consideration of fair coexistence.

    - For single switching point, for the gap from DL transmission to UL transmission exceeds 25us: one-shot LBT is used

      - Further study needed on how many one-shot LBT attempts is allowed for granted UL transmission

    - FFS: For multiple switching points, for the gap from DL transmission to UL transmission exceeds 25us, one-shot LBT is used. Regulations for this option.

Agreement:

- Study FBE (as in the ETSI BRAN specifications) based frame structure

  - Identify the changes needed to support FBE operation of NR-U

    - Restrictions/conditions on when FBE option can be used will be further identified, in consideration of fair coexistence.

  - Strive to minimize the change from current NR design

- Agreement:

- Benefits of using a signal that facilitates its detection with low complexity can be investigated including all/part of the following scenarios/use cases:

  - UE power saving

  - Improved coexistence

  - Spatial reuse at least within the same operator network

  - Serving cell transmission burst acquisition

  - FFS: further usage scenarios

### Potential physical layer channel design

#### DL Signals and Channels

Including DL data channel, DL control channel, DL RS, Sync channel, etc.

R1-1805916	DL channel and signals in NR unlicensed band	Huawei, HiSilicon

R1-1806084	Discussion on physical DL channel design in unlicensed spectrum	vivo

R1-1806106	On DL Signals and Channels for NR-U	Nokia, Nokia Shanghai Bell

R1-1806251	On DL Signals and channels	Ericsson

R1-1806315	DL Physical Channel and Signal Design for NR-U Operations	CATT

R1-1806410	Discussion on SS/PBCH block design in NR-U operation	Spreadtrum Communications

R1-1806460	Consideration on DL reference signals and channels design for NR-U	ZTE Corporation

R1-1806542	Potential enhancements to NR DL signals and channels for unlicensed operation	Intel Corporation

R1-1806568	Considerations on DL signals and channels for NR unlicensed operations	Sony

R1-1806643	Physical layer design of DL signals and channels for NR unlicensed operation	LG Electronics

R1-1806759	Downlink signal and channel design for NR-U	Samsung

R1-1806795	On downlink transmission detection for NR-U operation	MediaTek Inc.

R1-1806850	DL design for NR-U	OPPO

R1-1806988	Physical layer channel design for NR-U	AT&T

R1-1807081	DL Signals and Channels for NR-U operation	NTT DOCOMO, INC.

R1-1807208	SS/PBCH blocks transmission for NR unlicensed	Xiaomi Technology

R1-1807225	Design Considerations for SSB in NR-U 	Convida Wireless LLC

R1-1807387	DL signals and channels for NR-U	Qualcomm Incorporated

Agreement:

- NR-U should have a signal that contains at least SS/PBCH block burst set transmission

  - FFS: Other channels and signals transmitted together as part of the signal

- The design of this signal should consider the following characteristics specific to unlicensed band operation

  - There are no gaps within the time span the signal is transmitted at least within a beam

    - FFS: Whether any gaps are needed for beam switching and, if needed, their duration

  - The occupied channel bandwidth is satisfied (although this may not be a requirement)

  - Strive to minimize the channel occupancy time of the signal

  - Characteristics that may facilitate fast channel access

#### UL Signals and Channels

Including UL data channel, UL control channel, UL RS, PRACH, etc.

R1-1805921	UL PHY channels for NR unlicensed	Huawei, HiSilicon

R1-1806085	Discussion on physical UL channel design in unlicensed spectrum	vivo

R1-1806107	On Uplink Signal and Channel Structures for NR-U	Nokia, Nokia Shanghai Bell

R1-1806252	On UL Signals and channels	Ericsson

R1-1806316	UL Physical Channel and Signal Design for NR-U Operations	CATT

R1-1806461	Consideration on UL reference signals and channels design for NR-U	ZTE Corporation

R1-1806543	Potential enhancements to NR UL signals and channels for unlicensed operation	Intel Corporation

R1-1806644	Physical layer design of UL signals and channels for NR unlicensed operation	LG Electronics

R1-1806760	Uplink signal and channel design for NR-U	Samsung

R1-1806796	On UL signals and channels design for NR-U operation	MediaTek Inc.

R1-1806851	UL design for NR-U	OPPO

R1-1807002	NR Unlicensed physical channel design considerations	Apple Inc.

R1-1807035	On UL Signals and Channels in NR-Unlicensed 	InterDigital, Inc.

R1-1807082	UL Signals and Channels for NR-U operation	NTT DOCOMO, INC.

R1-1807157	On PUCCH for NR-U	Charter Communications

R1-1807388	UL signals and channels for NR-U	Qualcomm Incorporated

Agreement:

- An interlaced waveform can have benefits in some scenarios including

  - Link budget limited cases with given PSD constraint

  - As one option to efficiently meet the occupied channel bandwidth requirement.

- A waveform contiguous in frequency may be adequate in some scenarios

  - To inherit legacy contiguous allocation designs.

Note: It is RAN1’s understanding that the temporal allowance of not meeting occupied channel bandwidth by regulation can be exploited if the minimum bandwidth requirement, e.g., 2 MHz, is satisfied.

Agreement:

- Support for Rel-15 NR PUCCH formats can be considered. Exclusion of the support of certain formats is to be identified.

  - Note: It is RAN1’s understanding that certain formats do not meet the minimum bandwidth requirement by regulation.

- It is identified that block-interlaced based PUSCH can be beneficial.

- It is beneficial to use the same interlace structure for PUCCH and PUSCH.

- The following aspects can be considered for interlace waveform based PUCCH design:

  - Flexible number of OFDM symbols

  - Flexible payload size

  - User multiplexing

  - Number of formats

Agreement:

- Support for Rel-15 NR PRACH formats can be considered. Exclusion of the support of certain formats is to be identified.

  - Note: It is RAN1’s understanding that certain formats do not meet the minimum bandwidth requirement by regulation.

- It is identified that interlaced based PRACH can be beneficial.

- The following aspects can be considered for Interlace waveform based PRACH design for 4-step random access:

  - Interlacing based on PRB or REs

  - Targeted cell sizes

  - Targeted PRACH capacity

  - Targeted false alarm and detection rates

  - Targeted timing estimation accuracy

  - Number of formats

  - Multiplexing with other channels such as block interlaced PUCCH and PUSCH

### Potential physical layer procedures

#### Channel access procedures

R1-1805919	Coexistence and channel access for NR unlicensed band operations	Huawei, HiSilicon

R1-1806086	Discussion on the channel access procedures	vivo

R1-1806108	Channel access and co-existence for NR-U operation	Nokia, Nokia Shanghai Bell

R1-1806253	Channel access mechanisms	Ericsson

R1-1806317	Channel Access Procedures for NR Unlicensed Operations	CATT

R1-1806412	Considerations on the channel access procedures for NR-U	Spreadtrum Communications

R1-1806462	Discussion on Channel access mechanism for NR-U	ZTE Corporation

R1-1806544	Potential designs for NR unlicensed channel access mechanism	Intel Corporation

R1-1806569	Considerations on channel access for NR unlicensed operations	Sony

R1-1806645	Channel access procedure for NR unlicensed operation	LG Electronics

R1-1806670	Discussion on subband based channel access procedures for NR-U	NEC

R1-1806761	Channel access procedure for NR-U	Samsung

R1-1806797	Channel access to NR-based unlicensed spectrum	MediaTek Inc.

R1-1806852	Channel access mechanisms on NR-U	OPPO

R1-1807685	Channel access enhancements for NR-U	AT&T	(R1-1806989)

R1-1807003	NR Unlicensed channel access procedure considerations	Apple Inc.

R1-1807036	On NR-Unlicensed channel access procedures	InterDigital, Inc.

R1-1807083	Channel access procedures for NR-U operation	NTT DOCOMO, INC.

R1-1807161	Discussion on NR unlicensed band operation	ITRI

R1-1807205	Considerations on LBT in NR-U	CATR

R1-1807230	Discussion on Procedures for LBT in NR-U	Convida Wireless LLC

R1-1807231	Design Considerations for Network Assisted Channel Access 	Convida Wireless LLC

R1-1807240	Discussion on Channel Access Indication in NR-U	Convida Wireless LLC

R1-1807389	Channel access procedure for NR-U	Qualcomm Incorporated

Agreement:

- LTE-LAA channel access mechanism is adopted as baseline for 5GHz

  - Further enhancements not precluded

- LTE-LAA channel access mechanism is adopted as starting point of the design for 6GHz

  - Further enhancements not precluded

- For 5GHz band, a no-LBT option is beneficial for NR-U, such as for supporting fast A/N feedback, and is permitted per regulation.

  - Restrictions/conditions on when no-LBT option can be used will be further identified, e.g., in consideration of fair coexistence.

- No-LBT option can be applied to 6GHz band if allowed by regulation

  - Restrictions/conditions on when no-LBT option can be used will be further identified, if fair coexistence criterion is defined for 6GHz band

Note: Channel access mechanisms need to comply with regulations and may therefore need to be adapted for particular frequency ranges.

Agreement:

- Initial active DL/UL BWP is approximately 20MHz for 5GHz band

  - The final value will be quantized to number of PRBs

- Initial active DL/UL BWP is approximately 20MHz for 6GHz band if similar channelization as 5GHz band is used for 6GHz band

- FFS: Initial active DL/UL BWP for other applicable bands, including 60GHz

#### Initial access and mobility

R1-1805920	Initial access in NR unlicensed	Huawei, HiSilicon

R1-1806087	Discussion on RACH procedure for NR-U	vivo

R1-1806109	On Initial Access and Mobility for NR-U	Nokia, Nokia Shanghai Bell

R1-1806254	On initial access, RRM, mobility and RLM	Ericsson

R1-1806318	Initial Access and Mobiity for NR Unlicensed Operations	CATT

Withdrawn

R1-1806463	Discussion on initial access and mobility for NR-U	ZTE Corporation

R1-1806545	Potential enhancements to NR initial access and mobility to support unlicensed operation	Intel Corporation

R1-1806570	Considerations on initial access procedures for NR unlicensed operations	Sony

R1-1806646	Initial access and mobility for NR unlicensed operation	LG Electronics

R1-1806762	Initial access and mobility procedure for NR-U	Samsung

R1-1806798	Considerations on initial access and mobility for NR-U	MediaTek Inc.

R1-1806853	Initial access procedure on NR-U	OPPO

R1-1806990	Beam measurement for NR-U	AT&T

R1-1807037	On Initial Access in NR-Unlicensed Spectrum 	InterDigital, Inc.

R1-1807142	Initial access and mobility for NR-U	ITL

R1-1807390	Initial access and mobility for NR-U	Qualcomm Incorporated

Agreement:

The following modifications to initial access procedures are beneficial

- Modifications to initial access procedures considering limitations on access to the channel based on LBT

  - Develop techniques to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure

- Enhancement to 4-step RACH

  - Mechanisms to handle reduced msg 1/2/3/4 transmission opportunities due to LBT failure

- 2-step RACH potentially has benefit for channel access

Agreement:

Potential modifications to RLM/RRM procedures due to reduced transmission opportunities for DL signals and channels due to LBT failure should be identified and studied.

Agreement:

Modifications to paging procedures due to reduced transmission opportunities for paging due to LBT failure are beneficial and should be identified and studied.

#### Potential HARQ enhancements

R1-1805918	HARQ enhencement in NR unlicensed	Huawei, HiSilicon

R1-1806088	Discussion on HARQ operation for NR-U	vivo

R1-1806110	HARQ Enhancements for NR-Unlicensed	Nokia, Nokia Shanghai Bell

R1-1806255	HARQ enhancements for NR-U	Ericsson

R1-1806319	HARQ Operations in NR-U	CATT

R1-1806346	HARQ-ACK transmission on unlicensed carriers	Lenovo, Motorola Mobility

R1-1806464	Discussion on scheduling and HARQ for NR-U	ZTE Corporation

R1-1806546	Potential enhancements to NR scheduling and HARQ to support unlicensed operation	Intel Corporation

R1-1806647	HARQ operation for NR unlicensed operation	LG Electronics

R1-1806671	Discussion on HARQ enhancements for NR-U	NEC

R1-1806763	HARQ enhancements for NR-U	Samsung

R1-1806854	HARQ procedure on NR-U	OPPO

R1-1807038	Discussion on HARQ Procedure in NR-Unlicensed 	InterDigital, Inc.

R1-1807158	HARQ Feedback Principles	Charter Communications

R1-1807206	Discussions on HARQ enhancement for NR-U	CATR

R1-1807304	Considerations on HARQ operation for NR-U	KT Corp.

Late submission

R1-1807391	Potential HARQ enhancements for NR-U	Qualcomm Incorporated

Agreement:

- Transmission of HARQ A/N for the corresponding data in the same shared COT is identified as beneficial

  - Strive to support transmitting all HARQ A/N for the corresponding data in the same shared COT, if possible, considering the current NR UE processing time required

    - Mechanisms to support this need to be identified

  - It is understood in some cases, the HARQ A/N has to be transmitted in a separate COT from the one the corresponding data was transmitted

    - Mechanisms to support this need to be identified

Agreement:

- Techniques to handle reduced HARQ A/N transmission opportunities for a given HARQ process due to LBT failure are identified as beneficial

  - Potential techniques include mechanisms to provide multiple and/or supplemental time and/or frequency domain transmission opportunities

Agreement:

- NR-U uses NR HARQ feedback mechanisms as baseline, and enhancements can be identified

- When UL HARQ feedback is transmitted on unlicensed band, NR-U considers mechanisms to support flexible triggering and multiplexing of HARQ feedback for one or more DL HARQ processes

Agreement:

- Scheduling multiple TTIs for PUSCH each using a separate UL grant in the same PDCCH monitoring occasion is identified as beneficial

- Scheduling multiple TTIs for PUSCH using a single UL grant is identified as beneficial and should be supported in NR-U

#### Potential enhancements to configured grants

R1-1805922	Transmission with configured grant in NR unlicensed band	Huawei, HiSilicon

R1-1806111	On Support of UL Transmission with Configured Grants in NR-U	Nokia, Nokia Shanghai Bell

R1-1806256	Configured UL grant for NR-U	Ericsson

R1-1806320	DL control design for NR-U	CATT

Withdrawn

R1-1806413	Consideration on autonomous UL transmission in NR-U operation	Spreadtrum Communications

R1-1806465	Discussion on configured grant for NR-U	ZTE Corporation

R1-1806547	Potential enhancements to NR configured grants to support unlicensed operation	Intel Corporation

R1-1806571	Considerations on Configured Grant for NR-U	Sony

R1-1806648	Discussion on configured grant for NR unlicensed operation	LG Electronics

R1-1806764	Enhancements on configured grant for NR-U	Samsung

R1-1807039	Discussion on Configured Grants in NR-Unlicensed 	InterDigital, Inc.

R1-1807392	Potential enhancements to configured grants for NR-U	Qualcomm Incorporated

Agreement:

- The following modifications to the configured grant procedures are beneficial

  - Removing dependencies of HARQ process information to the timing

    - Introducing UCI on PUSCH to carry HARQ process ID, NDI, RVID

  - Introducing Downlink Feedback Information (DFI) including HARQ feedback for configured grant transmission

  - Increased flexibility on time domain resource allocation for the configured grant transmissions

  - Supporting retransmissions without explicit UL grant

#### Other

R1-1805923	BWP operation in NR unlicesned band	Huawei, HiSilicon

R1-1806112	On Wideband Operation for NR-U	Nokia, Nokia Shanghai Bell

R1-1806548	Potential designs to support spatial reuse for NR-unlicensed.	Intel Corporation

R1-1806855	Paging transmission on NR-U	OPPO

R1-1806856	SS/PBCH block transmission on NR-U	OPPO

R1-1806968	BWP operation in unlicensed spectrum	InterDigital, Inc.

R1-1807237	Discussion on NR-U in Wider Band	Convida Wireless LLC

### Other

R1-1805989	Waveform considerations for NR-U in the 60GHz band	Mitsubishi Electric RCE

R1-1806257	On NR operation in 60GHz unlicensed band	Ericsson

R1-1806258	On PRACH for NR in Unlicensed Spectrum	Ericsson

R1-1806259	On interlace design for NR-U uplinks	Ericsson

R1-1806260	On SRS design for NR unlicensed operation	Ericsson

R1-1806261	NR-U PUCCH Design	Ericsson

R1-1806444	On Maximum TX Power for UL Interlace Structures	Nokia, Nokia Shanghai Bell

R1-1806549	Initial thoughts on technology neutral design for greenfield	Intel Corporation

R1-1806906	Zero-Autocorrelation zone of frequency-interlaced PRACH	Huawei, HiSilicon

R1-1807040	Discussion on Potential Waveform Solutions for NR Unlicensed 	InterDigital, Inc.

R1-1807229	Discussion on SSB design for NR-U	Potevio

## Study on Integrated Access and Backhaul for NR - SID in RP-172290

### Enhancements to support NR backhaul links

R1-1806649	Discussions on NR IAB support	LG Electronics

- Proposal 1: It is supported that access and backhaul use different numerology.

  - Dynamic BWP switching in backhaul link is not supported

  - Enhancements on scheduling in consideration of multi-beam or dual connectivity can be further considered.

  - At least semi-static SFI is supported for backhaul link.

  - Non-slot scheduling is supported in backhaul link.

- Proposal 2: Dynamic point selection and coordinated muting are further studied for both cases of same or different cell IDs between donor and relay nodes. Intra-frequency dual connectivity can be considered between donor and relay node with different cell IDs.

- Proposal 3: Cell selection/measurement mechanisms on unequal priority between donor and relay nodes need to be further studied. The number of hops need to be also taken into account.

- Proposal 4: For the initial access, min(RSRP) from DgNB to UE can be considered.

- Proposal 5: Study resource/contents of channels/signals for initial access procedure of IAB-node.

- Proposal 6: D/U alignment between parent node and child node can be considered and symbol boundary among simultaneous Tx/Rx links should be aligned in one node.

- Proposal 7: It should be further studied that resource coordination to minimize cross-link interference in IAB scenario.

- Proposal 8: In determining timing for backhaul and access links, it is necessary to consider potential impacts of dynamic path change in backhaul link on access link performance. It seems desirable that timing of access link is not changed even if backhaul link path may change.

- Proposal 9: Consider Option (a), (b) and (c) in Section 4.2 for scheduling IAB links.

Decision: The document is noted.

R1-1806660	NR support for IAB	Nokia, Nokia Shanghai Bell

- Observation 1: Resource allocation schemes developed for different access link scenarios can be reused for BH link.

- Observation 2: Detected cells and related measurement results obtained during the cell search/selection phase can be utilized as the basis for the cell monitoring during the active IAB operation.

- Observation 3: GC-PDCCH can enable interference coordination between DgNB cell and RN cell(s) with dynamic split between Access/BH in such that the network can be operated free from cross-link interference

- Observation 4: NR cell selection and initial access procedures with TA based IAB timing adjustment can be used as the baseline for IAB synchronization.

- Proposal 1: Support half-duplex IAB scenarios with both TDM and SDM/FDM between Access and BH links.

- Proposal 2: GC-PDCCH enhancements are needed to facilitate dynamic resource allocation between the backhaul and access links as well as related interference coordination.

- Proposal 3: RN processing time capabilities need to be discussed as part of the IAB studies.

- Proposal 4: To enable inter-IAB node monitoring there shall be means to configure measurement opportunities for some IAB nodes matching with the reference signal transmissions of the monitored IAB nodes.

- Proposal 5: RAN1 is asked to consider CSI-RS measurements as a suitable option for IAB monitoring.

- Proposal 6: Cross-link interference management related to half-duplex operation with SDM/FDM between access and backhaul needs to be considered as part of NR studies.

- Proposal 7: Develop a unified cross-link interference management framework under single WI, e.g. flexible duplexing or MIMO, covering all potential use-cases, e.g. dynamic TDD and IAB, etc.

- Proposal 8. TA based synchronization can be taken as basis for IAB synchronization supporting also multi-hop scenarios.

- Proposal 9. Spectrum efficiency improvements can be considered generic NR features and do not have to be studied separately for IAB cases.

Decision: The document is noted.

R1-1806970	Details of physical layer enhancements for IAB	AT&T

- Proposal 1: Orthogonal time periods are required for SSB transmissions intended for backhaul link discovery and maintenance with a possibly different periodicity of transmission and separately configured from the parameters/resources used by access UEs.

- Proposal 2: IAB should support coordination mechanisms for the partitioning and configuration of CSI-RS resources for topology and route management measurements across multiple backhaul hops.

- Proposal 3: IAB should support coordination mechanisms for the partitioning of time slots and frequency resources across multiple backhaul hops to support orthogonal resources for access and backhaul links at a given IAB node.

- Proposal 4: IAB should support frame structure coordination mechanisms which enable alignment of DL transmission with UL reception slots as well as alignment of DL reception with UL transmission slots at the IAB node.

- Proposal 5: IAB supports dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment at the IAB node UE-f.

- Proposal 6: CLI mitigation techniques such as advanced receivers, transmitter coordination, etc. should be studied and prioritized in terms of complexity and performance.

- Proposal 7: CLI measurements such as short term and long term measurements, multiple antenna and beamforming based measurements should be studied to enable CLI mitigation in IAB.

- Proposal 8: RAN1 should study over the air synchronization of IAB nodes using existing Rel. 15 initial access signal such as PSS, SSS, etc., as well as study enhancements to the synchronization signal to allow for multiple hops.

- Proposal 9: IAB nodes should use timing advance received from the parent node to align its access subframes. The following options should be supported for over the air synchronization:

  - Option 1: The relative timing equal to TA/2 between the DL subframe on the backhaul and DL subframe on the access

  - Option 2: FFS

- Proposal 10: Study DL and UL power control enhancements to allow for intra-panel FDM and SDM of backhaul and access links.

- Proposal 11: IAB should support access and backhaul traffic multiplexing on single link (DL or UL) using multi-user MIMO transmission schemes.

- Proposal 12: Consider to support 8 layers of PUSCH transmission in the spec.

Decision: The document is noted.

R1-1807393	Enhancements to support NR backhaul links	Qualcomm Incorporated

- Proposal 1.1: NR studies how to handle multiplexing of RACH transmission from access and backhaul links.

- Proposal 2.1: simple modifications to the NR R15 RRM framework should be considered to make it more suitable for backhaul operations.

  - FFS: new values for CSI-RS/SMTC periodicity, maximum number of SMTC configured per frequency, flexible time-domain location of the SSBs within a SMTC.

- Proposal 2.2: IAB-nodes should adopt a TX/RX coordination (muting pattern) to enable discovering/measuring other IAB-nodes while being discoverable/measurable by the other IAB-nodes.

- Proposal 2.3: further study the details of the configuration of the TX/RX coordination and its required signaling.

- Proposal 2.4: adopt the proposed framework in Table 3 for initial acquisition, inter-IAB-node discovery and BH measurements.

Table 3: reference signals used for various BH operations

| BH procedure | Baseline ref signals | Additional ref signals |
| --- | --- | --- |
| Initial acquisition | CD SSBs | Off-raster SSBs |
| Inter-IAB-node discovery | Off-raster SSBs | CD SSBs, CSI-RS |
| BH measurements | On-demand CSI-RS | CD SSBs, off-raster SSBs |

- Proposal 3.1: Over-the-air (OTA) synchronization (over Uu interface) can be used to synchronize a multi-hop IAB network to the IAB-donors.

- Proposal 3.2: network should synchronize the IAB-donors using the available techniques, e.g. using GPS/GNSS, Ethernet, etc.

- Proposal 4.1: the slot boundaries across multi-hop IAB network should be aligned.

- Proposal 5.1: Further study is required to understand whether and how a DU of an IAB-node can ensure no DL/UL transmissions on an unallocated resource unit by using existing NR framework.

- Proposal 5.2: For resource partitioning on time-domain, the partitioning granularity, whether to be a symbol, a mini-slot, a slot or multiple slots, and its impact on various procedures shall be studied.

- Proposal 6.1: donor and relay maintain separate cell IDs.

- Proposal 6.2: during NR-IAB SI, we should prioritize discussions of the design aspects that are fundamentally required for NR-IAB operation. Discussing enhancements (e.g. CLI, higher modulation, multi-link/multi-TRP, etc) should be deferred to WI or leverage a unified study with other NR features.

Decision: The document is noted.

R1-1807703	Summary of 7.7.1 Enhancements to support NR backhaul links	AT&T

Decision: The document is noted.

Proposals:

- To support the half-duplex constraint from the perspective of a given IAB node, IAB supports candidate backhaul link discovery (after initial access) which utilizes resources that are orthogonal in time from those used by access UEs for access link discovery.

- The following solutions for candidate IAB node discovery can be further considered:

  - TDM of SSBs (e.g. depending on hop order, cell ID, etc.)

  - SSB muting across IAB nodes

  - Multiplexing of SSBs for access UEs and IABs within a half-frame or across half-frames

  - Additional IAB node discovery signal TDM with SSB (e.g. CSI-RS)

  - Use of off-raster SSBs

  - Different transmission periodicity compared to the periodicity used by access UEs

- Further study coordination mechanisms for different solutions

Agreements:

- IAB supports SSB and CSI-RS based RRM measurements. The following features can be considered:

  - Mechanisms for coordination of RS transmission and measurement occasions for IAB nodes

  - Enhancements of SMTC and CSI-RS configurations for IAB

- An IAB-node supports mechanisms for detecting/recovering from backhaul link failure based on Rel-15 mechanisms.

  - Study enhancements to RLM RS and associated procedures for IAB

Agreements:

- IAB supports TDM, FDM, and SDM between Access and BH links at an IAB node, subject to a half-duplex constraint. Further study the following solutions for the different multiplexing options:

  - Mechanisms for orthogonal partitioning of time slots or frequency resources between access and backhaul links across one or multiple hops

  - Utilization of different DL/UL slot configurations for access and backhaul links

  - DL and UL power control enhancements and timing requirements to allow for intra-panel FDM and SDM of backhaul and access links.

  - Interference management including cross-link interference

  - Note: the level of required enhancement or optimization for the different options is FFS

Agreements:

- IAB supports TA-based synchronization between IAB nodes, including across multiple backhaul hops

  - Enhancements to existing mechanisms can be further studied

- The following cases should be further studied:

  - Case 1: DL transmission timing alignment across IAB nodes and donor nodes

  - Case 2: DL and UL transmission timing is aligned within an IAB node

  - Case 3: DL and UL reception timing is aligned within an IAB node

  - Case 4: within an IAB node, when transmitting using case 2 while when receiving using case 3

  - Case 5: Case 1 for access link timing and Case 4 for backhaul link timing within an IAB node in different time slots

- Further study the following levels of alignment between IAB nodes/donor nodes or within an IAB node:

  - Slot alignment

  - Symbol-level alignment

  - No alignment

- Further consider the impact of different cases on TDM/FDM/SDM multiplexing of access and backhaul links, cross-link interference, and impact on access UEs

Thursday session

R1-1807769	Summary of 7.7.1 Enhancements to support NR backhaul links	AT&T

Decision: The document is noted.

Agreements:

- To support the half-duplex constraint from the perspective of a given IAB node, IAB supports detection and measurement of candidate backhaul links (after initial access) which utilizes resources that are orthogonal in time from those used by access UEs for cell detection and measurement.

- The following solutions can be further considered:

  - TDM of SSBs (e.g. depending on hop order, cell ID, etc.)

  - SSB muting across IAB nodes

  - Multiplexing of SSBs for access UEs and IABs within a half-frame or across half-frames

  - Additional IAB node discovery signal TDM with SSB (e.g. CSI-RS)

  - Use of off-raster SSBs

  - Different transmission periodicity compared to the periodicity used by access UEs

- Further study coordination mechanisms for different solutions

Agreement: Study mechanisms for multiplexing of RACH transmissions from UEs and RACH transmissions from IAB nodes.

Agreements:

- Downlink IAB transmissions (transmissions from an IAB node to child IAB nodes and UEs directly under the IAB node) should be scheduled by the IAB node itself.

- Uplink IAB transmission (transmissions from an IAB node to its parent node) should be scheduled by the parent node.

- Semi-static (on the timescale of RRC signalling) should be supported for resource (frequency, time in terms of slot/slot format, etc.) coordination between IAB nodes.

- The following aspects should be further studied:

  - Distributed or centralized coordination mechanisms

  - Resource granularity of the required signalling (e.g. TDD configuration pattern)

  - Exchange of L1 and/or L3 measurements between IAB nodes

  - Exchange of topology related information (e.g. hop order) impacting RAN1 study

  - Resource (frequency, time in terms of slot/slot format, etc.) coordination which is faster than semi-static coordination

Agreements:

- CLI mitigation techniques including advanced receivers and transmitter coordination should be studied and prioritized in terms of complexity and performance.

- CLI mitigation techniques should be able to manage the following inter IAB node interference scenarios:

  - Case 1: Victim IAB node is receiving in DL in the backhaul link, interfering IAB node is transmitting in UL in the backhaul link.

  - Case 2: Victim IAB node is receiving in DL in the backhaul link, interfering IAB node is transmitting in DL in the access link.

  - Case 3: Victim IAB node is receiving in UL in the access link, interfering IAB node is transmitting in UL in the backhaul link.

  - Case 4: Victim IAB node is receiving in UL in the access link, interfering IAB node is transmitting in DL in the access link.

- Note: In this case access links include links to and from the IAB node to child IAB nodes and UEs which are served by the IAB node

- CLI measurements such as short-term and long term measurements, and multiple-antenna and beamforming based measurements should be studied to enable CLI mitigation in IAB.

- Mechanisms for inter IAB node CLI measurement need to be able to capture Cases 1-4.

Agreement:

- Interference experienced at the IAB node in case of FDM/SDM reception between access and backhaul links at a given IAB node should be further studied.

R1-1805925	Physical layer enhancement consideration on IAB	Huawei, HiSilicon

R1-1806024	Overview of physical layer enhancements for IAB	ZTE

R1-1806089	Enhancements to support NR backhaul link	vivo

R1-1806321	NR Physical Layer design for IAB backhual link	CATT

R1-1806374	Discussion on enhancements for IAB	CMCC

R1-1806551	PHY layer enhancements for NR IAB	Intel Corporation

R1-1806572	Discussion on NR enhancements to support IAB	Sony

R1-1806765	Necessary Enhancements for NR IAB	Samsung

R1-1806821	On enhancements for IAB	China Telecommunications

R1-1806859	Discussion of backhaul link enhancement for IAB	OPPO

R1-1807221	Physical-layer aspects related to IAB	Ericsson

### Evaluation methodology for NR IAB

Including deployment scenarios, evaluation assumptions, etc.

R1-1806552	Evaluation methodology for NR IAB	Intel Corporation

- Observation 1: In heterogeneous deployment, cases with two backhaul hops can be observed for ISD = 200m, 500m, 1000m. The number of backhaul hops increases as the macro layer ISD increases.

- Observation 2: Increasing the number of large-scale realization N from 1 to 5 with Alternative 1 leverages the pathloss gain added to the backhaul link and the effect of applying Alternative 1 to backhaul channel realizations appears to be different from low to high pathloss environments.

- Observation 3: Varying the number of N from 1 to 5 with Alternative 1 does not have much impact on the distribution of the number of backhaul hops.

- Proposal 1: The remaining issues of IAB deployment scenarios need to be addressed.

|  | Homogeneous IAB scenario | Heterogeneous IAB scenario |
| --- | --- | --- |
| RN deployment | Planned dropping | Random or planned dropping |
| RN’s antenna | Three sectors/panels | Two sectors/panels and three sectors/panels |
| RN’s sector/panel boresight | Planned orientation | Random or planned orientation |
| Macro layer ISD | 200m, 500m |  |

- Proposal 2: In NR IAB simulation, use Alternative 1 for path loss and LOS probability generation in the backhaul link.

- Proposal 3: Use the following steps in generating IAB network.

  - Step 0: Deploy a serving node set A with L DNs and an unassociated node set B with M RNs in the network area.

  - Step 1: Calculate the RSRP values of size |A|×|B|×N between the node in A and the node in B for N channel realizations.

  - Step 2: Sort RSRP(l, m, n) and pick (l*, m*, n*) = argmax{RSRP(l, m, n)}.

  - Step 3: Associate nodes l* and m*. The associated and non-associated channels between the node m* and the nodes in the set A are determined using n*-th channel realization.

  - Step 4: Add node m* into set A and remove node m* from set B.

  - Step 5: Repeat Steps 1-4 until all RNs are associated (i.e., until the set B is empty).

- Proposal 4: IAB node association could be based on RSRP of the immediate backhaul link and the backhaul load and number of hops throughout the multiple backhaul hops.

- Proposal 5: The following performance metrics are used for FTP traffics in IAB evaluation.

  - Per-hop resource utilization in the backhaul link and access link.

  - E2E UPT considering the finished packets and E2E packet delivery time through relaying [3].

Decision: The document is noted.

R1-1807642	Evaluation Methodology for NR IAB	Samsung	(rev of R1-1806766)

- Proposal 1: IAB should be evaluated with the following assumptions.

  - Physically fixed relays (imply that the benefit of planning should be captured in modelling)

  - In-band and out-of-band scenarios

  - NR access over NR backhaul

  - 30GHz for NR backhaul

  - Support of multiple backhaul hops

  - Topology adaptation to enable robust operation, e.g., mitigate blockage and load variation on backhaul links

  - Maximize the reuse of Rel-15 NR specifications

- Observation: For the “HetNet” scenario, replacing macro TRP with micro TRP for the donor BS can be considered as an additional option, if evaluation of topologies with larger number of hops on average is necessary.

- Proposal 2: The evaluation assumptions for the heterogeneous network is according to Table 1 in Sec 3.1.

- Proposal 3: Adopt Alt 1 with N=3 to determine the pathloss for links between the IAB node and other IAB nodes/donors.

  - Alt. 1: Determine the pathloss for links between the IAB node and candidate serving IAB nodes/donors based on N (value FFS but <= 5) independent large-scale channel realizations (taking into account LOS/NLOS probability and shadow fading).

  - Select the realization that results in the minimum pathloss between the IAB node and the selected serving IAB node/donor.

Decision: The document is noted.

R1-1807704	Summary of 7.7.2 Evaluation Methodology for NR IAB	AT&T

Decision: The document is noted.

Agreement:

- UEs in outage (which is defined as when UEs with traffic to be served but no packets have been delivered to higher layers by the end of the simulation) are included in the CDF for UPT.

Agreements:

- Update the previous agreements as follows:

  - Per-link SNR and Geometry can be reported per hop level

    - Detailed definition of per-link SNR FFS

  - Min(Geometry) of all links for a given UE route (access and one or more backhaul links) between a donor and UE can be reported

Thursday session

R1-1807770	Summary of 7.7.2 Evaluation Methodology for NR IAB	AT&T

Decision: The TP in R1-1807770 is endorsed.

R1-1805924	IAB system evaluation methodology and preliminary results	Huawei, HiSilicon

R1-1806025	IAB scenarios and evaluations	ZTE

R1-1806375	Discussion on evaluation methodology for IAB	CMCC

R1-1806650	Discussions on evaluation methodology on NR IAB	LG Electronics

R1-1806661	Evaluation Methodology for NR IAB	Nokia, Nokia Shanghai Bell

R1-1806971	Remaining details of IAB evaluation methodology	AT&T

R1-1807224	Duplexing options for IAB	Ericsson

R1-1807394	Evaluation methodology for NR IAB	Qualcomm Incorporated

### Other

Thursday

TP to capture IAB agreements to be prepared by Thomas (AT&T)

Friday

R1-1807850	TP for 38.874 capturing RAN1 agreements on IAB	AT&T

Decision: The document is endorsed.

R1-1806026	Discussion on IAB node initial access process	ZTE

R1-1806662	Dynamic resource allocation between backhaul and access link	Nokia, Nokia Shanghai Bell

R1-1806663	Measurements for IAB	Nokia, Nokia Shanghai Bell

R1-1806664	IAB Synchronization	Nokia, Nokia Shanghai Bell

R1-1806665	On Cross-link Interference Management in IA	Nokia, Nokia Shanghai Bell

R1-1806833	Utilising sidelink in IAB	OPPO

R1-1806907	On the performance of IAB solutions	Huawei, HiSilicon

R1-1806972	System level simulation results of IAB	AT&T

R1-1807226	Node failure in AB half duplexing	Nanjing Ericsson Panda Com Ltd

R1-1807227	Interference overview of IAB architecture	Ericsson

R1-1807395	Inter-IAB-node discovery	Qualcomm Incorporated

R1-1807396	Network synchronization for multi-hop IAB	Qualcomm Incorporated

R1-1807398	Inter-IAB-node discovery	Qualcomm Incorporated

R1-1807399	Network synchronization for multi-hop IAB	Qualcomm Incorporated

## Study on Self Evaluation towards IMT-2020 Submission

### Evaluation methodology

R1-1807758	Summary on offline discussion on eMBB evaluation method and parameters for IMT-2020 self-evaluation	Huawei

Decision: The document is noted.

Agreement: The proposals in Section 2 of R1-1807758 are agreed.

R1-1807759	Summary on offline discussion on mMTC evaluation method and parameters for IMT-2020 self-evaluation	Huawei

Decision: The document is noted.

Agreement: The proposals in Section 2 of R1-1807759 are agreed.

R1-1807760	Summary on offline discussion on URLLC evaluation method and parameters for IMT-2020 self-evaluation	Huawei

Discussion: CATT requested more time for reviewing and checking

Decision: The document is noted.

Agreement: The proposals in Section 2 of R1-1807760 are agreed with the following update:

- “…pre-processing SINR is encouraged to be used in system….”

R1-1805926	Consideration on evaluation features and parameters for IMT-2020 self evaluation	Huawei, HiSilicon

R1-1806118	Consideration on Evaluation methodology of IMT-2020 self-evaluation	ZTE

R1-1806322	Discussions on evaluation methodology for self-evaluation	CATT

R1-1806430	On IMT-2020 Evaluation methodology	Ericsson

R1-1806553	On the IMT-2020 self-evaluation performance metrics and evaluation methods	Intel Corporation

R1-1806767	Evaluation methodology for IMT-2020 submission	Samsung

R1-1806865	Consideration on mobility evaluation method	OPPO

R1-1807283	On Evaluation of the Mobility Parameter for IMT-2020	Nokia, Nokia Shanghai Bell

### Other

Including evaluation results

R1-1805927	Considerations and evaluation results for IMT-2020 for eMBB spectral efficiency	Huawei, HiSilicon

R1-1805928	IMT-2020 initial evaluation results for user experienced data rate	Huawei, HiSilicon

R1-1805929	Considerations and evaluation results for IMT-2020 for area traffic capacity	Huawei, HiSilicon

R1-1805930	Considerations and evaluation results for IMT-2020 for high mobility	Huawei, HiSilicon

R1-1805931	Considerations and evaluation results for IMT-2020 for peak spectral efficiency and peak data rate	Huawei, HiSilicon

R1-1805932	Considerations and evaluation results for IMT-2020 for network energy efficiency	Huawei, HiSilicon

R1-1805933	Considerations and evaluation results for IMT-2020 for UE energy efficiency	Huawei, HiSilicon

R1-1805934	Considerations and evaluation results for IMT-2020 for mMTC connection density	Huawei, HiSilicon

R1-1805935	Considerations and evaluation results for IMT-2020 for reliability	Huawei, HiSilicon

R1-1805936	Considerations and evaluation results for IMT-2020 for control plane latency	Huawei, HiSilicon

R1-1805937	Considerations and evaluation results for IMT-2020 for user plane latency	Huawei, HiSilicon

R1-1805938	Consideration on self evaluation of IMT-2020 for link budget	Huawei, HiSilicon

R1-1806113	Updated UMi calibration results for user experience data rate in the multi-layer network deployment	ZTE

R1-1806114	Updated DL spectral efficiency in the eMBB usage scenario for IMT-2020 self evaluation	ZTE

R1-1806115	Link curve performance of LDPC coding in IMT-2020 self evaluation	ZTE

R1-1806116	Considerations on the evaluated details and parameters for IMT-2020 self evaluation	ZTE

R1-1806117	Updated UL spectral efficiency in the eMBB usage scenario for IMT-2020 self evaluation	ZTE

R1-1806119	Consideration on Evaluation of mobility and reliability of IMT-2020 self-evaluation	ZTE

R1-1806323	Discussions on detailed link budget evaluation methodology	CATT

R1-1806324	Discussions on calculation of shadow fading margin  for link budget	CATT

R1-1806325	Discussions on penetration margin and SD for link budget	CATT

R1-1806326	Discussions on system level simulation configurations and technical features for self evaluation	CATT

R1-1806327	Initial self evaluation on peak data rate and peak spectrum efficiency	CATT

R1-1806328	Initial self evaluation for Indoor Hotspot-eMBB	CATT

R1-1806329	Initial self evaluation for Dense Urban-eMBB	CATT

R1-1806330	Initial self evaluation for Rural-eMBB	CATT

R1-1806331	Initial self evaluation on control plane latency	CATT

R1-1806332	Initial self evaluation on user plane latency	CATT

R1-1806333	Discussions and initial self evaluation on energy efficiency	CATT

R1-1806334	Initial self evaluation on mobility	CATT

R1-1806376	Initial self evaluation results for eMBB spectrum efficiency	CMCC

R1-1806377	Initial self evaluation results for mobility in eMBB scenario	CMCC

R1-1806378	Initial self evaluation results for connection density in mMTC scenario	CMCC

R1-1806379	Initial self evaluation results for reliability in URLLC scenario	CMCC

R1-1806380	Initial link-level self evaluation results for mobility in eMBB scenario	CMCC

R1-1806431	An overview of the IMT-2020 Evaluations	Ericsson

R1-1806432	IMT-2020 Self-Evaluation: NR Link Budgets	Ericsson

R1-1806434	IMT 2020 Evaluations: On eMBB user experienced data rate with NR	Ericsson

R1-1806435	IMT 2020 Evaluations: On eMBB area traffic capacity with NR	Ericsson

R1-1806436	IMT-2020 self-evaluation: mMTC non-full-buffer connection density	Ericsson

R1-1806437	IMT-2020 self-evaluation: mMTC full buffer connection density	Ericsson

R1-1806438	IMT-2020 self-evaluation: Mobility evaluations for NR	Ericsson

R1-1806439	IMT-2020 self-evaluation: Peak data rate and peak spectrum efficiency evaluations for NR	Ericsson

R1-1806440	IMT-2020 self evaluation: Radio Network Energy Performance	Ericsson

R1-1806441	IMT 2020 Evaluations - CP latency in NR	Ericsson

R1-1806442	IMT 2020 Evaluations - UP latency in NR	Ericsson

R1-1806443	IMT 2020 Evaluations - Reliability in NR	Ericsson

R1-1806554	Initial Results for IMT-2020 self-evaluation	Intel Corporation

R1-1806651	Initial evaluation results of IMT-2020 self-evaluation for high mobility	LG Electronics

R1-1806652	Initial evaluation results of Rural-eMBB for IMT-2020 self-evaluation	LG Electronics

R1-1806653	Initial evaluation results of Indoor Hotspot-eMBB for IMT-2020 self-evaluation	LG Electronics

R1-1806654	Initial evaluation results of Dense Urban-eMBB for IMT-2020 self-evaluation	LG Electronics

R1-1806822	Initial self-evaluation for Indoor Hotspot-eMBB	China Telecommunications

R1-1806823	Link level performance for LDPC coding with BP decoder	China Telecommunications

Withdrawn

R1-1807084	Evaluation results of Indoor Hotspot-eMBB for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1807085	Evaluation results of dense urban-eMBB for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1807086	Evaluation results of rural-eMBB for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1807087	Preliminary evaluation results of mobility for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1807284	Spectral Efficiency Results for the IMT-2020 Self-Evaluation	Nokia, Nokia Shanghai Bell

R1-1807285	IMT-2020 Reliability Evaluation Results (URLLC)	Nokia, Nokia Shanghai Bell

R1-1807286	IMT-2020 Connection Density Evaluation Results (mMTC)	Nokia, Nokia Shanghai Bell

R1-1807287	IMT-2020 self evaluation: Control plane latency	Nokia, Nokia Shanghai Bell

R1-1807288	IMT-2020 self evaluation: User plane latency	Nokia, Nokia Shanghai Bell

R1-1807289	IMT-2020 self evaluation: Network energy efficiency	Nokia, Nokia Shanghai Bell

R1-1807609	IMT 2020 Evaluations: On eMBB spectral efficiency with NR	Ericsson	(R1-1806433)

R1-1807680	Initial evaluation results of the Dense Urban-eMBB scenario for IMT-2020 evaluation	Motorola Mobility, Lenovo

## Other

R1-1806090	Measurement results and analysis on UE power consumption	vivo

R1-1806091	NR UE power saving	vivo

R1-1806803	On UE power saving for NR	MediaTek Inc.

R1-1806888	On coexistence between LTE and NR	Huawei, HiSilicon

# Closing of the meeting

RAN1 chair commented that RAN1#93 was the last meeting for Rel-15. At this point in time, none knows what's going to be approved in the upcoming plenary with regards to the next release. Together RAN1 made a good progress, and should be proud of the achievement; RAN1 chair expressed his appreciation for the mutual trust between each other...

From RAN1 perspective, Release 15 is completed.

RAN1 chair thanked the host (Samsung) for the location and the well organization through the full week, especially the free lunch was very nice. Thanks to all the delegates for their great efforts, to the feature leads for their implication. Apologizes for scheduling so many offline sessions outside the main meeting rooms.

Meeting was closed at 17:30.

See you all in Gothenburg.

# Annex A:	List of Tdocs at RAN1 #93

Please see excel file attached to this report

# Annex B:	List of CRs agreed at RAN1 #93

| TS/TR | CR | Rev | Rel | Title | Cat | Vsn | @ Mtg | TD# | Source to WG | Work Item |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36.213 | 1076 | - | Rel-13 | Correction of a typo further to CR0594 faulty implementation | F | 13.9.0 | RAN1#93 | R1-1807423 | ETSI (MCC) | LTE_CA_enh_b5C-Core |
| 36.213 | 1077 | - | Rel-14 | Correction of a typo further to CR0594 faulty implementation | A | 14.6.0 | RAN1#93 | R1-1807424 | ETSI (MCC) | LTE_CA_enh_b5C-Core |
| 36.213 | 1078 | - | Rel-15 | Correction of a typo further to CR0594 faulty implementatio | A | 15.1.0 | RAN1#93 | R1-1807425 | ETSI (MCC) | LTE_CA_enh_b5C-Core |
| 36.213 | 1079 | - | Rel-13 | Clarification on beta offset for more than 22 HARQ-Ack bits | F | 13.9.0 | RAN1#93 | R1-1807432 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1080 | - | Rel-14 | Clarification on beta offset for more than 22 HARQ-Ack bits | A | 14.6.0 | RAN1#93 | R1-1807433 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1081 | - | Rel-15 | Clarification on beta offset for more than 22 HARQ-Ack bits | A | 15.1.0 | RAN1#93 | R1-1807434 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1082 | - | Rel-13 | Clarification on simultaneousAckNackAndCQI for format 4-5 and format 3 | F | 13.9.0 | RAN1#93 | R1-1807435 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1083 | - | Rel-14 | Clarification on simultaneousAckNackAndCQI for format 4-5 and format 3 | A | 14.6.0 | RAN1#93 | R1-1807436 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1084 | - | Rel-15 | Clarification on simultaneousAckNackAndCQI for format 4-5 and format 3 | A | 15.1.0 | RAN1#93 | R1-1807437 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1085 | - | Rel-13 | Clarification on PUCCH formats | F | 13.9.0 | RAN1#93 | R1-1807438 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1086 | - | Rel-14 | Clarification on PUCCH formats | A | 14.6.0 | RAN1#93 | R1-1807439 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1087 | - | Rel-15 | Clarification on PUCCH formats | A | 15.1.0 | RAN1#93 | R1-1807440 | Qualcomm Incorporated | LTE_CA_enh_b5C-Core |
| 36.213 | 1088 | - | Rel-13 | Correction on PUCCH configuration with CEModeA or CEModeB | F | 13.9.0 | RAN1#93 | R1-1807441 | Huawei Tech.(UK) Co., Ltd | LTE_MTCe2_L1-Core |
| 36.213 | 1089 | - | Rel-14 | Correction on PUCCH configuration with CEModeA or CEModeB | A | 14.6.0 | RAN1#93 | R1-1807442 | Huawei Tech.(UK) Co., Ltd | LTE_MTCe2_L1-Core |
| 36.213 | 1090 | - | Rel-15 | Correction on PUCCH configuration with CEModeA or CEModeB | A | 15.1.0 | RAN1#93 | R1-1807443 | Huawei Tech.(UK) Co., Ltd | LTE_MTCe2_L1-Core |
| 36.213 | 1095 | - | Rel-14 | Corrections in Physical Resource Block numbering to RA Type0 for FeMTC UEs | F | 14.6.0 | RAN1#93 | R1-1807480 | Samsung | LTE_feMTC-Core |
| 36.213 | 1096 | - | Rel-15 | Corrections in Physical Resource Block numbering to RA Type0 for FeMTC UEs | A | 15.1.0 | RAN1#93 | R1-1807481 | Samsung | LTE_feMTC-Core |
| 36.211 | 448 | - | Rel-15 | Introduction Rel-15 Further NB-IoT enhancements | B | 15.1.0 | RAN1#93 | R1-1807939 | Ericsson | NB_IOTenh2-Core |
| 36.212 | 293 | - | Rel-15 | Introduction of further enhancements of NB-IoT | B | 15.1.0 | RAN1#93 | R1-1807917 | Huawei | NB_IOTenh2-Core |
| 36.213 | 1104 | 1 | Rel-15 | Introduction of Rel-15 Further NB-IoT enhancements in 36.213, s14 | B | 15.1.0 | RAN1#93 | R1-1807940 | Motorola Mobility | NB_IOTenh2-Core |
| 36.214 | 50 | 1 | Rel-15 | Introduction Rel-15 Further NB-IoT enhancements | B | 15.1.0 | RAN1#93 | R1-1807941 | Ericsson | NB_IOTenh2-Core |
| 36.211 | 442 | 1 | Rel-13 | Clarification on CSI-RS resource aggregation | F | 13.9.0 | RAN1#93 | R1-1807587 | Qualcomm Incorporated | LTE_EBF_FDMIMO-Core |
| 36.211 | 443 | 1 | Rel-14 | Clarification on CSI-RS resource aggregation | A | 14.6.0 | RAN1#93 | R1-1807588 | Qualcomm Incorporated | LTE_EBF_FDMIMO-Core |
| 36.211 | 444 | 1 | Rel-15 | Clarification on CSI-RS resource aggregation | A | 15.1.0 | RAN1#93 | R1-1807589 | Qualcomm Incorporated | LTE_EBF_FDMIMO-Core |
| 36.211 | 449 | - | Rel-15 | Correction on layer-to-port mapping for feCoMP | F | 15.1.0 | RAN1#93 | R1-1807943 | Ericsson | feCOMP_LTE-Core |
| 36.213 | 1106 | - | Rel-15 | Corrections to feCoMP in 36.213, s06-s07 | F | 15.1.0 | RAN1#93 | R1-1807933 | Motorola Mobility | feCOMP_LTE-Core |
| 36.213 | 1093 | - | Rel-14 | Correction on RV determination for eVoLTE | F | 14.6.0 | RAN1#93 | R1-1807448 | Huawei, HiSilicon, CMCC | LTE_VoLTE_ViLTE_enh-Core |
| 36.213 | 1094 | - | Rel-15 | Correction on RV determination for eVoLTE | A | 15.1.0 | RAN1#93 | R1-1807449 | Huawei, HiSilicon, CMCC | LTE_VoLTE_ViLTE_enh-Core |
| 36.211 | 440 | 1 | Rel-15 | Corrections to sTTI and SPT implementation | F | 15.1.0 | RAN1#93 | R1-1807925 | Ericsson | LTE_sTTIandPT-Core |
| 36.212 | 280 | 2 | Rel-15 | Corrections on shortened processing time and shortened TTI in 36 212 | F | 15.1.0 | RAN1#93 | R1-1807913 | Huawei | LTE_sTTIandPT-Core |
| 36.213 | 1049 | 2 | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s00-s05 | F | 15.1.0 | RAN1#93 | R1-1807926 | Motorola Mobility Germany GmbH | LTE_sTTIandPT-Core |
| 36.213 | 1050 | 2 | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s06-s07 | F | 15.1.0 | RAN1#93 | R1-1807927 | Motorola Mobility Germany GmbH | LTE_sTTIandPT-Core |
| 36.213 | 1051 | 2 | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s10-s13 | F | 15.1.0 | RAN1#93 | R1-1807928 | Motorola Mobility Germany GmbH | LTE_sTTIandPT-Core |
| 36.213 | 1074 | 1 | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s08-s09 | F | 15.1.0 | RAN1#93 | R1-1807929 | Motorola Mobility Germany GmbH | LTE_sTTIandPT-Core |
| 36.213 | 1044 | 1 | Rel-14 | Support of early contention resolution | F | 14.6.0 | RAN1#93 | R1-1807966 | Qualcomm | TEI14, NB_IOT-Core |
| 36.213 | 1045 | 1 | Rel-15 | Support of early contention resolution | A | 15.1.0 | RAN1#93 | R1-1807967 | Qualcomm | TEI14, NB_IOT-Core |
| 36.213 | 1114 | - | Rel-15 | Correction for EN-DC Coexistence into TS36.213 s06-s07 | F | 15.1.0 | RAN1#93 | R1-1807952 | Motorola Mobility | NR_newRAT-Core |
| 36.213 | 1115 | - | Rel-15 | Correction for EN-DC Coexistence into TS36.213 s08-s09 | F | 15.1.0 | RAN1#93 | R1-1807953 | Motorola Mobility | NR_newRAT-Core |
| 36.213 | 1116 | - | Rel-15 | Correction for EN-DC Coexistence into TS36.213 s00-s05 | F | 15.1.0 | RAN1#93 | R1-1807964 | Motorola Mobility | NR_newRAT-Core |
| 36.213 | 1117 | - | Rel-15 | Correction for EN-DC Coexistence into TS36.213 s10-s13 | F | 15.1.0 | RAN1#93 | R1-1807965 | Motorola Mobility | NR_newRAT-Core |
| 38.202 | 2 | 1 | Rel-15 | CR to 38.202 capturing the RAN1#92bis and RAN1#93 meeting agreements | F | 15.1.0 | RAN1#93 | R1-1807954 | Qualcomm | NR_newRAT-Core |
| 38.211 | 2 | 1 | Rel-15 | CR to 38.211 capturing the RAN1#92bis and RAN1#93 meeting agreements | F | 15.1.0 | RAN1#93 | R1-1807955 | Ericsson | NR_newRAT-Core |
| 38.212 | 2 | 1 | Rel-15 | CR to 38.212 capturing the RAN1#92bis and RAN1#93 meeting agreements | F | 15.1.1 | RAN1#93 | R1-1807956 | Huawei | NR_newRAT-Core |
| 38.213 | 2 | 1 | Rel-15 | CR to 38.213 capturing the RAN1#92bis and RAN1#93 meeting agreements | F | 15.1.0 | RAN1#93 | R1-1807957 | Samsung | NR_newRAT-Core |
| 38.214 | 2 | 1 | Rel-15 | CR to 38.214 capturing the RAN1#92bis and RAN1#93 meeting agreements | F | 15.1.0 | RAN1#93 | R1-1807958 | Nokia | NR_newRAT-Core |
| 38.214 | 4 | - | Rel-15 | CR to 38.214: maintenance according to agreed Rel 15 features | F | 15.1.0 | RAN1#93 | R1-1807968 | Nokia | NR_newRAT-Core |
| 38.215 | 3 | 1 | Rel-15 | CR to 38.215 capturing the RAN1#92bis and RAN1#93 meeting agreements | F | 15.1.0 | RAN1#93 | R1-1807959 | Intel | NR_newRAT-Core |
| 36.212 | 284 | 1 | Rel-15 | Introduction of high capacity stationary wireless link and 1024QAM into 36.212 | B | 15.1.0 | RAN1#93 | R1-1807915 | Huawei | LTE_1024QAM_DL-Core |
| 36.213 | 1053 | 1 | Rel-15 | Introduction of enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE to 36.213 | B | 15.1.0 | RAN1#93 | R1-1807942 | Motorola Mobility Germany GmbH | LTE_1024QAM_DL-Core |
| 36.211 | 447 | - | Rel-15 | Introduction of Even Further Enhanced MTC for LTE | B | 15.1.0 | RAN1#93 | R1-1807935 | Ericsson | LTE_eMTC4-Core |
| 36.212 | 292 | - | Rel-15 | Introduction of even further enhancements of eMTC | B | 15.1.0 | RAN1#93 | R1-1807916 | Huawei | LTE_eMTC4-Core |
| 36.213 | 1101 | 1 | Rel-15 | Introduction of even further enhanced MTC for LTE in 36.213, s00-s05 | B | 15.1.0 | RAN1#93 | R1-1807936 | Motorola Mobility | LTE_eMTC4-Core |
| 36.213 | 1102 | 1 | Rel-15 | Introduction of even further enhanced MTC for LTE in 36.213, s06-s07 | B | 15.1.0 | RAN1#93 | R1-1807937 | Motorola Mobility | LTE_eMTC4-Core |
| 36.213 | 1103 | 1 | Rel-15 | Introduction of even further enhanced MTC for LTE in 36.213, s08-s09 | B | 15.1.0 | RAN1#93 | R1-1807938 | Motorola Mobility | LTE_eMTC4-Core |
| 36.213 | 1100 | - | Rel-15 | Short CQI Reporting for EuCA | B | 15.1.0 | RAN1#93 | R1-1807552 | Nokia, Nokia Shanghai Bell | LTE_euCA-Core |
| 36.201 | 26 | - | Rel-15 | Introduction of V2X Phase 2 enhancements into 36.201 | B | 15.0.0 | RAN1#93 | R1-1807414 | Nokia, Nokia Shanghai Bell | LTE_eV2X-Core |
| 36.212 | 294 | - | Rel-15 | Introduction of V2X Phase 2 based on LTE | B | 15.1.0 | RAN1#93 | R1-1807918 | Huawei | LTE_eV2X-Core |
| 36.213 | 1107 | - | Rel-15 | Addition of V2X Phase 2 based on LTE in 36.213, s14 | B | 15.1.0 | RAN1#93 | R1-1807944 | Motorola Mobility, LG Electronics | LTE_eV2X-Core |
| 36.211 | 450 | - | Rel-15 | Introduction of HRLLC into 36.211 | B | 15.1.0 | RAN1#93 | R1-1807945 | Ericsson | LTE_HRLLC-Core |
| 36.212 | 295 | - | Rel-15 | Introduction of HRLLC into 36.212 | B | 15.1.0 | RAN1#93 | R1-1807919 | Huawei | LTE_HRLLC-Core |
| 36.213 | 1108 | - | Rel-15 | Introduction of HRLLC for LTE in 36.213, s07-1 | B | 15.1.0 | RAN1#93 | R1-1807946 | Motorola Mobility | LTE_HRLLC-Core |
| 36.213 | 1109 | - | Rel-15 | Introduction of HRLLC for LTE in 36.213, s07-3 | B | 15.1.0 | RAN1#93 | R1-1807947 | Motorola Mobility | LTE_HRLLC-Core |
| 36.213 | 1110 | - | Rel-15 | Introduction of HRLLC for LTE in 36.213, s08 | B | 15.1.0 | RAN1#93 | R1-1807948 | Motorola Mobility | LTE_HRLLC-Core |
| 36.213 | 1111 | - | Rel-15 | Introduction of HRLLC for LTE in 36.213, s09 | B | 15.1.0 | RAN1#93 | R1-1807949 | Motorola Mobility | LTE_HRLLC-Core |
| 36.213 | 1112 | - | Rel-15 | Introduction of HRLLC for LTE in 36.213, s10 | B | 15.1.0 | RAN1#93 | R1-1807950 | Motorola Mobility | LTE_HRLLC-Core |
| 36.213 | 1091 | - | Rel-14 | Correction to timing advance | F | 14.6.0 | RAN1#93 | R1-1807446 | Qualcomm Incorporated | LTE_MTCe2_L1-Core |
| 36.213 | 1092 | - | Rel-15 | Correction to timing advance | A | 15.1.0 | RAN1#93 | R1-1807447 | Qualcomm Incorporated | LTE_MTCe2_L1-Core |
| 36.211 | 441 | 1 | Rel-15 | Introduction of enhancements to operation in unlicensed spectrum | B | 15.1.0 | RAN1#93 | R1-1807930 | Ericsson | LTE_unlic-Core |
| 36.212 | 290 | 2 | Rel-15 | Introduction of FeLAA into 36.212 | B | 15.1.0 | RAN1#93 | R1-1807914 | Huawei | LTE_unlic-Core |
| 36.213 | 1055 | 2 | Rel-15 | Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s06-s09 | B | 15.1.0 | RAN1#93 | R1-1807931 | Motorola Mobility Germany GmbH | LTE_unlic-Core |
| 36.213 | 1098 | - | Rel-14 | Clarification of NPUSCH reception in 2 NB-IoT HARQ processes | F | 14.6.0 | RAN1#93 | R1-1807547 | Huawei, HiSilicon, Ericsson | NB_IOTenh-Core |
| 36.213 | 1099 | - | Rel-15 | Clarification of NPUSCH reception in 2 NB-IoT HARQ processes | A | 15.1.0 | RAN1#93 | R1-1807548 | Huawei, HiSilicon, Ericsson | NB_IOTenh-Core |
| 36.212 | 291 | - | Rel-15 | Introduce definition of non-BL/CE in Definitions section | F | 15.1.0 | RAN1#93 | R1-1807499 | Ericsson | LTE_MTCe2_L1-Core |
| 36.212 | 296 | - | Rel-15 | Introduction of modulation enhancements | B | 15.1.0 | RAN1#93 | R1-1807922 | Qualcomm Incorporated | TEI15 |
| 36.213 | 1075 | 1 | Rel-15 | Introduction of SRS antenna switching for 4 ports in TS36.213 | B | 15.1.0 | RAN1#93 | R1-1807504 | Qualcomm, Huawei, HiSilicon | TEI15 |
| 36.213 | 1097 | - | Rel-15 | Introduction of advanced CSI CBSR in TS 36.213 | B | 15.1.0 | RAN1#93 | R1-1807506 | Ericsson | TEI15 |
| 36.213 | 1105 | - | Rel-15 | Introduction of modulation enhancements | B | 15.1.0 | RAN1#93 | R1-1807923 | Qualcomm Incorporated | TEI15 |
| 38.202 | 3 | - | Rel-15 | CR to 38.202 capturing the RAN1#92bis and RAN1#93 meeting agreements related to URLLC | B | 15.1.0 | RAN1#93 | R1-1807961 | Qualcomm | NR_newRAT-Core |
| 38.212 | 3 | - | Rel-15 | CR to 38.212 capturing the RAN1#92bis and RAN1#93 meeting agreements related to URLLC | B | 15.1.1 | RAN1#93 | R1-1807962 | Huawei | NR_newRAT-Core |
| 38.214 | 3 | - | Rel-15 | CR to 38.214 capturing the RAN1#92bis and RAN1#93 meeting agreements related to URLLC | B | 15.1.0 | RAN1#93 | R1-1807963 | Nokia | NR_newRAT-Core |
| 36.213 | 1113 | - | Rel-15 | Introduction of Aerial for LTE in 36.213, s00-s05 | B | 15.1.0 | RAN1#93 | R1-1807951 | Motorola Mobility | LTE_Aerial-Core |

# Annex C-1:	List of Outgoing LSs from RAN1 #93

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-1807457 | LS on introduction of advanced CSI CBSR and associated UE capability | RAN1, Ericsson | Rel-15 | TEI15 |  | RAN2 |  |  |  |
| R1-1807488 | LS on shortened TTI and processing time for LTE | Ericsson | Rel-15 | LTE_sTTIandPT-Core |  | RAN2 |  |  |  |
| R1-1807492 | LS on Ultra Reliable Low Latency Communication for LTE | RAN1, Qualcomm | Rel-15 | LTE_HRLLC-Core |  | RAN2 |  |  |  |
| R1-1807519 | LS on RRC parameter list for Ultra Reliable Low Latency Communication for LTE | RAN1, Ericsson | Rel-15 | LTE_HRLLC-Core |  | RAN2 |  |  |  |
| R1-1807520 | Reply LS on RLM Qin/Qout Targets for URLLC | RAN1, Nokia | Rel-15 | LTE_HRLLC-Core | R4-1805042 | RAN4 | RAN2 |  |  |
| R1-1807543 | LS on capability signalling for maximum number of ports for advanced CSI | RAN1, Qualcomm Incorporated | Rel-14 | LTE_eFDMIMO-Core |  | RAN2 |  |  |  |
| R1-1807549 | LS on details of new categories for 1024QAM | RAN1, Qualcomm | Rel-15 | LTE_1024QAM_DL-Core |  | RAN2 |  |  |  |
| R1-1807551 | LS on parameters and capabilities for SRS antenna switching | RAN1, Qualcomm | Rel-15 | TEI15 |  | RAN2 |  |  |  |
| R1-1807566 | LS on additional agreements for shortened TTI and processing time for LTE | Ericsson | Rel-15 | LTE_sTTIandPT-Core |  | RAN2 |  |  |  |
| R1-1807567 | LS on Rel-14 NPRS enhancements | RAN1, Qualcomm | Rel-14 | NB_IOTenh-Core |  | RAN2, RAN4 |  |  |  |
| R1-1807572 | [DRAFT] LS on NPBCH for RRM measurements | Huawei | Rel-15 | NB_IOTenh2-Core |  | RAN4 |  |  |  |
| R1-1807575 | LS on updated RRC and LPP parameter list for further NB-IoT enhancements | RAN1, Huawei | Rel-15 | NB_IOTenh2-Core |  | RAN2, RAN3 |  |  |  |
| R1-1807580 | LS on RRC parameter list for L1 configuration of Rel-15 LTE-MTC | RAN1, Ericsson | Rel-15 | LTE_eMTC4-Core |  | RAN2 |  |  |  |
| R1-1807650 | Reply LS on messagePowerOffsetGroupB | RAN1, Ericsson | Rel-15 | NR_newRAT-Core | R2-1803860 | RAN2 |  |  |  |
| R1-1807651 | Reply LS on formula or table for L1 data rate | RAN1, Intel Corporation | Rel-15 | NR_newRAT-Core | R2-1801668 | RAN2 |  |  |  |
| R1-1807676 | LS reply on subcarrier spacing for BWPs and TDD configurations | RAN1, MediaTek Inc. | Rel-15 | NR_newRAT-Core | R2-1806505 | RAN2 |  |  |  |
| R1-1807718 | Reply LS on PC5 transmission mechanism selection | RAN1, LG Electronics | Rel-15 | LTE_eV2X-Core | S2-183968 | SA2, RAN2 |  |  |  |
| R1-1807719 | LS to RAN about TR38.811 chap 6 text proposal completion | RAN1, Thales | Rel-15 | FS_NR_nonterr_nw |  | RAN |  |  |  |
| R1-1807720 | Signalling Aspects Related to Pi/2 BPSK | RAN1, Qualcomm | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807722 | LS on adjacent TDM combinations of SRS for antenna switching and other UL physical channels | RAN1, Qualcomm | Rel-15 | NR_newRAT-Core |  | RAN4 |  |  |  |
| R1-1807723 | Reply LS on removal of CSI-RS-for-tracking from TCI-State | RAN1, ZTE | Rel-15 | NR_newRAT-Core | R2-1806495 | RAN2 |  |  |  |
| R1-1807725 | Reply LS on beam failure recovery | RAN1, Ericsson | Rel-15 | NR_newRAT-Core | R2-1806223 | RAN2 |  |  |  |
| R1-1807726 | [DRAFT] LS on DMRS positions of PUSCH | RAN1, vivo | Rel-15 | NR_newRAT-Core |  | RAN4 |  |  |  |
| R1-1807727 | LS on configuration of non-PMI port index | RAN1, ZTE | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807763 | LS on simultaneous PDSCH reception with Paging | RAN1, Huawei | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807764 | LS on numberOfRA-PreamblesGroupA | RAN1, Qualcomm | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807767 | LS on useServingCellTimingForSync definition | RAN1, Intel Corporation | Rel-15 | NR_newRAT-Core |  | RAN2, RAN4 |  |  |  |
| R1-1807831 | Reply LS on RMSI CORESET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth | RAN1, CATT | Rel-15 | NR_newRAT-Core |  | RAN4 | RAN2 |  |  |
| R1-1807842 | Reply LS on MSG3 size for NR | RAN1, Ericsson | Rel-15 | NR_newRAT-Core | R2-1806508 | RAN2 |  |  |  |
| R1-1807858 | Reply LS on physical layer aspects of Rel-15 NB-IoT and eMTC for TS 36.300 | RAN1, Huawei | Rel-15 | NB_IOTenh2-Core, LTE_eMTC4-Core | R2-1808848 | RAN2 |  |  |  |
| R1-1807868 | LS reply on Coexistence of NR-NR and NR-LTE networks with synchronised, unsynchronised and semi-synchronised operation in 3400-3800 MHz | RAN1, Huawei | Rel-15 | NR_newRAT-Core | R1-1806022 | RAN | RAN4 |  |  |
| R1-1807870 | Reply LS on the transmission of video over PC5 interface | RAN1, Huawei | Rel-15 | FS_mV2X | S4-180633 | SA4 | RAN2, SA1 |  |  |
| R1-1807871 | Reply LS on simultaneous configuration of EN-DC and NR PUCCH Scell | RAN1, NTT DOCOMO | Rel-15 | NR_newRAT-Core | R2-1801568 | RAN2, RAN4 |  |  |  |
| R1-1807873 | Reply LS on bandwidth configuration for initial BWP | RAN1, MediaTek | Rel-15 | NR_newRAT-Core | R2-1808990 | RAN2 | RAN4 |  |  |
| R1-1807882 | LS on aperiodic SRS | RAN1, ZTE | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807883 | LS on BFR search space configuration | RAN1, MediaTek | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807887 | LS on NR CSI-RS | RAN1, Huawei | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807892 | Reply LS on the design of PRACH table | RAN1, Qualcomm | Rel-15 | NR_newRAT-Core | R2-1806227 | RAN2 |  |  |  |
| R1-1807893 | LS on Initial Active BWP for Pattern 2 and Pattern 3 | RAN1, Huawei | Rel-15 | NR_newRAT-Core |  | RAN2, RAN4 |  |  |  |
| R1-1807896 | LS on updated NR UE features | RAN1, NTT DOCOMO, AT&T | Rel-15 | NR_newRAT-Core |  | RAN, RAN2, RAN4 | RAN3 |  |  |
| R1-1807903 | LS on the size of DCI format 2-1 | RAN1, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807907 | Reply to LS on Defining reserved resources in NR-SIB1 | RAN1, Nokia | Rel-15 | NR_newRAT-Core | R2-1806497 | RAN2 |  |  |  |
| R1-1807908 | Reply LS on LTE peak data rate for EN-DC | RAN1, Huawei |  |  | R2-1803950 | RAN2 | RAN4 |  |  |
| R1-1807909 | LS on CSI-RS for L3 mobility | RAN1, Intel | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807910 | LS on signalling of transmitter-side DC location | RAN1, MediaTek | Rel-15 | NR_newRAT-Core |  | RAN2 | RAN4 |  |  |
| R1-1807912 | LS on RRC parameter k0 | RAN1, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807921 | LS on RRC Parameters for NR URLLC | RAN1, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807924 | LS on introduction of modulation enhancements for LTE | RAN1, Qualcomm | Rel-15 | TEI15 |  | RAN2 |  |  |  |
| R1-1807960 | Reply LS on Type 3 UE capabilities | NTT DOCOMO, AT&T | Rel-15 | NR_newRAT-Core | R2-1806507 | RAN2, RAN4 |  |  |  |
| R1-1807969 | LS on removing RRC parameters related to multi-TRP | RAN1, Nokia | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807970 | LS on CSI measurement and reporting in DRX active time | RAN1, Nokia | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1807972 | LS on LTE V2X UE Categories and Reception Capabilities | RAN1, Intel | Rel-15 | LTE_eV2X-Core |  | RAN2 | RAN4 |  |  |
| R1-1807974 | Reply LS on RAN4 agreement on intraband EN-DC A-MPR | RAN1, Qualcomm | Rel-15 | NR_newRAT-Core | R4-1808144 | RAN4 |  |  |  |

# Annex C-2:	List of Incoming LSs from RAN1 #93

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-1805802 | Reply LS on beam failure recovery | RAN2, Ericsson | Rel-15 | NR_newRAT-Core | R1-1803348 | RAN1 |  | R2-1806223 |  |
| R1-1805803 | LS on the design of PRACH table | RAN2, Huawei | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1806227 |  |
| R1-1805804 | LS on further RAN2 agreements for euCA | RAN2, vivo | Rel-15 | LTE_euCA-Core |  | RAN1, RAN4 |  | R2-1806244 |  |
| R1-1805805 | LS on DL channel quality reporting in MSG3 | RAN2, Ericsson | Rel-14 | NB_IOTenh-Core |  | RAN4 | RAN1 | R2-1806258 |  |
| R1-1805806 | LS on feLAA agreements | RAN2, Nokia | Rel-15 | LTE_unlic-Core |  | RAN1 |  | R2-1806488 |  |
| R1-1805807 | LS on SSB-based Beam Management, Beam Failure Detection, RLM | RAN2, ZTE | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1806493 |  |
| R1-1805808 | LS on Removal of csi-RS-for-tracking from TCI-State | RAN2, ZTE | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1806495 |  |
| R1-1805809 | LS on Defining reserved resources in NR-SIB1 | RAN2, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1806497 |  |
| R1-1805810 | Reply LS on Simultaneous PDSCH Reception with Paging | RAN2, Qualcomm | Rel-15 | NR_newRAT-Core | R1-1803537 | RAN1 |  | R2-1806500 |  |
| R1-1805811 | LS to RAN1 on Removing RLM-RS Restriction in TS 38.331 ASN.1 | RAN2, Samsung | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1806504 |  |
| R1-1805812 | LS on subcarrier spacing for BWPs and TDD configurations | RAN2, Nokia | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1806505 |  |
| R1-1805813 | LS on Type 3 UE capabilities | RAN2, NTT DOCOMO | Rel-15 | NR_newRAT-Core | R1-1803480 | RAN1, RAN4 |  | R2-1806507 |  |
| R1-1805814 | Reply LS on MSG3 size for NR | RAN2, Huawei | Rel-15 | NR_newRAT-Core | R1-1805657 | RAN1 |  | R2-1806508 |  |
| R1-1805815 | LS response on serving cell measurement relaxation for NB-IOT | RAN4, Ericsson | Rel-15 | NB_IOTenh2-Core | R1-1803150 | RAN1 |  | R4-1805013 |  |
| R1-1805816 | LS on RLM Qin/Qout Targets for URLLC | RAN4, Ericsson | Rel-15 | LTE_HRLLC-Core |  | RAN1 | RAN2 | R4-1805042 |  |
| R1-1805817 | Reply LS on SRS switching | RAN4, Huawei | Rel-15 | NR_newRAT-Core | R1-1803549 | RAN1 | RAN2 | R4-1805660 |  |
| R1-1805818 | LS on UE Rx/Tx Transition Times | RAN4, Qualcomm | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R4-1805766 |  |
| R1-1805819 | Reply LS on narrowband measurement | RAN4, Huawei | Rel-15 | NB_IOTenh2-Core | R1-1803151 | RAN1, RAN2 |  | R4-1805950 |  |
| R1-1805820 | Reply LS on CRS muting in eFeMTC | RAN4, Huawei | Rel-15 | LTE_eMTC4-Core | R1-1706734 | RAN1 | RAN2 | R4-1805953 |  |
| R1-1805821 | LS reply to RAN1 on P_0 ranges on UL power control | RAN4, ZTE | Rel-15 | NR_newRAT-Core | R1-1721608 | RAN1 | RAN2 | R4-1806009 |  |
| R1-1805822 | LS on network-based CRS interference mitigation | RAN4, Ericsson | Rel-15 | LTE_NW_CRS_IM |  | RAN2 | RAN1 | R4-1806018 |  |
| R1-1805823 | Reply LS on PC5 transmission mechanism selection | SA2, LG Electronics | Rel-15 | LTE_eV2X-Core | R2-1804090 | RAN2, RAN1 |  | S2-183968 |  |
| R1-1806022 | Coexistence of NR-NR and NR-LTE networks with synchronised, unsynchronised and semi-synchronised operation in 3400-3800 MHz | CEPT ECC PT1 | Rel-15 | NR_newRAT-Core |  | RAN1, RAN4 | RAN |  | R1-1807868 |
| R1-1806456 | LS on minimum number of RX antennae for NR vehicular UEs | 5GAA WG2, LG Electronics | Rel-15 |  |  | RAN4, RAN | RAN1, RAN2 | 5GAA_A-180148 |  |
| R1-1807524 | LS response on NB-IoT downlink channel quality determination and report | RAN4, Ericsson | Rel-14 | NB_IOT-Core, TEI14 | R1-1803163 | RAN1, RAN2 |  | R4-1807937 |  |
| R1-1807614 | LS on Removal of LTE specific terminology from Group Communication System Enablers TS 22.468 | SA1, Nokia | Rel-15 | MCOver |  | SA2, SA3, SA4, SA5, SA6, CT1, CT3, CT4, CT6, RAN1, RAN2, RAN3, RAN4, RAN5, RAN6 | SA, CT, RAN | S1-181249 |  |
| R1-1807627 | LS on L2 measurement specification for NR in R15 | SA5, Ericsson | Rel-15 | NETSLICE-ADPM5G |  | RAN2, RAN3 | RAN1 | S5-183622 |  |
| R1-1807683 | LS on updates for TS 36.300 | RAN2, Huawei | Rel-15 | NB-IoTenh2-Core, LTE_eMTC4-Core |  | RAN1, RAN3 |  | R2-1808848 |  |
| R1-1807731 | LS on Bandwidth configuration for initial BWP | RAN2, NTT DOCOMO | Rel-15 | NR_newRAT-Core |  | RAN1 | RAN4 | R2-1808990 |  |
| R1-1807732 | Reply LS on PDCCH common search space configurations | RAN2, NTT DOCOMO | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1808992 | R1-1805440 |
| R1-1807737 | Reply LS on signalling | RAN3 |  |  |  | RAN1, RAN2, RAN4, RAN5, RAN6, SA2 |  |  |  |
| R1-1807804 | LS on the feasibility of 1 symbol GP for 60KHz in FR1 and 120KHz in FR2 | RAN4, MediaTek | Rel-15 | NR_newRAT-Core |  | RAN1 | RAN2 |  |  |
| R1-1807805 | LS on RAN4 agreement on intraband EN-DC A-MPR | RAN4, Nokia | Rel-15 | NR_newRAT-Core |  | RAN1 |  |  |  |
| R1-1807806 | LS on RMSI CORESET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth | RAN4, CATT, Samsung | Rel-15 | NR_newRAT-Core |  | RAN1 | RAN2 |  |  |
| R1-1807898 | LS on Lead Time for Beam Switching | RAN4, Qualcomm | Rel-15 | NR_newRAT-Core |  | RAN, RAN1 |  | R4-1808542 |  |

# Annex D:	List of Approved updated WIDs

None

# Annex E:	List of draft TSs/TRs agreed at RAN1 #93

| Tdoc Number | Title | Source | Conclusion/Decision |
| --- | --- | --- | --- |
| R1-1806178 | Draft TR 37.885 v0.2.0 Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR | LG Electronics Inc. | Endorsed |
| R1-1807869 | TR 37.885 v1.0.0 Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR | LG Electronics | Endorsed |
| R1-1807932 | TS 37.213 v1.0.0 Physical layer procedures for shared spectrum channel access | Ericsson | Endorsed |

# Annex F:	List of actions

- Outgoing LS

[93-NR-03]  – Abjorn (Ericsson)

Send an LS to RAN2 to inform the above agreements and removal of parameter k_0, for email approval by May 30th

Done: As per decision posted on May 31st, the final LS is approved in:

R1-1807912	LS on RRC parameter k0	RAN1, Ericsson

[93-NR-01] – Peter (Qualcomm)

R1-1807805	LS on RAN4 agreement on intraband EN-DC A-MPR	RAN4, Nokia

Email discussion on whether any response is needed and, if needed, the response LS details until June 29th, 2018.

Done: As per decision posted on July 17th, the conclusion in the latest summary regarding whether a response LS is needed is agreed in:

R1-1807973	Summary of email discussion [93-NR-01] LS on intra-band EN-DC MPR, A-MPR	Qualcomm

LS is approved in:

R1-1807974	Reply LS on RAN4 agreement on intraband EN-DC A-MPR	RAN1, Qualcomm	(reply to R4-1808144)

[93-NR-04] - Kazuaki (NTT DOCOMO)/ Xiaoyi (ATT)

R1-1807847	Draft reply LS on Type 3 UE capabilities	NTT DOCOMO, AT&T

Email discussion to conclude Type 3 UE capabilities and to approve reply LS by May 31st

Done: As per decision posted on June 5th, the final LS is approved in R1-1807960.

[93-NR-06] – Yufei (Ericsson)

R1-1807874	[Draft] LS on RRC Parameters for NR URLLC	Ericsson	(rev. of R1-1807822)

Email approval till May 31st

Done: As per decision posted on June 1st, the final LS (inc. the list of parameters for NR URLLC) is approved in:

R1-1807920	RRC parameter list for NR URLLC	Ericsson

R1-1807921	LS on RRC Parameters for NR URLLC	RAN1, Ericsson

[93-NR-02-38.214] - Mihai (Nokia)

NR maintenance

Done: The following LSs are approved:

R1-1807969	LS on removing RRC parameters related to multi-TRP	RAN1, Nokia	Rel-15	NR_newRAT-Core	to RAN2

R1-1807970	LS on CSI measurement and reporting in DRX active time	RAN1, Nokia	Rel-15	NR_newRAT-Core	to RAN2

[93-LTE-15] – Alexey (Intel)

Email approval till June 7th of LS to RAN2 regarding outcomes of [93-LTE-12] & [93-LTE-13], which impacts TS36.306. Also discussion/approval removing the brackets in the table of finalization of TX/RX parameters for V2X UE Sidelink Categories

Done: As per decision posted on June 8th, the final LS is approved in R1-1807972.

- CR approval

[93-LTE-01] - Yutao Sui (Ericsson)

Email approval for CR on new NPRS sequence design until May 30th, 2018

Done: As per decision posted on June 1st, the CRs are for further discussion in the next RAN1 meeting in order to solve the overlapping of the legacy and new NPRS configurations issue. In addition, they could not be presented to plenary for approval because LPP (and LPPa) and the corresponding RAN2 CRs are missing. The reserved Tdoc# R1-1807591 (36.211, Rel-14) and R1-1807592 (36.211, Rel-15) are withdrawn. The latest draft CR is in R1-1807934 for information.

[93-LTE-02-spec#]

Drafts for LTE CRs to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

sTTI: Updated cat-F editor CRs for 211, 212, 213 based on endorsed TPs, agreements, and working assumptions.

Done: The following CRs are agreed:

R1-1807913	Corrections on shortened processing time and shortened TTI in 36 212	Huawei	36.212	15.1.0	LTE_sTTIandPT-Core	CR0280r2, F

R1-1807925	Corrections to sTTI and SPT implementation	Ericsson	36.211	15.1.0	LTE_sTTIandPT-Core	CR0440r1, F

R1-1807926	Corrections for shortened processing time and shortened TTI in 36.213, s00-s05	Motorola Mobility	36.213	15.1.0	LTE_sTTIandPT-Core	CR1049r2, F

R1-1807927	Corrections for shortened processing time and shortened TTI in 36.213, s06-s07	Motorola Mobility	36.213	15.1.0	LTE_sTTIandPT-Core	CR1050r2, F

R1-1807928	Corrections for shortened processing time and shortened TTI in 36.213, s10-s13	Motorola Mobility	36.213	15.1.0	LTE_sTTIandPT-Core	CR1051r2, F

R1-1807929	Corrections for shortened processing time and shortened TTI in 36.213, s08-s09	Motorola Mobility	36.213	15.1.0	LTE_sTTIandPT-Core	CR1074r1, F

[93-LTE-03-spec#]

Drafts for LTE CRs to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

FeLAA: Updated cat-F editor CRs for 211, 212, 36.213, and 37.213 based on endorsed TPs, agreements, and working assumptions.

Done: The following CRs are agreed:

R1-1807914	Introduction of FeLAA into 36.212	Huawei	36.212	15.1.0	LTE_unlic-Core	CR0290r2, B

R1-1807930	Introduction of enhancements to operation in unlicensed spectrum	Ericsson	36.211	15.1.0	LTE_unlic-Core	CR0441r1, B

R1-1807931	Introduction of Enhancements to LTE operation in unlicensed spectrum into 36.213 s06-s09	Motorola Mobility	36.213	15.1.0	LTE_unlic-Core	CR1055r2, B

In addition, TS 37.213 is endorsed as version 1.00 for one-step approval in

R1-1807932	TS 37.213 v1.0.0 Physical layer procedures for shared spectrum channel access	Ericsson	37.213	1.0.0	LTE_unlic-Core

[93-LTE-04-spec#]

Drafts for LTE CRs to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

HCS: New cat-F editor CRs for 212, 213 based on endorsed TPs

Done: The following CRs are agreed:

R1-1807915	Introduction of high capacity stationary wireless link and 1024QAM into 36.212	Huawei	36.212	15.1.0	LTE_1024QAM_DL-Core	CR0284r1, B

R1-1807942	Introduction of enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE to 36.213	Motorola Mobility	36.213	15.1.0	LTE_1024QAM_DL-Core	CR1053r1, B

[93-LTE-05-spec#]

Drafts for LTE CRs to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

FeCoMP: New cat-F editor CR for 213 based on endorsed TPs

Done: The following CRs are agreed:

R1-1807943	Correction on layer-to-port mapping for feCoMP	Ericsson	36.211	15.1.0	feCOMP_LTE-Core	CR0449, F

R1-1807933	Corrections to feCoMP in 36.213, s06-s07	Motorola Mobility	36.213	15.1.0	feCOMP_LTE-Core	CR1106, F

[93-LTE-06-spec#]

Drafts for LTE CRs to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

eMTC: Updated cat-B editor CRs for 211, 212, 213 based on agreements and working assumptions.

Done: The following CRs are agreed:

R1-1807971	Agreement summary for Rel-15 Even further enhanced MTC for LTE	WI rapporteur (Ericsson)	(rev. of R1-1807581)

R1-1807935	Introduction of Even Further Enhanced MTC for LTE	Ericsson	36.211	15.1.0	LTE_eMTC4-Core	CR0447, B

R1-1807916	Introduction of even further enhancements of eMTC	Huawei	36.212	15.1.0	LTE_eMTC4-Core	CR0292, B

R1-1807936	Introduction of even further enhanced MTC for LTE in 36.213, s00-s05	Motorola Mobility	36.213	15.1.0	LTE_eMTC4-Core	CR1101r1, B

R1-1807937	Introduction of even further enhanced MTC for LTE in 36.213, s06-s07	Motorola Mobility	36.213	15.1.0	LTE_eMTC4-Core	CR1102r1, B

R1-1807938	Introduction of even further enhanced MTC for LTE in 36.213, s08-s09	Motorola Mobility	36.213	15.1.0	LTE_eMTC4-Core	CR1103r1, B

[93-LTE-07-spec#]

Drafts for LTE CRs to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

NB-IoT: Updated cat-B editor CRs for 211, 212, 213, 214 based on agreements and working assumptions.

Done: The following CRs are agreed:

R1-1807590	RAN1 agreements for Rel-15 Further NB-IoT enhancements	WI rapporteur (Ericsson)

R1-1807939	Introduction Rel-15 Further NB-IoT enhancements	Ericsson	36.211	15.1.0	NB_IOTenh2-Core	CR0448, B

R1-1807917	Introduction of further enhancements of NB-IoT	Huawei	36.212	15.1.0	NB_IOTenh2-Core	CR0293, B

R1-1807940	Introduction of Rel-15 Further NB-IoT enhancements in 36.213, s14	Motorola Mobility	36.213	15.1.0	NB_IOTenh2-Core	CR1104r1, B

R1-1807941	Introduction Rel-15 Further NB-IoT enhancements	Ericsson	36.214	15.1.0	NB_IOTenh2-Core	CR0050r1, B

[93-LTE-08-spec#]

Drafts for LTE CRs to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

eV2X: Updated cat-B editor CRs for 212, 213 based on agreements and working assumptions.

(Rapporteur to provide updated list of agreements and working assumptions by Saturday May 26)

Done: The following CRs are agreed:

R1-1807918	Introduction of V2X Phase 2 based on LTE	Huawei	36.212	15.1.0	LTE_eV2X-Core	CR0294, B

R1-1807944	Addition of V2X Phase 2 based on LTE in 36.213, s14	Motorola Mobility, LG Electronics	36.213	15.1.0	LTE_eV2X-Core	CR1107, B

[93-LTE-09-spec#]

Drafts for LTE CRs to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

HRLLC: New cat-B editor CRs for 211, 212, 213 based on agreements and working assumptions.

(Rapporteur to provide list of agreements and working assumptions by Saturday May 26)

Done: The following CRs are agreed:

R1-1807582	RAN1 decisions for WI Ultra Reliable Low Latency Communication for LTE (LTE_HRLLC) – per meeting	Ericsson

R1-1807583	RAN1 decisions for WI Ultra Reliable Low Latency Communication for LTE (LTE_HRLLC) – per topic	Ericsson

R1-1807919	Introduction of HRLLC into 36.212	Huawei	36.212	15.1.0	LTE_HRLLC-Core	CR0295, B

R1-1807945	Introduction of HRLLC into 36.211	Ericsson	36.211	15.1.0	LTE_HRLLC-Core	CR0450, B

R1-1807946	Introduction of HRLLC for LTE in 36.213, s07-1	Motorola Mobility	36.213	15.1.0	LTE_HRLLC-Core	CR1108, B

R1-1807947	Introduction of HRLLC for LTE in 36.213, s07-3	Motorola Mobility	36.213	15.1.0	LTE_HRLLC-Core	CR1109, B

R1-1807948	Introduction of HRLLC for LTE in 36.213, s08	Motorola Mobility	36.213	15.1.0	LTE_HRLLC-Core	CR1110, B

R1-1807949	Introduction of HRLLC for LTE in 36.213, s09	Motorola Mobility	36.213	15.1.0	LTE_HRLLC-Core	CR1111, B

R1-1807950	Introduction of HRLLC for LTE in 36.213, s10	Motorola Mobility	36.213	15.1.0	LTE_HRLLC-Core	CR1112, B

[93-LTE-10-spec#]

Drafts for LTE CRs to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

Aerials: New cat-B editor CR for 213 based on endorsed TPs.

Done: The following CR is agreed:

R1-1807951	Introduction of Aerial for LTE in 36.213, s00-s05	Motorola Mobility	36.213	15.1.0	LTE_Aerial-Core	CR1113, B

[93-LTE-11-spec#]

Drafts for LTE CRs to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

EN-DC: 36.213 CRs

Done: The following CRs are agreed:

R1-1807952	Correction for EN-DC Coexistence into TS36.213 s06-s07	Motorola Mobility	36.213	15.1.0	NR_newRAT-Core	CR1114, F

R1-1807953	Correction for EN-DC Coexistence into TS36.213 s08-s09	Motorola Mobility	36.213	15.1.0	NR_newRAT-Core	CR1115, F

R1-1807964	Correction for EN-DC Coexistence into TS36.213 s00-s05	Motorola Mobility	36.213	15.1.0	NR_newRAT-Core	CR1116, F

R1-1807965	Correction for EN-DC Coexistence into TS36.213 s10-s13	Motorola Mobility	36.213	15.1.0	NR_newRAT-Core	CR1117, F

[93-LTE-14] - Alberto (Qualcomm)

Email approval to decide the value range of α, the contents for rows 42-58, and whether entries 56-58 have scaling applied or not, agree to CR, and send LS to RAN2 until May 31st, 2018

Done: As per decision posted on June 1st, the following CRs are agreed.

R1-1807922	Introduction of modulation enhancements	Qualcomm Incorporated	36.212	15.1.0	TEI15	CR0296, B

R1-1807923	Introduction of modulation enhancements	Qualcomm Incorporated	36.213	15.1.0	TEI15	CR1105, B

LS to RAN2 is approved in:

R1-1807924	LS on introduction of modulation enhancements for LTE	RAN1, Qualcomm

[93-NR-02-spec#]

NR maintenance

Editor to prepare CRs (capturing agreements and working assumptions): drafts to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

Done: The following CRs are agreed:

R1-1807954	CR to 38.202 capturing the RAN1#92bis and RAN1#93 meeting agreements	Qualcomm	38.202	15.1.0	NR_newRAT-Core	CR0002r1, F

R1-1807955	CR to 38.211 capturing the RAN1#92bis and RAN1#93 meeting agreements	Ericsson	38.211	15.1.0	NR_newRAT-Core	CR0002r1, F

R1-1807956	CR to 38.212 capturing the RAN1#92bis and RAN1#93 meeting agreements	Huawei	38.212	15.1.1	NR_newRAT-Core	CR0002r1, F

R1-1807957	CR to 38.213 capturing the RAN1#92bis and RAN1#93 meeting agreements	Samsung	38.213	15.1.0	NR_newRAT-Core	CR0002r1, F

R1-1807958	CR to 38.214 capturing the RAN1#92bis and RAN1#93 meeting agreements	Nokia	38.214	15.1.0	NR_newRAT-Core	CR0002r1, F

R1-1807959	CR to 38.215 capturing the RAN1#92bis and RAN1#93 meeting agreements	Intel	38.215	15.1.0	NR_newRAT-Core	CR0003r1, F

R1-1807968	CR to 38.214: maintenance according to agreed Rel 15 features	Nokia	38.214	15.1.0	NR_newRAT-Core	CR0004, F

[93-NR-05-spec#]

NR URLLC

Editor to prepare CRs: drafts to be distributed by Thursday May 31st, to be agreed by Tuesday June 5th.

Done: The following CRs are agreed:

R1-1807961	CR to 38.202 capturing the RAN1#92bis and RAN1#93 meeting agreements related to URLLC	Qualcomm	38.202	15.1.0	NR_newRAT-Core	CR0003, B

R1-1807962	CR to 38.212 capturing the RAN1#92bis and RAN1#93 meeting agreements related to URLLC	Huawei	38.212	15.1.1	NR_newRAT-Core	CR0003, B

R1-1807963	CR to 38.214 capturing the RAN1#92bis and RAN1#93 meeting agreements related to URLLC	Nokia		38.214	15.1.0	NR_newRAT-Core	CR0003, B

- Miscellaneous

[93-LTE-12] - Alexey (Intel)

Email discussion to finalize on the following by May 31 (Alexey, Intel)

Transmission of physical parameter values set by ue-CategorySL-C-TX

Reception of physical parameter values set by ue-CategorySL-C-RX

Done: As per decision posted on June 1st, the following is agreed:

Agreements

- TX Parameters

| UE SL-C-TX Category | Maximum number of SL-SCH transport block bits transmitted within a TTI | Maximum number of bits of a SL-SCH transport block transmitted within a TTI | Maximum number of supported layers for spatial multiplexing in SL-C | Comments |
| --- | --- | --- | --- | --- |
| SL-C-TX Category 1 | 25456 | 25456 | 1 |  |
| SL-C-TX Category 2 | 31704 | 31704 | 1 | 20MHz aggregated BW 16QAM |
| SL-C-TX Category 3 | [32856 + 16416 = 49272] | [32856] | 1 | 30MHz aggregated BW 16QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |
| SL-C-TX Category 4 | [48936] | [48936] | 1 | 20MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz and 20MHz) |
| SL-C-TX Category 5 | [48936 + 24496= 73432] [24496 + 24496 + 24496 = 73488] | [48936] | 1 | 30MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |

- RX Parameters

| UE SL-C-RX Category | Maximum number of SL-SCH transport block bits received within a TTI | Maximum number of bits of a SL-SCH transport block received within a TTI | Total number of soft channel bits | Comments |
| --- | --- | --- | --- | --- |
| SL-C-RX Category 1 | 25456 | 25456 |  |  |
| SL-C-RX Category 2 | 31704 | 31704 | 737280 | 20MHz aggregated BW 16QAM |
| SL-C-RX Category 3 | [48936] | [48936] | [995328] | 20MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz and 20MHz) |
| SL-C-RX Category 4 | [48936 + 24496= 73432] [24496 + 24496 + 24496 = 73488] | [48936] | [1492992] | 30MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |

[93-LTE-13] - Alexey (Intel)

Email discussion by May 31 on V2X Sidelink Processing Capabilities – Indication per CC (Alexey, Intel)

Done: As per decision posted on June 2nd, the following is agreed:

Agreements

- Define minimum UE reception capability vs amount of aggregated BW per sidelink RX band combination as provided in table below:

| Aggregated RX BW by UE, MHz | V2X High Reception Capability | V2X Default Reception Capability |
| --- | --- | --- |
| 10, 20 (Rel.14, Rel.15) | 20 PSCCHs and 136 RBs | 10 PSCCHs and 100 RBs |
| 30 (Rel.15) | 30 PSCCHs and 204 RBs | 15 PSCCHs and 150 RBs |

- UE reception capability (V2X High/ V2X Default) is indicated per sidelink RX band combination

[93-NR-07] – Li (ZTE)

NR NOMA: Companies to share the calibration results by August 1st, to be consolidated for RAN1#94 contribution

Done: As per decision posted on August 9th, an email discussion summary is to be prepared as input to next meeting.

[93-NR-08]

NR NOMA

Email discussion for the templates of reporting the link-level evaluation results, to start email discussion from June 18th, aim to finish by 8/3

Done: As per decision posted on August 9th, an email discussion summary is to be prepared as input to next meeting.

[93-NR-09] - Jing (Qualcomm)

NR Unlic

Email discussion on calibration of parameters A and X targeting a single setting of parameters for both outdoor scenarios until August 2nd, 2018

Done: As per decision posted on August 9th, an email discussion summary is to be prepared as input to next meeting.

# Annex G:	List of participants at RAN1 #93

Please see excel file attached to this report

# Annex H:	TSG RAN WG1 meetings in 2018 – 2019

| TITLE | TYPE | DATES | LOCATION | CTRY |
| --- | --- | --- | --- | --- |
| 3GPPRAN1#94 | WG | 20 – 24 Aug 2018 | Gothenburg | Sweden |
| 3GPPRAN1#94bis | WG | 8 – 12 Oct 2018 | Chengdu | China |
| 3GPPRAN1#95 | WG | 12 – 16 Nov 2018 | Spokane | US |
| 3GPPRAN1-AH-1901-TBC | AH | 21 - 25 Jan 2019 | Taipei | Taiwan |
| 3GPPRAN1#96 | WG | 25 Feb - 1 Mar 2019 | Athens | Greece |
| 3GPPRAN1#96bis | WG | 8 - 12 Apr 2019 | China | China |
| 3GPPRAN1#97 | WG | 13 - 17 May 2019 | US | US |
| 3GPPRAN1-AH-1906-TBC | AH | 24 - 28 Jun 2019 | TBD |  |
| 3GPPRAN1#98 | WG | 26 - 30 Aug 2019 | Prague | CZ |
| 3GPPRAN1#98bis | WG | 14 - 18 Oct 2019 | China | China |
| 3GPPRAN1#99 | WG | 18 - 22 Nov 2019 | US | US |

| MEETING TYPES |  |  |
| --- | --- | --- |
| AH = Ad Hoc | ST = Startup Meeting | RG = Rapporteurs Group |
| JM = Joint | WG = Working Group | SG = Steering Group |
| PM = Preparatory Meeting | CM = Chairmen's meeting | TG = Task Group |
| RM = Resolution Meeting | OR = Ordinary | XO = Extraordinary |

End of document