Source: 	MCC Support

Title:	Final Report of 3GPP TSG RAN WG1 #94 v1.0.0

(Gothenburg, Sweden, 20th – 24th August 2018)

Document for:	Approval

Table of contents

1	Opening of the meeting	6

1.1	Call for IPR	6

1.2	Competition law statement	6

1.3	Network usage conditions	6

1.4	Check-in for Registered Delegates	6

1.5	Aspects related to RAN1 Meeting Management	7

2	Approval of Agenda	7

3	Highlights from RAN plenary	7

4	Approval of Minutes from previous meeting	7

5	Incoming Liaison Statements	7

6	E-UTRA	10

6.1	Maintenance of E-UTRA Releases 8 – 15	10

6.1.1	Maintenance of E-UTRA Release 8 – 14	10

6.1.2	Maintenance of Release 15 Shortened TTI and processing time for LTE	13

6.1.2.1	Maintenance for shortened processing time for 1ms TTI	13

6.1.2.2	Maintenance for shortened TTI with shortened processing time	13

6.1.2.2.1	Aspects related to interaction between different TTI lengths	13

6.1.2.2.2	Aspects related to DL control channel	13

6.1.2.2.3	Aspects related to UL control channel	13

6.1.2.2.4	Aspects related to DL data channel	13

6.1.2.2.5	Aspects related to UL data channel	14

6.1.2.2.6	Aspects related to FS2	15

6.1.2.3	Other	15

6.1.3	Maintenance of Release 15 3GPP V2X Phase 2	15

6.1.3.1	Maintenance for Carrier Aggregation (up to 8 PC5 carriers)	15

6.1.3.2	Maintenance for Support for 64-QAM	15

6.1.3.3	Maintenance for Resource pool sharing between mode-3 and mode-4 users	16

6.1.3.4	Maintenance for Maximum time reduction between packet arrival at layer 1 and resource selection for transmission	16

6.1.3.5	Other	16

6.1.4	Maintenance of Release 15 Even further enhanced MTC for LTE	16

6.1.4.1	Maintenance for Resynchronization signal	16

6.1.4.2	Maintenance for Early data transmission	16

6.1.4.3	Maintenance for Wake-up signal	17

6.1.4.4	Maintenance for PUSCH sub-PRB allocation	17

6.1.4.5	Maintenance for Flexible PDSCH/PUSCH starting PRB	18

6.1.4.6	Other	19

6.1.5	Maintenance of Release 15 Further enhancements of NB-IoT	19

6.1.5.1	Maintenance for Latency and power consumption reduction	19

6.1.5.1.1	Power consumption reduction for paging and connected-mode DRX	19

6.1.5.1.2	Data transmission during the random access procedure	20

6.1.5.1.3	Physical layer scheduling request	20

6.1.5.1.4	Semi-persistent scheduling	20

6.1.5.2	Maintenance for NPRACH range enhancements	21

6.1.5.3	Maintenance for TDD	21

6.1.5.3.1	Downlink aspects	21

6.1.5.3.2	Uplink aspects	22

6.1.5.3.3	Common aspects	22

6.1.5.4	Other	23

6.1.6	Maintenance of Release 15 Ultra Reliable Low Latency Communication for LTE	23

6.1.6.1	Maintenance for Blind/HARQ-less Repetition for Scheduled DL-SCH Operation	23

6.1.6.2	Maintenance for Repetition Enhancements for UL SPS Operation	23

6.1.6.3	Other	24

6.1.7	LTE Release 15 UE features	24

6.1.8	Other	25

6.2	LTE Release 16	26

6.2.1	Additional MTC Enhancements	26

6.2.1.1	UE-group wake-up signal	26

6.2.1.2	Support for transmission in preconfigured UL resources	27

6.2.1.3	Scheduling of multiple DL/UL transport blocks	28

6.2.1.4	Coexistence of LTE-MTC with NR	28

6.2.1.5	Support of Quality report in Msg3	29

6.2.1.6	MPDCCH performance improvement	29

6.2.1.7	CE mode A and B improvements for non-BL UEs	29

6.2.1.8	Use of LTE Control Channel Region for DL Transmission	30

6.2.1.9	Use of RSS for measurement improvements	30

6.2.2	Additional Enhancements for NB-IoT	31

6.2.2.1	UE-group wake-up signal	31

6.2.2.2	Support for transmission in preconfigured UL resources	31

6.2.2.3	Scheduling of multiple DL/UL transport blocks	32

6.2.2.4	Coexistence of NB-IoT with NR	32

6.2.2.5	Support of Quality report in Msg3 for non-anchor access	33

6.2.2.6	Presence of NRS on a non-anchor carrier for paging	33

6.2.2.7	Other	33

6.2.3	DL MIMO efficiency enhancements for LTE	33

6.2.3.1	Support of enhancing SRS capacity and coverage	34

6.2.3.1.1	Additional SRS symbols	34

6.2.3.1.2	Virtual cell ID for SRS	35

6.2.3.1.3	Other	35

6.2.3.2	Other	35

6.2.4	Other	35

7	NR	35

7.1	Maintenance of Release 15 NR	35

7.1.1	Maintenance for Initial access and mobility	36

7.1.1.1	Maintenance for Downlink signals and channels	36

7.1.1.2	Maintenance for Physical random access channel and procedure	40

7.1.1.3	Maintenance for Mobility procedure	43

7.1.1.4	Other	47

7.1.2	Maintenance for MIMO	47

7.1.2.1	Maintenance for Multi-antenna scheme	47

7.1.2.2	Maintenance for CSI acquisition	51

7.1.2.3	Maintenance for beam management	62

7.1.2.4	Maintenance for Reference signals and QCL	65

7.1.2.5	Other	84

7.1.3	Maintenance for Scheduling/HARQ aspects	85

7.1.3.1	Maintenance for physical downlink control channel	85

7.1.3.2	Maintenance for physical uplink control channel	91

7.1.3.3	Maintenance for DL/UL data scheduling and HARQ procedure	95

7.1.3.4	Maintenance for carrier aggregation and bandwidth parts	103

7.1.3.5	Other	107

7.1.4	Maintenance for NR-LTE co-existence	108

7.1.5	Maintenance for UL power control	110

7.1.6	Maintenance for URLLC	112

7.1.7	NR Release 15 UE features	112

7.1.8	Other	114

7.2	NR in Release 16	114

7.2.1	Study on non-orthogonal multiple access for NR	114

7.2.1.1	Transmitter side signal processing schemes for NOMA	114

7.2.1.2	Receivers for NOMA	116

7.2.1.3	Procedures related to NOMA	118

7.2.1.4	Link and system level performance evaluation for NOMA	119

7.2.1.5	Other	122

7.2.2	Study on NR-based Access to Unlicensed Spectrum	122

7.2.2.1	Simulation Methodology for NR-U operation	123

7.2.2.2	Frame structure for NR-U operation	123

7.2.2.3	Potential physical layer channel design	124

7.2.2.3.1	DL Signals and Channels	124

7.2.2.3.2	UL Signals and Channels	125

7.2.2.4	Potential physical layer procedures	126

7.2.2.4.1	Channel access procedures	126

7.2.2.4.2	Initial access and mobility	126

7.2.2.4.3	Potential HARQ enhancements	127

7.2.2.4.4	Potential enhancements to configured grants	128

7.2.2.4.5	Other	129

7.2.2.5	Other	129

7.2.3	Study on Integrated Access and Backhaul for NR	129

7.2.3.1	Enhancements to support NR backhaul links	129

7.2.3.2	Evaluation methodology for NR IAB	136

7.2.3.3	Other	137

7.2.4	Study on NR V2X	138

7.2.4.1	Sidelink design	138

7.2.4.1.1	Support of unicast, groupcast and broadcast	138

7.2.4.1.2	Physical layer structures and procedure(s)	140

7.2.4.1.3	Synchronization mechanism	143

7.2.4.1.4	Resource allocation mechanism	145

7.2.4.1.5	Other	147

7.2.4.2	Evaluation of Uu for advanced V2X use cases	147

7.2.4.3	Uu-based sidelink resource allocation/configuration	148

7.2.4.3.1	Enhancements of LTE Uu and NR Uu to control NR sidelink	149

7.2.4.3.2	Enhancements of NR Uu to control LTE sidelink	150

7.2.4.3.3	Other	150

7.2.4.4	QoS management	150

7.2.4.5	Coexistence	151

7.2.4.6	Other	152

7.2.5	Study on Remote Interference Management for NR	152

7.2.5.1	Mechanisms for improving network robustness	157

7.2.5.2	Mechanisms for identifying strong gNB interferers	157

7.2.5.3	Other	158

7.2.6	Study on Physical Layer Enhancements for NR URLLC	159

7.2.6.1	Layer 1 enhancements	159

7.2.6.2	UL inter UE Tx prioritization/multiplexing	163

7.2.6.3	Enhanced UL grant-free transmissions	165

7.2.6.4	Other	166

7.2.7	Study on Self Evaluation towards IMT-2020 Submission	167

7.2.7.1	Evaluation methodology	168

7.2.7.2	Other	168

7.2.8	Other	170

7.3	Other	171

8	Closing of the meeting	171

Annex A:	List of Tdocs at RAN1 #94	172

Annex B:	List of CRs agreed at RAN1 #94	173

Annex C-1:	List of Outgoing LSs from RAN1 #94	177

Annex C-2:	List of Incoming LSs from RAN1 #94	179

Annex D:	List of Approved updated WIDs	183

Annex E:	List of draft TSs/TRs agreed at RAN1 #94	184

Annex F:	List of actions	185

Annex G:	List of participants at RAN1 #94	193

Annex H:	TSG RAN WG1 meetings in 2018 – 2019	194

Main facts summary

The 3GPP TSG WG RAN1#94 meeting, hosted by the European Friends of 3GPP, was held at the Gothia Towers Hotel, Göteborg, Sweden.

The meeting started at 9:00 on Monday 20th August and finished at 17:07 on Friday 24th August 2018.

The number of attending delegates, having confirmed their participation through the electronic check-in application, was 499.

Note that the number of registered was 622.

ETSI Workshop "Writing World Class standards" took place during the week (organizer: Milan Zoric from ETSI).

- Total number of attendees was 65.

- Per RAN group, split is as follows: RAN 1 (28), RAN2 (21), RAN3 (1), RAN4 (10), RAN5 (5)

Milan was very pleased with the atmosphere and attentiveness of attendees as well as with questions raised and discussions that took place. Thanks you all who have participated.

The schedule of the week was as follows:

The list of action points that require RAN1 post meeting follow-up is listed in Annex F (end of document).

The number of contributions for this meeting was 2017.

Note: The amount of documents includes those discussed during the email discussion session post meeting.

|  | Contribution is agreed/approved |
| --- | --- |
|  | Contribution is not pursued (no consensus) |
|  | Further discussion is required – come-back needed |
|  | Working assumption |
|  | Specific action needed from MCC |
|  | Contribution is for email discussion/approval |

# Opening of the meeting

Mr Wanshi Chen (RAN1 Chairman) welcomed the participants of the RAN WG1 #94 meeting and opened the meeting at 09:00.

Ms Sorour Falahati (Ericsson) on behalf of the European Friends of 3GPP, Apple, BlackBerry, Deutsche Telekom, Ericsson, Huawei, Intel, InterDigital, KPN, Motorola, Nokia, Orange, Qualcomm, SIMAlliance, Sony, Telefonica, Telenor, Thales, TIM, Vodafone, welcomed the delegates, and detailed the domestic arrangements for the full week.

## Call for IPR

The attention of the members of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The members take note that they are hereby invited:

-	to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

-	to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms (e.g. see the ETSI IPR forms http://webapp.etsi.org/Ipr/).

## Competition law statement

The Chairman also drew Member’s attention to the fact that 3GPP activities are subject to antitrust and competition laws and that compliance with said laws is therefore required of any participant of this WG meeting including the Chairman and Vice Chairmen. In case of question, please contact your legal counsel.

The present meeting will be conducted with strict impartiality and in the interests of 3GPP.

Furthermore, the Chairman reminded Members that timely submission of work items/contributions in advance of WG meetings is important to allow for full and fair consideration of such matters.

## Network usage conditions

The PCG has laid down the following network usage conditions:

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network. Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting chairman should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself. 1. DON’T place your WiFi device in ad-hoc mode 2. DON’T set up a personal hotspot in the meeting room 3. DO try 802.11a if your WiFi device supports it 4. DON’T manually allocate an IP address 5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

## Check-in for Registered Delegates

The attention of the delegates to this meeting was drawn to the fact that it is not permitted to check in other delegates on their behalf. In the event of technical difficulties preventing check-in, delegates should present themselves in person to the Secretary.

## Aspects related to RAN1 Meeting Management

Delegates are encouraged to check R1-1721392 for some thoughts on RAN1 meeting management.

RAN1 chair recalled the absolute necessity to follow the rules laid down in the above document.

# Approval of Agenda

R1-1808190	Draft Agenda of RAN1#94 meeting	RAN1 Chair

Wanshi Chen (RAN1 Chair) proposed the agenda for the meeting, as well the schedule of the week.

Discussion: No comments.

Decision: The agenda is approved.

Announcement was made regarding the ETSI Worshop "Writing World Class standards"

The workshop aims at improving the overall quality of our deliverables; as contributors to the RAN1 specifications, companies are invited to participate. Brief description of the workshop content includes:

- What makes a World Class Standard

- Well written requirements

  - Complete, precise, unambiguous, necessary, etc.

- Use of shall, should and may

- Requirements and their context and conditions

- Typical causes of misunderstanding

  - Poor language, wrong document structure, ambiguity, verbosity, vagueness, etc.

# Highlights from RAN plenary

R1-1808002	Highlights from RAN#80	RAN1 Chair

Discussion: Ad-Jan 2019 meeting is confirmed - RAN1 chair clarified that no LTE, no NR CRs will be considered and the ad-hoc meeting schedule should be focused on other NR stuff (mainly new SI...)

Intel: Why is the uncertainty on NOMA higher than to other features? RAN1 chair --> refer to RAN plenary discussion, mainly the way to make progress is the issue.

Decision: The document is noted.

# Approval of Minutes from previous meeting

R1-1808001	Report of RAN1#93 meeting	MCC (ETSI)

The document was presented by Patrick Merias from MCC (ETSI Mobile Competence Center) and provides the report from last RAN1#93 meeting in Busan.

Discussion: MCC recalled that all CRs presented for approval at RAN#80 were duly implemented. Nevertheless, eV2X CR to 36.211 (draft CR endorsed as a result of post RAN1#92bis email discussion prior to RAN1#93) was accidently forgotten. Formal CR should be treated in the corresponding LTE maintenance session (see 6.1.3).

Decision: The document is approved.

# Incoming Liaison Statements

R1-1808179	LS on Joint ETSI - OSA Workshop: Open Implementations & Standardization	ETSI

Joint ETSI - OSA Event on Open Implementations and Standardization. Preceded by a one day training on 11 December, the Joint ETSI - OSA Workshop: Open Implementations and Standardization will take place on 12-13 December 2018 at ETSI premises in Sophia Antipolis, France.

Decision: The document is noted.

LTE

// 6.1.1 related

R1-1808022	Wi-Fi Alliance Coexistence Test Plan for LTE-LAA Version 1.1.10	Wi-Fi Alliance

R1-1808023	Reply LS on PSCCH/PSSCH subframe numbering issue in partial network coverage	RAN2, Ericsson

//6.1.3 related

R1-1808027	Reply LS to RAN1 on RRC parameters for V2X Phase 2	RAN2, LG Electronics

//6.1.5 related

R1-1808005	LS response on NB-IoT downlink channel quality determination and report	RAN4, Ericsson

R1-1808011	Reply LS on Narrowband measurement accuracy improvements	RAN4, Huawei

R1-1808024	Reply LS on narrowband measurement	RAN2, Huawei

// 6.1.9 related

R1-1808032	LS on final RAN2 agreements for euCA	RAN2, Nokia

The following LSs are noted (not presented online unless explicitly requested)

R1-1808006	Reply LS on updates for TS 36.300	RAN3, Huawei

R1-1808013	LS on eFeMTC CRS muting	RAN4, Huawei

R1-1808026	Reply LS on PC5 transmission mechanism selection	RAN2, LG Electronics

R1-1808031	Reply LS on CP latency reduction	RAN2, Ericsson

NR

// General

R1-1808003	LS on the feasibility of 1 symbol GP for 60KHz in FR1 and 120KHz in FR2	RAN4, MediaTek

Discussion: Panasonic --> RAN1 understanding is that GP length is assumed per UE, the LS content seems different?

MediaTek will check with RAN4 and clarify offline the RAN4 requirements

Decision: The document is noted.

R1-1808176	LS on clarification of useServingCellTimingForSync	RAN4, MediaTek

R1-1809228	[draft] Response LS on clarification of useServingCellTimingForSync	LG Electronics Inc.

//7.1.1 related

R1-1808004	LS on RMSI CORESET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth	RAN4, CATT, Samsung

R1-1808025	Reply LS on RMSI 9773SET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth	RAN2, CATT

R1-1808860	LS on ssb-PositionsInBurst mismatch in SIB1 and ServingcellConfigCommon	RAN2, vivo

Decision: The document is noted. Reply LS is necessary

R1-1808214	Discussion on ssb-PositionsInBurst mismatch in SIB1 and ServingcellConfigCommon	vivo

R1-1809710	Draft Reply LS Discussion on ssb-PositionsInBurst mismatch in SIB1 and ServingcellConfigCommon	Nokia, Nokia Shanghai Bell	(Revision of R1-1808964)

R1-1809255	Draft Reply LS on ssb-PositionsInBurst mismatch in SIB1 and ServingcellConfigCommon	Ericsson

R1-1808028	Reply LS on Agreements on Paging	RAN2, Qualcomm

R1-1808172	LS on Further Agreements on Paging Occasions	RAN2, Qualcomm

Discussion: Debate on the fact that RAN2 sent that LS from July ad-hoc meeting, and agreed in principle by email after the ad-hoc a CR not fully aligned with the LS.

Companies are encouraged to check with their RAN2 colleagues to make sure there is no unconsistency in RAN1 understanding.

Decision: The document is noted.

R1-1809226	[draft] Response LS on Further Agreements on Paging Occasions	LG Electronics Inc.

R1-1809374	[Draft] LS Reply on Further Agreements on Paging Occasions	Nokia, Nokia Shanghai Bell

R1-1808169	LS on short message transmission using DCI	RAN2, Ericsson

Decision: The document is noted. Reply LS is necessary

R1-1808454	[draft] Response LS on short message transmission using DCI	LG Electronics

R1-1808566	On short message transmission using DCI	Nokia, Nokia Shanghai Bell

R1-1808567	[DRAFT] Reply LS On short message transmission using DCI	Nokia, Nokia Shanghai Bell

R1-1808168	LS on overlapping SIB1 and SI messages	RAN2, Ericsson

Decision: The document is noted. Reply LS is necessary

R1-1808650	[DRAFT] Reply LS on Overlapping SIB1 and SI messages	Intel Corporation

R1-1809372	[Draft] LS Reply on overlapping SIB1 and SI messages	Nokia, Nokia Shanghai Bell

R1-1808167	Reply LS on numberOfRA-PreamblesGroupA	RAN2, Nokia

RAN2 reached a conclusion the current specification is clear and no changes are being made. The proposal by RAN1 would have added more overhead unnecessarily to the RRC signaling.

No impact to RAN1 spec

Decision: The document is noted.

R1-1808175	LS on RA preambles mapping to SSBs	RAN2, ZTE

R1-1808963	Draft Reply LS on RA preambles mapping to SSBs	Nokia, Nokia Shanghai Bell

//7.1.2 related

R1-1808166	LS on termination of contention-free BFR	RAN2, Intel

Decision: The document is noted. Reply LS is necessary (also related to 7.1.3.1).

R1-1808366	Draft reply LS to RAN2 on termination of contention-free BFR	CATT

R1-1808649	Draft reply to LS on beam failure recovery	Intel Corporation

R1-1808729	DRAFT Reply LS on termination of contention-free BFR	Samsung

Withdrawn

R1-1808884	Discussion on the reply to “LS on termination of contention-free BFR”	Guangdong OPPO Mobile Telecom

R1-1809234	[DRAFT] LS Response to RAN2 LS to RAN1 on LS on termination of contention-free	Nokia, Nokia Shanghai Bell

R1-1809395	Draft reply LS on termination of contention-free BFR	Motorola Mobility, Lenovo

//7.1.3 related

(in response to R1-1807732 (R2-1808992))

R1-1809133	Draft reply LS on PDCCH common search space configurations	NTT DOCOMO, INC.

R1-1808173	LS on additional TDD configuration periodicities	RAN2, Nokia

Decision: The document is noted. Reply LS is necessary. See 7.1.3.1

R1-1809390	On Additional TDD configuration periodicities	Nokia, Nokia Shanghai Bell, SPRINT Corporation

R1-1809391	DRAFT Response LS on additional TDD configuration periodicities	Nokia, Nokia Shanghai Bell, SPRINT Corporation

R1-1808171	Reply LS on Bandwidth configuration for initial BWP	RAN2, NTT DOCOMO

RAN2 agreed to support both options. Both options are supported for a UE capable of basic BWP operation with restriction (#6-1) as mandatory without IOT bit.

Option 1:	BWP-DownlinkCommon/UplinkCommon for initialDownlink/UplinkBWP (BWP ID #0) plus BWP-Downlink/Uplink for one configured downlink/uplinkBWP (BWP ID #1);

Option 2:	BWP-DownlinkCommon/UplinkCommon and BWP-DownlinkDedicated/UplinkDedicated for initialDownlink/UplinkBWP (BWP ID #0).

Decision: The document is noted. Check offline how this is going to be applied and corresponding spec impacts, if any.

R1-1808174	LS on Active DL BWPs overlapping/non-overlapping with Initial DL BWP	RAN2, ZTE

Decision: The document is noted. Reply LS is necessary (related to many other aspects)

R1-1808651	[DRAFT] Reply LS on Active DL BWPs overlapping/non-overlapping with Initial DL BWP	Intel Corporation

R1-1808947	Draft reply LS on Active DL BWPs overlapping/non-overlapping with Initial DL BWP	Huawei, HiSilicon

R1-1809373	[Draft] LS Reply on Active DL BWPs overlapping/non-overlapping with Initial DL BWP	Nokia, Nokia Shanghai Bell

R1-1809396	Draft reply LS on Active DL BWPs overlapping/non-overlapping with Initial DL BWP	Motorola Mobility, Lenovo

// 7.1.4 related

R1-1808007	LS on NR Cell Information signalled over X2 for EN-DC	RAN3, Ericsson

R1-1808029	Reply LS on UE power class and UE capability on SRS carrier switch	RAN2, NTT DOCOMO

Decision: The document is noted. Reply LS is necessary

R1-1809124	Draft reply LS on UE power class and UE capability on SRS carrier switch	Huawei

//7.1.7 related

R1-1808017	LS on UE feature list	RAN4, NTT DOCOMO, Intel

R1-1808020	LS on NR UE feature list	RAN, NTT DOCOMO

R1-1808859	LS on fallback interpretation of UE FeatureSetDown(Up)LinkPerCC capability fields	RAN2, Intel

// 7.2.4 related

R1-1808008	Additional Reply LS on eV2X evaluation methodology	SAE C-V2X TC Advanced Applications Task-Force, AT&T

R1-1809720	LS on Prioritised Use Cases and Requirements for consideration in Rel-16 NR-V2X	5GAA WG1

// Others

R1-1808303	Liaison ESOA (EMEA Satellite Operators Association) on 5G for Satellite Systems	ESOA, Thales

The following LSs are noted (not presented online unless explicitly requested)

R1-1808009	LS reply on UL timing difference in EN-DC	RAN4, Intel Corporation

R1-1808014	LS on 2RX requirements for vehicle mounted NR UE	RAN4, LG Electronics

R1-1808015	RAN4 response LS on Coexistence of NR-NR and NR-LTE networks	RAN4, Qualcomm Incorporated

R1-1808018	LS reply on Coexistence of NR-NR and NR-LTE networks with synchronised, unsynchronised and semi-synchronised operation in 3400-3800 MHz	RAN, Huawei

R1-1808019	LS reply on minimum number of RX antennae for NR vehicular Ues	RAN, Samsung (Harman)

R1-1808021	LS on L2 measurement specification for NR in R15	SA5, Ericsson

R1-1808030	LS on RAN2 progress on ANR	RAN2, vivo

R1-1808170	Reply LS on NR Cell Information signalled over X2 for EN-DC	RAN2, Ericsson

R1-1808177	LS on potential enhancement of DR-mode of operation	SA2, Intel

R1-1808178	LS on the transmission of video over PC5 interface	SA4, Samsung

R1-1808180	IEEE 802.11 WG LS to 3GPP RAN4 on certain channel combinations for LAA in 5GHz	IEEE 802.11 Working Group

R1-1808181	IEEE 802.11 WG LS to 3GPP RAN in relation to a proposed joint Coexistence Workshop	IEEE 802.11 Working Group

Discussion: Should be further discussed in RAN as proposed timeslots for the workshop may not fit with RAN1 future schedule.

Decision: The document is noted.

LS received during the week.

R1-1809876	LS on OSI acquisition	RAN2, Qualcomm

Decision: The document is noted.

R1-1809892	LS on default configuration for L1 parameter	RAN2, NTT DOCOMO

RAN2 asks RAN1 to define the default value for the L1 parameters required during the transition from RRC IDLE/INACTIVE to RRC CONNECTED state but for which the values are not provided via MIB or SIB.

Decision: The document is noted.

R1-1809942	LS on UE capability for simultaneousTxSUL-NonSUL	RAN2, Huawei

Decision: The document is noted. Treated under 7.1.7

R1-1809948	LS response on NPBCH-based RRM measurement for FeNB-IoT UE	RAN4, Qualcomm

Decision: The document is noted. Treated under 6.1.5

R1-1810001	Reply LS on SRS carrier switching	RAN4, Huawei

Decision: The document is noted.

# E-UTRA

## Maintenance of E-UTRA Releases 8 – 15

For 6.1.2, 6.1.3, 6.1.4, 6.1.5, 6.1.6,

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

Friday

Editors to prepare CRs incorporating the TPs/agreements/working assumptions by August 30th for 6.1.2/6.1.3/6.1.4/6.1.5/6.1.6, respectively, for comments and endorsement d by Sept. 4th

### Maintenance of E-UTRA Release 8 – 14

Friday

R1-1809664	Chairman's notes of AI 6.1.1 Maintenance of E-UTRA Release 8 – 14	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content is incorporated below.

NB-IoT

NPRS

R1-1808113	On handling new and legacy NPRS for in-band operation mode	Huawei, HiSilicon

R1-1808114	Correction to NPRS mapping	Huawei, HiSilicon

R1-1808992	draft CR on NPRS enhancement	Ericsson

R1-1809000	Remaining details for enhanced NPRS	Qualcomm Incorporated

Discuss further offline. Come back later

R1-1809592	WF on NPRS Remaining Issues	Qualcomm, Huawei, HiSilicon, Ericsson

Agreement:

- If the periodicity configured for legacy NPRS is equal to that of new NPRS,

  - The UE is not expected to be configured with overlapped REs between legacy NPRS and new NPRS.

- Otherwise,

  - A RE configured for legacy NPRS shall not be used for new NPRS.

R1-1809599 (Revision of R1-1808114)	Correction to NPRS mapping	Huawei, HiSilicon, Qualcomm, Ericsson

Final CR agreed in R1-1809605 (Rel-14, TS36.211, CR0454), R1-1809606 (Rel-15, TS36.211, CR0455)

R1-1808994	Transmission of NPSS/NSSS/NPBCH	Qualcomm Incorporated

Conclusion:

- The described behaviour as in R1-1808994 is the common understanding in RAN1

  - No CR is necessary

R1-1809003	Correction for interference randomization for NB-IoT	Qualcomm Incorporated

The draft CR in R1-1809003 is endorsed (with correction of typo “randomziaiton” in the reason for change) – final CR agreed in R1-1809535 (CR 0452, Rel-14), & R1-1809536 (CR 0453, Rel-15)

eMTC

R1-1808995	PRG for eMTC	Qualcomm Incorporated

The draft CR is endorsed in principle (need to move the sentence starting with “otherwise….” before the table to the non-BL/CE case). Come back later for final CR (R1-1809537, CR 1121, Rel-13), (R1-1809538, CR 1122, Rel-14) , (R1-1809539, CR 1123, Rel-15)

Final CR agreed (R1-1809537, CR 1121, Rel-13), (R1-1809538, CR 1122, Rel-14) , (R1-1809539, CR 1123, Rel-15)

R1-1808996	Correction for HARQ-ACK delay in eMTC	Qualcomm Incorporated

Discuss further offline. Come back later

R1-1809635 (Revision of 1808996)	Correction for HARQ-ACK delay in eMTC	Qualcomm Incorporated

Final CRs agreed in R1-1809653 (TS36.213, Rel-14, CR1156) and R1-1809654 (TS36.213, Rel-15, CR1157)

R1-1809004	Correction on DCI for HARQ-ACK bundling	Qualcomm Incorporated

The draft CR is endorsed in principle. Further discuss offline to refine the wording. Come back later. R1-1809540 (CR0298, Rel-14), R1-1809541 (CR0299, Rel-15)

Final CR approved in R1-1809540 (CR0298, Rel-14), R1-1809541 (CR0299, Rel-15)

R1-1809378	Clarification of number of PDSCH repetitions when R_CSI is 1 in LTE-MTC	Ericsson

Conclusion:

- For the issue raised in R1-1809378, the agreements cited are applicable only to TDD.

  - No CR is necessary

eCA

R1-1808593	Correction on TDD HARQ-ACK reporting procedure for different UL/DL configurations	Huawei, HiSilicon

The draft CR is endorsed. Final CRs agreed in R1-1809542 (Rel-13, 1124), R1-1809543 (Rel-14, 1125), R1-1809544 (Rel-15, 1126)

R1-1808594	Correction on PUCCH format 4 HARQ-ACK procedure without adaptive codebook	Huawei, HiSilicon

The CR is agreed. Shadow CRs for Rel-14 & 15 agreed in R1-1809545 (Rel-14, 1127), R1-1809546 (Rel-15, 1128)

MUST

R1-1808652	Draft CR: Corrections to modulation signalling in MUST	Intel Corporation

Discuss further offline. Come back later

R1-1809610 (Revision of R1-1808652)	Draft CR: Corrections to modulation signalling in MUST	Intel Corporation

Final CRs agreed in R1-1809636 (TS36.212, Rel-14, CR0302) and R1-1809637 (TS36.212, Rel-15, CR0303)

MIMO

R1-1808653	Draft CR: Correction to PDSCH RE mapping in TM10	Intel Corporation

The draft CR is endorsed. Final CRs agreed in R1-1809547 (Rel-13, 1129), R1-1809548 (Rel-14, 1130), R1-1809549 (Rel-15, 1131)

R1-1808997	Rate matching for A-CSI-RS	Qualcomm Incorporated

Discuss further offline. Come back later

Decision: For email approval till 8/31 – Alberto (Qualcomm)

R1-1809005	Correction on advanced CSI codebook	Qualcomm Incorporated

The CR is endorsed (by updating the WI code by removing the dash in between). Final CRs agreed in R1-1809550 (Rel-14, CR1132), R1-1809551 (Rel-15, CR1133)

SRS carrier switching

R1-1808993	HARQ reference configuration for SRS carrier switching	Qualcomm Incorporated

The draft CR is endorsed in principle, with the following points for further discussion:

- Changes in 7.3.3 & 10.1.2A (neither/nor vs. either/or or other ways)

- Changes in Section 13

- Conditions for the parameter

Final CRs to be prepared in R1-1809552 (Rel-14, CR 1134), R1-1809553 (Rel-15, CR 1135)

Decision: CRs are postponed as proponent couldn’t find enough time to converge in all the details.

R1-1809001	Discussion on CA capability for SRS carrier switching	Qualcomm Incorporated

For the first issue, further discussion offline. Come back later

The second TP raised in R1-1809001 is endorsed. Final CRs agreed in R1-1809554 (Rel-14, CR 1136) & R1-1809555 (Rel-15, CR 1137)

feMBMS

R1-1808998	Capability for FeMBMS	Qualcomm Incorporated

The CR is endorsed in principle (further refine wording offline). Final CRs agreed in R1-1809556 (Rel-14, CR1138) & R1-1809557 (Rel-15, CR1139)

UL Capacity Enhanements

R1-1808999	SRS transmission in UpPTS for special subframe configuration 10	Qualcomm Incorporated

The CR is endorsed in principle (further refine wording offline, & further check whether or not there are other missing parts. Note that the impact spec should be 36.213). Final CRs agreed in R1-1809558 (Rel-14, CR1140), R1-1809559 (Rel-15, CR1141)

R1-1809006	HARQ-ACK payload size determination for crs-less special subframe configuration 10	Qualcomm Incorporated

The draft CR is endorsed. Final CRs agreed in R1-1809560 (Rel-14, CR1142), R1-1809561 (Rel-15, CR1143)

Antenna Selection

R1-1809002	Discussion on closed loop PUSCH antenna selection	Qualcomm Incorporated

Continue discussion offline. Come back later.

Agreement:

Clarify that the intended UE behaviour for closed loop PUSCH antenna selection is “Most recent command” = “latest DCI received at or before n-4” for Rel-14 and later UEs.

R1-1809607	Clarification for antenna selection	Qualcomm Incorporated

Final CRs agreed in R1-1809638 (TS 36.213, Rel-14, CR1148) and in R1-1809639 (TS36.213, Rel-15, CR1149)

R1-1809244	Correction on transmit antenna selection	CMCC

Continue discussion offline. Come back later.

R1-1809633	Change request on UE transmit antenna selection	CMCC

Final CRs agreed in R1-1809640 (TS36.213, Rel-12, CR1150), R1-1809641 (TS36.213, Rel-13, CR1151), R1-1809642 (TS36.213, Rel-14, CR1152) and R1-1809643 (TS36.213, Rel-15, CR1153)

R1-1809634	Change request on 1T2R antenna switching of SRS	CMCC

Final CR agreed in R1-1809644 (TS36.213, Rel-15, CR1154)

### Maintenance of Release 15 Shortened TTI and processing time for LTE

LTE_sTTIandPT-Core; WID in RP-171468.

Friday

R1-1809665	Chairman's notes of AI 6.1.2 Maintenance of Release 15 Shortened TTI and processing time for LTE	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content is incorporated below.

R1-1809597	Outcome of sTTI offline	Ericsson

#### Maintenance for shortened processing time for 1ms TTI

None

#### Maintenance for shortened TTI with shortened processing time

R1-1809625	DRAFT LS on additional UE capabilities for sTTI	Ericsson

R1-1809683	DRAFT LS on sTTI	Ericsson

Decision: The draft LS is endorsed and final LS is approved in R1-1809687.

##### Aspects related to interaction between different TTI lengths

R1-1809293	TP on collision handling for subslot-PUSCH	Guangdong OPPO Mobile Telecom, CAICT

R1-1809601 (Revision of R1-1809293)	TP on collision handling for subslot-PUSCH		Oppo

R1-1809367	Corrections on interaction between different TTI lengths	Huawei, HiSilicon

Conclusion:

UE processing of MSG3 PUSCH is not interrupted by sPUSCH grants (as per current specification).

##### Aspects related to DL control channel

R1-1809241	Correction on the starting SCCE index for an aggregation level	Ericsson

The text proposals in R1-1809241 are endorsed.

##### Aspects related to UL control channel

R1-1809216	Aspects related to UL control channel	Qualcomm Incorporated

The text proposal in R1-1809216 is endorsed with the following modification:

Add SR after HARQ-ACK in “ is the total number of HARQ-ACK bits including CRC bits transmitted on PUCCH format 4 in slot/subslot I”

R1-1809218	Corrections on PRB determination for subslot PUCCH format 1a/1b	ZTE

R1-1809350	Corrections on UCI mapping in 36.212	Huawei, HiSilicon, LG Electronics, Nokia, Nokia Shanghai Bell

The text proposals in R1-1809350 are endorsed.

##### Aspects related to DL data channel

R1-1808455	Corrections related to CSI feedback	LG Electronics

The text proposal in Section 2.2 of R1-1808455 is endorsed.

Agreement:

UE capability on the maximum number of CSI processes to be updated is defined separately for

- {slot,slot}

- {subslot,slot}

- {subslot,subslot} processimg timeline set 1

- {subslot,subslot} processimg timeline set 2

Note: This replaces the existing UE capability on the maximum number of CSI processes to be updated for sTTI. RAN2 should be informed about this.

R1-1809217	Aspects related to DL data channel	Qualcomm Incorporated

Late submission

R1-1809354	Correction on rate matching for SPS sPDSCH	Huawei, HiSilicon

Agreement:

The sPDCCH rate matching for dynamic retransmissions of SPS sPDSCH applies the same mode as for scheduled sPDSCH scrambled with C-RNTI.

R1-1809579	Correction on rate matching for SPS sPDSCH	Huawei, HiSilicon

The text proposal in R1-1809579 is endorsed.

R1-1809364	Corrections on SFBC sPDSCH RE mapping	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics

The text proposal in R1-1809364 is endorsed.

R1-1809365	Corrections on spreading sequence for sPDSCH DMRS	Huawei, HiSilicon, LG Electronics, Nokia, Nokia Shanghai Bell, ZTE, Ericsson

The text proposal in R1-1809365 is endorsed.

R1-1809384	Corrections to CSI reporting for shorter TTI in 36.213 – TP#1	Nokia, Nokia Shanghai Bell, Ericsson

R1-1809578 (Revision of R1-1809384)	Corrections to CSI reporting for shorter TTI in 36.213 – TP#1	Nokia, Nokia Shanghai Bell

The text proposal in R1-1809578 is endorsed.

R1-1809385	Correction to slot/subslot PDSCH RBG numbering/order for Resource Allocation Type 0 in 36.213 – TP#2	Nokia, Nokia Shanghai Bell, LG Electronics, Ericsson, Huawei, HiSilicon

The text proposal in R1-1809385 is endorsed

R1-1809386	Correction to PUSCH/PDSCH processing skipping in case of DCI Format 7-X reception in 36.213 – TP#4	Nokia, Nokia Shanghai Bell, LG Electronics, Ericsson, Huawei, HiSilicon

The text proposal in R1-1809386 is endorsed

##### Aspects related to UL data channel

R1-1809007	Aspects related to UL Data Channel	Qualcomm Incorporated

Agreement:

The sRBG size and starting point granularity for 2/3os and 7os sPUSCH is for 1.4 MHz and 3 MHz as per table below (the same as legacy).

|  | 1.4 MHz | 3 MHz |
| --- | --- | --- |
| sRBG size | 1 | 1 |
| Starting point granularity | 1 | 1 |

Agreement:

For PUSCH without UL-SCH, define a UE capability per UL sTTI length whether to support BPRE derivation based on the actual derived O_CQI. Define an RRC parameter uplinkPower-CSIPayload to configure a UE to derive BPRE based on either the actual value of O_CQI or the largest value of O_CQI across all RI values. In case the UE does not support the capability, the UE will derive BPRE based on the largest value of O_CQI across all RI values.

The text proposal in section 3 of R1-1809007 is endorsed

R1-1809581	Clarification for UL Resource Allocation Type 0	Qualcomm Incorporated

The text proposal in R1-1809581 is endorsed

Agreement:

In case of sPUSCH in UpPTS, the following TBS scaling factors apply depending on the number of symbols, X:

- X ≤ 3: 0.125

- X > 3: 0.375

R1-1809580	TBS Scaling for Slot TTI in UpPTS	Qualcomm Incorporated

The text proposal in R1-1809580 is endorsed

R1-1809351	Corrections on UL power control in case of UL DMRS sharing	Huawei, HiSilicon

R1-1809366	Corrections on RV determination for sPUSCH	Huawei, HiSilicon, LG Electronics, Nokia, Nokia Shanghai Bell, ZTE, Ericsson

The text proposal in R1-1809366 is endorsed.

R1-1809387	Correction of transport block size determination for sPUSCH in 36.213 – TP#3	Nokia, Nokia Shanghai Bell, LG Electronics, Ericsson, Huawei, HiSilicon

The text proposal in R1-1809387 is endorsed

R1-1809388	Correction of UL HARQ process definition for SPT PUSCH & sPUSCH in 36.213 – TP#5	Nokia, Nokia Shanghai Bell, Ericsson

Agreement:

Define a pseudo HARQ-ID to subframe mapping for synchronous HARQ (HARQ process pool sharing) for improved asynchronous and synchronous HARQ coexistence.

R1-1809389	Correction to sPUSCH resource allocation in 36.213 – TP#15	Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, LG Electronics, Ericsson

The text proposal in R1-1809389 is endorsed

##### Aspects related to FS2

R1-1808456	eIMTA support with sTTI operation	LG Electronics

The text proposal in R1-1808456 is endorsed

R1-1809363	Corrections on DAI timing for FS2 with sTTI	Huawei, HiSilicon, Ericsson

The text proposals in R1-1809363 are endorsed

#### Other

R1-1808457	Corrections on collision handling between subslot UL SPS and SPUCCH	LG Electronics

The text proposal in R1-1808457 is endorsed

R1-1809352	Corrections on HARQ-ACK feedback for DL SPS	Huawei, HiSilicon

The text proposal in R1-1809352 is endorsed

R1-1809353	Remaining issues on SPS	Huawei, HiSilicon

### Maintenance of Release 15 3GPP V2X Phase 2

LTE_eV2X-Core; WID in RP-171740.

Friday

R1-1809666	Chairman's notes of AI 6.1.3 Maintenance of Release 15 3GPP V2X Phase 2	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content is incorporated below.

R1-1808328	Introduction of eV2X	Ericsson

In the e-mail discussion following RAN1#92bis, draft CR for introducing eV2X functionality in 36.211 was endorsed. However, by mistake, the endorsed CR was submitted in R1-1807408 as a draft CR and not a final CR, resulting in the CR package to RAN including the CRs for 36.212 and 36.213 only but not 36.211. Thus, the current RAN1 specifications are incomplete in terms of eV2X support.

The changes in the current CR (R1-1808328) is identical to the already endorsed draft CR in R1-1807408 but is based on version 15.2.0 instead of 15.1.0.

Decision: CR0451 to 36.211 Rel-15 is agreed.

#### Maintenance for Carrier Aggregation (up to 8 PC5 carriers)

R1-1808902	Discussion on synchronization carrier configuration	Guangdong OPPO Mobile Telecom

#### Maintenance for Support for 64-QAM

R1-1808873	Text proposal to support 64-QAM in eV2X	Guangdong OPPO Mobile Telecom

#### Maintenance for Resource pool sharing between mode-3 and mode-4 users

R1-1808903	Discussion on necessary L1 support for mode 3 sensing and reporting	Guangdong OPPO Mobile Telecom

#### Maintenance for Maximum time reduction between packet arrival at layer 1 and resource selection for transmission

R1-1808874	Text proposal to support maximum time reduction in eV2X	Guangdong OPPO Mobile Telecom

#### Other

R1-1808367	Corrections to the PSCCH decoding capability in TS 36.213	CATT

Agreed in R1-1809656 (rev1, CR1119, TS36.213, Rel-15)

Friday: Spec 36.213 editor commented that this is an individual CR, proponent did not follow the guidance set in the agenda of the meeting.

RAN1 chair: The decision is cancelled, CR is declared "not pursued" while the corresponding changes will be incorporated by editor.

R1-1809292	[Draft] LS on RRC parameters SL_SLSS_Tx_Enabled for V2X Phase 2	Guangdong OPPO Mobile Telecom

### Maintenance of Release 15 Even further enhanced MTC for LTE

LTE_eMTC4-Core; WID in RP-172811.

Friday

R1-1809667	Chairman's notes of AI 6.1.4 Maintenance of Release 15 Even further enhanced MTC for LTE	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content is incorporated below.

#### Maintenance for Resynchronization signal

R1-1808623	Remaining collision issues for RSS transmission	ZTE

R1-1808984	Maintenance issues for Resynchronization signal for Rel-15 LTE-MTC	Ericsson

R1-1809008	Maintenance for RSS	Qualcomm Incorporated

R1-1809531	Feature summary of 6.1.4.1 Maintenance for Resynchronization signal	Ericsson

R1-1809591	Remaining issues for 6.1.4.1 Maintenance for Resynchronization signal	Ericsson

Agreement:

The whole RSS of the subframe is dropped if any RSS PRB pair overlaps with any PSS/SSS/PBCH PRB pair.

Agreement:

The whole RSS of the subframe is dropped if any RSS PRB pair overlaps with any PRB pair carrying SIB1-BR or other SI.

Agreement:

A UE may assume that RSS punctures PRB pairs belonging to MPDCCH not associated with a RAR and PRB pairs belonging to PDSCH not addressed by RA-RNTI.

Agreement:

A UE may assume that the whole WUS subframe is dropped if WUS PRB pairs partially or fully overlap with RSS PRB pairs

R1-1809618	Text proposals for 6.1.4.1 Maintenance for Resynchronization signal	Ericsson

The text proposals in R1-1809618 are endorsed.

#### Maintenance for Early data transmission

R1-1808125	Corrections on EDT for eMTC	Huawei, HiSilicon

R1-1808624	Remaining issue for MTC EDT	ZTE

R1-1808985	Maintenance issues for Early data transmission for Rel-15 LTE-MTC	Ericsson

R1-1809565	Feature lead summary of Maintenance for Early data transmission in eMTC	Huawei

Agreement:

In 36.213, specify the number of PUSCH repetition for EDT Msg3 retransmissions, similarly as done in 36.213 for NB-IoT

R1-1809645	Updated feature lead summary of Maintenance for Early data transmission in eMTC	Huawei

The text proposal in Section 3.2 of R1-1809645 is endorsed with the following modifications in both places where the text occurs in the text proposal.

“….is the smallest integer multiple of M  that is equal to or larger than where  is the TBS corresponding to the PUSCH transmission scheduled by the Random Access Response Grant, and  is the value of the higher layer parameter edt-TBS-r15. M = 4 if > 4, M = 1 otherwise”

The text proposal in Section 3.5 of R1-1809645 is endorsed with a modification to change “is set to’true’” to “set to true” in the captions of the two tables.

Agreement:

The following text proposal is agreed:

------------------------------------ Start of Text proposal---------------------

6.2 Random Access Response Grant

- <unchanged part>

The repetition number field in the random access response grant configured by higher layers indicates the repetition level () for the initial transmission of Msg3 PUSCH as given in Table 6.2-C for CEmodeA and Table 6.2-D for CEmodeB, where

-	 is determined by higher layer parameter pusch-maxNumRepetitionCEmodeA-r13 if it is signaled, otherwise  = 8,

-	 is determined by higher layer parameter pusch-maxNumRepetitionCEmodeB-r13 if it is signaled, otherwise  = 512.

- If the higher layers indicate EDT to the physical layer as defined in [8] and if the UE is configured with higher layer parameter edt-smallTBS-Enabled-r15, the repetition number for the initial transmission of Msg3 PUSCH is the smallest integer multiple of M  that is equal to or larger than where  is the TBS of Msg3 PUSCH as determined in subclause 8.6.2, and  is the value of the higher layer parameter edt-TBS-r15. M = 4 if > 4, M = 1 otherwise.

- <unchanged part>

----------------------------------- end of Text proposal-----------------------

#### Maintenance for Wake-up signal

R1-1808126	Corrections on MWUS	Huawei, HiSilicon, Ericsson, Qualcomm, LG, Samsung, ZTE

R1-1808986	Maintenance issues for Wake-up signal for Rel-15 LTE-MTC	Ericsson

R1-1809009	Maintenance for wake-up signal	Qualcomm Incorporated

R1-1809520	Summary of 6.1.4.3 Maintenance for MWUS	Qualcomm Incorporated

The text proposals in R1-1808126 are endorsed.

The text proposal in Section 2.3 of R1-1809520 is endorsed.

R1-1809657	Remaining issues of 6.1.4.3 Maintenance for MWUS		Qualcomm Incorporated

Agreement:

===== start of TP =====

6.11B.2	Mapping to resource elements

The same antenna port shall be used for all symbols of the MWUS within a subframe. The UE shall not assume that the MWUS is transmitted on the same antenna port as any of the downlink reference signals or synchronization signals. If only one CRS port is configured by the eNB, the UE may assume the transmission of all MWUS subframes is using the same antenna port; otherwise, the UE may assume the same antenna port is used for MWUS transmission in DL subframes w0+2n and w0+2n+1, where w0 is the first DL subframe of the MWUS transmission as specified in [4], and n=0,1,…. .

===== end of TP =====

R1-1809661	Draft LS on WUS transmission scheme	Qualcomm

#### Maintenance for PUSCH sub-PRB allocation

R1-1808625	Remaining issue for Sub-PRB allocation	ZTE

R1-1808987	Maintenance issues for PUSCH sub-PRB allocation for Rel-15 LTE-MTC	Ericsson

R1-1809010	Maintenance for PUSCH subPRB allocation	Qualcomm Incorporated

R1-1809104	Discussion on RV determination for sub-PRB allocation	Sharp

R1-1809529	Rel15 MTC Sub-PRB PUSCH Maintenance Summary	Sierra Wireless

Agreement:

Cyclic repetition is not supported for PUSCH sub-PRB transmissions in CE Mode B

Agreement:

For CE Mode A and B, RV cycling is supported for PUSCH sub-PRB repetitions.

- RV changes per repetition which occurs every (RU Size *number of RU) transmission subframes.

- For CE Mode A, the first RV is indicated in DCI and subsequent RVs continue following the RV order 0, 2, 3, 1;

- For CE Mode B, the first RV is always 0 and the RV order is always 0, 2, 3, 1.

The text proposals in Section 2.3, 2.4, 2.5, 2.6, 2.7, of R1-1809529 are endorsed.

The text proposal in Section 2.8 excluding the changes to Section 8.1.6 of TS36.213 are endorsed.

R1-1809627	Rel15 Part II - MTC Sub-PRB PUSCH Maintenance Summary	Sierra Wireless

The text proposal in Section 2.1 of R1-1809627 is endorsed.

Agreement:

For Sub-PRB in CE Mode A, the UE considers the DCI valid even if the number of transmitted subframes is greater than 'pusch-maxNumRepetitionCEmodeA'.

#### Maintenance for Flexible PDSCH/PUSCH starting PRB

R1-1808127	Corrections on more flexible PDSCH/PUSCH resource allocation	Huawei, HiSilicon

R1-1808988	Maintenance issues for Flexible PDSCH/PUSCH starting PRB for Rel-15 LTE-MTC	Ericsson

R1-1809011	Maintenance for flexible starting PRB	Qualcomm Incorporated

R1-1809526	Summary of Flexible PDSCH/PUSCH starting PRB	Samsung

Agreement:

Update the size of RA field in TS 36.212 clause 5.3.3.1.10 to ceil{log2(6 NULRB)} for FDD

Agreement:

If frequency hopping is enabled for PDSCH, the PDSCH shall be transmitted in subframe  within the  consecutive downlink subframes using the PRB resources of the narrowband nNB(i) with same RIV as that of narrowband nNB(i0).

Agreement:

If frequency hopping is enabled for PUSCH, and the UE is not configured with ce-PUSCH-FlexibleStartPRB-AllocConfig, PUSCH is transmitted in subframe  within the   consecutive uplink subframes using the  PRB resources of the narrowband nNB(i)  with same RIV as that of narrowband nNB(i0).

Agreement:

- For PDSCH CEModeA: Propose to reduce the allocated length to satisfy the condition that LCRBs shall not exceed (NRBDL-nNB,0-RBstart).

- For PUSCH CEModeA: LCRBs shall not exceed (NRBUL-RBSTART) .

- For PUSCH CEModeB: Limit the starting/end RB for ‘110’ and ‘111’ within system bandwidth.

R1-1809628	Summary of Flexible PDSCH/PUSCH starting PRB	Samsung

Agreement:

L=2,3,4,5,6 are supported for flexible PUSCH starting PRB in TDD.

Agreement:

Update the size of RA field in TS 36.212 clause 5.3.3.1.10 to log25×NRBUL for TDD.

Agreement:

- For PDSCH CEModeA: if a RIV entry is to align the starting RB to the RBG boundaries, a set of two PRGs starts from the corresponding RBstart; if a RIV entry is to align the ending RB to the RBG boundaries, a set of two PRG ends at the corresponding RBstart+LCRBs-1.

- For PDSCH CEModeB: the narrowband is shifted to align with the RBG boundaries. The set of two PRGs are aligned with the shifted narrowband.

The text proposals in Section 5 of R1-1809628 are endorsed.

#### Other

R1-1808123	Discussion on flexible DL starting PRB allocation in CEMode A	Huawei, HiSilicon

R1-1808124	Discussion on the flexible starting PRB for PUSCH resource allocation in FDD	Huawei, HiSilicon

R1-1808561	Discussion on the flexible starting PRB for PUSCH resource allocation in TDD	Huawei, HiSilicon

### Maintenance of Release 15 Further enhancements of NB-IoT

NB_IOTenh2-Core; WID in RP-172063.

Friday

R1-1809668	Chairman's notes of AI 6.1.5 Maintenance of Release 15 Further enhancements of NB-IoT	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content is incorporated below.

R1-1809678	[DRAFT] LS on additional RRC parameters for Rel-15 NB-IoT	Huawei

Decision: The draft LS is endorsed and final LS is approved in R1-1809679.

R1-1809948	LS response on NPBCH-based RRM measurement for FeNB-IoT UE	RAN4, Qualcomm

R1-1809685	Introduction of NPBCH based measurements	Qualcomm Incorporated

The text proposal in R1-1809685 is endorsed.

R1-1809686	[DRAFT] LS on NPBCH-based measurements	Qualcomm Incorporated

Decision: The draft LS is endorsed and final LS is approved in R1-1809689.

R1-1809126	Clarification on the cyclic repetition L of Early Data Transmission	NTT DOCOMO, INC.

#### Maintenance for Latency and power consumption reduction

R1-1809502	Correction to NRS subframes and additional SIB1-NB-to-subframe mapping	LG Electronics

Revision of R1-1808458

R1-1809603	Correction to NRS subframes and additional SIB1-NB-to-subframe mapping	LG Electronics

The text proposals in R1-1809603 are endorsed.

##### Power consumption reduction for paging and connected-mode DRX

R1-1808128	Corrections on NWUS	Huawei, HiSilicon, Ericsson, Qualcomm, LG, Samsung, ZTE

R1-1808989	Maintenance issues for Wake-up signal for Rel-15 NB-IoT	Ericsson

R1-1809012	Maintenance for wake-up signal	Qualcomm Incorporated

R1-1809566	Feature lead summary on NB-IoT Rel-15 NWUS maintenance	Huawei, HiSilicon

Agreement:

The text proposals in R1-1808128 are agreed.

R1-1809662	Updated feature lead summary on NB-IoT Rel-15 NWUS maintenance	Huawei, HiSilicon

The following text proposal is endorsed:

-------------------------------------------- Beginning of TP -------------------------------------------------

10.2.6B.2	Mapping to resource elements

The same antenna port shall be used for all symbols of the NWUS within a subframe. The UE shall not assume that the NWUS is transmitted on the same antenna port as any of the downlink reference signals or synchronization signals. If only one NRS port is configured by the eNB, the UE may assume the transmission of all NWUS subframes is using the same antenna port; otherwise, the UE may assume the same antenna port is used for NWUS transmission in DL subframes w0+2n and w0+2n+1, where w0 is the first DL subframe of the NWUS transmission as specified in [4], and n=0,1,…. .

-------------------------------------------- End of TP ----------------------------------------------------------

##### Data transmission during the random access procedure

R1-1808129	Corrections on EDT for NB-IoT	Huawei, HiSilicon

R1-1809567	Feature lead summary of EDT during RACH in feNB-IoT	Huawei, HiSilicon

The text proposal in Section 3.1 of R1-1809567 is endorsed with modifications to change “were selected” to “would have been selected” and “would end” to “would have ended”.

The text proposal in Section 3.2 of R1-1809567 is agreed excluding the insertion of “16.3.3 and”.

R1-1809684	Updated feature lead summary of EDT during RACH in feNB-IoT	Huawei, HiSilicon

The text proposals in Sections 3.5.1 and 3.5.2 of R1-1809684 are endorsed.

R1-1809663	Clarification of TBS for retransmission of EDT Msg3	Sharp

##### Physical layer scheduling request

R1-1808130	Corrections on physical layer SR	Huawei, HiSilicon

R1-1808626	Remaining issues for scheduling request	ZTE

R1-1809568	Feature lead summary of physical layer SR in feNB-IoT	Huawei, HiSilicon

The text proposal in Section 2.2, 2.3 of R1-1809568 is endorsed.

Conclusion:

For an NB-IOT UE, in case an NPDCCH candidate partially or fully overlaps with an uplink SR transmission, the prioritization between the two is up to UE implementation.

Agreement:

The following text proposal is endorsed.

---------------------------------------------- Start of Text Proposal ------------------------------------------

-------------------------------------------- Unchanged parts omitted -----------------------------------------

16.5.3	UE procedure for transmitting SR

If the UE is configured with higher layer parameter sr-without-HARQ-ACK-Config, the UE is configured with Narrowband Random access channel parameters (NPRACH configuration) for SR transmission by higher layers.

The UE shall, if requested by higher layers for transmitting SR, start transmission of a narrowband random access preamble on the configured NB-IoT carrier at the next available NPRACH resource, unless the transmission would overlap with any subframe(s) of NPDSCH reception. The narrowband preamble is transmitted on the allocated subcarrier and a number of NPRACH repetitions for the associated NPRACH repetition level as indicated by higher layers. The narrowband preamble is transmitted with transmission power PNPRACH , as determined in subclause 16.3.2, commencing on the indicated NPRACH resource.

-------------------------------------------- Unchanged parts omitted -----------------------------------------

----------------------------------------------- End of Text Proposal ------------------------------------------

##### Semi-persistent scheduling

R1-1808131	Corrections on SPS	Huawei, HiSilicon

R1-1808627	Maintenance for semi-persistence scheduling	ZTE

R1-1809569	Feature lead summary of SPS in NB-IoT	Huawei, HiSilicon

Agreement:

For collisions between UL SPS transmission for BSR and NPDSCH, BSR is not transmitted, and remains pending. Adopt the TP below.

---------------------------------------------Start of Text Proposal----------------------------------------

16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel

< Unchanged parts are omitted >

Table 16.5.1-1A: for DCI format N0 for TDD.

| 0 | 0 |
| --- | --- |
| 1 | 8 |
| 2 | 16 |
| 3 | 32 |

If a NPUSCH transmission without a corresponding NPDCCH collides partially or fully with a NPDSCH transmission, the NPUSCH transmission is dropped.

< Unchanged parts are omitted >

-------------------------------------------- End of Text Proposal ----------------------------------

Conclusion:

For an NB-IOT UE, in case an NPDCCH candidate partially or fully overlaps with an UL SPS transmission, the prioritization between the two is up to UE implementation.

Conclusion:

The time between SPS NPUSCHs is always equal to semiPersistSchedIntervalUL i.e. not related to the scheduling delay.

#### Maintenance for NPRACH range enhancements

R1-1808132	Corrections on NPRACH range enhancements	Huawei, HiSilicon

R1-1808459	Discussion on NPRACH preamble formats for NPDCCH order	LG Electronics

R1-1809013	Maintance for NPRACH range enhancements	Qualcomm Incorporated

R1-1809563	Summary of Rel15 Maintenance for NPRACH range enhancements	Ericsson

The text proposals in Sections 2.1, 2.2, 2.4 of R1-1809563 are agreed.

Agreement:

The following text proposal is endorsed.

===== start TP =====

16.1.2	Timing synchronization

Upon reception of a timing advance command, the UE shall adjust uplink transmission timing for NPUSCH based on the received timing advance command.

The timing advance command indicates the change of the uplink timing relative to the current uplink timing as multiples of 16. The start timing of the random access preamble is specified in [3].

In case of random access response, an 11-bit timing advance command [8], TA, indicates NTA values by index values of TA = 0, 1, 2, ..., 1536, where an amount of the time alignment is given by NTA = TA 16. NTA is defined in [3].

===== end of TP =====

#### Maintenance for TDD

##### Downlink aspects

R1-1808133	Corrections on DL aspects for TDD	Huawei, HiSilicon

R1-1808460	Correction on RE mapping rule in a DwPTS	LG Electronics

R1-1808628	Remaining issue for TDD DL aspects	ZTE

R1-1809014	Maintenance for DL aspects of TDD	Qualcomm Incorporated

R1-1809230	Maintenance issues for TDD DL aspects for Rel-15 NB-IoT	Ericsson

R1-1809613	Feature lead summary of DL aspects for TDD NB-IoT	ZTE

R1-1809631	Feature lead summary of DL aspects for TDD NB-IoT	ZTE

The text proposals in Section 1.1, 2.2, of R1-1809631 are endorsed.

Agreement:

Clarify that the starting OFDM symbol in the special subframe for NPDCCH/NPDSCH with repetition shall be same as that of the normal DL subframe

Agreement:

NPDSCH/NPDCCH with repetitions is rate matched in a special subframe around REs that are used for NRS in a downlink subframe but not carrying NRS in a special subframe when l= NsymbDL-2, NsymbDL-1.

Agreement:

For NPDCCH with repetitions, NPDCCH is punctured by NRS REs in a special subframe when l= NsymbDL-5, NsymbDL-4. Also specify that rate matching is done according to the available REs within DwPTS for NPDCCH without repetition

Agreement:

Add the operation of insertion of <NIL> elements for NPDCCH with repetitions in the spec for TDD.

R1-1809675	Feature lead summary of DL aspects for TDD NB-IoT	ZTE

R1-1809690	Text proposal for DL aspects for TDD NB-IoT	Huawei, HiSilicon, ZTE

The text proposals in R1-1809690 are endorsed.

##### Uplink aspects

R1-1808134	Corrections on UL aspects for TDD	Huawei, HiSilicon

R1-1808461	Clarification on valid UL subframe caused by symbol groups dropping	LG Electronics

R1-1808629	Maintenance for TDD UL aspects	ZTE

R1-1808990	Maintenance issues for TDD UL aspects for Rel-15 NB-IoT	Ericsson

R1-1809015	Maintenance for UL aspects of TDD	Qualcomm Incorporated

R1-1809632	Feature lead summary of maintenance of UL aspects for TDD NB-IoT	ZTE

The text proposal in Section 1 of R1-1809632 is endorsed.

R1-1809655	DRAFT LS on UL PRB to DL PRB center offset for TDD NB-IoT	Ericsson

R1-1809680	DRAFT LS on UL PRB to DL PRB center offset for TDD NB-IoT	Ericsson

Decision: Final LS is approved in R1-1809681 with a modification to change “Study Item:	NB_IOTenh2” to “Work Item: NB_IOTenh2-Core”

The text proposal in Section 3.1 of R1-1809632 is endorsed with “… by the higher layer parameter” changed to “… by higher layers”.

Conclusion:

It is common understanding in RAN1 that NPUSCH is only transmitted in NB-IoT uplink subframe for TDD.

The text proposals in Sections 4.1, 4.2, 4.3, 5 of R1-1809632 is endorsed.

Agreement:

Text capturing the following should be added in TS36.211 (it is up to the editor how to include the text in the specifications):

The valid uplink subframes which are not used for NPRACH transmission due to insufficient number of consecutive valid uplink subframes to transmit G symbol groups back-to-back are not part of the NPRACH resource.

##### Common aspects

R1-1808135	Corrections on common aspects for TDD	Huawei, HiSilicon

R1-1808991	Maintenance issues for TDD DL/UL common aspects for Rel-15 NB-IoT	Ericsson

R1-1809016	Maintenance for Common aspects of TDD	Qualcomm Incorporated

R1-1809629	Summary of TDD Common aspects	LG Electronics

Agreement:

Update the section of 10.0.1.2 in TS36.211 to capture the agreement that DwPTS in special subframe configuration 0 and 5 for normal cyclic prefix is not used for NPDCCH and NPDSCH transmission for inband operation mode.

The text proposal in Annex A-2 is endorsed. Spec editor to check on alignment of parameter names with other specifications.

Conclusion:

It is common understanding that the special subframe configuration 0 and 5 are not indicated as NB-IoT DL subframe for inband NB-IoT carrier.

Agreement:

Update the section of 16.6 in TS36.213 to capture the agreement that UL/DL interlaced transmission and reception for single-HARQ capable UE is not supported.

The text proposal in Annex B-2 is endorsed.

Agreement:

Update the sections of 10.2.3.4 and 10.2.5.5 in TS36.211 in order to implement interference randomization for NPDSCH and NPDCCH, respectively, in TDD without capabilities.

- For NPDSCH, the relevant TP can be found in Section 2 of R1-1809016 and also in Annex C-2

- For NPDCCH, the relevant TP can be found in Section 2 of R1-1809016 and also in Annex D-2

Note: Editor to update these sections on interference randomization to be consistent with the FDD case

Agreement:

Update the section of 16.2.1.1.1 in TS36.213 in order to define an enhanced random access power control mechanism in TDD to be the same as FDD.

The text proposal in Annex E of R1-1809629 is endorsed.

Agreement:

Add section 10.2.2.4 in TS36.211 for description of guard period for TDD which UE needs to switch the direction of transceiver from uplink to downlink.

The text proposal for Section 10.2.2.4 in TS36.211 in R1-1809629 is endorsed. The relevant TP can be found in Annex G-3

#### Other

R1-1808562	Corrections on additional SIB1-NB transmission	Huawei, HiSilicon

The text proposal in Annex A of R1-1808562 is endorsed.

R1-1808630	Clarification of valid subframe issue for additional SIB-NB transmission	ZTE

### Maintenance of Release 15 Ultra Reliable Low Latency Communication for LTE

LTE_HRLLC-Core; WID in RP-181259.

Friday

R1-1809669	Chairman's notes of AI 6.1.6 Maintenance of Release 15 Ultra Reliable Low Latency Communication for LTE	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content is incorporated below.

R1-1809598	Outcome of HRLLC Offline	Ericsson

R1-1809682	DRAFT LS on additional UE capabilities for HRLLC	Ericsson

R1-1809688	DRAFT LS on HRLLC	Ericsson

Decision: The draft LS is endorsed and final LS is approved in R1-1809691.

#### Maintenance for Blind/HARQ-less Repetition for Scheduled DL-SCH Operation

R1-1808462	Corrections on blind/HARQ-less PDSCH repetition	LG Electronics

The text proposals in Sections 2.2, 3.2 and 5.2 of R1-1808462 are endorsed.

R1-1809649 (Revision of 1809630)	Corrections on PDSCH repetition for TDD	LG Electronics

The text proposal in R1-1809649 is endorsed.

R1-1809355	Text proposal to section 7.1 of TS 36.213 on the configurable HARQ process numbers	Huawei, HiSilicon

#### Maintenance for Repetition Enhancements for UL SPS Operation

R1-1808463	Corrections on power control for UL SPS operation	LG Electronics

The text proposal in R1-1808463 is endorsed.

Agreement:

- An RRC parameter for TPC index is to be introduced for subframe UL SPS configurations

- If a UE is configured with multiple UL SPS configurations in a serving cell, the same TPC index for DCI format 3/3A applies to all the UL SPS configurations in the serving cell

R1-1809242	On multiple UL SPS configuration handling for URLLC	Ericsson

Conclusion:

In the event of a resource conflict in the same serving cell between the initial transmisions within a configured grant bundle from multiple different UL SPS configurations configured with Uplink Semi-Persistent Scheduling C-RNTI, the UE behaviour is undefined

R1-1809357	Clarification on validation of UL SPS with repetition for one UL SPS configuration	Huawei, HiSilicon, LG Electronics, Nokia, Nokia Shanghai Bell

#### Other

R1-1808912	Further discussion on CFI configuration for LTE HRLLC	Guangdong OPPO Mobile Telecom

Agreement:

For TDD, semi-static CFI pattern per subframe in the radioframe is adopted in addition to existing signalling of CFI per MBSFN and non-MBSFN subframes. Only one CFI signaling shall be configured (either per MBSF/non-MBSFN subframe, or, per subframe in the radio frame). The corresponding RRC parameter names are cfi-Radioframe-patternSubframe and cfi-Radioframe-patternSlot.

Agreement:

When the UE is configured with RRC parameters and CFI pattern in the radio frame for subframe PDSCH and CFI pattern in the radio frame for slot PDSCH, the two configured values shall be the same.

R1-1809583	TP to 36.211 and 36.213 on handling of CFI pattern	Oppo

The text proposals in R1-1809583 are endorsed.

R1-1808913	Ls on RRC parameter list for HRLLC	Guangdong OPPO Mobile Telecom

R1-1808914	List of physical layer related RRC parameters for LTE HRLLC	Guangdong OPPO Mobile Telecom

R1-1809356	Clarification on subframe-PDSCH repetition on MBSFN or non-MBSFN subframes	Huawei, HiSilicon, LG Electronics

Agreement:

MBSFN subframes are treated similarly as UL subframes for TDD i.e. the subframe based. PDSCH repetition resumes after MBSFN subframe(s)

R1-1809582	Clarification on subframe-PDSCH repetition on MBSFN or non-MBSFN subframes	Huawei, HiSilicon

The text proposals in R1-1809582 are endorsed.

R1-1809358	Clarification on UE procedure for receiving PDSCH when configured with repetition	Huawei, HiSilicon

R1-1809359	Clarification on specification impact of no more than one DCI validation for UL SPS	Huawei, HiSilicon

R1-1809360	Clarification on redundancy version determination for UL SPS with repetition	Huawei, HiSilicon,

LG Electronics

The text proposal in R1-1809360 is endorsed.

R1-1809361	Clarification on resource mapping for HARQ-less PDSCH repetition	Huawei, HiSilicon

R1-1809362	Clarification on resource mapping for UL SPS with repetition	Huawei, HiSilicon

### LTE Release 15 UE features

Friday

R1-1809670	Chairman's notes of AI 6.1.7 LTE Release 15 UE feature	Ad-hoc chair (NTT DOCOMO)

Decision: The document is endorsed, content is incorporated below.

R1-1809017	Views on Rel-15 UE features	Qualcomm Incorporated

R1-1809127	LTE UE features	NTT DOCOMO, INC.

Late submission

R1-1809243	UE feature list for LTE release 15	Ericsson

Late submission

R1-1809368	Views on LTE Rel-15 UE features	Huawei, HiSilicon

R1-1809619	Rel-15 LTE UE feature list	NTT DOCOMO, INC.

R1-1809616	Draft LS on Rel-15 LTE UE feature list 	NTTDOCOMO, INC

Final LS is agreed in R1-1809617.

### Other

Including any potential CRs for Rel-15 Enhancements to LTE operation in unlicensed spectrum, Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE, Further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE, Enhanced LTE Support for Aerial Vehicles, Enhancing CA Utilization, modulation enhancements, 2Tx/4Tx SRS switching, CP latency reduction, etc

Friday

R1-1809671	Chairman's notes of AI 6.1.8 Others	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content is incorporated below.

FeLAA

R1-1808587	Discussion on the partial PUSCH mode field for FeLAA in 36.212	Huawei, HiSilicon

R1-1808139	Correction on the partial PUSCH mode field for FeLAA in 36.212	Huawei, HiSilicon

Final CR agreed in R1-1809576 (Rel-15, CR0300, TS 36.212)

R1-1808140	Correction on starting position of Partial PUSCH Mode 1 for FeLAA in 37.213	Huawei, HiSilicon

R1-1809623	Correction on starting position of Partial PUSCH Mode 1 for FeLAA in 37.213	Huawei, HiSilicon, Ericsson

Final CR agreed in R1-1809647 (TS37.213, Rel-15, CR0002)

R1-1808464	Discussion on AUL-initiated COT sharing for LAA	LG Electronics

R1-1809626	Correction on COT length for AUL transmission	LG Electronics

Final CR agreed in R1-1809648 (TS37.213, Rel-15, CR0003)

R1-1808585	Discussion on the AUL-UCI payload for FeLAA in 36.212	Huawei, HiSilicon

R1-1808595	Correction on the AUL-UCI payload for FeLAA in 36.212	Huawei, HiSilicon

R1-1808589	Correction on the interpretation of HARQ-ACK bitmap for FeLAA in 36.212	Huawei, HiSilicon

R1-1809577	Correction on the interpretation of HARQ-ACK bitmap for FeLAA in 36.212	Huawei, HiSilicon

R1-1808590	Correction on DL CP extention for FeLAA in 36.211	Huawei, HiSilicon

R1-1809602	Correction on DL CP extension for FeLAA in 36.211	Huawei, HiSilicon

Final CR agreed in R1-1809650 (TS36.211, Rel-15, CR0456)

R1-1808591	Correction on RRC parameters for FeLAA in 37.213	Huawei, HiSilicon

R1-1809281	Correction to indication of UE-to-eNodeB COT sharing	Nokia, Nokia Shanghai Bell, Ericsson

R1-1809622	Correction on RRC parameters for FeLAA in 37.213	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson

R1-1809280	Correction to determination of ending symbol for AUL transmissions	Nokia, Nokia Shanghai Bell, Ericsson

R1-1809620	Correction to determination of ending symbol for AUL transmissions	Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon

Final CR agreed in R1-1809651 (TS36.211, Rel-15, CR0457)

R1-1809369	Correction to AUL-UCI resource allocation in 36.213	Nokia, Nokia Shanghai Bell

R1-1809621	Correction to AUL-UCI resource allocation in 36.213	Nokia, Nokia Shanghai Bell

Final CR agreed in R1-1809652 (TS36.213, Rel-15, CR1155)

FeCoMP

R1-1808141	Correction on CRI definition in 36.213	Huawei, HiSilicon

R1-1808586	Discussion on aperiodic CSI triggering for feCoMP	Huawei, HiSilicon

R1-1808592	Correction on aperiodic CSI triggering for feCoMP in 36.213	Huawei, HiSilicon

Final CR agreed in R1-1809586 (Rel-15, TS36.213, CR1144)

Modulation enhancements

R1-1808654	Correction to alternative 6-bit MCS	Intel Corporation

R1-1809021	Remaining details for modulation enhancements	Qualcomm Incorporated

Agreement:

UE is not expected to be scheduled with a modulation order higher than the configured CQI table

The text proposal in Section 2 of R1-1809021 is endorsed.

Final CR capturing the agreement and the TP to be submitted R1-1809587 (TS36.213, Rel-15, CR1145)

Final CR agreed in R1-1809608 (revision of R1-1809587 to capture 1024 QAM table) (TS36.213, Rel-15, CR1145rev1)

1024 QAM

R1-1809018	Correction of RRC parameter name for 1024QAM rate matching	Qualcomm Incorporated

Final CR agreed in R1-1809588 (TS36.212, Rel-15, CR0301)

R1-1809019	Applicability of 1024QAM to SPS C-RNTI	Qualcomm Incorporated

Final CR agreed in R1-1809589 (TS36.213, Rel-15, CR1146)

R1-1809020	Removal of R_CSI form 1024QAM CSI table	Qualcomm Incorporated

Final CR agreed in R1-1809590 (TS36.213, Rel-15, CR1147)

R1-1809298	Motivation for Correction to CSI reporting for 1024QAM (36.213)	Nokia, Nokia Shanghai Bell

R1-1809299	Correction to CSI reporting for 1024QAM	Nokia, Nokia Shanghai Bell

R1-1809604	[Draft] LS on Restriction of 1024QAM CSI reporting	Nokia

Decision: The draft LS is endorsed and final LS is approved in R1-1809609

Others

R1-1808584	Discussion on PRG size for TM9 and TM10	Huawei, HiSilicon

R1-1809472	On configurable wideband PRG size for TM9 and TM10	Ericsson

R1-1808588	Introduction of configured PRG size for TM9 and TM10 in 36.213	Huawei, HiSilicon

R1-1809249	Introduction of NR-SS-SINR	Ericsson

## LTE Release 16

### Additional MTC Enhancements

LTE_eMTC5-Core; WID in RP-181450. Please refer to the WID for detailed scoping

Friday

R1-1809672	Chairman's notes of AI 6.2.1 Additional MTC Enhancements	Ad-hoc chair (Samsung)

Decision: The document is endorsed, content is incorporated below.

R1-1808033	WI work plan for Rel-16 LTE-MTC	Ericsson

#### UE-group wake-up signal

R1-1809521	Summary of 6.2.1.1 UE group MWUS	Qualcomm

Agreement

UE-group WUSs are only multiplexed in the same NB as associated PO

- FFS TDM/FDM/CDM for UE-group MWUS multiplexing

Agreement

- UE-group MWUS is supported based on eNB’s and UE’s capability.

- Whether the network supports UE-group MWUS is done by higher layer signalling.

- FFS: The number of UE groups is configured by SIB.

- Note that the UE-group MWUS is UE optional

Agreement

Rel-16 UE-group MWUS sequence should consider at least

- Fallback to legacy UE behavior

- Inter-cell interference randomization

- UE group ID for different UE-group MWUS

- Reuse of Rel-15 sequences is not precluded

- Effect of sequence detection on UE complexity

Agreement

Study the RAN1 consequence of UE-grouping on the following basis:

- UE ID

- Coverage

- DRX/eDRX

- Gap configuration

- Services

R1-1808034	UE-group wake-up signal in LTE-MTC	Ericsson

R1-1808121	Support of sub-groups for MWUS	Huawei, HiSilicon

R1-1808215	On UE-group wake-up signal for eMTC	vivo

R1-1808347	UE-group wake-up signal for MTC	Sony

R1-1808430	UE-group wake-up signal for MTC	Nokia, Nokia Shanghai Bell

R1-1808465	Discussion on UE-grouping wake up signal in MTC	LG Electronics

R1-1808631	Discussion on Wake-up signal for MTC	ZTE

R1-1808655	UE-group wake-up signal for eMTC	Intel Corporation

R1-1808730	UE-group wake-up signal for eMTC	Samsung

R1-1809022	UE-group wake-up signal	Qualcomm Incorporated

R1-1809128	UE-group wake-up signal for Rel.16 IoT	NTT DOCOMO, INC.

#### Support for transmission in preconfigured UL resources

R1-1809528	LTE-M Preconfigured UL Resources Summary	Sierra Wireless

Agreement

Idle mode based pre-configured UL resources is supported for UEs in possession of a valid TA

- FFS: Validation mechanism for TA

- FFS: How the pre-configured UL resources is acquired

Agreement

For transmission in preconfigured UL resources, the UE may use the latest TA of which its validity can be confirmed

Agreement

Study both shared and dedicated resource for preconfigured UL resources. If both shared and dedicated resources are supported, strive for commonality in design of both resource types.

Agreement

HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered:

- Whether to support HARQ;

  - If supported, details of HARQ design including the number of HARQ processes;

- Whether ACK/NACK is necessary

Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures.

R1-1808035	Support for transmission in preconfigured UL resources in LTE-MTC	Ericsson

R1-1808118	UL transmission in preconfigured resource	Huawei, HiSilicon

R1-1808348	Transmission in preconfigured UL resources	Sony

R1-1808357	Pre-configured UL Resources Design Considerations	Sierra Wireless, S.A.

R1-1808431	Transmission in preconfigured UL resources	Nokia, Nokia Shanghai Bell

R1-1808466	Discussion on preconfigured UL resources in MTC	LG Electronics

R1-1808632	Support for transmission in preconfigured UL resources for MTC	ZTE

R1-1808656	UL transmission in preconfigured resources for eMTC	Intel Corporation

R1-1808731	Discussion on transmission in preconfigured UL reosurces for MTC	Samsung

R1-1809023	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

R1-1809129	UL transmission scheme in preconfigured resources	NTT DOCOMO, INC.

#### Scheduling of multiple DL/UL transport blocks

R1-1809562	Feature lead summary Scheduling of multiple DL UL transport blocks	Ericsson

Agreement

Specify scheduling of multiple transport blocks for both CE Mode A and B

Agreement

The possibility of scheduling multiple DL/UL transport blocks is configured via RRC. Details TBD

Agreement

When scheduling of multiple TBs is enabled, the number of scheduled transport blocks (>= 1) should be dynamically selected via DCI. The maximum number of scheduled transport blocks with one single DCI is [TBD].

- The number of blind decodes for MPDCCH is not increased as a result of scheduling multiple TBs

Conclusion

When multiple TBs are scheduled by one DCI, study interleaving amongst TBs from different HARQ process in cases of repetitions

- Companies are encouraged to submit evaluation results in the next RAN1 meeting

Agreement

One DCI to schedule multiple TBs for SC-MCCH is not supported

R1-1809615	Updated Feature lead summary Scheduling of multiple DL UL transport

blocks (Revision of R1-1809562)	Ericsson

Working assumption

For unicast, when multiple DL/UL transport blocks are assigned by a single DCI, each transport block corresponds to a unique HARQ process.

R1-1808036	Scheduling of multiple DL/UL transport blocks in LTE-MTC	Ericsson

R1-1808117	Scheduling of multiple transport blocks	Huawei, HiSilicon

R1-1808349	Support of scheduling of multiple DL / UL transport blocks for MTC	Sony

R1-1808355	Multiple TB Grant Design for Unicast	Sierra Wireless, S.A.

R1-1808432	Scheduling of multiple DL/UL transport blocks	Nokia, Nokia Shanghai Bell

R1-1808467	Discussion on multiple transmission blocks scheduling in MTC	LG Electronics

R1-1808557	Design of scheduling of multiple DL/UL transport blocks for Rel.16 MTC	Lenovo, Motorola Mobility

R1-1808633	Consideration on scheduling enhancement for MTC	ZTE

R1-1808732	Discussion on scheduling of multiple TBs for MTC	Samsung

R1-1809024	Scheduling of multiple DL/UL transport blocks	Qualcomm Incorporated

#### Coexistence of LTE-MTC with NR

R1-1809564	Feature lead summary for Coexistence of LTE-MTC with NR	Ericsson

Observation: From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and eMTC

Agreement

- RAN1 studies additional specification enhancement for improving the performance of coexistence of eMTC with NR.

R1-1808037	Coexistence of LTE-MTC with NR	Ericsson

R1-1808122	On eMTC co-existence with NR	Huawei, HiSilicon

R1-1808350	Coexistence of eMTC with NR	Sony

R1-1808433	Coexistence of eMTC with NR	Nokia, Nokia Shanghai Bell

R1-1808468	Discussion on coexistence of LTE-MTC with NR	LG Electronics

R1-1808634	Coexistence of LTE-MTC with NR	ZTE

R1-1809025	Coexistence of LTE-MTC with NR	Qualcomm Incorporated

#### Support of Quality report in Msg3

R1-1809527	Feature summary of 6.2.1.5 Support of quality report in Msg3	Samsung

Agreement

Prioritize the following alternatives for DL quality report in Msg3 in MTC, for CE Mode A and CE Mode B separately:

- CQI (for CE Mode A)

- Repetition number related to UE decoding of actual or hypothetical MPDCCH/PDSCH

  - FFS if aggregation level needs to be reported when repetition number equal to 1

- RSRP/RSRQ

Companies are encouraged to provide details and/or performance evaluation results

Agreement

Whether the DL quality report is included in Msg3 is indicated in SIB and/or RAR.

Above applies in case the UE supports DL quality report in Msg3.

R1-1808038	Support of quality report in Msg3 in LTE-MTC	Ericsson

R1-1808119	Channel quality reporting in Msg3	Huawei, HiSilicon

R1-1808351	On providing quality report in Message 3	Sony

R1-1808434	Support of Quality report in Msg3	Nokia, Nokia Shanghai Bell

R1-1808469	Downlink channel quality report during random access procedure	LG Electronics

R1-1808635	Specify Msg3 quality reporting at least for EDT	ZTE

R1-1808733	Discussion on quality report in Msg3 for MTC	Samsung

R1-1809026	Support of Quality report in msg3	Qualcomm Incorporated

R1-1809469	DL quality reporting in Msg3 for eMTC	Intel Corporation

#### MPDCCH performance improvement

R1-1809570	Draft summary of MPDCCH performance improvement	Huawei, HiSi

Agreement

CRS for improving channel estimation on USS MPDCCH is supported

Agreement

The relation between the MPDCCH DMRS and CRS is defined for enabling improved channel estimation. FFS the details on specifying the relation (e.g., whether the ‘definition’ means predefined in specification or done by signalling, relation between the CRS ports and DMRS ports, power offset between DMRS-CRS).

Agreement

eNB enables/disables “CRS for improving channel estimation on MPDCCH” via dedicated or broadcasted RRC signaling. FFS the details on the signaling.

Agreement

CRS for improving channel estimation on Type0-MPDCCH CSS MPDCCH is supported.

R1-1808039	MPDCCH performance improvement in LTE-MTC	Ericsson

R1-1808120	On MPDCCH performance improvement	Huawei, HiSilicon

R1-1808352	On MPDCCH performance improvement	Sony

R1-1808435	MPDCCH performance improvement	Nokia, Nokia Shanghai Bell

R1-1808470	Discussion on MPDCCH performance improvement	LG Electronics

R1-1808636	Consideration on MPDCCH improvement by using CRS	ZTE

R1-1808657	MPDCCH performance improvement	Intel Corporation

R1-1808734	Discussion on MPDCCH performance improvement using CRS for MTC	Samsung

R1-1809027	Usage of CRS for MPDCCH	Qualcomm Incorporated

#### CE mode A and B improvements for non-BL UEs

R1-1809534	Summary of CE mode A and B improvements for non-BL UEs	LG Electronics

Agreement

Study on the performance benefit of dual layer DL reception when it is supported for non-BL UEs in CE mode A. Companies are encouraged to submit evaluation results.

- UE complexity should be also considered

- Prioritize SNR region relevant to CE mode A

Agreement

Study on the performance benefit of CSI-RS based CSI feedback when it is supported for non-BL UEs in CE mode A.

- UE complexity should be also considered

- Prioritize SNR region relevant to CE mode A

R1-1808040	CE mode A and B improvements for LTE-MTC non-BL UEs	Ericsson

R1-1808115	Coverage enhancement for Non-BL UE	Huawei, HiSilicon

R1-1808436	CE mode A and B improvements for non-BL UEs	Nokia, Nokia Shanghai Bell

R1-1808471	CE mode improvements for non-BL Ues	LG Electronics

R1-1808637	Discussion on CE mode A and B improvements for non-BL UEs	ZTE

R1-1808658	Improvement for non-BL UEs in CE mode	Intel Corporation

R1-1808735	Discussion on CE Mode A and Mode B improvements for non-BL UEs	Samsung

R1-1809028	CE Mode A and B improvements for non-BL Ues	Qualcomm Incorporated

#### Use of LTE Control Channel Region for DL Transmission

R1-1809530	Use of LTE Control Channel Region for DL Transmission Summary	Nokia, Nokia Shanghai Bell

Agreement

FFS whether and for what broadcast transmission (e.g. SIBs, paging, SC-PTM, Msg2, etc) use of LTE control channel region for MPDCCH and PDSCH are applied

Agreement

The following options can be considered for transmission of MPDCCH and PDSCH in LTE control channel region:

For MPDCCH

- Option 1: All or part of the MPDCCH are mapped into the DL control region.

- Option 2: MPDCCH are rate-matched to all OFDM symbols. FFS on RE mapping details.

For PDSCH

- Option A: All or part of the PDSCH are mapped into the DL control region.

- Option B: PDSCH are rate-matched to all OFDM symbols. FFS on RE mapping details.

R1-1808041	Use of LTE control channel region for DL transmission in LTE-MTC	Ericsson

R1-1808116	DL transmission on LTE control channel region	Huawei, HiSilicon

R1-1808353	DL transmission in the LTE control channel region	Sony

R1-1808437	Consideration on the usage of LTE control channel region	Nokia, Nokia Shanghai Bell

R1-1808472	On the use of LTE control channel region for LTE-MTC DL transmission	LG Electronics

R1-1808558	Design of Use of LTE Control Channel Region for DL Transmission	Lenovo, Motorola Mobility

R1-1808638	Discussion on use of LTE control channel region for DL transmission	ZTE

R1-1808659	Standalone deployment for eMTC	Intel Corporation

R1-1808736	Discussion on use of control channel region	Samsung

R1-1809029	Use of control region for eMTC Ues	Qualcomm Incorporated

R1-1809130	DL transmission in LTE control channel region	NTT DOCOMO, INC.

R1-1809173	Initial views on the use of LTE control channel region for DL transmission	Xiaomi Communications

#### Use of RSS for measurement improvements

R1-1809503	Summary on the use of RSS for measurement improvements	Sony

Agreement

From RAN1 perspective, it is feasible to use RSS for measuring RSRP for cells at least for IDLE mode mobility

- RAN1 to identify the related parameters in the next RAN1 meeting

R1-1808042	Use of RSS for measurement improvements in LTE-MTC	Ericsson

R1-1808354	On Measurement performance improvements for MTC	Sony

R1-1808438	Use of RSS for measurement improvements	Nokia, Nokia Shanghai Bell

R1-1808473	Discussion on the use of RSS for measurement improvement	LG Electronics

R1-1808560	Considerations on improving DL measurement accuracy	Huawei, HiSilicon

R1-1808639	Discussion on use of RSS for measurement improvement	ZTE

R1-1809030	Use of RSS for RRM measurements	Qualcomm Incorporated

### Additional Enhancements for NB-IoT

NB_IOTenh3-Core; WID in RP-181451. Please refer to the WID for detailed scoping

Friday

R1-1809673	Chairman's notes of AI 6.2.2 Additional Enhancements for NB-IoT	Ad-hoc chair (Samsung)

Decision: The document is endorsed, content is incorporated below.

R1-1808106	Additional enhancements for NB-IoT workplan	Huawei, HiSilicon

#### UE-group wake-up signal

R1-1809532	Feature summary of 6.2.2.1 UE-group wake-up signal	Ericsson

Agreement:

UE-group WUSs are only multiplexed in the same carrier as associated PO

- FFS TDM/CDM for UE-group NWUS multiplexing

Agreement

Rel-16 UE-group NWUS sequence should consider at least

- Fallback to legacy UE behavior

- Inter-cell interference randomization

- UE group ID for different UE-group NWUS

- Reuse of Rel-15 sequences is not precluded

- Effect of sequence detection on UE complexity

Agreement

Study the RAN1 consequence of UE-grouping on the following basis:

- UE ID

- Coverage

- DRX/eDRX

- Gap configuration

- Services

R1-1808043	UE-group wake-up signal in NB-IoT	Ericsson

R1-1808107	On support for UE-group wake-up signal	Huawei, HiSilicon

R1-1808216	On UE-group wake-up signal for NB-IoT	vivo

R1-1808439	UE group wake-up signal for NB-IoT	Nokia, Nokia Shanghai Bell

R1-1808474	Discussion on UE-grouping wake up signal in NB-IoT	LG Electronics

R1-1808640	Discussion on Wake-up signal for NB-IoT	ZTE

R1-1808660	UE-group wake-up signal for NB-IoT	Intel Corporation

R1-1808737	UE-group wake-up signal for eMTC	Samsung

R1-1808959	UE-Group WUS in NB-IoT	MediaTek Inc.

R1-1809031	UE-group wake-up signal	Qualcomm Incorporated

R1-1809131	UE-group wake-up signal for Rel.16 IoT	NTT DOCOMO, INC.

R1-1809483	UE-group wake-up signal for NB-IoT	Sony

#### Support for transmission in preconfigured UL resources

R1-1809571	Feature lead summary of Support for transmission in preconfigured UL resources	Huawei, HiSi

Agreement

Idle mode based pre-configured UL resources is supported for UEs in possession of a valid TA

- FFS: Validation mechanism for TA

- FFS: How the pre-configured UL resources is acquired

Agreement

For transmission in preconfigured UL resources, the UE may use the latest TA of which its validity can be confirmed

Agreement

Study both shared and dedicated resource for preconfigured UL resources. If both shared and dedicated resources are supported, strive for commonality in design of both resource types.

Agreement

HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered:

- Whether to support HARQ;

  - If supported, details of HARQ design including the number of HARQ processes;

- Whether ACK/NACK is necessary

Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures.

R1-1808044	Support for transmission in preconfigured UL resources in NB-IoT	Ericsson

R1-1808108	On support for transmission in preconfigured UL resources	Huawei, HiSilicon

R1-1808358	Pre-configured UL Resources Design Considerations	Sierra Wireless, S.A.

R1-1808440	Preconfigured Grant for Uplink transmission	Nokia, Nokia Shanghai Bell

R1-1808475	Discussion on preconfigured UL resources in NB-IoT	LG Electronics

R1-1808641	Support for transmission in preconfigured UL resources for NB-IoT	ZTE

R1-1808661	UL transmission in preconfigured resources for NB-IoT	Intel Corporation

R1-1808738	Discussion on transmission in preconfigured UL reosurces for NB-IOT	Samsung

R1-1808960	Early Data Transmission on Preconfigured UL resources in NB-IoT	MediaTek Inc.

R1-1809032	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

R1-1809132	UL transmission scheme in preconfigured resources	NTT DOCOMO, INC.

#### Scheduling of multiple DL/UL transport blocks

R1-1809523	Feature lead summary of scheduling enhancement for NB-IoT	ZTE

Agreement

For unicast, scheduling multiple DL/UL transport blocks with single DCI is supported.

Agreement

- One DCI to schedule multiple TBs for SC-MCCH is not supported

Agreement

For Unicast, the possibility of scheduling multiple DL/UL transport blocks is configured via RRC. Details TBD

Agreement

For unicast, the number of TBs scheduled should be dynamically indicated in the DCI, the maximum number of TBs is FFS

R1-1808045	Scheduling of multiple DL/UL transport blocks in NB-IoT	Ericsson

R1-1808109	Discussion on scheduling multiple DL/UL transport blocks for SC-PTM and unicast	Huawei, HiSilicon

R1-1808356	Multiple TB Grant Design for Unicast	Sierra Wireless, S.A.

R1-1808441	Scheduling of multiple DL/UL transport blocks	Nokia, Nokia Shanghai Bell

R1-1808476	Discussion on multiple transmission blocks scheduling in NB-IoT	LG Electronics

R1-1808559	Design of scheduling of multiple DL/UL transport blocks for Rel.16 NBIoT	Lenovo, Motorola Mobility

R1-1808642	Consideration on scheduling enhancement for NB-IoT	ZTE

R1-1808739	Discission on scheduling of multiple TBs for NB-IoT	Samsung

R1-1809033	Scheduling of multiple DL/UL transport blocks	Qualcomm Incorporated

#### Coexistence of NB-IoT with NR

R1-1809572	Feature lead summary of Coexistence of NB-IoT with NR	Huawei, HiSi

Observation

From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and NB-IoT

Note that similar observation will be made for eMTC

Agreement

RAN1 studies additional specification enhancement for improving the performance of coexistence of NB-IoT with NR.

Note that similar agreement will be made for eMTC

R1-1808046	Coexistence of NB-IoT with NR	Ericsson

R1-1808112	Discussion on NB-IoT coexistence with NR	Huawei, HiSilicon

R1-1808442	Coexistence of NB-IoT with NR	Nokia, Nokia Shanghai Bell

R1-1808477	Discussion on coexistence of NB-IoT with NR	LG Electronics

R1-1808643	Discussion on coexistence of NB-IoT with NR	ZTE

R1-1808958	Coexistence of NB-IoT with NR	MediaTek Inc.

R1-1809034	Coexistence of NB-IoT with NR	Qualcomm Incorporated

#### Support of Quality report in Msg3 for non-anchor access

R1-1809600	Feature lead summary on support of quality report in Msg3 for non-anchor access	Huawei, HiSi

Agreement

For channel quality report in Msg3 on non-anchor access, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1%

- FFS: Whether the details on the hypothetical NPDCCH are specified or not

Working Assumption

For channel quality report in Msg3 on non-anchor access, UE performs the channel quality measurement on the carrier it monitors to receive Msg2 (i.e. RAR)

- FFS: Whether the UE performs measurement on other carriers

Agreement

For non-anchor access, RAN1 further studies how UEs report the measured channel quality

R1-1808047	Support of quality report in Msg3 for non-anchor access in NB-IoT	Ericsson

R1-1808110	On support of Msg3 quality reporting for non-anchor access	Huawei, HiSilicon

R1-1808443	Support of Quality report in Msg3 for non-anchor access	Nokia, Nokia Shanghai Bell

R1-1808478	Downlink channel quality report during random access procedure on a non-anchor carrier	LG Electronics

R1-1808644	Support of Quality report in Msg3 for non-anchor access	ZTE

R1-1808740	Discussion on quality report in Msg3 for NB-IoT	Samsung

R1-1808961	Quality report in Msg3 on non-anchor carrier	MediaTek Inc.

R1-1808962	NRS support on non-anchor carrier	MediaTek Inc.

R1-1809035	Support of quality report in msg3 for non-anchor	Qualcomm Incorporated

R1-1809471	DL quality reporting in Msg3 for non-anchor access in NB-IoT	Intel Corporation

#### Presence of NRS on a non-anchor carrier for paging

R1-1809522	Feature lead summary of presence of NRS on a non-anchor carrier for paging	Qualcomm

Agreement (confirmed by Friday)

The presence of NRS on subframes which will contain NRS even when no paging NPDCCH is transmitted is enabled by broadcast signalling.

- Above applies only for non-anchor carriers

Agreement (confirmed by Friday)

For the objective on presence of NRS on a non-anchor carrier for paging, consider the following use case:

- Early termination of NPDCCH / WUS

Above is not intended to restrict the study of other use cases in other WGs which may or may not have RAN1 specification impact

R1-1808048	Presence of NRS on a non-anchor carrier for paging in NB-IoT	Ericsson

R1-1808111	NRS presence on non-anchor carriers for paging	Huawei, HiSilicon

R1-1808444	Presence of NRS on a non-anchor carrier for paging	Nokia, Nokia Shanghai Bell

R1-1808479	Discussion on NRS on an a non-anchor carrier	LG Electronics

R1-1808645	NRS on non-anchor PRB	ZTE

R1-1808741	Discussion on NRS presence on non-anchor carrier	Samsung

R1-1809036	Presence of NRS on a non-anchor carrier for paging	Qualcomm Incorporated

#### Other

R1-1808921	Early UL Transmission for NB-IoT and HD-FDD eMTC	Sequans Communications

### DL MIMO efficiency enhancements for LTE

LTE_DL_MIMO_EE-Core; WID in RP-181485. Please refer to the WID for detailed scoping

Friday

R1-1809674	Chairman's notes of AI 6.2.3 DL MIMO efficiency enhancements for LTE	Ad-hoc chair (Samsung)

Decision: The document is endorsed, content is incorporated below.

R1-1808138	Work plan of DL MIMO efficiency enhancements for LTE	Huawei, HiSilicon

#### Support of enhancing SRS capacity and coverage

R1-1808368	Discussion on SRS capacity and coverage enhancement	CATT

##### Additional SRS symbols

R1-1809584	Feature summary on additional SRS symbols for LTE	Huawei, HiSi

Agreement

The work for additional SRS symbols in this WI should consider the following scenarios

- TDD for non-CA

- TDD only CA

- FDD-TDD CA

RAN1 will strive for a common framework for all scenarios

R1-1809659	Outcome of offline discussion for additional SRS symbols	Huawei, HiSi

Agreement

The time location of possible additional SRS symbols in one normal UL subframe for a cell include the following

- Option 1: All symbols in one slot can be used for SRS from cell perspective

  - E.g., The other slot in the subframe may be used for PUSCH transmission for sTTI-capable UE.

- Option 2: All symbols in one subframe can be used for SRS from cell perspective

- Option 3: A subset of symbols in one slot can be used for SRS from cell perspective

  - E.g., The other slot in the subframe may be used for PUSCH transmission for sTTI-capable UE.

- Other options are not precluded

- FFS whether it has spec impacts or not for cell-specific configuration

- Companies are encouraged to study and evaluate considering

  - Performance (eg, DL performance improvement, degradation on UL performance including legacy UL for legacy UEs)

  - Specification impact

  - UE complexity

  - RF impact

- RAN1 will finalize on the performance metrics and evaluation methodology in RAN1#94bis

- System level parameters, evaluation methodology, and the need for system level evaluation can be discussed in the next RAN1 meeting

Conclusion

For link level evaluation, companies are encouraged to submit evaluations results based on following assumptions as a starting point, which is to be checked in next RAN1 meeting.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 20 MHz ( or 10MHz) |
| Duplex | TDD, LTE Configuration 2 |
| Subcarrier spacing | 15 kHz |
| Polarization | eNB: X pole (+/-45° ) UE: + pol (0/90°) |
| Macro eNB antenna array | (M,N,P) = (2,4,2) with vertical spacing 0.8λ and horizontal spacing 0.5λ |
| Macro eNB antenna element model (only applicable to CDL and other details FFS) | Elevation beamwidth = 65º |
|  | Azimuth beamwidth = 65º |
| UE Speed | 3km/h, 30km/h, 60km/h |
| UE antenna | 2 receive antennas Isotropic (0 dBi) |
| CP | Normal CP |

R1-1808136	Discussion on SRS transmission on more than one symbols of normal UL subframe	Huawei, HiSilicon

R1-1808192	Discussion on additional SRS symbols	ZTE

R1-1808227	Discussion on SRS enhancements	vivo

R1-1808369	Consideration on multiple SRS symbols in a subframe	CATT

R1-1809574	Views on additional symbols for SRS	Mitsubishi Electric Co.	(revision of R1-1808445)

R1-1808480	Enhancements on SRS resource allocation	LG Electronics

R1-1808547	Additional SRS symbols	Lenovo, Motorola Mobility

R1-1808662	Additional SRS symbols	Intel Corporation

R1-1808742	Discussions on additional SRS symbols	Samsung

R1-1809037	Additional SRS symbols	Qualcomm Incorporated

R1-1809191	On multi-symbol SRS enhancements	Ericsson

R1-1809215	A potential scenario for additional SRS symbols	SoftBank Corp.

R1-1809376	Discussion on introduction of additional SRS symbols	Nokia, Nokia Shanghai Bell

##### Virtual cell ID for SRS

R1-1809585	Feature summary on virtual cell ID for SRS	Huawei, HiSilicon

Agreement

For SRS, nIDRS=nIDSRSif nIDSRSis configured by higher layer, otherwise nIDRS=NIDcell, where nIDRS is within the range from 0 to 503.

R1-1808137	On virtual cell ID for SRS sequence generation	Huawei, HiSilicon

R1-1808193	Discussion on virtual cell ID for SRS	ZTE

R1-1808446	Views on SRS virtual cell ID	Mitsubishi Electric Co.

R1-1808481	Discussion on VCID for SRS sequence	LG Electronics

R1-1808548	Virtual cell ID for SRS	Lenovo, Motorola Mobility

R1-1808663	Virtual cell ID support for SRS	Intel Corporation

R1-1808743	Discussions on virtual cell ID for SRS	Samsung

R1-1808957	Virtual Cell ID for SRS Capacity Enhancements	MediaTek Inc.

R1-1809038	Virtual cell ID for SRS	Qualcomm Incorporated

R1-1809192	On virtual cell ID for SRS	Ericsson

R1-1809377	Introduction of virtual cell id for SRS	Nokia, Nokia Shanghai Bell

##### Other

R1-1809193	Simulation assumptions and evaluation methodology for SRS enhancements	Ericsson

#### Other

R1-1809194	Possible enhancements to Advanced CSI reporting	Ericsson

### Other

R1-1809229	Virtual cell ID for SRS	Qualcomm Incorporated

Withdrawn

# NR

## Maintenance of Release 15 NR

WID in RP-181474

Only essential corrections. Notes:

- No individual CRs are allowed. Only text proposals are allowed.

  - Later on editors will aggregate all endorsed text proposals for a jumbo CR per spec

- Editors are encouraged to submit editor’s CR before the meeting reflecting additional comments received after the endorsed & frozen NR specifications. If submitted, please submit under agenda 7.

R1-1808744	Updates to TS 38.213	Samsung

R1-1808329	TP to 38.211 on handling of initial BWP configuration (response to R2-1810943)	Ericsson

R1-1808071	Corrections for TR38.811 Chapter 6 Non-Terrestrial Networks channel models	Huawei, HiSilicon

Monday guidance

Conclusion:

- Editors to check the specifictions w.r.t. whether there are cases when the corresponding RRC parameter name is missing. If so, to make necessary updates to include the RRC parameter name in the first occurrence in the specs.

- Editors to update the definition of symbols (Section 3.2) and abbreviations as necessary in the corresponding specs

- Notes: companies are encouraged to inform the editors regarding potential updates for the above two bullets

Wednesday

Conclusion:

- Companies are encouraged to check editor errors within and across different specs, and notify the editors of misalignments to be captured in editors’ CR update

  - One example, in 38.211, the following update is necessary

Table 6.3.2.4.1-2: Orthogonal sequences  for PUCCH format 1.

Friday

Editors to prepare CRs incorporating the TPs/agreements/working assumptions by August 30th , for review/comment and endorsement by Sept. 4th.

### Maintenance for Initial access and mobility

#### Maintenance for Downlink signals and channels

Including sync, PBCH, RMSI, OSI, and Paging.

R1-1809703	Summary of synchronization signals and PBCH	Ericsson

Decision: The document is noted.

Issue of k_SSB: discuss offline

Conclusion:

- Editor to clarify that in the first occurrence of u_0 the largest subcarrier spacing configuration among all subcarrier spacing configurations provided in the cell-specific higher layer parameter scs-SpecificCarrierList

Agreement:

- Agree to the TP to section 7.4.3.1 of 38.211

===== start of TP =====

For an SS/PBCH block, the UE shall assume

-	antenna port  is used for transmission of PSS, SSS, PBCH and DM-RS for PBCH,

-	the same cyclic prefix length and subcarrier spacing for the PSS, SSS, PBCH and DM-RS for PBCH,

===== end of TP =====

Issue regarding SSB Position Indication in ServingcellConfigCommon

Proposal:

From the above cases, if the subcarrier spacing of SS/PBCH blocks is not provided with the higher layer parameter subcarrierSpacing, the applicable ones for a cell depend on a respective frequency band, as provided in [8-1, TS 38.101-1] and [8-2, TS 38.101-2]. Case B shall be applied when a 30kHz subcarrier spacing for the SS/PBCH block is indicated from higher layer for frequency bands with a default 15kHz subcarrier spacing provided in [8-1, TS 38.101-1]. A same case applies for all SS/PBCH blocks on the cell.

Also discuss some conditions for intra-band case

Wednesday

R1-1809808	Summary of synchronization signals and PBCH	Ericsson

Decision: The document is noted.

Proposal from offline:

- k_SSB shall be restricted to:

  - for {SSB SCS, RMSI SCS} = {30 kHz, 15 kHz}, {30 kHz, 30 kHz}, {120 kHz, 60 kHz}, or {240 kHz, 120 kHz}, k_SSB should be restricted as an integer multiple of 2;

  - for {SSB SCS, RMSI SCS} = {240 kHz, 60 kHz}, k_SSB should be restricted as an integer multiple of 4.

- Add a table to section 7.4.3.1 of 38.211 with the above restrictions in k_SSB

- Send LS to RAN4 informing them of this decision (Draft in R1-1809809)

Concerns of LGE, Huawei

RAN4 may have already define such restriction

Discuss further offline

Proposals for the LS from RAN2(R1-1808860):

For the first action:

- The functionality of the 8+8 group bitmap format for SSB position indicated in SIB1 was intended to provide cell specific indication of SSB transmission that should be used during initial access procedures and possibly measurement purposes. The original intention of the full bitmap format for SSB position indicated in dedicated RRC to CONNECTED mode UEs are for PDSCH rate-matching purposes only, and the additional flexibility is to provide flexible resource utilization of resources occupied by SSB for PDSCH transmission.

For the second action:

- Although originally it’s RAN1’s understanding that the target cell can provide an updated full bitmap for rate matching purposes, if needed, for enhanced flexibility, it however creates some complexity in ensuring consistency in the signaled SSB position between the two methods (8+8 vs. 64-bit bitmap), especially in light of the necessary PRACH procedure. As a result, RAN1 agrees that the two signaling methods shall indicate the same SSB position in Rel-15.

Agreements:

- At least for FR2, if the subcarrier spacing of SS/PBCH blocks is provided with the higher layer parameter subcarrierSpacing, a UE is not expected to be configured with different subcarrier spacings of SS/PBCH blocks for cells in intra-band contiguous carrier aggregation.

  - Check further offline regarding FR1

- To adopt the following TP to section 4.1 of 38.213:

===== start of TP =====

From the above cases, if the subcarrier spacing of SS/PBCH blocks is not provided by the higher layer parameter subcarrierSpacing, the applicable ones for a cell depend on a respective frequency band, as provided in [8-1, TS 38.101-1] and [8-2, TS 38.101-2]. A same case applies for all SS/PBCH blocks on the cell.  If 30kHz SS/PBCH block subcarrier spacing is indicated by higher layer parameter subcarrierSpacing, Case B shall be applied for frequency bands with only 15kHz SS/PBCH block subcarrier spacing as specified in [8-1, TS 38.101-1], and the case specified for 30kHz SS/PBCH block subcarrier spacing in [8-1, TS 38.101-1] shall be applied for bands with 30 kHz SS/PBCH block subcarrier spacing or both 15kHz and 30 kHz SS/PBCH block subcarrier spacing as specified in [8-1, TS 38.101-1].

===== end of TP =====

Thursday

R1-1809879	Summary of synchronization signals and PBCH	Ericsson

Decision: The document is noted.

Conclusion:

- It is RAN1’s understanding that subcarrier alignment can be achieved by setting point A correctly

- UE’s are not expected to handle cases where point A has been set so that subcarriers are not aligned

- As an example, by configuring k_SSB to the values below, alignment can be achieved.

  - for {SSB SCS, RMSI SCS} = {30 kHz, 15 kHz}, {30 kHz, 30 kHz}, {120 kHz, 60 kHz}, or {240 kHz, 120 kHz}, k_SSB should be restricted as an integer multiple of 2;

  - for {SSB SCS, RMSI SCS} = {240 kHz, 60 kHz}, k_SSB should be restricted as an integer multiple of 4

- No RAN1 specification impact.

Agreements:

- For the LS from RAN2(R1-1808860):

  - For the first action:

    - The functionality of the 8+8 group bitmap format for SSB position indicated in SIB1 was intended to provide cell specific indication of SSB transmission that should be used for SSB-RO association and rate matching purpose for both IDLE, INACTIVE, and CONNECTED UEs. The original intention of the full bitmap format for SSB position indicated in dedicated RRC to CONNECTED mode UEs are for UE-specific PDSCH rate-matching purposes only, and the additional flexibility is to provide flexible resource utilization of resources occupied by SSB for PDSCH transmission.

  - For the second action:

    - Based on the above, from RAN1’s perspective, the solution would have been to have the 8+8 bitmap in ServingCellConfigCommonSIB and ServingCellConfigCommon and the full 64-bit bitmap in ServingCellConfig, but RAN1 realizes that this is a too late change for Rel-15. Although originally it was RAN1’s intention that the target cell can provide an updated full bitmap for rate matching purposes only, if needed, the current signaling creates some complexity in ensuring consistency in the signaled SSB position between the two methods (8+8 vs. 64-bit bitmap), especially related to the RACH procedure. As a result, RAN1 agrees that the two signaling methods shall indicate the same SSB position in Rel-15.

  - No RAN1 specification impact from the above conclusion.

- Reply LS to RAN2 in R1-1809880.

Agreement: The TP in section 7.3 of R1-1809879 is endorsed (38.213)

Friday

R1-1809880	Draft reply LS on ssb-PositionsInBurst mismatch in SIB1 and ServingcellConfigCommon	Ericsson

Decision: The draft LS is endorsed and final LS is approved in R1-1809958.

R1-1809709	Summary of Remaining RMSI Issues	CATT

Decision: The document is noted.

Conclusion:

- Editor to correct the format issue with respect to the caption of table 13.2 of 38.213.

Wednesday

R1-1809794	Summary of Remaining RMSI Issues	CATT

Decision: The document is noted.

Agreements:

- It is RAN1 understanding that UE is not required to decode SI messages until SIB1 has been successfully decoded.

  - No RAN1 spec impact

- From RAN1 perspective, it is not required to avoid supporting transmission of SIB1 and SI message in the same SI transmission window

  - Use one reserved bit in DCI with CRC scrambled by SI-RNTI to indicate UE whether SIB1 or SI message is scheduled

Friday

R1-1809854	Summary of Remaining RMSI Issues	CATT

Decision: The document is noted.

Agreement: The TP in Section 2 of R1-1809854 for Section 7.4.3.1 of 38.211 is endorsed.

R1-1809714	Summary of Other System Information Delivery	Samsung

Decision: The document is noted.

Issue regarding whether or not to avoid supporting transmission of SIB1 and SI message in the same SI transmission window

Wednesday

R1-1809782	Summary of Other System Information Delivery	Samsung

Decision: The document is noted.

Agreements:

To adopt the following TP to 7.3.1.2.1 of 38.212:

===== start of TP =====

- Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

- System information indicator – 1 bit as defined in Table 7.3.1.2.1-2.

- Reserved bits – [1615] bits

/// texts omitted

Table 7.3.1.2.1-2: System information indicator

| Bit field | System information to be present |
| --- | --- |
| 0 | SIB1 |
| 1 | SI message |

===== end of TP =====

R1-1809783	[draft] LS response on overlapping SIB1 and SI message	Samsung

Decision: The draft LS is endorsed and final LS is approved in R1-1809850.

Agreements:

- If a UE is not provided higher layer parameter searchSpaceOtherSystemInformation for Type0A-PDCCH common search space set, the UE determines the Type0A-PDCCH common search space set as the same as Type0-PDCCH common search space set (i.e., default Type0A-PDCCH common search space set).

- If a UE is not provided higher layer parameter pagingSearchSpace for Type2-PDCCH common search space set, the UE determines the Type2-PDCCH common search space set as the same as Type0-PDCCH common search space set (i.e., default Type2-PDCCH common search space set).

- Separate default Type0A-PDCCH common search space set is not needed.

- Separate default Type2-PDCCH common search space set is not needed.

  - Note: the above 2 sub-bullets have no spec impact

R1-1809737	Summary on Paging	Huawei

Decision: The document is noted.

Agreements:

- Remove the square bracket from ‘[8]’ from TS 38.212, section 7.3.1.2.1

  - It implies that there are 6 spare bits dedicated for future use

  - Note: when scheduling information is not present, all other information fields (other than the short message indicator) are reserved which can be potentially for future use

- Send reply RAN2 of RAN1 decision above (R1-1809760)

Wednesday

R1-1809760	Reply LS on short message transmission using DCI	RAN1, Huawei

Decision: The document is approved.

Friday

R1-1809925	Summary on Paging	Huawei

Agreements:

Regarding the issues related to RAN2 LS in R1-1808172:

- It is RAN1 understanding that the Paging Frame (PF) is a starting point for the first PO (defined by ‘S’ valid PDCCH monitoring occasions). In addition, the concatenated POs corresponding to one PF can overlap into the next radio frame(s) and/or next PF(s). Based on this understanding, RAN1 has the following questions:

  - Is the first PDCCH monitoring occasion of the first PO of a PF starting from the starting location of the pagingSearchSpace in that frame?

  - RAN1 would like to understand the motivation of introducing PF_offset? Is it intended to avoid overlapping such as to avoid the collision of valid PDCCH monitoring occasion and e.g. SSB transmission.

- Within the UE PO for the non-default association, RAN1 is considering the valid PDCCH monitoring occasion associated to which SSB index to limit the UE monitoring time.

- RAN1 is also discussing if paging PDSCH resource allocation impacts the validity of paging PDCCH monitoring occasion.

Draft LS in R1-1809959

R1-1809959	[draft] Reply LS on Further Agreements on Paging Occasions	Huawei

Decision: The draft LS is endorsed (updating “RAN2 answers” to “RAN2 to answer”) and final LS is approved in R1-1809965.

R1-1809721	Maintenance for Downlink signals and channels	ZTE	(Revision of R1-1808163)

R1-1808217	Remaining issues on broadcast signals and channels	vivo

R1-1809722	Remaining issues on downlink signals and channels for initial access	MediaTek Inc.	(Revision of R1-1808268)

R1-1808370	Remaining issues of broadcasting signals and channels	CATT

R1-1808482	Maintenance for Downlink signals and channels	LG Electronics

R1-1808622	Remaining issue on synchronization signal	NEC Corporation

R1-1808664	Corrections to Initial Access DL Channels and Signals	Intel Corporation

R1-1808745	Remaining Issues on Synchronization and System information	Samsung

R1-1808890	Maintenance for Downlink signals and channels	Guangdong OPPO Mobile Telecom

R1-1808922	Remaining issues on downlink signals and channels	Huawei, HiSilicon

R1-1809105	Remaining issues on OFDM baseband signal generation	Sharp

R1-1809134	Maintenance for NR initial access	NTT DOCOMO, INC.

R1-1809245	Discussion on remaining issues of SSB and RMSI	CMCC

R1-1809700	Maintenance for Downlink signals and channels	Ericsson	(Revision of R1-1809256)

R1-1809370	On issues related to DL signals and channels	Nokia, Nokia Shanghai Bell

R1-1809417	Maintenance for downlink signals and channels for initial access	Qualcomm Incorporated

#### Maintenance for Physical random access channel and procedure

R1-1809736	Maintenance for physical random access channel	Convida

Decision: The document is noted.

Proposals:

For unpaired spectrum, a PRACH occasion in a PRACH slot is invalid if it precedes a SS/PBCH block in the PRACH slot or starts less than symbols after a last SS/PBCH block transmission symbol, where  is provided in Table 8.2-2.

If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is also provided higher layer parameter tdd-UL-DL-ConfigurationCommon2, a PRACH occasion in a PRACH slot is also invalid if

-	it is not within UL symbols, orand

-	it does not precede a SS/PBCH block in the PRACH slot and starts at leastless than  symbols after a last downlink symbol and at least symbols after a last SS/PBCH block transmission symbol, where  is provided in Table 8.2-2.

- To discuss further offline the issue of “Only the valid PRACH occasions are used for performing association between SSB and PRACH occasions”.

Agreements:

- To adopt the followingTP in 38.211:

Table 6.3.3.2-3: Random access configurations for FR1 and unpaired spectrum.

| 95 | A2 | 16 2 | 1 | 2,3,4,7,8,9 | 0 | 1 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum.

| … |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 134 | B4 | 1 | 0 | 3,5,7,9,11,13 | 02 | 1 | 1 | 12 |

Wednesday

R1-1809785	Maintenance for physical random access channel	Convida

Decision: The document is noted.

Agreements:

- To adopt the following TP of 5.3.2 of 38.211:

===== start of TP =====

===== end of TP =====

Agreements:

- To adopt the following TP to section 8.1 of 38.213:

===== start of TP =====

For paired spectrum all PRACH occasions are valid.

For unpaired spectrum, if a UE is not provided higher layer parameter tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least symbols after a last SS/PBCH block transmission symbol, where  is provided in Table 8.2-2.

If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is also provided higher layer parameter tdd-UL-DL-ConfigurationCommon2, a PRACH occasion in a PRACH slot is valid if

===== end of TP =====

Friday

R1-1809919	Maintenance for physical random access channel	Convida

Decision: The document is noted. Companies are encouraged to check section 3.1 of R1-1809919 – aim to conclude in RAN1#94bis

R1-1809756	Summary of maintenance for PRACH procedures	Qualcomm

Decision: The document is noted.

Agreements:

- To adopt the following TP to 38.213, Section 8.1:

===== start of TP =====

A UE is provided a number  of SS/PBCH blocks associated with one PRACH occasion and a number  of contention based preambles per SS/PBCH block per valid PRACH occasion by higher layer parameter ssb-perRACH-OccasionAndCB-PreamblesPerSSB. If , one SS/PBCH block is mapped to  consecutive valid PRACH occasions. If ,  contention based preambles with consecutive indexes associated with SS/PBCH block , , per valid PRACH occasion start from preamble index n.64/N n totalNumberOfRA-Preambles/N, where totalNumberOfRA-Preambles is an integer multiple of N. SS/PBCH block indexes are mapped to valid PRACH occasions in the following order where the parameters are described in [4, TS 38.211].

===== end of TP =====

Agreements:

- To adopt the following TP to 38.213, Section 10.1:

===== start of TP =====

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE is not expected to monitor a PDCCH for Type0/0A/2/3-PDCCH common search space, if the SS/PBCH block or the CSI-RS the UE selects for PRACH association, as described in Subclause 8.1, does not have same QCL-TypeD [6, TS 38.214] with a DM-RS for monitoring the PDCCH for Type0/0A/2/3-PDCCH common search space, and if the reception of the PDCCH or PDSCH scrambled by the RNTIs associated for Type0/0A/2/3-PDCCH common search space overlapped with PDCCH or PDSCH scrambled by the RNTIs associated with Type1-PDCCH common search space in at least one symbol.

===== end of TP =====

Proposals:

- To adopt the following TP to Section 8.2 of 38.213:

If a UE detects a DCI format 1_0 with the CRC scrambled by the corresponding RA-RNTI and receives the corresponding PDSCH that includes the DL-SCH transport block, the UE may assume that the PDSCH and the PDCCH scheduling the PDSCH have same DM-RS antenna port quasi co-location properties, as described in [6, 38.214], as for a SS/PBCH block or a CSI-RS resource the UE used for PRACH association as described in Subclause 8.1, regardless of whether or not the higher layer parameter TCI-StatesPDCCH is provided for the CORESET where the PDCCH is received. If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers non-contention based random access procedure, the UE may assume that the PDCCH and the PDCCH order have same DM-RS antenna port quasi co-location properties.

Proposals:

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with a CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest control resource set the UE is configured for Type1-PDCCH common search space, as defined in Subclause 10.1, that is zero at least  symbols after the last symbol of the RACH occasion corresponding to the preamble sequence  PRACH transmission, where  is defined in [10, TS 38.133] and  is the subcarrier spacing configuration for Type1-PDCCH common search space. The length of the window in number of slots, based on the subcarrier spacing for Type1-PDCCH common search space as defined in Subclause 10.1, is provided by higher layer parameter ra-ResponseWindow.

Wednesday

R1-1809845	Summary of maintenance for PRACH procedures	Qualcomm

Decision: The document is noted.

Agreements:

- Adopt the following TP to Section 8.2 of 38.213 (change shown in red color)

===== start of TP =====

If a UE detects a DCI format 1_0 with the CRC scrambled by the corresponding RA-RNTI and receives the corresponding PDSCH that includes the DL-SCH transport block, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, 38.214], as for a SS/PBCH block or a CSI-RS resource the UE used for PRACH association as described in Subclause 8.1, regardless of whether or not the higher layer parameter TCI-StatesPDCCH is provided for the CORESET where the PDCCH is received. If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers non-contention based random access procedure, the UE may assume that the PDCCH and the PDCCH order have same DM-RS antenna port quasi co-location properties.

===== end of TP =====

Agreement:

- The 1st valid RACH occasion in one association period is used as the starting RACH occasion for ordering in PDCCH order case.

Agreements:

The indexing group of PRACH Mask index should be consecutive PRACH occasions per SSB, i.e., SS/PBCH blocks to PRACH occasions mapping cycle per SSB.

- The indexing of PRACH Mask index should be reset after consecutive PRACH occasions per SSB, i.e., SS/PBCH blocks to PRACH occasions mapping cycle per SSB.

- UE selects the next available PRACH occasion after UE can prepare Msg1 transmission.

Agreements:

- Repetition across multiple slots is not supported for message 3.

- To adopt the following TP to 8.3 of 38.213:

===== start of TP =====

A UE transmits an UL-SCH in an Msg3 PUSCH scheduled by a RAR grant in a corresponding RAR message using redundancy version number 0. Retransmissions, if any, of the UL-SCH in an Msg3 PUSCH are scheduled by a DCI format 0_0 with CRC scrambled by a TC-RNTI provided in the corresponding RAR message [11, TS 38.321].

When the UE is configured with aggregationFactorUL, aggregationFactorUL=1 is assumed for Msg3 PUSCH transmission, regardless of which value of aggregationFactorUL is configured for PUSCH for the active BWP.

===== end of TP =====

Proposal:

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with a CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest control resource set the UE is configured for Type1-PDCCH common search space, as defined in Subclause 10.1, that is one at least  symbols, where the symbol duration is based on the subcarrier spacing for Type1-PDCCH common search space as defined in Subclause 10.1, after the last symbol of the RACH occasion corresponding to the preamble sequence  PRACH transmission, where  is defined in [10, TS 38.133] and  is the subcarrier spacing configuration for Type1-PDCCH common search space. The length of the window in number of slots, based on the subcarrier spacing for Type1-PDCCH common search space as defined in Subclause 10.1, is provided by higher layer parameter ra-ResponseWindow.

Friday

R1-1809903	Summary of maintenance for PRACH procedures	Qualcomm

Decision: The document is noted.

Agreements:

- To adopt the following TP to section 8, 38.213:

===== start of TP =====

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with a CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest control resource set the UE is configured for Type1-PDCCH common search space, as defined in Subclause 10.1, that is at least one at least  symbols, where the symbol duration is based on the subcarrier spacing for Type1-PDCCH common search space as defined in Subclause 10.1, after the last symbol of the RACH occasion corresponding to the preamble sequence  PRACH transmission, where  is defined in [10, TS 38.133] and  is the subcarrier spacing configuration for Type1-PDCCH common search space. The length of the window in number of slots, based on the subcarrier spacing for Type1-PDCCH common search space as defined in Subclause 10.1, is provided by higher layer parameter ra-ResponseWindow.

===== end of TP =====

R1-1808164	Maintenance for Physical random access channel and procedure	ZTE

R1-1809512	Remaining issues in PRACH design and RACH procedure	MediaTek Inc.	(Revision of R1-1808267)

R1-1808310	Corrections on RACH	Fujitsu

R1-1808371	Remaining issues of physical random access channel and procedure	CATT

R1-1808483	Maintenance for Physical random access channel and procedure	LG Electronics

R1-1808542	Remaining issues on RACH procedure	vivo

R1-1808665	Corrections to Random Access Channel and Procedures	Intel Corporation

R1-1808746	Remaining Issues on Random Access	Samsung

R1-1808804	Remaining issues on Physical random access channel and procedure	Spreadtrum Communications

R1-1808829	Discussion on ssb-PositionsInBurst mismatch in SIB1 and ServingcellConfigCommon	CMCC

R1-1808891	Maintenance for Physical random access channel and procedure	Guangdong OPPO Mobile Telecom

R1-1808923	Remaining issues on physical random access channel and procedure	Huawei, HiSilicon

R1-1808965	Remaining details on NR Random Access	Nokia, Nokia Shanghai Bell

R1-1809135	Maintenance for NR random access	NTT DOCOMO, INC.

R1-1809257	Maintenance for Physical random access channel and procedure	Ericsson

R1-1809725	Maintenance for Physical random access channel and procedure	Qualcomm Incorporated	(Revision of R1-1809418)

R1-1809474	Correction on RACH procedure	ITL

#### Maintenance for Mobility procedure

Including RRM and RLM

R1-1809732	Summary of discussion for NR measurements for mobility	Intel

Decision: The document is noted.

Agreements:

- Adopt the following text proposals

----------------------------------Text Proposal for Section 7.1.4.5 in TS38.211------------------------------------

----omitted---- Zero-power (ZP) and non-zero-power (NZP) CSI-RS are defined - for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE or CSI-RS-Resource-Mobility in the CSI-RS-ResourceConfigMobility IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3 ----omitted---- ….. at the start of each OFDM symbol where ns,fμ is the slot number within a radio frame, is the OFDM symbol number within a slot, and equals the higher-layer parameter scramblingID or sequenceGenerationConfig. ----omitted---- The value of is given by the higher-layer parameter density in the CSI-RS-ResourceMapping IE or the CSI-RS-ResourceConfigMobility IE and the number of ports X is given by the higher-layer parameter nrofPorts. ----omitted---- The time-domain locations l0∈2, 3, …, 12 and l1∈2, 3, …, 12 are provided by the higher-layer parameters firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain2, respectively, in the CSI-RS-ResourceMapping IE or the CSI-RS-ResourceConfigMobility IE and defined relative to the start of a slot. The frequency-domain location is given by a bitmap provided by the higher-layer parameter frequencyDomainAllocation in the CSI-RS-ResourceMapping IE or the CSI-RS-ResourceConfigMobility IE with the bitmap and value of in Table 7.4.1.5.3-1 given by - , for row 1 of Table 7.4.1.5.3-1 - , for row 2 of Table 7.4.1.5.3-1 - , for row 4 of Table 7.4.1.5.3-1 - , for all other cases where is the bit number of the bit in the bitmap set to one, repeated across every of the resource blocks configured for CSI-RS reception by the UE when . The starting position and number of the resource blocks in which the UE shall assume that CSI-RS is transmitted are given by the higher-layer parameters freqBand and density in the CSI-RS-ResourceMapping IE for the bandwidth part given by the higher-layer parameter bwp-Id in the CSI-ResourceConfig IE, or given by the higer-layer parameters csi-rs-MeasurementBW and density in the CSI-RS-ResourceConfigMobility IE where the the startPRB given by csi-rs-MeasurementBW is relative to common resource block 0. ----omitted---- where the periodicity (in slots) and slot offset are obtained from the higher layer parameter CSI-ResourcePeriodicityAndOffset or slotconfig. The UE shall assume that CSI-RS is transmitted in a candidate slot only if all OFDM symbols of that slot corresponding to the configured CSI-RS resource are classified as 'downlink'.

Agreements:

- Adopt the following text proposal to 38.215:

Table 5.1.3-1: NR Carrier RSSI measurement symbols

| OFDM signal indication endSymbol SS-RSSI-MeasurementSymbolConfig | Symbol indexes |
| --- | --- |
| 0 | {0,1} |
| 1 | {0,1,2,..,10,11} |
| 2 | {0,1,2,…, 5} |
| 3 | {0,1,2,…, 7} |

Agreements:

- To adopt the following TP to TS 38.213 Section 5

===== start of TP =====

If the UE is not provided higher layer parameter RadioLinkMonitoringRS and the UE is provided by higher layer parameter TCI-state for PDCCH one or more RSs that include one or more of a CSI-RS and/or a SS/PBCH block

-	the UE uses for radio link monitoring the RS provided for the active TCI state for PDCCH if the active TCI state for PDCCH includes only one RS

-	if the active TCI state for PDCCH includes two RS, the UE expects that one RS has QCL-TypeD and the UE uses the one RS for radio link monitoring; the UE does not expect both RS to have QCL-TypeD

-	the UE is not required to use for radio link monitoring an aperiodic or semi-persistent RS

===== end of TP =====

Wednesday

R1-1809793	Summary of offline discussion for NR mobility	Intel

Decision: The document is noted.

Proposal:

- When the configured C-DRX cycle is 80msec or larger, UE does not expect CSI-RS resource outside the C-DRX ON period are available for RRM measurements. Otherwise, UE may assume CSI-RS resources are always available.

Agreements:

- Adopt the following text proposal for TS38.214, 5.1.6.1.3:

===== start of TP =====

For frequency range 2 the associatedSSB is either present for all configured CSI-RS resources or not present for any configured CSI-RS resources per frequency layer.

For any CSI-RS resource configuration, UE shall assume that the value for parameter cdm-Type is 'No CDM', and there is only one anntenna port.

===== end of TP =====

Agreements:

- Adopt the following text proposal for 38.215 Section 5.1.3

===== start of TP =====

| Definition | <omitted> If indicated by higher-layers, for a half-frame with SS/PBCH blocks the NR Carrier RSSI is measured in slots within a half frame with SS/PBCH blocks that are indicated by the higher layer parameter measurementSlots and in from OFDM symbols of the indicated slots and the OFDM symbols are given by Table 5.1.3-1. - For intra-frequency measurements, NR Carrier RSSI is measured with timing reference corresponding to the serving cell in the frequency layer - For inter-frequency measurements, NR Carrier RSSI is measured with timing reference corresponding to any cell in the target frequency layer Otherwise, if measurement gap is not used, NR Carrier RSSI is measured from OFDM symbols within SMTC window duration and, if measurement gap is used, NR Carrier RSSI is measured from OFDM symbols corresponding to overlapped time span between SMTC window duration and minimum measurement time within the measurement gap. <omitted> |
| --- | --- |

===== end of TP =====

Agreements:

- Send an LS to RAN2, informing them that density parameter in CSI-RS-CellMobility is required for measurement of CSI-RS. RAN1 recommends that RAN2 make appropriate actions to make sure the value of density is available in all cases. If a default value is to be determined by RAN2, RAN1 recommends use of d3 as the default value.

- LS to be drafted in R1-1809851

Friday - R1-1809851	[DRAFT] LS on density parameter for CSI-RS-ResourceConfigMobility	Intel

Decision: The draft LS is endorsed (assuming RAN4 being deleted from ACTION) and final LS is approved in R1-1809960.

Agreements:

- Send an LS to RAN4 providing motivation for the second BLER pair thresholds for RLM.

  - Draft LS in R1-1809813

- The following is the motivation of the second BLER pair threshold:

  - It has been observed that in LTE, when semi-persistent scheduling is used, the service quality can be adequate also when the SINR is below the level corresponding to a PDCCH BLER of 10%. The most prominent example is for VoLTE.

  - In these cases, it has been observed that the UE triggers RLF also when the voice quality is quite adequate. In other words, RLF may be triggered prematurely.

  - To avoid this situation, RAN1 introduced the additional BLER threshold values for RLM. The main idea was thus that the additional BLER values would be higher than the default values.

R1-1809813	Draft LS on second BLER pair for RLM	Ericsson

Decision: The draft LS is endorsed and final LS is approved in R1-1809853.

Friday session

R1-1809933	Summary of offline discussion for NR mobility	Intel

Decision: The document is noted.

Agreements:

- Adopt the following text proposal for TS38.214 section 5.1.6.1.3

===== start of TP =====

If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the UE may base the timing of the CSI-RS resource on the timing of the cell given by the cellId of the CSI-RS resource configuration. Additionally, for a given CSI-RS resource, if the associated SS/PBCH block is configured but not detected by the UE, the UE is not required to monitor the corresponding CSI-RS resource. The higher layer parameter isQuasiColocated indicates whether the associated SS/PBCH block given by the associatedSSB and the CSI-RS resource(s) are quasi co-located with respect to ['QCL-TypeD'].

If the UE is configured with DRX, UE is not required to perform measurement of CSI-RS resources other than during the active time for measurements based on CSI-RS-Resource-Mobility.

If the UE is configured with DRX and DRX cycle in use is larger than 80 msec, UE may not expect CSI-RS resources are available other than during the active time for measurements based on CSI-RS-Resource-Mobility. Otherwise, UE may assume CSI-RS are available for measurements based on CSI-RS-Resource-Mobility.

===== end of TP =====

Working asusmption:

- In frequency range 1 unpaired spectrum, UE is not required to perform intra-frequency neighbor cell RRM measurement over SSB or CSI-RS for mobility when UE detects a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 triggering the UE to transmit in UL in at least one of the symbols where the SSB or CSI-RS for RRM measurement on neighbor cell is transmitted.

  - Note: this is not intended to have any impact on existing overlapping/overwriting rules related to SFI

Draft LS in R1-1809963, to RAN4 regarding the above and provide feedback if there is any concern.

R1-1809963	[DRAFT] LS on collision of RRM measurement resources with uplink transmissions in FR1 TDD	vivo

Decision: The draft LS is endorsed with the following update:

Send an LS to RAN4 regarding the above and provide feedback if there is any concern

Final LS is approved in R1-1810008.

Proposal:

- To adopt the following TP to section 5 of 38.213:

The downlink radio link quality of the primary cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. If UE is configured for operation in initial DL BWP and UE is configured to perform monitoring of radio link quality of SS/PBCH block(s) when SS/PBCH block and control resource set multiplexing pattern 2 or 3 is configured in PBCH, UE is expected to perform RLM by monitoring the associated SS/PBCH block. Otherwise, theThe UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the primary cell.

Proposal:

- Adopt the following text proposal of Section 5, 38.213

For a CSI-RS resource configuration, the higher layer parameter powerControlOffsetSS is not applicable and a UE expects to be provided only 'No CDM' from higher layer parameter cdm-Type, only '1' and '3' from higher layer parameter density, and only '1 port' from higher layer parameter nrofPorts [6, TS 38.214].

If a UE is configured with multiple DL BWPs for the serving cell and DL BWP is switched from one to another as defined in Subclause 12, UE updates the RS for radio link monitoring from RadioLinkMonitoringRS or TCI-state for PDCCH associated with the previous DL BWP to RadioLinkMonitoringRS or TCI-state for PDCCH associated with the current DL BWP.

R1-1809985	Summary of Office discussion for NR mobility	Intel

Decision: The document is noted.

Agreements:

- Adopt the following text proposal, section 5 of 38.213

===== start of TP =====

The downlink radio link quality of the primary cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the primary cell. When SS/PBCH block and control resource set multiplexing pattern 2 or 3 is configured in PBCH and initial DL BWP is active, UE is expected to perform RLM by monitoring the associated SS/PBCH block.

===== end of TP =====

Agreements:

- Adopt the following text proposal, section 5 of 38.213

===== start of TP =====

For a CSI-RS resource configuration, the higher layer parameter powerControlOffsetSS is not applicable and a UE expects to be provided only 'No CDM' from higher layer parameter cdm-Type, only '1' and '3' from higher layer parameter density, and only '1 port' from higher layer parameter nrofPorts [6, TS 38.214].

If a UE is configured with multiple DL BWPs for the serving cell and a new DL BWP is activated, UE uses the RS(s) configured by RadioLinkMonitoringRS for the activated DL BWP or RS(s) provided for the active TCI-state for PDCCH if RadioLinkMonitoringRS is not configured in the activated DL BWP.

===== end of TP =====

R1-1808165	Maintenance for Mobility procedure	ZTE

R1-1808218	Remaining issues on mobility procedure	vivo

R1-1808269	Remaining issues in RRM measurement procedure	MediaTek Inc.

R1-1808372	Remaining issues of Mobility procedure	CATT

R1-1808484	Maintenance for Mobility procedure	LG Electronics

R1-1808666	Corrections to NR mobility measurements	Intel Corporation

R1-1808747	Remaining Issues on Measurements for Mobility Management	Samsung

R1-1808892	Remaining issues on NR RRM and RLM	Guangdong OPPO Mobile Telecom

R1-1808924	Remaining issues on mobility procedure	Huawei, HiSilicon

R1-1809136	Maintenance for NR mobility procedure	NTT DOCOMO, INC.

R1-1809258	Maintenance for Mobility procedure	Ericsson

R1-1809371	Issues related to RRM and RLM	Nokia, Nokia Shanghai Bell

R1-1809419	Maintenance for mobility procedure	Qualcomm Incorporated

#### Other

R1-1808925	Correction on TA offset determination in TS38.213	Huawei, HiSilicon

R1-1808929	On RMSI PRB grid	Huawei, HiSilicon

R1-1809420	Maintenance for TA	Qualcomm Incorporated

### Maintenance for MIMO

Friday

R1-1809913	Chairman's notes of AI 7.1.2 Maintenance for MIMO	Ad-hoc chair (Samsung)

Decision: The document is endorsed, content is incorporated below.

#### Maintenance for Multi-antenna scheme

Including codeword mapping, codebook & non-codebook based tx for UL, PRB bundling for DL, etc.

Please use separate sections for each of the respective sub-topic to facilitate reviews and summary.

R1-1809493	Summary of codeword mapping	Samsung

R1-1809742	Issue Summary on Codebook based UL transmission	Intel

R1-1809895	Summary for further discussion on codeword mapping	Samsung

Agreement

Agree on the following text proposal for 5.1.3.2 of TS38.214

----------------------------------------------------Start of TP ----------------------------------------------------------------

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then a transport block one of the two transport blocks is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block, otherwise the transport block is enabled. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

----------------------------------------------------End of TP ----------------------------------------------------------------

Conclusion

Note that when two codeword transmission is configured, ‘IMCS = 26 and rvid = 1’ on both transport blocks is a valid downlink assignment for DCI 1_1

R1-1809744	Summary of issues on UL non-codebook based transmission	Nokia, Nokia Shanghai Bell

Agreement:

Agree this TP to 6.1.1.2 of 38.214:

----------------------------------------------------Start of TP ----------------------------------------------------------------

For non-codebook based transmission, the UE does not expect to be configured with both spatialRelationInfo for SRS resource and associatedCSI-RS in SRS-ConfigSRS-ResourceSet for SRS resource set.

For non-codebook based transmission, the UE can be scheduled with DCI format 0_1 when at least one SRS resource is configured in SRS-ResourceSet with usage set to 'nonCodebook'.

----------------------------------------------------End of TP ----------------------------------------------------------------

Agreement

Adopt the following TP for Subclause 6.1.1.2 of TS38.214:

----------------------------------------------------Start of TP ----------------------------------------------------------------

6.1.1.2	Non-Codebook based UL transmission

For non-codebook based transmission, the UE can determine its PUSCH precoder and transmission rank based on the wideband SRI when multiple SRS resources are configured, where the SRI is given by the SRS resource indicator in DCI according to subclause 7.3.1.1.2 of [5, 38.212], or the SRI is given by srs-ResourceIndicator according to subclause 6.1.2.3. The UE shall use one or multiple SRS resources for SRS transmission, where the number of SRS resources which can be configured to the UE for simultaneously transmission in the same RBs is a UE capability. Only one SRS port for each SRS resource is configured. Only one SRS resource set can be configured with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. The maximum number of SRS resources that can be configured for non-codebook based uplink transmission is 4. The indicated SRI in slot n is associated with the most recent transmission of SRS resource(s) identified by the SRI, where the SRS transmission is prior to the PDCCH carrying the SRI before slot n.

----------------------------------------------------End of TP ----------------------------------------------------------------

Agreement

Adopt the following TP for Subclause 6.1.1.2 of TS38.214:

----------------------------------------------------Start of TP ----------------------------------------------------------------

-	If aperiodic SRS resource set is configured, the associated NZP-CSI-RS is indicated via SRS request field in DCI format 0_1 and 1_1, where AperiodicSRS-ResourceTrigger (indicating the association between aperiodic SRS triggering state and SRS resource sets), triggered SRS resource(s) srs-ResourceSetId, csi-RS (indicating the associated NZP-CSI-RS-ResourceId) are higher layer configured in SRS-ResourceSet. A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the aperiodic NZP-CSI-RS resource and the first symbol of the aperiodic SRS transmission is less than 42 OFDM symbols.

----------------------------------------------------End of TP ----------------------------------------------------------------

Agreement

Adopt the following TP for Subclause 6.1.1.2 of TS38.214:

----------------------------------------------------Start of TP ----------------------------------------------------------------

-	If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of the SRS request field is not '00' as in Table 7.3.1.1.2-24 of [5, TS 38.212]. The CSI-RS is located in the same slot as the SRS request field. If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, any of the TCI states configured in the scheduled CC shall not be configured with 'QCL-TypeD'.

----------------------------------------------------End of TP ----------------------------------------------------------------

Agreement

Agree the TP to 6.1.1.2 of 38.214

----------------------------------------------------Start of TP ----------------------------------------------------------------

6.1.1.2	Non-Codebook based UL transmission

<unrelated part omitted>

The UE shall perform one-to-one mapping from the indicated SRI(s) to the indicated DM-RS ports(s) given by DCI format 0_1 or ConfiguredGrantConfig according to subclause 6.1.2.3 in increasing order.

<unrelated part omitted>

----------------------------------------------------End of TP ----------------------------------------------------------------

Agreement

Confirm the working assumption of TP to 38.214, Section 6.1.1.2:

----------------------------------------------------Start of TP ----------------------------------------------------------------

6.1.1.2		Non-Codebook based UL transmission

For non-codebook based transmission, the UE can determine its PUSCH precoder and transmission rank based on the wideband SRI given by SRS resource indicator field from the DCI when multiple SRS resources are configured, where the SRI is given by the SRS resource indicator in DCI according to subclause 7.3.1.1.2 of [TS 38.212], or the SRI is given by srs-ResourceIndicator according to subclause 6.1.2.3.

----------------------------------------------------End of TP ----------------------------------------------------------------

R1-1809740	Summary on Issues of PRB bundling for DL	vivo

Agreement

Text proposal for 38.214 Section 3.3 and 5.1.2.3

----------------------------------------------------Start of TP ----------------------------------------------------------------

3.3	Abbreviations

< Unchanged parts are omitted >

PRG	Physical resource block group Precoding resource block group

< Unchanged parts are omitted >

5.1.2.3	Physical resource block (PRB) bundling

< Unchanged parts are omitted >

If  is determined as "wideband", the UE is not expected to be scheduled with non-contiguous resource allocation PRBs and the UE may assume that the same precoding is applied to the allocated resource.

If  is determined as one of the values among {2, 4}, Precoding Resource Block Group (PRGs) partitions the bandwidth part i with  consecutive PRBs. Actual number of consecutive PRBs in each PRG could be one or more.

The first PRG size is given by and the last PRG size given by  if , and the last PRG size is if .

The UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a PRG.

< Unchanged parts are omitted >

-	is set to '1' and two values are configured for the first set of  values as 'n2-wideband' (corresponding to two  values 2 and wideband) or 'n4-wideband' (corresponding to two  values 4 and wideband), the UE shall use the value when receiving PDSCH scheduled by the same DCI as follows:

< Unchanged parts are omitted >

----------------------------------------------------End of TP ----------------------------------------------------------------

R1-1809896	Corrections for UL MIMO for single antenna port, DCI format, and antenna	Ericsson, Nokia, LG Electronics

Agreement

Text proposal for 38.214:

----------------------------------------------------Start of TP 1a ----------------------------------------------------------------

6.1.1	Transmission schemes

Two transmission schemes are supported for PUSCH: codebook based transmission and non-codebook based transmission. The UE is configured with codebook based transmission when the higher layer parameter txConfig in PUSCH-Config  is set to 'codebook', the UE is configured non-codebook based transmission when the higher layer parameter txConfig is set to 'nonCodebook'. If the higher layer parameter txConfig is not configured, the UE is not expected to be scheduled by DCI format 0_1. the PUSCH transmission is based on one PUSCH antenna port, which is triggered by DCI format 0_0. If PUSCH is scheduled by DCI format 0_0, the PUSCH transmission is based on a single antenna port.

----------------------------------------------------End of TP 1a ----------------------------------------------------------------

----------------------------------------------------Start of TP 1b----------------------------------------------------------------

6.1.1.1 Codebook based UL transmission

For codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1 or semi-statically configured to operate according to Subclause 6.1.2.3.  If this if PUSCH is scheduled by DCI format 0_1, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank from the DCI, given by DCI fields of SRS resource indicator and Precoding information and number of layers in subclause 7.3.1.1.2 of [TS 38.212]., where the The TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} and that corresponds to the SRS resource. The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Subclause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI before slot n.

----------------------------------------------------End of TP 1b ----------------------------------------------------------------

----------------------------------------------------Start of TP 1c ----------------------------------------------------------------

6.1.1.2 Non-Codebook based UL transmission

For non-codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1 or semi-statically configured to operate according to Subclause 6.1.2.3.  the The UE can determine its PUSCH precoder and transmission rank based on the wideband SRI when multiple SRS resources are configured, where the SRI is given by the SRS resource indicator in DCI according to subclause 7.3.1.1.2 of [5, 38.212], or the SRI is given by srs-ResourceIndicator according to subclause 6.1.2.3. The UE shall use one or multiple SRS resources for SRS transmission, where the number of SRS resources which can be configured to the UE for simultaneously transmission in the same RBs is a UE capability. Only one SRS port for each SRS resource is configured. Only one SRS resource set can be configured with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. The maximum number of SRS resources that can be configured for non-codebook based uplink transmission is 4. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI before slot n.

----------------------------------------------------End of TP 1c ----------------------------------------------------------------

Agreement

Text proposal for TS38.211

----------------------------------------------------Start of TP ----------------------------------------------------------------

6.3.1.5	Precoding

The block of vectors ,  shall be precoded according to

where , . The set of antenna ports  shall be determined according to the procedure in [6, TS 38.214].

For non-codebook-based transmission, the precoding matrix  equals the identity matrix.

For codebook-based transmission, the precoding matrix  is given by  for single-layer transmission on a single antenna port, otherwise by Tables 6.3.1.5-1 to 6.3.1.5-7 with the TPMI index obtained from the DCI scheduling the uplink transmission or the higher layer parameters according to the procedure in [6, TS 38.214].

<unrelated part omitted>

----------------------------------------------------End of TP ----------------------------------------------------------------

Agreement

Text proposal for TS38.212

----------------------------------------------------Start of TP ----------------------------------------------------------------

7.3.1.1.2	Format 0_1

<unrelated part omitted>

-	Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters transformPrecoder, maxRank, and codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters transformPrecoder, maxRank, and codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters transformPrecoder, maxRank and codebookSubset;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters transformPrecoder, maxRank and codebookSubset.

<unrelated part omitted>

----------------------------------------------------End of TP ----------------------------------------------------------------

Agreement

Text proposal for TS38.214

----------------------------------------------------Start of TP ----------------------------------------------------------------

6.1.1.1	Codebook based UL transmission

<unrelated part omitted>

or codebook based transmission, the UE determines its codebook subsets based on TPMI and upon the reception of higher layer parameter codebookSubset in PUSCH-Config  which may be configured with ' fullyAndPartialAndNonCoherent ', or 'partialAndNonCoherent' , or 'nonCoherent' depending on the UE capability. The maximum transmission rank may be configured by the higher parameter maxRank in PUSCH-Config .

A UE reporting its UE capability of 'partialAndNonCoherent' transmission shall not expect to be configured by codebookSubset with ' fullyAndPartialAndNonCoherent '.

A UE reporting its UE capability of ' nonCoherent ' transmission shall not expect to be configured by codebookSubset with ' fullyAndPartialAndNonCoherent ' or with 'partialAndNonCoherent '.

A UE shall not expect to be configured with the higher layer parameter codebookSubset set to 'partialAndNonCoherent' when higher layer parameter nrofSRS-Ports in an SRS-ResourceSet with usage set to ' codebook ' indicates that two SRS antenna ports are configured.

For codebook based transmission, the UE may be configured with a single SRS-ResourceSet set to ' codebook ' and only one SRS resource can be indicated based on the SRI from within the SRS resource set. The maximum number of configured SRS resources for codebook based transmission is 2. If aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of aperiodidc SRS resources.

When multiple SRS resources are configured by SRS-ResourceSet with usage set to ' codebook ', the UE shall expect that higher layer parameters nrofSRS-Ports in SRS-ResourceSet in SRS-ResourceSet shall be configured with the same value for all these SRS resources.

----------------------------------------------------End of TP ----------------------------------------------------------------

R1-1809911	Further discussions on non-codebook based uplink transmission	Nokia, Nokia Shanghai Bell

Agreement

Agree this TP to 6.2.1 of 38.214:

----------------------------------------------------Start of TP ----------------------------------------------------------------

6.2.1	UE sounding procedure

The UE can be configured with one or more Sounding Reference Symbol (SRS) resource sets as configured by the higher layer parameter SRS-ResourceSet. For each SRS resource set, a UE may be configured with SRS resources (higher laterlayer parameter SRS-Resource), where the maximum value of K is indicated by [SRS_capability [13, 38.306]]. The SRS resource set applicability is configured by the higher layer parameter SRS-SetUseusage in SRS-ResourceSet. When the higher layer parameter usageSRS-SetUse is set to 'BeamManagement', only one SRS resource in each of multiple SRS sets can be transmitted at a given time instant. The SRS resources in different SRS resource sets can be transmitted simultaneously.

For aperiodic SRS at least one state of the DCI field is used to select at least one out of the configured SRS resource set

…

----------------------------------------------------End of TP ----------------------------------------------------------------

R1-1808146	Remaining issues on multi-antenna schemes	Huawei, HiSilicon

R1-1808194	Maintenance for Multi-antenna scheme	ZTE

R1-1808219	Remaining issues on multi-antenna scheme	vivo

R1-1808311	Corrections on Codebook based transmission	Fujitsu

R1-1808373	Issues on transmission schemes	CATT

R1-1808485	Text proposals on multi-antenna scheme	LG Electronics

R1-1808549	Maintenance for codebook and non-codebook based UL transmission	Lenovo, Motorola Mobility

R1-1808667	Remaining Issues on Multi-antenna Scheme	Intel Corporation

R1-1808799	Remaining issues on PRB bundling for DL	Spreadtrum Communications

R1-1808880	Text proposals for multi-antenna scheme	Guangdong OPPO Mobile Telecom

R1-1809052	Text Proposal on MIMO Codeword Mapping	AT&T

R1-1809195	Maintenance for Multi-antenna scheme	Ericsson

R1-1809235	Remaining issues on multi-antenna schemes	Nokia, Nokia Shanghai Bell

R1-1809421	Maintenance for multi-antenna scheme	Qualcomm Incorporated

R1-1809494	Corrections on transmission schemes	Samsung	(rev. of R1-1808748)

R1-1809852	Correction on UL antenna port mapping	Nokia, Nokia Shanghai Bell

#### Maintenance for CSI acquisition

Including CSI measurement & reporting, CQI & MCS, etc.

Please use separate sections for each of the respective sub-topic to facilitate reviews and summary.

R1-1809771	Feature lead summary on CSI measurement	ZTE

Agreement

Section 5.2.1.4.2 of TS 38.214:

----------------------------------------------------Start of TP ----------------------------------------------------------------

[Unchanged text are omitted]

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-CQI':

-	the UE is configured with higher layer parameter non-PMI-PortIndication contained in a CSI-ReportConfig, where r ports are indicated in the order of layer ordering for rank r and each CSI-RS resource in the CSI resource setting is linked to the CSI-ReportConfig based on the order of the associated NZP-CSI-RS-ResourceId in the linked CSI resource setting for channel measurement given by higher layer parameter resourcesForChannelMeasurement. The higher layer parameter non-PMI-PortIndication contains a sequence  of port indices, where  are the CSI-RS port indices associated with rank ν and  where is the number of ports in the CSI-RS resource.

-	When calculating the CQI for a rank, the UE shall use the ports indicated for that rank for the selected CSI-RS resource. The precoder for the indicated ports shall be assumed to be the identity matrix scaled by  1v.

----------------------------------------------------End of TP ----------------------------------------------------------------

Agreement

----------------------------------------------------Start of TP for 38.214 Section 5.2.1.4.2--------------------------

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-CQI':

-	If the UE is configured with higher layer parameter non-PMI-PortIndication contained in a CSI-ReportConfig, where r ports are indicated in the order of layer ordering for rank r and each CSI-RS resource in the CSI resource setting is linked to the CSI-ReportConfig based on the order of the associated NZP-CSI-RS-ResourceId in the linked CSI resource setting for channel measurement given by higher layer parameter resourcesForChannelMeasurement. The configured higher layer parameter non-PMI-PortIndication contains a sequence  of port indices, where  are the CSI-RS port indices associated with rank ν and  where is the number of ports in the CSI-RS resource.

-	If the UE is not configured with higher layer parameter non-PMI-PortIndication, the UE assumes, for each CSI-RS resource in the CSI resource setting linked to the CSI-ReportConfig, that the CSI-RS port indices  are associated with ranks  where is the number of ports in the CSI-RS resource.

----------------------------------------------------End of TP for 38.214 Section 5.2.1.4.2--------------------------

Agreement

- One DCI 0_1 trigger is not allowed to trigger reports with the same report configuration ID. Multiple CSI reports with the same ID cannot be simultaneously triggered

Up to the RAN1 editor to implement the above agreement

Agreement

----------------------------------------------------Start of TP for 38.214 -------------------------------------------------

< Start of the text proposal >

5.2.2.4 Channel State Information – Interference Measurement (CSI-IM)

< Unchanged parts are omitted>

The following parameters are configured via higher layer parameter CSI-IM-Resource for each CSI-IM resource configuration:

-	csi-IM-ResourceId determines CSI-IM resource configuration identity

-	subcarrierLocation-p0 or subcarrierLocation-p1 defines subcarrier occupancy of the CSI-IM resource within a slot for csi-IM-ResourceElementPattern set to 'pattern0' or 'pattern1', respectively.

-	symbolLocation-p0 or symbolLocation-p1 defines OFDM symbol location of the CSI-IM resource within a slot for csi-IM-ResourceElementPattern set to 'pattern0' or 'pattern1', respectively.

< End of the text proposal >

----------------------------------------------------End of TP for 38.214 --------------------------------------------------

Agreement

----------------------------------------------------Start of TP for 38.214---------------------------------------------------

5.2.1.2	Resource settings

ach CSI Resource Setting CSI-ResourceConfig contains a configuration of S≥1 CSI Resource Sets (given by higher layer parameter csi-RS-ResourceSetList), with each CSI Resource Set consisting of CSI-RS resources (comprised of either NZP CSI-RS or CSI-IM) or SS/PBCH Block resources used for L1-RSRP computation. Each CSI Resource Setting is located in the DL BWP identified by the higher layer parameter bwp-id, and all CSI Resource Settings linked to a CSI Report Setting have the same DL BWP.

-------------------   omitted  -----------------

The UE may assume that the NZP CSI-RS resource(s) for channel measurement and the CSI-IM resource(s) for interference measurement configured for one CSI reporting are resource-wise QCLed with respect to 'QCL-TypeD'.  When NZP CSI-RS resource(s) is used for interference measurement, the UE may assume that the NZP CSI-RS resource for channel measurement and the CSI-IM resource and/or NZP CSI-RS resource(s) for interference measurement configured for one CSI reporting are QCLed with respect to 'QCL-TypeD'.

----------------------------------------------------End of TP for 38.214 --------------------------------------------------

Agreement

----------------------------------------------------Start of TP for 38.214----------------------------------------------------

5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS

-------------------   omitted  -----------------

If the UE is configured with a single carrier for uplink, the UE is not expected to transmit more than one aperiodic CSI report triggered by different DCIs on overlapping OFDM symbols.

----------------------------------------------------End of TP for 38.214 --------------------------------------------------

Agreement

----------------------------------------------------Start of TP for 38.214----------------------------------------------------

5.2.1	Channel state information framework

The time and frequency resources that can be used by the UE to report CSI are controlled by the gNB. CSI may consist of Channel Quality Indicator (CQI), precoding matrix indicator (PMI), CSI-RS resource indicator (CRI), SS/PBCH Block Resource indicator (SSBRI), layer indicator (LI), rank indicator (RI) and/or L1-RSRP.

or CQI, PMI, CRI, SSBRI, LI, RI, L1-RSRP, a UE is configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings, M≥1 CSI-ResourceConfig Resource Settings, and one or two list(s) of trigger states (given by the higher layer parameters CSI-AperiodicTriggerStateList and CSI-SemiPersistentOnPUSCH-TriggerStateList). Each trigger state in CSI-AperiodicTriggerStateList contains a list of associated CSI-ReportConfigs indicating the Resource Set IDs for channel and optionally for interference. Each trigger state in CSI-SemiPersistentOnPUSCH-TriggerStateList contains one associated CSI-ReportConfig.

----------------------------------------------------End of TP for 38.214 --------------------------------------------------

R1-1809828	Summary of views on CSI reporting v4	Ericsson

Agreement

----------------------------------------------------Start of TP for 38.214 section 5.2.1.4.1----------------------------

For semi-persistent or periodic CSI, each CSI-ReportConfig is linked to periodic or semi-persistent Resource Setting(s):

-	When one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is configured, the Resource Setting is for channel measurement for L1-RSRP computation.

-	When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference) is used for interference measurement performed on CSI-IM.

A UE is not expected to be configured with more than 1 CSI-RS resource in resource set for channel measurements for a CSI-ReportConfig with the higher layer parameter codebookType set to 'typeII' or to 'typeII-PortSelection’.

If interference measurement is performed on CSI-IM, each CSI-RS resource for channel measurement is resource-wise associated with a CSI-IM resource by the ordering of the CSI-RS resource and CSI-IM resource in the corresponding resource sets. The number of CSI-RS resources for channel measurement equals to the number of CSI-IM resources.

----------------------------------------------------End of TP for 38.214 section 5.2.1.4.1----------------------------

Agreement

Capture the following text proposal to section 5.2.1.5.1 of TS 38.214.

----------------------------------------------------Start of TP ---------------------------------------------------------------

For CSI-RS resource sets associated with Resource Settings configured with the higher layer parameter resourceType set to 'aperiodic', periodic', or semi-persistent', trigger states for Reporting Setting(s) (configured with the higher layer parameter reportConfigType set to 'aperiodic') and/or Resource Setting for channel and/or interference measurement on one or more component carriers are configured using the higher layer parameter CSI-AperiodicTriggerStateList. For aperiodic CSI report triggering, a single set of CSI triggering states are higher layer configured, wherein the CSI triggering states can be associated with any candidate DL BWP. A UE is not expected to receive more than one DCI with non-zero CSI request per slot. A UE is not expected to receive more than one aperiodic CSI report request for transmission in a given slot. A UE is not expected to be triggered with a CSI report for a non-active DL BWP. A trigger state is initiated using the CSI request field in DCI.

----------------------------------------------------End of TP ---------------------------------------------------------------

Agreement

-----------------------------------------------------Start of TP for 38.214-------------------------------------------------

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS

For semi-persistent reporting on PUSCH, a set of trigger states are higher layer configured by CSI-SemiPersistentOnPUSCH-TriggerStateList, where the CSI request field in DCI scrambled with SP-CSI-RNTI activates one of the trigger states.

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType.

Semi-persistent reporting on PUCCH is activated by an activation command [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. When the HARQ-ACK corresponding to the PDSCH carrying the activation command is transmitted in slot n, the indicated semi-persistent Reporting Setting should be applied starting from slot .

For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent' .

-	when a UE receives an activation command [10, TS 38.321] for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the HARQ-ACK corresponding to the PDSCH carrying the selection command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from slot n+3Nslotsubframe,µ+1. If a TCI-State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP.

-	when a UE receives a deactivation command [10, TS 38.321] for activated CSI-RS/CSI-IM resource set(s) associated with configured CSI resource setting(s), and when  the HARQ-ACK corresponding to the PDSCH carrying the selection command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource set(s) shall apply starting from slot n+3Nslotsubframe,µ+1.

-------------------   omitted  -----------------

A UE validates, for semi-persistent CSI activation or release, a DL semi-persistent assignment PDCCH on a DCI only if the following conditions are met:

-	the CRC parity bits of the DCI format are scrambled with a SP-CSI-RNTI provided by higher layer parameter sp-csi-RNTI

-	Special fields for the DCI format are set according to Table 5.2.1.5.2-1 or Table 5.2.1.5.2-2.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of semi-persistent CSI transmission on PUSCH. If validation is not achieved, the UE considers the DCI format as having been detected with a non-matching CRC.

-------------------   omitted  -----------------

f the UE has an active semi-persistent CSI-RS/CSI-IM resource configuration, or an active semi-persistent ZP CSI-RS resource set configuration, and has not received a deactivation command, the activated semi-persistent CSI-RS/CSI-IM resource set or the activated semi-persistent ZP CSI-RS resource set configurations are considered to be active when the corresponding DL BWP is active, otherwise they are considered suspended.

If the UE is configured with carrier deactivation, the following configurations in the carrier in activated state would also be deactivated and need re-activation configuration(s): semi-persistent CSI-RS/CSI-IM resource, semi-persistent CSI reporting on PUCCH, semi-persistent SRS, semi-persistent ZP CSI-RS resource set.

-----------------------------------------------------End of TP for 38.214-------------------------------------------------

Agreement

- Refine the CPU occupation for A-CSI report based on P/SP CSI-RS as starting from the first symbol after the PDCCH triggering the report until the last symbol of the PUSCH carrying the report.

Conclusion

RAN1 editors to check and if necessary implement the following into the specification:

- Reports are assigned to CPUs on the symbol where CPU occupancy starts

Agreement

- Capture the following text proposal to section 5.2.1.6 of TS 38.214

----------------------------------------------------Start of TP ---------------------------------------------------------------

The UE indicates the number of supported simultaneous CSI calculations NCPU. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports across all configured cells. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If a UE receives an aperiodic CSI request for N CSI reports, where each CSI report n=0, …, N-1 has Ks(n)CSI-RS resources in the CSI-RS resource set for channel measurementcorresponds to  the UE is not required to update the N-M requested CSI reports with lowest priority (according to Subclause 5.2.5), where  0≤M≤N is the largest value such that n=0MKs(n)OCPU(n)≤NCPU-L holds.

The UE is not required to update a periodic or semi-persistent CSI report if NCPU-L≤Ks<OCPU on the earliest one of the first symbol of the latest CSI-RS/CSI-RS resource earlier than the corresponding CSI reference resource of the CSI report.

Processing of a CSI report occupies a number of CPUs for a number of symbols as follows:

-	OCPU=0 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with high layer parameter trs-Info configured.

-	OCPU=1 for aA CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', ‘ssb-Index-RSRP’, or 'none' (and CSI-RS-ResourceSet with high layer parameter trs-Info not configured) occupies one CPU.

-	OCPU=Ks for aA CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cri-RI-PMI-CQI’, ‘cri-RI-i1’, ‘cri-RI-i1-CQI’, ‘cri-RI-CQI’, or ‘cri-RI-LI-PMI-CQI’, 'cri-RSRP' or 'none' occupies Ks CPUs, where  the number of CSI-RS resources in the CSI-.RS resource set for channel measurement.

-	OCPU=NCPU for aA CSI report aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'TypeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', occupies NCPU CPUs.

The CPU(s) are occupied for a number of OFDM symbols as follows:

-	A periodic or semi-persistent CSI report used with periodic or semi-persistent CSI-RS occupies CPU(s) from the first symbol of the earliest one of the for each CSI-RS/CSI-IM resource for channel or interference measurement, respective latest CSI-RS/CSI-IM occasion no later than the corresponding CSI reference resource, if applicable, until the last symbol of the PUSCH/PUCCH carrying the report.

-	An aperiodic CSI report used with periodic or semi-persistent CSI-RS occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM resource for channel or interference measurement respective latest CSI-RS/CSI-IM occasions no later than the corresponding CSI reference resource until the last symbol of the PUSCH carrying the report.

--	An aperiodic CSI report used with aperiodic CSI-RS occupies a CPU from the first symbol after the PDCCH triggering the CSI report until the last symbol of the PUSCH carrying the report.

----------------------------------------------------End of TP ---------------------------------------------------------------

Agreement

----------------------------------------------------Start of TP to Section 5.2.2.1.1 of 38.214-----------------------

The CSI reference resource for a serving cell is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CQI value relates.

-	In the time domain, for a UE configured with a single CSI resource set for the serving cell, the CSI reference resource for a CSI reporting in uplink slot n’ is defined by a single downlink slot n-nCQI_ref,

-    where  n=n'⋅2μDL2μULand μDL and μUL are the subcarrier spacing configurations for DL and UL,  respectively

-	where for periodic and semi-persistent CSI reporting

-	if a single CSI-RS resource is configured for channel measurement nCQI_ref is the smallest value greater than or equal to 4⋅2min µDL, µUL2μDL, such that it corresponds to a valid downlink slot, or

-	if multiple CSI-RS resources are configured for channel measurement nCQI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot.

-	where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCQI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCQI_ref is the smallest value greater than or equal to , such that slot n-nCQI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Subclause 5.4.

-	when periodic or semi-persistent CSI-RS/CSI-IM is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting.

----------------------------------------------------End of TP to Section 5.2.2.1.1 of 38.214-----------------------

Agreement

----------------------------------------------------Start of TP to Section 5.2.2.2 of 38.214-----------------------

5.2.2.2.1	Type I Single-Panel Codebook

< Unchanged part omitted >

For UE configured with higher layer parameter codebookType set to 'typeI-SinglePanel', tThe bitmap parameter typeI-SinglePanel-ri-Restriction forms the bit sequence  where  is the LSB and  is the MSB. When  is zero, , PMI and RI reporting are not allowed to correspond to any precoder associated with  layers.

< Unchanged part omitted >

5.2.2.2.4	Type II Port Selection Codebook

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031}, and the UE configured with higher layer parameter codebookType set to 'typeII-PortSelection'

----------------------------------------------------End of TP to Section 5.2.2.2 of 38.214-----------------------

Agreement

Adopt the text proposal for TS38.214 section 5.2.3(CSI reporting using PUSCH) which is provided in R1-1809828 section 2.7.3 (page 19~20).

Agreement

----------------------------------------------------Start of TP ------------------------------------------------------------

5.2.4 CSI reporting using PUCCH

< Unchanged parts are omitted >

A UE is not expected to report CSI with a payload size larger than 115 bits when configured with PUCCH format 4.

For CSI reports transmitted on a PUCCH, if all CSI reports consist of one part, the UE may omit a portion of CSI reports. Omission of CSI is according to the priority order determined from the Prii,CSI(y,k,c,s) value as defined in Subclause 5.2.5. CSI report is omitted beginning with the lowest priority level until the CSI report code rate is less or equal to  configured by higher layer parameter maxCodeRate.

If any of the CSI report consist of two parts, the UE may omit a portion of Part 2 CSI is omitted. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1. Part 2 CSI is omitted beginning with the lowest priority level until the Part 2 CSI code rate is less or equal to  configured by higher layer parameter maxCodeRate.

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

----------------------------------------------------Start of TP for 38.214----------------------------------------------

5.2.4	CSI reporting using PUCCH

A UE is semi-statically configured by higher layers to perform periodic CSI Reporting on the PUCCH. A UE can be configured by higher layers for multiple periodic CSI Reports corresponding to multiple higher layer configured CSI Reporting Settings, where the associated CSI Resource Settings are higher layer configured. Periodic CSI reporting on PUCCH formats 2, 3, 4 supports Type I CSI with wideband granularity.

A UE shall perform semi-persistent CSI reporting on the PUCCH applied starting from slot n+3Nslotsubframe,µ+1 after the HARQ-ACK corresponding to the PDSCH carrying the selection command [10, TS 38.321] is transmitted in slot n. The selection command will contain one or more Reporting Settings where the associated CSI Resource Settings are configured. Semi-persistent CSI reporting on the PUCCH supports Type I CSI. Semi-persistent CSI reporting on the PUCCH format 2 supports Type I CSI with wideband frequency granularity. Semi-persistent CSI reporting on PUCCH formats 3 or 4 supports Type I CSI with wideband and sub-band frequency granularities and Type II CSI Part 1.

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for TS38.214

----------------------------------------------------Start of TP ------------------------------------------------------------

5.2.5	Priority rules for CSI reports

< Unchanged parts are omitted >

A first CSI report is said to have priority over second CSI report if the associated  value is lower for the first report than for the second report.

Two CSI reports are said to collide if the time occupancy of the physical channels scheduled to carry the CSI reports overlap in at least one OFDM symbol and are transmitted on the same carrier. When a UE is configured to transmit two colliding CSI reports,

if y values are different between the two CSI reports, the following rules apply (for CSI reports transmitted on PUSCH, as described in Subclause 5.2.3; for CSI reports transmitted on PUCCH, as described in Subclause 5.2.4):

-	The CSI report with higher  value shall not be sent by the UE,

otherwise, the two CSI reports are multiplexed or either is dropped based on the priority values, as described in Subclause 9.2.5.2 in 38.213.

If a semi-persistent CSI report to be carried on PUSCH collides with PUSCH data transmission, when the starting symbols between the two channels are aligned, the CSI report shall not be transmitted by the UE.

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

- The values of Z1,Z1' for CSI timing requirement 1 for 15kHz SCS is Z1,Z1'=(10,8) symbols

  - Note: Huawei can accept this solution as a compromise for the purpose of progress

Agreement

----------------------------------------------------Start of TP Section 5.4 of 38.214--------------------------------

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, the UE shall provide valid CSI report(s),

-	if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts no earlier than at symbol Zref , and

-	if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts no earlier than at symbol Z'ref,

where Zref is defined as the next uplink symbol with its CP starting after  after the end of the last symbol of the PDCCH triggering the CSI report, and where Z'ref, is defined as the next uplink symbol with its CP starting after  after the end of the last symbol in time of the latest of: the last symbol of aperiodic CSI-RS resource for channel measurements, the last symbol of aperiodic CSI-IM used for interference measurements, and the last symbol of aperiodic NZP CSI-RS for interference measurement, when aperiodic CSI-RS is used for channel measurement for triggered CSI report n,

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts no earlier than at symbol Zref,

-	the UE may ignore the scheduling DCI if no HARQ-ACK or transport block is multiplexed on the PUSCH, or

-	the UE drops the CSI for the triggered CSI reports if either HARQ-ACK or transport block is multiplexed on the PUSCH,

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, if the first uplink symbol to carry the corresponding CSI report including the effect of the timing advance, starts no earlier than at symbol Z'ref for report n,

-	the UE may ignore the scheduling DCI if the number of triggered reports is one and no HARQ-ACK or transport block is multiplexed on the PUSCH

-	Otherwise, the UE is not required to update the CSI for the triggered CSI report n.

Z and Z' are defined as:

Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Subclause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as

-	(Z1,Z1') of the table 5.4-1 if the CSI is triggered without a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied (according to Subclause 5.2.1.6) and the CSI to be transmitted  is a single CSI and corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'TypeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or

-	(Z1,Z1') of the table 5.4-2 if the CSI is triggered without a PUSCH with either a transport block or HARQ-ACK or both and the CSI to be transmitted corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'TypeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or

-	If reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP',  Z'mis according to UE reported capability and  Zmis FFS, or

-	(Z2,Z2') of table 5.4-2 otherwise.

-	µ of table 5.4-1 and table 5.4-2 corresponds to the min (µPDCCH, µCSI-RS, µUL) where the µPDCCH corresponds to the subcarrier spacing of the PDCCH with which the DCI was transmitted and µUL corresponds to the subcarrier spacing of the PUSCH with which the CSI report is to be transmitted and µCSI-RS corresponds to the minimum subcarrier spacing of the aperiodic CSI-RS triggered by the DCI

-	d = 0 if the CSI is not multiplexed with a PUSCH with either a transport block or HARQ-ACK of both. If CSI is multiplexed with a PUSCH with either a transport block or HARQ-ACK of both, d = 2 for µ = 0,1, d = 3 for µ = 2 and d = 4 for µ = 3

- Table 5.4-1: CSI computation delay requirement 1

|  | Z1 [symbols] |  |
| --- | --- | --- |
|  | Z1 | Z'1 |
| 0 | [9 or 10] | [7 or 8] |
| 1 | 13 | 11 |
| 2 | 25 | 21 |
| 3 | 43 | 36 |

- Table 5.4-2: CSI computation delay requirement 2

|  | Z1 [symbols] |  | Z2 [symbols] |  |
| --- | --- | --- | --- | --- |
|  | Z1 | Z'1 | Z2 | Z'2 |
| 0 | 22 | 16 | 40 | 37 |
| 1 | 33 | 30 | 72 | 69 |
| 2 | 44 | 42 | 141 | 140 |
| 3 | 97 | 85 | 152 | 140 |

----------------------------------------------------End of TP Section 5.4 of 38.214--------------------------------

Agreement

Text proposal for TS38.214

----------------------------------------------------Start of TP ------------------------------------------------------------

6.1.2.1 Resource allocation in time domain

< Unchanged parts are omitted>

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in sub-clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission.

hen the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a CSI request field on a DCI, the Time-domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table which is defined by the higher layer configured pusch-TimeDomainResourceAllocationList in pusch-Config. The indexed row defines the start and length indicator SLIV, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as K2=maxjYj(m+1), where Yj, j=0,…,NRep-1, are the corresponding list entries  Yj, j=0,…,NRep-1of the higher layer parameter reportSlotConfigreportSlotOffsetList in CSI-ReportConfig for the triggered CSI Reporting Settings and Yj(m+1) is the (m+1)th entry of .

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

----------------------------------------------------Start of TP Section 6.3.1.1.2 of 38.212-----------------------

6.3.1.1.2	CSI only

< Unchanged part omitted >

Table 6.3.1.1.2-7: Mapping order of CSI fields of one CSI report, pmi-FormatIndicator=widebandPMI and cqi-FormatIndicator=widebandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI as in Tables 6.3.1.1.2-3/4, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4, if reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4, if reported |
|  | Zero padding bits , if needed |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or codebook index for 2 antenna ports according to Subclause 5.2.2.2.1 in [6, TS 38.214], if reported |
|  | Wideband CQI as in Tables 6.3.1.1.2-3/4/5, if reported |

< Unchanged part omitted >

Table 6.3.1.1.2-10: Mapping order of CSI fields of one CSI report, CSI part 2 wideband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or codebook index for 2 antenna ports according to Subclause 5.2.2.2.1 in [6, TS 38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.1.1.2-11: Mapping order of CSI fields of one CSI report, CSI part 2 subband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2 or codebook index for 2 antenna ports according to Subclause 5.2.2.2.1 in [6, TS 38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2 or codebook index for 2 antenna ports according to Subclause 5.2.2.2.1 in [6, TS 38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |

----------------------------------------------------End of TP Section 6.3.1.1.2 of 38.212-----------------------

----------------------------------------------------Start of TP Section 6.3.2.1.2 of 38.212-----------------------

6.3.2.1.2	CSI

< Unchanged part omitted >

Table 6.3.2.1.2-4: Mapping order of CSI fields of one CSI report, CSI part 2 wideband

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2 or codebook index for 2 antenna ports according to Subclause 5.2.2.2.1 in [6, TS 38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.2.1.2-5: Mapping order of CSI fields of one CSI report, CSI part 2 subband

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2 or codebook index for 2 antenna ports according to Subclause 5.2.2.2.1 in [6, TS 38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2 or codebook index for 2 antenna ports according to Subclause 5.2.2.2.1 in [6, TS 38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |

----------------------------------------------------End of TP Section 6.3.2.1.2 of 38.212-----------------------

Agreement

----------------------------------------------------Start of TP for 38.212--------------------------------------------

Section 6.3.2.1.2

[Unchanged text are omitted]

The bitwidth for PMI of codebookType=typeII is provided in Tables 6.3.2.1.2-1, where the values of , , , , , , and  are given by Subclause 5.2.2.2.3 in [6, TS 38.214].

Table 6.3.2.1.2-1: PMI of codebookType= typeII

|  | Information fields X1 for wideband PMI |  |  |  |  |  | Information fields X2 per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank=1 SBAmp off |  |  |  |  | N/A | N/A |  | N/A | N/A | N/A |
| Rank=2 SBAmp off |  |  |  |  |  |  |  |  | N/A | N/A |
| Rank=1 SBAmp on |  |  |  |  | N/A | N/A |  | N/A |  | N/A |
| Rank=2 SBAmp on |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType= typeII-PortSelection is provided in Tables 6.3.2.1.2-2, where the values of , , , , , , and  are given by Subclause 5.2.2.2.4 in [6, TS 38.214].

Table 6.3.2.1.2-2: PMI of codebookType= typeII-PortSelection

|  | Information fields X1 for wideband PMI |  |  |  |  | Information fields X2 per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank=1 SBAmp off |  |  |  | N/A | N/A |  | N/A | N/A | N/A |
| Rank=2 SBAmp off |  |  |  |  |  |  |  | N/A | N/A |
| Rank=1 SBAmp on |  |  |  | N/A | N/A |  | N/A |  | N/A |
| Rank=2 SBAmp on |  |  |  |  |  |  |  |  |  |

----------------------------------------------------End of TP for 38.212--------------------------------------------

Agreement

----------------------------------------------------Start of TP for 38.212--------------------------------------------

6.3.2.1.2	CSI

The bitwidth for PMI of codebookType=typeI-SinglePanel and codebookType=typeI-MultiPanel is specified in Subclause 6.3.2.1.1.

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel and codebookType=typeI-MultiPanel is specified in Subclause 6.3.2.1.1.

The bitwidth for PMI of codebookType=typeII is provided in Tables 6.3.2.1.2-1, where the values of , , , , , , and  are given by Subclause 5.2.2.2.3 in [6, TS 38.214].

----------------------------------------------------End of TP for 38.212--------------------------------------------

R1-1809741	Summary of CQI/MCS	AT&T

R1-1809868	Offline summary on CSI measurement	ZTE

Conclusion

From RAN1 point of view, the following is assumed

- To determine MAC-CE activation time, the end of transmission of ACK for the PDSCH carrying the MAC-CE shall be the end of the last slot of the PUCCH or PUSCH transmission carrying ACK for the transport block(s) associated with the PDSCH.

- In case of CBG-based PDSCH, the ACK for determining MAC-CE activation time refers to the last ACK for all CBGs, regardless which CBGs carry MAC-CE bits.

There is no RAN1 specification impact from the above understanding

Agreement

UE is not expected to be configured with more than 64 NZP CSI-RS resources in a resource setting for channel measurement linked to non-PMI feedback.

- There is no impact to RAN2 specifications

R1-1809912	Comeback issues for CSI reporting	Ericsson

Agreement

----------------------------------------------------Start of TP for 38.214--------------------------------------------

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS

For semi-persistent reporting on PUSCH, a set of semi-persistent Reporting settings are higher layer configured by CSI-SemiPersistentOnPUSCH-TriggerStateList, the CSI request field in DCI scrambled with SP-CSI-RNTI activates one of the semi-persistent CSI reports and the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. A UE is not expected to receive a DCI scrambled with SP-CSI-RNTI activating one semi-persistent CSI report with the same CSI-ReportConfigId as in a semi-persistent CSI report which is activated by a previously received DCI scrambled with SP-CSI-RNTI.

----------------------------------------------------End of TP for 38.214--------------------------------------------

Agreement

----------------------------------------------------Start of TP for 38.214--------------------------------------------

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS

-------------------   omitted  -----------------

A codepoint of the CSI request field in the DCI is mapped to a SP-CSI triggering state according to the order of the CSI triggering state ID of the configured trigger states, with codepoint '0' mapped to the triggering state having the smallest triggering state ID. A UE validates, for semi-persistent CSI activation or release, a DL semi-persistent assignment PDCCH on a DCI only if the following conditions are met:

-	the CRC parity bits of the DCI format are scrambled with a SP-CSI-RNTI provided by higher layer parameter sp-csi-RNTI

-	Special fields for the DCI format are set according to Table 5.2.1.5.2-1 or Table 5.2.1.5.2-2.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of semi-persistent CSI transmission on PUSCH. If validation is not achieved, the UE considers the DCI format as having been detected with a non-matching CRC.

-------------------   omitted  -----------------

----------------------------------------------------End of TP for 38.214--------------------------------------------

Agreement

----------------------------------------------------Start of TP for 38.212--------------------------------------------

6.3.1.1.2	CSI only

-------------------   omitted  -----------------

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel is provided in Tables 6.3.1.1.2-3.

Table 6.3.1.1.2-3: RI, LI, CQI, and CRI of codebookType=typeI-SinglePanel

| Field | Bitwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 antenna port | 2 antenna ports | 4 antenna ports | >4 antenna ports |  |
|  |  |  |  | Rank1~4 | Rank5~8 |
| Rank Indicator | 0 |  |  |  |  |
| Layer Indicator | 0 |  |  |  |  |
| Wide-band CQI | 4 | 4 | 4 | 4 | 8 |
| Subband differential CQI | 2 | 2 | 2 | 2 | 4 |
| CRI |  |  |  |  |  |

in Table 6.3.1.1.2-3 is the number of allowed rank indicator values according to Subclause 5.2.2.2.1 [6, TS 38.214]. The value of  is the number of CSI-RS resources in the corresponding resource set.

----------------------------------------------------End of TP for 38.212--------------------------------------------

Agreement

Send an LS to RAN2 to convey that nrofCQIsPerReport is not necessary from RAN1 perspective

R1-1809955	[Draft] LS on the number of CQI reports	Intel

Decision: The draft LS is endorsed and final LS is approved in R1-1809993

For the next RAN1 meeting:

- Discuss the following issues in RAN1#94bis:

  - CPU occupancy starting time for L1-RSRP reporting

  - Z timing requirement (from triggering PDCCH to start of PUSCH) for L1-RSRP reporting

  - Solution to avoid transmission of CSI for inactive/suspended BWP without changing the definition of valid downlink slot of the CSI reference resource.

R1-1809422	Maintenance for CSI acquisition	Qualcomm Incorporated

Further revised in R1-1809501

R1-1808143	Remaining issues on CSI acquisition	Huawei, HiSilicon

R1-1808220	Remaining issues on CSI acquisition	vivo

R1-1808263	Remaining issues in CSI acquisition	MediaTek Inc.

R1-1808362	Remaining issues on CSI reporting	NEC

R1-1808374	Issues on CSI acquisition	CATT

R1-1808486	Text proposals on CSI acquisition	LG Electronics

R1-1808550	Maintenance on CSI reporting	Lenovo, Motorola Mobility

R1-1808668	Remaining issues on CSI reporting	Intel Corporation

R1-1808749	Issues on CSI reporting	Samsung

R1-1808800	Remaining issues on CSI acquisition	Spreadtrum Communications

R1-1808885	Text proposals for CSI acquisition/reporting	Guangdong OPPO Mobile Telecom

R1-1809053	Text Proposal on Type 1 Codebook	AT&T

Withdrawn

R1-1809137	Maintenance for CSI acquisition	NTT DOCOMO, INC.

R1-1809196	Maintenance for CSI acquisition	Ericsson

R1-1809236	Remaining issues on CSI acquisition	Nokia, Nokia Shanghai Bell

R1-1809518	Maintenance for CSI acquisition	ZTE	(rev of R1-1808195)

#### Maintenance for beam management

Including measurement, reporting, beam failure recovery, etc.

Please use separate sections for each of the respective sub-topic to facilitate reviews and summary.

R1-1809805	Summary #1 on Remaing issues on Beam Failure Recovery	MediaTek

Agreement

Concerning the questions in RAN2 LS (R1-1808166) on BFR

Question 1: After UE sending PRACH for contention-free BFR, does the UE continue monitoring PDCCH candidates in configured search spaces monitored before PRACH, in addition to the search space indicated by recoverySearchSpaceId?

- RAN1 repsonse: Yes, all configured search spaces before dedicated BFR PRACH transmission for BFR are monitored

Question 2: If the answer to Question 1 is “Yes”, is the BFR RACH procedure considered successfully completed only if PDCCH is received in search space indicated by recoverySearchSpaceId?

- RAN1 repsonse: Yes, only when a PDCCH transmission addressed to C-RNTI is received on the search space indicated by higher layer parameter recovoerySearchSpaceId, i.e., SS-BFR, UE considers a contention-free BFR procedure is successfully terminated

Response LS to RAN2 is approved in R1-1809887

Agreement

Text proposal for TS38.213, section 6: Remove the following text

----------------------------------------------------Start of TP ------------------------------------------------------------

< Unchanged parts are omitted >

If the UE is not provided a control resource set for a search space set provided recoverySearchSpaceId or if the UE is not provided recoverySearchSpaceId, the UE does not expect to.

< Unchanged parts are omitted >

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

----------------------------------------------------Start of TP TS38.213 Section 6--------------------------------

< Unchanged parts are omitted >

The UE expects the set  to include up to two RS indexes and, if there are two RS indexes in a TCI state, the set  includes only RS indexes with QCL-TypeD configuration for the corresponding TCI states.

< Unchanged parts are omitted >

----------------------------------------------------End of TP TS38.213 Section 6--------------------------------

R1-1809864	Feature lead summary for beam management – Thursday	Ericsson

Agreement

NW may schedule an ap-CSI-RS with a shorter delay than a threshold, and the UE applies a default QCL assumption

- At least applies for CSI-RS for CSI acquisition

- All CSI-RS symbols in one resource set should be below the threshold or all CSI-RS symbols in one resource set should be above the threshold

- FFS: Default QCL assumption

- FFS: Exact threshold with reference to UE capability

Agreement

When TCIpresentinDCI is enabled, the TCI field in DCI in the scheduling CC points to the activated TCI states in the scheduled CC/BWP

Agreement

The indicated TCI should be based on the candidate TCI states in the slot with the scheduled PDSCH

- Applies for single slot PDSCH

Agreement

PUSCH on a cell cannot be scheduled by DCI format 0_0 without configured PUCCH resource on the same cell with spatial relation info in FR2.

- This does not preclude the deactivation DCI for configured grant on a SCell

Agreement

----------------------------------------------------Start of TP for 38.214 Section 5.2.1.2-------------------------

Each CSI Resource Setting CSI-ResourceConfig contains a configuration of a list of S≥1 CSI Resource Sets (given by higher layer parameter csi-RS-ResourceSetList), where the list is comprised of references to either or both of NZP CSI-RS resource set(s) and SS/PBCH block set(s) or the list is comprised of references to CSI-IM resource set(s). with each CSI Resource Set consisting of CSI-RS resources (comprised of either NZP CSI-RS or CSI-IM) and SS/PBCH Block resources used for L1-RSRP computation. Each CSI Resource Setting is located in the DL BWP identified by the higher layer parameter bwp-Id, and all CSI Resource Settings linked to a CSI Report Setting have the same DL BWP.

----------------------------------------------------End of TP for 38.214 Section 5.2.1.2-------------------------

Agreements

Increase the maximum number of configurable TCI states to 128 per CC

- Requires change to RAN2 specifications

- Support of up to 128 is UE optional

Send an LS to RAN

R1-1809908	[DRAFT] LS on maximum number of TCI states	Ericsson

Decision: The draft LS is endorsed and final LS is approved in  R1-1809888

For RAN1#94bis:

Companies are encouraged to check whether the existing radio resource control element ControlResourceSet could be used to configure TCI state for CORESET #0 or not

R1-1809926	Summary #2 on Remaing issues on Beam Failure Recovery	MediaTek

Agreement

Downselect among the following two alternatives in RAN1#94bis

- Alt 1: K symbols after successfully receiving BFR gNB response, the PUCCH transmissions shall use the same spatial filter as the PRACH transmission until the UE receives an activation or reconfiguration of PUCCH-Spatialrelationinfo of at least one of configured PUCCH resources

  - Note: The latency of RRC or MAC CE configuration is included as part of time duration for applying the same spatial filter as the PRACH transmission

  - FFS: value of K

- Alt 4: K symbols after successfully receiving BFR gNB response, the transmissions of PUCCH resources for HARQ ACK/NACK feedback of a corresponding DL PDSCH scheduled from SearchSpace-BFR shall use the same spatial filter as the PRACH transmission until the UE receives a MAC-CE activation or reconfiguration of PUCCH-Spatialrelationinfo of at least one of configured PUCCH resources

  - FFS: value of K

  - Note: The latency of RRC or MAC CE configuration is included as part of time duration for applying the same spatial filter as the PRACH transmission

R1-1808145	Remaining issues on beam management and beam failure recovery	Huawei, HiSilicon

R1-1808196	Maintenance for beam management	ZTE

R1-1808221	Remaining issues on beam management	vivo

R1-1808264	Remaining issues for beam management and beam failure recovery	MediaTek Inc.

R1-1808289	Remaining issues on Beam Failure Recovery	China Telecommunications

R1-1808330	Remaining issues on beam management and beam failure recovery	Sony

R1-1808375	Issues on beam management	CATT

R1-1808487	Text proposals on BM and BFR	LG Electronics

R1-1808669	Remaining Issues on Beam Management	Intel Corporation

R1-1808750	Remaining Issues on multi-beam operation	Samsung

R1-1808830	Discussion on remaining issues for beam management	CMCC

R1-1808886	Text proposals for beam management	Guangdong OPPO Mobile Telecom

R1-1809077	Remaining issues on beam management 	InterDigital, Inc.

R1-1809106	Remaining issues on beam management	Sharp

R1-1809116	Remaining issues on beam management	MTI

R1-1809138	Remaining issues on beam management	NTT DOCOMO, INC.

R1-1809197	Maintenance for beam management	Ericsson

R1-1809225	Remaining issues on beam management	ASUSTEK COMPUTER (SHANGHAI)

R1-1809237	Remaining issues on beam management	Nokia, Nokia Shanghai Bell

R1-1809711	Maintenance for beam management	Qualcomm Incorporated 	(rev of R1-1809423)

R1-1809759	Feature lead summary for beam management	Ericsson

#### Maintenance for Reference signals and QCL

Including CSI-RS, DM-RS, PT-RS, SRS, TRS, QCL, multiplexing of different types of RSs, etc.

Please use separate sections for each of the respective sub-topic to facilitate reviews and summary.

R1-1809743	Feature lead summary of remaining issues for RS multiplexing	Huawei, HiSilicon

Agreement

Text change for section 5.1.6.1.2 of TS 38.214

----------------------------------------------------Start of TP ------------------------------------------------------------

< Unchanged parts are omitted >

If the UE is configured with the CSI-RS resource in the same OFDM symbol(s) as an SS/PBCH block, the UE may assume that the CSI-RS and the SS/PBCH block are quasi co-located with 'QCL-TypeD' if 'QCL-TypeD' is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap with those of the SS/PBCH block, and the UE shall expect that the same subcarrier spacing is used for both the CSI-RS and the SS/PBCH block.

< Unchanged parts are omitted >

----------------------------------------------------End of TP ------------------------------------------------------------

R1-1809749	Remaining issues on CSI-RS #1	Huawei, HiSi

Agreement

Text proposal for Section 7.4.1.5.3 in 38.211:

----------------------------------------------------Start of TP ------------------------------------------------------------

The time-domain locations l0∈0, 1, 2, 3, …, 12, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameters firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain2, respectively, in the CSI-RS-ResourceMapping IE and defined relative to the start of a slot.

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for Section 5.1.4.2 in 38.214:

----------------------------------------------------Start of TP ------------------------------------------------------------

----------------------------------------------------End of TP ------------------------------------------------------------

Text proposal for Section 5.1.4.2 in 38.214:

----------------------------------------------------Start of TP ------------------------------------------------------------

-	The ZP-CSI-RS time domain behavior can be configured to periodic, semi-persistent or aperiodic by higher parameters within PDSCH-config. All the resources in a ZP CSI-RS resource set are configured with the same time domain behavior ('periodic', 'semi-persistent', 'aperiodic').

----------------------------------------------------End of TP ------------------------------------------------------------

Text proposal for Section 5.1.4.2 in 38.214:

----------------------------------------------------Start of TP ------------------------------------------------------------

For a UE configured with a list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList, is configured

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for Section 3.2 in 38.211:

----------------------------------------------------Start of TP ------------------------------------------------------------

System frame number; see clause 4.3.1

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for Section 5.1.6.1.2 in 38.214:

----------------------------------------------------Start of TP ------------------------------------------------------------

If a UE is configured with a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on', the UE may assume that the CSI-RS resources, described in Subclause 5.2.2.3.1, within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter, where the CSI-RS resources in the NZP-CSI-RS-ResourceSet are transmitted in different OFDM symbols. If repetition is set to 'off', the UE shall not assume that the CSI-RS resources within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter.

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

The possible agreement in section 2.14 of R1-1809749 is agreed (text proposal for Section 5.1.6.1.2 in 38.214).

----------------------------------------------------Start of TP ------------------------------------------------------------

Table 7.4.1.5.3-2: The sequences  and  for cdm-Type equal to 'no CDM'.

| Index | wf0 | wt0 |
| --- | --- | --- |
| 0 | 1 | 1 |

Table 7.4.1.5.3-3: The sequences  and  for cdm-Type equal to 'FD-CDM2'.

| Index | wf0wf1 | wt0 |
| --- | --- | --- |
| 0 |  | 1 |
| 1 |  | 1 |

Table 7.4.1.5.3-4: The sequences  and  for cdm-Type equal to 'CDM4'.

| Index | wf0wf1 | wt0wt1 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Table 7.4.1.5.3-5: The sequences  and  for cdm-Type equal to 'CDM8'.

| Index | wf0wf1 | wt0wt1wt2wt3 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |

----------------------------------------------------End of TP ------------------------------------------------------------

R1-1809762	Summary of DMRS Issues	Qualcomm

Agreement:

Text proposal for section 7.4.1.1.2 in 38.211

----------------------------------------------------Start of TP -----------------------------------------------------------

7.4.1.1.2	Mapping to physical resources

The UE shall assume the PDSCH DM-RS being mapped to physical resources according to configuration type 1 or configuration type 2 as given by the higher-layer parameter dmrs-Type.

The UE shall assume the sequence  is scaled by a factor  to conform with the transmission power specified in [6, TS 38.214] and mapped to resource elements  according to

where , , and  are given by Tables 7.4.1.1.2-1 and 7.4.1.1.2-2 and the following conditions are fulfilled:

-	the resource elements are within the common resource blocks allocated for PDSCH transmission

...

For PDSCH mapping type B

-	if the PDSCH duration is 2, 4, or 7 OFDM symbols for normal cyclic prefix or 2, 4, 6 OFDM symbols for extended cyclic prefix, and the PDSCH allocation collides with resources reserved for a search space set associated with a CORESET,  shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET and

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

The following text proposal for 4.1 of 38.214 is agreed.

----------------------------------------------------Start of TP -----------------------------------------------------------

4.1	Power allocation for downlink

For downlink DM-RS associated with PDSCH, the UE may assume the ratio of PDSCH EPRE to DM-RS EPRE ( [dB]) is given by Table 4.1-1 according to the number of DM-RS CDM groups without data as described in Subclause 5.1.6.2. The DM-RS scaling factor  specified in Subclause 7.4.1.1.2 of [4, TS 38.211] is given by  .

- Table 4.1-1: The ratio of PDSCH EPRE to DM-RS EPRE

| Number of DM-RS CDM groups without data | DM-RS configuration type 1 | DM-RS configuration type 2 |
| --- | --- | --- |
| 1 | 0 dB | 0 dB |
| 2 | -3 dB | -3 dB |
| 3 | - | -4.77 dB |

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

The following text proposal for 6.2.2 of 38.214 is agreed.

----------------------------------------------------Start of TP -----------------------------------------------------------

6.2.2	UE DM-RS transmission procedure

For uplink DM-RS with PUSCH, the UE may assume the ratio of PUSCH EPRE to DM-RS EPRE ( [dB]) is given by Table 6.2.2-1 according to the number of DM-RS CDM groups without data. The DM-RS scaling factor  specified in subclause 6.4.1.1.3 of [4, TS 38.211] is given by .

Table 6.2.2-1: The ratio of PUSCH EPRE to DM-RS EPRE

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

The following text proposal for section 6.4.1.1.1.1 of 38.211 is agreed.

----------------------------------------------------Start of TP -----------------------------------------------------------

6.4.1.1.1.1	Sequence generation when transform precoding is disabled

If transform precoding for PUSCH is not enabled, the sequence  shall be generated according to

- .

where the pseudo-random sequence  is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

where  is the OFDM symbol number within the slot,  is the slot number within a frame, and

-	 are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1;

-	 is given by the higher-layer parameter scramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI or CS-RNTI;

-	 otherwise.

The quantity  nSCID∈0,1is indicated by the DM-RS initialization field, if present, in the DCI associated with the PUSCH transmission if DCI format 0_1 in [4, TS 38.212] is used, otherwise nSCID=0.

----------------------------------------------------End of TP ------------------------------------------------------------

R1-1809792	Summary of DMRS Issues	Qualcomm

Agreement

----------------------------------------------------Start of TP for 38.211-----------------------------------------------

6.4.1.1.3	Precoding and mapping to physical resources

or PUSCH mapping type A, the case dmrs-AdditionalPosition equal to ‘ pos3’ is only supported when dmrs-TypeA-Position is equal to ‘ pos2’. For PUSCH mapping type A, duration of 4 symbols in Table 6.4.1.1.3-4 is only applicable for dmrs-TypeA-Position equals ‘pos2’.

The time-domain index  and the supported antenna ports  are given by Tables 6.4.1.1.3-3 through 6.4.1.1.3-6:

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is not configured and frequency hopping is disabled, the tables shall be used according to single-symbol DM-RS

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to ‘len2’ and frequency hopping is disabled, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used

-	if the higher-layer parameter dmrs-AdditionalPosition is not set to ‘pos0’ and frequency hopping is enabled according to clause 7.3.1.1.2 in [4, TS 38.212], Tables 6.4.1.1.3-6 and 6.4.1.1.3-7 shall be used assuming dmrs-AdditionalPosition is equal to ‘pos1’ for each hop.

/***************************** Omitted part *******************************/

7.4.1.1.2	Mapping to physical resources

The reference point for  and the position  of the first DM-RS symbol depends on the mapping type:

-	for PDSCH mapping type A:

-	 is defined relative to the start of the slot

-	 if the higher-layer parameter dmrs-TypeA-Position is equal to ‘pos3’ and  otherwise

-	for PDSCH mapping type B:

-	 is defined relative to the start of the scheduled PDSCH resources

-

The position(s) of the DM-RS symbols is given by  and

-	for PDSCH mapping type A, the duration is between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PDSCH resources in the slot

-	for PDSCH mapping type B, the duration is the number of OFDM symbols of the scheduled PDSCH resources as signalled

and according to Tables 7.4.1.1.2-3 and 7.4.1.1.2-4. The case dmrs-AdditionalPosition equal to ‘pos3’ is only supported when dmrs-TypeA-Position is equal to ‘pos2’. For PDSCH mapping type A, duration of 3 and 4 symbols in Tables 7.4.1.1.2-3 and 7.4.1.1.2-4 respectively is only applicable when dmrs-TypeA-Position is equal to ‘pos2’.

For PDSCH mapping type B

-	if the PDSCH duration is 2, 4, or 7 OFDM symbols for normal cyclic prefix or 2, 4, 6 OFDM symbols for extended cyclic prefix, and the PDSCH allocation collides with resources reserved for a CORESET,  shall be incremented such that the first DM-RS symbol occurs immediately after the CORESETand

-	if the PDSCH duration is 4 symbols, the UE is not expected to receive a DM-RS symbol beyond the third symbol,

-	if the PDSCH duration is 7 symbols for normal cyclic prefix or 6 symbols for extended cyclic prefix,

-	the UE is not expected to receive the first DM-RS beyond the fourth symbol, and

-	if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted.

-	if the PDSCH duration is 2 or 4 OFDM symbols, only single-symbol DM-RS is supported.

The time-domain index  and the supported antenna ports  are given by Table 7.4.1.1.2-5 where

-	single-symbol DM-RS is used if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is not configured,

-	single-symbol or double-symbol DM-RS is determined by the associated DCI if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is equal to ‘len2’

In absence of CSI-RS configuration, and unless otherwise configured, the UE may assume PDSCH DM-RS and SS/PBCH block to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters. The UE may assume that the PDSCH DM-RS within the same CDM group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx. The UE may assume that DMRS ports associated with a PDSCH are QCL with QCL Type A, Type D (when applicable) and average gain.

----------------------------------------------------End of TP for 38.211-----------------------------------------------

Agreement

----------------------------------------------------Start of TP for 38.214-----------------------------------------------

6.2.2	UE DM-RS transmission procedure

If frequency hopping is enabled:

-	The UE shall assume dmrs-AdditionalPosition equal to 'pos1' and up to one additional DM-RS can be transmitted according to PUSCH duration.

For the UE-specific reference signals generation as defined in Subclause 6.4.1.1 of [4, TS 38.211], a UE can be configured by higher layers with one or two scrambling identity(s),  nIDDMRS,i i = 0,1 which are the same for both PUSCH mapping Type A and Type B.

When transmitting PUSCH scheduled by PDCCH format 0_1 with CRC scrambled by C-RNTI or CS-RNTI,

-	the UE may be configured with higher layer parameter dmrs-Type in DMRS-UplinkConfig, and the configured DM-RS configuration type is used for transmitting PUSCH in as defined in Subclause 6.4.1.1 of [4, TS 38.211].

-	the UE may be configured with the maximum number of front-loaded DM-RS symbols for PUSCH by higher layer parameter maxLength in DMRS-UplinkConfig.

- if maxLength is not configured, single-symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be ‘pos0’, ‘pos1’ , ‘pos2’ or ‘pos3’.

- if maxLength is configured, either single-symbol DM-RS or double symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0' or 'pos1'.

- and, the UE shall transmit a number of additional DM-RS as specified in Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 in -Subclause 6.4.1.1.3 of [4, TS 38.211].

----------------------------------------------------End of TP for 38.214-----------------------------------------------

Agreement

- Agree on the following text proposal for TS 38.211 v15.2.0 Section 6.4.1.1.1.3

----------------------------------------------------Start of TP -----------------------------------------------------------

The position(s) of the DM-RS symbols is given by  and

-	the signalled duration between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PUSCH resources in the slot for PUSCH mapping type A according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if inter-slot frequency hopping is used or intra-slot frequency hopping is not used, or

-	the signalled duration of scheduled PUSCH resources for PUSCH mapping type B according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if inter-slot frequency hopping is used or intra-slot frequency hopping is not used, or

-	the signalled duration per hop according to Table 6.4.1.1.3-6 if intra-slot frequency hopping is used.

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is not configured, the tables shall be used according to single-symbol DM-RS

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to ‘len2’, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used

-	if the higher-layer parameter dmrs-AdditionalPosition is equal to or more than 1 not set to ‘pos0’ and intra-slot frequency hopping is enabled according to clause 7.3.1.1.2 in [4, TS 38.212] and by higher layer, Tables 6.4.1.1.3-6 shall be used assuming dmrs-AdditionalPosition is equal to ‘pos1’ for each hop.

For PUSCH mapping type A, the case dmrs-AdditionalPosition equal to 3‘pos3’ is only supported when dmrs-TypeA-Position is equal to 2‘pos2’. For PUSCH mapping type A, duration of 4 symbols in Table 6.4.1.1.3-4 is only applicable for dmrs-AdditionalPosition equals ‘pos1’.

The time-domain index  and the supported antenna ports  are given by Tables 6.4.1.1.3-35. through 6.4.1.1.3-6:

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 1 and frequency hopping is disabled, the tables shall be used according to single-symbol DM-RS

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 2 and frequency hopping is disabled, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used

-	if the higher-layer parameter dmrs-AdditionalPosition is equal to or more than 1 and frequency hopping is enabled according to clause 7.3.1.1.2 in [4, TS 38.212], Tables 6.4.1.1.3-6 and 6.4.1.1.3-7 shall be used assuming dmrs-AdditionalPosition is 1 for each hop.

Table 6.4.1.1.3-3: PUSCH DM-RS positions within slot  for single-symbol DM-RS and intra-slot frequency hopping disabled and frequency hopping disabled.

| Duration in symbols | DM-RS positions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUSCH mapping type A |  |  |  | PUSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| <4 | - | - | - | - |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 6 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 7 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 8 |  | , 7 | , 7 | , 7 |  | , 6 | , 3, 6 | , 3, 6 |
| 9 |  | , 7 | , 7 | , 7 |  | , 6 | , 3, 6 | , 3, 6 |
| 10 |  | , 9 | , 6, 9 | , 6, 9 |  | , 8 | , 4, 8 | , 3, 6, 9 |
| 11 |  | , 9 | , 6, 9 | , 6, 9 |  | , 8 | , 4, 8 | , 3, 6, 9 |
| 12 |  | , 9 | , 6, 9 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |
| 13 |  | , 11 | , 7, 11 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |
| 14 |  | , 11 | , 7, 11 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |

Table 6.4.1.1.3-4: PUSCH DM-RS positions within slot  for double-symbol DM-RS and intra-slot frequency hopping disabled and frequency hopping disabled.

| Duration in symbols | DM-RS positions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUSCH mapping type A |  |  |  | PUSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| <4 | - | - |  |  | - | - |  |  |
| 4 |  |  |  |  | - | - |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  | , 5 |  |  |
| 9 |  |  |  |  |  | , 5 |  |  |
| 10 |  | , 8 |  |  |  | , 7 |  |  |
| 11 |  | , 8 |  |  |  | , 7 |  |  |
| 12 |  | , 8 |  |  |  | , 9 |  |  |
| 13 |  | , 10 |  |  |  | , 9 |  |  |
| 14 |  | , 10 |  |  |  | , 9 |  |  |

Table 6.4.1.1.3-6: PUSCH DM-RS positions within slot  for single-symbol DM-RS and intra-slot frequency hopping enabled.

| Duration in symbols | DM-RS positions l |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUSCH mapping type A |  |  |  |  |  |  |  | PUSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 1 |  |
|  | 1st hop | 2nd hop | 1st hop | 2nd hop | 1st hop | 2nd hop | 1st hop | 2nd hop | 1st hop | 2nd hop | 1st hop | 2nd hop |
| ≤3 | - | - | - | - | - | - | - | - | 0 | 0 | 0 | 0 |
| 4 | 2 | 0 | 2 | 0 | 3 | 0 | 3 | 0 | 0 | 0 | 0 | 0 |
| 5, 6 | 2 | 0 | 2 | 0, 4 | 3 | 0 | 3 | 0, 4 | 0 | 0 | 0, 4 | 0, 4 |
| 7 | 2 | 0 | 2, 6 | 0, 4 | 3 | 0 | 3 | 0, 4 | 0 | 0 | 0, 4 | 0, 4 |

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

TP1 for section 7.4.1.1.1 in 38.211

7.4.1.1.1 Sequence generation … The quantity nSCID∈0, 1is given by the DM-RS sequence initialization field in the DCI associated with the PDSCH transmission if DCI format 1_1 in [4, TS 38.212] is used, otherwise nSCID=0.

TP2 for section 7.3.1.1.2 in 38.212

7.3.1.1.2 Format 0_1 ... DMRS sequence initialization – 0 bit if the higher layer parameter transformPrecoder=enabled; 1 bit if the higher layer parameter transformPrecoder=disabled.

TP3 for section 7.3.1.2.2 in 38.212

7.3.1.2.2 Format 1_1 ... - DMRS sequence initialization – 1 bit.

Agreement

Agree to the following text proposal for TS38.211, section 7.3.1.4

The set of antenna ports  shall be determined according to the procedure in [4, TS 38.212]

R1-1809777	Feature lead summary of PT-RS	Ericsson

Agreement

Text proposal for TS38.214

----------------------------------------------------Start of TP -----------------------------------------------------------

4.1	Power allocation for downlink

<<<<<<<<<<<<<<<<<<<<<<<<omitted>>>>>>>>>>>>>>>>>>>

When the UE is scheduled with a PT-RS ports associated with the PDSCHand when the PT-RS port is associated to DM-RS ports,

-	if the UE is configured with the higher layer parameter epre-Ratio, the ratio of PT-RS EPRE to PDSCH EPRE per layer per RE for PT-RS port () is given by Table 4.1-2 according to the epre-Ratio, the PT-RS scaling factor specified in subclause 7.4.1.2.2 of [4, TS 38.211] is given by.

-	otherwise, the UE shall assume epre-Ratio is set to state '0' in Table 4.1-2 if not configured.

Table 4.1-2: PT-RS-to-PDSCH EPRE ratio per layer for PT-RS port i ()

| epre-Ratio | The number of PDSCH layers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| 0 | 0 | 3 | 4.77 | 6 | 7 | 7.78 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | reserved |  |  |  |  |  |
| 3 | reserved |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<omitted>>>>>>>>>>>>>>>>>>>

When the UE is scheduled with PT-RS ports associated with the PDSCH and when the PT-RS port is associated to DM-RS ports,

-	if the UE is configured with the higher layer parameter epre-Ratio, the ratio of PT-RS EPRE to PDSCH EPRE per layer per RE for PT-RS port () is given by Table 4.1-2 according to the epre-Ratio, the PT-RS scaling factor specified in subclause 7.4.1.2.2 of [4, TS 38.211] is given by.

-	otherwise, the UE shall assume epre-Ratio is set to state '0' in Table 4.1-2 if not configured.

Table 4.1-2: PT-RS-to-PDSCH EPRE ratio per layer for PT-RS port i ()

| epre-Ratio | The number of PDSCH layers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| 0 | 0 | 3 | 4.77 | 6 | 7 | 7.78 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | reserved |  |  |  |  |  |
| 3 | reserved |  |  |  |  |  |

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for TS38.214

----------------------------------------------------Start of TP -----------------------------------------------------------

6.2.3.1	UE PT-RS transmission procedure when transform precoding is not enabled

<<<<<<<<<<<<<<<<<<<<<<<<omitted>>>>>>>>>>>>>>>>>>>

The higher layer parameter PTRS-UplinkConfig provides the parameters ptrs-MCSi, i=1,2,3 and with values in 0-29 when MCS Table 5.1.3.1-1 is configured and 0-28 when MCS Table 5.1.3.1-2 is configured, respectively. ptrs-MCS4 is not explicitly configured by higher layers but assumed 29 when MCS Table 5.1.3.1-1 is configured and 28 when MCS Table 5.1.3.1-2 is configured. The higher layer parameter PTRS-UplinkConfig provides the parameters NRBi i=0,1 with values in range 01-276.

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement Text proposal for TS38.212

----------------------------------------------------Start of TP -----------------------------------------------------------

7.3.1.1.2	 Format 0_1

< Unchanged parts are omitted >

Table 7.3.1.1.2-25: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 0 1st scheduled DMRS port |
| 1 | 1 2nd scheduled DMRS port |
| 2 | 2 3rd scheduled DMRS port |
| 3 | 3 4th scheduled DMRS port |

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for TS38.214, section 6.2.3

“by DCI parameter PTRS-DMRS association as shown in DCI format 0_1 described in Subclause 7.3.1 of [5, TS38.212].”

Agreement

Text proposal for TS38.214

----------------------------------------------------Start of TP -----------------------------------------------------------

6.2.3	UE PT-RS transmission procedure

If a UE is not configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, the UE shall not transmit PT-RS. The PTRS may only be present if RNTI equals C-RNTI, CS-RNTI, SP-CSI-RNTI or new-RNTI.

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

The TP for section 5.1.6.3 and 6.2.3 in 38.214 as follows:

----------------------------------------------------Start of TP -----------------------------------------------------------

5.1.6.3	PT-RS reception procedure

< Unchanged parts are omitted >

If a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-DownlinkConfig,

-	the higher layer parameters timeDensity and frequencyDensity in PTRS-DownlinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, respectively.

-	if either or both of the additional higher layer parameters timeDensity and frequencyDensity are configured, and the RNTI equals new-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS antenna ports' presence and pattern are a function of the corresponding scheduled MCS of the corresponding codeword and scheduled bandwidth in corresponding bandwidth part as shown in Table 5.1.6.3-1, and Table 5.1.6.3-2, and Table 5.1.6.3-3,

<…>

-	otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals new-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2, and

< Unchanged parts are omitted >

The higher layer parameter PTRS-DownlinkConfig provides the parameters ptrs-MCSi, i=1,2,3 and with values in range 0-29 when MCS Table 5.1.3.1-1 or MCS Table 5.1.3.1-3 is used configured and 0-28 when MCS Table 5.1.3.1-2 is used configured, respectively. ptrs-MCS4 is not explicitly configured by higher layers but assumed 29 when MCS Table 5.1.3.1-1 or MCS Table 5.1.3.1-3 is used configured and 28 when MCS Table 5.1.3.1-2 is used configured, respectively. The higher layer parameter frequencyDensity in PTRS-DownlinkConfig provides the parameters NRBi i=0,1 with values in range 1-276.

< Unchanged parts are omitted >

When a UE is receiving PDSCH for retransmission, if the UE is scheduled with an MCS index greater than V, where V=28 for MCS Table 5.1.3.1-1 and Table 5.1.3.1-3, and V=27 for MCS Table 5.1.3.1-2 respectively,

< Unchanged parts are omitted >

6.2.3	UE PT-RS transmission procedure

If a UE is not configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, the UE shall not transmit PT-RS. The PTRS may only be present if RNTI equals new-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI.

6.2.3.1	UE PT-RS transmission procedure when transform precoding is not enabled

< Unchanged parts are omitted >

The higher layer parameter PTRS-UplinkConfig provides the parameters ptrs-MCSi, i=1,2,3 and with values in 0-29 when MCS Table 5.1.3.1-1 or Table 5.1.3.1-3 is used configured, and 0-28 when MCS Table 5.1.3.1-2 is used configured, respectively. ptrs-MCS4 is not explicitly configured by higher layers but assumed 29 when MCS Table 5.1.3.1-1 or Table 5.1.3.1-3 is used configured, and 28 when MCS Table 5.1.3.1-2 is used configured. The higher layer parameter PTRS-UplinkConfig provides the parameters NRBi i=0,1 with values in range 0-276.

< Unchanged parts are omitted >

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

The TP for TS 38.214 v15.2.0 Section 5.1.6.3 as follows

----------------------------------------------------Start of TP -----------------------------------------------------------

If a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-DownlinkConfig,

-	the higher layer parameters timeDensity and frequencyDensity in PTRS-DownlinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, respectively.

-	if either or both of the additional higher layer parameters timeDensity and frequencyDensity are configured, and the RNTI equals C-RNTI or CS-RNTI, the UE shall assume the PT-RS antenna ports' presence and pattern are a function of the corresponding scheduled MCS of the corresponding codeword and scheduled bandwidth in corresponding bandwidth part as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2,

-	if the higher layer parameter timeDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume LPT-RS = 1.

-	if the higher layer parameter frequencyDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume KPT-RS = 2.

-	otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals C-RNTI or CS-RNTI, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2, and the UE shall assume PT-RS is not present when

-	The UE shall assume PT-RS is not present when,

-	the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or

-	the scheduled MCS from Table 5.1.3.1-2 is smaller than 5, or

-	the number of scheduled RBs is smaller than 3, or

-	otherwise, if the RNTI equals RA-RNTI, SI-RNTI, or P-RNTI, the UE shall assume PT-RS is not present

----------------------------------------------------End of TP ------------------------------------------------------------

R1-1809519	Summary of SRS	Sony

Agreement

Text proposal on SRS resource configuration in 38.214 version 15.2.0

< Start of the text proposal > 6.2 UE reference symbolsignal (RS) procedure 6.2.1 UE sounding procedure The UE can be configured with one or more Sounding Reference Signal (SRS) resource sets as configured by the higher layer parameter SRS-ResourceSet. For each SRS resource set, a UE may be configured with SRS resources (higher layer parameter SRS-Resource), where the maximum value of K is indicated by [SRS_capability [13, 38.306]]. The SRS resource set applicability is configured by the higher layer parameter SRS-SetUse. When the higher layer parameter SRS-SetUse is set to 'BeamManagement', only one SRS resource in each of multiple SRS sets can be transmitted at a given time instant. The SRS resources in different SRS resource sets can be transmitted simultaneously. For aperiodic SRS at least one state of the DCI field is used to select at least one out of the configured SRS resource sets. < Unchanged parts are omitted > The UE is not expected to be configured with different time domain behavior for SRS resources in the same SRS resource set. The UE is also not expected to be configured with different time domain behavior between SRS resource and associated SRS resources set. The 2-bit SRS request field [5 TS38.212] in DCI format 0_1, 1_1 indicates the triggered SRS resource set given in Table 7.3.1.1.2-24 of [5, TS 38212]. The 2-bit SRS request field [5, TS38.212] in DCI format 2_3 indicates the triggered SRS resource set given in Subclause 11.4 of [6, TS 38.213]. < End of the text proposal >

Agreement

Text proposal on SRS rule in 38.214 version 15.2.0

< Start of the text proposal > 6.2.1 UE sounding procedure < Unchanged parts are omitted > In case of intra-band carrier aggregation, a UE is not expected to be configured with SRS and PUSCH/UL DM-RS/UL PT-RS/PUCCH in the same symbol. < End of the text proposal >

For further discussion in RAN1#94bis

Simultaneous SRS transmission on a different CC in intra-band or inter-band CA

Agreement

Text proposal on SRS rule in 38.214 version 15.2.0

< Start of the text proposal > 6.2.1 UE sounding procedure < Unchanged parts are omitted > The 2-bit SRS request field [5 TS38.212] in DCI format 0_1, 1_1 indicates the triggered SRS resource set given in Table 7.3.1.1.2-24 of [5, TS 38212]. The 2-bit SRS request field [5, TS38.212] in DCI format 2_3 indicates the triggered SRS resource set given in Subclause 11.4 of [6, TS 38.213]. For PUCCH formats 0 and 2, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s) or if aperiodic SRS is configured and PUCCH consists of beam failure request. A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only. < End of the text proposal >

Agreement

Text proposal on SRS in 38.214 version 15.2.0

----------------------------------------------------Start of TP -----------------------------------------------------------

< Unchanged parts are omitted >

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent symbols within the last 6 symbols of the slot, where all antenna ports of the SRS resources are mapped to each symbol of the resource.

< Unchanged parts are omitted >

6.2.1.1	UE SRS frequency hopping procedure

AFor a given SRS resource, the UE is configured with repetition factor R∈{1,2,4} by higher layer parameter resourceMapping in SRS-Resource where R≤Ns. When frequency hopping within an SRS resource in each slot is not configured (R=Ns), all antenna ports of the SRS resource in each slot are mapped in each of the  symbols to the same set of subcarriers in the same set of PRBs. When frequency hopping within as SRS resource in each slot is configured without repetition (R=1), according to the SRS hopping parameters , and defined in Subclause 6.4.1.4 of [4, TS 38.211], all antenna ports of the SRS resource in each slot are mapped to different sets of subcarriers in each OFDM symbol, where the same transmission comb value is assumed for different sets of subcarriers. When both frequency hopping and repetition within an SRS resource in each slot are configured (Ns=4, R=2), all antenna ports of the SRS resource in each slot are mapped to the same set of subcarriers within each pair of R adjacent OFDM symbols, and frequency hopping across the two pairs is according to the SRS hopping parameters , and .

< Unchanged parts are omitted >

6.2.1.3	UE sounding procedure between component carriers

< Unchanged parts are omitted >

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE’s indicated uplink carrier aggregation capability included in the SRS_capability [13, TS 38.306].

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for section 6.2.1.1 in 38.214

----------------------------------------------------Start of TP -----------------------------------------------------------

6.2.1.1	UE SRS frequency hopping procedure

A UE may be configured to transmit an SRS resource on  adjacent symbols within the last six symbols of a slot, where all antenna ports of the SRS resource are mapped to each symbol of the resource. For a given SRS resource, the UE is configured with repetition factor R∈{1,2,4} by higher layer parameter resourceMapping in SRS-Resource where R≤Ns. When frequency hopping within an SRS resource in each slot is not configured (R=Ns), each of the antenna ports of the SRS resource in each slot is mapped in all the  symbols to the same set of subcarriers in the same set of PRBs. When frequency hopping within an SRS resource in each slot is configured without repetition (R=1), according to the SRS hopping parameters , and defined in Subclause 6.4.1.4 of [4, TS 38.211], each of the antenna ports of the SRS resource in each slot is mapped to different sets of subcarriers in each OFDM symbol, where the same transmission comb value is assumed for different sets of subcarriers. When both frequency hopping and repetition within an SRS resource in each slot are configured (Ns=4, R=2), each of the antenna ports of the SRS resource in each slot is mapped to the same set of subcarriers within each pair of R adjacent OFDM symbols, and frequency hopping across the two pairs is according to the SRS hopping parameters , and .

A UE may be configured  adjacent symbol aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across  symbols when frequency hopping is configured with R=1. A UE may be configured  adjacent symbols aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across two pairs of R adjacent OFDM symbols, when frequency hopping is configured with R=2. Each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each pair of R adjacent OFDM symbols of the resource.

A UE may be configured symbol periodic or semi-persistent SRS resource with inter-slot hopping within a bandwidth part, where the SRS resource occupies the same symbol location in each slot. A UE may be configured symbol periodic or semi-persistent SRS resource with intra-slot and inter-slot hopping within a bandwidth part, where the N-symbol SRS resource occupies the same symbol location(s) in each slot. For Ns=4, when frequency hopping is configured with R=2, intra-slot and inter-slot hopping is supported with each of the antenna ports of the SRS resource mapped to different sets of subcarriers across two pairs of R adjacent OFDM symbol(s) of the resource in each slot. Each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each pair of R adjacent OFDM symbols of the resource in each slot. For Ns= R, when frequency hopping is configured, inter-slot frequency hopping is supported with each of the antenna ports of the SRS resource mapped to the same set of subcarriers in R adjacent OFDM symbol(s) of the resource in each slot

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal on SRS resource configuration in 38.211 version 15.2.0

----------------------------------------------------Start of TP -----------------------------------------------------------

6.4.1.4	Sounding reference signal

< Unchanged parts are omitted >

The frequency-domain starting position  is defined by

where

The frequency domain shift value  adjusts the SRS allocation to align with respect to the common resource block grid  four and is contained in the higher-layer parameter freqDomainShift in the SRS-Config IE. The transmission comb offset  is contained in the higher-layer parameter transmisisonComb transmissionComb in the SRS-Config IE and  is a frequency position index.

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for TS 38.214 V15.2.0 (2018-06)

----------------------------------------------------Start of TP -----------------------------------------------------------

6.2.1	UE sounding procedure

< Unchanged parts are omitted >

For PUCCH formats 0 and 2, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s) or if aperiodic SRS is configured and PUCCH consists of beam failure request. A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with semi-persistent or periodic PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only.

< Unchanged parts are omitted >

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for TS 38.211

----------------------------------------------------Start of TP -----------------------------------------------------------

6.4.1.4.1	SRS resource

An SRS resource is configured by the SRS-Resource IE and consists of

-	 antenna ports , where pi=i+1000, and where the number of antenna ports is given by the higher layer parameter nrofSRS-Ports

-	 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping

-	, the starting position in the time domain given by  where the offset counts symbols backwards from the end of the slot and is given by the field startPosition contained in the higher layer parameter resourceMapping and

-	, the frequency-domain starting position of the sounding reference signal

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for 38.214 section 6.2.1 and section 6.2.1.3.

----------------------------------------------------Start of TP -----------------------------------------------------------

6.2.1	UE sounding procedure

The UE can be configured with one or more Sounding Reference Symbol (SRS) resource sets as configured by the higher layer parameter SRS-ResourceSet. For each SRS resource set, a UE may be configured with SRS resources (higher laterlayer parameter SRS-Resource), where the maximum value of K is indicated by [SRS_capability [13, 38.306]]. The SRS resource set applicability is configured by the higher layer parameter usage. When the higher layer parameter usage is set to ‘BeamManagement’, only one SRS resource in each of multiple SRS sets can be transmitted at a given time instant. The SRS resources in different SRS resource sets can be transmitted simultaneously.

For aperiodic SRS at least one state of the DCI field is used to select at least one out of the configured SRS resource set(s).

The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource.

-	srs-ResourceId determines SRS resource configuration identify.

-	Number of SRS ports as defined by the higher layer parameter nrofSRS-Ports and described in Subclause 6.4.1.4 of [4, TS 38.211].

-	Time domain eriodic of SRS resource configuration as indicated by the higher layer parameter  resourceType, which can be periodic, semi-persistent, aperiodic SRS transmission as defined in Subclause 6.4.1.4 of [4, TS 38.211].

-	Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p  or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent. The UE shall not expect to be configured with SRS resources in the same SRS resource set SRS-ResourceSet with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to ‘aperiodic’, a slot level offset is defined by the higher layer parameter slotOffset.

-	Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping and described in Subclause 6.4.1.4 of [4, TS 38.211].

-	SRS bandwidth and , as defined by the higher layer parameter freqHopping and described in Subclause 6.4.1.4 of [4, TS 38.211].

-	Frequency hopping bandwidth, , as defined by the higher layer parameter freqHopping and described in Subclause 6.4.1.4 of [4, TS 38.211].

-	Defining frequency domain position and configurable shift to align SRS allocation to 4 PRB grid, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, respectively, and described in Subclause 6.4.1.4 of [4, TS 38.211].

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2 or cyclicShift-n4 for transmission comb value 2 and 4, respectively, and described in Subclause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb value as defined by the higher layer parameter transmissionComb described in Subclause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb offset as defined by the higher layer parameter combOffset-n2 or combOffset-n4 for transmission comb value 2 or 4, respectively, and described in Subclause 6.4.1.4 of [4, TS 38.211].

-	SRS sequence ID as defined by the higher layer parameter sequenceId in Subclause 6.4.1.4 of [4].

-	The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo, if configured, contains the ID of the reference RS. The reference RS can be an SS/PBCH block, CSI-RS or an SRS configured on the same or different component carrier and/or bandwidth part as the target SRS.

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying a location within the last 6 symbols of the slot.

When PUSCH and SRS are transmitted in the same slot, the UE may be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to ‘periodic’:

-	if the UE is configured with the higher layer parameter spatialRelationInfo containing the ID of a reference ‘ssb-Index’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo contains the ID of a reference ‘csi-RS-Index’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo containing the ID of a reference ‘srs’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to ‘semi-persistent’:

-	when a UE receives an activation command [10, TS 38.321] for an SRS resource, and when the HARQ-ACK corresponding to the PDSCH carrying the selection command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission corresponding to the configured SRS resource set shall be applied starting from slot . n+3Nslotsubframe,µ+1. The activation command also contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the activated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource, or SRS resource configured on the same or different component carrier and/or bandwidth part as the SRS resource(s) in the SRS resource set.

-	if an SRS resource in the activated resource set is configured with the higher layer parameter spatialRelationInfo, the UE shall assume that the ID of the reference signal in the activation command overrides the one configured in spatialRelationInfo.

-	when a UE receives a deactivation command [10, TS 38.321] for an activated SRS resource set, and when the HARQ-ACK corresponding to the PDSCH carrying the selection command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of SRS transmission corresponding to the deactivated SRS resource set shall apply starting from slot n+3Nslotsubframe,µ+1..

-	if the UE is configured with the higher layer parameter spatialRelationInfo containing the ID of a reference ‘ssb-Index’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo contains the ID of a reference ‘csi-RS-Index’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo contains the ID of a reference ‘srs’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS.

If the UE has an active semi-persistent SRS resource configuration and has not received a deactivation command, the semi-persistent SRS configuration is considered to be active in the UL BWP which is active , otherwise it is considered suspended.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to ‘aperiodic’:

-	the UE receives a configuration of SRS resource sets,

-	the UE receives a downlink DCI, a group common DCI, or an uplink DCI based command where a codepoint of the DCI may trigger one or more SRS resource set(s). The minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 + 42.

-	if the UE is configured with the higher layer parameter spatialRelationInfo containing the ID of a reference ‘ssb-Index’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo contains the ID of a reference ‘csi-RS-Index’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, or of the latest reference aperiodic CSI-RS. If the higher layer parameter spatialRelationInfo contains the ID of a reference ‘srs’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS or of the reference aperiodic SRS.

The 2-bit SRS request field [5 TS38.212] in DCI format 0_1, 1_1 indicates the triggered SRS resource set given in Table 7.3.1.1.2-24 of [5, TS 38212]. The 2-bit SRS request field [5, TS38.212] in DCI format 2_3 indicates the triggered SRS resource set given in Subclause 7.3 of [5, TS 38.212].

For PUCCH formats 0 and 2, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s) or if aperiodic SRS is configured and PUCCH consists of beam failure request. A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with semi-persistent or periodic PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only.

A UE is not expected to be configured with aperiodic SRS and PUCCH formats 0 or 2 with aperiodic CSI report in the same symbol.

In case of intra-band carrier aggregation, a UE is not expected to be configured with SRS and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats 1, 3 or 4 in the same symbol.

In case of intra-band carrier aggregation, AaUE shall not transmit simultaneously SRS resource(s) and PRACH.

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to ‘antennaSwitching,’ and a guard period of Y symbols is configured according to Subclause 6.2.1.2, the UE shall use the same priority rules as defined above during the guard period as if SRS was configured.

----------------------------------------------------End of TP ------------------------------------------------------------

----------------------------------------------------Start of TP -----------------------------------------------------------

6.2.1.3	UE sounding procedure between component carriers

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI and/or PRACH happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE’s indicated uplink carrier aggregation capability included in the SRS_capability [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit a periodic/semi-persistent  SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE’s indicated uplink carrier aggregation capability included in the SRS_capability [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUCCH/PUSCH transmission carrying periodic CSI comprising only CQI/PMI, and/or SRS transmission on another serving cell configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) on the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE’s indicated uplink carrier aggregation capability included in the SRS_capability [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) on the carrier of the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE’s indicated uplink carrier aggregation capability included in the SRS_capability [13, TS 38.306].

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to ‘typeA’, and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers, where the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to ‘antenna switching’ and higher layer parameter resourceType in SRS-ResourceSet set to ‘aperiodic’.

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to ‘typeB’ without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells with aperiodic SRS triggered in the DCI, and the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to ‘antenna switching’ and higher layer parameter resourceType in SRS-ResourceSet set to ‘aperiodic’.

A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL), the UE temporarily suspends the uplink transmission on carrier c2.

If the UE is not configured for PUSCH/PUCCH transmission on carrier c1 with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on carrier c1 and serving cell c2, the UE is not expected to be configured or indicated with SRS resource(s) such that SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) would collide with the REs corresponding to the SS/PBCH blocks configured for the UE or the slots belonging to a control resource set indicated by [SystemInformationBlockType0] or [SystemInformationBlockType1] on serving cell c2.

For n-th (n ≥ 1) aperiodic SRS transmission on a cell c, upon detection of a positive SRS request on a grant, the UE shall commence this SRS transmission on the configured symbol and slot provided

-	it is no earlier than the summation of

-	the maximum time duration between the two durations spanned by N OFDM symbols of the numerology of cell c and the cell carrying the grant respectively, and

-	the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL,

-	it does not collide with any previous SRS transmissions, or interruption due to UL or DL RF retuning time.

Otherwise, n-th SRS transmission is dropped, where N is the reported capability as the minimum time interval in unit of symbols, between the DCI triggering and aperiodic SRS transmission.

In case of inter-band carrier aggregation, a UE can simultaneously transmit SRS and PUCCH/PUSCH across component carriers in different bands subject to the UE’s capability.

In case of inter-band carrier aggregation, a UE can simultaneously transmit PRACH and SRS/PUCCH/PUSCH across component carriers in different bands subject to UE’s capability.

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for 6.2.1 in TS38.214

----------------------------------------------------Start of TP -----------------------------------------------------------

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying a location within the last 6 symbols of the slot.

When PUSCH and SRS are transmitted in the same slot, the UE can only be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS.

----------------------------------------------------End of TP ------------------------------------------------------------

Agreement

Text proposal for 6.2.1 in TS38.214

----------------------------------------------------Start of TP ------------------------------------------------------------

/************************ Omitted part **************************/

For PUCCH formats 0 and 2, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s) or if aperiodic SRS is configured and PUCCH consists of beam failure request. A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with semi-persistent or periodic PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only.

In case of intra-band carrier aggregation, a UE is not expected to be configured with SRS and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats 1, 3 or 4 in the same symbol.

In case of intra-band carrier aggregation, a UE shall not transmit simultaneously SRS resource(s) and PRACH.

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to ‘antennaSwitching,’ and a guard period of Y symbols is configured according to Subclause 6.2.1.2, the UE shall use the same priority rules as defined above during the guard period as if SRS was configured.

----------------------------------------------------End of TP ------------------------------------------------------------

R1-1809757	Summary #1 for TRS	MediaTek

Agreement

Text proposal for TS38.214

----------------------------------------------------Start of TP ------------------------------------------------------------

5.1.6.1.1	CSI-RS for tracking

----------------omitted part-----------------

For a NZP-CSI-RS-ResourceSet configured with the higher layer parameter trs-Info, the UE shall assume the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same. For frequency range 1, the UE may be configured with one or more NZP-CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. For frequency range 2 the UE may be configured with one or more NZP-CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic CSI-RS resources in one slot or with a NZP-CSI-RS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot.

A UE configured with NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-Info may have the CSI-RS resources configured as:

-	Periodic, with the CSI-RS resources in the NZP-CSI-RS-ResourceSet configured with same periodicity, bandwidth and subcarrier location

-	Periodic CSI-RS resource in one set and aperiodic CSI-RS resources in a second set, with the aperiodic CSI-RS and periodic CSI-RS resource having the same bandwidth (with same RB location) and the aperiodic CSI-RS being ‘QCL-Type-A’ and ‘QCL-TypeD’, where applicable, with the periodic CSI-RS resources. The UE does not expect that the scheduling offset between the triggering DCI and the first symbol of the aperiodic CSI-RS resources is smaller than the UE reported ThresholdSched-Offset. The UE shall expect that the periodic CSI-RS resource set and aperiodic CSI-RS resource set are configured with the same number of CSI-RS resources. For the aperiodic CSI-RS resource set if triggered, and if the associated periodic CSI-RS resource set is configured with four periodic CSI-RS resources with two consecutive slots with two periodic CSI-RS resources in each slot, the higher layer parameter aperiodicTriggeringOffset indicates the triggering offset for the first slot for the first two CSI-RS resources in the set.

----------------------------------------------------End of TP -------------------------------------------------------------

Conclusion

No consensus on the support of semi-persistent TRS in Rel-15

Agreement

Text proposal for TS38.214

----------------------------------------------------Start of TP ------------------------------------------------------------

5.1.6.1.1	CSI-RS for tracking

< Unchanged parts are omitted >

Each CSI-RS resource, defined in Subclause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter NZP-CSI-RS-Resource with the following restrictions:

-	the periodicity and slot offset, as given by the higher layer parameter periodicityAndOffset configured by NZP-CSI-RS-Resource, is one of slots where 10, 20, 40, or 80 and where µ is defined in Subclause 4.3 of [4, TS 38.211].

----------------------------------------------------End of TP -------------------------------------------------------------

Agreement

Text proposal for TS38.214

----------------------------------------------------Start of TP ------------------------------------------------------------

5.1.6.1.1	CSI-RS for tracking

Periodic CSI-RS resource in one set and aperiodic CSI-RS resources in a second set, with the aperiodic CSI-RS and periodic CSI-RS resource having the same bandwidth (with same RB location) and the aperiodic CSI-RS being ‘QCL-Type-A’ and ‘QCL-TypeD’, where applicable, with the periodic CSI-RS resources. For frequency range 2, The UE does not expect that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is smaller than the UE reported ThresholdSched-Offset

----------------------------------------------------End of TP -------------------------------------------------------------

R1-1809758	Feature lead summary on QCL	Nokia

Agreement

The following text proposal is agreed

----------------------------------------------------Start of TP ------------------------------------------------------------

---- Unchanged parts omitted, section 5.1.5 of 38.214----

For a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with the same CSI-RS resource, or

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with an SS/PBCH block, or

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'QCL-TypeB' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info when 'QCL-TypeD' is not applicable.

---- Unchanged parts omitted, section 5.1.5 of 38.214----

----------------------------------------------------End of TP -------------------------------------------------------------

Agreement

TP in Section 5.1.5 of 38.214 for the determination of QCL assumption before RRC configuration

----------------------------------------------------Start of TP ------------------------------------------------------------

5.1.5	Antenna ports quasi co-location

< Unchanged parts are omitted >

For both the cases when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold Threshold-Sched-Offset, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are configured for the UE. If none of configured TCI states contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

----------------------------------------------------End of TP -------------------------------------------------------------

Agreement

Text proposal for TS38.214

----------------------------------------------------Start of TP ------------------------------------------------------------

5.2.1.4.3	L1-RSRP Reporting

For L1-RSRP computation

-	the UE may be configured with CSI-RS resources, SS/PBCH Block resources or both CSI-RS and SS/PBCH block resources, when resource-wise quasi co-located with 'QCL-Type A C' and 'QCL-TypeD' when applicable.

----------------------------------------------------End of TP -------------------------------------------------------------

R1-1809842	Remaining issues on CSI-RS #2	Huawei, HiSilicon

Agreement

Text proposal for Section 5.2.2.3.1 in 38.214:

< Start of text proposal > repetition in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and defines whether UE can assume the CSI-RS resources within the NZP-CSI-RS ResourceSet are transmitted with the same downlink spatial domain transmission filter or not as described in Subclause 5.1.6.1.2. and can be configured only when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'cri-RSRP' or 'none'. - qcl-InfoPeriodicCSI-RS contains a reference to a TCI-State indicating QCL source RS(s) and QCL type(s). If the TCI-State is configured with a reference to an RS with 'QCL-TypeD' association, that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic located in the same or different CC/DL BWP. - trs-Info in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and for which the UE can assume that the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same as described in Subclause 5.1.6.1.1 and can be configured when reporting setting is not configured or when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'none'. < End of text proposal >

Agreement

- For multi-slot and single-slot PDSCH scheduling, the triggered aperiodic ZP CSI-RS is applied to all the slot(s) of the scheduled PDSCH.

Agreement

- UE does not perform rate matching nor assumes puncturing on PDSCH REs overlapped with the REs for the triggered aperiodic NZP CSI-RS resource

- UE does not rate match PDSCH around the overlapped aperiodic ZP-CSI-RS resource scheduled by a DL DCI other than the one which scheduled this PDSCH

For further discussion until the next RAN1 meeting:

UE is not expected to be configured with NZP-CSI-RS resource(s) without functionality

- FFS on whether specification change is needed

For reference the following is provided for consideration until the next RAN1 meeting

A configured NZP-CSIRS resource falls under at least one of the cases below:

1) a NZP-CSI-RS resource is included in a configured NZP-CSI-RS resource set with a configured field “repetition” or “trs-info”

2) a NZP-CSI-RS resource is included in a configured NZP-CSI-RS resource set which is included by a configured CSI-ResourceConfig included in a configured CSI-ReportConfig

3) a NZP-CSI-RS resource is included in a configured “RadioLinkMonitoringRS” IE

4) a NZP-CSI-RS resource is included in a configured PUSCH-PathlossReferenceRS

5) a NZP-CSI-RS resource is included in a configured PUCCH-PathlossReferenceRS

6) a NZP-CSI-RS resource is included in a configured SRS-ResourceSet

7) a NZP-CSI-RS resource is included in a configured "SRS-SpatialRelationInfo"

R1-1809891	Summary of DMRS Issues	Qualcomm

Agreement

DMRS of the PDSCH cannot be outside the scheduled OFDM symbols of the PDSCH

Agreement

TP: 38.211, v 15.2.0 Section 7.4.1.1.2

----------------------------------------------------Start of TP ------------------------------------------------------------

For PDSCH mapping type B

-	if the PDSCH duration is 2, 4, or 7 OFDM symbols for normal cyclic prefix or 2, 4, 6 OFDM symbols for extended cyclic prefix, and the PDSCH allocation collides with resources reserved for a CORESET,  shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET and

-	if the PDSCH duration is 2 symbols, the UE is not expected to receive a DM-RS symbol beyond the second symbol,

-	if the PDSCH duration is 4 symbols, the UE is not expected to receive a DM-RS symbol beyond the third symbol,

-	if the PDSCH duration is 7 symbols for normal cyclic prefix or 6 symbols for extended cyclic prefix,

-	the UE is not expected to receive the first DM-RS beyond the fourth symbol, and

-	if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted.

-	if the PDSCH duration is 2 or 4 OFDM symbols, only single-symbol DM-RS is supported.

----------------------------------------------------End of TP ------------------------------------------------------------

R1-1809897	Feature lead summary 2 of PT-RS	Ericsson

Agreement

TP for 38.214

----------------------------------------------------Start of TP ------------------------------------------------------------

6.2.3.1	UE PT-RS transmission procedure when transform precoding is not enabled

<Unrelated part omitted>

For PT-RS, the transmit power of PT-RS is derived from , which is the power ratio between power of PUSCH and power of PT-RS per port.

When the UE is scheduled with Qp={1,2} PT-RS port(s) in uplink and the number of scheduled layers is ,

-	If the UE is configured with higher layer parameter ptrs-Power, the PUSCH to PT-RS power ratio per layer per RE  is given by , where  is shown in the Table 6.2.3.1-3 according to the higher layer parameter ptrs-Power, the PT-RS scaling factor  specified in subclause 6.4.1.2.2.1 of [4, TS 38.211] is given by and also on the Precoding Information and Number of Layers TPMI field in DCI.

-	The UE shall assume ptrs-Power in PTRS-UplinkConfig is set to state "00" in Table 6.2.3.1-3 if not configured or in case of non-codebook based PUSCH.

----------------------------------------------------End of TP -------------------------------------------------------------

R1-1809814	Summary of SRS	Sony

Agreement

- For SRS for CB PUSCH and antenna switching on FR1, the minimal time internal between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 symbols

  - Support of zero slot offset for aperiodic SRS triggering is UE capability

  - The time interval in units of OFDM symbols is counted based on the minimum subcarrier spacing between PDCCH and SRS

Above agreement does not change the switching time for SUL/UL

Above agreement only affects UE capability signalling from RAN2 perspective

Agreement

SRS dropping rule for same carrier

Aperiodic SRS resources > Semi-persistent SRS resources > Periodic SRS

- FFS: Whether Dropped SRS resource shall be at resource level or symbol level

Agreement

----------------------------------------------------Start of TP for 38.214--------------------------------------------------

-    If the UE receives the DCI triggering aperiodic SRS in slot n, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot  where k is configured via higher layer parameter slotoffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission,  and  are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively.

-	if the UE is configured with the higher layer parameter spatialRelationInfo containing the ID of a reference 'ssb-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo contains the ID of a reference 'csi-RS-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, or of the latest reference aperiodic CSI-RS. If the higher layer parameter spatialRelationInfo contains the ID of a reference 'srs', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS or of the reference aperiodic SRS.

----------------------------------------------------End of TP for 38.214---------------------------------------------------

Agreement

Send LS to RAN2 and RAN4:

- From UE implementation perspective, RAN1 recommends RF retuning time(s) should be per band per band combination to signal retuning/interruption time from the band to other bands in the band combination.

- To avoid performance loss, a single value (the maximum among all band combinations) for all band combinations is not recommended.

R1-1809931	[Draft] reply LS on UE power class and UE capability on SRS carrier switch	Huawei

Decision: The draft LS is endorsed and final LS is approved in R1-1809889.

R1-1808144	Remaining issues on reference signals and QCL	Huawei, HiSilicon

R1-1808197	Maintenance for Reference signals and QCL	ZTE

R1-1808222	Remaining issues on reference signals and QCL	vivo

R1-1808312	Corrections on SRS antenna ports	Fujitsu

R1-1808376	Issues on reference signals	CATT

R1-1808488	Text proposals on reference signals	LG Electronics

R1-1808670	Corrections to reference signals and QCL	Intel Corporation

R1-1808751	Corrections on reference signals and QCL	Samsung

R1-1808801	Remaining issues on Reference signals	Spreadtrum Communications

R1-1808831	Discussion on remaining issues on SRS	CMCC

R1-1808881	Text proposals for RS and QCL	Guangdong OPPO Mobile Telecom

R1-1809054	Text Proposal on Reference Signal	AT&T

Withdrawn

R1-1809107	Remaining issues on SRS	Sharp

R1-1809139	Maintenance for Reference signals and QCL	NTT DOCOMO, INC.

R1-1809198	Maintenance for RS and QCL	Ericsson

R1-1809238	Remaining issues on reference signals and QCL	Nokia, Nokia Shanghai Bell

R1-1809504	Maintenance for RS	Qualcomm Incorporated	(rev of R1-1809424)

#### Other

Including handling simultaneous transmission and reception of different channels and reference signals

R1-1809820	Summary of simultaneous Rx and Tx	Intel

Agreement

RAN1 expects that UE behavior for receiving the following combinations will be clarified in RAN4

- SSB-RRM/SSB-RLM/SSB-BFD/SSB-L1-RSRP in combination with PDSCH/PDCCH

- (CSI-RS RLM, CSI-RS RRM, CSIRS-BFD) in combination with (SSB SSB-RRM/SSB-RLM/SSB-BFD/SSB-L1-RSRP)

- (CSIRS-RRM, CSIRS-L1-RSRP-Rep-OFF) in combination with PDSCH

- Immediate RAN1 actions are not needed. RAN1 may discuss if needed following RAN4 decisions.

Send an LS to RAN4 (Bishwarup to draft LS)

Agreement

Send an LS to RAN4 with the following questions:

Question #1:

RAN1 requests RAN4 to clarify UE behaviour, if necessary, when a channel or RS in group 1 overlaps with a channel or RS in group 2 on the same OFDM symbol in the same serving cell or in different serving cells in the case of CA in FR2.

| Case | Channels/RS in group 1 | Channels/RS in group 2 |
| --- | --- | --- |
| 1 | SSB for L3 measurements, SSB for RLM, SSB for BFD, SSB for L1-RSRP measurements | CSI-RS for RLM, CSI-RS for L3 measurements, CSIRS for BFD, CSI-RS for L1-RSRP |
| 2 | CSIRS for L3 measurements, CSIRS with repetition=OFF for L1-RSRP measurements | PDSCH |
| 3 | CSIRS for L3 measurements, CSIRS with repetition=OFF for L1-RSRP measurements | PDCCH |
| 4 | CSI-RS for RLM, CSI-RS for L3 measurements, CSIRS for BFD, CSI-RS for L1-RSRP measurements | CSI-RS for RLM, CSI-RS for L3 measurements, CSIRS for BFD, CSI-RS for L1-RSRP measurements |

Question #2:

If scheduling restriction due to Rx beamforming is applied on certain symbols, is the NW allowed to schedule PDSCH that spans a time duration containing these symbols but with configured rate matching resources used for protecting the symbols such that the UE does not receive PDSCH on symbols with scheduling restriction applied?

R1-1809881	draft LS on UE behaviour on reception of channels or RS in the same OFDM symbol	Intel

Decision: The draft LS is endorsed and final LS is approved in R1-1809890.

R1-1809882	Guidance on simultaneous reception of channels or RS	Intel

Companies are encouraged to check the tdoc for discussions on simultaneous TX/RX in future RAN1 meetings

R1-1808142	Remaining issues on simultaneous reception or transmission over CCs and BWPs	Huawei, HiSilicon

R1-1808198	Simultaneous transmission and reception of channels/signals	ZTE

R1-1808199	Enhancements on multi-beam operation	ZTE

R1-1808201	Enhancements on Type II CSI feedback for MU-MIMO	ZTE

R1-1808202	Enhancements on multi-TRP transmission/reception	ZTE

R1-1808223	Remaining issues on simultaneous reception of DL/UL physical channels and RSs	vivo

R1-1808265	Discussions on simultaneous reception and transmission across CCs	MediaTek Inc.

R1-1808377	Discussion on simultaneous transmission and reception of different channels/signals	CATT

R1-1808489	Remaining issues on simultaneous Tx/Rx	LG Electronics

R1-1808671	Simultaneous Tx and Rx of channels and RS	Intel Corporation

R1-1808752	Simultaneous transmission and reception of physical channels and reference signals	Samsung

R1-1808832	Discussion on beam failure recovery	CMCC

R1-1809140	Simultaneous Tx/Rx of UL/DL physical channels and reference signals	NTT DOCOMO, INC.

R1-1809199	On simultaneous transmission and reception in FR2	Ericsson

R1-1809239	Correction on UL Antenna Port Indexing	Nokia, Nokia Shanghai Bell

R1-1809240	On simultaneous transmission and reception of multiplexed channels	Nokia, Nokia Shanghai Bell

R1-1809425	Details on  simultaneous reception of PHY channels and RS in FR2	Qualcomm Incorporated

### Maintenance for Scheduling/HARQ aspects

#### Maintenance for physical downlink control channel

Including PDCCH structure, search space, GC-PDCCH, DCI contents and formats, etc.

R1-1809766	Offline summary for PDCCH structure and search space	NTT DOCOMO

Decision: The document is noted.

Conclusion:

- It is understood that the parameter “duration” in the search space configuration is applicable to any CSS configurations.

Agreements:

- PDCCH BD/CCE counting is only based on the configured PDCCH decoding candidates (i.e., irrespective of whether or not a PDCCH decoding candidate is dropped, e.g., due to collision with other channels/signals)

  - Check further offline on potential spec impact

Proposals:

- Send an LS to RAN2 regarding:

  - Whether RAN1’s understanding of the limitation of RAN2 signalling of # of decoding candidates configured for the cross-carrier scheduled carriers is correct or not

    - If so, indicate RAN1’s original intention and the limitations of the current RAN2 signalling (to give some example(s)), and ask RAN2 for possible update of the signalling to reflect RAN1’s intention, if feasible (in terms of ASN.1 impact)

      - If not feasible, RAN1 will discuss further on how to handle

- Also discuss further offline same vs. different numerologies

Wednesday session

R1-1809855	Offline summary for PDCCH structure and search space	NTT DOCOMO

Decision: The document is noted.

Conclusion:

- No new RRC signaling for TCI-state configuration for the CORESET#0 is introduced.

- No new MAC-CE for TCI-state indication for the CORESET#0 is introduced.

- FFS: Whether the existing RRC configuration and/or existing MAC-CE field can be used to configure/indicate dedicated TCI-state or SSB index for the CORESET#0

- FFS: Whether/how to support BFR for the CORESET#0

Agreements:

- From RAN1 perspective, the parameters monitoringPeriodicity and nrofPDCCH-Candidates under dci-Format2-3 in the RRC IE SearchSpace have no use. It is up to RAN2 to delete or disable these two parameters

  - Note: No RAN1 spec impact is expected

- Send LS to RAN2 about the agreements – R1-1809856

R1-1809856	Draft LS on DCI format 2_3	NTT DOCOMO

Friday decision: The draft LS is endorsed and final LS is approved in R1-1809966.

Agreements: RAN1#93 agreements are updated as follows (to cover Case 5):

- For self-scheduling with same numerology or for cross-carrier scheduling with the same numerology for all the DL serving cells, and the number of DL-CCs is more than 4 and with up to T DL-CCs where the UE reports BD capability of y < T, the limit of BDs/CCEs per CC per slot is

  - The total number of BDs/CCEs across CCs is based on UE BD capability. It can be split across CCs, subject to the non-CA limit on each CC.

  - For SCell, NW ensures no overbooking based on non-CA case occurs. For cross-carrier scheduling, BDs/CCEs overlapped across DL serving cells are independently counted (i.e., counted per serving cell).

Proposals:

- For a UE monitors multiple search spaces associated with different CORESETs,

  - if the monitoring occasions of the search spaces are overlapped in time [or are separated less than Threshold-Sched-Offset in time], the UE may assume that the DM-RS of PDCCHs in the search spaces are quasi co-located with the RS(s) in the TCI state with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the lowest CORESET-ID among the CORESETs associated with the search spaces.

Agreement:

- Adopt following TP for TS 38.213 Section 10.

===== start of TP =====

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG except for PDCCH monitoring for Type0/0A/2-PDCCH common search spaces, in which case the UE is not required to apply the procedures in this clause for SCG.

===== end of TP =====

Agreements:

- Adopt following TP for TS38.211 Subclause 7.4.1.3.2.

===== start of TP =====

The reference point for  is

-	subcarrier 0 of the lowest-numbered resource block in the CORESET if the CORESET is configured by the PBCH or by the controlResourceSetZero in the PDCCH-ConfigCommon SIB1,

-	subcarrier 0 in common resource block 0 otherwise

===== end of TP =====

Agreements:

- To adopt the following TP to 38.213:

===== start of TP =====

10	UE procedure for receiving control information

[…]

If consistent control information is not detected in a PDCCH, the PDCCH is discarded.

[…]

===== end of TP =====

Agreement:

- The TP at the end of Section 3.5 in R1-1809855 is endorsed

Conclusion:

- At least for self-scheduling and for cross-carrier scheduling with the same numerology for all the DL serving cells, total number of CCEs or BDs corresponding to the remaining PDCCH candidates after PDCCH candidates are dropped based on the non-CA limit for the PCell (PSCell) and the configured PDCCH candidates for SCells is guaranteed by network to be no more than the CA limit.

Agreements:

- To adopt the following TP to TS 38.213 Section 10 (subject to Edtior’s refining of wording):

===== start of TP =====

If the higher layer parameter monitoringSymbolsWithinSlot indicates to a UE only one PDCCH monitoring occasion within a slot to monitor PDCCH of all search space sets on a span of up to 3 consecutive OFDM symbols at the same position in every slot, the UE does not expect to be configured with a PDCCH subcarrier spacing other than 15 kHz for the corresponding search space set  if the control resource set  associated with the search space if the span includes at least one symbol after the third symbol of the in any slot.

===== end of TP =====

Proposals:

- Adopt following TP for TS 38.213 Section 10.

If a UE is provided one or more search space sets by corresponding one or more higher layer parameters searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and the UE is provided with a C-RNTI or a CS-RNTI, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with the C-RNTI or the CS-RNTI in the one or more search space sets whenever a DCI format associated with SI-RNTI, RA-RNTI, or P-RNTI, is monitored.

Friday session

Agreements:

- Send LS to RAN2:

  - RAN1 is discussing how the UE knows the number of PDCCH candidates for each scheduled cell in case of cross-carrier scheduling. RAN1’s understanding is that:

    - Without ASN.1 signaling change with possible field description updates:

      - Search space with ID#s configuration configured on a BWP of a scheduled cell is used to determine the number of PDCCH candidate(s) for each aggregation level monitored in a search space with ID#s of a BWP of the scheduling cell for the BWP of the scheduled cell

    - Otherwise, RAN1 would like to know whether it is possible to realize previous RAN1 conclusion by modifying the ASN.1, e.g., update of crossCarrierSchedulingConfig, PDCCH-Config, SearchSpace, etc.

      - If not possible, RAN1 will discuss further on how to handle.

- In the LS, will include the previous RAN1 conclusion accordingly.

R1-1809968	Draft LS on cross-carrier scheduling	NTT DOCOMO

Friday decision: The draft LS is endorsed and final LS is approved in R1-1810009.

Conclusion:

- The following is RAN1’s common understanding (no spec impact):

  - For initial DL BWP of the PCell, PDCCH-ConfigCommon is provided by SIB1, and for other DL BWPs, PDCCH-ConfigCommon is provided by dedicated RRC signaling. For a SCell, PDCCH-ConfigCommon for any DL BWP is provided by dedicated RRC signalling. Then, PDCCH-ConfigCommon other than the initial DL BWP can be different from the PDCCH-ConfigCommon provided by SIB1. However, PDCCH-ConfigCommon has a description “The IE PDCCH-ConfigCommon is used to configure cell specific PDCCH parameters provided in SIB as well as during handover and PSCell/SCell addition”. RAN1 view is that the PDCCH-ConfigCommon can be provided by dedicated RRC signalling also for each DL BWP of any cell including the PCell.

  - searchSpaceSIB1, searchSpaceOtherSystemInformation, paging-SeachSpace, and ra-SeachSpace can be associated with controlResourceSetZero or commonControlResourceSet in the PDCCH-ConfigCommon, or can be associated with controlResourceSet configured in the PDCCH-Config. However, the current description of commonControlResourceSet does not allow association to the searchSpaceSIB1, searchSpaceOtherSystemInformation, and paging-SeachSpace. Moreover, it is not clear whether searchSpaceSIB1, searchSpaceOtherSystemInformation, paging-SeachSpace, and ra-SeachSpace can be associate with controlResourceSet in the PDCCH-Config. RAN1’s understanding is that these are possible.

  - After RRC connection setup, UE monitors common search space(s) based on the search space configuration(s) provided by PDCCH-ConfigCommon (which are Type0/0A/1/2-PDCCH common search spaces) and by PDCCH-Config (which is Type3 PDCCH-common search space).

  - If a UE is configured to monitor DCI format 0_0/1_0 in a CSS in the SearchSpace of the PDCCH-Config on the PCell, the UE monitors all the RNTIs that UE is configured to monitor, based on the SearchSpace configuration.

Send an LS to RAN2 regarding the first and the second sub-bullets above.

R1-1809969	Draft LS on higher layer configuration for search space monitoring	NTT DOCOMO

Friday decision: The draft LS is endorsed with the update “review if the above”, and final LS is approved in R1-1810010.

R1-1809970 	Draft reply LS on PDCCH common search space configurations	NTT DOCOMO

Friday decision: The draft LS is endorsed (by accepting all change marks), and final LS is approved in R1-1809971.

Agreements:

- To adopt the following TP to Section 11.1 of 38.213:

===== start of TP =====

-	If the UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS in the set of symbols of the slot, the UE receives the PDCCH, the PDSCH, or the CSI-RS if

-	the UE does not detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot, or

Otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS in the set of symbols of the slot.

-	If the UE is configured by higher layers to transmit a periodic SRS, or a PUCCH, or a PUSCH, or a PRACH in the set of symbols in the slot, the UE transmits the periodic SRS, or the PUCCH, or the PUSCH, or the PRACH in a symbol in the set of symbols of the slot if

-	the UE does not detect a DCI format 1_0, DCI format 1_1, or DCI format 0_1 that indicates to the UE to receive PDSCH or CSI-RS in the set of symbols in the slot, or

-	the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 that indicates to the UE to receive PDSCH or CSI-RS in at least one symbol in the set of symbols in the slot and a number of symbols between a last symbol of a control resource set where the UE detects the DCI format 1_0, DCI format 1_1, or DCI format 0_1 and the symbol is smaller than the PUSCH preparation time N2 for the corresponding PUSCH timing capability.

Otherwise, the UE does not transmit the periodic SRS, or the PUCCH, or the PUSCH, or the PRACH in the symbol.

===== end of TP =====

R1-1809769	Summary of GC-PDCCH carrying SFI	Qualcomm

Decision: The document is noted.

Agreement:

- In 38.213 Section 11.1.1, change log2(maxSFIindex) to  max⁡(log2maxSFIindex+1,1) when computing the SFI field length.

Conclusion:

- On search space configuration of DCI format 2_0, currently 38.331 says

dci-Format2-0							SEQUENCE {

nrofCandidates-SFI						SEQUENCE {

aggregationLevel1						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel2						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel4						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel8						ENUMERATED {n1, n2}											OPTIONAL,	-- Need R

aggregationLevel16						ENUMERATED {n1, n2}											OPTIONAL	-- Need R

},

...

}

- RAN1 agreement is only support one AL. The 38.331 seems to allow multiple, which conflicts with the RAN1 agreement

- Send LS to RAN2 to update nrofCandidates-SFI accordingly. Draft LS in R1-1809761.

R1-1809761	[draft] LS on search space configuration for DCI format 2_0	Qualcomm

Decision: The draft LS is endorsed (by updating “change the 38.331”). Final LS is approved in R1-1809774.

Wednesday session

R1-1809835	Summary of contributions for GC-PDCCH carrying SFI	Qualcomm

Decision: The document is noted.

Agreements: Update the #92bis agreement as follows:

For cancellation of RRC configured transmission or reception by SFI or DCI, the cancellation is for a unit of transmission/reception if any OFDM symbol within the unit is cancelled by SFI.

- For RRC configured CSI-RS resource set, the cancellation unit is the CSI-RS resource set

- For RRC configured PDSCH and PUSCH with slot aggregation, the cancellation unit is the whole PDSCH or PUSCH within a slot

- For RRC configured PDSCH, PUCCH, and PUSCH without slot aggregation, the cancellation unit is the whole PDSCH, PUCCH, and PUSCH

- For RRC configured SRS transmission, the cancellation unit is OFDM symbol

Agreements:

- To adopt the following TP:

======================= start of Text Proposal in Section 11.1 in TS 38.213=======================

[…]

A UE expects .

A value  msec is valid only for . A value  msec is valid only for  or . A value  msec is valid only for , or , or .

A slot configuration period of P+P2  msec includes first  slots and second S2=P2∙2μref,2 slots. From the  slots, a first  slots include only downlink symbols and a last  include only uplink symbols. The  symbols after the first  slots are downlink symbols. The  symbols before the last  slots are uplink symbols. The remaining  are flexible symbols.

[…]

For unpaired spectrum operation for a UE on a serving cell, the UE is provided by higher layer parameter subcarrierSpacing a reference subcarrier spacing configuration of  for each slot format in a combination of slot formats indicated by a SFI-index field value in DCI format 2_0. The UE expects that for a reference subcarrier spacing configuration of  and for an active DL BWP or an active UL BWP with subcarrier spacing configuration of , it is . Each slot format in the combination of slot formats indicated by the SFI-index field value in DCI format 2_0 is applicable to  consecutive slots in the active DL BWP or the active UL BWP where the first slot starts at a same time as a first slot for the reference subcarrier spacing configuration of  and each downlink or flexible or uplink symbol for the reference subcarrier spacing configuration of  corresponds to  consecutive downlink or flexible or uplink symbols for the subcarrier spacing configuration .

[…]

If a UE is configured by higher layers to transmit periodic SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in at least one symbol in the set of symbols, then

-	the UE does not expect to cancel the transmission in symbols from the subset of symbols that occur, relative to a last symbol of a control resource set where the UE detects the DCI format 2_0, or the DCI format 1_0, or the DCI format 1_1, or the DCI format 0_1 after a number of symbols that is smaller than the PUSCH preparation time N2 for the corresponding PUSCH timing capability [6, TS 38.214];

-	the UE cancels the PUCCH, or PUSCH, or PRACH transmission in the remaining symbols of the slot and cancels the periodic SRS transmission in the subset of symbols of the slot.

A PUSCH preparation time N2 throughout Subclause 11.1.1 is defined with respect to the subcarrier spacing of the PDCCH providing DCI format 2_0.

======================= end of Text Proposal in Section 11.1 in TS 38.213=======================

Agreements:

- To adopt the following TP:

======================= Start of Text Proposal in Section 11.1 in TS 38.213 ======================

< --------------------------------------omitted--------------------------------------------------- >

For a set of symbols of a slot, a UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols in the slot as downlink and to detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 , or UL grant carried by RAR, indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS, in the set of symbols of the slot.

< --------------------------------------omitted------------------------------------------------------>

For a set of symbols of a slot indicated to a UE as flexible by higher layer parameters tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationCommon2, and tdd-UL-DL-ConfigDedicated, when provided to the UE, or when higher layer parameters tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationCommon2, and tdd-UL-DL-ConfigDedicated are not provided to the UE, and if the UE detects a DCI format2_0 providing a format for the slot using a slot format value other than 255

< ----------------------------------omitted--------------------------------------------------------- >

-     If an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible and the UE detects a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3  , or UL grant carried by RAR, indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot the UE transmits the PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot.

-     If an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible, and the UE does not detect a DCI format 1_0, DCI format 1_1, or DCI format 0_1indicating to the UE to receive PDSCH or CSI-RS, or the UE does not detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 , or UL grant carried by RAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot, the UE does not transmit or receive in the set of symbols of the slot.

< ---------------------------------------omitted--------------------------------------------------------- >

-     A UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as downlink and also detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 , or UL grant carried by RAR indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH, in one or more symbols from the set of symbols of the slot.

======================= end of Text Proposal in Section 11.1 in TS 38.213 ======================

R1-1809701	Summary of 7.1.3.1 (DCI contents and formats) 	Ericsson

Decision: The document is noted.

Agreement:

- To adopt the following TP for DCI format 1-1 in 38.212:

===== start of TP =====

VRB-to-PRB mapping – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if interleaved VRB-to-PRB mapping is not configured by higher layers;

-	1 bit according to Table 7.3.1.1.2-33 otherwise, only applicable to resource allocation type 1, as defined in Subclause 7.3.1.6  of [4, TS 38.211].

===== end of TP =====

Agreements:

- In TS 38.212:

  - Replace “transformPrecoder = enabled” by “transform precoder is enabled”

  - Replace “transformPrecoder = disabled” by “transform precoder is disabled”

Agreement:

- In 38.212, change the heading of Table 7.3.1.2.1-1 from “PUSCH frequency hopping” to “Short Message indicator”

Agreements:

- To adopt the following TP in Section 6.1.2.2.2 of 38.214

===== start of TP =====

In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active carrier bandwidth part of size  PRBs except for the case when DCI format 0_0 is decoded in any common search space in which case the initial bandwidth part of size  shall be used.

===== end of TP =====

Tuesday session

R1-1809797	[DRAFT] Response LS on additional TDD configuration periodicities	Nokia, Nokia Shanghai Bell

Decision: The draft LS is endorsed and final LS is approved in R1-1809812.

Friday session

R1-1809983	On offline discussion on DCI	Ericsson

Decision: The document is noted.

Agreement:

- Capture the following conclusion in 38.213 (with the clarification that “DCI” refers to “consistent DCI”)

  - 7) Processing no more than one DCI with each RNTI in each of Type 0 CSS, Type 0A CSS, Type 1 CSS, Type 2 CSS, Type 3 CSS excluding unicast DCI per slot

R1-1808069	Remaining issues on DL control	Huawei, HiSilicon

R1-1809490	Remaining issues on NR DL Control	ZTE	(revision of R1-1808206)

R1-1808224	Remaining issues on PDCCH	vivo

R1-1808262	Remaining issues in physical downlink control channel	MediaTek Inc.

R1-1808378	Corrections to NR PDCCH	CATT

R1-1808490	Remaining issues on downlink control channel	LG Electronics

R1-1808614	NR TDD Configurations for LTE-NR co-existence	Apple Inc.

R1-1808672	Outstanding aspects on NR PDCCH	Intel Corporation

Withdrawn

R1-1808753	Remaining Issues on PDCCH and Search Space Design	Samsung

R1-1808796	Remaining issues on PDCCH	Spreadtrum Communications

R1-1808833	Discussion on remaining issues for PDCCH and search space	CMCC

R1-1808869	Maintenance for physical downlink control channel	Panasonic Corporation

R1-1808952	On remaining details of DL control	Nokia, Nokia Shanghai Bell

R1-1809108	Remaining issues on DCI format	Sharp

R1-1809141	Maintenance for physical downlink control channel	NTT DOCOMO, INC.

R1-1809246	Remaining issues on downlink control channel	ASUSTEK COMPUTER (SHANGHAI)

R1-1809405	Maintenance issues of physical downlink control channel	Ericsson

R1-1809426	Maintenance for physical downlink control channel	Qualcomm Incorporated

#### Maintenance for physical uplink control channel

Including short/long PUCCH, UCI multiplexing, resource allocation, etc.

R1-1809751	Summary of Maintenance Issues on Short PUCCH and UCI Multiplexing	Ericsson

Decision: The document is noted.

Agreements:

- To adopt the following TP (Section 6.3.2.2.1 of 38.211)

===== start of TP =====

where  is given by the higher-layer parameter hoppingId, if the higher-layer parameter hoppingId is configured; otherwise, .

-	if pucch-GroupHopping equals 'enable'

where the pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with  where  is given by the higher-layer parameter hoppingId, if the higher-layer parameter hoppingId is configured; otherwise, .

-	if pucch-GroupHopping equals 'disable'

where the pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with cinit=25nID30+nID mod 30 where  is given by the higher-layer parameter hoppingId, if the higher-layer parameter hoppingId is configured; otherwise, ..

===== end of TP =====

Agreements:

- To adopt the following TP (Section 6.3.2.2.2 of 38.211)

===== start of TP =====

where the pseudo-random sequence  is defined by subclause 5.2.1. The pseudo-random sequence generator shall be initialized with , where  is given by the higher-layer parameter hoppingId, if the higher-layer parameter hoppingId is configured; otherwise, ..

===== end of TP =====

Agreement:

- In Subclause 6.4.1.3.2 of TS 38.211, replace  with

R1-1809739	Review Summary for Al 7.1.3.2 related to long PUCCH	Huawei

Decision: The document is noted.

Agreements:

For SP-CSI on configured PUSCH without UL-SCH colliding with a PUSCH with UL-SCH with overlapped symbols in time,

- If the timeline requirement for handling overlapped UCI over PUSCH is satisfied, drop SP-CSI and transmit PUSCH

- Otherwise, it is an error case. It is up to UE implementation to handle the error case.

Agreement:

- UE is not expected to be indicated ‘UL-SCH indicator’ = “0” and ‘CSI request’ = “zero(s)” (not triggered) in DCI format 0-1.

Wednesday

Conclusion (from offline consensus):

- It is clarified that NR Rel-15 supports multiple groups of overlapping PUCCH/PUSCH channels in a slot.

Thursday

R1-1809802	Review Summary for Al 7.1.3.2 related to long PUCCH	Huawei

Decision: The document is noted.

Agreements:

When multi-slot PUCCH with repetition overlap with multi-slot PUCCH with repetition in time within the same PUCCH group

- The UE is not expected to have two overlapped multi-slot PUCCH transmissions with the same starting slot carrying UCI with the same proirity

- For UCI with the same priority, the PUCCH that starts in an earlier slot is transmitted.

- For UCI with different priority, the UCI PUCCH with higher priority is transmitted in overlapping slots if timeline requirement is met.

- Priority is defined as follows: HARQ-ACK > SR > CSI with higher priority >CSI with lower priority

- In overlapping slots, the de-prioritized UCI is dropped without any postponing of the transmission

- There is no impact on the transmission of the de-priortized UCI in the remaining non-overlapping slots

- FFS: multi-slot PUCCH with repetition overlap with single-slot PUCCH in time

Agreements:

- Pseudo-code in 9.2.5 of TS38.213 is applicable to overlapping PUCCH resources semi-statically configured

  - Note: timeline requirement for UCI multiplexing is not applied if none of these PUCCH resources correspond to DL SPS HARQ-ACK

Agreements:

- When a UE is not configured with multi-CSI-PUCCH report,

  - When SR occasion and/or DL SPS HARQ-ACK and two non-overlapping CSI PUCCH resources contained in Resource A and Set X (as defined in RAN1 #93 meeting) decided by the pseudo code (as in 9.2.5 of TS38.213) in a slot

    - DL SPS HARQ-ACK/SR is multiplexed on the CSI PUCCH resource with higher priority, and CSI on the CSI PUCCH resource with lower priority is dropped

      - In case of presence of DL SPS HARQ-ACK, multiplexing occurs if the corresponding timeline requirement is met, otherwise, it is considered as an error case

R1-1809806	Summary of offline discussions on UCI multiplexing	Ericsson

Decision: The document is noted.

Proposals:

- Multiplex DL SPS AN/SR on CSI resource subject to timeline constraints as follows:

  - When HARQ-ACK for SPS PDSCH overlaps with configured PUCCH resource(s) for CSI and/or SR and/or configured PUSCH in a slot, the timeline for UCI multiplexing is defined such that

    - the earliest symbol among these PUCCHs/PUSCH starts no earlier than N1+X symbols after the last symbol of the corresponding SPS-PDSCH(s).

        - X is defined as agreed in RAN1#93.

Agreements:

- When grant-based PUSCH overlaps with configured PUCCH resource(s) for CSI and/or SR in a slot, the timeline requirement for UCI multiplexing on PUCCH or PUSCH is satisfied if the earliest symbol among these PUCCHs/PUSCH starts no earlier than N2+Y symbols after the last symbol of the DCI scheduling PUSCH.

  - Y is defined as agreed in RAN1#93.

Agreements:

- When PUSCH without corresponding DCI overlaps with configured PUCCH resource(s) for CSI and/or SR in a slot,

  - The timeline requirement for UCI multiplexing is not applicable

  - The pseudo-code in 9.2.5 of TS38.213 is applicable to resolve overlapping among PUCCH resources (if any)

  - After resolving overlapping among PUCCH resources (if any), the PUCCH UCI is multiplexed on the PUSCH without corresponding DCI if overlapping occurs.

  - Note: this does not apply to PUSCH as part of the activation

R1-1809927	Summary of PUCCH maintenance	Qualcomm

Decision: The document is noted.

Agreements:

- The TP as in proposal 7 of R1-1809927 is endorsed (38.212)

- The TP as in proposal 8a of R1-1809927 is endorsed (38.212)

- The TP as in Section 2.3.3 of R1-1809927 is endorsed (38.213)

R1-1809827	Offline summary on  maintenance for R15 NR PUCCH resource allocation	OPPO

Decision: The document is noted.

Agreements:

- To adopt the following TP to 9.2.1 of 38.213

===== start of TP =====

If a UE does not have dedicated PUCCH resource configuration, provided by higher layer parameter PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by higher layer parameter pucch-ResourceCommon in SystemInformationBlockType1 through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial active UL BWP of  PRBs provided by SystemInformationBlockType1. The PUCCH resource set is provided by higher layer parameter PUCCH-Resource-Common and includes sixteen resources, each corresponding to a PUCCH format, a first symbol, a duration, a PRB offset , and a cyclic shift index set for a PUCCH transmission. An orthogonal cover code with index 0 is used for a PUCCH resource with PUCCH format 1 in Table 9.2.1-1. The UE transmits a PUCCH using frequency hopping. The UE transmits the PUCCH using the same spatial domain transmission filter as for the Msg3 PUSCH transmission.

===== end of TP =====

Proposals:

- To adopt the following TP to 9.2.1 of 38.213

If the UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a DCI format 1_0 or DCI format 1_1, the UE determines a PUCCH resource with index ,  , as , where  is a number of CCEs in a control resource set of a PDCCH reception conveying the DCI format 1_0 or DCI format 1_1, as described in Subclause 10.1,  is the index of a first CCE for the PDCCH reception, and  is a value of the PUCCH resource indicator field in the DCI format 1_0 or DCI format 1_1.

Agreements:

- To adopt the following TP to 9.2.1 of 38.213

===== start of TP =====

A UE can be configured up to four sets of PUCCH resources by higher layer parameter PUCCH-ResourceSet. A PUCCH resource set is associated with a PUCCH resource set index provided by higher layer parameter pucch-ResourceSetId, with a set of PUCCH resource indexes provided by higher layer parameter resourceList that provides a set of pucch-ResourceId used in the PUCCH resource set, and with a maximum number of UCI information bits the UE can transmit using a PUCCH resource in the PUCCH resource set provided by higher layer parameter maxPayloadMinus1. For the first PUCCH resource set, the maximum number of UCI information bits is fixed to two. A maximum number of PUCCH resource indexes for a set of PUCCH resources is provided by higher layer parameter maxNrofPUCCH-ResourcesPerSet. The maximum number of PUCCH resources in the first PUCCH resource set is 32 and the maximum number of PUCCH resources in the other sets of PUCCH resources is 8.

===== end of TP =====

Agreements:

- To adopt the following TP to 9.2.1 of 38.213

===== start of TP =====

-	the UE determines the PRB index of the PUCCH transmission in the first hop as  and the PRB index of the PUCCH transmission in the second hop as , where  is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes set

-	the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as

===== end of TP =====

Friday session

R1-1809932	Summary of Maintenance Issues on Short PUCCH	Ericsson

Decision: The document is noted.

Agreements:

- When HARQ-ACK corresponding to SPS PDSCH without scheduling DCI overlaps only with configured PUCCH resource(s) for CSI and/or SR and/or PUSCH resources with configured grant in a slot,

  - The timeline requirement for UCI multiplexing is satisfied if the earliest symbol among these PUCCH(s)/PUSCH(s) starts no earlier than N1+X symbols after the last symbol of the corresponding SPS-PDSCH(s).

    - X is defined as agreed in RAN1#93.

  - The pseudo-code in 9.2.5 of TS38.213 is applicable to resolve overlapping among PUCCH resources (if any)

  - After resolving overlapping among PUCCH resources (if any), the PUCCH UCI is multiplexed on the overlapping PUSCH resource with configured grant (if any).

  - Note: this does not apply to PUSCH or SPS PDSCH as part of the activation

Agreements:

- The TP in section 3.1 of R1-1809932 for 38.211, subclause 6.3.2.2.2 and 6.4.1.3.2.1, is endorsed

- The TP in section 3.2 of R1-1809932 for 38.211, subclause 6.3.2.5.1, is endorsed

Agreement:

- To adopt the following TP to 6.4.1.3.2.1 of 38.211:

===== start of TP =====

…………………….

is given by the higher-layer parameter scramblingID0 in the DMRS-UplinkConfig IE if provided and by NIDcell otherwise. If a UE is provided both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, scramblingID0 is provided by dmrs-UplinkForPUSCH-MappingTypeB.

===== end of TP =====

R1-1809994	Summary of maintenance issues on frequency hopping	Ericsson

Decision: The document is noted. further revised in R1-1810011.

Agreement:

- The two TPs in R1-1810011 are endorsed (38.211)

R1-1809976	Summary of PUCCH maintenance	Qualcomm

Decision: The document is noted. Further revised in R1-1810003.

Agreements:

- For the determination of the number of PRBs in 38.213 section 9.2.5.1 and 9.2.5.2, OCRC=11if the number of UCI bits is larger than or equal to 360; otherwise OCRC is the number of CRC bits calculated based on the number of UCI bits, as determined in 38.212.

R1-1809517	Summary of RAN1#94 Tdocs on maintenance for R15 NR PUCCH resource allocation	OPPO

R1-1809707	Specification Update on UL Control Incorporating Proposals in RAN1#94	Samsung

R1-1808103	Remaining issues for physical uplink control channel	Huawei, HiSilicon

R1-1808207	Remaining issues on NR PUCCH	ZTE

R1-1808225	Remaining issues on PUCCH	vivo

R1-1808260	Remaining issues of PUCCH	MediaTek Inc.

R1-1808313	Corrections on Subclause 9.2 UCI reporting in PUCCH in TS 38.213	Fujitsu

R1-1808363	Remaining issues on PUCCH	NEC

R1-1809713	Corrections to UCI feedback procedures	CATT	(revision of R1-1808379)

R1-1808491	Correction on PUCCH and UCI multiplexing for NR	LG Electronics

R1-1808620	Remaining issues on physical uplink control channel	Panasonic Corporation

R1-1808673	Remaining details on NR PUCCH	Intel Corporation

R1-1808754	Remaining Issues on PUCCH	Samsung

R1-1808802	Remaining issues on UCI multiplexing	Spreadtrum Communications

R1-1808812	Remaining issues about PUCCH	ETRI

R1-1808893	Remaining issues for UL control	Guangdong OPPO Mobile Telecom

R1-1808966	Remaining details on NR Physical UL Control Channel	Nokia, Nokia Shanghai Bell

R1-1809513	Remaining issues on PUCCH	Sharp	(revision of R1-1809109)

R1-1809142	Maintenance for physical uplink control channel	NTT DOCOMO, INC.

R1-1809321	Remaining issues on PUCCH	WILUS Inc.

R1-1809406	Maintenance issues of physical uplink control channel	Ericsson

R1-1809427	CRs for PUCCH	Qualcomm Incorporated

#### Maintenance for DL/UL data scheduling and HARQ procedure

Including DL/UL resource allocation, DL/UL scheduling and HARQ management, CBG, UL data transmission procedure, soft-buffer management, multiplexing data with different tx durations, etc.

R1-1809702	Summary of 7.1.3.3 (resource allocation)	Ericsson

Decision: The document is noted.

Proposal: Conclude whether interleaved VRB-to-PRB mapping is necessary for SIB1 prior to discussing which of the (potentially mutually incompatible) specification changes to adopt.

Interleaved VRB-to-PRB mapping is necessary for SIB1

Yes: Huawei, CATT

No: Ericsson

R1-1808105	Remaining issues on data scheduling and HARQ	Huawei, HiSilicon

- Observation 1: Compared with non-interleaved VRB-to-PRB mapping, when the bandwidth of initial DL BWP is 24/48/96 RBs, 0.7/1.4/2.3 dB gain for SIBx (x ≥ 1) transmission is achieved by interleaved VRB-to-PRB mapping.

- Proposal 1: NR supports VRB-to-PRB interleaving for SIBx transmission.

Decision: The document is noted.

Agreements:

- To adopt the following TP to Section 4.3.2 of 38.211

===== start of TP =====

A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than NRx-TxTc after the end of the last received downlink symbol in the same cell where NRx-Tx is given by [TS 38.101] Table 4.3.2-3.

A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same cell where  NTx-Rx  is given by Table 4.3.2-3.

Table 4.3.2-3: Transition time NRx-Tx and NTx-Rx.

| Transition time | FR1 | FR2 |
| --- | --- | --- |
| NRx-Tx | 25600 | 13792 |
| NTx-Rx | 25600 | 13792 |

===== end of TP =====

Proposals:

- To adopt the following TP to section 5.1.2.2.2 of 38.214:

In downlink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved or interleaved virtual resource blocks within the active bandwidth part of size  PRBs except for the case when DCI format 1_0 is decoded in any common search space in CORESET 0 in which case the initial bandwidth part of size of the BWP shall be used.

Agreements:

- To adopt the following TP to Section 8.3 of 38.213:

===== start of TP =====

The Msg3 PUSCH frequency resource allocation field is interpreted as follows:

- if NsizeBWP ≤ 180

- Truncate the Msg3 PUSCH frequency resource allocation to its b least significant bits, where b=⎾log2(NsizeBWP (NsizeBWP+1)/2)˥, and interpret the truncated frequency resource allocation according to the rules for DCI format 0_0

- else

- Insert b most significant bits with value set to ‘0’ after the NUL_hop hopping bits in the Msg3 PUSCH frequency resource allocation, where the number of hopping bits NUL_hop is zero when the frequency hopping flag is not set to 1, and is defined in Table 8.3-1 when the hopping flag bit is set to1, and b=(⎾log2(NsizeBWP (NsizeBWP+1)/2)˥ - 14), and interpret the expanded frequency resource allocation according to the rules for DCI format 0_0

-  end if

===== end of TP =====

Agreements:

- Adopt the TP below for 38.214, Section 5.1.3.2:

===== start of TP =====

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH.

The UE is not expected to receive a PDSCH assigned by a PDCCH with CRC scrambled by SI-RNTI with a TBS exceeding 2976 bits.

For the PDSCH assigned by a PDCCH with DCI format 1_0 with CRC scrambled by P-RNTI, or RA-RNTI, TBS determination follows the steps 1-4 with the following modification in step 2: a scaling  is applied in the calculation of Ninfo, where the scaling factor is determined based on the TB scaling field in the DCI as in Table 5.1.3.2-3.

===== end of TP =====

Agreements:

- To adopt the following TP to 7.4.1.1.2 of 38.211:

===== start of TP =====

The reference point for  is

- for PDSCH transmission carrying SIB1, subcarrier 0 of the lowest-numbered common resource block in the CORESET 0 configured by the PBCH if the corresponding PDCCH is associated with CORESET 0 and Type0-PDCCH common search space and is addressed to SI-RNTI,

- otherwise, subcarrier 0 in common resource block 0

===== end of TP =====

Agreements:

- To adopt the following TP of 8.2 of 38.213:

===== start of TP =====

If higher layer parameter pusch-AllocationList is provided by pusch-ConfigCommon, a value of the Msg3 PUSCH time resource allocation field indicates an entry to a table provided by pusch-AllocationList; otherwise a value of the Msg3 PUSCH time resource allocation field indicates an entry to default time resource allocation table [6, 38.214].

===== end of TP =====

Proposals:

- To adopt the following TP of 5.1.2.1 of 38.214:

If a bandwidth part indicator field is not configured in the scheduling DCI, the slot and symbol indexing for downlink type 0 and type 1 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI, the slot and symbol indexing for downlink type 0 and type 1 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI.

When the UE is configured with aggregationFactorDL > 1, the same symbol allocation is applied across the aggregationFactorDL consecutive slots. The UE may expect that the TB is repeated within each symbol allocation among each of the aggregationFactorDL consecutive slots and the PDSCH is limited to a single transmission layer. The redundancy version to be applied on the nth transmission occasion of the TB is determined according to table 5.1.2.1-2.

Wednesday session

R1-1809839	Outcome of offline discussion on 7.1.3.3 (resource allocation)	Ericsson

Decision: The document is noted.

Agreement:

- The UE counts the number of DCI sizes across slots based on RRC configuration for PDCCH, without taking into account PDCCH candidate dropping by overbooking handling.

Agreements:

- The TPs in Section 2 of R1-1809839 are endorsed

Thursday session

R1-1809928	Outcome of offline discussion on 7.1.3.3 (resource allocation) – part II	Ericsson

Decision: The document is noted.

Agreements:

- Adopt the text proposal in Section 2 of R1-1809928 for 38.211

- Remove the bracket in 38.212 around the VRB-to-PRB bit

Agreements:

For OSI and paging:

- if PDSCH-ConfigCommon has provided a table

  - Use this table

- else

  - the same table (A, B, or C) as used by SIB1

Adopt the TP below for 38.214

Table 5.1.2.1.1-1: Applicable PDSCH time domain resource allocation

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | pdsch-ConfigCommon includes pdsch-TimeDomainAllocationList | pdsch-Config includes pdsch-TimeDomainAllocationList | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- |
| SI-RNTI | Type0 common | 1 | - | - | Default A for normal CP |
|  |  | 2 | - | - | Default B |
|  |  | 3 | - | - | Default C |
| SI-RNTI | Type0A common | 1 | No | - | Default A |
|  |  | 2 | No | - | Default B |
|  |  | 3 | No | - | Default C |
|  |  | 1,2,3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdschConfigCommon |
| RA-RNTI, TC-RNTI, | Type1 common | 1, 2, 3 | No | - | Default A |
|  |  | 1, 2, 3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdschConfigCommon |
| P-RNTI | Type2 common | 1 | No | - | Default A |
|  |  | 2 | No | - | Default B |
|  |  | 3 | No | - | Default C |
|  |  | 1,2,3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdschConfigCommon |
| C-RNTI, CS-RNTI | Any common search space associated with CORESET#0 | 1, 2, 3 | No | - | Default A |
|  |  | 1, 2, 3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdschConfigCommon |
| C-RNTI, CS-RNTI | Any common search space not associated with CORESET#0 UE specific search space | 1,2,3 | No | No | Default A |
|  |  | 1,2,3 | Yes | No | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
|  |  | 1,2,3 | No/Yes | Yes | pdsch-TimeDomainAllocationList provided in pdsch-Config |

Friday session

R1-1809977	Outcome of offline session on data rates and TB size limitation	Ericsson

Decision: The document is noted.

Working assumption:

A UE is not required to handle overlapped PDSCH/PUSCH transmissions not fulfilling the following condition at all points in time:

\sum_{j=0}^{N-1}{CjKr,j/Dj} ≤ DataRate

- Note: the summation may need to be refined – e.g., considering summation over R code blocks indexed by r

- Cj' is the number of scheduled code blocks of the transport block in the j-th PDSCH/PUSCH (from 38.212)

- Kr,j is the number of  bits in r-th code block of the j-th PDSCH/PUSCH

- DjDj is the duration of the j-th PDSCH/PUSCH (= #symbols * symbol duration)

- DataRate is computed by the approximate date rate given by Subcluase 4.1.2 in [13, TS 38.306] based on the following:

  - Applying per frequency range per CG only

    - N is the number of configured NR carriers of the frequency range of the CG

- The PDSCHs/PUSCHs for j = 0 to N-1 overlapping or partially overlapping PDSCH/PUSCH transmission

- FFS In addition to limiting the number of information bits as above, constraints on the number of coded bits may be needed.

- Symbol duration Ts =10-3/(Nsymbslot2), where  is the numerology for the PDSCH/PUSCH and Nsymbslot is the number of symbols per slot

R1-1809997	Summary of offline discussion on LBRM	Ericsson

Discussion: Some companies can not agree as it may change the behaviour in RAN2 spec.

Decision: The document is noted.

Agreement:

- The proposals in R1-1809997 is taken as a working assumption

R1-1809776	Summary for DL/UL data scheduling maintenance	Qualcomm

Decision: The document is noted.

Agreements:

The following agreement is extended to include RRC_INACTIVE. Text proposals for 38.214 and 38.202 should also be adopted to capture additional related specification details and corrections.

- For UE behavior in RRC_IDLE and RRC_INACTIVE, if PDSCHs among SI-RNTI PDSCH, P-RNTI PDSCH, and RA-RNTI/TC-RNTI PDSCH are overlapped with at least one symbol for a given UE from the primary cell

  - NR supports UE to decode up to two PDSCH simultaneously in Pcell, but not more than two (if more than two are received, the decoding prioritization is up to UE implementation)

  - Note: the above assumes that all the PDSCHs to be decoded by the UE have non-overlapped PRBs

Agreements:

- If a first DCI has indicated a first PUCCH resource for HARQ-ACK bits associated to the PDSCH scheduled by this DCI

  - If due to a second DCI, additional HARQ-ACK bits are to be multiplexed on the same slot as the first PUCCH resource, the UE is not expected to multiplex additional HARQ-ACK bits if the last symbol of second DCI is not earlier than N OFDM symbols before first symbol of the first PUCCH resource, where for UE Capability #1 the N values are:

    - 15 kHz case: 8

    - 30 kHz case: 10

    - 60 kHz case: 17

    - 120 kHz case: 20

    - FFS values to be used for Capability #2 (e.g., 3, 4.5, and 9 for 15kHz, 30kHz, and 60kHz respectively)

Agreement:

- For Capability #2, the processing time for UCI multiplexing should be extended by d symbols using the same values and conditions as used for Capability #1.

Agreements:

- The processing requirement for HARQ-ACK in response to DL SPS release is N OFDM symbols from last symbol of the corresponding PDCCH for UE Capability #2:

  - 15 kHz case: 5

  - 30 kHz case: 5.5

  - 60 kHz case (FR1 only): 11

Thursday session

R1-1809894	Summary for DL/UL data scheduling maintenance	Qualcomm

Decision: The document is noted.

Agreements:

- Confirm the working assumption for N1 with Capability #2 and the following change

  - (working assumption) For PDSCH mapping type B with 4 or 2 symbols

    - N1 processing time is increased by ‘d’ symbols relative to the case of PDSCH with 7 symbols, where ‘d’ is the amount of time-domain overlap in symbols between the scheduling PDCCH and the scheduled PDSCH

      - FFS: handling of 3-symbol CORESET where first 2 symbols of CORESET are overlapped with a 2-symbol PDSCH

      - For a DCI received in a 3-symbol CORESET where first 2 symbols of CORESET are overlapped with 2-symbol PDSCH scheduled by this DCI, then d=3

Agreements:

- For a UE indicating Capability #2 N1 with maximum scheduled RB allocation of 136 RBs

  - The UE may skip decoding a number of PDSCHs with last symbol within 10 symbols before the start of a PDSCH that is scheduled to follow Capability #2, if any of those PDSCHs are scheduled with more than 136 RBs with 30kHz SCS and following Capability #1 processing time.

Agreements:

- To adopt the following TP to 5.1 of 38.214

===== start of TP =====

On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI or CS-RNTI requires Capability #2 processing time according to subclause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI or CS-RNTI.

On a frequency range 2 cell, the UE is not expected to decode a PDSCH scheduled with C-RNTI or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time in non-overlapping PRBs.

The UE is expected to decode a PDSCH scheduled with C-RNTI or CS-RNTI during a process of autonomous SI acquisition.

===== end of TP =====

Agreements:

- Send LS to RAN2 to requesting them to make [0..15] applicable for K1 – draft LS in R1-1809930 (Joseph, QC)

  - Note that from 38.331 vf2, RAN2 already has [0…15] as values.

dl-DataToUL-ACK							SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)											OPTIONAL,	-- Need M

- But in the field description below, the range is limited. Removing this limitation in the field description will enable the requested change.

PUCCH-Config field descriptions

dl-DataToUL-ACK List of timing for given PDSCH to the DL ACK. In this version of the specification only the values [0..8] are applicable. Corresponds to L1 parameter 'Slot-timing-value-K1' (see TS 38.213, section FFS_Section).

Friday session

R1-1809930	[DRAFT] LS on K1 Slot Offset	Qualcomm

Decision: The draft LS is endorsed and final LS is approved in R1-1809982.

Observation:

The following are noted by RAN1 as already supported in 38.331

- The scheduling parameters K0 and K2 may be configured from the set of values {0,1,2,3,…,32}

Working assumption:

- The UE is not required to store more than 16 scheduling DCI for PDSCH on a given cell, among all scheduling DCI received prior and up to the current slot

  - FFS the applicability to broadcast PDSCH or not

  - FFS the applicability to activation/deactivation DCI

- The UE is not required to store more than 16 scheduling DCI for PUSCH on a given cell, among all scheduling DCI received prior and up to the current slot

R1-1809953	UE Processing Time and UCI Multiplexing TPs for 38.213 and 38.214	Qualcomm

Decision: The document is noted. Companies are encouraged to check R1-1809953.

R1-1809767	Offline summary for UL data transmission procedure	NTT DOCOMO

Decision: The document is noted.

Proposals:

- To adopt the following TP to 8.3 of 38.213:

An UL BWP, as described in Subclause 12 and in [4, TS 38.211], for Msg3 PUSCH transmission is indicated by higher layers. When determining the frequency domain resource allocation for Msg3 PUSCH transmission within the active UL BWP, the initial UL BWP shall be used, if the active BWP and the initial BWP have the same  and the active BWP includes all the PRBs of the initial BWP, or the active BWP is the initial BWP. Otherwise the active UL BWP shall be used.

Proposals:

- For MSG3 transmission and retransmission, when transform precoding is enabled, UE ignores the previously configured UE-specific parameters by dedicated signalling (for sequence hopping and mcs-tableTransformProcoder) but only applies cell-specific parameters. Sequence hopping should be considered as disabled for DMRS sequence generation, MCS table for transform precoder applies the value 64QAM. Further, groupHoppingEnabledTransformPrecoding in PUSCH-ConfigCommon should be interpreted as follows: (1) absent: both group hopping and sequence hopping disabled; (2) enabled: group hopping enabled, sequence hopping disabled; (3) disabled: group hopping disabled, sequence hopping disabled

- For PUSCH transmission including configured grant, if transform precoding is enabled, it follows the same default configuration as MSG3 transform precoding if dedicated signalling parameters are absent. Sequence hopping should be considered as disabled for DMRS sequence generation, MCS table for transform precoder applies the value 64QAM

In RAN2, there is an on-going discussion on how the default assumptions/values for L1 parameters are captured in the specs. They may ask us to check/capture the default assumptions/values. Once above is agreed, the agreements captured in the same way as other L1 parameters.

Thursday session

R1-1809898	Corrections on UL data transmission procedure	NTT DOCOMO

Decision: The document is noted.

Agreements:

- To adopt the following TP to 8.3 of 38.213

===== start of TP =====

An UL BWP, as described in Subclause 12 and in [4, TS 38.211], for Msg3 PUSCH transmission is indicated by higher layers. When determining the frequency domain resource allocation for Msg3 PUSCH transmission within the active UL BWP, the initial UL BWP shall be used, if the active UL BWP and the initial UL BWP have the same SCS and the same CP length and the active UL BWP includes all the PRBs of the initial UL BWP, or the active UL BWP is the initial UL BWP.

Otherwise the RB numbering for the scheduled PUSCH starts from starting RB of the active UL BWP and the maximum number of RBs possible for resource allocation is given by the size of the initial UL BWP.

===== end of TP =====

Agreements:

- For MSG3 transmission and retransmission, when transform precoding is enabled, UE ignores the previously configured UE-specific parameters by dedicated signalling (for sequence hopping) but only applies cell-specific parameters. Sequence hopping should be considered as disabled for DMRS sequence generation. Further, groupHoppingEnabledTransformPrecoding in PUSCH-ConfigCommon should be interpreted as follows: (1) absent: both group hopping and sequence hopping disabled; (2) enabled: group hopping enabled, sequence hopping disabled.

- For PUSCH transmission including configured grant, if transform precoding is enabled, it follows the same default configuration as MSG3 transform precoding if dedicated signalling parameters are absent. Sequence hopping should be considered as disabled for DMRS sequence generation.

Agreements:

- The TP in Section 2.3 in R1-1809898 is endorsed (38.214)

- The TP in Section 2.4 in R1-1809898 is endorsed (38.213)

- For a scheduled retransmission of a TB previously transmitted on a configured PUSCH, the UE follows

- Option 1: the set of parameters provided in configuredGrantConfig IE that are common to both Type1 and Type2 configured grant – i.e. except those provided specifically for Type1 in rrc-ConfiguredUplinkGrant IE. For all other parameters not provided in configuredGrantConfig, including those in rrc-ConfiguredUplinkGrant, the UE follows the transmission configuration provided by the PUSCH-Config IE.

  - Pros: simple and no additional issues.

  - Cons: May lose some scheduling flexibility

- Option 2: the set of transmission parameters provided by the PUSCH-Config IE.

  - Pros: May have some scheduling flexibility

  - Cons:

    - ambiguity on the DCI scrambled by CS-RNTI is used for (de)activation or re-transmission

R1-1809765	Summary on CBG based retransmission	LG Electronics

Decision: The document is noted.

R1-1809734	Summary of 7.1.3.3 (Soft Buffer Management)	Ericsson

Decision: The document is noted.

R1-1809763	Summary of multiplexing data with different tx durations	vivo

Decision: The document is noted.

Agreements:

- To adopt the following TP of 11.2 of 38.213 (Section 3 of R1-1809763):

===== start of TP =====

11.2	Interrupted transmission indication

< Unchanged parts are omitted >

If a UE detects a DCI format 2_1 for a serving cell from the configured set of serving cells, the UE may assume that no transmission to the UE is present in PRBs and in symbols that are indicated by DCI format 2_1, from a set of PRBs and a set of symbols of the last monitoring period. The indication by the DCI format 2_1 is not applicable to receptions of SS/PBCH blocks.

The set of PRBs is equal to the active DL BWP as defined in Subclause 12 and includes  PRBs.

If a UE detects a DCI format 2_1 in a PDCCH transmitted in a control resource set in a slot, the set of symbols  is the last  symbols prior to the first symbol of the control resource set in the slot where  is the PDCCH monitoring periodicity provided by the value of higher layer parameter monitoringSlotPeriodicityAndOffset, as described in Subclause 10.1,  is the number of symbols per slot, is the subcarrier spacing configuration for a serving cell with mapping to a respective field in the DCI format 2_1,  is the subcarrier spacing configuration of the DL BWP where the UE receives the PDCCH conveying the DCI format 2_1. If the UE is configured with higher layer parameters tdd-UL-DL-ConfigurationCommon or tdd-UL-DL- ConfigurationCommon2, symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL- ConfigurationCommon2 are excluded from the last  symbols prior to the first symbol of the control resource set in the slot. The resulting set of symbols includes a number of symbols that is denoted as .

The UE does not expect to be provided values of , , and  resulting to a value of  that is not an integer. For DCI format 2_1, the UE does not expect to be configured with more than one PDCCH monitoring occasions in a slot, by higher layer parameter monitoringSymbolsWithinSlot.

< Unchanged parts are omitted >

===== end of TP =====

Wednesday

R1-1809838	Summary for DL/UL data scheduling maintenance	Qualcomm

Decision: The document is noted.

Agreements:

The following UE capabilities are added:

- Duplicate 5-11,5-11a,5-11b for Capability #2

  - Note: 5-11,5-11a,5-11b will be associated only with Capability #1

- Duplicate 5-12,5-12a,5-12b for Capability #2

  - Note: 5-12,5-12a,5-12b will be associated only with Capability #1

R1-1808210	Remaining Issues of NR scheduling and HARQ process	ZTE

R1-1808226	Remaining issues on scheduling and HARQ	vivo

R1-1808261	Remaining issues of DLUL data scheduling and HARQ procedure	MediaTek Inc.

R1-1808380	Corrections to DL/UL scheduling and HARQ management	CATT

R1-1808492	Discussion on DL/UL data scheduling and HARQ procedure	LG ELECTRONICS

R1-1809731	NR scheduling and HARQ procedures	Intel Corporation	(revision of R1-1808674)

R1-1808755	Remaining Issues on UL/DL Scheduling	Samsung

R1-1808808	Remaining issues on DL/UL data scheduling and HARQ procedure	Spreadtrum Communications

R1-1808870	Maintenance for DL/UL data scheduling and HARQ procedure	Panasonic Corporation

R1-1808887	Text proposal for DL/UL data scheduling and HARQ procedure	Guangdong OPPO Mobile Telecom

R1-1809051	38.214 Text Proposal to align PUSCH-AllocationList and PDSCH-AllocationList to ASN.1	SPRINT Corporation

R1-1809060	Remaining details on PDSCH time domain resource allocation	AT&T

R1-1809110	Remaining issues on Msg3 PUSCH frequency resource allocation	Sharp

R1-1809143	Maintenance for DL/UL data scheduling and HARQ procedure	NTT DOCOMO, INC.

R1-1809297	Discussion on Remaining issues on HARQ-ACK bundling and multiplexing	China Telecommunications

R1-1809322	Remining issues on DL/UL data scheduling and HARQ procedure	WILUS Inc.

R1-1809392	Remaining issues on resource allocation and UE processing times	Nokia, Nokia Shanghai Bell

R1-1809407	Maintenance issues of DL/UL data scheduling and HARQ procedure	Ericsson

R1-1809428	Maintenance for DL/UL data scheduling and HARQ procedure	Qualcomm Incorporated

R1-1809949	Proposal for Data rate and TBS	Ericsson

R1-1809957	Above Peak-Rate Skip-Decoding Rule	Qualcomm Incorporated

R1-1809980	General principle of re-transmission of PUSCH with configured grant	NTT DOCOMO

#### Maintenance for carrier aggregation and bandwidth parts

R1-1809773	Summary of Bandwidth Part Remaining Issues	MediaTek

Decision: The document is noted.

Agreements:

- Issue #1: In case multiplexing pattern 2 or 3 is used for the initial DL BWP, for an active DL BWP that overlaps with the initial DL BWP but not with the SS/PBCH block associated to the initial DL BWP, is it possible for the UE to:

  - Question #1a: Perform BM based on the SS/PBCH block associated to the initial DL BWP?

    - Answer: No, a UE is not expected to perform BM based on the SS/PBCH block outside the active DL BWP. Note that the scenario can be supported with appropriate CSI-RS configuration.

  - Question #1b: Perform BFD based on the SS/PBCH block associated to the initial DL BWP?

    - Answer: No, a UE is not expected to perform BFD based on the SS/PBCH block outside the active DL BWP. Note that the scenario can be supported with appropriate CSI-RS configuration.

  - Question #1c: Perform RLM based on the SS/PBCH block associated to the initial DL BWP?

    - Answer: No, a UE is not expected to perform RLM based on the SS/PBCH block outside the active DL BWP. Note that the scenario can be supported with appropriate CSI-RS configuration.

  - Question #1d: Monitor the SS/PBCH block associated to the initial DL BWP to receive broadcast SI / paging based on CORESET#0 and searchspace#0? (assuming the same SCS is used in the active BWP as that of initial BWP)

    - Answer: No, a UE is not expected to monitor the SS/PBCH block outside the active DL BWP and associated to the initial DL BWP to receive broadcast SI / paging based on CORESET#0 and searchspace#0.

- Issue #2: For an active DL BWP that overlaps with neither the initial DL BWP nor with the SS/PBCH block associated to the initial DL BWP (from now on: "non-overlapping" active DL BWP), RAN2 understands that additional CORESET/search space(s) need to be configured in the active DL BWP to provide SI broadcast and paging to a connected UE. What is not clear is whether an SS/PBCH block necessarily needs to be associated to the additional CORESET/search space(s) and, specifically, if a UE needs to monitor such SS/PBCH block.

  - Question #2a: In a "non-overlapping" active DL BWP, does a UE necessarily need to monitor an SS/PBCH block associated to the additional CORESET/search space to be able to receive SI broadcast? If yes, could this be the SS/PBCH block associated to the initial DL BWP? And under which conditions, if any, could the UE monitor it for this purpose?

    - Answer: No, a UE does not necessarily need to monitor an SS/PBCH block associated to the additional CORESET/search space to be able to receive SI broadcast. The UE can be configured with TCI states for the additional CORESET/search space to enable SI broadcast reception.

  - Question #2b: In a "non-overlapping" active DL BWP, does a UE necessarily need to monitor an SS/PBCH block associated to the additional CORESET/search space to be able to receive paging? If yes, could this be the SS/PBCH block associated to the initial DL BWP? And under which conditions, if any, could the UE monitor it for this purpose?

    - Answer: No, a UE does not necessarily need to monitor an SS/PBCH block associated to the additional CORESET/search space to be able to receive paging. The UE can be configured with TCI states for the additional CORESET/search space to enable paging reception.

R1-1809800	[Draft] reply LS on active DL BWPs overlapping/non-overlapping with initial DL BWP	MediaTek

Decision: The document is endorsed and final LS is approved in R1-1809810.

Offline Proposal:

- For the support of Option #2 at least in PSCell & SCell, modify the agreements in RAN1#92bis as follows.

  - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by

    - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWPCORESET0

    - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled

      - Otherwise, for format 0-0/1-0, the size is given by the initial DL BWPCORESET0

  - For DCI format 1-0 in CSS with P-RNTI, SI-RNTI, RA-RNTI, C-RNTI, CS-RNTI, or TC-RNTI:

    - the RB numbering for the scheduled PDSCH starts from the lowest RB in the CORESET the DCI is received in

    - the maximum number of RBs possible to indicate in the DCI is given by the size of the initial DL BWPCORESET0.

    - Note: P-RNTI & SI-RNTI are only applicable to PCell (if supported)

  - Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 as defined by the initial BWP in CSS

- Send LS to RAN2 stating that there is no consensus on whether to support Option #2 in PCell in RAN1

Thursday session

R1-1809929	Summary of Bandwidth Part Remaining Issues	MediaTek

Decision: The document is noted.

Agreements:

- To adopt the following TP to section 12 of 38.213:

===== start of TP =====

If a UE is configured by higher layer parameter bwp-InactivityTimer a timer value for the primary cell [11, TS 38.321] and the timer is running, the UE increments the timer every interval of 1 millisecond for frequency range 1 or every 0.5 milliseconds for frequency range 2 if the UE does not detect a DCI format for PDSCH reception on the primary cell for paired spectrum operation or if the UE does not detect a DCI format for PDSCH reception or a DCI format for PUSCH transmission on the primary cell for unpaired spectrum operation the restarting conditions in [11, TS 38.321] are not met during the interval [11, TS 38.321].

If a UE is configured by higher layer parameter BWP-InactivityTimerbwp-InactivityTimer a timer value for a secondary cell [11, TS 38.321] and the timer is running, the UE increments the timer every interval of 1 millisecond for frequency range 1 or every 0.5 milliseconds for frequency range 2 if the UE does not detect a DCI format for PDSCH reception on the secondary cell for paired spectrum operation or if the UE does not detect a DCI format for PDSCH reception or a DCI format for PUSCH transmission on the secondary cell the restarting conditions in [11, TS 38.321] are not met during the interval for unpaired spectrum operation during the interval. The UE may deactivate the secondary cell when the timer expires.

===== end of TP =====

Agreements:

- To adopt the following TP to section 12 of 38.213:

===== start of TP =====

-	set the active UL BWP or DL BWP to the UL BWP or DL BWP indicated by the bandwidth part indicator in the DCI format 0_1 or DCI format 1_1, respectively.

A UE does not expect to detect a DCI format 1_1 or DCI format 0_1 indicating active DL or UL BWP change with a time domain resource assignment field value providing a slot offset K0 or K2 for corresponding PDSCH reception or PUSCH transmission that is smaller than the active DL or UL BWP switching delay required by the UE [TS 38.133].

If a UE detects DCI format 1_1 indicating active DL BWP change for a cell, the UE is not required to receive or transmit in that cell during a time duration from the end of the 3rd OFDM symbol of a slot containing the PDCCH carrying DCI format 1_1 indicating active DL BWP change in the scheduling cell till the beginning of a slot indicated by K0 in the DCI format 1_1.

If a UE detects DCI format 0_1 indicating active UL BWP change for a cell, the UE is not required to receive or transmit in that cell during a time duration from the end of the 3rd OFDM symbol of a slot containing the PDCCH carrying DCI format 0_1 indicating active UL BWP change in the scheduling cell till the beginning of a slot indicated by K2 in the DCI format 0_1.

A UE expects to detect a DCI format 0_1 indicating active UL BWP change, or a DCI format 1_1 indicating active DL BWP change, only if a corresponding PDCCH is received within the first 3 symbols of a slot.

===== end of TP =====

Agreements:

- To adopt the following TP in section 12 of 38.213

===== start of TP =====

If a UE is configured by higher layer parameter BWP-InactivityTimer a timer value for a secondary cell [11, TS 38.321] and the timer is running, the UE increments the timer every interval of 1 millisecond for frequency range 1 or every 0.5 milliseconds for frequency range 2 if the UE does not detect a DCI format for PDSCH reception on the secondary cell for paired spectrum operation or if the UE does not detect a DCI format for PDSCH reception or a DCI format for PUSCH transmission on the secondary cell for unpaired spectrum operation during the interval. The UE may deactivate the secondary cell when the timer expires.

In a cell where the active DL BWP changes due to BWP inactivity timer expiration, a UE is not required to receive or transmit in that cell during a time duration from the beginning of the subframe for frequency range 1 or the half of a subframe for frequency range 2 immediately after a BWP inactivity timer expires till the beginning of the slot the UE is able receive DL signals or transmit UL signals to accommodate the BWP switching delay required by the UE [TS 38.133].

===== end of TP =====

Proposals:

- Modify the agreements in RAN1#92bis as follows.

  - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by

    - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWPCORESET#0

    - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled

      - Otherwise, for format 0-0/1-0, the size is given by the initial DL BWPCORESET#0

  - For DCI format 1-0 in CSS with P-RNTI, SI-RNTI, RA-RNTI, C-RNTI, CS-RNTI, or TC-RNTI:

    - the RB numbering for the scheduled PDSCH starts from the lowest RB in the CORESET the DCI is received in

    - the maximum number of RBs possible to indicate in the DCI is given by the size of the initial DL BWPCORESET#0.

  - Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 as defined by the initial BWP in CSS

- For PSCell and SCell,

  - The initial DL BWP of a cell can be configured to be the same as or larger than CORESET#0 of the cell

  - The initial DL BWP of a cell includes CORESET#0 of the cell in frequency domain

- Note (earlier agreements) For PCell,

  - The initial DL BWP of a cell is defined as the frequency location and bandwidth of CORESET#0 of the cell, which was agreed and captured in the current spec

- There is no consensus on whether to support Option #2 in PCell

- Send LS to RAN2 stating that there is no consensus on whether to support Option #2 in PCell in RAN1 (including informing the above agreements)

Friday session

R1-1809961	Summary of Bandwidth Part Remaining Issues	MediaTek

Decision: The document is noted.

Agreements:

- Modify the agreements in RAN1#92bis as follows.

  - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by

    - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWPCORESET#0

    - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled

      - Otherwise, for format 0-0/1-0, the size is given by the initial DL BWPCORESET#0

  - For DCI format 1-0 in CSS with P-RNTI, SI-RNTI, RA-RNTI, C-RNTI, CS-RNTI, or TC-RNTI:

    - the RB numbering for the scheduled PDSCH starts from the lowest RB in the CORESET the DCI is received in

    - the maximum number of RBs possible to indicate in the DCI is given by the size of the initial DL BWPCORESET#0.

  - Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 as defined by the initial BWP in CSS

- Modify the agreements in RAN1#91 as follows if Option #2 is supported in PCell

  - CORESET configured by RMSI is confined within the initial DL BWPthe bandwidth of CORESET#0

- For PCell, the initial DL BWP can be configured in SIB1 to be the same as or different with the initial DL BWP as initially defined by CORESET#0

  - The initial DL BWP configured in SIB1 includes the bandwidth of CORESET#0

  - If the initial DL BWP configured by SIB1 is different with the initial DL BWP as initially defined by CORESET#0, the configuration of the initial DL BWP configured by SIB1 is applicable after the initial access

- For PSCell and SCell, the initial DL BWP of a cell can be configured to be the same as or different with the initial DL BWP as defined by CORESET#0 of the cell

  - The initial DL BWP of a cell includes the bandwidth of CORESET#0 of the cell

- Note: RAN1 assumes that the above agreements related to PCell, PSCell and SCell have no RAN1 specification impact and will be captured in RAN2 specs

- Send LS to RAN2 to ask RAN2 to capture the above agreements related to PCell, PSCell and SCell in RAN2 specs

R1-1810000	[Draft] LS on bandwidth configuration for initial BWP	MediaTek

Decision: The document is endorsed and final LS is approved in R1-1810002.

R1-1809811	CA Offline Session 1	Samsung

Decision: The document is noted.

Agreement:

- Use firstActiveDownlinkBWP as reference BWP for a deactivated SCell for semi-static HARQ codebook determination.

Thursday session

R1-1809848	CA Offline Session 2	Samsung

Decision: The document is noted.

Agreements:

- To support fallback operation for semi-static HARQ-ACK codebook on a PUSCH subject to the following conditions

  - DAI in UL grant is equal to 0 (when PUSCH is in response to detection of an UL grant)

  - Include in section 9.1.2.2 the following (exact text subject to editorial adjustments)

    - If a UE reports HARQ-ACK information in a PUSCH only for a SPS PDSCH release or only for a PDSCH reception within the  occasions for candidate PDSCH receptions, as determined in Subclause 9.1.2.1, that is scheduled by DCI format 1_0 with a counter downlink assignment indicator (DAI) field value of 1 on the PCell, the UE determines a HARQ-ACK codebook only for the SPS PDSCH release or only for the PDSCH reception

Agreements:

- For determination of Type 1 (semi-static) HARQ-ACK codebook, the SLIV values are the union of the SLIV values in the default table and the SLIV values in the table provided by UE-dedicated RRC

Friday session

R1-1809988	CA Offline Session 3	Samsung

Decision: The document is noted. Companies are encouraged to check slide 4 of R1-1809988.

Agreements:

- The UE multiplexes HARQ-ACK in any slot of a multi-slot PUSCH transmission where the UE would otherwise transmit HARQ-ACK in a single slot PUCCH transmission, based on the HARQ timeline

- DAI is applicable in any slot where the UE would transmit HARQ-ACK

R1-1809706	Specification Update on HARQ-ACK Codebooks Incorporating Proposals in RAN1#94	Samsung

R1-1809752	Remaining issues on bandwidth part and CA	Huawei, HiSilicon	(revision of R1-1808104)

R1-1808208	Remaining issues on BWP	ZTE

R1-1808270	Remaining issues on bandwidth part operation	MediaTek Inc.

R1-1808381	Corrections on carrier aggregation and bandwidth part operation	CATT

R1-1808493	Correction on CA and BWP operation for NR	LG Electronics

R1-1808756	Remaining Issues on BWP and CA	Samsung

R1-1808805	Remaining issues on bandwidth parts and carrier aggregation	Spreadtrum Communications

R1-1808894	Text proposals for HARQ-ACK transmission	Guangdong OPPO Mobile Telecom

R1-1808953	On remaining details of BWPs and CA	Nokia, Nokia Shanghai Bell

R1-1809144	Maintenance for carrier aggregation and bandwidth parts	NTT DOCOMO, INC.

R1-1809183	Latency due to bandwidth part switching	ITRI

R1-1809247	Remaining issue on BWP	ASUSTEK COMPUTER (SHANGHAI)

R1-1809323	Remaining issues on HARQ-ACK transmission on CA	WILUS Inc.

R1-1809408	Remaining issues related to CA and BWP operation	Ericsson

R1-1809429	Maintenance for carrier aggregation and bandwidth parts	Qualcomm Incorporated

#### Other

Including rate matching, TA, etc.

R1-1809746	Summary of maintenance related to rate-matching in NR	Nokia, Nokia Shanghai Bell

Decision: The document is noted.

Agreements:

- To adopt the following TP (Section 2.1 of R1-1809746):

===== start of TP =====

7.4.3.1	Time-frequency structure of an SS/PBCH block

< Unchanged parts are omitted >

The UE may assume that the complex-valued symbols corresponding to resource elements that are part of a common resource block partially or fully overlapping with an SS/PBCH block and not used for SS/PBCH transmission are set to zero in the OFDM symbols that partially or fully overlapping with OFDM symbols where SS/PBCH is transmitted.

< Unchanged parts are omitted >

===== end of TP =====

Agreement:

Update the following agreement as follows:

When a PDCCH candidate collides with any RB-level resource (RateMatchPattern) configured without controlResourceSetId, where PDSCH is semi-statically not mapped, as described in TS38.214 Section 5.1.4.1 then

- The UE is not required to monitor the PDCCH candidate overlapping with the resource even only by one RE

- Note: the current spec already reflects the above updated agreements

Thursday session

R1-1809846	Summary of offline on maintenance related to rate-matching in NR	Nokia, Nokia Shanghai Bell

Decision: The document is noted.

Agreements:

- To adopt the following TP to 5.1.4 of 38.214:

===== start of TP =====

within a BWP, a frequency domain resource of a CORESET with controlResourceSetId and time domain resource determined by the higher layer parameters monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot of search-space-sets configured by SearchSpace and time domain resource of search-space-set zero configured by pdcch-ConfigSIB1 or searchSpaceZero associated with the CORESET with controlResourceSetId as well as CORESET duration configured by ControlResourceSet with controlResourceSetId. This resource not available for PDSCH can be included in one or two groups of resource sets (higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2).

===== end of TP =====

Agreements:

- To adopt the following TP to 5.1.4.1 of 38.214:

===== start of TP =====

If a PDSCH scheduled by a PDCCH would overlap with resources in the CORESET containing the PDCCH, the resources corresponding to a union of the detected PDCCH that scheduled the PDSCH and associated PDCCH DM-RS are not available for the PDSCH. When precoderGranularity configured in a CORESET where the PDCCH was detected is equal to allContiguousRBs, the associated PDCCH DMRS are DMRS in all REGs of the CORESET. Otherwise, the associated DMRS are the DMRS in REGs of the PDCCH.

===== end of TP =====

Agreements:

- When configured with higher-layer parameter RateMatchPatternLTE-CRS for a serving cell in unpaired spectrum, UE assumes LTE CRS are present for rate matching in DL and flexible symbols based on NR TDD UL/DL semi-static configuration.

  - Note: Also subject to MBSFN configured in RateMatchPatternLTE-CRS.

  - Further check whether there is any specification impact

Proposal:

- When UE is configured to rate-match around the LTE CRS, it rate-matches also around the center carrier provided with the configuration in the symbols containing LTE CRS

Agreements:

- To adopt the following TP to 7.3.1.2.2 of 38.212:

===== start of TP =====

  - Rate matching indicator – 0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2.

===== end of TP =====

Discuss further Section 3.2 of R1-1809846

Friday

Agreement:

- The TP in Section 3.2 in R1-1809846 (before the wording “offline consensus”) for Section 5.1.4.1 of 38.214 is endorsed

Wednesday

R1-1809824	Summary of TA related aspects	Panasonic

Decision: The document is noted.

Thursday

R1-1809910	Summary of TA related aspects	Panasonic

Decision: The document is noted.

Agreement: The TP in Section 3 of R1-1809910 is endorsed (38.213)

R1-1808209	Remaining aspects for rate matching	ZTE

R1-1808314	Corrections on Type 1 HARQ-ACK codebook on PUCCH	Fujitsu

R1-1808315	Corrections on HARQ-ACK codebooks on PUSCH	Fujitsu

R1-1808382	Corrections on rate matching	CATT

R1-1808494	TA command reception with dynamic BWP change	LG ELECTRONICS

R1-1808803	Remaining issues on PDSCH rate matching	Spreadtrum Communications

R1-1808871	TP on rate matching Subclause 5.1.4 in TS38.214	Panasonic Corporation

R1-1808928	On the number of CRC bits for PUCCH resource determination	Huawei, HiSilicon

Withdrawn

R1-1808944	Corrections on configured grant related procedures	Huawei, HiSilicon

R1-1808954	On remaining details of rate-matching in NR	Nokia, Nokia Shanghai Bell

R1-1808967	URLLC aspects for overlapped UL transmissions of single UE	Nokia, Nokia Shanghai Bell

R1-1809328	On VRB-to-PRB interleaving for SIB transmission	Huawei, HiSilicon

R1-1809329	Remaining issues on reserved resources and rate matching	Huawei, HiSilicon

R1-1809330	Remaining issues on search space	Huawei, HiSilicon

R1-1809331	Remaining issues on DCI budget for CA	Huawei, HiSilicon

R1-1809332	Remaining issues on ECP	Huawei, HiSilicon

R1-1809505	Remaining issues on PDSCH rate matching in CORESET	Qualcomm Incorporated	(R1-1809430)

### Maintenance for NR-LTE co-existence

Including architecture option 4 related issues for both NR and LTE, if any (please refer to RP-181473 for detailed scoping)

Friday

R1-1809914	Chairman's notes of AI 7.1.4 Maintenance for NR-LTE co-existence	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content is incorporated below.

R1-1808266	MPR and A-MPR considerations for EN-DC UE	MediaTek Inc.

Revised in R1-1809730

R1-1808383	Remaining issues of LTE-NR Dual Connectivity	CATT

R1-1808495	Remaining issues on NR LTE coexistence	LG ELECTRONICS

Agreement:

A UE is not expected to be configured with sTTI and/or shortened processing time operation on a carrier that is part of an EN-DC configuration in Rel-15.

R1-1808675	Remaining issues of NR-LTE co-existence	Intel Corporation

R1-1808757	Remaining Issues on LTE-NR Coexistence	Samsung

R1-1808872	Maintenance for NR-LTE co-existence	Panasonic Corporation

R1-1808930	Remaining issues on EN-DC, NE-DC, and SUL	Huawei, HiSilicon

Agreement:

For a serving cell configured with SUL, the same uplink carrier is used for both initial transmission and re-transmission(s) of a TB in Rel-15.

R1-1809075	Power control for Option 4 DC	AT&T

R1-1809102	MPR and A-MPR considerations in EN-DC dynamic power sharing	SPRINT Corporation

R1-1809393	Intra-band EN-DC UE Tx power requirements	Nokia, Nokia Shanghai Bell

R1-1809397	Maintenance for NR-LTE co-existence	Motorola Mobility, Lenovo

R1-1809409	Remaining issues related to LTE-NR coexistence	Ericsson

R1-1809847 (Revision of R1-1809409)	Remaining issues related to LTE-NR coexistence	Ericsson

R1-1809431	Maintenance for NR-LTE co-existence	Qualcomm Incorporated

R1-1809831	Clarifications for EN-DC power control	Qualcomm

R1-1809830	Clarification for FR1-FR2 power control in EN-DC	Qualcomm

The text proposal on slide 3 of R1-1809830 is endorsed with the indentation of the last sub-bullet modified to be at the same level as the previous two main sub-bullets (i.e., no change to the indentation in the current specification).

Note: Currently the definition of Pcmax,c is incomplete in the RAN4 specifications (TS38.101-3) for the EN-DC case. If PLTE, PNR are incorporated in the Pcmax,c definition in RAN4, the specification text addressed by the above agreement should be revised to take that into account.

Continue offline discussions on the following topics:

A-MPR on intra-band EN-DC (Qualcomm)

R1-1809833	Proposals on intra-band EN-DC MPR handling	Qualcomm

Phase continuity issues (Hong)

R1-1809823	Summary on the phase continuity issue of intra-band EN-DC  Intel

Agreement:

UE capability signalling that indicates per band per band combination whether the UE can handle (motivated by impacts of PA phase discontinuity) cases with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers is beneficial and is introduced.

- Only applicable to intra-band EN-DC, intra-band CA, FDM-based ULSUP (UL Sharing from UE Perspective)

  - Note: Normal UE behaviour is assumed when the UE does not indicate the lack of this capability through the capability signalling or when the capability is not applicable

- RAN4 to determine whether a UE that cannot handle such cases needs different performance requirements or whether these cases are treated as error cases.

- For intra-band EN-DC and FDM-based ULSUP, whether the capability is applicable also when NR is operated with different numerology than LTE should be determined in RAN4

- RAN4 to determine in which bands, in which band-combinations (including in which bands in the band-combination) the capability is applicable

  - Note: This is not intended to change other UE capabilities and minimum performance requirements related to the number of antennas and PAs as per the RAN4 requirements.

R1-1809967	[Draft] LS on UE capability for NR-LTE co-existence	Intel

Final LS approved in R1-1809992 with the following modifications:

- Change title to “LS on NR-LTE co-existence

- Add the agreement on sTTI and SPT to the draft LS in R1-1809967

- LS to be addressed to RAN4 and RAN2 with the same action, i.e., to take the agreements into account in their work.

R1-1809832	Limitations on EN-DC PHR	Qualcomm

R1-1809843	Offline discussion on LTE sTTI for EN-DC	LG

R1-1809885	Offline discussion on LTE sTTI for EN-DC LG

R1-1809874	Issue on EN-DC Dynamic Power Sharing with LTE SPS UL Skipping	Motorola Mobility, Lenovo

R1-1809954	Issue on EN-DC Dynamic Power Sharing with LTE SPS UL Skipping	Motorola Mobility, Lenovo

### Maintenance for UL power control

Including both non-CA and CA aspects, NR-NR DC related power control issues (if any) (please refer to RP-181473 for detailed scoping), etc.

Please use separate sections for each of the respective sub-topic to facilitate reviews and summary.

Friday

R1-1809915	Chairman's notes of AI 7.1.5 Maintenance for UL power control	Ad-hoc chair (Samsung)

Decision: The document is endorsed, content is incorporated below.

R1-1809772	Summary for Al 7.1.5 NR UL power control in non-CA aspects	ZTE

Conclusion

Editor to reflect the following existing agreement into the specification:

A UE is not expected to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions

Agreement

Following working assumption is confirmed

For PUCCH, K value for non-scheduled UL transmission is the minimum of the common configured K2 values

Agreement

The text in the paragraph on the PUSCH power control in {38.213: 7.1.1	UE behaviour}.

- If the PUSCH transmission is configured by higher layer parameter ConfiguredGrantConfig, is a number of symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by higher layer parameter k2 in PUSCH-ConfigCommon and for UL BWP of carrier of serving cell

Agreement

The text in the paragraph on the PUCCH power control in {38.213:7.2.1	UE behaviour}.

- If the PUCCH transmission is not in response to a detection by the UE of a DCI format 1_0 or DCI format 1_1, is a number of symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by higher layer parameter k2 in PUSCH-ConfigCommon and for UL BWP of carrier of serving cell

Agreement

The text in the paragraph on the SRS power control in {38.213: 7.3.1	UE behaviour}.

- if the SRS transmission is semi-persistent or periodic, is a number of symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by higher layer parameter k2 in PUSCH-ConfigCommon and for UL BWP of carrier of serving cell

Agreement

Editor to update the specification to reflect the following existing agreement:

- If {P0, alpha} is reconfigured for one closed loop process by higher layer change, the CL PC is reset.

- If {P0, alpha} is changed for one closed loop process by DCI, the CL PC is not reset.

Conclusion:

If the UE has reached  for UL BWP  of carrier  of serving cell  for closed loop l, the UE does not accumulate positive TPC commands for closed-loop l of UL BWP  of carrier  of serving cell .

- Up to the editor to capture the above conclusion

Agreement:

Power control for PRACH BFR should be captured in the specifications. Editor to come up with wording. Vivo tdoc provides some reference for TP.

Agreement

UE-specific P0 for PUCCH should be assumed as 0 before UE specific RRC configuration

Agreement

For PUCCH format 1, 10log10(OUCI) factor is added in ΔTF,b,f,c (i)

- FFS: Whether the same factor is added for PUCCH format 0

R1-1809778	Summary of NR UL power control – CA/DC aspects	Samsung

Agreement

In NR-NR DC where one cell group is in FR1 and the other in FR2, the two cell groups perform power control independently according to section 7 of 38.213.

- Applies for synchronous NR-NR DC for late drop of Rel-15 per RAN agreement

- It is RAN1 understanding that a common power limit for FR1+FR2 is not defined in RAN4 specifications

- It is up to the editor to whether and how to capture the above

Send an LS to RAN4 to inform them of the above agreement

R1-1809873	[Draft] LS on power control for NR(FR1)-NR(FR2) Dual Connectivity	Ericsson

Decision: The draft LS is endorsed and final LS is approved in R1-1809886.

R1-1809124	[Draft] reply LS on UE power class and UE capability on SRS carrier switch	Huawei

Power control issues to be resolved in the next RAN1 meeting on whether any specification change is needed or not:

- Issue#1: Cross-carrier indication for power control parameters for PUCCH, SRS and PUSCH

- Issue#2: UE behavior for PHR on SUL/non-SUL, e.g.,

  - For UEs capable of simultaneous transmission of PUSCH at SUL/non-SUL and transmission of SRS at the other UL carrier, if PHR is triggered, which type of PHR to report

- Issue#3: How to set P0_NOMINAL_PUSCH,f,c(j) for virtual PHR

  - Regarding the cell-specific P0 for virtual PHR, the cell-specific P0 for grant-free transmission should be used if P0alphasetindex = 0 is configured in higher layer parameter ConfiguredGrantConfig; otherwise, the cell-specific P0 for grant-based transmission should be used.

- Issue#4: Linkage between DL BWP and UL BWP for paired spectrum, e.g.,

  - PL is measured using reference signal for DL BWP which has the same BWP ID as UL BWP or PL is measured using reference signal for the active DL BWP.

Agreement

Selection of UL carrier for virtual SRS PHR

- For the PC setting for virtual SRS PHR in case of CA/DC, for a cell configured with non-SUL and SUL, a default UL carrier is selected according to the following rules:

  - The UL carrier configured for PUCCH if present, otherwise the non-SUL carrier

R1-1808070	Discussion on UE power control	Huawei, HiSilicon

R1-1808200	Maintenance for UL power control	ZTE

R1-1808228	Remaining issues on UL power control	vivo

R1-1808384	Remaining issues of NR power control	CATT

R1-1808597	Power scaling with transmissions associated to different MCS tables	InterDigital, Inc.

R1-1808676	Remaining Issues on UL Power Control	Intel Corporation

R1-1808758	Remaining Issues on UL Power Control	Samsung

R1-1808882	Text proposals for uplink power control	Guangdong OPPO Mobile Telecom

R1-1809074	Power control for Option 2 DC	AT&T

R1-1809145	Correction on EN-DC power sharing	NTT DOCOMO, INC.

R1-1809282	Further clarification on NR PUSCH power control	Nokia, Nokia Shanghai Bell

R1-1809295	Remaining issues on PUSCH power control	ASUSTeK

R1-1809398	Maintenance for UL power control	Motorola Mobility, Lenovo

Late submission

R1-1809410	Remaining issues on Power Control	Ericsson

R1-1809432	Maintenace for NR power control	Qualcomm Incorporated

### Maintenance for URLLC

Focusing on CQI/MCS aspects

R1-1809784	Summary of 7.1.6 Maintenance for URLLC	Ericsson

Decision: The document is noted.

R1-1809836	Text Proposal to 38.300 for URLLC	Ericsson

Decision: The document is noted.

Proposal

16.1 URLLC

16.1.4	CQI and MCS

For channel state reporting, A CQI table for target block error rate 10-5 is introduced.

For scheduling data packets with higher reliability, 64QAM MCS tables containing entries with lower spectral efficiency are introduced for both downlink and uplink. The tables are different for CP-OFDM and DFT-s-OFDM. The MCS tables can be configured semi-statically or dynamically. The dynamic signalling of MCS table is supported by configuring UE with new-RNTI, where new-RNTI indicates the 64QAM MCS tables with entries of lower spectral efficiency.

Offline discussion to idenfity impact of “new-RNTI” to NR specs such as 38.211, 38.213 and 38.214 and to capture in a document for the editors to take into account in the spec update.

Friday session

R1-1809869	Text Proposal to 38.300 for URLLC	Ericsson

Decision: The document is noted. The TP to 38.300 in R1-1809869 is endorsed. Send an LS to RAN2.

R1-1809984	[Draft] LS on Text Proposal to 38.300 for URLLC	Ericsson

Decision: The document is endorsed and final LS is approved in R1-1809991

R1-1809871	Text Proposal to Capture MCS-C-RNTI	EricssonAgreements:

Decision: The document is noted.

Agreements:

- Use “MCS-C-RNTI” instead of placeholder name “new-RNTI” in RAN1 specifications and TPs.

- The TPs in R1-1809871 are endorsed (38.211, 213, & 214)

R1-1809870	Text Proposal for MCS Determination	Ericsson

Decision: The document is noted.

Agreement:

- The TPs in R1-1809870 are endorsed

R1-1809837	Text Proposal for MCS determination	Ericsson

R1-1808496	Discussion on MCS table selection for URLLC	LG Electronics

R1-1808945	Correction on interrupted transmission indication in 213	Huawei, HiSilicon

R1-1809042	Maintenance of Rel-15 URLLC	Ericsson

R1-1809146	Maintenance for URLLC	NTT DOCOMO, INC.

R1-1809172	Corrections on MCS table determination	Nokia, Nokia Shanghai Bell

R1-1809733	Text proposal on physical layer mechanisms for URLLC in 38.300	Huawei, HiSilicon	(R1-1809349)

### NR Release 15 UE features

Including both eMBB and URLLC

Friday

R1-1809918	Chairman's notes of AI 7.1.7 UE features	Ad-hoc chair (NTT DOCOMO)

Decision: The document is endorsed, content is incorporated below.

R1-1808097	Discussion on remaining issues of UE features	Huawei, HiSilicon

R1-1808229	Rel-15 UE features for eMBB and URLLC	vivo

R1-1809723	Remaining issues in NR R-15 UE features	MediaTek Inc.

Revision of R1-1808259

R1-1808327	Remaining issues on Rel-15 NR UE features	ZTE

R1-1809506	On Rel-15 UE features	CATT

Revision of R1-1808385

R1-1808497	Discussion on UE features including URLLC	LG Electronics

R1-1808610	Discussions on NR UE feature list	Apple Inc.

R1-1808759	Remaining Issues on UE Features for Rel-15	Samsung

R1-1808974	On NR UE feature list	Ericsson

R1-1809147	Remaining issues on NR UE features	NTT DOCOMO, INC.

R1-1809712	On selected UE features	Nokia, Nokia Shanghai Bell

Revision of R1-1809394

R1-1809433	NR Features and Capabilities	Qualcomm Incorporated

R1-1809486	Rel-15 NR UE feature list	Intel Corporation

Conclusion

How/if the following component is captured in the spec will be discussed in the CR sessions.

7) Processing no more than one DCI with each RNTI in each of Type 0 CSS, Type 0A CSS, Type 1 CSS, Type 2 CSS, Type 3 CSS excluding unicast DCI per slot

Agreement

- RAN1 asks RAN2 to define the capability signalling for 1-8 in the LS.

- RAN1 and RAN2 will check other potential missing capability signalling.

R1-1809883	Proposal on Rel-15 UE feature group 6-23, 4-19 and 8-2	Intel

R1-1809905	Remaining issues for non-MIMO features and capabilities	Qualcomm

R1-1809884	Remaining issues on BWP features in R15 NR	MediaTek

Come back in RAN1#94bis

R1-1809942	LS on UE capability for simultaneousTxSUL-NonSUL	RAN2, Huawei

Possible agreement

RAN1 understanding is that simultaneousTxSUL-NonSUL capability is per band combination

Draft reply LS will be drafted by Zukang (Huawei)

Friday session

Agreement: RAN1 understanding is that simultaneousTxSUL-NonSUL capability is per band per band combination.

R1-1809999	Draft reply LS on UE capability for simultaneousTxSUL-NonSUL	Huawei

Decision: The document is endorsed with the following update:

In a band combination including SUL and multiple other NR bands, RAN1’s understanding is that the SUL carrier can be paired with a non-SUL carrier in any of the NR bands in the band combination. Thus,

Final LS is approved in R1-1810004.

R1-1809922	New UE features list for NR	NTT DOCOMO, AT&T

R1-1809989	UE UL beam capability	Qualcomm

Agreements

- R1-1809922 is endorsed, with the following updates:

  - Incorporating R1-1809989

  - Remove yellow-highlight of 2-24 (by updating “configured to measure”)

    - 2-24, add component 1A (which is the same as component 2)

  - 6-23, the tdoc # is R1-1809992

Agreements:

- Change [5-5b?] as indicated in red

|  | 2 [5-5b]? | UE PDSCH processing capability #2 with scheduling limitation for 30kHz-SCS | 1) Support of carrier aggregation? Capability #2 supported only if 1 carrier configured in the band (independent of #carriers configured in other bands) 2) Max PDSCH BW of 136 PRBs 3) N1 based on Table 5.3-2 of TS 38.214 for 30 kHz SCS |
| --- | --- | --- | --- |

The updated new UE feature list for NR is agreed in R1-1809998.

R1-1809923	[Draft] LS on new UE features list for NR	NTT DOCOMO, AT&T

Decision: The document is endorsed (by updating the new UE feature list tdoc #), and final LS is approved in R1-1810005.

### Other

R1-1808316	Clarification on UCI multiplexing on PUSCH	Fujitsu

R1-1809411	PUSCH power scaling in UL power control	Ericsson

R1-1809412	SRS power scaling in UL power control	Ericsson

R1-1809491	A comparison of polar and LDPC codes for URLLC	Polaran

R1-1810007	Correction and confirmation of some evaluation assumption parameters	LG Electronics

## NR in Release 16

### Study on non-orthogonal multiple access for NR

FS_NR_NOMA; SID in RP-181403. Please refer to the SID for detailed scoping

R1-1808150	Text proposal to capture agreements made in RAN1#93 to TR 38.812 (NOMA)	ZTE

Decision: The text proposal is endorsed.

MCC post meeting: Clean-up of the version endorsed in RAN1#94 and creation of v0.1.0 as baseline for further update (R1-1810046).

#### Transmitter side signal processing schemes for NOMA

R1-1809753	Discussion on the design of SCMA	Huawei, HiSilicon	(rev of R1-1808049)

- Observation 1: The structured symbol dependency in the bits-to-symbols mapping of SCMA provides more flexibility and better distance properties than sequence or repetition based linear spreading.

- Observation 2: The bits-to-symbols mapping design in SCMA provides more modulation granularity than the sequence or repetition based linear spreading.

- Observation 3: UE-specific sparse mapping of SCMA provides good tradeoff between the coding gain and the multi-user interference mitigation capability.

- Observation 4: SCMA provides flexible and scalable solution for NoMA transmission to address versatile KPIs in different application scenarios.

- Observation 5: With both CP-OFDM and DFT-s-OFDM waveform, SCMA can provide large SNR gain over sequence spreading based NoMA schemes with layer and/or spreading factor adaptation in all the cases simulated.

- Observation 6: Multi-layer sequence spreading based NoMA schemes without sparsity increase the PAPR for DFT-s-OFDM, compared to QAM baseline.

- Proposal 1: SCMA should be a candidate NoMA scheme of Rel-16 NoMA SI and captured in the TR.

Decision: The document is noted.

R1-1809148	Transmitter design for uplink NOMA	NTT DOCOMO, INC.

- Observation 1: UE-specific bit-level scrambling has already been supported in NR Rel-15. UE-specific bit-level interleaving can be equivalent to UE-specific symbol-level interleaving.

- Observation 2: Less iteration is needed for NOMA scheme with symbol-level spreading and lower decoding complexity can be expected compared to bit-level design.

- Observation 3: Higher throughput or larger overloading can be obtained by power domain NOMA than OMA.

- Observation 4 Low code rate coding may not be sure to give a significantly better performance than repetition but has a higher decoding complexity.

- Observation 5 Symbol level design is a more suitable way for NR NOMA schemes.

- Observation 6: Sequences targeting to generalized welch-bound equality under any powers can achieve optimal cross-correlation performance.

- Observation 7: NOMA with user grouping can achieve much higher sum throughput than existing NOMA schemes with WBE sequences and equal SNR.

- Observation 8: NOMA with user grouping has better flexibility and robustness on user number and TBS than existing NOMA schemes with WBE sequences and equal SNR.

- Proposal 1 Adopt power assignment and multi-branch based data processing units for NOMA.

- Proposal 2: Sequence grouping based on cross-correlations can be further discussed.

- Proposal 3: NOMA with user grouping should be considered as the candidate technique for further performance enhancement.

- Proposal 4: Sequences meeting generalized welch-bound equality (GWBE) have low cross-correlation and should be considered as one candidate of sequence pools for NOMA.

Decision: The document is noted.

R1-1809434	Transmitter side signaling processing schemes for NOMA	Qualcomm Incorporated

- Observation 1

  - When the network operates in grant-based mode, transmission schemes proposed for NOMA can be applied to MU-MIMO as well. The gain of NOMA over MU-MIMO in spectral efficiency is questionable, especially for underloading scenarios.

  - When the network operates in grant-free mode and the UL access is contention-free, the gain of NOMA over MU-MIMO in spectral efficiency is questionable.

  - The most significant gain of NOMA over MU-MIMO can be achieved in the following scenarios:

    - contention-based, grant-free transmission

    - small data transmission from RRC_INACTIVE state

- Proposal 1: NR NOMA solutions achieving performance gains over NR Rel-15 MU-MIMO should be prioritized in the study and evaluation.

- Proposal 2:  For normal cell coverage and DL synchronized UE, solutions capable of achieving a better tradeoff than 4-step RACH and CP extension in performance, latency and complexity should be studied for asynchronized NOMA.

- Proposal 3: The asynchronized NOMA transmission scheme in normal cell coverage, such as two-step RACH, should be studied, wherein MSG1 carries MA signature and small data, and UE does not need to perform timing advance prior to MSG1 transmission.

- Proposal 4: For grant-based UL transmission, the degree of freedom can be defined as the number of orthogonal physical resource units in time, frequency and space domains, which is measured for a given TTI, a given transmission bandwidth and a given number of transceiver antennas.

- Proposal 5: For synchronized NOMA transmission, TX solutions incapable of UE overloading and/or unsuitable for scalable configuration should be deprioritized and FFS whether they have significant performance gains over OMA or MU-MIMO with the same transceiver complexity.

- Proposal 6: NR NOMA should re-use linear modulations compliant with NR Rel-15. NR NOMA should consider the use of group or cell-specific scrambling at symbol level, multi-branch transmission and power domain multiplexing to optimize the tradeoff in performance and transceiver complexity.

- Proposal 7:  NOMA transmission scheme should provide flexibility and scalability to support different spectral efficiencies and overloading factors. To reduce the description complexity and memory size of transceivers, it should consider:

  - a closed form design for the short spreading codes that achieves Welch bound equality (WBE) for arbitrary spreading factor (K) and overloading ratio (N/K>1);

  - a closed from design for the long scrambling codes that is compliant with NR Rel-15;

  - reuse the same spreading codes and scrambling codes for both DFT-s-OFDM and CF-OFDM waveforms;

  - reuse linear modulations compliant with NR Rel-15 Tx chain.

- Proposal 8:   Intra-cell and inter-cell interference in NOMA transmission can be mitigated by UE grouping, power control, symbol-level scrambling, and the hopping of MA signatures. Specifically:

  - NOMA UEs within the same cell can be partitioned into multiple groups. Same or different short spreading codes achieving WBE can be applied to each group. Group-specific scrambling and power domain multiplexing can be applied to each UE group, wherein the configuration of scrambling codes can be made as a function of cell ID and UE group ID.

  - To average the intra-cell and inter-cell interference, the use of short spreading codes and long scrambling codes can be hopping in time domain.

- Proposal 9: The design of linear spreading scheme for NOMA should consider the inclusion of symbol-wise scrambling to improve the PAPR performance. The PAPR performance of QPSK can be used as a baseline.

Decision: The document is noted.

Wednesday session

R1-1809786	Offline summary of transmitter processing for NOMA	ZTE

Decision: The document is noted.

Agreements:

- Capture Section 2 and Annex in R1-1809786 into TR38.812.

  - Companies are encouraged to check the exact content of Section 2 and Annex in R1-1809786 and to provide comments if any by Friday

  - (Update on Friday) R1-1809974 – section 2 and Annex are agreed

- Comparnies to provide more detailed description of the proposed scheme (to the extent that solid simulations can be performed) by email by 8/28, and to be commented by 8/31.  These schemes are to be captured in R1-1809844 – Li (ZTE)

Friday session

R1-1809974	Updated summary of transmitter side signal processing schemes for NOMA	ZTE

Decision: see the agreements above.

R1-1808151	Transmitter side designs for NOMA	ZTE

R1-1808230	Discussion on NOMA transmitters	vivo

R1-1808331	Consideration on the transmitter side signal processing for NOMA	Sony

R1-1808386	NOMA transmitter side signal processing	CATT

R1-1808499	Transmitter side signal processing schemes for NCMA	LG Electronics

R1-1808563	Discussion on NOMA Schemes for NR	NEC

R1-1808677	NOMA transmission scheme	Intel Corporation

R1-1809499	Transmitter side signal processing schemes for NOMA	Samsung (rev of R1-1808760)

R1-1808834	Discussion on NoMA transmitter schemes	CMCC

R1-1809515	New Coding Scheme for NOMA with Traffic Control	Sequans Communications	(rev of R1-1808948)

R1-1808955	Discussion on general symbol-to-resource mapping for NR-NOMA	Fraunhofer HHI, Fraunhofer IIS

R1-1808968	Considerations on NOMA Transmitter	Nokia, Nokia Shanghai Bell

R1-1808975	TX schemes for NoMA	Ericsson

R1-1809056	On Transmitter Side signal Processing for NOMA	AT&T

R1-1809841	Discussion on Transmit Timing Offset for NOMA Evaluation	InterDigital, Inc.	(rev of R1-1809078)

#### Receivers for NOMA

R1-1808231	Receiver design considerations for NOMA	vivo

- Proposal 1: NR Rx discussion should focus on the typical receiver classification and should not take too much details of receiver implementation for performance and complexity comparison.

- Proposal 2: Performance of different Tx schemes are compared under the same Rx complexity class

- Proposal 3: For asynchronous scenarios, Rx classification could be based on the same rule as synchronous cases.

- Proposal 4: Tx scheme performance comparison should mainly be based on MMSE-IRC considering the commercial interest of the industry.

Decision: The document is noted.

R1-1808969	Receiver considerations for UL NOMA	Nokia, Nokia Shanghai Bell

- Observation 1: The choice of receiver architecture influences the size and latency of the receiver.

- Observation 2: Latency of the receiver depends on the computational complexity of each block, and the number of iterations.

- Proposal 1: The overall computational complexity is a good indication of the latency at a constant receiver size.

- Observation 3: The overall computation complexity depends on the complexity of each functional block of the receiver, the number of iterations and number of users processed per iteration.

- Proposal 2: For computational complexity analysis of the receiver for different NOMA schemes use Table 1 of x8969.

- Proposal 3: Use parameters in Table 2 of x8969 for computation complexity evaluation assuming eMBB user.

- Proposal 4: For NOMA evaluation consider an MMES-based Hard IC receiver.

- Proposal 5: For NOMA evaluation consider an ESE-based Soft IC receiver.

Decision: The document is noted.

R1-1809180	Discussion on Receiver for NOMA	KDDI Corporation

- Proposal 1: RAN1 should set a target performance to evaluate the receiver complexity for fair comparison.

- Proposal 2: Receiver evaluation relating to complexity should be based on the mMTC scenario as the first priority.

- Proposal 3: Performance evaluation for asynchronous transmission should take the additional signal processing into account, such as multiple FFT timing decision, multiple timing FFT, multi-user detection for each FFT timing and interference mitigation among different FFT timings, and should clarify the effectiveness from the aspects of performance gain by neglecting TA procedure and receiver complexity

- Proposal 4: For asynchronous transmission, effect of interference from asynchronous UEs to UEs for synchronous transmission including grant-based access which are simultaneously operated in the same system bandwidth should be evaluated

Decision: The document is noted.

Wednesday session

R1-1809787	Offline summary of receiver for NOMA	ZTE

Decision: The document is noted.

Thursday session

R1-1809859	Updated offline summary of receiver for NOMA	ZTE

Decision: The document is noted.

Agreements:

- The following table for computation complexity analysis of the receiver as the starting point, entries can be updated till RAN1#94bis.

Table I	Template of Receiver Computation Complexity breakup

| Receiver component | Detailed component | Computation in parametric number of usages, O(.) analysis, [impact factor] |  |  |
| --- | --- | --- | --- | --- |
|  |  | Receiver type 1 | Receiver type 2 | … |
| Detector | UE detection |  |  |  |
|  | Channel estimation |  |  |  |
|  | Rx combining, if any |  |  |  |
|  | Covariance matrix calculation, if any |  |  |  |
|  | Demodulation weight computation, if any |  |  |  |
|  | UE ordering, if any |  |  |  |
|  | Demodulation, if any |  |  |  |
|  | Soft information generation, if any |  |  |  |
|  | Soft symbol reconstruction, if any |  |  |  |
|  | Message passing, if any |  |  |  |
|  | Others |  |  |  |
| Decoder | LDPC decoding |  |  |  |
| Interference cancellation | Symbol reconstruction(Including FFT operations for DFT-S-OFDM waveform), if any |  |  |  |
|  | LLR to probability conversion, if any |  |  |  |
|  | Interference cancellation |  |  |  |
|  | LDPC encoding, if any |  |  |  |
|  | Others |  |  |  |

- The impact factor is to be estimated based on the analysis of computation, memory size, hardware and software implementation, etc.

  - If/How and which entries are to be combined/compared in order to get the total complexity estimate is FFS.

- Companies may provide the impact factor

  - The impact factor is for each cell

- The rows in the above table are subject to potential re-finement, e.g., adding new row(s), merge some rows, etc.

- Note: the numbers may or may not be a function of UL waveform

- FFS whether or not to add row(s) for memory blocks

R1-1808050	Discussion on the design of NoMA receiver	Huawei, HiSilicon

R1-1808152	Multi-user advanced receivers for NOMA	ZTE

R1-1808387	Discussion on NOMA receivers	CATT

R1-1808500	Considerations on the receiver types and NoMA schemes	LG Electronics

R1-1808678	NOMA receiver structure	Intel Corporation

R1-1808761	Receivers for NOMA	Samsung

R1-1808976	Receiver design for NOMA	Ericsson

R1-1809057	Receiver techniques for NOMA	AT&T

R1-1809079	Complexity Analysis of IDMA NOMA Reception 	InterDigital, Inc.

R1-1809149	Receiver for uplink NOMA	NTT DOCOMO, INC.

R1-1809435	Receivers for NOMA	Qualcomm Incorporated

#### Procedures related to NOMA

R1-1808153	Procedures related to NOMA	ZTE

- Observation 1: Configured grant transmission is not suited to eMBB small data or mMTC scenario due to its limitation in conveying an efficient small data transmission, in particular sporadically.

- Observation 2: NOMA based UL transmission in RRC-inactive can well support the usage in all cases with maximizing the benefits on signaling/latency and power saving.

- Observation 3: Collision of RS allocation/selection could happen in both configured-grant and purely grant free transmission.

- Observation 4: RS enhancement with directly increasing the overhead is not efficient.

- Observation 5: DM-RS enhancement via either reducing the density in frequency domain or introducing quasi-orthogonal sequence has negative impacts on the performance.

- Observation 6:  In case of fixed MA/RS allocation, up to 24 UEs can be supported with enhanced RS.

- Observation 7: In case of randomly selection of MA/RS allocation with timing offset within CP, simultaneously transmission with medium number of UE can be well supported with enhanced RS.

- Observation 8: The necessity of the enhancement on link adaption for NOMA based UL transmission should be further verified.

- Observation 9: Minor gain from network perspective can be expected with introducing the mechanism to enable the switching between NOMA and OMA.

- Proposal 1: UL data transmission from UE_INACTIVE state should be also studied to fully utilize the benefits of NOMA techniques.

- Proposal 2: Enhancement on the RS with different numerology should be considered to achieving the large pool size with supporting both UE detection and channel estimation.

- Proposal 3: The mechanism of HARQ via both explicit and implicit way should be considered for supporting NOMA transmission.

- Proposal 4: Additional enhancements, e.g., MA hooping or power ramping, can be considered to ensure the performance of re-transmission.

- Proposal 5: Study on the link adaption and transmission scheme switching should be down-prioritized in NOMA SI.

Decision: The document is noted.

R1-1808288	On NOMA procedures for Grant Free	Orange

- Observation 1: grant free NOMA benefits at least mMTC and URLLC

- Observation 2: To perform a grant-free uplink transmission, a UE has access to at least one pool of resources pre-allocated for grant-free access which are shared between several UEs. In mMTC scenarios, many UEs could share the same pool.

- Observation 3: Blind resource selection is very often considered for uplink grant free transmission, even if sub-optimal, it is due to the UE inability to acquire information on current grant-free resource use

- Observation 4: The gNB knows the information of the use of grant-free resources that UE are lacking. This lack of information leads them to perform sub-optimal blind resource selection.

- Proposal 1: The ability by the gNB to broadcast a real-time view of the use of the resource pool for grant-free access currently in operation should be considered, and its associated benefits should be evaluated with respect to a fixed or a pure random grant free resource selection.

- Proposal 2: Including into the broadcasted resource pool for grant-free access relevant statistical information on the use made by UEs configured to these resource pools should be considered and its associated benefits should be evaluated with respect to a pure random or fixed grant free resource selection. Which information is relevant depends on the considered NOMA techniques and should be further studied.

Decision: The document is noted.

Wednesday session

R1-1808762	Procedures related consideration to NOMA	Samsung

- Observation 1: Purely no TA acquisition, i.e., Type 3, suffers from severe ISI, ICI and channel estimation degradation in NR OFDM based system. Additional means are needed in order to support type 3 and should be carefully investigated.

- Observation 2: pure asynchronized case may need large CP overhead in order to support grant free transmission, subject to cell radius.

- Observation 3: Grant-based NoMA is more efficient way to schedule a number of UEs in the same resources compared to MU-MIMO.

- Observation 4: Grant-based NoMA is a good supplement to remedy weak points of grant-free NoMA.

- Observation 5: For grant-based NoMA, downlink control signalling should be minimized to support huge connection density at least for mMTC.

- Proposal 1: The channel structure of a preamble preceding a data payload can be considered for grant-free transmission based non-orthogonal multiple access.

- Proposal 2: the resource configuration for grant free transmission should consider the GFO indication, preamble/DMRS indication and MA signature indication.

- Proposal 3: Means to determine the GF-RNTI need to be studied.

- Proposal 4: For grant-based NoMA, an enhancement in downlink control siganling needs to be further studied.

Decision: The document is noted.

Friday session

R1-1809788	Offline summary of procedures for NOMA	ZTE

Decision: The document is noted.

Agreements:

- Consider mechanism to handle or mitigate the collision on MA signature/RS/resource, if needed

- FFS whether the number of configured MA signature/RS/resource from UE perspective can be 1 or multiple

- FFS whether multiple sets of MA signature/RS/resource can be configured to a UE

R1-1808051	Discussion on NoMA related procedures	Huawei, HiSilicon

R1-1808232	Discussion on NOMA procedures	vivo

R1-1808290	Considerations on procedures related to NOMA	Fujitsu

R1-1808332	Considerations on NOMA related procedures	Sony

R1-1808388	Discussion on NOMA procedures	CATT

R1-1808501	Discussion on procedures related to NOMA	LG Electronics

R1-1808564	Considerations on NOMA related procedures	NEC

R1-1808679	NOMA related procedure	Intel Corporation

R1-1808835	Discussion on NoMA related procedures	CMCC

R1-1808915	Consideration on NoMA procedure	Guangdong OPPO Mobile Telecom

R1-1808970	Procedures to be considered for NOMA operation	Nokia, Nokia Shanghai Bell

R1-1808977	Procedures for NoMA	Ericsson

R1-1809058	On NOMA Procedures	AT&T

R1-1809080	On NOMA Related Procedures in NR 	InterDigital, Inc.

R1-1809150	Extended DMRS design and NOMA related procedures	NTT DOCOMO, INC.

R1-1809181	Discussion on procedures related to NOMA 	KDDI Corporation

R1-1809190	Discussion on NOMA related procedures	ITRI

R1-1809283	Considerations on Procedures of Non-orthogonal Multiple Access Schemes	CAICT

R1-1809316	Considerations on MA Preamble Design for NOMA	KT Corp.

R1-1809379	Design consideration on NOMA procedure 	Convida Wireless LLC

R1-1809436	Procedures related to NOMA	Qualcomm Incorporated

R1-1809487	NOMA Procedure Considerations 	Sierra Wireless, S.A.

#### Link and system level performance evaluation for NOMA

R1-1808159	Email discussion on the templates of reporting LLS evaluation results	ZTE

Decision: The document is noted.

R1-1809789	Offline summary of performance evaluations for NOMA	ZTE

Decision: The document is noted.

Agreements:

- An exemplary list of simulation cases are included in the companion spreadsheet ‘template 1’ in R1-1809789, for initial collection of BLER vs. SNR curves.

  - Companies can select among the list of simulation conditions in templates 1 & 2 when performing initial link level simulations

    - Companies are encouraged to simulate enough cases to support a broad understanding for scenarios under study in NOMA

    - Additional simulation cases may be captured in template 1.

  - For unequal SNR distribution within range [x - a, x + a] (dB), per UE SNR is the average SNR in dB, i.e. x (dB)

- Adopt the companion spreadsheet ‘template 2’ in R1-1809789 as the template for collecting the initial evaluation results of per UE SNR at the target BLER level (in addition to BLER vs. SNR curve).

- CM/PAPR results as proposed for ‘template 3’ in R1-1809789 can be collected

  - It is FFS how the CM/PAPR relates to UE performance tradeoffs, PA backoff, and UE power saving

  - It is FFS how to compare the CM/PAPR results using this template, e.g. modulation order should be aligned or not

  - PAPR is reported as the CCDF of instantaneous power divided by mean power over all the samples.

Agreements:

- Refine the relative SINR and/or I(inter-cell interference)NR values used for link level simulations to reflect those observed in a cell, if needed

  - The extent of the refinement, if any, is to be determined according to evaluations.

Agreements:

- Further study how many NOMA UEs can be multiplexed in the same PRBs in practical multi-cell deployments by system-level evaluations, taking inter-cell interference and per UE performance into account

R1-1809790	Email discussion on the calibration results of reference OMA single user cases	ZTE	(rev of R1-1808160)

Decision: The document is noted.

Agreements:

- Adopt the calibration results in R1-1809790, to be captured in TR38.812.

  - To update the legends of the results by replacing companies’ names with the reference indexing

  - Further update is possible

- Further alignment on the decoding algorithm for NOMA evaluations can be considered in the future, if necessary.

  - Note: this is not intended to revise the corresponding calibration results

Thursday session

R1-1809860	Updated offline summary of performance evaluations for NOMA	ZTE

Decision: The document is noted.

Agreements:

- Determine the value y for the evaluation with non-zero timing offset (including asynchronous)

  - For Case 1: y = NCP/2

  - For Case 2: y = 1.5*NCP

Agreements:

- For random MA signature (including RS) in LLS, companies report the details of the chosen Option(s):

  - Opt 1: Fixed number of UEs, with each UE randomly selects a MA signature from a pre-configured MA signature pool

    - Number of potential UEs and the pool size should be reported

  - Opt 2: Fixed number of randomly activated UEs, with each potential UE’s MA signature pre-configured.

    - Number of potential UEs and the pool size should be reported

  - Realistic UE/MA signature detection should be performed.

  - DMRS extension, if any

  - FFS whether to align the pool size for performance evaluations.

Agreements:

- Adopt the additional assumptions in the following table for SLS calibration:

| Parameters | Assumptions |
| --- | --- |
| Minimum distance between UE and BS | 35m for Case 1, 10m for Case 2 and Case 3 |
| Building penetration loss | Follow the evaluation assumptions |
| TXRU mapping to antenna elements on BS side | One TXRU per vertical dimension per polarization |
| TXRU mapping weights on BS side | TXRU virtualization only in the vertical dimension, i.e., sub-array partition model with 1D virtualization, refer to TR36.897 |
| BS Tx power | 46 dBm for 500m/1732m ISD; 41 dBm for 200m ISD |
| Polarized antenna modeling | Model-2 in TR36.873 |
| Bandwidth | 10MHz |
| UE antenna configuration | 1 (vertical polarization) |
| UT array orientation | uniformly distributed on [0,360] degree |
| UE receiver noise figure | 9dB |
| UE distribution and antenna height | Follow the evaluation assumptions |
| Handover margin | 0dB |
| Wrapping method | Geographical distance based wrapping |
| UE attachment | Based on RSRP |

Agreements:

- The target higher layer system PDR to be used to evaluate the supported system capability in terms of high layer system PAR for mMTC or eMBB scenarios is 1%

- The target percentage of users satisfying reliability and latency requirements to be used to evaluate the supported system capability in terms per UE PAR for URLLC scenario is 95%

Agreements:

- At least for eMBB, companies are encouraged to report the distribution of PDCCH RB utilization (including PDCCH outage) in each slot at each gNB, which can be used for representing the signalling overhead for grants.

  - If used, companies report the system bandwidth and the scheduling mechanism used in simulation.

    - The simulation should contain both DL and UL transmissions.

Email discussion on SLS calibration and templates for collecting the SLS evaluation results, by 09/07 – Li (ZTE)

Friday session

R1-1809975	Updated offline summary of performance evaluations for NOMA	ZTE

Decision: The document is noted.

Agreements

- For performance metric of Sum throughput v.s. SNR at given BLER level, for a given pair of {per UE SE, # of UEs}

  - Same TBS is assumed for each curve

  - Total SNR is used

    - Total SNR is simplified as sum of average SNR conducted in dB, i.e. x+10*log10(N) (dB), where N is the number of UEs.

Agreement:

- Confirm frequency offset for 4GHz carrier frequency as

  - uniform distribution between -140 and 140 Hz

Agreements:

- For non-zero timing offset (for  asynchronous)

  - For all UEs in Case 1 or all UEs in Case 2, TO values for each UE for each transmission are i.i.d from uniform distribution [0, y], and independent between UEs.

  - For mixed sync and async, X% of UEs with zero TO and (100–X)% with non-zero TO.

    - X = 80

    - Other values are not precluded

  - Note: Companies should provide the details of receiver structure and TO estimation.

Agreements:

- The packet size for NOMA evaluations in eMBB scenario

  - 50~600 bytes Pareto distribution, with shaping parameter alpha = 1.5

- Clarification of ‘‘8 dBi, including 3dB cable loss” for both system-level evaluation and calibration as

  - 8dBi, 0dB cable loss

- In the case of packet segmentation, use 5 bytes packet segmentation overhead for each TB in the SLS evaluation of the NoMA schemes.

  - Company report the details of packet segmentation

R1-1809754	Further discussion on NoMA evaluations	Huawei, HiSilicon	(rev of R1-1808052)

R1-1808154	Prelimenary system-level simulation results for NOMA	ZTE

R1-1808204	Link-Level Simulation Results for ACMA with Synchronous, Asynchronous transmission	HUGHES Network Systems Ltd

R1-1808205	Partially Asynchronous and Multi-stream Transmission of ACMA	HUGHES Network Systems Ltd

R1-1808233	Evaluations for NOMA	vivo

R1-1808389	Link level and initial system level evaluation results of NOMA	CATT

R1-1808502	Further LLS evaluation for NCMA in mMTC	LG Electronics

R1-1808680	Performance evaluation for NOMA	Intel Corporation

R1-1809500	Performance evaluation for NoMA	Samsung	(rev of R1-1808763)

R1-1808971	Evaluations for NOCA	Nokia, Nokia Shanghai Bell

R1-1808978	Evaluation methodology and metrics for NoMA	Ericsson

R1-1809059	Link Level Performance for NOMA	AT&T

R1-1809081	System Level Simulation Methodology in mMTC Scenario 	InterDigital, Inc.

R1-1809151	Link and system level simulation results and analysis of UL NOMA	NTT DOCOMO, INC.

R1-1809284	Further LLS results of NOMA schemes	CAICT

R1-1809437	System level evaluation for NOMA	Qualcomm Incorporated

R1-1809438	Link level evaluation for NOMA	Qualcomm Incorporated

#### Other

R1-1808053	Evaluation results for OMA calibration	Huawei, HiSilicon

R1-1808054	Evaluation results for mMTC scenario	Huawei, HiSilicon

R1-1808055	Evaluation results for URLLC scenario	Huawei, HiSilicon

R1-1808056	Evaluation results for eMBB scenario	Huawei, HiSilicon

R1-1808155	Link-level simulation results for mMTC scenraio	ZTE

R1-1808156	Link-level simulation results for URLLC scenraio	ZTE

R1-1808157	Link-level simulation results for eMBB scenraio	ZTE

R1-1808158	Physical layer abstraction for NOMA SLS	ZTE

R1-1808390	Link level evaluation results of OMA	CATT

R1-1808391	System level calibration of NOMA	CATT

R1-1808541	A comparison of polar and LDPC codes for URLLC	Polaran

Withdrawn

R1-1808972	Initial Link Level Simulation Results of IDMA	Nokia, Nokia Shanghai Bell

R1-1808979	DMRS design for NoMA	Ericsson

R1-1808980	Impairments model for link and system evaluations	Ericsson

R1-1808981	Link level evaluations for MU-MIMO	Ericsson

R1-1808982	Observations on NOMA traffic models	Ericsson

R1-1808983	MU-MIMO operation in NR with configured grants	Ericsson

R1-1809082	On the impact of timing offset in NOMA Evaluation	InterDigital, Inc.

R1-1809083	Aspects of IDMA NOMA Transmitter 	InterDigital, Inc.

R1-1809084	Link Level Simulation Results on IDMA OFDM/DFT-s-OFDM 	InterDigital, Inc.

R1-1809152	LLS evaluation results of UGMA	NTT DOCOMO, INC.

R1-1809182	LLS evaluation results for OMA calibration	KDDI Corporation

### Study on NR-based Access to Unlicensed Spectrum

FS_NR_unlic; SID in RP-181339. Please refer to the SID for detailed scoping

Friday

R1-1809916	"Chairman's notes of AI 7.2.2	 Study on NR-based Access to Unlicensed Spectrum"	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content is incorporated below.

R1-1809439	TR38.889 v0.0.2	Qualcomm Incorporated

R1-1809795	TR38.889 v0.0.3	Qualcomm Incorporated

Version v0.0.3 of TR38.889 is endorsed.

MCC post meeting: Clean-up of the version endorsed in RAN1#94 and creation of v0.1.0 as baseline for further update (R1-1810047).

#### Simulation Methodology for NR-U operation

R1-1808317	Simulation Methodology and  Preliminary Results for NR-U	ZTE

R1-1808503	Simulation methodology for NR unlicensed operation	LG Electronics

R1-1808681	On calibrating the outdoor simulation models for NR-unlicensed	Intel Corporation

R1-1808764	Remaining simulation methodology for NR-U	Samsung

R1-1808816	Simulation Methodology for NR-U operation	Nokia, Nokia Shanghai Bell

R1-1808926	Simulation assumptions and preliminary results for NR unlicensed bands	Huawei, HiSilicon

R1-1809085	Simulation and calibration for NR-U outdoor sub-7GHz 	InterDigital, Inc.

R1-1809200	NR-U evaluation methodology	Ericsson

R1-1809440	Email discussion summary for sub7 outdoor evalution calibration	Qualcomm Incorporated

R1-1809441	Simulation methodology for NR unlicensed	Qualcomm Incorporated

R1-1809764	Feature summary on Simulation Methodology for NR-U operation	Qualcomm Incorporated

Agreements:

- The base metrics for NR-U evaluation are the same as in LTE-LAA in TR 36.889.

- For coexistence evaluations below 7GHz, for parameters not covered by previous agreements, the evaluations assumptions specified for LTE (e)LAA coexistence evaluations apply.

  - For example, the minimum distance between a small cell and a UE, and between two UEs is three meters.

Agreement:

- For coexistence evaluation, WiFi+WiFi, WiFi+NR-U and NR-U+NR-U evaluations are baseline with equal priority.

Agreement:

A=1.0 for scenario 1 and A=1.5 for scenario 2

Note: The agreed scenarios are the baseline for generation of results to be included in the TR. Contributions based on other scenarios are not precluded from being considered for discussion and decisions on NR-U design.

#### Frame structure for NR-U operation

R1-1808058	NR numerology and frame structure for unlicensed bands	Huawei, HiSilicon

R1-1808234	Frame structure for NR-U operation	vivo

R1-1808271	On frame structure design for NR-U operation	MediaTek Inc.

R1-1808291	On flexible starting positions and related PDCCH monitoring for NR-U	Fujitsu

R1-1808318	Discussion on frame structure for NR-U	ZTE

R1-1808333	Considerations on Frame Structure for NR Unlicensed Operations	Sony

R1-1808392	Frame Structure for NR Unlicencensed Operations	CATT

R1-1808504	Frame structure for NR unlicensed operation	LG Electronics

R1-1808543	Frame structure for NR-U operation	Lenovo, Motorola Mobility

R1-1808618	Numerology for NR-U	Panasonic Corporation

R1-1808682	On the NR-unlicensed frame structure	Intel Corporation

R1-1808765	Frame structure for NR-U	Samsung

R1-1808817	Frame structure for NR-U operation	Nokia, Nokia Shanghai Bell

R1-1808896	Frame structure for NR-U	Guangdong OPPO Mobile Telecom

R1-1809061	Frame structure for NR-U operation	AT&T

R1-1809086	On NR-U frame structure 	InterDigital, Inc.

R1-1809201	Frame Structure for NR-U	Ericsson

R1-1809220	Discussion on the frame structure for NR-U	Xiaomi

R1-1809227	Discussion on frame structure for NR-U operation	ASUSTEK COMPUTER (SHANGHAI)

R1-1809285	Discussions on frame structure design in NR-U	CAICT

R1-1809476	Frame structure for NR-U operation	Qualcomm Incorporated

R1-1809738	Summary on frame structure for NR-U	LG Electronics

Agreement:

- It is identified that being able to operate all DL signal/channels with the same numerology for a carrier and at least for intra-band CA on serving cells on unlicensed bands has at least the following benefits (at least for standalone operation, FFS whether this is benefit is realizable for inter-operator measurements)

  - Lower implementation complexity (e.g., a single FFT, no switching gaps)

  - Lower specification impact

  - No need for gaps for measurements on frequencies with a configured serving cell in unlicensed bands

- It is identified that being able to operate all UL signal/channels with the same numerology for a carrier and at least for intra-band CA on serving cells on unlicensed bands has at least the following benefits

  - Lower implementation complexity (e.g., a single FFT, no switching gaps)

  - Lower specification impact

  - Common interlace structure

  - No need for gaps for transmission of SRS on a configured serving cell in unlicensed bands

  - FFS: PRACH benefits

- FFS: same numerology for DL and UL considering switching gap

R1-1809972	Summary #2 on frame structure for NR-U	LG Electronics

#### Potential physical layer channel design

##### DL Signals and Channels

Including DL data channel, DL control channel, DL RS, Sync channel, etc.

R1-1808059	DL channels and signals in NR unlicensed band	Huawei, HiSilicon

R1-1808235	Discussion on physical DL channel design in unlicensed spectrum	vivo

R1-1808272	On downlink transmission detection in NR-U	MediaTek Inc.

R1-1808319	Considerations on DL reference signals and channels design for NR-U	ZTE

R1-1808334	Considerations on DL Signals and Channels for NR unlicensed operation	Sony

R1-1808393	DL Physical Channel and Signal Design for NR Unlicensed Operations	CATT

R1-1808505	Physical layer design of DL signals and channels for NR unlicensed operation	LG Electronics

R1-1808612	On DL signals and channels for NR-U	Apple Inc.

R1-1808683	Enhancements to NR DL signals and channels for unlicensed operation	Intel Corporation

R1-1808766	Downlink signal and channel design for NR-U	Samsung

R1-1808807	Discussion on SS_PBCH block in NR-U operation	Spreadtrum Communications

R1-1808897	DL design for NR-U	Guangdong OPPO Mobile Telecom

R1-1808917	On DL Signals and Channels for NR-U	Nokia, Nokia Shanghai Bell

R1-1809062	NR-U design for DL signals and channels	AT&T

R1-1809087	On downlink signals and channels for NR-U 	InterDigital, Inc.

R1-1809184	Discussion on SSB transmission for NR-U	ITRI

R1-1809202	On DL signals and channels	Ericsson

R1-1809380	Discussion on partial slot transmission in NR-U	TCL Communication Ltd.

R1-1809477	DL signals and channels for NR-U	Qualcomm Incorporated

R1-1809791	Feature lead summary for NR-U DL Signals and Channels	Motorola Mobility, Lenovo

R1-1809822	Feature lead summary for NR-U DL Signals and Channels	Motorola Mobility, Lenovo

Agreement:

- Inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) in addition to the SS/PBCH burst set in one contiguous burst (tentatively referred to as the NR-U DRS) can be beneficial for

- Meeting OCB requirement

- Compacting signals in time domain to limit the required number of channel access and for short channel occupancy

- Support of stand-alone NR-U deployments

- Support of automatic neighbour relations (ANR) functionality in an NR-U deployment

- Resolution of PCI confusion in an NR-U deployment

- Note: The NR-U DRS (it can be called something else in the future) can include signals and channels that are required for cell acquisition etc. and is not limited only to reference signals

- The transmission of additional signals such as OSI and paging within the NR-U DRS is allowed and can be beneficial

- Note: This does not imply that RMSI-CORESET+PDSCH and CSI-RS can only be transmitted as part of the NR-U DRS, and does not imply that these are necessarily part of all NR-U DRS transmissions.

##### UL Signals and Channels

Including UL data channel, UL control channel, UL RS, PRACH, etc.

R1-1808060	UL PHY channels for NR unlicensed	Huawei, HiSilicon

R1-1808236	Discussion on physical UL channel design in unlicensed spectrum	vivo

R1-1808273	On UL signals and channels in NR-U	MediaTek Inc.

R1-1808292	On UL signals and channels design for NR-U	Fujitsu

R1-1808320	Considerations on UL reference signals and channels design for NR-U	ZTE

R1-1808394	UL Physical Channel and Signal Design for NR Unlicensed Operations	CATT

R1-1808506	Physical layer design of UL signals and channels for NR unlicensed operation	LG Electronics

R1-1808544	UL signals and channels for NR-U operation	Lenovo, Motorola Mobility

R1-1808613	On UL signals and channels for NR-U	Apple Inc.

R1-1808619	Resource allocation for NR-U UL channels	Panasonic Corporation

R1-1808684	Enhancements to NR UL signals and channels for unlicensed operation	Intel Corporation

R1-1808767	Uplink signal and channel design for NR-U	Samsung

R1-1808798	Discussion on UL Signals and Channels in NR-U	Spreadtrum Communications

R1-1808818	On uplink signal and channel structures for NR-U	Nokia, Nokia Shanghai Bell

R1-1808898	UL design for NR-U	Guangdong OPPO Mobile Telecom

R1-1809088	On uplink signals and channels for NR-U 	InterDigital, Inc.

R1-1809153	UL Signals and Channels for NR-U operation	NTT DOCOMO, INC.

R1-1809203	On UL signals and channels	Ericsson

R1-1809324	Discussion on UL signals and channels for NR-U operation	WILUS Inc.

R1-1809478	UL signals and channels for NR-U	Qualcomm Incorporated

R1-1809726	Feature lead summary for UL Signals and Channels	Ericsson

Agreement:

- For scenarios in which a block-interlaced waveform is used for PUCCH/PUSCH, it has been identified that from FDM-based user-multiplexing standpoint it can be beneficial to have UL channels on a common interlace structure, at least for PUSCH, PUCCH, associated DMRS, and potentially PRACH

  - Note: This is only from a user-multiplexing perspective. Other aspects of PRACH design need to be considered, i.e., timing estimation accuracy, miss detection rate, PAPR, RACH capacity, transmission power

- For scenarios in which a contiguous allocation for PUSCH and PUCCH is used, it is beneficial to use contiguous resource allocation for PRACH

- FFS: Potential LBT blocking due to TA difference between FDM’d PUSCH, PUCCH, and PRACH

R1-1809819	Feature lead summary 2 for UL Signals and Channels	Ericsson

Agreement:

- For scenarios in which a block-interlaced waveform is used for UL transmission, a PRB-based block-interlace design has been identified as beneficial at least for 15 and 30 kHz SCS, and potentially for 60 kHz SCS

  - Link budget limited cases with given PSD constraint

    - It is observed that power boosting gains decrease with increasing SCS

  - As one option to efficiently meet the occupied channel bandwidth requirement

  - Comparatively less specification impact than Sub-PRB interlace design

- Design for 60 kHz requires further discussion, e.g., sub-PRB vs. PRB-based block interlace designs

- The following has been observed for sub-PRB block interlace designs

  - In some scenarios sub-PRB interlacing can be beneficial in terms of power boosting

    - FFS: scenario details, e.g., small resource allocations

  - Sub-PRB interlace design has at least the following specification impact:

    - Reference signal design (e.g., DMRS)

    - Channel estimation aspects

    - Resource allocation

Agreement:

- It has been identified as beneficial to support a block-interlaced structure in which the number of interlaces (M) decreases with increasing SCS, and the nominal number of PRBs per interlace (N) is similar for each SCS (in a given bandwidth) at least for 15 and 30 kHz SCS, and potentially 60 kHz depending on supported interlace design

- FFS: M and N for each supported SCS

- FFS: 60 kHz in case a sub-PRB interlace is introduced

Agreement:

- From a RAN1 perspective it has been identified that supporting a non-uniform interlace structure in which the number of PRBs per interlace is allowed to be different for different interlaces is beneficial from a spectrum utilization point of view

  - FFS: Exact number of PRBs per interlace for supported value(s) of M and N

    - Note: M is the number of interlaces and N is the nominal number of PRBs per interlace in a given bandwidth

  - FFS: Whether or not there are issues in the interlace design in the resource allocation to 2^n1*3^n2*5^n3 in the case of DFT-s-OFDM

#### Potential physical layer procedures

##### Channel access procedures

R1-1808061	Coexistence and channel access for NR unlicensed band operations	Huawei, HiSilicon

R1-1808237	Discussion on the channel access procedures	vivo

R1-1808253	Discussion on synchronized user multiplexing for NR-U	TCL Communication Ltd.

R1-1808274	On channel access procedure in NR-U	MediaTek Inc.

R1-1808293	On channel access procedure for NR-U	Fujitsu

R1-1808321	Discussion on Channel access mechanism for NR-U	ZTE

R1-1808335	Considerations on channel access for NR unlicensed operations	Sony

R1-1808359	Discussion on subband based channel access procedures for NR-U	NEC

R1-1808395	Channel Access Procedures for NR Unlicensed Operations	CATT

R1-1808507	Channel access procedure for NR unlicensed operation	LG Electronics

R1-1808611	Channel access procedures for NR-U	Apple Inc.

R1-1808685	Enhancements to channel access mechanism for NR-unlicensed	Intel Corporation

R1-1808768	Channel access procedure for NR-U	Samsung

R1-1808819	Channel access and co-existence for NR-U operation	Nokia, Nokia Shanghai Bell

R1-1808899	Channel access mechanisms on NR-U	Guangdong OPPO Mobile Telecom

R1-1809063	Enhancements for the NR-U channel access procedure	AT&T

R1-1809089	Channel access procedure and coexistence in NR-U 	InterDigital, Inc.

R1-1809111	Channel access procedures for NR unlicensed operation	Sharp

R1-1809154	Channel access procedures for NR-U operation	NTT DOCOMO, INC.

R1-1809204	Channel access mechanisms for NR-U	Ericsson

R1-1809248	Channel access procedure in NR-U	ASUSTEK COMPUTER (SHANGHAI)

R1-1809296	Discussion on proposed channel access schemes for NR-Unlicensed	BROADCOM LTD

R1-1809300	Extensions for Channel Access Procedures	Motorola Mobility, Lenovo

R1-1809325	Discussion on channel access procedure for NR-U operation	WILUS Inc.

R1-1809381	Design considerations on channel accessing	Convida Wireless LLC

R1-1809479	Channel access procedure for NR-U	Qualcomm Incorporated

R1-1809821	WF on the use of No-LBT and one-shot LBT for transmissions outside a shared COT	Broadcom, CableLabs, BlackBerry, HPE (Aruba), Ruckus

R1-1809801	Feature Lead’s Summary on Channel Access Procedures	Nokia, Nokia Shanghai Bell

Agreement:

In addition to aspects considered in LTE LAA, CWS adjustment procedure in NR-U may additionally consider at least the following aspects:

- CBG based HARQ-ACK operation,

- NR scheduling and HARQ-feedback delays and processing times

- wideband (>20 MHz) operation including BWPs

- Configured grant operation

##### Initial access and mobility

R1-1808062	Initial access in NR unlicensed	Huawei, HiSilicon

R1-1808238	Discussion on initial access and mobility procedure for NR-U	vivo

R1-1808275	On initial access procedure in NR-U	MediaTek Inc.

R1-1808322	Discussion on initial access and mobility for NR-U	ZTE

R1-1808336	Considerations on initial access and mobility for NR unlicensed operations	Sony

R1-1808396	Initial Access and Mobiity for NR Unlicensed Operations	CATT

R1-1808508	Initial access and mobility for NR unlicensed operation	LG Electronics

R1-1808686	Enhancements to initial access and mobility for NR-unlicensed	Intel Corporation

R1-1808769	Initial access and mobility procedure for NR-U	Samsung

R1-1808900	Initial access procedure on NR-U	Guangdong OPPO Mobile Telecom

R1-1808920	Discussion on initial access for NR-U	Potevio

R1-1808951	On Initial Access and Mobility for NR-U	Nokia, Nokia Shanghai Bell

R1-1809064	NR-U design for initial access and mobility	AT&T

R1-1809090	On Initial Access and Mobility for NR-U 	InterDigital, Inc.

R1-1809179	SI Transmission in NR-U	Charter Communications

R1-1809185	Enhancement of configured grant for NR unlicensed band	ITRI

R1-1809205	On initial access, RRM, mobility and RLM	Ericsson

R1-1809219	Initial access in NR Unlicensed	Xiaomi Technology

R1-1809492	Diversity in RACH transmissions	Motorola Mobility, Lenovo	(Revision of R1-1809315)

R1-1809326	Discussion on initial access and mobility for NR-U operation	WILUS Inc.

R1-1809399	Initial access and mobility in NR-U	Motorola Mobility, Lenovo

Withdrawn

R1-1809402	Discussion on initial access in NR-U	KT Corp.

R1-1809473	Initial access and mobility for NR-U	ITL

R1-1809480	Initial access and mobility for NR-U	Qualcomm Incorporated

R1-1809727	Feature lead summary #1 of initial access and mobility	Charter Communications

R1-1809826 	Feature lead summary #1 of initial access and mobility	Charter Communications

Agreement:

- It is recommended to define a mechanism to transmit SSBs dropped due to LBT failure

- Following are examples of candidate mechanisms for further consideration with enhancements or modifications not precluded:

  - Alt-1: Shift SSB(s) in time to the next transmission instance

  - Alt-2: Cyclically wrap the SSBs dropped due to LBT failure around to the end of the burst set transmission

  - Alt-3: Network to flexibly position SSB index and indicate the timing information

  - Other alternatives are not precluded

- It is recommended to define a mechanism for UE(s) to determine the timing and QCL assumptions from the detected SSB

Agreement:

If preamble transmissions are dropped due to LBT failure, then

- From a RAN1 perspective, it is recommended that preamble power ramping is not performed and that the preamble transmission counter is not incremented

Agreement:

- In some scenarios it is beneficial for the maximum RAR window size to be extended beyond 10 ms to increase robustness to DL LBT failure

  - FFS: Value of maximum RAR window size

Agreement:

It is beneficial to support reporting of RSSI

- FFS: The time and frequency resources on which RSSI is measured

##### Potential HARQ enhancements

R1-1808063	HARQ enhancement in NR unlicensed	Huawei, HiSilicon

R1-1808239	Discussion on HARQ operation for NR-U	vivo

R1-1808276	Enhancements on HARQ for NR-U operation	MediaTek Inc.

R1-1808323	Discussion on scheduling and HARQ for NR-U	ZTE

R1-1808337	Considerations on HARQ enhancements for NR unlicensed operations	Sony

R1-1808360	Discussion on HARQ enhancements for NR-U	NEC

R1-1808397	HARQ Operations in NR-U	CATT

R1-1808509	HARQ procedure for NR unlicensed operation	LG Electronics

R1-1808545	HARQ-ACK enhancement for NR-U	Lenovo, Motorola Mobility

R1-1808687	Enhancements to HARQ for NR-unlicensed	Intel Corporation

R1-1808770	HARQ enhancements for NR-U	Samsung

R1-1808811	Consideration on HARQ enhancements in NR-U	Spreadtrum Communications

R1-1808820	HARQ Operation in NR-Unlicensed	Nokia, Nokia Shanghai Bell

R1-1808895	HARQ enhancements for NR-U	Guangdong OPPO Mobile Telecom

R1-1809065	HARQ enhancements for NR-U	AT&T

R1-1809091	On HARQ enhancements for NR-U 	InterDigital, Inc.

R1-1809186	Potential HARQ enhancements for NR-U	ITRI

R1-1809206	HARQ enhancements for NR-U	Ericsson

R1-1809286	Discussions on HARQ enhancement for NR-U	CAICT

R1-1809403	Considerations on HARQ operation for NR-U	KT Corp.

R1-1809481	Potential HARQ enhancements for NR-U	Qualcomm Incorporated

R1-1809750	Feature lead summary of HARQ enhancement in NR-U	Huawei, HiSilicon

R1-1809921	Feature lead summary of HARQ enhancement in NR-U	Huawei, HiSilicon

Agreement:

- NR-U should support both:

  - HARQ feedback corresponding to some or all the PDSCHs of a channel occupancy can be reported in the same channel occupancy

    - It is found beneficial to extend the PDSCH-to-HARQ-timing to support indicating timings up to the end of the longest COT allowed by regulations, one or more of the following would be needed:

      - Allow values larger than 15 by RRC signaling (FFS the largest value needed)

        - Note: in some cases this may point outside of the COT

      - Allow more bits for the PDSCH-to-HARQ-timing-indicator

  - HARQ feedback corresponding to PDSCHs of a channel occupancy can be reported outside of that channel occupancy. These possible candidate solutions can be considered:

    - Alt1: gNB requests/triggers feedback for PDSCH from earlier COT(s)

    - Alt2: UE is configured to report HARQ feedback for PDSCH from earlier COT(s) without an explicit request/trigger

    - Alt3: by PDSCH-to-HARQ-timing-indicator in the DCI scheduling the PDSCH

    - Note: the alternatives above are at least applicable for the case where there is no HARQ feedback expected in the same channel occupancy as the PDSCH

    - Study the impact of the above candidate solutions on the HARQ codebook

##### Potential enhancements to configured grants

R1-1808064	Transmission with configured grant in NR unlicensed band	Huawei, HiSilicon

R1-1808240	Discussion on the enhancements to configured grants	vivo

R1-1808277	Discussion on NR-U configured grant	MediaTek Inc.

R1-1808324	Discussion on configured grant for NR-U	ZTE

R1-1808338	Considerations on Enhancements to Configured Grants for NR Unlicensed operation	Sony

R1-1808361	Discussion on configured grants in NR-U	NEC

R1-1808510	Discussion on configured grant for NR unlicensed operation	LG Electronics

R1-1808688	Enhancements to NR configured grants for unlicensed operation	Intel Corporation

R1-1808771	Enhancements on configured grant for NR-U	Samsung

R1-1808806	Consideration on autonomous UL transmission in NR-U operation	Spreadtrum Communications

R1-1808821	On support of UL transmission with configured grants in NR-U	Nokia, Nokia Shanghai Bell

R1-1808901	Configured grant transmission on NR-U	Guangdong OPPO Mobile Telecom

R1-1809092	Configured grant enhancements in NR-U 	InterDigital, Inc.

R1-1809207	Configured UL grant for NR-U	Ericsson

R1-1809382	Autonomous Uplink Transmission in NR Unlicensed	Convida Wireless LLC

R1-1809482	Potential enhancements to configured grants for NR-U	Qualcomm Incorporated

R1-1809747	Summary of views on the potential enhancements to configured grants	Intel Corporation

Agreement:

Allowing consecutive configured grant resources in time without any gaps in between the resources and non-consecutive configured grant resources (not necessarily periodic) with gaps in between the resources is beneficial and should be considered for NR in unlicensed spectrum

R1-1809825	Summary of views on the potential enhancements to configured grants	Intel Corporation

Conclusion:

There is no necessity to exclude Type-1 or Type-2 configured grant mechanism for operation of NR in unlicensed spectrum.

Agreement:

UE selects the HARQ process ID from an RRC configured set of HARQ IDs for NR-unlicensed configured grant transmission.

Agreement:

It is identified to be beneficial to support DFI to include pending HARQ ACK feedback for prior configured grant transmissions from the same UE.

- FFS: DFI to include HARQ ACK feedback for scheduled UL transmissions using HARQ IDs configured for NR-unlicensed configured grant transmission.

Agreement:

- Retransmission via same configured grant resource is supported for a HARQ process that was initially transmitted via configured grant resource.

- Retransmission via resource scheduled by UL grant is supported for a HARQ process that was initially transmitted via configured grant resource.

Agreement:

UE may autonomously initiate retransmission for a HARQ process that was initially transmitted via configured grant mechanism for NR-unlicensed when one of the following conditions is met:

- Reception of NACK feedback via DFI for the corresponding HARQ process

- FFS: No reception of feedback from gNB upon the timer expiration.

  - To introduce a new timer or reuse configuredGrantTimer.

Agreement:

It is identified to be beneficial to consider UE multiplexing and collision avoidance mechanisms between configured grant transmissions and between configured grant and scheduled grant transmissions.

- FFS: detailed mechanism.

Agreement:

NR-unlicensed configured grant transmission is not allowed during the time when it overlaps with occasions configured for potential NR-U DRS of the serving cell irrespective of the configured time domain resource for configured grant transmission.

##### Other

R1-1808065	BWP operation in NR unlicensed band	Huawei, HiSilicon

R1-1808825	On Wideband Operation for NR-U	Nokia, Nokia Shanghai Bell

R1-1809208	On SRS design for unlicensed operation	Ericsson

R1-1809209	On interlace design for NR-U uplinks	Ericsson

R1-1809210	Indoor coexistence evaluation results for NR-U and WiFi	Ericsson

#### Other

R1-1808066	Zero-Autocorrelation zone of frequency-interlaced PRACH	Huawei, HiSilicon

R1-1808257	Discussion on user multiplexing for NR-U	TCL Communication Ltd.

R1-1808258	Discussion on Frequency Reuse for NR-U	TCL Communication Ltd.

R1-1808689	On the consideration for spatial reuse for NR-unlicensed	Intel Corporation

R1-1808690	Initial design on technology neutral preamble for unlicensed operation	Intel Corporation

R1-1808826	On Maximum TX Power for UL Interlace Structures	Nokia, Nokia Shanghai Bell

R1-1809223	Wideband operation in NR unlicensed	PANASONIC

### Study on Integrated Access and Backhaul for NR

FS_NR_IAB; SID in RP_181349. Please refer to the SID for detailed scoping

R1-1809488	Work plan for Integrated Access and Backhaul Study Item		AT&T, KDDI, Qualcomm, Samsung

Decision: The document is noted.

R1-1809489	TR38.874, Version 0.4.0	AT&T, KDDI, Qualcomm, Samsung

Discussion:Sharp: is the document content the same for all WG? AT&T --> yes, RAN2 lead WI - the TR provides the current status of progress.

Decision: The document is noted.

#### Enhancements to support NR backhaul links

R1-1808101	Physical layer design for NR IAB	Huawei, HiSilicon

- Proposal 1: Study whether the maximum number of candidate SSBs in Rel-15 is sufficient to serve both access UEs and IAB nodes for initial access.

- Proposal 2: Support TDM of SSBs for IAB nodes and access UEs considering the following two options (1) TDM within a SSB burst set; (2) TDM across SSB burst sets with dedicated periodicity and offset settings for IAB nodes.

- Proposal 3: The SSBs for IAB node discovery and RRM measurement can be placed at the frequency location different from those for initial access, in order not to cause UE confusion.

  - Off-raster frequency position can be an option for further study

- Proposal 4: The SSBs for IAB node discovery/measurement among IAB nodes can be overlapped.

- Proposal 5: For a given IAB node, its SSB transmission should be muted to facilitate the measurement for other IAB nodes if its SMTC window is overlapped with its SSB transmission.

- Proposal 6: For IAB node discovery and backhaul link RRM measurement, the principle defined in Rel-15 should be a starting point and further optimization can be studied for backhaul links to save measurement overhead

  - Longer SMTC period and CSI-RS measurement period

  - Aperiodic measurement configuration

- Proposal 7: Faster beam failure recovery mechanism for backhaul link should be studied.

- Proposal 8: Beam failure notification mechanism to child node and child node’s behavior should be studied.

- Proposal 9: Semi-static slot configuration for each IAB node should be supported,

  - For L2 IAB, the configuration is determined by the CU of IAB donor according to centralized coordination, and configured by the CU to the DU of is associated IAB nodes

  - For L3 IAB, the configuration is determined by each IAB node according to distributed coordination

- Proposal 10: An IAB node can be configured with a set of slots for backhaul and access transmission including the link directions.

- Proposal 11: For the backhaul-access dynamic sharing slot, the PDCCH time location for IAB node MT’s backhaul link and the PDCCH for its DU’s access link should meet the following requirement

  - The backhaul link PDCCH (BH-PDCCH) should be ahead of its associated PDSCH/PUSCH

  - The backhaul link PDCCH (BH-PDCCH) should be ahead of the access link PDCCH (AC-PDCCH)

- Proposal 12: To support FDM and SDM between backhaul and access links, enhanced downlink power control scheme on backhaul link should be studied in order to solve the power imbalance issue between backhaul and access links.

- Proposal 13: For FR1, it should be studied whether TA/2 timing adjustment for OTA based DL Tx timing synchronization can work for multi-hop topology.

- Proposal 14: It should be studied how the DL Tx timing of IAB node can be configured in an accurately and timely manner to avoid timing error propagation and system performance degradation.

- Proposal 15: It should be studied how to maintain IAB node local DL Tx timing in case of route switching after it finishes initial DL Tx timing configuration.

- Proposal 16: In order to support symbol alignment between access and backhaul link within IAB node, additional TA adjustment on IAB node or UE should be further studied.

- Proposal 17: In addition to symbol alignment, other mechanisms to enable SDM based interference mitigation should also be studied, at least including

  - DMRS orthogonal configuration between access and backhaul links

  - Interference mitigation receiver

- Proposal 18: Both distributed and centralized interference management schemes should be studied for inter-IAB CLI management.

- Proposal 19: RAN1 should study the enabling mechanisms to support the inter-IAB node CLI mitigation method, at least including

  - Time/frequency coordination

  - Beam coordination

  - Power control

- Proposal 20: Up to 1024QAM should be considered for backhaul link.

- Proposal 21: Phase noise compensation performance improvement on backhaul link should be investigated to support higher order modulation above 64QAM. Two possible directions are: 1) PT-RS redesign 2) Compensation algorithm improvement, e.g., CPE with ICI.

Decision: The document is noted.

R1-1808578	NR support for IAB	Nokia, Nokia Shanghai Bell

- Observation 1: IAB node discovery can be handled by (access UE) SSB search in the initial access, followed by CSI-RS detection in later phases.

- Observation 2: NM-patterns for IAB SSB TX-RX would provide discovery opportunities for a group of IAB nodes within a fixed number of SSB set TX/RX windows but co-ordination for pattern reuse would be needed.

- Observation 3: From access UEs’ point of view, IAB SSBs could be handled the same way as access UE SSBs that are not connected to SIB1.

- Observation 4: CSI-RS based IAB link monitoring of candidate links can be done like UE measurements for mobility except that the maximum periodicity could be larger for IAB.

- Observation 5: Resource allocation schemes developed for different access link scenarios can be reused for BH link.

- Observation 6: Depending on the IAB architecture and protocol choices, RRC signalling may need to be enhanced to support multiple hops with centralized resource pool coordination.

- Observation 7: Random access of the UE part of the IAB node is less frequent compared to access UEs.

- Observation 8: For sharing RACH resources between IAB and UE access, the maximum round-trip time (i.e. cyclic shift step) should be estimated based on the IAB hop length rather than the maximum path loss supported by IAB node for a RACH format.

- Proposal 1: RAN1 is asked to consider the need for IAB specific SSB transmissions and if the CSI-RS (or SRS) based detection and monitoring of the IAB nodes is sufficient between nodes that have done cell selection and network access.

- Proposal 2: Distributed resource pool coordination scheme is the baseline resource allocation scheme. It can be enhanced by centralized coordination by IAB-donor.

- Proposal 3: Granularity for the resource pool coordination between IAB-donor and IAB-nodes is one slot.

- Proposal 4: GC-PDCCH enhancements are needed to facilitate dynamic resource allocation between the backhaul and access links as well as centralized resource coordination.

- Proposal 5: RN processing time capabilities need to be discussed as part of the IAB studies.

- Proposal 6: Multiplexing RACH transmissions of access UEs and IAB nodes can be handled within the Rel-15 framework.

- Proposal 7: It should be clarified whether there will be standard impacts related to use of different timing options or could they be left for implementation options.

- Proposal 8: Options leading to synchronized network can be taken as a baseline for IAB operation and deployment. The other options can be left for later studies and/or choices for implementation.

Decision: The document is noted.

R1-1808691	PHY layer enhancement to support NR IAB	Intel Corporation

- Observation 1: Different PRACH Configuration Index does not guarantee the orthogonality of PRACH transmitting time in an IAB RN and its child node.

- Observation 2: The semi-static configuration on cell-specific DL-UL-F configurations and the set of slot format combinations in Rel-15 NR design is expected to be sufficient for time-domain resource allocation for IAB nodes.

- Observation 3: Current NR design support frequency resource allocation across IAB node in RB level.

- Observation 4: IAB relaying is effective in improving coverage. But the user perceived throughput could be undermined due to half-duplexed constraint.

- Proposal 1: For NR IAB node discovery and RRM, use Solutions 1 or 5 as baseline solution and use Solution 3 as a supplementary solution.

- Proposal 2: To coordinate the SSB resource or periodicity across IAB nodes, either RRC or F1-AP signalling can be applied.

- Proposal 3: For UE or RN cell selection, the backhaul link condition should be factored in. The following two options are preferred considering latency, power consumption and specification effort:

  - Option 1a: use cellBarred indication in MIB;

  - Option 2: adjust RSRP threshold in initial access based on backhaul link condition.

- Proposal 4: For Rel-16, no additional specification effort is needed for IAB node selection considering backhaul link condition. Further enhancement can be considered in future releases.

- Proposal 5: For NR IAB synchronization, support Case 1 for NR IAB.

- Proposal 6: For an IAB RN to operate in half-duplex mode, the PRACH Configuration Index of the IAB RN and its child node needs to be carefully chosen to guarantee the orthogonality among adjacent hops.

- Proposal 7: Study approaches to offset the impact of half-duplex constraint on per link and overall system performance.

- Proposal 8: Consider link adaptation based on measurement that emulates the upcoming interference level for NR IAB CLI management.

- Proposal 9: Clarify that in NR IAB, if FDM and SDM are supported, it should assume that the IAB node has multiple antenna panels and multiple basebands each serving different frequency bands or transmission directions.

- Proposal 10: Study mechanisms on IAB node power saving.

Decision: The document is noted.

R1-1809442	Enhancements to support NR backhaul links	Qualcomm Incorporated

--- RACH design ---

Observation 1.1: The ratio of backhaul link distance to access link distance might be much higher than two in NR.

Observation 1.2: If access and backhaul links use same resources for RACH, supportable number of cyclic shifts for each ZC root sequence decrease significantly.

- This, in turn, leads to the use of higher number of ZC root sequences in each cells and higher interference during RACH transmission.

Observation 1.3: Periodicity of backhaul RACH resources, that are orthogonal to access RACH resources, can be extended to reduce overhead.

Proposal 1.1: NR allows network the flexibility to configure access and backhaul RACH resources with different occasions and periodicities.

- Different mechanisms can be considered to enable this, e.g., additional bits can be introduced in RMSI to indicate backhaul CBRA occasions.

--- inter-relay discovery and measurements ---

Proposal 2.1: to address the half-duplex constraint in the IAB-node discovery and measurements, support different TX/RX coordination patterns. The patterns may be (pseudo-)random, semi-persistent/periodic, or dynamically configured.

Proposal 2.2: support configuring the TX/RX pattern both centrally (baseline) and in a distributed manner.

- FFS the required signaling.

Observation 2.1: Sporadic SSB transmissions, following a muting pattern, may confuse the initial access UEs, if they are transmitted on a sync raster.

Proposal 2.3: use off-raster SSBs for IAB discovery and measurements.

Proposal 2.4: simple modifications to the NR R15 RRM framework should be considered to make it more suitable for backhaul operations, such as

- configuring new values for CSI-RS/SMTC periodicity (e.g. larger than 160 msec),

- increasing the maximum number of SMTC configured per frequency,

- supporting more flexible time-domain location of the SSBs within a SMTC,

- leveraging SMTC design to configure transmission windows (STTC).

Proposal 2.5: adopt the proposed framework in Table 5 for initial acquisition, inter-IAB-node discovery and BH measurements.

--- link failure recovery ---

Observation 3.1: In Rel-15, RLF is declared after a N310 consecutive OOS are detected and T310 timer runs for the configured time without receiving N311 consecutive IS indications.

- In Rel-15, failure of beam recovery procedure leads to RLF declaration.

- In Rel-15, Success of beam recovery procedure does not impact RLF procedure.

Observation 3.2: Radio link monitoring and candidate beam reference signals can be different.

- Communication can happen via candidate beam reference signal even if radio link monitoring reference fail.

Observation 3.3: Declaring RLF is costly for an IAB UEF node.

- The corresponding IAB DU will have to release all resources of its child nodes.

Proposal 3.1: NR supports success of beam recovery procedure stopping the T310 timer and preventing IAB UEF to declare RLF.

--- over-the-air synchronization ---

Observation 4.1: TA-based OTA synchronization can support multi-hop IAB network (4~5 hops) for mmw bands. TA-based OTA synchronization is not sufficient to support multiple hops in lower bands.

Proposal 4.1: To tighten the OTA timing error, especially in lower bands, RAN1 should consider the following solutions:

- Using wider band signals (UL and DL) to achieve more accurate timing estimation

- Enhancing TA, e.g. by reducing its granularity and increasing the number of bits

Proposal 4.2: IAB TR should mention TA-based OTA synchronization can support up to [5] hops in MMW bands.

Proposal 4.3: IAB network should use other synchronization techniques, such as GNSS and PTP, along with OTA techniques to achieve tight network synchronization.

Proposal 4.4: it should be further studied how timing adjustment of an IAB-node (following OTA synchronization) impacts the operation of its child IAB-nodes and UEs.

--- multi-hop timing alignment ---

Proposal 5.1: the timing alignment case 1 (where slot boundaries across multi-hop IAB network are aligned) is adopted as the baseline.

- The other cases that allow simultaneous transmissions and/or receptions over two adjacent hops should also be supported.

- It is left to network, which case to select.

--- resource allocation and coordination ---

Proposal 6.1: A semi-static resource partitioning shall be supported as a baseline approach, where a set of resources, called schedulable resources, are determined for each IAB-node to schedule its child nodes.

- To support a centralized implementation of the semi-static resource partitioning schemes. New upper-layer (F1-AP) signaling is needed to indicate the required reports for selecting schedulable resources and provide the selected resources.

  - Exact signaling format is out of RAN1 scope.

- The granularity for schedulable resource pattern shall be configurable.

- A schedulable resource pattern for an IAB-node can be associated with only a subset of its child links.

- Interaction between schedulable resource pattern and the resource allocation (e.g. RRC-configured resources) shall be further studied, including e.g. conflict resolution rules.

Proposal 6.2: A dynamic coordination approach shall be supported with potentially using enhanced slot format indication framework.

--- cross-link interference ---

Observation 7.1: in a homogenous IAB network, the worst-case SINR (due to BH-to-BH interference) is sufficiently high: >20dB at 10% and >30dB at 50%.

Observation 7.2: IAB CLI scenarios are very similar to cross-link (in flexible TDD) and inter-cell interference scenarios in the access network.

Proposal 7.1: IAB CLI study should be unified with other CLI scenarios and worked out in the CLI WI.

--- power control ---

Proposal 8.1: the power of the concurrent access and backhaul links (multiplexed in frequency or spatial)

- should be controlled semi-statically – at least for the transmissions that cannot dynamically change the power (like DL RS for RRM, periodic CSI-RS)

- can be determined dynamically for the remaining transmissions.

- FFS: enhancements to allow more efficient SDM/FDM of the access and backhaul links.

Decision: The document is noted.

Thursday session

R1-1809857	Summary of 7.2.3.1 Enhancements to support NR backhaul links	AT&T

Decision: The document is noted.

Agreements:

- In case of SA deployments, initial IAB node discovery by the MT (Stage 1) follows the Rel.15 procedure for cell search and initial access based on SSBs available for access UEs without additional required specification support.

  - FFS: how to support NSA deployment

  - FFS whether Stage 2 enhancements (if any) can be applied to Stage 1

Agreements:

- For the purpose of inter-IAB node and donor detection after the IAB node DU becomes active (Stage 2) at least one of the following solutions should be supported:

  - SSB-based solutions (Solution 1):

      - Solution 1-A) Reusing the same set of SSBs used for access UEs

      - Solution 1-B) Use of SSBs which are orthogonal (TDM and/or FDM) with SSBs used for access UEs

    - Mechanisms to support half-duplex transmission/measurement of SSBs (e.g. muting patterns) for Solution 1-A) or Solution 1-B)

    - Further study potential impacts of the above solutions on access UEs performing initial access/in IDLE mode, including:

      - Cell detection/measurement performance impact due to loss of SSB occasions due to muting

      - Discovery of SSBs by access UEs which are intended only for IAB node discovery

  - CSI-RS based solutions (Solution 2)

    - Feasibility of CSI-RS only based discovery in case of unsynchronized network operation

- Further study enhancements to existing configurations (e.g. SMTC and CSI-RS configuration) and inter-node coordination (e.g. F1) for Solutions 1) or 2) and possibility of aperiodic transmission of SSBs/CSI-RS

Agreements:

- IAB supports the ability of network flexibility to configure backhaul RACH resources with different occasions, periodicities, and/or formats, compared to access RACH resources without impacting Rel.15 UEs

  - Further study mechanisms under current PRACH design framework to ensure that after initial access, IAB nodes and access UE of its mother node can be configured or identify TDMed PRACH occasions.

  - Further study the need for new RACH formats/configurations specific for IAB node random access

Agreements:

- For the purpose of backhaul link measurements IAB supports both SSB and CSI-RS for backhaul link RSRP/RSRQ RRM measurements. Further consider the following aspects:

  - Enhancements to Rel.15 CSI-RS and SSB measurement configurations and required coordination

Agreements:

- To support RLM/RLF procedures for IAB nodes, the following should be further studied:

  - Enhancements to support interaction between Beam Failure Recovery success indication and RLF

  - Enhancements to existing beam management procedures for faster beam switching/coordination/recovery to avoid backhaul link outages should be considered for IAB nodes

Agreements:

- Study the need for additional backhaul link condition notification mechanism from the parent IAB node DU to the child IAB node as well as corresponding IAB node behavior.

  - E.g., if the parent IAB node’s backhaul link fails (RLF or BF)

  - Note: this study is intended to focus on RAN1 aspecs only (any higher layers aspects are to be handled by other WGs)

Agreements:

Capture the following definitions in the TR:

- Access link: a link between an access UE and an IAB node or IAB donor (LA,DL or LA,UL)

- Backhaul link: a link between an IAB node and an IAB child node (LC,DL or LC,UL)or IAB parent node (LP,DL or LP,UL)

- Note: the IAB node may have its functions for UL access and child BH respectively in the same location or different locations, which is not the scope of RAN1 discussion

- Note: For a given BH link for an IAB node, it may be a parent BH or a child BH, depending on the topology/architecture, which is not the scope of RAN1 disucssion

Proposals:

To support TDM of parent and child links, where child links can include access links to UEs and/or backhaul links to child IAB nodes, a pattern which is specific to an IAB node is configured which indicates resources in time available for parent and child links:

- Further study details of the adaptation period and granularity (e.g. slot or symbol-level) of the pattern provided to the IAB node, including

  - Explicit or implicit indication of the resources

  - Enhancements to existing signaling mechanisms to indicate the pattern

  - Potential need to differentiate the resources for UL child backhaul and UL access transmissions in time

- Further study the indication of resources within the configuration which can be dynamically and flexibly used for parent and child links by the IAB node, including

  - The need to consider the scheduling delay or information required to be available for the use of flexible resources

  - Mechanisms to schedule flexible resources (e.g. GC-PDCCH)

- Note: this does not address the question of how access and child backhaul links are multiplexed by an IAB node

Agreements:

An IAB node can support the following cases using existing MU-MIMO or sectorization mechanisms:

- Case A: multiplexing of DL transmissions to access UEs and child IAB nodes

- Case B: multiplexing of UL transmissions from access UEs and child IAB nodes

Agreements:

Clarify the SDM scenario definition:

- SDM Tx: An IAB node simultaneous transmits in the DL (to an access UE and/or child IAB node) and transmits in the UL (to a parent IAB node)

- SDM Rx: An IAB node simultaneous receives in the DL (a transmission from a parent node) and receives in the UL (from an access UE and/or child IAB node)

For the support of SDM, further study the following aspects:

- Transmit power coordination between parent and child links

- Considerations of single panel vs. multi-panel operation (single or multiple baseband)

- Requirements of symbol-level timing alignment within an IAB node (e.g. Case #6/Case #7)

Agreements:

- At least Case #1 is supported for both access and backhaul link transmission timing.

- Further study includes additionally the following two cases (in addition to other cases #2/3/4/5)

  - Case #6 (Case#1 DL transmission timing + Case #2 UL transmission timing):

    - the DL transmission timing for all IAB nodes is aligned with the parent IAB node or donor DL timing (e.g. TA/2 adjustment as in Case #1)

    - the UL transmission timing of an IAB node can be aligned with the IAB node’s DL transmission timing

  - Case #7 (Case#1 DL transmission timing + Case #3 UL reception timing):

    - the DL transmission timing for all IAB nodes is aligned with the parent IAB node or donor DL timing (e.g. TA/2 adjustment as in Case #1)

    - the UL reception timing of an IAB node can be aligned with the IAB node’s DL reception timing

  - FFS: TA required for IAB nodes to support these cases

  - For Case #6 and Case #7 further consider the potential impact of imperfect timing adjustment, overhead of required DL/UL switching gaps, and scheduling impact on access UEs and child IAB nodes

  - Study to include aspects (including feasibility) when the IAB node is connected to one or multiple parent nodes

Friday session

R1-1809941	Summary of 7.2.3.1 Enhancements to support NR backhaul links	AT&T

Decision: The document is noted.

Agreements:

- For the support of TDM, at least the following cases are supported:

|  | TDM Between: |  |  |
| --- | --- | --- | --- |
| Case | Link 1 | Link 2 | Supported by a pattern? |
| 1 | LP,DL | LC,DL | Yes |
| 2 | LP,UL | LC,UL | Yes |
| 3 | LP,DL | LC,UL | Yes |
| 4 | LP,UL | LC,DL | Yes |
| 5 | LP,DL | LA,DL | Yes |
| 6 | LP,UL | LA,UL | Yes |
| 7 | LP,DL | LA,UL | Yes |
| 8 | LP,UL | LA,DL | Yes |
| 9 | LP,DL | LA,DL and LC,DL | Yes |
| 10 | LP,UL | LA,UL and LC,UL | Yes |
| 11 | LP,DL | LA,UL and LC,UL | Yes |
| 12 | LP,UL | LA,DL and LA,DL | Yes |
| 13 | LC,DL | LA,DL | * At least Rel. 15 mechanisms can be used, FFS enhancements |
| 14 | LC,UL | LA,UL | * At least Rel. 15 mechanisms can be used, FFS enhancements |
| 15 | LC,DL | LA,UL | * At least Rel. 15 mechanisms can be used, FFS enhancements |
| 16 | LC,UL | LA,DL | * At least Rel. 15 mechanisms can be used, FFS enhancements |

Note: A given pattern may include support for multiple cases, details FFS.

At least for Cases 1-12, an IAB node is configured with IAB-node specific resources in time available for the links:

- Further study details of the adaptation period and granularity (e.g. slot or symbol-level) of the pattern provided to the IAB node, including

  - Explicit or implicit indication of the resources

  - Enhancements to existing signaling mechanisms to indicate the pattern

- Further study the indication of resources within the configuration which can be dynamically and flexibly used for different links, including

  - The need to consider the scheduling delay, IAB node processing delays, or information required to be available for the use of flexible resources

  - Mechanisms to schedule flexible resources (e.g. GC-PDCCH)

R1-1808241	Enhancements to support NR backhaul links	vivo

R1-1808339	Discussion on Enhancements to support NR backhaul links	Sony

R1-1808398	NR Physical Layer design for IAB backhual link	CATT

R1-1808511	Discussions on NR IAB support	LG Electronics

R1-1808551	Discussion on resource partitioning for IAB network	Lenovo, Motorola Mobility

R1-1808772	Necessary Enhancements for NR IAB	Samsung

R1-1808836	Discussion on enhancements to support NR Backhaul links	CMCC

R1-1808879	Discussion of backhaul link  enhancement for IAB	Guangdong OPPO Mobile Telecom

R1-1808918	Discussion on IAB node discovery and measurement	Potevio

R1-1809072	Enhancements to support NR backhaul links	AT&T

R1-1809103	Discussion on physical layer enhancements for IAB	ZTE, Sanechips

R1-1809155	Discussion on enhancements to support NR backhaul links	NTT DOCOMO, INC.

R1-1809231	IAB physical layer enhancements for backhaul link management	Ericsson

R1-1809250	Discussion on Timing Alignment for IAB nodes	ASUSTEK COMPUTER (SHANGHAI)

#### Evaluation methodology for NR IAB

Including remaining details on deployment scenarios and evaluation assumptions, evaluation results, etc.

R1-1809073	Evaluation methodology and simulation results for NR IAB	AT&T

Observation 1: Multi-hop backhaul links experience significant gains in RSRP compared to access links.

Observation 2: Multi-hop backhaul networks benefit from resource allocation partitioning across backhaul links.

Observation 3: IAB provides coverage and system performance improvements even when nodes are subject to a half-duplex constraint.

Proposal 1: Capture the observations in this contribution in TR 38.874. RAN1 should further study potential gains and techniques for resource allocation and interference mitigation across multiple backhaul hops.

Decision: The document is noted.

R1-1809817	Evaluation Methodology for NR IAB	Samsung	(rev of R1-1808773)

The contribution presents initial evaluation results on IAB topology constructions for the heterogeneous scenario (200m and 500m inter macro BS distances and 3 micro BSs per macro BS) based on the agreed evaluation assumptions from the last meeting.

For 200m inter macro BS distance, it can be observed that ~66% of the relay nodes are directly connected to the macro BS (first hop relay nodes), 28% of the relay nodes are connected to the first hop relay nodes, and 6% of the relay nodes are connected to the second hop relay nodes. For 500m inter macro BS distance, it can be observed that ~70% of the relay nodes are directly connected to the macro BS (first hop relay nodes), 25% of the relay nodes are connected to the first hop relay nodes, and 5% of the relay nodes are connected to the second hop relay nodes. Finally, the long term SINR CDFs of the wireless backhaul links under half-duplex constraint per node are provided.

Decision: The document is noted.

R1-1809232	More details on IAB evaluation methodology	Ericsson

- Observation 1: When the IAB node is assumed to have 3 panels, the current two options of adding panel orientation do not serve properly to multi-hop topology formation. Option 1 which adds panel orientation after topology formation may change the link strength towards the potential child nodes and therefore make the current topology no longer optimal. Option 2 which uses random orientation degrades the advantage of using directional antenna panels.

- Observation 2: In case of multi-hop topology, using only the per-link pass-loss or RSRP to determine the parent node does not consider the end-to-end performance achieved by the IAB chain from the donor all the way to the outermost IAB nod.

- Observation 3: Multi-hop IAB deployment improves the connection between the UE and the serving IAB donor comparing to the single-hop relay.

- Proposal 1: The panel orientation of IAB nodes should be included in the process of topology formation.

- Proposal 2: The decision of parent node for a given IAB node should be based on the end-to-end performance from the donor to the IAB node instead of only based on the link performance between the parent node and the IAB node.

- Proposal 3: The decision of associating an IAB node to a parent node should not only consider for maximizing the performance of users served by the new IAB node, but also consider for minimizing the impact on the performance of all affected higher-level nodes that have already been in the network as well as the users served by those IAB nodes.

- Proposal 4: Adding weights of each IAB node regarding the served traffic or the number of hops to topology formation should be studied to avoid link congestion.

- Proposal 5: The tradeoff between the connection improvement brought by the multi-hop deployment and the associated disadvantages, e.g., increased overhead, aggregated traffic, changed interference environment, etc., should be further investigated.

Decision: The document is noted.

Thursday session

R1-1809816	Summary of 7.2.3.2 Evaluation Methodology for NR IAB	AT&T

Decision: The document is noted.

Agreements:

- Update the Heterogeneous scenario with the following additional network dropping assumptions:

| Distance | ISD 500m | ISD 200m |
| --- | --- | --- |
| Minimum distance between Micro TRPs | 40m | 40m |
| Minimum distance between Macro TRP and UE | 35m | 10m |
| Minimum distance between Micro TRP and UE | 10m | 10m |
| Minimum distance between Micro TRPs and Macro TRP | 40 m | 20m |

Agreements:

The following factors can be considered as input to the IAB node parent-node selection, in addition to parent-node RSRP as measured by the IAB node

- Number of hops to between the candidate parent node to the donor node

- “Capacity” measures (downlink and uplink) of links on the path between the candidate parent node to the donor node

  - e.g. min RSRP of a route, harmonic mean of RSRP, Shannon capacity of the link, IAB node capability

- Load (downlink and uplink) of the candidate parent node as well as nodes on the paths between the candidate parent node to the donor node

  - Examples: Number of IAB nodes and access UEs served by a certain node

Agreements:

Add the following to the list of reported metrics for IAB evaluations:

- Distribution of minimum backhaul link RSRP of a given route between an IAB node and IAB donor

- Distribution of number of child IAB nodes per IAB node and per IAB donor

- Distribution of number of access UEs per IAB donor

- Hop count distribution

Agreements:

- At least for the purpose of IAB evaluations, when the IAB node has multiple panels, RAN1 assumes access and backhaul traffic can be sent on any panel, subject to the per IAB-node half duplex constraint.

Friday session

R1-1809978	TP for 38.874 on Evaluation Methodology for NR IAB	AT&T

Decision: The document is noted.

Agreement:

- The TP in Section 2 of R1-1809978 to 38.874 is endorsed

R1-1808090	System performance evaluation in multi-hop topology IAB network	Huawei, HiSilicon

R1-1808512	Remaining issue on evaluation assumptions in IAB scenario	LG ELECTRONICS

R1-1808579	Evaluation of IAB	Nokia, Nokia Shanghai Bell

R1-1808692	Evaluation methodology for NR IAB	Intel Corporation

R1-1809443	Evaluation methodology for NR IAB	Qualcomm Incorporated

#### Other

R1-1808085	Consideration on IAB node discovery and RRM measurement	Huawei, HiSilicon

Withdrawn

R1-1808086	On resource coordination and dynamic scheduling in IAB	Huawei, HiSilicon

Withdrawn

R1-1808087	On IAB node synchronization and timing alignment	Huawei, HiSilicon

R1-1808088	Consideration on cross-link interference in IAB	Huawei, HiSilicon

R1-1808089	On high order modulation in IAB	Huawei, HiSilicon

R1-1808513	Discussions on discovery among NR IAB nodes	LG Electronics

R1-1808514	Discussions on access and backhaul link timing for NR IAB	LG Electronics

R1-1808515	Discussions on node behavior for IAB link management	LG Electronics

R1-1808516	Discussions on HARQ-ACK transmission for NR IAB	LG Electronics

R1-1808517	Initial access procedure in IAB scenario	LG ELECTRONICS

R1-1808552	Discussion on discovery and measurement for IAB network	Lenovo, Motorola Mobility

R1-1808580	Resource allocation between backhaul and access link	Nokia, Nokia Shanghai Bell

R1-1808581	Measurements with IAB	Nokia, Nokia Shanghai Bell

R1-1808582	IAB Timing	Nokia, Nokia Shanghai Bell

R1-1808583	Enhancements on RACH for IAB	Nokia, Nokia Shanghai Bell

R1-1808774	CLI Management in NR IAB	Samsung

R1-1808904	Utilising sidelink in IAB	Guangdong OPPO Mobile Telecom

R1-1809233	Transmission timing of IAB nodes	Ericsson

R1-1809444	IAB resource management	Qualcomm Incorporated

R1-1809445	Network synchronization for multi-hop IAB	Qualcomm Incorporated

### Study on NR V2X

FS_NR_V2X; SID in RP-181480. Please refer to the SID for detailed scoping

R1-1808518	Work plan of Study on NR V2X	LG Electronics

Discussion: CATT don't think LTE PC5 design should be reused

Decision: The document is noted. For information.

R1-1808942	TR for Study on NR V2X	Huawei, HiSilicon

Decision: The skeleton of TR 38.885 is endorsed.

#### Sidelink design

R1-1808092	Overview of general PHY sidelink design	Huawei, HiSilicon,

R1-1808905	Discussion on V2X use cases and priority of work in RAN1	Guangdong OPPO Mobile Telecom

R1-1809043	Overview of NR Sidelink Design	Nokia, Nokia Shanghai Bell

##### Support of unicast, groupcast and broadcast

R1-1808307	Sidelink design for unicast and groupcast	Fraunhofer HHI

Observation 1: Existing blind retransmission mechanisms in V2X broadcast are not spectrally efficient to support V2X unicast.

Proposal 1: Study different CQI reporting mechanisms and its implications for in-coverage NR V2X UEs.

Proposal 2: Study the implications for introducing HARQ for unicast and groupcast SL in NR V2X.

Proposal 3: Evaluate the suitability of blind retransmission mechanisms for NR V2X.

Decision: The document is noted.

R1-1808519	Discussion on support of unicast, groupcast and broadcast for sidelink	LG Electronics

Proposal 1: HARQ feedback is supported at least for unicast. FFS for groupcast and broadcast.

Proposal 2: Further study is needed whether to adopt a separate discovery channel.

Observation 1: Identifier for communication can be included in physical control channel.

Decision: The document is noted.

R1-1808775	Discussion on unicast, groupcast and broadcast for NR V2X	Samsung

Observation 1:

- High layer unicast/groupcast causes useless decoding and increases UE power consumption.

- It is better to use a ID in physical layer to identify a unicast/groupcast transmission. An ID is also required to support HARQ transmission and other physical layer functions.

Observation 2:

- Discovery is needed before setting up unicast transmission of a pair of UEs.

- HARQ based data transmission could be more resource efficient. Further discussion on how to support HARQ-ACK/CSI feedback in NR V2X.

Observation 3:

- Discussion on how to distinguish between groupcast and broadcast is needed.Discovery procedure can be used to form a group of UEs doing groupcast transmission.

- Add/release of UEs to a groupcast group may be needed.

- Further study on HARQ-ACK feedback and increasing retransmission efficiency. Further study on obtaining channel state information.

Observation 4:

- Strive to have a common design to support unicast, groupcast and broadcast.

Decision: The document is noted.

R1-1809156	Support of unicast, groupcast and broadcast	NTT DOCOMO, INC.

- Proposal 1: Support UE grouping procedure in NR V2X.

- Proposal 2: Rel-13 D2D relay UE discovery and selection procedure can be a start point to study UE grouping procedure.

- Proposal 3: Study the necessity to specify discovery channel (PSDCH) to support UE grouping procedure.

- Proposal 4: Study UE grouping procedure based on base station instruction, e.g., using SL measurement report.

- Proposal 5: Study link level QoS management mechanism at least for unicast and/or groupcast transmission, e.g., HARQ-ACK feedback, CSI feedback, SL pathloss compensation and closed-loop power control.

  - FFS: Method to distinguish unicast, groupcast and broadcast in PHY layer

- Proposal 6: Study the following PSCCH/PSSCH configuration for unicast, groupcast and broadcast transmission.

  - Common PSCCH/PSSCH resource configuration among unicast, groupcast and broadcast

  - Dedicated PSCCH/PSSCH resource configured for unicast, groupcast and broadcast respectively

Decision: The document is noted.

Tuesday session conclusion: Continue discussion offline – Hanbyul (LGE)

Wednesday

R1-1809798	Offline summary for 7.2.4.1.1 Support of unicast, groupcast and broadcast	LG Electronics

Decision: The document is noted.

Agreements:

- RAN1 assumes that higher layer decides if a certain data has to be transmitted in a unicast, groupcast, or broadcast manner and inform the physical layer of the decision. For a transmission for unicast or groupcast, RAN1 assumes that the UE has established the session to which the transmission belongs to. Note that RAN1 has not made agreement about the difference among transmissions in unicast, groupcast, and broadcast manner.

- RAN1 assumes that the physical layer knows the following information for a certain transmission belonging to a unicast or groupcast session. Note RAN1 has not made agreement about the usage of this information.

  - ID

    - Groupcast: destination group ID, FFS: source ID

    - Unicast: destination ID, FFS: source ID

    - HARQ process ID (FFS for groupcast)

  - RAN1 can continue discussion on other information

Send an LS to RAN2 and SA2 for the above agreements – Hanbyul, R1-1809834

Agreements:

- RAN1 to study the following topics for the SL enhancement for unicast and/or groupcast. Other topics are not precluded.

- HARQ feedback

- CSI acquisition

- Open loop and/or closed-loop power control

- Link adaptation

- Multi-antenna transmission scheme

Thursday session

R1-1809834	[Draft] LS on support of unicast, groupcast, and broadcast	LG Electronics

Decision: The draft LS is endorsed by updating action to “provide feedback if necessary”. Final LS is approved in R1-1809907.

R1-1808091	Overview of sidelink unicast, groupcast and broadcast support for NR V2X	Huawei, HiSilicon,

R1-1808242	Considerations on sidelink unicast, groupcast and broadcast	vivo

R1-1808340	Discussion on support of unicast, groupcast and broadcast in NR V2X	Sony

R1-1808399	Support of unicast, groupcast and broadcast in NR sidelink	CATT

R1-1808553	Support of unicast, groupcast and broadcast for NR V2X	Lenovo, Motorola Mobility

R1-1808602	Support of unicast, groupcast and broadcast in NR V2X	ZTE

R1-1808646	Discussion on supporting unicast, groupcast and broadcast via NR sidelink	Panasonic

R1-1808693	Support of sidelink unicast, groupcast and broadcast modes for NR V2X communication	Intel Corporation

R1-1808722	Design issues for NR V2X unicast, groupcast and broadcast	ITRI

R1-1808837	Discussion on support of V2X unicast transmission	CMCC

R1-1808875	Discussion of sidelink unicast, groupcast and broadcast for NR-V2X	Guangdong OPPO Mobile Telecom

R1-1809044	Initial View on Support of unicast, groupcast and broadcast for NR Sidelink	Nokia, Nokia Shanghai Bell

R1-1809066	V2X sidelink design in supporting unicast, groupcast and broadcast	AT&T

R1-1809093	NR Sidelink Support of Unicast Groupcast and Broadcast 	InterDigital, Inc.

R1-1809174	On support of sidelink unicast, groupcast and broadcast	Xiaomi Communications

R1-1809301	Support of unicast, groupcast and broadcast	Ericsson

R1-1809446	Support for unicast, groupcast and broadcast	Qualcomm Incorporated

##### Physical layer structures and procedure(s)

R1-1808554	Physical layer structures and procedure in NR V2X	Lenovo, Motorola Mobility

- Proposal 1: The following sidelink physical signal/channel should be studied in NR V2X:

  - Synchronization signal and sidelink broadcast channel

  - Sidelink control channel

  - Sidelink data channel

- Proposal 2: Reuse the following sidelink communication modes for NR V2X:

  - Scheduling based communication mode

  - UE autonomous resource selection based communication mode

- Proposal 3: Link adaptation mechanism should be studied for both unicast and groupcast communication.

- Proposal 4: Flexible numerology on different frequency spectrum can be studied.

- Proposal 5: For the waveform, DFT-s-OFDM and CP-OFDM can be studied.

Decision: The document is noted.

R1-1808694	Sidelink Physical Layer Structure and Procedures for NR V2X Communication	Intel Corporation

- Further discuss if there are benefits to support different SCS options for the sidelink of eV2X use cases

- Based on the assumption of ~0.5 km maximum communication distance, a maximum delay spread of 1 μs and a timing offset error analysis, a CP size not leading to degradation of the system performance should be selected

- At least for FR1, consider OFDM as the baseline waveform for both PSCCH and PSSCH

  - SC-FDMA is considered only if significant benefits over OFDM are observed

- Consider a common DMRS design for PSSCH and PSCCH in the case of FDM between PSSCH and PSCCH

- Consider the PDCCH reference structure as a starting point for PSCCH in TDM mode and introduce enhancements if the evaluation shows insufficient performance

- Prioritize the evaluation of transparent schemes for transmit diversity

  - Study the effects of the bundling size on the different schemes, as well as give recommendation for a useful bundling parametrization of the different schemes

- For a SCS of 30 kHz or 60 kHz consider to reuse the existing DMRS patterns in PDSCH at least as a baseline for the initial study

- If 15 kHz is selected for the ITS band, consider to define new DMRS patterns to improve demodulation performance

- Consider to support up to 8 antenna ports from a system perspective with orthogonal reference signals to support spatial multiplexing of UEs as well as to resolve collisions of different devices transmitting in the same resources

- Evaluate open loop spatial multiplexing as a primary option for spatial multiplexing.

- Evaluate transparent transmit diversity schemes for PSSCH

Decision: The document is noted.

R1-1809447	Design principles for physical layer aspects of NR V2X	Qualcomm Incorporated

- Principle 1: NR-V2X sidelink should be based on synchronous communications between UEs, so that the resource wastage due to asynchronous operation can be avoided.

- Principle 2: NR-V2X sidelink operates either in license exempt ITS spectrum or utilizes uplink spectrum for

- Principle 3: CP-OFDM is used for NR-V2X sidelink.

- Principle 4: Similar to NR Uu, NR-V2X sidelink supports multiple numerologies. The exact numerology used should be (pre-)configurable and should apply to all UEs using a common pool of time and frequency resources.

- Principle 5: Sidelink control channel is introduced to provide detail transmission characteristics of data channel.

- Principle 6: To reduce the delay in data channel decoding control channel is TDMed with data channel for NR-V2X sidelink.

- Principle 7: Similar to NR Uu, front loaded DMRS and variable number of DMRS are used for NR-V2X sidelink. Control channel can indicate the DMRS structure used in the transmission.

- Principle 9: Introduce HARQ feedback channel for efficient unicast & groupcast.

- Principle 10: Introduce channel state feedback to support special multiplexing for unicast.

- Principle 11: Introduce implementation friendly synchronization channel for enabling communication when GNSS coverage is challenging.

- Principle 12: Similar to LTE-V2X, resource pool definition for NR-V2X sidelink is required to indicate time and frequency resources that can be used by UE to select resource for sidelink transmission.

- Principle 13: Sub-channel based resource pool design is used for NR-V2X sidelink.

- Principle 14: Similar to NR Uu a radio frame of fixed 10ms, a subframe of fixed 1 ms and a slot of fixed 14 OFDM symbols are used for NR-V2X sidelink. Where basic scheduling unit is a slot.

- Principle 15: No need to define mini slots for NR-V2X sidelink as due to dominated mechanical aspects, vehicular communication latency below few ms is not required.

- Principle 16: NR-V2X sidelink should be able to operate out of coverage, partial coverage and in-coverage.

- Principle 17: UE autonomous resource selection and gNB scheduled resource allocation should be supported for NR-V2X sidelink.

- Principle 18: Allow LTE network to allocate NR V2X sidelink.

- Principle 19: NR network should also be able to allocate LTE V2X sidelink.

- Principle 20: Study the feasibility to enable UE-type RSU scheduling for sidelink transmissions.

- Principle 21: To support periodic, aperiodic, fixed and variable size packet from one transmission to another transmission a common framework based on per slot scheduling is used which is flexible in utilising time and frequency resources used for a transmission.

Decision: The document is noted.

R1-1809507	Discussion on physical layer structure for NR V2X	Xiaomi Communications	(rev of R1-1809175)

- Observation 1: In order to better match the time-varying characteristics of high-speed channel, the density of DMRS in time domain should be increased.

- Observation 2: The benefit of higher SCS on combating Doppler effect may not be dominant for H-type DMRS structure.

Decision: The document is noted.

Tuesday session conclusion: Continue discussion offline – Hanbyul (LGE)

Thursday session

R1-1809799	Offline summary for 7.2.4.1.2 Physical layer structures and procedure(s)	LG Electronics

Decision: The document is noted.

Agreements:

- At least PSCCH and PSSCH are defined for NR V2X. PSCCH at least carries information necessary to decode PSSCH.

  - Note: PSBCH will be discussed in the synchronization agenda.

- RAN1 continues study on the necessity of other channels.

- Further study on

  - Whether/which sidelink feedback information is carried by PSCCH or by another channel/signal.

  - Whether/which information to assist resource allocation and/or schedule UE’s transmission resource(s) is carried by PSCCH or by another channel/signal.

  - PSCCH format(s) and content(s) for unicast, groupcast, and broadcast

Agreements:

- RAN1 to continue study on the physical channel considering at least the following aspects:

- Waveform

  - Candidates: CP-OFDM, DFT-s-OFDM

  - Proposals from companies:

    - CP-OFDM only

    - Support both

  - Consideration points:

    - Different channel can have different waveform?

    - Benefit and impact of supporting only one waveform and supporting both waveforms

- Subcarrier spacing

  - Candidates for further study are:

    - FR1: 15 kHz, 30 kHz, 60 kHz, 120 kHz

    - FR2: 30 kHz, 60 kHz, 120 kHz, 240 kHz

  - Companies are encouraged to consider the potential issues and benefit of introducing new subcarrier spacing.

- CP length

- RS design

  - Candidates are:

    - DM-RS

      - DM-RS defined in Rel-15 NR Uu is the starting point.

    - PT-RS

    - CSI-RS

    - SRS

    - AGC training signal

- Channel coding

  - For data, channel coding defined for data in Rel-15 NR Uu is the starting point.

  - For control, channel coding defined for control in Rel-15 NR Uu is the starting point.

- Modulation

- RE mapping and rate-matching

- Scrambling

Agreement:

- RAN1 continues study on the necessity, benefits and relationship between bandwidth part and resource pool.

Agreement:

Agree the following assumptions as tentative assumptions for the simulation at least till RAN1#94bis

- AGC

  - Up to [15] us in FR1. Up to [10] us in FR2.

- TX/RX switching time

  - [13] us in FR1 and [7] us in FR2

- Time error

  - Up to [0.4] us between a UE and its synchronization reference

- Frequency error

  - Up to [0.1] PPM between a UE and its synchronization reference

To send an LS to RAN4 for feedback (to be included in the IBE LS)

R1-1809748	[Draft] LS on IBE model for V2X	Ericsson, Intel

Decision: The document is noted.

Friday session

R1-1809964	Updated offline summary for 7.2.4.1.2 Physical layer structures and procedure(s)	LG Electronics

Decision: The document is noted.

Agreements:

RAN1 to continue study on multiplexing physical channels considering at least the above aspects:

- Multiplexing of PSCCH and the associated PSSCH (here, the “associated” means that the PSCCH at least carries information necessary to decode the PSSCH).

  - Study further the following options:

    - Option 1: PSCCH and the associated PSSCH are transmitted using non-overlapping time resources.

      - Option 1A: The frequency resources used by the two channels are the same.

      - Option 1B: The frequency resources used by the two channels can be different.

    - Option 2: PSCCH and the associated PSSCH are transmitted using non-overlapping frequency resources in the all the time resources used for transmission. The time resources used by the two channels are the same.

    - Option 3: A part of PSCCH and the associated PSSCH are transmitted using overlapping time resources in non-overlapping frequency resources, but another part of the associated PSSCH and/or another part of the PSCCH are transmitted using non-overlapping time resources.

Illustration of the above options:

R1-1809945	[Draft] LS on IBE model for V2X	Ericsson, Intel

Discussion: Huawei do not understand why Intel is indicated as contact - SI rapporteur is enough

Decision: The draft LS is endorsed (by removing the 2nd contact name), and final LS is approved in R1-1810006.

R1-1808093	PHY sidelink structure and procedure for NR V2X	Huawei, HiSilicon

R1-1808203	On sidelink design for NR V2X	Mitsubishi Electric RCE

R1-1808243	Physical layer structure and procedure for NR sidelink	vivo

R1-1808280	On NR V2X physical channel design issues	MediaTek Inc.

R1-1808294	SLS Study on Realization of Stringent Reliability in Mode-4 NR-V2X	Fujitsu

R1-1808295	Considerations on sidelink physical layer structures	Fujitsu

R1-1808400	Physical layer structure and procedure in NR V2X	CATT

R1-1808520	Discussion on sidelink physical layer structures and procedures	LG Electronics

R1-1808603	Discussion on NR Sidelink Physical layer structures	ZTE

R1-1808616	NR sidelink design	NEC

R1-1808647	Discussion on physical layer structures and procedure(s) of NR sidelink	Panasonic

R1-1808776	Discussion on physical layer structures and procedures for NR V2X	Samsung

R1-1808813	Sidelink Design for NR V2X	ETRI

R1-1808876	Discussion of physical layer structure and procedure for NR-V2X	Guangdong OPPO Mobile Telecom

R1-1809039	Discussion on beam management for NR-V2X sidelink in millimeter-wave bands	TOYOTA InfoTechnology Center

R1-1809045	Initial View on NR V2X Sidelink Physical Layer Structures and Procedures	Nokia, Nokia Shanghai Bell

R1-1809067	V2X sidelink PHY structure and procedures	AT&T

R1-1809094	NR Sidelink Physical Layer Structures and Procedures	InterDigital, Inc.

R1-1809157	Physical layer structures and procedure(s)	NTT DOCOMO, INC.

R1-1809302	Physical layer design of NR sidelink	Ericsson

R1-1809507	Discussion on physical layer structure for NR V2X	Xiaomi Communications	(rev of R1-1809175)

##### Synchronization mechanism

R1-1808401	Considerations on synchronization mechanism in NR V2X	CATT

- Proposal 1: The design of NR V2X synchronization mechanism needs to take into consideration of the SSB structure and the beamforming of SSB for extended coverage.

- Proposal 2: NR V2X synchronization priority order should consider the following issues:

- The NR V2X synchronization priority order should consider NR gNB-based and GNSS-based synchronization with similar synchronization priority order to that of LTE V2X.

  - The accuracy of different synchronization priority order should be further investigated with NR SSB design with different duty cycle.

  - The limitation of UE capability should be considered in the multi-carrier synchronization mechanism of NR-V2X.

- Proposal 3: The PBCH payload content of NR can be modified in NR V2X for adapting different traffic model and scenarios.

Decision: The document is noted.

R1-1808604	Discussion about synchronization mechanism for NR V2X	ZTE

- Proposal 1: The synchronization mechanism in LTE V2X can be used as the starting point for NR V2X synchronization study and to be modified considering new characteristics in NR, e.g., multiple numerologies, frame structure and beam transmission.

- Proposal 2: SS/PBCH block in NR can be reused as the starting point for Resource pattern of NR-based SLSS/PSBCH.  Other patterns would not be precluded.

- Proposal 3: Beam sweeping should be considered for transmission of NR-based SLSS/PSBCH.

- Proposal 4: The contents of PSBCH should be studied considering the new characteristics of the network coverage scenario.

- Proposal 5: Mechanisms for synchronization reference selection should be based on network coverage scenarios in NR V2X.

Decision: The document is noted.

R1-1809046	Initial View on NR V2X Sidelink Synchronization	Nokia, Nokia Shanghai Bell

- Proposal 1: Synchronous operation is used in NR sidelink V2X. Asynchronous operation is not supported.

- Proposal 2: GNSS-based, network-based and device-based synchronization is supported in NR V2X sidelink operation.

- Proposal 3: Study methods that reduce problems related to switching of synchronization source and using different synchronization sources in the V2X UE for SL and Uu.

- Proposal 4: Study interference between sidelink and Uu taking into account potential timing difference between SL and Uu.

- Proposal 5: Study beam-based sidelink synchronization channel design.

Decision: The document is noted.

Tuesday session conclusion: Continue discussion offline – Fangcheng (CATT)

Thursday session

R1-1809818	Summary of offline discussion on sidelink synchronization in NR V2X	CATT

Decision: The document is noted.

Agreements:

- NR V2X Sidelink Synchronization includes at least the following

  - Sidelink synchronization signal(s)

  - PSBCH

  - Sidelink synchronization sources and procedure(s)

    - Synchronization sources – at least GNSS, gNB, eNB, UE, LTE UE

      - Note: this doesn’t mean all of them are to be supported

Friday session

R1-1809996	Summary of Offline Discussion on Sidelink Synchronization in NR V2X	CATT

Decision: The document is noted. Above agreement is updated.

R1-1808094	Sidelink synchronization mechanisms for NR V2X	Huawei, HiSilicon

R1-1808244	NR sidelink synchronization mechanism	vivo

R1-1808282	On NR V2X synchronization issues	MediaTek Inc.

R1-1808521	Discussion on sidelink synchronization mechanism	LG Electronics

R1-1808555	Synchronization mechanism in NR V2X	Lenovo, Motorola Mobility

R1-1808695	Sidelink synchronization for NR V2X communication	Intel Corporation

R1-1808723	Considerations of Synchronization Enhancement for NR V2X Communication	ITRI

R1-1808777	Discussion on Synchronization mechanisms for NR sidelink	Samsung

R1-1808814	Discussion on NR V2X sidelink synchronization	ETRI

R1-1808877	Discussion of synchronization mechanism for NR-V2X	Guangdong OPPO Mobile Telecom

R1-1809068	Sidelink based synchronization mechanism	AT&T

R1-1809095	On V2X Synchronization in NR 	InterDigital, Inc.

R1-1809158	Synchronization mechanism	NTT DOCOMO, INC.

R1-1809177	On synchronization for NR V2X	Xiaomi Communications

R1-1809254	Considerations on sidelink synchronization signal for NR V2X	ITL

R1-1809303	On Synchronization for Sidelink V2X	Ericsson

R1-1809448	Synchronization design for NR V2X	Qualcomm Incorporated

R1-1809475	Discussion on NR V2X Synchronization mechanism	ITL

##### Resource allocation mechanism

R1-1808522	Discussion on sidelink resource allocation mechanism	LG Electronics

- Proposal 1: RAN1 studies enhancement of NR SL resource allocation including support of LTE features such as e.g., PPPP based sensing and resource reservation, sensing metric (e.g., PSSCH-RSRP, S-RSSI), FDMed transmission of SA and data, zoning, congestion control.

- Proposal 2: RAN1 studies solution of NR sidelink resource allocation for handling a traffic type with time-varying size and/or generation interval.

- Proposal 3: RAN1 studies mechanism of feedback and adaptive retransmission number (more than 1) to support the advanced V2X services

- Proposal 4: RAN1 studies mechanism of resource allocation and congestion control when using flexible frame structure for NR sidelink.

- Proposal 5:  RAN1 studies impact of distributed antenna panel on resource allocation and congestion control.

Decision: The document is noted.

R1-1808696	Sidelink resource allocation mechanisms for NR V2X communication	Intel Corporation

- For periodic traffic, study benefits of additional enhancements to LTE V2X sensing and resource selection procedure including the following aspects:

  - Collisions handling in case of simultaneous resource (re)-selection;

  - Configurable sensing window (e.g. based on maximum reservation interval in the system);

  - Dynamic resource selection window (e.g. derived based on packet delay budget);

  - Physical structure and multiplexing of PSCCH and PSSCH;

  - Resource (re)-selection conditions and triggers;

  - Priority handling enhancements;

- For aperiodic traffic, further study benefits and details of small scale sensing schemes, including:

  - Collisions handling when multiple UEs compete for resources;

  - Channel access criteria and conditions;

  - Sidelink resource and channel access granularity in time;

  - Resource reservation mechanisms;

  - Priority handling mechanisms.

- Further study benefits of large and small scale channel sensing procedures as well as their combination in application to periodic, aperiodic and mixed traffic models

- Further study technical challenges and benefits of sub-slot level channel access and resource granularity for sidelink communication (e.g. ½-slot and ¼-slot)

- Further study how to efficiently enable sidelink resource reservation/preemption mechanisms for eV2X sidelink communication.

- Concept of sidelink resource pools is reused and introduced for eV2X sidelink communication

- Strive for sidelink design that supports TDM and FDM between PSCCH and PSSCH from UE perspective

- From system perspective, control and shared channel resources can overlap in time and frequency within sidelink resource pool

- PSCCH and PSSCH resources are separately configured within the same resource pool

Decision: The document is noted.

R1-1809304	Radio Resource Management for NR Sidelink	Ericsson

- Observation 1: When the sidelink utilizes licensed spectrum resources, it must not degrade the performance of cellular services.

- Observation 2: Sidelink RRM techniques should consider the specific characteristics of unicast and broadcast communications.

- Observation 3: Sidelink RRM techniques should consider the coexistence of unicast, multicast and/or broadcast communication.

- Observation 4: In case of no network coverage for NR, UEs follow distributed RA mechanisms like LTE mode-4.

- Observation 5: In the case of network coverage, network control RRM helps limiting the interference and providing the required QoS.

- Observation 6: LTE SL mode-4 resource allocation is not suitable for the traffic models considered in eV2X.

- Proposal 1: NR supports an RRM configuration for unicast transmissions that is designed according to the radio resource management techniques described as alternative 2 above.

- Proposal 2: RAN1 studies one-shot and resource reservation schemes to tackle variation in the traffic patterns and intensity.

- Proposal 3: RAN1 studies solutions to reduce resource wastage when future resources reserved for sidelink transmissions are not used.

- Proposal 4: RAN1 studies the possibility of enabling pre-emption based on QoS.

- Proposal 5: RAN1 studies the possibility of enabling a UE to reserve multiple consecutive resources for transmission.

- Proposal 6: RAN1 strives to minimize the collision probability due to random resource selection.

- Proposal 7: RAN1 studies solutions to limit sidelink bandwidth fragmentation.

- Proposal 8: RAN1 studies the possibility of different levels of network control.

Decision: The document is noted.

R1-1809449	Initial perspectives on resource allocation	Qualcomm Incorporated

- Proposal 1:  For NR V2X, like LTE V2X both scheduled and autonomous resource allocation need to be defined. Like LTE V2X, the NR V2X autonomous resource allocation should be able to operate independent of network.

- Proposal 2: NR V2X autonomous resource allocation should be able to support arbitrary traffic patterns and target very high reliability service requirements.

- Proposal 3: Study the performance of autonomous resource allocation based on LBT signals on potentially every slot.

- Proposal 4: Study the request/response type handshake mechanisms to further improve reliability for unicast/groupcast transmissions.

- Proposal 5: Study the feasibility to enable UE-type RSU scheduling for NR sidelink transmissions.

Decision: The document is noted.

Wednesday session conclusion: Continue discussion offline - Alexey (Intel)

Thursday session

R1-1809867	Offline Summary for NR-V2X Agenda Item - 7.2.4.1.4 Resource Allocation Mechanism	Intel

Decision: The document is noted.

Agreements:

At least two sidelink resource allocation modes are defined for NR-V2X sidelink communication

  - Mode 1: Base station schedules sidelink resource(s) to be used by UE for sidelink transmission(s)

  - Mode 2: UE determines (i.e. base station does not schedule) sidelink transmission resource(s) within sidelink resources configured by base station/network or pre-configured sidelink resources

Notes:

  - eNB control of NR sidelink and gNB control of LTE sidelink resources will be separately considered in corresponding agenda items.

  - Mode-2 definition covers potential sidelink radio-layer functionality or resource allocation sub-modes (subject to further refinement including merging of some or all of them) where

    - UE autonomously selects sidelink resource for transmission

    - UE assists sidelink resource selection for other UE(s)

    - UE is configured with NR configured grant (type-1 like) for sidelink transmission

    - UE schedules sidelink transmissions of other UEs

RAN1 to continue study details of resource allocation modes for NR-V2X sidelink communication

R1-1808245	NR sidelink resource allocation	vivo

R1-1808281	On NR V2X resource allocation	MediaTek Inc.

R1-1808296	Discussion on Subframe-based Resource Selection Parameters in Mode-4 NR-V2X	Fujitsu

R1-1808297	Resource Allocation for NR V2X Sidelink Communication considering Low Latency Requirement	Fujitsu

R1-1808309	Enhancements for NR V2X Resource Allocation Mechanism	Fraunhofer HHI

R1-1808341	Initial considerations on NR V2X resource allocation mechanism	Sony

R1-1808402	Considerations on resource allocation mechanism in NR V2X	CATT

R1-1808556	Resource allocation mechanism in NR V2X	Lenovo, Motorola Mobility

R1-1808599	Resource allocation for NR V2X	InterDigital, Inc.

R1-1808605	Discussion on NR Sidelink Physical resource pool allocation	ZTE

R1-1808648	Discussion on resource allocation mechanism for NR sidelink	Panasonic

R1-1808724	Considering geo-based resource management techniques for NR V2X communication	ITRI

R1-1808778	Discussion on resource allocation mechanisms for NR V2X	Samsung

R1-1808838	Discussion on consideration for V2X resource allocation	CMCC

R1-1808906	Discussion on resource allocation in NR-V2X	Guangdong OPPO Mobile Telecom

R1-1808939	UE autonomous sidelink allocation for NR V2X	Huawei, HiSilicon

R1-1809047	Initial View on NR V2X Sidelink Resource Allocation	Nokia, Nokia Shanghai Bell

R1-1809069	Resource allocation mechanism	AT&T

R1-1809159	Resource allocation mechanism	NTT DOCOMO, INC.

R1-1809178	On resource allocation for NR V2x sidelink communications	Xiaomi Communications

R1-1809287	Considerations on Resource Allocation of NR V2X Sidelink	CAICT

R1-1809383	Discussion on Sidelink Resource Allocation	Convida Wireless LLC

##### Other

Including any remaining aspects of simulation assumptions

R1-1809946	Correction and confirmation of some evaluation assumption parameters	LG Electronics

Decision: The document is endorsed. Final CR in R1-1810007 (CR0002r1 to 37.885)

R1-1809950	Offline summary for remaining evaluation assumptions	LG Electronics

Decision: The document is noted.

Agreement:

- Agree CRs to 37.885 in R1-1809924 and R1-1810007

R1-1809924	Correction of optional antenna element patterns for vehicular UEs	Fraunhofer HHI, Fraunhofer IIS, General Motors, Nokia, Nokia Shanghai Bell, LG Electronics

Decision: CR0001r2 is agreed.

Conclusion:

It is understood in RAN1 that:

- Regarding whether LOS/NLOS/NLOSv status can be different in different panels, the common understanding is that this status is defined per link between vehicles and not different in the panels of the same vehicle.

- Regarding how fading channel is generated across panels, the common understanding is that one set of clusters is generated and used for both panels.

RAN1 to have an email discussion until the next meeting to complement the existing simulation assumptions, using R1-1809950 and R1-1809944 (traffic model) at a starting point– Hanbyul (LGE)

R1-1808298	Discussion on LLS parameters of fast fading model	Fujitsu

R1-1808523	Discussion on remaining aspects of simulation assumptions	LG Electronics

R1-1808697	Remaining aspects of eV2X evaluation methodology and assumptions	Intel Corporation

R1-1808779	Discussion on AGC settling issue for NR sidelink	Samsung

R1-1808937	Sidelink control channel design of NR V2X	Huawei, HiSilicon

R1-1808938	Sidelink data channel design of NR V2X	Huawei, HiSilicon

R1-1808940	Preliminary system level evaluations on sidelink for NR V2X	Huawei, HiSilicon

R1-1808946	Preliminary link level evaluations on sidelink for NR V2X	Huawei, HiSilicon

R1-1809048	Required Clarifications on Simulation Assumptions	Nokia, Nokia Shanghai Bell

R1-1809305	Link level simulations of DMRS design for NR V2X	Ericsson

R1-1809306	Considerations and Design Principles for NR Sidelink	Ericsson

R1-1809803	Open issues of evaluation assumptions for NR V2X	Ericsson	(rev of R1-1809307)

R1-1809450	Baseline simulation assumptions and scenarios	Qualcomm Incorporated

#### Evaluation of Uu for advanced V2X use cases

R1-1808102	Evaluation and enhancement of NR Uu and LTE Uu for advanced V2X use cases	Huawei, HiSilicon

- Observation 1: NR R15 Uu could support about 80% downlink coverage at 99.999% reliability in Highway deployments.

- Observation 2: NR R15 Uu could support about 60% downlink coverage at 99.999% reliability in Urban Grid deployments.

- Observation 3: NR R15 Uu could support about 90% uplink coverage at 99.999% reliability in Highway deployments.

- Observation 4: NR R15 Uu could support about 55% uplink coverage at 99.999% reliability in Urban Grid deployments.

- Observation 5: DL RRM measurement needs to be improved to meet 0 ms interruption requirement.

- Proposal 1: To support V2X scenarios, NR Uu should support multicast via Group-RNTI and the scheduling multiple groups of multicast services in the same subframe

- Proposal 2: For the evaluation of NR Uu multicast transmission, the vehicle UEs at a road intersection area can be formed as a UE group. The packet size of FTP3 model could be 0.5Mbytes.

- Proposal 3: Mini-slot based repetition within a slot should be supported for both grant based PUSCH and grant-free PUSCH. The mini-slot repetition pattern can be studied.

- Proposal 4: Explicit HARQ-ACK feedback should be introduced in URLLC as an overhead saving method to allow UE to perform initial transmission or non-adaptive retransmission in a grant-free manner.

- Proposal 5: The scheduling/HARQ scheduling limitation in Release 15 URLCC should be removed for Release 16.

- Proposal 6: A-CSI report on short PUCCH should be triggered by DL DCI.

- Proposal 7: To reduce NR mobility latency, UL RRM measurement should be considered.

- Proposal 8: NR V2X should support grant-free UL transmission for RRC-Idle/Inactive V-UEs.

- Proposal 9: NR Uu UL data rate should be considered for V-UEs based on the reliability and latency requirements.

- Proposal 10: To increase the reliability and reducing the collision occasions in contention-based grant-free communication, V-UE geo-location should be taken into account.

- Observation 6: LTE Uu evaluations show 62.5% and 48.3% UEs could be supported at the 99.999% reliability level for downlink for highway and urban grid scenario, respectively.

- Observation 7: LTE Uu evaluations show 60% and 40% UEs could be supported at the 99.999% reliability level for uplink for highway and urban grid scenario, respectively.

- Proposal 11: Study the feasibility of reducing DCI size for HARQ-less transmission to support the advanced V2X service of remote driving.

- Proposal 12: Study the feasibility of disabling downlink HARQ-ACK feedback in order to support the advanced V2X service of remote driving.

Decision: The document is noted.

R1-1808283	On NR Uu to support NR V2X services	MediaTek Inc.

- Proposal 1: Study for NR V2X unicast communications via NR uu interface can be deprioritized or leveraged the study in Rel16 eURLLC SI.

- Proposal 2:  NR uu interface to support NR V2X  broadcast/multicast communications can be studied in a separated SI/WI for a generic mechanism to support broadcast/multicast services rather than restriction to NR V2X.

- Proposal 3: NR uu interface enhancement for facilitating NR V2X sidelink communications is prioritized for study and support.

Decision: The document is noted.

R1-1809308	Study of Uu feasibility for advanced V2X use cases	Ericsson

- Observation 1: High reliability can be achieved for both 15 kHz and 30 kHz SCS at fairly low SNR with 250 km/h UE moving speed. That means, the current NR Uu physical layer can achieve the desired reliability for advanced V2X use cases.

- Observation 2: 10ms latency requirement can be met in most times for 15 kHz SCS and 6.5 ms can be achieved for 30 kHz SCS. In other words, the current NR Uu physical layer can achieve the desired latency for advanced V2X use cases.

- Observation 3: Current NR Uu physical layer can achieve the desired eV2X data rate at low SNR (-4 dB) with 100 MHz system bandwidth.

Decision: The document is noted.

Wednesday session conclusion: Continue discussion offline - Ricardo (Ericsson)

Thursday session

R1-1809909	Feature Lead Summary AI 7.2.4.2 v1	Ericsson

Decision: The document is noted.

Friday session

R1-1809944	Feature Lead Summary AI 7.2.4.2 v2	Ericsson

Decision: The document is noted.

R1-1809893	Evaluation and enhancement of NR Uu and LTE Uu for advanced V2X use cases	Huawei, HiSilicon

R1-1808403	Evaluation of Uu for advanced V2X use cases	CATT

R1-1808524	Discussion on evaluation of Uu for advanced V2X use cases	LG Electronics

R1-1808698	Analysis of NR Uu support for advanced V2X use cases	Intel Corporation

R1-1808725	Evaluations of Uu Capability in Supporting Advanced V2X Use Cases	ITRI

R1-1808780	Discussion on Uu enhancements for advanced V2X use cases	Samsung

#### Uu-based sidelink resource allocation/configuration

R1-1808726	Discussion on Uu-based sidelink resource allocation and configuration	ITRI

Thursday session

R1-1809878	Summary of AI: 7.2.4.3 Uu-based sidelink resource allocation/configuration	Huawei, HiSilicon

Decision: The document is noted.

Agreements:

- NR Uu can assign NR sidelink resources for the following:

  - Shared licensed carrier between Uu and NR sidelink

  - Dedicated NR sidelink carrier

Agreements:

- Study at least the following NR sidelink resource allocation techniques:

  - Dynamic resource allocation

  - Activation/deactivation based

    - E.g., semi-persistent scheduling allocation or NR grant free type-2

  - RRC (pre-)configured

    - E.g., configured NR grant type-1, UE autonomous selection of resource(s) from resources configured by RRC

- RAN1 will study the level of network control, e.g., whether the UE may select other parameters (e.g., MCS) and/or the exact transmission resources, and whether the selection is autonomous or not

##### Enhancements of LTE Uu and NR Uu to control NR sidelink

R1-1808095	Discussion on NR sidelink resource allocation by NR Uu and LTE Uu	Huawei, HiSilicon

- Proposal 1: NR SL should support both grant-free and grant-based resource allocation modes.

- Proposal 2: NR should support configuration of SL grant-free resources for in-coverage UEs and pre-configuration of SL grant-free resources for out-of-coverage UEs.

- Proposal 3: For enhanced reliability and latency, NR should support pre-/configuration of SL transmission patterns indicating the repetition location of a given TB for out-of-coverage/in-coverage UEs.

  - Study how many retransmissions are needed.

- Proposal 4: Study dynamic and semi-persistent grant-based SL resource allocation mechanisms for in-coverage and out-of-coverage UEs.

- Proposal 5: For reliability and latency enhancement, NR grant-based sidelink transmission should support allocation of SL transmission patterns indicating the repetition location of a given TB.

- Proposal 6: In case of unicast and groupcast transmissions, consider whether scheduled UEs may report measurements, or resources selected based on such measurements, to gNB to support sidelink scheduler.

- Observation 1: A radio resource may be reused by nearby UEs in case of non-isotropic transmissions.

- Proposal 7: Study whether non-orthogonal resources can be assigned to UEs based on the directionality of their intended transmissions.

- Proposal 8: LTE enhancements to enable NR sidelink control by LTE Uuinclude provision of DCI format 5A contents and SIB/RRC configurations.

Decision: The document is noted.

R1-1808246	Enhancements of LTE Uu and NR Uu to control NR sidelink	vivo

- Proposal 1: It should be clarified whether the scenario that evolved eNB controls the NR sidelink is practical, and if yes, what the desired behavior is.

- Proposal 2: The eNB broadcasts the necessary NR sidelink configurations in SIB, at least for the mode-4 operation in EN-DC case.

- Proposal 3: If the network is used as the synchronization source in the case of EN-DC, the network should indicate either the eNB or the gNB is used as the NR sidelink synchronization source to the UE.

- Proposal 4: The UE should be able to indicate capability of not supporting simultaneous sidelink and uplink operations to network.

Decision: The document is noted.

R1-1809049	Initial View on Enhancements of LTE Uu and NR Uu to control NR sidelink	Nokia, Nokia Shanghai Bell

- Proposal 1: Support both dynamic scheduling and semi-persistent scheduling/configured grant.

- Proposal 2: Support resource pool sharing between gNB-scheduled and UE-autonomous UEs from the beginning.

- Proposal 3: Consider support for sidelink unicast/groupcast enhancements in the work on Uu-based sidelink resource allocation.

Decision: The document is noted.

Wednesday session conclusion: Continue discussion offline - Philippe (Huawei)

R1-1809775	Discussion on Sidelink Bi-mode Transmission in NR-V2X	Fujitsu	(revision of R1-1808299)

R1-1808300	Discussion on latency performance of Uu-based sidelink resource allocation	Fujitsu

R1-1808404	On LTE Uu and NR Uu control NR sidelink in NR V2X	CATT

R1-1808525	Discussion on LTE Uu and NR Uu enhancements to control NR sidelink	LG Electronics

R1-1808606	Discussion on LTE/NR Uu based resource allocation/configuration for NR sidelink	ZTE

R1-1808699	Enhancements of NR and LTE Uu link to control NR sidelink	Intel Corporation

R1-1808727	Issues in NR Uu for Mode 3 NR Sidelink Resource Access	ITRI

R1-1808781	Discussion on Enhancements of LTE Uu and NR Uu to control NR sidelink	Samsung

R1-1808878	Enhancement of LTE Uu and NR Uu to control NR sidelink	Guangdong OPPO Mobile Telecom

R1-1809070	Enhancements of LTE Uu and NR Uu to control NR sidelink	AT&T

Withdrawn

R1-1809096	Uu Interface Enhancement for NR Sidelink 	InterDigital, Inc.

R1-1809160	Enhancements of LTE Uu and NR Uu to control NR sidelink	NTT DOCOMO, INC.

R1-1809309	On Network Control of NR Sidelink	Ericsson

R1-1809451	Enhancements of LTE Uu and NR Uu to control NR sidelink	Qualcomm Incorporated

##### Enhancements of NR Uu to control LTE sidelink

R1-1808839	Discussion on NR Uu control LTE sidelink	CMCC

- Proposal 1: It’s necessary to study the case of NR Uu control LTE sidelink in NR V2X.

- Proposal 2: In NR-V2X, the architecture of NR-LTE DC can be used as a baseline to support the case of NR Uu control LTE sidelink.

- Proposal 3: In NR-V2X, we need further study the mechanism of NR Uu control LTE sidelink and identify the necessary enhancements for mode 3 and mode 4 UE respectively.

Decision: The document is noted.

R1-1808907	Enhancement of NR Uu to control LTE sidelink	Guangdong OPPO Mobile Telecom

- Proposal 1: For NR Uu to schedule a UE on LTE sidelink, a Tx start timing indicator can be added to the new DCI format (for NR Uu) to allow a NR capable UE to transmit earlier than n+4 subframe.

- Proposal 2: A single DCI format that can be dynamically used for scheduling resources on either NR sidelink or LTE sidelink should be considered for NR Uu to reduce UE processing complexity, time and power consumption of blind decoding of different SL scheduling grants (DCIs).

Decision: The document is noted.

R1-1809071	Enhancements of NR Uu to control LTE sidelink	AT&T

- Proposal-1: Dual connectivity framework can support NR UU interface as MCG to control NR and LTE sidelink as SCG.

- Proposal-2: The connection between local manager and UE is a “diet” connection

Decision: The document is noted.

Wednesday session conclusion: Continue discussion offline - Philippe (Huawei)

R1-1808096	Discussion on NR Uu to control LTE PC5	Huawei, HiSilicon

R1-1808405	On NR Uu control LTE sidelink in NR V2X	CATT

R1-1808526	Discussion on NR Uu enhancements to control LTE sidelink	LG Electronics

R1-1808607	Discussion on NR Uu based resource allocation/configuration for LTE sidelink	ZTE

R1-1808700	Enhancements of NR Uu link to control LTE sidelink	Intel Corporation

R1-1808782	Discussion on Enhancements of NR Uu to control LTE sidelink	Samsung

R1-1809097	NR Uu Interface Enhancement for LTE Sidelink 	InterDigital, Inc.

R1-1809310	On NR Uu controlling LTE sidelink	Ericsson

R1-1809452	Enhancements of NR Uu to control LTE sidelink	Qualcomm Incorporated

##### Other

R1-1809311	On Link Selection for Unicast V2X Communications	Ericsson

#### QoS management

R1-1808600	QoS Management for NR V2X	InterDigital, Inc.

- Observation 1: How to translate VQI and bearer information into parameters used by PHY will be determined by RAN2.

- Proposal 1: NR V2X supports mode 4 transmission prioritization based on priority information in SCI.

- Proposal 2: NR V2X supports mode 4 resource selection based on maximum delay budget for a packet.

- Proposal 3: NR V2X supports mechanism(s) to achieve different target error rate depending on the QoS characteristics of the data to be transmitted.

- Proposal 4: NR V2X supports methods to achieve different minimum communication range depending on the QoS characteristics of the data to be transmitted.

- Proposal 5: NR V2X supports methods to efficiently multiplex UEs with different data rate requirements on sidelink

Decision: The document is noted.

R1-1809312	QoS framework for NR V2X	Ericsson

- Observation 1: V2X service KPIs are already covered in 5GS QoS management for Uu.

- Observation 2: LTE SL UEs may handle logical channel prioritization differently depending on their implementations.

- Observation 3: A more concrete admission control mechanism in SL is needed.

- Proposal 1: 5GS QoS framework is used for V2X services over Uu-interface.

- Proposal 2: A unified QoS handling rule, e.g. map a packet to a corresponding QoS flow, shall be used for NR SL UEs.

- Proposal 3: RAN1 studies mechanisms to allow the gNB to monitor the resource utilization of SL UEs and mechanisms to allow the gNB to pre-empt transmissions of SL UEs.

- Proposal 4: In Mode4, a pre-emption signalling can be designed. UEs with high priority packets can pre-empt some resources used for low priority packets before starting its own transmission.

- Proposal 5: RAN1 assumes that RAN2 leads the studies on enhancing SL QoS management including unifying QoS handling and introducing proper admission control functionality.

Decision: The document is noted.

Wednesday session conclusion: Continue discussion offline - Torsten (Nokia)

Thursday session

R1-1809840	Offline Summary for 7.2.4.4 QoS Management FS_NR_V2X	Nokia, Nokia Shanghai Bell

Decision: The document is noted.

Agreement: From RAN1 perspective, at least the following QoS-related parameters relevant to physical layer studies are considered:

- Priority

- latency

- reliability

R1-1808406	QoS management in NR V2X	CATT

R1-1808527	Discussion on QoS management	LG Electronics

R1-1808540	QoS Management for NR V2X	Fraunhofer IIS

R1-1808608	Discussion on QoS Management	ZTE

R1-1808701	QoS management for NR V2X communication	Intel Corporation

R1-1808728	Discussion QoS requirement for V2X in Mode 3 like and Mode 4 like	ITRI

R1-1808783	Discussion on QoS management for NR V2X	Samsung

R1-1809050	Initial View on NR V2X QoS Management	Nokia, Nokia Shanghai Bell

R1-1809161	QoS management	NTT DOCOMO, INC.

R1-1809336	QoS management for NR V2X	Huawei, HiSilicon

R1-1809453	Design aspects and requirements for QoS	Qualcomm Incorporated

#### Coexistence

Including feasibility of coexistence mechanisms & advanced V2X services provided by NR V2X under co-existence

R1-1808941	Discussion on co-existence for NR V2X and LTE V2X	Huawei, HiSilicon

- Proposal 1: for coexistence studies, consider the case where the NR and LTE carriers share the same or different band.

- Proposal 2: Send an LS to RAN4 to ask them to study the carrier spacing/filtering requirements for simultaneous RX/TX.

- Observation 1: For LTE Mode-4 sidelink CA transmission:

  - For intra-band sidelink CA, transmission may be interrupted when only part of the TX carriers are configured and activated simultaneously.

  - For inter-band sidelink CA, there can be significant inter-band interruption time. Further discussions are needed in RAN4.

- Observation 2: If OFDM is used for NR SL, RAN4 may need to evaluate MPR when the same PA is used on non-adjacent carriers.

Decision: The document is noted.

R1-1809454	Co-existence principles for NR-V2X and LTE-V2X	Qualcomm Incorporated

- Observation 1: LTE-V2X and NR-V2X in device coexistence challenges has the same nature as half duplex challenges of LTE-V2X multi-channel operation.

- Observation 2: The main difference here lies in the fact that LTE-V2X and NR-V2X are two different RATs, so they can have different signal design, different physical layer procedure and may be implemented in UE in separated stacks.

- Observation 3: NR sidelink may have other types of traffic than SPS traffic. Some R14-R15 solution optimized for SPS traffic may not be optimal here.

- Observation 4: Inter RAT sidelink scheduling may be supported for mode 3. In device coexistence issue needed to be considered in such kind of solution.

Decision: The document is noted.

Wednesday session conclusion: Continue discussion offline - Arjun (Qualcomm)

Thursday session

R1-1809904	Summary of Coexistence Aspects in NR-V2X Study	Qualcomm

Decision: The document is noted.

Proposal:

- For the study of LTE-V2X and NR-V2X co-existence, half-dupelx constraints are assumed at least for the intra-band case

Friday session

R1-1809990	Summary of Coexistence Aspects in NR-V2X Study	Qualcomm

Decision: The document is noted.

Agreements:

For the study of LTE-V2X and NR-V2X sidelink co-existence, at least the following scenarios are considered from the UEs perspective:

- LTE sidelink and NR sidelink do not have any coordinated procedures

- LTE sidelink and NR sidelink have coordinated procedures and half-duplex constraints are assumed

  - RAN1 will focus on this scenario in the SI

Agreements:

RAN1 focus on at least the following potential solutions for coexistence at least until the next meeting:

- TDM of LTE V2X and NR V2X sidelink transmissions

- FDM of LTE V2X and NR V2X sidelink transmissions

R1-1808247	Coexistence mechanism for LTE and NR V2X	vivo

R1-1808407	Coexistence of NR sidelink and LTE sidelink in V2X	CATT

R1-1808528	Discussion on coexistence mechanisms	LG Electronics

R1-1808609	In device coexistence between NR sidelink and LTE sidelink	ZTE

R1-1808702	Coexistence mechanisms for eV2X services	Intel Corporation

R1-1808784	Discussion on Coexistence between LTE and NR PC5	Samsung

R1-1809076	Coexistence of Different V2X Services	AT&T

R1-1809098	The Coexistence between NR V2X and LTE V2X 	InterDigital, Inc.

R1-1809313	On coexistence aspects of NR V2X	Ericsson

#### Other

R1-1808601	Vehicle Platooning Use Case for NR V2X	InterDigital, Inc.

R1-1809807	Correction of optional antenna element patterns for vehicular UEs	Fraunhofer HHI, Fraunhofer IIS, General Motors	(rev of R1-1809253)

R1-1809314	On the Objectives of NR V2X System	Ericsson

R1-1809334	NR Uu support for advanced V2X use cases	Huawei, HiSilicon

R1-1809335	LTE Uu support for advanced V2X use cases	Huawei, HiSilicon

R1-1809485	Configuration and signalling principles for NR sidelink	Ericsson

### Study on Remote Interference Management for NR

FS_NR_RIM; SID in RP-181430. Please refer to the SID for detailed scoping

R1-1808840	Work plan for Rel-16 SI on NR RIM	CMCC

Decision: The document is noted.

R1-1809375	Draft skeleton of TR 38.866 Study on remote interference management for NR	CMCC

Discussion: Ericsson suggested offline to restructure the TR, mainly clause 6.

Decision: The document is noted. Discuss further offline

Thursday session

R1-1809872	Draft skeleton of TR 38.866 Study on remote interference management for NR	CMCC

Decision: The TR 38.866 v0.0.0 is endorsed.

R1-1809865	Draft summary on Study on NR-RIM	CMCC

Decision: The document is noted.

Agreements:

- To include the following in the TR: As shown in Figure 1, it is assumed in the RIM study that the whole network with synchronized macro cells has a common understanding on a DL transmission boundary (denotes as the 1st reference point) which indicates the ending boundary of the DL transmission, and an UL reception boundary (denotes as the 2nd reference point) which denotes the starting boundary of the first allowed UL reception within a DL-UL transmission periodicity.

  - The boundary may be considered for RS design

  - The 1st reference point locates before the 2nd reference point.

Figure 1. Illustration of DL and UL transmission boundaries within a DL-UL transmission periodicity

Agreements:

- In terms of the IoT (interference over thermal) increase between two sets of gNBs causing remote interference to each other, two scenarios should be considered for NR-RIM,

  - Scenario #1: IoT increases are detectable by one or more gNBs in both sets,

  - Scenario #2: IoT increase is detectable by one or more gNBs in only one set.

Agreements:

Framework-1, Framework-2.1, Framework-2.2 below are used as starting point for further study, using Framework-0 as basis for comparison.

Note:

- Not all the steps need to be included when making use of a given framework.

- Mechanisms for improving network robustness at both victim and aggressor side can be studied under the NR-RIM frameworks.

  - A victim cell may take actions applying remote mitigation scheme. This detail is FFS

- An aggressor may also be a victim (and vice versa) at least for Scenario #1

            - Framework-0

Workflow of Framework-0

Step 0: Atmospheric ducting phenomenon happens and the remote interference appears

Step 1:

- Victim experiences “sloping” like IoT increase and start RS transmission

- Aggressor starts monitoring RS as configured by OAM

Step 2: Upon reception of RS, Aggressor reports the detected RS to OAM

Step 3: OAM sends remote interference mitigation scheme to Aggressor

Step 4: Aggressor applies remote interference mitigation scheme

Step 5: OAM stops RS monitoring and restores original config. at aggressor side and stop RS transmission at victim side.

            - Framework-1

Workflow of Framework-1

Step 0: Atmospheric ducting phenomenon happens and the remote interference appears

Step 1:

- Victim experiences “sloping” like IoT increase and start RS transmission/monitoring

  - This RS marked as RS-1 is used to assist aggressor(s) to recognize that they are causing remote interference to the victim and to detect/deduce how many UL resources of the victim are impacted by the aggressors.

- Aggressor starts monitoring RS as configured by OAM or when it experiences remote interference with “sloping” IoT increase.

Step 2: Upon reception of RS-1, Aggressor starts remote interference mitigation solutions such as muting some DL transmission symbols and transmits RS to inform victim that the atmospheric ducting phenomenon still exist

- This RS marked as RS-2 is used to assist the victim to decide whether the atmospheric ducting phenomenon still exist.

  - It does not preclude the possibility of using RS-2 for other purposes, pending on further study.

Step 3: Victim continues RS-1 transmission while receiving RS-2. Upon “disappearance” of RS-2, victim stops RS transmission

Step 4: Aggressor continue remote interference mitigation while receiving RS-1. Upon “disappearance” of RS-1, Aggressor restores original configuration when “disappearance” of RS-1.

Note: Although RS-1 and RS-2 carry different functionalities, it might be beneficial to achieve a common design for RS-1 and RS-2.

            - Framework-2.1

Workflow of Framework-2.1

Step 0: Atmospheric ducting phenomenon happens and the remote interference appears

Step 1:

- Victim experiences “sloping” like IoT increase and start RS transmission

  - A set of gNBs might use the same RS, which may carry the set ID.

- Aggressor starts monitoring RS as configured by OAM or when it experiences remote interference with “sloping” IoT increase.

Step 2: Upon reception of RS, Aggressor informs the set of victim gNB(s) the reception of RS through backhaul and apply interference mitigation scheme

- Message exchange in Step 2 could include other information, pending on further study.

Step 3: Upon “disappearance” of RS, Aggressor informs the set of Victim gNB(s) the “disappearance” of RS through backhaul and restore original configuration.

Step 4: Victim stop RS transmission upon the reception of the “disappearance of RS” info through backhaul

            - Framework-2.2

Workflow of Framework-2.2

Step 0: Atmospheric ducting phenomenon happens and the remote interference appears

Step 1:

- Victim experiences “sloping” like IoT increase and start RS transmission

  - A set of gNBs might use the same RS, which may carry the set ID.

- Aggressor starts monitoring RS as configured by OAM or when it experiences remote interference with “sloping” IoT increase.

Step 2: Upon reception of RS, Aggressor informs the set of victim gNB(s) the reception of RS through backhaul

Step 3: Upon reception of the “reception of RS” info received in the backhaul, victim sends info to assist RIM coordination

Step 4: Aggressor applies remote interference mitigation scheme

Step 5: Upon “disappearance” of RS, Aggressor informs Victim the “disappearance” of RS through backhaul.

Step 6: Victim stop RS transmission upon the reception of the “disappearance of RS” info through backhaul

Agreements:

- Inform RAN3 that three frameworks are used as in RAN1 as a starting point for further study. Following information will also be included is the LS.

  - The distance between gNB aggressor and gNB victim can be up to 300 km.

  - Action to RAN3: to provide feedback regarding feasibility of the frameworks

- Draft LS in R1-1809875

Agreements:

- For RIM SI, the evaluation is to be carried out via link-level simulation to evaluate the performance of the reference signals in the NR-RIM frameworks.

Friday session

R1-1809875	[Draft] LS on NR-RIM frameworks	CMCC

Decision: The draft LS is endorsed and final LS is approved in R1-1809987.

R1-1809973	Draft summary on Study on NR-RIM	CMCC

Decision: The document is noted.

Agreements:

For simulation evaluation of reference signals in the NR-RIM frameworks

- Following Descriptions of the RS should be provided

  - RS sequence

  - Length of RS sequence

  - Time/frequency pattern of RS

    - Time pattern (number of symbols)

    - Frequency pattern

- Following analytical metrics of the RS should be provided

  - The complexity of reference signal detection at gNB

  - Overhead

  - Impact on UEs

  - Others

- Simulation

  - Simulation parameters

    - SCS: 30 kHz (mandatory) / 15 KHz (optional)

    - Simulation bandwidth: 20 MHz

    - gNB MIMO configuration: 1T1R (mandatory)/1T2R(optional)

    - Frequency offset: 0 Hz

    - FFT size: to be provided

    - Length of detection window Lsymbol: to be provided

    - Channel model:

      - Option1: AWGN with random complex phase

      - Option2: TDL-E (K-factor = [22] dB, DS = [30] ns, Doppler [0] Hz)

      - FFS: whether one of the two options or both options are mandatory.

    - Delay of received RS: When multiple RSs arrive in the detection window, the arrival time of the i-th RS respect to the start of the detection window, △i , is uniformly distributed within [-Lsymbol, Lsymbol], where Lsymbol is the length of UL symbol based on the numerology of RS.

    - Power of received RS:

      - Option1: Pi of multiple RSs have a power offset with respect to the reference power P0, where the power offset is randomly selected from [-0.5dB, 0.5dB].

      - Use option1 as starting point for evaluation, FFS other option(s), e.g., different power offset ranges.

  - Simulation cases and related metrics

    - Case 1: Single RS + AWGN (mandatory)

      - Metric: the minimum SNR where detection probability of [90%] and a false alarm requirement of [1%]

      - FFS: successful detection time, e.g., one-shot.

    - Case 2: Multiple RS + AWGN (mandatory)

      - Number of total RSs arrived within one detection window: FFS

        - Number of base sequences arrived within the detection window: FFS

      - Metric: FFS.

Email discussion on additional simulation assumptions for RIM till Sept 7th  – Hua (CMCC)

R1-1808927	Design considerations for remote interference management	Huawei, HiSilicon

#### Mechanisms for improving network robustness

R1-1808529	Discussion on mechanism for NR RIM support	LG Electronics

- Proposal 1: Study characteristics of remote CLI in asymmetric interference scenario.

- Proposal 2: Multiple mitigation techniques are supported in RIM scenario. Mechanisms to identify which technique is proper are supported. For example, victim can recommend mitigation technique based on aggressor’s information such as load and the number of gNBs.

- Proposal 3: Existing reference signals can be starting points. It should be further studied the necessary of dedicated reference signal.

- Proposal 4: For backhaul signaling, at least the followings are considered

  - Reference signal configuration

  - Cell/group/cluster ID of aggressor/victim gNB

  - Level of remote CLI

  - Potential RIM techniques

Decision: The document is noted.

R1-1809162	Views on remote interference management for NR	NTT DOCOMO, INC.

- Observation 1: The number of symbols that may suffer remote interference are related to the following factors:

  - Propagation distance between aggressive gNB and victim gNB

  - Guard period in semi-static TDD configuration

  - Subcarrier spacing configuration

- Observation 2: For remote interference management, both interference mitigation at aggressive gNBs and victim gNBs should be studied.

- Observation 3: For remote interference management, the interference mitigation schemes in TD-LTE network can be used as a starting point.

- Observation 4: For remote interference management, the cross-link interference mitigation techniques in flexible duplex such as power control, advanced receiver, etc. should be studied.

Decision: The document is noted.

R1-1809455	Mechanisms for improving network robustness	Qualcomm Incorporated

- Proposal: UE autonomously chooses a new RACH configuration that is robust to remote interference when it senses the remote interference.

Discussion: Questioning whether the choice should be made by the network side, not the UE

Decision: The document is noted.

Tuesday session conclusion: Discuss further offline - Hua (CMCC)

R1-1808067	Discussion on remote interference mitigation mechanisms	Huawei, HiSilicon

R1-1808325	Consideration on RIM framework and mechanisms for improving network robustness	ZTE

R1-1808342	Considerations on remote interference management	Sony

R1-1808408	Views on remote interference management	CATT

R1-1808703	Remote interference management schemes	Intel Corporation

R1-1808785	Mechanisms for improving network robustness	Samsung

R1-1808822	On mechanism for improving network robustness in the presence of remote interference	Ericsson

R1-1809221	Remote interference management for NR	Nokia, Nokia Shanghai Bell

#### Mechanisms for identifying strong gNB interferers

R1-1808841	Discussion on framework for RIM	CMCC

- Proposal 1. For both Framework-1 (w/o backhaul assistance) and Framework-2 (w/ backhaul assistance), characteristics, functionalities and requirements of RS design should be carefully studied.

- Proposal 2. Select one of the frameworks for NR-RIM.

- Proposal 3. Send LS to RAN3 to study potential additional coordination among gNBs for mitigating remote interference based on the proposed Framework-2.

Decision: The document is noted.

R1-1808823	On mechanism for identifying strong gNB interferers	Ericsson

- Proposal 1: Support some level of gNB/gNB group identification, at least to add observability from OAM of the presence of remote interference in the NW

- Proposal 2: Further study what size of the ID space to enable group or individual gNB victim/aggressor identification is needed

- Proposal 3: The following baseline procedure shall be used for RS transmission/monitoring: i) RI identification (victim); ii) RS Tx (victim); iii) RS monitoring (potential aggressor)

- Proposal 4: Requirements on the reference signal should be: i) not to cause ambiguity with legacy UL transmissions; ii) have low overhead; iii) have acceptable (false) detection performance and complexity; iv) be detectable without OFDM symbol alignment; v) enable (group) gNB identification; vi) enable propagation delay estimation; vii) low cubic metric/PAPR

- Proposal 5: Decide, based on evaluations and gNB complexity, which gNB detection window should be the assumption for the reference signal design

- Proposal 6: Decide how to partition RIM RS sounding resources using TDM, FDM and CDM based on agreed RIM interference characteristics

Decision: The document is noted.

R1-1808326	Consideration on reference signal design for remote interference management	ZTE

- Observation 1: The cyclic prefix of SS/PBCH block is not enough to provide protection against multi-path effect in remote interference management.

- Observation 2: On the ability to counter multi-path effect, it seems that CSI-RS is too sparse and the length of CSI-RS sequence is not long enough for remote interference management.

- Observation 3: Considering the detection complexity and ability to counter multi-path delay, PRACH preamble is also inappropriate for RIM.

- Observation 4: The legacy reference signals including SS/PBCH, CSI-RS and PRACH preamble cannot meet all the requirements.

- Proposal 1: The reference signal should provide performance with sufficiently good auto-correlation and cross-correlation, low false alarming rate and miss detection probability and so on.

- Proposal 2:  The overhead of reference signal should be taken into consideration to ensure that the impact to the throughput of the network is minimized

- Proposal 3: The impact between reference signal for RIM and existing reference signals should be minimized.

- Proposal 4: The reference signal for RIM should be well designed to counter the serious multi-path delay spread.

- Proposal 5:  It is necessary to distinguish the two reference signals in Scheme-1.

- Proposal 6: The RIM RS in Scheme-2 should convey victim gNB ID information for victim identification and inter-gNB communications through backhaul.

- Proposal 7:  The reference signal should be able to assist the aggressor gNB to identity how far away the impacted gNB is.

- Proposal 8:  A new and dedicated reference signal is needed in remote interference management.

- Proposal 9: Reference signal with a longer cyclic prefix or special structure would be needed to counter the multi-path delay spread in RIM.

- Proposal 10: Reference signal with a configurable periodicity is preferred to satisfy different requirements.

- Proposal 11: The frequency band of RIM reference signal needs further study to balance the overhead and performance requirements.

- Proposal 12: The reference signal for RIM may still reuse the generator, pattern and so on of the legacy reference signals with some necessary modifications.

- Proposal 13: In order to reduce the NR-RIM standardization workload and work out a feasible scheme to mitigate remote interference, we should strive for a unified RS design if multiple reference signals are needed.

- Proposal 14:  The simulation assumption and performance metrics in Appendix A1 could be used to evaluate the RIM-RS.

Decision: The document is noted.

Tuesday session conclusion: Discuss further offline - Hua (CMCC)

R1-1808068	Discussion on reference signal design for identifying remote interference	Huawei, HiSilicon

R1-1808409	Discussion on identification of strong gNB interferers	CATT

R1-1808530	Discussion on procedure for NR RIM	LG Electronics

R1-1808704	Mechanism for identifying remote gNB interference	Intel Corporation

R1-1808786	Mechaisms for identifying strong gNB interferers	Samsung

R1-1809222	Considerations on identifying strong gNB interferers	Nokia, Nokia Shanghai Bell

R1-1809252	Some Considerations on Remote Interference Management for NR	China Telecommunications

R1-1809456	Mechanisms for identifying strong gNB interferers	Qualcomm Incorporated

R1-1809463	On remote interference management	CEWiT

#### Other

R1-1808824	Evaluation methodology and assumptions for RIM reference signal evaluations	Ericsson

R1-1808842	Discussion on RS design for RIM	CMCC

R1-1808843	Discussion on evaluation assumption for RIM	CMCC

### Study on Physical Layer Enhancements for NR URLLC

FS_NR_L1enh_URLLC; SID in RP-181477. Please refer to the SID for detailed scoping

R1-1808098	Draft skeleton of TR 38.824: Study on physical layer enhancements for NR URLLC	Huawei, HiSilicon

Discussion: Annex 2 structure was discussed - Transport industry should be removed acc. to some companies (already covered by V2X).

Decision: The document is noted.

Proposals: The draft skeleton is endorsed with the following updates:

- Removing subsections A.2.1/2/3

- Updating the section # for link level simulation assumptions to A.3

  - Remove the subsections under the link level simulation assumptions

Discuss further offline

Thursday

R1-1809861	Draft skeleton of TR 38.824: Study on physical layer enhancements for NR URLLC	Huawei, HiSilicon

Decision: The TR 38.824 v0.0.0 is endorsed.

R1-1808943	Work plan for Rel-16 SI on physical layer enhancements for URLLC	Huawei, HiSilicon

Discussion: Comment from RAN1 chair --> no TUs have been allocated to the Jan 19 ad-hoc meeting

Decision: The document is noted.

#### Layer 1 enhancements

Including necessary evaluations and potential enhancements to PDCCH, UCI, PUSCH, & scheduling/HARQ/CSI processing timeline

R1-1809516	L1 enhancements for URLLC	Huawei, HiSilicon	(R1-1808162)

Proposal 1: Take the requirements in Table 1 as the starting point for Rel-16 NR URLLC evaluation with the following assumptions:

- 1 ms core network delay for factory automation

- 3 ms core network delay for power distribution and transport industry

Proposal 2: The performance metric for Rel-16 NR URLLC evaluation includes at least the percentage of users satisfying reliability and latency requirements. The target percentage should be 100%.

Proposal 3: Take the simulation settings in Table 2 as the starting point for Rel-16 NR URLLC system level evaluation for transport industry.

Proposal 4: Take the simulation settings in Table 3 as the starting point for Rel-16 NR URLLC system level evaluation for electrical power distribution.

Proposal 5: Take the simulation settings in Table 4 as the starting point for Rel-16 NR URLLC system level evaluation for factory automation.

For baseline performance achievable with Rel-15 URLLC,

Observation 1: For 40 MHz bandwidth and 10 vehicles per cell, from downlink transmission perspective about 58% and 85% vehicles could achieve the required 2 ms latency and 99.999% reliability in Urban Macro and Highway deployments respectively.

Observation 2: For 40 MHz bandwidth and 10 vehicles per cell, from uplink transmission perspective about 47% and 80% vehicles could achieve the required 2 ms latency and 99.999% reliability in Urban Macro and Highway deployments respectively.

Observation 3: For 40 MHz bandwidth and 10 UEs per cell for differential protection, from downlink transmission perspective about 77% UEs could achieve the required 2 ms latency and 99.999% reliability in Urban Macro deployment.

For PDCCH enhancements,

Observation 4: The single user reliability of 99.999% can be fulfilled with AL=8 for the payload size of 40 bits without considering any SINR margin.

Observation 5: A large amount of users cannot satisfy the requirement due to PDCCH blocking. PDCCH enhancements are needed for Rel-16 NR URLLC to reduce the PDCCH blocking probability.

Observation 6: PDCCH blocking is decreased significantly by using compact DCI.

Proposal 6: Study mechanism(s) to support compact DCI for Rel-16 URLLC.

Observation 7: PDCCH blocking probability is decreased significantly with PDCCH repetition.

Proposal 7: Study mechanism(s) to support PDCCH repetition with fast feedback for R16 URLLC.

For PUSCH enhancements,

Proposal 8: Study mechanism(s) to support mini-slot based repetition for both grant based PUSCH and grant-free PUSCH.

Proposal 9: Intra-slot and inter-slot frequency hopping pattern for mini-slot based repetition can be studied by taking into account the diversity gain and the DMRS overhead.

For UCI enhancements,

Observation 8: Restricting one PUCCH transmission for HARQ-ACK within one slot as done in Rel-15 would increase latency, which is unacceptable at least for some URLLC use cases with low latency requirement and short packet arrival interval.

Observation 9: The Rel-15 HARQ feedback method with configured semi-static HARQ-ACK codebook is not applied to intra-UE DL multiplexing.

Proposal 10: Study enhanced HARQ feedback for Rel-16 URLLC. Enhancements can include at least the following aspects:

- Support of more than one PUCCH transmission possibilities within one slot.

- HARQ-ACK feedback with semi-static codebook is extended to apply to intra-UE DL multiplexing.

For CSI enhancements,

Proposal 11: Study mechanism(s) to support A-CSI report on short PUCCH triggered by DL DCI for Rel-16 NR URLLC.

- Whether and how to separate the timing/resources for HARQ-ACK feedback and aCSI feedback triggered by the same DL DCI could be studied.

- How to enable fast CSI measurement for URLLC traffic needs to be studied.

For enhancements on processing timeline,

Observation 10: The DL system resource efficiency could be low if the UE is not allowed to receive a new scheduled unicast PDSCH transmission with a HARQ ID before the HARQ-ACK for an earlier transmission with the same HARQ ID is transmitted.

Observation 11: The latency for URLLC UE is too long if the scheduled unicast PDSCH transmission of  A comes before the scheduled unicast PDSCH transmission of  B,  and then the (baseline capability) UE is not allowed to send the HARQ-ACK for A after the HARQ-ACK for B.

Observation 12: The latency for URLLC UE is too large if the DL/UL grant scheduling unicast PDSCH/PUSCH transmission A comes before (in time) the DL/UL grant scheduling unicast PUSCH transmission B, and the UE is not allowed to be scheduled such that PDSCH/PUSCH for B is before the PDSCH/PUSCH for A.

Proposal 12: The scheduling/HARQ scheduling limitation in Release 15 should be removed for Release 16 URLLC, and the UE can have the following behavior:

- For each HARQ process ID, the UE can receive a scheduled unicast PDSCH transmission with the same HARQ process ID before the end of the expected transmission of the HARQ-ACK for an earlier transmission on the same HARQ process ID.

- For any two HARQ process IDs A and B for a given cell, if scheduled unicast PDSCH transmission for A comes before the scheduled unicast PDSCH transmission for B then the (baseline capability) UE can be triggered to send the HARQ-ACK for B before the HARQ-ACK for A.

- For any two HARQ process IDs A and B for a given cell, if the DL/UL grant scheduling unicast PDSCH/PUSCH transmission A comes before (in time) the DL/UL grant scheduling unicast PUSCH transmission B, UE can be scheduled such that PDSCH/PUSCH for B is before the PDSCH/PUSCH for A.

Decision: The document is noted.

R1-1808787	Layer 1 enhancement for URLLC	Samsung

- Proposal 1: A Rel-16 UE supports the same number of PDCCH candidates and the same number of non-overlapping CCEs per slot as a Rel-15 UE.

- Proposal 2: Study DCI formats for Rel-16 URLLC with configurable field sizes.

- Proposal 3: Study PDSCH receptions without associated DCI formats (DCI format with size 0).

- Proposal 4: Study design of DCI formats for Rel-16 URLLC that enable scheduling of simultaneous PDSCH receptions by multiple UEs.

- Proposal 5: Study a need for PDCCH repetitions while also considering PDSCH coverage, introduction of CCE aggregation levels larger than 16, limit on number of non-overlapping CCEs per slot, and simultaneous configuration of CORESETs with different number of symbols.

- Proposal 6: Study support of multiple PUCCH format 0/2 transmissions for HARQ-ACK in a slot.

- Proposal 7: Study the support of simultaneous PUSCH and PUCCH transmissions.

- Proposal 8: If simultaneous PUSCH and PUCCH transmissions by a UE is not default operation, when a grant-free PUSCH transmission from a UE overlaps in time with a PUCCH transmission with HARQ-ACK from the UE and the UE cannot multiplex the HARQ-ACK in the PUSCH, the UE drops the PUSCH or the PUCCH transmission based on network configuration.

- Proposal 9: If simultaneous PUSCH and PUCCH transmissions by a UE is not default operation, study corrections in determining number of REs for HARQ-ACK (or UCI) multiplexing in a PUSCH. If HARQ-ACK BLER larger than data BLER is allowed, introduce  values smaller than 1. If HARQ-ACK multiplexing in one PUSCH of multiple PUSCHs with a same TB is allowed, study increasing the number of REs for multiplexing HARQ-ACK in the PUSCH.

- Proposal 10: If simultaneous PUSCH and PUCCH transmissions by a UE is not default operation and the UE would simultaneously transmit HARQ-ACK and data in a PUSCH that includes only symbols with DMRS, consider transmitting either only data in the PUSCH or only HARQ-ACK in the PUCCH.

- Proposal 11: Consider mini-slot level repetition for grant-free UL transmission.

- Proposal 12: For mini-slot level repetition, further study the hopping methods according to different repetition methods with two hops as a baseline.

- Proposal 13: Consider whether or not it is necessary to satisfy the minimum latency requirement for eRULLC with lower subcarrier spacing for grant-based PDSCH and grant-free PUSCH.

- Proposal 14: If it is necessary to satisfy the minimum latency requirement for eRULLC with lower subcarrier spacing for grant-based PDSCH and grant-free PUSCH, study new scheduling and HARQ processing time.

- Proposal 15: Study new scheduling and HARQ processing time for eURLLC to satisfy latency requirements for grant-based PUSCH.

- Proposal 16: Study whether or not to support out-of-order HARQ procedures.

Decision: The document is noted.

R1-1809457	Layer 1 enhancements	Qualcomm Incorporated

- Proposal 1: Adopt a PHY-layer signalling for differentiating the channels with different traffic types in Rel. 16 URLLC.

- Proposal 2: To enable fast scheduling for URLLC, the number of non-overlapping CCEs per slot of a serving cell can be revisited.

- Proposal 3: Designing a compact DCI format can be considered for Rel. 16 URLLC.

- Proposal 4: If the PDCCH reliability cannot be satisfied even with the largest defined AL, the gains from PDCCH repetition can be studied for NR URLLC. The starting point could be to evaluate PDCCH repetition (in CC, time, and TRP domain with increased diversity order) that does not require combining across different PDCCH occasions by the UE.

- Proposal 5: To reduce the URLLC latency and improve its PUCCH coverage, multiple HARQ-ACK reporting per slot can be supported.

- Observation 1: The benefit of A-CSI triggering via a DL grant is two-fold: (1) There is no need to spend resources on sending an UL grant to only trigger the A-CSI report, and (2) the up-to-date CSI report can be made available at the gNB to make proper scheduling decisions for TB re-transmissions.

- Proposal 6: For Rel. 16 URLLC, allow for triggering A-CSI reporting via the downlink grant.

- Proposal 7: Study the achievable gains and suitable operating scenarios for enabling mini-slot level repetition and frequency-domain hopping.

- Proposal 8: Reducing the UE’s (N1/N2) and gNB’s (N3/N4) processing timelines can be considered for Rel. 16 URLLC.

- Proposal 9: Consider reducing the CSI computation timeline and study the possible relaxations needed to achieve it for Rel. 16 URLLC.

Decision: The document is noted.

RAN1 Chair comment: Only 1TU to that meeting makes very difficult looking at more contributions. Companies are encouraged to discuss further offline  – Chengyan (Huawei)

Thursday

R1-1809862	Summary of 7.2.6.1 Layer 1 enhancements	Huawei, HiSilicon

Decision: The document is noted.

R1-1809863	Text proposal on capturing specific use case and requirements for Rel-16 NR URLLC in TR 38.824	Huawei, HiSilicon

Decision: The document is noted.

Friday session

R1-1809951	Offline discussion summary of 7.2.6.1 Layer 1 enhancements	Huawei, HiSilicon

Decision: The document is noted.

Agreements:

- Select one or more representative use case(s) for the prioritized use cases in the SID and/or the Rel-15 enabled use case for evaluation, which use case(s) to evaluate is up to companies.

  - Further discussion how/whether to capture them in the TR

  - Further discussion other detailed simulation assumptions

The following table of representative use cases for selection for evaluation is an example as the starting point for further discussion:

| Use case (Clause #) | Reliability (%) | Latency (ms) | # of UEs (per cell) | Data packet size and traffic model | Description |
| --- | --- | --- | --- | --- | --- |
| Transport Industry (22.186: 5.5) | [99.999] | [5] (end to end latency) | [30] | DL: [TBD] byte; ftp model 3 with arrival interval [TBD] s UL: [TBD] byte; Periodic with arrival interval [TBD] s | Remote driving |
| Power distribution (22.804:5.6.4 &5.6.6) | 99.9999 | 5(end to end latency) | 8 | [80] byte ftp model 3 with arrival interval 100ms | Power distribution grid fault and outage management |
|  | [99.999] | 15(end to end latency) | 8 | 250 byte Periodic and deterministic with arrival interval 0.833 ms | Differential protection |
| Factory automation (22.804: 5.3.2) | 99.9999 | [2](end to end latency) | [4, 40] | 20 byte, 50 byte Periodic and deterministic traffic model | Motion control |
| Rel-15 enabled use case (e.g. AR/VR) | 99.999 | [1ms] (air interface delay) | 1, 5, 10, 20 | [32, 256] bytes FTP model 2/3 or periodic with different arrival rates | Companies report the combination of the requirement |

  - All the entries in the above table are subject to further discussion which can be revisited in the next meeting

  - Note: The details on above the requirements can refer to R1-1809337.

  - Note: 3ms ~ 10ms CN delay for differential protection (i.e. power distribution case 2) could be considered.

  - Note: Rel-15 higher layer mechanisms for reliability may be applicable for achieving the reliability requirement

  - Note: The reliability and latency are as defined in 22.186.

  - Note: For AR/VR, the requirement can refer to section 7.2.3 in TS 22.261.

  - Note: FFS whether the packet size is based on application layer or L2/L3. The packet size listed in the table needs to further discussed, especially depending on the outcome of whether the packet size is based on application layer or L2/L3

  - Further discussion on how to map the requirements (e.g., reliability, latency, etc.) to RAN-level requirements

Email discussion on additional simuliation assumptions till Sept 7th (including proposals 4/5/6 in R1-1809951) – Chengyan (Huawei)

Agreement:

- Further discussion till next meeting regarding whether/how to evaluate the number of users, the % of users, etc., satisfying reliability and latency requirements.

Agreements:

- Companies are encouraged to report the CDF of UE geometry

- Further discussion whether/how to re-use the deployment and channel models in the existing TRs (e.g. 38.802, 37.885 and 38.901)

Agreements:

Further evaluate the potential PDCCH enhancements for NR Rel-16 URLLC.

- Further evaluate PDCCH reliability

- Further evaluate PDCCH blocking

  - Companies describe the resource utilization

- Complexity should be considered

- Latency of the enhancement(s) should be considered

Agreement:

- Study further how to enable more than one PUCCH for HARQ-ACK transmission within a slot.

Agreements:

Study further whether/how to enable enhanced reporting procedure/feedback for HARQ-ACK.

- Enhanced HARQ-ACK multiplexing on PUSCH and PUCCH

- Finer indication for HARQ feedback timing, e.g. symbol-level, half-slot, etc.

  - Note: this may be related to more than one PUCCH for HARQ-ACK tx within a slot

- Other enablers are not precluded

Agreements:

Study the need for enhanced CSI reporting/measurement mechanisms. E.g.,

- DMRS based CSI

- A-CSI on PUCCH

- Trigger by DL assignment

- Enhanced CSI reporting mode

- Other approaches are not precluded

R1-1808147	Enhancements to Uplink and Downlink Physical Channels for NR URLLC	Ericsson

R1-1808161	Achieving ultra-low latency in grant-based URLLC transmission with minimum overhead	Idaho National Laboratory

R1-1808191	On compact DCI for URLLC	Mitsubishi Electric RCE

R1-1808211	URLLC PHY enhancements	ZTE

R1-1808248	Layer 1 enhancements for URLLC	vivo

R1-1808255	Discussion on enhanced reliability	TCL Communication Ltd.

R1-1808285	Study and evaluations of NR control channels for URLLC	MediaTek Inc.

R1-1808301	On improved reliability of UCI piggybacked PUSCH	Fujitsu

R1-1808308	HARQ Processing Timeline Enhancements for NR URLLC	Fraunhofer HHI

R1-1808343	On Layer 1 enhancement for URLLC	Sony

R1-1808410	Discussion on PHY enhancements for Rel-16 URLLC	CATT

R1-1808531	Discussion on layer 1 enhancements	LG Electronics

R1-1808568	On the L1 enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

R1-1808617	Enhancements to PDCCH	NEC

R1-1808705	Views on evaluation methodology and Layer 1 enhancements for Rel-16 eURLLC	Intel Corporation

R1-1808810	Consideration on URLLC physical layer enhancement	Spreadtrum Communications

R1-1808815	Discussion on PDCCH enhancements for URLLC	ETRI

R1-1808827	On NR URLLC L1 enhancements	Panasonic

R1-1808844	Discussion on out-of-order scheduling and HARQ	CMCC

R1-1808854	PDCCH enhancements for URLLC	China Telecommunications

R1-1808908	Layer 1 enhancement for URLLC	Guangdong OPPO Mobile Telecom

R1-1808919	Discussion on UL pre-emption for URLLC reliability	Potevio

R1-1809055	Overview of Physical Layer enhancements for NR URLLC	AT&T

R1-1809099	Views on potential physical layer enhancements for URLLC 	InterDigital, Inc.

R1-1809112	PUCCH enhancements for URLLC HARQ-ACK feedback	Sharp

R1-1809125	HARQ enhancement for URLLC	III

R1-1809163	Physical Layer Enhancements for NR URLLC	NTT DOCOMO, INC.

R1-1809224	PDCCH combining strategy	TCL Communication Ltd

Withdrawn

R1-1809288	Considerations on PDCCH enhancements to support URLLC	CAICT

R1-1809289	Considerations on UCI transmission enhancements to support URLLC	CAICT

R1-1809291	Discussion on the PDCCH repetition for NR URLLC	Xiaomi

R1-1809318	Considerations for PDCCH enhancements	Sequans Communications

R1-1809320	PUCCH enhancements	Sequans Communications

R1-1809400	Layer 1 URLLC Enhancements	Motorola Mobility, Lenovo

R1-1809981	Summary of 7.2.6.1 Layer 1 enhancements	Huawei, HiSilicon

#### UL inter UE Tx prioritization/multiplexing

R1-1808286	Evaluation and analysis for UL multiplexing between eMBB and URLLC	MediaTek Inc.

- Proposal 1: Uplink preemption indication should not be considered in Rel-16 unless URLLC performance can be guaranteed to achieve its minimum L1 latency requirement.

- Proposal 2: It is essential to model the following in order to realistically evaluate the performance of uplink preemption schemes in a system-level simulation environment:

  - Accurate eMBB processing timeline modeling (including, at least, PI monitoring periodicity and eMBB processing capability)

  - HARQ re-transmission modeling with soft-combining capability (at least for eMBB UEs).

- Proposal 3: System-level simulation studies for uplink multiplexing should consider the scenario where only some of the eMBB UEs are capable of supporting preemption indication.

- Proposal 4: Further enhancements should be investigated for power control and re-scheduling methods in Rel-16 to improve UL dynamic multiplexing between eMBB and URLLC.

Decision: The document is noted.

R1-1808706	On inter-UE multiplexing in UL	Intel Corporation

- Proposal 1: Mechanisms for UL inter-UE multiplexing should not be limited to cater to only multiplexing of different service types (eMBB vs. URLLC), but also consider the general problem of multiplexing services with different QoS requirements.

- Proposal 2: NR provides sufficient mechanisms to configure different power control parameters for different services either dynamically or semi-statically

- Observation 1: Dynamic reduction of eMBB power can be better classified as a generalized form of interruption indication and should be discussed in that context rather than power control techniques.

- Proposal 3: Support of interruption or continuation indication can be further studied, subject to application time of interruption indication being at least N2 according to the supported UE minimum processing time capability as a starting point

  - Further studies are needed, taking UE power consumption and monitoring complexity, and reliability of indication into account

  - FFS: Details of UE specific or group-based signaling, implicit/explicit indication, etc.

  - FFS: Feasibility of application time shorter than N2

Decision: The document is noted.

Tuesday session conclusion: Continue offline discussion – Xueming (vivo)

Friday session

R1-1809995	Offline discussion for UL inter UE Tx prioritization/multiplexing	vivo

Decision: The document is noted.

Agreements:

- RAN1 to study the potential enhancements for UL inter UE Tx prioritization/multiplexing

  - Performance study of the enhanced UL inter UE Tx prioritization/multiplexing mechanisms using Re-15 mechanisms as the performance benchmark

    - The use cases and scenarios adopted in L1 enhancements for URLLC are considered for the evaluation of UL inter UE Tx prioritization/multiplexing

    - Other factors to be considered such as overhead, capability, etc.

  - Study the UE UL cancelation mechanisms, including at least the following aspects

    - The potential mechanisms may include UE UL cancelation/pausing indication, UL continuation indication, UL re-scheduling indication

    - Physical channel/signal used for the UL cancelation indication

    - UE Processing timeline for the UL cancelation indication

    - UE monitoring behaviours for the UL cancelation indication

    - UE PDCCH monitoring capability, if the UL cancelation indication is by PDCCH

    - Methods to ensure the reliability of the indication for UE UL cancelation

  - Study the UL power control enhancements

  - Study other enhancements for the multiplexing between a grant-based UL transmission from a UE and a grant-free UL transmission from another UE

R1-1808099	UL inter-UE transmission prioritization and multiplexing	Huawei, HiSilicon

R1-1808148	Inter-UE Prioritization and Multiplexing of  UL Transmissions	Ericsson

R1-1808212	On Inter-UE multiplexing between eMBB and URLLC	ZTE

R1-1808249	UL inter UE Tx prioritization for URLLC	vivo

R1-1808302	Discussion on UL preemption indication	Fujitsu

R1-1808306	UL multiplexing for URLLC	China Telecommunications

R1-1808344	On UL inter UE pre-emption	Sony

R1-1808411	Discussion on inter-UE UL multiplexing	CATT

R1-1808447	Discussions on UL inter-UE Tx multiplexing	Mitsubishi Electric Co.

R1-1808532	Discussion on UL inter UE Tx prioritization	LG Electronics

R1-1808565	UL inter UE Tx prioritization/multiplexing of eMBB and URLLC	NEC

R1-1808569	On UL inter-UE multiplexing between eMBB and URLLC	Nokia, Nokia Shanghai Bell

R1-1808788	Uplink inter UE multiplexing/prioritization for enhanced URLLC	Samsung

R1-1808797	Discussion on UL inter UE Tx prioritization/multiplexing	Spreadtrum Communications

R1-1808845	Discussion on UL inter UE Tx prioritization/multiplexing	CMCC

R1-1808868	Discussion on Inter UE UL multiplexing	III

R1-1808909	Consideration on UL inter UE Tx prioritization and multiplexing	Guangdong OPPO Mobile Telecom

R1-1808916	 Discussion on UL inter UE Tx multiplexing	ASUSTEK COMPUTER (SHANGHAI)

R1-1809100	Discussion on UL inter-UE multiplexing for URLLC 	InterDigital, Inc.

R1-1809113	Priority handling and multiplexing among PUSCHs with different latency and reliability requirements	Sharp

R1-1809164	Uplink transmission prioritization/multiplexing for NR URLLC	NTT DOCOMO, INC.

R1-1809327	Considerations for UL multiplexing with different reliability requirements	Sequans Communications

R1-1809404	Considerations on UL eMBB/URLLC multiplexing	KT Corp.

R1-1809458	UL inter UE Tx prioritization-multiplexing	Qualcomm Incorporated

#### Enhanced UL grant-free transmissions

R1-1809165	Enhanced UL transmission with configured grant for URLLC	NTT DOCOMO, INC.

- Proposal 1: Study multiple configured grant configurations on a serving cell.

  - The initial transmission of a TB shall start at the first transmission occasion of the K transmissions.

  - The initial transmission of a TB for different configured grant configurations is on different TTIs.

  - Different HARQ process IDs are used for different configured grant configurations.

  - The UE shall not transmit PUSCH simultaneously using different configured grant configurations on a serving cell.

- Proposal 2: Study mini-slot repetitions as the promising candidates for URLLC enhancements and capture the benefits and advantages of them in the TR.

  - PUSCH repetitions shorter than one repetition per slot (e.g., repetitions within a slot).

  - PUSCH repetitions with multiple-TRPs.

    - Enabling different QCLs/precoders/beams across repetitions.

- Proposal 3: Study further detailed options of PUSCH repetition.

  - Frequency-hopping

    - E.g., the number of repetitions in the first hop is floor(N/2), the number of repetitions in the 2nd hop is ceiling (N/2) where N is the number of repetitions within a slot

  - Time-domain allocation taking into account slot boundary

    - the resources for repetition are in consecutive available UL or flexible symbols within one slot; if one repetition is about to cross the slot boundary, it is postponed to the earliest available symbol for PUSCH transmission in the next slot

Observation 1:

- Unless strong benefit is identified, explicit positive HARQ-ACK feedback from gNB to UE is not supported.

  - UL grant scrambled by C-RNTI or new-RNTI scheduling the new TB transmission of the same HARQ process can indicate “ACK”

  - UL grant scrambled by CS-RNTI scheduling the same TB initially transmitted without grant can indicate “NACK”

    - Above UL grant scheduling the new transmission or retransmission can be used during and after the K repetition

- If strong benefit is identified, explicit positive HARQ-ACK feedback from gNB to UE is realized by a PDCCH.

Decision: The document is noted.

R1-1808910	Grant free transmission enhancement	Guangdong OPPO Mobile Telecom

- Proposal 1: Explicit HARQ-ACK can improve transmission reliability due to it can recognize miss-detection of PUSCH.

- Proposal 2: Physical channel design for explicit HARQ-ACK should be considered. Group-common DCI and Sequence based channel can be studied to support explicit HARQ-ACK feedback.

- Proposal 3:Mini-slot repetition can reduce latency.

- Proposal 4: Mini-slot hopping enhancement can reduce collision.

- Proposal 5: Restriction on piggybacked UCI type and bit number in grant free is necessary for URLLC.

Decision: The document is noted.

Thursday conclusion: Continue offline discussion – Lihui (NTT DOCOMO)

Friday session

R1-1809979	Summary of 7.2.6.3 Enhanced UL grant-free transmissions	NTT DOCOMO

Decision: The document is noted.

Agreements:

- Study further whether/how multiple active configured grants for a BWP of a serving cell.

  - Identify potential specification impacts and options for both type 1 and type 2

    - At least Activation/deactivation mechanism for Type2

      - E.g., whether each configuration is activated/deactivated or multiple configurations are activated/deactivated

    - Study how to support repetitions with multiple configurations for a BWP of a serving cell

    - FFS HARQ process ID determination for both type 1 and type 2

    - FFS other specification impacts for both type 1 and type 2

  - Study the performance impacts

Agreements:

- Study further whether/how on ensuring K repetitions.

- Study further on PUSCH repetitions within a slot for configured grant.

R1-1808100	Enhanced UL configured grant transmissions	Huawei, HiSilicon

R1-1808149	Enhancements to UL Grant-free Transmission for NR URLLC	Ericsson

R1-1808213	Enhanced UL grant-free transmissions	ZTE

R1-1808250	Enhanced UL grant-free transmissions  for URLLC	vivo

R1-1808287	Evaluation of configured-grant for NR URLLC	MediaTek Inc.

R1-1808304	Discussion on the Reliability Enhancement for Grant-free Transmission	China Telecommunications

R1-1808345	On Enhancement of UL grant-free transmissions	Sony

R1-1808412	On enhancements to configured UL grant operation	CATT

R1-1808533	Discussion on enhancement for grant-free transmission	LG Electronics

R1-1808570	On Configured Grant enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

R1-1808621	Discussion on enhanced UL grant-free transmissions	Panasonic Corporation

R1-1808707	Enhancements to configured grant PUSCH	Intel Corporation

R1-1808789	Potential enhancement for UL grant-free transmission	Samsung

R1-1808809	Consideration on enhanced UL grant-free transmissions	Spreadtrum Communications

R1-1809101	Views on enhanced UL grant-free transmissions for URLLC 	InterDigital, Inc.

R1-1809114	Reliability enhancements for UL configured grant	Sharp

R1-1809294	Discussion on Configured Grant Enhancements	III

R1-1809319	Enhancements for grant free transmissions	Sequans Communications

R1-1809459	Enhanced SPS and grant-free transmissions	Qualcomm Incorporated

R1-1809464	Discussion on enhanced UL grant-free transmissions	KDDI Corporation

#### Other

R1-1808251	Other enhancements for URLLC	vivo

R1-1808256	Discussion on two stage feedback for URLLC	TCL Communication Ltd.

R1-1808346	Multiple active BWPs	Sony

R1-1808534	Discussion on differentiation of eMBB and URLLC services	LG Electronics

R1-1808571	On compact DCI for NR URLLC	Nokia, Nokia Shanghai Bell

R1-1808572	On PDCCH repetition for NR URLLC	Nokia, Nokia Shanghai Bell

R1-1808573	On the PDCCH limits on the number of CCEs for channel estimation and BDs for URLLC	Nokia, Nokia Shanghai Bell

R1-1808574	On enhanced HARQ-ACK feedback for URLLC	Nokia, Nokia Shanghai Bell

R1-1808575	CQI reporting mode enhancements for URLLC	Nokia, Nokia Shanghai Bell

R1-1808576	On scheduled PUSCH (& PDSCH) repetition enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

R1-1808577	On the out-of-order HARQ-ACK support for URLLC	Nokia, Nokia Shanghai Bell

R1-1808596	HARQ-ACK feedback for URLLC	InterDigital, Inc.

R1-1808598	URLLC UCI prioritization	InterDigital, Inc.

R1-1808828	On BWP hopping for repetitions and retransmission in NR URLLC	Panasonic

R1-1808846	Discussion on remaining issues of new RNTI for URLLC	CMCC

R1-1808888	Resource allocation enhancements for NR URLLC	Guangdong OPPO Mobile Telecom

R1-1808889	On eMBB and URLLC data differentiation	Guangdong OPPO Mobile Telecom

R1-1808911	Consideration on UL intra UE Tx prioritization and multiplexing	Guangdong OPPO Mobile Telecom

R1-1808973	Enhancements for eMBB/URLLC multiplexing in DL	Sequans Communications

R1-1809040	Latency Evaluation of Rel-15 URLLC	Ericsson

R1-1809041	Reliability Evaluation of Rel-15 URLLC	Ericsson

R1-1809251	Discussion on HARQ-ACK enhancement for URLLC	LG Electronics Inc.

R1-1809317	Considerations for DL automatic repetition	Sequans Communications

R1-1809337	Evaluation assumptions and methodology for Rel-16 NR URLLC	Huawei,China Southern Power Grid, HiSilicon

R1-1809338	Simulation results on the baseline performance achievable with Rel-15 URLLC for the identified use cases	Huawei, HiSilicon

R1-1809339	PDCCH enhancements for URLLC	Huawei, HiSilicon

R1-1809340	Compact DCI  for URLLC	Huawei, HiSilicon

R1-1809341	PDCCH repetition for URLLC	Huawei, HiSilicon

R1-1809342	HARQ-ACK enhancements for URLLC	Huawei, HiSilicon

R1-1809343	CSI enhancements for URLLC	Huawei, HiSilicon

R1-1809344	PUSCH enhancements for URLLC	Huawei, HiSilicon

R1-1809345	Enhancements to scheduling/HARQ/CSI processing timeline for URLLC	Huawei, HiSilicon

R1-1809346	Discussion on time synchronization impact in physical layer	Huawei, China Southern Power Grid, HiSilicon

R1-1809347	Potential RAN1 impacts on intra-UE prioritization	Huawei, HiSilicon

R1-1809413	UL Intra-UE Pre-emption for Rel-16	Ericsson

R1-1809414	Time synchronization	Ericsson

R1-1809460	eURLLC Considerations for Factory Automation	Qualcomm Incorporated

### Study on Self Evaluation towards IMT-2020 Submission

FS_5G_eval; SID in RP-181318. Please refer to the SID for detailed scoping

Friday

R1-1809917	Chairman's notes of AI 7.2.7  Study on Self Evaluation towards IMT-2020 Submission	Ad-hoc chair (Ericsson)

Decision: The document is endorsed, content is incorporated below.

R1-1808861	Evaluation results and observations of DL spectral efficiency for IMT2020 self evaluation	MediaTek Inc.

R1-1808862	Evaluation results and observations of UL spectral efficiency for IMT2020 self evaluation	MediaTek Inc.

R1-1809940	Summary on discussion on IMT-2020 evaluation for reliability	Huawei

Agreement:

- The evaluation results in R1-1809940 are endorsed by RAN1.

- Text proposal to TR37.910 is developed based on the results in R1-1809940 that fulfill ITU-R requirements.

R1-1809939	Summary on discussion on IMT-2020 evaluation for mMTC connection density	Huawei

Agreement:

- The evaluation results in R1-1809939 are endorsed by RAN1.

- Text proposal to TR37.910 is developed based on the results in R1-1809939 that fulfill ITU-R requirements.

- For non-full buffer SLS, early data transmission and RRC Resume procedure will be captured as two transmission schemes.

R1-1809938	Summary on discussion on IMT-2020 evaluation for energy efficiency	Huawei

The text proposal in R1-1809938 is endorsed.

R1-1809937	Summary on discussion on IMT-2020 evaluation for eMBB mobility evaluation	Huawei

Agreement:

- The evaluation results in R1-1809937 are endorsed by RAN1.

- Text proposal to TR37.910 is developed based on the results in R1-1809937 that fulfill ITU-R requirements.

R1-1809936	Summary on discussion on IMT-2020 evaluation for eMBB user experienced data rate	Huawei

Agreement:

- The evaluation results in R1-1809936 are endorsed by RAN1.

- Text proposal to TR37.910 is developed based on the results in R1-1809936 as well as the evaluation of 5%-tile user spectral efficiency (for single band) that fulfill ITU-R requirements.

R1-1809935	Summary on discussion on IMT-2020 evaluation for eMBB spectral efficiency	Huawei

Agreement:

- The results will be based on the following overhead assumptions for simulation bandwidth:

  - For DL: 2 OFDM symbols for PDCCH; other components are based on companies’ schemes.

  - For UL: PUCCH format 4 for CSI feedback and format 0 for ACK/NACK feedback; other components are based on companies’ schemes.

  - GP will be taken into account in overhead; 50% of GP symbols are considered as DL overhead; 50% of GP symbols are considered as UL overhead.

- The DL spectral efficiency will be further scaled up to larger than simulation bandwidth by using the agreed guard band ratio scaling method in R1-1803386 and the PDCCH overhead reduction model agreed in Section 2.4 in R1-1807758.

Email approval to compile and endorse the eMBB spectral efficiency results submitted by August 24th, 2018, until August 29th, 2018 (Wu Yong)

R1-1809934	Summary on discussion on IMT-2020 evaluation for peak data rate and peak spectral efficiency	Huawei

Agreement:

The NR parts (Section 5.1.1, Section 5.2.1, and Section B.x.1) of the text proposal in R1-1809934 are endorsed by RAN1.

Email approval to derive the LTE peak spectral efficiency and peak data rate results with the text proposals in R1-1809934 as the starting point, August 29th, 2018 (Wu Yong)

R1-1809947	Summary on discussion on IMT-2020 evaluation for CP and UP latency	Huawei

Agreement:

The text proposal on user plane latency in R1-1809947 is endorsed by RAN1.

Email approval to derive the NR control plane latency evaluation results, based on the summary of CP latency evaluation parameters in R1-1809947, August 29, 2018 (Wu Yong).

#### Evaluation methodology

R1-1808072	Consideration on IMT-2020 self evaluation	Huawei, HiSilicon

R1-1808413	Remaining issues on evaluation methodology for self-evaluation	CATT

R1-1808535	Discussion on evaluation methodology for reliability	LG Electronics

R1-1808708	Evaluation Methodology for IMT-2020 Self-Evaluation	Intel Corporation

R1-1808790	Evaluation methodology for IMT-2020 self-evaluation	Samsung

R1-1808863	IMT-2020 Mobility Evaluation Results	Nokia, Nokia Shanghai Bell

R1-1809259	IMT-2020 self-evaluation: Evaluation Methodology for Mobility	Ericsson

R1-1809728	Consideration on Evaluation methodology of IMT-2020 self-evaluation	ZTE	(rev of R1-1809728)

#### Other

Including evaluation results

R1-1808057	LTE evaluation parameters and configurations for Rural-eMBB test environment	Huawei, HiSilicon

R1-1809508	Considerations and evaluation results for IMT-2020 for eMBB spectral efficiency	Huawei, HiSilicon

Revision of R1-1808073

R1-1809509	Considerations and evaluation results for IMT-2020 for user experienced data rate	Huawei, HiSilicon

Revision of R1-1808074

R1-1808075	Considerations and evaluation results for IMT-2020 for area traffic capacity	Huawei, HiSilicon

R1-1808076	Considerations and evaluation results for IMT-2020 for high mobility	Huawei, HiSilicon

R1-1808077	Considerations and evaluation results for IMT-2020 for peak spectral efficiency and peak data rate	Huawei, HiSilicon

R1-1808078	Considerations and evaluation results for IMT-2020 for network energy efficiency	Huawei, HiSilicon

R1-1808079	Considerations and evaluation results for IMT-2020 for UE energy efficiency	Huawei, HiSilicon

R1-1808080	Considerations and evaluation results for IMT-2020 for mMTC connection density	Huawei, HiSilicon

R1-1809510	Considerations and evaluation results for IMT-2020 for reliability	Huawei, HiSilicon

Revision of R1-1808081

R1-1808082	Considerations and evaluation results for IMT-2020 for control plane latency	Huawei, HiSilicon

R1-1808083	Considerations and evaluation results for IMT-2020 for user plane latency	Huawei, HiSilicon

R1-1809511	Consideration on self evaluation of IMT-2020 for link budget	Huawei, HiSilicon

Revision of R1-1808084

R1-1808182	Updated UMi calibration results for user experience data rate in the multi-layer network deployment	ZTE

R1-1808183	Updated DL spectral efficiency in the eMBB usage scenario for IMT-2020 self evaluation	ZTE

R1-1808184	Link curve performance of LDPC coding in IMT-2020 self evaluation	ZTE

R1-1808185	Updated UL spectral efficiency in the eMBB usage scenario for IMT-2020 self evaluation	ZTE

R1-1808187	Consideration on Evaluation of mobility of IMT-2020 self-evaluation	ZTE

R1-1808188	Consideration on Evaluation of reliability of IMT-2020 self-evaluation	ZTE

R1-1808189	Consideration on Evaluation of connection density of IMT-2020 self-evaluation	ZTE

R1-1808364	Evaluation results for eMBB spectrum efficiency in TDD	NEC

R1-1808365	Evaluation results for eMBB spectrum efficiency in FDD	NEC

R1-1808414	Discussions on detailed link budget evaluation methodology	CATT

R1-1808415	Discussions on calculation of shadow fading margin  for link budget	CATT

R1-1808416	Discussions on penetration margin and SD for link budget	CATT

R1-1808417	Discussions on overhead for spectrum efficiency	CATT

R1-1808418	Self-evaluation on Peak Data Rate and Peak Spectrum Efficiency for NR	CATT

R1-1809719	Self-evaluation on Peak Data Rate and Peak Spectrum Efficiency for LTE	CATT

Revision of R1-1808419

R1-1808420	Self-evaluation on Spectrum Efficiency and Area Traffic Capacity in Indoor Hotspot-eMBB	CATT

R1-1808421	Self-evaluation on Spectrum Efficiency and User Experienced Data Rate in Dense Urban-eMBB	CATT

R1-1808422	Self-evaluation on NR Spectrum Efficiency in  Rural-eMBB	CATT

R1-1808423	Self-evaluation on LTE Spectrum Efficiency in  Rural-eMBB	CATT

R1-1808424	Self-evaluation on CP latency	CATT

R1-1809717	Self-evaluation on UP latency for NR	CATT

Revision of R1-1808425

R1-1809718	Self-evaluation on UP latency for LTE	CATT

Revision of R1-1808426

R1-1808427	Self-evaluation on Energy Efficiency	CATT

R1-1809715	Self-evaluation on Mobility	CATT

Revision of R1-1808428

R1-1809716	Self-evaluation on Reliability	CATT

Revision of R1-1808429

R1-1808448	IMT-2020 self-evaluation: UP latency analysis for FDD and dynamic TDD with UE processing capability 1 (eMBB)	Nokia, Nokia Shanghai Bell

R1-1808449	IMT-2020 self-evaluation: UP latency analysis for FDD and dynamic TDD with UE processing capability 2 (URLLC)	Nokia, Nokia Shanghai Bell

R1-1808450	IMT-2020 self-evaluation: UP latency analysis for static TDD with UE processing capability 1 (eMBB)	Nokia, Nokia Shanghai Bell

R1-1808451	IMT-2020 self-evaluation: UP latency analysis for static TDD with UE processing capability 2 (URLLC)	Nokia, Nokia Shanghai Bell

R1-1808452	IMT-2020 Self Evaluation: CP latency	Nokia, Nokia Shanghai Bell

R1-1808453	IMT-2020 self evaluation: Network energy efficiency	Nokia, Nokia Shanghai Bell

R1-1808536	Evaluation results of Dense Urban-eMBB for IMT-2020 self-evaluation	LG Electronics

R1-1808537	Evaluation results of Rural-eMBB for IMT-2020 self-evaluation	LG Electronics

R1-1808538	Evaluation results of Indoor Hotspot-eMBB for IMT-2020 self-evaluation	LG Electronics

R1-1808539	Initial evaluation results of IMT-2020 self-evaluation for high mobility	LG Electronics

R1-1808546	Initial self evaluation results for Dense Urban-eMBB UL spectrum efficiency	Lenovo, Motorola Mobility

R1-1808709	Peak Data Rate and Peak Spectral Efficiency Evaluations for NR	Intel Corporation

R1-1808710	Spectral Efficiency Evaluation for NR eMBB	Intel Corporation

R1-1808711	Mobility Evaluation for NR eMBB	Intel Corporation

R1-1808712	Area Traffic Capacity Evaluation for NR Indoor Hotspot eMBB	Intel Corporation

R1-1808713	User Experienced Data Rate Evaluation for NR Dense Urban eMBB	Intel Corporation

R1-1808714	Summary of Email Discussion on NR SLS Overhead Calculation for IMT-2020 Self-Evaluation	Intel Corporation

R1-1808715	Link Budget for NR	Intel Corporation

R1-1808716	Connection Density Evaluation for mMTC	Intel Corporation

R1-1809708	Reliability Evaluation for URLLC	Intel Corporation

Revision of R1-1808717

R1-1808791	Evaluation results of peak spectral efficiency and peak data rate for IMT-2020 submission	Samsung

R1-1808792	Evaluation results of Indoor Hotspot-eMBB for IMT-2020 self-evaluation	Samsung

R1-1808793	Evaluation results of Dense Urban-eMBB for IMT-2020 self-evaluation	Samsung

R1-1808794	Evaluation results of Rural-eMBB for IMT-2020 self-evaluation	Samsung

R1-1808795	Evaluation results of mobility for IMT-2020 submission	Samsung

R1-1808847	Initial evaluation results for downlink eMBB spectrum efficiency	CMCC

R1-1808848	Initial system evaluation results for mobility in eMBB scenario	CMCC

R1-1808849	Initial system evaluation results for reliability in URLLC scenario	CMCC

R1-1808850	Initial evaluation results for uplink eMBB spectrum efficiency	CMCC

R1-1808851	Initial link-level self evaluation results for reliability in URLLC scenario	CMCC

R1-1808852	Initial link-level self evaluation results for mobility in eMBB scenario	CMCC

R1-1808855	Initial results of Dense Urban-eMBB for IMT-2020 self-evaluation	China Telecommunications

R1-1808856	Initial results of Rural-eMBB for IMT-2020 self-evaluation	China Telecommunications

R1-1808857	Updated results of Indoor Hotspot-eMBB for IMT-2020 self-evaluation	China Telecommunications

R1-1808858	Link level performance for LDPC coding with BP decoder	China Telecommunications

R1-1808864	Spectral Efficiency Results for the IMT-2020 Self-Evaluation	Nokia, Nokia Shanghai Bell

R1-1808865	IMT-2020 Reliability Evaluation Results (URLLC)	Nokia, Nokia Shanghai Bell

R1-1808866	IMT-2020 Connection Density Evaluation Results (mMTC)	Nokia, Nokia Shanghai Bell

R1-1808867	IMT-2020 Link Budgets	Nokia, Nokia Shanghai Bell

R1-1808883	Initial evaluation results for eMBB spectral efficiency  for IMT-2020	Guangdong OPPO Mobile Telecom

R1-1808931	Summary of offline discussion on IMT-2020 self evaluation for peak spectral efficiency and peak data rate	Huawei

R1-1808932	Summary of offline discussion on IMT-2020 self evaluation for network energy efficiency	Huawei

R1-1808933	Summary of offline discussion on IMT-2020 self evaluation for UE energy efficiency	Huawei

R1-1808934	Summary of offline discussion on IMT-2020 self evaluation for control plane latency	Huawei

R1-1808935	Summary of offline discussion on IMT-2020 self evaluation for user plane latency	Huawei

R1-1808936	Summary of offline discussion on IMT-2020 self evaluation for link budget	Huawei

R1-1809514	Initial evaluation results for IMT2020 self-evaluation Reliability	Sharp

Revision of R1-1809115

R1-1809166	Updated evaluation results of Indoor Hotspot-eMBB for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1809167	Updated evaluation results of dense urban-eMBB for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1809168	Updated evaluation results of rural-eMBB for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1809169	Preliminary evaluation results of mobility for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1809170	Preliminary evaluation results of connection density for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1809171	Preliminary evaluation results of reliability for IMT-2020 self-evaluation	NTT DOCOMO, INC.

R1-1809176	Discussion on NR link budget evaluation	CHTTL

R1-1809497	Evaluation results of Indoor Hotspot-eMBB spectral efficiency for IMT-2020 self-evaluation	ITRI

Revision of R1-1809187

R1-1809188	Evaluation results of Dense Urban-eMBB spectral efficiency for IMT-2020 self-evaluation	ITRI

R1-1809189	Evaluation results of Rural-eMBB spectral efficiency for IMT-2020 self-evaluation	ITRI

R1-1809260	An overview of the IMT-2020 Evaluations - LTE	Ericsson

R1-1809261	An overview of the IMT-2020 Evaluations - NR	Ericsson

R1-1809262	IMT-2020 self-evaluation: mMTC full buffer connection density	Ericsson

R1-1809263	IMT-2020 self-evaluation: mMTC non-full buffer connection density for LTE-MTC and NB-IoT	Ericsson

R1-1809264	IMT-2020 self-evaluation: Mobility evaluations for LTE	Ericsson

R1-1809265	IMT-2020 self-evaluation: Mobility evaluations for NR	Ericsson

R1-1809266	IMT-2020 Self-Evaluation: NR Link Budgets	Ericsson

R1-1809267	IMT-2020 self-evaluation: On eMBB spectral efficiency - LTE	Ericsson

R1-1809268	IMT-2020 self-evaluation: On eMBB spectral efficiency - NR	Ericsson

R1-1809269	IMT-2020 self-evaluation: On eMBB user experienced data rate - NR	Ericsson

R1-1809270	IMT-2020 self-evaluation: On eMBB area traffic capacity - NR	Ericsson

R1-1809271	IMT-2020 self-evaluation: Peak data rate and peak spectrum efficiency evaluations for LTE	Ericsson

R1-1809272	IMT-2020 self-evaluation: Peak data rate and peak spectrum efficiency evaluations for NR	Ericsson

R1-1809273	IMT-2020 self-evaluation: Radio Network Energy Performance - LTE	Ericsson

R1-1809274	IMT-2020 self-evaluation: Radio Network Energy Performance - NR	Ericsson

R1-1809275	IMT-2020 self-evaluation: Reliability in NR	Ericsson

R1-1809276	IMT-2020 self-evaluation: Reliability in LTE	Ericsson

R1-1809277	IMT-2020 self-evaluation: UP latency in NR	Ericsson

R1-1809278	IMT-2020 self-evaluation: UP latency in LTE	Ericsson

R1-1809279	IMT-2020 self-evaluation: CP latency in NR	Ericsson

R1-1809290	Self evaluation on peak spectral efficiency of FR1 towards IMT-2020	CAICT

R1-1809495	IMT-2020 Indoor Downlink Spectral Efficiency Results	Motorola Mobility, Lenovo

Revision of R1-1809401

R1-1809461	IMT-2020 eMBB spectral efficiency evaluations	Qualcomm Incorporated

R1-1809465	Rural-eMBB results for IMT-2020 self-evaluation	CEWiT

### Other

R1-1808252	On NR UE power saving	vivo

R1-1808615	Power model for evaluating UE power saving techniques	Apple Inc.

R1-1808718	On RS Enhancement to Support Multi-TRP Transmission	Intel Corporation

R1-1808719	On UL/DL Beam Management Latency and Overhead Reduction	Intel Corporation

R1-1808720	On SCell Beam Failure Recovery	Intel Corporation

R1-1808721	On Interference-aware Beam Measurement and Reporting	Intel Corporation

R1-1808853	Discussion on scenarios and enhancements for NR positioning	CMCC

R1-1808949	Enhancements on CSI reporting and codebook design	Huawei, HiSilicon

R1-1808950	Enhancements on Type II CSI for more than 2 layers	Huawei, HiSilicon

R1-1809117	Enhancements on multi-TRP/panel transmission in NR	Huawei, HiSilicon

R1-1809118	Discussion on multi-TRP/panel transmission for reliability and robustness in NR	Huawei, HiSilicon

R1-1809119	Beam management enhancements for latency and overhead reduction	Huawei, HiSilicon

R1-1809120	Discussion on BFR for Scell	Huawei, HiSilicon

R1-1809121	Discussion on reciprocity based CSI acquisition to reduce CSI overhead	Huawei, HiSilicon

R1-1809122	Discussion on panel-based UL beam selection	Huawei, HiSilicon

R1-1809123	Beam measurement and reporting using L1-RSRQ and SINR	Huawei, HiSilicon

R1-1809211	Why PAPR is important for BS transmitters	Ericsson

R1-1809212	On PAPR issue for CSI-RS	Ericsson

R1-1809213	On PAPR issue for DMRS	Ericsson

R1-1809214	On signalling reduction for beam management	Ericsson

R1-1809333	General considerations on UE power saving in Rel-16	Huawei, HiSilicon

R1-1809348	Considerations on NR positioning	Huawei, HiSilicon

R1-1809415	On fast SRS precoder updation	CEWiT

R1-1809462	Views on UE power saving	Qualcomm Incorporated

R1-1809470	On BWP configuration for efficient spectrum utilization	CEWiT

## Other

R1-1808278	Modelling power consumption in NR modems	MediaTek Inc.

R1-1808279	UE power consumption adaptation based on traffic type	MediaTek Inc.

R1-1808284	View on NR positioning	MediaTek Inc.

R1-1809416	Link and system level performance evaluation for NR-NTN	THALES

# Closing of the meeting

RAN1 chair commented that the number of CRs at RAN1#94 was higher than he expected, and thanked all the group for the efforts improving the quality of RAN1 specs.

Thanks to Vice-chairmen, including Kazuaki-san and Xiaoyi for chairing the parallel sessions, MCC for support and EF3 for hosting facilities.

Meeting was closed at 17:07.

See you all in Chengdu.

# Annex A:	List of Tdocs at RAN1 #94

Please see excel file attached to this report

# Annex B:	List of CRs agreed at RAN1 #94

| TS/TR | CR | rev | Rel | Title | Cat | Vsn | @ Mtg | TD# | Source to WG | Work Item |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36.211 | 451 | - | Rel-15 | Introduction of eV2X | B | 15.2.0 | R1#94 | R1-1808328 | Ericsson | LTE_eV2X-Core |
| 36.211 | 452 | - | Rel-14 | Correction for interference randomization for NB-IoT | F | 14.7.0 | R1#94 | R1-1809535 | Qualcomm Incorporated | NB_IOTenh-Core |
| 36.211 | 453 | - | Rel-15 | Correction for interference randomization for NB-IoT | A | 15.2.0 | R1#94 | R1-1809536 | Qualcomm Incorporated | NB_IOTenh-Core |
| 36.211 | 454 | - | Rel-14 | Correction to NPRS mapping | F | 14.7.0 | R1#94 | R1-1809605 | Huawei, HiSilicon, Qualcomm, Ericsson | NB_IOTenh-Core |
| 36.211 | 455 | - | Rel-15 | Correction to NPRS mapping | A | 15.2.0 | R1#94 | R1-1809606 | Huawei, HiSilicon, Qualcomm, Ericsson | NB_IOTenh-Core |
| 36.211 | 456 | - | Rel-15 | Correction on DL CP extension for FeLAA in 36.211 | F | 15.2.0 | R1#94 | R1-1809650 | Huawei, HiSilicon | LTE_unlic-Core |
| 36.211 | 457 | - | Rel-15 | Correction to determination of ending symbol for AUL transmissions | F | 15.2.0 | R1#94 | R1-1809651 | Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon | LTE_unlic-Core |
| 36.211 | 458 |  | Rel-14 | Rate matching for aperiodic CSI-RS | F | 14.7.0 | R1#94 | R1-1810015 | Qualcomm Incorporated | LTE_eFDMIMO-Core |
| 36.211 | 459 |  | Rel-15 | Rate matching for aperiodic CSI-RS | A | 15.2.0 | R1#94 | R1-1810016 | Qualcomm Incorporated | LTE_eFDMIMO-Core |
| 36.211 | 460 | - | Rel-15 | Corrections to sTTI and SPT implementation | F | 15.2.0 | R1#94 | R1-1810023 | Ericsson | LTE_sTTIandPT-Core |
| 36.211 | 461 | - | Rel-15 | Corrections to eMTC | F | 15.2.0 | R1#94 | R1-1810030 | Ericsson | LTE_eMTC4-Core |
| 36.211 | 462 | - | Rel-15 | Corrections to NB-IoT | F | 15.2.0 | R1#94 | R1-1810035 | Ericsson | NB_IOTenh2-Core |
| 36.211 | 463 | - | Rel-15 | Corrections to LTE HRLLC implementation | F | 15.2.0 | R1#94 | R1-1810040 | Ericsson | LTE_HRLLC-Core |
| 36.212 | 297 | 1 | Rel-15 | Correction on the interpretation of HARQ-ACK bitmap for FeLAA in 36.212 | F | 15.2.1 | R1#94 | R1-1809577 | Huawei, HiSilicon | LTE_unlic-Core |
| 36.212 | 298 | - | Rel-14 | Correction on DCI for HARQ-ACK bundling | F | 14.6.0 | R1#94 | R1-1809540 | Qualcomm Incorporated | LTE_feMTC-Core |
| 36.212 | 299 | - | Rel-15 | Correction on DCI for HARQ-ACK bundling | A | 15.2.1 | R1#94 | R1-1809541 | Qualcomm Incorporated | LTE_feMTC-Core |
| 36.212 | 300 | - | Rel-15 | Correction on the partial PUSCH mode field for FeLAA in 36.212 | F | 15.2.1 | R1#94 | 1-1809576 | Huawei, HiSilicon | LTE_unlic-Core |
| 36.212 | 301 | - | Rel-15 | Correction of RRC parameter name for 1024QAM rate matching | F | 15.2.1 | R1#94 | R1-1809588 | Qualcomm Incorporated | LTE_1024QAM_DL-Core |
| 36.212 | 302 | - | Rel-14 | Corrections to modulation signalling in MUST | F | 14.6.0 | R1#94 | R1-1809636 | Intel Corporation | LTE_MUST-Core |
| 36.212 | 303 | - | Rel-15 | Corrections to modulation signalling in MUST | A | 15.2.1 | R1#94 | R1-1809637 | Intel Corporation | LTE_MUST-Core |
| 36.212 | 304 | - | Rel-15 | Corrections on shortened processing time and shortened TTI in 36.212 | F | 15.2.1 | R1#94 | R1-1810024 | Huawei | LTE_sTTIandPT-Core |
| 36.212 | 305 | - | Rel-15 | Corrections to Even further enhanced MTC for LTE | F | 15.2.1 | R1#94 | R1-1810031 | Huawei | LTE_eMTC4-Core |
| 36.212 | 306 | - | Rel-15 | Corrections to Further Enhancements of NB-IoT | F | 15.2.1 | R1#94 | R1-1810036 | Huawei | NB_IOTenh2-Core |
| 36.212 | 307 | - | Rel-15 | Corrections on HRLLC in 36.212 | D | 15.2.1 | R1#94 | R1-1810041 | Huawei | LTE_HRLLC-Core |
| 36.213 | 1118 | - | Rel-15 | Correction on CRI definition in 36.213 | F | 15.2.0 | R1#94 | R1-1808141 | Huawei, HiSilicon | feCOMP_LTE-Core |
| 36.213 | 1120 | - | Rel-13 | Correction on PUCCH format 4 HARQ-ACK procedure without adaptive codebook | F | 13.10.0 | R1#94 | R1-1808594 | Huawei, HiSilicon | LTE_CA_enh_b5C-Core |
| 36.213 | 1121 |  | Rel-13 | PRG for eMTC | F | 13.10.0 | R1#94 | R1-1809537 | Qualcomm Incorporated | LTE_MTCe2_L1-Core |
| 36.213 | 1122 |  | Rel-14 | PRG for eMTC | A | 14.7.0 | R1#94 | R1-1809538 | Qualcomm Incorporated | LTE_MTCe2_L1-Core |
| 36.213 | 1123 |  | Rel-15 | PRG for eMTC | A | 15.2.0 | R1#94 | R1-1809539 | Qualcomm Incorporated | LTE_MTCe2_L1-Core |
| 36.213 | 1124 | - | Rel-13 | Correction on TDD HARQ-ACK reporting procedure for different UL/DL configurations | F | 13.10.0 | R1#94 | R1-1809542 | Huawei, HiSilicon | LTE_CA_enh_b5C-Core |
| 36.213 | 1125 | - | Rel-14 | Correction on TDD HARQ-ACK reporting procedure for different UL/DL configurations | A | 14.7.0 | R1#94 | R1-1809543 | Huawei, HiSilicon | LTE_CA_enh_b5C-Core |
| 36.213 | 1126 | - | Rel-15 | Correction on TDD HARQ-ACK reporting procedure for different UL/DL configurations | A | 15.2.0 | R1#94 | R1-1809544 | Huawei, HiSilicon | LTE_CA_enh_b5C-Core |
| 36.213 | 1127 | - | Rel-14 | Correction on PUCCH format 4 HARQ-ACK procedure without adaptive codebook | A | 14.7.0 | R1#94 | R1-1809545 | Huawei, HiSilicon | LTE_CA_enh_b5C-Core |
| 36.213 | 1128 | - | Rel-15 | Correction on PUCCH format 4 HARQ-ACK procedure without adaptive codebook | A | 15.2.0 | R1#94 | R1-1809546 | Huawei, HiSilicon | LTE_CA_enh_b5C-Core |
| 36.213 | 1129 | - | Rel-13 | Correction to PDSCH RE mapping in TM10 | F | 13.10.0 | R1#94 | R1-1809547 | Intel Corporation | LTE_EBF_FDMIMO-Core |
| 36.213 | 1130 | - | Rel-14 | Correction to PDSCH RE mapping in TM10 | A | 14.7.0 | R1#94 | R1-1809548 | Intel Corporation | LTE_EBF_FDMIMO-Core |
| 36.213 | 1131 | - | Rel-15 | Correction to PDSCH RE mapping in TM10 | A | 15.2.0 | R1#94 | R1-1809549 | Intel Corporation | LTE_EBF_FDMIMO-Core |
| 36.213 | 1132 |  | Rel-14 | Correction on advanced CSI codebook | F | 14.7.0 | R1#94 | R1-1809550 | Qualcomm Incorporated | LTE_eFDMIMO-Core |
| 36.213 | 1133 |  | Rel-15 | Correction on advanced CSI codebook | A | 15.2.0 | R1#94 | R1-1809551 | Qualcomm Incorporated | LTE_eFDMIMO-Core |
| 36.213 | 1136 | - | Rel-14 | Corrections related to capabilities for SRS Carrier Switching | F | 14.7.0 | R1#94 | R1-1809554 | Qualcomm Incorporated | LTE_SRS_switch-Core |
| 36.213 | 1137 | - | Rel-15 | Corrections related to capabilities for SRS Carrier Switching | A | 15.2.0 | R1#94 | R1-1809555 | Qualcomm Incorporated | LTE_SRS_switch-Core |
| 36.213 | 1138 | - | Rel-14 | Capability for FeMBMS | F | 14.7.0 | R1#94 | R1-1809556 | Qualcomm Incorporated | MBMS_LTE_enh2-Core |
| 36.213 | 1139 | - | Rel-15 | Capability for FeMBMS | A | 15.2.0 | R1#94 | R1-1809557 | Qualcomm Incorporated | MBMS_LTE_enh2-Core |
| 36.213 | 1140 | - | Rel-14 | SRS transmission in UpPTS for special subframe configuration 10 | F | 14.7.0 | R1#94 | R1-1809558 | Qualcomm Incorporated, Huawei, HiSilicon | LTE_UL_CAP_enh-Core |
| 36.213 | 1141 | - | Rel-15 | SRS transmission in UpPTS for special subframe configuration 10 | A | 15.2.0 | R1#94 | R1-1809559 | Qualcomm Incorporated, Huawei, HiSilicon | LTE_UL_CAP_enh-Core |
| 36.213 | 1142 | - | Rel-14 | HARQ-ACK payload size determination for crs-less special subframe configuration 10 | F | 14.7.0 | R1#94 | R1-1809560 | Qualcomm Incorporated | LTE_UL_CAP_enh-Core |
| 36.213 | 1143 | - | Rel-15 | HARQ-ACK payload size determination for crs-less special subframe configuration 10 | A | 15.2.0 | R1#94 | R1-1809561 | Qualcomm Incorporated | LTE_UL_CAP_enh-Core |
| 36.213 | 1144 | - | Rel-15 | Correction on aperiodic CSI triggering for feCoMP in 36.213 | F | 15.2.0 | R1#94 | R1-1809586 | Huawei, HiSilicon | feCOMP_LTE-Core |
| 36.213 | 1145 | 1 | Rel-15 | Corrections related to modulation enhancements | F | 15.2.0 | R1#94 | R1-1809608 | Qualcomm Incorporated | TEI15 |
| 36.213 | 1146 | - | Rel-15 | Applicability of 1024QAM to SPS C-RNTI | F | 15.2.0 | R1#94 | R1-1809589 | Qualcomm Incorporated | LTE_1024QAM_DL-Core |
| 36.213 | 1147 | - | Rel-15 | Removal of R_CSI from 1024QAM CQI table | F | 15.2.0 | R1#94 | R1-1809590 | Qualcomm Incorporated | LTE_1024QAM_DL-Core |
| 36.213 | 1148 | - | Rel-14 | Clarification for antenna selection | F | 14.7.0 | R1#94 | R1-1809638 | Qualcomm incorporated | LTE-Phys, TEI14 |
| 36.213 | 1149 | - | Rel-15 | Clarification for antenna selection | A | 15.2.0 | R1#94 | R1-1809639 | Qualcomm incorporated | LTE-Phys, TEI14 |
| 36.213 | 1150 | - | Rel-12 | Change request on UE transmit antenna selection | F | 12.12.0 | R1#94 | R1-1809640 | CMCC | LTE_UL_MIMO-Core, TEI12 |
| 36.213 | 1151 | - | Rel-13 | Change request on UE transmit antenna selection | A | 13.10.0 | R1#94 | R1-1809641 | CMCC | LTE_UL_MIMO-Core, TEI12 |
| 36.213 | 1152 | - | Rel-14 | Change request on UE transmit antenna selection | A | 14.7.0 | R1#94 | R1-1809642 | CMCC | LTE_UL_MIMO-Core, TEI12 |
| 36.213 | 1153 | - | Rel-15 | Change request on UE transmit antenna selection | A | 15.2.0 | R1#94 | R1-1809643 | CMCC | LTE_UL_MIMO-Core, TEI12 |
| 36.213 | 1154 | - | Rel-15 | Change request on 1T2R antenna switching of SRS | B | 15.2.0 | R1#94 | R1-1809644 | CMCC | TEI15 |
| 36.213 | 1155 | - | Rel-15 | Correction to AUL-UCI resource allocation in 36.213 | F | 15.2.0 | R1#94 | R1-1809652 | Nokia, Nokia Shanghai Bell | LTE_unlic-Core |
| 36.213 | 1156 | - | Rel-15 | Correction for HARQ-ACK delay in eMTC | F | 14.7.0 | R1#94 | R1-1809653 | Qualcomm Incorporated, Ericsson | LTE_feMTC-Core |
| 36.213 | 1157 | - | Rel-15 | Correction for HARQ-ACK delay in eMTC | A | 15.2.0 | R1#94 | R1-1809654 | Qualcomm Incorporated, Ericsson | LTE_feMTC-Core |
| 36.213 | 1158 | - | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s00-s05 | F | 15.2.0 | R1#94 | R1-1810025 | Motorola Mobility | LTE_sTTIandPT-Core |
| 36.213 | 1159 | - | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s06-s07 | F | 15.2.0 | R1#94 | R1-1810026 | Motorola Mobility | LTE_sTTIandPT-Core |
| 36.213 | 1160 | - | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s08-s09 | F | 15.2.0 | R1#94 | R1-1810027 | Motorola Mobility | LTE_sTTIandPT-Core |
| 36.213 | 1161 | - | Rel-15 | Corrections for shortened processing time and shortened TTI in 36.213, s10-s13 | F | 15.2.0 | R1#94 | R1-1810028 | Motorola Mobility | LTE_sTTIandPT-Core |
| 36.213 | 1162 | - | Rel-15 | Correction to V2X Phase 2 based on LTE in 36.213, s14 | F | 15.2.0 | R1#94 | R1-1810029 | Motorola Mobility | LTE_eV2X-Core |
| 36.213 | 1163 | - | Rel-15 | Corrections to even further enhanced MTC for LTE in 36.213, s00-s05 | F | 15.2.0 | R1#94 | R1-1810032 | Motorola Mobility | LTE_eMTC4-Core |
| 36.213 | 1164 | - | Rel-15 | Corrections to even further enhanced MTC for LTE in 36.213, s06-s07 | F | 15.2.0 | R1#94 | R1-1810033 | Motorola Mobility | LTE_eMTC4-Core |
| 36.213 | 1165 | - | Rel-15 | Corrections to even further enhanced MTC for LTE in 36.213, s08-s09 | F | 15.2.0 | R1#94 | R1-1810034 | Motorola Mobility | LTE_eMTC4-Core |
| 36.213 | 1166 | - | Rel-15 | Editorial corrections to Rel-15 Further NB-IoT enhancements in 36.213, s2 | D | 15.2.0 | R1#94 | R1-1810037 | Motorola Mobility | NB_IOTenh2-Core |
| 36.213 | 1167 | - | Rel-15 | Corrections to Rel-15 Further NB-IoT enhancements in 36.213, s14 | F | 15.2.0 | R1#94 | R1-1810038 | Motorola Mobility | NB_IOTenh2-Core |
| 36.213 | 1168 | - | Rel-15 | Corrections to HRLLC in 36.213, s00-s05 | F | 15.2.0 | R1#94 | R1-1810042 | Motorola Mobility | LTE_HRLLC-Core |
| 36.213 | 1169 | - | Rel-15 | Corrections to HRLLC in 36.213, s06-s07 | F | 15.2.0 | R1#94 | R1-1810043 | Motorola Mobility | LTE_HRLLC-Core |
| 36.213 | 1170 | - | Rel-15 | Corrections to HRLLC in 36.213, s08-s09 | F | 15.2.0 | R1#94 | R1-1810044 | Motorola Mobility | LTE_HRLLC-Core |
| 36.213 | 1171 | - | Rel-15 | Corrections to even further enhanced MTC for LTE in 36.213, s14-sxx | F | 15.2.0 | R1#94 | R1-1810045 | Motorola Mobility | LTE_eMTC4-Core |
| 36.214 | 51 | - | Rel-15 | Introduction of NR-SS-SINR | B | 15.2.0 | R1#94 | R1-1809249 | Ericsson | NR_newRAT-Core |
| 36.214 | 52 | - | Rel-15 | Introduction of NPBCH based NRSRP measurement | F | 15.2.0 | R1#94 | R1-1810039 | Ericsson | NB_IOTenh2-Core |
| 37.213 | 1 | 1 | Rel-15 | Correction on RRC parameters for FeLAA in 37.213 | F | 15.0.0 | R1#94 | R1-1809622 | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson | LTE_unlic-Core |
| 37.213 | 2 | - | Rel-15 | Correction on starting position of Partial PUSCH Mode 1 for FeLAA in 37.213 | F | 15.0.0 | R1#94 | R1-1809647 | Huawei, HiSilicon, Ericsson | LTE_unlic-Core |
| 37.213 | 3 | - | Rel-15 | Correction on COT length for AUL transmission | F | 15.0.0 | R1#94 | R1-1809648 | LG Electronics | LTE_unlic-Core |
| 37.885 | 1 | 2 | Rel-15 | Correction of optional antenna element patterns for vehicular UEs | F | 15.0.0 | R1#94 | R1-1809924 | Fraunhofer HHI, Fraunhofer IIS, General Motors, Nokia, Nokia Shanghai Bell, LG Electronics | FS_LTE_NR_V2X_eval |
| 37.885 | 2 | 1 | Rel-15 | Correction and confirmation of some evaluation assumption parameters | F | 15.0.0 | R1#94 | R1-1810007 | LG Electronics | FS_LTE_NR_V2X_eval |
| 38.202 | 4 | - | Rel-15 | CR to 38.202 capturing the RAN1#94 meeting agreements | F | 15.2.0 | R1#94 | R1-1810017 | Qualcomm | NR_newRAT-Core |
| 38.211 | 3 | - | Rel-15 | CR to 38.211 capturing the RAN1#94 meeting agreements | F | 15.2.0 | R1#94 | R1-1810018 | Ericsson | NR_newRAT-Core |
| 38.212 | 4 | - | Rel-15 | CR to 38.212 capturing the RAN1#94 meeting agreements | F | 15.2.0 | R1#94 | R1-1810019 | Huawei | NR_newRAT-Core |
| 38.213 | 5 | - | Rel-15 | CR to 38.213 capturing the RAN1#94 meeting agreements | F | 15.2.0 | R1#94 | R1-1810020 | Samsung | NR_newRAT-Core |
| 38.214 | 5 | - | Rel-15 | CR to 38.214 capturing the RAN1#94 meeting agreements | F | 15.2.0 | R1#94 | R1-1810021 | Nokia | NR_newRAT-Core |
| 38.215 | 4 | - | Rel-15 | CR to 38.215 capturing the RAN1#94 meeting agreements | F | 15.2.0 | R1#94 | R1-1810022 | Intel | NR_newRAT-Core |

# Annex C-1:	List of Outgoing LSs from RAN1 #94

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-1809609 | LS on Restriction of 1024QAM CSI reporting | RAN1, Nokia, Nokia Shanghai Bell | Rel-15 | LTE_1024QAM_DL-Core, LTE_sTTIandPT-Core |  | RAN2 |  |  |  |
| R1-1809617 | LS on UE feature list | RAN1, NTT DOCOMO, INC | Rel-15 | TEI15 |  | RAN2, RAN4 | RAN3 |  |  |
| R1-1809679 | LS on additional RRC parameters for Rel-15 NB-IoT | RAN1, Huawei, HiSilicon | Rel-15 | NB_IOTenh2-Core |  | RAN2 |  |  |  |
| R1-1809681 | LS on UL PRB to DL PRB center offset for TDD NB-IoT | RAN1, Ericsson | Rel-15 | NB_IOTenh2-Core |  | RAN4 | RAN2 |  |  |
| R1-1809687 | LS on sTTI | RAN1, Ericsson | Rel-15 | LTE_sTTIandPT-Core |  | RAN2 |  |  |  |
| R1-1809689 | LS on RRC parameter for NPBCH based measurements | RAN1, Qualcomm | Rel-15 | NB_IOTenh2-Core |  | RAN2 |  |  |  |
| R1-1809691 | LS on HRLLC | RAN1, Ericsson | Rel-15 | LTE_HRLLC-Core |  | RAN2 |  |  |  |
| R1-1809760 | Reply LS on short message transmission using DCI | RAN1, Huawei | Rel-15 | NR_newRAT-Core | R2-1810937 | RAN2 |  |  |  |
| R1-1809774 | LS on search space configuration for DCI format 2_0 | RAN1, Qualcomm | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1809810 | Reply LS on active DL BWPs overlapping/non-overlapping with initial DL BWP | RAN1, MediaTek | Rel-15 | NR_newRAT-Core | R2-1810968 | RAN2 |  |  |  |
| R1-1809812 | Response LS on additional TDD configuration periodicities | RAN1, Nokia, Nokia Shanghai Bell, SPRINT Corporation | Rel-15 | NR_newRAT-Core | R2-1810963 | RAN2 | RAN4 |  |  |
| R1-1809850 | LS response on overlapping SIB1 and SI messages | RAN1, Samsung | Rel-15 | NR_newRAT-Core | R2-1810936 | RAN2 |  |  |  |
| R1-1809853 | LS on second BLER pair for RLM | RAN1, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN4 |  |  |  |
| R1-1809886 | LS on power control for NR(FR1) - NR(FR2) dual connectivity | RAN1, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN4 |  |  |  |
| R1-1809887 | Reply LS on termination of contention-free BFR | RAN1, MediaTek | Rel-15 | NR_newRAT-Core | R2-1810807 | RAN2 |  |  |  |
| R1-1809888 | LS on maximum number of TCI states | RAN1, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1809889 | Reply LS on UE power class and UE capability on SRS carrier switch | RAN1, Huawei | Rel-15 | NR_newRAT-Core | R2-1809180 | RAN2, RAN4 |  |  |  |
| R1-1809890 | LS on UE behaviour on reception of channels or RS in the same OFDM symbol | RAN1, Intel | Rel-15 | NR_newRAT-Core |  | RAN4 |  |  |  |
| R1-1809907 | LS on support of unicast, groupcast, and broadcast | RAN1, LG Electronics | Rel-16 | FS_NR_V2X |  | RAN2, SA2 |  |  |  |
| R1-1809958 | Reply LS on ssb-PositionsInBurst mismatch in SIB1 and ServingcellConfigCommon | RAN1, Ericsson | Rel-15 | NR_newRAT-Core | R2-1810976 | RAN2 |  |  |  |
| R1-1809960 | LS on density parameter for CSI-RS-ResourceConfigMobility | RAN1, Intel | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1809965 | Reply LS on Further Agreements on Paging Occasions | RAN1, Huawei | Rel-15 | NR_newRAT-Core | R2-1810954 | RAN2 |  |  |  |
| R1-1809966 | LS on DCI format 2_3 | RAN1, NTT DOCOMO | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1809971 | Reply LS on PDCCH common search space configurations | RAN1, NTT DOCOMO | Rel-15 | NR_newRAT-Core | R2-1808992 | RAN2 |  |  |  |
| R1-1809982 | LS on K1 Slot Offset | RAN1, Qualcomm | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1809987 | LS on NR-RIM frameworks | RAN1, CMCC | Rel-16 | FS_NR_RIM |  | RAN3 |  |  |  |
| R1-1809991 | LS on Text Proposal to 38.300 for URLLC | RAN1, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1809992 | LS on NR-LTE coexistence | RAN1, Intel, LG Electronics | Rel-15 | NR_newRAT-Core |  | RAN2, RAN4 |  |  |  |
| R1-1809993 | LS on the number of CQI reports | RAN1, Intel | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1810002 | LS on bandwidth configuration for initial BWP | RAN1, MediaTek | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1810004 | LS on UE capability for simultaneousTxSUL-NonSUL | RAN1, Huawei | Rel-15 | NR_newRAT-Core |  | RAN2 | RAN4 |  |  |
| R1-1810005 | LS on new UE features list for NR | RAN1, NTT DOCOMO, AT&T | Rel-15 | NR_newRAT-Core |  | RAN, RAN2, RAN4 |  |  |  |
| R1-1810006 | LS on IBE model for V2X | RAN1, Ericsson | Rel-16 | FS_NR_V2X |  | RAN4 |  |  |  |
| R1-1810008 | LS on collision of RRM measurement resources with uplink transmissions in FR1 TDD | RAN1, vivo | Rel-15 | NR_newRAT-Core |  | RAN4 |  |  |  |
| R1-1810009 | LS on cross-carrier scheduling | RAN1, NTT DOCOMO | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |
| R1-1810010 | LS on higher layer configuration for search space monitoring | RAN1, NTT DOCOMO | Rel-15 | NR_newRAT-Core |  | RAN2 |  |  |  |

# Annex C-2:	List of Incoming LSs from RAN1 #94

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-1808003 | LS on the feasibility of 1 symbol GP for 60KHz in FR1 and 120KHz in FR2 | RAN4, MediaTek | Rel-15 | NR_newRAT-Core |  | RAN1 | RAN2 | R4-1806541 |  |
| R1-1808004 | LS on RMSI CORESET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth | RAN4, CATT, Samsung | Rel-15 | NR_newRAT-Core |  | RAN1 | RAN2 | R4-1808263 |  |
| R1-1808005 | LS response on NB-IoT downlink channel quality determination and report | RAN4, Ericsson | Rel-14 | NB_IOT-Core, TEI14 | R1-1803163 | RAN1, RAN2 |  | R4-1807937 |  |
| R1-1808006 | Reply LS on updates for TS 36.300 | RAN3, Huawei | Rel-15 | NB_IOTenh2-Core, LTE_eMTC4-Core | R2-1808848 | RAN2 | RAN1 | R3-183572 |  |
| R1-1808007 | LS on NR Cell Information signalled over X2 for EN-DC | RAN3, Ericsson | Rel-15 | 5GS_Ph1 |  | RAN1, RAN2, RAN4 |  | R3-183569 |  |
| R1-1808008 | Additional Reply LS on eV2X evaluation methodology | SAE C-V2X TC Advanced Applications Task-Force, AT&T | Rel-15 | FS_LTE_NR_V2X_eval | R1-1719239 | RAN, RAN1, SA1 |  | SAE C-V2X AA TF additional Reply LS 05282018 |  |
| R1-1808009 | LS reply on UL timing difference in EN-DC | RAN4, Intel Corporation | Rel-15 | NR_newRAT-Core | R2-1804102 | RAN2 | RAN1 | R4-1806315 |  |
| R1-1808011 | Reply LS on Narrowband measurement accuracy improvements | RAN4, Huawei | Rel-15 | NB_IOTenh2-Core | R1-1805434 | RAN1 | RAN2 | R4-1807328 |  |
| R1-1808013 | LS on eFeMTC CRS muting | RAN4, Huawei | Rel-15 | LTE_eMTC4-Core |  | RAN2 | RAN1 | R4-1808032 |  |
| R1-1808014 | LS on 2RX requirements for vehicle mounted NR UE | RAN4, LG Electronics | Rel-15 | NR_newRAT-Core |  | RAN | RAN1 | R4-1808188 |  |
| R1-1808015 | RAN4 response LS on Coexistence of NR-NR and NR-LTE networks | RAN4, Qualcomm Incorporated | Rel-15 | NR_newRAT-Core | R4-1806116 | RAN | RAN1 | R4-1808253 |  |
| R1-1808017 | LS on UE feature list | RAN4, NTT DOCOMO, Intel | Rel-15 | NR_newRAT-Core |  | RAN1, RAN2 | RAN3 | R4-1808535 |  |
| R1-1808018 | LS reply on Coexistence of NR-NR and NR-LTE networks with synchronised, unsynchronised and semi-synchronised operation in 3400-3800 MHz | RAN, Huawei | Rel-15 | NR_newRAT | CEPT_ECC_PT1_LS_180420 | CEPT ECC PT1 | RAN1, RAN4 | RP-181389 |  |
| R1-1808019 | LS reply on minimum number of RX antennae for NR vehicular Ues | RAN, Samsung (Harman) | Rel-15 | NR_newRAT | 5GAA_A-180148 | 5GAA, 5GAA WG2 | RAN1, RAN4 | RP-181468 |  |
| R1-1808020 | LS on NR UE feature list | RAN, NTT DOCOMO | Rel-15 | NR_newRAT-Core | R1-1807896, R2-1809153, R4-1808552 | RAN1, RAN2, RAN4 | RAN3 | RP-181484 |  |
| R1-1808021 | LS on L2 measurement specification for NR in R15 | SA5, Ericsson | Rel-15 | NETSLICE-ADPM5G |  | RAN2, RAN3 | RAN1 | S5-184337 |  |
| R1-1808022 | Wi-Fi Alliance Coexistence Test Plan for LTE-LAA Version 1.1.10 | Wi-Fi Alliance | Rel-13 | LTE_LAA | RP-172843 | RAN, RAN4, RAN1 | IEEE 802.11 WG | 20180615 Wi-Fi Alliance Coexistence Test Plan for LTE-LAA v1.1.10 |  |
| R1-1808023 | Reply LS on PSCCH/PSSCH subframe numbering issue in partial network coverage | RAN2, Ericsson | Rel-14 | LTE_V2X-Core | R1-1803078 | RAN1 |  | R2-1808901 |  |
| R1-1808024 | Reply LS on narrowband measurement | RAN2, Huawei | Rel-15 | NB_IOTenh2-Core | R4-1805950 | RAN4, RAN1 |  | R2-1809096 |  |
| R1-1808025 | Reply LS on RMSI CORESET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth | RAN2, CATT | Rel-15 | NR_newRAT-Core | R1-1805712 | RAN1 | RAN4 | R2-1809134 |  |
| R1-1808026 | Reply LS on PC5 transmission mechanism selection | RAN2, LG Electronics | Rel-15 | LTE_eV2X-Core | S2-183968 | SA2 | RAN1 | R2-1809166 |  |
| R1-1808027 | Reply LS to RAN1 on RRC parameters for V2X Phase 2 | RAN2, LG Electronics | Rel-15 | LTE_eV2X-Core | R1-1805620 | RAN1 | SA2 | R2-1809167 |  |
| R1-1808028 | Reply LS on Agreements on Paging | RAN2, Qualcomm | Rel-15 | NR_newRAT-Core | R1-1801280 | RAN1 |  | R2-1809174 |  |
| R1-1808029 | Reply LS on UE power class and UE capability on SRS carrier switch | RAN2, NTT DOCOMO | Rel-15 | NR_newRAT-Core | R4-1805929, R4-1805660 | RAN4, RAN1 |  | R2-1809180 |  |
| R1-1808030 | LS on RAN2 progress on ANR | RAN2, vivo | Rel-15 | NR_newRAT-Core |  | RAN3, RAN4 | RAN1, SA5 | R2-1809226 |  |
| R1-1808031 | Reply LS on CP latency reduction | RAN2, Ericsson | Rel-15 | TEI15 | RP-172840 | RAN | RAN1 | R2-1809244 |  |
| R1-1808032 | LS on final RAN2 agreements for euCA | RAN2, Nokia | Rel-15 | LTE_euCA-Core |  | RAN1, RAN4 |  | R2-1809271 |  |
| R1-1808166 | LS on termination of contention-free BFR | RAN2, Intel | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1810807 |  |
| R1-1808167 | Reply LS on numberOfRA-PreamblesGroupA | RAN2, Nokia | Rel-15 | NR_newRAT-Core | R1-1807764 | RAN1 |  | R2-1810894 |  |
| R1-1808168 | LS on overlapping SIB1 and SI messages | RAN2, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1810936 |  |
| R1-1808169 | LS on short message transmission using DCI | RAN2, Ericsson | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1810937 |  |
| R1-1808170 | Reply LS on NR Cell Information signalled over X2 for EN-DC | RAN2, Ericsson | Rel-15 | NR_newRAT-Core | R3-183569 | RAN3 | RAN1, RAN4 | R2-1810942 |  |
| R1-1808171 | Reply LS on Bandwidth configuration for initial BWP | RAN2, NTT DOCOMO | Rel-15 | NR_newRAT-Core | R1-1807873 | RAN1 | RAN4 | R2-1810943 |  |
| R1-1808172 | LS on Further Agreements on Paging Occasions | RAN2, Qualcomm | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1810954 |  |
| R1-1808173 | LS on additional TDD configuration periodicities | RAN2, Nokia | Rel-15 | NR_newRAT-Core |  | RAN1 | RAN4 | R2-1810963 |  |
| R1-1808174 | LS on Active DL BWPs overlapping/non-overlapping with Initial DL BWP | RAN2, ZTE | Rel-15 | NR_newRAT-Core | R1-1807893 | RAN1 | RAN4 | R2-1810968 |  |
| R1-1808175 | LS on RA preambles mapping to SSBs | RAN2, ZTE | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1810969 |  |
| R1-1808176 | LS on clarification of useServingCellTimingForSync | RAN4, MediaTek | Rel-15 | NR_newRAT-Core |  | RAN1, RAN2 |  | R4-1809409 |  |
| R1-1808177 | LS on potential enhancement of DR-mode of operation | SA2, Intel | Rel-16 |  |  | RAN2 | RAN1, RAN4 | S2-187629 |  |
| R1-1808178 | LS on the transmission of video over PC5 interface | SA4, Samsung | Rel-15 | FS_mV2X |  | RAN2 | RAN1, SA1 | S4-180980 |  |
| R1-1808179 | LS on Joint ETSI - OSA Workshop: Open Implementations & Standardization | ETSI |  |  |  | RAN, RAN1, RAN2, RAN3, RAN4, RAN5, CT, CT1, CT3, CT4, CT6, SA, SA3, SA5, SA6 |  | ETSI_CL18_3463 |  |
| R1-1808180 | IEEE 802.11 WG LS to 3GPP RAN4 on certain channel combinations for LAA in 5GHz | IEEE 802.11 Working Group | Rel-13 | LTE_LAA |  | RAN4 | RAN1 | 2018-07-13-LS from IEEE 802.11 to 3GPP RAN4 |  |
| R1-1808181 | IEEE 802.11 WG LS to 3GPP RAN in relation to a proposed joint Coexistence Workshop | IEEE 802.11 Working Group | Rel-15 | FS_NR_unlic |  | RAN | RAN1 | 2018-07-13-LiS from IEEE 802.11 to 3GPP-coex-workshop |  |
| R1-1808303 | Liaison ESOA (EMEA Satellite Operators Association) on 5G for Satellite Systems | ESOA, Thales | Rel-16 |  |  | SA1, SA2, RAN1, RAN2, RAN3 | SA, RAN | ESOA-LS-2018-1 |  |
| R1-1808859 | LS on fallback interpretation of UE FeatureSetDown(Up)LinkPerCC capability fields | RAN2, Intel | Rel-15 | NR_newRAT-Core |  | RAN1, RAN4 |  | R2-1810975 |  |
| R1-1808860 | LS on ssb-PositionsInBurst mismatch in SIB1 and ServingcellConfigCommon | RAN2, vivo | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1810976 |  |
| R1-1809720 | LS on Prioritised Use Cases and Requirements for consideration in Rel-16 NR-V2X | 5GAA WG1 | Rel-16 | FS_NR_V2X |  | RAN1, RAN2, SA1 | 5GAA WG2, 5GAA WG4, RAN, RAN3, SA2 | 5GAA_T-180181 |  |
| R1-1809876 | LS on OSI acquisition | RAN2, Qualcomm | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1813326 |  |
| R1-1809892 | LS on default configuration for L1 parameter | RAN2, NTT DOCOMO | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1813319 |  |
| R1-1809942 | LS on UE capability for simultaneousTxSUL-NonSUL | RAN2, Huawei | Rel-15 | NR_newRAT-Core |  | RAN1 | RAN4 | R2-1813366 |  |
| R1-1809948 | LS response on NPBCH-based RRM measurement for FeNB-IoT UE | RAN4, Qualcomm | Rel-15 | NB_IOTenh2-Core |  | RAN1, RAN2 |  |  |  |
| R1-1810001 | Reply LS on SRS carrier switching | RAN4, Huawei | Rel-15 | NR_newRAT-Core |  | RAN1, RAN2 |  |  |  |

# Annex D:	List of Approved updated WIDs

None

# Annex E:	List of draft TSs/TRs agreed at RAN1 #94

| Tdoc Number | Title | Source | Conclusion/Decision |
| --- | --- | --- | --- |
| R1-1808150 | Text proposal to capture agreements made in RAN1#93 to TR 38.812 (NOMA) | ZTE | Endorsed as v0.0.3 |
| R1-1808942 | TR for Study on NR V2X | Huawei, HiSilicon | Endorsed as v0.0.0 |
| R1-1809795 | TR 38.889 v0.0.3 Study on NR-based access to unlicensed spectrum | Qualcomm | Endorsed as v0.0.3 |
| R1-1809861 | Draft skeleton of TR 38.824: Study on physical layer enhancements for NR URLLC | Huawei, HiSilicon | Endorsed as v0.0.0 |
| R1-1809872 | Draft skeleton of TR 38.866 Study on remote interference management for NR | CMCC | Endorsed as v0.0.0 |
| R1-1810046 | TR 38.812 (NOMA) v0.1.0 | ZTE | Created by MCC as baseline for further updates - aiming at one step approval in RAN#82 (Dec. 2018) |
| R1-1810047 | TR 38.889 v0.1.0 Study on NR-based access to unlicensed spectrum | Qualcomm | Created by MCC as baseline for further updates - aiming at one step approval in RAN#82 (Dec. 2018) |

# Annex F:	List of actions

- Outgoing LS

- CR approval

[94-LTE-01-spec#] for 6.1.2	Maintenance of Release 15 Shortened TTI and processing time for LTE		LTE_sTTIandPT-Core

Done: According to the decision posted on Sept 5th, the following CRs are agreed:

R1-1810023	Corrections to sTTI and SPT implementation	Ericsson		36.211	15.2.0	CR0460, F

R1-1810024	Corrections on shortened processing time and shortened TTI in 36.212	Huawei	36.212	15.2.0	CR0304, F

R1-1810025	Corrections for shortened processing time and shortened TTI in 36.213, s00-s05	Motorola Mobility	36.213	15.2.0	CR1158, F

R1-1810026	Corrections for shortened processing time and shortened TTI in 36.213, s06-s07	Motorola Mobility	36.213	15.2.0	CR1159, F

R1-1810027	Corrections for shortened processing time and shortened TTI in 36.213, s08-s09	Motorola Mobility	36.213	15.2.0	CR1160, F

R1-1810028	Corrections for shortened processing time and shortened TTI in 36.213, s10-s13	Motorola Mobility	36.213	15.2.0	CR1161, F

[94-LTE-02-spec#] for 6.1.3	Maintenance of Release 15 3GPP V2X Phase 2	LTE_eV2X-Core

Done: According to the decision posted on Sept 5th, the following CR is agreed:

R1-1810029	Correction to V2X Phase 2 based on LTE in 36.213, s14	Motorola Mobility	36.213	15.2.0	CR1162, F

[94-LTE-03-spec#] for 6.1.4	Maintenance of Release 15 Even further enhanced MTC for LTE	LTE_eMTC4-Core

Done: According to the decision posted on Sept 5th, the following CRs are agreed:

R1-1810030	Corrections to eMTC	Ericsson		36.211	15.2.0	CR0461, F

R1-1810031	Corrections to Even further enhanced MTC for LTE	Huawei	36.212	15.2.0	CR0305, F

R1-1810032	Corrections to even further enhanced MTC for LTE in 36.213, s00-s05	Motorola Mobility	36.213	15.2.0	CR1163, D

R1-1810033	Corrections to even further enhanced MTC for LTE in 36.213, s06-s07	Motorola Mobility	36.213	15.2.0	CR1164, F

R1-1810034	Corrections to even further enhanced MTC for LTE in 36.213, s08-s09	Motorola Mobility	36.213	15.2.0	CR1165, F

R1-1810045	Corrections to even further enhanced MTC for LTE in 36.213, s14-sxx	Motorola Mobility	36.213	15.2.0	CR1171, F

[94-LTE-04-spec#] for 6.1.5	Maintenance of Release 15 Further enhancements of NB-IoT	NB_IOTenh2-Core

Done: According to the decision posted on Sept 5th, the following CRs are agreed:

R1-1810035	Corrections to NB-IoT	Ericsson		Rel-15	36.211	15.2.0	CR0462, F

R1-1810036	Corrections to Further Enhancements of NB-IoT	Huawei	Rel-15	36.212	15.2.0	CR0306, F

R1-1810037	Editorial corrections to Rel-15 Further NB-IoT enhancements in 36.213, s2	Motorola Mobility	Rel-15	36.213	15.2.0	CR1166, D

R1-1810038	Corrections to Rel-15 Further NB-IoT enhancements in 36.213, s14	Motorola Mobility	Rel-15	36.213	15.2.0	CR1167, F

R1-1810039	Introduction of NPBCH based NRSRP measurement	Ericsson		Rel-15	36.214	15.2.0	CR0052, F

[94-LTE-05-spec#] for 6.1.6	Maintenance of Release 15 Ultra Reliable Low Latency Communication for LTE	LTE_HRLLC-Core

Done: According to the decision posted on Sept 5th, the following CRs are agreed:

R1-1810040	Corrections to LTE HRLLC implementation	Ericsson		Rel-15	36.211	15.2.0	CR0463, F

R1-1810041	Corrections on HRLLC in 36.212	Huawei	Rel-15	36.212	15.2.0	CR0307, D

R1-1810042	Corrections to HRLLC in 36.213, s00-s05	Motorola Mobility	Rel-15	36.213	15.2.0	CR1168, F

R1-1810043	Corrections to HRLLC in 36.213, s06-s07	Motorola Mobility	Rel-15	36.213	15.2.0	CR1169, F

R1-1810044	Corrections to HRLLC in 36.213, s08-s09	Motorola Mobility	Rel-15	36.213	15.2.0	CR1170, F

[94-LTE-06] – Alberto (Qualcomm)

R1-1808997	Rate matching for A-CSI-RS	Qualcomm Incorporated

Email approval till August 31st

Done: According to the decision posted on Sept 1st, the following is agreed:

Agreements

- The draft CR in R1-1808997 is endorsed. Final CRs are agreed in R1-1810015 (36.211 CR0458, Rel-14, F) and R1-1810016 (36.211 CR0459, Rel-15, A).

- NOTE: For the scenario treated in R1-1808997, the UE may not perform puncturing (e.g. zero out LLRs) in REs that are not covered by the ZP CSI-RS in the downlink DCI but are indicated to be used for aperiodic NZP CSI-RS in the uplink DCI.

[94-NR-01-spec#]

Editors to prepare CRs incorporating the TPs/agreements/work assumptions by August 30th , to be commented and endorsed by Sept. 4th

Done: According to the decision posted on Sept 5th, the following CRs are agreed:

R1-1810017	CR to 38.202 capturing the RAN1#94 meeting agreements	Qualcomm	Rel-15	38.202	15.2.0	NR_newRAT-Core	CR0004, F

R1-1810018	CR to 38.211 capturing the RAN1#94 meeting agreements	Ericsson		Rel-15	38.211	15.2.0	NR_newRAT-Core	CR0003, F

R1-1810019	CR to 38.212 capturing the RAN1#94 meeting agreements	Huawei		Rel-15	38.212	15.2.0	NR_newRAT-Core	CR0004, F

R1-1810020	CR to 38.213 capturing the RAN1#94 meeting agreements	Samsung	Rel-15	38.213	15.2.0	NR_newRAT-Core	CR0005, F

R1-1810021	CR to 38.214 capturing the RAN1#94 meeting agreements	Nokia		Rel-15	38.214	15.2.0	NR_newRAT-Core	CR0005, F

R1-1810022	CR to 38.215 capturing the RAN1#94 meeting agreements	Intel		Rel-15	38.215	15.2.0	NR_newRAT-Core	CR0004, F

- Miscellaneous

[94-NR-02] – Li (ZTE)

NOMA: Companies to provide more detailed description of the proposed scheme (to the extent that solid simulations can be performed) by email by 8/28, and to be commented by 8/31. These schemes are to be captured in R1-1809844

Done: According to decision posted on Sept.6th, simulations can be carried out based on the proposed NOMA schemes in:

R1-1809844	Email discussion to collect detailed description of the NOMA schemes	ZTE

[94-NR-03] – Li (ZTE)

NOMA: Email discussion on SLS calibration and templates for collecting the SLS evaluation results, by 09/07

Done: According to decision posted on Sept.18th, the following is agreed:

Agreements:

- Adopt the calibration results in Section 2 (as in R1-1810048), to be captured in TR38.812.

  - Calibration results can be further updated as necessary.

- Adopt the companion spreadsheet (as in R1-1810048) as the template for the collection of SLS evaluation results.

Agreements:

- Companies are encouraged to report the PDF of multiplexed UEs per transmission duration (e.g. duration of a slot/mini-slot) at the PAR corresponding to the target system operation point, i.e.,

  - For mMTC and eMBB scenarios:

    - PDF of multiplexed UEs per transmission duration at the system PAR corresponding to the 1% system PDR.

  - For URLLC scenario:

    - PDF of multiplexed UEs per transmission duration at the per UE PAR corresponding to the 95% satisfied ratio.

  - FFS how to use the PDF for performance evaluations

  - Other evaluation methodologies for studying the practical number of NOMA UEs multiplexed in the same PRBs taking inter-cell interference and per UE performance into account are not precluded.

[94-NR-04] – Hanbyul (LGE)

V2X: RAN1 to have an email discussion until the next meeting to complement the existing simulation assumptions, using R1-1809950 and R1-1809944 (traffic model) at a starting point

Done: Progress to be checked in RAN1#94bis meeting.

[94-NR-05] – Hua (CMCC)

RIM: Email discussion on additional simulation assumption for RIM till 9/7

Done: According to decision posted on Sept.18th, a summary (R1-1810049) captures the following agreements:

Agreements:

- For channel model: Option 1 is mandatory for RS comparison. Companies are also encouraged to provide results using Option 2 with K_factor = 22 dB, DS=30 ns, Doppler 0 Hz.

- For delay of received RS: Clarify that the arrival time of each RS is generated at least in the resolution of sample duration of the OFDM symbol.

Agreement: For power of received RS, Option1(as in last RAN1 agreement): Pi of multiple RSs have a power offset with respect to the reference power P0, where the power offset is randomly selected from [-0.5dB, 0.5dB] is used as starting point for multiple RS evaluation. The SNR for each RS is defined based on P0.

Agreements:

For Case 1 single RS+AWGN, the metric is the minimum SNR required for one-shot detection with 90% detection probability and 1% false alarm requirement. The assumptions for Case 1 are summarized as follows:

|  | Total number of sequences used in the network (N_seq) | Number of sequences arriving within the window (n) | Number of total RSs arriving within the window (N) |
| --- | --- | --- | --- |
| Case 1 | 1 | 1 | 1 |

Note: This case is mainly used to compare and calibrate performance of same RS design. The above figures will be used for simulations to 94bis and determine whether further refinement is needed.

Agreements: At least Case 2-1 and Case 2-2A are evaluated for the next meeting. Evaluation of Case 2-2B depending on further agreements whether to support gNB group for Framework 2.1 and 2.2, and whether Case 2-1 and 2-2A is sufficient to cover Case 2-2B in terms of error detection, wherein,

- Case 2-1 (Single sequence): All RSs received within the detection window correspond to the same sequence. Number of total RS base sequence is only 1

- Case 2-2A (Multiple sequences): Each RS received by the detector corresponds to a different sequence.

- Case 2-2B (Multiple sequences): The number of distinctive sequences is smaller than the number of RSs received by the detector. Multiple RSs may correspond to the same sequence.

Agreements:

For Case 2-1, the number of RS arrived within the detection window is N=10 as starting point. Companies are also encouraged to provide results under more values of N. The assumptions for Case 2-1 are summarized as follows:

|  | Total number of sequences used in the network (N_seq) | Number of sequences arriving within the window (n) | Number of total RSs arriving within the window (N) |
| --- | --- | --- | --- |
| Case 2-1 | 1 | 1 | 10 as starting point |

Agreements: The assumptions for Case 2-2A and Case 2-2B are summarized as follows:

| Case 2-2A: Each RS received by the detector corresponds to a different sequence. |  |  |  |
| --- | --- | --- | --- |
|  | Number of sequences used in the network (N_seq) | Number of sequences within the detection window (n) | Number of total RSs arriving within one detection window (N*n) |
| Case 2-2A | 8 as starting point | 1,2,4,8 1 | n |
| Case 2-2B | 8 as starting point | 1,2,4,8 1 | 10*n as starting point |
| NOTE 1: Separate simulation runs NOTE 2: Evaluation of Case 2-2B depending on further agreements whether to support gNB group for Framework 2.1 and 2.2, and whether Case 2-1 and 2-2A is sufficient to cover Case 2-2B in terms of error detection. |  |  |  |

Agreements:

At least detection probability of option 1 and 2 will be evaluated for next meeting, wherein,

- Detection probability of option 1: Worst case detection probability of all sequences asmink Pd,k, where Pd,k = Prob{sequence i is detected in a detection window | sequence k is present the detection window}

- Detection probability of option 2: Average detection probability of all the sequences Pd,k

For the options of error detection probability or miss detection probability, companies are encouraged to provide evaluation results and report exact definition of the corresponding metric in the next meeting, so as to determine the final aligned metric(s).

Agreements:

- For single sequence case (Case 2-1), gNB attempts to detect only one sequence per detection window.

- For multiple sequence case (Case 2-2A and Case 2-2B) The RS detector should attempt to detect all possible RS sequences arriving in the detection window, where no advanced receiver algorithm is adopted for RS detection.

Agreement:

- False detection rate of RS is evaluated by only AWGN input to the receiver, i.e. modelling thermal noise, and should be no larger than 1% for all evaluation cases.

[94-NR-06] – Chengyan (Huawei)

URLLC: Email discussion on additional simuliation assumptions till 9/7 (including proposals 4/5/6 in R1-1809951)

Done: According to decision posted on Sept.20th, the following is agreed. Note that these agreements are by no means to limit further discussion. RAN1 will further discuss and try to complete evaluation assumptions in RAN1#94bis.

Agreements:

- Additional assumptions for evaluation:

  - Companies describe overhead modeling (e.g. PDCCH overhead) used by companies in the simulation

  - Companies describe modification to channel models if any

  - Companies describe power control mechanisms

- 1 ms air interface latency is assumed for evaluation for factory automation, with the assumption of 1 ms CN delay in 2 ms end-to-end latency.

  - Other values for evaluation are not precluded

- In evaluations, it is assumed that the packet size is based on L2/L3 SDU in the evaluation

  - FFS header overhead

- FFS whether to describe the following assumptions:

  - Duplex mode: FDD or TDD (DL/UL configuration)

  - Re-dropping or discarding UEs which do not satisfy certain channel quality if any

  - Blockage due to moving metal parts for channel model for factory automation

  - Other assumptions like TTI size, gNB/UE processing time, CSI measurement and reporting

- Companies are encouraged to provide evaluation on baseline performance achievable with Rel-15 NR URLLC, for the prioritized URLLC use cases identified in the Rel-16 URLLC SID.

- Take the simulation settings in the following table as the starting point for Rel-16 NR URLLC system level evaluation for urban macro for power distribution:

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m Note: Other value (e.g. 150 m) is not precluded |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | FFS antenna ports (e.g., 4 Tx/4 Rx antenna, etc.) (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1); dH = 0.5λ, dV = 0.8λ; 102 degree for 500m ISD |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | FFS antenna ports (e.g., 2 Tx/4 Rx antenna ports, etc) Panel model 1: Mg=1, Ng=1, P=2, dH=0.5 |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi as starting point |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| BS receiver | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. |
| Parameters with the value not defined in 38.802 |  |
| Number of UEs per cell | Up to 10 |
| Simulation bandwidth | 40 MHz |
| SCS | 30 kHz Note: Other values for evaluation are not precluded. |
| UE distribution | 100% of users are outdoors |
| UE power control | Companies report the PC mechanisms used for URLLC. |
| HARQ/repetition | Companies report (including HARQ mechanisms). |
| Channel estimation | Realistic |

- Take the simulation settings in the following table as the starting point for Rel-16 NR URLLC system level evaluation for indoor hot-spot for factory automation:

| Parameters | Value |
| --- | --- |
| Inter-BS distance | 20m |
| Carrier frequency | 4 GHz |
| UE Tx power | 23dBm |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 5dB |
| BS antenna configurations | FFS antenna ports (e.g., 4 Tx/4 Rx antenna ports, etc) (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH = dV = 0.5 λ for 4GHz |
| BS antenna height | [3] m Note: Companies report the modification of the layout |
| UE antenna configuration | FFS antenna ports (e.g., 2 Tx/4 Rx antenna ports, etc.) Panel model 1: Mg = 1, Ng = 1, P = 2, dH = 0.5 |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) Note: Companies report the modification of the layout |
| UE antenna gain | 0dBi as starting point |
| BS Tx power | 24 dBm per 20 MHz |
| BS receiver | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. |
| UE receiver noise figure | 9 dB |
| Parameters with the value not defined directly for factory automation in 38.802 |  |
| SCS | 30 kHz Note: Other values for evaluation are not precluded. |
| Simulation bandwidth | 40 MHz |
| Layout | Single layer as defined in 38.802 Indoor floor: [(3, 6, 12) BSs per 120 m x 50 m] Note: Companies report the modification of the layout |
| Channel model | ITU InH for 4 GHz Companies report the modification of the channel model |
| Number of UEs per cell | Up to [40] |
| UE distribution | 100% of users are indoor: 3 km/h and/or 30 km/h UE-speed Note: which one to use is up to companies and other value(s) are not precluded |
| UE power control | Companies report the PC mechanisms used for URLLC. |
| HARQ/repetition | Companies report (including HARQ mechanisms). |
| Channel estimation | Realistic |

- Evaluation of 30 GHz carrier frequency is not precluded.

[94-NR-07] – Wu Yong (Huawei)

ITU: Email approval to compile and endorse the eMBB spectral efficiency results submitted by August 24, 2018, until August 29, 2018

Done: According to decision posted on Sept.6th, the evaluation results are endorsed in:

R1-1810014	Summary of IMT-2020 evaluation results of eMBB spectral efficiency	Huawei

[94-NR-08] – Wu Yong (Huawei)

ITU: Email approval to derive the LTE peak spectral efficiency and peak data rate results with the text proposals in R1-1809934 as the starting point, by August 29th , 2018

Done: According to decision posted on August 30th, the TP is endorsed as:

R1-1810012	Text proposal to TR37.910 for LTE peak data rate and peak spectral efficiency	Huawei

[94-NR-09] – Wu Yong (Huawei)

ITU: Email approval to derive the NR control plane latency evaluation results, based on the summary of CP latency evaluation parameters in R1-1809947

, by August 29th, 2018.

Done: According to decision posted on August 30th, the TP is endorsed as:

R1-1810013	Text proposal to TR37.910 for NR control plane latency	Huawei

# Annex G:	List of participants at RAN1 #94

Please see excel file attached to this report

# Annex H:	TSG RAN WG1 meetings in 2018 – 2019

| TITLE | TYPE | DATES | LOCATION | CTRY |
| --- | --- | --- | --- | --- |
| 3GPPRAN1#94bis | WG | 8 – 12 Oct 2018 | Chengdu | China |
| 3GPPRAN1#95 | WG | 12 – 16 Nov 2018 | Spokane | US |
| 3GPPRAN1-AH-1901 | AH | 21 - 25 Jan 2019 | Taipei | Taiwan |
| 3GPPRAN1#96 | WG | 25 Feb - 1 Mar 2019 | Athens | Greece |
| 3GPPRAN1#96bis | WG | 8 - 12 Apr 2019 | China | China |
| 3GPPRAN1#97 | WG | 13 - 17 May 2019 | US | US |
| 3GPPRAN1-AH-1906-TBC | AH | 24 - 28 Jun 2019 | TBD |  |
| 3GPPRAN1#98 | WG | 26 - 30 Aug 2019 | Prague | CZ |
| 3GPPRAN1#98bis | WG | 14 - 18 Oct 2019 | China | China |
| 3GPPRAN1#99 | WG | 18 - 22 Nov 2019 | US | US |

| MEETING TYPES |  |  |
| --- | --- | --- |
| AH = Ad Hoc | ST = Startup Meeting | RG = Rapporteurs Group |
| JM = Joint | WG = Working Group | SG = Steering Group |
| PM = Preparatory Meeting | CM = Chairmen's meeting | TG = Task Group |
| RM = Resolution Meeting | OR = Ordinary | XO = Extraordinary |

End of document